Improving Road Safety: Assessing Curve Geometry of Road Network in Rivers State University Main Campus, Port Harcourt, Nigeria

Abstract

The insertion of curves in road designs is of utmost importance for the safety of the driver and commuters. It provides a smooth transition when the roadway changes direction and also ensures that proper and safe sight distances are maintained for the speed which the road will accommodate. Road curves can be horizontal or vertical, each serving a crucial role in safe navigation. The study investigate the curve parameters of selected road network within the main campus of Rivers State University, Nkpolu Oroworukwo, Port Harcourt, Rivers State. The study utilizes the quantitative data collection approach which is based on ground survey data acquisition.  Field data were acquired with the use of the Total Station (Kolida KTS 440L) instrument. The Field data were reduced to obtain horizontal and vertical angles and further processing and map production were carried out using specialized surveying and mapping software. Accuracies of approximately 1:16,000 and 0.008 were achieved for traversing and levelling operations respectively and it is within the degree of allowable minimum accuracy of 1:5000 for traversing as stipulated by the Surveyors Council of Nigeria. Findings from the study revealed that the sections of the road network under study covers a total length and width of approximately 2.93km and 7.00m respectively. The parameters for 5 horizontal curves and 6 vertical curves were determined and will be useful to the University Estate and Works Department in ascertaining if the curves parameters noted were in line with the design prior to construction works.

Key words: road designs, Road Network, Curve Parameters, Surveying

- Introduction

Surveying is a measurement science and involves the determination of the measurement data, the reduction and interpretation of the data to usable form, and, conversely, the establishment of relative position and size according to given measurement requirements (Eze, Lawson and Pepple, 2024). The determination of the relative horizontal and vertical position of points on the earth surface is imperative, as these points serves as control and bench marks during construction activities and land boundaries definition (Eze, Hart and Eke, 2022).

Curve are the transitions which are provided at the intersection of two straight lines. Where the fieldwork is composed of straight lines, setting out is comparatively straight forward (Ezigbo, 2022). With roads, railways and pipelines, two straights will normally be connected by a curve where there is a change of direction (Calvi, 2014). The purpose of these curves is to deflect a vehicle travelling along of the straights safely and comfortably through a deflection angle to enable it to continue its journey along the other straight. While horizontal curves indicates a change in the direction of road, vertical curves refers to a change in the slope or elevation of a road (Ai and Tsai, 2015; Ghilani and Wolf, 2008).

The introduction of horizontal and vertical curves into road design is critical in ensuring the safety of vehicle, drivers and commuters (Ryan et al, 2022). It is an overall design control for horizontal alignments in roadway design that may equal or exceed the legal statutory speed limit. The level of service is directly related to the speed of operation. It should meet driver expectations and be consistent with the facility’s functional classification and location (Uren and Price, 2010).

Design speed selection is a critical decision that should be done at the beginning of the planning and design process. This speed should balance safety, mobility, and efficiency with potential environmental quality, economics, aesthetics, as well as social and political impacts (Malaghan, Pawar and Dia, 2020). Roadway design features (curve radii, super elevation, sight distance, etc.) are impacted by the design speed, as well as other characteristics not directly related to speed. Therefore, any changes to design speed may affect many roadway design elements (Gregory, 2015).

Hu, Li, Lui and Adanu posited that design speeds for rural roads should be as high as practicable to supply an optimal degree of driving safety and operational efficiency. Findings from Dhar and Hassan, (2019) indicates that drivers operate quite comfortably at speeds that are higher than typical design speeds. Lower design speeds may be appropriate for certain urban roadways such as residential streets, school zones (Tanyes et al, 2022). Traffic calming techniques have proven to be a viable option for residential traffic operations. Designers should evaluate high speed compatibility with safety (pedestrians, driveways, parking, etc.) for urban arterials (Qianqian, 2014; Ghilani and Wolf, 2008).

It is very necessary that appropriate safety measures be adopted in the introduction of horizontal and vertical curves to make the infrastructure road user friendly and decrease the risks of hazardous circumstances.

Notations of a Typical Circular Curve

Figure 1: A typical Circular curve (Ezigbo, 2022; Uren and Price, 2010).

As showed in figure 1, the analysis of the curve parameters for a typical curve relationships, the required equations as provided in Ezigbo, (2022) and Uren and Price, 2010) are as follow;

- Tangent lengths IT and IU: in triangle IUO

R tan(θ2) = IUIO = IUR

Hence IU = IT = R tan (θ2)							(1.1)

- External distance, PI: in triangle IUO

cos(θ2) = RIO

or IO = Rcos⁡(θ/2) =R sec(θ2)

But PI = OI – OP = OI – R

Hence PI = R sec(θ2) - R = R[secθ2-1]			(1.2)

- Mid-ordinate, PS: in triangle TSO

cos(θ2) = OSOT

or OS = OT cos(θ2) =R cos(θ2)

But PS = OP – OS = R – R cos(θ2) = R[1 – cos(θ2)]			(1.3)

- Long chord, TU: in triangle USO

sin(θ2) = USUO = USR

or US = R sin(θ2)

But TU = US + TS and US = TS

Hence TU = 2R sin(θ2)			(1.4)

The length of the circular curve, Lc, can be obtained from one of the two formulae depending on whether the curve is a radius curve or a degree curve:

For a curve of radius R:

Lc = Rθ metres			(1.5)

Where R is in metres and θ is in radians.

For a D̊ degree curve:

Lc = 100(θD) metres			(1.6)

Where θ and D are in the same units, either degrees or radians.

## Q = is any point on the circular curve

## S = is the Mid-point of the long cord

## P = is the Mid-point of the circular curve

## θ = deflection angle = external angle at I

## R = radius of curvature

## O = Centre of Curvature

## I = Intersection point

## T&U = Tangent point

## Intersection angle = (180-θ) = internal angle at I

## Long cord TU = It is straight line joining TU

## Tangent length = IT&IU (IT = IU)

## Tangential angle, ITQ = angle from the tangent at T (or U) to any point on the curve.

## Mid-ordinate = PS: It is the greatest offset from the long chord to the curve.

## Radius angle = angle TOU = deflection angle

## External distance = PI: Is the shortest distance from the intersection point to the curve.

## You and Easa illustrated the relation between roadway reverse curve and roadside spiraled curve for roadway circular curve 2 as showed in Figure 2. The study noted that a spiraled roadside curve can be determined for roadway circular curve 2 as follows;

## From figure 2, Roadway circular curve 2 has a radius of R2. Its curve centre C2, (XC2 and YC2).

## Where the coordinates of point XC2 and YC2 are given by;

## XC2 = XpI1 + T1cos (I1) + R2sin (I1)                                                          (1.7)

## YC2 = XpI1 + T1sin (I1) + R2cos (I1)                                                         (1.8)

## Station TC2 or CT2 has a tangent length, T1 from the point of intersection PI1, and a tangent length, T2, from the point of intersection PI2.

## But T2 = R2 tan (I1/2)                                                                                 (1.9)

## Where; I2 is the angle of intersecting tangents at PI2 for roadway circular curve 2.

## The coordinates of PI2, (XP2 and YP2) are given by;

## XP2 = XpI1 + (T1 + T2) cos (I1)                                                          (1.91)

## YP2 = XpI1 + (T1 + T2) sin (I1)                                                         (1.92)

## Figure 2: Relation between roadway reverse curve LC>S and roadside spiraled curve, roadway circular curve 2 (You and Easa, 2018).

The study investigated the parameters and extent of curves under study. The as-built metrology of the curves in Rivers State University will provide the adequate safety measures that can be adopted for smooth transition when the road changes direction.

The significance of this study is such that provide information about the status and parameters of the curves under study, enhance safe transportation and efficient traffic control of vehicle moving along the study area. The findings of this study will also be useful to the university Estate and Works Department, as well as the Physical Planning Department, especially in future planning, construction and management of engineering projects in the Campus.

Study Area Description

The study area is situated within Rivers State University Main Campus, Nkpolu-Oroworukwu, Port Harcourt, in Port Harcourt City Local Government Area. It is geographically located is between latitude 04̊  47’ 30’’ to latitude 04˚  48’  20’’ and longitude 06˚  58'  35”  to longitude 06˚  59’  05”. It is bounded at the west by Mgbouosimini community, at the south by Eagle Island, at the east by mile 3 diobu community and at the North by Nkpolu-Oroworukwu community (Pepple, Eze and Zagha, 2024).

As showed in figure 2, the study area covers approximately 2.88km and stretches from the main gate and passes through parts of the University circular road and connects to Road A, B, and G extensions respectively.

Figure 3: Map of Rivers State University Main Campus and Environs. (Othophoto Map of Rivers State 2021.

- Materials and Methods

Table 1: Equipment and Hardware Selection

| S/N | Description of Items | Instrument Type | No. of Item |
| --- | --- | --- | --- |
| 1 | Total station and its accessories | (Kolida KTS 440L) | 1 |
| 2 | Tripod stand | Leica TRI100 | 1 |
| 3 | Prisms and tracking rod | GDST OEM 6771112 | 2, 2 |
| 4 | 100m Measuring Tape | PANYI Steel Tape PJ9 | 1 |
| 5 | Nails and Harmer |  |  |
| 6 | Automatic Level | Leica (NA24) | 1 |
| 7 | Field Book and stationaries |  |  |
| 8 | Laptop Computer | Dell Latitude E6540 | 1 |

Table 2: Software Selections

| S/N | Description of Items | Version |
| --- | --- | --- |
| 1 | AutoCAD | 2007 |
| 2 | Kolida data downloading Software | 2022 |
| 3 | AutoCAD 3D | 2019 |
| 4 | Microsoft office | 2007 |

Secondary Data Sources

Secondary data used for field data acquisition consists Northings and Easting coordinates of identified control points in the study area. As contained in table 3, the coordinates were projected in Universal Transverse Mercator (UTM, Zone 32N). The secondary data was collected from the Department of Surveying and Geomatics, Rivers state university, River state.

Table 3: Spatial Data of Control Points

| Station ID | Northings (m) | Easting (m) | Reduced level (m) |
| --- | --- | --- | --- |
| SVG GPS 002 | 530903.200 | 275962.761 | 11.986 |
| SVG WGPS 003 | 530933.315 | 275992.841 | 8.876 |
| SVG WGPS 004 | 530938.002 | 275855.250 |  |
| SVG 005/2018/01 | 5309428.721 | 276428.721 | 11.986 |
| SVG 005/2018/04 | 530099.624 | 275958.695 |  |
| SVG 005/2018/05 | 530103.649 | 275648.306 |  |

Primary Data Acquisition

This constitute the methods adopted for field data acquisition, the stepwise approach of the research methods are as follows:

Test of Survey Instrument

The reliability of the Kolida KTS440L total station instrument was tested to see if the instrument was in good working condition and also if it could give the required accuracy. The result of the instrument test is shown in table 4:

Table 4: Collimation Test

| Instrument Face | Horizontal Circle Reading | Vertical Circle Reading |
| --- | --- | --- |
| Face left | 332 ̊ 17’ 54’’ | 88 ̊ 14’ 30.3’’ |
| Face right | 172 ̊ 17’ 57’’ | 271 ̊ 44’ 57’’ |
|  | Difference = 00 ̊ 00’ 03’’ | 00 ̊ 00’ 30’’ |

The results in table 4 indicate that the instrument was in good condition and could be used to obtain the required primary data for the study.

Control (In-situ) Check

The control used as a referenced points for primary data acquisition were checked to ascertain if they in-situ and its reliability to reference field observations. The coordinates of control points used were referenced to the Universal Transverse Mercator (UTM) Zone 32N. The total station was set on SVG 002 with all necessary temporary adjustments appropriately carried out. As showed in figure 4, a back sight was taken by observing the reflector vertically placed on the back control station SVG WGPS 004 with the horizontal circle readings properly read and recorded. Thereafter, the total station was turned toward observing reflector vertically placed on the fore control station SVG WGPS 003 with the vertical circle reading properly read and recorded respectively.

Figure 4: Sketch layout of the control used

Table 5: Angular method for Control Check.

| Station | Sight | Face | HCR | Horizontal angle | Mean | Vertical angle readings |
| --- | --- | --- | --- | --- | --- | --- |
| SVG 002 | SVG WGPS 004 | FL1 | 00 ̊ 00’00’’ |  |  | 89 ̊ 53’ 58’’ |
|  | SVG WGPS 003 | FL2 | 117 ̊ 02’ 00’’ | 117 ̊ 02’ 00’’ |  |  |
|  | SVG WGPS 003 | FR2 | 297 ̊ 02’ 02’’ |  | 117 ̊ 01’ 59.5’’ |  |
|  | SVG WGPS 004 | FR1 | 180 ̊ 00’ 03’’ | 117 ̊ 01’ 59’’ |  | 270 ̊ 05’ 54’’ |

Horizontal angle (Ha) of FL = FL2 - FL1                                                      (2.1)

Horizontal angle (Ha) of FR = FR2 - FR1                                                     (2.2)

Mean angle (Ma) = FL2 - FLI + FR2 - FR1 / 2                                             (2.3)

Where; HCR = Horizontal Circle Readings,

FL = Face Left and FR = Face Right

Reconnaissance Survey

A holistic view of the road network with curves under study was done. Instrument Stations Station Selected where based on inter-visibility and free from obstacle and likely obstruction from human and vehicular movements. A recce diagram shows all details of features in the study area was produced and assisted greatly in primary data acquisition.

Primary sources

Traversing

The traverse was done in three loops, this includes Loop 1, Loop 2 and Loop 3 respectively. Traversing was carried out based on the basic principles working from whole to part Loop 1 started at SVG 002, 003, 004 near the faculty of environmental sciences open field which run through to main gate and back to faculty of environmental sciences SVG002. The result of loop 1 is shown in table 6.

Leveling

Profile levelling was adopted with interval of at twenty-five meters (25m) using a Leica Automatic Level (NA24) model.

The height of collimation method of levelling data reduction was used for reduction of points observed in the field. Consider the model;

HI = RL of BM + BS of BM                                                                       (2.4)

RL of observed Points = HI – IS or HI – FS                                               (2.5)

Whereas;

HI = Height of Collimation, RL = Reduced Level, BS = Back Sight reading, IS = Intermediate Sight readings and FS = Fore Sight reading

The following mathematical models were used to for traverse computations:

N = Lcosθ                                                                                                    (2.6)

E = Lsinθ                                                                                                     (2.7)

(∆E)2+(∆N)2

The bearing and distance can be gotten from the following mathematical models;

tan-1 ( ∆E∆N)

L =				                                                   (2.8)

Bearing =						                            (2.9)

Where; N = Difference in Northings, E = Difference in Easting, L = Horizontal distance between two established points with known values, and θ = bearing of a line between two known points.

- Results

The results of the study are presented in tables and map and are contained in tables 6 and 7, and figure 4 and 5.

| Station From | Bearings | Distances (m) | N (m) | E (m) | Northings(m) | Eastings(m) | Station To |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 530903.200 | 275962.761 | WGPS002 |
|  | 44˚ 58’ 0’’ | 42.564 | 30.115 | 30.08 | 530933.315 | 275992.841 | WGPS003 |
| GPS 003 | 67˚ 20’ 12’’ | 42.276 | 16.289 | 39.011 | 530949.637 | 276031.841 | Peg 1 |
| Peg 1 | 89˚ 59’ 26’’ | 130.508 | 0.021 | 130.507 | 530949.658 | 276162.358 | Peg 2 |
| Peg 2 | 89˚ 47’ 21’’ | 193.003 | 0.710 | 193.001 | 530950.368 | 276355.357 | Peg 3 |
| Peg 3 | 96˚ 56’ 30’’ | 73.857 | -8.926 | 73.315 | 530941.459 | 276428.672 | SVG 005 |
| SVG 005 | 274˚ 50’ 5’’ | 41.581 | 3.504 | -41.433 | 530944.969 | 276387.238 | Peg 4 |
| Peg 4 | 268˚ 57’19’’ | 134.699 | -2.455 | -134.67 | 530942.518 | 276252.561 | Peg 5 |
| Peg 5 | 269˚ 33’16’’ | 158.172 | -1.229 | -158.16 | 530941.221 | 276094.393 | Peg 6 |
| Peg 6 | 250˚ 15’33’’ | 94.777 | -32.01 | -89.206 | 530909.278 | 276005.186 | Peg 7 |
| Peg 7 | 261˚ 44’42’’ | 42.878 | -6.156 | -42.433 | 530903.135 | 275962.269 | SVG 002 |
| SVG 002 | 44˚ 58’ 00’’ | 42.568 | 30.117 | 30.089 | 530933.315 | 275992.841 | WGPS 003 |

Table 6: Specimen of Traverse Computation

Table 7: Coordinates and reduced level of some Points for Loop 1

| Station ID | Northings(m) | Eastings(m) | Reduced levels(m) |
| --- | --- | --- | --- |
| SVG002 | 530903.200 | 275962.761 | 11.986 |
| WGPS003 | 530933.315 | 275992.841 | 8.876 |
| PEG1 | 530940.404 | 276024.482 | 9.502 |
| PEG2 | 530955.036 | 276253.162 | 10.497 |
| Station ID | Northings(m) | Eastings(m) | Reduced levels(m) |
| SVG005/001 | 531094.098 | 276428.721 | 11.986 |
| PEG3 | 5301008.437 | 276484.455 | 11.624 |
| PEG4 | 531213.551 | 276640.299 | 11.599 |
| PEG5 | 531146.682 | 276559.450 | 11.610 |
| PEG6 | 530988.125 | 276442.656 | 11.625 |
| PEG7 | 530955.015 | 276417.448 | 11.902 |
| PEG8 | 530949.205 | 276282.391 | 10.925 |
| PEG9 | 530949.637 | 276031.851 | 11.982 |
| PEG10 | 530947.925 | 275839.998 | 11.626 |

Table 8: Coordinates and reduced level of some Points for Loop 2

| Station ID | Northings(m) | Eastings(m) | Reduced levels(m) |
| --- | --- | --- | --- |
| SVG002 | 530903.200 | 275962.761 | 11.986 |
| WGPS003 | 530933.315 | 275992.841 | 8.876 |
| PEG1 | 530947.925 | 275839.998 | 11.629 |
| PEG2 | 530940.520 | 27644.032 | 9.143 |
| PEG3 | 530742.921 | 275646.502 | 7.645 |
| PEG4 | 530543.043 | 275649.074 | 6.910 |
| PEG5 | 530543.043 | 275651.624 | 8.064 |
| PEG6 | 530343.136 | 2756653.995 | 10.133 |
| SVG005/05 | 530103.649 | 275648.306 | 10.123 |

Table 9: Coordinates and reduced level of some Points for Loop 3

| Station ID | Northings(m) | Eastings(m) | Reduced levels(m) |
| --- | --- | --- | --- |
| SVG005/05 | 530103.649 | 275648.306 | 10.123 |
| PEG1 | 530018.121 | 275690.996 | 8.633 |
| PEG2 | 529877.612 | 275833.089 | 8.174 |
| PEG3 | 529762.836 | 275947.054 | 6.146 |
| PEG4 | 529685.669 | 275846.001 | 6.148 |
| PEG5 | 529834.776 | 275907.421 | 6.724 |
| PEG6 | 529834.216 | 276032.009 | 4.580 |
| PEG7 | 529958.490 | 276072.037 | 5.077 |
| PEG8 | 530065.908 | 275970.748 | 6.565 |
| SVG05/04 | 530099.621 | 275958.691 | 7.191 |

Figure 5: Specimen of Horizontal Curve (A) Identified in the Study Area

Curve (A) Parameters

The parameters of curve A is shown below;

Chainage distance 0+375 - 0+750

Begin: 530945.287mN                            276367.485mE

Radial Point: 531045.286mN                  276367.322mE

End: 530984.745mN                               276446.913mE

PI: 530945.367mN                                  276416.961mE

Included angle = 52-38-53,         Radius = 100.000m

Tangent length = 49.475m,         Arc length = 91.888m

Chord length = 88.689m,            External secant = 11.570m

Mid ordinate = 10.370m,           Degree of curve = 57-17-45

Curve B is located within road B.

Figure 6: Specimen of Horizontal and Vertical Curve B determined in the Study Area

Curve (B) Parameters

The parameters of curve B is shown below

Chainage distance 1+1875-2+250

Begin: 530081.124mN                               275668.873mE

Radial Point: 530076.127mN                     275743.706mE

End: 530023.002mN                                  275690.765mE

PI: 530047.084mN                                     275666.600mE

Included angle = 48-55-12,                    Radius = 75.000m

Tangent length = 34.116m,                   Arc length = 64.036m

Chord length = 62.109m, 	               External secant = 7.395m

Mid ordinate = 6.731m, 	               Degree of curve = 76-23-40

Discussion

The Field data were reduced to obtain horizontal and vertical angles and further processing and map production were carried out using Auto-CAD 2007 and Auto-CAD 3D Software. An accuracy of approximately 1:16,000 and 0.008 were achieved for traversing and levelling operations respectively, which was within the allowable degree of accuracy. The findings revealed that the sections of the road network under study covers a total length and width of approximately 2.93km and 7.00m respectively. The geometry for 5 horizontal curves and 6 vertical curves were determined and will be useful to the University Estate and Works Department in ascertaining if the curves parameters noted were in line with the design prior to construction works.

Table 6 is a specimen of traverse information computed using the first principle of field data manipulation. The table consists of final corrected coordinates of some points traversed, traverse point identities, distances and directions. Tables 7, 8, and 9 contains summary of information about traverse loop 1, 2 and 3 respectively. The information revealed that the highest elevation was11.986m and the lowest elevation was 4.580m. Figures 5 and 6 is are specimens of some horizontal and vertical curves identified with parameters determined.

Curve (A) stretches from chainage distance of 0+375 to 0+750 with projected coordinates of 530945.287m.N, 276367.485m.E to 530984.745m.N, 276446.913m.E. The radial point and point of intersection was defined by 531045.286mN, 276367.322mE and 530945.367m.N                           , 276416.961m.E respectively with included angle of 52o 38’ 53” and radius of 100.000m. Other parameters of curve (A) were also determined as follows; the tangent length was 34.116m, arc length was 64.036m, chord length was 62.109m, external secant was 7.395, Mid-ordinate was 6.731m, 	 and Degree of curve 76o 23’ 40” respectively.

Wheras curve (B) stretches from chainage distance of 1+1875 to 2+250 with projected coordinates of 530082.124m.N, 275743.706m.E to 530047.081m.N, 275666.600m.E. The radial point and point of intersection was defined by 530076.127mN, 275743.706mE and 530945.367m.N, 276416.961m.E respectively, with included angle of 48o 55’ 12” and radius of 75.000m. Other parameters of curve (B) were also determined as follows; the tangent length was 34.116m, arc length was 64.036m, chord length was 62.109m, external secant was 7.395, Mid-ordinate was 6.731m, 	 and Degree of curve 76o 23’ 40” respectively.

- Conclusion

The traversing and levelling techniques used yielded the required results that addresses the aim and objectives of the study. The expected degree of accuracy for every stage of the project work was achieved. The desire to define the alignment of the road, determination of existing curves and its parameters of selected road network within-the main campus of Rivers State University was achieved.

- Recommendation

The following recommendations are highlighted;

- The curve parameters determined in this study should be used to validate the curve as-designed prior to road construction and check for possible variation.

- There is need to improve the drainage system and regular maintenance to forestall incidence of flood that could damage sections of the road network with low elevation.

Acknowledgement

We sincerely acknowledge the Department of Surveying and Geomatics, Rivers State University for providing us with necessary secondary data which consists of the coordinate of established monuments use as reference points for field data acquisition.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

REFRERENCES

Ai, C. and Tsai, Y. (2015). Automatic Horizontal Curve Identification and Mearsurement using GPS Data. Journal of Transportation Engineering, 141(2), http://doi.org/10.1061/(asce)te.1943.5436.0000740

Calvi, A, (2014). A Study on Driving Performance along Horizontal Curves of Rural Roads. Journal of Transportation Safety and Security, 7(8):243-267. http://doi.org/10.1080/19439962.2024.952468

Dhahir, B. and Hassan, Y. (2019). Using Horizontal Curve Speed Reduction Extracted from the Naturalistic Drivind Study to Predict Curve Collision Frequency. Accid and Prev, 123(2019):190-199

Eze, P. I., Lawson, N. and Pepple G. T. (2024). Investigation of Drainage Inverts levels for optimal Functionality in a Section of Onne Multipurpose Terminal: Rivers State, Nigeria. IRE 1705508 Iconic Research and Engineering Journals. IRE Journals, 7(8):219-225. ISSN: 2456-8880 online. https://www.irejournals.com/paper-details/1705508

Eze P. I; Hart, L, and Eke, S. N. (2022). Assessment of Relative Accuracy of Selected Total Station Instrument for a Closed Traverse Survey. Journal of Communications Technology and Electronics, 8(5), 72-78. https://www.questjournals.org/jecer/papers/vol8-issue5/L08057278.pdf

Ezeigbo, C. U. (2022). Fundamental of Surveying and Geomatics, with Special Application to Special Surveys. El’Demak publishers, 76 Robinson Street, Uwani, Enugu State, Nigeria. ISBN: 978-978-8436-26-3

Ghilani, C. D. and Wolf, P. R. (2008). Elementary Surveying and Introduction, An Introduction to Geomatics (13th Ed.), New York; Pearson Publisher, 2012. Pp 468-469, 470-471, 504-505.

Gregory J. T. (2015). Roadway Horizontal Alignment. Continuing Education and Development, Inc. 22 stonewall court, woodcliff lake, NJ07677.

Hart and Eze, (2024). Imperative of a 4-Dimensional Spatial Data for Land Development Planning, Design and Construction in Nigeria. Conference paper, Nigeria Institution of Surveyors (NIS) Annual General Meeting, Edo State, Nigeria. https://www.researchgate.net/publication/382557874_Imperative_of_a_4-Dimensional_Spatial_Data_for_Land_Development_Planning_Design_and_Construction_in_Nigeria

Hu, Q., Li, X., Lui, J. and Adanu. E. N. (2021). A low-cost approach to identify hazard curvature for Local Road Networks using Open Source Data. Journal of Transportation Research, Interdisciplinary perspective, 10(1), june 2021. http://doi.org/10.1016.2021.100393

Kulkaarni, P. P. and Mandal, J. N. (2022). Strength Evaluation of Soil Stabilized with Nano Silica-Cement mixes as Road Construction Materials. Journal of Construction and Building Materials, 314:125363. http://doi.org/10.1016/j.conbuildmat.2021.125363

Malaghan, V., Pawar, D. S. and Dia, H. (2020). Modelling Operating Speed using Continous Speed Profiles in Two-lane Rural Highway in India. Journal of Transportation Engineering, part A, Systems, 146(11). http://doi.org/10.1061/JTEPBS.0402024

Musa, J.I., Otachi, M. Y., Saliu, I. I., Muhammed, I. M., Kuti, I. A., Ezekiel, L. P. and Joseph, M. S. (2019). Assessment of Urban Drainage Systems in Nigeria: A Review. Department of Agricutural and Bioresources Engineering, Federal University of Technology, Minna, Niger State.

Pepple G. T., Eze, P. I. and Zagha B. F. (2024). Facility Mapping of Street Lighting in Rivers State University Main Campus, Port Harcourt: Nigeria. International Journal of Scientific Research and Engineering Development, 7(1):496-50. https://ijsred.com/volume7/issue1/IJSRED-V7I1P58.pdf

Qianqian, Z. (2014). A Review of Sustainable Urban Drainage Systems Consedering the Climate Change and Urbanization Impacts. Water, 2014: (6):976-992.

Ryan, A., Hennessy, E. A., Kwon, W., Fitzpatrick, C. and Michael, A. K. (2022). Driver Performance at Horizontal Curves: Bridging Critical Research Gaps to increase Safety. Traffic Safety Research, an Interdisciplinary Journal, 3.00014. http://doi.org/10.55329/imji8901

Schofield, W. (2001). Engineering Surveying. Butterworth-Heinemann. ISBN 978-0- 7506-	4987-2.

Tanyes, H., Gorum, T., Kirschbaum, D. and Lombarde, L. (2022). Could Road Construction be mor Hazardous than an Earthquake in terms of Mass Movement. Journal of Natural Hazard, 2(5); http://doi.org/10.1007/s11069-021-05199-2

Wakjira, T. D. (2018). A Review of Sustainability of Urban Drainage Traits and Consequences. Journal of Sedimentary Environment 3(3):131-137. http://doi.org/10:12957/jse.2018.3782

Uren, J. and Price, W. F. (2010), “Surveying for Engineers” Fifth Edition Published by Palgrave Macmillan.

You C. Q. and Easa, S. M. (2018). Innovative Roadside Design Curve of Lateral Clearance Roadway Reverse Horizontal Curves. Journal of Transportation Engineering, 5(4):268-283. http://doi.org/10.1061/j.jtte.2017.09.004