Optimizing Energy Recovery in Hybrid Electric Vehicles Using A-ECMS Strategy

.

ABSTRACT

The fuel efficiency of plug-in hybrid electric vehicles (PHEVs) is largely influenced by their energy management strategies. This paper presents, for the first time, a dual static adaptive A-ECMS (Adaptive Equivalent Consumption Minimization Strategy) energy management controller designed to address energy management challenges in dual-motor PHEVs. The robustness of this controller was also evaluated under perturbations. In the experiments, the performance of the A-ECMS strategy was compared against the ECMS strategy and the RB (Rule-Based) strategy, the latter serving as an ideal optimization benchmark. Additionally, we analyzed the equivalent factors of A-ECMS and examined the impact of initial equivalent factors on the experimental outcomes. The results demonstrated that, compared to the ECMS strategy, the A-ECMS approach enhanced fuel economy by 7.8% and reduced CO2 emissions by 15.14 g/km. To further assess the robustness of this method, 5% random noise was introduced into typical operating conditions. Even under these conditions, the A-ECMS strategy still achieved a 6.9% improvement in fuel economy and a 14.39% reduction in CO2 emissions compared to the ECMS strategy. From an economic perspective, the proposed A-ECMS strategy led to cost savings of 30.17% and 28.56% in two different scenarios relative to the ECMS strategy. These findings highlight the effectiveness of the A-ECMS strategy in enhancing fuel efficiency, reducing CO2 emissions, and lowering operational costs for PHEVs. This study provides valuable insights for future research in energy management strategies for hybrid electric vehicles.

Keywords: Electronically controlled transmission, Carbon reduction, Plug-in hybrid vehicle, Dual-motor hybrid vehicle

1. INTRODUCTION

The widespread reliance on fossil fuels remains a primary cause of both environmental pollution and energy depletion [1, 2]. Growing concerns about energy sustainability and ecological preservation have driven researchers and engineers to explore more efficient and environmentally friendly alternatives in the automotive sector.

Over the past decade, vehicle electrification has emerged as a key technological advancement to mitigate these challenges. Various types of electrified vehicles, including hybrid electric vehicles (HEVs) [3], battery electric vehicles (BEVs) [4], and plug-in hybrid electric vehicles (PHEVs) [5], have been developed to improve energy efficiency and reduce emissions. Among them, PHEVs, which integrate dual power sources, provide greater flexibility in power distribution based on driving conditions, making them a promising solution in modern transportation.

The design of powertrain systems allows for different combinations of energy storage systems (ESS), such as battery-engine hybrids [6] or fuel cell-battery configurations [7]. Depending on the energy sources used, hybrid vehicles can be further categorized into petrol-electric hybrids, hydraulic hybrids, and electro-hydraulic hybrids. These systems offer advantages such as high energy conversion efficiency, rapid power delivery, and effective short-term energy storage, meeting the dynamic demands of vehicle operation [8]. Specifically, electro-hydraulic hybrid vehicles, which incorporate both batteries and hydraulic accumulators, benefit from the high energy density of batteries and the superior power density of hydraulic accumulators. This combination enhances adaptability to challenging driving conditions involving frequent starts and shifts [9].

One of the critical challenges in hybrid vehicle design is maximizing the efficiency of energy utilization while managing the complexity of the system. Achieving optimal energy distribution across different ESS requires considering multiple constraints, including battery state of charge (SOC), health status, internal combustion engine (ICE) and generator operating limits, and powertrain dynamic characteristics [10–13]. Another major difficulty in energy management is the uncertainty of real-time driving conditions, which limits the ability of vehicle control systems to make precise energy allocation decisions [14].

At the control level, various algorithms are employed to determine vehicle power demands and distribute energy efficiently among available sources [15]. While these strategies strive to balance emission reduction and economic efficiency, they often involve significant computational complexity, making real-time implementation on commercial vehicles challenging and costly [16]. Global optimal control methods, based on optimization theory, serve as benchmarks for evaluating other strategies and provide a theoretical foundation for control system design [17]. Although neural networks can approximate optimal solutions with reduced computational demand, they require extensive training datasets for effective deployment [18]. The effectiveness of hybrid vehicle performance is closely tied to energy management strategies, as achieving optimal fuel efficiency and emission reduction remains a crucial challenge not addressed by conventional vehicles [19].

Energy management strategies for hybrid vehicles are typically classified into rule-based methods, instantaneous optimization approaches, and global optimization techniques [20]. These strategies aim to minimize energy losses, reduce equivalent fuel consumption, or optimize a combination of fuel economy and emissions [21]. However, instantaneous optimization may lead to suboptimal solutions due to local optimization pitfalls, while global optimization, although theoretically superior, is computationally intensive and unsuitable for real-time vehicle applications [22].

Recent studies have explored advanced control strategies for hybrid vehicles. For example, in [23], an online corrective predictive energy management system (EMS) was proposed for series hybrid electric tracked vehicles (HETVs). Deep Reinforcement Learning (DRL), which integrates reinforcement learning (RL) and neural networks (NN), has gained popularity in energy management research. In [24], a Deep Q-Learning (DQL) algorithm was applied to series HETV power management, demonstrating a 5.96% fuel consumption reduction compared to traditional RL approaches. Meanwhile, [25] found that the Dyna-H algorithm achieved faster convergence than the Dyna method, although incomplete state-action training in Dyna-H could impact adaptability.

Different learning approaches, including supervised learning, unsupervised learning, RL, deep learning, and DRL, have been applied to hybrid vehicle control. In [26], RL-based EMSs were evaluated against dynamic programming (DP) algorithms, verifying their effectiveness and adaptability. Reference [27] utilized Markov chains to derive transition probabilities for RL-based EMSs and compared their performance with DP methods. In [28], fuzzy encoding predictors were introduced to forecast power demands, while [29] leveraged DRL and Actor-Critic DRL techniques to develop data-driven EMS models tailored to diverse driving conditions. Similarly, [30] proposed a DRL-based power allocation strategy using a stochastic driver model, demonstrating its potential in enhancing system intelligence and adaptability.

In real-world applications, differences in battery SOC inevitably affect performance and longevity. In [31], a DRL-based approach was implemented to optimize SOC balancing across battery packs, extending battery lifespan and minimizing maintenance requirements. Reference [32] combined RL and NN techniques for power distribution in electric vehicles, showing notable efficiency improvements. In [33], DQL was employed for hybrid electric bus power management, achieving faster convergence and better adaptability compared to Q-learning.

Hybrid Energy Storage Systems (HESS), which integrate supercapacitors with batteries, have gained significant attention for their role in extending battery lifespan and improving overall system performance [34]. Supercapacitors, known for their high power density and long cycle life, serve as effective auxiliary storage components. Their power density surpasses that of conventional batteries by a factor of two to three [35], and they exhibit superior thermal stability, operating within a temperature range of -40°C to 70°C. Researchers have increasingly explored the integration of supercapacitors with batteries in HESS configurations to meet both high-energy and high-power demands [36–39]. The complementary nature of these energy sources enhances efficiency and dynamic response. In [40], a novel real-time load power allocation method was proposed for HESS, along with a battery lifespan prediction model.

In addition to energy storage advancements, transmission system optimization plays a crucial role in improving hybrid vehicle efficiency. In [41], a torque control strategy was developed for a planetary gear dual-motor transmission system, incorporating a stiffness-dynamics model to mitigate speed fluctuations and reduce shifting impact. Reference [42] proposed a multi-speed transmission control algorithm to enhance vehicle efficiency and ensure smooth gear transitions. A clutchless automatic transmission system for electric vehicles was introduced in [43] and validated through experimental testing. Meanwhile, [44] investigated dual-control strategies for a 48V P3 hybrid AMT vehicle, refining electric mode switching and shifting processes to enhance driving experience. In [45], a single-motor dual-clutch hybrid powertrain was analyzed, integrating nonlinear clutch characteristics to improve shifting performance. Further studies in [46, 47] examined dual-clutch operation during shifting, while [48] established a three-degree-of-freedom transmission dynamics model to analyze transient upshift behavior.

To further optimize plug-in hybrid electric vehicle performance, this study proposes an energy recovery strategy for dual-motor PHEVs based on electro-mechanical automatic transmission. Experimental validation confirms its feasibility, providing a foundation for future research in energy-efficient hybrid vehicle technologies.

2. Exploring dual static adaptive optimization strategy using PMP framework

2.1 Modeling stochastic operating conditions mathematically.

This study introduces random noise into the original sample operating conditions to simulate the stochastic factors that vehicles may encounter during driving. Subsequently, the sample operating conditions with added random noise are processed using smoothing functions to simulate the driving behavior of drivers. To ensure the authenticity of the driving conditions, specific modeling processes are implemented with maximum and minimum acceleration boundaries set. This ensures that the driving distance and average speed under random conditions remain consistent with the sample conditions. The specific modeling process is illustrated in Figure 1.

At this point, the driving conditions are represented as:

(1)

In the equation,  represents the velocity of the driving conditions after adding random noise,  denotes the velocity under the original driving sample conditions,  is a function of the number of elements in the computation vector, and  signifies the noise intensity, indicating the complexity of traffic conditions, measured in [dBW] units. A larger value of  indicates a greater influence of random factors on the velocity, indicating a more complex current traffic scenario.

Due to the intense oscillations in velocity caused by the added random noise, which is inconsistent with actual driving conditions, a local weighted scatterplot smoothing method is employed to smooth the velocity data. The smoothing principle can be described as follows:

(2)

In the equation, m represents the window width of the function, which determines the smoothness of the velocity curve. It is used to describe the driving style of the driver. A larger value of m leads to a smoother driving process.  denotes the weight function of the velocity scatter at time t, which can be expressed as:

(3)

The maximum acceleration capability of the powertrain system under the current torque is not only dependent on factors such as vehicle mass, slope resistance, and wheel rotational inertia but also on the current driving speed and road gradient. Therefore, the maximum acceleration of the vehicle at the current moment is a function of velocity and road gradient variations and can be expressed as:

(4)

In the equation,  represents the maximum torque that the powertrain system can provide at the current velocity, and can be expressed as:

(5)

In the equation,  and  respectively represent the maximum torque that the internal combustion engine (ICE) and the electric motor (EM) can provide at the current vehicle speed.

Figure 1 Illustrates the mathematical modeling process of random driving conditions

To maintain the authenticity of driving conditions, this study sets the maximum acceleration boundary under random conditions . According to the modeling process illustrated in Figure 1, four random operating conditions are generated, and the resulting random driving conditions are depicted in Figure 2, with their acceleration curves shown in Figure 3. In each random operating condition, the vehicle's acceleration exceeds the limit, indicating that they are all within the range of the power system's capabilities.

Figure 2 The four random driving conditions generated based on the sample operating conditions

Figure 3 The acceleration curves for the random driving conditions

2.2 Modeling the structure of plug-in hybrid electric vehicles

In modern society, plug-in hybrid electric vehicles (PHEVs) play a crucial role. Despite the powerful driving force for socio-economic development provided by automobiles, the significant energy consumption and severe air pollution they bring about have undeniable negative impacts on the living environment. PHEVs not only exhibit excellent fuel economy but also significantly reduce air pollution. The object of this study is a plug-in dual-motor hybrid electric vehicle based on electronically controlled mechanical automatic transmission (AMT). This novel hybrid vehicle consists of an engine, a battery pack, two drive motors, and an electronically controlled mechanical automatic transmission, as illustrated in the diagram.

In this powertrain system, there are also two pathways for power transmission. The first pathway involves the engine transmitting power through the clutch to Motor1, then to the AMT(Automated Manual Transmission), followed by Motor2, and finally to the differential, which transmits the power to the wheels, driving the vehicle forward. The second pathway starts from the battery, supplying power to Motor1, and then follows the same route as the first pathway.  Additionally, the Energy Management System (EMS) is connected to the Engine Control Unit(ECU), Motor Control unit(MCU), and Battery Control Unit(BCU), continuously monitoring system status and making real-time adjustments according to optimization strategies to ensure the vehicle operates in an optimal state.

To systematically analyze the optimization capability of the dual-static adaptive ECMS control strategy, identify its underlying causes of error, and provide a more accurate description of the energy state of the battery (state of charge, SOC), we introduce a concept called the equivalent factor . The equivalent factor plays a crucial role in the energy allocation exchange between the engine and the electric motor in the control strategy. By introducing the concept of the equivalent factor, we can equate the consumed electricity to a certain quantity of fuel. This approach not only simplifies calculations but also allows us to compute the equivalent fuel consumption rate, which is the sum of the actual fuel consumption and the fuel equivalent of the consumed electricity.

By introducing the equivalent factor, the energy exchange between the internal combustion engine and the electric motor is standardized, achieving consistency in system energy management. This method not only simplifies the previous energy management processes but also enhances the efficiency and performance of the system.

The equivalent factor  enables the correlation between the cost of electrical power and fuel consumption cost. The transient optimization energy cost function in ECMS can be described as:

(6)

The value of the equivalent factor  affects the maintenance of power and the effectiveness of control strategies. If the chosen equivalent factor is too large, the use of electrical energy will result in excessive costs, thereby limiting the potential of plug-in hybrid systems to some extent. Conversely, if the equivalent factor is chosen to be too small, the stored energy in the battery will be depleted prematurely and may not meet the control requirements. Therefore, there is a need to develop new methods to determine the optimal solution for the equivalent factor, and further investigate its impact on the State of Charge (SOC) of the battery in plug-in hybrid electric vehicles.

Using the PMP principle, the optimal solution can be derived. For the optimization problem that only considers energy consumption, the state equation of the system can be rewritten according to the definition of the battery circuit model:

(7)

In this equation,  represents the amount of energy that can be stored in the battery, and its units .

The Hamiltonian function can be reformulated as:

(8)

Combining equations (6) and (8), we can obtain:

(9)

Equation (9) represents the connection between ECMS and PMP. Although ECMS is an engineering-based heuristic algorithm, its optimization theory can be explained using the PMP theory. In the PMP strategy, the Hamiltonian function in equation (9) is solved by hitting the target method with covariance t, adjusting the covariance value at each moment as the operating conditions change to obtain the optimal solution. Similarly, the optimal equivalent factor in ECMS can be obtained through iterative search, but it also requires prior knowledge of driving condition information. How to find an approximation of the optimal equivalent factor under unknown operating conditions is a focus of ECMS research. Adaptive ECMS based on SOC feedback is a simple yet effective method that can be implemented using a PI (Proportional and Integral) controller:

(10)

In the equation,  represents the initial equivalent factor,   and  are the proportional and integral coefficients of the controller, respectively. Based on the driving distance of the driving task, the reference SOC of the PHEV after one charge can be planned as follows:

(11)

In the equation,  represents the total distance traveled between two charging tasks of the PHEV.  denotes the distance of a driving task, which is the distance traveled by the vehicle after charging.

2.3 Developing a two-state adaptive optimization strategy using PMP framework

After studying the adaptability of the PI controller, to explore a simple and effective multi-objective adaptive solution, we first directly incorporate the battery aging term into the ECMS optimization strategy. We embed the aging model into the optimization framework to obtain a reasonable weighting factor based on PMP. The performance of the single-state adaptive strategy in the multi-objective adaptive optimization process was tested in simulation scenarios composed of random conditions 1 and random conditions 4. The instantaneous cost function can be represented as:

(12)

To track the battery capacity degradation trajectory of the ideal compromise solution, the state variable  is introduced. According to the PMP principle, the Hamiltonian function can be transformed into:

(13)

In the equation, , , and  is the covariance vector corresponding to the state variable . Then, the Hamiltonian function can be transformed as follows:

(14)

The instantaneous optimization function with dual adaptive factors can be represented as:

(15)

Combining equations (14) and (15), we obtain:

(16)

For a multi-input multi-output (MIMO) system, decentralized PID control is one of the most commonly used control schemes. In this paper, dual PI controllers are employed for DA-ECMS with dual-state adaptive control, and their adaptive rules are designed as follows:

(17)

In the equation,  and  represent the scaling factors of the two PI controllers,  and  represent the integral factors,  and  represent the reference values of the effective throughput.  and  are the initial equivalent factors and can be expressed as:

(18)

In the equation,  and  are the initial values of the optimal covariance, and it is noteworthy that only one state variable, battery SOC, is involved in the PMP offline optimization process. Therefore, the ideal compromise solution obtained according to PMP.  is the desired  terminal value at the end of the driving task, and the reference  trajectory can be planned as:

(19)

In the equation, represents the floor function, which returns the largest integer less than or equal to the specified value.  represents the actual distance traveled by the vehicle, and  represents the total distance traveled by the vehicle between charging tasks.

accumulates with the increase of effective throughput ah driving distance. If it is assumed that there is an approximate linear relationship between effective throughput ah and driving distance, the reference trajectory can be expressed as:

(20)

In the equation,  is the optimal effective throughput corresponding to the ideal compromise solution at the end of the driving task, and  is the effective throughput  generated as the SOC rises from  to  once it is charged.

3. study on influence of parameters in A-Ecms strategy

3.1 Discussion on the influence of initial equivalent factor on experiments

In this study, we first examined the optimization outcomes of Equivalent Consumption Minimization Strategy (ECMS) under different initial equivalence factors, excluding the influence of the PI controller, to assess their impact on battery SOC. Six initial equivalence factors were selected for the experiment: 2.1, 2.2, 2.3, 2.4, 2.5, and 2.6. The findings suggest that as the equivalence factor increases, the vehicle's power performance shows a slight enhancement. Specifically, when the equivalence factor rises from 2.1 to 2.6, the power performance improves from 4.46 to 4.94, reflecting a 9.717% increase. However, this improvement comes at the cost of reduced fuel economy, as indicated by a decline in MPGe. When the equivalence factor increases from 2.1 to 2.6, the MPGe decreases from 52.70 to 47.62, marking a reduction of 9.639%.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 4 Effects of different initial equivalence factors on SOC, performance, MPGe, and fuel price

Table 1

SOC, performance, MPGe and fuel price for different initial equivalence factors

|  | SOC | Performance | MPGe | Fuel price |
| --- | --- | --- | --- | --- |
| 2.1 | 15.7% | 4.46 | 52.70 | 0.3836 |
| 2.2 | 23.0% | 4.59 | 51.27 | 0.3779 |
| 2.3 | 28.4% | 4.68 | 50.27 | 0.3706 |
| 2.4 | 33.5% | 4.77 | 49.35 | 0.3664 |
| 2.5 | 38.9% | 4.86 | 48.38 | 0.3624 |
| 2.6 | 43.1% | 4.94 | 47.62 | 0.3555 |

The variation in the initial equivalence factor results in deviations in the dual-static adaptive ECMS control strategy. Therefore, selecting the appropriate initial equivalence factor is crucial for studying the optimized vehicle performance and fuel economy of dual-motor plug-in hybrid electric vehicles. In this study, we use the initial value of the optimal covariance trajectory as the basis for calculating the optimal initial equivalence factor：

(40)

3.2 Discussion on the impact of control factors of PI controller on experiments

To ensure convergence to the reference value, it is necessary to adjust the adaptive parameters, proportional factor  and integral factor , in Equation (10) appropriately. Therefore, it is necessary to study the influence mechanism of each adaptive parameter on the optimization results. To make the comparison more evident, the initial equivalence factor is set to . In the driving task composed of random condition 4, the influence of the proportional factor on SOC and the adaptive equivalence factor was studied with the integral factor of the PI controller set to zero. As shown in Figure 5, with the increase of the proportional factor, the rate of SOC decrease gradually decreases. The principle of SOC feedback-based adaptive ECMS execution is illustrated in Figure 5.

When studying the impact of the PI controller on the performance and fuel economy of plug-in hybrid electric vehicles in the dual-static adaptive ECMS control strategy, we separately treat the proportional factor and the integral factor as single variables to better observe their influence on the performance of the dual-static adaptive ECMS control strategy. In this study, two sets of experiments will be conducted: the first set involves keeping the integral gain at zero and varying the proportional gain ; the second set involves keeping the proportional gain at zero and adjusting the integral gain .

From Table 2, it can be observed that keeping the integral factor  constant, as the proportional factor  increases, the value of  gradually decreases. This implies that the vehicle can accomplish the driving task using less electrical energy. Furthermore, with an increase in the integral factor , the difference in SOC caused by the proportional factor  is 10.15%, while their combined effect on battery performance reaches 19.87%. When the integral factor  is 0.75, the variation in battery SOC is only 1.1% with  values of 0 and 0.75, while the impact on battery performance is 4.73%.

Figure 6 depicts a three-dimensional schematic diagram illustrating the impact of the control factors of the PI controller on SOC. Overall, from the graph, it can be observed that SOC is more sensitive to the integral factor . As the integral factor  increases, the control capability of the PI controller gradually strengthens, resulting in smaller variations in SOC. Along the Y-axis (), it can be noted that as the integral factor  decreases, the influence of the proportional factor  on the PI controller gradually increases.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 5 Influence of different PI scale factors on SOC

Figure 6 Influence of different PI control factors on

Table 2

Different  variations under various PI control factors

| PI control factors | =0 | =0.25 | =0.5 | =0.75 |
| --- | --- | --- | --- | --- |
| =0 | 51.08% | 48.79% | 44.46% | 40.93% |
| =0.25 | 32.72% | 30.69% | 29.42% | 27.45% |
| =0.5 | 25.54% | 30.69% | 24.75% | 24.52% |
| =0.75 | 23.27% | 22.94% | 22.50% | 22.17% |

To better investigate the impact of PI control factors on the dual-static adaptive ECMS strategy, it is important to select the sensitive region of the PI controller in the experiment. From previous experiments, we learned that the integral factor  has a greater influence on the PI controller, and when the integral factor, the battery SOC is most sensitive to changes in the proportional factor , with the fastest response. Therefore, our experiment is conducted with a fixed integral factor .

4. Comparison of experimental results for A-ECMS strategy, ECMS strategy, and RB strategy

In this study, to evaluate the effectiveness of the designed dual-static adaptive ECMS controller, it will be compared with two widely used energy strategies to investigate potential fuel economy and carbon emission reduction. The two strategies chosen for comparison are rule-based strategy and ECMS strategy.

parameters of the rule-based controller are typically obtained through optimization techniques. The battery's State of Charge (SOC) and load power determine the operating points of the engine and generator, as shown in Table 3.

Although the Rule-Based (RB) strategy boasts advantages such as clear logic and ease of implementation, its formulation relies on predefined rules and thresholds. Consequently, static rules cannot dynamically adjust to varying driving conditions and vehicle states, making it challenging to ensure globally optimal energy management. RB strategy typically only considers the current vehicle state and driving demands, without forecasting future driving conditions such as upcoming inclines or traffic situations, which limits its ability to optimize energy management.

Table 3

Rule-Based Control Strategy

| Judgment | State of the Battery | Power supply |
| --- | --- | --- |
|  | Charging |  |
|  | Not-working |  |
|  | Discharging |  |
|  | Not-working |  |

In the table,  represents the target power demand;  denotes the rated power of the engine;  indicates the maximum power that the engine can provide;  represents the efficiency of the engine when operating;  denotes the effective output power of the engine;  represents the rated power of the battery;  represents the total power that the engine and the battery can provide (ignoring the impact of vehicle electronic devices on the total power in the experiment);  and  represent the minimum and maximum charging states of the battery.

4.1 The ECMS strategy

The Equivalent Consumption Minimization Strategy (ECMS) is derived from the Pontryagin's Minimum Principle (PMP) and is a chronological method. Due to its fast computation speed and the absence of a requirement for the global power request of the drive configuration, ECMS is practically implementable. Generally, ECMS is able to identify a series of candidate solutions to the PHEV energy management problem by establishing a set of necessary conditions from PMP. These candidate solutions must not only satisfy these necessary conditions but also be subject to restrictions imposed by defined state constraints. Through the interaction of these two principles, the optimal control strategy can be found using a trial-and-error method.

However, in ECMS, the optimal control solution can only be achieved through perfect coordination of equivalent factors. Equivalent factors are used to scale between electrical cost and fuel cost in the Hamiltonian function defined by the minimum principle and cannot be predetermined a priori. Moreover, ECMS is highly stringent, and even slight deviations from the optimal equivalent factor values may result in unacceptable operations of the vehicle outside the SOC boundaries.

In this chapter, two different scenarios were established to simulate different usage environments, aiming to compare the experimental results of the A-ECMS strategy, ECMS strategy, and RB strategy.

4.1.1 Scenario 1: UDDS driving cycle

In this scenario, a typical UDDS (Urban Dynamometer Driving Schedule) driving cycle was selected to test the performance of the three strategies under the same conditions. Figure 7 displays the speed distribution of the typical driving cycle used in this study. The two images in Figure 8 respectively illustrate the engine input-output power and the battery power under the UDDS driving cycle.

Figure 7 UDDS driving cycle

| (a) Engine power demand | (b) Battery power demand |
| --- | --- |

Figure 8Vehicle power demand under UDDS

Under the typical UDDS driving cycle, traditional ECMS strategy, RB strategy, and the dual-static adaptive A-ECMS strategy designed in this paper were tested separately. For the RB strategy, the target SOC was set to 45%. Therefore, when the SOC of the vehicle battery reached around 45%, the RB strategy would initiate battery sustainment strategy. According to the experiment, it was observed that at the end of the test, the battery SOC value under the RB strategy was 42.5%.

Under the same conditions as depicted in Figure 9, the fuel economy, battery SOC, CO2 emissions, and vehicle performance achieved by the dual-static adaptive A-ECMS algorithm were compared with the ECMS strategy and rule-based (RB) algorithm.

As shown in Table 4, the improved dual-static adaptive A-ECMS strategy can reduce fuel consumption by 7.8% compared to the traditional ECMS strategy. Since the RB strategy is applicable only to known road conditions and is difficult to use in reality, it is generally used as a reference optimization target for other research strategies. Analyzing its behavior can provide possible improvements for controller design.

From the four images depicted in Figure 9, it can be observed that the dual-static adaptive A-ECMS strategy designed in this paper achieved excellent results in terms of battery SOC reduction, with a decrease of only 30.3%. The energy consumption of the ECMS strategy was 204.3% of that of the A-ECMS strategy, while the RB strategy consumed 156.8% of the energy consumed by the A-ECMS strategy.

In the experiment, we also tested the CO2 emissions under the three strategies. The emission under the ECMS-based strategy was 95.6 g/km, while under the improved dual-static adaptive A-ECMS strategy, it was reduced to 80.4 g/km. For the RB strategy, the emission was 57.9 g/km. Compared to the ECMS strategy, the A-ECMS strategy reduced CO2 emissions by 15.9%!

Through experimental testing, the improved dual-static adaptive A-ECMS strategy not only improved vehicle performance but also optimized the allocation of engine power and battery power during PHEV operation. This resulted in a 7.8% increase in fuel economy and a reduction of approximately 31.6% in energy consumption. Furthermore, it also lowered CO2 emissions, reducing them by 15.2 g per kilometer compared to the ECMS strategy.

Table 4

Fuel consumption, electricity consumption, CO2 emissions, and vehicle performance under Scenario 1.

| Control Strategy | Fuel Consumption(L/100Km) | Fuel Economy(%) | Electric Consumption() |
| --- | --- | --- | --- |
| ECMS | 4.98 | 100% | 61.9% |
| A-ECMS | 4.59 | 92.2% | 30.3% |
| RB | 3.71 | 74.5% | 47.5% |
| Control Strategy | Electric Economy(%) | CO2 emission(g/Km) | MPGe |
| ECMS | 204.3% | 95.6 | 45.4 |
| A-ECMS | 100% | 80.4 | 49.1 |
| RB | 156.8% | 57.9 | 62.1 |

(a)

(b)

(c)

(d)

Figure 9Results of ECMS, A-ECMS, and RB under Scenario 1

4.1.2 Scenario 2: Adding 5% random noise.

s

Figure 10 Scenario 2 utilizes a cyclic operation condition.

From Table 5, it can be observed that under Scenario 2, the fuel consumption of the improved dual-static adaptive A-ECMS strategy proposed in this paper is reduced by 6.9% compared to the ECMS strategy. From the four images shown in Figure 11, we can see that the final SOC values for the three strategies are as follows: for the ECMS strategy, SOC is 24%; for the A-ECMS strategy, it is 58.95%; and for the RB strategy, it is 39.68%. Comparing the SOC consumption based on the A-ECMS strategy as the baseline, the SOC consumption for the ECMS strategy is 212.56%, and for the RB strategy, it is 162.06%. The PHEV based on the A-ECMS strategy, while maintaining good vehicle performance during driving, also reduces CO2 emissions by 14.39 g/km compared to the ECMS strategy.

The initial conditions for the three strategies are consistent with those of Scenario 1. From Figure 11(b), it can be observed that the differences in battery SOC among the three strategies are not significant until around 250s. Under the RB strategy, the battery SOC reaches 45%, the set threshold, at 500s, after which it operates in charge-sustaining (CS) mode. Under the ECMS strategy, the battery SOC decreases rapidly, and by the end of the experiment, it has already dropped to around 24%. Compared to the former two strategies, the improved A-ECMS strategy proposed in this paper optimizes the power distribution of the battery, resulting in a slower decrease in SOC.

Figures 11(a), (c), and (d) depict the performance of the three strategies in terms of fuel consumption, CO2 emissions, and vehicle performance. Within the three test ranges, the performance of the A-ECMS strategy is slightly better than that of the ECMS strategy. However, it is evident from the three plots that the stability of the RB strategy is not as good as that of the A-ECMS strategy during long-term testing. Starting from around 500s, the PHEV based on the RB strategy shows a significant increase in fuel consumption and CO2 emissions, as well as a decline in vehicle performance. This is because after 500s, the vehicle based on the RB strategy transitions from an energy consumption mode to a charge-sustaining mode. In the charge-sustaining mode, the vehicle primarily relies on the engine as the main power source, while the battery acts as a power auxiliary, resulting in increased fuel consumption, higher CO2 emissions, and decreased vehicle performance.

Table 5

Results of the three strategies under Scenario 2

| Control Strategy | Fuel Consumption(L/100Km) | Fuel Economy(%) | Electric Consumption() |
| --- | --- | --- | --- |
| ECMS | 6.39 | 100% | 66% |
| A-ECMS | 5.95 | 93.1% | 31.05% |
| RB | 5.26 | 82.3% | 50.32% |
| Control Strategy | Electric Economy(%) | CO2 emission(g/Km) | MPGe |
| ECMS | 212.56% | 116.84 | 35.66 |
| A-ECMS | 100% | 102.45 | 38.18 |
| RB | 162.06% | 90.89 | 44.91 |

(a)

(b)

(c)

(d)

Figure 11 Experimental results of ECMS, A-ECMS, and RB under Scenario 2.

5 Conclusion

This study developed a plug-in hybrid electric vehicle (PHEV) featuring a dual-motor hybrid powertrain integrated with an electrically controlled mechanical automatic transmission. A dual-static adaptive A-ECMS energy management strategy was designed, along with a dedicated energy management controller based on this approach. The research primarily focused on optimizing the key parameter—equivalent factor—in the A-ECMS strategy. Additionally, the study evaluated the environmental impact of the proposed controller, particularly in terms of CO2 emissions, highlighting its potential contribution to carbon and emission reduction efforts.

To effectively validate the proposed methodology, Chapter Four presents a comparative experimental study involving the A-ECMS, ECMS, and RB strategies under two distinct operating scenarios. Initially, the structure and modeling of the PHEV with an electrically controlled mechanical automatic transmission were developed. Based on this model, three energy management strategies were implemented. Simulation results under the UDDS driving cycle revealed that the A-ECMS algorithm improved fuel economy by 7.8% compared to the ECMS approach, alongside a CO2 emissions reduction of 15.14 g/km.

Furthermore, to assess the robustness of the A-ECMS strategy under varying conditions, a 5% random noise perturbation was introduced into typical operating conditions. The simulation findings demonstrated that, despite these perturbations, the A-ECMS strategy still enhanced fuel economy by 6.9% and reduced CO2 emissions by 14.39 g/km compared to the ECMS strategy. These results confirm the strong robustness of the proposed energy management controller.

In the second experimental scenario, the overall stability of the A-ECMS strategy was found to surpass that of the RB strategy. The RB strategy, upon reaching a predefined SOC threshold, shifts into a battery charge maintenance mode, where the engine becomes the dominant power source while the battery functions as an auxiliary. This shift leads to a notable increase in fuel consumption and CO2 emissions, ultimately compromising the stability of the powertrain system.

From an economic perspective, the A-ECMS strategy demonstrated significant cost-saving potential, reducing expenses by 30.17% and 28.56% compared to the ECMS strategy across the two scenarios. These findings underscore the effectiveness of the proposed strategy in enhancing both economic and environmental sustainability in PHEVs.

Ethical approval (where ever applicable)

It is stated that the research was carried out based on ethical standards.

References

- Liu T, Wang B, Yang C. Online Markov chain-based energy management for a hybrid tracked vehicle with speedy Q-learning. Energy 2018;160:544–55.

- Ramachandran S, Stimming U. Well to wheel analysis of low carbon alternatives for road traffic. Energy Environ Sci 2015;8:3313–24.

- Song Z, Zhang X, Li J, Hofmann H, Ouyang M, Du J. Component sizing optimization of plug-in hybrid electric vehicles with the hybrid energy storage system. Energy 2018;144:393–403.

- Zou Y, Liu T, Sun F, Peng H. Comparative study of dynamic programming and Pontryagin’s minimum principle on energy management for a parallel hybrid electric vehicle. Energies 2013;6:2305–18.

- Momen F, Rahman K, Son Y. Electrical propulsion system design of Chevrolet Bolt battery electric vehicle. IEEE Trans Ind Appl 2018;55:376–84.

- Pourabdollah M, Egardt B, Murgovski N, Grauers A. Convex optimization methods for powertrain sizing of electrified vehicles by using different levels of modeling details. IEEE Trans Veh Technol 2017;67:1881–93.

- Wu X, Hu X, Yin X, Li L, Zeng Z, Pickert V. Convex programming energy management and components sizing of a plug-in fuel cell urban logistics vehicle. J Power Sources 2019;423:358–66.

- Khodaparastan, M., Mohamed, A. A., & Brandauer, W. (2019). Recuperation of regenerative braking energy in electric rail transit systems. IEEE Transactions on Intelligent Transportation Systems, 20(8), 2831-2847.

- Zhou, D., Al-Durra, A., Matraji, I., Ravey, A., & Gao, F. (2018). Online energy management strategy of fuel cell hybrid electric vehicles: a fractional-order extremum seeking method. IEEE Transactions on Industrial Electronics, 65(8), 6787-6799.

- Tang X, Jia T, Hu X, et al. Naturalistic data-driven predictive energy management for plug-in hybrid electric vehicles. IEEE Transactions on Transportation Electrification 2021;7(2):497–508.

- Johri R, Filipi Z. Optimal energy management of a series hybrid vehicle with combined fuel economy and low-emission objectives. Proc Inst Mech Eng - Part D J Automob Eng 2014;228:1424–39.

- Castaings A, Lhomme W, Trigui R, Bouscayrol A. Comparison of energy management strategies of a battery/supercapacitors system for electric vehicle under real-time constraints. Appl Energy 2016;163:190–200.

- Althaher S, Mancarella P, Mutale J. Automated demand response from home energy management system under dynamic pricing and power and comfort constraints. IEEE Trans Smart Grid 2015;6:1874–83.

- Langari R, Won J-S. Intelligent energy management agent for a parallel hybrid vehicle-part I: system architecture and design of the driving situation identification process. IEEE Trans Veh Technol 2005;54:925–34.

- Yue, M., Jemei, S., Gouriveau, R., & Zerhouni, N. (2019). Review on health-conscious energy management strategies for fuel cell hybrid electric vehicles: Degradation models and strategies. International Journal of Hydrogen Energy, 44(13), 6844-6861.

- Xing, Y., & Lv, C. (2019). Dynamic state estimation for the advanced brake system of electric vehicles by using deep recurrent neural networks. IEEE Transactions on Industrial Electronics, 67(11), 9536-9547.

- Caineng, Z. O. U., Xiong, B., Huaqing, X. U. E., Zheng, D., Zhixin, G. E., Ying, W. A. N. G., ... & Songtao, W. U. (2021). The role of new energy in carbon neutral. Petroleum exploration and development, 48(2),

- Sanguesa, J. A., Torres-Sanz, V., Garrido, P., Martinez, F. J., & Marquez-Barja, J. M. (2021). A review on electric vehicles: Technologies and challenges. Smart Cities, 4(1), 372-404.

- Cano, Z. P., Banham, D., Ye, S., Hintennach, A., Lu, J., Fowler, M., & Chen, Z. (2018). Batteries and fuel cells for emerging electric vehicle markets. Nature Energy, 3(4),  279-289.

- Chen, M., Ma, X., Chen, B., Arsenault, R., Karlson, P., Simon, N., & Wang, Y. (2019). Recycling end-of-life electric vehicle lithium-ion batteries. Joule, 3(11), 2622-2646.

- Kovač, A., Paranos, M., & Marciuš, D. (2021). Hydrogen in energy transition: A review. International Journal of Hydrogen Energy, 46(16), 10016-10035.

- Reitz, R. D., Ogawa, H., Payri, R., Fansler, T., Kokjohn, S., Moriyoshi, Y., ... & Zhao, H. (2020). IJER editorial: The future of the internal combustion engine. International Journal of Engine Research, 21(1), 3-10.

- Harper, G., Sommerville, R., Kendrick, E., Driscoll, L., Slater, P., Stolkin, R., ... & Anderson, P. (2019). Recycling lithium-ion batteries from electric vehicles. nature, 575(7781), 75-86.

- Tang X, Duan Z, Hu X, et al. Improving ride comfort and fuel economy of connected hybrid electric vehicles based on traffic signals and real road information. IEEE Trans Veh Technol 2021;7(4):3101–12.

- Du G, Zou Y, Zhang X, Liu T, Wu J, He D. Deep reinforcement learning based energy management for a hybrid electric vehicle.Energy.2020;201;117591.

- Shuai B, Zhou Q, Li J, He Y, Li Z, Williams H, Xu H, Shuai S. Heuristic action execution for energy efficient charge-sustaining control of connected hybrid vehicles with model-free double Q-learning. Appl Energy 2020;267:114900.

- Liu T, Zou Y, Liu D, Sun F. Reinforcement learning of adaptive energy management with transition probability for a hybrid electric tracked vehicle. IEEE Trans Ind Electron 2015;62:7837–46.

- Zou Y, Liu T, Liu D, Sun F. Reinforcement learning-based real-time energy management for a hybrid tracked vehicle. Appl Energy 2016;171:372–82.

- Liu T, Hu X. A Bi-level control for energy efficiency improvement of a hybrid tracked vehicle. IEEE Trans Ind Informatics 2018;14:1616–25.

- Yu H, Kuang M, McGee R. Trip-oriented energy management control strategy for plug-in hybrid electric vehicles. IEEE Trans Contr Syst Technol 2013;22:1323–36.

- Ozatay E, Onori S, Wollaeger J, Ozguner U, Rizzoni G, Filev D, Di Cairano S. Cloud-based velocity profile optimization for everyday driving: a dynamic-programming-based solution. IEEE Trans Intell Transport Syst 2014;15:2491–505.

- Zhang, Q., Wang, L.J., Li, G., Liu, Y., 2020. A real-time energy management control strategy for battery and supercapacitor hybrid energy storage systems of pure electric vehicles. J. Energy Storage 31, 101721.

- Zhang, L., Wang, Z.P., Sun, F.C., Dorrell, D.G., 2014. Online parameter identification of ultracapacitor models using the extended Kalman filter. Energies 7 (5), 3204–3217.

- Zhou, H.H., Bhattacharya, T., Tran, D., Siew, T.S.T. Khambadkone, A.M., 2010.Composite energy storage system involving battery and ultracapacitor with dynamic energy management in microgrid applications. IEEE Trans. Power Electron. 26 (3), 923–930.

- Wang, B., Xu, J., Cao, B.G., Zhou, X., 2015. A novel multimode hybrid energy storage system and its energy management strategy for electric vehicles. J. Power Sources 281, 432–443.

- Zhang, S., Xiong, R., Cao, J.Y., 2016. Battery durability and longevity based power management for plug-in hybrid electric vehicle with hybrid energy storage system. Appl. Energy 179, 316–328.

- Castaings, A., Lhomme, W., Trigui, R., Bouscayrol, A., 2016. Comparison of energy management strategies of a battery/supercapacitors system for electric vehicle under real time constraints. Appl. Energy 163, 190–200.

- Li, J.W., Xiong, R., Mu, H., Cornélusse, B., Vanderbemden, P., Ernst, D., Yuan, W.J.2018. Design and real-time test of a hybrid energy storage system in the microgrid with the benefit of improving the battery lifetime. Appl. Energy 218, 470–478.

- Sellali, M., Abdeddaim, S., Betka, A., Djerdir, S., Tiar, M., 2019. Fuzzy–super twisting control implementation of battery/super capacitor for electric vehicles.ISA Trans. 95, 243–253.

- Amy, T., Kong, H., Offer, G., Longo, S., 2016. Regularized MPC for power management of hybrid energy storage systems with applications in electric vehicles. IFAC-Papers OnLine 49 (11), 265–270.

- Wu J, He H, Peng J, Li Y, Li Z. Continuous reinforcement learning of energy management with deep Q network for a power split hybrid electric bus. Appl Energy 2018;222:799–811.

- Gillingham, K. T., Knittel, C. R., Li, J., Ovaere, M., & Reguant, M. (2020). The short-run and long-run effects of Covid-19 on energy and the environment. Joule, 4(7), 1337-1341.

- Lv, C., Hu, X., Sangiovanni-Vincentelli, A., Li, Y., Martinez, C. M., & Cao, D. (2018). Driving-style-based codesign optimization of an automated electric vehicle: A cyber-physical system approach. IEEE Transactions on Industrial Electronics, 66(4), 2965-2975.

- Baars, J., Domenech, T., Bleischwitz, R., Melin, H. E., & Heidrich, O. (2021). Circular economy strategies for electric vehicle batteries reduce reliance on raw materials. Nature Sustainability, 4(1), 71-79.