Short Research Article

PHYSICOCHEMICAL ANALYSIS OF THE LIMNETIC AND LITTORAL ZONES OF

LAKE PINAMALOY, DON CARLOS,

BUKIDNON, PHILIPPINES

.

ABSTRACT

Lake Pinamaloy is an important freshwater resource in Bukidnon, Philippines, providing water for fishing, household use, and recreation. This study assessed its water quality by measuring temperature, pH, electrical conductivity (EC), total dissolved solids (TDS), salinity, and turbidity at five sampling stations in both the limnetic and littoral zones. Most of the results—pH at 6.78, EC at 61.13 µS/cm, TDS at 29.6 mg/L, and salinity at 30.33 mg/L—were within acceptable levels. However, turbidity had a high average of 20.08 NTU, exceeding the recommended limit of 5 NTU, which may affect aquatic life. Significant differences in temperature, turbidity, and salinity were observed between the sampling stations, likely due to natural factors or human activities. These findings provide valuable baseline data for monitoring and management. Future studies should identify pollution sources through land use mapping and watershed analysis, and include biological indicators like plankton or macroinvertebrates to better understand the lake’s ecological health.

Keywords: freshwater ecosystem, in-situ analysis, Lake Pinamaloy, physicochemical parameters, water quality

1. INTRODUCTION

Lakes serve as critical freshwater resources for agriculture, industry, and biodiversity conservation (Cull, 2023). According to the United Nations Environment Programme (UNEP), lakes contain 90% of the planet’s surface freshwater, making them essential in supporting wildlife, sustaining ecosystems, and meeting water demands across sectors (UNEP, 2022). However, growing human activities have contributed to the degradation of many lake systems, particularly in developing regions where pollution and overexploitation of resources are common issues (Brillo, 2015). Monitoring physicochemical parameters such as temperature, pH, turbidity, salinity, total dissolved solids (TDS), and electrical conductivity is important for evaluating water quality and ecological health. Assessing the physicochemical characteristics of lake water is important in evaluating its quality, ecological status, and suitability for different uses.

Lake Pinamaloy, located in Don Carlos, Bukidnon, is one of the major freshwater lakes in Mindanao, Philippines. It holds ecological, cultural, and economic significance for the surrounding communities. Despite its importance, there is limited research on its water quality, especially its physicochemical properties. Fluctuations in temperature, pH, turbidity, salinity, total dissolved solids (TDS), and electrical conductivity can show signs of pollution or chemical contamination in freshwater systems. Elevated turbidity and TDS often result from surface runoff and sedimentation, while shifts in pH and conductivity are commonly linked to agricultural or industrial discharges (Palamuleni and Akoth, 2015). In Lake Lanao, nutrient loading and poor waste management have led to significant changes in water quality (Angagao et al., 2017). Understanding parameters such as temperature, pH, turbidity, salinity, total dissolved solids (TDS), and electrical conductivity provides awareness into the lake’s ecological condition and the impact of anthropogenic activities.

Physicochemical parameters are important for maintaining aquatic ecosystems balanced and productive. Temperature and pH affect biological activities, such as metabolism and the availability of nutrients in the water. Turbidity affects light penetration, which impacts photosynthesis and the growth of aquatic vegetation. Salinity, total dissolved solids (TDS), and electrical conductivity show the concentration of dissolved ions and possible pollutants, showing the chemical condition of the water  (Leiva-Tafur et al., 2022).  Shifts in these factors often show signs of environmental stress due to pollution or human activities, this emphasizes the importance of regular assessments for the protection and sustainable management of freshwater systems (Nirmala et al., 2022).

Studies in similar freshwater bodies such as Kalawaig Creek in Bukidnon, physicochemical analysis of Kalawaig Creek provided essential baseline data, examining parameters such as temperature, total dissolved solids, pH, dissolved oxygen, and total alkalinity to evaluate water conditions and environmental health (Damasco et al., 2024). Additionally, limnological research on LakeWood examined stratification and water quality, emphasizing the role of such studies in understanding seasonal and ecological dynamics (Buarao, 2021). Moreover, investigations into molluscan fauna in lakes such as Lake Lakewood demonstrated how physicochemical conditions influence aquatic biodiversity and the distribution of sensitive species (Mahinay-Cardente et al., 2024). Despite the presence of comparable freshwater ecosystems in the region, Lake Pinamaloy has not been thoroughly studied.

This study aims to assess the physicochemical properties of the limnetic and littoral zones of Lake Pinamaloy by measuring key parameters: temperature, pH, turbidity, salinity, TDS, and electrical conductivity. By analyzing these parameters, the study seeks to establish baseline data that will aid in understanding the current water quality and its trends over time. The research will also identify potential sources of pollution or contamination affecting the lake’s ecosystem.

2.  Methodology

2.1 Research design

This study used a quantitative descriptive research design to assess water quality in Lake Pinamaloy. In situ measurements of temperature, pH, electrical conductivity (EC), total dissolved solids (TDS), salinity, and turbidity were taken from five sampling stations in the limnetic and littoral zones. The independent variables were the physico-chemical parameters, while the dependent variable was the water quality condition across the different stations. The study described the current water conditions and compared results between stations to identify differences.

2.2 Entry Protocol

Prior to the conduct of the study, all necessary permissions and ethical clearances were obtained. Formal approval was secured from the Lake Pinamaloy Management Office to ensure compliance with local environmental and regulatory guidelines. Institutional consent was also granted by the administration of San Isidro College. Informed consent was obtained from participants or their legal guardians, following a clear explanation of the study’s purpose, scope, and duration. All procedures were conducted in accordance with ethical research standards to ensure transparency, accountability, and respect for all stakeholders involved.

2.3 Study Site

Lake Pinamaloy (7°58′45″N, 125°05′49″E) is a 60-hectare tectonic freshwater lake located in Don Carlos, Bukidnon, Philippines, at an elevation of 312 meters above sea level. The lake is primarily sustained by surface runoff and serves as a key resource for potable water, agricultural irrigation, and local tourism due to its natural landscape and ecological significance (Mahinay-Cardente et al., 2024).

Figure 1 Location and Sampling Stations in Lake Pinamaloy, Don Carlos, Bukidnon

Data collection was conducted from February to March 2025. During this period, five sampling stations were established across the lake to represent spatial variability. Two stations were situated in the limnetic zone (open water), and three stations were located in the littoral zone (nearshore areas). At each station, triplicate in situ measurements of selected physico-chemical parameters were conducted to ensure reliability and accuracy of data.

2.4 Identification and Collection of Water Samples Of Physicochemical Parameters

Adapting the protocols of Igloria et al. (2024), the assessment of physico-chemical parameters was conducted in situ at each designated sampling station in Lake Pinamaloy. Triplicate readings were taken per station to reduce variability and ensure the reliability of the results. The parameters measured included turbidity, water temperature (°C), pH, total dissolved solids (TDS), electrical conductivity (EC), and salinity—providing a broad assessment of the lake’s water quality.

Measurements were conducted using a digital multiparameter water quality tester. To ensure accuracy and consistency, the device was calibrated before and after each use in accordance with the manufacturer’s guidelines. The average of the triplicate readings for each parameter was calculated and used for statistical analysis to assess spatial variations between stations.

2.5 Statistical Treatment of Data

Descriptive statistics were used to summarize the measurements of each physico-chemical parameter across sampling stations. To determine whether there were significant differences in water quality among the stations, one-way analysis of variance (ANOVA) was performed, followed by Tukey’s post hoc test for pairwise comparisons of means. A significance level of 0.05 was applied. Prior to conducting ANOVA, the Shapiro-Wilk test was used to assess the normality of data distribution and ensure that statistical assumptions were met.

2.6 Documentation

The research process was thoroughly documented through photographs capturing key stages such as site observation, water sampling, and in situ measurement of physico-chemical parameters. All data were recorded systematically in both digital formats and handwritten field logs to ensure accuracy, consistency, and data backup. These records served as references for verification and analysis.

2.7 Ethical Considerations

This study followed established ethical guidelines to uphold the credibility, integrity, and transparency of the research. Emphasis was placed on objectivity and accuracy, particularly in the collection, interpretation, and reporting of data. Fieldwork was conducted in accordance with safety protocols to minimize risks to the researchers and the environment. Compliance with the Data Privacy Act was ensured by implementing safeguards to protect any personal or sensitive information. All data was securely stored and accessible only to authorized personnel. Any personal identifiers were removed or anonymized to maintain confidentiality and privacy throughout the research process.

2.8 Precautionary Measures

To ensure the safety and well-being of all participants during fieldwork, precautionary measures were strictly implemented. Researchers wore appropriate personal protective equipment (PPE), including gloves, boots, and masks when necessary. Fieldwork was conducted during safe weather conditions to avoid potential hazards, and sampling equipment was sanitized before and after use to prevent contamination.

3. results and discussion

3.1 Physicochemical Properties

Figure 2. Physicochemical properties of water across the limnetic and littoral zones

Table 1. Physicochemical Measurements Across Sampling Stations

| Parameters | Sampling Sites |  |  |  |  | Average | Standard | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A1 | A2 | B1 | B2 | B3 |  |  |  |
| Temperature (℃) | 27.37 | 27.70 | 29.23 | 28.67 | 27.83 | 28.16 | 25℃-31℃ | Passed |
| pH | 7.12 | 6.97 | 6.62 | 6.57 | 6.61 | 6.78 | 6.5-8.5 | Passed |
| Conductivity (μS/cm) | 57.00 | 63.33 | 46.67 | 66.67 | 72.00 | 61.13 | <1,500 μS/cm | Passed |
| TDS (mg/L) | 28.33 | 31.00 | 22.00 | 31.67 | 35.00 | 29.60 | <1000 mg/L | Passed |
| Salinity (mg/L) | 28.00 | 30.67 | 23.67 | 34.00 | 35.33 | 30.33 | <500 mg/L | Passed |
| Turbidity (NTU) | 9.20 | 8.67 | 32.4 | 5.10 | 48.63 | 20.08 | <5 NTU | Did not pass |

Figure 2 shows the physicochemical results from five sampling stations, with A1 and A2 representing the limnetic zone and B1, B2, and B3 representing the littoral zone. Temperature readings are close across all stations, with slightly higher values observed in the littoral zone. pH levels are slightly higher in the limnetic zone, but all stations fall within a slightly acidic to neutral range. Conductivity is higher in the littoral stations, particularly in B3, while B1 has the lowest value. A similar trend is seen in total dissolved solids and salinity, where B2 and B3 record the highest levels and B1 the lowest. The most noticeable difference is in turbidity, with B3 and B1 showing much higher levels, indicating more particles in the water, while the limnetic stations have significantly lower turbidity, indicating clearer water. Overall, the results show that the littoral zone generally has higher concentrations of dissolved substances and suspended particles compared to the limnetic zone. Supporting this, Table 1 presents a summary of these physicochemical measurements, including temperature, pH, conductivity, total dissolved solids, salinity, and turbidity, and compares the values from each station against standard values to evaluate water quality. It also provides the average for each parameter and indicates whether the readings pass or fail based on compliance with established water quality limits.

The temperature across the sampling stations ranges from 27.37°C to 29.23°C, with an average of 28.16°C, all within the acceptable range of 25°C to 31°C, indicating stable thermal conditions favorable for aquatic life. pH levels range from 6.57 to 7.12, averaging 6.78, within the permissible range of 6.5 to 8.5, suggesting slightly acidic to neutral water quality suitable for freshwater ecosystems. Conductivity values range from 46.67 to 72 μS/cm, with an average of 61.13 μS/cm, well below the 1,500 μS/cm limit, reflecting low ion concentrations, while TDS levels range from 22.00 to 35.00 mg/L, with an average of 29.60 mg/L, below the 1,000 mg/L limit, indicating minimal dissolved solids. Salinity levels are also low, between 23.67 and 35.33 mg/L, well below the 500 mg/L threshold, consistent with freshwater characteristics. However, turbidity values vary widely, ranging from 5.10 NTU to 48.63 NTU, with an average of 20.08 NTU, exceeding the acceptable limit of 5 NTU at stations B1 and B3. High turbidity suggests elevated sediment or pollutant levels, potentially linked to local environmental factors affecting water clarity.

Excess nutrients from agricultural runoff, urban wastewater, and industrial discharges drive eutrophication, leading to algal blooms that increase turbidity by reducing light penetration (Hietala et al., 2004; Wiik et al., 2015). Surface runoff from rainfall or flooding transports sediments into littoral zones, as seen in hurricanes that raise turbidity by introducing tannin-stained water and sediments (Timbs & Kolterman, 2023). Urban and industrial wastewater also contributes suspended solids, disrupting aquatic ecosystems, as observed in Lake Ladoga (Romanov & Arshanitsa, 2023). Additionally, construction projects like dams alter hydrodynamics, redistributing sediments and increasing turbidity (Makaoui et al., 2018). Wind-carried pollutants can also settle into water bodies, as seen in Lake Baikal (Ia & Ib, 2020). Submerged aquatic vegetation (SAV), crucial for stabilizing sediments, is often lost due to herbivory or low light, worsening turbidity (Timbs & Kolterman, 2023).

High turbidity reduces light availability for photosynthesis, disrupts food webs, and impairs fish reproduction, as noted in Lake Ladoga, where pollution-induced turbidity caused fish toxicosis (Romanov & Arshanitsa, 2023). Nutrient enrichment further diminishes buffer capacity against algal blooms in vegetated zones (Heitala, 2004). Stable temperatures are vital for regulating aquatic metabolic rates, enabling species to thrive without stress from extreme fluctuations (Guzzo et al., 2017). Similarly, pH stability indicates environmental health, with sudden changes signaling pollution or ecological disturbances (Santhanam & Dhyani, 2022).

Table 2. Mean, variance, standard deviation, and standard error of Water Quality Parameters

The data presented in the table provides an overview of the mean values for the water quality parameters measured across the five sampling stations, along with their corresponding variance, standard deviation, and standard error. The statistical results highlight variations in water quality, with some parameters exhibiting greater fluctuations across locations.

| Parameter | Mean | Variance | Std. Dev. | Std. Err. |
| --- | --- | --- | --- | --- |
| Temperature | 28.16 | 0.55 | 0.74 | 0.191 |
| pH | 6.78 | 0.36 | 0.60 | 0.156 |
| EC | 61.13 | 154.12 | 12.41 | 3.210 |
| TDS | 29.60 | 34.84 | 5.90 | 1.524 |
| Salinity | 30.33 | 29.70 | 5.45 | 1.407 |
| Turbidity | 20.08 | 342.29 | 18.50 | 4.780 |

Table 2 presents the descriptive statistical values for the water quality parameters measured across the five sampling stations. The mean values summarize the central tendency of the data, while variance, standard deviation, and standard error provide insights into the level of variability for each parameter. Temperature shows low variance (0.55) and standard deviation (0.734), indicating minimal fluctuations across stations. Similarly, pH values remain stable, with a mean of 6.78 and a standard deviation of 0.60, suggesting a consistent acidity level throughout the lake.

To better understand these variations, the sampling stations are categorized into two zones: limnetic (A1, A2) and littoral (B1, B2, B3). The limnetic zone, characterized by deeper open water, generally exhibits more stable temperature and water chemistry due to its depth and reduced exposure to external influences (Shariff, 2014). In contrast, the littoral zone, being shallow and nearshore, is subject to greater environmental fluctuations, sediment disturbances, and human activity (Hoverman, 2012). Studies on freshwater lakes have shown that nearshore areas often experience more variation in turbidity and electrical conductivity due to their interaction with surrounding land (Obegi et al., 2021; Leidonald et al., 2024).

Electrical conductivity (EC) exhibits the highest variance (154.12), indicating notable differences in ion concentration across stations, while turbidity has a variance of 342.29 and a standard deviation of 18.50, reflecting significant fluctuations in water clarity. Turbidity values are notably higher in the littoral zone (B1 and B3) compared to the limnetic zone (A1 and A2), showing that nearshore areas experience greater sediment resuspension due to wave action and watershed inputs. Organic matter accumulation, such as decaying vegetation in the littoral zone, may also contribute to increased turbidity levels (Austin et al., 2017). The standard error values further emphasize the reliability of these means, with lower values suggesting higher precision in measurement. Greater fluctuations in EC and salinity in the littoral zone indicate stronger external influences such as runoff and groundwater seepage, while studies in tropical lakes confirm that fluctuations in ion concentrations often correlate with seasonal changes and human activity (Obegi et al., 2021; Leidonald et al., 2024).

The stability of temperature and pH suggests that Lake Pinamaloy provides a consistent habitat for aquatic life. According to Easwaramoorthy et al. (2024), minimal temperature variation in lakes helps maintain fish metabolism and reproductive cycles. However, the high EC and salinity variability may indicate localized influences such as agricultural runoff or varying sediment composition (Guan et al., 2022; Chianca et al., 2023; Olaojo & Oladunjoye, 2023; Pratiwi et al., 2024). Greater fluctuations in turbidity confirm localized disturbances that may impact light penetration and photosynthesis. Research by Erhenhi et al. (2024) and Motunrayo (2024) suggests that high turbidity reduces dissolved oxygen levels, affecting aquatic biodiversity.

Table 3. One-Way ANOVA Results for Water Quality Parameters

| Parameter | F-value | p-value | Statistically Significant? |
| --- | --- | --- | --- |
| Temperature | 91.57 | <0.0001 | Yes |
| pH | 0.35 | 0.8372 | No |
| EC | 2.26 | 0.1517 | No |
| TDS | 2.37 | 0.1390 | No |
| Salinity | 4.46 | 0.0346 | Yes |
| Turbidity | 19.81 | 0.0003 | Yes |

Table 3 presents the one-way ANOVA results, analyzing whether significant differences exist in water quality parameters across the five sampling stations. The F-values reflect the degree of variation among stations, with higher F-values indicating greater differences between stations relative to variations within each station. The corresponding p-values determine whether these differences are statistically significant.

The ANOVA results indicate statistically significant differences among stations for temperature (p < 0.0001), turbidity (p = 0.0003), and salinity (p = 0.0346), while pH (p = 0.8372), electrical conductivity (EC) (p = 0.1517), and total dissolved solids (TDS) (p = 0.1390) remain relatively stable. This stability is particularly evident in the limnetic zone (A1, A2), where deeper, open waters experience less variation in temperature and ion concentrations due to greater depth and reduced exposure to external disturbances (Payler et al., 2019; Mackenzie et al., 2024). In contrast, the littoral zone (B1, B2, B3), located near the shoreline, exhibits greater variability in turbidity and salinity. This variability results from exposure to physical forces such as waves and tides, biological processes like algal blooms, and human influences, including runoff, all of which contribute to fluctuating water conditions in shoreline areas compared to more stable deep-water regions (Cho, 2007; Austin et al., 2017).

These variations underscore the littoral zone’s vulnerability to environmental changes and their ecological consequences. Increased turbidity can reduce light penetration, affecting photosynthesis and dissolved oxygen levels, which are crucial for aquatic life (Lunt & Smee, 2019; Nunes et al., 2022; Garcia et al., 2024). Meanwhile, the relative stability of pH, EC, and TDS across zones suggests that the lake’s natural buffering and mixing processes help maintain a chemical balance, preventing extreme fluctuations in water composition (Sobczyński & Joniak, 2013; Sojka et al., 2020; DeSellas et al., 2023).

These findings align with previous studies showing that temperature remains more stable in deeper limnetic zones due to reduced exposure to surface influences (Heling et al., 2018; Miller, 2020; Sulawesty & Yustiawati, 2021). Research also confirms that littoral zones typically experience greater turbidity fluctuations due to wave-induced sediment resuspension and watershed runoff (Biswas et al., 2021; Abhishek et al., 2024). Additionally, studies on tropical lakes emphasize that salinity differences often reflect localized groundwater seepage and human activity near shorelines (Dalkıran & Zünbülgil-Ünsal, 2023; Malashenkov et al., 2024). These findings reinforce the role of internal mixing mechanisms in maintaining chemical stability in freshwater lakes, consistent with previous research (Lazarus et al., 2023; Wand’arhasima et al., 2025).

4. Conclusions and Recommendations

The results of the study show that the physicochemical parameters of Lake Pinamaloy, specifically temperature, pH, electrical conductivity (EC), total dissolved solids (TDS), salinity, and turbidity are mostly within acceptable standards for freshwater ecosystems. However, turbidity exceeded the recommended limit of <5 NTU. The elevated turbidity suggests a high level of suspended particles, which may affect aquatic organisms and overall water clarity. In conclusion, Lake Pinamaloy maintains generally good water quality, but the high turbidity indicates potential environmental stress and pollution sources that need to be addressed. It is recommended that regular water quality monitoring be conducted to detect changes over time, and that efforts be made to control runoff, erosion, and human activities around the lake. Future research should explore the seasonal variations in the physicochemical parameters of Lake Pinamaloy to assess how water quality changes throughout the year. It is also recommended to expand the scope by including biological indicators, such as plankton or macroinvertebrates, to gain a more comprehensive understanding of the lake's ecological health. Further studies could investigate the potential sources of turbidity and other pollutants through land use mapping and watershed analysis. Lastly, incorporating community perceptions and stakeholder involvement in future studies can help link scientific findings with local conservation efforts and policy development

References

Cull, M. (2023). Why Lakes Are Important Resources. Discover Magazine. https://www.discovermagazine.com/planet-earth/why-lakes-are-important-resources?utm

UNEP. (2022). The interlinked threats facing lakes and why we need to protect them. UNEP. https://www.unep.org/news-and-stories/story/interlinked-threats-facing-lakes-and-why-we-need-protect-them

Brillo, B. B. C. (2015). The Status of Philippine Lake Studies: Scholarly Deficit in Social Science and Small-Lake Research. Asia-Pacific Social Science Review, 15(1). https://doi.org/10.59588/2350-8329.1045

Palamuleni, L., & Akoth, M. (2015). Physico-Chemical and Microbial Analysis of Selected Borehole Water in Mahikeng, South Africa. International Journal of Environmental Research and Public Health, 12(8), 8619–8630. https://doi.org/10.3390/ijerph120808619

Angagao, N. B., Quiao, M. A. D., Roa, E. C., & Prado, G. I. (2017). Water Quality Assessment of the South-Eastern Part of Lake Lanao, Philippines. International Letters of Natural Sciences, 63, 34–41. https://doi.org/10.18052/www.scipress.com/ilns.63.34

Leiva-Tafur, D., Goñas, M., Culqui, L., Santa Cruz, C., Rascón, J., & Oliva-Cruz, M. (2022). Spatiotemporal distribution of physicochemical parameters and toxic elements in Lake Pomacochas , Amazonas, Peru. Frontiers in Environmental Science, 10. https://doi.org/10.3389/fenvs.2022.885591

Nirmala, K., Senthil Kumar, P., Ambujam, N. K., & Srinivasalu, S. (2022). Assessment of physico-chemical parameters of surface waters of a tropical brackish water lake in South Asia. Environmental Research, 214, 113958. https://doi.org/10.1016/j.envres.2022.113958

Damasco, L. A., Jean, V., Quilang, E. B., Cutor, M., Benedict, L., & Jay, I. (2024). Rapid Physicochemical Assessment of Kalawaig Creek in Bukidnon, Philippines. Asian Journal of Biological and Life Sciences, 13(1), 176–186. https://doi.org/10.5530/ajbls.2024.13.23

Buarao, M. (2021). Limnology of Lake Wood: An Ancestral Lake of the Subanen Tribe. Dost.gov.ph.https://philjournalsci.dost.gov.ph/home-1/108-vol-150-no-5-october-2021/1482-limnology-of-lake-wood-an-ancestral-lake-of-the-subanen-tribe

Mahinay-Cardente, C. L., Torres, M. A. J., & Demayo, C. G. (2024). Molluscan fauna in selected freshwater lakes in Mindanao, Philippines. Aquaculture, Aquarium, Conservation & Legislation, 17(Issue 4). https://bioflux.com.ro/docs/2024.1460-1470.pdf

Department of Environment and Natural Resources. (2016). Water quality guidelines and general effluent standards of 2016 (DENR Administrative Order No. 2016-06).

Igloria, J. L., Labonite-Seriño, E. K., Magbanua, F. S., & Pogado, F. O. (2024). Macroinvertebrate community as bioindicator of water quality of Tambis River, Palompon, Leyte, Philippines.

Hietala, J., Vakkilainen, K., & Kairesalo, T. (2004). Community resistance and change to nutrient enrichment and fish manipulation in a vegetated lake littoral. Freshwater Biology, 49(12), 1525–1537. https://doi.org/10.1111/j.1365-2427.2004.01303.x

Timbs, R., & Kolterman, D. (2023). Herbivory impacts Vallisneria americana recovery in the lower St. Johns River, Florida. Journal of Aquatic Plant Management, 61(2), 63–65. https://doi.org/10.57257/japm-d-23-00003

Romanov, A. Y., & Arshanitsa, N. M. (2023). Ecological and ichthyotoxicological state of the southern water area of Lake Ladoga. International Journal of Veterinary Medicine, 2, 171–178. https://doi.org/10.52419/issn2072-2419.2023.2.171

Makaoui, A., Idrissi, M., Agouzouk, A., Larissi, J., Baibai, T., Ouehabi, Z. E., Laamel, M. A., Bessa, I., Ettahiri, O., & Hilmi, K. (2018). Etat oceanographique de la lagune de Oualidia, Maroc (2011-2012). European Scientific Journal ESJ, 14(18), 93. https://doi.org/10.19044/esj.2018.v14n18p93

Ia, B., & Ib, V. (2020). Snow pollution in the water area of the world heritage lake. International Journal of Hydrology, 4(4), 165–174. https://doi.org/10.15406/ijh.2020.04.00242

Guzzo, M. M., Blanchfield, P. J., & Rennie, M. D. (2017). Behavioral responses to annual temperature variation alter the dominant energy pathway, growth, and condition of a cold-water predator. Proceedings of the National Academy of Sciences, 114(37), 9912–9917. https://doi.org/10.1073/pnas.1702584114

Santhanam, H., & Dhyani, S. (2022). Lake ecosystems as proxies of change in a post-pandemic era. Environmental Sustainability, 5(3), 389–393. https://doi.org/10.1007/s42398-022-00242-9

Shariff, M. (2014). Littoral and limnetic phytoplanktonibution and biodiversity in a tropical man-made lake, Malaysia. Advanced Studies in Biology; Hikari, Ltd. https://www.academia.edu/116882816/Littoral_and_limnetic_phytoplankton_distribution_and_biodiversity_in_a_tropical_man_made_lake_Malaysia

Obegi, B. N., Ogendi, G. M., Omondi, R., Siriba, B. J., Morara, G. N., Rindoria, N. M., & Orina, P. (2021). Characteristic Relationships between Phosphorous Accrual, Ecosystem Aspects and Water Level Fluctuations in Tropical Lakes: Naivasha Ramsar Site, Kenya. Journal of Geoscience and Environment Protection, 09(01), 53–67. https://doi.org/10.4236/gep.2021.91004

Leidonald, R., Muhtadi, A. M., Susetya, I. E., Dewinta, A. F., Hasibuan, J. S., Simamora, L. F., & Saville, R. (2024). The dynamics and water quality status of a tropical coastal lake in Anak Laut Lake, Singkil Indonesia. Indonesian Journal of Marine Sciences, 29(3), 329–339. https://doi.org/10.14710/ik.ijms.29.3.329-339

Austin, Å. N., Hansen, J. P., Donadi, S., & Eklöf, J. S. (2017). Relationships between aquatic vegetation and water turbidity: A field survey across seasons and spatial scales. PLoS ONE, 12(, e0181419. https://doi.org/10.1371/journal.pone.0181419

Easwaramoorthy, Mellissa & Thompson, William & Fraz, Shamaila & Nederveen, Joshua & Hartenstein, Peyton & Laframboise, Lisa & Manzon, Richard & Somers, Chris & Wilson, Joanna. (2024). Elevated temperature during rearing diminishes swimming and disturbs the metabolism of yellow perch larvae. 10.1101/2024.10.30.621200.

Guan, Y., Grote, K., Schott, J., & Leverett, K. (2022). Prediction of Soil Water Content and Electrical Conductivity Using Random Forest Methods with UAV Multispectral and Ground-Coupled Geophysical Data. Remote Sensing, 14(4), 1023. https://doi.org/10.3390/rs14041023

Chianca, C. G. C., Batista, R. O., De Melo, S. B., Da S Dias, N., & De F a De Matos, M. (2023). Spatial variability of water electrical conductivity in underground dam areas in the semi-arid region of Rio Grande do Norte, Brazil. Revista Caatinga, 36(3), 639–649. https://doi.org/10.1590/1983-21252023v36n317rc

Olaojo, A. A., & Oladunjoye, M. A. (2023). Resolution of spatial variability of apparent electrical conductivity in agricultural soil assessment. Arabian Journal of Geosciences, 16(9). https://doi.org/10.1007/s12517-023-11650-4

Pratiwi, O. A., Nurwidyanto, M. I., & Widada, S. (2024). Analysis of groundwater salinity distribution based on electrical conductivity (EC) and hydrochemical approaches to deep wells in Sayung subdistrict, Demak Central Java. Cognizance Journal of Multidisciplinary Studies, 4(10), 53–61. https://doi.org/10.47760/cognizance.2024.v04i10.004

Erhenhi, O. H., Enwemiwe, V. N., Micheal, O. E., Aghahowa, E., Esiwo, E., Obi, C. C., & Omokoro, F. (2024). Studies on the impact of microplastics in freshwater systems: Biota could be vital indicators in Delta State, Nigeria. UTTAR PRADESH JOURNAL OF ZOOLOGY, 45(14), 34–44. https://doi.org/10.56557/upjoz/2024/v45i144177

Motunrayo, A. M. (2024). Effects of environmental variables on some aquatic biota of Jabi Lake, federal capital territory, Abuja. International Journal of Life Science and Agriculture Research, 03(03), 171–176. https://doi.org/10.55677/ijlsar/v03i3y2024-08

Payler, S. J., Biddle, J. F., Lollar, B. S., Fox-Powell, M. G., Edwards, T., Ngwenya, B. T., Paling, S. M., & Cockell, C. S. (2019). An ionic limit to life in the deep subsurface. Frontiers in Microbiology, 10. https://doi.org/10.3389/fmicb.2019.00426

Mackenzie, T, F., Duxbury, C, A., Byrne, & Howard, R. (2024, December 30). Seawater | Composition, Properties, Distribution, & facts. Encyclopedia Britannica. https://www.britannica.com/science/seawater

Cho, H. J. (2007). Effects of prevailing winds on turbidity of a shallow estuary. International Journal of Environmental Research and Public Health, 4(2), 185–192. https://doi.org/10.3390/ijerph2007040014

Lunt, J., & Smee, D. L. (2019). Turbidity alters estuarine biodiversity and species composition. ICES Journal of Marine Science, 77(1), 379–387. https://doi.org/10.1093/icesjms/fsz214

Nunes, P., Roland, F., Amado, A. M., Da Silva Resende, N., & Cardoso, S. J. (2022). Responses of phytoplanktonic chlorophyll-A composition to inorganic turbidity caused by mine tailings. Frontiers in Environmental Science, 9. https://doi.org/10.3389/fenvs.2021.605838

Sobczyński, T., & Joniak, T. (2013). The Variability and Stability of Water Chemistry in a Deep Temperate Lake: Results of Long-Term Study of Eutrophication. Polish Journal of Environmental Studies, 22(1), 227–237. https://www.pjoes.com/pdf-88973-22832?filename=The%20Variability%20and.pdf

Sojka, M., Choiński, A., Ptak, M., & Siepak, M. (2020). The variability of lake water chemistry in the Bory Tucholskie National Park (Northern Poland). Water, 12(2), 394. https://doi.org/10.3390/w12020394

DeSellas, A. M., Paterson, A. M., Rühland, K. M., & Smol, J. P. (2023). Lake water chemistry and its relationship to shoreline residential development and natural landscape features in Algonquin Provincial Park, Ontario, Canada. Canadian Journal of Fisheries and Aquatic Sciences, 81(2), 137–153. https://doi.org/10.1139/cjfas-2023-0103

Heling, C. L., Stelzer, R. S., Drecktrah, H. G., & Koenigs, R. P. (2018). Spatial variation of benthic invertebrates at the whole-ecosystem scale in a large eutrophic lake. Freshwater Science, 37(3), 605–617. https://doi.org/10.1086/699386

Miller, T. I. (2020). <span>Assessment of water quality using chemometric methods - a case study of Rusałka Lake, NW-Poland</span> Ecologica Montenegrina, 27, 80–89. https://doi.org/10.37828/em.2020.27.10

Sulawesty, F., & Yustiawati, Y. (2021). Spatial variation of phytoplankton and some physico-chemical variables in Lake Tempe, South Sulawesi. IOP Conference Series Earth and Environmental Science, 744(1), 012084. https://doi.org/10.1088/1755-1315/744/1/012084

Biswas, T. K., Karim, F., Kumar, A., Wilkinson, S., Guerschman, J., Rees, G., McInerney, P., Zampatti, B., Sullivan, A., Nyman, P., Sheridan, G. J., & Joehnk, K. (2021). 2019–2020 Bushfire impacts on sediment and contaminant transport following rainfall in the Upper Murray River catchment. Integrated Environmental Assessment and Management, 17(6), 1203–1214. https://doi.org/10.1002/ieam.4492

Abhishek, C., Nikola, G., & Ulrike, G. (2024). Effect of rainfall on water parameters in recreational lakes in Heidelberg, Germany. IgMin Research, 2(2), 121–126. https://doi.org/10.61927/igmin153

Dalkıran, N., & Zünbülgil-Ünsal, B. (2023). Environmental heterogeneity and salinity gradient impacted the alpha and beta diversities of diatom assemblages in a coastal Delta wetland. Water, 15(19), 3414. https://doi.org/10.3390/w15193414

Malashenkov, D. V., Vörös, L., Duisen, A., Dashkova, V., Abilkas, A., Vorobjev, I. A., & Barteneva, N. S. (2024). Assessing Aral Sea residual lake system: impact of fluctuating salinity on phytoplankton communities. bioRxiv (Cold Spring Harbor Laboratory). https://doi.org/10.1101/2024.09.08.611860

Lazarus, O. T., Anaero-Nweke, G. N., Green, A. F., Amachree, D., & Mbambo, S. W. (2023). Physical and chemical variables of freshwater swamp ponds of Engenni, Ahoada-West, Niger Delta, Nigeria. Asian Journal of Fisheries and Aquatic Research, 22(5), 22–30. https://doi.org/10.9734/ajfar/2023/v22i5583

Wiik, E., Bennion, H., Sayer, C. D., Davidson, T. A., McGowan, S., Patmore, I. R., & Clarke, S. J. (2015). Ecological sensitivity of marl lakes to nutrient enrichment: evidence from Hawes Water, UK. Freshwater Biology, 60(11), 2226–2247. https://doi.org/10.1111/fwb.12650