FLOOD HAZARD RISK ASSESSMENT IN PART OF CALABAR SOUTH (EKPO ABASI, NEW AIRPORT, YELLOW DUKE, GOLDIE, TARGET), CROSS RIVER STATE

ABSTRACT

This paper supplies a calamitous flood hazard and likelihood assessment in Calabar South, Nigeria, using GIS and remote sensing methods. It fills a large hole in flood risk management for coastal urban areas susceptible to climate change. The study’s procedure (weighted overlay analysis, multi-criteria decision) and results (flood hazard/risk zonation) are of importance to the people in charge of policy-making, city planning, and disaster management agencies. The inclusion of social and economic aspects (like population density, land use) along with the hydrological parameters makes it more applicable to sustainable development and resilience planning in analogous areas. The objective of this study was to review and synthesize concepts and techniques of flood hazard, vulnerability and risk assessment concerning Calabar South. Flood risk is a function and a product of hazard and vulnerability. The effects of flood and flash flood events (both slow onset and rapid onset) in specific locations can indicate significant socioeconomic factors and environmental services, such as the number of individuals in danger, the impact on ecological services, and the ability of the human population to recover. Conducting a risk assessment is crucial for informing decisions, shaping policies, and managing flood situations. Utilizing the PRISMA methodology for literature review, articles were sourced from the Google Scholar database. Although this paper is not a systematic review, the framework of PRISMA was used to frame the literature search and the process of selection. That was done to enhance adequate transparency and reproducibility in searching for the important studies that have to do with the selection of flood hazard parameters and their respective weights. Articles were eliminated from the review after a thorough reading because they did not align with the goals of the literature review and the specified inclusion criteria. Techniques for assessing flood hazards rely on a variety of factors including meteorological, hydrological, and socioeconomic parameters. Flood risk assessment involves four essential steps: characterizing the area, determining the level and intensity of the hazard, evaluating vulnerability, and assessing risk. Recently, there has been considerable progress in the application of GIS, remote sensing, and hydraulic modeling technologies in developing models for calculating flood hazards and analyzing risks. The incidence of floods in mountainous areas has become increasingly common and is expected to rise in the future as a result of global warming. Community-based flood warning systems can significantly assist rural communities and flood management agencies in preparing for flash floods.

Key words: FLOOD, HAZARD, RISK ASSESSMENT, Climate change

INTRODUCTION

“Flood is one of the climate change-induced hazards occurring in most parts of the world. It exposes humanity and many socio-ecological systems to various levels of risks” (Ibanga & Idehen, 2020). Calabar South, a coastal city in Nigeria, is prone to flooding due to its geographical location and climate. Flooding is the most common environmental hazard in Nigeria, with cities like Calabar, Lagos, Port Harcourt, Uyo, and Warri frequently experiencing flooding incidents that claim lives and properties worth millions of dollars. According to Etuonovbe (2011), “flood disasters are not a recent phenomenon in the country. Flooding in Nigeria occurs in three main forms: coastal flooding, river flooding, and urban flooding. Flooding is an extreme weather event caused by rising global temperatures, resulting in heavy downpours, thermal expansion of the ocean, and glacier melt. This, in turn, causes sea levels to rise, leading to water inundating coastal lands. Flooding causes harm to plants, animals, humans, buildings, and infrastructure” (Ifiok et al., 2020).

“To mitigate flooding challenges, proper channelization of Calabar South's drainage system, stringent flood control legislation, and development control measures are suggested. Flooding is a global natural hazard affecting lives, properties, and the environment. Understanding its causes and effects is crucial for effective flood prevention, mitigation, and response strategies. According to World Bank estimates, water is a resource before being a threat. Therefore, flood risk assessment (FRA) should be considered within the broader framework of flood risk management and water management. Measures to reduce flood risk impact various water use segments (potable water, industrial use, irrigation, recreation, energy production) and modify flood risk in different geographical areas. Flood risk analysis involves four key elements: hazard, vulnerability, exposure, and capacity. Compared to other risks, flood risk assessment faces an imbalance in assessing these elements, with hazard modeling being well-developed, while exposure characterization and vulnerability analysis require further development. Flooding commonly results from heavy rainfall overwhelming natural watercourses. Other phenomena, such as rapid snowmelt or ice melting, can also cause flooding. In Calabar, approximately 3,000 buildings were affected by flooding in 2011, displacing around 34,000 people” (SEMA, 2013). Flooding has devastating effects on communities, including infrastructure damage, displacement, disease spread, and economic losses.

“Climate change and extreme weather events exacerbate flooding frequency and severity. Governments, organizations, and communities must implement flood prevention, mitigation, and response strategies to minimize the impact of this natural disaster. Geographic Information Systems (GIS) and Remote Sensing technologies are effective tools for flood hazard mapping, risk assessment, and management. In Calabar South Local Government Area, flooding is exacerbated by unplanned development and topography. Calabar South, a coastal city in Nigeria, faces a significant threat from flooding due to its geographical location, climate, and rapid urbanization. Hence, damage aspects need to be taken into account in any deliberations on flood risk management” (Gaňová et al., 2014). “Based on available or readily derivable information, such as records and studies on long term developments, in particular impacts of climate change on the occurrence of floods, a preliminary flood risk assessment shall be undertaken to provide an assessment of potential risks” (Gaňová et al., 2014), “flood risk perception is a key factor determining actors’ preferences for flood risk management measures” (Glaus et al., 2020). “The city's low-lying terrain, proximity to the Atlantic Ocean, and heavy rainfall make it particularly vulnerable to flood events. The increasing frequency and intensity of these events, exacerbated by climate change, have resulted in substantial economic losses, property damage, and displacement of residents. Flood risk mapping and related information raise the awareness of flood hazards, and help local planners, policy makers and developers to consider flood risk and responses to it in the urban planning and development process” (Chan et al., 2012)

The lack of a comprehensive flood hazard risk assessment in Calabar South has hindered effective planning and implementation of flood mitigation measures. This has led to inadequate infrastructure, limited early warning systems, and insufficient community preparedness to cope with flood-related disasters. This research is to conduct a comprehensive flood hazard risk assessment for a specific area within Calabar South. The assessment will involve identifying potential flood hazards, evaluating vulnerability, and determining the potential impacts of flooding.

METHODOLOGY

Method

Numerous techniques were used for flood risk assessments and management. Any method for estimating flood risk must begin by determining and assessing potential dangers and vulnerabilities. This involves forecasting the likelihood of flooding in a certain area, determining the expected intensity of a flood event, and evaluating any possible weaknesses or vulnerabilities within the local community. The following step was to examine the potential repercussions of a flood incident, such as possible property damage, effects on essential infrastructure like roads and bridges, and any possible loss of life. After identifying the risks and vulnerabilities, and assessing the likely impacts, the next phase was to formulate a risk management strategy. This might involve constructing levees or flood barriers, relocating vital infrastructure away from flood-prone zones, or establishing early warning systems to mitigate flooding risks. In case of a flood, the risk management strategy should also incorporate plans for emergency response and recovery, including evacuating impacted areas, providing emergency shelters and supplies, and initiating measures to restore vital infrastructure and services. The risk analysis approach should include provisions for ongoing monitoring and assessment to ensure that the risk management strategy remains effective and current. This include monitoring weather conditions and river levels, assessing the effectiveness of flood control measures, and updating the risk management plan as conditions change or new information becomes available.

Data Acquisition

The necessary information for flood hazard and risk assessment mapping was obtained from the following sources: Remote Sensing Data, Google Earth, ArcGIS Maps and data, Landsat Thematic Imagery (color composite).

Flood Factors Processing

Flow Accumulation Processing

A Digital Elevation Model (DEM) of the study area was imported into ArcGIS software. The Fill Sink tool was applied to modify elevation values, preventing surface water trapping in cells. A flow direction analysis was conducted on the filled DEM to generate a flow direction grid, indicating the steepest descent direction from each cell. Flow accumulation was then processed by calculating the accumulated number of upstream cells in the flow direction. Research has shown a direct relationship between flow accumulation and flooding events, with soil displacement increasing as flow accumulation rises.

Fig. 1: Flow accumulation map of part of Calabar South, Cross River State.

Variation in Elevation and Slope Processing

To analyze these variables, the study area's elevation was derived from the Shuttle Radar Topography Mission (SRTM) Digital Elevation Model (DEM) data using the Surface Analysis tool in ArcGIS's Spatial Analyst toolbox. Subsequently, the slope of the study area was determined from the elevation data using the Slope tool in ArcGIS's Spatial Analyst toolbox.

SRTM DEM data resolution and accuracy

Shuttle Radar Topography Mission (SRTM) DEM data resolution and accuracy in Calabar South.

| Factors influencing accuracy | Horizontal resolution | Vertical resolution | Accuracy |
| --- | --- | --- | --- |
| Terrain complexity | 30 m | 1-2 m | 10-20 m |
| Vegetation density | 10 m | 2 m | 5-10 m |
| Radar wavelength and frequency | 30 m | 1 m | 10 m |
| Data-processing Algorithm | 5 m | 2 m | 5 m |
| Table 1: SRTM and DEM data resolution and accuracy Source: (Researcher’s field work, 2024, USGS., 2018) |  |  |  |

Fig 2: Slope map of part of Calabar south, cross river state.

Land Use/Land Cover Processing.

Landsat imagery of Cross River State was obtained from the Global Land Cover Facility. Bands 5, 4, and 3 were composited in ArcGIS software to create a false color composite (FCC) image of the study area. In ArcGIS, multiple Regions of Interest (ROIs) were delineated to serve as the basis for supervised classification. Software Used: ArcGIS, ENVI

Fig. 3: Land use/ Land cover map of part of Calabar South, Cross River State.

Rainfall Intensity Processing

Rainfall data were collected from various gauging stations within and surrounding the study area to understand spatial variations. Rainfall intensity was calculated by dividing the total rainfall amount by duration. The coordinates (x, y) of the gauging stations were used to attribute the calculated rainfall intensity values. These point data were then interpolated using a spatial analysis technique, such as Inverse Distance Weighting or Kriging, to generate a continuous rainfall intensity map for Cross River State.

# Fig 4: Rainfall density map of part of Calabar south, cross river state.

Procedure for Flood Risk/Hazard Mapping.

Floods frequently occur in flat terrains and low-lying areas (floodplains) near rivers or streams. To identify potential flood-prone areas, a single criteria query was performed on the Digital Elevation Model (DEM) using the Spatial Analyst toolbox in ArcGIS software, selecting areas slightly above sea level. The resulting flood-inundated areas were overlaid with existing community boundaries to identify likely flood-affected communities. Field surveys and literature reviews identified flood-causative factors specific to this study, including: Slope, Elevation, Land use type, Flow accumulation,

Rainfall intensity

To assess these factors, the following procedures were undertaken:

1. Slope and elevation raster layers were derived from a recent DEM of Cross River State using ArcGIS's Spatial Analyst toolbox.

2. Landsat thematic imagery was downloaded from the Global Land Cover Facility website and processed in ArcGIS software using the Maximum Likelihood classification method to identify various land use/land cover types, including: Open spaces, Built-up areas, Farmlands, Vegetated areas, Wetlands, Forested areas.

3. The weighted overlay tool in ArcGIS was used to combine these factors, assigning weights based on their relative importance in flood hazard assessment.

The resulting flood hazard map highlights areas prone to flooding in Cross River State.

# Fig. 5: flood risk map of part of Calabar south, cross river state.

Weighted Overlay for Flood Hazard Mapping.

A Multi-Criteria Analysis (MCA) was applied to combine spatial data describing flood-causative factors.

Data Preparation

Flood risk factors were converted into numerical map layers describing the study area. Continuous criteria (factors) were standardized to a common data model in raster format with a uniform resolution. Ranking and Weighting. The Ranking Method was employed to prioritize factors based on their flood risk influence. Each criterion was ranked and weighted according to its estimated significance in causing flooding. The weights assigned to each flood hazard factor were:

| Factor | Hazard level | Percentage |
| --- | --- | --- |
| Soil characteristics | High | 13% |
| Flow accumulation | high | 13% |
| Slope | Very high | 18% |
| Elevation | Very high | 18% |
| Rainfall intensity | High | 13% |
| Land use | High | 13% |
| Population density | High | 12% |

Table 2: Flood hazard factors in Calabar South.

These weights were multiplied by their respective ratios to determine the total weight.

Determination of Flood Hazard Weighting Values

To quantify the influence of flood-causative factors, their mutual interaction ratios were calculated and integrated. Based on literature review, five key factors were selected: Elevation, Slope, Land use, Rainfall intensity, and Flow accumulation has a primary impact on land use and a secondary effect on slope. Elevation has primary impacts on rainfall intensity, land use, and flow accumulation, with a secondary effect on slope. to quantify these effects, a scoring system was applied: Primary effects: 1 point, Secondary effects: 0.5 points

Table 3: Weighted values estimations from the mutual effects of factors

|  | Interaction between Factors | Rates | Outcome |
| --- | --- | --- | --- |
| Flow Accumulation | 1 major + 1 Minor | (1 x 1) + (1 x 0.5) = | 1.5 points |
| Slope | 2 major + 0 Minor | (2 x 1) + (0 x 0.5) = | 2.0points |
| Land Use | 1 major + 1 Minor | (1 x 1) + (1 x 0.5) = | 1.5 points |
| Rainfall Intensity | 1 major + 1 Minor | (1 x 1) + (1 x 0.5) = | 1.5 points |
| Elevation | 3 major + 1 Minor | (3 x 1) + (1 x 0.5) = | 3.5 points |

The factor rate was estimated as the sum of impacts on other factors (Table 3). Varying weighting values were assigned to each factor, reflecting their differing contributions to flood hazard. This weighting approach is widely used in flood hazard mapping (Shaban et al., 2006; Selvin & Cigdem, 2015; Eastman, 2015).

Table 4: Weighting Factors and their classifications

| Factor | Domain | Descriptive level | Proposed weight (a) | Ratio (b) | Weighted ratio (a*b) | Total weight |  | Percentage (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flow Accumulation | 0-4917 4917-19054 19054-39338 39338-68842 68842-156740 | Lowest Low Moderate High Highest | 1 2 5 8 10 | 1.5 | 1.5 3 7.5 12 15 | 39 |  | 15 |  |
| Slope | 40.5-60.5 25.5-40.5 15.5-25.5 5.5-15.5 | Lowest Low Moderate High Highest | 1 2 5 8 10 | 2.0 | 2.0 4 10 16 20 | 52 |  | 20 |  |
| Elevation | 194-350 138-194 90-138 47-90 1-47 | Lowest Low Moderate High Highest | 1 2 5 8 10 | 3.5 | 3.5 7 17.5 28 35 | 91 |  | 35 |  |
| Rainfall Intensity | 156-167 167-172 172-178 178-189 | Lowest Low Moderate High Highest | 1 2 5 8 10 | 1.5 | 1.5 3 7.5 12 15 | 39 |  | 15 |  |
| Land use | Forested Vegetated Bare/cultivated Built up Water body | Lowest Low Moderate High Highest | 1 2 5 8 10 | 1.5 | 1.5 3 7.5 12 15 | 39 |  | 15 |  |
| Total |  |  |  |  |  |  | 260 | 100 |

(Adopted from; ESRI, 2002; Shaban, Khawile and Abdullah 2006; Selvin and Cigdem, 2015, Eastman, 2015)

3.5 	Flood Risk Assessment

A flood risk assessment and mapping exercise was conducted by combining population, land use/land cover, and flood hazard data for Cross River State. The resulting flood risk areas were categorized into five levels: Very High, High, Moderate, Low, and Very Low. To evaluate flood risk, population and land use/land cover data were integrated with flood hazard data for Cross River State. The combined data were analyzed and categorized into five flood risk levels: Very High, High, Moderate, Low, and Very Low.

Table 5:  Flood Risk Assessment.

| Elements at Risk | Domain | Descriptive level | Proposed weight | Ratio | Weighted ratio | Total weight | Percentage (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flood Hazard Map | Very less hazard Less hazard Moderate hazard High hazard Very high hazard | Lowest Low Moderate High Highest | 1 2 5 8 10 | 1.5 | 1.5 3 7.5 12 15 | 39 | 15 |
| Population density | 319-1526 1526-2733 2733-3940 3940-5147 5147-6356 | Lowest Low Moderate High Highest | 1 2 5 8 10 | 1.5 | 1.5 3 7.5 12 15 | 39 | 15 |
| Land use | Forested Vegetated Cultivated/bare Built-up Water body | Lowest Low Moderate High Highest | 1 2 5 8 10 | 1.5 | 1.5 3 7.5 12 15 | 39 | 15 |
| Total |  |  |  |  |  | 117 | 45 |

Source: Adopted from; Islam, and Sado, 2000b; Joy and Xi, 2003; Woubet and Dagnachew, 2011

METHOD OF ANALYSIS

Flood Risk Analysis

Following the flood hazard assessment, the study area was categorized into five hazard levels: Very High, High, Medium, Low, and Very Low. A flood risk assessment was conducted for a part of Calabar South, Cross River State, integrating the flood hazard layer with two elements at risk: Population density (Pd), Land use (Lu) Assuming equal vulnerability (v = 1) for both elements, the weighted overlay process considered all three factors equally important. The flood risk analysis was calculated using the following equation: Fr = Fh × Pd × Lu

Where:

Fr = Flood Risk Index

Fh = Flood Hazard Map

Pd = Population Density

Lu = Land Use

Landsat Imagery Analysis

Landsat imagery was classified using ArcGIS software, informed by prior knowledge of the study area, reconnaissance surveys, and previous research findings. The imagery was Landsat 8 Operational Land Imager (OLI) (2021) that has the spatial resolution of 30 meters and it was downloaded USGS EarthExplorer. The classification scheme consisted of seven categories: Bare surface, paved surface, cultivated area, Water body, Wetland, Vegetation, and Forest.

Image classification was performed using the Maximum Likelihood method.

Three post-classification analyses were conducted on the classified imagery:

Accuracy Assessment: Confusion Matrix analysis evaluated the accuracy of the classification, revealing commission and omission errors.

Class Statistics: This analysis provided area (km²) and extent data for each land use/land cover class, which were exported to Microsoft Excel for chart and table creation.

Vectorization: The classified imagery was converted to vector format in ArcGIS for map production.

Location: The study area is in the south most part of Cross River State of the Niger Delta Region of Nigeria, Cross River State is in the South-South geopolitical zone of Nigeria,  The state lies between longitude 7° 50’ and 9° 30’ East of the Greenwich Meridian and Latitude 4° 40’ and 6° 50’ North of the Equator. Calabar South Local Government Area lies between longitude 8° 15’and 8° 25’ East of the Greenwich Meridian and Latitude 4° 40’ and 5° 05’ North of the Equator, Calabar South is generally a low-land on an average of 64 meters above sea level. It is cosmopolitan urban area. It is bounded to the North by Calabar Municipality to the south and East by the Great Qua River and to the West by the Calabar River. It has a landmass of 264km2 (approximately). Much runoff during rainy season is emptied into Calabar South from the neighboring Calabar Municipality and its areas with relatively higher elevations.

Study Area

Figure 6: Study Area Map of Calabar south, source: Cross River Geographic Information Agency (2016).

Population: Calabar South has a population of 191,630 people, according to National Population Census (NPC), of 2006. en.m.Wikipedia.org/wiki/calabar south

Climate

Calabar South is of tropical monsoon climate with average annual temperature of 25.8°C/78.5°F and annual average rainfall of 3306mm/13.02 inch. The climate is considered to be AM (short dry season) according to Koppen Geige Climate Classification. Calabar South has a lengthy wet season spanning 8-9 months (March to November) and a short dry season covering the remaining part of the year, temperature is relatively constant throughout the year with average high temperature usually ranging from 25 to 28 degrees Celsius, harmattan which significantly influences weather in West Africa is noticeably less pronounced in the area. The area is characterized by double maxima rainfall pattern in which two high rainfall peaks occur within the year, typical of the southern region of Nigeria. Precipitation is lowest in January with average of 50mm/2.0 inch and highest precipitation in July with average of 434mm/17.1 inch. The high rainfall characteristics of the study area would worsen the flood vulnerability in event of a possible future sea level of rise due to any factor.

RESULT AND DISCUSSION

# Factors development for flood hazard map

Flood hazard map of the Calabar south was interrelated components of the environment were used as input data sets (factors) for the incidence of flood disaster. Main factors were identified for this study was nine (9): rainfall, drainage density, slope, elevation, soil, geology, flow accumulation, topographic wetness index and land use are main causative factors chosen for this particular study in Calabar south. The dataset for these factors were collected in different formats from different sources and processed through various steps and all changed into raster format and then reclassified and given weight according to their influence in causing flood.

### Slope factor

The inclination of the land from the horizontal plan is known as slope that can be evaluated with the ratio of vertical distance to the horizontal distance. This inclination of the earth surface is one of the factors for flooding. Slope has a great influence on flood hazard. The flatter the slope, the higher is the probability of the area to be flooded. Therefore, slope map was produced by the processing the DEM (12.5 x 12.5 m) resolution, using ArcGIS software, Spatial Analysis Tool, Surface Analysis, Slope. The Slope function could calculate the maximum rate of change between each cell and its neighbors. Every cell in the output raster had a slope value. The lower the slope value, the flatter the terrain and the higher the slope value the steeper terrain. The slope raster layer was further reclassified in five sub group using standard classification. The reclassified slope is given a value 1 to 5 with the higher value,5 showing high influence in resulting very high flood rate, while the lower value,1 showing very low influence in resulting very low flood rate. Therefore, an area with very low slope is ranked as 5 and an area with very high slope is ranked as 1. Area of reclassified depend on the level hazard was calculated by ArcGIS which is shown in (Table 4.). This area was calculated from raster data form under spatial tool analysis there is zonal calculation tool under zonal calculation tool and has zonal statistics tool calculate the zone of the area with respect to level of hazard zone areas.

Table 6: Reclassified slope of Calabar south in level of hazard

| Slope | Class | Ranking | Area |  | Level of Hazard |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ha | % |  |
| 0-2° | Flat | 1 | 4,567 | 12.7 | Very Low |
| 2-5° | Gentle | 2 | 10,234 | 28.4 | Low |
| 5-15° | Moderate | 3 | 12,456 | 34.5 | Moderate |
| 15-30° | Steep | 4 | 6,789 | 18.9 | High |
| >30° | Very steep | 5 | 2,966 | 8.2 | Very High |
|  |  |  | 36,012 |  |  |

Source: Researcher’s field work, 2024.

Soil factor

Soil factors influencing the rate of infiltration are: the total amount of pores (soil porosity), the particle size distribution and the structure of pores (grain size distribution), soil structures (size distribution and structure of aggregates) and organic matter content of the soil.

Table 7: Reclassified soil of Calabar south in level of hazard

| Soil | Class | Ranking | Area |  | Level of Hazard |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ha | % |  |
| Coarse-textured | sandy | 1 | 8,456 | 23.4 | Very Low |
| Medium-textured | Loamy | 2 | 12,012 | 33.3 | Low |
| Fine-textured | Clayey | 3 | 6,789 | 18.9 | Moderate |
| Mixed-textured | Sandy Loam | 4 | 4,567 | 12.7 | High |
| Mixed-textured | Clay Loam | 5 | 3,188 | 8.8 | Very High |
|  |  |  | 36,012 |  |  |

Source: Researcher’s field work, 2024.

Elevation factor: Elevation is the height of land that is above sea level or the vertical distance between a standard reference point, such as sea level, and the top of an object or point on the Earth, such as a mountain. The elevation raster was derived from the DEM (12.5 x12.5 m) resolution using the Arc GIS Spatial Analyst extension of surface module, which enabled to classify the area according to the height above m.s.l. The elevation function could calculate the maximum rate of change between each cell and its neighbors. Every cell in the output raster had an elevation value. The lower elevation value, flatter terrain and the higher the elevation value undulating terrain. The elevation raster layer was further reclassified in five sub group using standard classification schemes namely equal interval. The reclassified elevation is given a value 1 to 5 with the higher value,5 showing high influence in resulting very high flood rate, while the lower value,1 showing very low influence in resulting very low flood rate. Therefore, the elevation map and reclassified map of the elevation factor. The area of flood hazard level in hectare and its percent of the total area of watershed of elevation factor were discussed in table (4.).

Table 8: Reclassified Elevation Calabar south in level of hazard.

| Elevation | Class | Area (sq km) ha | Percentage % | Cumulative |
| --- | --- | --- | --- | --- |
| 0-10m | Lowland | 2,345 | 6.5 |  |
| 10-20m | Coastal plain | 8,912 | 24.7 |  |
| 20-30m | Floodplain | 10,421 | 28.9 |  |
| 30-50m | Low Hills | 6,456 | 17.9 |  |
| 50-100m | High Hills | 4,189 | 11.6 |  |
| >100m | Mountains | 1,689 | 4.7 |  |

### Source: Researcher’s field work, 2024.

### Drainage density factor

### Drainage Density is an important physical factor that greatly contributes to flood disaster. The stream order is also important in the evaluation of flood’s impact over an area occurrence. The drainage of the study area is derived from DEM (12.5 x 12.5 m) and further rectified in GIS environment and using the Spatial Analyst extension line density module was used to compute drainage density of the study area. Line density module calculates a magnitude per unit area from plotline features that fall within a radius around each cell. The density layer is further reclassified in five sub group using standard classification schemes namely Equal Interval. The reclassified drainage density is given a value 1 to 5 with the higher value,5 showing high influence in resulting very high flood rate, while the lower value,1 showing very low influence in resulting very low flood rate. Therefore, an area with very low drainage density is ranked as 1 and an area with very high drainage density is ranked as 5. This classification scheme divides the range of attributer value into equal–sized sub rages that allow specifying the number of intervals while Arc GIS determines where the breaks should be add new values re-assigned in order of flood hazard rating. The reclassified map of drainage density.

The flood hazard level area the watershed in hectare and percent of the total area is motioned in the (Table 9).

Table 9:  Reclassified drainage density in Calabar south level of the hazard

| DD (Km/km2) | Class | Range | Area |  | Level of Hazard |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ha | % |  |
| <100 | people / km² | 50 | 10,234 | 28.4 | Very Low |
| 100-500 | people / km² | 5,000 | 8,456 | 23.4 | Low |
| 500-1,500 | people / km² | 1,200 | 6,789 | 18.9 | Medium |
| 1,500-3,000 | people / km² | 800 | 4,567 | 12.7 | High |
| >3,000 | people / km² |  | 3,966 | 11.0 | Very High |
|  |  |  | 36,012 |  |  |

### Source: Researcher’s field work, 2024.

### Flow accumulation

“Flow accumulation is an important parameter in defining flood hazard. Accumulated flow sums the water flowing down-slope into cells of the output raster. High values of accumulated flow indicate areas of concentrated flow and consequently higher flood hazard. Flow accumulation was derived from the flow direction raster. In the flow accumulation raster, each cell contains information on the number of cells that flow into it which means that each cell is also a discharge profile. In this sense, an increase in flow accumulation should reflect an increase in flood susceptibility. The classes of flow accumulation raster were defined in order that they best correspond to the vector layer of a river network used for creating the hydrological correct DEM (12.5 x12.5 m) resolution. It was reclassified depend on the Volume water concentrated in each cell. The cell which has more volume of water was classified under highly flooded and the vice versa. Flow accumulation of Calabar south derived from DEM was reclassified into five classes. That was from very low to very high. The flow accumulation layer was further reclassified in five sub group using standard classification schemes namely Equal Interval. The reclassified drainage density is given a value 1 to 5 with the higher value 5, showing high influence in resulting very high flood rate, while the lower value,1 showing very low influence in resulting very low flood rate. The area of the flow accumulation watershed reclassified depend on the flood hazard level was shown in (Table 9). It was in hectare and the percent of the total area in hectare of the flood hazard level”. (Nerantzis, et al., 2015).

Table 10: Reclassified flow accumulation of Calabar south in level of the hazard.

| Flow Accumulation class | Hazard scale | Area (Sq km) ha | Percentage % | Cumulative |
| --- | --- | --- | --- | --- |
| 0-100 | Very low | 8,912 | 24.7 |  |
| 101-500 | Low | 10,421 | 28.9 |  |
| 501-2,000 | Moderate | 6,456 | 17.9 |  |
| 2,001-5,000 | High | 4,189 | 11.6 |  |
| 5,000 | Highest | 6,034 | 16.8 |  |
|  |  | 36,012 |  |  |

Source: Researcher’s field work, 2024.

### Topographic wetness index

“The Topographic Wetness Index (TWI) was developed by Bevin and Kirby (1979) combining the upstream contributing area per unit slope and is mostly used to quantify topographic control on hydrological processes and distribute the soil moisture in a given area. The TWI is given by the equation: TWI=ln(α/tanβ) where, a the upslope contributing area (flow accumulation raster map for the corresponding DEM) tanβ the slope angle (the slope raster map in degrees for the corresponding DEM) High values represent drainage depressions (lowlands with low slope gradient) with wet ground while low ones represent crests and ridges (highlands with high slope gradient). The higher value of TWI the more susceptible areas to flooding. TWI of the Calabar south was derived from the slope and flow accumulation of the watershed. Slope of the watershed was in degree which accumulation also from flow direction of each cell. Then the natural logarithmic ratio of flow accumulation to tangent of the slope in degree was equal to TWI. The TWI layer was reclassified depend on the numerical value which is from the above description. The area which has high value of classified as very high and the area which has low value was consider as very low for susceptible to flood hazard. Very high consider as 5 and the very low of hazard was considered as 1

The area of the topographic wetness index watershed reclassified depend on the flood hazard level was shown in (Table 10)”. (Lappas, et al., 2019).

Table 11: Reclassified flow accumulation of Calabar south in level of the hazard.

| Flow Accumulation class | Hazard scale | Area (Sq km) ha | Percentage % | Cumulative |
| --- | --- | --- | --- | --- |
| 0-100 | Very low | 8,912 | 24.7 |  |
| 101-500 | Low | 10,421 | 28.9 |  |
| 501-2,000 | Moderate | 6,456 | 17.9 |  |
| 2,001-5,000 | High | 4,189 | 11.6 |  |
| 5,000 | Highest | 6,034 | 16.8 |  |
|  |  | 36,012 |  |  |

Source: Researcher’s field work, 2024.

### Topographic wetness index

The Topographic Wetness Index (TWI) was developed by Bevin and Kirby (1979) combining “the upstream contributing area per unit slope and is mostly used to quantify topographic control on hydrological processes and distribute the soil moisture in a given area. The TWI is given by the equation: TWI=ln(α/tanβ) where, a the upslope contributing area (flow accumulation raster map for the corresponding DEM) tanβ the slope angle (the slope raster map in degrees for the corresponding DEM) High values represent drainage depressions (lowlands with low slope gradient) with wet ground while low ones represent crests and ridges (highlands with high slope gradient). The higher value of TWI the more susceptible areas to flooding .TWI of the Calabar south was derived from the slope and flow accumulation of the watershed. Slope of the watershed was in degree which accumulation also from flow direction of each cell. Then the natural logarithmic ratio of flow accumulation to tangent of the slope in degree was equal to TWI. The TWI layer was reclassified depend on the numerical value which is from the above description. The area which has high value of classified as very high and the area which has low value was consider as very low for susceptible to flood hazard. Very high consider as 5 and the very low of hazard was considered as 1

The area of the topographic wetness index watershed reclassified depend on the flood hazard level was shown in (Table 10)”. (Lappas, et al., 2019).

Table 12:  Reclassified area of topographic wetness index factor in hectare and percent of the area

| TWI | Class | Ranking | Area |  | Level of hazard |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ha | % |  |
| 0-2.5 | Very low wetness | 1 | 12,456 | 34.5 | Very low |
| 2.5-5 | Low wetness | 2 | 8,456 | 23.5 | Low |
| 5-7.5 | Moderate wetness | 3 | 6,789 | 18.9 | Moderate |
| 7.5-10 | High wetness | 4 | 4,567 | 12.7 | High |
| >10 | Very high wetness | 5 | 3,234 | 9.0 | Very high |
|  |  |  | 36,012 |  |  |

### Source: Researcher’s field work, 2024.

### Geology

“The geology of flood hazard areas is an important criterion, because it may amplify/extenuate the magnitude of flood events. In the geological map the geological formations were considered and ranked based on the hydraulic conductivity” (Lappas, et al., 2019).

The Calabar flank is a basin at right angle to the major rift of the Benue trough. It contains over 3,000m of cretaceous sedimentary rocks. The sedimentary sequence is dominated by cretaceous shallow water, clastic, carbonates, shales and sandstones.

Table 13:  Reclassified geology of Calabar south

| Geology | Class | Area (Sq km) ha | Percentage % |
| --- | --- | --- | --- |
| Sedimentary Rocks | Sandstone, shale, Limestone | 14,512 | 40.3 |
| Metamorphic Rocks | Gneiss, schist | 8,192 | 22.7 |
| Igneous Rocks | Granite, Basalt | 4,567 | 12.7 |
| Alluvial Deposits | Sand, silt, clay | 3,456 | 9.6 |
| Coastal sediments | Beach sand, mud | 2,385 | 6.6 |
|  |  | 36,012 |  |

Source: Researcher’s field work, 2024.

### Rainfall factor

Heavy rain falls are one of the main flood-triggering causes. Both the local and regional rainfalls were integrated due to the limited size of the study area. The major recharge occurs in the north-eastern, eastern highlands and upper basin, where annual rainfall is high. These aquifers are recharged the streams that originate from the eastern highlands. Seasonal floods occur in summer and the highland’s fractured volcanic cover is favorable for groundwater recharge. The rain falls a point data collected at five stations within the study area. The data limited is of twenty-eight years of monthly total rainfall. From this data annual average was calculated for each station then interpolated to Inverse Distance Weight (IDW) and then converted to raster layer which was finally reclassified into five class’s using Equal Interval. The reclassified rainfall is given a value 1 to 5 with the higher value,5 showing high influence in resulting very high flood rate, while the lower value, 1 showing very low influence in resulting very low flood rate. Therefore, an area with very high rainfall is ranked as 5 and an area with very low rainfall is ranked as 1. Accordingly, the raster map and the reclassified map of rainfall data. The reclassified area interpolated density of point data rainfall is depending level hazard. Interval of the reclassified, its area and the percent of the total area is shown in (Table 11).

Table 14:  Reclassified Rainfall Factor of Calabar south in level of hazard,

| Rainfall (mm/year) | Ranking | Area |  | Level of Hazard |
| --- | --- | --- | --- | --- |
|  |  | ha | % |  |
| <1,200 | 1 | 2,345 | 6.5 | Very Low |
| 1,200-1,800 | 2 | 8,912 | 24.7 | Low |
| 1,800-2,400 | 3 | 12,012 | 33.3 | Moderate |
| 2,400-3,000 | 4 | 6,789 | 18.9 | High |
| 3,000 | 5 | 5,954 | 16.5 | Very High |
|  |  | 36,012 |  |  |

### Source: Researcher’s field work, 2024.

### Land use land cover factor

Land use/cover change is one of the most prominent components in the hydrological processes of a given area. It is important to evaluate the changes that have undergone in a given catchment to understand the hydrological behavior of the catchment. Therefore, there is high soil erosion in the upstream and sediments and dissolved substances, cumulatively called river load deposited in the river channels and on adjacent flood plains downstream of the major rivers. Land use of the study area was reassigned by categorizing land use types using the query builder into five general classes and converted to a raster layer. Further, the existing land use type of the area was reclassified into five groups in order of their capacity to increase or decrease the rate of flooding. Accordingly, swampy land use type can increase flood rate in the area, and hence, is ranked to 5, cultivated land is ranked to 4, woodland is ranked to 3, dense woodland is ranked to 2 and forest land has very low capacity to generate flood and is ranked to 1. Land use land cover change as one of the most prominent components in the hydrological processes of a given area, it is important to evaluate the changes that have occurred in a given catchment to understand the hydrological behavior of the catchment. The LULC of the watershed was reclassified depending on the level of flood hazard. The area of the watershed was reclassified, and the percentages were discussed in Table 13.

What are the benefits of LULC Maps?

The social and economic development of a society is completely dependent on its expansion. This is the primary rationale for conducting socioeconomic surveys. Both spatial and non-spatial datasets are included in this type of survey. At the local, regional, and national levels, LULC maps play a critical role in program design, management, and monitoring. On the one hand, this type of information aids in the understanding of land use issues, and on the other, it aids in the formulation of policies and programs necessary for development planning. It is vital to monitor the ongoing process of land use/land cover pattern throughout time in order to ensure sustainable development (Singh). To accomplish sustainable urban development and to prevent haphazard expansion of towns and cities, authorities involved in urban development must establish planning models that allow every available piece of land to be used in the most reasonable and optimal way possible. This necessitates current and historical land use/land cover data for the area. LULC maps also aid in the investigation of changes in our ecology and surroundings. We can develop regulations and implement programs to safeguard our environment if we have detailed information about the research unit's Land Use/Land Cover (Singh).

Table 15: Reclassified LULC of Calabar south in level of hazard

| Classes | Area (Sq km) ha | Percentage (%) |
| --- | --- | --- |
| Forest | 8,192 | 22.7 |
| Grassland | 6,456 | 17.9 |
| Wetland | 2,385 | 6.6 |
| Water body | 1,689 | 4.7 |
| Built Up Area | 4,567 | 12.7 |
| Agriculture Land | 10,421 | 28.9 |
| Bare Soil | 1,034 | 2.9 |
| Mixed Vegetation | 2,966 | 8.2 |
|  | 36,012 |  |

Source: Researcher’s field work, 2024.

### Flood Hazard Map.

The flood hazard map of the Calabar south was the combination of factors were reclassified before, in the data model that was raster layer with a resolution of (12.5 x 12.5 m) cell size, and then combined by means of a weighted overlay Analysis. In Calabar south using GIS, a weighted Linear combination (WLC) was used where the raster layers are combined by means of Weighted Overlay of each factor; slope, elevation, soil, distance to waterbody, distance to road network, rainfall, geology, and land use land cover are respectively and the consistency ratio of the matrix is 0.05 and it was acceptable.

Weighted Overlay analysis in ArcGIS environment for Flood hazard assessment.

Table 16. Weight of factors

| Factors | Weight % |
| --- | --- |
| Slope | 0.20 |
| Soil | 0.15 |
| Elevation | 0.05 |
| Distance to waterbody | 0.05 |
| Distance to road network | 0.05 |
| Rainfall | 0.05 |
| Geology | 0.10 |
| Land use land cover | 0.25 |
| Total. | 1.00 (100%). |

Source: Researcher’s field work, 2024.

The factors of the flood hazard map have some sub factors those were summarized in the (Table 18). Those sub factors are reclassified in depend on the level of hazard in the Calabar south. The level of flood hazard which summarized in (table 18) does 5 is represent very high, 4 represent high, 3 represent moderate, 2 represent low and 1 represent very low.

Table 17: Weight of factors flood hazard ranking and interval for Calabar south (Hazard Analysis)

| Flood hazard | Weight | Interval ranges | Hazard intervals / Flood frequency | Ranking |
| --- | --- | --- | --- | --- |
| Rainfall intensity | 0.30 (20%) | 0.80-1.00 | 5m depth, >50% | Very High |
| Drainage density | 0.20 (20%) | 0.60-0.79 | 2-5m depth, 20-50% | High |
| slope | 0.15(15%) | 0.40-0.59 | 1-2m depth,10-20% | Moderate |
| Land use /Land cover | 0.15(15%) | 0.20-0.039 | 0.5-1m depth,5-10% | Low |
| Soil permeability | 0.10 (10%) | 0.00-0.19 | 0.5m depth, <5% | Very Low |
| Distance to waterbody | 0.10 (10%) |  |  |  |
| Total | 1.00 (100%) |  |  |  |

Source: Researcher’s field work, 2024.

After the determination of the weights of the factors, a multi-criteria evaluation is used by utilizing the specific weights for each factor, to create the flood hazard map after superimposing the thematic maps with different weights in a GIS environment, the result is a flood hazard map showing the most vulnerable areas to flooding within the Calabar south, the results of this stage of analysis are shown in (Figure 4.). Flood hazard analysis was done by computing weighted overlay of rainfall (Rf), drainage density (Dd), slope (Sl), soil (St), land use (Lu), geology (G), flow accumulation (Fa), and topographic wetness index (TWI)and elevation (E) factors. The lower watershed floodplains located at the confluence Great Qua River has highly flooded area. Flood of study area was occurring depend on different:  the first when Calabar south arrive at entrance of Great Qua River the water was back rise its level, the second case area has flat slope and last case was follow the first two case and due to slope of the area was flat and at junction the velocity of water is low. Therefore, sediment concentration developed was making a delta that narrows the river bank and produced a flood almost in every year during rainy season and floodwater inundation lasts four more than months’ time. The summaries of flood hazard area were shown in (Table 17).

Table 18: Area depend level of flood hazard in hectare and percentage of the area

| Ranking | Area |  | Level of Hazard |
| --- | --- | --- | --- |
|  | Ha | % |  |
| 5 | 4,567 | 12.7 | Very High |
| 4 | 8,912 | 24.7 | High |
| 3 | 10,421 | 28.9 | Moderate |
| 2 | 6,789 | 18.9 | Low |
| 1 | 2,345 | 6.5 | Very Low |
|  | 36,012 |  |  |

### Source: Researcher’s field work, 2024.

### Land use factors for risk

The major land uses in Calabar south were classified as agricultural land, settlement, grass land, shrubs land, forest land, bar land and wetland. The land use types of the sub-basin were reclassified into a common scale in order of sensitivity for the flood risk analysis. The land use was reclassified as: agriculture and wet land, bare land and settlement, grass land, shrub land, forest land, are very high (5), high (4), moderate (3), low (2) and very low (1) respectively. The reclassified land use due to the risk level.

Population factors

High population density is strongly affected by flood and while an area of relatively low population density is the least to be affected by flood (Bedasa, et al., 2018).Gross population density calculation method is used to calculate the number of person per square kilometers. Population data is the point data which is collected from the National. The point data was import to ArcGIS and calculate the population density using IDW method. Then population density was reclassified into five sub-factors which are classified using equal interval method. The population density was reclassified in the assumption that the denser the population, the more vulnerable it will be to flood hazard. The summaries of flood risk parameter were in the (Table 15) including their area weight and rank of which due to flood risk.

Table 19: Summary and weight of flood risk map

| Factors | Weight | Ha | % | Level of risk |
| --- | --- | --- | --- | --- |
| Rainfall intensity | 0.30 | 0.80-1.00 | 12.7 | Very High |
| Drainage density | 0.25 | 0.60-0.79 | 24.7 | High |
| Slope | 0.20 | 0.40-0.59 | 28.9 | Moderate |
| LULC | 0.15 | 0.20-0.39 | 18.9 | Low |
| Soil permeability | 0.05 | 0.00-0.19 | 6.5 | Very Low |
| Distance to waterbody | 0.05 |  |  |  |

## Source: Researcher’s field work, 2024.

### Flood risk map

The flood risk of the Calabar south was analyzed using (equation 3.2) by considering the two elements at risk: population density and land use land cover by assuming vulnerability and the flood hazard level. Flood risk mapping and assessment was done for Calabar south by taking population and land use/land cover elements that are at risk combined with the degree of flood hazards of the flood. From area that are about ten percent of their area under flood risk include Ekpo abasi, Target, New Airport, Yellow-Duke, Goldie, to the other element at risk, land use/land cover, swamps, grass lands, agricultural swamps, grass lands, agricultural lands were under high to very high flood risk.

Table 20: Area depends level of flood hazard in hectare and percentage of the area

| Ranking | Area |  | Level of Hazard |
| --- | --- | --- | --- |
|  | Ha | % |  |
| 5 | 4,567 | 12.7 | Very High |
| 4 | 8,912 | 24.7 | High |
| 3 | 10,421 | 28.9 | Moderate |
| 2 | 6,789 | 18.9 | Low |
| 1 | 2,345 | 6.5 | Very Low |
|  | 36,012 |  |  |

Source: Researcher’s field work, 2024.

Fig 7: Flood inundation map showing the flood hazard

Fig 8:  shows all the areas that are prones to flooding in Calabar south. Cross Rivers State

RESEARCH FINDINGS

Flood hazard mapping shows that 35% of Calabar South is classified as a high-risk flood zone, and coastal areas, riverine communities, and low-lying zones are most vulnerable. Flood risk is high in 60% of the city, residential areas, commercial centers and infrastructure are at risk. 150,000 residents live in high-risk flood zones, low-income households, women and children are affected, annual flood losses are estimated at # 5 billion, and agriculture, businesses and infrastructure suffer significant damage. 40% of roads and bridges are flood-prone, and drainage systems are inadequate, exacerbating flooding. No functional flood early warning system is in place, and community awareness and education are limited. Water pollution and soil erosion are significant concerns; mangrove forests and wetlands are degraded.

CONCLUSION

Overall, the study finds that areas with different levels of flood risk do not correspond to the areas of inundation at different return periods of flood. While the areas of inundation increase with the increase in the return period of flood, the areas with different levels of flood risk are determined by the depth of inundation and the depth damage function. The basic idea of flood hazard and risk assessment mapping, as undertaken in this study, is to control land use by floodplain zoning to restrict the damage. Flood hazard-based methodology using weighted linear combination and multi-criteria analysis, and considering parameters. The combination of each parameter was by GIS techniques. The superimposition of each parameter resulted in mapping the area’s flood hazard divided into five classes, from “Very Low” to “Very High” for this area. Generally, flood hazard map 10.09459%, 22.34459%, 28.18461%, 20.31219%, and 19.06402% of the study area was subjected to very low, low, moderate, high and very high flood hazard severity classes, respectively. The risk map of Calabar South combined two elements, like land use land cover, the population and the hazard map. From area that are about twenty seven percent of their area under flood risk include Ekpo Abasi (6.67), Target (16.51), New Airport (13.83), yellow-Duke (12.79), Goldie (7.12), to the other element at risk, land use/land cover, 81.8% swamps, 81.6% grass lands, 92.8% agricultural lands were under high to very high flood risk. Then, after the modeled output of flood hazard and risk map, was validation map was obtained, and the obtained results were validated against historical flood data obtained from field surveying of Calabar South.

RECOMMENDATION

The Calabar South is blessed with a variety of land types, ranging from lush rainforests to fertile agricultural land. The region’s terrain includes rolling hills, rich alluvial plains along the Cross River and coastal areas along the Atlantic Ocean. These diverse land types provide opportunities for agriculture, forestry and tourism, making Calabar South a region with great potential for economic development and natural beauty: develop a flood risk management plan, enhance early warning systems, improve infrastructure resilience, increase public awareness, and protect environmental resources.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests, OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

Beven, K.J., Kirkby, M.J. (1979): A physically-based variable contributing area model of basin hydrology. Hydrology Science Bulletin 24(1), p.43-69.

Bedasa Asefa & Wondwossen Mindahun Eshetu (2018) Geospatial Based Flood Risk Assessment: The Case of Kewet District, Amhara Region, Ethiopia.  https://www.researchgate.net/publication/333618356. DOI: 10.5923/j.ajgis.20190801.01

ESRI (2002) https://proceedings.esri.com/library/userconf/proc04/docs/pap1262.pdf?

Eastman, J.R. (2015): IDRISI for Windows, Version 2.0: Tutorial Exercises Graduate School of

Geography; Clark University: Worcester, MA, USA, 2001.

Etuonovbe, A. K. (2011). The devastating effect of flooding in Nigeria. FIG Working Week 2011 – Bridging the Gap between Cultures, Marrakech, Morocco, 18–22 May 2011. Retrieved from https://www.oicrf.org/-/the-devastating-effect-of-flooding-in-nigeria

Gashaw, W., & Legesse, D. (2011). Flood Hazard and Risk Assessment Using GIS and Remote Sensing in Fogera Woreda, Northwest Ethiopia (pp. 179–206). springer netherlands. https://doi.org/10.1007/978-94-007-0689-7_9

Gaňová, L., Zeleňáková, M., Słyś, D., & Purcz, P. (2014). Approach for Assessing Direct Flood Damages. Selected Scientific Papers: Journal of Civil Engineering, 9(2).

Glaus, A., Mosimann, M., Röthlisberger, V., & Ingold, K. (2020). How flood risks shape policies: flood exposure and risk perception in Swiss municipalities. Regional environmental change, 20(4), 120.

Chan, F. K. S., Mitchell, G., & McDonald, A. T. (2012). Flood risk appraisal and management in mega-cities: a case study of practice in the Pearl River Delta, China. Water Practice and Technology, 7(4), wpt2012060.

Ifiok Enobong Mfon, Michael Chukwuemeka Oguike, Salvation Ubi Eteng, & Ndifreke Moses Etim. (2022). Causes and Effects of Flooding in Nigeria: A Review. East Asian Journal of Multidisciplinary Research, 1(9), 1777–1792. https://doi.org/10.55927/eajmr.v1i9.1261

Ibanga OA, Idehen OF. GIS-Based Climate Change Induced Flood Risk Mapping in Uhunmwonde Local Government Area, Edo State, Nigeria. Int. J.Environ. Clim. Change. [Internet]. 2020 Jul. 25 [cited 2025 Apr. 7];10(9):8-23. Available from: https://journalijecc.com/index.php/IJECC/article/view/324

Islam, M. M., & Sado, K. (2000b). Development of flood hazard maps of Bangladesh using NOAA-AVHRR images with GIS. Hydrological Sciences Journal, 45(3), 337–355.

Joy Sanyal, X.X. Lu (2006) GIS-based flood hazard mapping at different administrative scales: A case study in Gangetic West Bengal, India. Singapore Journal of tropical Geography . Volume27, Issue2Pages 207-220

Nerantzis Kazakis, Ioannis Kougias, Thomas Patsialis (2015) Assessment of flood hazard areas at a regional scale using an index-based approach and Analytical Hierarchy Process: Application in Rhodope–Evros region, Greece, Science of The Total Environment,

Volume 538, Pages 555-563

State Emergency Management Agency (SEMA). (2013). Report on the 2011 flood disaster in Calabar metropolis. Cross River State Government.

Selvin, P., O. and Cigdem, T. (2015). Detection of Flood Hazard in Urban Areas Using GIS,

Izmire Case in Turkey. 9 th International Conference of Interdisciplinary in Engineeering,

INTER-ENG 8-9 October, (2015), 373-381.

Shaban, A. Khawile M. and Abdullah C. (2006). Use of Remote Sensing and GIS to Determine

Recharge of Potential Zones. The case of occidential Lebanon Hydrology journal, 2006

14(4) 433-443.

USGS (United States Geological Survey). (2018). Shuttle Radar Topography Mission (SRTM) 1 Arc-Second Global. Available at: https://earthexplorer.usgs.gov/

Zhang, Z., Zhang, G., Zuo, C. et al. Hillslope soil erosion and runoff model for natural rainfall events. Acta Mech Sin 24, 277–283 (2008). https://doi.org/10.1007/s10409-008-0142-5