1

2

3

4

5

6

7

8

9

10

11

12

13

14	.

- Development and Evaluation of Wearable Smart Gloves for Real-Time Sign Language Translation

# 15	ABSTRACT

Aims: To design and execute a two-way sign language translator aimed at enhancing communication between hearing and deaf-Dumb individuals. The device seeks to address accessibility challenges by accurately translating sign language into audio for hearing people.

Study Design: A technological research and development study focused on gesture recognition and audio generation.

Place and Duration of Study: Conducted at [The Higher Institute of Engineering and Technology in New Damietta.] over [January 2025 to December 2025].

Methodology: The translator uses a hand motion detection technology to interpret sign language and generates corresponding text outputs. For reverse translation, the device converts textual input into audio using a pre-designed audio database, including both hearing and deaf-Dumb individuals, to validate usability and accuracy. Performance was assessed based on parameters like recognition accuracy and ease of use.

Results: User testing demonstrated high device accuracy, Participants rated the device as user-friendly and effective in facilitating communication. The findings underline the potential of the system in bridging communication gaps in everyday interactions for deaf-Dumb individuals.

Challenges: One of the challenges encountered during the development of this device was effectively communicating its functionality to the target users and ensuring their adaptation to its use.

Conclusions: This two-way sign language translator presents a promising solution to improve accessibility and communication for deaf-Dumb individuals. It combines innovative gesture recognition with intuitive audio generation to address a significant societal need. Looking forward, our vision is to see this innovation adopted broadly, not only by individuals with disabilities but also by the general population, fostering a more inclusive society.

17

| Keywords | Gloves | speech disabilities |
| --- | --- | --- |
|  | Hearing disabilities | Artificial Intelligence |

18

19

20

21

22

23

24

26

27

28

29

30

31

32

33

# 1. INTRODUCTION

- As illustrated in Figure 1, the everyday lives of people with impairments, including deaf-Dumb

- people, have significantly improved as a result of technological advancements. One of the most

- important components of human contact is communication, and its absence can lead to

- loneliness and social rejection. Communication barriers between deaf-Dumb people and the

- hearing community frequently result in miscommunications and restricted access

- to services and information [1]. Sign languages have been utilized by humans for

- communication and message delivery, especially by deaf societies. Facial emotions,

- hand shapes, and hand orientation and movement can all be used to communicate a speaker's

- ideas through sign language. utilized concurrently. Nonetheless, very few regular people are

- familiar with sign language. As a result, it could be difficult for people who regularly communicate

- using sign language to have conversations or just to share their opinions with others Thus,

- solutions have been created to help deaf communities hear or communicate with others as a

- result of the quick development of technology. Deafness and other communication impairments

- can be helped by a variety of over-the-counter hearing aids, such as behind-the-ear, in-the-ear,

- and canal aids. Although hearing aids are Although these devices are helpful, employing

- one could make the user uneasy or make background noise audible. Scientists have therefore

- been developing a

- variety of methods to translate motions in sign language. Wearable technology and vision- based

- systems are the two primary methods. Vision-based systems use image processing techniques

- such as feature extraction to identify hand and finger

- movements. This describes a number of additional studies on the application of a visionbased

- system for translating sign language.

- To eliminate cables, the wearable will be connected to a microcontroller, and the

- The smart glove will translate sign language into spoken or written speech using a number of

- techniques. Mobile phones with Bluetooth capabilities are used in this study to show translated

- audio and text [2].

- The American Sign Language alphabet was used in this paper, as indicated in Figure 2. The

- major goal of this endeavor is to create a wearable device that will facilitate communication

- between deaf groups and the general public while taking user comfort into account. The following

- is a description of how the paper is structured, hardware component section II, Hardware circuit

- design in section III, Results and Discussion section IV, The conclusion provided in V[3].

# Smart Gloves

- The smart gloves are just regular gloves that have sensors built into them. Flex sensors and

- gyroscopes make up the majority of these sensors. A microcontroller that receives input from the

- sensors, processes the information, and outputs the results as text controls these sensors.

- The gloves have one gyroscopes and five flex sensors. Figure 3 [4] illustrates how these

- A - Suggested Framework

- Figure 4 shows that every component is linked to an Arduino Uno. The HC-05 Bluetooth connects the Arduino Uno to the mobile phone. Hand gesture-sensing data from

- accelerometers and flex sensors is transmitted to an Arduino Uno, which processes and compares the data. The generated data is then communicated to a mobile device via HC05 Bluetooth, where the output is shown. There are two sections to the suggested system:-

- 1 - The transmitter section

- Devices like the Arduino Nano, HC-05 Bluetooth Module, accelerometers, and Flex sensors are included in this section.

- 2-Section of the Receiver

- This section's gadget is a mobile phone.[3]

- Block Diagram

- 	Accelerometer sensors measure linear hand motions in the X-axis and produce

- various X values that correspond to the X-axis movement

- 	Following the use of five accelerometers to detect the gesture, all sensor data are then

- handled by the MCU node.

.

# Components of hardware

- Five flex sensors, one for each finger, were used in the construction of a smart glove that

- translated ASL characters into spoken and written characters for a mobile phone application.

- The accelerometer in the back of the hand indicated whether the hand was positioned vertically

- or horizontally, an Arduino nano was used to write the code, and an HC-05 Bluetooth was used

- to link the glove to the phone [6].

- A - The Arduino nanocontroller

- Based on the ATmega328P,

- the Arduino Nano (Fig. 5(a))

- is a comprehensive, compact, and breadboard-friendly board. It has six digital I/O pins,

- fourteen 16MHz CLK speed and analog input pins. It serves as the SSG-Sys's brain in this

- paper, processing sensor input and providing text-based outputs.

- b) Flex sensor

- In essence, a flex sensor (Fig. 5(b)) is a variable resistor, and when the sensor is bent, its

- terminal resistance rises. Five 2.2-inch flex sensors were affixed to the glove's fingertips for this

- study. The fingers bend in response to a gesture, and

- the Arduino Nano receives a particular value from each sensor.

- c) Bluetooth module HC-05

- The user can view the data on the mobile app that is transformed on the Arduino board with the

- help of the HC-05 Bluetooth module (Fig. 5(c)). It is quite

- simple to combine with microcontrollers since the Serial Port Protocol (SPP) is how it operates.

- d) 10k Ohm resistor

- One part of the circuit that limits the flow of electrical current is the 10k ohm resistor (Fig.

- 5(d)). It is a crucial element frequently utilized in circuits and electronic projects.

- e) A speedometer called the MPU-6050

- The accelerometer sensor (Fig. 5(e)) is utilized to monitor the hand's movement and direction.

- The three-axis accelerometer found in each MPU-6050 is accountable for the X, Y, and Z axes'

- linear acceleration measurements. To detect the bending motion, it is placed on the wrist.

- f) A breadboard

- plastic board with many tiny holes used to build and test circuits is called a breadboard

- (Fig. 5(f)). It can be used to supply a variety of devices. with strength.

- g) Wire jumpers

- To connect components on the breadboard, a jump wire (Fig. 5(g)) is a single electrical wire or a

- collection of them in a cable. and the header pins of the Arduino.

- h) USB cord

- The smart glove cannot function without a USB cable (Fig. 5(h)), which powers the Arduino

- Nano by connecting to a laptop.

- i) The glove

- The right-hand glove (Fig. 5(i)) utilized in this research is black, composed of cotton, and flexible

- enough to allow the user to wiggle his fingers. 5 organically when making motions

- j) Android smartphones

- mobile phone using the Android operating system, the Android mobile (Fig. 5(j)) has

- more sophisticated computing power and greater capacity for communication than a

- standard phone [7].

- Fig. 5: SSG-Sys hardware components a breadboard, an Arduino nano, a flex sensor, an HC-05

- Bluetooth module, a resistor, an accelerometer, jumper wires, a USB cable, an Android

- smartphone, and a glove.[7]

# Components of software

- The suggested SSG-Sys software components are described in depth in the section that follows.

- A) Software for Fritzing

- Users can create electronic circuits with this open-source program. It helps them move from

- testing prototypes to building more environmentally friendly circuits. "0.9.8" is the version

- utilized in this work. Using Fritzing software, the circuit design of the suggested SSG-Sys is

- displayed in Fig. 6.

- A.Proteus software

- B.A program called Proteus is used to simulate different projects before they are

- implemented in hardware. It has sensors, additional electronic parts, and other non- Additional

- elements that can be added to the Proteus's library area by downloading and adding

- particular libraries. The Proteus software version "8.10" is used in this work. The

- Proteus software simulation of the suggested SSG-Sys is displayed in Fig. 7.

- C.The Arduino IDE

- It is the application that opens, writes, modifies, and uploads the Arduino board's source

- code. Although it was developed using Java, the application makes use of Sketch, a

- programming language that is comparable to C and C++. The Arduino IDE version

- "2.1.0" is used in this study. The Arduino IDE's sample code for the suggested SSG-Sys

- is seen in Fig. 8.

- D.The second inventor

- It is an open-source online application that lets people make apps that work on Android

- handsets. Users must first add buttons, graphics, and other interface components to their

- programs (Fig. 7(a) illustrates this). Second, they must use plain language instruction blocks that

- fit together like jigsaw pieces to add logic and procedures (Fig. 7(b)). [7].

# VOCAL GESTURES

- The selected vocabulary includes gestures and positions that correspond to 24 ASL(American Sign Language) letters,

- syllables, or phrases. This illustrates the sensorized glove's ability to recognize both static and

- dynamic motions and shows a possible use case that could improve the naturalness of human

- interactions by instantly translating ASL into voice or text. The 13 dynamic gestures and 11

- static stances are shown in Fig. 10. Naturally, ASL highlights the need to recognize hand motion

- and posture.I and J or A and Sorry are two examples of vocabulary entry pairings that share the

- same stance but have distinct dynamics. There are minor variations in hand positions,

- orientations, or motion instructions among other sets, such Eat, Home, and Thank You. Certain

- gestures, like "please" or "yes," are recurring motions that may or may not be repeated. The

- majority of the letters are in still, motionless attitudes. Thus, the selected language collectively

- tests the system's capacity to integrate motion and position data for multi-class gesture

- recognition [8].

- In order to identify a variety of stances and dynamic actions, a vocabulary consisting of 24 ASL

- letters and words was chosen (Fig. 10). This results in a useful corpus for assessing how well

- the embedded glove system combines accelerometer-based motion information with strain-

- based pose information [8].

# System Overview

- An overview of the suggested wearable sensor glove for the sign language translation system is

- shown in Figure 11. Three components make up the system: output, data processing, and input

- sensors. The smart glove's input sensors include five (5) units of

- 4.5inch flexible sensors on the back of each finger and a GY-61 accelerometer sensor on the

- back of the hand. An Arduino Nano microcontroller is connected to each sensor in order to

- process data. The ATmega328 single chip 8-bit microcontroller, a 5V operating voltage, and a

- processor clock with a speed of 16 MHz are all used in the tiny and compact Arduino Nano board.

- Its eight analogue pins are sufficient to connect to the input sensors needed for this task. The

- microcontroller processes and digitizes the data from the input sensors. It then recognizes the

- gestures and converts them into words, which are then sent to an Android-based smartphone

- application over an HC-05 Bluetooth module. The words will appear on the smartphone

- application if the Arduino Nano sends them there successfully, and the speaker on the

- smartphone will play the appropriate voices for the words [9].

- Performance and accuracy

- The letters being recognized determine how effectively this system works and how accurate it is.

- Because they are so similar to one another, letters like "M," "N," and "T" might be inconvenient.

- On the other hand, statistically, this only occurs around 5% of the time. "C," "O," and "E" have

- issues that are comparable to those of those three letters.

- In this instance, the issue is not just that they are extremely similar, but also that the joints are

- positioned in the midst of the range, which means that the fingers are neither flexed nor

- extended, and there is most likely a presence of oscillations from muscular twitching. Even

- though the minimal distance classifier or the provided criteria largely outperform this, it still

- occurs perhaps 80% to 85% of the time. Generally speaking, the other letters are separated

- from one another with great care, and the addition of the minimal distance classifier minimizes

- decision-making errors [10].

# Flowchart of The Project

- The project's flowchart was created at the planning stage to make sure everything would go

- according to plan. Figure (12,13) displays the project's flowchart. After the research is finished,

- the project will begin with software development. GUIs and code are also a part of the software

- development process. C++ will be the coding language utilized for this

- project because Arduino will be used [17]. After software development is finished, the code must

- be tested and run to find any coding or graphical user interface (GUI) flaws. Any errors must be

- corrected before proceeding to the next stage. The following stage is hardware development. As

- part of hardware development, the Flex and Arduino sensors are assembled [6,9]. These two

- pieces of equipment will help collect vibration data from the tool. The hardware and software

- components need to be connected. After thesoftware and hardware have been integrated, the

- system will be tested once more to find any potential issues. Any errors must be fixed prior to the

- experiment [11].

# Results and Discussion

- Initial sensor calibration revealed that three of the sensors recorded a nearly constant increase in

- voltage when fingers were stretched straight for five seconds, a similar but relatively higher voltage

- when fingers were closed and fully bent for five seconds, and a similar pattern of voltage rising

- when fingers were moved from straight to fully bent in all three sensors. Additionally, their

- beginning and ending voltages were consistent with the ones that came before them, confirming

- the reliability and consistency of the outcome (Figure 14). While several images of the glove's

- operational video are shown in Figure 6, certain graphs acquired during data acquisition for

- specific alphabets are displayed in Figure 15.

- Due to variations in skin tone and the inability to differentiate between hands and faces, the

- machine vision-based method may yield inconsistent results. Additionally, the user must cover

- their arms with clothing that has full sleeves. Furthermore, this approach may be negatively

- impacted by lighting effects. SignSpeak, on the other hand, offers a cameraindependent solution

- that is just as accurate in bright and dim lighting. identical to this, a boosted classifier tree approach

- for hand shape detection may produce unreliable results when hand photos have very basic and

- identical backgrounds for both training and test databases, but the SignSpeak approach is not

- image-based.While SignSpeak is a standalone glove that doesn't require a Kinect, gesture

- recognition using Kinect necessitates the configuration of a Kinect with sensors to gather both

- RGB and depth data for gesture translation. We first calculated the Euclidean distance of each

- set from the average of 23 training sets to assess the reproducibility of experimental

- measurements of SignSpeak. 85% was the result of calculating the percentage of these

- discrepancies. Later, by repairing one of the flex sensors that were positioned at the wrist bent,

- the accuracy was increased. Accuracy for an unskilled user was found to be approximately 92%

- after additional training sets were obtained [13].

# CONCLUSION

- The GloSign glove, which converts sign language motions to letters and words, is proposed in this

- paper. Using the recognized letters and words, the system can also construct sentences. This

- glove decodes sign language gestures using a flex sensor and an IMU. The IBM Watson IoT

- platform receives these sensor data. The KNN machine learning technique is used to differentiate

- between movements that are similar or challenging. Sentences are then formed by combining the

- letters that were detected from the gestures.To address the errors in letter detection at the word

- and sentence levels, these sentences are subjected to an additional error correction layer known

- as the gesture fix method. Lastly, the system's output is translated to speech for convenience and

- shown on the screen. More research is required to increase the system's speed and accuracy so

- that it can help with more proficient sign language gesture verification [14].

# References

- A. Siddiqui, A. Noor, M.I. Saleem, " Efficient Learning for Hearing-Impaired: Two Way Sign Language Translator Using AI", Pakistan Journal of Engineering Technology and Science (PJETS), Vol.12, NO.1,2024.

- A.J. Abougarair , W.A. Arebi ," Smart glove for sign language translation", International Robotics & Automation Journal, Vol.8, NO.3,2022.

- P. Mishra, M. Hamza, S. Singh, D. Drivedi , R. Singh, "Sign Language Translator Glove", JOURNAL OF EMERGING TECHNOLOGIES AND INNOVATIVE

RESEARCH (JETIR) , Vol.10, NO.4, 2023.

- K.Y. Reddy, D. Moses," SMART GLOVES TO UNDERSTAND SIGN LANGUAGE",	INTERNATIONAL	JOURNALOFCREATIVERESEARCH THOUGHTS (IJCRT), Vol.9 ,NO.12, 2021.

- P. Mishra, M. Hamza, S. Singh, D. Drivedi , R. Singh, "Sign Language Translator Glove", JOURNAL OF EMERGING TECHNOLOGIES AND INNOVATIVE

RESEARCH (JETIR) , Vol.10, NO.4, 2023.

- A.Z. Shukor,M.F. Miskon,M.H. Jamaluddin,F.B. Ali@Ibrahim,M.F. Ali@Ibrahim,M.B. Bahar," ANewData Glove Approach for Malaysian Sign Language Detection", Procedia Computer Science,Vol.8,2015.

- S.A. Shaban, D.L. Elsheweikh, "An Intelligent Android System for Automatic Sign Language Recognition and Learning", Journal of Advances in Information Technology, Vol. 15, No. 8, 2024.

- J.DelPreto, J.	Hughes, M.	D’Aria, M.	de	Fazio,	D. Rus,	"

AwearableSmartGloveandItsApplicationofPose

andGestureDetectiontoSignLanguageClassification",	IEEE	ROBOTICS AND AUTOMATION LETTERS. PREPRINT VERSION. ACCEPTED JUNE, 2022.

- R. Ambar, S. Salim, M.H. Abd Wahab, M.M. Abdul Jamil, T.C. Phing, " Development of a Wearable	Sensor Glove for Real-Time Sign Language Translation", Annals of Emerging Technologies in Computing (AETiC), Vol. 7, No. 5, 2023.

- V. Martin, " Design and Implementation of a System for Automatic Sign Language Translation", Faculty of Electrical Engineering and Information Technologies SS Cyril and Methodius University, Karpos 2 BB, MK-1000 Skopje, R. Macedonia.

- A.I. Mohd Thaim, N. Sazali, K. Kadirgama,A.S. Jamaludin,F.M. Turan, N. Ab. Razak," Smart Glove for Sign Language Translation", Journal of Advanced Research in Applied Mechanics,VOL.112,NO.1,2023.

- S.C.	Bodda, P.	Gupta, G.	Joshi,  A.	Chaturvedi," ANEWARCHITECTUREFORHAND-WORNSIGN

LANGUAGETOSPEECHTRANSLATOR.", arXiv:2009.03988v1 [cs.HC] 8 Sep 2020.

- J. Bukhari,M. Rehman, S.I. Malik,A. M. Kamboh, A. Salman," American Sign Language Translation through Sensory Glove; SignSpeak", International Journal of u- and e- Service, Science and Technology, Vol.8, No.1 ,2015.

- S.M. Biju,O. Al-Khatib,H.Z. Sheikh, F. Oroumchian, " Glove based wearable devices for sign language-GloSign", IAES International Journal of Artificial Intelligence (IJ- AI), Vol. 12, No. 4, December