Physicochemical and Bacteriological Analysis of Wastewater from the Atakpamé Regional Hospital Center (TOGO)

Abstract:

Our study was carried out on the analysis of wastewater from different departments of a regional hospital. Specifically, it was to determine the contents of certain chemical, physical and bacteriological parameters in the wastewaters. Based on our previous work, the processes deployed for the analyses of these parameters mentioned above are standardized methods, as the Association Française de Normalisation (AFNOR). The results obtained showed that some services such as laboratory, general medicine and surgery produced more chemical pollutants leading to very high values of conductivity, chemical oxygen demand and biological oxygen demand. The conductivities found in the wastewaters of laboratory and general medicine were 3950 µS/cm, 3870 µS/cm respectively. The chemical oxygen demand COD was 3750 mg/L in the surgery and 1550 mg/L for the maternity. The surgery department recorded the high content of biological oxygen demand (BOD) at 946 mg/L. Unfortunately, the COD/BOD ratio was greater than 2, which indicated that organic matter was difficult to biodegrade. The results of the analyses revealed that the wastewater contained some germs with the proportions exceeding the standards recommended by WHO. Total coliforms, thermotolerant coliforms and fecal spectrococci were identified at the concentrations of 40,000, 24,000, 200,000 CFU/mL in the general medicine department, which was the most polluted area. Moreover, the laboratory remained the least polluted environment due to the use of chemical reagents that were irritating to the microorganisms (bacteria, viruses, microbes). According to these results, the wastewaters must be treated before any recovery or agricultural use to avoid food contamination via vegetables.

Keywords: wastewater, physicochemical, bacteriological, coliforms, conductivity, COD and BOD

- INTRODUCTION

The current environmental emergency obliges all sectors to change their practices and adopt a sustainable approach. Developing countries faced significant challenges in sanitation, particularly wastewater management. The healthcare sector, although dedicated to the care and protection of populations, knows negative environmental impacts. Medical waste production, energy and water consumption, and greenhouse gas emissions are among the challenges of the healthcare section. The lack of appropriate infrastructures leads to disorganized management of this water, causing the harmful impacts on public health and the environment. Treating wastewater before its release into the environment constitutes a major challenge for many countries. In Africa, most drainage systems connected to wastewater treatment plants are often faulty, and raw wastewaters are generally discharged into lowland areas. Thus, all structures such as hospitals, breweries, food manufacturing and processing plants, including slaughterhouses, discharge large quantities of organic and inorganic wastes into the environment without prior treatment. Previous studies around the world have focused on these wastewater issues particularly in the hospitals: HASSANI Abdelkader hospital in Algeria (Ramdani et al., 2019), Thailand (Danchaivijitr et al., 2005), Rio Janeiro in Brazil (Santoro et al., 2015).

Worldwide, wastewater treatment prior to discharge represents a major challenge for many countries. This difficulty is even more acute in developing countries, which not only suffer from a lack of capital but are also faced with uncontrolled urbanization and industrialization. Attempts to combat wastewater discharge into the environment are described in the literature. There are some examples: in Burkina Faso, with the implementation of a combined system (sand + coconut substrate) (Kone et al., 2012), the use of membrane bioreactors to degrade organic pollutants in industrial water (Sawadogo, 2018), the installation of reed filters to treat domestic wastewater (Liénard et al., 2004).

Togo has established a legal framework for environmental protection, including the Environmental Code 1988 and the Framework Law 2008. Policies such as the National Environmental Policy (1998) and the National Environmental Action Plan (2000) have been adopted. The Public Health Law 2009 and the Water Code 2010 regulate the discharge of pollutants into water, prohibiting discharges without prior authorization and imposing strict restrictions on the discharge of wastewater and hazardous wastes. However, Togo lacks more specific standards for wastewater discharge, which limits the effectiveness of water pollution management regulations. Thus, all structures such as hospitals are specifically required to treat liquid waste before releasing it into the receiving environment.  It is in this dynamic that our study is part of which aims to characterize the effluents of the Regional Hospital of ATAKPAME in the Plateaux Region of TOGO. Specifically, it is a question of determining the contents of chemical and bacteriological parameters in the different departments of the hospital by referring to the WHO standards.

- MATERIALS AND METHODS

  - Study area

The Atakpamé Regional Hospital Center, established in 1969, is located in the “Plateaux Region” of Togo as shown in the Fig. 1. The geographic coordinates of the study area taken using a GPS (Global Positioning System) correspond to 7° 31' 57" north latitude and 1° 7' 35" east longitude.

### Fig. 1. Map of Atakpamé Regional Hospital Center

  - Materials

The sampling campaign was conducted in July 2024. The wastewater samples were collected in the septic tanks of the following key departments: General Medicine, Analytical Laboratory, Maternity, Pediatric and Surgery. For each septic tank, two 02 liters samples were taken. The first, packaged in polyethylene bottles, were intended for physicochemical analyses and the second, placed in sterile bottles, were reserved for microbiological analyses.

  - Methods

To avoid any biological, physical, or even chemical transformation of the samples, each bottle was transported in coolers equipped with cold accumulators at a temperature between 2°C and 8°C, in order to cool the samples, but also to protect them from light. The samples were analyzed within 48 hours fater sampling.

    - Determination of physical and chemical parameters

Temperature, pH and conductivity are analyzed directly on the site of sampling and other parameters of wastewaters such as chemical oxygen demand (COD), biological oxygen demand (BOD) were analyzed using the standards methods developed according our previous work (Ouéda et al., 2025). In a reaction flask 500 mL, 10 mL of sample, 5 mL of potassium dichromate solution (0.040 mol/L) and 15 mL of concentrated sulfuric acid solution containing silver sulfate are introduced successively, shaking the reaction flask thoroughly. Then the reaction flasks are placed in the heating block and brought to a boil (150°C ± 5°C) for 2 hours. After heating, they are removed from the heating block and then cooled, before transferring the contents of the flask into an Erlenmeyer flask 250 mL. The flasks are rinsed, collecting the washings and diluted with approximately 75 mL of water. 2 to 3 drops of ferroin are added and followed by titration with the iron (II) sulfate and ammonium solution (Mohr's salt).

A BOD meter AL606 based on the mercury-free manometric method with a BOD display from 0 to 4000 mg/L was used to measure the concentration. The automated device (BOD meter) stores pressure values over several days. Each bottle has its own pressure probe and it is possible to monitor the evolution of oxygen consumption. 2 to 3 grains of potassium hydroxide are placed in the basket to be placed on the bottle. Hermetically sealed, the bottles are incubated at 20°C for 5 days with constant stirring.

Suspended Solids (MES): Suspended Solids measurement is carried out using the filtration method developed by Rodier et al., (2009). The measurement of MES is carried out by the method of filtration of a volume of water V = 0.5 L on a filter paper acting as a filter membrane. After filtration of this sample, the membrane is then dried in an oven at 105 °C for a period of 2 h. After drying, the mass M1 (g) of the dry membrane is measured using a balance; the mass M0 (g) of the paper being known beforehand (Rodier et al., 2009).

Volatile suspended solids (VSS): These represent the organic fraction of suspended solids and are obtained by calcining these solids at 525°C for 2 hours. The difference in weight between the suspended solids at 105 °C and 525 °C gives the loss on ignition and corresponds to the VSS content in mg/L of water (Metahri, 2012).

    - Determination of microbiological parameters

Total coliforms (30 °C), thermotolerant coliforms (44 °C), fecal streptococci (37 °C): These microbiological parameters were analyzed in the wastewaters collected from the septic tanks of the hospital services using the routine standardized methods of the French Association for Standardization (AFNOR) adopted by the member states of the West African Economic and Monetary Union (UEMOA). Different conditions to analyze the germs in the wastewater’s samples were shown in the table. 1.

Table. 1. Culture media according to their temperature, incubation time and their characteristic germs

| Targeted germs | Culture Medium | Temperature | Incubation time | Coloration |
| --- | --- | --- | --- | --- |
| Total Coliforms | VRBL | 30 °C | 24 h | Pink-Red of Φ˃0.5 mm |
| Thermotolerant Coliforms | VRBL | 44°C | 24 h | Pink-Red of Φ˃0.5 mm |
| Fecal Streptococci | Slanetz & Bartley | 37°C 24 h - | 24 h-48 h | Coffee Brown |

VRBL: Violet Red Neutral Bile Lactose

- RESULTS AND DISCUSSION

  - Results

    - physicochemical parameters of the analyzed raw wastewater

The physicochemical composition of wastewater from the different collection sites is shown in Table 2. The temperatures recorded at all sites are in accordance with standard norms. Wastewater pH values ​​ranging from 6.92 to 7.91 are within the WHO recommended range (6.5 to 8.5). The conductivity of all wastewater exceeds the WHO guidelines, with a high value obtained in the laboratory (3950 µS/cm) and the lowest value (1123 µS/cm) observed at the surgery. COD and BOD5 values ​​provide an index of organic pollution in wastewater. Indeed, high concentrations of COD and BOD5 are recorded at the surgery with respective values ​​of 3750 and 946 mg/L. Furthermore, low levels are obtained in the analysis laboratory with respective concentrations of 160 and 59 mg/L. Logically, the analysis shows that the wastewater from surgery has high levels (36800 g/L) of suspended solids (MES) as well as COD and BOB5. The other departments also have the concentrations more than the WHO standards with the lowest concentration collected in the analysis laboratory.

Table. 2. Results of the physicochemical parameters of the analyzed raw wastewater

| Parameters | Surgery | Laboratory | General Medicine | Maternity | Pediatric | WHO’ Value |
| --- | --- | --- | --- | --- | --- | --- |
| Temperature (°C) | 21 | 21.5 | 21.6 | 21.6 | 21.2 | ˂ 30 |
| pH | 7.2 | 7.65 | 7.09 | 6.92 | 7.91 | 6.5-8.5 |
| Conductivity (µS/cm) | 1123 | 3950 | 3870 | 3200 | 3690 | ˂ 2000 |
| COD (mg/L) | 3750 | 160 | 800 | 1550 | 530 | ˂ 90 |
| BOD5 (mg/L) | 946 | 59 | 765 | 668 | 107 | ˂ 30 |
| COD/BOD | 3.96 | 2.71 | 1.04 | 2.32 | 4.95 |  |
| MES (mg/L) | 36800 | 60 | 367 | 306 | 133 | ˂ 20 |

    - Bacteriological parameters of the analyzed raw wastewater

Table 3 shows the total coliforms (TC) levels in the wastewater studied. It appears that the TC content varies from 40,000 to 440 CFU/mL with the highest value observed at the general medicine level. All samples contain TC higher than the WHO standard standards (10,000 CFU/mL) for wastewater except for the surgery and laboratory samples. The contamination quotient (sample content/WHO standard) is less than 1 at the following sites: surgery and laboratory. It is greater than 1 in the other samples, synonymous with high contamination.

Table 3. Total Coliforms in the site of collection

| Site of sampling | Surgery | Laboratory | General Medicine | Maternity | Pediatric | WHO’ Value |
| --- | --- | --- | --- | --- | --- | --- |
| Total coliforms (CFU/mL) | 5000 | 440 | 40000 | 20000 | 15000 | 10000 |
| Ratio (Sample value/WHO ‘value) | 0.5 | 0.044 | 4 | 2 | 1.5 |  |

The table 4 shows the content of thermotolerant coliforms in the different wastewaters. According to WHO standards, the acceptable level of thermotolerant coliforms (TTC) in wastewater is 2000 CFU/mL. The sample from general medicine has a high TTC concentration of 24000 CFU/mL. Departments such as surgery and laboratory show low TCC levels of 1400 and 300 CFU/mL respectively and with contamination ratios less than 1.

Table 4. Thermotolerant coliforms in the site of collection

| Site of sampling | Surgery | Laboratory | General Medicine | Maternity | Pediatric | WHO’ Value |
| --- | --- | --- | --- | --- | --- | --- |
| Thermotolerant Coliforms (CFU/mL) | 1400 | 300 | 24000 | 2100 | 5500 | 2000 |
| Ratio (Sample value/WHO ‘value) | 0.7 | 0.15 | 12 | 1.05 | 2.75 |  |

Contrary to TC and TTC, we observe through Table 5 that the levels of fecal streptococci (FS) in the analyzed samples exceed the standard standards set at 1000 CFU/mL. The high level is observed at the general medicine (200,000 CFU/mL) as for other germs. The contamination quotient varies from 3.1 to 200 and the lowest contamination is recorded in the laboratory.

Table 5. Fecal Streptococci in the site of collection

| Site of sampling | Surgery | Laboratory | General Medicine | Maternity | Pediatric | WHO’ Value |
| --- | --- | --- | --- | --- | --- | --- |
| Fecal Streptococci (CFU/mL) | 38000 | 3100 | 200000 | 27000 | 21000 | 1000 |
| Ratio (Sample value/WHO ‘value) | 38 | 3.1 | 200 | 27 | 21 |  |

  - Discussion

    - Physicochemical parameters

Temperature is a determining factor in the development of microorganisms such as bacteria, microbes and viruses. In this study, it remains below 30 °C (WHO standard) in all samples and could be due to the sampling period (July). The month of July corresponds to the rainy season in the Plateau region where our study was carried out. The pH of the samples is close to neutral (7), an environment favorable to the proliferation of bacteria. A similar study on fish pond wastewater found pH values ​​between 6.77 and 8.43 (Sule et al., 2016). A study of wastewater from chemical and medical laboratories reported pH values ​​of 6.6 and 8.1, respectively (Akintunde et al., 2023). Alexandre et al. (2020) were analyzed wastewater from a hospital and found pH values ​​between 7.25 and 8.12.

The conductivity of wastewater provides an indication of the mineralization of this water. Indeed, the conductivity of the samples analyzed shows that they contain a high level of mineral salts (ions). The high conductivity of wastewater from the laboratory (3950 µS/cm) could be explained by the use of chemical reagents for the various analyses. Similar results were obtained by  Akintunde et al. (2023), this high conductivity in the laboratory would be due to the use of large quantities of dissolved biomedical salts as reagents. Surgical wastewater has low conductivity (1123 µS/cm), which is due to the fact that mineralized solutions or ionic chemical reagents are rarely used in this department. Previous works have reported results where the conductivity value between 297 - 324 µS/cm remains very low compared to the results of our study (Emmanuel et al., 2009).

Measuring COD and BOD provides an idea of ​​the amount of oxygen needed to stabilize wastewater. Biological oxygen demand (BOD) measures the oxygen required by microorganisms during the decomposition of organic matter. Chemical oxygen demand (COD) is the measure of the amount of oxygen required to decompose organic and inorganic materials in the presence of potassium dichromate and concentrated sulfuric acid (Salem et al., 2011). Based on these definitions, the high levels of COD (3750 mg/L) and BOD (946 mg/L) in surgery would be due to the use of organic solvents such as disinfectants and detergents. Similar observations are noted in other services such as general medicine and maternity. The laboratory, renowned for the use of ionic chemical reagents, has low COD (160 mg/L) and BOD (59 mg/L) values, which indicate low use of organic products and even production of organic wastes. According to literature results, COD/BOD ratios greater than 2 indicate that the wastewater contains organic matter that is difficult to biodegrade (Ramdani et al., 2019). Therefore, the wastewater studied poses a threat to the environment, except for effluents from general medicine, whose ratio is less than 2. A study carried out on the effluents of a hospital in western Algeria revealed chemical pollution with respective COD and BOD contents equal to 879 mgO2/L and 850 mgO2/L (Ramdani et al., 2019).

The suspended solids evaluated in our study are significantly higher than the WHO standard (20 mg/L). This could be explained by the high organic matter content of this wastewater, as evidenced by the COD and BOD parameters. Apart from surgery, our results remain comparable to those reported in the literature: industrial wastewater in Nigeria 0-2740 mg/L; hospital wastewater in Algeria (420-640), groundwater contaminated by hospital wastewater in Haiti 12.80 - 33.20.

    - Bacteriological parameters

Wastewater from the surgery and the analysis laboratory contains lower levels of total coliforms. These low levels are believed to be due to the excessive use of disinfectants in these two departments, which eliminate germs. These include mineral detergents in the laboratory (high conductivity) and organic disinfectants in the surgery (high COD). The contamination quotient clearly shows that the laboratory (0.044) remains by far the least. Our results remain lower than some reported in the literature, 1600000 CFU/mL in polluted site (Asfaw et al., 2017). Another study had reported the presence of total coliforms in the hospital wastewater with the high value reaching 108 CFU/mL (A. Chukwu et al., 2018). Comparatively, our results on the total coliform content can be compared to those of the wastewater from the University of Benin where the authors found concentrations varying between 12,000-16,000 MPN/100 mL

As the bacteria’s TCs, thermotolerant coliforms (TC) are less prevalent in surgical and laboratory collection sites, with risk quotients less than 1. In fact, these wastewaters respected the WHO recommendations. The presence of TCs indicates fecal contamination; wastewater from other services therefore poses an environmental threat. Closed at similar conclusions, (Ramdani et al., 2019) had concluded that these wastewater discharges from hospitals pose a serious threat to human health.

The high presence of fecal streptococci in the septic tank indicates that the process of degradation of organic matter and elimination of pathogenic bacteria is not taking place correctly. It is obvious that the risk of germ contamination in wastewater is very high, particularly in general medicine, with a quotient of 200, well above the standards. This water cannot be used for any purpose without prior treatment. A study conducted on fish pond wastewater showed that the fecal coliform content was 1000 CFU/mL, a lower value than in our study (Sule et al., 2016). Analyses of hospital wastewater showed Fecal Streptococci concentrations of 21,000 FCU/mL in the same range as some values ​​in our study, particularly in pediatric wastewaters (Tahiri et al., 2012). In Burkina Faso, a microbiological analysis of wastewater in the dry and hot season showed results of FS with contents ranging from 650-45000 CFU/100 mL (Nitiema et al., 2013). Previous work has shown that maximum FS concentrations in wastewater are in the order of 1000 MPN/100 mL in 2001 and 900000 in 2002 (Emmanuel, 2003).

- CONCLUSION

Our study is of great importance because it raises the alarm on wastewater management that can contribute to environmental protection. Analyses reveal significant pollution of raw wastewater collected from septic tanks, but some values (pH) ​​are relatively acceptable according to WHO standards. The results showed that wastewater from all sampling sites has physical, chemical and bacteriological parameters that exceed the standards recommended by the World Health Organization (WHO). Pollution varies in nature depending on the department's objectives. Chemical pollution is noticeable in departments that use chemical reagents, such as the analysis laboratory, where conductivity is high, and surgery, where organic discharges exceed standards. Furthermore, bacteriological pollution is remarkable in services such as general medicine with high fecal contamination through the levels of fecal streptococci. Not all of this wastewater meets WHO standards; its release into the environment would therefore pose a serious threat to the environment. Finally, any potential recovery of this wastewater requires the implementation of a treatment system aimed at reducing the level of chemical and fecal contamination.

REFERENCES

Chukwu, A. V., Oti, B. V., Nnadozie, N. T., Nsemoh, E. H., Ajegena, K. B. (2018). Microbiological and Antimicrobial Analysis of Hospital Wastewater Discharged into the Soil Environment. South Asian Journal of Research in Microbiology, 1(2), 1–11. https://doi.org/10.9734/sajrm/2018/v1i2755

Akintunde, E. A., Atobatele, B. O., Akinola, O. T., Olufunmilayo, O., Olutona, O. (2023). Physico-chemical, Heavy Metal and Microbial Analysis of Wastewater Discharged from Selected Laboratories in Bowen University, Nigeria. https://www.researchsquare.com/article/rs-3423976/latest. https://doi.org/10.21203/rs.3.rs-3423976/v1

Alexandre, B., Maniragaba, A., Christophe, M., Bazambanza, A. (2020). Physico-Chemical and Bacteriological Analysis of Waste water from Hospital “Case of Centre University Teaching Hospital of Kigali.” Fields of Interests, 6(3), 23–31.

Asfaw, T., Negash, L., Kahsay, A., & Weldu, Y. (2017). Antibiotic resistant bacteria from treated and untreated hospital wastewater at Ayder Referral Hospital, Mekelle, North Ethiopia. Advances in Microbiology, 7(12), 871–886.

Danchaivijitr, S., Wongchanapai, W., Assanasen, S., Jintanothaitavorn, D. (2005). Microbial and heavy metal contamination of treated hospital wastewater in Thailand. J Med Assoc Thai, 88(10), S59-64.

Emmanuel, E. (2003). Evaluation des risques sanitaires et ecotoxicologiques lies aux effluents hospitaliers, 194.p.

Emmanuel, E., Pierre, M. G., & Perrodin, Y. (2009). Groundwater contamination by microbiological and chemical substances released from hospital wastewater: Health risk assessment for drinking water consumers. Environment International, 35(4), 718–726.

Kone, M., Bonou, L., Koulidiati, J., Joly, P., Sodre, S., & Bouvet, Y. (2012). Traitement d’eaux usées urbaines par infiltration-percolation sur sable et sur substrat de coco après un bassin anaérobie de lagune sous climat tropical. Revue Des Sciences de l’Eau, 25(2), 139–151.

Liénard, A., Boutin, C., Molle, P., Racault, Y., Brissaud, F., & Picot, B. (2004). Filtres plantés de roseaux à flux vertical et lagunage naturel en traitement d’eaux usées domestiques en France: Comparaison des performances et des contraintes d’exploitation en termes de pérennité et fiabilité. Sciences Eaux & Territoires, Spécial Ingénieries-EAT-20, 87–99.

Metahri, M. S. (2012). Elimination siultanée de la pollution azotée et phosphatée des eaux usées traitées par des procédés mixtes: Cas de la STEP est de la ville de Tizi Ouzou [PhD Thesis, Universite Mouloud Mammeri]. https://dspace.ummto.dz/bitstreams/4a5f5a23-d72d-4ad4-b1c5-ca786ee312d5/download

Nitiema, L. W., Boubacar, S., Dramane, Z., Kabore, A., Noël, P. J., Traoré, A. S., & Dianou, D. (2013). Microbial quality of wastewater used in urban truck farming and health risks issues in developing countries: Case study of Ouagadougou in Burkina Faso. Journal of Environmental Protection, 4(6), 575–584.

Ouéda, N., Bawa, K. E., Tchakala, I., & Bawa, M. L. (2025). Contrôle de la qualité physico-chimique et bactériologique des sources d’eau (puits et forages) en saison hivernale d’une zone inondable dans le district de Lomé: cas de Zogbedji. Asian Journal of Science and Technology, 16(2), 13509–13515.

Ramdani, N., Benouis, K., Hamou, A., Boufadi, M. Y., & Lousdad, A. (2019). Physicochemical and bacteriological characterization of hospital effluents and their impact on the environment. Arabixiv. https://doi.org/10.31221/osf.io/rbka9

Rodier, J., Legube, B., & Merlet, N. (2009). L’Analyse de l’eau 9è édition. Entièrement Mise à Jour, Dunod, Paris.

Salem, I. B., Ouardani, I., Hassine, M., & Aouni, M. (2011). Bacteriological and physico-chemical assessment of wastewater in different region of Tunisia: Impact on human health. BMC Research Notes, 4(1), 144. https://doi.org/10.1186/1756-0500-4-144

Santoro, D. O., Cardoso, A. M., Coutinho, F. H., Pinto, L. H., Vieira, R. P., Albano, R. M., & Clementino, M. M. (2015). Diversity and antibiotic resistance profiles of Pseudomonads from a hospital wastewater treatment plant. Journal of Applied Microbiology, 119(6), 1527–1540.

Sawadogo, B. (2018). Traitement des eaux usées industrielles par des procédés membranaires sous climat sahélien: Cas des eaux usées de brasserie au Burkina Faso [PhD Thesis, Université Montpellier; Institut international d’ingénierie de l’eau et de l …]. https://theses.hal.science/tel-02071743/, 119.p

Sule, I. O., Agbabiaka, T. O., Ahmed, R. N., Saliu, B. K., & Olayinka, K. J. (2016). Bacteriological and physicochemical analysis of waste water from fish ponds. Ethiopian Journal of Environmental Studies and Management, 9(2).

Tahiri, E. M., Benaabidate, L., Nejjari, C., & Benbrahim, K. F. (2012). Assessment of physicochemical and biological parameters of Al Ghassani hospital wastewaters, Fez—Morocco. J. Mater. Environ. Sci, 3(1), 115–120.