The Novel Household Waste Measurement Approach and Strategies for Sustainable Prevention: A Comprehensive Review

ABSTRACT

Food waste is a pressing global issue, with approximately one-third of food produced wasted annually, causal to economic losses, and environmental degradation. This paper examines the challenges of measuring household food waste, current methodologies, and proposes a novel quantification approach to address existing limitations.

The United Nations’ Sustainable Development Goal 12.3 aims to cut up per capita food waste by 2030, necessitating accurate measurement to inform effective policies. However, inconsistencies in definitions, fragmented data, and methodological discrepancies hinder reliable estimates, with global food waste estimates ranging from 10% to 50%. Current measurement methods include top-down approaches, self-reporting, and direct measurements. Each method has strengths, such as large data access in top-down approaches, but faces challenges like inaccuracy in self-reports due to social desirability bias or logistical issues in physical surveys, including waste degradation and sampling shared bins.

A proposed method focuses on daily doorstep waste collection from individual households, minimizing degradation, side flows, and time gaps between disposal and sorting. This approach enhances accuracy by preserving waste composition and linking waste to its source, facilitating behavioral studies. The paper also explores environmental impacts, such as methane emissions from landfilled food waste, and socio-economic drivers like industrialization, urbanization, and cultural attitudes.

Effective food waste prevention requires multifaceted policies addressing values, skills, and logistics, tailored to diverse communities. Improved measurement and prevention strategies are critical for sustainable food systems and global food security.

Keywords: Food waste management, household waste measurement, sustainable prevention, doorstep waste collection

Graphic abstract

Water pollution

Climate change

Solid waste

Biodiversity loss

Environment

Industrialization

Urbanization

Modern agriculture

Food waste

Doorstep food waste collection measurement

Sustainable food waste control and prevention

Demography

Waste management policy

Behavior Consumption

Food culture

Economy

society

1. INTROODUCTION

There exist 17 goals established by the United Nations (UN) as a global sound to action aimed to protect the planet and ensure that every individual experiencing peace and development. The goal number twelve reports the issue of warranting sustainable consumption and production strategies. A central point of this goal is unsustainable patterns of waste generation. Target 12.3 calls for the reduction of food loss, which refers to the decrease in edible food mass throughout the entire supply chain (Gustavsson et al., 2011), and cutting in half per capita global food waste, which refers to final consumption (Gustavsson et al., 2011), by 2030 (UN, 2015).

The increasing concern over food loss and waste can be divided into three main aspects. The first of these concerns is food security needed to feed the up warding population, which is predicted to reach around 10 billion by 2050 (UN, 2017). Research has shown that the threat is so severe with indications that the world will need 70% to 100% more food by 2050 Gomiero et al. (2011); (McKenzie & Williams, 2015) . Cutting food loss and waste is a plausible solution for ensuring global food security (Carthy et al., 2022). Monetary loss is the second aspect of this concern. Annually, it is estimated that the US loses $1.3billion due to food loss and waste alone (Buzby et al., 2014), while it amounts to $750billion globally (FAO, 2013). Financial losses per household reach $566–$593 a year in Italy and the United Kingdom, respectively (Secondi et al., 2015). Additionally, there are others externalities which basically has to do with the carbon footprint of global loss and food waste.

According to data from the Food and Agriculture Organization (FAO), a third of global food production is lost every year. nevertheless, there is no consensus on that proportion, and estimates actually range between 10% and 50% of total global food production (Parfitt et al., 2010). There is a major data and methodology gap even though research on food loss and waste has significantly increased with time (Chen et al., 2017; Xue et al., 2017), (Nicastro & Carillo, 2021) which might be the reason there are unresolved discrepancies in data. Data regarding the volume of food waste is fragmented and inconsistent (Bellemare et al., 2017; Hoj, 2012; Roodhuyzen et al., 2017), and different studies have employed different definitions and methods (Chaboud & Daviron, 2017; Garcia-Garcia et al., 2017; Hebrok & Boks, 2017; Janssen et al., 2017; Parfitt et al., 2010; Ponis et al., 2017; Ramukhwatho et al., 2018; Richter & Bokelmann, 2017). Therefore, researchers have stressed the need for commonly agreed-upon and improved metrics for food waste (Stenmarck et al., 2016; Williams et al., 2015).

This paper focuses on the current measurement methods, their advantages and disadvantages and also some of the new methods that can be used and how they address the challenges of the current methods.

# 2. LITERACTURE RIVIEW

# Food Waste Definition

Studying and quantifying food waste is difficult (Hoehn et al., 2023) as definitions of food waste are not universally agreed upon (Lebersorger & Schneider, 2011) (Boiteau & Pingali, 2023). Materials included, means of production and management approaches are the basis on which some of the different categorizations are generated (Gjerris & Gaiani, 2013). Multiple terms have been used inter changeably, such as food loss, food waste, and kitchen waste (Schneider, 2013). Also, often the same terms are used, but with different meanings (Gjerris & Gaiani, 2013), (Mohamed Yusoff et al., 2024)

# Quantification of Food Waste

For the development of effective, well-planned food waste management policies, quantification of the magnitude of food waste is essential, and it can be used to determine if future food waste recovery and prevention efforts considerably change the residual waste stream (Thyberg & Tonjes, 2015; Thyberg et al., 2015). Understanding the extent of food waste may provide a drive for people to change their attitudes and potentially their behaviors toward food waste. However, definitional issues, the absence of sound quantification methods, and a general lack of imperative or political drive have led to considerable data gaps regarding food waste quantities (Parfitt et al., 2010), (Aitken et al., 2024) There have been a range of diverse methodologies used to quantify food wasted, with each method having its downsides.

# Drivers of Food Waste Generation

There are many sources for the generation of food waste. Food waste is generated from the residential, institutional and commercial sectors even though detailed information on the exact causes are limited (Lebersorger & Schneider, 2011). In the developed world, particularly the U.S., increases in the volume, availability, accessibility, affordability, and caloric density of food have led to increase over consumption and waste (Blair & Sobal, 2006). There seems to be little understanding regarding what food is, where it comes from and what its production entails (Stuart, 2009). Decisions regarding what is too good to throw away is affected by culture and personal choice and these perceptions change over time. Some socio-demographic factors have also been associated with the increase in food wastage. Striking differences in attitudes toward food and food waste have been documented both within and across nations (Stuart, 2009), (Giordano & Franco, 2021) . Thus, food waste generation is a function of cultural, personal, political, geographic, and economic forces that influence behavior in specific ways (Pearson et al., 2013) and it may differ from person to person, year to year, or from society to society.

    - Industrialization

The industrialization of food systems results in a transition of food production and preparation from the home to the factory and from handcraft to purchasing (Strasser, 2000), this affects the foods that people consume, the types and quantities of food waste and contributes to the distancing of people from food production and preparation (Marimuthu et al., 2024). In areas with industrialized food systems with large amounts of food processing, people often purchase pre-made foods, or canned and frozen vegetables. As a result, packaging becomes more common in household waste while peapods and corn husks for example become industrial wastes.

Increased frequency of eating at restaurants and consumption of takeout food (commercially prepared but consumed at home) (Sobal, 1999) has been observed in the developed world. This can be directly linked to the dramatic rise of two-earner households, leading to little available time for food selection and preparation. As a result of more food being prepared and consumed in restaurants, there may be some shift in food waste from homes to the commercial sector. It is estimated that almost half the U.S. food budget is spent eating away from home; USDA estimated that in 2012, $672 billion was spent for food prepared in the home and $630 billion was spent on food outside of the home. This indicates a dramatic change from the early twentieth century where almost all food was prepared within the home; in 1929, $15.3 billion was spent on food in the home and $3.5 billion was spent on food from outside (USDA, 2013). It is less likely for adults to waste food that they have prepared themselves or food that a loved one has prepared. In cultures based on handwork, handmade things are valuable as they embody many hours of labor. People who have not created or prepared something themselves, or watched a loved one do so, value labor less than those who have, and therefore, are more likely to throw it away (Strasser, 2000). As food preparation and consumption is increasingly done in restaurants, factories, or supermarkets, there is likely to be shifts in the types and quantities of food waste generated by residences, industry, and commercial establishments.

    - Economic Growth

Higher incomes have generally been associated with the consumption of a more varied diet (Drewnowski, 1999; Pingali & Khwaja, 2004). There has been a decline in starchy food staples and a diversification in of diet toward more meats, dairy, fish and poultry which is associated with the growth in household incomes (Fischler, 1999; Parfitt et al., 2010), per Bennett’s Law, food share of starchy staples decreases as income increases (Bennett, 1941). There has been a documentation of a worldwide trend with decreases in rice consumption and increases in consumption of protein and energy rich foods and convenience foods. Particularly, Asian diets are shifting toward more Western foods (Pingali & Khwaja, 2004). Western diets, with vulnerable, shorter shelf-life foods, are associated with greater food waste and a greater drain on environmental resources (Lundqvist et al., 2008). Rathje and Murphy (2001) note that the more repetitive the diet, the less food wasted and diet diversification may lead to more food waste.

As incomes rise, people may be able to waste food because food expenditures are not considerable portions of their income. Food is relatively inexpensive compared to other expenses such as housing in wealthy countries like the U.S. and people can afford to waste food (Pearson et al., 2013). It is suggested by the FAO that the careless attitude of consumers who can afford to waste food is a huge contributor to household food wastage (Gustavsson et al., 2011).

    - Urbanization

Urbanization requires extensions of food supply systems (Parfitt et al., 2010). Food waste may increase because urbanization leads to diet diversification and a disconnection from food sources. Urbanization has increased substantially in the U.S.; in 1790, five percent of Americans lived in urban areas, by 1890 it was 35 percent, and in 2010, it was 81 percent (Bureau, 2012). Urbanization is expected to continue increasing globally with concentrated, population dense urban food systems are different from those of dispersed, low density rural systems (Solomons & Gross, 1995). Fewer farms and farmers are in urban areas so, fewer people interact directly with agricultural processes or live near places where food is produced, hindering knowledge about food origins. This promotes disconnections from food (Parfitt et al., 2010) as people have no sense of what their food is made of or how it was produced (Fischler, 1999). Lebersorger and Schneider (2011) found residual waste from urban Austrian households contained significantly more food waste than rural areas.

    - Globalization

There has been a change in food systems due to the shift from local to regional to global foods in terms of quantity, type, cost, variety and desirability (Hawkes, 2006). Globalization basically means the linkage of formerly local, national and regional phenomena in organizational arrangements at a global scale (Sobal, 1999). Globalization in food supply was made possible by social and technological changes occurring after the industrialization of food supply (Robertson, 1990). Globalization has been associated with the consumption of fewer locally produced plant foods and more imported and processed foods, particularly animal products (Pingali & Khwaja, 2004; Sobal, 1999). Food now travels long distances (Pretty et al., 2005), and to more supermarkets in place of small, local markets, and so consumers purchase more non local foods. Dietary changes as a result of globalization affects the type of food disposed; people may also be more likely to waste food as they do not have a deep connection and understanding of it.

## Cultural Factors

Culture plays a fundamental role in shaping food, eating, and nutrition (Rozin, 2005; Sobal, 1999), as well as waste generation. Cultural habits and attitudes are the major factors attributed to the amount of food a society wastes. Different foods and food part are considered as edible in different cultures while some are thrown away (Strasser, 2000). Pollan (2007) points out that some cultures, particularly the U.S. and Australia, have weak food traditions of their own, meaning there are few longstanding rules and rituals about what to eat and when to eat it, and there are weak connections between the production and preparation of food and its consumption. The U.S. has an unhealthy relationship with food and the food culture in the U.S. places little value on food, leading to waste according to Bloom (2010). Other societies have a strong appreciation for food, including production and preparation. The French for instance have deep food cultures which have developed over long periods of time (Gatley et al., 2014). French attitudes toward food tend to emphasize moderation and quality, rather than abundance and quantity as in the U.S. (Rozin, 2005). Countries with deep food cultures tend to be more resistant to change primarily due to strong values surrounding what foods can be grown during certain seasons and how foods are prepared. Changes brought about by modernization of the food supply system have less effects on deep food cultures.

Furthermore, there are cultural differences in daily food practices which may affect wastage. For instance, there may be cross-national differences in shopping patterns in terms of the amount of food purchased in a single trip, the number of days between shopping trips, and the amount of food stored in the household (Neff et al., 2015). Compared to developed countries, consumers in developing countries generally buy smaller amounts of food each time they shop, often times, they buy just enough meals for that day (Pearson et al., 2013), which may reduce waste.

    - Socio-Demographic Factors

Surveys of attitudes and behaviors have shown some correlations between food wasting behaviors and certain socio demographic characteristics (Pearson et al., 2013), even though there is no clear consensus on how more waste is generated by socio-demographic factors. To get a better understanding of how wastage patterns may change as demographics change, there needs to be an understanding of demographic patterns such as aging population. Food waste generation has been shown to be affected by age with young people wasting more than older people (Cox & Downing, 2007; Hamilton et al., 2005). Food waste fell sharply as age increased in Australia; among 18–24-year-olds, 38 percent of respondents wasted more than $30 (Australian) on fresh food over two weeks, compared to seven percent of people aged 70 and up (Hamilton et al., 2005). People over the age of 65 wasted considerably less food than the rest of the population in the U.K. Older people felt that food wasting was wrong which may be based on the fact that many people of this age group experienced austerity and food rationing during the World War II, establishing attitudes against wastefulness (Quested et al., 2013). It is unknown if current young people will waste less as their knowledge, attitudes, and lifestyle change as they age (Pearson et al., 2013).

Family composition and household size significantly affect food waste generation. Households with children waste more than households without children (Cox & Downing, 2007; Hamilton et al., 2005; Parizeau et al., 2015). Children often did not want to finish their food in Swedish households which was a common cause for food waste. Smaller households waste more per capita than larger households (Baker et al., 2009; Parizeau et al., 2015; Williams et al., 2012), especially those where people live alone. Koivupuro et al. (2012) found no significant difference in waste per capita based on house hold size, but people that lived alone generated the most waste per capita. Women that lived alone in particular generated the most food waste per capita. Jorissen et al. (2015) also found that single person households wasted the most per capita.

Food is wasted across all levels of income (Pearson et al., 2013). Lower food waste has been found in low-income compared to high income households (Cox & Downing, 2007) and food waste has also been shown to increase with household income (Baker et al., 2009). However, others found little or no correlation between income and food wastage (Koivupuro et al., 2012; Van Garde & Woodburn, 1987; Wenlock et al., 1980).

# 4. Environmental Impacts of Food Waste Disposal

Depending on how food waste is managed, it may have negative environmental impacts at the end of its life. In landfills, food waste converts to methane, greenhouse gas with a global warming potential 25 times greater than carbon dioxide (CO2) on a 100-year time scale (IPCC, 2007). In the U.S., even though landfills capture methane to create energy, fugitive emissions and landfills without collection system cause landfills to be the third largest source of anthropogenic methane in the U.S. (USEPA, 2011). Food waste tends to degrade faster than other landfilled organic materials, has a high methane yield, and does not contribute to considerable biogenic sequestration in landfills (Levis & Barlaz, 2011); thus, the priority should be reducing the amount of food waste that is landfilled. Treatment of food waste with waste-to-energy incineration (WTE) is not considered to be energetically favorable due to the high moisture content of food waste (which results in a lower heating value than other materials). Also, WTE does not capture valuable nutrients within food waste and environmental pollution problems may arise from inefficient air pollution and control measures. As a result, methods other than WTE for the handling of food waste are preferred (Pham et al., 2015).

Energy and compost are some of the benefits of food waste if managed through composting or anaerobic digestion or in landfills with efficient gas collection systems (Yaser et al., 2022). Through the use of informal routes for the management of food waste such as donating it to charity or feeding it to pets may also provide some environmental benefits (Reynolds et al., 2014; Reynolds et al., 2015). Reducing and diverting food waste from disposal may be a means to increase stagnant recycling rates and improve the overall environmental performance of waste management systems (Farahdiba et al., 2023)

# 2.5. Policies Driving Food Waste Generation

There are policies which contribute to food waste by mandating food disposal under certain conditions or by preventing its redistribution elsewhere. Even though they may lead to increased food wastage, these policies aim to achieve some overall benefit for instance, food safety and enhanced nutrition. Also, reuse or redistribution may be discouraged by litigation concerns. As a result, there is tension between the need for food safety and nutrition and the desire to reduce food waste (Watson & Meah, 2012), (Phonthanukitithaworn et al., 2024).

A policy which may lead to food wastage is the 2010 Healthy, Hunger-Free Kids Act which required USDA to update nutrition standards of the National School Lunch and Breakfast Program. The revised standard emphasized nutritional quality improvements for student meals. Substantial food wastage has been the criticism of this policy because students dislike the new meals and are throwing away fruits and vegetables that they are required to take (Jalonick, 2014). 45 percent of served food and beverages were discarded by students in an elementary school after the implementation of the policy (Byker et al., 2014). However, an evaluation of food waste before and after the implementation of the standards by Cohen et al. found substantial amounts of food waste before and after. In middle schools, Shwartz et al. (2015) found that the standard reduced plate waste, so it is unclear whether the standard causes increased food wastage.

# 2.5.1. Policies for Food Waste Prevention

Food waste is a complex, interdisciplinary and international issue which have profound effects for global sustainability. An examination of the diverse ranges of food wastage drivers and behaviors provides an insight into the best ways to achieve a successful food waste prevention, which can reverse the trend of increased food wastage. There is little research that directly addresses the factors that motivate food waste prevention behaviors (Graham-Rowe et al., 2014). The prevention of waste requires a change in people’s behavior, both collectively for instance, in companies and individually (Wilson, 1996), (Shen et al., 2023)

2.5.1.1 A Multi-Faceted Approach

Waste prevention policies should target the circumstances and actions that lead to food wastage and should be informed by motivations for waste production. Multiple prevention mechanisms are simultaneously need to be addressed because prevention is not created by one, but by many behaviors (Cox et al., 2010). Also, multiple policy approaches will target different parts of the population, thus providing greater opportunities to engage more people (Quested et al., 2013). This is necessary because different populations will respond differently to prevention initiatives. For instance, economically and socially deprived communities, particularly those in high-rise, high-density housing, will require exceptional efforts and additional resources to drive behavior changes to prevent food waste, according to Rispo et al. (2015). Hence, it can be concluded that a combination of prevention policies is imperative to prevent food waste.

2.5.1.2. Selecting The Best Policy Approach

Food waste prevention policies are obstructed by regulatory, social and political obstacles. Some of these challenges were outlined by Thyberg and Tonjes (2015) which include poor public participation, lack of efficient indicators to monitor performance and uncertainty regarding policy outcomes. There is no one-size-fits-all solution to food waste; policy measures to address it should be custom tailored for each individual situation, integrate community needs, and involve a package of several measures addressing Values, Skills and Logistics. Holistic approaches which integrate education, financial aspects, and logistical improvements across food and waste systems are ideal.

Evaluations of food waste prevention policies are scarce, as a result, it is unclear which mechanisms to prevent food waste is most effective. Due to the inherent difficultly in studying and implementing waste prevention, there has been little quantitative work assessing its environmental impacts (Gentil et al., 2011). Also, food waste prevention initiatives are still in its early stages so comprehensive data are not yet available and this further complicates food waste prevention

The amount of food waste disposed can be substantially reduced by food waste prevention policies making it an effective alternative to collection and treatment of wastes economically, socially and environmentally. However, even with rigorous prevention programs, food waste from residential, institutional, and commercial sectors will never be eliminated because some food waste is unavoidable (e.g., peels) (Schott & Andersson, 2015), and redistribution of edible food to feed humans may be unfeasible due to food perishability and high transport or distribution costs (Buzby et al., 2014).

- FOOD WASTE MEASUREMENT METHODS

Assessment of food waste generation along the supply chain is troubled with considerable uncertainties (Amicarelli & Bux, 2020; Bräutigam et al., 2014). Definition systems and classification methods vary, which make it difficult, if not impossible to compare food waste studies. Thus, establishing a valid estimation of the extent of food waste remains major challenge (Bräutigam et al., 2014; Girotto et al., 2015; Parfitt et al., 2010; Stenmarck et al., 2016; Williams et al., 2015), (Kwakye et al., 2024).

There are three classifications for current food waste measurement. A top-down approach is the first, in which data is based on mass balance or extrapolation of waste data-bases (Beretta et al., 2013; Bradley et al., 2009; Bräutigam et al., 2014). The main strength of this method is the access to large amounts of data, which could facilitate comparability, although when combining different databases, this advantage might be diminished (Moller et al., 2014).

The second type of food waste measurement focuses on self-reporting methods, including questionnaires (Jörissen et al., 2015; Lorenz et al., 2017; Ponis et al., 2017; Quested et al., 2013; Stefan et al., 2013) food waste diaries (Koivupuro et al., 2012; Langley et al., 2010; Quested et al., 2013; Richter & Bokelmann, 2017; Williams et al., 2012), and interviews (Aschemann-Witzel et al., 2015; Glanz, 2008; Parfitt et al., 2010; Ramukhwatho et al., 2018). Using questionnaires requires a good sample design and large sample sizes (Zorpas & Lasaridi, 2013). This method focuses mainly on less invasive, time consuming and expensive than direct measurements (Hoj, 2012; Visschers et al., 2016). Although, it is based on the assumption that respondents can recall accurately the specific waste events and assess the amount of food wasted (Hoj, 2012). From a realistic point of view, respondents might not really know the exact amounts of waste they produce which may undermine the reliability of this method (Visschers et al., 2016; Xue et al., 2017).

A good tool for providing close observations of food waste practices and amounts are interviews (Moller et al., 2014); yet they are prone to biases similar to those of self-reporting. Inter views also dictate small sample sizes that might compromise representativeness (Zorpas & Lasaridi, 2013).

In order to accurately measure food waste weight and composition, it is widely agreed that one must record them as closely as possible to the point at which the food enters the waste stream (Langley et al., 2010). Food waste diaries, in which waste is recorded each time it is produced, put this notion into practice. However, food diaries are also subject to self-reporting biases (Xue et al., 2017). For instance, the “observer effect” (Langley et al., 2010); is a form of social desirability bias whereby people change their waste discarding habits or underreport their waste in order to present themselves in a positive light in accordance with the well-known social desirability and experimenter demand effects (Hebrok & Boks, 2017; Hoj, 2012; Langen et al., 2015; Langley et al., 2010; Neff et al., 2015; Porpino, 2016). Food waste diaries are time-consuming and costly hence they are mainly suitable for short sampling periods and small sizes which makes up another challenge of using them (Xue et al., 2017; Zorpas & Lasaridi, 2013).

Overall, the ability to provide firsthand insights into food discarding habits stands as the main strength of self-reporting methods (Richter & Bokelmann, 2017). Large samples may be used at relatively low cost when questionnaires are employed. The main disadvantage of self-reports their inaccuracy in measuring waste levels that tend to be understated, e.g., due to social desirability, causing respondents’ conservation efforts and environmental awareness to be exaggerated. Interviews and waste diaries are also costly and only suitable for small samples.

The third type of measurement involves direct measurement involves direct measurement, also known as physical waste surveys (Edjabou et al., 2016; Fehr & Romão, 2001; Hanssen et al., 2016; Lebersorger & Schneider, 2011; Parizeau et al., 2015; Quested et al., 2013; Schott & Andersson, 2015), which is the focal point of this paper.

# 3.1. Physical Household Waste Surveys

In a physical waste survey carried out by Elimelech et al. (2018), different waste fractions and components are classified into different categories such as edible/ inedible fractions and then weighed (Moller et al., 2014). It is considered as a more objective and accurate method than self-reporting methods, as it is usually carried out by a third party who has expertise in field waste surveys (Jörissen et al., 2015; Langley et al., 2010; Lebersorger & Schneider, 2011).

Measurement of municipal solid waste (MSW) composition is a well-known and standardized practice. No, yet  standardized guidelines have been developed for conducting physical surveys of food waste (Jörissen et al., 2015; Langley et al., 2010; Lebersorger & Schneider, 2011). Food waste analysis require higher sorting and classification resolution compared to MSW surveys. For example, the identification of individual food items such as different types of fruit and vegetables, dairy products, food leftovers and so on. There is need for adjustment during the application of physical waste survey methods to food waste quantification and classification, although, accumulated knowledge from measuring MSW can certainly be an added advantage. Below are some of the challenges in measuring household food waste using physical waste surveys.

# 3.1.1. Challenges in Measuring Household Food Waste Using Physical Waste Surveys

3.1.1.1. Sampling Shared Waste Bins

In sampling single-family houses, each waste bin can be distinctly associated with a particular household (Hanssen et al., 2016; Quested et al., 2013). Sampling multi-apartment buildings or mixed-use development areas such as commercial and residential buildings where shared waste bins are in place and it is not clear which household uses which use which waste bin or whether the bin also serves other people such as local businesses or public utility is where the challenge arises (Dahlén & Lagerkvist, 2008). The absence of clarity could influence variation in results. It also causes a difficulty in the ability to differentiate between sectors and (Dahlén & Lagerkvist, 2008; Sahimaa et al., 2015) link waste bags and their source of production. Current practices include: (1) Extracting subsamples from shared waste bins. For example, when sampling a 1100-liter waste bin, only a subsample of 240-liter is analyzed (European Commission, 2004; Lebersorger and Schneider, 2011; Seagull Environmental Planning Ltd., 2014); (2) Sorting samples collected from multi-apartment buildings in ‘‘batches” (Edjabou et al., 2016). Both of these practices, however, exclude the option of drawing conclusions at the household level.

3.1.1.2. The Age of the Waste Sample

The age of the sample is influenced by the time gap between the waste’s disposal and its sampling and sorting time. A bin to be sampled includes all the waste accumulated in the bin at the time of sampling, as a result, the actual age of the waste samples is uncertain and may vary from a few minutes to days. Thus, there is an uncertainty in the identification, separation and classification of each waste item (Langley et al., 2010).

In order to avoid compromising the sample’s physical and chemical properties through processes such as waste compaction, wetting of the waste’s dry components, dehydration of the wet residue and other biochemical processes that can change the waste’s composition, Dahlén and Lagerkvist (2008) recommend that each sample be sorted within two days from the sampling day, prefer ably on the same day. Lebersorger and Schneider (2011) argue that a violation of the two-day time limit seems tolerable, considering logistical considerations such as collection intervals, the size of the sorting team, and the like. Both Hanssen et al. (2016) and Edjabou et al. (2016) reported on sorting samples within a week of the sampling day. It should be noted that a week’s interval might raise severe barriers to the sorting process and the accuracy of the identification and separation of the waste components, especially in hot weather countries.

3.1.1.3. Degradability of Organic Matter

Season and climatic conditions are the major determinants of degradation which very much depends on collection intervals which could be as long as 14 days, according to Langley et al. (2010), or 42 days, according to Lebersorger and Schneider (2011). Hanssen et al. (2016) argue that cold weather conditions eliminate evaporation. Edjabou et al. (2016) claim that food waste degradation is significantly minimized when the waste is sorted within a week of the sampling day. However, due to natural degradation, some waste items may ultimately be inseparable (Jorissen et al., 2015; Langley et al., 2010).

3.1.1.4. Waste Compaction

Physical waste surveys are generally based on samples taken from ordinary waste-collection vehicles or waste bins (Dahlén and Lagerkvist, 2008; European Commission, 2004). Collection vehicles mix and compact MSW, thus decreasing its particle size and contaminating individual waste components (Dahlén and Lagerkvist, 2008; European Commission, 2004; Lebersorger and Schneider, 2011). Some degree of compaction also occurs at the waste bin level as a result of gravity and long collection intervals. This makes identification of different waste fractions challenging (Lebersorger & Schneider, 2011).

3.1.1.5. Unknown Side Flows of Waste Materials

Sampling rubbish from collection vehicles or waste bins does not guarantee that all waste streams have been included. Because of the usage of waste disposals, yard composters, or even the ownership of a pet, some waste products are not recorded since they do not reach the trash can (Dahlén & Lagerkvist, 2008; Jörissen et al., 2015; Quested et al., 2013). Waste & Resource Action Program (WRAP) estimates that a fifth of all household food waste. Even with a proper synergy between waste sample-taking and collection routes, it is not unreasonable to assume that some waste materials will be lost when discarded after sample collection or due to the use of other disposal routes such as kitchen sink, home composting or pet food.

## The New Method of Quantifying Household Food Waste

The physical waste survey methods outlined above suggest the need for improved measurement methods. The methods are based on the principle of measuring waste of individual households on a daily basis. It includes the following elements: (i) capturing food waste at the point at which it enters the waste stream; (ii) collecting waste samples at the doorstep (iii) using the individual household as a sampling unit; (iv) collecting and sorting waste daily.

    - Doorstep Sample Collection

This involves the collection of waste samples from the resident’s doorstep in labelled bags. This provides maximum proximity to the source of the waste produced and guarantees that the waste is as close as it can be to its original state (Laurieri et al., 2020). Also, it eliminates the need coordination between waste sampling and collection routes, and decreases the probability of waste side flows(Mohamed Yusoff et al., 2024).

      - A Single Household as a Sampling Unit

A direct link can be drawn between the waste-production source and the waste sample when each household is taken as an individual sampling unit. This practice also makes it possible for diverse samples consisting of single and multi-apartment buildings in rural or urban areas and sample households located in mixed-use development areas, that is commercial and residential areas to all be assembled. The amounts of food waste and MSW produced on a household level can be documented by researchers. Furthermore, the identification of households allows further investigation of households’ waste-disposal behaviors, attitudes, and habits, for instance, by employing complementary questionnaires.

      - Sample Collection On a Daily Basis

Samples are collected using a trailer that avoids compaction on a daily basis for one week. This ensure samples are preserved with their original composition and density with minimal mixing between different waste streams making identification and sorting process easier. The practice of daily sample collection has been rejected by some scholars on the grounds of cost and inconvenience (Jörissen et al., 2015; Langley et al., 2010). Nonetheless, it has the potential to increase the relative accuracy of the sample, which can subsequently reduce the waste sample size, thus contributing to technical feasibility.

      - Sample Sorting On a Daily Basis

Sorting takes place within a few hours of collection. This process eliminates the need to store samples. The time interval between waste discarding, sampling and sorting is also eliminated. Thus, uncertainties that restrict the waste items’ identification is resolved and inaccuracies regarding the samples’ physical and chemical properties are avoided.

      - Capturing all Waste Streams

The likelihood of waste material side flows is decreased when all waste streams generated in a household are collected on a daily basis, with the exception of liquid food waste, which is typically discarded into the drainage system or used as pet food leftovers. This method can be applied to any given municipal waste-collection scheme (mixed and/or source separated collection schemes) since all waste streams are collected. Also, this method has another advantage of the ability to calculate the average amount of MSW produced on the household level, for instance, by dividing the total amount of waste produced by a single household during the course of the study by the number of sampling days (kg of waste per household per day).

Table 1. The main challenges in estimating food waste, and the solutions suggested by the new method.

| Challenges in sampling food waste | Implications | Proposed solution in the new method |
| --- | --- | --- |
| Proximity to the waste-production source | Decreased accuracy | Sampling waste from the household’s doorstep |
| Long time gaps between waste discarding, sample collection, and sample sorting | Reduces the ability to identify, separate, and classify each waste item | Eliminating time gaps by collecting, sampling, and sorting waste on the day of disposal |
| Degradability of organic matter | Reduces the ability to identify, separate, and classify each waste item | Samples are collected on a daily basis, ensuring ‘‘fresh” samples that preserve the original composition and density |
| Unknown side flows of waste materials | Inaccurate description of waste composition and quantity | samples that preserve the original composition and density - Doorstep sample collection decreases the probability of waste side flows - All waste streams are collected |
| Sampling of shared waste bins | - Difficult to determine the exact number of households using a certain waste bin - Increased variation in waste composition | Each household represents a sampling unit, so there is a clear linkage between the waste-production source and its sample |
| Almost impossible to collect data on waste-discarding behaviors and practices that affect waste production | Difficult to evaluate the relationship between household behavior and waste production | Sampling waste at the household level grants access to valuable data regarding waste behaviors, attitudes, and habits |

In research carried out on food waste, the classification procedure used is given below.

Food waste was classified as any raw or cooked food (Monier et al., 2010).

# IMPORTANCE OF FOOD WASTE PREVENTION

There is need for the sound understanding of the importance of studying food waste because it provides a foundation for the development of policies to address it. Teaching people about the implications of food waste can change their perceptions and attitudes towards it, potentially yielding behavior that can reduce waste. The four primary motivations for studying food waste are summarized below;

4.1. Environmental Impacts of Food Production, Storage and Transportation

There is a substantial environmental burden associated with food production, packaging, distribution and marketing. Food production affects the environment to the detriment of humans, animals, plants ad ecosystems generally (Gjerris & Gaiani, 2013). There has been a shift from local and seasonal food toward imported, non-seasonal fruits and vegetables, increasing transportation and energy use. More food processing as a result of growing population has led to increased energy and material inputs. The increased demand for resource intensive foods such as meats, makes the environmental impacts greater.

Food production and distribution requires large amounts of energy and other resources (Cuéllar & Webber, 2010). Key environmental risk areas include water, soil, and air. Food production can contribute to water pollution and eutrophication, particularly due to the seepage of nutrients, such as manure and fertilizers, into the broader environment.

The wastage of edible food means that all of the resources that went into growing, producing, processing and transporting that food are also wasted resulting in needless environmental impact (Gustavsson et al., 2011). The production of this lost and wasted food globally has been estimated to account for 24 percent of total freshwater resources used in food production, 23 percent of global cropland, and 23 percent of global fertilizer use (Kummu et al., 2012). In the U.S., the production of wasted food requires the expenditure of over 25 percent of the total freshwater used in the U.S., about 300 million barrels of oil (Hall et al., 2009), and represents two percent of annual energy consumption (Cuéllar & Webber, 2010). Venkat (2011) estimated that 112.92 million metric tons of carbon dioxide equivalent per year were emitted from the production, processing, and disposal of avoidable food waste in the U.S.

4.2. Economic Losses

All individuals and organizations involved in the food supply chain is affected by the economic impacts of throwing away food.

Table 2. The financial estimated cost of food wasted and lost food from research.

| Country | Year | Estimate | Sectors included |
| --- | --- | --- | --- |
| New Zealand | 2015 | $589 million/year | Avoidable household waste (WasteMinz, 2015) |
| Australia | 2015 | $5.8 billion/year | All sectors (Food Wise, 2015) |
| Global | 2013 | $750 billion/ year | All sectors (seafood excluded (FAO, 2013) |
| U.K. | 2012 | $18.3 billion/year, $689/household/year | Household (WRAP, 2013) |
| U.S. | 2011 | $197.7 billion/year, $643.3/person/year | Avoidable distribution, retail and consumer waste (Venkat, 2011) |
| U.S. | 2010 | $161.6 billion/year, 1249 calories/person/day | Avoidable retail and consumer food waste (Buzby et al., 2014) |
| Canada | 2010 | $21.1 billion/year | All sectors (Gooch et al., 2010) |
| U.S. | 2008 | $165.6 billion/year, $390/person/year | Avoidable retail and consumer food waste (Buzby and Hyman, 2012) |

4.3 Food Insecurity

Food security is the availability of and access to sufficient and healthy foods. Good nutrition is imperative for the wellbeing of individuals and nations (Soussana, 2014). Even though there appears to be sufficient food available to feed the world’s population, nearly 11 percent of the global population still suffers from food insecurity (FAO, 2015). Due to this high prevalence of food insecurity, food wastage has an important ethical dimension (Gjerris & Gaiani, 2013). Resources could be used to help feed the hungry if food resources were better managed and wastes were minimized. If all available food waste in Australia were rescued by charity, it could feed 921 thousand people for a year, Reynolds et al. (2015) theoretically estimated.

Also, food loss and waste amplify the environmental impact of food production along the entire supply chain by requiring more production than is needed based on market demand. Therefore, reducing food waste, while maintaining current production levels, could help meet global food needs.

5. CONCLUSION

Increasingly citizens, scientists, businesses, institutions, and policy makers are realizing that the current food system is unsustainable and changes are required if the world will be able to support a population of over nine billion by 2050. An increasingly important strategy to help feed this growing human population is reducing food waste (Godfray et al., 2010). Food wastage is a global phenomenon and it is likely going to keep growing if it is not curbed by prevention policies. An urgent priority is the understanding of the implications of food waste and adjusting attitudes and behaviors towards food in order to prevent it.

Even though complicated and challenging, quantitative measurement of food waste samples is critically important for waste management. The method proposed above in this paper was designed to address the challenges of connecting the household’s food waste to its source and keeping the waste fresh. This method has the potential to become a promising tool for addressing the challenges of food waste measurement.

6. RECOMMENDATIONS

Improving food waste measurement through the proposed daily doorstep collection method, and addressing prevention through multifaceted behavior driven policies, aim to tackle the identified challenges (inconsistent definitions, shared bin sampling, and degradation) while promoting sustainable practices to meet global goals like SDG 12.3. following recommendation are suggested:

Standardized Food Waste Definitions promoting universally agreed-upon definitions for food waste, distinguishing between avoidable (e.g., edible food) and unavoidable (e.g., bones, peels) waste, to enhance consistency and comparability across studies and regions should be developed.

in addition, Governments should create standardized protocols for physical household waste surveys, incorporating high-resolution sorting (e.g., classifying specific food types like vegetables, dairy, or meat) to improve accuracy compared to general municipal solid waste surveys. Furthermore, combine doorstep waste collection with questionnaires or interviews to study household waste-disposal behaviors, attitudes, and cultural factors, enabling targeted interventions to reduce avoidable food waste should be enhanced.

Furthermore, Public Awareness Campaigns, educate households about the environmental impacts of food waste, such as methane emissions from landfills and resource wastage to encourage Multifaceted Prevention Policies targeting values (e.g., cultural appreciation of food), skills (e.g., meal planning and storage techniques), and logistics (e.g., improved supply chain efficiency), tailored to diverse communities, especially in high-density or economically deprived areas should be promoted.

Longitudinal Research, collaboration with universities, NGOs, and research institutions to fund longitudinal studies and build comprehensive databases on food waste quantities, drivers, and prevention outcomes, addressing data gaps and supporting evidence-based policymaking should be invested in.

DISCLAIMER (ARTIFICIAL INTELLIGENCE)

Author(s) hereby declare that no generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc) and text-to-image generators have been used during writing or editing of this manuscript.

REFERENCES

Aitken, J. A., Sprenger, A., Alaybek, B., Mika, G., Hartman, H., Leets, L., Maese, E., & Davoodi, T. (2024). Surveys and Diaries and Scales, Oh My! A Critical Analysis of Household Food Waste Measurement. Sustainability, 16(3). https://doi.org/10.3390/su16030968

Amicarelli, V., & Bux, C. (2020). Food waste measurement toward a fair, healthy and environmental-friendly food system: a critical review. British Food Journal, 123(8), 2907-2935. https://doi.org/10.1108/bfj-07-2020-0658

Aschemann-Witzel, J., De Hooge, I., Amani, P., Bech-Larsen, T., & Oostindjer, M. (2015). Consumer-related food waste: Causes and potential for action. Sustainability, 7(6), 6457-6477.

Baker, D., Fear, J., & Denniss, R. (2009). What a waste–An analysis of household expenditure on food.

Bellemare, M. F., Çakir, M., Peterson, H. H., Novak, L., & Rudi, J. (2017). On the measurement of food waste. In: Wiley Online Library.

Beretta, C., Stoessel, F., Baier, U., & Hellweg, S. (2013). Quantifying food losses and the potential for reduction in Switzerland. Waste management, 33(3), 764-773.

Blair, D., & Sobal, J. (2006). Luxus consumption: Wasting food resources through overeating. Agriculture and Human Values, 23, 63-74.

Bloom, J. (2010). American wasteland: How America throws away nearly half of its food (and what we can do about it). Da Capo Lifelong Books.

Boiteau, J. M., & Pingali, P. (2023). Can we agree on a food loss and waste definition? An assessment of definitional elements for a globally applicable framework. Global Food Security, 37. https://doi.org/10.1016/j.gfs.2023.100677

Bradley, P., Thomas, C., Druckman, A., & Jackson, T. (2009). Accounting for food waste: comparative analysis within the UK. Proceedings of the Institution of Civil Engineers-Waste and Resource Management,

Bräutigam, K.-R., Jörissen, J., & Priefer, C. (2014). The extent of food waste generation across EU-27: Different calculation methods and the reliability of their results. Waste management & research, 32(8), 683-694.

Bureau, U. C. (2012). Growth in Urban Population Outpaces Rest of Nation Census Bureau Reports. Washington, DC: US Department of Commerce.

Buzby, J. C., Farah-Wells, H., & Hyman, J. (2014). The estimated amount, value, and calories of postharvest food losses at the retail and consumer levels in the United States. USDA-ERS Economic Information Bulletin(121).

Byker, C. J., Farris, A. R., Marcenelle, M., Davis, G. C., & Serrano, E. L. (2014). Food waste in a school nutrition program after implementation of new lunch program guidelines. Journal of nutrition education and behavior, 46(5), 406-411.

Carthy, U. M., Uysal, I., Badia-Melis, R., Mercier, S., O'Donnell, C., & Ktenioudaki, A. (2022). Corrigendum to ‘Global food security – Issues, challenges and technological solutions’ [Trends in Food Science & Technology 77 (2018) 11–20]. Trends in Food Science & Technology, 123. https://doi.org/10.1016/j.tifs.2022.03.029

Chaboud, G., & Daviron, B. (2017). Food losses and waste: Navigating the inconsistencies. Global Food Security, 12, 1-7.

Chen, H., Jiang, W., Yang, Y., Yang, Y., & Man, X. (2017). State of the art on food waste research: a bibliometrics study from 1997 to 2014. Journal of Cleaner Production, 140, 840-846.

Cox, J., & Downing, P. (2007). Food behaviour consumer research: quantitative phase. Banbury: WRAP.

Cox, J., Giorgi, S., Sharp, V., Strange, K., Wilson, D. C., & Blakey, N. (2010). Household waste prevention—a review of evidence. Waste management & research, 28(3), 193-219.

Cuéllar, A. D., & Webber, M. E. (2010). Wasted food, wasted energy: the embedded energy in food waste in the United States. Environmental Science & Technology, 44(16), 6464-6469.

Dahlén, L., & Lagerkvist, A. (2008). Methods for household waste composition studies. Waste management, 28(7), 1100-1112.

Drewnowski, A. (1999). Fat and sugar in the global diet: dietary diversity in the nutrition transition. Food in Global History, Westview, Boulder.

Edjabou, M. E., Petersen, C., Scheutz, C., & Astrup, T. F. (2016). Food waste from Danish households: Generation and composition. Waste management, 52, 256-268.

Elimelech, E., Ayalon, O., & Ert, E. (2018). What gets measured gets managed: A new method of measuring household food waste. Waste management, 76, 68-81.

Farahdiba, A. U., Warmadewanthi, I. D. A. A., Fransiscus, Y., Rosyidah, E., Hermana, J., & Yuniarto, A. (2023). The present and proposed sustainable food waste treatment technology in Indonesia: A review. Environmental Technology & Innovation, 32. https://doi.org/10.1016/j.eti.2023.103256

Fehr, M., & Romão, D. (2001). Measurement of fruit and vegetable losses in Brazil: a case study. Environment, Development and Sustainability, 3, 253-263.

Fischler, C. (1999). Chapter 11: The'Mad Cow'Crisis: a Global Perspective. Food in Global History. Westview Press, Boulder, CO, USA.

Garcia-Garcia, G., Woolley, E., Rahimifard, S., Colwill, J., White, R., & Needham, L. (2017). A methodology for sustainable management of food waste. Waste and biomass valorization, 8(6), 2209-2227.

Gatley, A., Caraher, M., & Lang, T. (2014). A qualitative, cross cultural examination of attitudes and behaviour in relation to cooking habits in France and Britain. Appetite, 75, 71-81.

Gentil, E. C., Gallo, D., & Christensen, T. H. (2011). Environmental evaluation of municipal waste prevention. Waste management, 31(12), 2371-2379.

Giordano, C., & Franco, S. (2021). Household Food Waste from an International Perspective. Sustainability, 13(9). https://doi.org/10.3390/su13095122

Girotto, F., Alibardi, L., & Cossu, R. (2015). Food waste generation and industrial uses: A review. Waste management, 45, 32-41.

Gjerris, M., & Gaiani, S. (2013). Household food waste in Nordic countries: Estimations and ethical implications. Etikk i praksis-Nordic Journal of Applied Ethics(1), 6-23.

Glanz, R. (2008). Causes of food waste generation in households: an empirical analysis. na.

Godfray, H. C. J., Beddington, J. R., Crute, I. R., Haddad, L., Lawrence, D., Muir, J. F., Pretty, J., Robinson, S., Thomas, S. M., & Toulmin, C. (2010). Food security: the challenge of feeding 9 billion people. science, 327(5967), 812-818.

Gomiero, T., Pimentel, D., & Paoletti, M. G. (2011). Environmental impact of different agricultural management practices: conventional vs. organic agriculture. Critical reviews in plant sciences, 30(1-2), 95-124.

Graham-Rowe, E., Jessop, D. C., & Sparks, P. (2014). Identifying motivations and barriers to minimising household food waste. Resources, Conservation and Recycling, 84, 15-23.

Gustavsson, J., Cederberg, C., Sonesson, U., Van Otterdijk, R., & Meybeck, A. (2011). Global food losses and food waste. In: FAO Rome.

Hall, K. D., Guo, J., Dore, M., & Chow, C. C. (2009). The progressive increase of food waste in America and its environmental impact. PloS one, 4(11), e7940.

Hamilton, C., Denniss, R., & Baker, D. G. (2005). Wasteful consumption in Australia. Australia Institute Canberra, Australia.

Hanssen, O. J., Syversen, F., & Stø, E. (2016). Edible food waste from Norwegian households—Detailed food waste composition analysis among households in two different regions in Norway. Resources, Conservation and Recycling, 109, 146-154.

Hawkes, C. (2006). Uneven dietary development: linking the policies and processes of globalization with the nutrition transition, obesity and diet-related chronic diseases. Globalization and health, 2, 1-18.

Hebrok, M., & Boks, C. (2017). Household food waste: Drivers and potential intervention points for design–An extensive review. Journal of Cleaner Production, 151, 380-392.

Hoehn, D., Vázquez-Rowe, I., Kahhat, R., Margallo, M., Laso, J., Fernández-Ríos, A., Ruiz-Salmón, I., & Aldaco, R. (2023). A critical review on food loss and waste quantification approaches: Is there a need to develop alternatives beyond the currently widespread pathways? Resources, Conservation and Recycling, 188. https://doi.org/10.1016/j.resconrec.2022.106671

Hoj, S. B. (2012). Metrics and measurement methods for the monitoring and evaluation of household food waste prevention interventions University of South Australia Australia].

Jalonick, M. (2014). GOP supports break from healthy school meal standards. Huffington Post.

Janssen, A. M., Nijenhuis-de Vries, M. A., Boer, E. P., & Kremer, S. (2017). Fresh, frozen, or ambient food equivalents and their impact on food waste generation in Dutch households. Waste management, 67, 298-307.

Jörissen, J., Priefer, C., & Bräutigam, K.-R. (2015). Food waste generation at household level: Results of a survey among employees of two European research centers in Italy and Germany. Sustainability, 7(3), 2695-2715.

Koivupuro, H. K., Hartikainen, H., Silvennoinen, K., Katajajuuri, J. M., Heikintalo, N., Reinikainen, A., & Jalkanen, L. (2012). Influence of socio‐demographical, behavioural and attitudinal factors on the amount of avoidable food waste generated in Finnish households. International journal of consumer studies, 36(2), 183-191.

Kummu, M., De Moel, H., Porkka, M., Siebert, S., Varis, O., & Ward, P. J. (2012). Lost food, wasted resources: Global food supply chain losses and their impacts on freshwater, cropland, and fertiliser use. Science of the total environment, 438, 477-489.

Kwakye, S. O., Amuah, E. E. Y., Ankoma, K. A., Agyemang, E. B., & Owusu, B.-G. (2024). Understanding the performance and challenges of solid waste management in an emerging megacity: Insights from the developing world. Environmental Challenges, 14. https://doi.org/10.1016/j.envc.2023.100805

Langen, N., Göbel, C., & Waskow, F. (2015). The effectiveness of advice and actions in reducing food waste. Proceedings of the Institution of Civil Engineers-Waste and Resource Management,

Langley, J., Yoxall, A., Heppell, G., Rodriguez, E. M., Bradbury, S., Lewis, R., Luxmoore, J., Hodzic, A., & Rowson, J. (2010). Food for Thought?—A UK pilot study testing a methodology for compositional domestic food waste analysis. Waste management & research, 28(3), 220-227.

Laurieri, N., Lucchese, A., Marino, A., & Digiesi, S. (2020). A Door-to-Door Waste Collection System Case Study: A Survey on its Sustainability and Effectiveness. Sustainability, 12(14). https://doi.org/10.3390/su12145520

Lebersorger, S., & Schneider, F. (2011). Discussion on the methodology for determining food waste in household waste composition studies. Waste management, 31(9-10), 1924-1933.

Levis, J. W., & Barlaz, M. A. (2011). Is biodegradability a desirable attribute for discarded solid waste? Perspectives from a national landfill greenhouse gas inventory model. Environmental Science & Technology, 45(13), 5470-5476.

Lorenz, B. A.-S., Hartmann, M., & Langen, N. (2017). What makes people leave their food? The interaction of personal and situational factors leading to plate leftovers in canteens. Appetite, 116, 45-56.

Lundqvist, J., De Fraiture, C., & Molden, D. (2008). Saving water: from field to fork: curbing losses and wastage in the food chain.

Marimuthu, S., Saikumar, A., & Badwaik, L. S. (2024). Food losses and wastage within food supply chain: a critical review of its generation, impact, and conversion techniques. Waste Disposal & Sustainable Energy, 6(4), 661-676. https://doi.org/10.1007/s42768-024-00200-7

McKenzie, F. C., & Williams, J. (2015). Sustainable food production: constraints, challenges and choices by 2050. Food Security, 7(2), 221-233. https://doi.org/10.1007/s12571-015-0441-1

Mohamed Yusoff, N. I. B., Godsell, J., & Woolley, E. (2024). Towards zero waste: A comprehensive framework for categorizing household food waste. Sustainable Production and Consumption, 48, 1-13. https://doi.org/10.1016/j.spc.2024.05.002

Monier, M. V., Mudgal, M. S., Escalon, M. V., O’Connor, M. C., Anderson, M. G., Montoux, M. H., Reisinger, M. H., Dolley, M. P., Ogilvie, M. S., & Morton, M. G. (2010). Project Team.

Neff, R. A., Spiker, M. L., & Truant, P. L. (2015). Wasted food: US consumers' reported awareness, attitudes, and behaviors. PloS one, 10(6), e0127881.

Nicastro, R., & Carillo, P. (2021). Food Loss and Waste Prevention Strategies from Farm to Fork. Sustainability, 13(10). https://doi.org/10.3390/su13105443

Parfitt, J., Barthel, M., & Macnaughton, S. (2010). Food waste within food supply chains: quantification and potential for change to 2050. Philosophical transactions of the royal society B: biological sciences, 365(1554), 3065-3081.

Parizeau, K., Von Massow, M., & Martin, R. (2015). Household-level dynamics of food waste production and related beliefs, attitudes, and behaviours in Guelph, Ontario. Waste management, 35, 207-217.

Pearson, D., Minehan, M., & Wakefield-Rann, R. (2013). Food waste in Australian households: Why does it occur. Aust. Pac. J. Reg. Food Stud, 3(1), 118-132.

Pham, T. P. T., Kaushik, R., Parshetti, G. K., Mahmood, R., & Balasubramanian, R. (2015). Food waste-to-energy conversion technologies: Current status and future directions. Waste management, 38, 399-408.

Phonthanukitithaworn, C., Maitree, N., & Naruetharadhol, P. (2024). Eco-innovation policies for food waste management: A European Union-ASEAN comparison. Journal of Open Innovation: Technology, Market, and Complexity, 10(2). https://doi.org/10.1016/j.joitmc.2024.100295

Pingali, P., & Khwaja, Y. (2004). Globalisation of Indian diets and the transformation of food supply systems.

Pollan, M. (2022). The omnivore's dilemma: a natural history of four meals. Findaway World.

Ponis, S. T., Papanikolaou, P.-A., Katimertzoglou, P., Ntalla, A. C., & Xenos, K. I. (2017). Household food waste in Greece: A questionnaire survey. Journal of Cleaner Production, 149, 1268-1277.

Porpino, G. (2016). Household food waste behavior: Avenues for future research. Journal of the Association for Consumer Research, 1(1), 41-51.

Pretty, J. N., Ball, A. S., Lang, T., & Morison, J. I. (2005). Farm costs and food miles: An assessment of the full cost of the UK weekly food basket. Food policy, 30(1), 1-19.

Quested, T., Ingle, R., & Parry, A. (2013). Household food and drink waste in the United Kingdom 2012. In: Wrap London.

Ramukhwatho, F., duPlessis, R., & Oelofse, S. (2018). Preliminary drivers associated with household food waste generation in South Africa. Applied Environmental Education & Communication, 17(3), 254-265.

Rathje, W. L., & Murphy, C. (2001). Rubbish!: the archaeology of garbage. University of Arizona Press.

Reynolds, C. J., Mavrakis, V., Davison, S., Høj, S. B., Vlaholias, E., Sharp, A., Thompson, K., Ward, P., Coveney, J., & Piantadosi, J. (2014). Estimating informal household food waste in developed countries: The case of Australia. Waste management & research, 32(12), 1254-1258.

Reynolds, C. J., Piantadosi, J., & Boland, J. (2015). Rescuing food from the organics waste stream to feed the food insecure: an economic and environmental assessment of Australian food rescue operations using environmentally extended waste input-output analysis. Sustainability, 7(4), 4707-4726.

Richter, B., & Bokelmann, W. (2017). Explorative study about the analysis of storing, purchasing and wasting food by using household diaries. Resources, Conservation and Recycling, 125, 181-187.

Rispo, A., Williams, I., & Shaw, P. (2015). Source segregation and food waste prevention activities in high-density households in a deprived urban area. Waste management, 44, 15-27.

Robertson, R. (1990). Mapping the global condition: Globalization as the central concept. Theory, Culture & Society, 7(2-3), 15-30.

Roodhuyzen, D., Luning, P. A., Fogliano, V., & Steenbekkers, L. (2017). Putting together the puzzle of consumer food waste: Towards an integral perspective. Trends in Food Science & Technology, 68, 37-50.

Rozin, P. (2005). The meaning of food in our lives: a cross-cultural perspective on eating and well-being. Journal of nutrition education and behavior, 37, S107-S112.

Sahimaa, O., Hupponen, M., Horttanainen, M., & Sorvari, J. (2015). Method for residual household waste composition studies. Waste management, 46, 3-14.

Schneider, F. (2013). Review of food waste prevention on an international level. Proceedings of the Institution of Civil Engineers-Waste and Resource Management,

Schott, A. B. S., & Andersson, T. (2015). Food waste minimization from a life-cycle perspective. Journal of environmental management, 147, 219-226.

Schwartz, M. B., Henderson, K. E., Read, M., Danna, N., & Ickovics, J. R. (2015). New school meal regulations increase fruit consumption and do not increase total plate waste. Childhood Obesity, 11(3), 242-247.

Secondi, L., Principato, L., & Laureti, T. (2015). Household food waste behaviour in EU-27 countries: A multilevel analysis. Food policy, 56, 25-40.

Shen, G., Li, Z., Hong, T., Ru, X., Wang, K., Gu, Y., Han, J., & Guo, Y. (2023). The status of the global food waste mitigation policies: experience and inspiration for China. Environ Dev Sustain, 1-29. https://doi.org/10.1007/s10668-023-03132-0

Sobal, J. (1999). Food system globalization, eating transformations, and nutrition transitions. Food in global history, 171-193.

Solomons, N. W., & Gross, R. (1995). Urban nutrition in developing countries. Nutrition reviews, 53(4), 90-95.

Soussana, J.-F. (2014). Research priorities for sustainable agri-food systems and life cycle assessment. Journal of Cleaner Production, 73, 19-23.

Stefan, V., Van Herpen, E., Tudoran, A. A., & Lähteenmäki, L. (2013). Avoiding food waste by Romanian consumers: The importance of planning and shopping routines. Food quality and preference, 28(1), 375-381.

Stenmarck, Â., Jensen, C., Quested, T., Moates, G., Buksti, M., Cseh, B., Juul, S., Parry, A., Politano, A., & Redlingshofer, B. (2016). Estimates of European food waste levels. IVL Swedish Environmental Research Institute.

Strasser, S. (2000). Waste and want: A social history of trash. Macmillan.

Stuart, T. (2009). Waste: Uncovering the global food scandal. WW Norton & Company.

Thyberg, K. L., & Tonjes, D. J. (2015). A management framework for municipal solid waste systems and its application to food waste prevention. Systems, 3(3), 133-151.

Thyberg, K. L., Tonjes, D. J., & Gurevitch, J. (2015). Quantification of food waste disposal in the United States: a meta-analysis. Environmental Science & Technology, 49(24), 13946-13953.

Van Garde, S. J., & Woodburn, M. J. (1987). Food discard practices of householders. Journal of the American Dietetic Association, 87(3), 322-329.

Venkat, K. (2012). The climate change and economic impacts of food waste in the United States. International Journal on Food System Dynamics, 2(4), 431-446.

Visschers, V. H., Wickli, N., & Siegrist, M. (2016). Sorting out food waste behaviour: A survey on the motivators and barriers of self-reported amounts of food waste in households. Journal of Environmental Psychology, 45, 66-78.

Watson, M., & Meah, A. (2012). Food, waste and safety: Negotiating conflicting social anxieties into the practices of domestic provisioning. The Sociological Review, 60(2_suppl), 102-120.

Wenlock, R., Buss, D., Derry, B., & Dixon, E. (1980). Household food wastage in Britain. British Journal of Nutrition, 43(1), 53-70.

Williams, H., Wikström, F., Otterbring, T., Löfgren, M., & Gustafsson, A. (2012). Reasons for household food waste with special attention to packaging. Journal of Cleaner Production, 24, 141-148.

Williams, I., Schneider, F., & Syversen, F. (2015). The" food waste challenge" can be solved. Waste management, 41, 1-2.

Wilson, D. C. (1996). Stick or carrot?: The use of policy measures to move waste management up the hierarchy. Waste management & research, 14(4), 385-398.

Xue, L., Liu, G., Parfitt, J., Liu, X., Van Herpen, E., Stenmarck, Å., O’Connor, C., Östergren, K., & Cheng, S. (2017). Missing food, missing data? A critical review of global food losses and food waste data. Environmental Science & Technology, 51(12), 6618-6633.

Yaser, A. Z., Lamaming, J., Suali, E., Rajin, M., Saalah, S., Kamin, Z., Safie, N. N., Aji, N. A. S., Wid, N., & Mustafa, A. (2022). Composting and Anaerobic Digestion of Food Waste and Sewage Sludge for Campus Sustainability: A Review. International Journal of Chemical Engineering, 2022, 1-14. https://doi.org/10.1155/2022/6455889

Zorpas, A. A., & Lasaridi, K. (2013). Measuring waste prevention. Waste management, 33(5), 1047-1056.