Energy Balance and Fuel Consumption Analysis in Rice Cultivation in Medak District

.

ABSTRACT

Aims: To study fuel energy and energy input-output analysis in Rice production system Study design: Cross-sectional descriptive study Place and Duration of Study: Medak district 2023 Methodology: Data for the study were gathered through face-to-face surveys conducted on sixty Rice-producing farms in Medak district. The selection of farms for the survey was determined using a simple random sampling method. After collecting the data, fuel energy and input-output energy ratios were calculated. Results:. Rice production cost averaged Rs. 44,000/ha with total energy input of 17,349.64 MJ/ha. N fertilizer (38.61%) and diesel fuel (23.37%) were major energy contributors, while seed contribution was minimal (0.41%). Grain yield reached 7,410 kg/ha, generating net energy of 3,270.64 MJ/ha and energy productivity of 0.43 kg MJ⁻¹. The output-input ratio was low (0.81) due to high fertilizer inputs. Specific energy was 2.34 MJ kg⁻¹ and energy intensiveness 0.39 MJ Rs⁻¹. Among machinery, cultivator had lowest fuel consumption while combine harvester had highest. Conclusion: The analysis reveals that rice production in Medak District faces significant energy efficiency challenges that require immediate attention. The key finding that the system consumes more energy than it produces (output-input ratio of 0.81) indicates an unsustainable production model that threatens both economic viability and environmental sustainability.

Keywords: Fuel energy, Rice, Energy intensiveness, Net energy

1. INTRODUCTION

Rice (Oryza sativa) is one of the most important, widely grown cereal crops in the world, and it feeds half of the world's population, especially in Asia, where it is a main diet. India is the second-largest producer  of rice after China with a production of 136.7 million tons from a 47 million hectare area in 2023-24 (https://upag.gov.in). Energy plays a pivotal role in agriculture, dating back to the era of subsistence farming. It’s widely acknowledged that agricultural production correlates positively with energy input (Taheri Garavand et al., 2010). Reduced energy consumption in crop production translates to lower production costs, particularly in developing countries where traditional methods persist, elevating production expenses. Agriculture consumes approximately 18.5 per cent of India's total energy consumption (Vijayakumar et al., 2023). Agriculture is both a significant consumer and producer of energy. Improving energy efficiency in agricultural production involves assessing the effectiveness of methods and techniques employed. Energy usage in agriculture has surged due to population growth, dwindling arable land, and aspirations for higher living standards (Kizilaslan, 2019). The sector, like others, relies heavily on resources such as electricity, fuels, natural gas, and coke. This dependence, coupled with capital-intensive technologies, is partly fueled by relatively low energy prices compared to the resources they substitute.

Efficient energy utilization boosts production, productivity, and contributes to the economic     viability and competitiveness of agriculture, especially in rural areas (Ozkan et al., 2007  and Singh et al., 2022).Energy use efficiency in rice-based cropping systems is a key determinant of sustainability and profitability. Various indicators, such as energy productivity, energy intensity,and energy balance, are used to assess the efficiency of energy utilization in rice production. Medak district's economic status is relatively lower compared to other districts in Telangana,. It was designated as one of India's most backward districts and receives funding from the Backward Regions Grant Fund Programme (BRGF). While it has a significant Gross Domestic Product (GDP), its per capita income is also lower than some other districts in Telangana state. In Medak district, Agriculture is the most predominant sector of     the district economy, as 80 percent of the population is engaged in Agriculture and     allied activities for their livelihood. The gross cropped area of the district is 352262 ha with 154731 no. of farm holdings.

2. material and methods

This research was undertaken within the Medak District of Telangana State, focusing on farms cultivating rice during the year 2023. Data for the study were gathered through face-to-face surveys conducted on sixty rice-producing farms in Medak district. The selection of farms for the survey was determined using a simple random sampling method. The formula for this method is outlined as follows

n=N*s*s*t*tN-1*d2+s2*t2

Where

n = the volume of sample,

s = the standard deviation,

t = the t value of the 95% confidence interval (1.96),

N = the number of farms belonging to the sampling frame and

d = desired margin of error or allowable error

Finally energy use efficiency, specific energy, energy productivity and net energy were

determined by applying standard equations (Hatirli et al., 2008 and Mohammad et al., 
      2010).

Energy use efficiency = (output energy[MJha-1])(input energy [MJha-1] …………………………..(1)

Specific energy    = (input energy[MJha-1   ])(Rice yield[Kgha-1])…………………………………(2)

Energy productivity=  (Rice yield[Kgha-1])(input energy[MJha-1])…………………………….(3)

Net energy=output energy (MJha-1) – input energy(MJha-1)…..	(4)

Energy intensiveness= Energy input  (MJha-1   )Cost of cultivation   (Rs ha-1 )    ……………………………(5)

Agrochemical energy ratio was calculated by applying Equations

Agrochemical energy ratio = input energy of agrochemicals(MJha-1)total input energy (MJ ha-1) ………..(6)

The following equation was used in the calculation of fuel consumption per hectare for each field operation. (Moerschner and Gerowitt, 2000):

ED = h × AFU × PEU  ×RU

where:

ED = Specific direct energy use (fuel) for a field operation, MJ ha-1.

h = Specific working hours per run, h ha-1

AFU = Average fuel use per working hour, L h-1

PEU = Specific energy value per litre of fuel, MJ L-1

RU = Runs, number of applications in the considered field operation

Table 1. Energy equivalents of input and output in Rice production systems.

| Equipment /inputs | Unit | Energy equivalents | Reference |
| --- | --- | --- | --- |
| A. Inputs |  |  |  |
| 1. Human Labor | H | 1.96 | (Ajay et al., 2025 and Yilmaz et al., 2005) |
| 2. Machinery | h | 62.50 | (Ajay et al., 2025 and Esengun et al., 2007) |
| 3. Diesel fuel | L | 51.33 | (Gaurang Meher Diljun et al., 2022 and Seyed et al., 2013) |
| 4. Chemical Fertilizer | Kg |  |  |
| (a) Nitrogen |  | 66.14 | (Gaurang Meher Diljun et al., 2022 Erdal et al., 2007 ) |
| (b) Phosphate (P2O5) |  | 12.44 | (Ajay et al., 2025 and Rafiee et al., 2010) |
| 5. FYM |  | 0.3 | (Seyed et al.,2013) |
| 6. Chemical |  | 120 | (Erdalet al.,2007 and Ozkan et al., 2007) |
| 7.Seed | Kg | 14.7 | ( Ventkat et al., 2024 and Ozkan et al., 2004 ) |
| B. Output |  |  |  |
| 1. Rice | Kg | 14.7 | (Ventkat et al., 2024 and Mandal et al., 2002) |

Table 2. Energy equivalents of input and output in Rice production systems in Medak district

| Quantity | Quantity per unit area (ha) | Total energy equivalents (MJha1-) | Percentage of total energy (%) |
| --- | --- | --- | --- |
| A. Inputs |  |  |  |
| 1. Human Labour (h) | 269 | 527.24 | 3.04 |
| 2. Machinery (h) | 17 | 1065.9 | 6.14 |
| 3. Diesel fuel(L) | 72 | 4054.32 | 23.37 |
| 4. Chemical Fertilizer(kg) |  |  |  |
| (a) Nitrogen | 101 | 6697.99 | 38.61 |
| (b) Phosphate (P2O5) | 114 | 1413.18 | 8.15 |
| (d) FYM | 9880 | 2964 | 17.08 |
| 5. Pesticides(kg) | 7.5 | 555 | 3.20 |
| 6. Seed(kg) | 20 | 72 |  |
| Total energy input(MJ) |  | 17349.64 | 100 |
| B. Output |  | 0 |  |
| 1. Rice | 7410 | 14079 | 100 |
| Total energy output(MJ) |  | 14079 | 100 |

Fig1. Percentage of energy inputs in Rice production system in Medak district

3. RESULTS AND DISCUSSION

The study unveiled that the average production cost per hectare of Rice crop amounted to Rs. 44,000. Table 2 presents a breakdown of inputs utilized and outputs in Rice production systems, along with their energy equivalents and percentages of the total energy input. Results indicated that the total energy input in Rice production systems was 17349.64 MJ/ha, which is in line with a study conducted in Punjab, India (energy input in the range 52,400 ± 13,000 MJ ha-1 (Sing et al.,2019). The, Nitrogen fertilizer employed in Rice production systems accounted for the highest share at 38.61% (see Fig. 1). Diesel fuel energy ranked second with 23.37% contribution to the total energy input. Seed, on the other hand, represented the smallest share of the total energy input at 0.41%. Additionally, the study observed a Rice grain yield of 7410 kg/ha, equating to a total energy equivalent of 17349.642.MJ/ha. Table 3 presented the energy indicators for Rice production systems.

Table 3. Indicators of energy use in Rice production systems

| Indicators | Unit | Quantity |
| --- | --- | --- |
| Inputs energy | MJha-1 | 17349.64 |
| Output energy | MJha-1 | 14079.00 |
| Grain yield | Kgha-1 | 7410.00 |
| Energy use efficiency |  | 0.81 |
| Specific energy | MJkg-1 | 2.34 |
| Energy productivity | KgMJ-1 | 0.43 |
| Agrochemical Energy Ratio | % | 0.33 |
| Net energy | MJha-1 | -3270.64 |
| Energy intensiveness | MJRs-1 | 0.39 |

Notably, from table 3 the energy efficiency, represented by the output-input ratio, was 0.81. If energy use efficiency is above 1, the production system generates energy The lower energy use efficiency observed in rice production systems can be attributed to the elevated energy inputs, particularly the consumption of Nitrogen fertilizer

From the study it is observed that in rice production systems, the energy productivity, denoting the grain yield per unit of energy input, was  0.43 kg MJ⁻¹, while the specific energy, indicating the input energy required per unit of grain yield, was 2.34 MJ kg⁻¹. A lower value of specific energy is desirable as it indicates higher energy efficiency in production. Put differently, for every MJ of input energy, 0.43 kg of rice grain was produced, or conversely, 2.33 MJ of energy was expended to yield one kilogram of grain. Furthermore, the system's net energy, is calculated as the output minus input,  which is amounted to 3270.64 MJ ha-1. The net energy is low because of less yield in the study area. A high agrochemical ratio implies a large agrochemical footprint and negative environmental effects such as nitrogen leaching, air and water pollution and GHG emissions (Pishgar-Komleh et al.,2013). In this study, the agrochemical energy ratio was 0.33% of the input energy, which is desirable. Additionally, the energy intensiveness, indicating the amount of energy produced per rupee spent, was computed at 0.39 MJ Rs-1, signifying that for each rupee invested, 0.39 MJ of energy could be generated. The energy consumption of different implements/machinery is shown in table 4 From table it can be concluded that cultivator consumed less fuel energy than puddler. Combine harvester consumed highest fuel energy.

Table 4 Fuel energy of implements/machinery

| Machine/Implement | Fuel energy MJ per ha |
| --- | --- |
| Cultivator | 985.43 |
| Puddler | 1666.78 |
| Combine harvester | 1875.12 |

CONCLUSION

Energy inputs and output was investigated for rice production system in Medak district. This study reveals significant opportunities for enhancing energy efficiency in agricultural operations. The findings indicate that the current energy use efficiency is 0.81, with a negative net energy of -3270.64 MJ ha⁻¹ and an energy productivity of 0.43 kg MJ⁻¹. There is need to increase energy use efficiency above 1, net energy and energy productivity for making rice production system more efficient. Energy use efficiency above 1.0 with positive net energy and enhance energy productivity is feasible by adopting integrated nutrient management, use appropriate-scale machinery, crop diversification to reduce nitrogen fertilizer, and adopting energy-conscious farming practices. Such improvements would not only enhance the energy productivity beyond the current 0.43 level but also contribute to sustainable agricultural intensification while reducing production costs for farmers in the region. In Medak district, effort should be made at increasing the level of rice production through mechanisation and move from the use of energy from non-renewable sources to renewable sources to attain a self-sufficient and sustainable rice production system.

References

Ajay kumar, baheliya., Neeraj kumar., Ram ratan.,  Sunil kumar.,Dharmendra kumar., and krishna kumar patel.(2025). Indian Journal of Agronomy.70:8-12

Babajide S. Kosemani.and  Isaac Bamgboye,A.(2020).Energy Input- output Analysis of Rice Production in Nigeria.Energy.

Demircan, V., Ekinci, K., Keener, D., Akbolat, C. and Ekinci, (2006). A case study from Isparta province. Energy Convers Manage. Energy and Economic Analysis of Sweet Cherry Production in Turkey. 47: 1761-1769.

Erdal, G., Esengun, K., Erdal, H. and Gunduz, O. (2007). Energy use and economical analysis of sugar beet production in Tokat province of Turkey. Energy. 32: 35-41.

Esengun, K., Gunduz, O. and Erdal, G. (2007). Input-output energy analysis in dry apricot production of Turkey. Energy Convers Manage. 48: 592-598.

Gaurang Meher Diljun., Vinay Shankar Prasad Sinha., Meena Chamola., Prashant Singh., Ashutosh Mishra and Rajendra Dobhal,(2022).Current Science .123:881-886

Seyed Mohammad Hossein Tabatabaie, Shahin Rafiee, Alireza Keyhani, Mohammad Davoud Heidari, (2013). Energy use pattern and sensitivity analysis of energy inputs and input costs for pear production in Iran. Renewable Energy. 51: 7-12.

Hatirli, S.A., Ozkan, B. And Fert, C. (2008).Energy inputs and crop yield relationship in greenhouse tomato production. Renew Energy. 31: 427- 438.

Khan, S., Khan, M.A. and Hanjra, J. Mu. (2009). Pathways to reduce the environmental footprints of water and energy input in food production. Food policy. 34: 141- 149.

Hilly, M., Adams, M. L., & Nelson, S. C. (2002). A study of digit fusion in the mouse embryo. Clinical and Experimental Allergy, 32(4), 489-498

Kizilaslan, H. (2009). Input-output energy analysis of cherries production in Tokat Province of Turkey. Ap- plied Energy. 86: 1354–1358.

Mandal, K.G., Saha, K.P.., Ghosh, P.K., Hati, K.M. and Bandyopadhyay, K.K. (2002) Bioenergy and eco- nomic analysis of soybean-based crop production systems in central India. Biomass Bioenergy. 23(5): 337-345.

Moerschner, J. and B. Gerowitt. (2000). Direct and in-direct energy use in arable farming- 
                in   Northern Germany. In: Weidema, B.P. and M.J.G Meeusen (Eds.), Agricultural 
               Data for Life      Cycle Assessments. The Hague, Agricultural Economics Research 
              Institute (LEI). Volume 1.   195 p.

Mohammad, A., Rafiee, S., Mohtasebi, SS. and Rafiee, H. (2010). Energy inputs-yield relationship and cost analysis of kiwifruit production in Iran. Renewable Energy. 35: 1071-1075.

Ozkan, B., Akcaoz, H. and Fert, C. (2004). Energy input– output analysis in Turkish agriculture. Renewable Energy. 29: 39-51.

Ozkan, B., Fert, C. and Karadeniz, C.F. (2007) Energy and cost analysis for greenhouse and open file grape production and open- field grape production. Energy. 32: 1-4.

Pishgar-Komleh, S. H., Omid, M. and Heidari, M. D.(2013).  On the study of energy use and GHG (greenhouse gas) emissions in greenhouse cucumber production in Yazd Province. Energy. 59: 63–71.

Rafiee, S., Mousavi, S.H. and Mohammadi, A. (2010). Modeling and sensitivity analysis of energy inputs for apple production in Iran. Energy. 35(8): 3301-6.

Singh, H., Mishra, D. and Nahar, N.M. (2002). Energy use pattern in production agriculture of typical village in arid zone, India-part-I. Energy Convers Manag. 43: 2275–2286.

Singh, P., Singh, G. and Sodhi, G. P. S. (2019) Energy auditing and optimization approach for improving energy efficiency of rice cultivation in south-western Punjab, India. Energy. 174, 269–279.

Streimikiene, D., Klevas, V. and Bubeliene, J. (2007). Use of EU structural funds for sustainable energy develop- ment in new EU member states. Renew Sustain En- ergy Rev. 116: 1167-87.

Taheri Garavand, A., Asakereh, A. and Haghani, K. (2010). Energy elevation and economic analysis of canola production in Iran a case study: Mazandaran prov- ince. International Journal of Environmental Sciences. 1(2): 236- 243.

Venkat,Regatti., Sai Mohan, S.,  Rahaman,S.,  Vinayak,M., Hari Babu,B.,  Rami Reddy,K.V.S. (2024).Energy Assessment of Manual Transplanting Rice and Dry Direct Seeding Rice Production Systems in Combined Nalgonda District, Telangana. Indian Journal of Agricultural Research.58: 95-100

Yilmaz, I., Akcaoz, H. and Ozkan, B. (2005) An analysis of energy use and input costs for cotton production in Turkey. Renewable Energy. 30(2): 145-55.

.