Poultry Waste Utilisation: A Nutrient-Rich Organic Resource for Sustainable Agriculture

ABSTRACT

Poultry waste is a good source of organic waste to raise crops like wheat due to its nutrient composition and availability. Fertiliser use is an integral part of the crop production system, and fertilisers are non-poisonous materials, unlike pesticides. Due to the high cost of synthetic fertiliser and demand for organic food, the farmers are more interested in adopting organic farming practices, and poultry manure will replace the fertiliser. This manure can serve as organic fertiliser, an effective soil amendment, a very good animal feed and as a good source of bio-fuel. Broiler litter can fulfil a sufficient amount of Fe, Cu, Mn and Zn; moreover, it may be used as a very good source of micronutrients for cotton production. Different methods can be used to turn over the poultry manure into a value-added product, such as compost and pellets. Studies have been done to determine the composition of the poultry manure. Balanced application should be applied on the wheat crop so that farmers do not use more than the amount required and do the cause leaching of productive soil. Comparative study of broiler and layer litter, where broiler litter is said to have more nutrient content, and comparative study of farm yard manure and poultry manure (PM) has been done, and the result found is that PM is best in its quality, with more nitrogen content in it. In the case of the housing system, deep litter gives better quality litter than cage and broiler manure. Everything is poison without a dose, so even if poultry manure has a good nutrient amount, it should be used sustainably to avoid its cons.

Keywords: Bio-fuel, Deep-litter, Leaching, Poultry waste, Organic waste

INTRODUCTION

During the last several decades, sustainable development has become one of the most important developmental priorities worldwide. “The concepts of sustainability and sustainable development have acquired great relevance in scientific research about environmental issues, policies linked to environmental management, and industrial and agricultural production, among others”(Ruggerio,2021; Quintana‐García et al.,2022). Although “the concept of sustainable development was officially promoted in the late 1980s, there is still no precise definition of this phenomenon” (Manesh et al., 2020). “Poultry manure, properly handled, is the most valuable of all manures produced by livestock. A large amount of livestock manure is generated worldwide, most of which is placed directly onto open land or treated as organic fertilisers for agricultural production in rural areas” (Zhang et al.,2022). “It has historically been used as a source of plant nutrients and as a soil amendment” (Kannan et al., 2015). “The number of intensive livestock and poultry farms is expected to increase substantially in future because of consumer demand” (Zhang et al.,2021; Peng et al.,2022). However, “in areas of intense poultry production, over-fertilisation of pasture land with poultry manure occurs” (Bamire & Amujoyegbe, 2004). “The result is suspected groundwater and surface water problems as excess nutrients run off the land or leach into groundwater supplies” (Moore et al., 1995). “To obtain maximum economic value of plant nutrients in poultry manure and to protect our water supplies from excessive nutrient runoff or leaching, poultry manure should be applied to match the nutrient needs of crops” (Sorathiya et al., 2014). “Fertiliser use is an integral part of the crop production system, and fertilisers are non-poisonous material, unlike pesticides” (Subedi & Dhakal, 2015). “It is well recognised that fertilisers are essential for the production of food and fibre crops to cater for the needs of ever ever-increasing population of the world” (Araji et al., 2001). Moreover, fertilisers not only help to maintain the fertility and productivity of cultivated lands but also improve the soil quality and, intern food production.

USE OF POULTRY MANURE:

1. Fertiliser: Poultry litter has been utilised as an organic fertiliser since it is a rich source of nutrients required for plant growth. It has a high concentration of important nutrients, particularly N, P, and K. This manure also contains trace minerals such as Cu and Zn, as well as other micro and macro nutrients. When manure decomposes in the soil, nutrients are released into the soil for plant absorption. Various investigations have revealed that poultry litter manure produces good crop yields. For example, chicken manure application increased corn yields in several trials. Poultry litter is a valuable source of plant nutrients in fertiliser trials with cotton in the United States. It is locally readily available fertiliser at a low cost and can be very useful in reducing vegetable production cost (Tewolde et al.,2005; Hachicha et al., 2006).

2. Soil amendment: Soil structure and general soil quality deteriorate as a result of excessive chemical fertiliser application and continued cultivation of the same crop in the same field. Several studies have shown that adding poultry manure and litter to soil enhances a variety of important soil qualities. Poultry manure has been shown to reduce soil bulk density while increasing water holding capacity, organic matter content, oxygen diffusion rate, and aggregate stability. “When poultry litter is utilised as a mulching material, it helps to keep the soil moist and prevents the surface feeding roots from drying up in the summer heat. Poultry dung can help mining tailings become more biologically fertile. Poultry litter is increasingly being utilised to restore land that has been affected by mining and other industrial activities”.(Frenzluebbers and Doraiswamy, 2007).

3. Animal feed: Manure from chickens can be fed to cattle and fish. Foreign elements in the litter, including plastic, feathers, and glass, must be removed before it can be used as feed. Poultry litter with an ash content of more than 28% cannot be utilised as feed, hence, a low ash content is required. Pathogenic microbes such as Clostridium, Salmonella, and Enterobacter spp. are found in unprocessed chicken litter, as well as feed additives that are supplied to the poultry diet and can be found in the litter as waste byproducts such as antibiotics, coccidiostats, and arsenicals. To use as feed, adequate processing is required to remove these contaminants from the litter. Protein, minerals, and fibre have been added to properly processed poultry dung and litter, which has been blended with cattle feed for these nutrients. In the U.S., they have been used as a useful ingredient for around 40 years.

4. Fuel source: Poultry litter with a moisture content of less than 15% can be burned directly as fuel to generate heat energy. Anaerobic digestion of poultry manure produces biogas, a very valuable combustible gas containing roughly 60% methane. Biogas created from poultry litter can be used for a variety of reasons, including motor fuel, electricity generation, and simple heat generation. Many states in the United States are producing large amounts of turkey and broiler waste to be used as renewable green energy sources.

CONVERSION OF POULTRY LITTER:

Composting: Because fresh poultry litter includes numerous pathogenic organisms, toxic chemicals, and weed seeds, it must be composted before being used as fertiliser. Fresh chicken dung should be blended with various carbon sources to compost. A bulking agent that aids aeration of the compost pile is also required. The pile temperature should be kept between 55 and 65 degrees Celsius for at least two weeks. Turning the pile on a regular basis is necessary to homogenise it; turning and maintaining a pasteurising temperature of 55-65°C for two weeks kills all weeds and pathogens and produces compost that is not fully mature. The resulting compost is unstable and has a strong odour, but it is still a good fertiliser free of weeds and pathogens.

Pellets out of manure: As poultry manure contains NPK in a renowned amount, which are very useful nutrients for plant growth, but the restriction is in using fresh manure in food crops, which can lead to unacceptable residues associated with fungi, parasites, pathogens and any noxious odour. This may cause adverse effects on the health of consumers. The long transportation of raw poultry manure is debatable. To convert this problem of dehydration of litter is a commendable substitute. “Pelleting is a process that eliminates microorganisms and odour in broiler litter. The pelleting of broiler litter was clearly preferable to the fresh broiler litter in terms of storage, handling, transport and field application due to decreased dry matter, electrical conductivity, urea N, K, Na, S, Fe, Cu and Cd contents. The litter of the layer and meat quails was also successfully pelleted in one study in Thailand” (Suppadit et al., 2011).

Table 1-Composition of pelleted broiler and layer litter from various countries:

| Nutrient% | Broiler manure pellets |  |  | Layer manure pellets |  |
| --- | --- | --- | --- | --- | --- |
| DM | 84.6 | - | - | - | - |
| Nitrogen | 4.16 | 2.57 | - | 3.28 | - |
| Phosphorous | 1.68 | 1.06 | 1.60 | 1.08 | 1.80 |
| Potassium | - | 1.01 | 0.90 | 1.52 | 1.00 |
| Ash | 19.9 | - | 13.27 | - | 12.65 |
| Calcium | 3.72 | 1.62 | 2.20 | 1.28 | 1.80 |
| Magnesium | - | 0.35 | 0.80 | 0.62 | 1.10 |
| Sulphur | - | 0.52 | 0.30 | 0.85 | 0.28 |
| Country | Thailand | Australia | Nigeria | Australia | Nigeria |

Drying of poultry manure reduces the presence and offensive odour by 65.3% and 69.3 %, respectively. Further, significant reductions in bacteria (65.6-99.8 %), yeast, mould (74.1-99.6 %) and E. coli (99.97 %) were achieved. Other than NPK, poultry manure also contains Mo, Ca, Mg, Cu, Fe, S, Mn, Bo, Co and Zn, which are lacking in commercial fertiliser and can be compensated by poultry manure. (Sorathiya et al.,2014)

- Poultry waste is a good organic source of nutrients for raising crops, such as wheat. Crop productivity is believed to increase by poultry waste application as a fertiliser due to its nutrient composition and availability.

- Poultry waste application brought higher yields to poultry waste users with comparatively low amounts of chemical fertilisers applied than to poultry waste non-users.

- Higher rates of returns in terms of revenue and overall increase in gross margins can be achieved due to the better plant growth impacts of poultry waste.

- Variables involved in the study, like land preparation cost, sowing method, chemical fertilisers, plant protection cost, labour cost and poultry waste showed significant effect on yield.

- Poultry waste had the most significant effect on the yield of wheat with highly significant behaviour (p<0.05) and coefficient value of 1.644. (Memon et al., 2016).

Table 2-Summaries of variables included in the analysis. (Memon. et.al 2016)

| Variables | Unit | Frequency | Minimum | Maximum | Mean | Standard deviation | Variance |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Land preparation | Rs. | 120 | 2350 | 4550 | 3578.38 | 552.659 | 305432.4 |
| seed | Kg. | 120 | 50 | 60 | 51.88 | 0.402 | 0.162 |
| Irrigation | no | 120 | 3 | 05 | 3.94 | 0.456 | 0.208 |
| Chemical fertilizers | kg | 120 | 55 | 110 | 81.1 | 13.996 | 195.889 |
| Poultry waste | trolley | 60 | 0.5 | 1.5 | 0.913 | 0.259 | 0.067 |
| Plant protection | Rs. | 120 | 0 | 1970 | 1060 | 525.959 | 276633.6 |
| labour | Rs. | 120 | 0 | 3500 | 2466.25 | 530.161 | 281071 |
| yield | mound | 120 | 35 | 48 | 40.95 | 2.319 | 5.376 |

“An annual fertilisation strategy with 1 to 2 tons/ac of poultry (turkey) litter and recommended supplemental nitrogen maximises nitrogen on-farm profitability for cultivated crop production off-site of poultry production facilities. Treatments with commercial (inorganic) fertiliser only and 3 tons/ac litter with supplemental nitrogen also produced relatively high profits, but greater litter rates rapidly decreased profits. The economically optimal range of litter rates coincides with the environmentally optimal rate in terms of minimising nutrient runoff, preventing rapid build-up of soil phosphorus and micronutrients, and providing a sustainable fertiliser alternative. In regions with concentrations of surplus litter, the present results support a win-win scenario for environmentally friendly litter utilisation. In this scenario, farmers and ranchers gain a cost-effective fertiliser alternative, the public saves remediation and water treatment costs, and the poultry industry enhances its public perception, avoids litigation, and increases its revenue potential”. (Harmelet al., 2005)

Table 3-Benefits of pelletizing poultry manure over dilute poultry manure (Karasahın, M. 2014)

| Components | Pelleted poultry manure | Components | Diluted poultry manure |
| --- | --- | --- | --- |
| Total n% | 2.30 | Total n% | 0.42 |
| P2o5 % | 5.86 | P (ppm) | 343.50 |
| K20 % | 3.31 | K (ppm) | 350.82 |
| Organic matter % | 61.40 | Organic matter % | 2.07 |

Poultry manure pelleting is much much-improved technique and give more benefits then using diluted poultry manure. As you can see in the above-mentioned table, the total nitrogen is more when pelletized. When pellets are applied to ground, there will be a slow release of nutrient contents as compared to diluted ones. When irrigation is done, the pellets will not run off, which is the biggest benefit of all.

EFFECT OF FEEDING POULTRY MANURE AND MILK YIELD IN HOLSTEIN COWS:

40 lactating cows were fed 0,5,9,11.7, or 17.4% processed poultry excreta in total mixed ration. The effect of treatment was evaluated on feed intake, BW, milk yield and composition. Processed poultry excreta is well accepted by cows even when included in the ration at 17.4% of total DM. Mean DM intake averaged 19.3,19.7,19.5, and 19.7kg/ d for cows fed 0,5.9,11.7, and 17.4% processed poultry excreta, respectively. Body weight change averaged 2.0,1.7-1.7and 1.4kg/ week from 90 to 180d lactation for cows fed from 0 to 17.4% processed poultry excreta, respectively. Fat-corrected milk (4%)yield was similar among treatment groups, averaging 24.2, 26.1, 24.6, and 25.1kg/ d for cows fed 0,5.9,11.7 and 17.4% processed poultry excreta. No differences were found for milk and fat yield or percentage fat among treatment means. No off-flavours in milk could be attributed to processed poultry excreta added to feed. Processed poultry excreta fed up to 17.4% of total mixed rations supported both adequate feed intake and high milk yield.

Table 4-Nutrient content of different types of poultry manure

| Sources | Typical moisture content | Typical (lbs/ton) | Nutrient | Content |
| --- | --- | --- | --- | --- |
|  |  | N | P | K |
| Layer | High | 35 | 40 | 20 |
| Pullet | Low | 40 | 45 | 40 |
| Breeder | High | 40 | 60 | 40 |
| Turkey | Low | 60 | 60 | 55 |
| Broiler | Low | 60 | 60 | 55 |

| Particular | Deep litter | Broiler | Cage litter |
| --- | --- | --- | --- |
| C/n ratio | 9.5-11.5 | 9.4-11.2 | 5.8-7.6 |
| Total n% | 1.70-2.20 | 2.40-3.60 | 3.66-5.30 |
| Total p2o5% | 1.41-1.81 | 1.56-2.80 | 1.54-2.90 |
| Total k205% | 0.93-1.30 | 1.40-2.31 | 2.5-2.90 |
| Fe (ppm) | 930-1380 | 9.70-1370 | 970-1450 |
| Zn (ppm) | 90-309 | 160-315 | 290-460 |
| Cu (ppm) | 24-42 | 27-47 | 80-172 |
| Mn (ppm) | 210-380 | 190-350 | 370-590 |
| Ca % | 0.90-1.10 | 0.86-1.11 | 0.80-1.02 |
| Mg % | 0.45-0.68 | 0.42-0.45 | 0.40-0.56 |

Table 5: Nutrient availability of poultry manure:

Table 6-Poultry manure (PM) vs farm yard manure (FYM) (Ghosh, P. K. et al., 2004):

| Chemical characteristics | Farm yard manure | Poultry manure |
| --- | --- | --- |
| Total n% | 0.73 | 2.14 |
| P% | 0.18 | 1.09 |
| K% | 0.71 | 1.23 |
| S% | 0.58 | 0.72 |
| Zn (mgkg-1) | 16 | 24 |
| Total carbon% | 25.9 | 31.3 |
| Lignin % | 13.8 | 14.1 |
| Cellulose % | 38.3 | 30.1 |

Application of farm yard manure is a way old process in India but its availability is at stake because of increase in cropping intensity and area, other competitive uses of cow dung. In central India, mechanization is increasing causing reduction of farm animal so only a small amount of FYM is available per ha. Further, FYM transportation is high and only a small amount of major nutrient is available compare to poultry manure. As day-by-day poultry industry is rising, we can take out more benefit from it. It is a cheap and organic source of manure which contains both macro and micro nutrients. As mentioned in the earlier table, it can increase the nitrogen content of soil, which will increase the soil porosity and enhance microbial activity. As PM contains a high amount of nutrients compared to FYM, only a small amount will also work, whereas if you add FYM, you will need a high quantity.

GUIDELINES TO USE POULTRY MANURE:

- Effects of poultry manure on water quality will be high if it is excessively used, which will result in NO3-N leaching into the tile drains and potentially contaminate both surface and groundwater (Dinneset at., 2002).

- Higher rate of pelletized poultry manure in combination with mineral fertiliser may affect the receiving water quality via leaching for nitrate and runoff for phosphate, and therefore appropriate rate of application must be determined.

- Poultry litter can have an impact on soil pH and liming due to varying amounts of calcium carbonate in poultry feed. (Mullins et al., 20020)

- Poultry litter should be evenly distributed over application sites at a rate not to exceed 11.2 metric tons/ha/year, with no more than 5.6 metric tons/ha/year in each application (as a rule of thumb, 30 A for one 16,000-ft2 house/year).

- Surface land application of poultry waste should not be undertaken when the soil is saturated, frozen, or covered with snow or during rainy weather.

- Poultry waste should not be applied on land with a greater than 15% slope or in any manner that will allow waste to enter the waters of the state.

- Surface and subsurface application of poultry waste should not be made within 8 m of rock outcrops; 30 m of streams, ponds, lakes, springs, sinkholes, wells, water supplies, and dwellings.

CONCLUSION

Best out of waste is always been a motto which we can apply to any field. For the sustainable use of our earth's products, we can always replace them through science to improve the environmental condition and their economic use. The production cost of fodder crops can be reduced by the use of poultry manure. The poultry manure can be easily stored and transported after drying and pelleting. It may be a good source of non-protein nitrogen (NPN) that can be fed to ruminants to reduce the feed cost. The manure pelleting industry can also generate employment for rural youth and poultry farmers in order to improve profit and the environment. The only thing to keep in mind while usage of PM should be used in limited amounts in order to save the groundwater water leaching. Everything is poison without a dose, so even if poultry manure has a good nutrient amount, it should be used sustainably to avoid its cons.

Principal outcome of the work:

- Poultry manure can be the new face of organic fertiliser, which has the property of soil amendment and bio-fuel.

- There is a comparison drawn between broiler litter-layer litter and farmyard manure-poultry manure, where broiler litter and poultry manure are found to be more nutritive.

- There can be different versions of poultry manure eg, pellets, which can be another source of income.

- Poultry manure can also be fed to cattle to supplement the nutritional requirements.

- It can be a futuristic approach for sustainability.

# COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

REFERENCE:

- Memon, I. N., M. I. Kumbhar, and S. Noonari. "Economics of Poultry Waste Use as a Fertilizer in Sindh Pakistan." Journal of Fisheries & Livestock Production (2016): 1-9.

- Harmel, R. Daren, B. Harmel, and M. C. Patterson. "On-farm agro-economic effects of fertilizing cropland with poultry litter." Journal of Applied Poultry Research 17.4 (2008): 545-555.

- Sorathiya, L. M., Fulsoundar, A. B., Tyagi, K. K., Patel, M. D., & Singh, R. R. (2014). Eco-friendly and modern methods of livestock waste recycling for enhancing farm profitability. International Journal of Recycling of Organic Waste in Agriculture, 3(1), 1-7.

- Amanullah, M. M., Somasundaram, E., Vaiyapuri, K., &Sathyamoorthi, K. (2007). Poultry manure to crops: A Review. Agricultural reviews-agricultural research communications centre India, 28(3), 216.

- Chapman, S. L. (1996). Soil and solid poultry waste nutrient management and water quality. Poultry Science, 75(7), 862-866.

- Tasneem, K., Tariq, M., Kamal, J., & Masood, A. (2004). Effectiveness of farmyard manure, poultry manure and nitrogen for corn productivity. International Journal of Agriculture and Biology, 6(2), 260-263.

- Tewolde, H. K. R. S., Sistani, K. R., & Rowe, D. E. (2005). Broiler litter as a micronutrient source for cotton: Concentrations in plant parts. Journal of environmental quality, 34(5), 1697-1706.

- Suppadit, T., Phumkokrak, N., &Poungsuk, P. (2012). The effect of using quail litter biochar on soybean (Glycine max [L.] Merr.) production. Chilean Journal of Agricultural Research, 72(2), 244.

- Harmel, R. D., & King, K. W. (2005). Uncertainty in measured sediment and nutrient flux in runoff from small agricultural watersheds. Transactions of the ASAE, 48(5), 1713-1721.

- Dinnes, D. L., Karlen, D. L., Jaynes, D. B., Kaspar, T. C., Hatfield, J. L., Colvin, T. S., &Cambardella, C. A. (2002). Nitrogen management strategies to reduce nitrate leaching in tile‐drained Midwestern soils. Agronomy journal, 94(1), 153-171.

- Franzluebbers, A.J. and Doraiswamy, P.C. (2007). Carbon sequestration and land degradation. Climate and land degradation. International Workshop on Climate and Land Degradation, 343-358.

- Mullins, G.L., E.S. Bendfeldt, and R.A. Clark. 2002. Poultry litter as a fertilizer and soil amendment. 13 June 2007

- Bamire, A. S., & Amujoyegbe, B. J. (2004). Economics of poultry manure utilization in land quality improvement among integrated poultry-maize-farmers in southwestern Nigeria. Journal of sustainable Agriculture, 23(3), 21-37.

- Moore Jr, P. A., Daniel, T. C., Sharpley, A. N., & Wood, C. W. (1995). Poultry manure management: Environmentally sound options. Journal of soil and water conservation, 50(3), 321-327.

- Manesh, M. K., Rezazadeh, A., & Kabiri, S. J. R. E. (2020). A feasibility study on the potential, economic, and environmental advantages of biogas production from poultry manure in Iran. Renewable Energy, 159, 87-106.

- Subedi, S., & Dhakal, D. (2015). Economics of poultry manure as an alternative to chemical fertilizer for agricultural production in Nepal. Journal of the Institute of Agriculture and Animal Science, 259-268.

- Sorathiya, L. M., Fulsoundar, A. B., Tyagi, K. K., Patel, M. D., & Singh, R. R. (2014). Eco-friendly and modern methods of livestock waste recycling for enhancing farm profitability. International Journal of Recycling of Organic Waste in Agriculture, 3, 1-7.

- Araji, A. A., Abdo, Z. O., & Joyce, P. (2001). Efficient use of animal manure on cropland–economic analysis. Bioresource technology, 79(2), 179-191.

- Kannan, P., Balasubramaniyan, P., Mahimairaj, P., & Prabukumar, G. (2015). Developing and evaluation of ecofriendly fertilizer from poultry waste to enhance pulse production in alfisols of semi-arid tropics. Communications in Soil Science and Plant Analysis, 46(2), 195-209.

- Hachicha, S., Chtourou, M., Medhioub, K., & Ammar, E. (2006). Compost of poultry manure and olive mill wastes as an alternative fertilizer. Agronomy for Sustainable Development, 26(2), 135-142.

Ruggerio, C. A. (2021). Sustainability and sustainable development: A review of principles and definitions. Science of the Total Environment, 786, 147481.

Zhang, X., Gong, Z., Allinson, G., Xiao, M., Li, X., Jia, C., & Ni, Z. (2022). Environmental risks caused by livestock and poultry farms to the soils: Comparison of swine, chicken, and cattle farms. Journal of Environmental Management, 317, 115320.

Zhang, J. B., Zhang, J., Li, J. H., Tomerlin, J. K., Xiao, X. P., Ur Rehman, K., ... & Yu, Z. N. (2021). Black soldier fly: A new vista for livestock and poultry manure management. Journal of Integrative Agriculture, 20(5), 1167-1179.

Peng, S., Zhang, H., Song, D., Chen, H., Lin, X., Wang, Y., & Ji, L. (2022). Distribution of antibiotic, heavy metals and antibiotic resistance genes in livestock and poultry feces from different scale of farms in Ningxia, China. Journal of Hazardous Materials, 440, 129719.

Quintana‐García, C., Marchante‐Lara, M., & Benavides‐Chicón, C. G. (2022). Towards sustainable development: Environmental innovation, cleaner production performance, and reputation. Corporate Social Responsibility and Environmental Management, 29(5), 1330-1340

Karasahın, M. (2014). Effects of cultivar and fertilization on plant and silage crude protein contents in maize. Journal of Agricultural Faculty of Gaziosmanpaşa University (JAFAG), 2014(2), 98-104.

Ghosh, P. K., Ramesh, P., Bandyopadhyay, K. K., Tripathi, A. K., Hati, K. M., Misra, A. K., & Acharya, C. L. (2004). Comparative effectiveness of cattle manure, poultry manure, phosphocompost and fertilizer-NPK on three cropping systems in vertisols of semi-arid tropics. I. Crop yields and system performance. Bioresource technology, 95(1), 77-83.