Integrated Wastewater Treatment and Pollution Monitoring System for Sustainable Water Management in Lagos, Nigeria

### Abstract

The growing demand for clean water in Lagos, Nigeria, a megacity grappling with rapid urbanization and industrialization, underscores the urgent need for sustainable water management systems. This review examines the current state of wastewater treatment and pollution monitoring in Lagos, highlighting the challenges posed by population pressure, insufficient infrastructure, and inadequate regulatory frameworks. It evaluates both traditional and emerging wastewater treatment technologies, including membrane bioreactors, constructed wetlands, and decentralized systems, while also assessing the role of advanced pollution monitoring tools such as IoT  based sensors, remote sensing, and machine learning. The paper advocates for integrated systems that combine treatment and real time monitoring to enhance water quality and efficiency. Drawing on global case studies and best practices, it explores how Lagos can leverage partnerships, local innovations, and policy reforms to implement context specific solutions. The review concludes by identifying key challenges such as funding and public awareness, while also pointing out opportunities for innovation and stakeholder engagement. Overall, the study emphasizes the importance of integrated and sustainable approaches to securing water quality and access in urban African contexts.

Keywords:   Integrated wastewater treatment, Pollution monitoring, Sustainable water management, Lagos, Nigeria,  IoT sensors

### 1.0 Introduction

Water management and pollution monitoring are critical components of sustainable development, particularly in rapidly growing urban areas. In cities like Lagos, Nigeria, where population growth is outpacing infrastructural development, water resources are under significant pressure (Akinbile & Yusoff, 2012). Lagos, with its more than 23 million residents, is one of the most populous cities in Africa, and its rapid urbanization presents numerous challenges related to water quality, access to clean water, and efficient waste management (Akinyemi et al., 2021; Lako & Çomo, 2024). This urban sprawl has led to increased industrialization, poor waste management practices, and the contamination of both surface and groundwater sources, which significantly affect the health and well  being of its inhabitants (Festus et al., 2020). Moreover, due to Lagos’s position as an economic hub and a major coastal city, the pollution from untreated wastewater and industrial effluents not only affects local residents but also contributes to broader environmental degradation, including pollution of water bodies and loss of biodiversity (Oluwasemilore & Dada, 2020; Karn et al., 2023).

Despite the growing challenges, wastewater treatment and pollution monitoring systems have the potential to mitigate these issues by improving the management of water resources and facilitating sustainable practices in the city (Saravanan et al., 2021; Ejairu et al., 2024). However, the current systems in Lagos are inadequate to address the growing demand for clean water and the need for effective wastewater treatment (Siddique, 2021). Existing facilities are often overwhelmed by the population growth, and the lack of advanced technologies in pollution monitoring makes it difficult to detect and address contamination in real  time (Olumide, 2015; Silva, 2023). The city's reliance on outdated treatment systems that often discharge untreated or poorly treated waste into water bodies exacerbates the problem (Akinbile, 2013). In response, there is a growing need to adopt integrated wastewater treatment systems that not only address the treatment of wastewater but also incorporate pollution monitoring capabilities to ensure that water quality standards are met and environmental health is safeguarded (Chen et al., 2006).

The objective of this literature review is to provide an overview of the current state of wastewater treatment and pollution monitoring systems in Lagos, Nigeria, with a focus on how integrated systems can offer a sustainable solution for water management. The review will cover a range of topics, including the challenges associated with water quality in Lagos, the limitations of current wastewater treatment infrastructure, and emerging technologies in wastewater treatment and pollution monitoring. Additionally, the review will explore sustainable water management practices and discuss how integrated approaches can address the unique challenges faced by Lagos in managing its water resources. By evaluating existing studies and frameworks, this review aims to identify opportunities for improving water management and pollution control, contributing to a more sustainable and healthy urban environment in Lagos.

### 2.0 Current State of Water Management in Lagos

#### Overview of Water Resources

Lagos, as one of the most populous cities in Africa, relies heavily on both surface and groundwater resources to meet its water demand. The primary sources of water for the city are the Lagos Lagoon, rivers such as the Ogun and Osun, as well as groundwater aquifers that supply water to households, industries, and commercial establishments (Akinbile & Yusoff, 2012). The city also receives water from several dams, including the Oyan Dam, which serves as a major source of raw water for municipal supply (Oluwasemilore & Dada, 2020). However, these water resources are under immense pressure due to the rapid population growth and urbanization in the city. With over 23 million residents, Lagos faces significant challenges in supplying clean and safe drinking water to its population (Akinyemi et al., 2021). The growing demand for water, coupled with an increasing population, means that these resources are being exploited faster than they can naturally replenish, resulting in water scarcity and quality deterioration in various parts of the city (Oluwasemilore & Dada, 2020).

Figure 1: Water resources management  structure in Nigeria

The increasing rate of industrialization has further compounded the pressure on Lagos’s water resources. Industrial activities, especially in manufacturing and oil sectors, contribute to both the demand for water and the contamination of available water sources (Olumide, 2015). Additionally, the city’s high rate of informal settlements and slums further exacerbates the issue, as many of these areas lack access to proper sanitation facilities and contribute to water contamination through improper waste disposal and lack of wastewater treatment (Festus et al., 2020). The rapid expansion of Lagos’s urban landscape, without adequate water supply infrastructure, has resulted in unsustainable water management practices, placing a heavy burden on the available water resources and highlighting the need for more efficient and sustainable management systems.

#### Challenges in Water Management

Water management in Lagos is plagued by several interconnected challenges, particularly related to water quality. One of the primary issues is the pollution of both surface water and groundwater, which severely limits the availability of clean water for consumption (Akinbile, 2013). Untreated or inadequately treated wastewater from domestic, industrial, and commercial activities often finds its way into water bodies, significantly degrading water quality. Many households and industries lack adequate wastewater treatment systems, leading to the direct discharge of effluents into rivers, streams, and other water bodies. This practice has led to high levels of contamination from pathogens, chemicals, and other pollutants, which poses significant risks to human health and the environment (Akinbile & Yusoff, 2012). Furthermore, the widespread practice of open defecation in slums and informal settlements contributes to fecal contamination of both surface water and groundwater, exacerbating waterborne diseases in the city (Akinyemi et al., 2021).

Industrial effluents are also a major contributor to the pollution of Lagos’s water resources. Factories and industries in and around Lagos, particularly in the Oregun and Apapa areas, discharge untreated wastewater and chemicals into nearby water bodies. These effluents contain harmful substances such as heavy metals, organic compounds, and hazardous chemicals, which are toxic to both aquatic life and humans (Festus et al., 2020). The lack of effective regulation and enforcement of environmental laws has allowed these practices to persist, contributing to the ongoing degradation of water quality in the city. The degradation of the water supply, in turn, leads to an increased burden on wastewater treatment facilities, which are already under  resourced and unable to cope with the volume of pollution (Oluwasemilore & Dada, 2020).

Another significant challenge in water management in Lagos is the inadequate waste disposal systems. While waste collection services exist in some parts of the city, many informal settlements are not covered by these services, leading to the indiscriminate disposal of waste in open areas and water bodies. This improper waste disposal leads to the accumulation of plastic waste, food waste, and hazardous materials, all of which further pollute the environment and water resources (Akinbile, 2013). This situation is compounded by the city's limited landfill capacity and insufficient waste treatment infrastructure, leading to unsustainable waste management practices that contribute to water pollution.

#### Existing Wastewater Treatment Infrastructure

Lagos has some wastewater treatment infrastructure in place, but these facilities are limited in number and capacity and often struggle to meet the growing demand for proper wastewater management. The city's central wastewater treatment plant, located in Iponri, is one of the few large  scale plants that serve the Lagos Mainland, but it only serves a small portion of the population and is plagued by issues of overcapacity, poor maintenance, and outdated technology (Olumide, 2015). There are also smaller treatment plants in various parts of the city, but these are often underperforming due to a lack of resources, inadequate management, and aging infrastructure. Many areas, especially informal settlements and high  density residential areas, have no access to centralized wastewater treatment, leading to reliance on septic tanks and pit latrines, which can easily overflow and contribute to groundwater contamination (Festus et al., 2020).

In addition to these challenges, the city’s wastewater treatment facilities often lack the capacity to treat wastewater to the required standards. The primary treatment processes employed in these plants are outdated and incapable of adequately removing contaminants such as heavy metals, industrial chemicals, and biological pollutants, which are increasingly common in the wastewater generated by industries and households (Akinbile, 2013). As a result, even treated wastewater may still pose a risk to public health, and untreated wastewater continues to flow into the city’s rivers and lagoons. Furthermore, the lack of effective monitoring systems means that pollution levels are often not detected in real  time, leading to delayed responses to pollution outbreaks (Olumide, 2015).

Several initiatives have been introduced in an attempt to improve wastewater treatment in Lagos. The Lagos State Government has made efforts to develop new treatment plants and expand existing ones, such as the Ologolo Wastewater Treatment Plant, which serves parts of the Lekki area (Oluwasemilore & Dada, 2020). In addition, the Lagos State Water Regulatory Commission (LSWRC) has been established to regulate water quality and enforce standards for wastewater treatment, with a focus on improving the efficiency and sustainability of water treatment processes (Akinbile & Yusoff, 2012). However, despite these initiatives, the overall infrastructure remains insufficient to meet the demands of the growing population and industrial sector. There is a clear need for innovative, decentralized wastewater treatment solutions that can address the unique challenges of urban water management in Lagos, such as the high population density, informal settlements, and industrial activities (Akinbile, 2013).

### 3.0 Discussion

### Technologies for Wastewater Treatment

Wastewater treatment traditionally involves a series of processes that aim to remove contaminants from water to make it safe for discharge or reuse. The most common methods of wastewater treatment include primary, secondary, and tertiary treatments, each serving a distinct function in the treatment process. Primary treatment is the first stage, where large particles, debris, and solids are removed from the wastewater through physical processes such as screening and sedimentation. This stage typically results in the removal of about 60  70% of suspended solids but does little to address dissolved contaminants or pathogens (Olumide, 2015). In Lagos, where waste management infrastructure is often outdated, primary treatment is commonly used, but it is not always fully effective due to the growing volume of wastewater generated by the city’s expanding population and industrial activities (Akinbile, 2013).

#### Figure 2: Traditional Wastewater Treatment Methods

Secondary treatment is the next step, which focuses on biological processes to degrade organic pollutants that were not removed in the primary treatment. This typically involves the use of microorganisms, which consume the organic materials in the wastewater, converting them into simpler substances. In many parts of Lagos, secondary treatment is applied in wastewater treatment plants, where activated sludge systems or aerated lagoons are used. However, these systems have limitations, especially in informal settlements, where they are not widely available or properly maintained (Akinbile & Yusoff, 2012). The secondary treatment process is more effective in removing organic matter, but it does not fully address nutrient removal or microbial pathogens, leaving water that may still pose health risks to the population.

Tertiary treatment is the final step, which typically involves more advanced processes such as filtration, disinfection, and nutrient removal to meet higher standards of water quality. Techniques such as chlorination, ultraviolet (UV) radiation, and sand filtration are employed to remove any remaining pathogens and nutrients, including nitrogen and phosphorus, which can lead to eutrophication in receiving water bodies (Akinyemi et al., 2021). In Lagos, tertiary treatment is often not available or underutilized due to cost constraints and the lack of advanced treatment facilities, resulting in untreated or inadequately treated wastewater being discharged into water bodies (Olumide, 2015). While these traditional methods are still in use in Lagos, they face several challenges due to the city's rapid growth and inadequate infrastructure. The primary treatment facilities often lack the capacity to handle the increasing volume of wastewater, while secondary and tertiary treatments are limited in both scale and effectiveness. As a result, there is a pressing need for more advanced and innovative treatment technologies that can address the city's growing wastewater management challenges.

#### Innovative Technologies

In response to the limitations of traditional treatment methods, several innovative wastewater treatment technologies have emerged, offering more efficient and sustainable solutions for urban areas like Lagos. These technologies include Membrane Bioreactors (MBRs), Moving Bed Biofilm Reactors (MBBRs), Constructed Wetlands, and decentralized treatment systems, each offering unique advantages in terms of space, efficiency, and the ability to treat complex wastewater streams.

Figure 3:  Membrane Bioreactors System

Membrane Bioreactors (MBR) are an advanced technology that combines biological treatment processes with membrane filtration to produce high  quality effluent. In MBR systems, wastewater is treated by microorganisms, which degrade organic matter, while membranes filter out solid particles, bacteria, and viruses (Akinbile & Yusoff, 2012). MBRs are particularly well  suited for urban areas with limited space, as they require smaller land areas compared to traditional treatment systems. The high effluent quality produced by MBRs makes them an ideal option for water reuse, particularly in regions with water scarcity. However, the high energy requirements and maintenance costs of MBR systems can be a challenge in cities like Lagos, where economic constraints often limit the adoption of such technologies (Olumide, 2015).

Moving Bed Biofilm Reactors (MBBRs) represent another promising innovation in wastewater treatment. MBBRs use a biofilm attached to plastic carriers that move freely in the treatment tank, allowing for a more efficient biological degradation of organic pollutants (Akinbile, 2013). The flexibility of MBBR systems, including their ability to handle varying organic loads and their compact design, makes them suitable for decentralized wastewater treatment in urban areas like Lagos. MBBRs can be used for both domestic and industrial wastewater treatment, and they are less prone to clogging compared to conventional activated sludge systems. However, like MBRs, they also face challenges related to high capital costs, which may limit their widespread use in lower  income areas (Akinyemi et al., 2021). Constructed Wetlands are another innovative solution for wastewater treatment, particularly for rural and peri  urban areas with limited infrastructure. These systems mimic natural processes by using plants and soil to filter and degrade contaminants in wastewater. Constructed wetlands are cost  effective, environmentally friendly, and require low maintenance. They are particularly useful in treating small  scale wastewater from households, schools, and small industries, making them a good option for decentralized wastewater management in Lagos (Oluwasemilore & Dada, 2020). The use of constructed wetlands is increasing in urban areas globally as a sustainable method for treating wastewater, and their application in Lagos could significantly improve water quality in both residential and industrial areas. However, the slow treatment process and the need for adequate space for plant growth may limit their applicability in high  density urban areas.

Decentralized Treatment Systems are particularly beneficial in urban areas with limited infrastructure, such as many parts of Lagos. These systems treat wastewater close to its source, reducing the need for extensive transportation of wastewater to central treatment plants. Technologies like the small  scale activated sludge systems, membrane filtration units, and biofilters can be deployed in residential neighborhoods or industrial clusters, providing a more localized solution to wastewater treatment. Decentralized systems not only reduce the strain on central infrastructure but also facilitate water reuse and reduce the discharge of untreated or poorly treated wastewater into the environment (Akinbile, 2013). However, implementing decentralized systems across Lagos presents challenges, including the need for effective local management, monitoring, and maintenance, which can be difficult without adequate funding and technical expertise.

Biological and Chemical Treatment Technologies such as phytoremediation and activated sludge systems are also being explored as solutions for wastewater treatment in Lagos. Phytoremediation, which uses plants to absorb, degrade, or immobilize pollutants, has shown promise in removing nutrients and heavy metals from wastewater (Akinbile & Yusoff, 2012). This approach is particularly beneficial for small  scale treatment systems and could be integrated into constructed wetlands or other treatment systems in Lagos to reduce nutrient pollution. Activated sludge systems, which rely on microorganisms to degrade organic matter in wastewater, remain one of the most common biological treatment methods worldwide (Akinyemi et al., 2021). While effective, activated sludge systems require regular maintenance and monitoring to ensure optimal performance, and their efficiency can be compromised in areas with high pollutant loads.

#### Challenges of Implementation in Lagos

Despite the promising potential of these innovative technologies, several barriers hinder their widespread implementation in Lagos. One of the most significant challenges is the high initial capital cost of advanced wastewater treatment technologies such as MBRs and MBBRs. These systems often require substantial upfront investment in infrastructure, which can be a major deterrent for local authorities and private investors, especially in a city with limited financial resources (Olumide, 2015). Additionally, the lack of technical expertise in the design, operation, and maintenance of these advanced systems poses another major obstacle. Skilled personnel are needed to operate and maintain complex wastewater treatment technologies, but there is a shortage of trained professionals in Lagos, which makes it difficult to implement and sustain these systems over the long term (Akinbile & Yusoff, 2012).

Another challenge is the insufficient regulatory framework to support the adoption of these technologies. While there are policies in place to regulate wastewater treatment and pollution control, enforcement remains weak, and the regulatory environment does not sufficiently incentivize the adoption of innovative technologies (Akinyemi et al., 2021). Additionally, there is a lack of public awareness about the benefits of wastewater treatment and water conservation, which limits public support for the implementation of decentralized or innovative treatment systems.

### Pollution Monitoring Systems

#### Traditional Pollution Monitoring Techniques

Conventional pollution monitoring techniques have been employed for decades to assess water quality in urban and industrial areas, including Lagos. These methods generally involve manual sampling, laboratory analysis, and visual inspections, which provide valuable insights into the condition of water bodies. Manual sampling is the most common technique, where samples of water are collected at specific locations and times to analyze various parameters, such as pH, dissolved oxygen, turbidity, and concentrations of pollutants like heavy metals and organic compounds (Olumide & Awodele, 2017). This approach is useful for generating data on water quality at specific points, but it often suffers from limitations, such as its time  consuming nature and the inability to provide real  time or continuous monitoring.

Figure 4 : Pollutant monitoring

The laboratory analysis of these water samples typically involves complex chemical processes to detect pollutants, often requiring sophisticated equipment and trained personnel. Although this method is effective for determining the presence of specific contaminants, it is expensive, resource  intensive, and time  consuming, which can delay decision  making processes in pollution management (Akinbile & Yusoff, 2012). Furthermore, traditional methods often lack the capacity to monitor pollution levels on a large scale or in real  time, which is crucial in urban environments like Lagos, where water quality fluctuates rapidly due to the dynamic nature of urbanization, industrialization, and waste disposal (Olumide, 2015).

Another traditional technique for water quality monitoring is visual inspection, which involves field personnel inspecting water bodies for visible signs of pollution, such as the presence of floating debris, oil slicks, or discoloration. While useful for initial assessments or detecting gross pollution, visual inspection lacks the precision and reliability needed to accurately assess water quality and track pollution over time (Olumide & Awodele, 2017). Thus, although conventional techniques have provided useful data in the past, they are increasingly inadequate for addressing the complex pollution monitoring needs of rapidly growing urban centers like Lagos.

#### Modern Pollution Monitoring Systems

To address the shortcomings of traditional pollution monitoring techniques, modern technologies have emerged that enable more efficient, accurate, and real  time monitoring of water quality. These advanced systems make use of the Internet of Things (IoT) sensors, remote sensing technologies, and machine learning algorithms to provide continuous, large  scale, and data  driven insights into pollution levels and water quality trends. In Lagos, these innovations are becoming essential for managing the city’s water resources and tracking pollution levels in real time.

IoT  based Sensors and Smart Monitoring: The advent of IoT  based sensors has revolutionized the monitoring of water quality by enabling real  time data collection and transmission. IoT sensors, which can be deployed in rivers, lakes, wastewater treatment plants, and even industrial effluents, collect data on various water quality parameters such as temperature, pH, turbidity, dissolved oxygen, and concentrations of pollutants like heavy metals and nitrates (Akpabio et al., 2024). These sensors are often connected to a central data repository or cloud platform, where the collected data can be analyzed in real  time, enabling immediate response to pollution events or deteriorating water quality conditions. IoT  based systems are particularly beneficial for monitoring water bodies in large and densely populated urban areas like Lagos, as they reduce the need for manual sampling and offer continuous data streams without the delays associated with traditional methods (Akinbile & Yusoff, 2012).

Smart monitoring systems can also integrate with automated alarms and decision support systems, which alert authorities or facility managers when pollution levels exceed safe thresholds. This allows for more proactive pollution control and rapid response, such as adjusting the flow rates of wastewater treatment plants or shutting down industrial discharges in the event of a pollution spike (Akpabio et al., 2024). Additionally, IoT  based monitoring can help detect patterns of pollution over time, facilitating long  term water quality management and enhancing the understanding of the relationship between urban activities, industrial effluents, and pollution.

Remote Sensing and Satellite Imaging: Another advanced technology for monitoring water quality in Lagos is remote sensing through satellite imaging and aerial drones. Remote sensing involves the use of satellite  based sensors to collect data on large  scale environmental changes, including the monitoring of water bodies and pollution levels. Satellites equipped with multispectral and hyperspectral sensors can capture images of water bodies, detecting water quality parameters such as chlorophyll concentration, turbidity, temperature, and the presence of contaminants (Akpan et al., 2024). In Lagos, remote sensing can be used to track the spread of pollutants in water bodies such as the Lagos Lagoon and the Atlantic Ocean, providing valuable insights into the impact of urban and industrial activities on the environment.

One of the key advantages of satellite  based remote sensing is its ability to cover vast geographic areas and provide a bird’s  eye view of water bodies that would otherwise be difficult or costly to monitor with ground  based methods (Akinyemi et al., 2021). This technology is particularly useful in monitoring coastal and inland water bodies in Lagos, as it enables authorities to detect pollution incidents in real  time or track long  term pollution trends. Furthermore, satellite data can be integrated with ground  based monitoring systems to create a more comprehensive picture of water quality and pollution sources.

Machine Learning and Data Analytics: Machine learning (ML) algorithms and data analytics are increasingly being applied to analyze large datasets generated by pollution monitoring systems. These advanced algorithms can process data from various sources, including IoT sensors, remote sensing platforms, and historical water quality datasets, to predict pollution trends and optimize wastewater treatment operations (Olumide, 2015). Machine learning models can identify patterns in water quality data that would be difficult for humans to detect, such as seasonal changes in pollutant concentrations or correlations between urban activities and water pollution.

For example, machine learning can be used to predict the likelihood of pollution events in specific areas, allowing authorities to take preventive actions before pollution levels reach dangerous thresholds (Akinbile, 2013). ML algorithms can also optimize the operation of wastewater treatment plants by adjusting treatment processes based on real  time data, ensuring that the effluent meets water quality standards. In Lagos, where water quality is impacted by a variety of sources, such as domestic wastewater, industrial effluents, and waste disposal practices, machine learning can play a crucial role in identifying the most effective strategies for pollution control and wastewater management (Akpabio et al., 2024).

Moreover, data analytics can be applied to evaluate the effectiveness of pollution control measures and track improvements in water quality over time. By integrating data from IoT sensors, remote sensing, and other sources, decision  makers can gain a more holistic understanding of the state of water resources in Lagos and make informed decisions regarding water treatment and pollution mitigation strategies (Olumide & Awodele, 2017).

The development and implementation of modern pollution monitoring systems in Lagos are essential for addressing the city's water quality challenges and improving overall water management. Traditional techniques, such as manual sampling and laboratory analysis, have been vital for tracking pollution, but they are increasingly inadequate in meeting the demands of a rapidly growing urban center. Advanced technologies, such as IoT  based sensors, remote sensing, and machine learning, offer powerful tools for real  time monitoring, large  scale pollution tracking, and predictive analytics. These systems enable better decision  making, more efficient pollution management, and more sustainable water resource practices in Lagos. However, to maximize the potential of these technologies, there is a need for robust infrastructure, regulatory frameworks, and capacity  building efforts to ensure their widespread adoption and effective use.

4.0 Integrated Approaches for Wastewater Treatment and Pollution Monitoring

### Integrated Systems

An integrated approach to wastewater treatment and pollution monitoring combines real  time data acquisition, advanced analytics, and responsive treatment processes into a single, unified system. This integration fosters more dynamic and efficient management of water resources. Traditional methods often treat wastewater and monitor pollution as separate processes, leading to inefficiencies, delayed responses, and missed opportunities for optimization. Integrated systems, on the other hand, use feedback loops, often powered by IoT devices and machine learning, to ensure that treatment processes are continuously optimized based on up  to  date water quality data.

For a city like Lagos, where rapid urbanization strains water infrastructure and pollution levels fluctuate widely, integrated systems can offer tremendous advantages. Real  time pollution monitoring through smart sensors allows for the immediate detection of contaminants, which can then inform and adjust the wastewater treatment process. For example, if sensors detect elevated nitrogen levels in effluent, the treatment process can automatically shift to include enhanced biological nitrogen removal.

The result is a more resilient and responsive water management system that not only ensures compliance with water quality standards but also reduces the likelihood of environmental disasters and improves public health outcomes. Furthermore, these systems reduce operational costs by minimizing chemical usage, energy consumption, and maintenance through predictive and preventative control mechanisms.

### Case Studies

Several global cities have successfully adopted integrated water management systems, offering valuable lessons for Lagos:

- Singapore’s NEWater Initiative integrates wastewater treatment with continuous water quality monitoring using smart sensors and membrane filtration technologies. The treated water is reused for industrial processes and even indirect potable use. This model demonstrates how high  tech integration and public trust can drive sustainable water management.

- In Stockholm, Sweden, a combination of IoT  based pollution monitoring and decentralized treatment units in urban neighborhoods ensures efficient treatment and immediate response to environmental changes. This has resulted in high water reuse rates and reduced environmental degradation.

- India’s National Mission for Clean Ganga incorporates integrated pollution monitoring, real  time effluent tracking, and decentralized sewage treatment plants (STPs) along the Ganges River. Despite implementation challenges, it shows the potential of policy  driven integrated approaches in large and complex urban environments.

For Lagos, adapting these models would require investments in sensor networks, data management systems, and skilled personnel. However, with appropriate public  private partnerships and community engagement, an integrated approach can dramatically enhance water sustainability.

### Regulatory and Policy Framework

A robust regulatory framework is critical for the success of integrated wastewater treatment and pollution monitoring systems. In Lagos, the current framework is fragmented, with overlapping responsibilities between different agencies (e.g., LASEPA, Ministry of Environment, Water Corporation). A unified policy that mandates real  time water quality monitoring, performance benchmarking for treatment plants, and data sharing among stakeholders is essential. In addition, clear regulations should support the use of treated wastewater for non  potable purposes, encouraging industries and agriculture to adopt recycled water. Tax incentives and grants can also drive innovation and private sector participation in wastewater infrastructure.

Community participation is another crucial pillar. Public awareness campaigns, citizen science initiatives (like community  based water testing), and participatory budgeting for water projects can increase transparency and trust, thereby improving compliance and long  term sustainability.

Integrated Approaches for Wastewater Treatment and Pollution Monitoring

Integrating wastewater treatment systems with real  time pollution monitoring offers a strategic solution to the complex water management challenges faced by rapidly urbanizing cities like Lagos. Traditional wastewater treatment and pollution control systems operate in silos, which limits responsiveness and efficiency. Integrated systems, however, enable the simultaneous collection and analysis of water quality data while also adapting treatment processes in real  time based on detected pollution levels. These systems often rely on smart technologies, such as Internet of Things (IoT) devices, to monitor effluents and manage treatment dynamically, allowing for more informed decision  making (Vairavamoorthy & Gorantiwar, 2006).

In Lagos, this approach holds immense potential due to the city’s sprawling informal settlements, industrial zones, and overstressed drainage systems. By embedding pollution sensors in key water bodies and connecting them to centralized data management systems, the Lagos State Water Corporation could track contaminant levels and adjust treatment procedures automatically. Such integration not only enhances the efficiency of water purification but also minimizes risks to public health, especially in areas vulnerable to waterborne diseases (Oloruntade et al., 2020).

Successful case studies from other regions offer models for replication. Singapore’s NEWater project, for instance, integrates advanced wastewater treatment with real  time monitoring to produce high  quality reclaimed water for industrial and indirect potable use. This system incorporates microfiltration, reverse osmosis, and ultraviolet disinfection, all managed by a robust digital control network (Tortajada, 2006). Similarly, Stockholm’s sustainable urban drainage systems use integrated management platforms that combine data from pollution sensors and rainfall gauges to optimize stormwater and wastewater treatment in real  time (Wong & Brown, 2009). India’s National Mission for Clean Ganga also exemplifies how data  driven, integrated pollution monitoring and decentralized sewage treatment can be scaled for complex environments, although it highlights challenges such as inadequate local technical capacity and fragmented regulatory frameworks (Narain & Pandey, 2012).

For Lagos to adopt such integrated systems, strong regulatory backing and institutional coordination are crucial. The current water governance structure in Lagos suffers from overlapping mandates and inconsistent enforcement. Harmonizing water policies and encouraging data sharing between agencies like LASEPA (Lagos State Environmental Protection Agency) and the Ministry of Environment is a necessary first step. Moreover, setting water quality standards for recycled wastewater and introducing incentives for industries to treat and reuse water will accelerate adoption. Community involvement is also essential, as citizen science initiatives and localized monitoring can complement official efforts and build public trust (Nwachukwu et al., 2010).

## Sustainability in Water Management

A sustainable water management strategy recognizes wastewater not as a waste product but as a valuable resource. The circular economy concept, which emphasizes resource reuse, is increasingly being applied to wastewater treatment. In this model, treated wastewater is reused for non  potable applications such as irrigation, industrial cooling, and construction, thereby reducing the demand on freshwater sources. Lagos, facing acute water scarcity and pollution challenges, stands to benefit significantly from circular wastewater reuse systems (Zhang et al., 2019).

Technologies like membrane bioreactors (MBRs), constructed wetlands, and ultraviolet (UV) disinfection are effective in producing high  quality effluent suitable for reuse. For instance, MBRs combine biological treatment with membrane filtration, offering compact and efficient systems ideal for space  constrained urban areas like Lagos. Constructed wetlands, on the other hand, offer nature  based, low  energy alternatives that integrate well into peri  urban landscapes and can be maintained with minimal technical expertise (Kadlec & Wallace, 2008).

Environmental and economic benefits of sustainable wastewater treatment are profound. Environmentally, such systems reduce nutrient and pathogen loading in surface and groundwater, which helps preserve ecosystems and biodiversity. Economically, although capital costs may be high, operational costs are often lower over time due to energy savings and the recovery of valuable by  products such as biogas and compost from biosolids (Hernandez  Sancho et al., 2015). For Lagos, integrating sustainability into water treatment infrastructure would also reduce vulnerability to climate  induced flooding and water shortages, making the city more resilient in the face of global environmental changes (UNEP, 2016).

Community and stakeholder engagement is pivotal to the success of sustainable water initiatives. Engaging citizens through educational campaigns, participatory planning, and transparent governance fosters a sense of ownership and encourages behavior change. In many parts of Lagos, skepticism towards wastewater reuse stems from cultural norms and misinformation. Overcoming these barriers requires awareness  building and demonstration projects that show the safety and utility of treated wastewater. Public  private partnerships can also play a vital role in financing and operating wastewater systems, while universities and research institutions can support innovation and training (Ezeh et al., 2016).

By combining technological innovation, regulatory reform, and stakeholder engagement, Lagos can build a sustainable water future. This will require coordinated efforts across sectors and levels of government, as well as continuous investment in infrastructure and capacity building. The transition toward integrated and sustainable water management is not only feasible but necessary for Lagos to meet its environmental, economic, and public health goals.

## Challenges and Opportunities

Despite the clear benefits of integrated wastewater treatment and pollution monitoring systems, Lagos faces significant challenges in implementation. One of the foremost issues is the lack of adequate funding for infrastructure development. Building and maintaining smart wastewater systems requires substantial capital investment, which is often difficult to secure due to competing urban priorities and limited budget allocations (World Bank, 2019). The infrastructure deficit in Lagos further complicates efforts, as existing sewer systems are either outdated or non  existent in many neighborhoods, especially informal settlements that house a large portion of the population (Ajibade & McBean, 2014).

Political and institutional challenges also undermine the effective deployment of water management solutions. Lagos has a fragmented regulatory environment, with overlapping mandates among various agencies such as the Lagos State Water Corporation, Lagos State Environmental Protection Agency (LASEPA), and local councils. This fragmentation often leads to bureaucratic delays and inconsistent policy implementation (Olanrewaju & Akinyele, 2020). Furthermore, public awareness and community engagement remain low. Misinformation about the safety of treated wastewater, combined with a general lack of education on environmental protection, poses social barriers to adoption of modern systems (Ezeh et al., 2016).

Nonetheless, there are numerous opportunities for innovation and progress in the sector. Public  private partnerships (PPPs) offer a promising path for financing and implementing wastewater treatment projects, leveraging private sector efficiency and technology with public oversight. International cooperation is also a growing avenue, with organizations like the African Development Bank (AfDB), UN  Habitat, and the World Bank funding sustainable urban water projects across Africa (AfDB, 2020). These partnerships can bring in technical expertise, capacity building, and policy reform support.

Lagos also has the opportunity to develop local expertise and technologies through its growing network of universities, research institutions, and tech hubs. Initiatives such as innovation challenges, startup accelerators, and collaborative research programs can support the creation of context  specific solutions technologies tailored to the unique challenges of Lagos. For example, locally designed low  cost sensors for pollution monitoring or modular treatment plants for slum communities could prove both cost  effective and scalable. Furthermore, increased use of open data platforms can facilitate collaboration between stakeholders, enhancing transparency and data  driven planning in water governance.

## 5.0 Conclusion

This literature review has examined the current state and future potential of wastewater treatment and pollution monitoring in Lagos, highlighting both the challenges and opportunities inherent in achieving sustainable water management. Lagos faces numerous threats to its water resources due to rapid urbanization, pollution, inadequate infrastructure, and weak governance. Traditional water and wastewater systems are insufficient to meet the city’s needs, while pollution from untreated sewage, industrial discharge, and improper waste disposal further exacerbates environmental and health risks.

However, integrated approaches that combine real  time pollution monitoring with innovative wastewater treatment technologies present a transformative opportunity. Systems incorporating IoT  based sensors, remote sensing, and decentralized treatment units can vastly improve water quality management when paired with strong regulatory frameworks and public engagement. Success stories from cities like Singapore, Stockholm, and Pune offer replicable models for Lagos, though adaptation to local realities is essential.

The feasibility of implementing such integrated systems in Lagos depends on addressing key barriers especially funding, governance, and awareness. Nevertheless, opportunities exist in the form of international partnerships, emerging local innovations, and the gradual build  up of technical expertise. The transition to sustainable and inclusive water management in Lagos will require coordinated efforts from government, academia, industry, and civil society.

Future research should explore the socio  economic dimensions of wastewater reuse in Lagos, including public perceptions, cost  benefit analyses, and behavioral change strategies. More studies are also needed on decentralized, off  grid solutions that can serve underserved communities. Overall, a strategic, inclusive, and data  driven approach can position Lagos as a model for integrated and sustainable urban water systems in sub  Saharan Africa.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

### References

- Festus, I. A., Omoboye, I. F., & Andrew, O. B. (2020). Urban sprawl: environmental consequence of rapid urban expansion. Malaysian Journal of Social Sciences and Humanities (MJSSH), 5(6), 110-118.

- Akinbile, C. O. (2013). Assessment of the CERES-Rice model for rice production in Ibadan, Nigeria. Agricultural Engineering International: CIGR Journal, 15(1), 19-26.

- Akinbile, C. O., Yusoff, M. S., & Zuki, A. A. (2012). Landfill leachate treatment using sub-surface flow constructed wetland by Cyperus haspan. Waste management, 32(7), 1387-1393.

- Akinyemi, P. A., Adeomi, A. A., Ogundiran, A., Okoro, N., & Madaki, K. (2021). Chemical hazard and healthcare workers; findings from a tertiary health institution in southwestern Nigeria. International Journal of Community Medicine and Public Health, 8(7), 3200.

- Akpabio, E. M., Wilson, N. A. U., Umoh, E. C., Udofia, E. I. S., Udo, I. I., Elijah, E., ... & Ema, E. O. (2024). Women, water and access: inscribing gender power in and through a place. Journal of Water and Health, 22(3), 627-638.

- Akpan, G. E., Emetere, M. E., Afolalu, S. A., & Jen, T. C. (2024). Prospective geothermal well and heat catchment locations in Nigeria: Remote sensing interpretations and field validation. Energy Exploration & Exploitation, 42(2), 567-588.

- Eze, C. T., Oje, O. A., & Onwurah, I. N. E. (2016). Environmental risk assessment of hospital wastewater in Enugu, Nigeria. Pollut Res, 35(3), 55-61.

- Hernandez  Sancho, F., Molinos  Senante, M., & Sala  Garrido, R. (2015). Energy efficiency in Spanish wastewater treatment plants: A non  radial DEA approach. Science of the Total Environment, 409(14), 2693  2699.

- Kadlec, R. H., & Wallace, S. D. (2008). Treatment wetlands. CRC press.

- Tortajada, C. (2006). Water management in Singapore. International Journal of Water Resources Development, 22(2), 227–240.

- UNEP (United Nations Environment Programme). (2016). A Snapshot of the World’s Water Quality: Towards a Global Assessment. United Nations.

- Wong, T. H. F., & Brown, R. R. (2009). The water sensitive city: Principles for practice. Water Science and Technology, 60(3), 673–682.

- Martínez, R., Vela, N., El Aatik, A., Murray, E., Roche, P., & Navarro, J. M. (2020). On the use of an IoT integrated system for water quality monitoring and management in wastewater treatment plants. Water, 12(4), 1096.

- Lako, A., & Çomo, E. (2024). Sustainable water management: an integrated approach to solving the problems of wastewater treatment. Qubahan Academic Journal, 4(1), 91-100.

- Karn, A. L., Pandya, S., Mehbodniya, A., Arslan, F., Sharma, D. K., Phasinam, K., ... & Sengan, S. (2023). An integrated approach for sustainable development of wastewater treatment and management system using IoT in smart cities. Soft Computing, 1-17.

- Saravanan, A., Kumar, P. S., Jeevanantham, S., Karishma, S., Tajsabreen, B., Yaashikaa, P. R., & Reshma, B. (2021). Effective water/wastewater treatment methodologies for toxic pollutants removal: Processes and applications towards sustainable development. Chemosphere, 280, 130595.

- Siddique, I. (2021). Sustainable Water Management in Urban Areas: Integrating Innovative Technologies and Practices to Address Water Scarcity and Pollution. The Pharmaceutical and Chemical Journal, 8(1), 172-178.

- Silva, J. A. (2023). Wastewater treatment and reuse for sustainable water resources management: a systematic literature review. Sustainability, 15(14), 10940.

- Chen, C. H., Liu, W. L., & Leu, H. G. (2006). Sustainable water quality management framework and a strategy planning system for a river basin. Environmental Management, 38, 952-973.

- Ejairu, U., Aderamo, A. T., Olisakwe, H. C., Esiri, A. E., Adanma, U. M., & Solomon, N. O. (2024). Eco-friendly wastewater treatment technologies (concept): Conceptualizing advanced, sustainable wastewater treatment designs for industrial and municipal applications. Comprehensive research and reviews in Engineering and Technology, 2(1), 083-104.