Concentration and Health Risk Assessment of Heavy Metal in Roadside Soil in Eket, Akwa Ibom State, Nigeria

ABSTRACT

Aims: This study determines the heavy metal impacts on roadside soils and its potential health hazards in Eket, Akwa Ibom State, Nigeria. Background: Roads affect natural environment to a large extent because automobile act as line sources of heavy metal pollutants. Emission from heavy traffic have been reported to contain Pb, Cd, Zn and Ni which are present in fuel as anti-knock agent. Heavy metals in the soil can also generate airborne particles and dust which may affect the quality of air. Inhalation of substantial quantities of heavy metal particle over period of time may add to human body burden of the metals and constitute health risk. Methodology: Roadside soil samples were collected from Mobile Matt (MM), Marina Junction (MJ), Uqua Junction (UJ) and Fongetok Junction (FJ) in Eket, a city in Akwa Ibom State of Nigeria and analysed using Atomic Absorption Spectrophotometer. Risk assessment quantification was calculated including potential ecological risk index Eri, health risk assessment, non-carcinogenic risk assessment and carcinogenic risk assessment. The concentrations of heavy metal were applied to assess the adult and children’s health risks both carcinogenic and non-carcinogenic. Hazard quotient (HQ) calculated to determine non-carcinogenic health risk for each individual heavy metal element. Results: The findings revealed that Eri is divided into three main categories, low, high and very high risk at the four sampling junctions in the study area. High and very high risk is due to high Potential Ecological Risk Index Eri of Cd 252.6 at MJ, 408 at UJ, 259.8 at FJ and 301.2 at MM. Risk index RI is also divided into considerable and high risk. High RI is due to high values of Eri caused by Cd and Pb at different sampling junctions. Heavy metals pose no harmful effect on both adult and children's health as HI < 1. The LCR for ingestion, inhalation and dermal contact for adults and children resulted from Pb and Cd. For Cd, LCR is within the threshold value of 1 × 10-6 – 1 × 10-4 which is widely considered acceptable except at MJ adult is 1 × 10-7. For Pb, LCR is in the range of 1 × 10-4 which is considered to have significant health effects on adults and children. Conclusion: It is therefore expedient that measures to combat further contamination of soil by Cd and Pb be taken to keep the environment clean and safe for humans.

Keywords: [Ecological Potential Risk; Heavy Metals; Risk Assessment; Roadside Soil; Vehicular Emissions)

1. INTRODUCTION

“Heavy metals are metallic chemical elements that have relatively high density and are toxic even at low concentrations. Through anthropogenic influence, geochemical and biochemical life span of most heavy metals are altered and pose negative effect to humans and environment as a result of their persistence, toxicity, biological accumulation and bio-magnification nature” (Nduka et al., 2023; Awoyemi et al., 2025). “Roads serve as the main linkage among communities through which foods and other important commodities are transported. It is one of the important amenities that plays a key role in improving social and economic activities. Nevertheless, road transport may result in heavy metal pollution, particularly on soils. Research reports have shown that roadside soils may be contaminated from industrial and energy production, vehicle exhaust, waste disposal as well as coal and fuel combustion” [1]. “Roads affect the natural environment to a large extent because automobiles act as line sources of heavy metal pollutants” [2]. “Roadside soil pollution in these studies may attributed to traffic. Thus, emissions from heavy traffic are a global phenomenon. The terrestrial environments act as sink for accumulation of these contaminants. Atmospheric particulates loaded with heavy metals are deposited on the roadside soil, water and dust through emissions from vehicular traffic, construction of road work, worn out fragments from buildings, rusting of galvanized metal structures and particle suspensions to the air from tyre and other automobile components” (Nduka et al., 2023; Ayaz et al., 2023).

“Heavy metals frequently reported in literature with regard to potential hazards and occurrences in contaminated soils are Cd, Cr, Pb, Zn, Fe and Cu” [3, 4]. “Emissions from heavy traffic have been reported to contain Pb, Cd, Zn and Ni which are present in the fuel as anti-knock agent”[5, 6]. “Vehicle exhausts, as well as industrial activities, emit heavy metals such that soils, plants and even residents along roads with heavy traffic loads are subjected to increasing levels of contamination with heavy metals” [7]. “Soils are critical in assessing the potential environmental impacts of automobile emissions and several researchers have indicated the need for a better understanding of heavy metal pollution of roadside soils [8, 9]. In Nigeria, over the years, the number of vehicles plying roads kept on increasing as more heavy trucks and trailers were being used for the transportation of goods across different zones because rail system was abandoned. In developing countries like Nigeria, improved road accessibility creates a variety of ancillary employment which ranges from vehicle repairs, vulcanizer and welders to auto-electricians, battery chargers and dealers in other facilitators of motor transportation. These activities send the heavy metals into the air and are subsequently deposited into nearby soils which are absorbed by plants” [10, 11]. “Heavy metals in the soil can also generate airborne particles and dust which may affect the quality of air. Inhalation of substantial quantities of heavy metal particles over period of time may add to the human body burden of the metals and constitute a health risk. Among the numerous environmental pollutants, heavy metals play an important role as its concentrations in soil, water and air are continuously increasing due to anthropogenic activities” [12]. “With rapid urbanization and the overexploitation of mineral resources, many heavy metals enter the soil. Heavy metals in soil do not easily degrade, and they accumulate in the food chain, potentially causing serious harm to human health” (Panqing et al., 2023; Mohamed AND Yarmoshenko, 2020).

“Although there are a number of studies on heavy metals in roadside dust. However, research reports on health risk assessment quantification on roadside dust is scarce. Health risk assessment of heavy metals will make known the pollution level of soil and planning the management strategy accordingly” [13]. “The increasing accumulation of heavy metals in both  soils  and plants  has  raised  concerns  about  potential  risks to    human    health,    particularly    through    the consumption    of    contaminated    food.    Food poisoning stands out as a predominant  route  for these  hazardous  chemicals  to  enter  the  human body” (Tripathi et al., 2024). The aims of this study were to assess the likely potential risk index of individual heavy metal (Eri) and the potential ecological risk index (RI) imposed by heavy metals and the health impact that metals in the roadside soil represent in one of the emerging cities of the Niger Delta Region of Nigeria.

2. material and methods

2.1 Location and Climate of the Study Area

The study area was Eket Local Government Area of Akwa Ibom State, Nigeria. Eket is situated between Latitudes 4 33’ and 4 45’ and Eastwards between Longitudes 7 52’ and 5 02’. Eket has tropical climate marked by two distinct seasons; dry (November - March) and wet (April – October) season. The wet season is usually interrupted by a short dry period in August. The temperature of Eket ranges from 23-31℃ and precipitation averages 3044mm.

2.2 Sample Collection and Analysis

Surface soils were collected in polythene bags from four road junctions in Eket city including, Mobile Matt (MM), Marina Junction (MJ), Uqua Junction (UJ) and Fongetok Junction (FJ). These locations were selected based on the magnitude of traffic and human activities within these locations. The collected soil samples were taken to the laboratory for heavy metals analyses. Atomic Absorption Spectrophotometer was used to analyse heavy metals including Cd, Cr, Cu, Fe, Mn, Ni, Pb, and Zn.

2.3 Heavy Metal Risk Assessment

2.3.1. Potential Ecological Risk Index

The toxicity and ecological hazards posed by heavy metals as proposed by [14] and first reported by [15] used to determine the potential risk of individual heavy metals on road side soils, Equation (1) as follows:

Eri = Cri × Tr i = (Cis/Cin) × Tir			(1)

Where; Tir is the toxic-response factor for a single heavy metal contamination and was taken as, 1 for Zn, 5 for Cu and Pb, and 30 for Cd as suggested by [16, 17]. Cir is an index for contamination of a given heavy metal, Cis is the present concentration of heavy metal in road site soil and Cin is the reference value of heavy metal in the soil. The reference values of the average shale in the urban environment used in this work are from [18]. These values are: 35.1 mg kg−1 for Cu, 0.5 mg kg−1 for Cd, 17 mg kg−1 for Pb, 59.9 mg kg−1 for Zn and 13.2 mg kg−1 for Ni. The sum of potential individual risks (Eir) is the potential ecological risk index (RI) and was calculated using Equation (2), which is defined as:

RI = ∑ Eri = ∑ Tr i (Cis/Cin) 			(2)

The potential ecological hazard and the risk criteria, as resulted in heavy metal accumulation in the soil, are classified into risk categories and given in Table 1.

Table 1. Classification of Potential Ecological Risk Index (Eir) and Risk index (RI)

| EiR | Risk classification | RI | Risk classification |
| --- | --- | --- | --- |
| EiR < 40 | Low risk | RI < 50 | Low risk |
| 40 ≤ EiR < 80 | Moderate risk | 50 ≤ RI < 200 | Moderate risk |
| 80 ≤ EiR < 160 | Considerable risk | 200 ≤ RI < 300 | Considerable risk |
| 160 ≤ EiR < 320 | High risk | RI ≥ 300 | High risk |
| EiR ≥ 320 | Very high risk |  |  |

2.3.2 Health Risk Assessment

Human beings are exposed to heavy metals in soil/dust in three key pathways [19, 20, 21]: (1) ingestion (Ding); (2) inhalation (Dinh); and (3) dermal contact [22, 23]. The health risks via the three pathways were detected using Equations (3–5):

Ding=C×Ring×ED×EFAT×BW×10-6			(3)

Dinh=C×Rinh×ED×EFAT×BW×PEF			(4)

Ddermal=C×AF×SA×ED×EF×ABSAT×BW×10-6		(5)

where C is the concentration of heavy metal in roadside soil (mg/kg), RIng is the ingestion rate (mg/day), ED is the exposure duration (years), EF is the exposure frequency (days/year), AT is the averaging time, BW is the average body weight (kg), (days), RInh is the inhalation rate (mg/cm2), PEF is the particle emission factor (m3/kg), AF is the skin adherence factor for soil (mg/cm2-day), SA is the surface area of the exposed skin that is in contact with the sample (cm2), and ABS is the dermal absorption factor (unit-less). Exposure factors used in the non-carcinogenic Risk Ddermal estimate are given in Table 2.

Table 2. Exposure Variables used in Non-Carcinogenic Exposure Ddermal Assessment [24]

| IngR | 100 mg/day (adult), 200 mg/day (children) |
| --- | --- |
| EF | 180 days |
| ED | 24 years (adult), 6 years (children) |
| BW | 70 kg (adult), 15 kg (children) |
| AT | 365 × ED adult/children |
| InhR | 20 mg/cm2 |
| PEF | 1.36 × 109 m3/kg |
| SA | 2145 cm2 event−1 (adult), 1150 cm2 event−1 (child) |
| AFsoil | 0.07 mg cm−2 day−12 (adult), 0.2 mg cm−2 day−1 (child) |
| ABS | 0.001 |

2.3.3 Non-Carcinogenic Risk Assessment

The concentrations of heavy metal were applied to assess the adult and children’s health risks both carcinogenic and non-carcinogenic. Hazard quotient (HQ) calculated to determine non-carcinogenic health risk for each individual heavy metal element are as described in Equation (6) [16].

HQ=DRFD				(6)

Where RFD reflects the chronic reference dose for each heavy metal (mg/kg-day) as given in Table 3 [20].

Table 3. Reference Dose RFD (mg/kg-day) for each Heavy Metal

| Heavy metal | RfD (mg/kg/day) |
| --- | --- |
| Cu | 0.0371 |
| Pb | 0.0035 |
| Zn | 0.3 |
| Ni | 0.91 |
| Cd | 0.001 |

A risk index (HI) was calculated to measure the risk of carcinogenic health effects posed by heavy metal. HI is the sum of three major pathways’ hazard quotient as shown in Equation (7) [21]:

HI=HQ=HQing+HQinh+HQdermal	 (7)

The values of HI are classified into two categories. When HI < 1 is no harmful effect to the health, while HI > 1 there is a potential for adverse effects on health.

2.3.4 Carcinogenic Risk Assessment

The lifetime cancer risk (LCR) was estimated to determine the health risk for carcinogenic heavy metal by calculating the cumulative life cancer risk rating using Equation (8) for each exposure pathway:

LCR=D×SF=CR=CRing+CRinh+CRdermal		(8)

Where, CR is the cancer risk and SF is the slope factor for carcinogenicity (by mg / kg day) was presented by [22]. For the related heavy metals Cd and Pb the SF values are 6.3, and 0.0085 mg/kg/day respectively [22]. In overall, the acceptable threshold value of LCR 1 × 10−4 is considered to have significant health effects while LCR of 1 × 10−6 to 1 × 10−4 is widely considered acceptable, and LCR below 1 × 10−6 is regarded as negligible [23].

3. results and discussion

3.1.1. Potential ecological risk index

Table 4 presents the calculated potential individual risk (Eri) of the heavy metals and the potential ecological risk index (RI). Relating Table 1 and 4, Eri is divided into three main categories; low, high and very high risk at the four sampling junctions in the study area. High and very high risk was due to high Eri of Cd of 252.6, 408, 259.8, and 301.2 at MJ, UJ, FJ, and MM respectively. RI according to Table 1, is divided into considerable and high risk. High RI is due to high values of Eri at different sampling junctions.

Table 4. Ecological risk assessment of heavy metals in road side soil

| Heavy metals | MJ | UJ | FJ | MM |
| --- | --- | --- | --- | --- |
| Cd | 252.6 | 408 | 259.8 | 301.2 |
| Cu | 1.574 | 2.165 | 1.513 | 1.712 |
| Ni | 12.880 | 10.824 | 9.421 | 8.826 |
| Pb | 0.210 | 0.205 | 0.224 | 0.212 |
| Zn | 10.451 | 7.895 | 7.747 | 6.449 |
| Risk Index (RI) = ∑ Eri | 277.715 | 429.089 | 278.089 | 6.449 |

3.1.2 Health risk assessment

Table 5 presents the calculated three main pathways of human exposure to heavy metals on roadside soil, via ingestion, inhalation and dermal contact pathways. Table 6 presents the hazard quotient (HQ) calculated to determine non-carcinogenic health risk for each heavy metal. Table 7 presents the risk index (HI) calculated to measure the risk of carcinogenic health effects posed by heavy metals. Table 8 presents the life cancer risk (LCR) calculated to determine the health risk for carcinogenic heavy metals. According to Table 9, heavy metals posed no harmful effect on both adult and children health as HI < 1. The LCR for ingestion, inhalation and dermal contact for adults and children resulted from Pb and Cd. For Cd, LCR was within the threshold value of 1 × 10-6 – 1 × 10-4 which is widely considered acceptable except at MJ adult was 1 × 10-7. For Pb, LCR was in the range of 1 × 10-4 which is considered to have significant health effects on adults and children. However, it is imperative to pay attention to possible health risks due to exposure of adult and children to high concentrations of Cd and Pb on roadside soil.

Table 5 Human exposure pathways to heavy metals via ingestion (Ing), inhalation (Inh) and dermal contact pathways

| Parameters | MJ (DIng) |  | UJ (DIng) |  | FJ (DIng) |  | MM (DIng) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adult | Children | Adult | Children | Adult | Children | Adult | Children |
| Cd | 2.97E-6 | 2.79E-5 | 4.79E-5 | 4.51E-5 | 3.05E-5 | 2.87E-5 | 3.54E-5 | 3.33E-5 |
| Cu | 7.79E-6 | 7.33E-5 | 1.07E-5 | 1.01E-4 | 7.48E-5 | 7.04E-5 | 1.66E-5 | 7.97E-5 |
| Ni | 1.96E-5 | 1.84E-4 | 1.48E-5 | 1.39E-4 | 1.45E-5 | 1.37E-4 | 1.21E-5 | 1.14E-4 |
| Pb | 3.09E-5 | 2.90E-4 | 2.59E-5 | 2.44E-4 | 2.26E-5 | 2.12E-4 | 2.11E-5 | 1.99E-4 |
| Zn | 8.67E-6 | 8.15E-5 | 9.86E-5 | 9.24E-5 | 9.45E-5 | 8.89E-5 | 8.93E-5 | 8.41E-5 |
|  | MJ (DInh) |  | UJ (DInh) |  | FJ (DInh) |  | MM (DInh) |  |
| Cd | 4.45E-10 | 2.06E-9 | 7.14E-10 | 3.33E-9 | 2.27E-10 | 1.06E-9 | 5.27E-10 | 2.47E-9 |
| Cu | 1.16E-9 | 5.41E-9 | 1.59E-9 | 7.45E-9 | 1.11E-9 | 5.20E-9 | 1.26E-9 | 5.89E-9 |
| Ni | 2.92E-9 | 1.36E-8 | 2.20E-9 | 1.02E-8 | 2.16E-9 | 1.01E-8 | 1.80E-9 | 8.40E-9 |
| Pb | 4.60E-9 | 2.15E-8 | 3.86E-9 | 1.80E-8 | 3.36E-9 | 1.57E-8 | 3.15E-9 | 1.47E-8 |
| Zn | 1.29E-9 | 6.03E-9 | 1.47E-9 | 6.86E-9 | 1.40E-9 | 6.57E-9 | 1.33E-9 | 6.21E-9 |
|  | MJ (Ddermal) |  | UJ (Ddermal) |  | FJ (Ddermal) |  | MM (Ddermal) |  |
| Cd | 4.45E-9 | 3.20E-8 | 7.19E-9 | 5.18E-8 | 4.58E-9 | 3.30E-8 | 2.95E-11 | 3.82E-8 |
| Cu | 1.16E-8 | 8.42E-8 | 1.60E-5 | 1.15E-7 | 1.12E-8 | 8.09E-8 | 1.27E-8 | 9.16E-8 |
| Ni | 2.94E-8 | 2.11E-7 | 2.22E-8 | 1.60E-7 | 2.18E-8 | 1.57E-7 | 1.81E-8 | 1.30E-7 |
| Pb | 4.63E-7 | 3.33E-7 | 3.89E-8 | 2.80E-7 | 3.38E-8 | 2.44E-7 | 3.17E-8 | 2.28E-7 |
| Zn | 1.30E-8 | 9.37E-5 | 1.48E-8 | 1.06E-7 | 1.41E-8 | 1.02E-7 | 1.34E-8 | 9.66E-8 |

Table 6. Hazard quotient (HQ) calculated to determine non-carcinogenic health risk for each individual heavy metals

| Parameters | MJ (HQIng) |  | UJ (HQIng) |  | FJ (HQIng) |  | MM (HQIng) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adult | Children | Adult | Children | Adult | Children | Adult | Children |
| Cd | 2.96E-3 | 2.79E-2 | 4.79E-3 | 4.5E-2 | 3.05E-3 | 2.87E-2 | 3.54E-3 | 3.33E-2 |
| Cu | 2.10E-4 | 1.97E-3 | 2.89E-4 | 2.72E-3 | 2.02E-4 | 2.61E-5 | 4.48E-4 | 2.15E-3 |
| Ni | 2.15E-5 | 2.03E-4 | 1.63E-5 | 1.50E-4 | 1.59E-5 | 1.24E-4 | 1.32E-5 | 1.25E-4 |
| Pb | 8.81E-3 | 8.30E-2 | 7.41E-3 | 6.97E-2 | 6.45E-3 | 7.43E-6 | 6.04E-3 | 5.68E-2 |
| Zn | 2.88E-5 | 2.72E-4 | 3.29E-5 | 3.09E-4 | 3.15E-5 | 2.96E-4 | 2.98E-5 | 2.80E-4 |
|  | MJ (HQInh) |  | UJ (HQInh) |  | FJ (HQInh) |  | MM (HQInh) |  |
| Cd | 4.45E-7 | 2.06E-6 | 7.14E-7 | 3.33E-6 | 2.27E-7 | 1.06E-6 | 5.27E-7 | 2.47E-6 |
| Cu | 3.12E-8 | 1.46E-7 | 4.30E-8 | 2.00E-7 | 3.00E-8 | 1.40E-7 | 3.40E-8 | 1.58E-7 |
| Ni | 3.20E-9 | 1.49E-8 | 2.42E-9 | 1.13E-8 | 2.37E-9 | 1.11E-8 | 1.97E-9 | 9.23E-9 |
| Pb | 1.31E-6 | 6.13E-6 | 1.10E-6 | 5.15E-6 | 9.61E-7 | 4.48E-6 | 9.00E-7 | 4.20E-6 |
| Zn | 4.30E-9 | 2.01E-8 | 4.92E-9 | 2.28E-8 | 4.69E-9 | 2.19E-8 | 4.43E-9 | 2.07E-8 |
|  | MJ (HQdermal) |  | UJ (HQdermal) |  | FJ (HQdermal) |  | MM (HQdermal) |  |
| Cd | 4.45E-6 | 3.20E-5 | 7.19E-6 | 5.18E-5 | 4.58E-6 | 3.30E-5 | 2.95E-8 | 3.82E-5 |
| Cu | 3.15E-7 | 2.27E-6 | 4.33E-8 | 3.12E-6 | 3.02E-7 | 2.18E-6 | 3.42E-7 | 2.47E-6 |
| Ni | 3.23E-8 | 2.32E-7 | 2.44E-8 | 1.75E-7 | 2.39E-8 | 1.72E-7 | 1.99E-8 | 1.43E-7 |
| Pb | 1.32E-5 | 9.54E-5 | 1.11E-5 | 8.01E-5 | 9.68E-6 | 2.68E-7 | 9.06E-6 | 6.53E-5 |
| Zn | 4.33E-8 | 3.12E-7 | 4.93E-8 | 3.55E-7 | 4.72E-8 | 3.40E-7 | 4.47E-8 | 3.22E-7 |

Table 7. Risk index (HI) calculated to measure the risk of carcinogenic health effects pose by heavy

|  | MJ (HI) |  | UJ (HI) |  | FJ (HI) |  | MM (HI) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adult | Children | Adult | Children | Adult | Children | Adult | Children |
| Cd | 2.96E-3 | 2.73E-2 | 4.79E-3 | 4.51E-2 | 3.05E-3 | 2.87E-2 | 3.59E-3 | 3.33E-2 |
| Cu | 2.10E-4 | 1.97E-3 | 2.80E-4 | 2.72E-3 | 2.02E-3 | 2.18E-2 | 4.48E-4 | 2.15E-3 |
| Ni | 2.15E-5 | 2.00E-4 | 1.63E-5 | 1.50E-4 | 1.59E-5 | 1.20E-4 | 1.33E-5 | 1.20E-4 |
| Pb | 8.82E-3 | 8.31E-2 | 7.42E-3 | 6.97E-2 | 6.45E-3 | 1.21E-5 | 6.04E-3 | 5.68E-2 |
| Zn | 2.88E-5 | 2.70E-4 | 3.29E-5 | 3.00E-3 | 3.15E-5 | 2.96E-4 | 2.98E-5 | 2.80E-4 |

Table 8. Life cancer risk (LCR) calculated to determine the health risk for carcinogenic heavy metals.

| Parameters | MJ (LCRIng) |  | UJ (LCRIng) |  | FJ (LCRIng) |  | MM (LCRIng) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adult | Children | Adult | Children | Adult | Children | Adult | Children |
| Cd | 4.71E-7 | 4.42E-6 | 7.60E-6 | 7.15E-6 | 4.84E-6 | 4.55E-6 | 5.61E-6 | 5.28E-6 |
| Pb | 3.63E-3 | 3.41E-2 | 3.04E-3 | 2.87E-2 | 2.65E-3 | 2.49E-2 | 2.48E-3 | 9.89E-3 |
|  | MJ (LCRInh) |  | UJ (LCRInh) |  | FJ (LCRInh) |  | MM (LCRInh) |  |
| Cd | 7.06E-11 | 3.27E-10 | 1.13E-10 | 5.29E-10 | 3.60E-11 | 1.68E-10 | 8.36E-11 | 3.92E-10 |
| Pb | 5.41E-7 | 2.52E-6 | 4.54E-7 | 2.12E-6 | 3.95E-7 | 1.84E-6 | 3.70E-7 | 1.73E-6 |
|  | MJ (LCRdermal) |  | UJ (LCRdermal) |  | FJ (LCRdermal) |  | MM (LCRdermal) |  |
| Cd | 7.06E-10 | 5.09E-9 | 1.14E-9 | 8.22E-9 | 7.26E-10 | 5.23E-9 | 4.68E-12 | 6.07E-9 |
| Pb | 5.45E-5 | 3.92E-5 | 4.57E-6 | 3.30E-5 | 3.98E-6 | 2.87E-5 | 3.73E-6 | 2.69E-5 |
|  | LCR = ∑LCR ing + inh + dermal |  | LCR = ∑LCR ing + inh + dermal |  | LCR = ∑LCR ing + inh + dermal |  | LCR = ∑LCR ing + inh + dermal |  |
| Cd | 4.71E-7 | 4.37E-5 | 7.60E-6 | 7.16E-6 | 4.84E-6 | 4.56E-6 | 5.61E-6 | 5.29E-6 |
| Pb | 3.68E-3 | 3.41E-2 | 3.00E-3 | 2.87E-2 | 2.65E-3 | 2.49E-2 | 2.48E-3 | 9.91E-3 |

4. Conclusion

This study showed the presence of heavy metals including Cd, Cr, Cu, Fe, Mn, Ni, Pb, and Zn in roadside soil samples taken from various sample locations. Eri is divided into three main categories, low, high and very high risk at the four sampling junctions in the study area. Heavy metals pose no harmful effect on both adult and children health as HI < 1. The LCR for ingestion, inhalation and dermal contact for adult and children resulted from Pb and Cd. For Cd, LCR is within the threshold value of 1 × 10-6 – 1 × 10-4 which is widely considered acceptable except at MJ adult is 1 × 10-7. For Pb, LCR is in the range of 1 × 10-4 which is considered to have significant health effects on adults and children. However, it is imperative to pay attention to possible health risks due to exposure of adults and children to high concentrations of Cd and Pb on roadside soil in the study area. Therefore, immediate counter measures to reduce Cd and Pb contamination in roadside soil within the study area should be taken. Also, value-added traffic flows through good road constructions that could lead to improved fuel efficiency and better engine performance, thus reducing volume of vehicles wearing and tearing and idling traffic should be applied.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

References

[1] Li, X.D., Poon, C.S. & Pui, S.L. (2001). Heavy metal contamination of urban soils and street

dusts Hong Kong. App. Geochem.,16: 1361-1368.

[2] Poszyler-Adamska, A. & Czemiak, A. (2007). Biological and chemical indication of       roadside ecotonic zones.L. Environ. Engg. Landscape Management,15(2): 113a-118a.

[3] Alloway, J.B. (1995). Soil pollution and land contamination. In: Harrison RM (Ed).       Pollution: Causes, effects and control. The Royal Society of Chemistry, Cambridge.

[4] Akoto, O., Ephraim, J.H. & Darko, G. (2008). Heavy metal pollution in surface soils in the

vicinity of abundant raiway servicing workshop in Kumasi, Ghana.Int. J. Environ. Res.2(4):

359-364.

[5] Suzuki, K., Yabuki, T. & Ono, Y. (2008). Roadside Rhododendron pulchrum leaves as      bioindicators of heavy metal pollution in traffic areas of Okayama. Japan. Environ. Monit.      Asses. 149:133-141.

[6] Atayese, M.O.; Eigbadon, A.I.; Oluwa, K.A. & Adesodun, J.K. (2009). Heavy metal

contamination of amaranthus grown along major highways in Lagos. Afr. Crop. Sci. J.16:225-235.

[7] Ghrefat, H. & Yusuf, N. (2006). Assessing Mn, Fe, Cu, Zn and Cd pollution in bottom       sediments of Wadi Al-Arab Dam, Jordan.Chemosphere65: 2114–2121.

[8] De kimple, C.R. & Morel, J.F. (2000). Urban soil Management: a growing Concern. Soil     Sci., 165: 31-40.

[9] Manta, D.S., Angelone, M., Bellanca, A., Neri, R. & Sprovieri, M. (2002). Heavy metal in        urban soils: A case study from the city of Palermo (Sicily), Italy. Sci. Total Environ., 300:       229-243.

[10] Adefolalu, A.A. (1980). ‘Transport and rural integrated development in: proceedings of         the National conference on: Integrated Rural Dev. Women Dev., 1: 294-299.

[11] Mabogunje, A.L. (1980). ‘Development process a spatial perspective’ Hutchinson and Co publishers Ltd. pp. 234-244.

[12] Gray, C.W., Mclaren, R.G. & Roberts, A.H.C. (2003). ‘Atmospheric accessions of heavy        Metals to some New Zealand pastoral soils’. Sci. Total Environ., 305: 105-115.

[13] Nta, S. A. (2021). Ecological Risk Assessment of Heavy Metals in Contaminated Soil from a Point Source. World Academy of Science, Engineering and Technology International Journal of Environmental  and Ecological Engineering, 15(3), 119-122.

[14] Zhu, H; Yuan, X-Z; Zeng G-M. (2012). Ecological Risk Assessment of Heavy Metals in Sediments Xiawan Port Based on Modified Potential Ecological Risk Index. Trans Nonferrous + Met Soc China. 22,1470–1477. doi:10.1016/S1003-632.

[15] akanson, L. (1980). Ecological Risk Index for Aquatic Pollution Control, a Sedimentological approach. Water Research. 14, 975– 1001.

[16] Guo, G; Wu, F; Xie, F. (2012). Spatial distribution and pollution assessment of heavy metals in urban soils from southwest China. J Environ Sci. 24, 410–418. doi:10.1016/S1001-0742(11)60762-6.

[17] Islam, A.R.M.T; Rakib, M.A; Islam M.S. (2015). Assessment of health hazard of metal concentration Groundwater of Bangladesh. Am Chem Sci J. 5:41–49. doi:10.9734/ ACSj/2015/13175.

[18] Fadigas, F.D.S; Nelson, M.B; Sobrinho, A. (2014). Estimation of reference values for Cadmium, Cobalt, Chromium, Copper, Nickel, Lead, and Zinc in Brazilian soils. Commun Soil Sci Plant Anal. 37:945 959. doi:10.1080/00103620 600583885.

[19] USEPA. (1989). Risk Assessment Guidance for Superfund Volume I Human Health Evaluation Manual (part A). I.

[20] USEPA. (1996). Acid Digestion of Sediments, Sludges, and Soils. 1–12.

[21] Zheng, N; Liu, J; Wang, Q, (2010). Health risk assessment of heavy metal exposure to street dust in the zinc Smelting district, Northeast of China. Sci Total Environ. 408, 726–733. doi:10.1016/j.scitotenv.2009.10.075.

[22] Lee, S; Lee, B; Kim, J. (2006). contamination in the abandoned metal mine areas, Korea. Environ Monit Assess. 233–244. doi:10.1007/s10661-005-9024-5.

[23] Luo, X; Ding, J; Xu, B. (2012). Incorporating bioaccessibility into human health risk assessments of heavy metals in urban park soils. Sci Total Environ. 424, 88–96. doi:10.1016/j.scitotenv.2012.02.053.

[24] USEPA. (2002). Supplemental guidance for developing soil screening office of emergency and remedial response.

Nduka, J. K., Umeh, T. C., Kelle, H. I., Mgbemena, M. N., Nnamani, R. A., & Okafor, P. C. (2023). Ecological and health risk assessment of heavy metals in roadside soil, dust and water of three economic zone in Enugu, Nigeria. Urban Climate, 51, 101627.

Ayaz, H., Nawaz, R., Nasim, I., Irshad, M. A., Irfan, A., Khurshid, I., ... & Bourhia, M. (2023). Comprehensive human health risk assessment of heavy metal contamination in urban soils: insights from selected metropolitan zones. Frontiers in Environmental Science, 11, 1260317.

Panqing, Y., Abliz, A., Xiaoli, S., & Aisaiduli, H. (2023). Human health-risk assessment of heavy metal–contaminated soil based on Monte Carlo simulation. Scientific Reports, 13(1), 7033.

A.R., Awoyemi, Opasola, O.A., Adiama, B.Y., Agboola, O.E, Awoyemi, A.G., Ekundayo, D.E., and Atimiwoaye, A.D. 2025. “Heavy Metal Contamination in Soils Near Waste Dumpsite in Ado-Ekiti, Nigeria Using Polllution and Geo-Accumulation Indices”. Asian Journal of Environment & Ecology 24 (3):76-86. https://doi.org/10.9734/ajee/2025/v24i3671.

Tripathi, Kaushlendra Mani, Deo Kumar, and Suraj Mishra. 2024. “Effect of Contamination of Heavy Metals in Soil and Its Mitigation Strategies: A Review”. International Journal of Plant & Soil Science 36 (7):135-46. https://doi.org/10.9734/ijpss/2024/v36i74715.

Mohamed Y. Hanfi & Ilia V. Yarmoshenko (2020) Health risk assessment quantification from heavy metals contamination in the urban soil and urban surface deposited sediment, Journal of Taibah University for Science, 14:1, 285-293, DOI: 10.1080/16583655.2020.1735735