Original Research Article

Relationship Between Compliance with Hearing Protection Use and Audiometry Results in Workers Exposed to High Noise Levels

.

ABSTRACT

Background: Noise exposure in the work environment is one of the main risk factors for hearing loss in industrial workers, especially when the noise level exceeds 85 dB. The use of hearing protection devices (HPD) is an important preventive measure, but their effectiveness is highly dependent on the level of compliance with their use by workers. Objective: This study aims to determine the relationship between HPD compliance and audiometry results in workers at PT Elangperdana. Method: This study used a quantitative approach with a cross-sectional design. A sample of 204 workers from the production department of PT Elangperdana who were exposed to noise above 85 dB was selected using a total sampling technique. Data were collected through noise intensity measurements using a sound level meter, questionnaires related to compliance with the use of HPD, and the latest audiometry test results. Data analysis was performed using the chi-square test with the help of SPSS. Results: The results of the data analysis showed that there was a significant relationship between noise exposure above 85 dB and compliance with the use of HPD on audiometry results (p <0.001). In addition, workers with working hours of more than 8 hours and working periods of more than 5 years have a higher prevalence of hearing loss. Conclusion: Excessive noise exposure in the work environment adversely affects hearing health, and compliance with using HPD is very important to prevent hearing loss.

Keywords: Hearing Protection Devices (HPD), Audiometry, Noise, Factory workers, Hearing Health

1. INTRODUCTION

The era of globalization has increased the use of modern technology in the industrial sector, which has positive and negative impacts. One of the negative impacts is noise that can interfere with employees' hearing health (Lekhman et al. 2021). In the modern industrial world, efficiency and productivity often require the use of heavy and high-tech machines. Although this supports the achievement of production targets, the increased use of machines also has an impact on the work environment, especially in the form of noise. Noise typically means unwanted sound, especially one that is loud, harsh, or disruptive, but has a significant effect on workers' health, especially the hearing system. The World Health Organization (WHO) states that excessive exposure to noise can cause permanent hearing loss, even at a young age (DeRuiter and Ramachandran 2021; TRANSMIGRASI 2011). In Indonesia, noise has become a major issue in occupational safety and health (OHS) (Simarmata and Hendrika 2025). Based on the Regulation of the Minister of Manpower and Transmigration No. PER.13/MEN/X/2011 concerning the Threshold Limit Value of Physical Factors in the Workplace, the maximum noise exposure limit is 85 dB(A) for 8 working hours. Exposure exceeding this limit, especially if it occurs continuously without adequate protection, can cause Induced Hearing Loss (NIHL), irreversible noise-induced hearing loss (Saha et al. 2022). Elangperdana Tire Industry is a company that operates in the tire industry, where personal protective equipment (PPE) is very important to protect workers from the risk of noise. Noise is an unwanted sound that can interfere with comfort and health. Prolonged exposure to noise in the work environment can cause Noise-Induced Hearing Loss (NIHL), which is hearing loss due to exposure to loud noises. In Indonesia, the noise threshold is set by the Regulation of the Minister of Manpower of the Republic of Indonesia Number 5 of 2018, which states that exposure to noise above 85 dB for more than 8 hours can increase the risk of hearing loss (Gopinath et al. 2021; Nurrokhmawati, Nataliningrum, and Anggraeny 2022)

Therefore, workers need to use hearing protection devices (HPD) to reduce these risks. Ear Protection Devices, such as ear muffs, ear plugs, and helmets, are specifically designed to reduce the intensity of noise received by the ear. In addition, audiometry is a test used to evaluate a person's hearing ability (Surianti 2012) involving measuring hearing sensitivity at various frequencies and sound intensities. Audiometry results can provide important information about the worker's hearing health condition and help in the early detection of hearing disorders (Demirtaş, Saygun, and Bayar Muluk 2022). Many cases show that even though HPD has been provided, workers are often reluctant to use it consistently due to reasons of comfort, habit, lack of awareness, or weak supervision. Audiometry examination, as one of the early diagnosis methods for NIHL, needs to be done routinely to monitor the hearing status of workers (Doutres et al. 2019). Unfortunately, many companies have not integrated audiometry results with evaluations of work behavior and environmental conditions. As a result, early detection of hearing loss is often missed until the condition is severe (Uddin et al. 2023). Based on this background, this study was conducted to systematically examine the relationship between noise exposure levels above 85 dB and compliance with the use of ear protection devices against audiometric results in PT Elangperdana workers. It is hoped that the results of this study can provide input for company management, K3 agencies, and other related parties in designing more effective preventive strategies for workers' hearing protection.

2. material and methods

This study used a cross-sectional analytical observational design to explore the relationship between noise exposure exceeding 85 dB and compliance with the use of hearing protection devices (HPD) on the results of audiometry examinations in workers at PT Elangperdana. The study was conducted at PT Elangperdana Tire Industry, located on Jl. Elang, Sukahati, Citeureup District, Bogor Regency, West Java on May 14 to 31, 2024. The population of this study was factory workers who were exposed to noise of more than 85 dB. Sampling was done using a purposive sampling technique, so that a sample of 204 workers was obtained who met the inclusion criteria, namely workers who had worked for at least 6 months, aged 18 to 55 years, and were willing to participate in this study. Data collection was carried out through three main stages: 1) Noise Measurement, 2) Questionnaire Distribution, 3) Audiometry Examination.Data were analyzed using the latest version of the SPSS program with the following stages 1) Univariate Analysis (to describe the distribution of variables, such as frequency of noise exposure, level of HPD compliance, and audiometry results); 2) Bivariate Analysis (using the Chi-square test to determine the relationship between Noise exposure and audiometry results, and Compliance with HPD use and audiometry results; 3) Multivariate Analysis (using logistic regression to determine the contribution of each independent variable to audiometry results simultaneously).

3. results and discussion

The data in table 1 shows that the majority of respondents are ≤ 44 years old (94.1%), male (99.5%), have a high school education (97.5%), have a working period of > 5 years (90.7%), and work ≤ 8 hours (70.6%). Most of the respondents sometimes use hearing protection devices (HPD) (51%).

Table 1. Characteristics of Research Workers

| Characteristic | Frequency (n) | Percentage (%) |
| --- | --- | --- |
| Age |  |  |
| ≤ 44 year 44 year | 192 12 | 94.1 5.9 |
| Gender |  |  |
| Male Female | 203 1 | 99.5 0.5 |
| Education |  |  |
| Junior High School/equivalent Senior High School/equivalent Bachelor's Degree/equivalent Master's Degree/equivalent Doctorate/equivalent | 1 199 3 0 1 | 0.5 97.5 1.5 0 0.5 |
| Work Period |  |  |
| ≤ 5 year 5 year | 19 185 | 9.3 90.7 |
| Length of working |  |  |
| ≤ 8 hour 8 hour | 144 60 | 70.6 29.4 |
| HPD Usage Compliance |  |  |
| Always Sometimes Never | 76 104 24 | 37.3 51.0 11.8 |

Table 2 shows the results of noise measurements at PT Elangperdana, showing that the noise intensity in four areas (testing tire, mill mixing, mill extruding, and mill calendar) exceeds the Threshold Limit Value (NAB) of 85 dB for exposure of 8 hours per day

Table 2. Noise measurement results at PT Elangperdana

| Measurement Point | Noise Intensity | Noise Type |
| --- | --- | --- |
| Tire testing area Mill mixing area Extruding mill area Calendar mill area | 102 96.4 88.8 90.3 | Continue Continue Continue Continue |

Table 3 shows the audiometry test results, showing that in the right ear, 84.8% of respondents had normal hearing, while 15.2% had mild deafness. In the left ear, 94.6% of respondents had normal hearing and 5.4% had mild deafness.

Table 3. Results of the audiometry examination of the right and left ears

| Audiometry Result Reference Category | Right Ear |  | Left Ear |  |
| --- | --- | --- | --- | --- |
|  | n | % | n | % |
| Normal Mild Deafness | 173 31 | 84.8 15.2 | 193 11 | 94.6 5.4 |
| Total | 204 | 100 | 204 | 100 |

Table 4 shows data on the majority of respondents, namely 90.7%, did not experience hearing complaints, while the most common complaint was tinnitus, which was experienced by 4.9% of respondents.

Table 4. Hearing complaints experienced by factory workers

| Hearing Complaints | Frequency (n) | Percentage (%) |
| --- | --- | --- |
| No hearing complaints Tinnitus Hearing fatigue Ears feel full Tinnitus and hearing fatigue | 185 10 6 1 1 | 90.7 4.9 2.9 0.5 0.5 |

Tables 5 and 6 show that in the age group ≤44 years, 164 workers had normal audiometry results and 28 had mild deafness. At age >44 years, 9 workers were normal and 3 had mild deafness. The p-value for the right ear was 0.306, indicating no significant relationship. For the left ear, 182 workers ≤44 years were normal and 10 had mild deafness; while >44 years, 11 were normal and 1 had mild deafness, with a p value of 0.644, also indicating no significant relationship.

Table 5. Relationship between age and right ear audiometry results

| Age | Reference Category |  | Total | P value |
| --- | --- | --- | --- | --- |
|  | Normal | Mild Deafness |  |  |
| ≤ 44 year 44 year | 164 9 | 28 3 | 192 12 | 0.306 |
| Total | 173 | 31 | 204 |  |

Table 6. Relationship between age and left ear audiometry results

| Age | Reference Category |  | Total | P value |
| --- | --- | --- | --- | --- |
|  | Normal | Mild Deafness |  |  |
| ≤ 44 year 44 year | 182 11 | 10 1 | 192 12 | 0.644 |
| Total | 193 | 11 | 204 |  |

In tables 7 and 8, it can be seen that in the group of ≤5 years of service, 108 workers had normal audiometry results, and 3 had mild deafness. In the group >5 years, 65 workers were normal and 28 had mild deafness. The p-value for the right ear was <0.001, indicating a significant relationship. For the left ear, in the group of ≤5 years of service, 109 workers were normal and 2 had mild deafness; while >5 years, 84 were normal and 9 had mild deafness, with a p-value <0.001, also indicating a significant relationship.

Table 7. Relationship between Work Period and right ear audiometry results

| Work Period | Reference Category |  | Total | P value |
| --- | --- | --- | --- | --- |
|  | Normal | Mild Deafness |  |  |
| ≤ 5 year 5 year | 108 65 | 3 28 | 111 93 | 0.001 |
| Total | 173 | 31 | 204 |  |

Table 8. Relationship between Work Period and left ear audiometry results

| Work Period | Reference Category |  | Total | P value |
| --- | --- | --- | --- | --- |
|  | Normal | Mild Deafness |  |  |
| ≤ 5 year 5 year | 109 84 | 2 9 | 111 93 | 0.001 |
| Total | 193 | 11 | 204 |  |

Tables 9 and 10 show that in the group of working hours ≤8 hours, 141 workers had normal audiometry results, and 3 had mild deafness. In the group >8 hours, 32 workers were normal and 28 had mild deafness. The p-value for the right ear was <0.001, indicating a significant relationship. For the left ear, in the group of working hours ≤8 hours, 142 workers were normal and 2 had mild deafness; while >8 hours, 51 were normal and 9 had mild deafness, with a p-value <0.001, also indicating a significant relationship

Table 9. Relationship between Working Hour and right ear audiometry results

| Working Hour | Reference Category |  | Total | P value |
| --- | --- | --- | --- | --- |
|  | Normal | Mild Deafness |  |  |
| ≤ 8 hour 8 hour | 141 32 | 3 28 | 144 60 | 0.001 |
| Total | 173 | 31 | 204 |  |

Table 10. Relationship between Working Hours and left ear audiometry results

| Working Hour | Reference Category |  | Total | P value |
| --- | --- | --- | --- | --- |
|  | Normal | Mild Deafness |  |  |
| ≤ 8 hour 8 hour | 142 51 | 2 9 | 144 60 | 0.001 |
| Total | 193 | 11 | 204 |  |

In the group that always used HPD, 76 workers had normal audiometry results, and none had mild deafness. In the group that sometimes used HPD, 94 workers were normal, and 10 had mild deafness; while in the group that never used HPD, 3 workers were normal, and 21 had mild deafness. The p-value for the right ear was <0.001, indicating a significant relationship. For the left ear, in the group that always used HPD, 76 workers were normal; sometimes, 102 were normal, and 2 had mild deafness; never, 15 were normal, and 9 had mild deafness, with a p-value <0.001, also indicating a significant relationship.

Table 11. Relationship between HPD compliance and right ear audiometry results

| HPD Usage Compliance | Reference Category |  | Total | P value |
| --- | --- | --- | --- | --- |
|  | Normal | Mild Deafness |  |  |
| always sometimes never | 76 94 3 | 0 10 21 | 76 104 24 | 0.001 |
| Total | 173 | 31 | 204 |  |

Table 12. Relationship between HPD compliance and left ear audiometry results

| HPD Usage Compliance | Reference Category |  | Total | P value |
| --- | --- | --- | --- | --- |
|  | Normal | Mild Deafness |  |  |
| always sometimes never | 76 102 15 | 0 2 9 | 76 104 24 | 0.001 |
| Total | 193 | 11 | 204 |  |

Discussion

Noise

The results of the study showed that high noise exposure in the work environment is directly related to decreased hearing ability. Research by Saha et al. found that 46.7% of factory workers exposed to noise above 85 dB experienced hearing loss. in line with the results of this study, which showed that workers at PT Elangperdana have a higher risk of hearing loss. Hearing Status Audiometry results showed more cases of mild deafness in the right ear (15.2%) than the left ear (5.4%). Research by Gopinath et al. also showed that workers in noisy environments are more susceptible to hearing loss in the right ear, where 16.2% of workers experienced mild deafness in the right ear and 9% in the left ear (Gong et al. 2021).

Hearing Complaints:

Tinnitus and hearing fatigue are common complaints among factory workers. A study by Nurrokhmawati et al. showed that 54% of factory workers experienced tinnitus, indicating that this condition is common among workers exposed to noise (Nurrokhmawati et al. 2022). Age and Audiometry Results 6 This study did not find a significant relationship between age and audiometry results, although age is generally associated with hearing loss. Surianti et al. found that there was no significant relationship between age and hearing complaints in respondents aged ≤40 years and >40 years, supporting the finding that other factors may be more influential in the age range studied (Surianti 2012).

Working Period and Audiometry Results

This study shows that workers with a working period of > 5 years have a higher risk of experiencing mild deafness. A study by Demirtaş et al. found that long-term noise exposure correlated with increased hearing loss, especially in environments exceeding 85 dB. Working Period and Audiometry Results (Demirtaş et al. 2022). There is a significant relationship between the working period and audiometry results. Workers who work more than 8 hours have a higher prevalence of mild deafness (Rachmantoko et al. 2021). Uddin et al. found that workers with exposure of more than 8 hours have a higher incidence of NIHL, supporting the results of this study (Uddin et al. 2023).

Compliance with HPD Use and Audiometry Results

There is a significant relationship between compliance with HPD use and audiometry results. Workers who always use HPD have normal audiometry results, while those who never use HPD are at high risk of mild deafness. Research by Gong et al. also showed that workers who inconsistently use HPD have a higher risk of NIHL

4. Conclusion

This study revealed a significant relationship between noise exposure above 85 dB and compliance with the use of Ear Protection Devices (HPD) on audiometry results of workers at PT Elangperdana. High noise levels in the production area increase the risk of hearing loss, especially for workers who do not routinely use HPD. Although age did not show a significant relationship, the factors of length of service and length of service were proven to be related to audiometry results. The results showed that workers with a work period of more than 5 years and a work period of 10 hours or more than 8 hours had a higher prevalence of mild deafness.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Demirtaş, Hasan, Meral Saygun, and Nuray Bayar Muluk. 2022. “Is There a Relationship between Occupational Noise and Hearing Levels, Sleep Quality, and QoL of the Factory Workers?” Neurological Sciences 43(2):1015–23. doi: 10.1007/s10072-021-05458-x.

- DeRuiter, Mark, and Virginia Ramachandran. 2021. Basic Audiometry Learning Manual. Plural Publishing.

- Doutres, Olivier, Franck Sgard, Jonathan Terroir, Nellie Perrin, Caroline Jolly, Chantal Gauvin, and Alessia Negrini. 2019. “A Critical Review of the Literature on Comfort of Hearing Protection Devices: Definition of Comfort and Identification of Its Main Attributes for Earplug Types.” International Journal of Audiology 58(12):824–33. doi: 10.1080/14992027.2019.1646930.

- Doutres, Olivier, Franck Sgard, Jonathan Terroir, Nellie Perrin, Caroline Jolly, Chantal Gauvin, and Alessia Negrini. 2022. “A Critical Review of the Literature on Comfort of Hearing Protection Devices: Analysis of the Comfort Measurement Variability.” International Journal of Occupational Safety and Ergonomics 28(1):447–58. doi: 10.1080/10803548.2020.1772546.

- Gong, Wei, Liangliang Zhao, Ling Li, Thais C. Morata, Wei Qiu, Huiling Amy Feng, and Baoli Zhu. 2021. “Evaluating the Effectiveness of Earplugs in Preventing Noise-Induced Hearing Loss in an Auto Parts Factory in China.” International Journal of Environmental Research and Public Health 18(13):7190.

- Gopinath, Bamini, Catherine McMahon, Diana Tang, George Burlutsky, and Paul Mitchell. 2021. “Workplace Noise Exposure and the Prevalence and 10-Year Incidence of Age-Related Hearing Loss.” PLoS One 16(7):e0255356.

- Lekhman, M., M. Shubitidze, A. Litvinov, I. Zanina, O. Pashkova, and N. Kobzeva. 2021. “A Dynamic Increase in the Impact of Noise on the Human Body, in Particular, on Cardiovascular Diseases.” P. 12110 in Vol. 1029. IOP Publishing.

- Nurrokhmawati, Yanti, Desire M. Nataliningrum, and Dian Anggraeny. 2022. “Noise Induced Hearing Loss among Furniture Factory Workers PT Chitose Cimahi.” ACTA Medical Health Sciences 1(1):10–18.

- Rachmantoko, R., Z. Afif, D. Rahmawati, R. Rakhmatiar, and S. Nandar Kurniawan. 2021. “Diabetic Neuropathic Pain. JPHV (Journal of Pain, Vertigo and Headache), 2 (1), 8–12.”

- Saha, Suranjit Kumar, M. M. Shahin Ul Islam, Md Hasnat Yousuf Zaki, Md Nizam Uddin, Mohammad Rezaul Quader, Sharmin Rahman, Prativa Saha, and Molla Sharfuddin Ahmad. 2022. “Mode, Pattern and Precipitating Factors of Acute Poisoning in a Southern District Hospital in Bangladesh.” Bangabandhu Sheikh Mujib Medical College Journal 1(2):85–89.

- Simarmata, Vidi Posdo Ahapta, and Wendy Hendrika. 2025. “The Relationship Of Knowledge, Attitudes, And Practices On The Use Of Personal Protective Equipment With The Incident Of Work Accidents.” Indonesian Journal of Global Health Research 7(2):823–32.

- Surianti, Rina. 2012. “Hubungan Tingkat Pajanan Kebisingan Dengan Keluhan Pendengaran Pada Pekerja Bagian Produksi PT Sanggar Sarana Baja Tahun 2012 (Skripsi).” Depok: Fakultas Kesehatan Masyarakat, Universitas Indonesia.

- TRANSMIGRASI, MENTERI TENAGA KERJA D. A. N. 2011. “Peraturan Menteri Tenaga Kerja Dan Transmigrasi Republik Indonesia.”

- Uddin, A. K. M. Shaif, L. Naher, M. L. Rahaman, and M. S. Islam. 2023. “Prevalence of Noise Induced Hearing Loss among the Textile Workers.” Bangladesh Journal of Otorhinolaryngology 29(2):89–95.