Studies on drying rate and characteristics of Babul gum (Acacia nelotica)

Abstract

The present study aimed to investigate the drying kinetics and behaviour of Babul gum (Acacia nilotica) using a conventional tray dryer, in order to identify optimal drying conditions for improved shelf life and product quality. Fresh exudate gum samples were collected from the field in the form of grits, cleaned, and uniformly spread in a thin layer for drying. Drying was performed at four different temperatures (50°C, 60°C, 70°C, and 80°C) with a constant airflow rate of 1.5 m/s. Drying attributes such as moisture content, drying rate, moisture ratio, and drying time were measured and analysed. The drying process for all conditions occurred exclusively in the falling rate period, indicating moisture movement controlled by internal diffusion. The drying time decreased significantly with increasing temperature, ranging from 2165 minutes at 50°C to 3395 minutes at 80°C. Moisture loss followed a nonlinear trend, with the fastest reduction occurring in the early stages of drying. Sensory evaluation showed that samples dried at 70°C had superior texture, appearance, and overall acceptability, making them the most suitable for storage and value-added applications. Based on the results, conventional tray drying at 70°C is recommended for effective moisture removal while retaining product quality.

Key words: Babul gum, acacia nelotica, Drying rate, Drying characteristics

1.	Introduction

“Gum is one of the important non-timber forest produce (NTFP) and viable source of income for thousands of forest dwellers, especially tribals in India. Gum production is a pillar of family economy and considered as an income-generating source that requires only a low input of work after the rainy season” (Hasalkar and Jadhav, 2004; Prasad and Giri, 2011). The major commercially important gums in good quantity are sourced from the central Indian forests, comprising of Madhya Pradesh, Chhattisgarh, Andhra Pradesh, Orissa, Jharkhand and Bihar and to some extent Gujarat and Rajasthan. Gums are primarily categorized as Grade-I of Karaya (Sterculia urens) and Grade-II of Dhawada (Anogeisus latifolia), Babool (Acacia indica) and Khair (Acacia catechu) in the Chhattisgarh state. The above four gums are nationalized gums in the state of Chhattisgarh.

“Acacia nelotica belonging to the family Leguminosae and sub family Mimosaceae. It is indigenously known as ‘Babul’ or ‘Kikar’ which is moderate sized with a spreading crown tree and is broadly scattered in tropical and subtropical countries” (Verma, 2016; Ali et al., 2012). “In India, natural babul forests are generally found in Maharashtra, Gujarat, Andhra Pradesh, Rajasthan, Haryana and Karnataka. However, scattered trees in groups occur naturally and also widely planted in almost all states and Union territories except north-eastern states and Kashmir” (Baishya et al., 2009).

Acacia nelotica has been recognized worldwide as a multipurpose tree and extensively used in traditional agro-forestry system. (National Academy of Sciences, 1980). “It is estimated that, there are roughly 1380 species of acacia worldwide, about two-third of them are native to Australia and rest are spread over tropical and subtropical regions of the world” (Maslin et al., 2003)

The babul tree, scientifically known as Acacia nilotica, is a versatile species thriving in diverse environments, particularly in regions with rainfall between 500 to 1250 mm. This medium-sized, thorny tree can reach heights of up to 15 meters and is characterized by its unique bark coloration, which varies from orange and green in younger trees to dark and rough in older specimens (Singhdoha et al., 2017). Its adaptability to alluvial and sandy loam soils makes it a valuable asset in agroforestry and land rehabilitation efforts. The bark, a tinge of orange and green (young tree), but older trees have dark, rough bark and tend to lose their thorns (Devi et al., 2023).

“A gum in general, is any water-soluble or water-swellable polysaccharide. It exudes from the wounds in bark. Exudate gums are produced by some trees, shrub and herbaceous perennials and are generally produced either naturally or in response to injury. The gum exudes from trees and shrubs in tear-like, striated nodules or amorphous lumps. It occurs in the form of rounded or ovoid tears and size up to 1cm and color varies from pale-yellow to brown or almost black. Gum production increases at high temperature and limited moisture. It generally exudes between the month of March and May” (Kipo et al., 2023).

Babul gum, derived from the acacia species, is rich in various polysaccharides and phenolic compounds, which contribute significantly to its medicinal properties. The gum contains substantial amounts of galactose, L-arabinose, and L-rhamnose, along with four aldobiouronic acids that are crucial for maintaining its structural integrity (Hongsing et al., 2012). Phenolic constituents such as m-digallic acid, gallic acid, and flavonoids like leucocyanidin and epicatechol are also present, contributing to its well-known antioxidant properties (Jourdes et al., 2012). Moreover, babul gum exhibits a high tannin content, including both condensed tannins and phlobatannins, which enhance its therapeutic potential (Jani et al., 2016).

In terms of its polysaccharide composition, babul gum contains elevated levels of galactose (42.57%), rhamnose (10.63%), and arabinose (21.42%), as reported by Hongsing et al. (2012). The phenolic compounds identified include gallic acid, protocatechuic acid, and various flavonoids (Zachariah et al., 2012), which contribute to its ability to scavenge free radicals, further supported by its tannin content Babul gum has traditionally been used for its anti-inflammatory and antimicrobial properties, showcasing its broad medicinal potential (Rather and Mohammad, 2015; Jani et al., 2016).

However, despite these benefits, the extraction and processing of babul gum can lead to variability in its chemical composition, potentially impacting its efficacy in medicinal applications (Raj et al., 2015). Gum production per tree is highly variable and influenced by environmental factors such as temperature, humidity, season, tree girth, and tapping methods. Maximum yields can reach up to 1 kg per year, but average yields are often much lower, sometimes just a few grams per tree (Raj et al., 2022; ).This variability poses challenges in ensuring consistent quality for therapeutic use.

“At the time of harvesting the initial moisture content of babul gum is higher and because of that quality of gum was lost during storage and transportation. Thus, to reduce these losses and to maintain the quality of gum, drying is important. There are two basic methods of drying, i.e. natural and mechanical. The natural (open sun drying) drying may results in contamination by dust and insects, therefore, most of the time conventional tray dryer is used for drying of agricultural products. The advantage of mechanical drying is to get quality product through proper hygienic conditions with reduced product loss. Therefore, the aim of the present research work is to study the drying kinetics of babul gum using a conventional  tray dryer, which provides hygienic and controlled drying conditions over traditional sun or open-air drying methods” (SETHI, I. B. et al., 2017).

2.	Materials and Methods

2.1	Collection of material

The exudate Babul gum sample was obtained from Network Project on Harvesting, Processing and Value Addition of Natural Resin and Gums, Indira Gandhi Krishi Vishwavidyalaya, Raipur, Chhattisgarh, India.

2.2	Experimental set-up

2.2.1	Drying setup

The conventional tray dryer (Shri Rudran instrument Co.) was used for conducting the drying experiments. It mainly consisted of a fan, air-heating chamber, temperature control unit, drying chamber, plenum chamber, hot air inlet and outlet with a maximum attainable temperature of 100°C, similar setup was also used by Tikhomirov et al., 2024. Fresh air is heated by electric heaters vertically fitted at the inlet of the dryer, air supplied by the fan was heated to the required temperature in the heating chamber, which was provided, with 8 heating coils. Inlet temperature of the air varied by adjusting the control unit system. The blower is powered by 1hp single phase electric motor with a direct online starter (Singh and Singh, 2014).

2.2.2	Drying procedure

The grits were dried in laboratory tray dryer at different air temperature i.e. 50, 60, 70 and 80°C at an inside constant air velocity. The dryer started about 30 minutes before each drying run at respective drying temperatures. A 25 g of the gum samples were weighed and uniformly spread onto pre-weighed petri dish in a thin layer. The loss in moisture was recorded using analytical balance at interval of 30 min. The weight of gum before drying i.e. at zero drying time, was considered as the initial weight. The gum containing dish were taken out, weighed and placed back after each drying interval in about 30 min to avoid any significant temperature variation in the dryer. Air velocity inside the dryer was kept constant during each

Fig:1 Babul Grits (before drying)

Fig:2 Babul Grits (after drying)

drying run (Singh and Singh, 2014).

2.4	Determination of initial moisture content (IMC)

Initial moisture content of gum sample was determined by drying at 105°C till their constant weight (Singh and Singh, 2014), using hot air oven. The moisture content was calculated as:

Moisture content db=Amount of moisture lossWeight of bone dried sample×100          …..(1)

The amount of moisture loss was calculated by taking a difference between the mass of the original sample and that of dried sample.

2.5	Determination of equilibrium moisture content

The sample was allowed to dry under actual drying conditions till the weight of sample became constant and moisture content of the dried sample was assumed to be the equilibrium moisture content (EMC).

2.6	Dry matter

Dry matter, left after complete removal of moisture from the product. The initial moisture content of various samples was determined by oven drying method, as described earlier. The dry matter percentage and weight of dry matter in sample were calculated as follows (Quirino et al., 2023):

Dry matter %=100-initial moisture content % wb                              …..(2)

Weight of dry matter=initial mass of sample ×dry matter (%)100                   …..(3)

2.7	Drying rate

The drying rate of babul gum grits was calculated using the following equation (Doymaz, 2014):

dmdt=Mt-Mt+∆t∆t                                                                                                         …..(4)

Where,

Mt + Δt  = Moisture content at t + Δt (g water/ g dry matter).

t = Time (min).

2.8	Moisture ratio

The moisture ratio (MR) of the samples is defined according to the following equation (Doymaz, 2014):

MR=Mt-MeMo-Me                                                                                                             …..(5)
Where,

Mt 	= Moisture content at any time (g water / g dry matter)

Mo 	= Initial moisture content (g water / g dry matter)

Me 	= Equilibrium moisture content (g water / g dry matter)

t 	= Drying time (min)

2.9	Sensory evaluation

The evaluation of organoleptic properties of prepared value-added products a 9-Point hedonic scale method was selected. A rating scale and test procedure have been derived from the theoretical basis. The scale has nine points; these points were given word descriptions from, “dislike extremely” to “like extremely”. The length of scale was determined experimentally. Replicate testing of products of varying hedonic value showed that responses were repeated more consistently when scale has 9, rather than 5, 7 or 11 points. The scale points were numbered from 1 to 9 and arithmetic mean of points checked is used as desired index. The panels of semi- trained judges consisting of 11 members were gave value added products samples for evaluation of organoleptic properties viz. appearance, colour, texture and overall acceptability. The following list was given to show us the details of the scale to judges (Wichchukit, S., and O'Mahony, M. 2015).

Hedonic Scale

9. Like extremely

8. Like very much

7. Like moderately

6. Like slightly

5. Neither like nor dislike

4. Dislike slightly

3. Dislike moderately

2. Dislike very much

1. Dislike extremely

The semi-trained panel of judges was selected among the professors and students for the sensory evolution from Faculty of Agricultural Engineering, IGKV, Raipur.

3.	Results and Discussion

3.1	Effect of temperature on drying behaviour of babul gum grits

3.1.1	Moisture reduction at different temperatures

The moisture content versus drying time curves for Babul gum grits at different drying air temperatures are presented in Figure 3. The initial moisture content (IMC) of the samples was 33.36% (dry basis). The drying process was conducted at four temperatures: 50°C, 60°C, 70°C, and 80°C. The final moisture contents achieved were 14.90%, 13.54%, 12.21%, and 10.97% (db), respectively.

A clear trend was observed in which higher drying temperatures resulted in lower final moisture contents and shorter drying durations. This indicates that the drying rate increased with increasing temperature, reducing the overall drying time.

Fig.3. Moisture content (%db) versus drying time (min)

3.1.2	Drying kinetics and moisture- time relationship

The moisture-time relationship exhibited a nonlinear pattern, characterized by a steep moisture loss in the initial phase of drying followed by a slower, gradual decrease. This behavior indicates that moisture removal occurred predominantly in the falling rate period, which is typical of drying governed by internal diffusion rather than surface evaporation.

Table 1 presents the predicted R² values for moisture content regression models at each drying temperature. The values ranged from 0.713 to 0.744, indicating a reasonably good model fit for describing moisture content changes during drying.

Table 1. Predicted R2 values of moisture content of Babul gum grits samples with different drying temperatures

| S. No. | Drying temperature (°C) | Drying time (min) | R2 |
| --- | --- | --- | --- |
| 1 | 50 | 2165 | 0.716 |
| 2 | 60 | 2615 | 0.720 |
| 3 | 70 | 3125 | 0.744 |
| 4 | 80 | 3395 | 0.713 |

3.1.3      Drying Rate Behaviour with Moisture Content

The drying rate versus moisture content curves are shown in Figure 4. The drying rate decreased nearly linearly with decreasing moisture content across all drying temperatures. Importantly, no constant rate drying period was observed, suggesting that the entire drying process occurred in the falling rate phase.

As temperature increased, drying rates improved, especially during the early drying stages. At 70°C, for instance, the drying rate decreased from 0.2777 to 0.0003 g water/g dry matter-min as the moisture content dropped from 33.36% to 12.21% (db), highlighting the impact of temperature on drying kinetics.

Fig. 4. Drying rate versus moisture content (db)

3.2	Sensory evaluation

The nine-point hedonic scale was used for product quality evaluation on the basis of appearance, colour, texture and overall acceptability. The samples were analysed through a group of 11 untrained peoples from Faculty of Agricultural Engineering, IGKV, Raipur. Fig. 5 shows the average points given to the all-dried samples named by dying product at 50°C to 80°C by the judges. There were no more differences in points given by judges to all products. The samples dried at 70°C was found better sensory appearance than other sample and suitable for long duration storage.

Fig. 5 Sensory attributes of all dried samples of Babul gum grits at different temperature

3.3       Identification of Optimal Drying Temperature

Across all temperatures, the drying process followed a consistent pattern initial rapid drying followed by a slower decline. The temperature of 70°C offered the best results in terms of drying efficiency. Which is further supported by sensory evaluation results discussed in Section 3.2. Therefore, 70°C can be considered an optimal drying condition for Babul gum under the studied parameters.

4.	Conclusion

Drying is necessity in developing countries like India where farmers are using traditionally evolved methods based upon natural sources like sun drying and wind drying, having major drawback of contamination. The application of drying technology on this locally available plant based biocompatible gum may boost its industrial use. The use of preservatives to prevent deterioration upon storage can be exempted by proper drying of gums. The drying kinetics of babul gum in a conventional tray dryer at three air temperatures (50, 60, 70 and 80ºC) was investigated. The curve for moisture content versus drying time was shows that the drying time was reduced when the drying temperature increased from 50 to 80°C. The dimensionless moisture ratio was observed to decrease uniformly with decrease in drying time. However, for all the temperatures, drying rate was highest during initial period of drying. The best result show at 70°C on the basis of sensory evaluation. This methodology best suitable for drying of gum at farm and industrial level.

Acknowledgement

Authors acknowledge the financial support received under Network Project on harvesting, processing and value addition of natural resins and gum, National Institute of Secondary Agriculture (ICAR-NISA), Namkum, Ranchi, Jharkhand.

Disclaimer (Artificial Intelligence Usage)

The author(s) hereby declare that generative AI technologies were used during the writing and editing of this manuscript to enhance language clarity, structure, and formatting. Specifically, ChatGPT (OpenAI, GPT-4.5 model) and GEN TEXT AI Assistant were utilized.

Details of AI Usage:

- ChatGPT (OpenAI, GPT-4.5) was used for Refining grammar, sentence structure, and technical clarity. Rewriting and restructuring the abstract and result discussion sections. Formatting content and improve readability.

- GEN TEXT AI Assistant was used for Typesetting assistance and Ensuring alignment with scientific document layout standards.

All inputs to AI tools were based on original research content, and final decisions on inclusion and interpretation of outputs were made by the author(s) to maintain scientific accuracy and integrity.

4. Reference

Ali, A., Akhtar, N., Khan, B.A., Khan, M.S., Rasul, A., Shahiq-uz-zaman, Khalid, N., Waseem, K., Mahmood, T., & Ali, L. (2012). Acacia nilotica: A plant of multipurpose medicinal uses. Journal of Medicinal Plants Research, 6, 1492-1496.

Baishya, R., Barik, S.K., & Upadhaya, K. (2009). Distribution pattern of aboveground biomass in natural and plantation forests of humid tropics in northeast India. Tropical Ecology, 50, 295-304.

Devi, A., Jhariya, M. K., Raj, A., Banerjee, A., Singh, K. P., & Singh, B. (2023). Acacia nilotica: A Promising Species for Soil Sustainability. Land and Environmental Management through Forestry, 339-353.

Doymaz, I. (2014). Drying kinetics and rehydration characteristics of convective hot‐air dried white button mushroom slices. Journal of Chemistry, 2014(1), 453175.

Hasalkar, S., & Jadhav, V.S. (2004). Role of Women in the Use of Non-Timber Forest Produce: A Review. Journal of Social Sciences, 8, 203 - 206.

Hongsing, P., Palanuvej, C., & Ruangrungsi, N. (2012). Chemical compositions and biological activities of selected exudate gums. J. Chem. Pharm. Res, 4, 4174-4180.

Jani, B. L., Devani, B. M., Vyas, D. M., & Akbari, S. H. (2016). Quality analysis of Acacia nilotica (Babul) gum exudates. International Journal of Food and Fermentation Technology, 6(2), 367-372.

Jourdes, M., Pouységu, L., Quideau, S., Mattivi, F., Truchado, P., & Tomás Barberán, F. A. (2012). Hydrolyzable tannins: gallotannins, ellagitannins, and ellagic acid. Handbook of analysis of active compounds in functional foods, 435-460.

Kipo, S. L., Ofori-Kwakye, K., Kuntworbe, N., Johnson, R., Boakye-Gyasi, M. E., Osei, Y. A., & Owusu, F. A. (2023). Physicochemical and microbiological characteristics of stem bark exudate gum of Cordia millenii tree in conventional release tablets. The Scientific World Journal, 2023(1), 9118067.

Maslin, B.R., Miller, J.T., Seigler and D.S. (2003). Overview of the generic status of Acacia (Leguminosae: Mimosoideae). Australian Systematic Botany, 16(1): 1-18.

Prasad, N., & Giri, S. K. (2011). Status of primary processing of gum karaya (Exudate gum) and scope of improvement. Journal of Non-Timber Forest Products, 18(3), 191-194.

Quirino, D. F., Lima, N. S., Palma, M. N., Franco, M. O., & Detmann, E. (2023). Evaluation of heating times for loss on drying at 105° C for estimation of laboratory dry matter in animal feeds. Journal of AOAC International, 106(2), 261-266.

Raj, A., & Jhariya, M. K. (2022). Effect of environmental variables on Acacia gum production in the tropics of Chhattisgarh, India. Environment, Development and Sustainability, 24(5), 6435-6448.

Raj, A., Haokip, V., & Chandrawanshi, S. (2015). Acacia nilotica :A Multipurpose Tree and Source of Indian Gum Arabic. South Indian Journal of Biological Sciences, 1, 66-69.

Rather, L., , S., & Mohammad, F. (2015). Acacia nilotica (L.): A review of its traditional uses, phytochemistry, and pharmacology. Sustainable Chemistry and Pharmacy, 2, 12-30.

Singh, J., and Singh, B. (2014). Drying Kinetics of Gum Karaya (sterculia urens) Using Convective Air Dryer. British J. Of Pharmaceutical Research. 4(13) 1647-1656.

Singhdoha, A., Dhillon, R. S., & Johar, V. (2017). Estimation of genetic diversity among Superior CPTs of Acacia nilotica. Int. J. Curr. Microbiol. App. Sci, 6(10), 1197-1202.

Tikhomirov, D., Khimenko, A., Kuzmichev, A., Budnikov, D., & Bolshev, V. (2024). Raising the Drying Unit for Fruits and Vegetables Energy Efficiency by Application of Thermoelectric Heat Pump. Agriculture, 14(6), 922.

Verma, S. (2016). A review on ethnomedicinal plant Acacia nilotica (Linn.) wild. Journal of Pharmacognosy and Phytochemistry, 5, 241-242.

Wichchukit, S., & O'Mahony, M. (2015). The 9‐point hedonic scale and hedonic ranking in food science: some reappraisals and alternatives. Journal of the Science of Food and Agriculture, 95(11), 2167-2178.

Zachariah, S. M., Viswanad, V., Aleykutty, N. A., Jaykar, B., & Halima, O. A. (2012). Free radical scavenging and antibacterial activity of Mirabilis jalapa Linn using in vitro models. Asian journal of pharmaceutical and clinical research, 5(3), 115-120.

SETHI, I. B., SEWHAG, M., & JAJORIA, M. (2017). Effect of Seed Rate on Chickpea Growth and Yield: A Review.