A REVIEW ON REMEDIATION OF HEAVY METALS

Abstract

Environmental contamination remains one of the most important global issues. Efforts are made to prevent the discharge of toxic substances into the environment. Many technologies for remediation of soil, surface water, and groundwater have been developed over time.The objective of the review is to highlight the various ways to remediate heavy metal contamination especially Chemical remediation technologies which are of tremendous interest because they can remove and decompose contaminants in contaminated areas.This review focusses on various remediation strategies, including precipitation, flocculation, adsorption/ ion exchange, chemical oxidation, soil washing among others.Remediation technologies are typically always integrated, however they can also be utilised alone. The type of pollutants and site conditions will determine which system is most cost-effective and efficient. Future Research should be directed towards improving existing and developing new environmentally friendly remedial solutions.

Keywords: Remediation, Heavy metals, Environmental pollution

Introduction.

Heavy metals are dangerous pollutants which occur either geogenic or due to anthropogenic activities; they can be harmful when present in a long time in the soil because they can easily undergo chemical transformation and can be assimilated by plants which when consumed by humans can be injurious to health (Shamsuddeen et al., 2024). Many scholars have showed the effect of some heavy metals on organisms, crops, soil and human health. (Iqbal et al.,2023 & Kumari et al.,2022). Heavy metals are nondegradable and can alter the structure, diversity, and function of the ecosystems which can pose serious threats to the environmental(Iqbal et al.,2023 & Younas et al., 2022).They have density higher than 5 g/cm3 and atomic mass greater than (>20), they are natural constituent of the soil that originate from the weathering of parent material of soil, called “Paedogenesis” Thou they occurred naturally, their concentration in the soil is low which increases gradually (Bakshi & Abhilash., 2020). Both  plant and  microbiome suffer when heavy metals are present in the soil. They cause stress that results in the production of ROS (Reactive Oxygen Species), which harms the vital macromolecules within an organism and has an impact on the ecology and nutritional value of crops and vegetation.The Challenges of environmental pollution has taken on social dimensions, especially the advent of  modern industrialization  which has  recently taken a dangerous trend. (Ozyigit et al., 2022). Soil pollution increases the level of heavy metals in the ecosystems.(Nkoh et al.,2022).Remediation of contaminated soils with heavy metals involves a wide range of approaches that can be broadly categorized into physical, chemical and biological methods (Rasafi et al.,2023 & Cai et al., 2017).  Many studies showed remediation technology of Contaminated Soil with heavy metals are classified under the following three techniques: (1) enhancing the mobility of heavy metals within the soil to facilitate separation (2) replacing and fixing the existing type of heavy metals in soil to reduce its mobility; (3) removal of heavy metals from farms and preventing them from further spreading and contaminating the environment (Xie et al.,2012). In heavy metal remediation some methods ‘Contain’ pollutants, e.g precipitation, sorption, drain system, slurry wall; Some methods remove pollutants e.g thermal, electrokinetic, air sparging, flushing, excavation and other methods degrade pollutants e.g chemical oxidation, permeable reactive barriers, phytoremediation and stimulated attenuation. (Javanovic et al., 2021).

HARMFUL EFFECT OF HEAVY METALS

Some of the harmful effects of some heavy metals on humans include kidney disease, headache, hypertension, itai itai disease, cancer of the lungs, prostate diseases of the bone, lymphocytosis, Azheimers disease, Anorexia and chronic nephropathy among others. The major concern regarding these heavy metal contaminants is that they are persistent, non-biodegradable and toxic in nature. (Amin et al., 2021). Heavy metal (loid) contamination has risen globally exponentially which negatively affect human health, this however has been the emerging challenge across the planet and pose a significant rise to food security and crop productivity especially in populous countries. (Rai et al., 2019).

METHODOLOGIES

STRATEGIES OF REMEDIATION

Before any remediation decision, the most important step is to conduct site characterization so as to determine the type and nature of contaminant as this will aid in decision making. A detail information of the contaminant has to be taken under consideration. After getting all the necessary data about the contaminant, then decision about remedial action is made which involve design, implementation and monitoring. It is equally important to note that each site may have different characteristics and as such they must be treated differently(Shamsuddeen et al., 2024). Remediation technologies are basically classified into two main strategies which are In situ remediation and ex situ remediation; Insitu remediation treats the pollutant in the original place without moving the polluted soil. In contrast ex situ remediation are carried out by excavation and removal of the polluted soil and taken elsewhere for treatment (Chen et al., 2022).

TYPES OF REMEDIATION

The major types of remediation include physical remediation, chemical remediation and biological remediation (Rasafi et al.,2023 & Cai et al., 2017).

PHYSICAL REMEDIATION

Physical remediation entails the removal of contaminated soil from the location. This technique effectively removes substantial quantities of soil rapidly and efficiently; nevertheless ,it can be expensive and may not be appropriate for many locations. Types of physical remediation encompass soil washing, soil replacement, Electrokinetic methods, and vitrification techniques.

Soil washing technique: This method of remediation involves treating the contaminated soil using physicals means by washing or replacing the contaminated soil (Yao et al., 2012). In soil washing the pollutants are usually dissolved or mobilized using water or a mixture of water and some chemical additives. The contaminants are then subsequently removed from the soil using physical techniques like flotation or sedimentation. Most importantly any Chemical to be use for soil washing must have the capability of breaking strong bonds between the soil and metals and at the same time dissolve the metals.  When soil contaminated with heavy metal is washed with aqueous solution of chelating reagent, it readily binds with the metal to form complex which is soluble in water and the metal get removed from the contaminated soil (Sun et al., 2020). EDTA is also widely used for washing of contaminated soil because of its abilities to form water soluble complexes with almost all heavy metal (Voglar & Lestan., 2014).

Soil replacement: The method of Soil replacement is easy, fastest and a direct way of physical remediation. The contaminated soil is carefully excavated, removed, treated and replaced directly or blended with uncontaminated soil using techniques such as landfilling, surface capping and capsulation. Barrier walls are inserted to prevent dispersion from the contaminated sites. Physical barriers that are constructed with impermeable materials like steel, cement, bentonite, and grout, are used for capping, vertical and horizontal containment. This method is straight forward and can easily reduce the movement of heavy metals from the soil into the groundwater (Cha,2024).

Electrokinetic Remediation:

The insitu method of electrokinetic remediation mobilizes and removes contaminants from low permeability soil using a low-intensity electric field. This method of remediation technology can separate contaminant from the soil by either electromigration, electropercolation and electrophoresis in the low density DC electric field (Sun et al., 2023). The contaminants migrate toward the electrodes where they are collected and extracted when an electrode is placed into the contaminated soil and a direct current is delivered to the electrodes. Adams (2022) showed that significant advancements have been achieved in the use of composite materials and conductive polymers for electrodes, which lead to more constant electric fields and faster rates of contaminant migration, in order to boost the electrodes' durability and effectiveness.

Vitrification : In the process of Vitrification, the contaminated soil is heated to a temperature of 1,200°C until it melts and forms into a material that resembles glass. This technique stops pollutants from seeping into the environment and lowers their bioavailability(Trifunović ,2021). Adam et al., (2023) described the use of fluxing agents to lower the melting temperature required for vitrification, and they demonstrated that doing so can reduce carbon emissions and energy costs. Furthermore, improvements in furnace design and the use of other energy sources, such as plasma arcs, have improved vitrification's effectiveness and environmental sustainability as a rehabilitation method.

Chemical remediation: Chemical remediation employs the use of chemicals in removal of heavy metals or contaminants. Pollutants, especially persistent organic chemicals can be eliminated or rendered immobile using chemical remediation technology. The mechanism of heavy metal remediation  include Adsorption, complexation, electrostatic attraction, ion exchange and precipitation processes, which caused the redistribution of Heavy metals from soil liquid phases to solid phases, restrict their mobility and bioavailability(Singh and Singh,2023).

Precipitation :The creation of a solid in solution or on the surface during the sorption process is known as precipitation. Insoluble compounds are created when metal ions dissolved in water and react with other substances. Various separation procedures can then be used to remove the precipitate that has developed. pH and concentration are the primary factors influencing the precipitation process. In alkaline liquids, heavy metals frequently create insoluble hydroxides. By adding appropriate precipitating agents, they can also precipitate as sulfides, carbonates, and sulfates. If environmental factors such as pH is altered, this process can be undone.  Sometimes, in situ precipitation of specific salts or oxides is used for remediation, which results in the co-precipitation and adsorption of pollutants ; e.g Insitu hydrous ferric oxide can be employed to treat subsurface iodine contamination (Wang et al.,2020a).

Complexation: In the complexe formation of heavy metals on the surface and within the pores of biochar, basic and acidic functional groups such as carbonyl, carboxylic, phenolic, and hydroxyl are essential. Compared to biochar produced by pyrolysis, which involves high temperatures, low temperature biochar has more polar functional groups.  Metal immobilization is caused by the formation of inner and outer sphere complexes with oxygenated (acid) functional groups. Complexation is the most common method of adsorption with charcoal for metals that are soluble.

Flocculation: Flocculation is the most popular technique for removing suspended particles from water. It uses flocculants, also known as clarifying or flocking agents, which speed up the agglomeration of colloidal particles, which are typically not subject to gravity and cannot be filtered.(Panalytical, 2024).

Adsorption and ion exchange: Adsorption is a commonly used remediation technique that can be used to remove both organic and inorganic pollutants, including pesticides and different hydrocarbons, as well as heavy metals and non-metallic ions. It is frequently characterized as a surface phenomenon where contaminants build up at the interface between two phases (Rashed., 2013). When a highly porous material (adsorbent) and a pollutant-containing solution (adsorbate) come into contact, the pollutants are deposited on the solid surface due to intermolecular forces of attraction. Adsorpion can either be Physical where adsorption doesn't depend on the molecules' electrical characteristics. Intermolecular forces between the adsorbent and the adsorbate are the cause of it. It can either be Chemical adsorption where the adsorbent and the adsorbate create a chemical connection and Electrostatic adsorption, also known as ion exchange where  adsorption is frequently categorized as ion exchange because it contains Coulombic attractive forces that produce ion-ion interactions. The process by which mobile ions from an external solution that comes into contact with the solid phase replace easily exchangeable ions in the solid phase is known as ion exchange(Ahmed et al.,2024).

Chemical Oxidation: In order to decompose organic contaminants, chemical oxidants are introduced to contaminated locations using the well-established process known as in situ chemical oxidation. Either less hazardous chemicals or carbon dioxide and water are produced from organic pollutants. The primary oxidants are ozone, permanganate, persulfate, hydrogen peroxide/Fenton's reagent and a few more readily available oxidants. They are preferred due to their affordability, oxidizing ability, non-toxicity, and availability. Because each of these oxidants might have a different reaction chemistry, the choice is based on the type and nature of the contaminant as well as the features of the polluted site. Chemical oxidation has been applied in recent years in conjunction with enzymes, inorganic catalysts, ultraviolet (UV) light, ultrasonic (US), or a mix of oxidants (Insitu ISCO,2022).

Biological remediation

To eliminate heavy metal pollutants from the environment, this method uses biological processes found in microbes and plants or both. When compared to other remediation techniques, bioremediation is a more economical and environmentally beneficial means of removing heavy metals and other contaminants from the environment. Chemical and physical techniques have shown to be costly and occasionally inefficient, particularly in situations with low metal concentrations. Additionally, these techniques typically result in large volumes of toxic sludge. Microorganisms used for the removal of heavy metals in contaminated soils are bacterial, fungi, yeast and algae. For better results Bioremediation is achieved successfully when many bacterial strains are utilized rather than a single strain of organism.(Khadka, 2023).

Mycoremediation: This is a form of bioremediation that employ the use of Fungi in bioremediation technique to remove heavy metals.  Heavy metals can be sequestered by fungi via biosorption and bioaccumulation. This method typically entails the introduction of fungus, also referred to as "bioremediation fungi" or "mycofilters," into the contaminated areas through a variety of methods, including enzymatic breakdown, bioaccumulation, or conversion into inert forms, these fungi have the capacity to degrade or change the pollutants into less dangerous compounds (Kumar et al., 2021).

Phytoremediation: Phytoremediation is a bioremediation method that uses plants to eliminate heavy metals from the environment When contaminants are present in the plant's root zone and cover a large region, this approach works better and more efficiently, This method include  strategies like phytoextraction, phytofiltration , phytostablization, phytovolatilization and phytodegradation. (Chai,2024)

Phytoextraction : A type of phytoremediation known as phytoextraction uses plants to remediate, eliminate, or break down harmful pollutants, typically metals that are harmful to living things even at low concentrations in soil or water.(Cha,2024., Tabassum & Mazar., 2023).  Additionally, it has been demonstrated that phytoextraction is a cost-effective and environmentally friendly method of removing heavy metals like lead, Zinc and Cadmium (Remigio et al., 2020).

Phytofiltration: This is another form of phytoremediaion which includes techniques like rhizofiltration, blastofiltration, and caulofiltration. Those techniques remove contaminants from water using plant systems (Chai,2024). Rhizofiltration focuses on using plant roots to actively absorb pollutants, including heavy metals, from water. According to Abdullahi (2015) rhizofiltration is a practical way to clean up water, particularly for pollutants like Cd and Cr. Blastofiltration employs certain plant seedlings to remove heavy metals like lead and cadmium from water by making use of the plant's capacity to absorb these metals and lower their concentration, thereby purifying the water. Caulofiltration uses plant shoots. Both methods are effective and environmentally beneficial ways to treat polluted water (Yan et al., 2020). Despite being primarily intended for water purification, rhizofiltration has proven effective over time in eliminating organic compounds and heavy metals from contaminated water sources. The special capacity of plants to absorb and accumulate heavy metals and other pollutants is what makes the natural filtration process work. To maintain the efficacy of rhizofiltration, this technique necessitates routine plant harvesting. The continual cleaning of the contaminated water is guaranteed by the continuous cycle of plant growth, absorption and harvest .  Rhizofiltration is a sustainable, economical, and environmentally beneficial way of treating water. The plant Typha latifolia's  rhizofiltration capacity for eliminating heavy metals from wastewater was examined by Wang et al., (2020) where he showed how successful the plant is at purifying contaminated water. Typha angustifolia is also another aquatic plant that exhibits high rhizofiltration ability. It can absorb Cd and Zn at rates of 12 mg per plant and 58 mg per plant, respectively.

Phytostabilization : This type of Phytoremediation is also referred to as "in-place activation,".The technique uses green plants to stabilise or localise soil metal pollutants inside their roots or close to the rhizosphere, hence reducing human exposure to them ;these method is economical, straightforward, environmentally benign, and do not necessitate substantial work (Chai,2024., Mazar & Tabassum., 2023). In addition to physically fortifying the soil against the effects of rain, erosion, and leaching, dense canopy plants and roots systems, like grass species, also stop contaminants from migrating off-site (Shen et al., 2022). The ability to alter the speciation of trace elements in the soil with the goal of decreasing their solubility and exchangeable fraction is one of phytostabilization's main objectives. Limiting the intake of trace elements by crops stabilises the plant canopy, and limiting plant mobility improves the ecosystem (Oladoye et al., 2022). By carefully selecting and using plant species that may form connections with soil contaminants, this technique effectively prevents heavy metals from moving to other locations or bodies of water (Aloud et al., (2022). By creating a barrier of defense, these plants keep pollutants where they are. According to research by Lorestani (2023), applying these amendments can significantly lower the bioavailability of copper and arsenic in contaminated soils, particularly when paired with the growth of white lupin. This suggests a workable strategy for the phytostabilization of polluted areas. phytostabilization alters the soil properties that influence pollutant speciation and immobilization.Because they contain compounds that interact with soil contaminants and aid in their precipitation or immobilization, plant exudates from plant roots are also crucial for phytostabilization.(Yan et al.,2020).

Phytovolatilization: This mehod is used to eliminate organic contaminants such as chlorinated solvents; heavy metals and other contaminants are mostly absorbed by plant roots, which are then transformed into volatile or gaseous forms before being released into the atmosphere by transpiration (Muthusaravanan et al., 2020). When working with volatile heavy metals and certain organic molecules, this approach works well. (Aloud et al., 2022). Plant species that are known to be able to absorb volatile pollutants through their roots are chosen for this approach. Only heavy metals like mercury and selenium, which may exist in the environment as gaseous species but do not necessarily do so, can be converted by plants into less dangerous or harmless gases within their tissues upon absorption.

Phytodegradation: This is the transformation or breakdown of metal contaminants by plant metabolic processes following uptake from the soils . In phytoremediation of organics, plant metabolism contributes to the contaminant reduction by transformation, break down, stabilisation or volatilising contaminant compounds from soil and groundwater. All phytoremediation technologies are not exclusive and may be used simultaneously, but the metal extraction depends on its bio available fraction in soil.(Chai, 2024).

Conclusion: The significance that remediation plays as a long-term remedy for heavy metal contamination in soil ecosystems is highlighted by economic factors, societal acceptance, and regulatory consequences. This review examined a number of heavy metal remediation methods, tactics, and strategies. Despite the potential of some of the remediation techniques outlined, it is important to recognize their limits, including their long-term efficacy and potential ecological effects. Future research should evaluate long-term sustainability, ecological integration, investigate synergies with microbial remediation and genetic engineering, focusing on various species of plants with enhanced metal absorption.

REFFRENCES

Abdullahi, M.S. (2015) Chapter 18-Soil Contamination, Remediation and Plants: Prospects and        Challenges; Remediation, S., Plants, H., Eds.; Academic Press: San Diego, CA, USA, ISBN 978-0-12-799937-1.

Amit Kumar , Ajar Nath Yadav , Raju Mondal , Divjot Kour et al (2021) Myco-remediation: A mechanistic understanding of contaminants alleviation from natural environment and future prospect. Chemosphere Volume 284 December 2021, 131325

Amin, I., Nazir, R., and Rather, M. A. (2021). Nano-bioremediation: an innovative approach for remedying heavy metals using fungi. J. Bioremediat. Biodegrad. (12), 2.

Adam, A. B., et al. (2022). Nanomaterial-enhanced soil washing: Advances in technology and application for heavy metal removal. Environmental Science & Technology, 56(11), 6583-6591.

Adam, A. B., Muhammad, J. A., & Kaugama, A. A (2023). Impact of heavy metals content found in irrigated red pepper in challawa industrial area. 5th international palandoken scientific studies congress 18-19 march erzurum / turkey.

Bakshi, M., and Abhilash, P. C. (2020). “Nanotechnology for soil remediation: revitalizing the tarnished resource,” in Nano-Materials as Photocatalysts for Degradation of Environmental Pollutants (Varanasi: Elsevier), 345–370. doi: 10.1016/B978-0-12-818598-8.00017-1

Cha H. Strategies for contaminated soil remediation using phytoremediation. J Biomed Imag Bioeng. 2024;8(1):202

Chemical Oxidation (In Situ - ISCO). (2022, April 28). Enviro Wiki, . Retrieved 23:15, March 8, 2025 from https://www.enviro.wiki/index.php?title=Chemical_Oxidation_(In_Situ_-_ISCO)&oldid=15912.

Corzo Remigio, A.; Chaney, R.L.; Baker, A.J.M.; Edraki, M.; Erskine, P.D.; Echevarria, G.(2020) Phytoextraction of High Value Elements and Contaminants from Mining and Mineral Wastes: Opportunities and Limitations. Plant Soil. 449, 11–37.

Iqbal, B., Zhao, X., Khan, K. Y., Javed, Q., Na- zar, M., Khan, I., Zhao, X., Li, G., Du, D. (2024). Microplastics meet invasive plants: unraveling the ecological hazards to agroecosystems. Science of ‏.167756 ,906 ,the Total Environment

Khan, F.I., Husain, T., Hejazi, R., (2004). J. Environ. Manage., 71, 95–122. doi:10.1016/j.jenvman.2004.02.003

Kumari, A., Kumari, P., Rajput, V.D., Sushkova, S.N., Minkina, T. (2022). Metal (loid) nanosorbents in restoration of polluted soils: geochemical, ecotoxi- cological, and remediation perspectives. Environ- ‏.246–235 ,)1(44 ,mental Geochemistry and Health

Malvern Panalytical(2024) What is flocculation in water treatment? https://www.malvernpanalytical.com/en/learn/knowledge-center/insights/what-is-flocculation-in-water-treatment

Mohamed Ahmed .,Mohammed Ahmed Reda Ahmed., Marwa., Abdelfattah Abdallah(2024) Applications of Ion Exchange in Environmental Remediation: Removal of Heavy Metals From Wastewater Remediation Journal 34(4). DOI: 10.1002/rem.21788

Muthusaravanan, S.; Sivarajasekar, N.; Vivek, J.S.; Vasudha Priyadharshini, S.; Paramasivan, T.; Dhakal, N.; Naushad, M.(2020) Research Updates on Heavy Metal Phytoremediation: Enhancements, Efficient Post-Harvesting Strategies and Economic Opportunities. In Green Materials for Wastewater Treatment; Naushad, E., Mu, L., Eds.; Springer International Publishing: Cham, Germany, pp. 191–222, ISBN 978-3-030-17724-9.

M. Cai, M. B. McBride, K. Li, e Z. Li,(2017) Bioaccessibility of As and Pb in orchard and urban soils amended with phosphate, Fe oxide and organic matter», Chemosphere, vol. 173, pp. 153–159, doi: 10.1016/j.chemosphere.2017.01.049.

Nkoh, J.N., Ajibade, F.O., Atakpa, E.O., Abdulaha- Al Baquy, M., Mia, S., Odii, E.C., Xu, R. (2022). Reduction of heavy metal uptake from polluted soils and associated health risks

Oladoye, P. O., Olowe, O. M., & Asemoloye, M. D. (2022). Phytoremediation technology and food security impacts of heavy metal contaminated soils: A review of literature. Chemosphere, 288, 132555. https://doi.org/10.1016/j.chemosphere .2021.132555

Ozyigit, I.I., Karahan, F., Yalcin, I.E., Hocaoglu- Ozyigit, A., Ilcim, A. (2022). Heavy metals and trace elements detected in the leaves of medicinal plants collected in the southeast part of Turkey. Arabian ‏.21–1 ,15 ,Journal of Geoscience

Rai, P. K., Lee, S. S., Zhang, M., Tsang, Y. F., and Kim, K. H. (2019). Heavy metals in food crops: health risks, fate, mechanisms, and management. Environ. Int. 125, 365–385. doi: 10.1016/j.envint.2019.01.067

Rashed, M.N., (2013). Adsorption technique for the removal of organic pollutants from water and wastewater, in: Rashed, M.N. (Ed.) Organic Pollutants - Monitoring, Risk and Treatment. InTech Open, Rijeka. doi:10.5772/54048.

Sangya Singh  and Shubham Singh (2023) Biochar Mediated heavy metal remediation from soil. Research Trends in Agriculture Science Volume I (ISBN: 978-93-88901-65-9) p 161-168

Shamsuddeen, R., Adam, A. B., Mathew, T. S., & Abubakar, M. Y. (2024). Environmental Remediation Using Nanoparticles: A Review. Kwaghe International Journal of Sciences and Technology, 1(1), 428-442.

Shen, X., Dai, M., Yang, J., Sun, L., Tan, X., Peng, C., ... & Naz, I. (2022). A critical review on the phytoremediation of heavy metals from environment: Performance and challenges.  Chemosphere, 291, https://doi.org/10.1016/j. chemosphere.2021.132979

Smriti Khadka (2023) Bioremediation: Factors, Types, Advantages, Disadvantages August 3,Mirobe note.

Sun, Tao, et al. (2020)"Optimizing extraction procedures for better removal of potentially toxic elements during EDTA-assisted soil washing."Journal of Soils and Sediments 20 (2020): 3417-3426.

Tabassum, I., Mazhar, S., & Sarfraz, B. (2009). Phytoremediation of Lead Contaminated Soil using Sorghum Plant in. Environ Sci, 21(10), 1432- 1437. https://doi.org/10.32350 /sir.63

Tijana Jovanović, Milica Petrović, Miloš Kostić, Danijela Bojić, Aleksandar Bojić (2021) Chemical remediation technologies   facta universitatis Series: Physics, Chemistry and Technology Vol. 19, Number 1, 2021, pp. 1 - 15 https://doi.org/10.2298/FUPCT2101001J

T. El Rasafi et al.,(2023) Recent progress on emerging technologies for trace elements-contaminated soil remediation», Chemosphere, vol. 341, p. 140121, doi: 10.1016/j.chemosphere.2023.140121.

Vanja Trifunovic (2021) Vitrification as a method of soil remediation. Zastita materijala 62(3):166-179. DOI: 10.5937/zasmat2103166T

Voglar, David, and Domen Lestan.(2014) "Chelant soil-washing technology for metal-contaminated soil. “Environmental technology 35.11 (2014): 1389- 1400.

Wang, Y., Liu, Y., Zhan, W., Zheng, K., Lian, M., Zhang, C.,  Xinling Ruan ., Tao Li. (2020). Long- term stabilization of Cd in agricultural soil using mercapto-functionalized nano- silica (MPTS/nano-silica): a three-year field study. Ecotoxicol. Environ. Saf. 197, 110600. doi: 10.1016/j.ecoenv.2020.110600

Wang, G., Szecsody, J.E., Avalos, N.M., Qafoku, N.P., Freedman, V.L.,(2020a). J. Contam. Hydrol., 235, 103705. doi:10.1016/j.jconhyd.2020.103705

Xie, Y.F., Cao, Y.Z., Zhang, D.D., Liu, X.J., Li, F.S.(2012)Engineering control technologies and its application in the risk management for contaminated sites. J. Environ. Eng. Technol. 2(1),pp. 51–59

Yan, A.; Wang, Y.; Tan, S.N.; Mohd Yusof, M.L.; Ghosh, S.; Chen, Z. (2020) Phytoremediation: A Promising Approach for Revegetation of Heavy Metal-Polluted Land. Front. Plant Sci. 2020, 11, 359.

Yao, J., Cheng, Y., Zhou, M., Zhao, S., Lin, S., Wang, X.,   Jiangjiexing Wu,  Sirong Li, Hui Wei (2018). ROS scavenging Mn 3 O 4 nanozymes for in vivo anti-inflammation. Chem. Sci. 9, 2927–2933. doi: 10.1039/C7SC05476A

Younas, H., Nazir, A., Bareen, F.E. (2022). Application of microbe-impregnated tannery solid waste biochar in soil enhances growth performance of sunflower. Environmental Science and Pollution ‏.57687–57669 ,)38(29

Y. Chen, H. Jiang, Y. Li, Y.Liu, Y. Chen, L. Chen, X. Luo, P. Tang, H. Yan, M. Zhao, Y.Yuan and S. Hou(2022) A critical review on EDTA washing in soil remediation for potentially toxic elements (PTEs) pollutants, Rev. in Environ. Sci. Biotech. Vol. 21, 2022, pp.399–423.

PanelZeying Sun, Miaomiao Zhao, Li Chen, Zhiyang Gong, Junjie Hu, Degang Ma(2023)Electrokinetic remediation for the removal of heavy metals in soil: Limitations, solutions and prospection. Science of The Total Environment ,   Volume 903, 10 December 2023, 165970.  https://doi.org/10.1016/j.scitotenv.2023.165970