Review Article

Evaluating Drinking Water Quality Standards and Technologies: A Comparative Analysis of Kenya, China and Germany.

Abstract

In this study, the drinking water quality regulations and technologies of Kenya, China, and Germany are compared to highlight the challenges and best practices in managing water quality across different levels of development. Access to clean water is a fundamental human right and is crucial for public health and sustainable development. Despite progress in some regions, many countries struggle to provide clean drinking water due to inadequate infrastructure, insufficient regulation, and emerging contaminants.

The regulatory frameworks, treatment technologies, monitoring and compliance methods, and access to drinking water in these three countries are examined. Kenya, a developing nation, faces challenges including limited resources, fragmented monitoring systems, and inadequate enforcement of water quality standards. Urban areas in China have made significant advancements due to stringent regulations and advanced treatment technologies, yet rural areas continue to face economic and infrastructural challenges. Germany, on the other hand, boasts nearly universal access to high-quality drinking water, supported by advanced monitoring devices to ensure quality. Although Kenya has established national drinking water quality standards, implementation and enforcement remain challenging, particularly in rural regions. While rural areas rely on simpler and less effective treatment methods, urban water treatment plants in China employ both conventional and modern technologies to meet strict criteria. In Germany, exceptional water quality is maintained through multi-barrier treatment and robust monitoring systems, achieving approximately 100% compliance.

To improve Kenya's drinking water quality, it is recommended to strengthen regulatory enforcement, invest in technical capacity and human resources, digitise water quality monitoring, enhance treatment infrastructure, adopt integrated and adaptive water management plans, improve data management and reporting, increase government investment, and promote public engagement and water safety. These recommendations, inspired by the experiences of China and Germany, aim to enhance Kenya's drinking water quality. The study emphasises the importance of tailored policy interventions and adopting best practices from industrialised nations to achieve universal access to safe and reliable drinking water. This comparative analysis serves as a valuable resource for policymakers, government institutions, non-governmental organisations, and international agencies working to improve drinking water quality by addressing the unique challenges faced by Kenya, China, and Germany.

Keywords:  Water, Drinking Water Quality, Water treatment technologies, Monitoring and compliance, Regulatory frameworks, China, Kenya, Germany

- Introduction

Safe water is necessary for health and well-being and is a basic human right(Jain, 2012)Access to safe drinking water is a very crucial issue when it comes to health and development at the National, regional and local levels globally. In some parts of the world, there is evidence showing yield in economic benefit as a result of investment in water supply and sanitation. This is attributed to lower health care costs due to a reduction in adverse health effects, which translates to cost saving that allows governments, households and communities to allocate the resources to other priorities that promote economic growth, stifling poverty as a result(Organization, 2022).

Sustainable Development Goal (SDG) 6.1 calls for universal and equitable access to safe and affordable drinking water, but signs of water scarcity and deteriorating water quality can be seen all over the world. According to the World Health Organisation (WHO), in 2022, 2.2 billion people accessed drinking water services that were not safely managed. This included 1.5 billion people with basic services (improved water source within a 30-minute round trip), 292 million people with limited services (improved water source requiring more than 30 minutes to collect water) 296 million people using water from unprotected wells and springs,115 million people collecting untreated surface water from lakes, ponds, rivers, and streams(Organization, 2023). These statistics clearly outline the need to address water infrastructure to supply dependable and safe water sources, water treatment technologies to guarantee water quality, and sustainable water management to alleviate scarcity and guarantee fair usage.

While developed countries such as Germany, Netherlands and Denmark just to mention a few have made significant progress in establishing and implementing stringent water regulations that necessitate adoption of advanced water treatment technology such as ozone and UV radiation, many developing countries continue to experience significant challenges such as inadequate water treatment, high operational costs, lack of delivery infrastructure and poor maintenance which make it difficult for them to implement the strict water regulations contributing to unsafe drinking thus, increasing risks of water-borne diseases (Bukhary et al., 2017; Kothe et al., 2024; Zhu & Balke, 2008).

This paper focuses on a comparative analysis of drinking water quality standards and technologies in three different countries, namely Kenya, China and Germany. These nations represent different levels of development and face diverse challenges relating to drinking water quality management. With an annual per capita availability of less than 1000 m3, Kenya is a developing nation that faces several difficulties, including water resource pollution, a lack of facilities for treating raw and drinking water, a poor water supply network and a lacklustre implementation of water policies and drinking water standards(Baloïtcha et al., 2022; Jones, 2014; Mulwa et al., 2021). On the other hand, China represents a rapidly developing nation that has adopted and implemented stringent drinking water regulations in urban areas, whose modern drinking water treatment plants (DWTP) combine both conventional and advanced water treatment technologies to obtain clean drinking water(Reese, 2019; Ren et al., 2023). However, it still faces a challenge in rural areas due to inadequate monitoring measures and primitive purification facilities for just a few (Ji et al., 2021). Germany stands out from the other two nations due to its sophisticated water treatment system and extensive regulatory framework. Furthermore, 100% of people have access to safe drinking water, and there is reliable and significant data monitoring taking place in the background(Reese, 2019).

The research focuses on the regulatory frameworks; each country establishes and enforces drinking water standards, treatment technologies; the effectiveness of different water purification methods from chlorination and sedimentation in Kenya to Ozone-Biological Activated Carbon(O3-BAC) in China and Germany which uses several advanced water treatment techniques such as Membrane filtration and UV filtration, monitoring and compliance to see how water quality is monitored and regulated and finally Public access and Equity which refers to the extent to which populations have access to safe drinking water. By comparing these aspects, the paper seeks to identify the best practices and lessons that Kenya can adopt from China and Germany to improve its drinking water quality. The findings will inform policymakers, government institutions, non-governmental organisations and international organisations on policy recommendations for the enhancement of regulatory enforcement, upgrading water treatment infrastructure and ensuring equitable access to clean water.

- Regulatory Frameworks for Drinking Water Quality (Kenya, China and Germany)

In the realm of public health and environmental protection, the quality of drinking water is a paramount concern. This research paper aims to analyse and compare the drinking water quality standards and policies in three distinct countries: Kenya, China, and Germany.  According to a review of global drinking water challenges, the availability of safe and clean drinking water remains a significant challenge in developing countries such as Kenya(Gadgil, 1998).

Firstly, it is crucial to understand the global context of drinking water quality. In 2004, a systematic review revealed that water quality often declines substantially between the source and the consumer, underscoring the need for household-level water treatment and safe storage. This finding is particularly relevant in developing countries, where water infrastructure and access to clean water can be limited (Gadgil, 1998).

In Kenya, a study on the Umoja Inner core Estate in Nairobi highlighted the pressing issue of water quality. The researchers found that the quality of water distributed in the area was compromised, with human health and livelihoods at risk due to inadequate, unreliable, and contaminated water supplies (Clasen & Mintz, 2004; Nyakundi et al., 2020). Similarly, a study on the influence of a quasi-delegated management framework on the sustainable distribution of quality drinking water in the same region revealed the challenges faced in managing the water distribution network and ensuring consistent quality(Nyakundi et al., 2020).

To address these challenges, the International Network to Promote Household Water Treatment and Safe Storage has emerged as a vital initiative. This network brings together stakeholders from various organizations to advance household-based water management strategies. The approach acknowledges that centralized water treatment and distribution systems are not always sufficient, especially in remote or underserved areas.

  - Regulatory Framework for Drinking Water Quality in Kenya

The regulatory framework for drinking water quality in Kenya is primarily governed by the Water Act of 2016. This act establishes the Water Resources Authority as the lead agency responsible for managing the country's water resources, including the development of standards and guidelines for drinking water quality. The Water Act of 2016 outlines the regulatory processes for water resource management, water supply, and sanitation services. It also mandates the establishment of the Water Services Regulatory Board, which is responsible for overseeing and regulating the provision of water and sewerage services throughout the country(Nyakundi et al., 2020).

In addition to the Water Act, Kenya has also adopted the Kenyan Standard 459 Part 1: 2007, which specifies the requirements for potable (drinking) water. These standards are aligned with the World Health Organization's Guidelines for Drinking-Water Quality and provide a comprehensive framework for monitoring and regulating the quality of drinking water in the country. Figure 1 illustrates the regulatory framework governing drinking water in Kenya.

Figure 1: Kenya Water Quality Framework

  - Regulatory Framework for Drinking Water Quality in China

In China, the rapid pace of industrialization and urbanization has placed immense strain on water resources. In contrast, Germany boasts a well-established water management system and stringent water quality standards.  While specific details on the Chinese and German water quality standards were not provided in the reference papers, it is generally understood that both countries have regulatory frameworks in place to ensure the safety of drinking water.

Furthermore, the economic aspects of water quality cannot be overlooked.  The importance of integrated water and wastewater management cannot be overstated(Santos et al., 2022). Water utilities worldwide are dedicated to providing safe and reliable drinking water to the public, a goal supported by water quality parameters developed by the World Health Organization (Hu et al., 2019). Moreover, the discharge of inefficiently treated wastewater into surface water sources poses a significant threat to both macro- and microflora and fauna, as well as the provision of good-quality water required for all socio-economic functions (Naidoo & Olaniran, 2014).

In addition to the chemical and biological aspects of water quality, the presence of phosphorus in the environment is a growing concern.  China's regulatory framework for drinking water quality is complex, with multiple agencies and regulations governing different aspects of water management. The primary legislation governing drinking water quality in China is the Law on Prevention and Control of Water Pollution, which was first enacted in 1984 and has been amended several times since then. The Ministry of Ecology and Environment is the main regulatory body responsible for overseeing water pollution control and ensuring the safety of drinking water.  China has also developed a series of national standards for drinking water quality, which are periodically updated to align with international best practices(Gastañaga, 2018). One key challenge in China's regulatory framework is the effective implementation and enforcement of these standards, particularly in rural and remote areas.

  - Regulatory Framework for Drinking Water Quality in Germany

Germany's regulatory framework for drinking water quality is well-established and firmly rooted in the European Union's Water Framework Directive. At the national level, the key legislation governing drinking water quality is the Drinking Water Ordinance, which sets stringent standards for the quality and safety of drinking water. The Federal Ministry of Health is the primary regulatory body responsible for overseeing the implementation of the Drinking Water Ordinance, with the Federal Environment Agency providing scientific support and advice(Federal Environment, 2025).

Germany's regulatory framework is characterized by a high degree of coordination and collaboration between federal, state, and local authorities. This ensures a consistent and harmonized approach to drinking water quality management across the country. The Drinking Water Ordinance is regularly updated to incorporate the latest scientific evidence and technological advancements, ensuring that Germany's drinking water standards remain among the most stringent in the world.

For this year’s report (Federal Environment, 2025), the measurement data from 2017 to 2019, as well as the complete analysis and results, are available in the FEA. A total of 14 specific parameters play a role in quality monitoring. One example is conductivity, which allows conclusions to be drawn about the salt content in the water. The measured values must lie within a prescribed range, and the same applies to the pH value of the water. The physical appearance is another important aspect: while clear and odorless water appears fresh and appealing, we perceive cloudy water as dirty and unappetizing. Therefore, German drinking water should not have any anomalies or inconsistencies in terms of odor, turbidity or coloration. Water quality also depends on the various substances contained in the water. For example, 1 litre of drinking water must not contain more than 20 µg of nickel. The limits for substances such as pesticides and bacteria in water are much stricter, as they have a direct impact on the health and safety of consumers. For example, the intestinal bacteria Escherichia coli and coliform bacteria must not be detectable in 100 ml of drinking water.

The results presented in the Federal Environment Agency's report show that the strict regulations of the German Drinking Water Ordinance were complied with by the sampling points with a success rate of almost 100 per cent (Federal Environment, 2025). Where limit values were exceeded, these were generally only temporary and very localized incidents. One of these rare exceptions related to the parameter mentioned above, namely nickel. This can be the result of installing unsuitable fittings, often without consulting a professional installation company that would have been able to assess the suitability of the fitting based on the corrosive and chemical properties of the local water. Despite routine checks, limit values may occasionally be exceeded, as was the case with the ‘coliform bacteria’ parameter in 2018. However, there was no immediate risk to health, as these are so-called ‘indicator bacteria’ that allow conclusions to be drawn about general changes in water quality and enable appropriate measures to be taken. When it comes to bacterial growth in drinking water, stagnation flushing is an important prevention mechanism, as bacteria multiply most rapidly when water stagnates in pipes.

- Drinking Water Quality Management Approaches

Effective drinking water quality management requires an integrated approach that addresses multiple interconnected aspects of water resource management. Integrated Water Resource Management (IWRM) serves as a holistic strategy that acknowledges the interconnectedness of water, land, and related resources, emphasizing basin-wide management, stakeholder participation, environmental values, and equitable resource access. Complementing IWRM, the catchment-to-consumer approach offers a comprehensive framework that spans the entire water supply chain, from source protection and hazard assessment to optimized treatment processes and continuous quality monitoring, ultimately safeguarding public health. Furthermore, adaptive management strategies reinforce these frameworks by providing an iterative decision-making process that involves continuous monitoring, scenario planning, risk assessment, stakeholder engagement, and regular evaluation and adjustment of management practices, thereby enhancing responsiveness to emerging challenges such as climate change, population growth, and shifting consumer demands. Collectively, these integrated, catchment-to-consumer, and adaptive approaches ensure the sustainable provision of safe, reliable drinking water, support ecosystem resilience, and promote sustainable development by effectively managing water quality at local and global scales. Figure 2 summarizes the different Drinking water quality Management approaches that can be adopted in Kenya.

Figure 2 Summary of effective Drinking Water Quality Management approaches.

The provision of safe and reliable drinking water is a critical challenge faced by many countries around the world. While the specific regulatory and management approaches may vary, there are common themes and best practices that can be learned from the experiences of different nations.

In Kenya, the regulatory framework for drinking water quality is still evolving, with multiple agencies and regulations governing different aspects of water management. The country has made progress in developing national standards for drinking water quality, but implementation and enforcement remain challenges, particularly in rural and remote areas (Hodgson & Manus, 2006).

China, on the other hand, has a well-developed regulatory framework for drinking water quality, with the Ministry of Ecology and Environment serving as the main regulatory body. China has also developed a series of national standards for drinking water quality, which are periodically updated to align with international best practices. The key challenge in China's regulatory framework is the effective implementation and enforcement of these standards, particularly in rural and remote areas(Knüppe, 2011). In contrast, Germany's regulatory framework for drinking water quality is widely regarded as one of the most robust and comprehensive in the world. The Drinking Water Ordinance, which is the primary legislation governing drinking water quality, sets stringent standards for water quality and safety.  The Federal Ministry of Health is the primary regulatory body responsible for overseeing the implementation of the Drinking Water Ordinance, with the Federal Environment Agency providing scientific support and advice. Germany's regulatory framework is characterized by a high degree of coordination and collaboration between federal, state, and local authorities.  This ensures a consistent and harmonized approach to drinking water quality management across the country.  The German regulatory framework is regularly updated to incorporate the latest scientific evidence and technological advancements, ensuring that the country's drinking water standards remain among the most stringent in the world. The regulation of drinking water quality involves a chain of processes that includes the creation of norms, their implementation among actors, and the monitoring of outcomes, with feedback to authorities for further action  (Mukherjee & Jensen, 2020)

# Types of Drinking Water Technologies in Kenya, China and Germany

# Kenya

In Kenya, the most common drinking water treatment technologies include conventional methods such as filtration, sedimentation, and chlorination.  These technologies are relatively simple and cost-effective, making them suitable for use in both urban and rural areas(Gupta & Singh, 2021). However, the effectiveness of these technologies can be limited by factors such as inadequate maintenance and the presence of emerging contaminants (Gupta & Singh, 2021). Table 1 illustrates the advantages and disadvantages of disinfectants used in certain conventional treatments.

In recent years, there has been growing interest in the use of more advanced treatment technologies, such as membrane filtration and UV disinfection, particularly in urban areas. These technologies offer a higher level of treatment and can remove a wider range of contaminants, but they are also more expensive and require more specialized expertise to operate and maintain(Gupta & Singh, 2021). Additionally, point-of-use water treatment technologies, such as household filters and solar disinfection, are becoming increasingly popular in rural areas, where access to centralized water treatment facilities is limited. A low-cost treatment solution such as sand water filters are also used to serve the drinking water need of rural families, offering a better alternative to reverse osmosis technology common for urban households (Gupta & Singh, 2021).

Table 1: Main advantages and disadvantages of conventional water disinfection technologies (Pichel et al., 2019)

| Disinfection Technologies | Advantages | Disadvantages/ limitations |
| --- | --- | --- |
| Chlorination | -Extremely effective for bacteria and virus inactivation -Residual protection against recontamination -Calcium and sodium hypochlorite require simple equipment and instrumentation | - DBPs formation - Ineffective against Cryptosporidium and Giardia cysts - Unsatisfactory taste and odor - Less effective in turbid and organic-rich waters - Dependence on pH - Chorine gas is hazardous and extremely corrosive - Requires trained operators - Sodium and calcium hypochlorite are more expensive than chlorine gas -Dependence on chemical access |
| Chloramination | -Lower DBPs than chlorine -Residual protection against re-contamination -Effective against biofilms formation in the distribution system | - Must be manufactured on-site -Less disinfection capability than other methods - Chloramines are harmful for patients undergoing dialysis, aquariums, and fish farming enterprises |
| Chloramination | -Less taste and odour concerns | - Requires trained operators - Dependence on chemical access |
| Chlorine dioxide | -Less dependence on pH than chlorine -Long-lasting residual activity -Effective against Giardia | - Requires on-site production - More expensive than chlorine - Chlorites and chlorates formation - Low level of inactivation of Cryptosporidium - Unsatisfactory taste and odour - Dependence on chemical access |
| Ozonation | -Effective against bacteria, viruses and protozoa (Giardia and Cryptosporidium) -No dependence on chemical access | - Requires on-site generation - Does not provide residual activity - Associated costs are high compared to other chemical disinfectants - Expensive to operate - Requires high energy input - Requires highly skilled maintenance - Formation of DBPs (bromate) |
| UV lamps | -Effective against virus, spores, and cysts -No taste and odor concerns -No harmful byproducts formation -No dependence on chemical access | - Low energy efficiency system - Lamps should be replaced every 6–12 months - Lamps contain mercury, which is toxic - Discarded lamps generate a waste management problem - Does not provide residual activity - Effectiveness of disinfection decrease because of chemical fouling, biological film formation, and inadequate transmittance of water |
| Pasteurization | -Simple to operate -No dependence on chemical access | - Requires a lot of fuel - Economically and environmental unsustainable |
| Filtration | -Reduces water turbidity and microorganism content -No dependence on chemical access -RO: high water quality output | - Pathogens elimination depends on the filter pore size - Filters must be cleaned regularly - RO: high energy consumption, high costs, chemicals addition for membranes cleaning - RO: not recommended for microorganism elimination |

# China

In China, the drinking water treatment technologies used vary depending on the region and the specific water quality challenges faced. In urban areas, conventional treatment methods such as coagulation, sedimentation, filtration, and disinfection are commonly used.  However, many water treatment plants are also incorporating more advanced technologies, such as activated carbon adsorption, membrane filtration, and advanced oxidation processes, to address emerging contaminants and improve water quality. In rural regions of China, the implementation of advanced treatment technologies is often constrained by economic factors and infrastructural limitations, resulting in a greater reliance on simpler, more affordable methods such as sand filtration, chlorination, and boiling, each of which present unique advantages and disadvantages. One important consideration in the selection of drinking water treatment technologies is the need to remove arsenic, a naturally occurring contaminant that is found in many groundwater sources in China (Elcik et al., 2013; Saxena et al., 2022). The use of technologies like coagulation, adsorption, and membrane filtration has been effective in addressing this challenge, but the high cost and technical complexity of these solutions have limited their widespread adoption, particularly in rural areas.

# Germany

Germany has a highly developed and centralized drinking water treatment infrastructure, with the majority of the population served by large-scale, municipally-operated water treatment plants. These plants employ a range of advanced treatment technologies, including coagulation, sedimentation, filtration, adsorption, and disinfection, to ensure the high quality of the water supplied to consumers. In addition to these conventional treatment methods, many German water treatment plants are also incorporating more advanced technologies, such as membrane filtration, ozonation, and advanced oxidation processes, to address emerging contaminants and improve the overall quality of the water. A key feature of Germany's drinking water treatment approach is the emphasis on multiple barriers, where multiple treatment steps are employed to ensure the removal of a wide range of contaminants. This redundancy helps to ensure the robust and reliable performance of the water treatment system, even in the face of unexpected changes or disruptions. In rural areas of Germany, where centralized water treatment facilities may not be available, the use of point-of-use treatment technologies, such as household filters and UV disinfection, is more common.  These technologies provide a cost-effective and reliable means of ensuring the safety of drinking water in decentralized settings.

Conventional drinking water treatment technologies like coagulation-flocculation are still widely used for water treatment (Shan, 2019). The usage of chemicals like aluminium sulfate and ferric chloride in chemical treatments is not common owing to the cost of maintenance and sludge management (Vasudevan et al., 2008). The removal efficiency of phosphate from wastewater may not exceed 30% in biological treatment plants (Vasudevan et al., 2008). Emerging treatment technologies, including membrane filtration, advanced oxidation processes, and UV irradiation, are promising options for enhancing public and environmental health, and can be used in combination to achieve higher efficacy (Zanacic et al., 2016). Ozone-assisted biological filtration systems are seen as a potential method for potable water treatment in small communities by reducing organic matter, inorganic nutrients and other exposure-related contaminants (Zanacic et al., 2016). Advanced oxidation processes, including the use of ozone, are important in breaking down persistent pollutants(Ghauch, 2017; Zanacic et al., 2016; Zhou & Smith, 2002). Advanced treatment facilities achieve high removal levels of pathogens and other constituents through processes like ozonation, chlorination, microfiltration, reverse osmosis, advanced oxidation, granular activated carbon filtration, and chlorination (Kantor et al., 2019). Ozone can pretreat distiller effluents, enhancing aerobic oxidation and reducing chemical oxygen demand(Kantor et al., 2019; Zanacic et al., 2016; Zhou & Smith, 2002). In experiments with arsenic removal, when initial arsenic concentration was 100 μg/L, the removal efficiency was around 56% for the UP150 membrane and around 80% for the UP005 membrane; high removal efficiencies demonstrate the potential of membrane technologies.

Current treatment processes do have limitations in terms of the removal of emerging organic micropollutants, including pharmaceuticals, personal care products, industrial chemicals, and pesticides. While conventional treatment methods can remove some of these contaminants, advanced oxidation processes and membrane technologies have shown higher removal efficiencies for many of these compounds. Future research and development are needed to further improve the effectiveness and cost-efficiency of drinking water treatment technologies, especially for emerging contaminants.

- Monitoring and Compliance (Kenya, China, and Germany)

Monitoring and compliance are important steps that need to be observed to obtain drinking water that is fit for human consumption. Monitoring water quality is crucial since it helps detect and mitigate the potential dangers that may arise as a result of harmful substances being found within the water sources. These toxic substances may range from pathogens such as E. coli, and heavy metals such as Lead, arsenic and fluoride. In addition, Water monitoring services help businesses, institutions, and water service providers comply with the water quality regulations that have been established by different governments and regulatory bodies. It also promotes Sustainable Water Management (Compliance, 2023)This section evaluates the enforcement of water regulations by governments and regulatory bodies in Germany, China and Kenya.

  - Monitoring and Compliance of Drinking Water Quality Standards in Kenya

In Kenya, water quality monitoring is primarily the responsibility of the Water Services Regulatory Board (WASREB). They are a regulatory state corporation established by the Water Act 2002, which was later repealed by the Water Act 2016 (board). This agency is tasked with ensuring Water Service Providers comply with the Kenyan standard 459, part 1:2007 for drinking water quality.

However, the implementation of the monitoring programs in the country faces significant challenges. This can be highlighted by the fact that only 73% of 54 million Kenyans have access to safe drinking water(Anderskov, 2023). Compliance with drinking water quality standards in Kenya is difficult to achieve due to several challenges, such as Limited resources and infrastructure, fragmented monitoring systems, challenges in data management and transparency, and insufficient water treatment practices.

Limited resources have proven to be a major challenge in ensuring compliance with drinking water standards in Kenya. This is highlighted by the lack of lab equipment, reagents, and skilled personnel. This challenge has led to inconsistent data collection since there is an irregular frequency of testing and reporting of water quality data. This contradicts the frequency provided by the national guidelines, which require Water Service Providers (WSPs) and Ministry of  Health (MoH) officials to conduct testing regularly and report the Water quality data to WASREB on a quarterly and annual basis(Kumpel et al., 2020).  In addition, some of the Water system operators, utility managers and County government officials lack technical expertise and knowledge regarding drinking water quality and national standards for drinking water, therefore making it difficult for them to provide proper water treatment practices and source protection(Project., 2024).

Poor enforcement of regulations has also emerged as a significant challenge. Some Water Service Providers report not receiving penalties for providing inconsistent data or for Water quality data that fails to meet the standards set by the Kenya Bureau of Standards. They suggest that if the WASREB were to conduct regular audits, it would encourage greater attention to compliance. In addition to regulatory enforcement, there are notable challenges in data management and transparency. Currently, water quality data is being collected and recorded through physical papers or photocopied templates, which take substantial time to be digitized by the managerial and M&E staff. The WSPs propose that investing in more computers and establishing a proper database, for example, Excel, could significantly improve record-keeping and data sharing. Such improvement will enhance transparency and facilitate efficient reporting to both the Water Service providers and the communities they serve(Kumpel et al., 2020).

Figure 3: Pathway to Improved Drinking Water Quality Through Enhanced Monitoring in Kenya

  - Monitoring and Compliance of Drinking Water Quality Standards in China.

Monitoring and compliance of drinking water in China is a multifaceted issue that involves technological, regulatory and participatory approaches. The country faces significant challenges due to it vast geography, diverse water sources, and rapid industrialization, which have led to water pollution and quality concerns. Various strategies are being employed to address these challenges, including real-time monitoring systems, citizen science initiatives and regulatory frameworks. These efforts aim to ensure safe drinking water and align with the Sustainable Development Goals.

The National Health Commission, in collaboration with the Ministry of Environment and Ecology, are responsible for monitoring and compliance of drinking water quality in China(Shi, 2020). They are guided by the strict Standards for Drinking Water Quality of China (SDWQ 2022). The document has a variety of indices covering biological, chemical, and physical risk factors in drinking water. In addition to providing indices that cover water quality, it contains management requirements for the comprehensive water supply process, establishing a technical foundation and criteria to ensure the safety of drinking water, as well as a scientific basis for formulating drinking water safety regulations and recommending specific public health protection actions(Han et al., 2023). At the end of 2018, monitoring of drinking water quality in both Urban and Rural areas in China had reached 90% of towns and townships, and all prefectures and counties. This highlights the significant improvement in the quality of drinking water in both urban and rural areas(Shi, 2020; Wang Fei, 2018).

In order to be compliant with the SDWQ 2022, China has implemented several technological approaches by incorporating more advanced water treatment technologies, such as activated carbon adsorption, membrane filtration, and advanced oxidation processes, to address emerging contaminants and improve water quality. Additionally, real time monitoring systems such as Internet of Things which ensures timely data response and accurate pollution alarms, GIS based systems that visualize and analyze spatial and temporal variations in water quality as well as providing early warning systems, and Satellite Remote Sensing which is used in monitoring various environmental parameter including water quality over large areas are being used in monitoring the drinking water quality. These projects also involve public participation in water quality monitoring, which helps in gathering extensive data and promoting community engagement. This approach has been particularly useful in monitoring freshwater and drinking water quality  (Guan et al., 2022; Shi et al., 2019; Wu et al., 2023; Zhao et al., 2017).

Despite all these developments in Monitoring and compliance in China there is still a disparity in water treatment between the Urban and Rural areas. This can be attributed to urban areas benefitting from advanced treatment technologies like ozonation and biologically activated carbon processes, which are often unaffordable for rural areas (Bei et al., 2019). Rural areas rely on simpler methods such as filtration, which may not be as effective (Zhang et al., 2020). Table 2, shows the Statistics of the Water Treatment Plants with an Advanced Treatment Process around China.

The recent unification of drinking water standards for urban and rural areas poses challenges, especially for rural regions with high organic pollutant levels. In addition to the problems brought about by the unification of drinking water standards, there are designed loopholes in policies managing various sources of pollution, including industrial, agricultural, and urban runoff, which complicate compliance with water quality standards(Guojun & Zhen, 2014; Ren et al., 2023). To solve these issues the relevant government should consider investing in advanced water treatment technologies and infrastructure especially in rural areas and also revise pollutant discharge control policies to address loopholes and ensure stricter enforcement.

Table 2. Statistics of the Water Treatment Plants with an Advanced Treatment Process around China (Bei et al., 2019)

| Regions | Provincial Areas | Number | Capacity (km3/day) |
| --- | --- | --- | --- |
| North China | Beijing Tianjin Hebei Shanxi Inner Mongolia | 6 5 8 1 1 | 3400 1200 1250 400 300 |
| North East China | Heilongjiang Jilin | 5 2 | 860 500 |
| East China | Shanghai Jiangsu Zhejiang Anhui Shandong | 12 79 24 1 13 | 4600 15000 5000 80 1600 |
| Central China | Henan Hunan | 3 5 | 1000 1400 |
| South China | Guangdong | 7 | 3000 |
| South West China | Yunnan | 1 | 300 |
| Northwest | / | 0 | 0 |
| Total |  | 173 | 39 890 |

  - Monitoring and Compliance of Drinking Water Quality Standards in Germany

German authorities employ a comprehensive and multi-faceted approach to ensure compliance with drinking water regulations and are guided by the Drinking Water Ordinance (Trinkwasserverordnung – TrinkwV), which serves as the core legal framework that governs drinking water quality, outlining the responsibilities of Water Suppliers and the regulatory authorities. Additionally, it also sets stringent standards for Microbiological, chemical and physical parameters of drinking water and mandates regular monitoring and sampling frequencies to ensure water quality(Grummt, 2007).

In Germany, the Gesundheitsamt (health office) serves as the main authority responsible for enforcing the Drinking Water Ordinance. Their role includes developing sampling plans, supervising the collection and testing of water samples, and ensuring that regulatory requirements are met at points of compliance which include: at the tap or draw -off point of a piped water supply, at the safety device of connected appliances attached to the water system that are not considered part of the installation, at the draw-off point of the water delivery vehicle, and sealed containers. Meanwhile, water suppliers are obligated to carry out routine sampling and analysis of drinking water quality. To fulfil this, they work in partnership with certified laboratories that meet established competence and quality assurance standards (Health, 2016). Sampling and analyses of the drinking water varies from parameters. For example radioactive substances and legionella  are required to be monitored annually while the monitoring of the other parameters can be extended to intervals of up to three years depending on the compliance history and operational conditions (Health, 2016).

Real-time monitoring is also being utilized to ensure that various water suppliers comply with regulations. These monitoring methods include sensor technologies, flow cytometry, and integrated monitoring systems. The integrated monitoring systems function across three tiers: tier 1 offers general monitoring, tier 2 improves detection and verification, and tier 3 confirms and identifies contaminants. Additionally, web-scraping is employed to aggregate decentralized data from different water supply regions, thereby offering a more holistic view of drinking water quality throughout Germany.

All these factors combined ensure that the compliance level of drinking water quality is at 99% all over the country.

- Policy Recommendations

  - Strengthen Regulatory Enforcement and Institutional Coordination

To fortify the management of drinking water quality in Kenya, it is imperative to augment regulatory enforcement and promote institutional cooperation.  Kenya should adopt Germany’s systematic methodology as outlined in the Trinkwasserverordnung by empowering regulatory bodies like the Water Services Regulatory Board (WASREB) to do regular audits, impose sanctions for non-compliance, and facilitate collaboration between national and county governments.  Explicit job delineation and accountability—characteristics of Germany’s framework—can markedly enhance compliance and supervision.

  - Invest in Technical Capacity and Human Resources

Investing in human capital and technical capacity is another priority. Both Germany and China highlight the value of skilled personnel in managing water systems. Kenya should establish comprehensive training programs for utility operators, water quality analysts, and local officials to ensure consistent and competent service delivery. In rural areas, where technical expertise is often limited, targeted capacity-building efforts modeled after China’s rural training programs are necessary to close this gap.

  - Modernizing and digitizing water quality monitoring

Modernizing and digitizing water quality monitoring is critical. Germany’s multi-tiered monitoring structure—ranging from general assessments to contaminant identification—and China's real-time technologies such as IoT sensors and GIS-based systems offer practical models. Kenya should adopt similar technologies to improve the accuracy and responsiveness of its monitoring efforts. Additionally, digitizing data records, transitioning from paper to digital platforms, and developing a centralized online database would enhance transparency, streamline reporting, and improve decision-making.

  - Enhance Treatment Infrastructure and Technology

Upgrading water treatment infrastructure is also vital. In urban areas, Kenya can adopt advanced technologies used in China and Germany, such as membrane filtration, UV disinfection, and ozonation, which offer higher contaminant removal efficiencies. For rural and underserved areas, Kenya should prioritize cost-effective, decentralized treatment options—such as sand filtration and solar disinfection—similar to China’s rural water strategies and Germany’s use of household-level treatment systems. Embracing Germany’s multi-barrier treatment approach can further enhance reliability and water safety.

  - Adopt Integrated and Adaptive Water Management Approaches

Integrated and adaptive water management strategies should be adopted to address Kenya’s complex water challenges. Germany’s catchment-to-consumer model and adaptive planning techniques offer a strong foundation for holistic water governance. These approaches involve continuous monitoring, stakeholder input, and the ability to adjust policies in response to climate change, demographic shifts, and emerging contaminants.

### Improve Data Management and Reporting Practices

Efficient data management is also a pressing need. Germany’s use of centralized digital tools and China’s web-scraping techniques for aggregating decentralized water quality data demonstrate how Kenya can improve information flow. Establishing a national water quality data platform would reduce reporting delays, improve inter-agency coordination, and support evidence-based policy making.

  - Increase Government Investment and Resource Mobilization

In terms of financing, Kenya must increase government budget allocations for water infrastructure and quality assurance. China’s large-scale investments in rural and urban water systems underscore the importance of sustained financial commitment. To bridge resource gaps, Kenya should also explore public-private partnerships and seek support from international donors and development organizations.

  - Promote Public Engagement and Water Safety Awareness

Finally, public awareness and engagement are critical for sustaining progress. Inspired by China’s citizen science initiatives and Germany’s public reporting requirements, Kenya can build community trust and participation through education campaigns on safe water practices and by encouraging local involvement in water quality monitoring. Publishing annual water quality reports would also foster transparency and empower consumers.

By adopting these combined recommendations, tailored to Kenya’s context yet informed by international best practices, the country can move toward achieving universal access to safe, reliable drinking water.

- Challenges and Barriers

While the proposed policy recommendations offer a roadmap for improving drinking water quality in Kenya, their implementation may be hindered by several significant challenges and barriers. Limited financial resources remain a critical obstacle. Upgrading treatment infrastructure, deploying advanced monitoring technologies, and digitizing data systems all require substantial capital investments that may exceed the budgetary capacity of county governments and water utilities. This financial strain is compounded by competing priorities in the national development agenda, such as health, education, and food security, which may divert attention and funding away from water sector reforms.

A second major challenge is the shortage of skilled personnel and technical expertise. Implementing complex technologies like membrane filtration or real-time water quality sensors requires highly trained operators, engineers, and technicians. Unfortunately, many counties in Kenya, especially in rural and marginalized regions, lack sufficient human capacity to operate and maintain such systems. Training programs will take time to scale up and may suffer from poor retention due to migration of trained staff to the private sector or urban areas.

Institutional fragmentation and weak coordination among key actors, such as the Ministry of Water, county governments, WASREB, and public health departments, pose further barriers. The current governance structure lacks streamlined communication and often results in overlapping mandates or implementation gaps. In contrast to Germany’s harmonized regulatory model, Kenya may struggle to align all stakeholders under a unified framework unless structural reforms are introduced.

Technological adaptation also presents difficulties. While real-time monitoring systems and integrated data platforms work well in Germany and China due to their robust digital infrastructure, Kenya still faces low levels of digitization, limited internet connectivity, and inconsistent electricity supply in many areas. These infrastructural gaps may render such technologies unreliable or infeasible in the short term.

Moreover, resistance to change—both from within institutions and from local communities—can delay progress. Some utility operators and local officials may be reluctant to adopt new practices due to institutional inertia, fear of increased accountability, or lack of understanding of the long-term benefits. Communities may also be hesitant to accept new water treatment methods or technologies without sufficient education and trust-building.

Policy enforcement is another weak link. Historically, Kenya has faced difficulties in enforcing environmental and water regulations. Even where standards exist, there is often a lack of monitoring, follow-up, and penalties for non-compliance, undermining the credibility of the regulatory framework. Without political will and legal reform, these enforcement challenges are likely to persist.

Finally, social inequality and geographical disparity may limit the equitable implementation of these recommendations. Rural and informal settlements, where water quality is typically poorest, often receive less investment and oversight. Ensuring that reforms do not disproportionately benefit urban or affluent areas will require a deliberate effort to prioritize vulnerable populations, something that both Germany and China have also struggled with, albeit to a lesser extent.

8. Conclusions

Access to safe and clean drinking water is a fundamental human right and essential for public health and sustainable development.  This comparative investigation of Kenya, China, and Germany has uncovered significant insights into how countries at varying levels of development tackle the difficulties of drinking water quality management.  Kenya, a developing country, encounters ongoing challenges such as constrained financial resources, insufficient infrastructure, disjointed regulation, and inconsistent enforcement.  Conversely, China has achieved significant advancements, especially in metropolitan regions, through the implementation of rigorous standards, the utilization of cutting-edge technologies, and the enhancement of public engagement.  Germany demonstrates optimal practices through its comprehensive regulatory framework, multi-layered monitoring systems, extensive utilization of innovative treatment technology, and nearly universal access to high-quality drinking water.

The study emphasizes that although Kenya has made notable progress in enhancing water quality standards and accessibility, substantial deficiencies persist in enforcement, infrastructure, technical capacity, and data administration.  Insights from China and Germany provide effective frameworks for change, encompassing the incorporation of real-time monitoring technologies, adaptive management tactics, and the establishment of robust regulatory control.

Policy recommendations derived from these worldwide practices underscore the necessity for Kenya to enhance enforcement mechanisms, invest in contemporary treatment technology, develop human resource capacity, digitize data systems, and include communities.  The path to implementation is intricate.  Kenya must surmount obstacles like insufficient money, institutional fragmentation, reluctance to change, and technology limitations, particularly in rural and neglected regions.

Enhancing drinking water quality in Kenya necessitates enduring political commitment, strategic investment, and a cooperative approach that includes all stakeholders, such as government entities, local authorities, civil society, and the corporate sector.  By utilizing global best practices and adapting them to its local environment, Kenya can gradually achieve its objective of providing safe, reliable, and equitable drinking water for all.

DISCLAIMER

The author(s) hereby declare that generative AI technologies, such as Large Language Models, have been used in the writing or editing of this manuscript. This explanation will include the name, version, model, and source of the generative AI technology, as well as all input prompts provided to it.

Details of AI usage are given below:

- Text-to-Image Generators

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

REFERENCES

Anderskov, C. (2023). Kenya/water The strategic sector cooperation (SSC) between Denmark and Kenya on water 2023-2033. The Danish Environmental Protection Agency. https://eng.mst.dk/about-the-danish-epa/global-cooperation/kenyacircular-economy/kenya-water

Baloïtcha, G. M. P., Mayabi, A. O., & Home, P. G. (2022). Evaluation of water quality and potential scaling of corrosion in the water supply using water quality and stability indices: A case study of Juja water distribution network, Kenya. Heliyon, 8(3).

Bei, E., Wu, X., Qiu, Y., Chen, C., & Zhang, X. (2019). A tale of two water supplies in China: finding practical solutions to urban and rural water supply problems. Accounts of chemical research, 52(4), 867-875.

board, W. S. R. Water Services Regulatory Board Background information. WASREB. https://wasreb.go.ke/who-we-are/#:~:text=Accordingly%2C%20Wasreb%20sets%20standards%20and,by%20allowing%20financing%20of%20operations%2C

Bukhary, S., Weidhaas, J., Ansari, K., Mahar, R. B., Pomeroy, C., VanDerslice, J. A., Burian, S., & Ahmad, S. (2017). Using Distributed Solar for Treatment of Drinking Water in Developing Countries. World Environmental and Water Resources Congress 2017,

Clasen, T. F., & Mintz, E. D. (2004). International network to promote household water treatment and safe storage. Emerging infectious diseases, 10(6), 1179.

Compliance, C. W. (2023). The Importance Of Water Monitoring Services For Clean And Safe Drinking Water. Canadian Water Compliance. https://www.canadianwatercompliance.ca/blogs/toronto-legionella-disinfecting-bacteria-water-testing-blog/1132817-the-importance-of-water-monitoring-services-for-clean-and-safe-drinking-water#:~:text=Monitoring%20water%20quality%20is%20crucial,ingested%20or%20inhaled%20through%20aerosols.

Elcik, H., Cakmakci, M., Sahinkaya, E., & Ozkaya, B. (2013). Arsenic removal from drinking water using low pressure membranes. Industrial & Engineering Chemistry Research, 52(29), 9958-9964.

Federal Environment, A. (2025). Report of the Federal Ministry of Health and the Federal Environment Agency to consumers on the quality of water intended for human consumption (drinking water) in Germany (2020_2022). https://www.umweltbundesamt.de/publikationen/bericht-des-bundesministeriums-fuer-gesundheit-des-5

Gadgil, A. (1998). Drinking water in developing countries. Annual review of energy and the environment, 23(1), 253-286.

Gastañaga, M. D. C. (2018). Water, sanitation and health. In (Vol. 35, pp. 181-182).

Ghauch, A. (2017). The importance of advanced oxidation processes in degrading persistent pollutants. Journal of Advanced Oxidation Technologies, 20(1), 20160197.

Grummt, H.-J. (2007). The drinking water quality in Germany in the period from 2002 to 2004. An overview: Eine Übersicht für die Jahre 2002–2004. Bundesgesundheitsblatt-Gesundheitsforschung-Gesundheitsschutz, 50, 276-283.

Guan, G., Wang, Y., Yang, L., Yue, J., Li, Q., Lin, J., & Liu, Q. (2022). Water-quality assessment and pollution-risk early-warning system based on web crawler technology and LSTM. International Journal of Environmental Research and Public Health, 19(18), 11818.

Guojun, S., & Zhen, Z. (2014). New National Standards for Drinking Water and the Reform of Policies Controlling Pollutant Discharge. Chinese Research Perspectives on the Environment, 3, 83-93.

Gupta, K., & Singh, S. (2021). SAND WATER FILTERS: A LOW-COST TREATMENT SOLUTION FOR WATER POLLUTION PROBLEMS IN RURAL REGIONS. International Journal of Technical Research & Science, Special(June), 61-64. https://doi.org/10.30780/specialissue-icaaset021/012

Han, J., Zhang, L., Ye, B., Gao, S., Yao, X., & Shi, X. (2023). The standards for drinking water quality of China (2022 edition) will take effect. China CDC Weekly, 5(13), 297.

Health, F. M. o. (2016). ORDINANCE ON THE QUALITY OF WATER INTENDED FOR HUMAN CONSUMPTION

(Trinkwasserverordnung – TrinkwV 2001)*,†) 2016chrome-extension://efaidnbmnnnibpcajpcglclefindmkaj/https://www.bundesgesundheitsministerium.de/fileadmin/Dateien/3_Downloads/E/Englische_Dateien/Drinking_Water_Ordinance.pdf

Hodgson, K., & Manus, L. (2006). A drinking water quality framework for South Africa. Water SA, 32(5).

Hu, G., Mian, H. R., Hager, J., & Sadiq, R. (2019). Water quality failure pathways. Encyclopedia of Water: Science, Technology, and Society, 1-12.

Jain, R. (2012). Providing safe drinking water: a challenge for humanity. In (Vol. 14, pp. 1-4): Springer.

Ji, Q., Dong, S., Cao, X., Liu, H., & Qu, J. (2021). Rural Drinking Water Supply in Poverty Alleviation Areas in China. Strategic Study of Chinese Academy of Engineering, 23(5), 163-168.

Jones, J. A. A. (2014). Water sustainability: a global perspective. Routledge.

Kantor, R. S., Miller, S. E., & Nelson, K. L. (2019). The water microbiome through a pilot scale advanced treatment facility for direct potable reuse. Frontiers in Microbiology, 10, 993.

Knüppe, K. (2011). The challenges facing sustainable and adaptive groundwater management in South Africa. Water SA, 37(1).

Kothe, A., Wachasunder, N., Rodge, A., Labhasetwar, P., & Maldhure, A. (2024). Trihalomethanes in developed and developing countries. Environmental Monitoring and Assessment, 196(1), 17.

Kumpel, E., MacLeod, C., Stuart, K., Cock-Esteb, A., Khush, R., & Peletz, R. (2020). From data to decisions: understanding information flows within regulatory water quality monitoring programs. NPJ Clean Water, 3(1), 38.

Mukherjee, M., & Jensen, O. (2020). Making water reuse safe: A comparative analysis of the development of regulation and technology uptake in the US and Australia. Safety Science, 121, 5-14.

Mulwa, F., Li, Z., & Fangninou, F. F. (2021). Water scarcity in Kenya: Current status, challenges and future solutions. Open Access Library Journal, 8(1), 1-15.

Naidoo, S., & Olaniran, A. O. (2014). Treated wastewater effluent as a source of microbial pollution of surface water resources. International Journal of Environmental Research and Public Health, 11(1), 249-270.

Nyakundi, V., Munala, G., Makworo, M., Shikuku, J., Ali, M., & Song’oro, E. (2020). Assessment of drinking water quality in Umoja Innercore Estate, Nairobi. Journal of Water Resource and Protection, 12(01), 36.

Organization, W. H. (2022). Guidelines for drinking-water quality: incorporating the first and second addenda. World Health Organization.

Organization, W. H. (2023). Drinking Water. World Health Organization. Retrieved 17th september 2023 from https://www.who.int/news-room/fact-sheets/detail/drinking-water

Pichel, N., Vivar, M., & Fuentes, M. (2019). The problem of drinking water access: A review of disinfection technologies with an emphasis on solar treatment methods. Chemosphere, 218, 1014-1030.

Project., R.-W. R. E. a. L. f. W. (2024). EVALUATING WATER QUALITY

ASSURANCE FUNDS IN KENYA:

BASELINE ASSESSMENT. chrome-extension://efaidnbmnnnibpcajpcglclefindmkaj/https://aquaya.org/wp-content/uploads/Evaluating-Water-Quality-Assurance-Funds-in-Kenya.pdf

Reese, M. (2019). Urban Water Governance in Europe and in China: Key Challenges, Benchmarks and Approaches. Urban Water Management for Future Cities: Technical and Institutional Aspects from Chinese and German Perspective, 249-282.

Ren, X., Wu, Q., Shu, J., Chen, C., Tiraferri, A., & Liu, B. (2023). Efficient removal of organic matters and typical odor substances in rural drinking water using Ozone-BAC-UF combined system to meet new water quality standards in China. Separation and Purification Technology, 327, 124899.

Santos, A. S. P., Pachawo, V., Melo, M. C., & Vieira, J. M. P. (2022). Progress on legal and practical aspects on water reuse with emphasis on drinking water–an overview. Water Supply, 22(3), 3000-3014.

Saxena, R., Ansari, S., Fatima, A., Srivastava, N., Singh, N., Maurya, V. K., & Rai, P. K. (2022). Microbes in drinking water: Control and prevention. In Current Directions in Water Scarcity Research (Vol. 6, pp. 203-222). Elsevier.

Shan, Y. (2019). Evaluation of treatment efficiency of coal mine drainage and impact on surrounding waters for selected coal mines. IOP Conference Series: Earth and Environmental Science,

Shi, W., Wang, Y., Tang, H., Liu, H., & Tang, D. (2019). Research on Water Quality Monitoring Technology of Industrial Park Based on Internet of Things and Cloud Computing. Journal of Physics: Conference Series,

Shi, X. (2020). The safety of drinking water in China: current status and future prospects. China CDC Weekly, 2(13), 210.

Vasudevan, S., Sozhan, G., Ravichandran, S., Jayaraj, J., Lakshmi, J., & Sheela, M. (2008). Studies on the removal of phosphate from drinking water by electrocoagulation process. Industrial & Engineering Chemistry Research, 47(6), 2018-2023.

Wang Fei, Z. W. (2018). A brief analysis of how to ensure rural drinking water safety and measures. Sichuan Cement, 5. https://doi.org/doi: 10.3969/j.issn.1007-6344.2018.05.294.

Wu, Y., Washbourne, C., & Haklay, M. (2023). Inspiring citizen science innovation for sustainable development goal 6 in water quality monitoring in China. Frontiers in Environmental Science, 11, 1234966.

Zanacic, E., Stavrinides, J., & McMartin, D. W. (2016). Field-analysis of potable water quality and ozone efficiency in ozone-assisted biological filtration systems for surface water treatment. Water Research, 104, 397-407.

Zhang, Z., Zhang, W., Hu, X., Li, K., Luo, P., Li, X., Xu, W., Li, S., & Duan, C. (2020). Evaluating the efficacy of point-of-use water treatment systems using the water quality index in rural southwest China. Water, 12(3), 867.

Zhao, S., Wang, Q., Li, Y., Liu, S., Wang, Z., Zhu, L., & Wang, Z. (2017). An overview of satellite remote sensing technology used in China’s environmental protection. Earth Science Informatics, 10, 137-148.

Zhou, H., & Smith, D. W. (2002). Advanced technologies in water and wastewater treatment. Journal of Environmental Engineering and Science, 1(4), 247-264.

Zhu, Y., & Balke, K.-D. (2008). Groundwater protection: What can we learn from Germany? Journal of Zhejiang University SCIENCE B, 9, 227-231.