Review Article

UBIQUITOUS PRESENCE OF MICROPLASTICS (MPs) IN SURFACE WATERS: A REVIEW OF OCCURRENCE, ABUNDANCES, SPATIAL DISTRIBUTION AND FUTURE EFFECTS IN THE SURFACE WATERS (KENYA)

ABSTRACT

Plastics pollution has slowly become a global concern over the years due to the worldwide technological trends in production and consumption of plastic products and materials. Surface water of natural water bodies, man-made waterways and other inter-connected reservoirs provides wide and extensive network of routes for the uncontrolled disposal and dispersal of macro and microplastics (MPs). There is a growing concern on the negative impacts from plastic-associated waste and litter in both marine and freshwater systems.  Microplastics are defined as plastic (primary or secondary) materials of size less than 5000µm. Kenya issued a nationwide ban on single use plastics in 2018 and the EU issued a ban on the same, in 2019. As a result, there is a growing research in understanding the scale of the microplastics in the environment, with relatively more data documented for marine than freshwater ecosystems, yet both act as active conduits of different types of MP’s. Several government and community initiatives are geared towards removal of litter along waterways and beaches and ensure control of plastic pollution by involvement of all stakeholders. However, less information is documented from specific ecosystems studies and environmental samples, on the types of MPs, their environmental concentrations, and the potential effects and relationships with environmental factors. Recent documented studies have focused on the coastal marine waters, and only a few studies on the diverse small and large inland aquatic ecosystems. Therefore, this review provides documented data on MPs occurrences, abundances, spatial distribution in Kenyan aquatic ecosystems, with an overall target of creating more informed stakeholders on the plastic litter menace in aquatic environments and the potential ecological effects of MPs for developing better future risk assessments. Results from this study will be useful in understanding future actions and solutions towards effective control and eradication of MP’s in  water resources.

Keywords:  Kenya, Microplastics, Surface water, Sediments, Abundance, Ecological risks; Toxicity; Polymers

Introduction

Plastics pollution has slowly become a global concern over the years due to the worldwide technological trends in production and consumption of plastic products and materials. Surface water of natural water bodies, man-made waterways and other inter-connected reservoirs provides wide and extensive network of routes for the uncontrolled disposal and dispersal of macro and microplastics (MPs). There is a growing concern on the negative impacts from plastic-associated waste and litter in both marine and freshwater systems. Micro-scale plastic particles were discovered in the marine environment in the early 1970s (Carpenter and Smith 1972), when initial exploration of marine microplastics commenced. Some of the reported hotspot concentration in water (102,000 particles/m3, Noren and Naustvoll 2010) and sediments (621,000 particles/Kg, Liebezeit and Dubaish, 2012) are global evidence of MPs pollution. In 2018, UN Environment programme designated the theme of World Environment day as “Beat Plastic Pollution”, affirming the global concerns. Microplastics are defined as plastic (primary or secondary) materials of size less than 5000µm. Kenya issued a nationwide ban on single use plastics in 2018 and the EU issued a ban on the same, in 2019. As a result, there is growing research in understanding the scale of the microplastics pollution in the environment, with a relatively more data documented for marine than freshwater ecosystems, yet both act as active conduits and reservoirs of different types of MP’s.

To date, the larger sized microplastic are the most studied and reported along the Kenyan coast (Okuku et al., 2022a,b; 2021; 2020a,b,c) and inland lakes (Okuku et al., 2024; Ogello et al., 2024). Several government and community initiatives are geared towards the removal of litter along waterways and beaches and ensure control of plastic pollution by involvement of all stakeholders. However, less information is documented from specific ecosystems studies and environmental samples, on the types of MPs, their environmental concentrations, and the potential effects and relationships with other environmental factors. Recent documented studies have focused on the coastal marine waters, and only a few studies on the diverse small and large inland aquatic ecosystems, especially lakes and rivers which provide a large surface area for MPs input, storage and remobilization after fragmentation into smaller particles.

Therefore, this review provides documented data on MPs occurrences, abundances, spatial distribution in Kenyan aquatic ecosystems, with an overall target of understanding the gaps in data; creating more informed stakeholders on the plastic litter menace in aquatic environments and the potential ecological effects of MPs; for developing better future risk assessments. Results from this study will be useful in understanding future actions and solutions towards effective control and eradication of MP’s in inland water resources for a sustainable fisheries; improved quality of the soil and aquatic ecosystems.

Materials and methods

The study analysed the accessible literature using online databases (Web of science, Scopus and Pubmed) on Plastics/macroplastics/microplastics/microbeads//nanoplastic/microplastics risk assessment/marine litter/marine debris, marine waters /freshwater/plastic polymers, and Kenya as key search words. All publications before March 2023 in studies conducted in the Kenya marine and inland waterbodies (surface water, sediments, soils, biota, risk assessment of microplastics) were selected for review.

Coastal marine areas in Kenya

The Indian Ocean coastline in Kenyan (Figure 1) is part of the international waters and supports some of the extensive tropical mangrove ecosystems. The coast forms the baseline for a sea area or Exclusive Economic Zone (EEZ) measuring 230,000 Km2, including a narrow continental shelf about 3-5 Km in width with a total area of about 19,100 Km2, of which some 11,000 Km2 is trawlable (Ruwa, 2006). The coastal area with 2.5 million people has a higher population density of 77 people per Km2, and is mainly concentrated in urbanised areas   such as Mombasa, Tiwi, Kwale, Msambweni, Shimoni (southcoast) and Kilifi, Malindi and Lamu in the north coast. Fishing grounds are located in the south (Majoreni and Vanga coasts) and around Lamu archipelago, Ungwana bay (Tana river delta), north Kenya bank, and Malindi bank, characterized by the convergence of the South-flowing Somali current and north flowing Est African current during the NE-Monsoon season of November- March, resulting in upwelling and nutrient enrichment (Ruwa, 2006).

Studies on the microplastics were conducted at three creeks in Mombasa county (Tudor and Port-Reitz), and Kilifi county (Mida creek). A detailed description of the creeks and climatic conditions are provided by Kitheka et al. (1999) and Obiero and Onyando (2013). Microplastics were determined in the commonly commercially exploited fish species. The dermersal fish species included Gerres oyena (Forsskal, 1775) (Gerreidae family), the seagrass parrot fish/reef fish Leptoscarus vaigiensis (Quay & Gaimard, 1824), tiger perch Terapon jarbua (Forsskal, 1775) (Terapontidae family) , and sea bream Acanthopagrus  berda (Forsskal, 1775) (Sparidae family). Acanthopagrus berda inhabits brackish waters and is among the commercially important fish from the Indian ocean (Shilta et al., 2018) greatly valued by consumers and is also considered of great aquaculture potential. Brackish water, estuaries, sandy beaches, reefs are common habitats for Gerres oyena. while Leptoscarus vaigiensis inhabits hard substrates and reefs (FAO, 2010). The pelagic fish, Rastreliger kanagurta (Cuvier, 1816) (Scombridae family) is found in bays, harbours and deep lagoons in turbid plankton rich waters (FAO, 2010).

Figure 1. A map of East Africa showing the major lakes, Indian ocean and the coastal marine waters in Kenya

Lake Naivasha

Lake Naivasha (Figure 2) receives inflows from rainfall and two major rivers (R. Malewa and Gilgil), which contribute over 90% of the water inputs. The lake is among the rift valley lakes which have already witnessed the cyclic water level fluctuations, similar to water level rises in Lakes Turkana, Baringo and Bogoria. Water level rise that was witnessed since September 2010 and that significantly affected the areas in 2013 and again 2019, resulted in expansion of the lakes surface area, with submergence of nearly all the riparian land and displacement of thousands of people, with huge socio-economic impacts (GOK and UNDP, 2021).

Lake Victoria

Lake Victoria (Figure 3) is a large but relatively shallow freshwater lake (3,440 Km shoreline, 40m mean depth; 80 maximum depth, 68,800 Km2 water surface area) with a watershed (195,000 Km2) extending into Western Kenya highlands, northern Tanzania, Southern Uganda, Rwanda and Burundi. The Kenyan watershed (42,460 Km2) is drained by many extensive river drainage systems, with a total discharge of 778.3 m3 (42.4% of the whole basin), overally amounting to about 20% of the total lake water input (precipitation contributes over 80%). Bathymetry updates show most of the inner shore areas and islands around the main lake lie at about 50m (Hamilton et al., 2022) water depth. The main seasonal rains (short and long rainy seasons) coupled with storm waters cause occasional flooding of river channels in the low-lying lakeshore areas, transporting diverse suspended solid wastes loads into the lake. These are active conduits of plastic wastes into downstream surface waters.

A decade ago the aquaculture production in Kenya grew from 1012 metric tonnes in 2003 to 21,487 metric tonnes in 2012 (FAO 2010). To enhance fish production due to the declining wild capture fisheries and address increasing demand of fish protein, the fisheries management has again introduced Nile tilapia cage farming since 2005. Therefore, the surface area of the lake currently supports cage aquaculture activities and cage structures at different sizes (volume of 4m3, 8m3, 10m3 and 131.3m3) and stocking densities (2250, 2500 and 3000 fingerlings per cage) (Aura et al., 2018), since the onset of tilapia cage culture in Lake Victoria (Kenya). According to unmanned aerial systems, satellite and GIS technologies study of 2019, the Kenyan portion supported over 4357 floating fish cages which covered 62,132 m2 (Hamilton et al., 2019, Aura et al. 2018). Also the COVID-19 health crisis of 2020-2021 impacted all sectors and according to respondents contacted, the immediate diverse control measures were found to impact on Homabay and Kisumu fishers and fish traders’ household (88 households) livelihood incomes (Okronipa et al. 2023; Fiorella et al. 2021, Aura et al., 2020), albeit other factors were important. This was also thought to cause increased vulnerability of surface waters to solid waste contamination from disposal of synthetic masks, a contributor to MPs.

Figure 3. Map of the Lake Victoria (Kenya) basin showing location of major towns and wastewater treatment plants.

Sample collection

- Sampling design

- Surface water collection

- Water column

- Trawl nets

- Sediment collection stainless steel sampler (e.g Ekman grab, ponar grab)

Extraction of MPs

- MPs Separation by density floatation

- Sample digestion using different treatments (mainly H2O2 & enzymes)

- Filtration stage

Identification and quantification of MPs

- Visual observation under dissecting microscope

- Classification of polymers (shape, colour)

- Advanced identification of polymer type

Figure 4: Stepwise sampling, extraction, identification and quantification of plastics polymers in environmental samples

NB: Web of science; Scopus and Pubmed online databases were used.and all publications before March 2023 for Kenyan marine and inland waterbodies were selected using keywords Plastics; marine water/freshwater; Kenya; macro/micro/nanoplastics; microbeads;marine debris/litter; plastic polymers risk

Figure 5. Sketch diagram of the major pathways and distribution of microplastics

Waterways, Ditches; Canals

Sand harvesting; Dredging operations

WWTPs; Biosolids application; Agriculture

Overland Over-land disposal; Shoreline erosion

Biological recycling (microbial; biofouling uptake)

Fragmentation off solids

Waterflow dynamics

Quarries; Surface runoffs; Storm waters

3. RESULTS AND DISCUSSION

There is a growing number of publications targeting MPs and evidence of  contamination, mostly in aquatic environments. Initial studies on the biota targeted fish species and zooplankton. The definition of MPs is standardized in size i.e micro = < 5,000 µm (Macro = > 20,000 µm; Meso = 5,000 - 20,000 µm, Micro = < 5,000 µm; Nano = < 100 nm ) in most studies.

The plastics debris / litter recorded from sediments and water of coastal marine waters and freshwater lakes is classified as MPs using abundance/occurrence data, size, colour, polymer type (but only a few studies). The distribution of most of the microplastics is linked to shoreline activities, but in general, data from lotic (rivererive) ecosystems is lacking.

The data on the MPs accumulation in biota is not readily available/accessible, however, a few isolated sudies report concentration in GIT (gastro intestinal tract) and tissues. There are no simplified indices developed for water quality; and there are no levels recommended for regulations on water quality and sediments;  and other analysed compartments reported.

3.1. MICROPLASTICS STUDIES

In the environment, macro and microplastics originate from different human activities, industrial wastes, wastewaters, surface runoffs and storm waters, and from agricultural sources, but can be transported and deposited in soils, air, surface marine and freshwater and deposited in bottom sediments. Several authors have endeavour to provide reviews of microplastics at varying times and regional scales in soils, oceanic/marine water and freshwater or both systems. Results from the web of science including 2010-March 2020 show a dismal number of publications for freshwater bodies (about 40%), although the articles regarding microplastic pollution exceed 3000 and 32% of them originated from China (14%) and the USA (18%). Other countries included in the search were Germany, Italy, France, Espania, Australia, Netherlands, Canada, Portugal and Brasil.

Most environmental studies rely on the FTIR and Raman spectroscopy for the identification and quantification of synthetic polymer particles due to their high effectiveness. Others employ other techniques such as Pyrolysis - Gas Chromatograpyh-Mass spectrometry (PyGC-MS) and liquid chromatogrphy. There are several variation in methods of collection of environmental samples for determination of microplastics . However the main steps remain the same in most studies, i.e collection of the target media of interest, initial preparation, separation and extraction, detection and description and final quantification (Figure 1).

Standardization of procedures for MPs analysis is of priority, and where standard methods are not employed, sample volumes used need to be specified for each sample type (solid or liquid sample). MPs are mostly of low density and undergo degradation to smaller particles or enter surface waters as microbeads. Larger plastic items sink and the gradient of MP concentration starts to be directly proportional to the water depth (Reisser et 2015). It is therefore to prioritize both the water column and sediments at different sizes in order to capture the true abundance of MPs.

Although Africa has a considerable low production of plastics, there is a high consumption of plastic products for packaging, construction and building, household appliances, agriculture, automotive electrical and electronic consumer products. In generl, PE, PP and PVC are the commonly used polymers.

The fate and effect of microplastics in the environment largely depends on their properties  and composition. Polystyrene (PS, 1.04-1.08 g/cm3), acrylonitrile-butadiene-styrene (ABS, 1.04-1.06 g/cm3), polyamide (PA, 1.13-1.16 g/cm3), polycarbonate (PC, 1.20-1.22 g/cm3), cellulose acetate (CA 1.3 g/cm3), polyethylene terephthalate (PET/PETE, 1.38-1.41 g/cm3) and polyvinylchloride (PVC, 1.38-1.41 g/cm3) polymers have densities greater than 1 g/cm3 (Lechner et al., 2014), whereas polypropylene (PP, 0.85-0.92 g/cm3), expanded polystrene (EPS, 0.01-0.04 g/cm3) low density polyethylene (LDPE, 0.89-0.93 g/cm3), high density polyethylene (HDPE, 0.94-0.98 g/cm3) are below than 1 g/cm3.

3.2: Microplastic characterization and chemical compositions in Lake Naivasha

The microplastics study focussed on surface lake water. Surface water samples were collected in triplicate at each sampling site by towing a zooplankton net with 150‐μm mesh and mouth diameter of 0.30 m. Sampling was done in January 2019, and the towing was conducted at an average low speed of 2.5 to 3.5 knots and a short distance of 100m because the plankton net clogged easily. The trawl residues collected in the detachable polyvinyl chloride cod end were thoroughly rinsed into glass bottles and transported to the laboratory at 4 °C.

Samples for microplastics analysis were collected from 7 study areas (Karati River mouth; Kasarani; Sher Karuturi discharge outlet; Kamere Beach; Elsamere Conservancy and Hippo Point) and reported as particles/Km2. The recovered microplastics were sorted into the 4 main shape categories, (fibers/filaments, fragments, films, and pellets as outlined by Viršek et al. (2016). Pellets were not present in all the samples.  Fibers/filaments were dominating (n = 69.5, ≈81%), and there was a significant difference (p<0.05) in concentration among the locations studied. Fibers and fragments were present in all 7 locations, whereas films were only present in 5 sites. Surfcae water MPs recovered from L. Naivasha were classified as fibres, fragment and films,. There were no significant differences among the locations for fragments (n = 13.5) and films (n = 2.5). The colours were mainly dominated by Black/brown, blue, red , green and white /transparent microplastics. Along the Kenyan coastal region, Kosore et al. (2018) reported white (51%) and black (26%) as the dominant colours of MPs in surface marine water.

The concentrations of MPs in surface water varied from 0.183 ± 0.017 to 0.633 ± 0.067 particles/m2 (1.56 – 5.38 ± 0.57 particles/m3). River Malewa recorded the highest levels and the lowest at Hippo point. The mean microplastic concentration was 0.407 ± 0.135 particles/m2 (3.46 ± 1.15 particles/m3) measured over 21 shoreline trawls. The high microplastic abundance detected in Lake Naivasha was attributed to plastic properties, meteorological and hydrodynamic conditions, and river‐line inputs. There are no repeated studies in the study area for making comparisons on temporal variations in MPs. Again MPs accumulation in bottom sediments are lacking in this study, but widely documented in other aquatic systems (van Cauwenberghe et al., 2013; Tsang et al. 2017) Therefore, studies in a few mountain lakes in Kenya can also provide an indication of “background levels” in inland waterbodies, for comparison with hotspots of microplastic pollution, in areas mostly affected by increasing urbanization, although MPs have been found in remote areas in oceans and other pristine terrestrial regions. This could be associated to human activities and uncollected garbage at such sites and potential redistribution by current and water circulation.

Polymer identification provides useful information on possible sources of plastic that have either weathered or disintegrated to yield microplastics (Desforges et al. 2014). Of the total 86 microplastic particles recovered, 20 were analyzed using FTIR spectroscopy, of which 18 were found to have a polymer composition (Table 1). Polyethylene (n = 3), polyethylene terephthalate (n = 2), polyester (n = 7), and polypropylene (n = 5) plastic polymers were recovered (Figure 5).

Similarly, because polyethylene and polypropylene polymers have been identified as the main components of fishing nets and lines (Wang et al. 2018), fishing activities could thus be hypothesized to contribute significantly to the concentration of microplastics in the lake. Polyethylene terephthalate and polyester are typically used in textile production of clothes, fleece, and blankets. These are released by washing and have been shown to be major sources of microplastic fibers (Hernandez et al. 2017), discharged into the lake by household waste discharge, surface run‐off, and atmospheric deposition. Interestingly, polyethylene terephthalate (1.37 g/cm3) was recovered from surface water of Lake Naivasha although it is known to be a “denser than water” polymer. This could be the result of other factors working together to influence the vertical distribution of microplastics in aquatic environments rather than particle density.

Relationships of MPs with prevailing environmental conditions

- In L. Naivasha study, there were no statistically significant correlations between microplastic abundance and salinity, temperature, conductivity, dissolved oxygen, and total dissolved solids. However, there were strong positive association between microplastic abundance and turbidity, total

Table 1: The abundance and composition of MPs in surface water and bottom sediments from Lake Naivasha (Kenya.

nitrogen, and total phosphorus. With increased settlement and urbanization around the lake and its catchments, large water abstractions for agriculture and domestic uses have increased pollution pressures, resulting in highly contaminated surface run‐off from the catchment areas into the lake (Ndungu et al. 2015). Furthermore, degradation in the catchment by activities such as forest clearance have led to soil erosion and the consequent transport of nutrients into the lake (Njiru et al. 2017). High volumes of pesticides and fertilizers, together with effluents from the horticultural farms and informal settlements around the lake, have been reported to increase heavy metal and nutrient levels in the lake (Mutia et al. 2012), and consequently the quantities of microplastics.

Conclusion

Microplastics were ubiquitously detected in the surface waters of Lake Naivasha, with the highest abundance being recovered from the major inflow river into the lake. This finding suggests that the lake is greatly influenced by river‐line influences, and that potential microplastic sources include, but are not limited to, ineffective treatment of industrial, household sewage, and land‐based pollution in the river Malewa's catchment. Similarly, anthropogenic activities in/around the lake, together with selected physicochemical parameters, were found to influence the microplastic pollution in Lake Naivasha. The high concentrations of microplastics found in Lake Naivasha raise concerns over the need to develop measures to prevent the release or input of plastics into our aquatic ecosystems, by improvements in plastic waste management.

Gaps/ Recommendations

Based on this study, the following are recommended:

- Domestic sewage should be treated and screened before being released into Malewa River.

- Awareness should be created to sensitize the locals on the importance of Lake Naivasha conservation from the micro-plastic contaminants.

- The County government together with other relevant environment conservation stakeholders should impose heavy fines to those found discharging micro-plastic contaminants into the lake.

3.3: Microplastics in the coastal marine waters (Kenya)

At the onset and progression of the COVID - 19 pandemic in 2020 onwards to 2021, it was envisaged that uncontrolled garbage disposal and runoff water would contribute huge volumes of wastes into the marine environment. A study was conducted at 100 days (June 19th 2020) since the first case was reported to document quantities, distribution and composition of street, beach and marine floating litter. Okuku et al., (2020a) documented large variations in beach and floating litter classified as COVID-19 related and non-COVID-19 marine litter along some coastal areas of Kilifi and Kwale. Along remote (0.00 to 5.6 x 10-2 items/m2) and urban (ranged from 0.00 to 3.8 x 10-2 items/m2) coastal beaches, COVID-19 related items were in low densities (0.1 items/m2) and represented only 0.43 % of the total litter (Okuku et al. 2020a,b), a reflection of the containment measures which restricted movement to recreational facilities in the three counties and hindered local and foreign tourism activities.

Coastal beach trawl surveys during the COVID-19 pandemic recorded no COVID-19 related items (macro litter > 25mm), but non –COVID -19 related litter types and densities (34.337 Km2) varied from foam/styrofoam (1,042 items/Km2), plastic line /fibers (82,109 items/Km2), hard plastic (137,950 Km2), and soft plastic (126,236 items/Km2). Catchment discharge through rivers, physical trapping by mangroves, sheltered bays, and strong currents experienced during the SE-Monsoon which tends to transport litter to the north, contributed to the high densities of non-COVID-19 marine litter (Okuku et al., 2020a). Plastics dominated the marine litter documented but their fate in the marine environment is not clearly documented in relation to the sources. The snapshot of impacts from COVID-19 were considered short-lived, but with increasing awareness on the effects of plastic pollution, there is need to develop more specific studies and data, to inform on household recycling (for recyclable refuse) and better longterm solid waste management options.

Table 2: The abundance and composition of MPs in fish tissues from coastal marine waters (Kenya).

Table 3: The abundance and composition of MPs in surface water and zooplankton from coastal marine waters (Kenya).

Table 4: The abundance and composition of MPs in Jelly fish (Crambionella orsini) from coastal marine surface waters (Kenya).

3.4: Microplastics in Lake Victoria

There exists very few studies on MPs from the lake. The first documented evidence of presence of MPs in the great lakes was reported by Biginagwa et al., (2016). Plastics were confirmed in the gastrointestinal tracts of fish (Nile perch Lates niloticus (11fish), and Nile tilapia, Oreochromis niloticus (7 fish). Plastics were confirmed in 20% individuals of each of the two fish species (Biginagwa et al., 2016) using attenuated total reflection-Fourier-Transform Infra Red (ATR-FTIR) spectroscopy. The secondary plastics polymers included PE, PU, PE/PP Co-polymer, and silicone rubber, which are commonly used in packaging, clothing, food and drink containers, insulation and industrial applications. Land and urban area wastes were the suggested as the main sources of the degraded MPs ingested by the fish. Despite the low number of publications on MPs, there is a wide awareness campaign and events  or public days on cleaning the beaches of Lake Victoria.

Debris of microplastics (0 to 1,102 particles/Kg dry weight sediments), mesoplastics (0 to 218 particles/Kg d.wt.) and macroplastics (0 to 100 particles /Kg d.wt.) were investigated by Egessa et al. (2020a) in shoreline sediments and also in bottom lake sediments (micro-0 to 108 particles/Kg d.wt., meso-0 to 33 particles/ Kg d.wt., and macro- 0-77 particles/Kg d.wt. ) around the northern shores of Lake Victoria (Uganda). Across the size groups (micro, meso and macroplastics) the main forms of MPs found were films, filaments, fragments foam and pellets, with dominance of the shoreline and bottom sediments by films (> 54%) and filaments (>55%) respectively. Dominant polymer types included PE, PP, PE/PETE, PA, and PVC. Micro and mesoplastics (films, fragments and total plastics) in shoreline sediments revealed a strong and significant association. The mean abundance plastic debris (micro, meso and macroplastics) in shoreline sediments ( 75.2±50.0, 16.7±8.1, 18.1±4.6 particles/Kg d.wt.) was highest in fish landing beaches than in recreational beach areas (1.5±0.6, 3.1±3.1, 3.8±3.8 particles/Kg d.wt.), and also lake sediments near fish landing beaches (9.5±2.6, 2.1±1.5, 7.7±4.5 particles/Kg d.wt) recorded higher abundances than those from recreational areas (0.7±0.7, 0.2±0.1, 0±0 particles/Kg d.wt) , respectively.

Similarly, a non-parametric Spearmans rank correlation showed a strong correlation between MPs recorded in shoreline (macroplastics) and lake sediments (microplastics), with fish landing areas as areas of concern with regards to plastic pollution.

Egessa et al., (2020b) also reported low microplastic (size range 0.3 - 4.9 mm) abundances at Kagera and Katonga areas (sites in vicinity of river inflows), with much higher abundances at Bwondha, Ddimo, Kasensero (sites within fish landing sites in a rural setting), Kasenyi, Gaba and Masese (sites in vicinity of fish landing and recreational beaches, and within urban and semi-urban setting) in northern shores of Lake Victoria (Uganda). Transect sampling was done using a floating mantra net They were mainly degradation products from larger plastics. Microplastics were recorded in all the study sites and the overall abundance of MPs ranged from 2,834 - 329,167 particles/Km2 (0.02 - 2.19 particles /m3). The highest and lowest abundances were recorded in Bwondha, Ddimo, Kasensero (103, 333 to 329,167 particles/m2 or 0.69 to 2.19 particles/m3) and Kagera and Katonga areas (2,834 to 20,840 particles/m2 or 0.02 to 0.14 particles/m3) respectively. Microplastic of less than 1mm occurred in high proportion (36%) and were dominated by low density polymers of PE and PP. The analysed MPs were of local origin and mainly associated with community activities.

Potential risks from microplastics exposure

Increasing demand for fish protein and a source of micronutrients and omega - 3 poly unsaturated fatty acids, means an increasing demand in aquaculture fish production as wild capture fisheries continue to dwindle. nutritional benefits of seafood and fish consumption are many, however, increasing presence of microplastics and potential trophic transfer of contaminant needs to be considered. There were no data on evaluation of microplastics as contaminant vectors (metallic contaminants or persistent bioaccumulative organic pollutants) or substrate for pathogenic microorganisms. From the published articles, there are few datasets on concentration of microplastics to allow evaluation of toxicity risks from exposure. Levels of microplastics in whole consumed aquatic species may be of concerns to human dietary exposure risks, although in most cases offals are removed from fish for food. The highest concentration of in some of the commonly identified additives or monomers found in MP’s and such as polybrominated diphenyl ethers (50 ng/g), nonylphenol (2,500 ng/g) octylphenol (50 ng/g) and contaminants such as non-dioxin – like polychlorinated biphenyls (2,970 ng/g) and polycyclic aromatic hydrocarbons (44,800 ng/g) used to evaluate human seafood dietary intakes, showed that MP’s contribute only a very small fraction of the total dietary intake of contaminants (i.e PBDE’s 0.0007 to 0.003 %; PAH’s 0.02 to 0.1 %; PCB’s 0.007 to 0.03 %) (EFSA 2008, 2011, 2012; JECFA 2016, 2006). However, according to EFSA (2016), after oral ingestion the largest faction (>90%) of the ingested microplastics (0.1-5000 µm) and nanoplastics (1-100 nm) are excreted via feaces, and only particles smaller than 150 µm may translocate across the gut epithelium causing systemic exposure. But still, commonly found additives and sorbed toxic chemical contaminants from the environment can contribute to toxicity in exposed individual species, due to the small size, large surface area, interaction with other organic contaminants, persistence of microplastics and presence of different polymers types. Future research should strive to provide more data through targeted monitoring of MP’s in environmental samples, to understand effects of nanoplastics and microplastics (<150 µm) in aquatic species utilized for food or feed and also target the endangered species. The widespread presence of microplastics and presence of different polymers in the environment calls for more investigations on marine sourced materials, to enable unravel the exposure routes to humans. The relevance and implications of microplastics exposure of seafoods species, and human health are still largely unknown (Bouwmeester et al. 2015, Koelmans et al., 2016). Established biofilms on MP’s alters the property of MP’s and can attract both organic and inorganic pollutant chemicals and pathogenic microbes (Glaser 2020), but conducted research suggests no conclusive evidence on adverse effect on human health from the pathogenic microbial community on microplastics. Besides presence of MPS’s in biota and potential trophic transfer, there are concerns of the potential presence of MP’s in other raw marine products such as seas salt. MP’s have been found in commercial salts from 128 brands and from 38 different countries spanning over five continents, with concentrations lower than those reported for blue mussels (Peixoto et al 2019). A study of Portugese saltworks showed a range of 595 to 5090 (sea salt) and 3325 to 6430 (well salt) microplastics /kg (Soares et al. 2020), dominated by fibres and fragments. by providing vital data and raising more awareness to the public about the “invisible” effects of this widely used plastic materials, we will be able to implement better solid waste management practices at household levels and encourage more reuse and recycling of wastes. There is a general paucity of data on MP’s concentrations and the lack of standardized methods/protocols may still make it difficult to compare concentrations on the same environmental samples or compartments across regions, ecosystems and study areas (Table 4).

Table 5. A comparison of concentrations across different ecosystems

Ecosystems		Concentration and composition

Water							Sediments			Biota

1.Lake Naivasha (a)	Plankton nets						Mean (0.407±0.135 part/m2)	NA

Mean (0.407±0.135 part/m2)				Range (0.23 to 0.7 part/m2)

Range (0.183±0.017 to 0.633±0.067 part/m2)

Composition: PP; PET; PES;				NA				NA		)

Fibre; fragments; films

2. Lake Victoria (b)	Mantra nets						NA				Fish nets

3. Coastal waters (c )	NA							NA				Tow nets (500 µm)

Jellyfish- R (0.012 to 0.05 MPs/g).

4. American Great Lakes (d)	Mantra trawl nets (333 µm)			NA				NA

Mean (43,000 part/Km2)

Lake Michigan (e)		Mean (17,000 part/Km2)			NA				NA

Lake Erie (f)			Range (43,000-466,000 part/Km2)		NA				NA

5. Lake Geneva (g)		Mantra nets (300 µm)				NA				NA

Mean (0.048 part/m2)				NA				NA

6. Lake Taihu (h)		Plankton nets

Mean (0.01 to 6.8 part/Km2)			NA				NA

7. Three Gorges Dam (TGD),	Plankton nets

(China) (i)			R (1597-12611 n/m3) 				R (25-300 n/m3)		NA

8. Lake Maggiore (j)		Mantra nets (100 µm)				NA				NA

R(0.02-0.29 part/m3)

9. Urban lakes,		36items/L					67items/Kg			Fish(17items/ind.)

Bangladesh (k)

NB: a = Migwi et al. 2020; b = Biginagwa et al. 2016; c =Awuor et al., 2021; d =Eriksen et al. 2013; e =Manson et al., 2020; f = Manson et al., (2017); g =; h = Su et al., 2016; i = Di & Wang 2018; j =Binelli et al., 2024 ; k= Fariha et al. 2023; Part = particles of microplastics; R = Range; ind=individuals; n = number;

CONCLUSIONS

There are several sources of MPs into freshwater and marine ecosystems, resulting in no clear conclusive source. The microplastics are widely distributed, but not well quantified in the different compartments of the surface waters in Kenya; despite the several publications which quanity macroplastic or macrolitter. There are also limited studies on effects of MPs on the aquatic fauna (fish , macroinvertebrates, zooplankton), and therefore more studies are of priority in the Kenyan surface waters. The notable potential impacts of concern from microplstics pollution includes:- loss of aesthetic quality of surface water/recreational use; -Ingestion by animals/toxicity from leachates or additives; -habitat degradation; light transperancy; surface for colonization; high potential for traslocation ; loss of biodiversity and trophic efficiency. The common major threats to tropical lake basins (i.e catchment degradation, climate change, landuse changes, wetland degradation, population expansion and urbanization) double up as factors accelerating transfer and accumulation of plastics.

DISCLAIMER (ARTIFICIAL INTELLIGENCE)

Author hereby declares that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, ETC) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

Alencastro, D. (2012). Pollution due to plastics and microplastics in Lake Geneva and in the Mediterranean Sea. Arch. Des. Sci., 65:157–164.

Andrady, A. L. (2011). Microplastics in the marine environment. Marine Pollution Bulletin, 62:1596–1605.

Andrady, A. L. & Neal, M. A. (2009). Applications and societal benefits of plastics. Philos. Trans. R. Soc. B. Biol. Sci., 364:1977–1984.

Aura, C. M., Nyamweya, C. S., Odoli, C. O., Owiti, H., Njiru, J. M., Otuo, P. W.et al. (2020). Consequences of calamities and their management: The case of COVID-19 pandemic and flooding on inland capture fisheries in Kenya, Journal of Great Lakes Research, https://doi.org/10.1016/j.jglr.2020.09.007

Aura, C. M., Musa, S., Yongo, E., Okechi, J. K., Njiru, J. M., Ogari, Z. et al. (2018). Integration of mapping and socio-economic status of cage culture: Towards balancing lake-use and culture fisheries in Lake Victoria, Kenya. Aquac Res. 2018;49: 532–545.

Awuor, W., Muthumbi, A., Deborah, V. R.A. (2021). Presence of microplastic in Jellyfish (Cambrionella Orsini) along the Kenyan coast. Western Indian Ocean J. Mar. Sci., 20:10.4314/wiojms.v20il.12

Bakir, A., Rowland, S. J. & Thompson, R. C. (2014). Enhanced desorption of persistent organic pollutants from microplastics under simulated physiological conditions. Environ Pollut 185:16–23.

Ballent, A., Corcoran, P. L., Madden, O., Helm, P. A., & Longstaffe, F. J. (2016). Sources and sinks of microplastics in Canadian Lake Ontario nearshore, tributary and beach sediments. Mar. Pollut. Bull., 110:383–395.

Biginagwa, F. J., Mayoma, B. S., Shashoua, Y., Syberg, K., & Khan, F. R. (2016). First evidence of microplastics in the African great lakes: Recovery from Lake Victoria Nile perch and Nile tilapia. J. Great Lakes Res., 42(1): 146-149; https://doi.org/10.1016/j.jglr.2015.10.012

Browne, M. A., Crump, P., Niven, S. J., Teuten, E., Tonkin, A., et. al., (2011). Accumulation of microplastic on shorelines worldwide: Sources and sinks. Environ. Sci. Technol., 45:9175–9179.

Binelli, A., Magni, S., Torre, C. D., Sbarberi, R., Cremones, C.. & Galafassi, S. (2024). Monthly variability of floating plastic contamination in Lake Maggoire (Northern Italy). Sci. Tot. Environ., 919:170740.

Carpenter, E.J., Smith, K., (1972). Plastics on the Sargasso sea surface. Science, 175:1240-1241.

Cole, M., Webb, H., Lindeque, P. K., Fileman E. S., Halsband, C. & Galloway, T. S. (2014). Isolation of microplastics in biota‐rich seawater samples and marine organisms. Sci Rep., 4:4528.

Desforges, J. P. W., Galbraith, M., Dangerfield, N. & Ross, P. S. (2014). Widespread distribution of microplastics in subsurface seawater in the NE Pacific Ocean. Mar. Pollut. Bull. 79:94–99.

Di, M. & Wang, J. (2018). Microplastics in surface waters and sediments of the Three Gorges Reservoir, China. Sci. Tot. Environ, 616-617: 1620-1627.

Dris, R., Gasperi, J., Rocher, V., Saad, M., Renault, N. & Tassin, B. (2015). Microplastic contamination in an urban area: A case study in Greater Paris. Environ. Chem. 12:592–599.

Egessa, R., Nankabirwa, A., Basooma, R. & Nabwire, R. (2020a). Occurrence, distribution and size relationships of plastic debris along shores and sediment of northern Lake Victoria. Environ. Pollut.,257: 113442,

Egessa, R., Nankabirwa A., Ocaya, H. & Pabire, W. G. (2020b) Microplastic pollution in surface water of Lake Victoria. Sci. Tot. Environ., 741, 140201.

Eriksen, M., Mason, S., Wilson, S., Box, C., Zellers, A. & Edwards, W. (2013). Microplastic pollution in the surface waters of the Laurentian Great Lakes. Mar. Pollut. Bull., 77:177–182.

FAO (2010). Food and Agriculture Organization. Report of the FAO expert workshop on on-farm feeding and feed management in aquaculture. Manila, The Philippines, 13–15 September 2010. FAO Fisheries and Aquaculture Report No. 949. Rome, FAO. 37.

Fariha, T. M., Rashidul Alam, A. K. M., Akbor, M. A. (2023). Abundances and characteristics of microplastics in major urban lakes of Dhaka, Bangladeshi, Heliyo, 9(4); e14587

Faure, F., Demars, C., Wieser, O., Kunz, M. & De Alencastro, L. F. (2015). Plastic pollution in Swiss surface waters: Nature and concentrations, interaction with pollutants. Environ Chem., 12:582–591.

Fiorella, K. J., Bageant, E. R., Mojica, L., Obuya, J. A., Ochieng, J. & Olela, P. (2021). Small-scale fishing households facing COVID-19: The case of Lake Victoria, Kenya. Fish. Res., 237, 105856.

Fischer, E. K., Paglialonga, L., Czech, E. & Tamminga, M. (2016). Microplastic pollution in lakes and lake shoreline sediments—A case study on Lake Bolsena and Lake Chiusi (central Italy). Environ. Pollut., 213:648–657.

Free, C. M., Jensen, O. P., Mason, S. A., Eriksen, M., Williamson, N. J. & Boldgiv, B. (2014). High‐levels of microplastic pollution in a large, remote, mountain lake. Mar. Pollut. Bull. 85:156–163.

Gauquie, J., Devriese, L., Robbens, J., De Witte, B. (2015). A qualitative screening and quantitative measurement of organic contaminants on different types of marine plastic debris. Chemosphere, 138:348–356.

Geyer, R., Jambeck, J. R. & Law, K. L. (2017). Production, use, and fate of all plastics ever made. Sci. Adv., 3:e1700782.

GOK and UNDP (2021). Rising Water Levels in Kenya’s Rift Valley Lakes, Turkwel Gorge Dam and Lake Victoria, The Short-Popular-Version, A scoping report. 44P. GOK and UNDP.

Hamilton, S. E., McGehee, D. D., Nyamweya, C., Ongore, C., Makori, A., Mangeni-Sande, R. et al., (2022) High – resolution bathymetries and shorelines for the great lakes of the White Nile basin. Scientific Data, 9: 642,

Hamilton, S. E., Gallo, S. M., Krach, N., Nyamweya, C., Okechi, J. et al. (2019). The use of unmanned aircraft systems and high-resolution satellite imagery to monitor tilapia fish-cage aquaculture expansion in Lake Victoria, Kenya. Bulletin of Marine Science, DOI:10.5343/bms.2019.0063

Hernandez, E., Nowack, B. & Mitrano, D. M. (2017). Polyester textiles as a source of microplastics from households: A mechanistic study to understand microfiber release during washing. Environ. Sci. Technol. 51:7036–7046.

Hidalgo‐Ruz, V., Gutow, L., Thompson, R. C. & Thiel, M. (2012). Microplastics in the marine environment: A review of the methods used for identification and quantification. Environ. Sci. Technol., 46:3060–3075.

Horton, A. A., Svendsen, C., Williams, R. J., Spurgeon, D. J. & Lahive, E. (2017). Large microplastic particles in sediments of tributaries of the River Thames, UK— Abundance, sources and methods for effective quantification. Mar. Pollut. Bull., 114:218–226.

Jiang, P., Zhao, S., Zhu, L. & Li, D. (2018). Microplastic‐associated bacterial assemblages in the intertidal zone of the Yangtze Estuary. Sci. Tot. Environ., 624:48–54.

Kitheka, J. U., Okemwa, E. N. & Kazungu, J. M. (1999) Monitoring of nutrient levels, turbidity and sediment transport at Port-Reitz creek in Kenya. IOC-SIDA GIPME/MARPOLMON Nutrients, sediment transport, and turbidity monitoring programme, report.

Kosore, C., Ojwang, L., Maghanga, J., Kamau, J., Kimeli, A., Omukoto J. et al., (2018). Occurrence and ingestion of microplastics by zooplankton in Kenya's marine environment: First documented evidence. Afri. J. Mar. Sci., 40:225–234.

La Daana, K. K., Officer, R., Lyashevska, O., Thompson, R. C. & O'Connor, I. (2017). Microplastic abundance, distribution and composition along a latitudinal gradient in the Atlantic Ocean. Mar. Pollut. Bull., 115:307–314.

Law, K. L. & Thompson, R. C. (2014). Microplastics in the seas. Mar. Pollut. Bull. 345:144–145.

Liebezeit, G. and Dubaish, F5, (2012). Microplastics in beaches of the aEast Frisian islands Spiekerog and Kachelotplate, Bull. Environ. Contam. Toxicol.

Li, J., Liu, H. & Chen, J. P. (2018). Microplastics in freshwater systems: A review on occurrence, environmental effects, and methods for microplastics detection. Water Res. 137:362–374.

Maes, T., Van der Meulen, M. D., Devriese, L. I., Leslie, H. A., Huvet, A., Frère, L. etal. (2017). Microplastics baseline surveys at the water surface and in sediments of the North‐East Atlantic. Front. Mar. Sci., 4:135.

Mason, S. A., Kammin, L., Eriksen, M., Aleid, G., Wilson, S., Box, C. etal., (2016). Pelagic plastic pollution within the surface waters of Lake Michigan, USA. J. Great Lakes Res 42:753–759.

Masura, J., Baker, J. E., Foster, G. D., Arthur, C. & Herring, C. (2015). Laboratory methods for the analysis of microplastics in the marine environment: Recommendations for quantifying synthetic particles in waters and sediments. NOAA Tech Memo. National Oceanic and Atmospheric Administration, Silver Spring, MD, USA.

Mato, Y., Isobe, T., Takada, H., Kanehiro, H., Ohtake, C. & Kaminuma, T. (2001). Plastic resin pellets as a transport medium for toxic chemicals in the marine environment. Environ. Sci. Technol., 35:318–324.

Migwi, F. K., Ogunah, J. A. & Kiratu, J. M. (2020). Occurrence and spatial distribution of microplastics in the surface water of Lake Naivasha, Kenya. Environ. Toxicol. Chem. 39: 765-774.

Mironga, J. M., Mathooko, J. M. & Onywere, S. M. (2012). Effect of water hyacinth infestation on the physicochemical characteristics of Lake Naivasha. Int. J. Hum. Soc. Sci., 2:103–113.

Mutia, T. M., Virani, M. Z., Moturi, W. N., Muyela, B., Mavura, W. J. & Lalah, J. O. (2012). Copper, lead and cadmium concentrations in surface water, sediment and fish, C. carpio, samples from Lake Naivasha: Effect of recent anthropogenic activities. Environ. Earth Sci., 67:1121–1130.

Mutungwa, D. M. 2011. Effects of climate change and human activity on lake levels in the Kenyan rift (case study Naivasha). BSc. Thesis. University of Nairobi, Nairobi, Kenya. Naivasha Water and Sanitation. 2019. Sanitation Services. Naivasha, Kenya. [cited 2019 October 24]. Available from: https://www.naivashawater.co.ke/sewer/

Ndungu, J., Augustijn, D. C. M., Hulscher, S. J. M. H., Fulanda, B., Kitaka, N. & Mathooko , J. M. (2015). A multivariate analysis of water quality in Lake Naivasha, Kenya. Mar. Freshw. Res., 66:177–186.

Nel, H. A. & Froneman, P. W. (2015). A quantitative analysis of microplastic pollution along the south‐eastern coastline of South Africa. Mar. Pollut. Bull., 101:274–279.

Noren, F. and Naustvoll, L. J. (2010). Survey of microscopic anthropogenic particles in Skagerrak (p. 21) Flodevigen, Norway. Institute of Marine Research.

Njiru, J., Waithaka, E. & Aloo, P. A. (2017). An overview of the current status of Lake Naivasha Fishery: Challenges and management strategies. Open Fish Sci. J. 10:1–11.

Obiero, J. P. & Onyando, J. O. (2013). Climate. In Developments in earth surface processes, 16: 39 - 50.

Ochieng, E. Z., Lalah, J. O. & Wandiga, S. O. (2007). Analysis of heavy metals in water and surface sediment in five Rift Valley lakes in Kenya for assessment of recent increase in anthropogenic activities. Bull. Environ. Contam. Toxicol., 79:570–576.

Ogello, E.O., Mbodze, M.M., Outa, N.O., Ajode, M.Z., Ojwando, W. (2024). Characterizing anthropogenic litter pollutants in African freshwater ecosystems: A case study of Dunga beach in Lake Victoria Kenya, Lakes & Reservoirs: Res.Manage., 29(1): e12454

First published:18 March 2024; https://doi.org/10.1111/lre.12454

Okuku, E., Kombo, M.M., Mwalugha, C.S., Chiphatsi, M.M.; Otieno, K., Chepkemboi, P., Owato, G.O., Otieno C.O. (2024) Anthropogenic litter pollution in selected African transboundary and Great rift valley lakes, Journal of Great Lakes Research;

Available online: 3 July 2024; https://doi.org/10.1016/j.jglr.2024.102382

Okuku, E. O., Owato, G., Kiteresi, L. I., Otieno, K., Kombo, M., Wanjeri, V., et al., (2022a). Are tropical estuaries a source of or a sink for marine litter? Evidence from Sabaki Estuary, Kenya, Mar. Pollut. Bull., 176, 113397, https://doi.org/10.1016/j.marpolbul.2022.113397.

Okuku, E. O., Owato, G., Otieno, K., Kombo, M. M., Mbuche, M., Gwada, B., et al. (2022b). The influence of monsoons on the spatial distribution and composition of floating marine litter, Mar. Pollut. Bull., 175, 113386, https://doi.org/10.1016/j.marpolbul.2022.113386.

Okuku, E. O., Kiteresi, L. I., Owato, G., Otieno, K., Omire, J., Kombo, M. M. et al.  (2021). Temporal trends of marine litter in a tropical recreational beach: A case study of Mkomani beach, Kenya, Mar. Pollut. Bull., 167, 112273, ttps://doi.org/10.1016/j.marpolbul.2021.112273.

Okuku, E.O., Kiteresi, L. I., Owato, G., Mwalugha, C., Mbuche, M., Gwada, B., et al., (2020a). The impacts of COVID-19 pandemic on marine litter pollution along the Kenya coast: A synthesis after 100 days following the first reported case in Kenya. Mar. Poll. Bull., 162, 111840.

Okuku, E. O., Kiteresi, L. I., Owato, G., Mwalugha, C., Omire, J., Mbuche, M. et al., (2020b) Baseline meso-litter pollution in selected coastal beaches of Kenya: Where do we concentrate our intervention efforts? Mar. Poll. Bull., 138:111420.

Okuku, E. O., Kiteresi, L. I., Owato, G., Mwalugha, C., Omire, J., Otieno, K. et al., (2020c). Marine macro-litter composition and distribution along the Kenyan Coast: The first-ever documented study, Mar. Pollut. Bull., 159, 111497, https://doi.org/10.1016/j.marpolbul.2020.111497.

Okronipa, H., Bageant, E. R., Baez, J., Onyango, H. O., Aura, C.M. & Fiorella, K. J. (2023). COVID-19 experiences of small-scale fishing households: The case of Lake Victoria, Kenya, Front. Sustain. Food Syst., 6: 987924

Omondi, D. O., Wairimu, M. A., Aketch, W. L., William, S. A., Trick, C. G. & Creed, I. F. (2015). Faecal pollution and solar purification of community water sources within Lake Naivasha basin, Kenya. J. Water Sanit. Hyg. Dev., 5:252–260.

PlasticsEurope. (2016). Plastics—The facts (2012). An analysis of European plastics production, demand and waste data. Paris, France. [cited 2019 May 16]. Available from: https://www.Plast.org/application/files/4315/1310/4805/plastic‐the‐fact‐2016.pdf

Reisser, J., Slat, B., Noble, K., du Plessis, K., Epp, M., Prioetti, M. et al., (2015). The vertical distribution of buoyant plastics at sea: an observational study in the North Atlantic Gyre. Biogeosciences 12, 1249-1256.

Ruwa, R. (2006). Coastal and offshore marine fisheries of Kenya: Sttus and opportunities KMFRI Technical report 2004/FP/1, http://hdl.handle.net/1834/7096, 44pp.

Science Advice for Policy by European Academies. (2019). A scientific perspective on microplastics in nature and society. Berlin, Germany.

Stoof‐Leichsenring, K. R., Junginger, A., Olaka, L. A., Tiedemann R. & Trauth, M. H. (2011). Environmental variability in Lake Naivasha, Kenya, over the last two centuries. J Paleolimnol., 45:353–367.

Su, L., Xue, Y., Li, L., Yang, D., Kolandhasamy P., Li, D. et al.,. (2016). Microplastics in Taihu Lake, China. Environ. Pollut., 216:711–719.

Tsang, Y. Y., Mak C. W., Liebich, C., Lam, S. W., Sze, E. T. P. & Chan, K. M. (2017). Microplastic pollution in the marine waters and sediments of Hong Kong. Mar. Pollut. Bull., 115:20–28.

Turner, A. & Holmes L. A. (2015). Adsorption of trace metals by microplastic pellets in fresh water. Environ. Chem., 12:600–610.

Van, A, Rochman CM, Flores, E. M., Hill, K. L., Vargas, E., Vargas, S. A. et al., (2012). Persistent organic pollutants in plastic marine debris found on beach in San Diego, California. Chemosphere, 86:258–263.

Van, Cauwenberghe L, Devriese L, Galgani F, Robbens J, Janssen CR. 2015.Microplastics in sediments: A review of techniques, occurrence and effects. Mar Environ Res 111:5–17.

Viršek, MK, Palatinus A, Koren Š, Peterlin M, Horvat P, Kržan A. (2016). Protocol for microplastics sampling on the sea surface and sample analysis. J Vis Exp 118:e55161.

Wang, W, Ndungu AW, Li Z, Wang J. (2017). Microplastics pollution in inland freshwaters of China: A case study in urban surface waters of Wuhan, China. Sci Total Environ 575:1369–1374.

Wang, W, Yuan W, Chen Y, Wang J. (2018). Microplastics in surface waters of Dongting Lake and Hong Lake, China. Sci Total Environ 633:539–545.

Wright, SL, Thompson RC, Galloway TS. (2013). The physical impacts of microplastics on marine organisms: A review. Environ Pollut 178:483–492.

Yang, D, Shi H, Li L, Li J, Jabeen K, Kolandhasamy P. (2015). Microplastic pollution in table salts from China. Environ Sci Technol 49:13622–13627.

Zhang, K., Gong, W., Lv, J., Xiong, X., Wu, C. (2015). Accumulation of floating microplastics behind the Three Gorges Dam. Environ. Poll., 204:117–123.

Zhao, S., Zhu. L. & Li, D. (2015). Microplastic in three urban estuaries, China. Environ Pollut 206:597–604.

Zhao, S., Zhu, L. & Li, D. (2016). Microscopic anthropogenic litter in terrestrial birds from Shanghai, China: Not only plastics but also natural fibers. Sci Total Environ 550:1110–1115.10 Environmental Toxicology and Chemistry, 2020;00:1–10.