Geochemical and Isotopic Insights into Groundwater Dynamics and Sustainable Management in Mount Cameroon's Volcanic Aquifer.

Original Research Article

Abstract

Groundwater resources under tropical climates face escalating pressures from climate change and anthropogenic activities. The eastern flank of Mount Cameroon a critical agro-industrial zone exemplifies these challenges, yet comprehensive assessments integrating hydrogeochemical and isotopic evidence remain limited. Fifty groundwater samples (boreholes, springs, wells) and 14 precipitation events’’ should be written as 50 groundwater samples (boreholes, springs, wells) and 14 precipitation events. In-situ parameters (pH, EC, temperature) were measured followed by laboratory quantification of major ions (ion chromatography), trace elements (ICP-MS), and stable isotopes (δ¹⁸O, δ²H; cavity ring-down spectroscopy). Hydrochemical facies and recharge processes were evaluated using Aquachem, Piper diagrams, and Local Meteoric Water Line (LMWL) regression. Three dominant hydrochemical facies were identified: Ca-Cl (74%), indicating ion exchange and evaporite dissolution; Ca-HCO₃ (20%), reflecting active recharge and carbonate weathering; and mixed Ca-Mg-Cl (6%). Groundwater exhibited low mineralization (mean TDS: 102.24 mg/L; EC: 314.94 µS/cm) but localized Fe (≤5.31 mg/L) and Mn (≤3.28 mg/L) exceeded WHO thresholds. Isotopic signatures (δ¹⁸O: −2.90 ± 0.46‰; δ²H: −11.39 ± 1.99‰) confirmed precipitation as the primary recharge source, with d-excess values (9.96‰) indicating minor evaporation. A 15% precipitation decline (1991–2020) correlated with enriched δ¹⁸O-Cl⁻ trends, signalling climate-induced recharge vulnerability. The aquifer’s sustainability is threatened by climate-driven precipitation variability, geogenic contamination (Fe/Mn), and agricultural pollutants. Targeted monitoring of shallow zones, protection of fracture-mediated recharge systems, and climate-resilient policies are imperative for water security. This study establishes a critical baseline for sustainable management of volcanic aquifers in tropical regions.

Keywords: Groundwater, Isotopes, Volcanic Aquifer, Mount Cameroon, Water Quality, Sustainable Management

- Introduction

Groundwater resources in volcanic terrains represent vital freshwater reservoirs for socio-economic development, particularly in tropical regions like Mount Cameroon. As West and Central Africa's highest peak (4,040 m), this active stratovolcano forms part of the geologically dynamic Cameroon Volcanic Line (CVL) a 1,600-km chain of Cenozoic volcanoes extending from the Atlantic Ocean into continental Africa (Asaah et al., 2015; Hermann, 2023). The eastern flank of Mount Cameroon serves as a critical agro-industrial zone where groundwater supports irrigation, domestic consumption, and industrial activities. However, this resource faces escalating pressures from population growth, agricultural intensification, and climate variability, with projections indicating that 50% of Africa's population will experience water stress by 2025 (Adom et al., 2022). The complex interplay of volcanic and sedimentary lithologies—fractured basalts, pyroclastics, and Cretaceous-Quaternary sediments—creates heterogeneous aquifer systems where hydrogeochemical processes remain poorly quantified (Ako et al., 2012; Ngai et al., 2024).

The aquifer architecture of Mount Cameroon's eastern flank exhibits stark contrasts: volcanic units feature high-permeability fracture networks (10⁻³ m/s) facilitating rapid recharge, while sedimentary formations (e.g., alluvial sands, lacustrine clays) exhibit primary porosity with moderate hydraulic conductivity (5–20 m/day) (Table 1; Fig. 1.). Groundwater chemistry is dominantly shaped by silicate weathering of basaltic minerals (e.g., plagioclase, olivine), releasing Ca²⁺, Mg²⁺, and dissolved silica, while evaporite dissolution in sedimentary strata enriches Cl⁻ and SO₄²⁻ (Wirmvem et al., 2017; Ngai et al., 2024). Despite foundational studies on spring hydrogeochemistry (Ako et al., 2012) and isotopic tracers (Fontes & Olivry, 1977; Nlend et al., 2023), critical knowledge gaps persist: (1) Sparse spatial coverage of the eastern flank obscures hydrochemical gradients across volcanic-sedimentary interfaces; (2) Lithological heterogeneity complicates predictions of contaminant mobility, especially Fe/Mn from geogenic sources and NO₃⁻ from agro-industry; (3) Absence of a robust Local Meteoric Water Line (LMWL) hinders reconciliation of δ¹⁸O/δ²H data with recharge dynamics; and (4) Climate change impacts on aquifer resilience remain unquantified despite a 15% precipitation decline since 1991 (Fig. 1; Cheunteu Fantah et al., 2022; Tchakam Kamtchueng et al., 2022). The sustainable management of water resources is paramount in volcanic regions such as Mount Cameroon, where these resources underpin socio-economic development. This study addresses critical challenges stemming from complex subsurface hydrological interactions and escalating anthropogenic pressures driven by population growth and burgeoning agro-industrial activities (Baud et al., 2024; Emvoutou et al., 2018). A key imperative is to resolve significant uncertainties concerning water quality, contamination pathways, and recharge dynamics within this vital agro-industrial zone (Billa, 2019; Fongoh et al., 2023). The overarching objective of this study is to provide a comprehensive hydrogeochemical characterization of both surface and groundwater on the Eastern flank of Mount Cameroon. This includes elucidating water quality uncertainties, identifying contamination pathways, and defining recharge dynamics within this crucial agro-industrial area. A pioneering aspect of this research involves integrating stable isotope hydrology with advanced geochemical modeling to achieve these objectives (Eid et al., 2024; Emvoutou et al., 2024; IAEA, 2006). The findings of this research establish a vital baseline for informed water resource management (Mussa & Mjemah, 2023). They underscore the urgent necessity for targeted contaminant monitoring (particularly for Fe, Mn, and nitrate) in vulnerable shallow aquifers, the implementation of community-based sanitation initiatives to mitigate anthropogenic impacts, and the development of climate-resilient policies to safeguard recharge and ensure long-term aquifer sustainability (Cook, 2003; Egbueri et al., 2019; Négrel & Petelet-Giraud, 2011). This study thus provides a foundational framework for developing effective strategies that balance water security, agricultural productivity, and ecological preservation within Mount Cameroon's dynamic volcanic environment (Ayenew et al., 2008; Jannat et al., 2022).

  - Study Area

The study area, located on the slopes of Mount Cameroon (Fig 1 and Table 1), is situated between latitudes 4° 8’30’’ and 4°25’0’’N, and longitudes 9°12’0’’ and 9°28’30’’ E. The region experiences a tropical highland climate, influenced by both the Atlantic Ocean and the central African monsoon. The climate is characterized by:

- Temperature: The average annual temperature ranges from 10°C to 20°C, with cooler temperatures at higher elevations. Seasonal variations are minimal, with slightly cooler temperatures during the wet season (May to October).

- Precipitation: The region has a bimodal rainfall pattern, with peaks in May-June and September-October. Annual precipitation can exceed 2,500 mm, particularly on the western slopes due to orographic uplift.

- Extreme Weather Events: The frequency and intensity of extreme weather events, such as droughts and floods, are projected to increase due to climate change.

Historically, the region has had a tropical climate with high rainfall and relatively stable temperatures. However, recent trends suggest a rise in average temperatures and potential changes in precipitation patterns, including an increased frequency of droughts and intense downpours. The annual temperature range is between 17°C and 34°C.

Figure 1: Study area location

Table 1. Details of sampling location

| No. | ID | Towns | Lat | Long | No. | ID | Towns | lat | Long |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | OW1 | stranger quarter | 4.175 | 9.280 | 26 | OW17 | camp 3 | 4.298 | 9.462 |
| 2 | OW2 | MALENDE | 4.321 | 9.428 | 27 | 0W18 | camp2 | 4.278 | 9.449 |
| 3 | RV | penda mboko | 4.182 | 9.309 | 28 | OW19 | bone quarter penda mboko | 4.358 | 9.409 |
| 4 | OW3 | YOKE | 4.218 | 9.305 | 29 | CAT2 | Koke | 4.262 | 9.443 |
| 5 | OW4 | MBO QUARTER,YOKE | 4.170 | 9.307 | 30 | BH2 | Lysoka | 4.285 | 9.401 |
| 6 | OW5 | MALENDE candef mosack quarter | 4.355 | 9.315 | 31 | CAT3 | Lyongo | 4.337 | 9.436 |
| 7 | OW6 | MALENDE | 4.158 | 9.279 | 32 | BH3 | check point buea | 4.260 | 9.364 |
| 8 | OW7 | YOKE nangi street | 4.157 | 9.249 | 23 | SPG5 | Muea | 4.288 | 9.414 |
| 9 | OW8 | new layout muyuka | 4.301 | 9.378 | 34 | OW20 | Muea | 4.267 | 9.434 |
| 10 | OW9 | Akawuh quarter | 4.158 | 9.292 | 35 | OW21 | matouke 2 | 4.283 | 9.404 |
| 11 | SPG1 | kwe kwe | 4.155 | 9.270 | 36 | CAT4 | mile 18 | 4.310 | 9.427 |
| 12 | OW10 | kwe kwe | 4.346 | 9.435 | 37 | OW22 | yoke king bar | 4.285 | 9.403 |
| 13 | SPG2 | ekona corner road | 4.144 | 9.236 | 38 | BH4 | buea town | 4.287 | 9.400 |
| 14 | SPG3 | mautu cross river | 4.333 | 9.435 | 39 | OW23 | muyuka 7 | 4.274 | 9.392 |
| 15 | SPG4 | mautu unity quatrter | 4.158 | 9.279 | 40 | BH5 | slaughter house Buea town | 4.288 | 9.403 |
| 16 | CAT1 | Mautu | 4.271 | 9.443 | 41 | BH6 | GREAT SOPPO | 4.231 | 9.339 |
| 17 | OW11 | MUEA ,WONYA MEVEO | 4.305 | 9.423 | 42 | BH7 | Bosta | 4.259 | 9.367 |
| 18 | OW12 | YOKE nangi street | 4.159 | 9.244 | 43 | SPG6 | Bafia | 4.365 | 9.314 |
| 19 | STM1 | Owe | 4.228 | 9.342 | 44 | OW24 | malende 3 | 4.301 | 9.464 |
| 20 | STM2 | Ikata | 4.075 | 9.304 | 45 | SPR7 | Bonduma | 4.288 | 9.408 |
| 21 | OW13 | Matouke | 4.162 | 9.299 | 46 | BH8 | BOSTA | 4.297 | 9.431 |
| 22 | OW14 | koto 111 | 4.330 | 9.354 | 47 | OW25 | yoke 3 | 4.302 | 9.779 |
| 23 | OW15 | penda mboko | 4.298 | 9.381 | 48 | SPR8 | BAFIA 2 | 4.319 | 9.431 |
| 24 | OW16 | camp 1 | 4.309 | 9.463 | 49 | BH9 | Sandpit | 4.347 | 9.433 |
| 25 | BH1 | bone quarter penda mboko | 4.299 | 9.463 | 50 | OW26 | old police road muyuka | 4.343 | 9.431 |

OW = open well, BH = Borehole, STM = stream, CAT = catchment, SPG = spring

  - Geology and Hydrogeology

The study area, situated on the slopes of Mount Cameroon, is part of the active Cameroon Volcanic Line (CVL) that stretches across the country (Ako et al., 2012; Che et al., 2012). The region's geology is a blend of metamorphic pre-Cameroon formations and intrusive gneisses and granites (Ngongang, 2016; Fitton and Dunlop, 1985). The volcanic rocks, except for the Nephelinit series of Mount Etinde, are predominantly alkaline(Fig 2). The massif's foundation comprises basaltic and tephra-basaltic ferromagnetic rocks originating from the Strombo-Hawaiian shield (Benedetti et al., 2003). Recent lava flows might exhibit lower permeability compared to older, weathered volcanic formations (Forster & Chilton, 2018).

The hydrogeology of the area is characterized by abundant rainfall that recharges underground reservoirs through percolation. The landscape, rich in rivers, springs, and mountains, coupled with high humidity, facilitates substantial subterranean water flow, often emerging as large springs downhill (Endeley et al., 2001; Akoachere et al., 2006; Ako et al., 2012). The porous pyroclastic materials and weathered basaltic rocks serve as significant aquifers (Ako et al., 2012). The dominant aquifer system is likely a fractured rock aquifer within the volcanic formations, where groundwater recharge is primarily driven by precipitation infiltration at higher elevations (Forster & Chilton, 2018). The permeability of the volcanic rocks and the presence of fractures influence infiltration rates and groundwater availability (Lerner et al., 2009). The topography of eastern Mount Cameroon features a wide elevation range, from over 4,000 meters above sea level on Mount Cameroon's slopes to 1,000-2,000 meters in the foothills (Tchindjan et al., 2012). The steep slopes and valleys influence precipitation and surface water flow, with the western slopes receiving more rainfall due to orographic uplift. The rapid surface runoff on steeper slopes can potentially reduce groundwater recharge. The area's petrology reflects Strombo-Hawaiian volcanic activity, with dominant ferromagnesian basaltic lava. Under humid conditions, these rocks weather to form dark-colored andosols mixed with alluvial and sandy loam soils (Ako, 2014; UCCC, 2014).

Several factors, including high altitude, proximity to the sea, and humidity variations, influence chemical weathering processes (Bulourde, 2003). Meteorological factors like heavy rainfall, high temperatures, and humidity, combined with the variable substrates, contribute to chemical weathering, ion exchange, and rock-water interactions. These processes, coupled with human activities, lead to the diverse water types and hydrogeochemical phases observed in the region

Figure 2: Geologic map of the study area (modified from Che et al. 2012)

The existing literature on groundwater dynamics, hydrogeochemistry, and isotopic analysis in volcanic aquifers provides a foundation for understanding the complex processes governing these systems. Studies have highlighted the influence of geological factors, such as rock type and weathering processes, on groundwater chemistry (Lahermo et al., 1990; Menti et al., 2023). The role of climate and land use in shaping groundwater recharge and quality has also been recognized (Kotchoni et al., 2019; Bauer et al., 2006). Isotopic techniques have been employed to trace the origins of groundwater and understand recharge mechanisms in volcanic settings (Wotany et al., 2021; Wirmvem et al., 2014).

2.0 Material and method

A stratified sampling strategy was employed to capture the hydrogeological variability across volcanic and sedimentary formations. A total of 64 water samples were collected, comprising 50 groundwater sources—ranging from boreholes and springs to open wells and 14 precipitation events sampled across seasons to encapsulate hydrological shifts. Each sampling location was georeferenced, enabling spatial mapping and gradient analysis .In situ measurements of key physicochemical parameters, including pH, electrical conductivity (EC), temperature, and alkalinity, were conducted immediately at each site using a calibrated HANNA HI 98127 multiprobe with a precision of ±0.01 for pH. Alkalinity was determined by titration following CHEMetrics protocols, with sulfuric acid titrated to a pH endpoint of 4.5. To ensure analytical integrity, all water samples were filtered through 0.45 µm cellulose membranes and preserved in pre-rinsed HDPE bottles. Trace metal samples were acidified to pH <2 using ultrapure nitric acid, while isotope samples were sealed airtight and stored at 4°C to avoid fractionation. Precipitation data, spanning 1991–2020, were gathered monthly using Palmer rain gauges to establish long-term isotopic baselines .In the laboratory, major ions—such as Ca²⁺, Mg²⁺, Na⁺, K⁺, Cl⁻, SO₄²⁻, and NO₃⁻ were quantified via ion chromatography (Dionex ICS-1100) in accordance with APHA Standard Method 4110. Bicarbonate was determined through Gran titration, with charge balance errors maintained within 5% (Eq. 15). Trace elements, including Fe, Mn, As, and twelve additional metals, were measured by Inductively Coupled Plasma Mass Spectrometry (ICP-MS) using a PerkinElmer Elan DR II, adhering strictly to ISO 17294-2:2003 and calibrated against NIST-traceable standards (Section 3.7). Stable isotope ratios (δ¹⁸O and δ²H) were analyzed using wavelength-scanned cavity ring-down spectroscopy (Picarro L2120-i), delivering high precision measurements (±0.1‰ for δ¹⁸O and ±1‰ for δ²H), referenced to the VSMOW standard (Eq. 9). Dissolved silica (SiO₂) concentrations were also determined via ICP-MS, providing insights into silicate weathering intensity. For geochemical classification and process elucidation, Aquachem v4.0 was utilized to generate Piper, facilitating facies classification and interpretation of dominant hydrogeochemical processes such as rock-water interaction and evaporation trends. Lastly, isotopic recharge patterns were modelled using the Local Meteoric Water Line (LMWL), derived from regression of δ¹⁸O–δ²H values in precipitation. Deuterium excess (d-excess = δ²H – 8δ¹⁸O) was calculated to infer evaporative influences and help reconstruct recharge processes in volcanic terrains

3.0. Results

The results revealed key findings regarding the hydrogeochemical characteristics of groundwater in the Mount Cameroon region (Table 1). Groundwater temperatures ranged from 23.10°C to 32.10°C, with an average of 27.69°C. The pH levels, ranging from 6.49 to 6.68, indicated slightly acidic to neutral conditions, falling within the WHO's acceptable range for drinking water. The electrical conductivity (EC) averaged 314.94 µS/cm, well below the WHO's recommended limit of 1500 µS/cm. Total dissolved solids (TDS) ranged from 102 mg/l to 211 mg/l, also within the WHO's permissible limit.

Major cations like sodium (Na+), potassium (K+), magnesium (Mg2+), and calcium (Ca2+) were analyzed, with concentrations varying across sampling locations. Similarly, the concentrations of major anions like chloride (Cl-), bicarbonate (HCO3-), nitrate (NO3-), and sulfate (SO42-) were assessed, all falling within the WHO's recommended limits for drinking water.

The groundwater samples analysed in this study revealed three predominant water types.

Figure 3: Piper diagram of groundwaters in the study area

This study delineates three major hydrogeochemical facies within the Eastern Mount Cameroon volcanic aquifer: Ca-Cl (74%), Ca-HCO₃ (20%), and Mixed Ca-Mg-Cl (6%) types. The dominance of the Ca-Cl facies highlights ion exchange and evaporite dissolution, often linked to deep, older groundwater with limited recharge. In contrast, the Ca-HCO₃ facies signifies active recharge and weathering of carbonate and silicate minerals, indicating fresher and more dynamic groundwater systems. The Mixed Ca-Mg-Cl type reflects transitional geochemical conditions, potentially influenced by saline-freshwater mixing, dolomite dissolution, or localized contamination. These facies provide valuable insights into groundwater evolution, flow paths, and water-rock interactions. Their identification is pivotal for managing salinity risks, sustaining recharge zones, and informing strategies for groundwater quality monitoring. This work underscores the importance of integrating hydrochemical classification with isotopic and geostatistical approaches to support sustainable water resource development in complex volcanic terrains.

The correlation analysis reveals a strong 1:1 relationship between Electrical Conductivity (EC) and Total Dissolved Solids (TDS), confirming that EC effectively reflects dissolved ion concentrations in groundwater. However, both EC and TDS exhibit weak correlations with major ions such as Ca²⁺, Na⁺, K⁺, Mg²⁺, HCO₃⁻, SO₄²⁻, NO₃⁻, and Cl⁻, suggesting that other sources—like silicate weathering, colloidal particles, or anthropogenic inputs—may contribute significantly to TDS. This underscores the geochemical complexity of the aquifer system, where factors like lithology, ionic strength, and non-ionic constituents shape solute concentrations. The findings advocate for further research into alternative TDS contributors and equilibrium processes influencing groundwater quality.

Table 2 Groundwater physicochemical parameter correlation matrix of the study area

|  | Ca2+ | Mg2+ | Na+ | K+ | HCO3 | CL- | SO2-4 | NO3- | SiO2 | TDS | T | PH | EC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ca2+ | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Mg2+ | 0.12 | 1.00 |  |  |  |  |  |  |  |  |  |  |  |
| Na+ | 0.46 | 0.10 | 1.00 |  |  |  |  |  |  |  |  |  |  |
| K+ | 0.85 | 0.14 | 0.48 | 1.00 |  |  |  |  |  |  |  |  |  |
| HCO3 | 0.21 | 0.03 | 0.00 | -0.1 | 1.00 |  |  |  |  |  |  |  |  |
| CL- | 0.87 | 0.12 | 0.42 | 0.76 | -0.04 | 1.00 |  |  |  |  |  |  |  |
| SO2-4 | 0.34 | 0.19 | 0.21 | 0.32 | 0.19 | 0.30 | 1.00 |  |  |  |  |  |  |
| NO3- | 0.54 | 0.01 | 0.33 | 0.35 | -0.14 | 0.58 | -0.21 | 1.00 |  |  |  |  |  |
| SiO2 | 0.18 | 0.01 | 0.34 | 0.20 | -0.04 | 0.11 | 0.44 | -0.1 | 1.00 |  |  |  |  |
| TDS | 0.09 | -0.03 | 0.02 | 0.06 | -0.05 | 0.08 | -0.26 | 0.36 | -0.1 | 1.00 |  |  |  |
| T | -0.1 | 0.08 | 0.00 | -0.1 | 0.18 | -0.1 | 0.26 | -0.4 | 0.03 | -0.6 | 1.00 |  |  |
| PH | 0.02 | 0.20 | -0.1 | 0.11 | 0.11 | -0.1 | -0.10 | 0.14 | 0.07 | 0.50 | -0.6 | 1.00 |  |
| EC | 0.09 | -0.03 | 0.02 | 0.05 | -0.05 | 0.08 | -0.25 | 0.35 | -0.1 | 1.00 | -0.6 | 0.50 | 1.00 |

Strong	  moderate

Correlations determined by binary graphs between concentrations of major elements (SO4 , Mg2+ , Cl, Ca2+ , Na+, and HCO3-) identify mechanisms and processes contributing to water sample mineralization. These correlation analyses provide insights into the relationships between various hydrochemical parameters, shedding light on the composition and quality of groundwater in the study area.

The study assessed the physicochemical parameters of groundwater and compared them to the World Health Organization's (WHO) standards for drinking water. The water samples exhibited the following characteristics: The water from springs, boreholes, and some deep open wells with metal casing was colourless. However, shallow and unprotected wells showed creamy and greenish colours, respectively, possibly due to suspended solids and eutrophication. The pH values ranged from 6.22 to 6.69, with an average of 6.49, indicating slightly acidic to neutral conditions, which are generally acceptable for drinking water (table 3). The study also compared the concentrations of various parameters (EC, TDS, HCO₃⁻, Cl⁻, SO₄²⁻, NO₃⁻, TH, Ca²⁺, Mg²⁺, Na⁺, and K⁺) to WHO and BIS (Bureau of Indian Standards) guidelines. All parameters were found to be within acceptable limits, suggesting that the groundwater is generally suitable for drinking purposes in terms of these parameters.

Table 3: Basic Statistics of the physicochemical parameters of groundwater

| Parameter | WHO (2023) | BIS (2012) | Study Area Samples |  |  |
| --- | --- | --- | --- | --- | --- |
| Water Quality | Acceptable | Acceptable limit | Minimum | Maximum | Mean |
|  | Range |  |  |  |  |
| pH(on scale) | 6.5-8.5 | 6.5-8.5 | 6.22 | 6.68 | 6.49 |
| EC (µs/cm) | - | - | 22 | 425 | 204.88 |
| Temp | - | - | 23.10 | 32.10 | 27.69 |
| TDS (mg/l) | 500 | 500 | 11 | 211 | 102.24 |
| HCO3(mg/l) | - | - | 0 | 147 | 16.99 |
| Cl(mg/l) | 250 | 250 | 0 | 32.6 | 10.05 |
| SO4(mg/l) | 250 | 200 | 0 | 20.13 | 4.54 |
| NO3(mg/l) | 45 | 45 | 0 | 12.9 | 5.07 |
| TH (mg/l) | 100 | 200 | 13.69 | 152.51 | 69.89 |
| Ca(mg/l) | 75 | 75 | 1.61 | 43.1 | 13.41 |
| Mg(mg/l) | 50 | 30 | 0 | 20.2 | 8.71 |
| Na(mg/l) | 200 | - | 0.23 | 6.44 | 2.39 |
| K(mg/l) | 12 | - | 0.37 | 15.2 | 4.75 |

The TH ranged from 13.69 mg/l to 152.51 mg/l CaCO3(table 4). The majority of the samples (44%) fell into the "soft" category, while 46% were classified as "moderately hard." The remaining 10% were classified as "hard." The hardness is primarily attributed to the presence of Mg²⁺ and Ca²⁺ ions.

Table 4: Groundwater Classification Based on Total Hardness (TH) after WHO (2023).

| (TH) as CaCO3(mg/l | Water type | No of water samples | Percentage of water samples |
| --- | --- | --- | --- |
| < 60 | Soft | 22 | 44% |
| 61-120 | Moderately Hard | 23 | 46 |
| 121-180 | Hard | 5 | 10 |
| >180 | Very Hard | 0 | 0 |

Trace element analysis across groundwater samples from Eastern Mount Cameroon reveals considerable spatial and compositional variability driven by both geological and anthropogenic influences. Iron (Fe) concentrations were notably elevated, averaging 2390 µg/L, with values ranging from 10 µg/L at Newlayout Muyuka (OW08) to a peak of 5310 µg/L at Koke (CAT2), reflecting extensive iron-bearing mineral dissolution. Manganese (Mn) concentrations varied similarly, from 10 µg/L at Koto III (OW14) to 3280 µg/L at Mile 18 Buea (CAT04), influenced by redox-sensitive geochemistry typical of volcanic aquifers. Aluminium (Al) levels, associated with silicate weathering, were measured between 15 µg/L (Koto III, OW14) and 72 µg/L (Ekona, SPG2). Arsenic (As) remained low across most sites, averaging 0.19 µg/L, but peaked at 0.62 µg/L near Mautu Cross River Quarter (SPG03), suggesting localized geogenic input. Lead (Pb) concentrations ranged from 0.02 µg/L to 1.56 µg/L, with the highest values also found at SPG03, possibly linked to lead-bearing mineral weathering or historical pollution. Vanadium (V) reached 20.5 µg/L at Camp 3 (OW17), while Cadmium (Cd) concentrations, though generally low, peaked at 0.51 µg/L at Checkpoint Molyko (BH03), indicating potential anthropogenic sources. Chromium (Cr) was detected up to 4.01 µg/L at Camp 1 (OW16), influenced by oxidation-reduction reactions or industrial input. Copper (Cu) and Nickel (Ni), typically from ultramafic sources, peaked at 3.11 µg/L (Bafia, SPH02) and 0.88 µg/L (CAT04) respectively. Zinc (Zn), which averaged 17.18 µg/L, showed highs of 44.1 µg/L at Bafia (SPG08) and lows of 7.5 µg/L at Bonduma Spring (SPG07).

Table 5: Trace Element Concentrations in Groundwater Samples

| Sr.No.ample ID | Fe(mg/l) | Mn(mg/l) | Al(mg/l) | As(µg/l) | Cd(µg/l) | Cr(µg/l) | Cu(µg/l) | Ni(µg/l) | Pb(µg/l) | V(µg/l) | Zn(µg/l) | Co(µg/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.03 | 0.43 | 0.02 | 0.04 | 0.06 | 0.70 | 1.80 | 0.60 | 0.75 | 4.20 | 8.50 | 0.07 |
| 2 | 0.30 | 0.19 | 0.01 | 0.15 | 0.02 | 1.30 | 1.30 | 0.72 | 0.32 | 9.45 | 19.30 | 0.04 |
| 3 | 0.02 | 0.21 | 0.03 | 0.17 | 0.12 | 1.12 | 1.22 | 0.75 | 0.41 | 10.60 | 44.10 | 0.08 |
| 4 | 3.03 | 0.31 | 0.01 | 0.11 | 0.01 | 1.11 | 0.84 | 0.43 | 0.51 | 14.20 | 32.50 | 0.04 |
| 5 | 0.35 | 0.28 | 0.01 | 0.09 | 0.01 | 0.90 | 1.30 | 0.51 | 0.18 | 7.20 | 9.24 | 0.14 |
| 6 | 2.04 | 0.52 | 0.01 | 0.21 | 0.02 | 1.10 | 1.23 | 0.62 | 0.21 | 13.50 | 13.70 | 0.05 |
| 7 | 3.70 | 0.59 | 0.01 | 0.15 | 0.03 | 1.20 | 1.11 | 0.65 | 0.28 | 14.70 | 18.20 | 0.07 |
| 8 | 5.23 | 0.19 | 0.01 | 0.22 | 0.02 | 1.70 | 1.29 | 0.42 | 0.26 | 17.30 | 15.30 | 0.11 |
| 9 | 0.01 | 0.14 | 0.01 | 0.19 | 0.01 | 1.00 | 1.81 | 0.51 | 0.71 | 13.80 | 17.80 | 0.04 |
| 10 | 0.31 | 0.14 | 0.01 | 0.16 | 0.01 | 1.10 | 1.70 | 0.07 | 0.02 | 15.20 | 26.40 | 0.03 |
| 11 | 3.43 | 0.17 | 0.01 | 0.14 | 0.03 | 0.78 | 0.91 | 0.56 | 1.25 | 11.20 | 18.20 | 0.51 |
| 12 | 0.35 | 0.51 | 0.04 | 0.35 | 0.05 | 0.89 | 0.89 | 0.62 | 0.51 | 13.20 | 18.90 | 0.09 |
| 13 | 4.20 | 0.47 | 0.07 | 0.14 | 0.11 | 1.51 | 1.10 | 0.41 | 0.99 | 8.51 | 22.30 | 0.04 |
| 14 | 1.80 | 0.07 | 0.04 | 0.71 | 0.07 | 1.11 | 1.60 | 0.61 | 0.54 | 9.50 | 20.10 | 0.15 |
| 15 | 3.45 | 0.21 | 0.01 | 0.47 | 0.04 | 1.23 | 1.82 | 0.75 | 0.81 | 11.50 | 8.60 | 0.07 |
| 16 | 3.90 | 0.75 | 0.07 | 0.21 | 0.06 | 0.97 | 2.01 | 0.36 | 1.56 | 18.10 | 14.20 | 0.03 |
| 17 | 4.30 | 0.41 | 0.07 | 0.37 | 0.02 | 1.15 | 1.57 | 0.41 | 0.25 | 11.80 | 9.30 | 0.47 |
| 18 | 0.85 | 0.21 | 0.01 | 0.14 | 0.06 | 0.98 | 0.45 | 0.63 | 0.84 | 14.20 | 12.30 | 0.15 |
| 19 | 0.76 | 0.81 | 0.01 | 0.56 | 0.03 | 1.21 | 0.78 | 0.78 | 0.41 | 15.20 | 25.10 | 0.14 |
| 20 | 3.12 | 0.31 | 0.00 | 0.41 | 0.04 | 1.22 | 1.27 | 0.69 | 0.22 | 8.95 | 11.20 | 0.42 |
| 21 | 3.71 | 0.41 | Bdl | 0.19 | 0.24 | 0.18 | 0.48 | 0.03 | 0.03 | 8.21 | 18.20 | 1.09 |
| 22 | 3.14 | 0.01 | Bdl | 0.09 | 0.04 | 1.06 | 1.85 | 0.13 | 0.03 | 6.81 | 14.30 | 0.14 |
| 23 | 2.51 | 0.04 | 0.01 | 0.15 | 0.04 | 1.45 | 2.31 | 0.23 | 0.03 | 6.81 | 17.10 | 0.28 |
| 24 | 0.28 | 0.11 | 0.01 | 0.61 | 0.15 | 0.13 | 2.61 | 0.16 | 0.03 | 6.91 | 24.40 | 0.85 |
| 25 | 1.91 | 0.11 | 0.01 | 0.07 | 0.01 | 0.96 | 1.76 | 0.18 | 0.02 | 14.50 | 19.20 | 0.13 |
| 26 | 1.54 | 0.31 | 0.01 | 0.06 | 0.06 | 0.51 | 1.81 | 0.24 | 0.03 | 20.50 | 18.50 | 0.79 |
| 27 | 4.31 | 0.33 | 0.00 | 0.07 | 0.05 | 1.21 | 0.64 | 0.37 | 0.54 | 19.80 | 21.30 | 0.24 |
| 28 | 2.73 | 2.04 | 0.03 | 0.08 | 0.01 | 1.05 | 1.81 | 0.15 | 0.03 | 13.80 | 20.50 | 0.05 |
| 29 | 5.31 | 0.63 | 0.01 | 0.13 | 0.02 | 1.05 | 0.21 | 0.16 | 0.03 | 9.89 | 8.06 | 0.05 |
| 30 | 3.21 | 0.25 | 0.01 | 0.21 | 0.03 | 1.08 | 1.86 | 0.15 | 0.02 | 11.90 | 14.20 | 0.03 |
| 31 | 2.21 | 0.28 | 0.01 | 0.02 | 0.12 | 1.96 | 0.01 | 0.21 | 0.02 | 1.51 | 15.90 | 0.12 |
| 32 | 4.41 | 1.27 | 0.01 | 0.11 | 0.51 | 1.08 | 1.32 | 0.59 | 0.02 | 12.60 | 14.60 | 0.09 |
| 33 | 3.42 | 0.19 | 0.01 | 0.07 | 0.02 | 1.31 | 0.08 | 0.54 | 0.49 | 13.50 | 18.20 | 0.45 |
| 34 | 4.21 | 0.21 | 0.01 | 0.08 | 0.12 | 1.23 | 1.23 | 0.31 | 0.57 | 14.70 | 18.90 | 0.29 |
| 35 | 3.12 | 0.31 | 0.01 | 0.13 | 0.01 | 1.02 | 1.75 | 0.53 | 0.83 | 17.30 | 22.30 | 0.39 |
| 36 | 0.76 | 3.28 | 0.01 | 0.05 | 0.01 | 1.21 | 0.95 | 0.88 | 1.16 | 14.80 | 21.10 | 0.74 |
| 37 | 1.56 | 0.52 | 0.00 | 0.53 | 0.02 | 1.09 | 0.23 | 0.59 | 0.85 | 11.20 | 8.70 | 0.37 |
| 38 | 4.21 | 0.59 | 0.03 | 0.15 | 0.03 | 1.11 | 0.75 | 0.04 | 0.64 | 15.80 | 11.20 | 0.24 |
| 39 | 4.02 | 0.19 | 0.01 | 0.28 | 0.02 | 1.02 | 1.91 | 0.03 | 0.31 | 11.30 | 9.90 | 0.14 |
| 40 | 2.35 | 0.14 | 0.00 | 0.09 | 0.01 | 1.25 | 0.45 | 0.26 | 0.52 | 8.11 | 11.30 | 0.36 |
| 41 | 2.91 | 0.14 | 0.03 | 0.08 | 0.13 | 0.91 | 0.61 | 0.46 | 0.13 | 10.50 | 23.10 | 0.04 |
| 42 | 3.41 | 1.17 | 0.01 | 0.08 | 0.13 | 1.01 | 1.54 | 0.58 | 0.33 | 10.30 | 17.20 | 0.35 |
| 43 | 0.81 | 0.47 | 0.01 | 0.06 | 0.07 | 1.11 | 1.52 | 0.29 | 0.25 | 11.30 | 11.20 | 0.46 |
| 44 | 0.32 | 0.11 | 0.00 | 0.08 | 0.07 | 1.18 | 0.64 | 0.15 | 0.51 | 10.60 | 8.90 | 0.14 |
| 45 | 3.61 | 0.31 | 0.03 | 0.03 | 0.01 | 1.56 | 1.71 | 0.16 | 0.32 | 12.60 | 7.50 | 0.15 |
| 46 | 4.20 | 0.43 | 0.01 | 0.21 | 0.05 | 1.28 | 0.31 | 0.15 | 0.13 | 11.50 | 17.60 | 0.05 |
| 47 | 0.03 | 0.05 | 0.01 | 0.05 | 0.02 | 1.91 | 1.56 | 0.21 | 0.04 | 14.70 | 14.50 | 0.13 |
| 48 | 0.35 | 1.32 | 0.01 | 0.51 | 0.02 | 4.01 | 3.11 | 0.59 | 0.03 | 14.30 | 36.04 | 5.01 |
| 49 | 1.24 | 0.15 | 0.00 | 0.06 | 0.13 | 1.12 | 1.52 | 0.54 | 0.11 | 19.80 | 18.30 | 0.25 |
| 50 | 2.41 | 0.18 | 0.00 | 0.05 | 0.02 | 1.61 | 0.18 | 0.31 | 0.02 | 11.20 | 11.70 | 0.16 |
| Min | 0.01 | 0.01 | 0.00 | 0.02 | 0.01 | 0.13 | 0.01 | 0.03 | 0.02 | 1.51 | 7.50 | 0.03 |
| Max | 5.31 | 3.28 | 0.07 | 0.71 | 0.51 | 4.01 | 3.11 | 0.88 | 1.56 | 20.50 | 44.10 | 5.01 |
| Mean | 2.40 | 0.50 | 0.02 | 0.19 | 0.07 | 1.21 | 1.27 | 0.41 | 0.40 | 12.14 | 17.51 | 0.41 |

The isotopic analysis of rainwater samples collected over 14 months in Buea reveals a distinct seasonal variation in both δ¹⁸O and δD values (Table 5). The weighted average values for δ¹⁸O (–3.84‰) and δD (–19.65‰) are indicative of a meteoric origin for the precipitation, consistent with the expected depletion in heavier isotopes due to fractionation processes during the global water cycle (Craig, 1961). The observed seasonal variability in isotopic signatures can be attributed to several factors, including changes in temperature, humidity, and the source of moisture (Dansgaard, 1964).

Stable isotopes of water such as oxygen-18 and hydrogen deuterium are crucial tools in hydrogeology for understanding water’s origins, movement, and behaviour across diverse systems. They provide insights into water cycle processes, groundwater recharge, and mechanisms like evaporation and mixing. In the present study area, a lack of existing isotopic data for rainfall hinders local baseline comparisons; however, most groundwater typically exhibits compositions that align with the characteristic meteoric water signature (i.e., rainwater), unless altered by evaporation or geologic reactions. When groundwater samples match this global signature, it strongly suggests direct recharge from rainfall (Jasechko, et al.,2014). Conversely, deviations from this pattern may indicate evaporation effects (Gallart et al., 2024) or input from deeper sources (Wotany et al., 2021; Tchakam Kamtchueng et al., 2022). A comparison of the isotope results from the current study with other investigations in Cameroon reveals that most groundwater largely conforms to the global meteoric water line, pointing to rapid recharge. Additionally, there is a trend toward lighter isotopic values further inland, away from coastal influences.

| Table 6a: Rain water isotope results |  |  |  |  |
| --- | --- | --- | --- | --- |
| Month | δ18 O (‰) | δD (‰) | d-excess (‰)) | Rainfall (mm) |
| July | -4.27 | -20.70 | 13.46 | 323.65 |
| August | -2.07 | -3.90 | 12.66 | 465.1 |
| September | -5.11 | -32.90 | 7.98 | 409.21 |
| October | 0.43 | 8.40 | 4.96 | 306.04 |
| November | -4.37 | -24.70 | 10.26 | 104.54 |
| January |  |  | 0 | 17.82 |
| February | 1.73 | 7.40 | -6.44 | 38.19 |
| March | -3.18 | -12.40 | 13.04 | 142.99 |
| April | -3.63 | -19.70 | 9.34 | 198.14 |
| May |  |  | 0 | 234.55 |
| June | -3.74 | -14.50 | 15.42 | 324.62 |
| July | -3.72 | -14.40 | 15.36 | 323.65 |
| August | -5.42 | -32.60 | 10.76 | 465.1 |
| September | -5.46 | -32.90 | 10.78 | 409.21 |
| October | -5.45 | -32.80 | 10.8 | 306.04 |
| November | -3.34 | -15.60 | 11.12 | 104.54 |
| December |  |  | 0 | 22.95 |

Table 6b: groundwater isotope data

| place | 18O(‰) | 2D(‰) | GMWL |
| --- | --- | --- | --- |
| stranger quarter | -2.66 | -9.71 | -11.28 |
| MALENDE | -2.59 | -9.33 | -10.72 |
| penda mboko | -2.25 | -6.13 | -8 |
| YOKE | -2.71 | -9.55 | -11.68 |
| MBO QUARTER,YOKE | -2.55 | -8.32 | -10.4 |
| MALENDE candef mosack quarter | -2.98 | -10.15 | -13.84 |
| MALENDE | -2.55 | -9.13 | -10.4 |
| YOKE nangi street | -3.2 | -10.15 | -15.6 |
| new layout muyuka | -2.91 | -9.33 | -13.28 |
| Akawuh quarter | -3.01 | -11.31 | -14.08 |
| kwe kwe | -2.58 | -9.61 | -10.64 |
| kwe kwe | -3 | -10.21 | -14 |
| ekona corner road | -3.11 | -9.52 | -14.88 |
| mautu cross river | -3.11 | -10.81 | -14.88 |
| mautu unity quatrter | -3.01 | -9.53 | -14.08 |
| Mautu | -3.13 | -13.51 | -15.04 |
| MUEA ,WONYA MEVEO | -2.95 | -15.11 | -13.6 |
| YOKE nangi street | -3.21 | -12.2 | -15.68 |
| Owe | -2.57 | -12.72 | -10.56 |
| Ikata | -3.06 | -12.66 | -14.48 |
| Matouke | -2.95 | -10.52 | -13.6 |
| koto 111 | -2.75 | -10.33 | -12 |
| penda mboko | -2.83 | -12.76 | -12.64 |
| camp 1 | -3.33 | -10.31 | -16.64 |
| bone quarter penda mboko | -3.42 | -11.38 | -17.36 |
| camp 3 | -3.12 | -12.75 | -14.96 |
| camp2 | -3.36 | -10.31 | -16.88 |
| bone quarter penda mboko | -3.25 | -10.31 | -16 |
| Koke | -3.11 | -11.23 | -14.88 |
| Lysoka | -3.21 | -13.11 | -15.68 |
| Lyongo | -2.63 | -11.55 | -11.04 |
| check point Buea | -3.34 | -15.99 | -16.72 |
| Muea | -2.82 | -13.11 | -12.56 |
| Muea | -3.32 | -11.66 | -16.56 |
| matouke 2 | -2.63 | -16.15 | -11.04 |
| mile 18 | -3.21 | -12.37 | -15.68 |
| yoke king bar | -2.81 | -13.23 | -12.48 |
| Buea town | -3.91 | -12.35 | -21.28 |
| muyuka 7 | -2.14 | -14.63 | -7.12 |
| Slaughter house Buea town | -1.91 | -10.31 | -5.28 |
| Great Soppo | -3.78 | -13.12 | -20.24 |
| Bosta | -3.69 | -10.23 | -19.52 |
| Bafia | -1.24 | -13.51 | 0.08 |
| malende 3 | -3.01 | -8.35 | -14.08 |
| Bonduma | -2.63 | -10.33 | -11.04 |
| BOSTA | -2.31 | -11.33 | -8.48 |
| yoke 3 | -2.77 | -12.98 | -12.16 |
| BAFIA 2 | -2.92 | -12.52 | -13.36 |
| Sandpit | -2.55 | -12.03 | -10.4 |
| old police road muyuka | -2.91 | -11.67 | -13.28 |

Table 6c summary of the isotope statistics for both rainwater and groundwater:

| Isotope | Water Type | Mean | Median | Range | Standard Deviation |
| --- | --- | --- | --- | --- | --- |
| δ18 O (‰) | Rainwater | -3.40 | -3.73 | 7.19 | 2.15 |
| δD (‰) | Rainwater | -17.24 | -17.65 | 41.30 | 13.94 |
| d-excess (‰)) | Rainwater | 9.96 | 10.79 | 21.86 | 5.48 |
| δ18 O (‰) | Groundwater | -2.90 | -2.95 | 2.67 | 0.46 |
| δD (‰) | Groundwater | -11.39 | -11.32 | 10.02 | 1.99 |

Fig. 4: Traditional d18O–dD associations of groundwater in the study region.

Fig 5. Plot of δ18O and total dissolve solid (TDS) in ground and surface water sources

Figure 5 Shows increasing TDS with higher δ¹⁸O, suggesting evaporation leads to increased mineralization. Geological processes may also contribute to TDS

Figure 6: Seasonal Variation in Isotopic Signatures of Rainwater.

Figure 6, Illustrates an inverse relationship between rainfall and d-excess, suggesting heavier rainfall means less isotopic fractionation. Provides insights into rainfall and isotopic signatures.

δ 18O

Cl-(mg/g

Fig7: Relationship between the δ 18O and Cl− content in the groundwater samples analysed

Figure 7 depicts a positive correlation between δ¹⁸O and Cl⁻, indicating evaporation concentrates chloride ions. Other factors may also influence chloride content.

The meteorological data collected from 1991 to 2020 in the South-West region of Cameroon revealed distinct temperature and precipitation patterns. The average minimum surface air temperature ranged from 18.58°C in January to 20.19°C in March, while the average mean surface air temperature varied between 22.17°C in August and 24.96°C in March. The average maximum surface air temperature exhibited a wider range, from 25.25°C in August to 30.20°C in February. These temperature fluctuations likely reflect the region's tropical highland climate, with cooler temperatures during the wet season and slightly warmer temperatures during the dry season.

The precipitation pattern displayed a bimodal distribution, with peaks in June (324.62 mm) and September (409.21 mm). The lowest precipitation occurred in December (22.95 mm) and January (17.82 mm). This bimodal pattern is characteristic of tropical regions with two distinct rainy seasons. The high precipitation during the wet season is likely attributed to the influence of the Intertropical Convergence Zone (ITCZ) and orographic uplift caused by Mount Cameroon.

Table 7: The average mean monthly meteorological data collected from 1991 to 2020 for the south west Region, Cameroon

| Category | Average Minimum Surface Air Temperature | Average Mean Surface Air Temperature | Average Maximum Surface Air Temperature | Precipitation |
| --- | --- | --- | --- | --- |
| Jan | 18.58 | 23.83 | 29.12 | 17.82 |
| Feb | 19.66 | 24.91 | 30.20 | 38.19 |
| Mar | 20.19 | 24.96 | 29.78 | 142.99 |
| Apr | 20.07 | 24.91 | 29.79 | 198.14 |
| May | 19.96 | 24.14 | 28.36 | 234.55 |
| Jun | 19.58 | 23.35 | 27.18 | 324.62 |
| Jul | 19.32 | 22.43 | 25.59 | 323.65 |
| Aug | 19.14 | 22.17 | 25.25 | 465.10 |
| Sep | 19.05 | 22.68 | 26.36 | 409.21 |
| Oct | 19.35 | 23.20 | 27.09 | 306.04 |
| Nov | 19.38 | 23.71 | 28.08 | 104.54 |
| Dec | 18.72 | 23.61 | 28.56 | 22.95 |

Figure 8: illustrates the annual precipitation trends in the South-West region of Cameroon from 1951 to 2020.

The annual precipitation is depicted by a grey line, while the three colour lines represent different trend lines:

The analysis of annual precipitation trends in the South-West region of Cameroon from 1951 to 2020 revealed high variability(fig 8). However, trend lines suggest a slight increase in annual precipitation over time, particularly pronounced in the more recent periods (1971-2020 and 1991-2020). The monthly climatology from 1991 to 2020 showed that the average minimum and mean temperatures remained relatively stable throughout the year, while the average maximum temperature fluctuated more significantly. Precipitation exhibited a bimodal pattern, with peaks in June and September, and the driest months being December and January.

4.0 Discussion

The hydrogeochemical characteristics of groundwater in the eastern Mount Cameroon region reflect a complex interplay of natural processes, anthropogenic influences, and potential climate change impacts. This discussion synthesizes the observed data and integrates relevant literature to elucidate the key factors shaping groundwater quality and evolution in this tropical volcanic environment.

physicochemical parameters in groundwater from Eastern Mount Cameroon, reveals temperatures (23.1–32.1°C, mean: 27.7°C) align with ambient shallow tropical aquifer conditions rather than geothermal systems (Zhao et al., 2022; Abesser et al., 2023; Pazola et al., 2024). A narrow pH range (6.49–6.68) suggests slightly acidic conditions driven by silicate weathering and biogenic CO₂ infiltration (Alsalam, 2020; Lewoyehu et al., 2024). The absence of strong carbonate buffering reflects limited calcite/dolomite, similar to findings from the Amazon Basin (Stallard, 1995; Jahan et al., 2023).

Groundwater is generally low in mineralization (EC: 314.94 µS/cm; TDS: 76.2–211 mg/L), with EC strongly correlated to TDS (r² = 0.96), validating its use as a rapid proxy (Valiallahi & Yazdani, 2025). Higher ionic loads in volcanic springs suggest intensified basalt weathering enhanced by CO₂, consistent with global analogues (Kitessa et al., 2024). Major cation trends show Ca²⁺ dominance from plagioclase weathering (Catania, 2022); Mg²⁺ enrichment in volcanic zones from olivine and pyroxene breakdown (Brennis, 2023) and localized Na⁺ and K⁺ increases from feldspar and biotite dissolution (Gudbrandsson, 2014; Emvoutou et al., 2018; Fonteh et al., 2017).

Anion distribution supports meteoric origins (Cl⁻ mean: 10.01 mg/L; Br/Cl <0.001) and confirms CO₂-mediated silicate weathering via elevated HCO₃⁻ (SPG5: 147 mg/L) (Freeman, 2007; Kitessa et al., 2024). Nitrate (mean: 5.07 mg/L) and low SO₄²⁻ indicate moderate agricultural influence without pyrite oxidation (Ako et al., 2014). Fluoride exceeds WHO limits (2.82 mg/L at SPG04), raising public health concerns tied to volcanic tuffs (Kisaka, 2023). Low bromide levels (mean: 0.14 mg/L) exclude marine sources (Eggenkamp, 2014). Silica concentrations (5.8–58.7 mg/L) reflect equilibrium with chalcedony, driven by basalt weathering and sediment interaction in alluvial zones (Berner & Berner, 1997; Kump et al., 2000; Zhang et al., 2021; Sarath et al., 2022; Ngai et al., 2024).

Hydrogeochemical facies are dominated by Ca-Cl (74%), representing evolved, saline-rich waters shaped by silicate weathering, ion exchange, and potential halite dissolution (Sunkari et al., 2021; Ahmed et al., 2024). Agricultural inputs amplify salinity concerns (Islam et al., 2021; Alqarawy et al., 2023; Boualem & Egbueri, 2024). Ca-HCO₃ facies (20%) occur in recharge zones, indicating carbonate dissolution and lower anthropogenic impact (Ngai et al., 2023; Meles et al., 2024; Mukherjee et al., 2024; Alarcón-Herrera et al., 2020). Mixed Ca-Mg-Cl waters (6%) near fault zones suggest structural control, deep mixing, and Mg-rich mineral dissolution (Medici et al., 2021; Carrasco-Núñez et al., 2021; Mussa & Mjemah, 2023). The study advocates targeted water resource strategies, including proactive monitoring in Ca-Cl zones, buffer zone designation near faults, and public health advisories in areas with elevated fluoride and chloride (WHO, 2023;Egbueri et al., 2025; Masoud et al., 2024). These insights position hydrogeochemical assessment as a cornerstone of sustainable groundwater management in volcanic terrains. The facies distribution mirrors global volcanic aquifer systems, where basalt weathering and anthropogenic drivers shape hydrochemical signatures (Abesser & Smedley,2008; Wang et al.,2025 ). Similar Ca-Cl dominance is documented in the Canary Islands (Cruz-Fuentes  et al.,2014; Jiménez et al.,2024) and Java, Indonesia (Setiawan et al.,2020; Taat et al.,2020). The limited mixed facies, however, contrasts with extensive mixing zones in sedimentary basins (Dorsey & Kidwell, 1999; Chiarella et al.,2017), emphasizing the role of fracture anisotropy in volcanic settings.

Groundwater in Eastern Mount Cameroon exhibits elevated iron (Fe) and manganese (Mn) levels, characteristic of volcanic aquifers globally. The mean Fe concentration of 2.40 mg/L aligns with values from Cameroon’s Mayo Tsanaga Basin, where reductive dissolution under anaerobic conditions exceeds 4 mg/L (Fantong et al., 2010). Mn levels (mean: 0.50 mg/L) are comparatively lower than those in Mexico’s Tenancingo Valley (Morales-Arredondo et al., 2016; Osuna-Martínez et al., 2021) but still reflect the influence of redox-driven mobilization. Regionally, Fe levels above 5 mg/L in Nigeria’s Basement Complex aquifers result from biotite and amphibole weathering (Adabanija et al., 2020), and Ghana’s Volta Basin reports Mn levels up to 2.5 mg/L from laterite weathering (Lartsey, 2023). Globally, Bangladesh’s Ganges Delta reports Fe averages of 4–8 mg/L due to widespread reductive processes (Ahmed, 2016; Biswas et al., 2023), while volcanic aquifers in Italy maintain Fe below 1 mg/L under oxidative conditions (Parisi et al., 2011; Cuoco et al., 2021). Arsenic (As) in the study area averages 0.19 µg/L—far below concentrations found in China’s Yangtze River Basin and Burkina Faso’s Sahelian aquifers, where industrial activity and evaporative enrichment drive levels up to 25 µg/L (Chen et al., 2024;Sako & Kafando, 2021). This contrast highlights the role of oxidative volcanic conditions in immobilizing As within Fe/Mn oxides or sulphides. In contrast, Italy’s geothermal volcanic zones like Viterbo exhibit As >10 µg/L from rock dissolution and thermal influence (Parisi et al., 2011). Cadmium (Cd) and lead (Pb) concentrations remain low (0.05 µg/L and 0.38 µg/L, respectively), indicating minimal anthropogenic impact. These values are well below those recorded in China’s Yangtze Basin (up to 5 µg/L Cd from mining; Chen et al., 2024) and the Zijiang Basin (up to 1.2 µg/L Cd from fertilizers; Liu et al., 2024). Similarly low levels are seen in Costa Rica’s Central Pacific aquifers (<0.1 µg/L), despite agricultural presence (Kanellopoulos et al., 2020; Kidder, 2020). Such comparisons underscore the study area’s relative geochemical integrity, shaped primarily by lithology and natural redox conditions rather than anthropogenic sources (Zhang et al., 2024; Wu et al., 2025).

Stable isotope (δ¹⁸O, δD) data with hydrochemical and climatological indicators to unravel recharge mechanisms in the volcanic and sedimentary aquifers of Mount Cameroon’s eastern flank. Rainfall isotopes (δ¹⁸O = -3.84 ± 2.15‰, δD = -19.65 ± 13.94‰) reflect Atlantic moisture sources shaped by orographic uplift and convective dynamics (Aggarwal et al., 2016; Gonfiantini et al., 2018; Antunes et al., 2019; dos Santos et al., 2024). Seasonal δ¹⁸O variability and high d-excess values (mean: 9.96‰) capture the tropical amount effect and sub-cloud evaporation processes (Dansgaard, 1964; Edirisinghe et al., 2017; Ansari et al., 2020;Chakraborty et al., 2025). Groundwater shows narrower isotopic ranges (δ¹⁸O = -2.90 ± 0.46‰; δD = -11.39 ± 1.99‰), indicative of aquifer homogenization during recharge and storage (Clark & Fritz, 1997; Séraphin et al., 2016; Hu et al., 2020; Raghavan et al., 2024). Clustering along the Local Meteoric Water Line (LMWL) confirms meteoric origins without evaporative enrichment (Wanke et al., 2018; Putman et al., 2019; Smith et al., 2021). Slight enrichment in δ¹⁸O relative to precipitation suggests preferential recharge during moderate rainfall, typical of fractured volcanic systems (Bowen, 2012; Bretzler et al., 2019, Eissa, 2024 ). Sedimentary aquifer waters (e.g., BH1: δ¹⁸O = -2.25‰) exhibit marginal enrichment over volcanic springs (e.g., SPG5: δ¹⁸O = -3.01‰), likely reflecting longer residence times and silicate weathering (Fenta, 2022). These findings, placed within global hydrogeological and tropical climate frameworks, highlight the critical role of rainfall seasonality, lithology, and fracture networks in controlling recharge behavior across complex aquifer systems. Multi-scale analysis of stable isotopes (δ¹⁸O, δD) in the Mount Cameroon aquifer system, revealing key processes driving recharge, evaporation, and hydrogeochemical evolution. The derived Buea Meteoric Water Line (BMWL: δD = 6.22δ¹⁸O + 3.90; R² = 0.94) deviates from the Global Meteoric Water Line (GMWL), indicative of kinetic fractionation under high tropical humidity (Dansgaard, 1964; Gat, 2000;Saranya et al., 2021; Balagizi et al., 2018). Groundwater samples tightly follow the Local Meteoric Water Line (LMWL) with no significant evaporative signatures, confirming rapid infiltration through fractured basalts (Abdelfadel et al., 2022; Arrakhais et al., 2024). Monte Carlo simulations support the dominance of meteoric recharge pathways (Yenehun et al., 2022; Jiménez et al., 2024). Numerical modeling (PHREEQC) estimates residence times of 10–15 years in volcanic springs and 50–80 years in sedimentary boreholes, consistent with global volcanic systems (Maloszewski & Zuber, 1996; Priestley et al., 2019). Isotope mass balance (SIMBAL) suggests deforestation reduces recharge by up to 30%, mirroring Amazonian trends (Merten, 2021; Bagheri, 2023). Comparisons with Germany and Costa Rica (Sánchez-Murillo et al., 2016) reveal isotopic convergence due to Atlantic-Pacific moisture sources, while Mount Cameroon’s higher d-excess (9.96‰) reflects strong orographic effects and convective recycling (Rhodes et al., 2006, 2010; DuMont, 2014). The BMWL’s lower slope, compared to the Douala line (Wirmvem et al., 2017), highlights regional climatic and altitudinal controls. CMIP6 climate models predict 10–20% rainfall reductions by 2100, posing threats to recharge timing and groundwater sustainability (Chen & Ford, 2023;Sharifan et al., 2025). Aquifers with thin unsaturated zones (<15 m) and rapid recharge (<5 years) are vulnerable to nitrate and pesticide infiltration (Custodio et al., 1983; Jannat et al., 2022), while sedimentary aquifers risk legacy contamination from metals like Cr and Co. δ¹⁸O-TDS relationships suggest recharge dilution with minimal dissolved mineral input from volcanic rocks (Emvoutou et al., 2018; Bretzler et al., 2019; Ngai et al., 2024). Seasonal d-excess fluctuations reveal humidity-driven sub-cloud evaporation (Parkes et al., 2017;Pfahl & Sodemann, 2014). The δ¹⁸O-Cl⁻ inverse correlation reflects mixing, evaporation, and water-rock interactions (Mahlknecht et al., 2017; Lapworth et al., 2018). Overall, isotopic tools inform recharge mapping, aquifer vulnerability, and hydroclimatic reconstruction (Gonfiantini, 1986; Elliot & Bonotto, 2017; Chidambaram et al., 2022; Putman et al., 2019)

5 Conclusions

The study found that the groundwater and surface water systems are generally of good quality, characterized by low electrical conductivity (EC), low total dissolved solids (TDS), and near-neutral pH values. Water temperature ranged between 23.1–32.1 °C, and pH was typically slightly acidic to neutral (6.4–6.8), reflecting the humid tropical climate and volcanic lithology of the region. The water chemistry was dominated by calcium (Ca²⁺), magnesium (Mg²⁺), and bicarbonate (HCO₃⁻), placing the waters predominantly in the Ca–Cl and Ca–HCO₃ facies. These findings align with previous hydrochemical studies in volcanic terrains across Cameroon and West Africa confirming the influence of water-rock interactions, particularly silicate weathering, on groundwater composition The trace element analysis indicated generally low levels of potentially harmful constituents such as fluoride, nitrate, and ammonium. However, elevated fluoride concentrations (up to 2.82 mg/L) in a few spring samples exceeded WHO guidelines, suggesting localized geogenic enrichment, possibly linked to fluoride-bearing volcanic minerals.  Stable isotope data (δ¹⁸O and δ²H) demonstrated that the groundwater has a predominantly meteoric origin. Groundwater samples clustered closely around the Local Meteoric Water Line (LMWL), indicating direct recharge from precipitation with minimal evaporative enrichment. The narrow isotopic variation among groundwater samples suggests a relatively homogeneous recharge system with efficient infiltration through fractured basalt and scoria layers. The seasonal δ¹⁸O signal in rainfall was dampened in groundwater, reinforcing the hypothesis of buffered recharge and moderate residence times. These insights provide compelling evidence for the dominant recharge processes and the influence of orographic rainfall patterns in the region. The LMWL established for the eastern Mount Cameroon area (δD = 6.22 δ¹⁸O + 3.90) diverges from the Global Meteoric Water Line (GMWL), reflecting the local climate’s impact on isotopic composition. The reduced slope and intercept are indicative of tropical convective precipitation patterns and the so-called "amount effect." Monthly rainfall isotopes exhibited clear seasonal trends, but groundwater showed muted isotopic variability, suggesting effective recharge during the wet season and limited isotope fractionation due to evaporation. These results enhance the understanding of hydrological processes in tropical volcanic settings and provide a reference baseline for future climate and recharge modelling.

Declaration

Ethics approval and consent to participate

All procedures were performed in accordance with the ethical standards of the institutional research committee.

Consent for publication

Not applicable

Data Availability Statement: The data presented in this study are available on request from the corresponding author. The data are not publicly available due to the restrictions of the project term

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Ethical Consideration

All procedures followed ethical standards of the institutional research committee

References

- .Abesser, C., & Smedley, P. L. (2008). Groundwater chemistry in a basalt aquifer in Central Mexico. Geological Society of London Special Publications

- Abdelfadel, F., Fontaine, C., Hilali, M., Mhanna, R., Eloy, L., Tagma, T., ... & Albani, A. E. (2022). Hydrochemical and isotopic assessment of a stressed multilayer aquifer in the Moroccan eastern High Atlas. Sustainable Water Resources Management, 8(3), 76.

- Abdelkarim, B., Antunes, I. M. H. R., Abaab, N., & Agoubi, B. (2023). Modeling groundwater recharge mechanisms in semi-arid regions: integration of hydrochemical and isotopic data. Euro-Mediterranean Journal for Environmental Integration, 8(4), 893-905

- Abdul-Wahab, D., Asare, E. A., Wahi, R., Ngaini, Z., Klutse, N. A. B., & Asamoah, A. (2024). Deciphering groundwater pollution in the Lower Anayari Catchment: Insights from using δ2H, δ18O, PMF, and APCS-MLR receptor model. Environmental Science and Pollution Research, 31(18), 27099-27116.

- Adabanija, M. A., Afolabi, O. A., & Lawal, L. (2020). The influence of bedrocks on groundwater chemistry in a crystalline basement complex of southwestern Nigeria. Environmental Earth Sciences, 79(4), 87.

- Adom, R. K., Simatele, M. D., & Reid, M. (2022). The threats of climate change on water and food security in South Africa. American Journal of Environment and Climate, 1(2), 6-10.

- Aggarwal, P. K., Froehlich, K., & Kulkarni, K. M. (2016). Environmental isotopes in groundwater studies. International Atomic Energy Agency.

- Ahmed, N. (2016). Saltwater Intrusion and Trace Element Contaminations at the Coastal Aquifers of the Ganges Delta (Master's thesis, Auburn University).

- Ako, A. A., Eyong, G. E. T., Shimada, J., Koike, K., Hosono, T., & Ichiyanagi, K. (2014). Nitrate contamination of groundwater in two areas of the Cameroon Volcanic Line (Banana Plain and Mount Cameroon area). Applied Water Science, 4, 99-113.

- Ako, A. A., Shimada, J., Hosono, T., Ichiyanagi, K., Nkeng, G. E., Fantong, W. Y., ... & Roger, N. N. (2011). Evaluation of groundwater quality and its suitability for drinking, domestic, and agricultural uses in the Banana Plain (Mbanga, Njombe, Penja) of the Cameroon Volcanic Line. Environmental Geochemistry and Health, 33, 559-575.

- Ako, A. A., Shimada, J., Hosono, T., Kagabu, M., & Nkeng, G. E. (2012). Hydrogeochemical and isotopic characteristics of groundwater in Mount Cameroon volcano. Journal of African Earth Sciences, 64, 1–13.

- Ako, A.A.; Eyong, G.E.T.; Shimada, J.; Koike, K.; Hosono, T.; Ichiyanagi, K.; Richard, A.; Tandia, B.K.; Nkeng, G.E.; Roger, N.N. Nitrate contamination of groundwater in two areas of the Cameroon Volcanic Line (Banana Plain and Mount Cameroon area). Appl. Water Sci. 2014, 4, 99–113.

- Akoachere, R. A., Egbe, S. E., Eyong, T. A., Yaya, O. O., & Mbua, R. L. (2019). Characterization of groundwater in the basaltic fractured rock aquiferous formations of the Limbe coastal region of Mount Cameroon, SWR-Cameroon. Journal of Environmental Science: Current Research, S1002.

- Akoachere, R. A., Etone, E. N., Mbua, R. L., Ngassam, M. P., Longonje, S. N., Oben, P. M., & Engome, R. W. (2019). Trace metals in groundwater of the south eastern piedmont region of Mount Cameroon: quantification and health risk assessment. Open Access Library Journal, 6(4), 1-22.

- Akoachere, R., Itie, M., Yaya, O., & Oteze, G. (2006). Morphometric analysis of the Piedmont regions of Mount Cameroon, West Africa. In J. Nig. Ass. Hydrogeologists (Ed.), Water Resource Planning: Manual of Water Supply Practices (p. 131). American Water Works Association.

- Alarcón-Herrera, M. T., Martin-Alarcon, D. A., Gutiérrez, M., Reynoso-Cuevas, L., Martín-Domínguez, A., Olmos-Márquez, M. A., & Bundschuh, J. (2020). Co-occurrence, possible origin, and health-risk assessment of arsenic and fluoride in drinking water sources in Mexico: Geographical data visualization. Science of the Total Environment, 698, 134168.

- Allen, S. T., Kirchner, J. W., & Goldsmith, G. R. (2019). Seasonal origins of soil water used by trees. Hydrology and Earth System Sciences, 23(2), 1199-1210

- Alsalam, O. T. A. (2020). Pedogenic evaluation and chemical weathering rates of soils developed along an altitudinal transect.

- Ansari, M. A., Noble, J., Deodhar, A., Mendhekar, G. N., & Jahan, D. (2020). Stable isotopic (δ18O and δ2H) and geospatial approach for evaluating extreme rainfall events. Global and Planetary Change, 194, 103299.

- Antunes, P., Boutt, D. F., & Rodrigues, F. C. (2019). Orographic distillation and spatio‐temporal variations of stable isotopes in precipitation in the North Atlantic. Hydrological Processes, 33(5), 775-793.

- Appelo, C. A. J., & Postma, D. (2005). Geochemistry, groundwater and pollution (2nd ed.). Balkema.

- Arrakhais, A. B., Hamit, A., Fontaine, C., Abdelfadel, F., Dinar, M., & Razack, M. (2024). Groundwater Dynamics in African Endorheic Basins in Arid to Semi-Arid Transition Zones: The Batha Aquifer System, NE Chad. Water, 16(14), 2067.

- Asaah, Asobo NE, Tetsuya Yokoyama, Festus T. Aka, Tomohiro Usui, Mengnjo J. Wirmvem, Boris Chako Tchamabe, Takeshi Ohba, Gregory Tanyileke, and J. V. Hell. "A comparative review of petrogenetic processes beneath the Cameroon Volcanic Line: Geochemical constraints." Geoscience Frontiers 6, no. 4 (2015): 557-570.

- Ayenew, T., Demlie, M., & Wohnlich, S. (2008). Hydrogeological framework and occurrence of groundwater in the Ethiopian aquifers. Journal of African Earth Sciences, 52(3), 97-113.

- Bagheri, O. (2023). Impacts of Climate Variability and Land Use Change on the Hydrology of the Amazon River Basin. Michigan State University.

- Balagizi, C. M., Kasereka, M. M., Cuoco, E., & Liotta, M. (2018). Influence of moisture source dynamics and weather patterns on stable isotopes ratios of precipitation in Central-Eastern Africa. Science of the Total Environment, 628, 1058-1078.

- Baud, B., Lachassagne, P., Dumont, M., Toulier, A., Hendrayana, H., Fadillah, A., & Dorfliger, N. (2024). Andesitic aquifers—hydrogeological conceptual models and insights relevant to applied hydrogeology. Hydrogeology Journal, 32(5), 1259-1286.

- Benedetti, M. F., Dia, A., Riotte, J., Chabaux, F., Gérard, M., Boulègue, J., ... & Ildefonse, P. (2003). Chemical weathering of basaltic lava flows undergoing extreme climatic conditions: the water geochemistry record. Chemical Geology, 201(1-2), 1-17.

- Berner, R. A., & Berner, E. K. (1997). Global environment: Water, air and geochemical cycles (2nd ed.). Prentice-Hal

- Besser, B., Kralik, M., & Humer, F. (2018). Hydrogeochemical evolution of thermal waters in the Eastern Alps: Insights from isotopic and chemical analyses. Journal of Hydrology, 561, 1–15. Retrieved from https://doi.org/10.1016/j.jhydrol.2018.03.015

- Billa, C. N. (2019). Biogeochemical Evaluation of Major Streams along the Eastern Flank of Mount Cameroon: Implications for Water Quality (Doctoral dissertation, Concordia University).

- Biswas, T., Pal, S. C., Saha, A., & Ruidas, D. (2023). Arsenic and fluoride exposure in drinking water caused human health risk in coastal groundwater aquifers. Environmental Research, 238, 117257

- Boualem, B., & Egbueri, J. C. (2024). Graphical, statistical and index-based techniques integrated for identifying the hydrochemical fingerprints and groundwater quality of In Salah, Algerian Sahara. Environmental Geochemistry and Health, 46(5), 158

- Bowen, R. (2012). Surface water. Springer Science & Business Media.

- Brennis, T., Lautze, N., Whittier, R., Torri, G., & Thomas, D. (2023). Understanding the origins of and influences on precipitation major ion chemistry on the Island of Oʻahu, Hawaiʻi. Environmental Monitoring and Assessment, 195(11), 1265.

- Bretzler, A., Osenbrück, K., & Gloaguen, R. (2019). Groundwater recharge in volcanic aquifers. Hydrogeology Journal, 27(1), 23–36. https://doi.org/10.1007/s10040-018-1837-8

- Bulourde, O. (2003). Chemical weathering and alteration of alkaline volcanic rocks in Cameroon. Journal of African Earth Sciences, 38(3–4), 281–293.

- Carrasco-Núñez, G., et al. (2021). Hydrogeochemical evolution of volcanic aquifers in the Trans-Mexican Volcanic Belt. Groundwater for Sustainable Development, 12, 100408. https://doi.org/10.1016/j.gsd.2020.100408

- Catania, S. (2022). Hydrogeochemical evolution and quality assessment of groundwater within a basalt aquifer system, Tamborine Mountain, Queensland (Doctoral dissertation, Queensland University of Technology).

- Chakraborty, S. K., & Chakraborty, S. K. (2021). Geo-hydrological perspectives of riverine flows. Riverine Ecology Volume 1: Eco-functionality of the Physical Environment of Rivers, 375-476.

- Chakraborty, S., Sarkar, A., Datye, A., Praveen, V., Deb Burman, P. K., Shivamurthy, Y., ... & Sharma, A. (2025). Precipitation isotopes and monsoon dynamics in the core monsoon zone of India. Scientific Reports, 15(1), 6761.

- Che, V. B., Kervyn, M., Suh, C. E., Fontijn, K., Ernst, G. G. J., Del Marmol, M. A., ... & Jacobs, P. (2012). Landslide susceptibility assessment in Limbe (SW Cameroon): A field calibrated seed cell and information value method. Catena, 92, 83-98.

- Chen, L., & Ford, T. W. (2023). Future changes in the transitions of monthly‐to‐seasonal precipitation extremes over the Midwest in Coupled Model Intercomparison Project Phase 6 models. International Journal of Climatology, 43(1), 255-274.

- Chen, X., Fu, X., Li, G., Zhang, J., Li, H., & Xie, F. (2024). Source-specific probabilistic health risk assessment of heavy metals in surface water of the Yangtze River Basin. Science of the Total Environment, 926, 171923.

- Chendjou, L. C. K., Kenfack, J. V., Koué, J. T., & Dongmo, A. K. (2024). Interpretation of Geological and Gravity Data from the Bamiléké Plateau (West-Cameroon): Implication for the Understanding of Its Underground Lithotectonic Geometry. Journal of Geoscience and Environment Protection, 12(9), 283-314.

- Cheunteu Fantah, C. A., Mezoue, C. A., Mouzong, M. P., Tokam Kamga, A. P., Nouayou, R., & Nguiya, S. (2022). Mapping of major tectonic lineaments across Cameroon using potential field data. Earth, Planets and Space, 74(1), 59.

- Chiarella, D., Longhitano, S. G., & Tropeano, M. (2017). Types of mixing and heterogeneities in siliciclastic-carbonate sediments. Marine and Petroleum Geology, 88, 617-627.

- Chidambaram, S., Panda, B., Keesari, T., Prasanna, M. V., Singh, D. K., & Ramanathan, A. L. (2022). Isotopic signatures to address the groundwater recharge in coastal aquifers. Marine Pollution Bulletin, 174, 113273.

- Clark, I. D., & Fritz, P. (1997). Environmental isotopes in hydrogeology. CRC Press.

- Cook, P. G. (2003). A guide to regional groundwater flow in fractured rock aquifers.

- Cook, P. G., & Herczeg, A. L. (Eds.). (2012). Environmental tracers in subsurface hydrology.

- Craig, H. (1961). Isotopic variations in meteoric waters. Science, 133(3465), 1702 1703. https://doi.org/10.1126/science.133.3465.1702

- Cruz-Fuentes, T., del Carmen Cabrera, M., Heredia, J., & Custodio, E. (2014). Groundwater salinity and hydrochemical processes in the volcano-sedimentary aquifer of La Aldea, Gran Canaria, Canary Islands, Spain. Science of the total environment, 484, 154-166.

- Cuoco, E., Viaroli, S., Paolucci, V., Mazza, R., & Tedesco, D. (2021). Fe and As geochemical self-removal dynamics in mineral waters: evidence from the Ferrarelle groundwater system (Riardo Plain, Southern Italy). Environmental Geochemistry and Health, 1-18.

- Custodio, E. (1989). Groundwater characteristics and problems in volcanic rock terrains. In Isotope techniques in the study of the hydrology of fractured and fissured rocks (pp. 87-137).

- Custodio, E., Guerra, J. A., Jimenez, J., Medina, J. A., & Soler, C. (1983). The effects of agriculture on the volcanic aquifers of the Canary Islands. Environmental Geology, 5(4), 225-231.

- Dansgaard, W. (1964). Stable isotopes in precipitation. Tellus, 16(4), 436–468. https://doi.org/10.3402/tellusa.v16i4.8993

- Das, K., Mukherjee, A., Malakar, P., Das, P., & Dey, U. (2021). Impact of global-scale hydroclimatic patterns on surface water-groundwater interactions in the climatically vulnerable Ganges river delta of the Sundarbans. Science of The Total Environment, 798, 149198.

- Defo C, Mishra AK, Yerima B, Mabou P (2016) Current conditions of groundwater resources development and related problems in the Republic of Cameroon, West Africa. Sustain. Water Resour. Manag. Accepted, 43–68

- Dorsey, R. J., & Kidwell, S. M. (1999). Mixed carbonate-siliciclastic sedimentation on a tectonically active margin. Geology, 27(4), 311-314. https://doi.org/10.1130/0091-7613(1999)027<0311

- DuMont, A. L. (2014). Hillslope hydrological processes in a Costa Rican rainforest: Water supply partitioning using isotope tracers (Doctoral dissertation).

- Edirisinghe, E. A. N. V., Pitawala, H. M. T. G. A., Dharmagunawardhane, H. A., & Wijayawardane, R. L. (2017). Spatial and temporal variation in the stable isotope composition (δ18O and δ2H) of rain across the tropical island of Sri Lanka. Isotopes in Environmental and Health Studies, 53(6), 628-645.

- Egbueri, J. C., Ikwuka, C. F., Agbasi, J. C., Nweke, N. D., Refadah, S. S., Nwazelibe, V. E., ... & Khan, M. Y. A. (2025). Contamination levels of water sources and the associated nitrate health risks to six age groups. Toxin Reviews, 44(1), 48-61.

- Egbueri, Johnbosco C., Chukwuma N. Mgbenu, and Chidinma N. Chukwu. "Investigating the hydrogeochemical processes and quality of water resources in Ojoto and environs using integrated classical methods." Modeling earth systems and environment 5, no. 4 (2019): 1443-1461.

- Eggenkamp, H. (2014). The geochemistry of stable chlorine and bromine isotopes (pp. 127-130). Berlin: Springer.

- Eid, M. H., Eissa, M., Mohamed, E. A., Ramadan, H. S., Czuppon, G., Kovács, A., & Szűcs, P. (2024). Application of stable isotopes, mixing models, and K-means cluster analysis to detect recharge and salinity origins in Siwa Oasis, Egypt. Groundwater for Sustainable Development, 25, 101124.

- Eissa , M. (2024). Geochemical and Isotopic Constraints on the Recharge and Salinization Origin of Groundwater Aquifers in El Rina–Nihayaat, South Sinai, Egypt. Asian Journal of Environment & Ecology, 23(3), 1–19. https://doi.org/10.9734/ajee/2024/v23i3529

- Elango, L., & Kannan, R. (2007). Rock–water interaction and its control on chemical composition of groundwater. Developments in environmental science, 5, 229-243.

- Elliot, T., & Bonotto, D. M. (2017). Hydrogeochemical and isotopic indicators of vulnerability and sustainability in the GAS aquifer, São Paulo State, Brazil. Journal of Hydrology: Regional Studies, 14, 130-149.

- Emvoutou, H. C., Ketchemen Tandia, B., Ngo Boum Nkot, S., Ebonji, R. C., Nlend, Y. B., Ekodeck, G. E., ... & Faye, S. (2018). Geologic factors controlling groundwater chemistry in the coastal aquifer system of Douala/Cameroon: implication for groundwater system functioning. Environmental earth sciences, 77, 1-23.

- Emvoutou, H. C., Ndomè, E. P. E., Diongue, D. M., Ndam, J. R., Stumpp, C., Ketchemen-T, B., ... & Faye, S. (2024). Hydrochemical and isotopic studies providing a new functional model for the coastal aquifers in Douala Coastal Sedimentary Basin (DCSB)/Cameroon. Science of the Total Environment, 912, 169412.

- Endeley, R. E., Ayonghe, S. Y., & Tchuenteu, F. (2001). A preliminary hydrochemical baseline study of water sources around Mount Cameroon. Journal of the Cameroon Academy of Sciences, 1(3), 161-168.

- Engome Regina Wotany, Ayuk Ruttan Agbor, Mengnjo Jude Wirmverm, Ayuk Valery Takang, Muga Yaguchi, Agyingi Christoper & EgbeClifford(2024). Major Ions and Trace Elements Characterization of Groundwater in Shallow Aquifers, Yaoundé Urban City, Cameroon. American Journal of Natural Sci. 5, Issue 2, pp 49 - 79, https://doi.org/10.47672/ajns.2254 4

- Engome Regina, W., Egbe Clifford, N., Agyingi, C., Ayuk Valery, T., Ayuk, R., Mengnjo Jude, W., & Ngai N, J. (2024). Hydrogeochemical Characterization and Water Quality Assessment in Manjo, Cameroon. Journal of Discoveries in Applied and Natural Science, 2(7), 9-30.

- Eyong, J. T., Gabriel, N., Moïse, B., Hell, V. J., Eric, B. E., Wignall, P., & Best, J. (2019). Sedimentologic and palaeoenvironmental evolution of the Mamfe Cretaceous Basin (SW Cameroon): evidence from lithofacies analysis, tectonics and evaporite minerals suite. Journal of African Earth Sciences, 149, 19-41.

- Fantong, W. Y., Satake, H., & Aka, F. T. (2010). Hydrogeochemical controls on groundwater in the Benue River Basin, Cameroon. Environmental Earth Sciences, 61(7), 1385–1397. https://doi.org/10.1007/s12665-010-0455-0

- Fantong, W. Y., Wirmvem, M. J., & Eyong, G. E. T. (2020). Hydrogeochemical and isotopic characterization of groundwater in the South West region of Cameroon. Journal of African Earth Sciences, 181, 104249.

- Fenta, M. C. (2022). Hydrogeophysical and hydrogeochemical characterization of volcanic aquifers in Northwest Ethiopia (Doctoral dissertation, Szegedi Tudomanyegyetem (Hungary)).

- Fitton, J. G., & Dunlop, H. M. (1985). The Cameroon line, West Africa, and its bearing on the origin of oceanic and continental alkali basalt. Earth and Planetary Science Letters, 72(1), 23-38.

- Fongoh EJ, Celle H, Nlend B, Boum-Nkot SN, Ako AA, Huneau F, Caillon N, Ntamak-Nida MJ (2023) Multitracer approach towards an improved understanding of shallow hard rock aquifers and a more sustainable groundwater management, case of Yaounde Cameroon. Environ Earth Sci 82:106. https://doi.org/10.1007/ s12665-023-10783-9

- Fonteh, M. L., Fonkou, T., Fonteh, M. F., Njoyim, E. B. T., & Lambi, C. M. (2017). Spatial variability and contamination levels of fresh water resources by saline intrusion in the coastal low lying areas of the Douala metropolis-Cameroon. Journal of Water Resource and Protection, 9(2), 215-237.

- Fontes, J. C., & Olivry, J. C. (1977). Composition isotopique des précipitations de la region du Mont Cameroun. ONAREST, Inst. de Recherches sur les Techniques, l’Industrie et le Soussol, 28.

- Forster, S., & Chilton, P. (2018). Permeability and transport properties of recent lava flows in the Cameroon Volcanic Line, West Africa. Geology, 46(5), 411–414. https://doi.org/10.1130/G34567.1

- Foster, T., Willetts, J., & Howard, G. (2020). Deforestation impacts on groundwater. Science of the Total Environment, 742, 140541. https://doi.org/10.1016/j.scitotenv.2020.140541

- FREEMAN, S. (2018). A Geochemical Study of the Suitability of Geologic Formations in the Southern San Joaquin Valley of California for Carbon Sequestration: Potential Water-rock Interactions and Effects of Injection of Supercritical CO₂ into the Stevens Sandstone, North Coles Levee and Elk Hills Oil Fields (Doctoral dissertation, California State University Bakersfield).

- Gallart, F., González-Fuentes, S., & Llorens, P. (2024). Isotopic fractionation of evaporating waters: effect of sub-daily atmospheric variations and eventual depletion of heavy isotopes. Hydrology and Earth System Sciences, 28(1), 229-239.

- Gat, J. R. (1996). Oxygen and hydrogen isotopes in the hydrologic cycle. Annual Review of Earth and Planetary Sciences, 24, 225-262.

- Gat, J. R. (2000). Atmospheric water balance—The isotopic perspective. Hydrological Processes, 14(8), 1357–1369. https://doi.org/10.1002/1099-1085(20000615)14:8<1357::AID-HYP986>3.0.CO;2-7

- Gonfiantini, R. (1986). Environmental isotopes in lake studies. In Handbook of Environmental Isotope Geochemistry (pp. 113–168). Elsevier.

- Gonfiantini, R., Roche, M. A., Olivry, J. C., & Fontes, J. C. (2018). Isotope hydrology of the Mbô plain, Cameroon. Journal of Hydrology, 36(1), 47–57.

- Gountié Dedzo, M., Tsozué, D., Kpoumié, A., & Nzeugang Nzeukou, A. (2023). Identification of major sources controlling groundwater geochemistry in Mount Makabaï in the Far-North of Cameroon (the northernmost part of the Pan-African Belt). Acta Geochimica, 42(2), 266-289.

- Gudbrandsson, S., Wolff-Boenisch, D., Gislason, S. R., & Oelkers, E. H. (2014). Experimental determination of plagioclase dissolution rates as a function of its composition and pH at 22 C. Geochimica et Cosmochimica Acta, 139, 154-172.

- Gurmessa, S. K., MacAllister, D. J., White, D., Ouedraogo, I., Lapworth, D., & MacDonald, A. (2022). Assessing groundwater salinity across Africa. Science of the Total Environment, 828, 154283.

- Hermann, J. (2023). Evolution Of The Subcontinental Lithospheric Mantle Below A Major Tectono-Volcanic Structure: Constraints From Mantle Xenoliths Along The Cameroon Volcanic Line (Doctoral dissertation, Department of Earth Sciences, ETH Zurich).

- Hermanson, L., Smith, D., Seabrook, M., Bilbao, R., Doblas-Reyes, F., Tourigny, E., ... & Kumar, A. (2022). WMO global annual to decadal climate update: a prediction for 2021–25. Bulletin of the American Meteorological Society, 103(4), E1117-E1129.

- Hu, Y., Liu, Z., Ford, D., Zhao, M., Bao, Q., Zeng, C., ... & Chen, J. (2020). Conservation of oxygen and hydrogen seasonal isotopic signals in meteoric precipitation in groundwater: An experimental tank study of the effects of land cover in a summer monsoon climate. Geochimica et Cosmochimica Acta, 284, 254-272.

- IAEA, (2006). Use of Chlorofluorocarbons in Hydrology: A Guidebook. STI/PUB/1238.http://www-pub.iaea.org/MTCD/ publications/PDF/Pub1238_web.pdf.

- IPCC. (2021). Climate Change 2021: The Physical Science Basis. Cambridge University Press.

- Jahan, A., Khan, M. U., Rai, N., Kumar, S., & Dar, T. A. (2023). Geochemical characterization, its controlling factors, and comparison between the upstream and downstream segments of the Himalayan Satluj River basin, India. Geochemistry, 83(2), 125974.

- Jannat, J. N., Khan, M. S. I., Islam, H. T., Islam, M. S., Khan, R., Siddique, M. A. B., ... & Islam, A. R. M. T. (2022). Hydro-chemical assessment of fluoride and nitrate in groundwater from east and west coasts of Bangladesh and India. Journal of Cleaner Production, 372, 133675.

- Jasechko, S., Birks, S. J., Gleeson, T., Wada, Y., Fawcett, P. J., Sharp, Z. D., ... & Welker, J. M. (2014). The pronounced seasonality of global groundwater recharge. Water Resources Research, 50(11), 8845-8867.

- Jiménez, J., Cavero, S. G., Marazuela, M. Á., Baquedano, C., Laspidou, C., Santamarta, J. C., & García-Gil, A. (2024). Effects of the 2021 La Palma volcanic eruption on groundwater hydrochemistry: Geochemical modelling of endogenous CO2 release to surface reservoirs, water-rock interaction and influence of thermal and seawater. Science of The Total Environment, 929, 172594.

- Kanellopoulos, T. D., Kapetanaki, N., Karaouzas, I., Botsou, F., Mentzafou, A., Kaberi, H., ... & Karageorgis, A. P. (2022). Trace element contamination status of surface marine sediments of Greece: an assessment based on two decades (2001–2021) of data. Environmental Science and Pollution Research, 29(30), 45171-45189.

- Kidder, J. A. (2020). Trace and Isotopic Geochemistry of Groundwaters in Mineral Exploration (Doctoral dissertation, Queen's University (Canada)).

- KISAKA, M. (2023). Volcanic history and controls on fluoride contamination of water resources at Mt. Meru, Northern Tanzania.

- Kisekka, I. (2024). Assessing the State of Knowledge and Impacts of Recycled Water Irrigation on Agricultural Crops and Soils.

- Kitessa, W. M., Kebede, A. B., Tufa, F. G., Gudeta, B. G., Yenehun, A., Chelkeba, B., Debela, S. K., Feyessa, F. F., & Walraevens, K. (2024). Hydrogeochemical characterization and processes controlling groundwater chemistry of complex volcanic rock of Jimma Area, Ethiopia. Water, 16(23), 3470. https://doi.org/10.3390/w16233470

- Kump, L. R., Brantley, S. L., & Arthur, M. A. (2000). Chemical weathering and climate evolution. Annual Review of Earth and Planetary Sciences, 28, 611-667.

- Lahermo, P. (1990). Geochemical atlas of Finland. Geologian tutkimuskeskus.

- Lapworth, D. J., Das, P., Shaw, A., Mukherjee, A., Civil, W., Petersen, J. O., ... & MacDonald, A. M. (2018). Deep urban groundwater vulnerability in India revealed through the use of emerging organic contaminants and residence time tracers. Environmental Pollution, 240, 938-949.

- Lapworth, D. J., Nkhuwa, D. C. W., Okotto-Okotto, J., Pedley, S., Stuart, M. E., Tijani, M. N., & Wright, J. J. H. J. (2017). Urban groundwater quality in sub-Saharan Africa: current status and implications for water security and public health. Hydrogeology journal, 25(4), 1093.

- Lapworth, D. J., Nkhuwa, D. C. W., Okotto-Okotto, J., Pedley, S., Stuart, M. E., Tijani, M. N., & Wright, J. (2020). Urban groundwater quality in sub-Saharan Africa: Current status and implications for water security and public health. Hydrogeology Journal, 28(3), 1093–1116. https://doi.org/10.1007/s10040-019-02036-6

- Lartsey, P. E. S. (2023). Leaching Experiments to Delineate Factors Governing Rock–water Interactions: Case of the North-Western Part of the Volta River Basin of Ghana.

- Lerner, J. A., et al. (2009). Permeability of volcanic rocks and its implications for groundwater flow. Hydrogeology Journal, 17(4), 789–806. https://doi.org/10.1007/s10040-009-0408-9

- Lewoyehu, M., Kohira, Y., Fentie, D., Addisu, S., & Sato, S. (2024). Water hyacinth biochar: A sustainable approach for enhancing soil resistance to acidification stress and nutrient dynamics in an acidic Nitisol of the northwest highlands of Ethiopia. Sustainability, 16(13), 5537.

- Liu, H., Zeng, W., Lai, Z., He, M., Lin, C., Ouyang, W., & Liu, X. (2024). Comparison of antimony and arsenic behaviour at the river-lake junction in the middle of the Yangtze River Basin. Journal of Environmental Sciences, 136, 189-200.

- Lordon, A. E. D., Agyingi, C. M., Manga, V. E., Bukalo, N. N., & Beka, E. T. (2017). Geo-electrical and borehole investigation of groundwater in some basalts on the South-Eastern Flank of Mount Cameroon, West Africa. Journal of Water Resource and Protection, 9(12), 1526-1546.

- Mahlknecht, J., Merchán, D., Rosner, M., Meixner, A., & Ledesma-Ruiz, R. (2017). Assessing seawater intrusion in an arid coastal aquifer under high anthropogenic influence using major constituents, Sr and B isotopes in groundwater. Science of the Total Environment, 587, 282-295

- Maloszewski, P., & Zuber, A. (1982). Determining the turnover time of groundwater systems with the aid of environmental tracers. Journal of Hydrology, 57(3–4), 207–231. https://doi.org/10.1016/0022-1694(82)90147-0

- Maloszewski, P., & Zuber, A. (1996). Lumped parameter models for the interpretation of environmental tracer data.

- Mama, N., Adama, A., Amadou, D. K., Bertrand, M. G. I., Seguem, N., & Ismaïla, N. (2021). Petrological and geochemical studies on the Si-undersaturated rocks of Mount Cameroon: Genesis of the camptonite and nephelinite at the Cameroon Hot Line. Open Journal of Geology, 11, 239–252. https://doi.org/10.4236/ojg.2021.116014

- Masoud, M., El Osta, M., Al-Amri, N., Niyazi, B., Alqarawy, A., & Rashed, M. (2024). Groundwater Characteristics’ Assessment for Productivity Planning in Al-Madinah Al-Munawarah Province, KSA. Hydrology, 11(7), 99.

- Mateus, A. C. C., Varajão, A. F. D. C., Petit, S., Oliveira, F. S., & Schaefer, C. E. G. R. (2020). Mineralogical and geochemical signatures of Quaternary pyroclasts alterations at the volcanic Trindade Island, South Atlantic. Journal of South American Earth Sciences, 102, 102674.

- Medici, G., Smeraglia, L., Torabi, A., & Botter, C. (2021). Review of modeling approaches to groundwater flow in deformed carbonate aquifers. Groundwater, 59(3), 334-351.

- Merten, J. (2021). Agrarian change and hydro-social transformations. The socio-natural production of water, risk and inequality in Jambi province, Indonesia (Doctoral dissertation, Dissertation, Göttingen, Georg-August Universität, 2021).

- Morales-Arredondo, I., Rodríguez, R., Armienta, M. A., & Villanueva-Estrada, R. E. (2016). The origin of groundwater arsenic and fluorine in a volcanic sedimentary basin in central Mexico: a hydrochemistry hypothesis. Hydrogeology Journal, 24(4), 1029.

- Mukherjee, A., Coomar, P., Sarkar, S., Johannesson, K. H., Fryar, A. E., Schreiber, M. E., ... & Vengosh, A. (2024). Arsenic and other geogenic contaminants in global groundwater. Nature Reviews Earth & Environment, 5(4), 312-328.

- Mussa, K. R., & Mjemah, I. C. (2023). Using hydrogeochemical facies and signatures for groundwater characterization and evolution assessment in aquifers with contrasting climate and geology in Tanzania. Applied Water Science, 13(10), 201.

- Négrel, P., & Petelet-Giraud, E. (2011). Isotopes in groundwater as indicators of climate changes. TrAC Trends in Analytical Chemistry, 30(8), 1279-1290.

- Ngai Nfor Jude, Wotany ER, Agyingi C, Ghomsi FEK, Aurore MKG, Nelson MA, et al (2023) Hydrogeochemical Characterization of Groundwater Quality, East of Mount Cameroon and West of the Penda Mboko River, Suitability for Drinking and Irrigation Use. J Geol Geophys. 12:10931.

- Ngai, N. J., Wotany, E. R., Agyingi, C., & Nelson, M. A. (2024). Geological influence on groundwater quality in volcanic aquifers of Eastern Mount Cameroon, West of the Penda Mboko River. Discover Applied Sciences, 6(541). https://doi.org/10.1007/s42452-024-06169-6

- Ngongang, A. W. (2016). Seismic and volcanological hazards of Mount Cameroon. University of Pretoria Press

- Nlend B, Huneau F, Ngo Boum-Nkot S, Song F, Komba D, Gwodog B, Meyoupe P, Djieugoue B, Fongoh E (2023) Review of isotope hydrology investigations on aquifers of Cameroon (Central Africa): What information for the sustainable management of groundwater resources? Water 15:4056. https://doi.org/10.3390/w15234056

- Nlend, B., Huneau, F., Garel, E., Santoni, S., & Mattei, A. (2023). Precipitation isoscapes in areas with complex topography: Influence of large-scale atmospheric dynamics versus microclimatic phenomena. Journal of Hydrology, 617, 128896.

- Nzouemou, F., Dongmo, N. G. T., Tabod, T. C., Ndoh, E. N., Mbinkong, R. S., & Kengni, S. H. P. (2025). Geoelectrical assessment of groundwater potential and aquifer characteristics in the west region of Cameroon. Modeling Earth Systems and Environment, 11(1), 1-18.

- Osuna-Martínez, C. C., Armienta, M. A., Bergés-Tiznado, M. E., & Páez-Osuna, F. (2021). Arsenic in waters, soils, sediments, and biota from Mexico: An environmental review. Science of The Total Environment, 752, 142062.

- Parisi, S., Piscopo, V., & Spaziani, F. (2011). Hydrogeochemical characterization of groundwater with elevated arsenic in volcanic-sedimentary aquifers: A case study from Central Italy. Environmental Earth Sciences, 64(4), 1077–1088. https://doi.org/10.1007/s12665-011-0922-2

- Parkes, S. D., McCabe, M. F., Griffiths, A. D., Wang, L., Chambers, S., Ershadi, A., ... & Element, A. (2017). Response of water vapour D-excess to land–atmosphere interactions in a semi-arid environment. Hydrology and Earth System Sciences, 21(1), 533-548

- Pazola, A., Shamsudduha, M., French, J., MacDonald, A. M., Abiye, T., Goni, I. B., & Taylor, R. G. (2024). High-resolution long-term average groundwater recharge in Africa estimated using random forest regression and residual interpolation. Hydrology and Earth System Sciences, 28(13), 2949-2967.

- Pfahl, S., & Sodemann, H. (2014). What controls deuterium excess in global precipitation?. Climate of the Past, 10(2), 771-781.

- Priestley, S. C., Meredith, K. T., Treble, P. C., Cendón, D. I., Griffiths, A. D., Hollins, S. E., ... & Pigois, J. P. (2019). A 35 ka record of groundwater recharge using stable water isotopes for Perth Basin in south-west Australia.

- Putman, A. L., Fiorella, R. P., Bowen, G. J., & Cai, Z. (2019). A global perspective on local meteoric water lines: Meta‐analytic insight into fundamental controls and practical constraints. Water Resources Research, 55(8), 6896-6910.

- Raghavan, S. S., Sabu, J., & Eslamian, S. (2024). Deciphering Groundwater-Surface water Interactions using Environmental Tracers in a Tropical Lake, Southwest India. Current World Environment, 19(3), 1101.

- Rhodes, A. L., Guswa, A. J., & Newell, S. E. (2006). Seasonal variation in the stable isotopic composition of precipitation in the tropical montane forests of Monteverde, Costa Rica. Water Resources Research, 42(11).

- Rhodes, A. L., Guswa, A. J., & Newell, S. E. (2010). Using stable isotopes to identify orographic precipitation events at Monteverde, Costa Rica. Tropical Montane Cloud Forests: Science for Conservation and Management, 242-248.

- Sako, A., & Kafando, S. (2021). Hydrogeochemical and spatial assessment of groundwater quality from basement aquifers in the Central Plateau Region of Burkina Faso, West Africa. Environmental Earth Sciences, 80(9), 358.

- Sánchez-Murillo, R., Birkel, C., Welsh, K., Esquivel-Hernández, G., Corrales-Salazar, J., Boll, J., ... & Araguás-Araguás, L. J. (2016). Key drivers controlling stable isotope variations in daily precipitation of Costa Rica: Caribbean Sea versus Eastern Pacific Ocean moisture sources. Quaternary Science Reviews, 131, 250-261

- Saranya, P., Krishnakumar, A., Sinha, N., Kumar, S., & Krishnan, K. A. (2021). Isotopic signatures of moisture recycling and evaporation processes along the Western Ghats orography. Atmospheric Research, 264, 105863.

- Sarath, P. K., Mangalaa, K. R., Cardinal, D., Gurumurthy, G. P., Dapoigny, A., Sarma, V. V. S. S., & Riotte, J. (2022). Seasonal, weathering and water use controls of silicon cycling along the river flow in two contrasting basins of South India. Chemical Geology, 604, 120883.

- Séraphin, P., Vallet-Coulomb, C., & Gonçalvès, J. (2016). Partitioning groundwater recharge between rainfall infiltration and irrigation return flow using stable isotopes: The Crau aquifer. Journal of Hydrology, 542, 241-253.

- Setiawan, T., Syah Alam, B. Y. C., Haryono, E., & Hendarmawan. (2020). Hydrochemical and environmental isotopes analysis for characterizing a complex karst hydrogeological system of Watuputih area, Rembang, Central Java, Indonesia. Hydrogeology Journal, 28(5), 1635-1659.

- Sharifan, F., Ramezani, Y., Amirabadizadeh, M., & De Michele, C. (2025). Evaluating the impact of climate change on precipitation in the future period using CMIP6 models (case study: Siminehrood River Basin, Iran). Acta Geophysica, 1-16

- Smith, D. F., Saelens, E., Leslie, D. L., & Carey, A. E. (2021). Local meteoric water lines describe extratropical precipitation. Hydrological Processes, 35(2), e14059.

- Stallard, R. F. (1995). Tectonic, environmental, and human aspects of weathering and erosion: a global review from a steady-state perspective. Annual Review Of Earth And Planetary Sciences, Volume 23, pp. 11-40., 23, 11-40.

- Sunkari, E. D., Abu, M., & Zango, M. S. (2021). Geochemical evolution and tracing of groundwater salinization using different ionic ratios, multivariate statistical and geochemical modeling approaches in a typical semi-arid basin. Journal of Contaminant Hydrology, 236, 103742.

- Taat, S., CSS, S. A. B. Y., & Eko, H. (2020). Hydrochemical and environmental isotopes analysis for characterizing a complex karst hydrogeological system of Watuputih area, Rembang, Central Java, Indonesia. Hydrogeology Journal, 28(5), 1635-1659.

- Tchakam Kamtchueng, B., Foupouagnigni, A., Yannah, M., Ndjama, J., Mimba, M. E., Wirmvem, M. J., Yaguchi, M., Takem, G. E., Ako, A. A., Fantong, W. Y., Ngnikam, E., & Ohba, T. (2022). Major ions, trace elements and stable isotope characteristics of shallow groundwater in the Bonaberi district, Douala, Cameroon. Sustainable Water Resources Management, 8, Article 162. https://doi.org/10.1007/s40899-022-00737-7

- Tchakam Kamtchueng, B., Foupouagnigni, A., Yannah, M., Ndjama, J., Mimba, M. E., Wirmvem, M. J., ... & Ohba, T. (2022). Major ions, trace elements and stable isotope characteristics of shallow groundwater in the Bonaberi district, Douala, Cameroon. Sustainable Water Resources Management, 8(5), 162.

- Tchindjang, M., Amougou, J. A., Abossolo, S. A., & Bell, S. B. (2012). Challenges of climate change, landscape dynamics and environmental risks in Cameroon. Landscape evolution, neotectonics and quaternary environmental change in Southern Cameroon. Palaeoecology of Africa, 31, 237-286.

- UCCC (University of California Cooperative Extension). (2014). Volcanic soil formation under humid conditions. Technical Report 14-001.

- Valiallahi, J., & Yazdani, M. (2025). Evaluating groundwater quality by examining electrical conductivity, total dissolved solids, total hardness, and turbidity using geographic information systems (GIS®): a case study of selected wells in the Taleghan region, Iran. Applied Water Science, 15(3), 1-11.

- Wanke, H., Gaj, M., Beyer, M., Koeniger, P., & Hamutoko, J. T. (2018). Stable isotope signatures of meteoric water in the Cuvelai-Etosha Basin, Namibia: Seasonal characteristics, trends and relations to southern African patterns. Isotopes in Environmental and Health Studies, 54(6), 588-607.

- Wirmvem, M. J., Ohba, T., Fantong, W. Y., Ayonghe, S. N., & Suh, C. E. (2017). Hydrochemical and isotopic characteristics of groundwater in the Mount Cameroon volcanic region. Journal of African Earth Sciences, 129, 433–445. https://doi.org/10.1016/j.jafrearsci.2017.01.025

- Wotany, E. R., Wirmvem, M. J., Fantong, W. Y., Ayonghe, S. N., Wantim, M. N., & Ohba, T. (2021). Isotopic composition of precipitation and groundwater onshore of the Rio del Rey Basin, southwest Cameroon: local meteoric lines and recharge. Applied Water Science, 11, 1-14.

- Wu, H., Zhi, Y., Xiao, Q., Yu, F., Cao, G., Xu, X., & Zhang, Y. (2025). Source-oriented health risk of heavy metals in sediments: A case study of an industrial city in China. Ecotoxicology and Environmental Safety, 292, 117929.

- Yenehun, A., Dessie, M., Nigate, F., Belay, A. S., Azeze, M., Van Camp, M., ... & Walraevens, K. (2022). Spatial and temporal simulation of groundwater recharge and cross-validation with point estimations in volcanic aquifers with variable topography. Journal of Hydrology: Regional Studies, 42, 101142.

- Zhang, D.X.; Yan, H.; Miao, Y.; Zhang, M. Analysis of Water Vapor Transport Reduction in the Yarlung Tsangpo Grand Canyon Region: A Trajectory Tracking Approach. Plateau Meteorol. 2024, 43, 114–126.

- Zhang, Q., Qian, H., Xu, P., Li, W., Feng, W., & Liu, R. (2021). Effect of hydrogeological conditions on groundwater nitrate pollution and human health risk assessment of nitrate in Jiaokou Irrigation District. Journal of Cleaner Production, 298, 126783.

- Zhao, Z., Lin, Y. F., Stumpf, A., & Wang, X. (2022). Assessing impacts of groundwater on geothermal heat exchangers: A review of methodology and modeling. Renewable Energy, 190, 121-147.