TREATMENT OF PETROCHEMICAL WASTE WATER USING FENTON OXIDATIVE METHOD

Abstract

The remediation of surface water contaminated with diesel using the Fenton oxidative method was investigated at ambient temperature (28- 34 °C) for effectiveness and optimum conditions were established using UV Spectroscopy. This study was aimed at investigating the effectiveness of Fenton oxidation in remediating diesel-contaminated  water. The optimization results for the Fenton-Oxidative showed 400,000 mg/L H2O2 and 300 mg/L FeSO4, and pH adjusted between 2.50-3.00 for all samples. At the end of the chemical remediation experiment, the Fenton-oxidative method proved to be incredibly quick, with the reaction being exothermic and following second order kinetics, 88.80% (from 4297.73 mg/L to 480.57mg/L) removal efficiency of total petroleum hydrocarbon as diesel was realized after one hour under water. The reaction followed a pseudo-first order kinetics with the rate constant of 1.16 x 104 min-1 mg/L. The Fenton-oxidative method for surface water polluted with diesel gave remediation efficiency of 39.62% (Cd), 53.23% (Ni), 21.79% (Pb), 38.89% (Hg), 68.75% (Se), and 82.65% (Cr). Except for Chromium and Selenium, other heavy metal concentrations exceeded the permissible levels recommended by World Health Organization (WHO).  This implies that Fenton oxidative method is ineffective in the removal of Cd, Pb, Ni, and Hg from surface water polluted with diesel. The values of the physicochemical parameters of the surface water contaminated with diesel were similar with the unpolluted surface water sample and World Health Organization (WHO) permissible limits. This implies that diesel polluted surface water remediated by Fenton-oxidative method needs post-treatment to improve on its portability for drinking/domestic and agricultural uses .Fenton oxidative technique makes use of what is readily available and cheap, thereby avoiding the use of more complex methods. In conclusion, the Fenton-oxidative method was found to be very effective in remediating surface water contaminated with diesel.

Keywords: Petrochemical waste water, Fenton-oxidative method, Diesel, TPH

1.0 Introduction

There has been a rapid increase in population worldwide resulting in increased demand for hydrocarbon products such as diesel, petrol, kerosene, pesticides and industrial chemicals [3]. Although many chemicals are utilized or destroyed, a high percentage is released into the air, water, and soil representing a potential environment hazard [1,8].

Water contamination is a serious global is that necessitates ongoing review and constant adjustments of water resources policies at all levels [9]. Petroleum hydrocarbons polluted water has been suggested to be the leading worldwide cause of illness and mortality; it causes the deaths of around 14,000 people daily. Processing and distribution of petroleum hydrocarbons as well as the of petroleum products when released, leads to contamination of soil [2] and these petroleum hydrocarbons end up in surface and ground  water [10]. Therefore, the water is also affected since the petroleum products are washed down into rivers, streams, oceans, and other water bodies or may sink down through the soil and contaminate ground water [11].

Changes in water properties such as turbidity, alkalinity, TDS, etc due to contamination with petroleum-derived substances lead to oxygen deficiency in the water which affects living organisms in the water and those who utilize the water for drinking, cooking, and other purposes [12].

Petroleum-derived substances are usually less dense than water and so they float or are found on top of the water during spillage and this inhibits light penetration which affects phytoplankton in the contaminated water during the process of photosynthesis [13].

Hydrocarbon spill is a serious threat to the ecology [7]. As reported by Dorn et al., [4], hydrocarbon contains substances that are toxic to the flora and fauna found in the ecosystem such as diesel which contain low molecular weight compounds and high proportion of saturated hydrocarbons that are usually more toxic than long chain hydrocarbons [14].

Diesel pollution is on the increase in Nigeria as well as in other developing countries [8]. Pollutants enter the environment directly as a result of accidents, spills during transportation, leakage from waste disposal or storage sites or from industrial facilities [6].

So therefore, Fenton oxidative method is used to remove these pollutants from the environment because it is eco-friendly and cost effective as compared with other methods [15].

2.0 Materials and Methods

2.1 Sampling Method

Surface water sample was collected using the grab sampling technique along the bank of the flowing River Bali which flows through the Bali main bridge in a 25 litre container from the River Bali [5].

2.2 Source of Diesel Oil

Diesel oil was purchased from Nigeria National Petroleum Corporation (NNPC) retail Station, Kofai, Jalingo Local Government Area, Taraba State and used as received.

2.3 Preparation of Samples

45 mL of the surface water sample was contaminated with 5 mL of diesel oil in multiple beakers to create 10% contamination.

2.4 Instrument Requirements

Different hydrocarbons in water displayed absorbance at particular wavelengths. Spectrophotometric measurements provided acceptable accuracy, sensitivity, reproducibility and linearity at different wavelengths used. A generator and uninterrupted power supply (UPS) device were used to guarantee stable electricity for reliable performance. In the study, a spectrophotometer was chosen over a Colorimeter to lessen the interference from undesired Chromogen.

2.5 Statistical Treatments

Samples were made in replicate of three to provide data for statistical treatment. Standard deviation, relative standard deviation, and coefficient of variation (CV) calculations were employed to checkmate indeterminate (random) error. Blank runs were also conducted to lessen the likelihood of determinate errors.

2.6 Optimization Study

Optimization study was conducted to determine the optimum conditions for the concentration of hydrogen peroxide, while that of pH was adopted from literature as used by Medjor et al., [5]. These conditions were then applied to kinetics studies.

2.6.1 Fenton-oxidative Method

Treatment Concentration: A 300 mg/L FeSO4 + 50,000 mg/L H2O2 in the ratio 1:5 by volume. A number of solutions of 10% contamination of diesel oil in 45 mL surface H2O were made and 6 cm3 of 300 mg/L FeSO4 + 30 cm3 of 50,000-500,000 mg/L H2O2 was added to each solution [5] and left for an hour until extraction and analysis. TPH as diesel was measured T-60 UV/Visible spectrophotometer at a wavelength of 350 nm following the laboratory procedure used by [5].

2.6.2 FeSO4 Concentration

The best concentration for FeSO4 used in the treatment solution was adopted from literature as used by [5].

2.6.3 pH Adjustment

The pH of all replicate water samples tainted with diesel was adjusted to pH values of between 2.50-3.00 using 1M H2SO4 and 1M NaOH and then was treated with the best treatment concentrations obtained from 2.6.1 and 2.6.2, kept for 1 hour until extraction and analysis. This is because Fenton reaction only works in an acidic medium (pH 2.00 and 4.00).

3.0 Results and Discussion

The findings indicate that diesel contaminated surface water provided the best possible remediation of 88.80 when treated with 400,000 mg/L H2O2 and 300 mg/L FeSO4 with each solution being adjusted to between pH 2.50 and pH 3.00. Table 1 shows the results that were obtained during the optimization studies and figure 1 shows the graph of % remediation against concentration of hydrogen peroxide used with the readings taken under water (without extraction with hexane).

Table 1: Results obtained from readings under H2O during optimization of H2O2

| S/N | Concentration of H2O2 (%) | Replicate 1 | Replicate 2 | Replicate 3 | TPH mean (mg/L) | Standard Deviation | RSD | CV | % Remediation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5.00 | 656.80 | 913.30 | 904.30 | 824.80 | 145.56 | 0.17643 | 17.643 | 80.80 |
| 2 | 10.00 | 1328.00 | 980.80 | 884.20 | 1064.33 | 233.39 | 0.21929 | 21.929 | 75.20 |
| 3 | 15.00 | 781.80 | 823.70 | 982.00 | 862.50 | 105.59 | 0.12242 | 12.242 | 79.90 |
| 4 | 20.00 | 882.10 | 1384.10 | 1142.10 | 1136.10 | 251.05 | 0.22098 | 22.098 | 73.60 |
| 5 | 25.00 | 591.90 | 528.20 | 599.80 | 573.30 | 39.26 | 0.06847 | 6.847 | 86.70 |
| 6 | 30.00 | 551.10 | 473.20 | 477.30 | 500.53 | 43.84 | 0.08759 | 8.759 | 88.40 |
| 7 | 35.00 | 745.20 | 467.00 | 439.70 | 550.63 | 169.05 | 0.30701 | 30.701 | 87.20 |
| 8 | 40.00 | 457.80 | 589.50 | 394.40 | 480.57 | 99.52 | 0.20709 | 20.709 | 88.80 |
| 9 | 45.00 | 759.30 | 521.20 | 416.90 | 565.80 | 175.50 | 0.31019 | 31.019 | 86.80 |
| 10 | 50.00 | 1335.10 | 490.60 | 659.10 | 828.26 | 446.94 | 0.53961 | 53.961 | 80.70 |

RSD = Relative Standard Deviation; CV = Coefficient of Variation

From Table 1, the concentration that gave the highest remediation was 40% H2O2. Also, a range of 25% to 45% H2O2 gave a very good result in remediation. This is in line with the optimum concentration used by Medjor et al., 2012 who used 25% in their studies with crude oil.

Figure 1: Optimization graph of % Remediation against concentration of H2O2

Table 2 Results obtained from organic layer during optimization of H2O2

| S/N | Concentration of H2O2 (%) | Replicate 1 | Replicate 2 | Replicate 3 | TPH mean (mg/L) | Standard Deviation | RSD | CV | % Remediation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5.00 | 3527.70 | 3432.90 | 3480.20 | 3480.27 | 47.40 | 0.0136 | 1.36 | 18.50 |
| 2 | 10.00 | 3603.00 | 3692.30 | 3453.40 | 3582.90 | 120.71 | 0.0336 | 3.36 | 16.10 |
| 3 | 15.00 | 3286.50 | 3535.80 | 3541.80 | 3454.70 | 145.69 | 0.0421 | 4.21 | 19.10 |
| 4 | 20.00 | 3324.10 | 3234.90 | 3247.40 | 3268.80 | 48.30 | 0.0147 | 1.47 | 23.50 |
| 5 | 25.00 | 3526.90 | 3378.40 | 3306.30 | 3403.87 | 112.48 | 0.0330 | 3.30 | 20.30 |
| 6 | 30.00 | 3555.50 | 3513.50 | 3424.20 | 3497.73 | 67.05 | 0.0191 | 1.91 | 18.10 |
| 7 | 35.00 | 3596.70 | 3534.50 | 3524.30 | 3551.83 | 39.19 | 0.0110 | 1.10 | 16.80 |
| 8 | 40.00 | 3580.80 | 3650.70 | 3637.20 | 3622.90 | 37.08 | 0.0102 | 1.02 | 15.20 |
| 9 | 45.00 | 3293.20 | 3447.20 | 3414.90 | 3385.10 | 81.21 | 0.0239 | 2.39 | 20.80 |
| 10 | 50.00 | 3326.50 | 3223.60 | 3193.40 | 3247.83 | 69.78 | 0.0214 | 2.14 | 24.00 |

RSD = Relative Standard Deviation; CV = Coefficient of Variation

From Table 2 above, the % remediation was quite low as compared to the % remediation obtained in Table 1. The values for the initial TPH without the introduction of the Fenton’s reagent were 4297.70 mg/L and 4271.30 mg/L for readings under water (without extraction) and under the organic (hexane) layer, respectively. The values are so close to each other such that one would expect the % remediation in the organic (hexane) layer and the % remediation for readings under water (without extraction) to be very close. The possible explanation for this drop in % remediation in the organic (hexane) layer as compared to the % remediation for readings under water (without extraction) is that there could be complexation between the Fenton’s reagents and the diesel or n-hexane or both.

Figure 2: Graph of % Remediation against H2O2 from organic layer readings in optimization of H2O2

Kinetics of Fenton oxidative method was examined for its order of reaction. The concentration of total petroleum hydrocarbon left was plotted versus time using the information from table Table 3 and represented in figure 3 and figure 4 for the second order profile. This strongly indicated that there was interaction between total petroleum hydrocarbon molecules and the reacting species of the treatment solution in the Fenton-oxidative method used in the study.

Table 3 Kinetics data for diesel form readings under water (without extraction0, in Fenton-oxidative method

| S/N | Time (min) | Replicate 1 | Replicate 2 | Replicate 3 | TPH Mean left as diesel (mg/L) | STDEV | Ln(B0/B1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 4455.40 | 4451.30 | 3986.50 | 4297.73 | 269.54 | - |
| 2 | 5.00 | 2320.20 | 1681.90 | 2960.20 | 2320.77 | 639.15 | 0.7889 |
| 3 | 10.00 | 2333.50 | 2076.40 | 2114.00 | 2174.63 | 138.86 | 0.8667 |
| 4 | 15.00 | 1905.10 | 1919.00 | 1806.20 | 1876.77 | 61.51 | 1.0469 |
| 5 | 30.00 | 1013.50 | 952.90 | 913.40 | 959.93 | 50.42 | 1.9828 |
| 6 | 45.00 | 702.10 | 953.20 | 952.20 | 869.17 | 144.68 | 2.1492 |
| 7 | 60.00 | 457.80 | 589.50 | 394.40 | 480.57 | 99.52 | 2.1908 |

STDEV = Standard Deviation; Ln = Natural log; B0 = Initial concentration of diesel; B1 = Concentration of diesel at time t.

Figure 3: Test for order of  reaction kinetics in Fenton oxidative method (for readings under water)

This result is in line with the findings of Medjor et al., 2012, who also reported a similar case of works of EPA, (2000).

Figure 4: Second order profile (for readings under water)

The rate constant of the Fenton-oxidative method employed in the treatment of diesel contaminated surface water samples was 1.16 x 104  mg-1L-1 min-1. The half-life of second-order reaction kinetics was 8.62 x 10-5 min-1. It was observed that an effective removal of diesel from the contaminated surface water samples required optimum period of 1 hour achieving a remediation of 88.80%.

Table 4 Kinetics data from organic layer for diesel in Fenton oxidative method

| S/N | Time (min) | Replicate 1 | Replicate 2 | Replicate 3 | TPH Mean (mg/L) | STDEV | Ln(B0/B1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 4446.50 | 4197.50 | 4169.80 | 4271.27 | 152.39 |  |
| 2 | 5.00 | 3467.10 | 3442.60 | 3376.80 | 3428.83 | 46.70 | 0.219 |
| 3 | 10.00 | 3588.50 | 3600.40 | 3587.00 | 3591.97 | 7.34 | 0.173 |
| 4 | 15.00 | 3455.80 | 3398.30 | 3398.70 | 3417.60 | 33.08 | 0.223 |
| 5 | 30.00 | 3854.00 | 3904.50 | 3921.80 | 3893.43 | 35.23 | 0.093 |
| 6 | 45.00 | 3760.20 | 3723.00 | 3685.90 | 3723.03 | 37.15 | 0.137 |
| 7 | 60.00 | 3580.80 | 3650.70 | 3637.20 | 3622.90 | 37.08 | 0.165 |

Key: STDEV = Standard Deviation; Ln = Natural log

Figure 5: A graph of kinetics of readings from organic layer

Figure 6: Order profile for organic layer

The result of the data obtained from readings of the organic layer were presented in Table 4 which was used in plotting a graph of TPH left against time as shown in figure 5. earlier investigators like [5] observed that TPH decreased that TPH decreased with time but that trend wasn’t seen in the organic layer readings. This could be as a result of complexation between the Fenton’s reagent and the diesel hydrocarbon. The supposed second-order profile in figure 6 was erratic which could also be due to complexation between Fenton’s reagent and the diesel.

3.4.1 Physicochemical Parameters

Table 5 summarized the results for electrical conductivity, NH4 alkalinity, turbidity, nitrate, chloride, phosphate, TDS, Ammonia, and heavy metals (Cd, Ni, Pb, Hg, Se, As, Cr, and V) determined in diesel unpolluted surface water sample, polluted surface water sample without treatment and polluted surface water sample remediated by Fenton-oxidative method.

Table 5 Physicochemical parameters of unpolluted, polluted, and remediated surface water samples.

| S/N | Physicochemical parameter | Surface water | Polluted surface water | Remediated surface water | Remediated back to surface water level | Within WHO or FEPA level | WHO/FEPA standards |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Turbidity (NTU) | 20.50 | 144.00 | 17.00 | √ | X | 5.00 |
| 2 | Alkalinity (mg/L) | 66.00 | 80.00 | Nil | √ | √ | 100.00 |
| 3 | Chloride (mg/L) | 0.80 | 0.40 | 0.60 | √ | √ | 250.00 |
| 4 | Nitrate (mg/L) | 1.68 | 2.37 | 1.69 | √ | √ | 50.00 |
| 5 | Phosphate (mg/L) | 0.14 | 0.17 | 0.13 | √ | √ | 5.00 |
| 6 | NH4 (mg/L) | 0.80 | 0.61 | 0.25 | √ | √ | 0.50 |
| 7 | TDS (mg/L) | 105.00 | 120.00 | 60.00 | √ | √ | 500.00 |
| 8 | Cd (mg/L) | 3.15 | 53.00 | 32.00 | X | X | 0.003 |
| 9 | Ni (mg/L) | 1.30 | 7.74 | 3.62 | X | X | 0.02 |
| 10 | Pb (mg/L) | 1.25 | 3.90 | 3.05 | X | X | 0.01 |
| 11 | Hg (mg/L) | 0.31 | 1.44 | 0.88 | X | X | 0.001 |
| 12 | V (mg/L) | Nil | Nil | Nil | Nil | Nil |  |
| 13 | Se (mg/L) | 0.02 | 0.16 | 0.05 | X | √ | 0.05 |
| 14 | As (mg/L) | Nil | Nil | Nil | Nil | Nil | 0.01 |
| 15 | Cr (mg/L) | 0.05 | 0.98 | 0.17 | X | √ | 0.50 |

√ = Yes; X = No

The turbidity of the surface water samples polluted with diesel was 144 NTU. This value indicates that the prepared sample was polluted which is in line with the value of turbidity in the work of Rump, H. H. (1991), who reported turbidity value of 100-275 NTU for polluted water. Diesel polluted surface water sample remediated by Fenton-oxidative method had turbidity value less than 20 NTU. This suggests low levels of dissolved suspended particles and very high quality water (WHO, 2010).

The Fenton-oxidative method for surface water polluted with diesel gave remediation efficiency of 39.62% (Cd); 53.23% (Ni); 21.79% (Pb); 38.89% (Hg); 68.75% (Se); and 82.65% (Cr). Except for Chromium and Selenium, where chemical leaching of the metal in an acidic medium was very effective, the concentration of other heavy metals exceeded the permissible levels recommended by World Health Organization (WHO). This implies that the Fenton-oxidative method is not effective in removing Cd, Pb, Ni, and Hg from surface water polluted with diesel as previous studies have shown [5].

In terms of the physicochemical parameters that were analysed, four out of the thirteen parameters that were determined were deviant i.e. they did not have values as close as those of the unpolluted surface water or the WHO permissible limits. This indicates that there was 69.23% remediation in terms of the physicochemical parameters.

4.0 Conclusion

From the results obtained and discussed, it shows clearly that remediation of diesel contaminated surface water occurred which to an extent is effective for most physicochemical parameters and in the removal of hydrocarbon from diesel contaminated surface water. There was 88.80% removal efficiency in using Fenton-oxidative method in the remediation studies. The remediated polluted surface water sample was also found to have most of its physicochemical parameters as close match with the unpolluted surface water sample and WHO permissible limits. It is therefore recommended that diesel polluted surface water remediated by Fenton-oxidative method needs post treatment especially for the removal of heavy metals to improve on its quality for drinking, domestic and Agricultural uses.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

References

1. Alexander, M. (1994). Biodegradation and Bioremediation. Academic Press.

2. Kong, S. H., Watts, R. J., & Choi, J. H. (1998). Treatment of petroleum-contaminated soils using iron mineral catalyzed hydrogen peroxide. Chemosphere, 37(8), 1473-1482.

3. Leethem, J. T. (2002). In Situ Chemical Oxidation of MtBE: A Successful Case Study of Remediation of a Large Gasoline Release. Soil and Sediment Contamination: An International Journal, 11(3), 450-451.

4. Scha Schafer, A. N., Snape, I., & Siciliano, S. D. (2009). Influence of liquid water and soil temperature on petroleum hydrocarbon toxicity in Antarctic soil. Environmental toxicology and chemistry, 28(7), 1409-1415.

5. Stephen, E., Okolo, M. O., & Akogu, E. A. (2011). Microbiological properties of oil impacted soil 36 months after diesel spill. International Journal of Education, Science, Mathematics and Environmental Studies, 3(1), 77-82.

6. W Watts, R. J., & Dilly, S. E. (1996). Evaluation of iron catalysts for the Fenton-like remediation of diesel-contaminated soils. Journal of Hazardous Materials, 51(1-3), 209-224.

7. Ayotamuno, M. J., Kogbara, R. B., & Agunwamba, J. C. (1996). Bioremediation of a petroleum-hydrocarbon polluted agricultural soil at various levels of soil tillage in Port Harcourt, Nigeria. Nigerian Journal of Technology, 35(2), 345–353.

8. Chakrabarty, A. M., Mylroie, J. R., Friello, D. A., & Vacca, J. G. (1998). Transformation of Pseudomonas putida and Escherichia coli with plasmid-linked drug-resistance factor DNA. Proceedings of the National Academy of Sciences of the United States of America, 75(1), 310–314.

9. Dorn, P. B., Vipond, T. E., & Salanitro, J. P. (1998). Assessment of the acute toxicity of crude oils in soils using earthworms. Chemosphere, 37(5), 845–860.

10.  Medjor W. O., Egharevba, F., Akpoveta, V. O., Ize-Iyamu, O. K., Jatto, E.O. (2012). Remediation of crude-oil contaminated groundwater by Fenton-Oxidative method.. International Journal of Environmental Sciences, 2(3): 0976–4402.

11. Riser-Roberts, E. (1998). Remediation of Petroleum Contaminated Soils: Biological, Physical, and Chemical Processes. Lewis Publishers.

12. Schafer, G., & Rippen, G. (1999). Remediation of petroleum contaminated soils-biological, physical and chemical processes. Environmental Sciences Europe, 11(4), 352.

13. Stephen, A. S., Aleer, S., Adetutu, E. M., Makadia, T. H., Patil, S., & Ball, A. S. (2011). Harnessing the hydrocarbon-degrading potential of contaminated soils for the bioremediation of waste engine oil. Water, Air, & Soil Pollution, 218(1), 121–130.

14. Anipsitakis, G. P., & Dionysiou, D. D. (2018). Radical generation by the interaction of transition metals with common oxidants. Environmental Science & Technology, 52(10), 5896–5912.

.