Design of Mini Tractor Operated Multipurpose Equipment for Groundnut Crop

ABSTRACT

A study was conducted in 2021 at the Department of Farm Machinery and Power Engineering, Dr. NTR College of Agricultural Engineering, Bapatla, Andhra Pradesh, to design, fabricate, and evaluate a cost-effective multipurpose agricultural implement. The equipment was developed in the departmental workshop and subsequently field-tested at the college research farm. Designed to address the needs of small and marginal farmers, the implement integrates sowing, spraying, and weeding functionalities into a single machine, significantly reducing the time, labor, and fuel requirements for row crop cultivation. The developed machine features a simple and robust design, with overall dimensions of 1800 × 600 × 1550 mm and a total weight of 150 kg. Tynes are mounted using nuts and bolts, allowing for easy adjustment of row spacing to suit various crops. The sowing unit ensures accurate seed placement at the desired depth, while the spraying and weeding units are configured for simultaneous operation. This makes the implement particularly suitable for narrow-row crops such as groundnut. Laboratory and field evaluations were conducted to assess the machine’s performance. The sowing unit delivered an average of 340 grams of seed per 20 ground wheel revolutions, corresponding to a seed rate of 120.2 kg ha⁻¹. The spraying unit, equipped with three adjustable nozzles, achieved a discharge rate ranging from 0.9 to 1.3 L min⁻¹ at operating pressures between 150 and 275 kPa. Field results confirmed the implement’s effectiveness in reducing operational costs and fuel usage by enabling combined spraying and weeding operations in a single pass.

Keywords: Groundnut, drudgery, field capacity, field efficiency, sowing, weeding

1.  INTRODUCTION

Agricultural mechanisation in India has come a long way in the last 50 years. India produces wide range of agricultural equipment needed to practice modern intensive farming. The overall farm mechanization in India is 40-45 % (National Round Table Conference, 2017) (i.e., tillage about 40%, seeding and planting about 30 %, plant protection 35-4 5%, harvesting and threshing about 60-70% for rice and wheat and less than 15 % for other crops). Farm mechanization helps to bring significant improvement in agricultural productivity. Farm mechanization saves inputs like seeds, fertilizers by 15-20% and increases cropping intensity by 5-20%. It increases efficiency of farm labour and reduces time of agricultural operation by 15-20%. Thus, there is strong need for mechanization of agricultural operations. Agricultural mechanisation has boosted agricultural production by reducing the power bottlenecks in agricultural operations, and enhancing land-use intensification. In fact, mechanisation can be a boon for small and inefficient farmers in developing countries and helps them cope with the shortage of labour and improve farm efficiency (Sarkar, 2020 and Aryal et al., 2021).

In India, 67% holdings are less than 1 ha (Singh, 2015) accounting for 19% of the total operated area whereas; over 86% holdings are less than 2 ha accounting for nearly 47% of the operating area (Anonymous, 2013). Fragmentation of farm holdings is major concern in this respect and the average size of holdings has shrunk from 2.82 ha in 1970–71 to 1.1 ha in 2010–11 (Khandetod, 2019). This will also not satisfy need of energy requirement of farming as compared to other countries in the world (Mehta and Pajnoo, 2013). There is a need of mechanization in farm operations at the same time it should be feasible for small scale farm holders.

India ranks among the largest producers of groundnut (Arachis hypogaea L.), accounting for approximately 6.8 million tonnes from a cultivated area of 4.5 million hectares, with major production concentrated in states like Gujarat, Andhra Pradesh, Tamil Nadu, and Karnataka. Groundnut is a crucial oilseed crop for Indian agriculture, contributing significantly to farmers’ incomes and rural livelihoods. Traditional groundnut cultivation practices, involving separate manual or animal-drawn operations for sowing, intercultural, and digging, are labor-intensive, time-consuming, and physically strenuous, particularly for women and aged farmers. This drudgery work results in increasing of heart rate, oxygen consumption rate, and energy expenditure rate which leads to health implications (Akki et al., 2021). These conventional methods result in high drudgery, low field efficiency, and increased production costs, often affecting timely operations and ultimately yield. The groundnut crop cultivation practices are land preparation, seeding, irrigation, weeding, fertigation, spraying, and harvesting. Almost in all operations there is a need of mechanization.

Commercially there are various machines that are available for sowing, intercultural and spraying operations. However, these machines have to be manually, or power operated to perform the required operations and moreover separate machines are used for every function. Practically with such little ownership of land, it is not possible for small and marginal farmers to purchase each implement for specific operation. So, instead of developing implements and operational specific machines, multipurpose farm equipment which can perform sowing, weeding, and spraying can reduce economic burden on small and marginal farmers.

To address the above challenges, a mini tractor operated multipurpose equipment was designed and developed specifically for small and marginal farmers cultivating groundnut. The objectives of the research are: i) To study existing sowing, inter cultural and spraying equipments; ii) To develop multipurpose equipment for row crops; iii) To evaluate equipment in laboratory and field conditions; and iv) To study cost economics of the multipurpose equipment with the existing practices.

2. MATERIALS AND METHODS

A multipurpose equipment was designed and fabricated in the Departmental Workshop of Farm Machinery and Power Engineering and evaluated at field of Dr. NTR College of Agricultural Engineering, Bapatla in the year of 2021.

2.1 Design Considerations

The seeding attachment furrow opener, frame, seed hopper, seed metering mechanism, chain and sprocket and ground wheel etc. The following sections describe the theoretical design process and fabrication involved.

2.1.1 Design of sowing unit

2.1.1.1. Size of the seed drill

Size of the seed drill usually depends on calculation of the total draft and number of furrow openers required based upon the power source available for operation of the seed drill (Ebrahim et al., 2008).

Accordingly, a trapezoidal cross section made by the furrow opener with a top width of 40 mm and bottom width of 25 mm and working at a depth of 35 mm respectively was assumed.

Cross section of furrow opener (A)=((a+b))/2 × h

Where,

a = Top width of furrow opener, mm

B = Bottom width of furrow, mm

h = Cutting depth of furrow opener, mm

Cross section of furrow opener (A)=13 cm2

The value for the average soil resistance is calculated by using the following formula

Average soil resistance (Fx) = A × Pk

Where,

A = Cross section of furrow, cm2

Pk = Specific soil resistance of heavy soil, kg cm-2

Specific soil resistance Pk of the soil when sowing to a depth of 15 cm for different soils is given as shown below (Allison, 1968);

Light soil: 0.12 kg cm-2

Medium soil: 0.15 kg cm-2

Heavy soil: 0.20 kg cm-2

Very Heavy soil: 0.35 kg cm-2

Therefore,

Average soil resistance (Fx) = 13 cm2×0.35 kg cm-2

Average soil resistance (Fx) = 4.55 kg

Assumed that the soil resistance to be 3–5 times higher than actual average soil resistance (Fx). (Shrivastava and Satyendra, 2011)

Draft at the tip of the tyne (Dt) = Fx×3(assumption)

= 4.55×3

Draft at the tip of the tyne (Dt) = 13.65 kgf

Since 1 kgf = 9.81 N

Total Draft (D) =13.65×9.81 N

Total Draft (D) =133.90 N

2.1.1.2. Furrow opener

Furrow openers are one of the most important components of any seed drill as they create a furrow to place the seed at required depth before dropping the seed. In case of direct seeding, the design of the furrow opener plays an important role as they affect the sowing depth and lateral scattering of the seeds.

Furrow openers of shovel type have been selected in the design of this equipment. The main aim is to open the furrow having 5 cm depth and 5 cm width so, that the seed can be placed at a depth of 2–5 cm and maintain row to row distance for groundnut crop is 30 cm (Reddy et al., 2015)

2.1.1.3. Design of furrow opener

In designing a furrow opener, the height of the furrow opener (H) plays an important role and it is calculated by using the following formula (Chaudhuri, 2001).

Height of the furrow opener, H=amax+H1+∆H

Where,

amax = Depth of the tool, cm

H1 = Length of the tyne, cm

∆H = Length of tyne used for fastening with the frame, cm

Here amax, H1, ∆H were considered as 5, 32 and 8 cm. Hence, the maximum height of furrow opener is calculated as 45 cm.

Distance (a) of the draft application on the furrow opener is said to be at a height of h/3 from the base of the opener.

Therefore, Distance of draft application,

A = h/3 = 45/3 = 15 cm

Moment arm length = h-a = 45-15 = 30 cm

Bending moment in tyne = Distance of draft application × Moment arm length (Kurtz et al., 1984)

D (h=a) = 13.65 (23.34) = 409.5 kg cm

= 409.5×9.81×10 N-mm = 40171.95 N-mm

By taking the value for factor of safety as 5, the maximum bending moment of tyne is given as

Maximum bending moment in tyne = Bending moment in tyne × Factor safety

= 40171.95×5 N-mm = 200859.75 N-mm

It is assumed that M.S flat tyne is used in the planter and fb for mild steel is 56 N/mm2

Section modulus, Z = Mb/Fb = 200589.75/56 = 3586.78 mm3

From this section modulus of tyne, the minimum thickness of the plate is taken and is given by the following equation

Z = (1/6) × t (H)2

Where,

Z = Section modulus of tyne, mm3

t = Thickness of the flat piece, mm

H = Height of the furrow opener

2438.89 = (1/6) × t (45)2

t = 10.6 mm

t = 10.6 mm = 1.6 cm

Hence, for the design of the furrow opener of the planter a M.S flat having minimum length of 45 cm and thickness of 1.6 cm was selected.

2.1.1.4. Design of frame

The components of the seed sowing are mounted on the main frame (chassis) which is supported a stand attached to the frame. The machine is also connected to the rigid hitch point of the prime mover. The design was based on the stresses produced on the front of frame (Lal and Dutta, 1995). Frame was subjected to torsion and bending due to induced draft (Ramya et al., 2020).

The total draft acting on the tyne measured as 13.65 kg. As three tynes are to be fastened on the frame. Total draft in kg.f is given as

Draft (D) =13.65×5 kg.f = 68.25 kg.f

Total Draft (D) = Draft (D) × Factor of safety

Torque acting on the bar = Draft (kg) × Factor of safety × Ground clearance (cm)

Taking ground clearance available 45 cm and Factor of safety for mild steel 3,

Torque, T = 68.25 × 3 × 45 = 9213.75 kg cm

In addition to the torque, bending moment would also be produced. The bar was considered as simple supported beam on the frame in between the five furrow openers.

Maximum bending moment (kg cm) = D × L

Where,

D = Total draft on the frame, kg cm = 204.75 kg.f

L = Length of the frame, cm = 180 cm

Maximum bending moment (kg cm) = 204.75 × 180 =36,855 kg cm

Maximum bending moment (kg cm) = 36,855 kg cm.

Equivalent torque due to torsion and bending moment,

Te = (Mmax2+T2)1/2

Te = (368552+9213.752)1/2

=37989.26 kg cm

2.1.1.5. Design of seed hopper

The capacity of hopper should be such that it gives minimum number of refilling per hectare area and does not cause high additional weight on the machine. The capacity of the hopper depends on the seed rate of different crops a 15 kg capacity hopper was designed.

Hopper capacity was calculated based on the quantity of the material to be filled in the box at a given bulk density (Varshney et al., 2004). The volume of the hopper is calculated as given below:

Let,

V = Volume of box, m3

ρ = Bulk density of material for groundnut = 286 kg/m3 (Reddy et al., 2015)

Q = Hopper capacity, kg= 15 kg

Then,

V = Q/ρ

= 15/283 = 0.065 m3 = 53000000 mm3

V = A×L

Where,

A = Area of cross section of box, mm2

L = Length of box (Assume 600 mm)

So,

A = V÷L

= 53000000 ÷ 700

= 75714.28 mm2 ≈ 75714 mm2

Section X is rectangular, section Y is trapezoidal, and Z is rectangular.

Cross section area of section X = B×H

Where,

B = Width of rectangular section, mm = 260

H = Height of rectangular section, mm =140

So,

Area of X = 60×140 = 36400 mm2

Cross section area of section Z = b’ x h’

Where,

b’ = Width of rectangular section, mm = 65

h’ = Height of rectangular section, mm = 140

Cross section area of section Y = 1/2×(a + b)×h

Where,

H = Height of trapezoidal section, mm

a = Top width of trapezoidal section, mm=260

b = Base width of trapezoidal section, mm = 65

So, Area of Y = 75714–36400–9100 = 30214 mm2

Area of Y = 1/2×(260+65)×h = 30214 mm2

h =185.9≈186 mm

The seed hopper was divided in three sections as upper, middle and bottom. The upper section is rectangular in cross section having width of 260 mm, height of 140 mm and length of 300 mm length as dimensions. The middle section is trapezoidal in cross section provided with 45o slopes from two sides for free flow of seed. Its dimensions were 260 mm of top width, 65 mm of bottom width, 186 mm of height and 300 mm of length. The bottom section is rectangular in cross section having width of 65 mm, height of 140 mm and length of 300 mm. The schematic diagram as shown in Figure 1.

Fig. 1. Designed CAD view of seed box

2.1.2 Seed sowing metering mechanism

The required seed metering mechanism for ground nut was designed. The required seed rate for groundnut crop was 100–130 kg ha-1 (Singh, 2017). As working width of developed equipment was 1.8 m, with ground wheel diameter 0.30 m, 7078 revolutions of drive wheel were required to cover area of one hectare.

Considering drive wheel to seed metering mechanism speed ratio as 1.5:1, the seed rate per revolution of metering mechanism was given by,

= 120/7078 = 0.0169 kg revolution-1

= 16.9 g revolution-1

Five furrow openers were provided on implement and hence seed rate per revolution per furrow opener was given by,

= 16.9/5 = 3.38 g revolution-1

3.38 grevolution-1 seed equivalence to seed of 11.8 cc revolution-1 in terms of volume. To suit the requirement of 3.38 g revolution-1 delivery of seed vertical rotor with cells on its periphery (Edge cell rotor) was chosen. Plastic metering rotor with 95 mm diameter, 12 mm width and 10 grooves on its periphery was selected based on its availability.

2.1.3 Chain and sprocket

A single strand bicycle chain was selected for transmitting power from ground wheel to metering shaft. The arrangement consisted one sprocket of 18 teeth and two sprockets of 12 teeth. The number chain links, length of chain and central distance between two sprockets were determined as discuss below.

The length of first chain (L) attached from ground wheel to 1st idler sprocket was calculated by using following formula (Khurmi and Gupta, 2011):

The number of links, m, where determined by following relationship.

m=(2Cd/P)+(T1+T2)/2+{(P(T1-T2)2)/(4π2×Cd )}

Where,

Cd = Centre distance, mm

P = Chain pitch, mm (12.5)

T1 = Number teeth of 1st sprocket

T2 = Number teeth of 2nd sprocket

On substituting the values of Cd, P, T1 and T2

m=(2×435/12.5)+(18+18/2)+{12.5(18-18)2)/(4π2×435)}=87.6 ≈88

The length of first chain (L1) was calculated as number of links into chain pitch i.e. 1075 mm. Similarly, the number of links, center to center distance between sprockets and length of Second chain (L2) measured to 388 mm.

Table 1. Specification of chains used for seed metering

| S. No. | Placement | Centre to Centre distance, mm | Number of links | Length of chain, mm |
| --- | --- | --- | --- | --- |
| L1 | Chain on ground wheel shaft to intermediate shaft | 435 | 88 | 1075 |
| L2 | Chain on intermediate shaft to main shaft | 157 | 112 | 388 |

2.1.4 Design of pegged type ground wheel

Pegged wheel was suitable for use under wet or sticky soils, where plain, lugged or even pneumatic wheels fail to work hence pegged type ground wheel was used in the present study. The diameter of the wheel and the number of pegs depends on the size of wheel. Here, the diameter of the ground wheel was selected as 40 cm and number of pegs on the ground wheel was 12 (Verma, 1986). The pegs were made up of mild steel round bars or flat bars having pointed edge towards the ground surface. The motion at any given point on the wheel follows a cycloid path. The pegs enter the ground vertically and leave the surface upwards. Hence, soil coming in contact with the peg was pushed downward but not taken upwards. The diameter of ground wheel is calculated by using the following equation

Diameter of ground wheel, D = (n×p)/π

Where,

D = Diameter of drive wheel, cm

n = No. of seed dropped in one revolution

p = Plant to plant distance, cm

Ground wheel of 30 cm diameter, having width and thickness of 5 and 1 cm respectively was selected and provided at back side of the main frame of the drill. Twelve pegs (10×4.0×0.5 cm2) (Figure 2) were provided on the outer periphery of the wheel to get sufficient grip during rotation.

Power developed due to rotation of the wheel was transmitted to the seed metering shafts with the help of two sets of chain and sprocket arrangement. As the drive wheel enters in to the soil it is considered that the half of the peg is inserted in to the soil for better gripping and to avoid skidding of drive wheel. Hence, diameter of the drive wheel was measured from the tip of peg at one end to the base of the peg of opposite side.

2.1.5 Design of power transmission system from ground wheel to metering device

The power required for the seed metering device is generated from the ground wheel and is transmitted by using chain and sprocket. In order to reduce the loss of power during transmission, a dual set of chain and sprocket arrangement one for transmitting the power form ground wheel to an idler shaft on which the second set of sprocket was arranged, from that in turn a chain was arranged to the metering shaft of the drill. The speed ratio of ground wheel and seed metering device is given as a function of number of teeth on the sprockets and is calculated by the following formula.

Ng/Ns = Ts/Tg (Khurmi and Gupta, 2005)

Here,

Ng = Number of revolutions of the ground wheel, rpm

Ns = Number of revolutions of the seed metering device, rpm

Tg = Number of teeth on the sprocket of ground wheel (= 18)

Ts = Number of teeth on the sprocket of the seed metering device (=18)

Ng/Ns = 18/18 = 1

Fig. 2. Line diagram of ground wheel

Thus, the speed ratio of 1:1 was used in the present study. It implies that number of revolutions made by the ground wheel in a minute is same as the one made by the seed metering device.

2.1.6 Design of weeding blade

2.1.6.1. Functional requirements of weeding blade

Different components of weeding machine were designed for its functional and structural requirement. Following functional requirements were considered for design of the weeding blade:

- The weeding blade should cut and uproot weeds present on surface in a single run.

- The weeding should minimum invert the soil with minimum disturbance.

- Damage to crop during weeding operation should be as low as possible.

- The weeding tool should be simple in design and construction, and efficient in its performance.

The following design parameters were considered during the development of sweep type weeding tool for various spacing of crop (Sharma and Mukesh, 2010). CAD representation of prototype with weeding blade as shown in Figure 3.

2.1.6.2. Design of weeding blade

The cutting width of sweep type blade were found by using the formula:

Sc = Zf+Zp

W = Zf = Sc-Zp

Where,

Sc = Crop spacing, cm

Zf = Effective soil failure zone, cm

Zp = Protection zone, cm = 6 cm

W = Width of sweep, cm

[Protection zone multiplied by 2 since protection zone has to be provided on both sides of crop]

Zf = 30-(6×2) = 18 cm

Zf = W+2d tan Øs

Where.

Zf = Effective soil failure zone, cm

W = Cutting width of sweep, cm

D = Depth of weeding, cm

Øs = Angle of internal friction which ranges between 10 o to 30 o depending upon type of soil (Laxminarayana and Vikas, 2018) = 20o

18 = W+2(6) tan 20o

W = 13.6 cm ≈ 14 cm

2.1.7 Design of spraying unit

2.1.7.1. Design for tank capacity

The capacity of tank of tractor drawn sprayers depends upon location of water supply, area covered, hydraulic lift capacity of tractor and boom length of sprayer. Generally, 100–500 litre capacity of tank was taken (Sharma and Mukesh, 2010). It should be such that it can provided liquid for treatment of one hectare area. It can be estimated as under

Qt = Db×t

Where,

Qt =Tank capacity, litres

Db =Total discharge rate of all nozzles of the boom, l/min

t = Duration of use, min

Qt = 2.25×60 = 135 lit

2.1.7.2. Design of sprayer tank size

The rectangular hollow frame was fabricated for mounting the sprayer tank in to the frame. The length, width and height of the frame were 440 mm, 240 mm and 310 mm respectively. The sprayer tank was designed to occupy solution 150 litres in one filling. Two PVC tanks of each 75-litre capacity were selected. Pump was arranged on the tractor with the help of frame and it was fixed on the top of the three-point linkage. Pump was to pressurise spray solution through and contributed through hose pipes to 3 nozzles.

2.1.8 Power requirement and drive arrangement of pump

Power requirement of spray pump was calculated by formula suggested by FAO 112/2 (Ramana and Jesudas, 2009)

Power requirement = (Output volume (Lpm) x pressure kPa ×0.746) / (44800×pump efficiency)

The power requirement of 0.71 kW is well within the tractor PTO power. Drive arrangement to pooled subtieady the HTP. The designated speed of 900 rpm of the HTP pump was attained by giving the pulley ratio which is mounted on the PTO splined shaft (540 rpm) and pump pulley. The V belt pulley used for transmitting power from PTO shaft to the HTP pump through A type belt.

Fig. 3. CAD representation of prototype with reversible furrow opener

Fig. 4. CAD representation of prototype with weeding blade

3. RESULTS AND DISCUSSION

The developed multipurpose equipment effectively performed sowing, spraying, and weeding operations in groundnut crop. The integrated design included an edge cell seed metering system, a 150 L adjustable height spraying unit, and a cast iron blade weeder. Field evaluation showed that combining spraying and weeding in a single pass reduced fuel use and labor, resulting in lower cultivation costs and improved operational efficiency.

Table 2 : Study of Crop Parameter

| S. No. | Groundnut crop parameters | Dimensions |  |
| --- | --- | --- | --- |
| 1 | Average plant height, cm |  | 25.3 |
| 2 | Average plant width, cm |  | 10.1 |
| 3 | Row to row spacing, cm |  | 30 |
| 4 | Plant to plant spacing, cm |  | 10 |

Table 3 :  Overall Specifications of Developed Equipment

| S. No. | Particular | Specification |
| --- | --- | --- |
| 1 | Name of implement | Multipurpose equipment |
| 2 | Make | ANGRAU |
| 3 | Model | Prototype |
| 4 | Type of power source | Mini tractor |
| 5 | overall width of implement, cm | 60 |
| 6 | Overall length of implement, cm | 180 |
| 7 | Overall height of implement, cm | 170 |
| 8 | No of tynes | 5 |
| 9 | Tynes, cm | 45 |
| 10 | Tyne width, cm | 5 |
| 11 | Tyne thickness, cm | 1.6 |
| 12 | Furrow opener(L×W×T), cm | 32×3×3 |
| 13 | Weeding blade (L×W×T), cm | 14×10×0.5 |
| 14 | No of ball bearing | 2 |
| 15 | No of pulleys & belts | 2 &1 |
| 16 | No of nozzles | 3 |
| 17 | Type of pump | Horizontal triplex piston pump |
| 18 | No of spray tanks | 2 |
| 19 | Each spray tank capacity, L | 75 |
| 20 | Type of nozzle | Solid cone |
| 21 | Nozzle spacing, mm | 30 |
| 22 | Type of linkage system | 3- point linkage |

Fig. 5 Developed multipurpose equipment with reversible tyne

Fig. 6 Developed multipurpose equipment with weeding blade

3.3 Laboratory Evaluation For Multipurpose Equipment

Laboratory tests were performed after fabrication of multipurpose equipment. During these tests, key parameters such as seed rate, discharge rate, and swath width were measured and evaluated.

3.3.1 Laboratory Evaluation for Seeding Unit

The seeding unit calibration was performed in the laboratory before the field assessment. Results are presented in Table 4. The average weight of the seeds obtained from seeding unit delivery tube after 20 ground wheel revolutions was 340 gm. The quality of seed required to sow seeds in one hectare of area with developed seeding unit is noticed to be 120.2 kg ha-1.

Table 4. Calculation of seed rate for developed seeding unit

| S. No. | No of revaluation of ground wheel | Weight of seed collected (kg) | Seed rate (kg ha-1) |
| --- | --- | --- | --- |
| 1 | 20 | 0.34 | 120.3 |
| 2 | 20 | 0.33 | 120.1 |
| 3 | 20 | 0.34 | 120.3 |
| 4 | 20 | 0.34 | 120.3 |

3.3.2 Laboratory Evaluation of Spraying Attachment

3.3.2.1 Discharge rate of spraying attachment

Operating pressures of 150, 200 and 275 kPa was used for performance evaluation of boom sprayer set up as per recommended pressures (Robert et al, 2013). Discharge rate of spraying attachment varied from 0.9 to 1.3 L min-1 as change in operating pressure of 150 to 275 kPa. Minimum discharge rate was obtained at 150 kPa and maximum at 275 kPa. Variations in discharge rate of nozzle were due to change in operating pressure. Fig. 7 shows the effect of operating pressure on discharge rate of all three nozzle.

Fig. 7 Effect of operating pressure on discharge rate of nozzle

3.3.2.2 Swath width of spraying attachment

Boom sprayer was tested at optimum working conditions such as 0.5 m height (Zhu et al.,2004) from target, operating pressures of 150, 200 and 275 kPa for solid cone nozzles. Swath width of spraying unit varied from 1.01 to 1.36 m and overlap with adjacent nozzle spray was varied from 23% to 44%. Maximum swath width and overlap was observed at operating pressure of 275 kPa. Mean values of swath width and overlap of sprayer was presented in Table 5.

Table 5 Mean values of swath width and overlap of spraying attachment

| Operating pressure (kPa) | Swath width of boom sprayer (m) | Overlap (%) | Overall swath with (m) |
| --- | --- | --- | --- |
| 150 | 1.7 | 23 | 1.01 |
| 200 | 2.07 | 33 | 1.13 |
| 275 | 2.42 | 44 | 1.36 |

4.  CONCLUSION

The developed multipurpose equipment, integrating sowing, spraying, and weeding functions, was successfully fabricated and tested in groundnut fields. The spraying unit, with a 150 L capacity and three adjustable nozzles, effectively covered three crop rows, while the weeding unit, equipped with a cast iron horizontal blade, efficiently removed inter-row weeds. Field tests demonstrated that simultaneous spraying and weeding operations reduced both fuel consumption and overall cultivation costs compared to separate operations. Laboratory evaluations confirmed the equipment's performance. The average seed delivery from the seeding unit was 340 g per 20 revolutions of the ground wheel, corresponding to a seed rate of 120.2 kg ha⁻¹. The discharge rate of the spraying unit varied from 0.9 to 1.3 L min⁻¹ across operating pressures of 150 to 275 kPa, with swath widths ranging from 1.01 to 1.36 m and nozzle overlap between 23% and 44%. These results highlight the effectiveness and efficiency of the developed equipment in reducing operational time and improving input utilization in row crop cultivation. Field trails revealed that using this multi-purpose equipment can reduce operational costs by 68%.

DISCLAIMER (ARTIFICIAL INTELLIGENCE)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc) and text-to-image generators have been used during writing or editing of this manuscript.

ACKNOWLEDGEMENT

The authors acknowledge Dr. NTR College of Agricultural Engineering, Acharya N.G. Ranga Agricultural University for funding and facilities provided to the research.

4. REFERENCES

Akki, S., Bangi, N., Batta, S., Basani, N., & Balachand, C. (2021). Ergonomical assessment on hand operated chaff cutter by rural women workers. International Journal of Bio-resource and Stress Management, 12(5), 490-495.

Allison, F.E., 1968. Soil aggregation. Some facts and fallacies as seen by a microbiologist. Soil Science 106, 136–143.

Anonymous, 2013. State of Indian Agriculture 2012-13. Directorate of Economics and Statistics, Department of Agriculture and Cooperation, Ministry of Agriculture, New Delhihttp://164.100.47.132/paperlaidfiles/AGRICULTURE/State%20.

Aryal, J. P., Thapa, G., & Simtowe, F. (2021). Mechanisation of small-scale farms in South Asia: Empirical evidence derived from farm households survey. Technology in Society, 65, 101591.

Chaudhuri, D., 2001. PM-power and machinery: performance evaluation of various types of furrow openers on seed drills-a review. Journal of Agricultural Engineering Research 79(2), 125–137.

Ebrahim A., Hamid Reza., Mohammad M and Kyeong U K. 2008. Development of a precision seed drill for oilseed rape. Turkish Journal of Agriculture and Forestry. 32. 451-458.

Khandetod, 2019. Mechanization in horticulture crops: present status and future scope. Advanced Agricultural Research & Technology 03, 92–103.

Khurmi R.S. and Gupta J.K., (2005). A Textbook of Machine Design. 4th Edition, Eurasia publishing house (PVT.) LTD. Ram Nagar, New Delhi, India

Khurmi, R.S., Gupta, J.K., 2011. Machine design. S. Chand publications. 120–180.

Kurtz G, Lester T and Paul C. 1984. Design of Agricultural Machinery, pp 245–55. John Willey and Sons, Singapore.

Lal, R., Dutta, A.C., 1995. Agricultural engineering (through worked examples), Saroj Prakasan, Allahabad.

Mehta, C.R., Pajnoo, R.K., 2013. Role of Japan in promotion of agricultural mechanization in India. Agricultural Mechanisation in Asia, America and Latin America 44(4), 15–17.

Ramana, C., Jesudas, M.D., 2009. Parasol sprayer for efficient chemical application in dwarf and semi dwarf mango orchards. Agricultural Mechanization in Asia, Africa, and Latin America 40(3), 9–14.

Ramya, K., Lokesh, M., Ravi, C.C.K., 2020. Design development of zero till cum fertilizer drill coulter type seed. Agrotechnology 09, 1–6.

Reddy, K.M., Kumar, D.V., Reddy, B.R., Reddy, B.S., Reddy, G.A., Munaswamy, V., 2015. Design and development of herbicide spraying technology while in sowing for groundnut. Progressive Agriculture 15(1), 21–27.

Robert, G., Pat, H., Shawn, D.A., Lloyd, H. and David, M. 2013. Nozzles: Selection and Sizing. Virginia Cooperative Extension. www.ext.vt.edu.

Sarkar, A. (2020). Agricultural mechanization in India: A study on the ownership and investment in farm machinery by cultivator households across agro-ecological regions. Millennial Asia, 11(2), 160-186.

Sharma, D.N., Mukesh, S., 2010. Farm machinery design principles and problems (2nd edition). Jain brothers.

Shrivasta, A.K., Sathendra, J., 2011. Modification and performance evaluation of tractor drawn improved till plant machine under vertisol. Agricultural Engineering International: the CIGR 13, 1–11.

Singh, S., Singh, R.S., Singh, S.P., 2015. Farm power availability on Indian. Farms, Agricultural Engineering Today 38(4), 44–52.

Singh, T.P., 2017. Farm Machinery. PHI learning private limited. 110.

Varshney, A.C., Kumar, U., Naik, R.K., 2004. Agricultural machine design, CIAE/2004/01.  210–245.

Verma, S.R., 1986. Seed cum fertilizer drills in India. International Conference on Small Farm Equipment for Developing Countries. International Rice Research Institute. 2–6 September, Philippines, 255–285.

Zhu, H., Dorner, J.W., Rowland, D.L., Derkshen, R.C. and Ozkan, H.E. 2004. Spray penetration into peanut canopies with hydraulic nozzle tips. Biosystems Engineering. 87(3): 275-283.