Sand Dynamics and Utilization in the Geographical Context of Fluvial Geomorphology and Watershed Hydrology

# ABSTRACT

This review paper delves into both the simple and complex relationships between sand dynamics, geographical landscapes and their significant implications for fluvial geomorphology and watershed hydrology. The conceptual framework classified sand and discussed its long and complex formation and transformation processes. The concepts of sand budget and sand dynamics were also addressed to explain their relevance in determining the amount of material that can be removed from an environment without causing undue erosion or degradation, either at the site or at a nearby location, upstream or downstream. Sand, a fundamental constituent of river systems, plays a pivotal role in shaping river morphology and influencing hydrological processes within watersheds.  This next section on sand mining and utilization synthesized a broad spectrum of research, integrating established findings from various geographical contexts to provide a comprehensive understanding of the spatio-temporal nature and magnitude of sand extraction globally. On the effects of sand mining, another section reviewed the geomorphological and hydrological mechanisms governing sand transport, deposition and erosion, elucidating their relationships with fluvial processes. It scrutinizes the influence of geomorphic factors such as channel morphology, sediment supply and discharge variability on sand movement and distribution within river systems. Furthermore, the section explored the diverse impacts of human interventions and anthropogenic activities on sand dynamics, shedding light on the alteration of natural sediment regimes and its repercussions on fluvial ecosystems. Moreover, the review investigated the utilization of sand resources and its physical and socio-economic implications, emphasizing the balance required between exploitation for construction purposes and preserving environmental integrity. Positive and carefully calculated approaches to sand mining leads to little or no consequences on the river ecosystem. The last section discussed management strategies aimed at sustainable sand extraction practices to mitigate adverse effects on river ecosystems and watershed hydrology. A framework by the Indian Ministry of Environment was presented on how to save watersheds from the impacts of anthropogenic activities. In summary, this review consolidated and synthesized the current state of knowledge regarding sand dynamics, identifying critical gaps and understanding avenues for future research for managing sand resources within river landscapes.

KEY WORDS: Sand formation, Sand Dynamics, Sand Budget, Watershed, Geomorphology, Hydrology,

# 1.0 INTRODUCTION

Sand has various definitions based on chemical, physical, and genetic perspectives. Chemically (Shaffer, 2006:2), it is mainly composed of silica (SiO₂), while physically, it is defined as loose, granular material between 0.0625 and 2.00 mm in size (Barksdale, 1991). Genetically, sand results from the weathering of rocks through mechanical forces (Meador and Lahyer, 1997). It is found in a range of environments such as riverbeds, shorelines, and deserts, and is classified along with gravel as aggregates—the second most consumed natural resource globally after water.

Sand deposits are widespread, but industrial-quality sands are rarer and often require extensive exploration (Pitchaiah, 2017). While deserts seem abundant in sand, not all sand types meet construction standards, leading to significant imports even in desert nations. For example, Qatar and the United Arab Emirates imported sand worth billions of dollars despite their desert environments, underscoring the disparity between sand availability and usability (Churchill, 2016). A good measure of aggregate consumption is cement production, as sand and gravel are core components of concrete. In 2012, global cement production reached 3.7 billion tonnes, which translates to about 25.9 to 29.6 billion tonnes of aggregates used. Countries like China, India, and the U.S. dominate global cement output, with China alone accounting for more than half of the production (Peduzzi, 2014). The demand for aggregates continues to rise with urban development and infrastructure expansion.

Beyond construction, aggregates are vital in land reclamation, shoreline protection, and road building. Global data for these uses is limited, but the scale is massive—China alone constructed over 146,000 km of roads in a single year. Considering all these uses, global aggregate consumption exceeds 40 billion tonnes annually, surpassing the sediment transported by all rivers combined (Peduzzi, 2014). This highlights humanity’s profound impact on Earth’s geological systems.

# 1.1 CONCEPTUAL FRAMEWORK

# 1.1.1 Classification of Sand

Gavriletea (2017) classified sand based on formation, composition, and grain size. Shaffer (2006) noted that sand’s physical characteristics (Table 1) reveal its origin, history, and uses. While sand can contain various minerals, industrial sand is mainly composed of quartz (Bolen, 1996; Shaffer, 2006). Tropical sands may also include olivine, ferromagnesian, and black oxide minerals, depending on local geology (Figure 1). Nonetheless, most sand formations mainly consist of quartz (Barksdale, 1991).

Figure 1: Classification of Sand	          Source: Gavriletea (2017).

Table 1: Structural Classification of Sand and other Materials

| SN | Soil aggregate | Size (mm) | Surface area (cm3/gm) | Visibility |
| --- | --- | --- | --- | --- |
| 1 | Clay | < 0.002 | 23,000 | Electron Microscope |
| 2 | Silt | 0.002-0.05 | 2,100 | Light Microscope |
| 3 | Very fine sand | 0.05-0.10 | - | Light Microscope |
| 4 | Fine sand | 0.10-0.25 | 210 | Natural Eyes |
| 5 | Medium sand | 0.25-0.50 | - | Natural Eyes |
| 6 | Coarse sand | 0.50-1.00 | 21 | Natural Eyes |
| 7 | Very coarse sand | 1.00-2.00 | - | Natural Eyes |
| 8 | Gravels | 2.00-100 | - | Natural Eyes |
| 9 | Boulders | >100 | - | Natural Eyes |

Source: Compiled from Barksdale (1991) and Shaffer (2006).

# 1.1.2 Sand Formation and Renewability

Sand is primarily a product of weathering and its distribution is basically a product of transportation; it is the final product of rock weathering which is an important part of the rock cycle (Adamu and Abdulazeez, 2019). Because it takes thousands to millions of years for sand to form via the rock cycle and weathering, it is virtually a non-renewable natural resource. Sand and its solidified form, sandstone, are both very important geologic products that have undergone extensive research (Pettijohn, 1975 in Shaffer, 2006). “Forces that produce sand and its parent materials occur all over the Earth thereby making its occurrence to be universal. Many sands and all the highest quality sand deposits are made of quartz. The two most abundant chemicals on Earth, Si (28%) and O (47%), combine to form quartz” (The Mineral Gallery, 2004; Abdulazeez,2016) which is the most common mineral in the crust of the Earth comprising an estimated 35% of all rocks (Shaffer, 2006).

The formation of sand begins with the weathering of quartz-bearing rocks—igneous, sedimentary, or metamorphic—and is part of the Earth's continuous rock cycle. Weathering separates mineral grains from cemented rock structures, after which transportation processes, primarily driven by water, carry the grains to different depositional environments. As sand grains travel, weaker minerals are removed, and the grains become smaller, more rounded, and chemically purer. Repeated transportation and deposition cycles lead to mature sands that are well-sorted and rich in quartz—sometimes exceeding 95% purity—making them valuable for various uses.

Rivers serve as the most efficient transporters of sediment, moving particles of all sizes through processes such as bedload movement, saltation, suspension, and solution. The transport capacity of a river depends on factors like water velocity, turbulence, and the roughness of the riverbed. River dynamics are complex, constantly reworking and redistributing sand deposits across floodplains and terraces. Some sand mining operations depend on this natural replenishment cycle, extracting sand from areas where sediments are continuously supplied by flowing water.

# 1.1.3 Sand Dynamics

“Many features of sand like mineral compositions, grain size distribution and grain roundness and other details are studied not only to understand its genesis and formation, but also to determine suitability for human uses” (Zdunczyk and Linkous, 1994). “Sand is a key component in a variety of construction products, including cement, mortar, tile, brick, glass, adhesives, and ceramics, as well as water filtering, chemical and metal processing, and the plastics sector. Due to population increase and rising living standards, these various uses resulted in exponential consumption growth, which is predicted to continue” (Gavriletea, 2017).

According to Shaffer (2006:10), “industrial sand, silica sand, high quartz sand, or simply silica are all terms used to describe pure quartz sands. Quartz sand is widely used for water filtration and the production of glass, with varied levels of purity and grain size required for different types of glass. Heat-resistant moulds or cores for casting metal parts are made by mixing raw, angular quartz sand with clay. Precision coatings, such as jewellery, high-precision items, or dental devices, are made with fine sand. Quartz is used in ceramics to provide toughness and robustness, as well as glazes. Quartz is used to manufacture refractory (temperature-resistant) bricks for lining furnaces, and fused quartz can be made into a variety of intricate shapes for laboratory apparatus” (Zdunczyk and Linkous, 1994; Bolen, 1996 and Shaffer, 2006). Shaffer (2006) summarized other salient uses of sand to man thus:

Refined quartz produces a huge range of silicon chemicals used in drugs, cleaners, and pharmaceuticals and also the silicon chips that power our computerized world. Quartz can provide silicon metal or ferrosilicon which is an alloying agent for various metals. Silicon carbide, an important abrasive made from quartz and natural quartz, both have been used for a myriad of abrasive tasks such as sandpaper. Quartz sand is used as a filler or extender for such products as paints, plastics, gels, and other suspensions. It imparts considerable toughness to rubber or plastics, and can provide some temperature resistance.

# 1.1.4 Sand Budget

“In general, sediment budget is defined as the material transport volume balance for a selected segment of the coast or river. The elements of the budget are processes that increase (sources) or decrease (sinks) the quantity of sand in a defined control volume” (Coastal Engineering Research Centre, 1984). “Determining the sand budget for a particular stream or catchment requires site-specific topographic, hydrologic, and hydraulic information. This information is used to determine the amount of sand that can be removed or mined from the area or environment without causing undue erosion or degradation, either at the site or at a nearby location, upstream or downstream” (Ab.Ghani et.al., 2003).

Sand and gravel mining in or near the channel alters the sediment budget and can cause significant temporal and spatial changes in channel hydraulics (Brown, 1996). “Minimizing the detrimental consequences of sand-and-gravel mining necessitates a thorough understanding of the channel's sensitivity to perturbations caused by mining. The definition of a reference state, i.e., a minimally acceptable or agreed-upon physical and biological condition of the channel, as well as the sand budget, is required for decisions on where to mine, how much, and how often” (Chauhan, 2010).

# 1.1.5 Sand Environments

“Sand mining occurs in various environments, such as beaches, inland dunes, ocean floors, river beds and deltaic regions” (Pitchaiah, 2017). “Sand was previously taken in land quarries and riverbeds all over the world; however, due to the depletion of inland resources, a shift to marine and coastal aggregates mining has occurred. River and marine aggregates are still the most used building and land reclamation materials” (Kondolf, 1997). Because the wind erosion process generates spherical granules that do not bind effectively, most desert sand cannot be used for concrete or land reclamation (Zhang et.al., 2006).

Worldwide, rivers are the preferred source of sand and gravel for a number of reasons listed by Koehnken and Rintoul (2018). River energy grinds rocks into gravels and sands, obviating the cost of mining, grinding, and sorting rock, and the material created by rivers tends to be durable minerals with angular shapes, which are ideal for construction. River sand deposits also have the advantages of being naturally sorted by grain size, being easily accessible, and being able to be carried cheaply by barges. There are cities located near rivers so transport costs are low (Koehnken and Rintoul, 2018).

# 1.2 SAND MINING AND UTILIZATION

“Sand and associated sediments mining is defined as the removal, excavating, extracting, quarrying or dredging of sand, gravel, rocks, boulders and other deposits from the bed, bank, or floodplain of a river, or from a river reserve” (Environmental Conservation Department, 2000). “Sand mining can also occur on coastal and inland environments like dunes, abandoned channels, gullies and deposits” (Dhakwa et.al., 2005). Going by the fact that it can be of different scales and dimensions, Saviour and Stalin (2012) simply defined sand mining as the process of removal of sand and gravel at a level that makes it become an environmental issue. Similarly, UNEP (2014) described it as “the temporary or permanent lowering of the productive capacity of land”.

# 1.2.1 Nature and Scope of Sand Mining

“Globally, sand mining is mainly carried out on rivers and associated environments where the material is readily available for extraction” (Gurubachan-Singh et.al., 2007). “Typically, in the case of inland sand mining, operations begin with the removal of overburden which is often hauled to the perimeter of the mine site and piled into beacons. Once the overburden has been removed, the sand is excavated. Depending upon the geological formation, blasting may be used to make the sand containing materials more amenable to excavation” (Hill and Kleynhans, 1999). Atejioye and Odeyemi (2018) noted that “large scale excavations involve heavy equipment or rubber-tired front-end loaders. In most other small-scale inland sand mining sites in Ado-Ekiti, Nigeria, the excavated sand materials are loaded on trucks by hired labourers using shovels” (Atejioye and Odeyemi, 2018).

“As sand is an essential component in the provision of shelter, the scope of its use is global. Sand mining is a universal phenomenon that no country in the world can claim to be completely free from its direct and indirect benefits and challenges” (Starnes, 1983). “Demand for sand and gravel relates to the increasing need for construction materials, which accounts for approximately 96% of the total amount of mined sand and gravel” (Langer, 1988). “The remaining 4% is used for foundry operations, glass manufacturing, abrasives, and filtration beds in water treatment facilities. Of the sand and gravel used in construction, approximately 43% is used for residential and other buildings” (Langer, 1988).

“According to the United States’ National Sand and Gravel Association, almost 91,000 kg of aggregate material (sand, gravel and crushed stone combined) are needed to construct a 6-room house, and approximately 14 million kilograms of aggregate are needed to construct a school or hospital” (Langer, 1988). “Although these values are approximations, they give an indication of the volume of material used in construction and how much our sand resources have to suffer” (Langer and Glanzman, 1993).

# 1.2.2 Spatial and Temporal Dimension of Sand Mining

One of the primary determinants of sand mining is location, which significantly influences sediment availability, extractability, movement, and replenishment. Time is also crucial, as seasonal variations affect mining activities—for instance, sand mining becomes more difficult during the rainy season (Abdulazeez, 2018). River channels and floodplains are vital sources of aggregates due to the durable and well-sorted nature of river-processed materials. The relevance of alluvial deposits depends on factors such as quality, location, processing needs, and the availability of alternative sources (Tepordei, 1992). In California, nearly all 120 million tonnes of construction aggregate produced annually are sourced from alluvial deposits (Carillo et al., 1990).

Understanding the spatial and temporal distribution, abundance, and quality of instream sand and gravel is essential for evaluating environmental issues linked to mining (Kondolf, 1995). Some experts propose limiting aggregate extraction to match the annual bed load to mitigate environmental damage. However, accurately estimating the annual bed load is complex due to its dynamic nature, varying hourly, daily, and annually (National Research Council, 1983).

Not all instream deposits are commercially viable; factors such as particle size, shape, hardness, chemical composition, and intended use determine their suitability. Commercial sand and gravel must be chemically inert and resistant to weathering and mechanical degradation (Kondolf, 1997). Instream gravel is particularly valued for its durability, having been naturally sorted and rounded through prolonged fluvial transport (Barksdale, 1991). Additionally, sediments in reservoirs, though largely untapped, are promising as building material sources. They offer the advantage of being size-sorted and their extraction helps reduce sediment accumulation, which in turn partially restores reservoir capacity (Kondolf, 1997). Economic considerations, particularly transportation, are also crucial. Transporting materials from extraction sites to demand areas constitutes the largest share of total mining costs. Consequently, sand and gravel extraction sites are generally located within 50–80 kilometers of the main demand zones and near transport routes to reduce expenses (Kondolf, 1997).

# 1.3 EFFECTS OF SAND MINING

Generally, the effects of sand mining can be positive or negative as well as immediate and long-term. Environmentally, there can be direct and indirect impacts of sand mining both on and off river ecosystems (Abdulazeez, 2018; Abdulazeez and Adamu, 2019). Direct impacts are those in which the extraction of material is directly responsible for the impact to the ecosystem, such as due to the loss of floodplain areas or removal of gravel, cobble or other habitat that is recognised as underpinning a specific ecosystem process. Indirect impacts are related to ecosystem changes that are promoted due to physical changes in the river system resulting from sand extraction. While studying the spatial concentration of rock minerals mining on the Lower Silesia region in Poland, Blachowski (2015:18) concluded that in the context of regional spatial policy, increase of rock minerals mining results in growth of regional economy but also leads to intensification of environmental and social problems.

# 1.3.1 Geomorphological and Hydrological Effects

“One of the principal causes of environmental impacts from in-stream mining is the removal of more sediment than the system can replenish” (Pitchaiah, 2017). “Even without morphological studies on the “impacts of instream sand mining on riverbank erosion, it has been established” (Sreebha, 2008) that when the rate of sand extraction exceeds the replenishment rate, significant and potentially irreversible changes occur in the hydraulic conditions and channel stability. The removal of material from a river channel can alter river hydraulics due to changes in the depth (Figure 2), width or slope of a river, which in turn can alter ecosystem processes” (Sreebha and Padmalal, 2010).

| Figure 2: Diagram of sand-and-gravel stream bed showing (A) the nick point that develops with a pit excavation, and (B) the upstream head cutting and downstream bed degradation that develop during high flows. Source: Hedge et.al. (2008) | Figure 3: Diagram of channel cross sections showing (A) a typical sand-gravel bar in relation to the low-flow channel, riparian zone and water table, and (B) the wide shallow channel that results from unrestricted mining characterized by bank erosion, braided flow and increased water temperatures. Source: Hedge et.al. (2008) |
| --- | --- |

“Head cutting mobilizes substantial quantities of streambed sediments which are then transported downstream to deposit in the excavated area and locations further downstream. In gravel-rich streams, effects downstream of mining sites may be short-lived when mining ends, because the balance between sediment input and transport at a site can re-establish itself relatively quickly” (Hedge et.al., 2008). “Effects in gravel-deficient streams may develop rapidly and persist for many years post-mining. Regardless of downstream effects, head cutting in both gravel-rich and gravel-deficient streams remains a major concern. Head cuts (Figure 3) often move long distances upstream and into tributaries, in some watersheds moving as far as the headwaters or until halted by geologic or man-made controls. A second form of bed degradation occurs when mineral extraction increases the flow capacity of the channel. A pit excavation locally increases flow depth (Figure 2) and a bar skimming operation increases flow width (Figure 3). Both conditions produce slower streamflow velocities and lower flow energies, causing sediments arriving from upstream to deposit at the mining site” (Hedge et.al., 2008). “As streamflow moves beyond the site and flow energies increase in response to the "normal" channel form downstream, the amount of transported sediment leaving the site is now less than the sediment carrying capacity of the flow. This sediment-deficient flow or "hungry" water picks up more sediment from the stream reach below the mining site, furthering the bed degradation process (Figure 3). This condition continues until the equilibrium between input and output of sediments at the site is re-established” (Hedge et.al., 2008).

“A similar effect occurs below dams, which trap sediment and release "hungry" water downstream, where channel incision usually ensues. Instream mineral excavation downstream of dams compounds this problem. Although other factors such as levees, bank protection and altered flow regimes also promote channel incision, mineral extraction rates in many streams are often orders-of-magnitude in excess of sediment supply from the watershed, suggesting that extraction is largely responsible for observed channel changes. Susceptibility to hungry-water effects depends on the rate of extraction relative to the rate of replenishment. Gravel-poor streams would be most susceptible to disturbance. Channel incision both causes vertical instability in the channel bed, and lateral instability in the form of accelerated stream bank erosion and channel widening. It increases stream bank heights, resulting in bank failure when the mechanical properties of the bank material cannot sustain the material weight. Channel widening causes shallowing of the streambed (Figure 3) as deep pools fill with gravel and other sediments” (Hedge et.al., 2008). Shallowing and widening of the channel also increase stream temperature extremes, and channel instability increases transport of sediments downstream. Mining-induced bed degradation and other channel changes may not develop for several years until major channel-adjustment flows occur, and adjustments may continue long after extraction has ended.

# 1.3.2 Inconsequential Sand Mining

“Generally, extraction of gravel can only cause problems and affect the ecologic functioning of rivers if particularly undertaken at the wrong time, place, manner and magnitude” (Rinaldi et.al., 2005). According to the Indian Ministry of Environment (2016) all other things being equal, sand mining will have little to no significant impact if the following are observed;

- Extracting gravel from an excavation that does not penetrate the water table and is located away from an active stream channel should cause little or no change to the natural hydrological processes unless the stream captures the pit during periods of flooding.

- In-stream gravel extraction from below the water level generally causes more changes to the natural hydrologic processes than limiting extraction to a reference point above it.

- In-stream extraction of sand and gravel below the deepest part of the channel (the thalweg) generally causes more changes to the natural hydrological processes than limiting extraction to a reference point above the thalweg.

- Excavating sand and gravel from a small straight channel with a narrow floodplain generally will have a greater impact on the natural hydrological processes than excavations on a braided channel with a wide floodplain.

- Extracting sand and gravel from a large river or stream will generally create less impact than extracting the same amount of material from a smaller river or stream.

# 1.4 SAND MINING AND WATERSHED RESPONSES

“The dynamics of probable responses or disturbances in a river system are often complex and unpredictable” (Schumm, 1991; Lord et.al., 2009). According to Schumm (1991), “the concepts of singularity, divergence, and convergence are additional traits of the complexity of fluvial systems important to establishing and understanding cause-effect relationships. Singularity acknowledges that well-established generalizations may not be applicable for site-specific predictions because of unexplained (or unknown) variations at a site” (Germanoski et.al., 2002). “Convergence is the condition where different processes (causes or stressors) can result in the same response (effect). Divergence is the condition where the same stressor (cause) can yield different responses (effects). In this case, two watersheds may respond differently (divergently) to a similar problem” (Gregory & Walling, 1987; Cooke & Doornkamp, 1990).

Fluvial systems can be affected by many single natural and anthropogenic stressors which are either within the watershed or as a result of external factors like climatic change. “Changes in some situations are local in nature and subtle” (Martin and Hess, 1986). “At the most basic level, the altered hydrology category includes any individual stressor that may affect pathway, rates, frequency, or location of water movement in a watershed” (Thorne, 2002). “This change in hydrology may be due to specific stressors such as forestry practices, withdrawal wash-water discharge, storm runoff, and dredging activities from improper sand and gravel operations which can increase the turbidity of streams. Any stressor that may affect the availability and transport of sediment falls in the altered sediment budget category. While basin-scale stressors are limited to the two general categories above, an additional category of stressor is included for channel-scale stressors—altered channel” (Rovira et.al., 2005). “This category includes individual stressors that directly affect the stream channel, such as dredging, trail crossings, channelization, and trampling of banks by grazing animals” (Waters, 1995).

# 1.4.1 The River System Equilibrium

Fluvial systems inherently undergo spatio-temporal changes due to various stressors. The U.S. Environmental Protection Agency (2002) notes that these changes and system responses can be immediate or delayed, direct or indirect, and vary in scale and impact—ranging from beneficial to detrimental. Despite this complexity, recurring patterns of response to disturbances are well-documented (Cottingham et al., 2003). One significant stressor is excessive sand removal, which disrupts the sediment continuity and natural equilibrium of stream channels (Yenn-Teo et al., 2017). Such disturbances can lead to adverse effects like riverbank erosion, riverbed degradation, encroachment on river buffer zones, and a decline in water quality. The severity of these impacts largely depends on the scale of sand extraction relative to the sediment supply and transport rates (Kondolf et al., 2001; Collins & Dunne, 1990; Rovira et al., 2005; Rinaldi et al., 2005).

Koehnken and Rintoul (2018) emphasize that sand mining alters sediment transport and deposits, triggering physical changes in river channels. Rivers naturally maintain a dynamic equilibrium between flow, slope, sediment size, and sediment supply. Altering any of these elements disrupts this balance, leading to erosion or deposition until a new equilibrium is reached (Langbein & Leopold, 1964). This dynamic is evident in seasonal sediment movements—sediment accumulates during low flows and remobilizes in high-flow periods, causing constant morphological adjustments.

Lane (1954) proposed a useful model for understanding river response to changes in sediment and flow dynamics. He suggested that river morphology is governed by a balance between four key factors: sediment load, sediment grain size, water discharge, and river slope. For instance, increasing sediment load or size without corresponding increases in slope or discharge causes deposition, while enhancing flow or slope without adjusting sediment load leads to erosion. This conceptual balance helps illustrate the multifaceted responses of rivers to interventions like sand mining and underscores the importance of maintaining equilibrium to sustain river health.

# 1.4.2 The Reservoirs’ Response

Dams significantly impact sediment dynamics and river morphology by trapping sediment and altering natural flow regimes. Regardless of their intended use, all dams impede sediment transport to varying degrees, disrupting the continuity of river systems and modifying flood peaks and seasonal flow patterns. These changes affect alluvial channels both upstream and downstream. Upstream of dams, bedload and suspended sediments are deposited in reservoirs, reducing their storage capacity and altering flow characteristics in backwater-influenced reaches (Brune, 1953).

Downstream, the released water, devoid of sediment, becomes "hungry water"—a term used to describe sediment-deficient flows with high erosive energy. This water erodes the riverbed and banks, leading to channel incision and coarsening of bed material until a new equilibrium is established (Kondolf, 1997). On average, such channels erode about 20% of their former sediment load annually due to this imbalance (Kondolf and Swanson, 1993). Additionally, reservoirs reduce downstream flood peaks, which may suppress the erosive effects of hungry water, encourage vegetation encroachment, narrow the channel, and allow fine sediment to settle.

These geomorphic changes have critical implications for sand and gravel mining. In sediment-starved reaches below dams, such mines are not replenished by upstream sediment flows but rely solely on inputs from downstream tributaries and localized erosion. Consequently, incision and bank erosion are most pronounced in these areas, heightening environmental risks associated with mining activities.

# 1.4.3 Water Quantity and Quality

Instream sand mining significantly impacts river water quality through increased turbidity, sedimentation, and potential oil leaks from machinery (NMFS, 1998). Dredging and excavation activities disturb sediments, elevating suspended solids that degrade water quality, particularly affecting downstream users relying on the river for domestic use. These suspended solids also raise the cost of water treatment (NMFS, 1998). Changes in flow velocity and stream morphology can alter water temperature and dissolved oxygen levels, further impacting aquatic life and human usage. In Peninsular Malaysia, studies have shown that physiochemical parameters and heavy metal concentrations in ex-mining areas often exceed permissible limits, indicating severe pollution (Ashraf et al., 2010). Additionally, sand mining alters riverbeds, forming deep pits and leading to a lowered groundwater table, which can dry up nearby drinking water wells. Bed degradation reduces streamflow elevation and floodplain water levels, affecting the natural hydrology. Furthermore, mining operations such as bar skimming expand flow width and slow stream velocities, promoting upstream sediment deposition at the mining site. These changes collectively degrade water quality, disrupt ecosystems, and impose economic burdens on water treatment infrastructure.

# 1.4.4 Abiotic Well-being and Biodiversity

Sand mining significantly alters soil properties and ecosystem integrity. It reduces infiltration rates, leading to waterlogging and poor soil aeration (Gurubachan-Singh et al., 2007; Saviour & Stalin, 2012). Ghosh (2002) observed notable differences between unmined and mined soils in Jharkhand: unmined soils were structured and cohesive, while mined soils were loose, less cohesive with disrupted profiles due to layer mixing. Beyond physical degradation, instream mining causes the loss of fertile land, timber, and wildlife habitats, with downstream effects including sedimentation and reduced water quality (Van Dolah et al., 1984). Habitat destruction from vegetation removal and soil profile disruption impacts both terrestrial and aquatic biodiversity, reducing faunal populations. Channel widening and habitat degradation persist until sediment balance is restored (Borges et al., 2002).

# 1.4.5 The Urbanization of Watersheds

Increased sand mining translates to rapid urbanization most likely within the watershed as more buildings and infrastructure come up. This comes with a response from the fluvial system. Urbanization can have a significant effect on channel characteristics within a watershed. The hydrology is vastly altered when vegetation is replaced with impervious surfaces like pavement and rooftops. Hammer’s (1972) study in Philadelphia examined the effect of impervious surfaces on stream channels and concluded that as little as 10% watershed impervious area can degrade channels. Increases in channel cross-sections were greatest in areas draining large impervious coverage such as parking lots and sewered streets where flood frequency increased the most (Krug and Goddard 1986).

# 1.5 ENVIRONMENTAL IMPACT ASSESSMENT FOR SAND MINING

Environmental assessment of sand mining activities is influenced by factors such as the operation’s scale, mining methods, and environmental context. Assessing impacts is often challenging due to the difficulty in quantifying many of the effects, particularly with river sand and stone mining. A key focus is determining whether a river can absorb mining activities while maintaining its geomorphic and ecological integrity. The assessment of environmental impacts from sand mining is guided by several criteria: the magnitude of change, which considers the spatial extent and importance of the impact; the permanence of the impact, indicating whether it is temporary or permanent; the reversibility of the condition, which assesses the potential for restoration or mitigation; and the cumulative nature of the impact, evaluating whether it results in isolated changes or contributes to broader, long-term or synergistic effects. These criteria help determine the overall environmental compatibility of sand mining activities and inform strategies for sustainable river resource management.

# 1.5.1 General EIA Approach to Sand and Gravel Mining

According to the Indian Ministry of Environment (2016), the following considerations summarized in Table 2 should be kept in mind for sand and gravel mining:

Table 2: Guidelines for Sand and Gravel Mining in Riverine Environments

| SN | Key Guideline/Activity | Condition/Specification | Purpose/Justification | Preferred/Restricted Area |
| --- | --- | --- | --- | --- |
| 1 | Identification of aggradation sites | Lease/Environmental Clearance holders may extract from these areas | To manage aggradation problems | Depositional parts of river reach |
| 2 | Distance between mining sites | Based on river's sediment replenishment rate; develop sediment rating curves | To balance sediment extraction with natural replenishment | Depends on site-specific sediment data |
| 3 | Mining season and location preference | Entire active channel during dry season; prefer abandoned channels or inactive floodplains | To reduce ecological impact | Avoid active channels, their deltas and floodplains |
| 4 | Depth and location of extraction | Based on river width and replenishment rate; avoid erosion-prone areas like concave banks | To ensure sustainable mining and reduce erosion | Prefer braided river systems within lateral migration area |
| 5 | Distance from hydraulic structures | 200–500 m; determined by local authorities; cross-section survey of 1 km upstream and downstream required | To protect infrastructure from structural damage | Away from bridges, intakes, pumping stations |
| 6 | Extraction from river bends | Downstream side only; retain upstream one-third of bar and riparian vegetation | To promote channel stability | River bends |
| 7 | Maintaining flood discharge capacity | Mining allowed to maintain flow based on surveyed history; prefer off-channel or floodplain areas | To avoid flood hazard and preserve natural flow | Away from channel banks; mining depth ≤ 3 m; distance from bank ≥ 3 m or 10% of river width |
| 8 | Burrow area placement | Should be river side of embankment; ≥25 m from toe of low embankment (<6 m), ≥50 m for higher embankments | To prevent embankment failure; burrow areas get naturally silted | Properly demarcated and geo-referenced burrow areas |

Source: Modified from Indian Ministry of Environment (2016)

# 1.5.2 Sustainable Sand and Gravel Mining

Sustainable sand mining guidelines aim to responsibly manage riverine resources for both present and future generations. Key principles include identifying suitable zones for mining—such as aggradation areas—while prohibiting extraction near erosion-prone zones and infrastructure. Scientific tools like satellite imagery and replenishment rate assessments help guide responsible extraction. The guidelines emphasize environmental protection, mineral conservation, and prevention of illegal mining to maintain ecological balance.

At the core of the policy is a District Survey Report, treating river stretches within each district as distinct ecological units. This report documents local sand sources, assesses demand and resource availability, and serves as a basis for determining permissible and restricted mining areas. Districts, as administrative units, are best placed to implement and monitor sustainable mining practices.

Periodic surveys by the District Environment Impact Assessment Authority (DEIAA), along with departments like Geology, Forest, and Public Works, support decision-making. These surveys provide data on river behavior, sand availability, and existing mining leases. Scientific studies—geomorphological, geological, and climatic—inform understanding of sediment transport and river dynamics, ensuring that sand extraction does not disrupt natural processes.

Mining zone selection considers river stability, sediment balance, and environmental risks. Reports must include safe mining depths and distances from riverbanks. A Sub-Divisional Committee comprising relevant stakeholders is tasked with inspecting proposed sites and making informed recommendations. Overall, these guidelines promote a systematic, science-based, and environmentally sustainable approach to sand mining.

# 1.5.3 Key Sand Mining Mitigation Measures

Key mitigation measures for sand mining activities focus on minimizing environmental impacts through improved operational practices, runoff control and additional safeguards (Table 3).

Table 3: Key Sand Mining Mitigation Measures

| Mitigation Category | Key Measures | Environmental Benefits |
| --- | --- | --- |
| Operational Practices | - Conduct mining during low flow and outside breeding seasons - Avoid operations during/after heavy rains and at night - Restrict mining to one riverbank with narrow access - Keep machinery clean before exit | - Reduces erosion - Protects aquatic life - Minimizes habitat disruption and noise pollution |
| Water Runoff & Discharge Control | - Build sedimentation ponds using earth/rock/concrete - Develop proper drainage networks - Implement slope protection and turfing | - Improves water quality - Prevents sediment pollution - Reduces downstream siltation risks |
| Other Environmental Measures | - Use a single, low-impact access route - Restore and revegetate site after use - Regulate site traffic with on-site parking - Spray water regularly for dust control - - Restrict work to non-sensitive hours | - Preserves vegetation - Minimizes air and noise pollution - Reduces public disturbance |

# 1.6 CONCLUSION

Comprehensive management of gravel and sand in river systems should be based on a recognition of the natural flow of sediment through the drainage network and the nature of impacts (to ecological resources and to infrastructure) likely to occur when the continuity of sediment is disrupted. A sediment budget should be developed for present and historical conditions as a fundamental basis for evaluation of these impacts, many of which are cumulative in nature. The cost of sediment-related impacts of existing and proposed water development projects and aggregate mines must be realistically assessed and included in economic evaluations of these projects.

The finite nature of high-quality alluvial gravel resources must be recognized, and high-quality PCC-grade aggregates should be reserved only for the uses demanding this quality material (such as concrete). Alternative sources should be used in less demanding applications (such as road subbase). The environmental costs of instream mining should be incorporated into the price of the product so that alternative sources that require more processing but have less environmental impact become more attractive. Instream mining should not be permitted in rivers downstream of dams by virtue of the lack of supply from upstream or in rivers with important salmon spawning (unless it can be shown that the extraction will not degrade habitat).

Disclaimer (Artificial intelligence)

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

- The main original content of the paper was wholly drafted without the single use of generative AI Technology.

- Realizing, it was too bulky (22 Pages), controlled use of ChatGPT was used to integrated few sections of the paper to decongest it (18 pages).

- Thereafter, the whole content was revised, revamped, edited and fine-tuned by the author, both pre and post review process.

# REFERENCES

Abdulazeez, A. (2023). Dynamics and Impacts of Sand Mining in the Watari River Basin in Kano State, Nigeria. A 	Ph. D Geography Thesis. Department of Geography, Faculty of Earth and Environmental Sciences, Federal 	University Dutsin-Ma (FUDMA), Katsina State.

Ab.Ghani, A., Zakaria, N.A., Abdullah, R., Mohd Sidek, L., Sinnakaudan, S.K. and Chang, C.K. (2003). 	Guidelines for Field Data Collection and Analysis of River Sediment, Department of Irrigation and 	Drainage Malaysia, Kuala Lumpur, 35.

Abdulazeez, A. (2018). A Spatial Assessment and Mapping of Groundwater Potentials in Parts of North-Western 	Kano State, Nigeria. Techno Science Africana Journal. 14(1), 103-114. ISSN: 2006-2273. Published by 	Department of Geography, Kano University of Science and Technology, Wudil. Available online at 	www.technoscienceafricana.com.

Abdulazeez, A. and Adamu A.U (2019): Spatial and Source Disparities of Groundwater Quality in Dawakin Tofa 	Local Government Area of Kano State, Nigeria, Proceedings of the 5th International Conference on 	Water Resource and Environment (WRE 2019) held from 16th to 19th July, 2019 at Macao Science 	Centre, Macao 	China. www.wreconf.org.

Ashraf, M.A., Maah M.J and Yusoff, I.B. (2010). Study of Water Quality and Heavy Metals in Soil and Water 	of 	Ex-Mining Area Bestari Jaya, Peninsular Malaysia.

Atejioye, A.A. and Odeyemi, C.A. (2018). Analysing Impact of Sand Mining in Ekiti State, Nigeria Using GIS 	for Sustainable Development. World Journal of Research and Review (WJRR) Volume-6, Issue-2, Pages 	26-31. ISSN:2455-3956.

Blachowski, J. (2015): “GIS-Based Spatial Assessment of Rock Minerals Mining - A Case Study of the 	Lower Silesia Region (SW Poland)”. Mining Science Journal, Volume 22, 2015, 07−22. 	www.miningscience.pwr.edu.pl ISSN 230470-9586 (print), ISSN 2084-35 (online).

Bolen, W.P. (1996). Sand and gravel, industrial. In 1995 Minerals yearbook (pp. 715-730). Reston, VA: U.S. 	Geological Survey.

Borges, P., Andrade, C., Freitas, M.C. (2002). Dune, Bluff and Beach erosion due to exhaustive sand mining – 	the 	case of Santa Barbara Beach, São Miguel (Azores, Portugal). J. Coastal Res., (ICS 2002 Proceedings) 	Special Issue., 36(1): 89-95.

Brierley, G.J. and Fryirs, K. (2014a). Developments in Earth Surface Processes, 18. 	http://dx.doi.org/10.1016/B978-	0-444-63402-3.00013-3. Copyright © 2014 Elsevier B.V.

Brown, A.G. (1996) Floodplain palaeoenvironments. In Floodplain Processes, edited by Anderson M G, Walling 	D E and Bates P D, 95-138. Chichester: Wiley.

Brune, G.M. (1953) Trap Efficiency of Reservoirs. Transactions, American Geophysical Union, 34, 407-418.
	http://dx.doi.org/10.1029/TR034i003p00407

Byrnes, M.R and Hiland, M.W. (1995). Large-scale sediment transport patterns on the continental shelf and 	influence on shoreline response: St. Andrew Sound, Georgia to Nassau Sound, Florida U.S.A. In: LIST, 	J.H. and TERWINDT, J.H.J. (eds.), Large-Scale Coastal Behaviour. Marine Geol., 126: 19-43.

Carillo, F.V., Davis, J.F., and Burnett J.L. (1990). California: Annual Report. US Department of Interior, Bureau 	of Mines. Washington, DC.

Chauhan, S.S. (2010). Mining, Development and Environment: A Case Study of Bijolia Mining Area in 	Rajasthan, India. Journal of Human Ecology, 31(1): 65-72.

Churchill, N. (2016). Strange but True: The UAE Imports Its Sand. Edgardaily.com. May 2019. Available online: 	https://edgardaily.com/en/life/2016/strange-but-true-the-uae-imports-its-sand-31668 (accessed May 	2019).

Coastal Engineering Research Centre (CERC) (1984). Shore Protection Manual. Volume I, CERC Dept. of the 	Army, U.S. Army Corps of Engineers, Washington.

Collins, B. and Dunne, T. (1990). Fluvial Geomorphology and River-Gravel Mining: A Guide for Planners, Case 	Studies Included. Special Publication 98, California Department of Conservation, Division of Mines and 	Geology.

Cooke, R.U. and Doornkamp, J.C. (1990) Geomorphology in Environmental Management. 2nd Edition, Oxford 	University Press, Oxford.

Dhakwa, C.A., Biney, C.A and K.A.A. De Graft Johnson. (2005). Impact of mining operations on the ecology 	of 	river Offin, Ghana.West African Journal of Applied Ecology, Volume 7.

Environmental Conservation Department (ECD) (2000), Environmental Impact Assessment (EIA) Guidelines 	for 	River Sand and Stone Mining. Sabah, Malaysia October 2000.

EPA (U.S. Environmental Protection Agency). (1976). Erosion and sediment control surface mining in the 	eastern 	United States. U.S. Environmental Protection Agency Technical Series Report EPA-625/3-76-	006, Washington, DC.

Fryirs, K. and Brierley, G.J. (2013). Geomorphic Analysis of River Systems. An Approach to Reading the 	Landscape. Blackwell, Oxford, UK.

Gavriletea, M.D. (2017). Environmental Impacts of Sand Exploitation. Analysis of Sand Market. Sustainability. 	9, 1118; doi:10.3390/su9071118 www.mdpi.com/journal/sustainability

Geological Survey of India (Undated) A Model Document on Impacts and Methodology of Systematic and 	Scientific Mining of The River Bed Material. www.gsi.gov.in

Germanoski, D., Miller, J.R. and Latham, D.D. (2002). The importance of event sequencing on the geomorphic 	impact of wildfire in the central Great Basin: Geological Society of America Abstracts with Programs, 	34(6), p. 319.

Goddard, J. (2007). Land Degradation and Rehabilitation. Sydney: University of South Wales Press.

Gregory, K.J. and Walling, D.E. editors, 1987: Human activity and environmental processes. Chichester: 	Wiley.

Gurubachan Singh, K.K. Mehta., R.C. Sharma. K.L Chawla., P.K. Joshi., N.P. Yaduvanchi (2005). Technical 	bulletin on Sand mining/no mining in Agricultural fields in Haryana.

Hammer, T. R. 1972. Stream channel enlargement due to urbanization. Water Resources Research. 8:6:1530-	1540.

Harris, N. (2003). Introduction. In Atlas of the World’s Deserts; Fitzroy Dearborn: New York, USA; pp.1–7.

Hess, D. (2012): Physical Geography: A Landscape Appreciation. 10th Edition. Published by PHI Learning Private 	Limited, New Delhi, India.

Hill, L. and Kleynrans, C.J. (1999). Authorisation and Licensing of Sand Mining/Gravel Extraction, in terms of 	Impacts on In-stream and Riparian Habitats. Journal of Mining Science. 15, 17-19.

Koehnken, L., and Rintoul, M. (2018). Impacts of Sand Mining on Ecosystem Structure, Process and Biodiversity 	in Rivers. WWF. ISBN: 978-2-940529-88-9

Kondolf, G.M. (1995). Managing bedload sediments in regulated rivers: examples from California, USA. 	Geophysical Monograph 89:165-176.

Kondolf, G.M. (1997). Hungry Water: Effects of Dams and Gravel Mining on River Channels, Profile, 	Environmental Management, 21(4): 533-551.

Kondolf, G.M. (1998). Environmental effects of aggregate extraction from river channels and floodplains, 	Aggregate resources – A global perspective: A.A, Balkema, Rotterdam, Netherlands, pp. 113-129.

Kondolf, G.M., and M.L. Swanson (1993). Channel adjustments to reservoir construction and instream gravel 	mining, 	Stony Creek, California. Environmental Geology and Water Science 21:256-269.

Kondolf, G.M., Smeltzer, M. and Kimball, L. (2001). Freshwater Gravel Mining and Dredging Issues. 	Washington Departments of Fish and Wildlife, Ecology, and Transportation, Olympia.

Krug, W.R. (1996), Simulation of Temporal Changes in Rainfall-Runoff Characteristics, Coon Creek Basin, 	Wisconsin. JAWRA Journal of the American Water Resources Association, 32: 745-	752. https://doi.org/10.1111/j.1752-1688.1996.tb03471.x

Langbein W.B and Leopold L.B (1964) Quasi-equilibrium states in channel morphology. American Journal of 	Science June 1964, 262 (6) 782-794; DOI: https://doi.org/10.2475/ajs.262.6.782

Langer W.H. (1988). Natural aggregates of the conterminous United States. U.S. Geol. Surv. Bull. 1594. U.S. 	Geological Survey, Denver, CO.

Langer, W. H., and Glanzman, V.M. (1993). Natural aggregate: building America's future. U.S. Geol. Surv. Cir. 	1110. U.S. Geological Survey, Denver, CO.

Leopold, L.B., Wolman, M.G. and Miller, J.P. (1964) Fluvial Processes in Geomorphology. Freeman, San 	Francisco, 522 p.

Lord, M.L., Germanoski, D. and Allmendinger, N.E. (2009). Fluvial geomorphology: Monitoring stream systems 	in response to a changing environment, in Young, R., and Norby, L., Geological Monitoring: Boulder, 	Colorado, Geological Society of America, p. 69–103, doi: 10.1130/2009. Monitoring (04).

Martin, C.R., and Hess, T.B. (1986). The impacts of sand and gravel dredging on trout and trout habitat in the 	Chattahoochee River, Georgia. Georgia Department of Natural Resources, Atlanta.

Meador, M.R. and Layher, A.O. (1997). Instream Sand and Gravel Mining: Environmental Issues and Regulatory 	Process in the United States. Fisheries Habitat 23(11).

Milliman, J.D. and Syvitski, J.P.M. (1992) Geomorphic/Tectonic Control of Sediment Discharge to the Ocean: 	The Importance of Small Mountainous Rivers. The Journal of Geology, 100, 525-544.
	http://dx.doi.org/10.1086/629606

Ministry of Environment, Forest and Climate Change (2016). Sustainable Sand Mining Management Guidelines.  	Government of India – 2016.

National Marine Fisheries Service (NMFS) (1998). National Gravel Extraction Policy. Online document: 	http://swr.ucsd.edu/hcd/gravelsw.htm.

National Research Council (1983). Risk Assessment in the Federal Government: Managing the Process. 	Washington, DC: The National Academy Press.

Newson, M.D. (1992). “Geomorphic Thresholds in Gravel-Bed Rivers: Refinement for an Era of 	Environmental Change.” In P. Billi, R. D. Hey, C. R. Thorne, and P. Tacconi, eds., Dynamics of 	Gravel-Bed Rivers. J. Wiley and Sons, U.K.

Nurhasan, U.  and Saputra, P.Y. (2018) Analysis of Sand Mining Areas in Lumajang Using WEBGIS. Advances 	in Economics, Business and Management Research, volume 59. International Conference on Energy 	and 	Mining Law (ICEML 2018).

Padmalal, D. and Maya, K. (2014). Sand mining: Environmental impacts and selected case studies. Springer. (E)

Pahuja, S. and Goswami, D. (2006): A Fluvial Geomorphology Perspective on the Knowledge Base of the 	Brahmaputra. Background Paper No. 3 commissioned as an input to the study “Development and 	Growth in Northeast India: The Natural Resources, Water, and Environment Nexus” August 2006.

Pettijohn, F. J. (1975). Sedimentary Rocks. New York: Harper and Row.

Pitchaiah, P.S (2017). Impacts of Sand Mining on Environment – A Review. SSRG International Journal of Geo 	informatics and Geological Science (SSRG-IJGGS) – Volume 4 Issue 1 Jan to April 2017 ISSN: 2393 - 	9206 www.internationaljournalssrg.org Page 1

Radford, J.Q., Bennett,A.F and Cheers, G.J (2005) Landscape-level thresholds of habitat cover for woodland-	dependent birds. Biological Conservation, Volume 124, Issue 3, Pages 317-337ISSN 0006-3207, 	https://doi.org/10.1016/j.biocon.2005.01.039.

Rinaldi, M., Wyzga, B. and Surian, N. (2005). Sediment Mining in Alluvial Channels: Physical Effects and 	Management Perspectives. River Research and Applications, 21, 805–828.

Rovira, A., Batalla, R.J. and Sala, M. (2005). Response of a River Sediment Budget after Historical Gravel 	Mining 	(The Lower Tordera, Ne Spain). River Research and Applications, 21, 829–847.

Saviour, M. N. and Stalin, P. (2012): Soil and Sand Mining: Causes, Consequences and Management. IOSR 	Journal 	of Pharmacy (IOSRPHR) ISSN: 2250-3013, Volume 2, Issue 4, PP 01-06. www.iosrphr.org

Schoof, J. (2014). Built on Sand: Is the Construction Boom Depleting Supplies?. Available online: 	http://www.detail-online.com/article/built-on-sand-is-the-construction-boom-depleting-supplies- 	16825/ 	(accessed on November 2019).

Schumm, S.A. (1991). To Interpret the Earth: Ten Ways to Be Wrong: Cambridge, Cambridge University Press, 	180 p.

Shaffer, N. R. (2006). The Time of Sands: Quartz-rich Sand Deposits as a Renewable Resource. Electronic Green 	Journal, 1(24). ISSN 1076-7975. eScholarship.org. Powered by the California Digital Library. 	University of California

Sreebha, S. (2008). Environmental impact of sand mining: A case study in the river catchments of Vembanad 	lake, Southwest India. Ph.D. Thesis of Cochin University of Science and Technology, Kochi.

Sreebha, S. and Padmalal, D. (2011). Environmental impact assessment of sand mining from the small catchment 	rivers in the Southwestern coast of India: A case study. Environmental Management, 47, 130–140. (E)

Starnes, L. B. (1983). Effects of surface mining on aquatic resources in North America. Fisheries 8(6):2-4.

The Mineral Gallery. (2004). The Tectosilicate sub-class. Retrieved November 5, 2019 from 	mineral.galleries.com.

Thorne, C. R. (2002). “Geomorphic Analysis of Large Alluvial Rivers.” Geomorphology 44:203–219.

United Nations Environment Programme-UNEP (2014). Sand, Rarer Than One Thinks. Available online: 	http://www.unep.org/pdf/UNEP_GEAS_March_2014.pdf (accessed on 24 May 2016).

USGS (U.S. Geological Survey). (1997). The Front Range Infrastructure Resources Project. U.S. Geological 	Survey, Denver, CO; http://webserver .cr.usgs.gov/frirp/frirp.html.

Van Dolah, R.F., Calder, D.R. and Knott, D.M. (1984). Effects of dredging and open-water disposal on benthic 	macroinvertebrates in a South Carolina estuary. Estuaries, 7(2): 28-37.

Waters, T. F. (1995). Sediment in streams: sources, biological effects, and control. Am. Fish. Soc. Monogr. 7, 	Bethesda, MD.

Yenn Teo, F., Noh, M.D., Ghani, M.N. and Zakaria, N.A. (2017):  River Sand Mining Capacity in Malaysia. 	Proceedings of the 37th IAHR World Congress August 13 – 18, 2017, Kuala Lumpur, Malaysia, P538-	546. ISSN 1562-6865 (Online) - ISSN 1063-7710 (Print).

Zdunczyk, M.J and Linkous, M.A (1994). Industrial Sand and Sandstone. In Industrial Minerals and 	Racks, 	6th 	Edition. Pp879-91

Zhang, G., Song, J., Yang, J., and Liu, X. (2006). Performance of mortar and concrete made with a fine 	aggregate of Desert sand. Building and Environment, 41(11), 1478–1481. doi: 	10.1016/j.buildenv.2005.05.033

Abdulazeez, A. (2016). A review on the impact of river and inland sand mining on Nigerian River basins. A phd thesis submitted at federal university Dutsinma, Katsina State Nigeria.