Production of Glucoamylase from Fungi using Agro Residues as a Substrate

ABSTRACT

Enzymes have been used in the production of consumables such as cheese, wine, beer, and vinegar, as well as in the development of products such as leather and linen. The discovery of fermentation processes over the latter part of the previous century, explicitly aimed at the production of enzymes utilising selective growth strains. Multiple microbial enzymes are well-identified catalysts for the synthesis of several products from a diverse range of substrates under regulated conditions. The need for large-scale production of enzymes from agro-residues for industrial application in Nigeria has become a source of worry to the enzyme-based industry. The aim of the study was to produce glucoamylase from fungi using agro-residue substrate. Fungi were isolated from soil and screened for glucoamylase production. Yam and potato peels were obtained and analysed for their composition. The peels were fermented using fungi positive for glucoamylase production, and process parameters were monitored. The data obtained from the compositional analysis of the agro residues were analysed using the One-way ANOVA to ascertain if any significant difference existed in the composition. The data obtained from the time-course study of glucoamylase activity at p-value <0.05 was ascertained. The result showed that the fungal population of the soil ranged from 1.0 x104CFU/g to 2.4 x104CFU/g. The isolated fungi were identified as Fusarium sp, Aspergillus niger, Rhizopus sp, Aspergillus sp and Penicillium sp. Had the highest potential for glucoamylase production on starch agar. The carbohydrate content of the yam and potato peels was 73.16 and 76.51%, respectively; protein content was 0.25 and 0.23%, respectively; and fibre content was 1.42 and 1.74%, respectively. The maximum yield of glucoamylase was recorded in Apergillus niger 37.13IU/ml while Aspergillus sp had glucoamylase yield of 26.12 IU/ml. At pH 6.0, a glucoamylase concentration of 24.18 IU/ml was obtained, and using a fungal inoculum size of 3 ml resulted in a glucoamylase concentration of 45.08 IU/ml. The carbon source (combination of yam and potato peel, 5g each) and nitrogen source, urea, had glucoamylase production of 32.17IU/ml and 58.36IU/ml, respectively. It is concluded that high yield of glucoamylase can be produced from yam and potato peels using A. niger.

Keywords: Agro-residues, Fungi, Glucoamylase, Production, Substrate

- INTRODUCTION

“Enzymes are the desirable metabolic catalyst that gives different endogenous biochemical reactions through a well-defined pathway. Enzymes are essential as they help catalyse biological processes without being consumed in the process. Their specificity to temperature, pressure and operational conditions has limited their production and usage. They have gained interest all over the world due to their vast application in biomedical, agronomics and industrial sectors. The use of enzymes in production processes limits the development of unwanted byproducts that are frequently seen in catalyst-assisted reactions. Enzymes have been used in the production of consumables such as cheese, wine, beer, and vinegar as well as in the development of products such as leather and linen” (Kalia et al., 2021; Sabino et al., 2021). “Additionally, enzymes have long been used to ease the intricate production steps/procedures of several industrial sectors. Nowadays, the demand for sustainable processes has extensively augmented the application of enzymes as catalysts in several industrial sectors” (Maghraby et al., 2023). “Production of leather or linen was dependent on either enzymes synthesised by randomly grown microbes or enzymes existing in additional preparation, such as the rumen of calves or papaya fruit” (Zambare, 2010). “Consequently, enzymes were not utilised in pure form. The discovery of fermentation processes over the latter part of the previous century, explicitly aimed at the production of enzymes utilising selective growth strains” (Muthusamy et al., 2022). “Enzymatic catalysis relies on the concerted action of amino acid residues, substrate, and sometimes water molecules arranged in or close to the active site. They operate cooperatively, promoting the binding of substrates and their chemical transformation, ultimately resulting in the release of the products” (Das et al., 2023; Hasan et al., 2024). “Multiple microbial enzymes are well-identified catalysts for the synthesis of several products from a diverse range of substrates under regulated conditions” (Singh et al., 2019). “Nowadays, several enzymes are well-defined, crystalline substances with extremely reproducible catalytic activities and physical properties. Among the enzyme-producing organisms, fungi-derived enzymes attained significant industrial applications due to high production yields at large-scale, ease of cultivation, and simultaneous biomass production, thereby half of the commercially utilised enzymes are from fungal origin” (Dhevagi et al., 2021). “Out of all the enzymes, proteases and amylases are of high industrial significance. Amylase accounts for 30% of the production of industrial enzymes worldwide” (Bamigboye et al., 2022). “Amylases are used to hydrolyse polysaccharides like starch into simple sugar constituents in the starch processing industry. As new opportunities in biotechnology have emerged, the range of amylase applications has expanded to include analytical chemistry, medical and pharmaceutical applications”. (Almulaiky, et al., 2021). “Amylases are one of the most essential enzymes and are very important in biotechnology; they are a class of industrial enzymes that account for around one quarter of the global enzyme market” (Kalia et al., 2021). “The key benefit of using microorganisms to produce amylases is the least expensive method of mass production, as well as the ease with which microbes can be influenced to produce desired enzymes. Other enzymes, such as carbohydrases, also termed glycosidases (glycoside hydrolases), are a group of enzymes that represent different enzymes involved in the hydrolysis and synthesis of carbohydrates” (Contesini et al., 2013). “Amylases are comprised of this group along with other enzymes like xylanases, cellulases, etc. It is one of the main industrially important groups of enzymes” (de Castro et al., 2010). There are several different sources of amylases, including plants, mammals, various fungi, bacteria, and actinomycetes (Kalia et al., 2021). “The greater yields produced in the shortest fermentation period, inexpensive bulk production capacity, and ease of manipulation of microbes to produce enzymes with desired properties are a few of the primary benefits of using microorganisms for amylase production. The optimal possible product concentration to develop throughout the fermentation processes is crucial to optimise the fermentation settings, especially with regard to physical and chemical characteristics. The cost of enzyme production in submerged fermentation is high, which necessitates the use of an alternative method. The potential for using agricultural wastes as renewable energy and carbon sources is immense. Agriculture waste can be used to produce feed for animals, biofuel, and in the medicinal industries. The wastes are mostly generated from preliminary operations such as peelings and cuttings. An enormous amount of this waste is disposed of into the environment, resulting in pollution hazards which are detrimental to biological and physical components of the environment” (Olakusehin and Oyedeji, 2021). “The initial step in the bioconversion of organic material into reducing sugars, such as glucose and xylose, is the saccharification of agricultural leftovers by microbial hydrolytic enzymes. Agricultural byproducts, which are widely regarded as the best substrate, are among the most promising residues for solid-state fermentation (SSF), making it an appealing alternative process. Solid state fermentation (SSF) method shows high volumetric productivity with lower capital cost and energy requirements, followed by increased concentration of product, less effluent and water output and very handful fermentation equipment with easier product recovery. SSF reduces the cost of recovery of enzymes and provides direct utilisation of substrates and biomass by the feedstock. Paddy husk, wheat bran, cassava peels, potato peels, rice processing waste and other starch containing wastes have gained importance as they support microbial growth during enzyme production” (Pasin et al., 2020) “Lignocellulosic agricultural biomass accounts for more than 60% of the total biomass produced in the form of wheat straw, rice straw, corn cob, corn fibers, para grass, kallar grass and maize stover” (Zambare, 2010). “Tons of agro-industrial residues are generated annually in agricultural countries like Pakistan” (Zambare, 2010). This residue contains significant quantities of biochemical fractions like cellulose, hemicellulose, and lignin to be converted into many valuable products for food and pharmaceuticals.

2.0 MATERIALS AND METHODS

## 2.1   Sample collection

The three (3) soil samples were collected from dumpsites within the Rivers State University. Soil samples were collected using a simple hand-held soil auger obtained from the Spring Research and Analytical Laboratory, Choba, Port Harcourt. Sterile plastic bags were purchased from the Everyday Supermarket, Choba, Rivers State, for the purpose of sample collection. The sample bags were labelled according to the location of the sample collection. The samples were transported in an ice chest to the Microbiology Laboratory, Rivers State University.

2.2 Isolation of Fungi

One gram of the soil sample was placed in 10 ml of sterile normal saline to obtain a soil suspension. The suspension was then subjected to a 10-fold serial dilution to a dilution of 10-4. Aliquot of 0.1 ml of the different dilutions was inoculated on a Potato Dextrose Agar (PDA) plate using the spread plate technique. Inoculated plates were incubated at ambient temperature (26-320C) for 3-5 days. After incubation, fungal colonies were characterised and isolated.

2.3 Macroscopic and Microscopic Examination of Isolated Fungi

The Colonial features of the isolated fungi, including top and reverse colour, form and presence/ absence of periphery, were noted and recorded. Also, slides of the fungi were prepared for microscopy using lactophenol cotton blue. The slides were viewed under the microscope using a 40x objective lens.

2.4 Collection of agro-residues as biomass

Yam peels and potato peels were obtained from the Mile 4 market in Obio-Akpor, Local Government Area of Rivers state. These agro residues were taken to the Microbiology Laboratory, Rivers State University, Nkpolu-Oroworokwu, Port Harcourt, Rivers State. They were dried and pulverised into granules using a mechanical grinder. The processed biomass was packaged, sealed and labelled accordingly.

2.5 Compositional Analysis of the Agro-Residues

2.5.1 Determination of crude protein

The crude protein content was determined using the Kjeldahl method. About 0.5g of agro residues was placed into a clean 250 ml conical flask, 15g of digestion catalyst was added, and 20 ml concentrated sulphuric acid was added. The mixture was heated to digest the content. The clear solution was then allowed to stand for 30 minutes and to cool. After that cooling was made up to 100 ml with distilled water. About 20 ml of the diluted digest was put in a heated distillation flask attached to a Liebig condenser connected to a receiver containing 10 ml of 2% boric acid indicator. About 40 ml of 40% Sodium hydroxide was injected into the digest via a syringe until the digest became strongly alkaline. The mixture was heated to a boil, and the distilled ammonia gas was condensed into the beaker containing the boric acid. The distillate was titrated with standard 0.1N Hydrochloric acid solution. The volume of hydrochloric acid added to effect this change was recorded as the titration value.

Calculation

%Nitrogen = Titre value X 0.1 X14X4

%Protein = %Nitrogen x 6.25

2.5.2 Determination of Carbohydrate

The carbohydrate content was determined using the Cleg Anthrone method. About 0.1g of the agro residues was placed into a 25 ml volumetric flask, 1 ml of distilled water and 1.3 ml of 62% perchloric acid were added and agitated for about 20 mins. Then the flask was made up to 25ml mark with distilled water. The solution formed was filtered through a filter paper. Then, 1 ml of the filtrate was transferred into a 10 ml test tube and diluted to volume with distilled water. 1 ml of the working solution was pipette into a clean test tube, and 5 ml of Anthrone reagent was added. Similarly, 1ml distilled water and 5ml Anthrone were mixed in a separate test tube to be used as a blank. A standard glucose solution of 0.1 ml was also treated with Anthrone reagent. The absorbance of all 3 solutions was read at 630nm. The carbohydrate content, as glucose, was then calculated using the formula below.

% CHO as glucose = 25 x absorbance of sample              x 100

Absorbance of standard glucose          1

2.5.3 Determination of Moisture Content

The moisture content was determined using the oven drying method. About one gram (1g) of the agro residues was placed into a clean, dried porcelain evaporating dish. The weight of the porcelain evaporating dish and sample was noted before drying. The dish was then placed in an oven set at a temperature of 1050C and allowed for six hours. Afterwards, the dish was brought out and placed in a desiccator so as to cool down to room temperature. The cooled dish was then weighed, and the moisture content was calculated as below

% Moisture = Weight of fresh sample – weight of dried sample   x   100

Weight of fresh sample				      1

2.5.4 Determination of Ash

About one gram (1g) of the dried agro residues was placed into a porcelain crucible, which was previously preheated and weighed. The crucible was placed in a muffled furnace set at a temperature of 630oC for three hours. Afterwards, the crucible was brought out and allowed to cool to room temperature, and re-weighed. The ash content was then calculated as below

% Ash = Weight of crucible + Ash sample – Weight of crucible     x 100

Weight of sample		                                 1

2.5.5 Determination of Crude Fibre

The total fibre content was determined by the Association of Official Analytical Chemists (1990) method. Two grams (2.0g) of agro residue was placed in a 500 mL beaker and was hydrolysed by adding 10 mL of 25% sulphuric acid and boiled for about 30 min on a hot plate. The mixture was filtered through a piece of clean white cloth and rinsed with hot distilled water. The residue was boiled again with 20 mL of 2.5% sodium hydroxide (NaOH) for 30 min, and was then filtered and rinsed with distilled water. Finally, the residue was collected and transferred into a crucible and dried in the oven to a constant weight. The weight of the fibre was then calculated using the formula below

Crude fiber (%) = 	Weight of fiber ×   100

Weight of sample

## 2.5.6 Determination of Crude Lipids

Approximately 4- 5g of sample was taken into the thimble, and a note of the sample weight was taken. A cotton was placed into the thimble in a way that covered the sample, and the thimble was folded to enclose the sample. A cellulose thimble (sample holder) was taken, and the sample was put inside the cellulose thimble. Thereafter, the weight of a cleaned and dried flat-bottom flask was taken. A Soxhlet extraction unit was taken, and the sample was placed in it. A sufficient amount of n-hexane was added. After the extraction of fat, the sample was taken out from the thimble. The flask was placed inside the oven to remove moisture and n-hexane. The temperature was set at 110 degrees Celsius and dried for 30 minutes. After 30 minutes, the dried flask was taken out and cooled in a desiccator. After cooling, the weight of the flask with the fat was taken.

Calculations.

Crude lipids content (%) = Weight of flask with fat -Weight clean dry flask ×   100

Weight of sample

2.6 Screening isolated fungi for glucoamylase activity

Spore inocula were prepared by adding 10-15 ml of sterile Tween 80 (0.8%) to slants of the fungal cultures and shaking vigorously. About 0.1 ml of the resulting suspension was introduced into wells 6mm within starch agar plates having 1% starch. The plates were incubated at ambient temperatures (26-320C) for 3 days. After incubation, the zone of clearance around the wells was measured with the aid of a millimetre scale.

2.7 Fermentation of the agro-residues

Five grams (5g) each of agro residues and 10 ml of mineral medium were placed in 250 ml cotton-plugged Erlenmeyer flasks, mixed homogenously and sterilised at 121oC for 15 min in an Autoclave. Thereafter, the mixtures were allowed to cool to room temperature and inoculated separately with 1ml spore suspension of Aspergillus niger, Aspergillus sp, and Penicillium sp. The inoculated mixtures were then incubated at ambient temperature (26-320C) for 5 days.

2.8 Enzyme extraction

After the incubation period, 0.1M citrate buffer was added at a ratio of 1:10, and agitated using a shaker set at 250 rpm for 30 minutes. The mixtures were then filtered through filter paper. The filtrates were then centrifuged at 5000 rpm for 20 minutes. The supernatant obtained was collected as the crude enzyme extract.

2.9 Partial Purification of Glucoamylase

2.9.1 Fractionation with Ammonium Sulphate

In a variety of saturated degrees of ammonium sulphate, a standard procedure was used to get the partially purified enzyme (Yandri et al., 2007). The raw extract was brought to 50% saturation of ammonium sulphate, and the precipitates were removed by centrifuging at 20,000g for 15 minutes. The resulting supernatant was raised to 70% ammonium sulphate saturation, and the precipitate was collected by centrifuging at 20,000g for 15 minutes. The resulting supernatant was raised to 90% ammonium sulphate saturation, and precipitate was also collected by centrifuging at 20,000g for 15 mins.  The resulting supernatants were subjected to enzyme assay.

2.10 Determination of Glucoamylase Activity

Spectrophotometric enzyme assay was used to determine the glucoamylase activity. The activities of the glucoamylase enzyme were expressed in International Units (IU). One IU is defined as one µmol of glucose (for α-amylase and glucoamylase activity) equivalents released per minute per ml under the assay conditions by using a glucose standard curve. Appropriate dilution factors were used during the estimation of enzyme activity. About 1ml of diluted enzyme extracts was added to test tubes containing 1% maltose in 0.1 M citrate buffer of pH 5.0 at 35°C and allowed for 30 minutes in an incubator. The enzyme reaction was halted by the 3 ml of 3, 5-dinitrosalicylic acid (DNS) reagent. The tubes were then heated for 15 minutes in a boiling water bath. One ml of a 40% solution of Rochelle salt was added to each tube prior to cooling to ambient temperature. The tube contents were made to 7 ml by adding distilled water. The Absorbance of the resulting solutions were then measured at 575 nm using a UV-Visible spectrophotometer. The buffer (0.1M citrate buffer) was used as a reference blank. The residual glucose concentrations were then determined through extrapolation using the absorbance measurements and a standard graph of glucose concentration (mg/ml) versus absorbance.

2.11 Effect of additional carbon sources on enzyme production

The effect of 1%w/v glucose, fructose, lactose, sucrose and maltose on production of glucoamylase using Aspergillus niger, Aspergillus sp and Penicillium sp and the agro-residues were determined following the procedures in section 2.7-2.10.

2.12 Effect of additional nitrogen sources on enzyme production

The effect of 1%w/v Ammonium nitrate, sodium nitrate, urea, ammonium sulphate and ammonium phosphate on production of glucoamylase using Aspergillus niger, Aspergillus sp and Penicillium sp and the agro-residues were determined following the procedures in section 2.7-2.10.

2.13 Effect of inoculum size on enzyme production

The effect of 1%, 2%, 3% and 4% of fungal spore suspension on production of glucoamylase using Aspergillus niger, Aspergillus sp and Penicillium sp and the agro-residues were determined following the procedures in section 2.7-2.10.

2.14 Effect of pH on enzyme activity

The substrate (starch) was prepared at different pH values ranging from 3.0 to 9.0 (3.0, 5.0, 6.0, 7.0, 8.0 and 9.0). Then it was assayed by the DNS method. The optimum pH for enzyme activity was determined using 0.02 M sodium acetate buffer pH 5.0 to 6.0, phosphate buffer pH 6.0 to 7.5 and Tris-HCl buffer pH 8.0 to 10.0 at intervals of 0.5. 0.5 ml of gelatinised starch solution (1%) was equilibrated with 1 ml of the buffers (20 mM) of respective pHs for 5 min at 27°C. 0.5 ml of the enzyme was added, and the resulting suspension was mixed properly and allowed to stand for 20 min at 35°C. The glucoamylase activity was assayed as described in section 2.9.

2.15 Statistical analysis

The data obtained from the compositional analysis of the agro residues were analysed using the One-way ANOVA to ascertain if any significant difference existed in the composition. The data obtained from the time-course study of glucoamylase activity at p-value <0.05 was ascertained.

## 3.0 RESULTS AND DISCUSSION

## 3.1 Fungal population in soil from the dumpsites

The fungal population in soil from the dumpsites is presented in Figure 1. In the figure, it can be seen that dumpsite soil 3 had the highest fungal population (2.4 × 104 CFU/g), while dumpsite soil 4 had the least fungal population (1.1 × 104 CFU/g).

3.2 Macroscopic and microscopic characteristics of the isolated fungi

The macroscopic and microscopic characteristics of the isolated fungi are presented in Table 1. Comparison of the characteristics (Hassan et al., 2014) indicated that the isolated fungi were Fusarium, Rhizopus, Aspergillus, and Penicillium species.

Figure 1: Fungal Population of the Dumpsite Soil

Table 1: Macroscopic and microscopic characteristics of isolated fungi

| Isolate Code |  | Macroscopic characteristics | Microscopic characteristics | Probable organism |
| --- | --- | --- | --- | --- |
|  | DS1 | White cottomy growth with yellow reverse | Canoe-shaped conidia, with septate hyphae | Fusarium sp |
|  | DS2 | Gray and black fluffy growth with brown reverse. | Aseptate hyphae with round conidia head | Rhizopus sp |
|  | DS3 | Black cottony growth with white radial periphery and brown reverse | Septate hyphae with round conidia head | Aspergillus niger |
|  | DS4 | Green, lawny growth with white radial periphery. | Septate branching hyphae with chain-like conidia | Penicillium sp |
|  | DS5 | Green cottony growth with white periphery | Septate hyphae with columnar conidia head | Aspergillus sp |
|  | DS6 | White cottomy growth with yellow reverse | Canoe-shaped conidia with septate branching hyphae | Fusarium sp |

3.3 Glucoamylase activity of isolated fungi

The glucoamylase activity of the isolated fungi is presented in Table 2, from the Table, it can be seen that Aspergillus niger had the highest glucoamylase activity with a clearance zone size of (18.3±0.4) mm, while Fusarium sp had the least glucoamylase activity with a clearance zone size of 2.1±0.2mm.

## 3.4 Proximate composition of Yam Peel and Potato Peel Residues

The proximate composition of the agro-residues is presented in Table 3. The ash content of the yam peel and potato peel was 4.94±0.04% and 5.06±0.06%, respectively. The moisture content of the yam peel and potato peel was 17.06±0.04% and 15.16±0.06%, respectively. Lipid content for yam peel and potato peel was 3.17±0.02% and 1.30±0.02%, respectively. Carbohydrate content was 73.16±0.05% and 76.51±0.01% for the yam peel and potato peel, respectively. The fibre content of the yam peel and potato peel was 1.42±0.03% and 1.74±0.02%, respectively. Protein content was 0.25±0.02 and 0.23±0.03 for the yam peel and potato peel, respectively.

Table 2: Glucoamylase activity of the isolated fungi

| Isolate code |  | Organism |  | Clearance zone (mm) |
| --- | --- | --- | --- | --- |
| S1 |  | Penicilium sp |  | 7.3±0.3D |
| S2 |  | Aspergillus niger |  | 18.3±0.4A |
| S3 |  | Fusarium sp |  | 2.1±0.2F |
| S4 |  | Rhizopus sp |  | 2.3±0.3F |
| S5 |  | Aspergillus niger |  | 15.2±0.2B |
| S6 |  | Aspergillus sp |  | 13.2±0.2C |
| Pr > F(Model) | < 0.0001 |  |
| Significant | Yes |  |

Key: Data presented as Mean ± Standard Deviation; Means that share the same superscripts do not have a significant difference

## Table 3: Proximate composition of Potato Peel and Yam Peel

| Parameter | Potato peel | Yam peel | p-value |
| --- | --- | --- | --- |
| Ash (%) | 5.06±0.06 | 4.94±0.04 | 0.192ᵃ |
| Moisture (%) | 15.16±0.06 | 17.06±0.04 | < .001ᵃ |
| Crude lipid (%) | 1.30±0.02 | 3.17±0.02 | < .001 |
| Crude fibre (%) | 1.74±0.02 | 1.42±0.03 | 0.004ᵃ |
| Crude protein (%) | 0.23±0.03 | 0.25±0.02 | 0.159ᵃ |
| Carbohydrate (%) | 76.51±0.01 | 73.16±0.05 | < .001ᵃ |

ᵃ Brown-Forsythe test is significant (p < .05), suggesting a violation of the equal variance assumption

# 3.5 Effect of Fermentation Periods on Glucoamylase Production using the agro-residues and fungi

The maximum yield 37.13IU/ml of glucoamylase was recorded in Apergillus niger while Aspergillus sp had glucoamylase yield of 26.12 IU/ml recorded on the 10th day of the fermentation period. The minimum yield 15.21IU/ml of glucoamylase was recorded in Penicillium sp on the 10th day of fermentation period. Additional increase in fermentation period decreased the production of glucoamylase (fig. 2).

3.6 Effect of pH on glucoamylase production using the agro-residues and the fungi

# The effect of pH on glucoamylase production using the agro-residues and Aspegillus niger, Penicillium     sp and Aspergillus sp is presented in fig 3. From the figure, it can be seen that maximum enzyme production for the 3 fungi were obtained at pH 6, with the highest production in Aspegillus niger   and the least in Penicillium sp.

# 3.7 Effect of inoculum size on the glucoamylase production using the fungi and the agro-residues

# The maximum glucoamylase production was observed at 3ml inoculums (fig. 4). Afterwards the production decreased in 4ml inoculum size.

# Figure 2: Effect of Fermentation Periods on Glucoamylase Production using the agro-residues and fungi

# Figure 3: Effect of pH on glucoamylase production using the Agro-residues and the fungi

# Figure 4: Effect of Inoculum Size on Glucoamylase Production using Agro-residues and the fungi

# 3.8 Effect of Different Carbon Sources on the Glucoamylase Production

The maximum glucoamylase enzyme production was observed when glucose was supplemented in the fermentation medium (fig. 5). The Aspergillus niger had highest glucoamylase production (32.17 IU/ml) while Penicillium sp had the least (12.04 IU/ml).

# 3.9 Effect of nitrogen sources on the glucoamylase production

The production of glucoamylase with the supplementation of various nitrogen sources is shown in Fig. 6. Maximum glucoamylase production was observed in urea-supplemented medium. Aspergillus niger had the highest enzyme production, while Penicillium had the least.

Figure 5: Effect of Different Carbon Sources on the Glucoamylase Production

Figure 6: Effect of Different Nitrogen Sources on the Glucoamylase Production

“Fungal populations in the dumpsite soil used in this study were in magnitudes of 104 CFU/g. The accessibility of fungal biodiversity in soil for the exploration of enzymes in industrial sectors is an area of research interest” (Ominyi et al., 2013). “In this study, 6 fungal isolates were obtained from different dumpsite soils and assessed for glucoamylase. Many reports show isolation of fungi from soil samples of different regions for the study of amylase” (Varalakshmi et al., 2009; Anto et al., 2006; Khokhar et al., 2011; Lawal et al., 2014). Kumar et al. (2015) reported “the isolation of soil mycoflora in agricultural fields at Tekkai Mandal, wherein the samples were collected from the rhizosphere and rhizoplane regions of different rice fields”. Similarly, Abdullah et al. (2016) isolated “fungi from 5 different regions in explosive institutes and contaminated sites in Riyadh province and reported that the isolated fungi in contaminated soils can be used as bioremediation agents for explosive materials”. In another study, Singh et al. (2014) reported “isolation of fungal strains from soil samples collected from different sites of the Jalandhar region of Punjab for the study of amylase activity and the production of fungal amylase was studied using cheap, readily available agricultural residues”. Ominyi et al. (2013) reported “26 fungal isolates showing starch hydrolysis test out of 120 fungi isolated from the soil of rice mill industrial areas, corn processing industries, rice milling and dumping ground soil of Nigeria. The variation in diversity index of collection sites indicates that soil harbours a plethora of fungal populations which can be used for industrial applications”. Jena et al. (2015) studied “the occurrence of culturable soil fungi in Odisha, wherein the study reported Shannon-Wiener indices to be 3.12 and 0.9425, respectively, in summer and winter seasons. Glucoamylase screening of the fungal isolates showed that the fungal isolates had the potential to produce glucoamylase. Aspergillus niger was observed to have the highest glucoamylase production potential while Fusarium had the least”. Similarly, Mishra and Dadhich (2010) isolated “15 filamentous fungi from the soil of different regions of Rajasthan state and assessed the isolates for amylase-producing ability”. In a similar observation, Ominyi et al. (2013) screened “the isolates from soil of rice mill industrial areas, corn processing industries, rice milling and dumping ground soil and reported 26 isolates to show maximum amylase production in the starch hydrolysis test of the potassium iodide method”. Sohail et al. (2005) studied “the screening of microbial populations for their ability to possess amylolytic potential and reported that a total of 130 fungal isolates were screened, and 21 fungal isolates from soil samples to be amylase producers. Aspergillus species was found to be most active amylase producer with a frequency of 16.15% of all the isolates. The proximate composition of yam peel and potato peel showed that the amount of carbohydrate within them is adequate to serve as substrate for enzyme production from fungi. The most appreciable enzyme production was observed with the use of potato peel as substrate” (Jahir and Sachin, 2011). “There are several reports of the use of agricultural wastes as cheap substrates for microbial α-amylase production” (Xu et al., 2008; Murthy et al., 2009; Jahir and Sachin, 2011). “The selection of an agricultural waste as substrate for enzyme production depends upon several factors, mainly related with cost and availability of the substrate. The solid substrate not only supplies the nutrients to the microbial culture growing in it but also serves as anchorage for the cells” (Nimkar et al., 2010). “Potato peel wastes contribute the major industrial food waste discarded in the environment, arising from potato processing and home wastes” (Shukla and Kar, 2006; Schieber et al., 2009). “Biotechnological applications of potato peel wastes are interesting not only from the point of view of a low-cost substrate, but also in solving problems related to their disposal” (Schieber et al., 2009). Maximum glucoamylase production was obtained with the agro-residues and Aspergillus niger. Other related studies (Nyamful et al., 2014; Keera et al., 2014) have shown “maximum amylase production using Aspergillus species. A slightly acidic pH of 6 was shown to have a positive effect on glucoamylase production”. This is consistent with the findings of Keera et al. (2014) and Imran et al. (2012), who achieved “maximum production effect at a slightly acidic pH of 5.5. An inoculum size of 3 ml was shown to have a positive effect on glucoamylase production”. In a similar research conducted by Imran et al. (2012), “maximum production was observed at 5 ml of inoculum size”. Similar result were reported by Puri et al. (2013) who “used agricultural byproducts like wheat bran and rice bran substrate for the synthesis of glucomaylase by solid state fermentation using Aspergillus niger. The maximum enzyme production was observed when glucose was supplemented in the fermentation medium, followed by fructose, and the least production was found in the maltose-supplemented medium”. Anto et al. (2006) also reported “enhanced enzyme production with glucose supplementation. Keera et al. (2014) also reported that the highest production was observed with glucose when Aspergillus oryzae FK 923 was used. The nitrogen source that showed have positive effect on the enzyme production was urea”. Keera et al. (2014) reported that “urea stimulated production of glucoamylase by A. niger. In another study (Nahar et al., 2008), urea was shown to enhance glucoamylase production by Rhizopus”.

## 4.0 CONCLUSION

The fungal cultures were isolated from dumpsite soil samples collected from Rivers State University dumpsites. Glucoamylase screening of the fungal isolates showed that the fungal isolates had the potential to produce glucoamylase. The yam peel and potato peel were supplemented as a carbon source and urea as a nitrogen source. Aspergillus niger was observed to have the highest glucoamylase production potential while Fusarium had the least. Maximum glucoamylase production using agro-residues and fungi was shown to occur using Aspergillus niger under the following fermentation parameters: fermentation duration 10days, pH 6, inoculum size 3 ml, supplementary carbon and nitrogen source- glucose and urea respectively.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

REFERENCES

Abdullah FM, Al-Dhabban and Bakhali AH (2016). Identification of contaminated soil from isolated fungi in Riyadh province. Life Science Journal; 13(2): 51-56.

Almulaiky, Y. Q., Khalil, N. M., & El-Shishtawy, R. M (2021). Hydroxyapatite-decorated ZrO2 for α-amylase immobilization: Toward the enhancement of enzyme stability and reusability. International Journal of Biological Macromolecule, 167:299-308. doi: 10.1016/j. ijbiomac.2020.11.150.

Anto, H., Trivedi, U. B., & Patel, K. C. (2006). Glucoamylase production by solid- state fermentation using rice flake manufacturing waste products as substrate. Bioresource Technology, 97(10), 1161–1166.

Bamigboye, C. O., Okonji, R. E., Oluremi, I. O., & James, V. (2022). Stain removing, juice-clarifying, and starch-liquefying potentials of amylase from Pleuro- tus tuberregium in submerged fermentation system. Journal of Genetic Engineering and Biotechnology, 20(1), 1–10.

Contesini, F. J., de Alencar Figueira, J., Kawaguti, H. Y., de Barros Fernandes, P. C., de Oliveira Carvalho, P., Nascimento, M. D. G., & Sato, H. H. (2013). Potential applications of carbohydrases immobilization in the food industry. International Journal of Molecular Sciences, 14(1), 1335-1369

De Castro, A., T. V., dos Reis Castilho, L., M., de Andréa & Freire, D. M. G. (2010). Use of mesophilic fungal amylases produced by solid-state fermen- tation in the cold hydrolysis of raw babassu cake starch. Applied Biochemistry and Biotechnology, 162(6), 1612–1625. https://doi.org/10. 1007/s12010-010-8942-z.

Imran M, Asad MZ, Gulfaraj M, Qureshi R, Gul H and Manzoor N (2012). Glucoamylase Production from Aspergillus niger by using Solid state fermentation process. Pakistan Journal of Botany; 44(6):2103- 2110.

Jahir AK and Sachin KY (2011). Production of alpha amylases by Aspergillus niger using cheaper substrates employing solid state fermentation. International Journal of Plant, Animal and Environmental Sciences, 1(3): 231-290.

Jena SK, Tayung T, Rath CC and Parida D (2015). Occurrence of culturable soil fungi in a tropical moist deciduous forest Similipal Biosphere Reserve, Odisha, India. Brazil Journal Microbiology; 46(1): 85-96.

Kalia, S., Bhattacharya, A., Prajapati, S. K et al. (2021). Utilization of starch effluent from a textile industry as a fungal growth supplement for enhanced α-amylase production for industrial application. Chemosphere. 279:130554. doi: 10.1016/j.chemosphere.2021.130554.

Keera A, Mouai F, Kahil Fade M, and Abedo A (2014). Hyper Production of Glucoamylase by Aspergillus oryzae FK- 923 under Solid State Fermentation. World Applied Sciences Journal, 30(11), 1447-1461.

Khokhar et al. 2011, Khokhar I, Mukhtar I and Mushtaq S (2011). Isolation and screening of amylolytic filamentous fungi. Journal Applied Science Environment Management 15(1): 203 -206.

Kumar PKR, Hemanth G, Shiny NP and Samauel KK (2015). Isolation and identification of soil mycoflora in agricultural fields at Tekkali Mandal in Srikakulam District. International Journal Advance Pharmacology and Biochemistry; 4(2): 484-490

Lawal A, Banjoko K, Olatope AM, Alebiosu SO, Orji FA, Suberu FA, Itoandon YL, Shittu EE, Adelaja KA, Ojo OD, Dike EN and Elemo GN (2014). Production and partial purification of glucoamylase from Aspergillus niger isolated from cassava peel soil in Nigeria. African Journal Biotechnology; 13(21): 2154-2158.

Mishra BK and Dadhich SK (2010). Production of amylase and xylanase enzyme from soil fungi of Rajasthan. Journal Advance Development Resource; 1(1): 21 -23

Murthy PS, Naidu MM, and Srinivas P (2009). Production of α-amylase under solid substrate fermentation utilizing coffee waste. Journal of Chemistry Technology and Biotechnology, 84: 1246-1249. DOI: http://dx.doi.org/10.1002/jctb.2142

Muthusamy, S., Ajit, S., Nath, A. V., Anupama Sekar, J., & Ramyaa Lakshmi, T. S. (2022). Enzymes from genetically modified organisms and their current applications in food development and food chain. In A. Dutt Tripathi, K. Novel Food Grade Enzymes, https://doi.org/10.1007/978-981-19-1288-7_13

Nahar S, Hossain F, Ferosa B and Hallm MA (2008). Production of glucoamylase by Rhizopus in liquid culture. Pakistan Journal of Botany; 40(4):1693-1698.

Nimkar MD, Deogade NG and Kawale M (2010). Production of alpha-amylase from Bacillus subtilis and Aspergillus niger using different agro waste by solid state fermentation. Asian Journal of Biotechnological Research, 1: 23-28.

Nyamful A, Moses E, Ankudey E and Woode M (2014). Soilid state fermentation by Aspergillus niger MENATE and Rhizopus MENACO11 for glucoamylase productionon Agriculture Residue. Biochemical Engineering Journal, 15(2), 107–115. https://doi.org/10.1016/S1369-703X(02)00192-4

Olakusehin, V.O. and Oyedeji, O. (2021). Production of α-amylase from Aspergillus flavus S2-OY using solid substrate fermentation of potato (Solanum tuberosum L.) peel. International Journal of Biological and Chemical Sciences. 15(5):1950-1967.

Ominyi MC, Ogbonna JC, Nwoba EG, Nwagi KE and Ukachi R (2013). Isolation and screening of amylase and glucoamylase producing fungi and their application in bioethanol production. International Journal Science Nation; 4(1): 44- 50.

Pasin, T. M., Dos Anjos Moreira. E., & de Lucas, R. C (2020). Novel amylase-producing fungus hydrolyzing wheat and brewing residues, Aspergillus carbonarius, discovered in tropical forest remnant. Folia Microbiologica. 65(1):173-184. doi: 10.1007/s12223-019-00720-4.

Puri S, Arora M, and Sarao L (2013). Production and optimization of amylase and glucoamylase using Aspergillus oryzae under solid state fermentation Authors: ISSN 2277–3843.

Sabino, T. P. F., Coelho, N. P. F., & Andrade, N. C (2021). Lignocellulosic materials as soil-cement brick reinforcement. Environmental Science and Pollution Research International, 29(15):21769-21788. doi: 10.1007/ s11356-021-17351-3.

Schieber A, Marleny D, Saldana A. 2009. Potato peels: A Source of Nutritionally and Pharmacologically Interesting Compounds-A review. Food, 3(2): 23-29

Shukla J and Kar R (2006). Potato Peel as a Solid Substrate for Thermostable α-amylase Production by Thermophilic bacillus Isolates. World Journal of Microbiology and Biotechnology, 22: 417-422. DOI: http://dx.doi.org/10.1007/s11274-005- 9049-5

Singh S, Singh S, Bali V, Sharma L and Mangla J (2014). Production of Fungal Amylases Using Cheap, Readily Available Agri-residues for Potential Application in Textile Industry. Biomedical Resource International: 215748.

Singh, R. S., Singh, T., & Pandey, A. (2019). Microbial enzymes—An overview. In Advances in Enzyme Technology, (pp. 1–40). DOI: 10.1016/B978-0-444-64114-4.00001-7

Sohail M, Ahmad A, Shahzad S and Khan SA (2005). A survey of amylolytic bacteria and fungi from native environmental samples. Pak. J. Bot 37(1): 155- 161.

Varalakshmi KN, Kumudini BS, Nandini BN, Solomon J, Suhas R, Mahesh B and Kavitha AP (2009). Production and characterization of alpha amylase from Aspergillus niger JGI 24 isolated in Bangalore. Polish Journal Microbiology; 58(1): 29- 36.

Xu H, Sun L, Zhao D, Zhang B, Shi Y and Wu Y (2008). Production of α-amylase by Aspergillus oryzae in solid substrate fermentation using spent brewing grains as substrate. Journal of Science and Food Agriculture, 88: 529-535. DOI: https://doi.org/10.1002/jsfa.3118

Zambare, V., 2010. Solid State Fermentation of Aspergillus oryzae for Glucoamylase production on agro residues. International Journal of Life Science, 4: 16-25.

Maghraby, Y. R., El-Shabasy, R. M., Ibrahim, A. H., & Azzazy, H. M. E. S. (2023). Enzyme immobilization technologies and industrial applications. ACS omega, 8(6), 5184-5196.

Das, S., Raucci, U., Neves, R. P., Ramos, M. J., & Parrinello, M. (2023). How and when does an enzyme react? Unraveling α-Amylase catalytic activity with enhanced sampling techniques. ACS Catalysis, 13(12), 8092-8098.

Hasan, K., Baroroh, U., Madhani, I. N., Muscifa, Z. S., Novianti, M. T., Abidin, M., ... & Subroto, T. (2024). Enzymatic performance of Aspergillus oryzae α-Amylase in the presence of organic solvents: activity, stability, and bioinformatic studies. Bioinformatics and Biology Insights, 18, 11779322241234767.

Dhevagi, P., Ramya, A., Priyatharshini, S., Geetha Thanuja, K., Ambreetha, S., & Nivetha, A. (2021). Industrially important fungal enzymes: productions and applications. Recent Trends in Mycological Research: Volume 2: Environmental and Industrial Perspective, 263-309.