# Original Research Article

# A Residual Network based on MLP and its

# 2	applications in energy forecasting

3

# 5	Abstract

6	After decades of development, the Multilayer Perceptron (MLP) has made significant strides.

7  However, many MLP models still encounter the issue of degradation as depth increases. Fortunately,

8  recent research has shown that the Residual Network (ResNet) can mitigate or even overcome this

9  phenomenon. In this paper, we propose a Residual Network based on MLP to enhance model appli-

10  cability and stability, thereby improving data forecasting accuracy. We employ the Adam algorithm

11  for model training and the Gridsearch algorithm for hyperparameter tuning. In the Application

12  Section, we validate the effectiveness of our proposed model using three real-world energy-related

13  cases and compare it with 10 other machine learning models. Our results demonstrate that the

14  proposed model outperforms others in all three cases, highlighting its versatility and robustness.

15	Keywords: Residual Network, Multilayer Perceptron, Adaptive Moment Estima-

16  tion, Gridsearch.

# 17	1	Introduction

18	In recent decades, the evolution of deep learning techniques has revolutionized

19  the field of artificial intelligence, paving the way for sophisticated models capable of

20  capturing complex patterns in data. Among these, Multilayer Perceptrons (MLPs) have

21  played a foundational role, serving as the building blocks for various neural network

22  architectures.

23	A layered network of perceptrons is first introduced by Frank Rosenblatt in his

24  book Perceptron [1] [2] [3]. The perceptrons in his book is composed of an input layer,

25  a hidden layer with randomized weights which did not learn, and an output layer with

26  learning connections. But this is seen as a extreme learning machine [4] but not a

27  deep learning network. Although this early form of MLP was not considered a deep

28  learning network, it laid the groundwork for subsequent advancements. In 1965, Alexey

29  Grigorevich Ivakhnenko and Valentin Lapa published the first deep-learning feedforward

30  network, known as the Group Method of Data Handling, which did not yet utilize

31  stochastic gradient descent [5] [6]. Two years later, Shun’ichi Amari introduced a deep-

32  learning network capable of classifying non-linearly separable pattern classes, marking

33  the first use of stochastic gradient descent in such networks [7]. And his team also

34  built a five-layered feedforward network, demonstrating the feasibility of deep learning

35  architectures. The modern backpropagation method, a crucial component of MLP

36  training, was first published in 1970 by Seppo Linnainmaa [8]. This efficient application

37  of a chain-rule-based supervised learning approach revolutionized the training of neural

38  networks by enabling the propagation of errors through the network to update the model

39  parameters. Subsequent improvements to the backpropagation algorithm, including its

40  standardization by Paul Werbos in 1982 [9], and experimental analyses conducted by

41  David E. Rumelhart et al. in 1985 [10], further solidified its importance in the field of

42  deep learning.

43	So far, The MLP model and its variants have been widely used in many fields ,

44  such as finance [11], bioinformatics [12], transportation [13], agricultur [14], medical [15]

45  and etc.. The MLP especially plays an important role in time series analysis including

46  regression and classification. In 2023, FINANNISA ZHAFIRA and etc. combine LSTM

47  and MLP to establish a model that can effectively reduce training costs [16]. In the

48  same year, Si-An Chen and etc. present Time-Series Mixer (TSMixer) which is a novel

49  architecture designed by stacking multi-layer perceptrons (MLPs) and prove its surpe-

50  rior performance on a real-world retail dataset [17]. Sujan Ghimire and etc. propose an

51  novel hybrid method which integrates convolutional neural network (CNN) with MLP

52  and forecasts global solar radiation (GSR) successfully [18].

53	However, the depth of the MLP model is limited by the vanishing gradient problem,

54  making it difficult to train deeper networks. To solve the problem of vanishing gradient,

55  in 1991, Sepp Hochreiter introduced skip connections or residual connections in the

56  long short-term memory (LSTM) recurrent neural network to solve this problem [29].

57  Subsequently, in 2015, the concept of Highway Networks was proposed, applying the

58  concept of forget gates in LSTM to the feedforward neural network, allowing information

59  to spread in the network and alleviating the vanishing gradient problem [30]. Then,

60  based on Highway Networks, ResNet further simplifies the structure, removes forget

61  gates, and uses simple skip connections directly, so that signals can be propagated

62  directly without the intervention of the gating mechanism. This structure has proven

63  to be very effective in training very deep neural networks [31].

64	This work aims to enhance the applicability of the mlp model so that it can adapt

65  to more situations.  The combination of the MLP and ResNet can synergizing the

66  strengths of two models, making the model has greater versatility and stability.

67	In the rest of the paper, the theory of MLP-ResNet and its solution will be shown

68  in Section 2; applications in 3 real-world cases in energy field will be represented in

69  Section 3; the conclusion of this paper is shown in Secton 4.

# 70	2	Theoretical Framework

## 71  2.1	Knowledge Background

### 72  2.1.1	Multiple Layer Perceptron (MLP)

73	Formally, an MLP consists of an input layer, one or more hidden layers, and an

74  output layer. Each layer is composed of numerous artificial neurons, also referred

75  to as perceptrons or nodes, interconnected via weighted connections. The primary

76  function of the MLP is to transform input data through successive layers of nonlinear

77  transformations, ultimately producing an output prediction. A simple mlp network

78  structure with one-hidden-layer is shown in Fig.(1).

Feature 1

Feature 2

Feature n

output

Input Layer

Hidden Layer

Output Layer

Fig. 1. Simple mlp network structure (single hidden layer)

79	Mathematically, the forward propagation process of an MLP can be expressed as

80  follows: for each layer l, the output x(l) is computed as the application of a nonlinear

81  activation function σ to the linear transformation of the previous layer’s output x(l−1),

82 incorporating weights W(l) and biases b(l) :

z(l) = W(l)x(l−1) + b(l)

x(l) = σ z(l)

(1)

83	During training, the parameters (weights and biases) of the MLP are optimized to

84  minimize a predefined loss function, typically through backpropagation and gradient-

85  based optimization techniques. Backpropagation involves the systematic calculation of

86  gradients with respect to the parameters of the network, facilitating parameter updates

87  in the direction that reduces the loss.

88	MLPs are characterized by their universal approximation capabilities, enabling

89  them to approximate arbitrary functions with sufficient capacity and data. However,

90  their effectiveness is contingent upon various factors, including network architecture

91  design, activation functions, optimization algorithms, and hyperparameter tuning.

92

93

94

95

96

97

98

99

100

101

102

103

104

105

106

107

108

109

### 2.1.2	Residual Network (ResNet)

Recent advancements in the field of image recognition have underscored the crit- ical role of network depth, particularly in Convolutional Neural Networks (CNNs), as elucidated by recent studies [32]. However, the efficacy of deeper networks is marred by a phenomenon termed degradation, wherein the accuracy of the model plateaus and subsequently declines rapidly with increasing depth. Notably, this degradation does not stem from overfitting but rather from optimization challenges.

Weight Layer

Weight Layer

Weight Layer

Activation Function

Weight Layer

+

Activation Function

Addressing this inherent limitation, ResNet (Residual Network) presents a pioneer- ing solution by introducing a residual learning framework. Unlike conventional CNNs where each layer aims to directly learn the underlying target function H(x), ResNet adopts a distinctive learning objective defined as F (x) := H(x) − x. This formulation epitomizes residual learning, where the network endeavors to learn the residual informa- tion of x in H(x). Distinguishing between the architectural setups of conventional CNN blocks and ResNet blocks is shown in Fig.(2). By reframing the learning task in terms of residual functions, ResNet facilitates more efficient optimization, as it is inherently easier to learn residuals than to directly learn complex target functions. This approach enables ResNet to navigate around the degradation issue by traversing a detour through residual learning pathways.

Activation Function

Activation Function

Fig. 2. Distinguishing between the architectural setups of conventional CNN blocks and ResNet blocks.

110	The core architectural feature of ResNet is the incorporation of "shortcut connec-

111  tions" or "skip connections," which facilitate identity mapping. Through these connec-

112  tions, the original input x is added directly to the output of the stacked layers, thereby

113  enabling the flow of information without significant alteration. This mechanism not only

114  fosters smoother gradient flow during backpropagation but also mitigates the vanishing

115  gradient problem commonly encountered in deep networks.

## 116  2.2	The proposed MLP-ResNet model

### 117  2.2.1	The representation of MLP-ResNet model and its solution

118	As we mentioned before, it’s apparent that the most existing machine learning

119  models including MLP often face degradation phenomenon which means the accuracy

120

121

122

123

n

n

n

n

124

of the model declines rapidly when increasing depth. In order to better forecast and enhance the MLP’s versatility, we propose the model which combines the MLP and the ResNet which is called MLP-ResNet (MLPRS) in this paper. The structure of the MLPRS is shown in Fig.(3) and the ⃝ with number means the neuron index in each layer. The number of MLPRS BLOCK depends on the depth of MLPRS.

1

1

1

2

Feature

1

1

2

2

+

2

Feature 2

2

MLPRS BLOCK

Feature 1

2

MLPRS BLOCK

1

1

output

n

n

n

Fig. 3. Structure of MLPRS

125

126

127

128

Suppose (X, y) is the input data of the model which X has n1 features, it is composed of (Xt, yt)(t = 1, 2, · · · , n) and we constuct the MLPRS model with k depth. The MLP in the proposed model has one-hidden-layer, we could obtain the output of MLPRS as follows:

h0 = W (1)X + b(1),	(2)

129  where h0 is the output of ResNet’s input layer.

130	When it come to the first MLPRS BLOCK, we could write the mathematical

131  expression as follows:

h1 = W (2)h0 + b(2),

1

h2 = δ(h1), h3 = W (3)h2 + b(3), h4 = W (4)h3 + h0,

(3)

132

133

134

135

where h1, h2, h3 are the state of the nerurons in MLP model (the input layer, hidden layer and output layer of MLP respectively), and h4 is the output of the first block. δ(·) is the activation function of the MLP model.

Similar to the first block, the formula of the second block could be written as

136

follows:

h5 = W (2)h4 + b(2),

2

h6 = δ(h5), h7 = W (3)h6 + b(3), h8 = W (4)h7 + h4,

(4)

137

138

139

140

141

142

143

144

145

146

After the iteration of k depth, the k-th block’s output h4k could be obtained:

k

h4k = W (4)h4n−1 + h4n−4.	(5)

Finally, we could calculate the output yˆ of the ResNet:

yˆ = W (5)h4n + b(5)	(6)

p

In Eq.(2)(3)(4)(5)(6), W (i)(i = 1, 2, 3, 4, 5) and b(j)(j = 1, 2, 3, 5) are the parame- ters of neural network in MLPRS. W (4)(p = 1, 2, · · · , k) is the k-th element in W (4).The specific shape of the parameter matrix are shown in Tab.1.

Table 1: The specific shape of the parameter matrix.

| parameter W (1) | W (2) | W (3) | W (4) | W (5) |
| --- | --- | --- | --- | --- |
| shape (n1, n2) | (n2, n3) | (n3, n2) | (1, k) | (n2, 1) |
| parameter b(1) | b(2) | b(3) | b(5) |  |
| shape (1, n2) | (1, n3) | (1, n2) | (1, 1) |  |

The activation function has a variety of choices which depends on specific applica- tion scenarios, here we use Tanh as the activation function of MLPRS. The output range of the Tanh function is within [−1, 1], which helps to reduce the problem of vanishing gradients and helps the network converge. The Tanh function δ(·) could be express as follows:

δ(x) =

ex − e−x

ex + e−x

(7)

### 147  2.2.2	Adam algorithm for training the MLP-ResNet model

148	Normally, the neural network could not obtain an analytical solution, and the

149  proposed MLPRS model in this paper is no exception. So we need to use optimization

150  algorithms to get its solution such as Gradient Descent [33], Stochastic Gradient Descent

151  [34], Adam [35]. In this paper, we introduce the Adam algorithm to train the proposed

152  model due to its efficiency, robustness, and adaptability.

153	First, we need to define the training error et at each point (Xt, yt):

et = yt − (W	h4n + b	),	(8)

(5)	(5)

154	Thus, we could obtain the sum of training error:

n

n

t

E(W , b) = 1 Σ e2 = eT e,	(9)

t=1

155  where W is composed of W (i) and b is composed of b(j). W and b are the parameters

156  whcih need to be solved by Adam.

157	Then, in order to complete Adam, we need to finish the gradient descent. So we

158  have to get the gradient of E(W , b):

159

where L is the gradient.

∂E

L = [	,

∂W

∂E ],	(10)

∂b

160

161

162

163

164

Different with ordinary gradient descent, it introduces the concept of the modified bias-corrected first moment estimate mˆ t and bias-corrected second raw moment estimate vˆt to speed up convergence, their mathematical expressions are as follows:

mt = µ1 · mt−1 + (1 − µ1) · L,	(11)

=	,	(13)

vt = µ2 · vt−1 + (1 − µ2) · L2,	(12)

165

mˆ t

t

vˆ

mt

1 − µ

t

vt

1

=	,	(14)

2

1 − µt

166  where µ1 and µ2 mean decay rates which are used to control the decay speed of the

167  first and second moments of the gradients, respectively.

168	Finally, we can obtain the iterative formula:

=

— l1 · √vˆ

.	(15)

+ ϵ

" W t+1  #

bt+1

" W t #

bt

mˆ t

t

169

170

where l1 is the learning rate of Adam and ϵ is a small constant. The complete algorithm of Adam is shown in Algorithm 1:

Algorithm 1: The complete process of Adam training MLPRS Input: E(W , b) (Eq.(9)), Learning rate l1, max_epochs Initialize: [W , b] → random();

µ1 → 0.9; µ2 → 0.999;

m0 → 0; v0 → 0;

epoch → 0;

1 while epoch < max_epochs do

171

2	ephoch = ephoch + 1 ;

3	L → Eq.(10);

"^  ^

4	mt, vt → Eq.(13)(14);

W t+1

5

bt+1

# → Eq.(15);

6 end

7 return [W , b]

### 172  2.2.3	Optimal Model Parameter Selection Using Gridsearch algorithm

173	In Section 2.2.2, we obtain the parameter set [W , b] through Adam algorithm, but

174  the depth k, learning rate l2 and the number of nunber of neurons nr(r = 2, 3) are still

175  need to be tuned. Here we introduce the Gridsearch algorithm to tune the parameters.

176	The basic principle of GridSearch is to exhaustively search all possible parameter

177  combinations in the parameter space, then perform cross-validation on each parameter

178  combination, and select the parameter combination with the best performance.

179	Suppose we have a model parameter space Θ, where each parameter combina-

180  tion can be represented by a vector θ which consisting of k, l2, n2, n3. Our goal is to

181  find the best parameter combination θ∗ given the training data set Dtrain, so that the

182  model can perform better on the validation data set Dval for optimal performance. Its

183  mathematical principle can be expressed as follows:

θ∗ = arg minf (θ, Dtrain, Dval),	(16)

θ∈Θ

184  where the term f (θ, Dtrain, Dval) represents the performance metric obtained by training

185  the model with parameter combination θ on the validation set Dval.  Here we use

186  Negative mean square error (NMSE) to calculate the metric:

1	 Σ	2

f (θ, Dtrain, Dval) = −	(yi − yˆi)	(17)

|Dval| i∈Dal

187	3	Applications

188	To verify the applicability and stability of the proposed model, we use 3-real-world

189  data in energy field. The data set has important practical significance and it will be

190  discussed further in later context. Moreover, min-max mapping is used to prevent

191  overflow here. The MAPE metric are used to measure the performance of the model

192  and its mathematical expression is as follows:

1 Σ |yˆ(t) − y(t)|

s

k∈U

|y(t)|

(18)

193  where U is the training or testing set and s is the length of U .

194	10 models are used to compare, and the information is shown in Tab.2.

Table 2: Information of models used for comparison

| Full Name | Abbreviation | Reference | Year |
| --- | --- | --- | --- |
| Gated Recurrent Unit | gru | [19] | 2017 |
| Random Forest Regression | rf | [20] | 2001 |
| Extreme Gradient Boosting | xgb | [21] | 2015 |
| Long Short-Term Memory | lstm | [22] | 2000 |
| Support Vector Regression | svr | [23] | 1996 |
| Convolution Neural Network | cnn | [24] | 2015 |
| Multilayer Perceptron | mlp | [25] | 2009 |
| CNN-LSTM | cnnlstm | [26] | 2019 |
| Convolutional LSTM | convlstm | [27] | 2017 |
| General Regression Neural Network | grnn | [28] | 2004 |

## 195  3.1	Case 1:Electricity Transformer Oil Temperature

196	In power distribution problems, the accuracy of voltage distribution is crucial. The

197  distribution of electricity needs to be adjusted according to the needs of different re-

198  gions, and this adjustment often depends on the continuous use of electricity. However,

199  predicting future demand in a specific region is a difficult task as it is affected by various

200  factors such as working days, holidays, seasons, weather, temperature, etc. Any incor-

201  rect prediction may damage the operation of the electrical transformer. As a result,

202  there is currently no very effective way to predict future electricity usage, and managers

203  are forced to make decisions based on empirical numbers, which are often higher than

204  actual demand. This results in unnecessary waste and depreciation of electricity and

205  equipment.

206	The transformer oil temperature can reflect the operating status of the electrical

207  transformer, so in this article we use oil temperature prediction to better solve the

208  voltage distribution problem to avoid unnecessary waste.

209	In this paper, we collect hourly transformer oil temperature data from 1:00 on June

210  1, 2018 to 19:00 on June 26, 2018 from the website https://github.com/zhouhaoyi/ETDataset.

211  The first 496 points are used to train and the rest 124 points are used to test. The

212  training plot is shown in Fig.4 and the testing plot is shown in Fig.5. The MAPE value

213  of all the models are presented in Table 3.

214	From the training and testing plot, we could observe that all the models have

215  good performance and the prediction curve are very close to the curve of raw data. In

216  addition, from Tab.3, it’s clearly that the proposed model has the smallest MAPE value

217  in testing. Only the testing values of gru and svr are close to MLP-ResNet, but both

218  of them perform worse than the proposed model in training. The rf model has the best

219  MAPE value in training but perform bad in testing.

!

!

!

"

!

!

!

!

!

!

!

"	 !

!

!

!

!

Fig. 4. Prediction values of Electricity Transformer Oil Temperature training set

Fig. 5. Prediction values of Electricity Transformer Oil Temperature testing set

Table 3: MAPE for training and testing of all the models in Case-1

| Model | MLP-ResNet | gru | rf | xgb | lstm | svr | cnn | mlp | cnnlstm | convlstm | grnn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Train | 3.623 | 3.684 | 1.620 | 3.509 | 3.800 | 4.038 | 4.190 | 5.744 | 3.649 | 3.695 | 3.221 |
| Test | 5.132 | 5.155 | 6.570 | 5.966 | 5.240 | 5.217 | 6.609 | 7.058 | 6.440 | 5.538 | 5.862 |

## 220  3.2	Case 2:U.S. Imports of Crude Oil and Petroleum Products

221	Petroleum and its products are a key component of the global economy and have

222  profound impacts on energy markets, trade balances and geopolitics. Therefore, accu-

223  rate forecasts and analysis of U.S. crude oil and petroleum product imports are critical

224  to the stability of international energy markets and the development of the global econ-

225  omy.

226	In this paper, we collect the monthly data from January 15, 1981 to June 15, 2023,

227  and the data comes from the U.S. Energy Information Administration. For convenience

228  of expression, we called the data as ICOP. The first 408 points are used for training

229  and the last 102 points are used for testing. The training and testing plot are shown in

230  Fig.6 and Fig.7. The MAPE value are shown in Tab.4.

231	All models’ curves shown in Fig.6 and Fig.7 also fit well both in training and

232  testing. From the MAPE table, while it is evident that MLP-ResNet performs best

233  during testing, its performance during training is suboptimal. The rf model achieves

234  the best MAPE value during training but performs poorly in testing.

Fig. 6. Prediction values of ICOP training set

!

!

Fig. 7. Prediction values of ICOP testing set

Table 4: MAPE for training and testing of all the models in Case-2

| Model | MLP-Resnet | gru | rf | xgb | lstm | svr | cnn | mlp | cnnlstm | convlstm | grnn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Train | 6.144 | 6.169 | 3.842 | 3.704 | 6.516 | 5.705 | 9.070 | 5.941 | 5.074 | 5.453 | 4.922 |
| Test | 4.495 | 4.580 | 4.818 | 4.637 | 4.531 | 4.754 | 4.709 | 4.748 | 4.996 | 4.514 | 4.528 |

## 235  3.3	Case 3:Inland Wind Turbine Power Generation

236	Wind energy is of great significance to combat climate change, reduce carbon emis-

237  sions, and achieve sustainable energy development. Forecasts of inland wind turbine

238  power generation can unveil the potential and feasibility of wind power generation in

239  the region. Similarly, comprehending the power generation of individual turbines in

240  inland wind farms aids in optimizing energy production and supply. By predicting

241  wind power generation and adjusting turbine operating parameters, energy utilization

242  efficiency can be enhanced, power generation costs can be minimized, and sustainable

243  energy production can be achieved.

244	Here, we use this hourly Inland Wind Turbine Power Generation data (IWTPG)

245  which is from September 11 to October 7, 2015, and the data is found from the website

246  https://zenodo.org/records/5516539. We use the first 420 points to train and the last

247  105 points to test. The training and testing plot are shown in Fig.8 and Fig.9. The

248  performance of the models are shown in Tab.5.

249	As the MAPE values shown in Tab.5, it’s apparent that MLP-ResNet has a sig-

250  nificantly lower training MAPE than the other models. Although MLP achieves the

251  second-best testing MAPE, its performance in training is much worse than that of MLP-

252  ResNet. Similar to the other two cases, the rf model also achieves the best training

253  MAPE value but performs poorly in testing.

!

!

Fig. 8. Prediction values of IWTPG training set

Fig. 9. Prediction values of IWTPG testing set

Table 5: MAPE for training and testing of all the models in Case-3

| Model | MLP-ResNet | gru | rf | xgb | lstm | svr | cnn | mlp | cnnlstm | convlstm | grnn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Train | 36.594 | 42.239 | 28.035 | 33.121 | 33.447 | 37.276 | 41.900 | 38.910 | 43.282 | 36.758 | 37.357 |
| Test | 34.914 | 59.740 | 44.011 | 38.140 | 69.743 | 39.190 | 50.164 | 36.553 | 53.151 | 44.628 | 39.438 |

254  3.4	Disscusion

255	Clearly, the proposed MLP-ResNet model performance on the training set is not

256  outstanding, but it performs the best in testing in 3 cases and the prediction curve of

257  MLP-ResNet is very close to the raw curve. This shows that it has strong generalization

258  ability and prediction accuracy. This result shows the reliability and stability of the

259  model in real scenarios. Meanwhile, the rf model achieves the best training MAPE

260  value, but it frequently performs worse in testing. The reason for this may be that the

261  model is not complex enough or overfitting occurs.

262	4	Conclusions

263	In this paper, we introduce the MLP-ResNet model, presenting a comprehensive

264  theoretical framework, model training methodology, and parameter tuning approach.

265  Moreover, according to 3 cases in Sec.3, it shows that the proposed model often performs

266  best in testing and has good versatility. The way combines the ResNet and MLP could

267  effectively improve the prediction accuracy and applicability. We believe that this kind

268  of approach could have deeper research in the future.

269	References

270	[1] Rosenblatt F. The perceptron: a probabilistic model for information storage and

271	organization in the brain[J]. Psychological review, 1958, 65(6): 386.

272	[2] Rosenblatt F. Principles of neurodynamics: Perceptrons and the theory of brain

273	mechanisms[M]. Washington, DC: Spartan books, 1962.

274	[3] Schmidhuber J. Annotated history of modern AI and Deep learning[J]. arXiv

275	preprint arXiv:2212.11279, 2022.

276	[4] Huang G B, Zhu Q Y, Siew C K. Extreme learning machine: theory and applica-

277	tions[J]. Neurocomputing, 2006, 70(1-3): 489-501.

278	[5] Ivakhnenko A G, Lapa V G. Cybernetic predicting devices[J]. (No Title), 1965.

279	[6] Ivakhnenko A G, Lapa V G. Cybernetics and forecasting techniques[J]. (No Title),

280	1967.

281	[7] Amari S. A theory of adaptive pattern classifiers[J]. IEEE Transactions on Elec-

282	tronic Computers, 1967 (3): 299-307.

283	[8] Rodríguez O H, Lopez Fernandez J M. A semiotic reflection on the didactics of the

284	chain rule[J]. The Mathematics Enthusiast, 2010, 7(2): 321-332.

285	[9] Werbos P J. Applications of advances in nonlinear sensitivity analysis[C]//System

286	Modeling and Optimization: Proceedings of the 10th IFIP Conference New York

287	City, USA, August 31–September 4, 1981. Berlin, Heidelberg: Springer Berlin

288	Heidelberg, 2005: 762-770.

289  [10] Rumelhart D E, Hinton G E, Williams R J. Learning internal representations by

290	error propagation[J]. 1985.

291  [11] Maqbool J, Aggarwal P, Kaur R, et al. Stock Prediction by Integrating Sentiment

292	Scores of Financial News and MLP-Regressor: A Machine Learning Approach[J].

293	Procedia Computer Science, 2023, 218: 1067-1078.

294  [12] Kang Y, Xu Y, Wang X, et al. HN-PPISP: a hybrid network based on MLP-Mixer

295	for protein–protein interaction site prediction[J]. Briefings in Bioinformatics, 2023,

296	24(1): bbac480.

297  [13] Li Y, Yin Z, Zheng Y, et al. Pose Estimation of Point Sets Using Residual MLP

298	in Intelligent Transportation Infrastructure[J]. IEEE Transactions on Intelligent

299	Transportation Systems, 2023.

300  [14] Samii A, Karami H, Ghazvinian H, et al. Comparison of DEEP-LSTM and MLP

301	Models in Estimation of Evaporation Pan for Arid Regions[J]. Journal of Soft

302	Computing in Civil Engineering, 2023, 7(2).

| 303 [15] | Gao S, Yang W, Xu M, et al. U-MLP: MLP-based ultralight refinement network |
| --- | --- |
| 304 | for medical image segmentation[J]. Computers in Biology and Medicine, 2023, 165: |
| 305 | 107460. |
| 306 [16] | ZHAFIRA F. OPTIMIZING LONG SHORT-TERM MEMORY NETWORKS |
| 307 | WITH MULTILAYER PERCEPTRON FOR TIME SERIES PREDICTION[D]. |
| 308 | Universitas Gadjah Mada, 2023. |
| 309 [17] | Chen S A, Li C L, Yoder N, et al. Tsmixer: An all-mlp architecture for time series |
| 310 | forecasting[J]. arXiv preprint arXiv:2303.06053, 2023. |
| 311 [18] | Ghimire S, Nguyen-Huy T, Prasad R, et al. Hybrid convolutional neural network- |
| 312 | multilayer perceptron model for solar radiation prediction[J]. Cognitive Computa- |
| 313 | tion, 2023, 15(2): 645-671. |
| 314 [19] | Dey R, Salem F M. Gate-variants of gated recurrent unit (GRU) neural net- |
| 315 | works[C]//2017 IEEE 60th international midwest symposium on circuits and sys- |
| 316 | tems (MWSCAS). IEEE, 2017: 1597-1600. |
| 317 [20] | Breiman L. Random forests[J]. Machine learning, 2001, 45: 5-32. |
| 318 [21] | Chen T, He T, Benesty M, et al. Xgboost: extreme gradient boosting[J]. R package |
| 319 | version 0.4-2, 2015, 1(4): 1-4. |
| 320 [22] | Gers F A, Schmidhuber J, Cummins F. Learning to forget: Continual prediction |
| 321 | with LSTM[J]. Neural computation, 2000, 12(10): 2451-2471. |
| 322 [23] | Drucker H, Burges C J, Kaufman L, et al. Support vector regression machines[J]. |
| 323 | Advances in neural information processing systems, 1996, 9. |
| 324 [24] | O’Shea K, Nash R. An introduction to convolutional neural networks[J]. arXiv |
| 325 | preprint arXiv:1511.08458, 2015. |
| 326 [25] | Popescu M C, Balas V E, Perescu-Popescu L, et al. Multilayer perceptron and |
| 327 | neural networks[J]. WSEAS Transactions on Circuits and Systems, 2009, 8(7): |
| 328 | 579-588. |
| 329 [26] | Kim T Y, Cho S B. Predicting residential energy consumption using CNN-LSTM |
| 330 | neural networks[J]. Energy, 2019, 182: 72-81. |
| 331 [27] | Kim S, Hong S, Joh M, et al. Deeprain: Convlstm network for precipitation pre- |
| 332 | diction using multichannel radar data[J]. arXiv preprint arXiv:1711.02316, 2017. |
| 333 [28] | Kulkarni S G, Chaudhary A K, Nandi S, et al. Modeling and monitoring of batch |
| 334 | processes using principal component analysis (PCA) assisted generalized regression |
| 335 | neural networks (GRNN)[J]. Biochemical Engineering Journal, 2004, 18(3): 193- |
| 336 | 210. |
| 337 [29] | Hochreiter S. Untersuchungen zu dynamischen neuronalen Netzen[J]. Diploma, |
| 338 | Technische Universität München, 1991, 91(1): 31. |

339  [30] Srivastava R K, Greff K, Schmidhuber J. Highway networks[J]. arXiv preprint

340	arXiv:1505.00387, 2015.

341  [31] He K, Zhang X, Ren S, et al. Identity mappings in deep residual net-

342	works[C]//Computer Vision–ECCV 2016: 14th European Conference, Amster-

343	dam, The Netherlands, October 11–14, 2016, Proceedings, Part IV 14. Springer

344	International Publishing, 2016: 630-645.

345  [32] He K, Zhang X, Ren S, et al. Deep residual learning for image recogni-

346	tion[C]//Proceedings of the IEEE conference on computer vision and pattern recog-

347	nition. 2016: 770-778.

348  [33] Andrychowicz M, Denil M, Gomez S, et al. Learning to learn by gradient descent

349	by gradient descent[J]. Advances in neural information processing systems, 2016,

350	29.

351  [34] Amari S. Backpropagation and stochastic gradient descent method[J]. Neurocom

352	puting, 1993, 5(4-5): 185-196.

353  [35] Kingma D P, Ba J. Adam: A method for stochastic optimization[J]. arXiv preprint

354	arXiv:1412.6980, 2014.