Temporal Variability of Potential Evapotranspiration in Response to Climate Change in Ambala District, Haryana, India

ABSTRACT

Potential evapotranspiration (PET) plays a critical role in irrigation planning and is needed for the determination of water demands of crops. The PET trends may have a direct influence on the production of crops, and thus identifying trends in Reference Evapotranspiration (RET) under climate change is important for understanding the effect of changing RET on agriculture. The present study was carried out with the following objectives: (1) to compute the Potential Evapotranspiration (mm/day) in Ambala using Thornthwaite formula, (2) to compute the evapotranspiration at Ambala under arid climatic conditions and (2) to investigate trends in PET using the Mann–Kendall (MK) non-parametric test. . In the present study, the non-parametric MK method was used for identifying trends in evapotranspiration and other climatic parameters. First, PET values were estimated through the Thornthwaite method for different time scales using meteorological data for 35 years from 1985 to 2019. PET was found to increase significantly at Ambala during annual, kharif, rabi, pre-monsoon, monsoon and post-monsoon time scales. On probing the causal meteorological parameters responsible for the observed PET trends in the Ambala, it was witnessed that wind speed dynamically influenced the observed PET changes at the annual time scale and all the seven seasons over the Ambala. The study showed highly significant trend in PET observed in Kharif season and found an increasing @ 0.02 mm/day, while Rabi season had increasing trend at less significance. In meteorological season pre-monsoon and post-monsoon season had highly significant increasing trend @ 0.03 mm/day and 0.01 mm/day respectively, whereas monsoon season had a less significance increasing trend. Contrary to all seasons, winter had a non-significant decreasing trend. The results of this study support that the evapotranspiration increases over Ambala. The present study will be useful for estimating the principal crop water requirements accurately under changing climates in the arid region of the Ambala.

KEYWORDS: Evapotranspiration, Potential evapotranspiration, meteorological parameters, irrigation

Introduction

Global warming has increasingly raised the concerns of both governments and the scientific community in recent decades. It is widely acknowledged that the ongoing changing patterns of precipitation and temperature have revealed an increased frequency of drought and the growing impact of aridity severity (Stefanidis & Alexandridis, 2021). “Water resources have become a prime concern for development and planning, including food production and flood control. The impact of climate change may be relatively severe with the reduction in water availability over different parts of the globe” (Jhajharia, et. Al., 2014). Kothawale and Rupa Kumar (2002) have reported that “the all India mean annual temperature has risen at a rate of 0.05 ∘C decade−1 during the last century, which is mostly due to the rise in maximum temperature (0.07°C decade−1) rather than because of the rise in minimum temperature (0.02°C decade−1). It is generally believed that a temperature rise may frequently lead to a rise in evapotranspiration”. “Recognizing that evapotranspiration is one of the basic components of the hydrologic cycle, the Food and Agriculture Organization (FAO) adopted the concept of reference evapotranspiration (RET) in the FAO guidelines for crop water requirements” based on studies by Doorenbos and Pruitt (1975, 1977), which are widely accepted for calculating evapotranspiration. The   change   in   climate   is   likely   to   have   a profound    effect    on    hydrological    cycle    viz.  precipitation,   evapotranspiration,   soil   moisture and  it  manifests  as  an  increase  in  global temperature, change in rainfall amount, increased  intensity  of  rainfall,  rising  sea  level, glacier  melting,  shifts  in  crop  growing  season, and   more   frequent   occurrences   of   extreme events  such  as  dry  spells,  wet  spells,  drought and flood (Thanushree et al., 2024).

Evapotranspiration (ET) is known as a key parameter reflecting the variations in the climate and is a major component of the meteorological and hydrological cycles (Luo  et al., 2014, Yan et al., 20220 and Yin  et al., 2020) . Given that ET is closely linked to land surface properties such as land use class (LUC), its influence on ET variability is highly relevant. Despite the recognized impacts of climate change and land use/land cover change (LUCC) on water cycles, accurately assessing their effects on ET is challenging due to the difficulty in linking hydrological impacts with LUCC and the inconsistency of land cover data (Ahmadpour et al., 2025). With the risk of heat wave events and their persistence intensifying in the recent past and increasing faster, the ET, precipitation, and other climatic variables have changed over the diverse regions (Lyakaremye  et al., 2021 and Ullah et al., 2022). In many hydrodynamic and water quality modeling system, the ET can be directly used as an input variable, so it is important to better understand the spatiotemporal features of ET and the contributing factors in the context of ongoing climate change. Furthermore, a better understanding of ET is beneficial for analysing the hydrodynamics and water quality, especially in terms of explaining the mechanisms for the variations in water quality, such as the variations in the balance of salinity and variations in contaminant concentrations. ET can be estimated by the reference evapotranspiration (ET0). It has been widely accepted that ET0 is an important factor in water balance and conversion and is thus its estimation and forecasting are important in water resources management (Traore et al., 2016 and Yan et al., 2021). For example, estimating crop water demand is a necessary step in irrigation scheduling and the design of agricultural water conservancy projects, and the estimation of crop water demand is mainly based on the calculation of ET0 (Tang et al., 2011 and Roy et al., 2020). “PET is the phase change of water, requiring a large amount of energy. Information on the amount of water required by different crops under a given set of region-specific climatic conditions will help greatly in irrigation planning and scheduling. RET is based primarily on measurements of radiation, vapour pressure, air temperature, relative humidity, and wind velocity over the evaporating surface” (Jhajharia et al., 2014). A clear understanding of the spatiotemporal variations in ET0 is of importance for better planning and management of water resources. Contrary to intuitive expectation, ET0 has been reported to decrease in many regions with increasing temperature (Burn and Hesch 2007), and this controversial behavior is known as the “evaporation paradox” (Liu, 2012). The “evaporation paradox” actually provides a straightforward indication that the ET0 variations are products of changes in multiple variables rather than any one variable, and it has attracted many researchers to investigate the spatiotemporal variations in ET0 and its contributing climatic factors

A number of researchers (Dinpashoh et al., 2011; Jhajharia et al., 2012) have analysed “trends in evapotranspiration under warmer climates around the globe using data of different durations at different locations under different types of climate”. Lawrimore and Peterson (2000), Golubev et al. (2001) and Roderick and Farquhar (2004) reported “significant decreases in either (both) pan evaporation (Epan) or (and) potential evapotranspiration (PET) over various parts of Russia and the United States, over India, over various parts of the United States, over a few sites in the United States and the former Soviet Union and Australia, respectively”. Similarly, several researchers from China reported decreasing trends in evapotranspiration (Zhang et al., 2007, 2009; Liu et al., 2010). Similarly, Jhajharia et al. (2009) and Zhang et al. (2014) reported “decreasing trends in Epan in the Chao Phraya River basin (Thailand), in northeast India and over the entire territory of China, respectively”. For other studies from the Indian subcontinent, Jhajharia et al. (2012) reported “significant decreasing trends in evapotranspiration over different parts of northeast India”.

“The PET trends may have a direct influence on the production of crops, and thus identifying trends in RET under climate change is important for understanding the effect of changing RET on agriculture. Bikaner, one of the well-known sites in the Thar Desert, has witnessed various changes due to urbanization and the extensive spread of irrigation canal networks of the Indira Gandhi Canal, carrying waters of the Satluj and Vayas Rivers through the states of Himachal Pradesh, Haryana and Punjab into the vast dry lands of the Thar Desert in the western part of Rajasthan, during the last few decades. Bikaner has witnessed significant increasing trends in minimum, maximum and mean temperatures at the annual time scale in the range of 0.1–0.4 ∘C decade−1 during the last 58 years from 1951 to 2008” (Choudhary et al., 2009). The reported increases in temperature in the Ambala area during this period provided the encouragement to determine if there would be any evapotranspiration increases under global warming scenarios in the arid climatic conditions of Ambala. As no information is available about trends in PET over Ambala, the present study was carried out with the following objectives: (1) to compute the Potential Evapotranspiration (mm/day) in Ambala using Thornthwaite formula, (2) to compute the evapotranspiration at Ambala under arid climatic conditions and (2) to investigate trends in PET using the Mann–Kendall (MK) non-parametric test.

Materials and methods

Study area and meteorological data

The present study was carried out at Department of Agricultural Meteorology, CCS HAU Hisar, to compute Spatio-temporal trend analysis of evapotranspiration at different location of Haryana viz. Ambala lies in different agroclimatic zone of Haryana. The meteorological data i.e., maximum and minimum temperature was collected from CSSRI Karnal, agro-met department, Hisar, IMD during period 1985-2019.

Mann–Kendall methods for trend analysis

In the present study, the non-parametric MK method was used for identifying trends in evapotranspiration and other climatic parameters, because this method is more suitable for non-normally distributed and censored data, and is less influenced by the presence of outliers in the data (Mann, 1945; Kendall, 1975). the Mann–Kendall test is widely used to estimate the monotonic trend of the hydro-meteorological time series and is used here for the detection of trends in PET. This has been carried out with the hypothesis (H0) that it of is no trend in the variable, but it has an alternative hypothesis (HA) that there is a monotonic trend in the said variable. Thus, MK statistic is the sum of the number of positive differences minus number of negative differences, which is calculated by using the given formula.  For the time series x1…., xn, the MK Test uses the following statistic:

Where,

Sgn (Xj-Xk) = 1 ifXj-Xk> 0

Sgn (Xj-Xk) = 0 ifXj-Xk = 0

Sgn (Xj-Xk) = -1 ifXj-Xk< 0

Further, anormalized test statistic was utilized to evaluate the statistical significance of the trend tendency of mean temperature and rainfall at 0.1, 0.05 and 0.001 significance level.

For having information of spatial distribution of PET for whole of Haryana state Geographical Information System (GIS) is used for interpolation. ArcGIS 10.4 software available at Department of Agriculture Meteorology, CCSHAU Hisar is used for this purpose.

The long-time sire’s daily temperature data was concealed in the monthly mean temperature. The monthly mean temperature was used for the computation of monthly PET by Thornthwaite method as given below:

Thornthwaite method for ET

E = 1.6 (10T/I)a

Thornthwaite method (1948) further gave the following formula for computing ET:

E = 1.6 (10T/I)a(D/12) (N/30)

Where, E = unadjusted PET, cm

T = mean monthly air temp, 0C

I = annual or seasonal heat index. It is the summation of 12 values of monthly heat indices.

i = (T/5)1.514

a = an empirical exponent

= 0.675 * 10-6  I3- 0.771 * 10-4 I2 + 1.79 * 10-2 I + 0.4924

Analysis of long-term temperature data (more than 30 years) was carried out to assess the ET. The daily temperature data for the periods of 35 years were employed to study the seasonal and annual ET trends in Ambala of Haryana state and presented in the results.

Results and Discussion

Ambala is located in northern part of Haryana and fall in eastern agro-climatic zone along with Karnal. During 1985-2019, the potential evapotranspiration of Ambala station observed maximum in June, 2012 i.e., 12.10 mm/day and minimum in January, 2003 i.e., 0.22 mm/day. Normal monthly potential evapotranspiration was maximum in June i.e., 9.02 mm/day and minimum in January i.e., 0.43 mm/day as shown in Table 1.

Table 1: Monthly Potential Evapotranspiration (mm/day) in Ambala using Thornthwaite formula (1985-2019)

| Year | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1985 | 0.47 | 1.02 | 2.97 | 5.59 | 9.40 | 10.20 | 6.94 | 6.48 | 5.58 | 2.95 | 1.35 | 0.74 |
| 1986 | 0.49 | 0.86 | 2.19 | 5.17 | 6.40 | 9.07 | 7.19 | 6.69 | 5.36 | 3.17 | 1.74 | 0.55 |
| 1987 | 0.53 | 1.11 | 2.37 | 5.35 | 5.66 | 9.41 | 9.34 | 7.89 | 6.85 | 3.96 | 1.49 | 0.60 |
| 1988 | 0.62 | 0.97 | 2.13 | 6.49 | 10.50 | 9.72 | 6.64 | 6.49 | 5.60 | 3.39 | 1.47 | 0.71 |
| 1989 | 0.41 | 0.85 | 2.29 | 4.46 | 8.21 | 7.88 | 7.66 | 6.12 | 5.63 | 3.62 | 1.58 | 0.58 |
| 1990 | 0.72 | 0.92 | 1.76 | 4.95 | 7.95 | 9.98 | 6.67 | 6.55 | 5.22 | 3.09 | 1.52 | 0.65 |
| 1991 | 0.41 | 1.01 | 2.22 | 4.35 | 8.56 | 8.13 | 8.55 | 6.54 | 5.49 | 3.12 | 1.27 | 0.61 |
| 1992 | 0.54 | 0.71 | 2.11 | 4.95 | 7.26 | 9.61 | 6.94 | 6.18 | 5.31 | 3.37 | 1.53 | 0.72 |
| 1993 | 0.45 | 1.17 | 1.69 | 4.89 | 8.81 | 9.71 | 7.13 | 7.58 | 4.69 | 2.89 | 1.40 | 0.55 |
| 1994 | 0.58 | 0.86 | 2.64 | 4.03 | 7.79 | 8.86 | 6.16 | 5.92 | 5.03 | 2.82 | 1.31 | 0.62 |
| 1995 | 0.34 | 0.69 | 1.66 | 4.07 | 8.42 | 10.35 | 7.45 | 5.80 | 5.59 | 3.82 | 1.63 | 0.75 |
| 1996 | 0.50 | 0.93 | 2.62 | 5.27 | 7.77 | 7.89 | 7.44 | 6.10 | 5.27 | 3.26 | 1.40 | 0.54 |
| 1997 | 0.40 | 0.91 | 2.20 | 4.24 | 6.62 | 7.43 | 7.43 | 6.12 | 5.60 | 2.74 | 1.47 | 0.44 |
| 1998 | 0.42 | 0.87 | 1.69 | 5.07 | 8.88 | 8.46 | 7.25 | 6.56 | 5.85 | 3.85 | 1.87 | 0.58 |
| 1999 | 0.38 | 1.11 | 2.53 | 6.93 | 8.39 | 8.18 | 6.90 | 6.22 | 5.48 | 3.52 | 1.59 | 0.57 |
| 2000 | 0.47 | 0.62 | 2.02 | 5.86 | 8.55 | 7.64 | 6.44 | 6.68 | 5.33 | 3.78 | 1.73 | 0.62 |
| 2001 | 0.24 | 0.85 | 2.13 | 4.96 | 7.87 | 7.55 | 7.50 | 8.29 | 6.55 | 5.46 | 2.83 | 1.25 |
| 2002 | 0.49 | 0.90 | 2.61 | 6.00 | 9.09 | 9.46 | 9.26 | 6.98 | 4.69 | 3.37 | 1.48 | 0.68 |
| 2003 | 0.22 | 0.98 | 2.21 | 5.77 | 7.81 | 9.22 | 6.95 | 6.79 | 5.38 | 3.24 | 1.35 | 0.58 |
| 2004 | 0.39 | 1.03 | 3.24 | 6.19 | 7.38 | 7.88 | 7.93 | 6.11 | 5.62 | 2.81 | 1.52 | 0.66 |
| 2005 | 0.34 | 0.85 | 2.17 | 5.24 | 8.35 | 9.70 | 8.38 | 6.90 | 5.29 | 3.32 | 1.38 | 0.52 |
| 2006 | 0.50 | 1.71 | 2.23 | 5.97 | 8.99 | 9.27 | 8.41 | 7.61 | 5.99 | 3.70 | 1.58 | 0.54 |
| 2007 | 0.48 | 0.90 | 2.08 | 6.27 | 8.02 | 8.65 | 7.62 | 6.81 | 5.50 | 3.10 | 1.40 | 0.50 |
| 2008 | 0.36 | 0.69 | 3.02 | 5.13 | 6.33 | 6.57 | 6.99 | 6.19 | 5.00 | 3.63 | 1.62 | 0.77 |
| 2009 | 0.55 | 1.09 | 2.58 | 5.43 | 7.42 | 9.83 | 7.33 | 6.56 | 5.23 | 3.38 | 1.30 | 0.49 |
| 2010 | 0.26 | 1.00 | 3.11 | 7.21 | 8.70 | 8.26 | 6.51 | 6.04 | 4.77 | 3.52 | 1.59 | 0.46 |
| 2011 | 0.31 | 1.03 | 2.62 | 4.98 | 8.74 | 7.64 | 7.51 | 6.92 | 5.85 | 3.80 | 1.79 | 0.57 |
| 2012 | 0.36 | 0.69 | 2.28 | 5.10 | 9.47 | 12.10 | 8.86 | 6.59 | 6.19 | 3.47 | 1.54 | 0.56 |
| 2013 | 0.30 | 0.76 | 2.29 | 5.44 | 9.69 | 8.33 | 7.88 | 6.69 | 5.98 | 4.12 | 1.46 | 0.63 |
| 2014 | 0.41 | 0.71 | 1.99 | 4.77 | 7.85 | 11.12 | 8.09 | 7.82 | 5.80 | 3.59 | 1.62 | 0.48 |
| 2015 | 0.33 | 1.15 | 2.03 | 4.72 | 8.87 | 9.06 | 7.50 | 6.70 | 5.97 | 3.89 | 1.83 | 0.66 |
| 2016 | 0.42 | 1.05 | 2.59 | 6.83 | 9.02 | 9.80 | 7.77 | 6.96 | 6.30 | 4.28 | 1.84 | 0.76 |
| 2017 | 0.53 | 1.13 | 2.42 | 6.69 | 9.64 | 9.01 | 8.34 | 7.20 | 6.08 | 4.08 | 1.46 | 0.68 |
| 2018 | 0.44 | 1.19 | 3.09 | 6.10 | 9.34 | 9.50 | 8.15 | 7.53 | 5.54 | 3.39 | 1.69 | 0.52 |
| 2019 | 0.52 | 0.87 | 1.83 | 5.85 | 8.20 | 10.37 | 7.14 | 6.70 | 6.19 | 3.54 | 1.76 | 0.37 |
| Normal | 0.43 | 0.95 | 2.33 | 5.44 | 8.28 | 9.02 | 7.55 | 6.72 | 5.60 | 3.52 | 1.58 | 0.61 |

“PET, an important parameter for irrigation planning, needs to be determined for estimation of the water demands of various crops and also for water resource development and use” (Jhajharia and Dhiman, 2006; Jhajharia et al., 2009a). The PET values estimated by the PM method at annual and seasonal time scales.

The highest and the lowest values of total PET for the annual duration were found to be 4.80 and 3.80 mm/day in 2018 and 1997, respectively. At the annual time scale, theP values were comparatively on the higher side starting from the late 1960s to the early 1970s over Ambala. The statistical properties of various parameters of PET were also obtained for annual and seasonal time scales (Table 2). These statistical properties of PET from 1985 to 2019 are useful in describing the characteristics of evapotranspiration over Ambala. The mean of the total PET was found to be 4.34 mm/day at the annual time scale. It is worthwhile mentioning that the total annual PET values over Ambala remained very high, i.e. above 4.34 mm for 15 years, i.e. 1955, 1987, 1988, 2001, 2022, 2005, 2006, 2012, 2013, 2014, 2015, 2016, 2017, 2018 and 2019 with the occurrence of the highest PET (4.80 mm/day) in 2018. On the other hand, the total annual PET values remained very low, i.e. varying from 3.80 to 4.31 mm/day. The above-mentioned results related to the annual PET values, both the highest and the lowest values, witnessed over Ambala.

At the seasonal time scale, the mean of PET values estimated by the PM method for the six different seasons varied from 0.69 mm/day (winter) to 7.22 mm/day (monsoon) over Ambala. The maximum (minimum) of the PET varied from 8.05 (6.30) for kharif, 2.13 (1.36) for Rabi, 1.11 (0.52) for winter, 6.25 (4.46) for pre-monsoon, 8.43 (6.19) for monsoon and 3.18 (1.55), respectively. The minimum and maximum values were observed for Kharif, season were 6.04 (2008) and 8.05 (2012) respectively, while the same for Rabi season 1.36 (1993, 1997) and 2.13 (2001) respectively.  Zhang et al. (2011) report “significant decreases in actual evapotranspiration in both ‘energy-limited’ Pearl River basin and ‘water-limited’ Yellow River basin of China. However, the middle and upper Yellow River basins witnessed RET increases and the Pearl River basin witnessed RET decreases”. Jhajharia et al. (2009) reported “decreasing Epan trends in humid environments of northeast India, mainly in the pre-monsoon and monsoon seasons”. Similarly, Zhang et al. (2014) identified “significant decreases in Epan in southern, central, southwestern, eastern and northwestern China. On the other hand, Epan increases were found to be spread over a few sites intermittently across China”.

Table 2: Seasonal Potential Evapotranspiration (mm/day) at Ambala during period 1985-2019

| Year | Annual | Kharif | Rabi | Winter | Premonsoon | Monsoon | Postmonsoon |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1985 | 4.47 | 7.36 | 1.58 | 0.74 | 5.98 | 7.30 | 1.68 |
| 1986 | 4.07 | 6.65 | 1.50 | 0.67 | 4.59 | 7.08 | 1.82 |
| 1987 | 4.55 | 7.42 | 1.68 | 0.82 | 4.46 | 8.37 | 2.02 |
| 1988 | 4.56 | 7.57 | 1.55 | 0.79 | 6.37 | 7.11 | 1.85 |
| 1989 | 4.11 | 6.66 | 1.56 | 0.63 | 4.98 | 6.82 | 1.93 |
| 1990 | 4.17 | 6.89 | 1.44 | 0.82 | 4.89 | 7.11 | 1.75 |
| 1991 | 4.19 | 6.94 | 1.44 | 0.71 | 5.04 | 7.18 | 1.67 |
| 1992 | 4.10 | 6.71 | 1.50 | 0.62 | 4.78 | 7.01 | 1.87 |
| 1993 | 4.25 | 7.14 | 1.36 | 0.81 | 5.13 | 7.28 | 1.61 |
| 1994 | 3.89 | 6.30 | 1.47 | 0.72 | 4.82 | 6.49 | 1.59 |
| 1995 | 4.22 | 6.95 | 1.48 | 0.52 | 4.72 | 7.30 | 2.07 |
| 1996 | 4.08 | 6.62 | 1.54 | 0.71 | 5.22 | 6.67 | 1.73 |
| 1997 | 3.80 | 6.24 | 1.36 | 0.66 | 4.35 | 6.65 | 1.55 |
| 1998 | 4.28 | 7.01 | 1.55 | 0.64 | 5.22 | 7.03 | 2.10 |
| 1999 | 4.32 | 7.02 | 1.62 | 0.75 | 5.95 | 6.69 | 1.89 |
| 2000 | 4.14 | 6.75 | 1.54 | 0.54 | 5.47 | 6.52 | 2.05 |
| 2001 | 4.62 | 7.12 | 2.13 | 0.55 | 4.98 | 7.48 | 3.18 |
| 2002 | 4.58 | 7.58 | 1.59 | 0.70 | 5.90 | 7.60 | 1.84 |
| 2003 | 4.21 | 6.99 | 1.43 | 0.60 | 5.26 | 7.09 | 1.72 |
| 2004 | 4.23 | 6.85 | 1.61 | 0.71 | 5.60 | 6.88 | 1.66 |
| 2005 | 4.37 | 7.31 | 1.43 | 0.59 | 5.25 | 7.57 | 1.74 |
| 2006 | 4.71 | 7.71 | 1.71 | 1.11 | 5.73 | 7.82 | 1.94 |
| 2007 | 4.28 | 7.15 | 1.41 | 0.69 | 5.46 | 7.15 | 1.67 |
| 2008 | 3.86 | 6.04 | 1.68 | 0.53 | 4.83 | 6.19 | 2.01 |
| 2009 | 4.27 | 6.97 | 1.56 | 0.82 | 5.14 | 7.24 | 1.72 |
| 2010 | 4.29 | 6.91 | 1.66 | 0.63 | 6.34 | 6.40 | 1.86 |
| 2011 | 4.31 | 6.94 | 1.69 | 0.67 | 5.44 | 6.98 | 2.06 |
| 2012 | 4.77 | 8.05 | 1.48 | 0.52 | 5.62 | 8.43 | 1.86 |
| 2013 | 4.46 | 7.33 | 1.59 | 0.53 | 5.81 | 7.22 | 2.07 |
| 2014 | 4.52 | 7.58 | 1.47 | 0.56 | 4.87 | 8.21 | 1.90 |
| 2015 | 4.39 | 7.14 | 1.65 | 0.74 | 5.20 | 7.31 | 2.13 |
| 2016 | 4.80 | 7.78 | 1.82 | 0.73 | 6.15 | 7.71 | 2.29 |
| 2017 | 4.77 | 7.83 | 1.72 | 0.83 | 6.25 | 7.66 | 2.07 |
| 2018 | 4.71 | 7.69 | 1.72 | 0.82 | 6.18 | 7.68 | 1.87 |
| 2019 | 4.44 | 7.41 | 1.48 | 0.70 | 5.29 | 7.60 | 1.89 |
| Normal | 4.34 | 7.10 | 1.57 | 0.69 | 5.35 | 7.22 | 1.90 |

Fig: 1. Temporal variation in PET at Ambala during different seasons since 1985-2019

The minimum and maximum monthly potential evapotranspiration at Ambala were found to be 0.22 and 12.10 mm in the months of January and June respectively. The Skewness and Kurtosis varied from -0.49 to 2.96 and -0.48 to 12.87 respectively. The mean, standard deviation (SD) and coefficient of variation (CV) varied from 0.43 to 9.02 mm, 0.11 to 1.15 mm and 8.83 to 25.19% respectively as showed in Table 3. Variation (CV) were higher in cold months i.e., December, January, February as compare to hot months.

Table 3 Descriptive Statistics for Monthly Potential Evapotranspiration at Ambala

| Month | Max | Min | Mean | SD | CV | Skewness | Kurtosis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Jan | 0.72 | 0.22 | 0.43 | 0.11 | 25.19 | 0.20 | 0.34 |
| Feb | 1.71 | 0.62 | 0.95 | 0.20 | 21.37 | 1.41 | 4.76 |
| Mar | 3.24 | 1.66 | 2.33 | 0.42 | 17.84 | 0.47 | -0.26 |
| Apr | 7.21 | 4.03 | 5.44 | 0.82 | 15.07 | 0.33 | -0.48 |
| May | 10.50 | 5.66 | 8.28 | 1.04 | 12.53 | -0.49 | 0.41 |
| Jun | 12.10 | 6.57 | 9.02 | 1.15 | 12.74 | 0.29 | 0.36 |
| Jul | 9.34 | 6.16 | 7.55 | 0.77 | 10.22 | 0.56 | -0.02 |
| Aug | 8.29 | 5.80 | 6.72 | 0.60 | 8.90 | 0.84 | 0.33 |
| Sep | 6.85 | 4.69 | 5.60 | 0.49 | 8.83 | 0.33 | 0.29 |
| Oct | 5.46 | 2.74 | 3.52 | 0.52 | 14.66 | 1.49 | 4.77 |
| Nov | 2.83 | 1.27 | 1.58 | 0.27 | 17.10 | 2.96 | 12.87 |
| Dec | 1.25 | 0.37 | 0.61 | 0.15 | 23.83 | 2.37 | 9.85 |

(Highest and lowest monthly values ever found during 1985-2019)

Ambala, as compared to all other station in this study, has shown a changing pattern of PET. Most of the months i.e., seven months have shown an increasing trend of PET (Table 4), wherein six months viz. April, May, July, September and November had a significant positive trend (0.1 level) and August and October had a highly significant trend (0.05 level of significance). As per Sen’s slope estimator, August and October months had increasing trend @ 0.017 & 0.018 mm/day, respectively. Besides, the monthly trend, the annual PET of Ambala had also shown a highly significant positive trend and found an increasing rate of 0.01 mm/day as shown in Table 4. PET of January and December found a decline.

Table 4 Trend analysis of Monthly Potential Evapotranspiration at Ambala during period 1985-2019

| Month | Mann-Kendall trend | Sen's slope estimate |
| --- | --- | --- |
|  | Test Z | Q |
| Jan | -1.704+ | -0.003 |
| Feb | 0.624 | 0.001 |
| Mar | 0.823 | 0.005 |
| Apr | 1.817+ | 0.027 |
| May | 1.817+ | 0.032 |
| Jun | 0.341 | 0.006 |
| Jul | 1.902+ | 0.022 |
| Aug | 2.215* | 0.016* |
| Sep | 1.761+ | 0.015 |
| Oct | 2.414* | 0.017* |
| Nov | 1.817+ | 0.006 |
| Dec | -1.476 | -0.002 |
| Annual | 3.124** | 0.011** |

+ = 0.1 level of significance, * = 0.05 level of significance, ** = 0.01 level of significance

The study showed a highly significant trend in PET observed in Kharif season and found increasing @ 0.02 mm/day, while Rabi season had an increasing trend at less significance (Table 5). In meteorological season pre-monsoon and post-monsoon seasons had highly significant increasing trends @ 0.03 mm/day and 0.01 mm/day, respectively, whereas the monsoon season had a less significant increasing trend. Contrary to all season, winter had a non-significant decreasing trend (Table 5).

Table 5: Trend analysis of Seasonal Potential Evapotranspiration at Ambala during period 1985-2019

| Season | Mann-Kendall trend | Sen's slope estimate |
| --- | --- | --- |
|  | Test Z | Q |
| Annual | 3.124** | 0.012** |
| Kharif | 2.698** | 0.019** |
| Rabi | 1.931+ | 0.005 |
| Winter | -0.738 | -0.001 |
| Pre-monsoon | 2.726** | 0.026** |
| Monsoon | 1.817+ | 0.016 |
| Post-monsoon | 2.073* | 0.006* |

+ = 0.1 level of significance, * = 0.05 level of significance, ** = 0.01 level of significance

Conclusions

Given the significance of potential evapotranspiration (PET) in water balance studies, irrigation planning, and reservoir operation, an effort has been made in this study to estimate PET over Ambala using the Penman–Monteith (PM) method. The trends in PET in Ambala's dry areas were examined in light of the changing climate after the PET was estimated using the PM method. Over Ambala, there were statistically significant declines in PET on both annual and seasonal time periods (kharif, rabi, winter, pre-monsoon, monsoon, and post-monsoon), as well as parallel declines in wind speed on both annual and seasonal time scales. The current study will be helpful in precisely estimating the primary crop water requirements in the arid Ambala region under changing climate conditions.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

Burn, D.H. and Hesch, N.M. (2007). Trends in evaporation for the Canadian Prairies. Journal of Hydrology 336, 61–73.

Choudhary, R.R., Jhajharia, D., Lal, M., Jain, J.K., Lunayach, A. and Choudhary, M.K. (2009). Climate and its variations over Bikaner since 1951–2008. Journal of Indian Geological Congr. 1(2): 79–86.

Dinpashoh, Y., Jhajharia, D., Fakheri-Fard, A., Singh, V.P. and Kahya, E. (2011). Trends in reference evapotranspiration over Iran. Journal of Hydrology 399: 422–433.

Doorenbos J, Pruitt WO. (1975). Guidelines for predicting crop water requirements. FAO Irrigation Drainage Paper No. 24, FAO: Rome; 168.

Doorenbos, J. and Pruitt, W.O. (1977). Crop water requirements. FAO Irrigation Drainage Paper No. 24, FAO: Rome; 144.

Golubev, V., Lawrimore, J.H., Groisman, P.Y., Speranskaya, N.A., Zhuravin, S.A. and Menne, M.J. (2001). Evaporation change over the contiguous United States and the former USSR: a reassessment. Geophysical research letters 28: 2665–2668.

Iyakaremye, V., Zeng, G., Yang, X., Zhang, G., Ullah, I., Gahigi, A., Vuguziga, F., Asfaw, T.G. and Ayugi, B. (2021). Increased high-temperature extremes and associated population exposure in Africa by the mid-21st century. Science of the Total Environment, 790, 148162.

Jhajharia, D., & Durbude, G. (2009). Assessing reference evapotranspiration by temperature-based methods for humid regions of Assam. differences, 94(38), 96-5.

Jhajharia, D., Dinpashoh, Y., Kahya, E., Singh, V.P. and Fakheri-Fard, A. (2012). Trends in reference evapotranspiration in the humid region of northeast India. Hydrological Processes 26: 421–435.

Kendall, M.G. (1975). Rank Correlation Methods (London: Charles Griffin) p 202.

Kothawale, D.R. and Rupa Kumar, K. (2002). Tropospheric temperature variations over India and links with the Indian summer monsoon: 1971–2000. Mausam 53(3): 289–308

Lawrimore, J. and Peterson, T. (2000). Pan evaporation in dry and humid regions of the United States. Journal of Hydrometeorology 1: 543–546.

Liu, Q., Yang, Z., Cui, B. and Sun, T. (2010). The temporal trends of reference evapotranspiration and its sensitivity to key meteorological variables in the Yellow River Basin, China. Hydrological Processes 24(15): 2171–2181.

Liu, C., Zhang, D., Liu, X. and Zhao, C. (2012). Spatial and temporal change in the potential evapotranspiration sensitivity to meteorological factors in China (1960–2007). Journal of Geosciences 22, 3–14

Luo, Y., Chang, X., Peng, S., Khan, S., Wang, W., Zheng, Q. and Cai, X. (2014). Short-term forecasting of daily reference evapotranspiration using the Hargreaves–Samani model and temperature forecasts. Agricultural Water Management, 136, 42–51.

Mann, H.B. (1945). Nonparametric tests against trend. Econometrica: Journal of the econometric society, 245-259.

Roderick, M.L. and Farquhar, G.D. (2004). Changes in Australian pan evaporation from 1970 to 2002. International Journal of Climatology, 24: 1077–1090.

Roy, D.K., Barzegar, R., Quilty, J. and Adamowski, J. (2020). Using ensembles of adaptive neuro-fuzzy inference system and optimization algorithms to predict reference evapotranspiration in subtropical climatic zones. Journal of Hydrology 591, 125509.

Tang, B., Tong, L., Kang, S. and Zhang, L. (2011). Impacts of climate variability on reference evapotranspiration over 58 years in the Haihe River basin of north China. Agricultural Water Management 98, 1660–1670.

Traore, S., Luo, Y. and Fipps, G. (2016). Deployment of artificial neural network for short-term forecasting of evapotranspiration using public weather forecast restricted messages. Agricultural Water Management 163, 363–379.

Ullah, I., Saleem, F., Iyakaremye, V., Yin, J., Ma, X., Syed, S., Hina, S., Asfaw, T.G. and Omer, A. (2022). Projected Changes in Socioeconomic Exposure to Heatwaves in South Asia Under Changing Climate. Earth’s Future 10, e2021EF002240.

Yan, S., Wu, L., Fan, J., Zhang, F., Zou, Y. and Wu, Y. (2021). A novel hybrid WOA-XGB model for estimating daily reference evapotranspiration using local and external meteorological data: Applications in arid and humid regions of China. Agricultural Water Management 244, 106594.

Yan, X. (2020). Mohammadian, A. Estimating future daily pan evaporation for Qatar using the Hargreaves model and statistically downscaled global climate model projections under RCP climate change scenarios. Arab. Journal of Geosciences, 13, 1–15.

Yin, J., Deng, Z., Ines, A.V., Wu, J. and Rasu, E. (2020). Forecast of short-term daily reference evapotranspiration under limited meteorological variables using a hybrid bi-directional long short-term memory model (Bi-LSTM). Agricultural Water Management 242, 106386.

Zhang, Q., Qi, T., Li, J., Singh, V.P. and Wang, Z. (2014). Spatiotemporal variations of pan evaporation in China during 1960-2005: changing patterns and causes. International Journal of Climatology, DOI: 10.1002/joc.4025.

Zhang, X., Ren, Y., Yin, Z.Y., Lin, Z. and Zheng, D. (2009). Spatial and temporal variation patterns of reference evapotranspiration across the Qinghai-Tibetan Plateau during 1971–2004. Journal of Geophysical Research: Atmospheres, 114: D15.

Zhang, Y., Liu, C., Tang, Y. and Yang, Y. (2007). Trends in pan evaporation and reference and actual evapotranspiration across the Tibetan Plateau. Journal of Geophysical Research: Atmospheres, 112: D12110, DOI: 10.1029/2006JD008161.

Stefanidis, S., & Alexandridis, V. (2021). Precipitation and potential evapotranspiration temporal variability and their relationship in two forest ecosystems in Greece. Hydrology, 8(4), 160.

Ahmadpour, S., Bayzidi, Y., & Trachte, K. (2025). Spatio-temporal patterns of evapotranspiration in the temperate Eastern German lowlands and its response to climate and land use change. Theoretical and Applied Climatology, 156(6), 307.

B. S., Thanushree, R. H. Patil, and K. G. Sumesh. 2024. “Projected Shifts in Potential Evapotranspiration under the Impact of Climate Change for India and Karnataka: A Jensen-Haise Approach”. International Journal of Environment and Climate Change 14 (12):332-41. https://doi.org/10.9734/ijecc/2024/v14i124629.

Jhajharia, D., Kumar, R., Dabral, P. P., Singh, V. P., Choudhary, R. R., & Dinpashoh, Y. (2014). Reference evapotranspiration under changing climate over the Thar Desert in India. Meteorological Applications, 22(3), 425-435.