Hybrid Approach for Fault Detection and Classification in Power Distribution Systems Using DWT and SVM with Real-Time Simulation

.

ABSTRACT

This study aims to develop a real-time model for detecting and classifying various fault types in power distribution networks to enhance reliability and operational efficiency. The proposed method addresses a range of fault scenarios, including single-phase-to-ground, line-to-line, double-line-to-ground, and three-phase faults, within a 33 kV distribution network. A hybrid approach combining Discrete Wavelet Transform (DWT) and Support Vector Machine (SVM) is introduced. Using the Debauchies-4 (Db4) wavelet, DWT effectively decomposes transient fault currents at the source terminal, capturing critical time-frequency domain features. Fault classification is performed using an SVM optimized with a Radial Basis Function (RBF) kernel and Particle Swarm Optimization (PSO), enabling precise mapping of data into higher-dimensional spaces for optimal separation. Validation conducted with MATLAB R2023b demonstrates a detection accuracy of 100% and a classification accuracy of 99%. Comparative analyses against models such as wavelet-based multilayer perceptron (WT-MLP), fuzzy logic, and discrete wavelet transform with artificial neural networks (DWT-ANN) on the IEEE 13-bus system highlight the proposed method's superior performance. This innovative approach proves to be robust, adaptable, and highly effective across diverse fault scenarios, offering significant improvements in accuracy and reliability for fault detection and classification in power distribution networks.

Keywords: [Fault detection, Power distribution systems, DWT, SVM.

1. INTRODUCTION

The electric power system is a vast and interconnected network that is particularly vulnerable to faults, especially within the distribution network, which experiences more frequent disruptions than the transmission and generation sectors. This vulnerability is exacerbated in regions like Nigeria, where maintenance is often overlooked (Oni et al., 2019). Faults in power systems can be either permanent or temporary, and their identification and classification are crucial before repairs can begin (Gonzalez et al., 2021). Temporary faults are sometimes cleared automatically through reclosing systems, but permanent faults require manual intervention. Accurate fault classification is essential for prompt maintenance, ensuring power restoration while improving reliability indices such as SAIDI, SAIFI, and CAIDI (Energy.gov, 2019). Short-circuit faults, one of the most common types, are caused by various factors such as lightning, severe weather, vegetation, and animal interference (Sarkar et al., 2020). Effective fault detection and classification are key to minimizing outages and enhancing the performance of protective systems, as noted by Kezunovic (2011). This paper focuses on improving fault detection and classification for the 33kV distribution lines from the Eket Transmission Injection Substation in Nigeria, with particular attention to the Etinan 33kV line, which serves rural areas and is more susceptible to faults due to its length and terrain.

The primary challenge in power distribution systems is delivering reliable service, which is often compromised by frequent faults caused by natural events, human activity, and environmental conditions (Akinola et al., 2021). These faults result in power outages, equipment damage, safety hazards, and reduced productivity (Muktar et al., 2019). Traditional methods of fault detection and classification, which rely on time-consuming manual inspections, are often inadequate for quick restoration. As such, there is an urgent need for a more efficient, accurate, and reliable method for detecting and classifying faults in the distribution network. This paper proposes a hybrid approach using Discrete Wavelet Transform (DWT) and Support Vector Machine (SVM) to address these challenges.

The primary objective of this paper is to develop a real-time model for detecting and classifying various fault types in power distribution networks. Specifically, the study aims to develop a DWT/SVM model for fault detection, a separate model for fault classification, and compare these models' performance with conventional methods. Additionally, the study will evaluate the accuracy and reliability of the DWT/SVM model using MATLAB simulations.

This study is significant due to the critical impact of faults on power reliability and customer satisfaction. In the case of the Etinan 33kV line, it can take days or even weeks for traditional troubleshooting methods to restore power after a fault. By introducing an advanced fault detection and classification system, this study aims to reduce downtime, improve repair efficiency, and enhance the overall reliability of the distribution network, thereby increasing customer satisfaction. Furthermore, the findings of this study will serve as a valuable resource for future research and development in the area of fault detection and power system optimization.

The scope of this study is limited to detecting and classifying short-circuit faults in power distribution lines using measurements of three-phase current during fault events. The classification algorithm will rely on data derived from DWT and SVM techniques. While fault location is typically a subsequent step after classification, this research will focus solely on detection and classification, leaving fault location for future studies. The types of faults under investigation include single-phase-to-ground, line-to-line, double-line-to-ground, and three-phase short-circuit faults.

2. material and methods

MATERIALS

The implementation of fault detection and classification using Discrete Wavelet Transform (DWT) and Support Vector Machine (SVM) required the following materials: Simulation Software including a 64-bit Windows 10 operating system, Microsoft Excel 2019, the Orange data mining platform, and MATLAB Simulink with relevant toolboxes like the Wavelet Toolbox for DWT, Statistics and Machine Learning Toolbox for SVM. Power System Model encompasses a distribution network model simulating various fault types (e.g., Single Line to Ground, Double Line to Ground) and tools within MATLAB Simulink to collect current and voltage data during faults. These fault data includes simulated current signals under different fault conditions and real-time data if available. For Data Preprocessing, DWT is used to extract key features, which serve as inputs for the SVM classification.

SVM classifiers were trained with the extracted features, and optimization hyperparameters. Hardware for real-time applications includes a Data Acquisition System for monitoring, Digital Relays for data collection, and high-performance computing resources,

METHOD

The methodology used in this study focused on the integration of Discrete Wavelet Transform (DWT) and Support Vector Machine (SVM) for fault detection (FD) and fault classification (FC) in power distribution networks. MATLAB R2023b/ Simulink was used to perform simulations on the network to generate dataset, fault and data analysis. Figure 2 and Figure 3 show the block diagram for DWT/SVM and MATLAB Simulink Simulation model for simple distribution network stimulated for dataset and generation of fault samples. The pre-processing phase involves feature extraction, which enhances the SVM's performance by reducing its input size while retaining all critical information from the current waveforms. Standard deviation (SD) is one of the key features used in this study to assess how data deviates from its mean, which helps in identifying significant patterns in the current signals. Current values from all three phases are sampled at a frequency of 20 kHz and recorded, both pre- and post-fault.

Figure: 1: Discrete wavelets transform decomposition tree

Sn

cG2

cG11

cH1

cH4

cG4

cH33

cG3

cH2

STEP 1

STEP 2

STEP 3

STEP 4

2.1 Discrete Wavelet Transform (DWT) Signal Decomposition

Fig 1 show the DWT decomposition tree employed to decompose the three-phase current signals into sets of coefficients that represent their time evolution within specific frequency bands. Compared to the Fourier transform, DWT is more effective in extracting transient signal information due to its ability to analyze both high- and low-frequency components over varying time windows (Meyer, 1993). In this study, the Daubechies (db4) wavelet was selected and used as the mother wavelet for its ability to extract primary properties of the input signal and eliminate harmonics (Daubechies, 1992). The decomposition process involves breaking down the signal into high-frequency (H1) and low-frequency (G1) elements, further decomposed to obtain higher resolution bands as shown in figure 1 This process continues until the optimal frequency band is reached, allowing for useful extraction.

2.1.1 Statistical Metrics for Fault Detection

Two key statistical metrics were employed to quantify the behavior of the signal during fault conditions: standard deviation and energy. Standard Deviation: The standard deviation (σ) of a signal measure its deviation from the mean. It is calculated as follows:

σ (t1 t2) = (t1t2xt-xdt)                                                                                                           (1)

Where t1 ,t2  is the time limit of the signal x(t) and x̄ is the means of the signal, calculated as:

x= 1t2- t1  t1t2xtdt							                          (2)

The mean x̄ represents the average value of the signal. Under normal operating conditions, x̄ is zero, but during faults, x̄ deviates from zero.

### Energy (E) of the signal x(t) over a time interval (t1, t2) is given by:

E = (t1 t2) = t1t2|xt|2dt	. The energy of a signal increases during fault conditions compared to its energy under normal conditions. This increase in energy is indicative of the higher signal amplitude associated with faults in a power.

Figure 2 Proposed Block Diagram for Fault Detection and classification

.

.

.

.

.

.

80

%

X

norm

20

%

T

rain data

Normalized

T

rain data

svm

classifier

Classifier

SVM Parameter

svm model for

fault detection and

classification

Fault Detection

Normalized

T

est Data

No fault

Faulty

Fault

Classification

R-G

Data

Generation

Per unit &

Current values

Distribution

Simulation

Y-G

B-G

R-Y

Y-B

R-B-G

R-B-G

B-R

B-R-G

R-Y-B

-

R-Y-B-G

T

est data

x - x min

Xmax

a

-Xmin

(3)

Figure 3.  MATLAB Simulink Simulation Model for Power Distribution Network.

Figure 4: Fault Detection and classification flow chart diagram

Yes

Start

IR, IY & IB

Signals

Introducing 3 Phase current signals

Signal decomposition using Db4, 4 levels

Feature extraction

Standard deviation

Feature selection

UsingRBF/PCA / PSO

Training of SVM classifier

Short circuit

Faults

No

Normal

Classify fault type

DWT

Analysis

Repeat the

Procedure

Detect fault

2.2 Fault Detection

The model was designed to detect faults in distribution networks and classify the specific fault type. Figure 4 show the flowchart of this process. DWT is applied to detect faults by analyzing the electrical signals for transient disturbances. When a fault occurs, it generates a sudden change in the current or voltage waveforms. DWT can effectively capture these transient disturbances due to its ability to localize both time and frequency components. The input data to the system includes three-phase current measurements during both fault and non-fault conditions. DWT-db4 wavelet, extracts key features from these measurements. The db4 wavelet was chosen for its regularity, smoothness, and compact support in time and frequency domains, making it ideal for fault detection. MATLAB R2023b is used to simulate current signals under various fault types (RG, YG, BG, etc.) and no-fault conditions. These simulations generate a dataset of post-fault current signals, which is then used to train the SVM classifier.

2.3 Fault Classification

The fault classification model aims to accurately identify different fault types using a DWT/SVM approach. The data samples include 1,200 fault samples and 1,000 non-fault samples, representing 11 fault types. The SVM classifier is trained to distinguish these faults using SD values extracted from DWT. Dimensionality reduction was done with the application of Principal Component Analysis features. This enhances model efficiency, reduces noise in the data and redundant dataset were filtered out.

2.3.1 SVM Classifier Design

SVM IN PHASE R,Y,B

SIGNAL

DISCOMPOSITION

USING DWT

CURRENT

MESUREMENT

FROM SENDING

END

OF THE

DISTRIBUTION

LINE

STATISTICAL FEATURE / SELECTION EXTRACTION

SVM-

G

SVM-

Y

SVM-

R

SVM-

B

SVM FOR GROUND

R,Y,B

FR

FY

FB

FG

Figure 5: Fault classification diagram using SVM classifiers.

Four Support Vector Machine (SVM) dyadic classifiers was employed to classify different fault types, as illustrated in Figure 5. Each classifier was trained with specific fault samples to identify faults in distinct phases: SVMR for phase R, SVMY for phase Y, SVMB for phase B, and SVMG for ground faults. The output from each SVM will be binary form, where a value of 1 indicates the presence of a fault in the respective phase or ground, while 0 signifies no fault detected. The SVM classifier uses a Radial Basis Function (RBF) kernel for handling nonlinear data in fault detection. A Particle Swarm Optimization (PSO) algorithm is employed to select the most relevant parameters to improve classification performance. The SVM classifiers are organized such that each classifier detects faults in a phase (R, Y, B) or ground faults (G). Each classifier outputs binary results, where 1 indicates a fault, and 0 indicates no fault. The hyperplane separating fault and non-fault data is optimized using quadratic programming methods.

2.3.2 PSO and Fault Detection Index

The process of PSO optimizes the SVM classifier by iteratively updating particle positions and velocities until an optimal solution is reached. An empirical index threshold of 0.03 is used to differentiate between ground faults and other types of faults. The fault index is calculated using the equation. Specific Current 𝐼𝑛𝑑𝑒𝑥 = (|Ia + Ib + Ic | )/Ia                         (4)

Where, 𝐼𝑎, 𝐼𝑏, and 𝐼𝑐 are the instantaneous values of the currents and it is use for fault detection. Classification accuracy (𝐶𝐴) is the key metric evaluate as:

𝐶𝐴 =  FCA/NoS                                                                                                                   (5)

Where, 𝐹𝐶A and NoS represent Fault Classification Accuracy and number of tested samples respectively.

3. results and discussion

3.1 Results

This chapter presents and analyzes the outcomes of the proposed fault detection and classification model. The model's performance was evaluated through simulations of various fault current signals using MATLAB R2023b. Figures 7.(a-d) showcase the simulated fault current signals for different fault scenarios.

3.1.1 Simulation of Fault Current Signals

The proposed model was tested under diverse fault conditions to assess its reliability and accuracy. The simulation results demonstrate the model's effectiveness in both fault detection and classification. Fault current signals generated for different scenarios are illustrated in Figures 5 (a-d).

##### Figure 6: Simulated Fault Current Signals Figure 6(a): SLG current fault signal  ,Figure 6(b): LL fault current signal, Figure 6(c): LLG fault current signal,Figure 6(d): LLL fault current signal

]

##### Figure 7 : Simulated Fault Current SignalsFigure 7(a): SLG current fault signal ,Figure 7(b): LL fault current signal, Figure 7(c): LLG fault current signal,Figure 7(d): LLL fault current signal

]

#### 3.1.2 Results Analysis

The proposed model successfully detects and classifies faults in the power distribution system. The model's performance was validated by comparing the simulation outcomes with expected results. Key findings include:

- Accuracy: The model achieved high accuracy in identifying and classifying various fault types.

- Speed: It showed fast detection and classification, critical for timely fault management.

- Robustness: The model performed well across different fault scenarios and conditions.

These simulations confirm that the fault detection and classification model, which integrates discrete wavelet transform (DWT) with support vector machine (SVM) classifiers, is highly effective for managing faults in power distribution systems. The model's high accuracy, speed, and robustness make it a valuable tool for improving system reliability and efficiency.

#### 3.1.3 Wavelet Decomposition

Wavelet decomposition was applied to phase current signals to extract useful features. The Daubechies-4 (db4) mother wavelet was selected due to its accuracy and efficiency in decomposing fault current signals.

- Approximation Coefficients: Represent low-frequency components, obtained by down-sampling signals filtered through low-frequency filters.

- Detail Coefficients: Represent high-frequency components, extracted by down-sampling signals filtered through high-frequency filters.

Figure 7 presents the level 4 detail coefficients for different fault types, such as Line-to-Ground (LG), Line-to-Line (LL), Double Line-to-Ground (LLG), and Three-Line (LLL) faults. These coefficients further validate the model’s effectiveness in capturing fault-specific characteristics

Figure 7: Level 4 Detail Coefficients for Various Fault Types SVM Classification Performance.

Current Signal, Detail Wavelet Coefficients (Level 1-4) and Approximation Coefficients

In addition, the classification performance of the Support Vector Machine (SVM) was optimized using standard deviation and median absolute deviation measures. Particle Swarm Optimization (PSO) was employed to determine the optimal feature parameters. The Radial Basis Function (RBF) kernel is employed to address nonlinearity in the dataset, particularly in scenarios involving high-dimensional input spaces. The RBF kernel is highly effective in handling non-linear datasets, a common challenge in fault detection tasks.  The dataset used for training and testing the SVM classifiers was split in an 80:20 ratio. As shown in Table 2, the model correctly classified 1185 out of 1200 test samples, resulting in an overall classification accuracy of 99%. Figure 7 provides a graphical representation of the classification results, showing consistent and accurate performance across all fault types.

Table 1:  SVM Fault Detection Matrix

| Fault Type |  | SVMR | SVMY | SVMB | SVMG | Accuracy (%) |
| --- | --- | --- | --- | --- | --- | --- |
| R-G |  | 1 | 0 | 0 | 1 | 100 |
| Y-G |  | 0 | 1 | 0 | 1 | 100 |
| B-G |  | 0 | 0 | 1 | 1 | 100 |
| R-Y |  | 1 | 1 | 0 | 0 | 100 |
| R-B |  | 1 | 0 | 1 | 0 | 100 |
| Y-B |  | 0 | 1 | 1 | 0 | 100 |
| R-Y-G |  | 1 | 1 | 0 | 1 | 100 |
| R-B-G |  | 1 | 0 | 1 | 1 | 100 |
| Y-B-G |  | 0 | 1 | 1 | 1 | 100 |
| R-Y-B |  | 1 | 1 | 1 | 0 | 100 |
| R-Y-B-G |  | 1 | 1 | 1 | 1 | 100 |
| No Fault |  | 0 | 0 | 0 | 0 | 100 |

Table 2:  SVM Classification Performance

| Fault Type | Tested Sample | Classified Correctly | Not Classified Correctly | Classification Accuracy (%) |
| --- | --- | --- | --- | --- |
| R-G | 100 | 100 | 0 | 100 |
| Y-G | 100 | 99 | 1 | 99 |
| B-G | 100 | 100 | 0 | 100 |
| R-Y | 100 | 100 | 0 | 100 |
| R-B | 100 | 99 | 1 | 99 |
| Y-B | 100 | 100 | 0 | 100 |
| R-Y-G | 100 | 98 | 2 | 100 |
| R-B-G | 100 | 99 | 1 | 99 |
| Y-B-G | 100 | 98 | 2 | 100 |
| R-Y-B | 100 | 100 | 0 | 100 |
| R-Y-B-G | 100 | 92 | 8 | 92 |
| No Fault | 100 | 100 | 0 | 100 |
| Total | 1200 | 1185 | 15 | 99 |

3.2 Discussion

The results of this study were compared with some good similar studies using different machine learning methods, such as Neural Networks (NN). For instance, Mahmud M.N. et al. (2015) achieved about 95% accuracy in fault detection and classification with a WT-MLP model. Another study by Majid J. et al. (2015) used DWT-Fuzzy Logic, obtaining a classification accuracy of around 95% despite variations in fault resistance and fault inception angle.

Additionally, a comparison was made with a DWT-ANN model by Naval K.S. et al. (2022), which achieved 99% accuracy but required large datasets for training, increasing computational time.

Table 3: Fault Detection and classification Results compared with other Models

| Model |  | Proposed DWT-SVM | ANN (WT-MLP) | DWT- FUZZY LOGIC | DWT-ANN |
| --- | --- | --- | --- | --- | --- |
| Detection (%) |  | 100 | above 95 | 95 | 99 |
| Classification (%) |  | 99 | 95 | 95 | 99 |
| No Fault Scenarios |  | 12 | 10 | 10 | 10 |

The superior performance of SVMs is attributed to their robust statistical foundation and ability to avoid overfitting. SVMs require fewer datasets for training and solve convex optimization problems, making them particularly suited for fault classification tasks.

3.3 Summary

The integration of Discrete Wavelet Transform (DWT) and Support Vector Machine (SVM) has proven to be an effective approach for fault detection and classification in power distribution networks. DWT's capability to decompose electrical signals into different frequency components enhances the extraction of transient fault features, while SVM provides robust classification with high accuracy. This synergy has been validated in numerous studies, demonstrating significant improvements in fault detection accuracy, reliability, and speed of response.

In real-world applications such as smart grids and high-voltage transmission lines, the DWT-SVM approach has shown remarkable success. The combination facilitates real-time, accurate fault detection essential for maintaining continuous power supply and optimizing energy distribution. The method’s robustness in handling complex and noisy data further enhances the stability and reliability of power networks.

While successful, challenges remain, including computational complexity, parameter selection, and handling noisy or incomplete data. However, advancements in feature extraction, improved SVM algorithms, and real-time fault detection systems are addressing these issues.

In this study, the proposed model detected and classified faults using half a cycle of post-fault signals. Key features of the model include:

4-level Daubechies-4 (db4) wavelet transform for feature extraction.

Dimension reduction using Principal Component Analysis (PCA), making large feature datasets simpler and more robust for analysis.

Radial Basis Function (RBF) kernel, allowing the SVM to separate classes in higher-dimensional spaces.

Particle Swarm Optimization (PSO) for parameter optimization, ensuring the selection of optimal classifier training parameters.

The model was tested using MATLAB R2023b simulations of both normal (no-fault) and 11 distinct fault types in a 33 kV distribution network. The three-phase current ratios served as input features for the SVM, which classified various fault types. This method's performance was compared with other machine learning algorithms, showing superior results.

4. Conclusion

The study concludes that the proposed model achieved 100% fault detection accuracy and 99% fault classification accuracy across 12 fault scenarios in test cases, demonstrating high performance and reliability. The model utilizes a lower sampling rate with current signals, simplifying its implementation while maintaining high stability across different fault types. Additionally, the model is rapid, reliable, and accurate, outperforming other models in similar studies and requiring fewer datasets for operation.

The following conclusions can be drawn:

1. The DWT-SVM method is highly suitable for fault detection and classification applications in power systems.

2. The system offers high accuracy, reliability, and robustness across various fault types and conditions.

3. The model's computational efficiency and reduced data requirements make it practical for real-time applications in smart grids and other power distribution systems.

4.1 Recommendations.

The following recommendations are proposed for future studies based on this study:

Utilize Raw Data from Injection Substations: To improve the real-world applicability of the model, it is recommended that raw data from actual injection substations be used in simulations. This would provide more accurate insights into the fault detection and classification processes, aligning the simulations with real-time operating conditions of power distribution lines.

Further study on Fault Location: Researchers are encouraged to extend the current DWT/SVM model to not only detect and classify faults but also to pinpoint their locations. Identifying the exact location of a fault is crucial for minimizing response times and improving power system reliability.

Test the Model in More Complex Power Distribution Topologies: The robustness of the proposed model should be tested on more complex and diverse power distribution topologies. Evaluating its performance under various network configurations and topological complexities will help assess its adaptability and effectiveness in different real-world scenarios.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References

Akinola, O. O., Salami, O. B., & Abolore, F. I. (2021). Assessment of power distribution system reliability in Nigeria. International Journal of Energy Economics and Policy, 11(1), 97-105.

Daubechies, I. (1992). Ten lectures on wavelets. SIAM.

Gonzalez, E., Garcia, A., & Garcia, S. (2021). A Review on Fault Detection and Diagnosis Methods for Electrical Systems. Energies, 14(6), 1581.

Kezunovic, M. (2011). Intelligent fault detection and classification in power systems. IEEE Power and Energy Society General Meeting, 2011, 1-8.

Majid, J., Fadhl, F., & Mahmood, F. (2015). DWT-Fuzzy Logic for fault detection in electrical systems. Energy Reports, 1, 1-8.

Meyer, Y. (1993). Wavelets: Algorithms and Applications. SIAM.

Mahmud, M. N., et al. (2015). WT-MLP for fault detection and classification. Energy Reports, 1, 1-8.

Muktar, M., Abdul Rahman, A. S., & Adamu, H. (2019). Power system reliability assessment: A review of existing methods and their applications. International Journal of Electrical Power & Energy Systems, 104, 500-511.

Naval, K. S., et al. (2022). DWT-ANN model for fault detection. Energies, 15(1), 1-15.

Oni, A. E., et al. (2019). An assessment of fault location techniques in power distribution systems. Journal of Electrical Engineering & Technology, 14(1), 215-225.

Sarkar, A., Saha, R., & Basak, B. (2020). Review on short-circuit fault detection in electrical networks. Journal of Electrical Engineering & Technology, 15(4), 1593-1602.

Energy.gov. (2019). Reliability Metrics. U.S. Department of Energy.Top of FormBottom of Form