Original Research Article

Modeling Nitrogen Dynamics in Maize under Drip Fertigation Using the HYDRUS Model: A Calibration and Validation Approach

ABSTRACT

Nutrient transport models simulate nutrient distribution in soil and help understand the relationship between nutrient application timing, crop uptake, yield, soil health, and groundwater pollution. Identifying a suitable model is crucial. A field experiment was conducted to model nitrogen dynamics using the HYDRUS-2D model in a split-plot design with three irrigation levels (0.6 ETc (I1), 0.8 ETc (I2), 1.0 ETc (I3)) and four nitrogen treatments (80% RDN (N1), 100% RDN (N2), 120% RDN (N3), and manual application (N4)) on sandy clay loam soil during kharif 2018, rabi 2018-19, and kharif 2019 at Dr N.T.R. College of Agricultural Engineering, Bapatla, Andhra Pradesh. The model was calibrated and validated for drip-fertigated maize. HYDRUS-2D uses Richard's equation for water flow and a non-equilibrium chemical transport model to simulate solute movement during transient flow in variably saturated porous media. The advection-dispersion equation was applied to simulate single non-reactive ion transport in a three-dimensional axi-symmetrical setup. Validation during kharif 2019 was done to simulate nitrogen concentration. Calibration results showed increased crop nitrogen uptake with higher N application from the development stage. Simulated and observed available nitrogen content up to 100 cm depth at 50 days after sowing (30 cm from emitter) showed good agreement, with R² = 0.981. Relative error ranged from -14.71% to -5.17%, averaging -8.62%. Root Mean Square Error (RMSE) was 0.103, and model efficiency was 0.98. Nitrogen availability was higher at 15 cm from the emitter. Therefore, HYDRUS can effectively simulate soil water and nutrient dynamics and is suitable for application in other crops and conditions.

INTRODUCTION

Conducting field experiments in huge number of soils with different fertigation levels for developing suitable design and management option is a costly and time-consuming affair. An accurately calibrated and validated nutrient transport model can downturn the time and cost needed for understanding the nutrient dynamics under drip fertigation system. Nutrient transport models could simulate these distributions in the soil. These models provide an understanding of the relationship between the amount and timing of nutrient application, the crop root uptake, yield and soil hazard and groundwater pollution (Antonopoules, 2001). However, identification of a suitable model is very important.

Among different available models, HYDRUS-2D has been extensively and successfully used to simulate nutrient transport in soils even for complicated problems. The HYDRUS model is a computer software package used to simulate the movement of water, heat, and solutes in variably saturated soils. It solves the Richards equation for water flow and advection–dispersion equations for solute transport, considering complex soil–plant–atmosphere interactions. Having both the flexibility to accommodate different types of boundary conditions for solute transport calculations and the capability to simultaneously consider root uptake of nutrients compared to other similar models (Li et al., 2015), the HYDRUS model can simulate solute dynamics in the fields under different solute management practices.

Hence, the present study has been planned at Dr NTR College of Agricultural Engineering, Bapatla to calibration and validation of hydrus-2d model for simulation of solute dynamics in drip fertigated maize crop in order to increase fertilizer use efficiency to avoid pollution of ground water in maize crop.

MATERIAL AND METHODS

LOCATION OF STUDY AREA

The field experiment was conducted during kharif 2018, rabi 2018-19 and kharif 2019 under maize crop at Dr. N. T. R. College of Agricultural Engineering, Bapatla, Guntur district of Andhra Pradesh State, India. Geographically the experimental site is located at latitude of 160 N and longitude of 880 E with an altitude of 6 m above mean sea level.

CLIMATE OF THE STUDY AREA

Bapatla is located in Southeastern part of Guntur district of Andhra Pradesh. It is very close to the coast and the town experiences hot summers and cool winters. The maximum temperature ranges between 400 C to 500 C in summer and the minimum temperature ranges between 180 C to 250 C in winter. The annual precipitation is about 700 - 1150 mm with an average of 940 mm of which 30.31 % is received during crop growing period of kharif 2018, 11% was received during crop growing period of rabi 2018-19. An amount of 48.02% is received during crop growing period of kharif 2019. The average relative humidity is 72.4 to 80.2 per cent and average wind speed is 72 to 132 km/day. Mean daily evaporation ranged between 2.4 to 4.0 mm/day during the crop growth period of the experiment. Hydrus-2D was calibrated for simulation of nitrogen distribution in the root zone of the maize crop for kharif 2018 with the observed nitrogen concentration in the field irrigated with 80% of crop water requirement and fertigated with 144.0 kg ha-1 of nitrogen.

BASIC PROPERTIES OF THE SOIL IN THE STUDY AREA

Soil samples were collected at every 15 cm layers from land surface till the soil depth of 90 cm using soil augur to characterize the soil. The collected soil samples were analysed in laboratory for determining the physical and chemical properties of the soil and are presented in Table 1 and 2.

Table 1 Physical properties of the experimental soil

| Soil depth from surface (cm) | Mineral content (% mass) |  |  | Textural class | Hydraulic conductivity (cm/h) | Bulk density (g/cm3) | Field capacity (% vol) | Permanent wilting point (% vol) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clay | Silt | Sand |  |  |  |  |  |
| 0-15 | 35 | 10 | 55 | Sandy clay loam | 0.94 | 1.37 | 21.48 | 6.73 |
| 15-30 | 35 | 10 | 55 | Sandy clay loam | 0.50 | 1.57 | 27.17 | 9.12 |
| 30-45 | 30 | 10 | 60 | Sandy clay | 0.46 | 1.53 | 28.24 | 10.56 |
| 45-60 | 35 | 5 | 60 | Sandy clay loam | 0.96 | 1.63 | 27.69 | 10.92 |
| 60-75 | 35 | 5 | 60 | Sandy clay loam | 0.96 | 1.63 | 27.73 | 11.61 |
| 75-90 | 30 | 5 | 65 | Sandy clay loam | 0.95 | 1.67 | 26.62 | 10.75 |

Table 2 Chemical properties of the experimental soil

| Soil depth from surface (cm) | pH | EC (ds m-1) | Organic carbon (%) | Available |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | N (kg ha-1) | P (kg ha-1) | K(kg ha-1) |
| 0-15 | 5.62 | 0.10 | 0.27 | 141.12 | 28.21 | 141.12 |
| 15-30 | 6.86 | 0.16 | 0.12 | 147.39 | 34.88 | 87.36 |
| 30-45 | 7.05 | 0.20 | 0.10 | 119.16 | 21.03 | 87.36 |
| 45-60 | 5.34 | 0.11 | 0.09 | 56.44 | 13.33 | 53.76 |
| 60-75 | 5.14 | 0.05 | 0.075 | 40.76 | 12.82 | 53.76 |
| 75-90 | 5.42 | 0.03 | 0.06 | 25.08 | 11.28 | 47.07 |

DESIGN AND LAYOUT OF EXPERIMENT

Field experiment was conducted with DEKALB DKC 8161 variety of hybrid maize under drip irrigation in split plot design consisting of three irrigation levels as main treatment and four fertigation levels as sub treatments with three replications during kharif 2018, rabi 2018-19 and kharif 2019 at the Field irrigation lab, Dr. NTR College of Agricultural Engineering, Bapatla. Maize seed was sown in a paired row system as plant to plant and row to row spacings were 20 cm and 40 cm, respectively [(80 cm + 40 cm) × 20 cm].

Treatments are as follows:

Main plots: Irrigation levels (I)

I1= 0.6 of the crop evapotranspiration

I2= 0.8 of the crop evapotranspiration

I3=1.0 of the crop evapotranspiration

Sub plots: Fertigation levels (N)

N1= Drip fertigation with 80% of recommended dose of nitrogen (CF)

N2= Drip fertigation with 100% of recommended dose of nitrogen (CF)

N3= Drip fertigation with 120% of recommended dose of nitrogen (CF)

N4= No drip fertigation (manual application) with 100% of recommended dose of        nitrogen (CF)

(CF= Conventional Fertilizers)

Description of Hydrus 2D

Graphical User Interface of HYDRUS 2D/3D, a software package for simulating water, heat, and solute movement in two- and three-dimensional variably saturated porous media. The software package includes the computational program and the interactive graphics-based user interface. The HYDRUS 2D/3D program numerically solves the Richards equation for variably saturated water flow and convection-dispersion equations for both heat and solute transport. The flow equation incorporates a sink term to account for water uptake by roots of plant. The solute transport equations consider advective-dispersive transport in the liquid phase, as well as diffusion in the gaseous phase. The transport equations also include provisions for nonlinear non equilibrium reactions between the solid and liquid phases, linear equilibrium reactions between the liquid and gaseous phases, zero-order production, and two first-order degradation reactions. In addition, physical non equilibrium solute transport can be accounted for by assuming a two-region, dual-porosity type formulation which partitions the liquid phase into mobile and immobile regions. Attachment/detachment theory, including filtration theory, is additionally included to enable simulations of the transport of viruses, colloids, and/or bacteria.

Governing Solute Transport Equation

For water flow Richard's equation was used in the Hydrus-2D. To simulate solute transport, Hydrus-2D uses two-dimensional non-equilibrium chemical transport of solutes involved in a sequential first order decay chain during transient water flow in a variably saturated rigid porous medium. For simulation of the transport of a single non reactive ion in homogeneous medium in three dimensional axi-symmetrical with polar coordinate system, advection-dispersion equation, Bear (1972) can be used

∂∂rθDrr∂C∂r+θDrz∂C∂z+1rθDrr∂C∂r+θDrz∂C∂z+∂∂zθDzz∂C∂z+θDrz∂C∂r-∂qrC∂r+qrCr+∂qzC∂z                                                                                   	 ------ (1)

Here

C = Solute concentration in the soil water, [ML−3]

q r and q z  = Components of the volumetric flux density, [LT-1]

D rr , D zz and D rz   = Components of the dispersion tensor. [L2T-1]

These components are given by (Bear 1972)

θDrr=εLqr2q+εTqz2q+θτDo 				------------ (2)

θDzz=εLqz2q+εTqr2q+θτDo 				------------ (3)

θDrz=εL-εTqrqzq                				 ------------ (4)

Where

|q| = Absolute value of the volumetric flux density, [LT−1]

ε L and ε T   = Longitudinal and transversal dispersivities, [L]

D o   = Molecular diffusion coefficient of the solute in free water, [L2T−1] and

τ = Tortuosity factor.

CALIBRATION AND VALIDATION OF HYDRUS-2D

Calibration is a process to establish the values of certain parameters. Model is executed by giving the nearest values of the parameters and predicted values are compared with observed values. Values of the said parameter are selected from the run when predicted and observed values are close enough. After calibration, model is validated with the seasonal observed data to examine its predictability.

Model was calibrated for hydraulic conductivity and dispersivity values for sandy clay loam soil. Based on the result of the kharif 2018, best treatment in terms of the yield and water use efficiency (I2N3) was selected to determine the spatial and temporal distribution of nitrogen at various growth stages. Calibration of the model was done for kharif 2018 and rabi 2018-19 using the values of nitrogen at various points, selected in the root zone with respect to the emitter. To validate the model, simulation was done for kharif 2019.

2.8 EVALUATION OF MODEL PERFORMANCE

Model performance was evaluated using dimensionless statistical performance criteria, viz., coefficient of determination (R2) and Root Mean Square Error (RMSE), Relative error (%) and Model efficiency.

2.8.1 Coefficient of Determination (R2)

R2=x-xy-y2x-xy-y2				                                   ---------- (5)

Where

x = Observed values

- = Mean of observed values

y = Simulated values

= Mean of simulated values

2.8.2 Root Mean Square Error (RMSE)

The model was evaluated by using root mean square error (RMSE) which quantified the differences between observed and simulated data.

RMSE=i=1NOi-PiN 			                                            ---------- (6)

Where

Oi = Observed value

Pi = Predicted value

N = Number of observations

2.8.3 Relative Error (%)

The relative error in percentage was calculated by using the following formula.

RE=|Pi-Oi|Oi×100 			                                 ---------- (7)

Where

Pi = ith simulated value

Oi= ith observed value

2.8.4 Model Efficiency

The measure of the deviation between model predictions and observed data is calculated by the following formula (Nash and Sutcliffe 1970).

EF=1-i=1nPi-Oi2i=1nOi-O2                                                                ---------- (8)

RESULTS AND DISCUSSIONS

Calibration of Hydrus 2D

Hydrus-2D was calibrated for simulation of nitrogen distribution in the root zone of the maize crop for kharif 2018 with the observed nitrogen concentration in the field irrigated with 80% of crop water requirement and fertigated with 144.0 kg ha-1 of nitrogen. The input parameters required by Hydrus-2D were determined by field experiment and data on few parameters were estimated from the model. The model was calibrated mainly for hydraulic conductivity and dispersivity values of the sandy clay loam soil. Model performed well with the measured hydraulic conductivity values. The initial value of longitudinal dispersivity was set to be one tenth of soil depth and the initial value of transverse dispersivity was set to be one tenth of longitudinal dispersivity. Calibrated values of longitudinal and transverse dispersivities for sandy clay loam soil were 0.5 and 0.05 cm. Results of the calibration for nitrogen content at 50 days after sowing is presented in Figures 1 (a) to 1 (b).

The crop N uptake increased in response to an increase in the application of N fertilizer from development stage. The simulated and observed nitrogen concentration content upto a depth of 60 cm in sandy clay loam soil at 50 days after sowing at emitter is shown in fig 1 (a). The nitrogen concentration decreased with depth and observed & simulated nitrogen concentration in the crop root zone exhibited similar trend. The scattered diagram (Fig. 1 (b)) has clearly indicated that there was good agreement between observed and simulated values with R2 as 0.963. The relative error varied from -20.979 to -8.69 with a average relative error of -14.62. The root mean square error is 0.158 and model efficiency is 0.957.

The simulated and observed nitrogen concentration upto a depth of 60 cm in sandy clay loam soil at 50 days after sowing and 15cm away from emitter is shown in fig 2 (a). The simulated nitrogen concentration is more than observed nitrogen content in the root zone but decreased with respect to depth. The model simulated very well the available nitrogen at 60 cm depth below soil surface. The scattered diagram (Fig. 2 (b)) has clearly indicated that there was good agreement between observed and simulated values with R2 as 0.985. Determination coefficient (R2) value explains that 98.5% of the variance was accounted. The relative error varied from -18.71 to -10.02 with a average relative error of -14.64. The root mean square error is 0.147 and model efficiency is 0.967.

The simulated and observed available nitrogen content upto a depth of 100 cm in sandy clay loam soil at 50 days after sowing at 30cm from emitter is shown in fig.3 (a). The nitrogen content decreased with depth and the model simulated well the available nitrogen particularly at 45 cm to 60 cm depth below soil surface. The scattered diagram (Fig. 3 (b)) has clearly indicated that there was good agreement between observed and simulated values with R2 as 0.981. The relative error varied from -14.71 to -5.17 with a average relative error of -8.62. The root mean square is 0.103 and model efficiency is 0.98. Nitrogen availability is more at 15 cm from emitter.

| Fig 1 (a) Simulated and observed nitrogen content at 50 days after sowing at emitter | Fig .2 (a) Simulated and observed nitrogen content at 50 days after sowing at 15 cm from emitter | Fig .3 (a) Simulated and observed nitrogen content at 50 days after sowing at 30 cm from emitter |
| --- | --- | --- |
| Fig .1 (b) Scattered diagram of nitrogen content at 50 days after sowing at emitter | Fig .2 (b) Scattered diagram of nitrogen content at 50 days after sowing at 15cm from emitter | Fig 3 (b) Scattered diagram of nitrogen content at 50 days after sowing at 30 cm from emitter |

4.8.2 Validation

To examine the performance of the model on seasonal basis, validation was carried out to simulate the nitrogen concentration during kharif 2019. The adjusted values of optimized parameters namely, saturated hydraulic conductivity, longitudinal and transverse dispersivities were used to validate the model using the kharif 2019 dataset. The results of simulation revealed that simulated and observed values of nitrogen concentration exhibited similar trend without much difference.

The simulated and observed nitrogen concentration up to a depth of 60 cm in sandy clay loam soil at 50 days after sowing at emitter is shown in fig 4 (a). The nitrogen concentration decreased with depth below root zone The scattered diagram (Fig..4 (b)) has clearly indicated that there was good agreement between observed and simulated values with R2 as 0.969. The relative error varied from -23.66 to -8.59 with a average relative error of -14.47. The root mean square error is 0.187 and model efficiency is 0.960.

Similarly, the simulated and observed nitrogen concentration upto a depth of 60 cm in sandy clay loam soil at 50 days after sowing at 15 cm from emitter is shown in fig.5 (a). The scattered diagram (Fig. 5 (b)) has clearly indicated that there was good agreement between observed and simulated values with R2 as 0.995. The relative error varied from -19.98 to -5.75 with a average relative error of -15.24. The root mean square is 0.158 and model efficiency is 0.94.

The simulated and observed nitrogen concentration upto a depth of 60 cm in sandy clay loam soil at 50 days after sowing at 30 cm from emitter is shown in fig 6 (a). The scattered diagram (Fig.6 (b)) has clearly indicated that there was good agreement between observed and simulated values with R2 as 0.999. The relative error varied from -16.22 to -11.58 with a average relative error of -13.27. The root mean square is 0.97 and model efficiency is 0.970.

| Fig .4 (a) Simulated and observed nitrogen content at 50 days after sowing at emitter | Fig .5 (a) Simulated and observed nitrogen content at 50 days after sowing at 15 cm from emitter | Fig 6 (a) Simulated and observed nitrogen content at 50 days after sowing at 30 cm from emitter |
| --- | --- | --- |
| Fig 4 (b) Scattered diagram of nitrogen content at 50 days after sowing at emitter | Fig .5 (b) Scattered diagram of nitrogen content at 50 days after sowing at 15cm from emitter | Fig .6 (b) Scattered diagram of nitrogen content at 50 days after sowing at 30 cm from emitter |

CONCLUSION

Finally, it can be concluded that there is a little over estimation of soil water content upto 40 cm depth below soil surface and then simulated very well upon 100 cm depth below root zone. Hence, Hydrus model can be applied to any crop to simulate soil water content in root zone.

References

- Antonopoulos, V.Z. 2001. Simulation of water and nitrogen balances of irrigation and fertilized Corn-crop soil. Journal of Irrigation and Drainage Engineering.  127 (2): 77-83.

- Li, X., Haibi. Shi, H., Simunek, J., Gong, X and Peng, Z. 2015. Modeling soil water dynamics in a drip-irrigated intercropping field under plastic mulch. Irrigation Science. 33: 289-302.

- Arbat, G. P., Lamm, F. R and Kheira, A.A.A. 2009. subsurface drip irrigation emitter spacing effects on soil water redistribution, corn yield, and water productivity. ASABE Annual Meeting as Paper No. 096578.

- Iqbal, M., Kamal, M.R., Soom, M.A.M., Yamin, M., Fazly, M., Hasfalina, C.M and Muhammed, H.H. 2020. HYDRUS-1D Simulation of Nitrogen Dynamics in Rainfed Sweet Corn Production. Applied Sciences. 10 (11): 3925-3940.

- Ranjbar, A., Rahimikhoob, A., Ebrahimian, H and Varavipour, M. 2019. Simulation of nitrogen uptake and distribution under furrows and ridges during the maize growth period using HYDRUS-2D. Journal of Irrigation Science. 37:            495–509.

- Karandish, F and Simunek, J. 2017. Two-dimensional modeling of nitrogen and water dynamics for various N-managed water-saving irrigation strategies using HYDRUS. Agricultural Water Management. 193: 174–190.

- Afua, A.M and Ramanathan, S.R. 2017. Hydrus (2D/3D) simulation of water flow through sandy loam soil under potato cultivation in southern Manitoba. Journal of Canadian Systems Engineering.  59(1):9-19.

- Baoli, X., Shao, D., Tan, X., Yang, X., Gu, W and Li, H. 2017. Evaluation of soil water percolation under different irrigation practices, antecedent moisture and ground water depths in paddy fields. Agricultural water management. 192: 149-158.

- Karandish, F and Simunek, J. 2017. Two-dimensional modeling of nitrogen and water dynamics for various N-managed water-saving irrigation strategies using HYDRUS. Agricultural Water Management. 193: 174–190.

- Mante, A.A and Ranjan, R.S. 2017. HYDRUS (2D/3D) simulation of water flow through sandy loam soil under potato cultivation in southern Manitoba. Journal of Canadian Biosystems Engineering. 59: 9-19.

- Mekala, C and Nambi, M.I. 2016. Experimental and simulation studies on nitrogen Dynamics in Unsaturated and saturated soil using HYDRUS-2D. Procedia Technology. 25: 122-129.

- Shahid, I., Andrey, K.G and Khan, H.Z. 2016. Estimating Nitrogen leaching losses after compost application in furrow irrigated soils of Pakistan Hydrus-2D software. Agricultural water management. 168: 85-95.

- Li, X., Haibi. Shi, H., Simunek, J., Gong, X and Peng, Z. 2015. Modeling soil water dynamics in a drip-irrigated intercropping field under plastic mulch. Irrigation Science. 33: 289-302.

- Li, Y., Simunek, J., Zhang, Z., Jing, L and Ni, L. 2015. Evaluation of nitrogen balance in a direct seeded rice field experiment using HYDRUS-1D. Agricultural Water Management. 148: 213-222.

- Tan, X., Shao, D., Gu, W and Liu, H. 2015. Field analysis of water and nitrogen fate in lowland paddy fields under different water managements using HYDRUS 1D. Agricultural Water Management. 150: 67-

- Wang, Z., Li, J and Li, Y. 2016. Assessing the effects of drip irrigation system uniformity and spatial variability in soil on nitrate leaching through simulation. American Society of Agricultural and Biological Engineers. 59 (1): 279-290.

- Wang, Z., Li, J and Li, Y. 2014. Simulation of nitrate leaching under varying drip system uniformities and precipitation patterns during the growing season of maize in the North China Plain. Agricultural Water Management. 142: 19–28.

- Wang, Z., Liu, F., Kang, S and Jensen, C.R. 2012. Alternate partial root-zone drying irrigation improves nitrogen nutrition in maize (Zea mays L.) leaves. Environmental and Experimental Botany. 75: 36–40.