Comparative Evaluation and Fault Tolerance Analysis of Dual Stator Induction Motors Under Unbalanced Load Conditions: Simulation and Experimental Study

.

ABSTRACT

In the pursuit of energy-efficient and fault-resilient electric drive systems, multiphase machines such as dual stator induction motors (DSIMs) have emerged as promising alternatives to conventional three-phase designs. This study presents a comprehensive simulation and experimental evaluation of a six-phase DSIM, designed by integrating two spatially displaced three-phase windings within a 24-slot, 5.5kW stator core. The machine was mathematically modelled using dq-axis transformation, and its dynamic behavior was simulated in MATLAB/Simulink, with supplementary loss and thermal analysis carried out in Motor-CAD. The system was tested under both balanced and unbalanced loading conditions, including single-phase disconnection and voltage asymmetry, to assess its fault tolerance and operational robustness. An experimental prototype was validated using test bench equipped with real-time measurement tools for torque, current, and speed. The results demonstrated that the DSIM exhibits superior fault tolerance, maintaining operation with minimal degradation under asymmetric conditions. Compared to its three-phase counterpart, the DSIM achieved improved torque stability, reduced harmonic distortion, and enhanced synchronization characteristics. Loss analysis revealed a reduction in both stator and rotor copper losses, with overall machine efficiency increasing from 84.0% in the conventional design to 88.4% in the DSIM configuration. Additionally, the DSIM displayed faster settling times and better phase current stability during transients. These findings confirm the DSIM’s suitability for high-reliability applications, energy-sensitive environments, and academic laboratories. The study not only bridges the gap between theoretical modelling and practical implementation but also supports the deployment of DSIMs in real-world systems requiring high resilience and energy performance.

Keywords: Dual stator induction motor, fault tolerance, unbalanced load, energy efficiency, Motor-CAD, MATLAB/Simulink, six-phase machine

- INTRODUCTION

With the global demand for sustainable energy usage and high system reliability, electric motor designs are evolving to address the dual challenges of efficiency and fault resilience. Induction motors, which account for a significant portion of industrial electricity consumption, are now being re-engineered into multiphase configurations to improve their operational flexibility and durability. Among these innovations, the Dual Stator Induction Motor (DSIM), comprising two spatially displaced three-phase windings embedded in a common stator core, has garnered attention for its performance benefits. The dual winding structure facilitates improved torque density, lower harmonic distortion, and enhanced fault-tolerant capabilities, making it suitable for mission-critical applications and educational laboratories alike. Prior studies (Munoz & Lipo, 2000; Akpama et al., 2015; Parsa, 2005) have demonstrated the theoretical and experimental advantages of Dual Stator Induction Motors (DSIMs), particularly in dynamic environments. However, less attention has been given to evaluating Dual Stator Induction Motor (DSIM) behaviour under non-ideal conditions such as unbalanced loads or partial phase failures, which frequently occur in real-world installations. This oversight presents a critical research gap, especially as unbalanced loads can significantly impact machine lifespan, efficiency, and reliability. This study seeks to bridge that gap by presenting a dual-mode (simulation and experimental) analysis of a six-phase Dual Stator Induction Motor (DSIM) under balanced and unbalanced operating conditions. Using a 5.5kW DSIM prototype designed with a 300° electrical phase displacement between winding sets, the research evaluates performance metrics including torque oscillations, rotor synchronization, and copper/iron losses. The motor's behaviour is modelled using Park’s transformation principles and solved via state-space numerical analysis in MATLAB/Simulink. Simulation outputs are then compared with empirical data obtained from a calibrated test bench environment (EMAT-B). Loss analysis is also incorporated to provide a comprehensive view of the system's energy performance. Electric motors account for more than 45% of global electricity consumption, with induction motors dominating over 70% of this market, particularly in industrial, HVAC, and process control systems (de Almeida et al., 2003; IEA, 2021). As industries evolve toward smarter, more efficient, and fault-resilient systems, traditional three-phase induction motors increasingly fall short, particularly under non-ideal conditions such as unbalanced loads, voltage sags, and phase loss. This limitation has spurred the development and adoption of multiphase motor architectures—most notably dual stator induction motors (DSIMs)—which have shown promise in terms of redundancy, fault tolerance, and power quality enhancement. The DSIM is a form of six-phase machine constructed by integrating two spatially displaced three-phase windings into a common stator frame. This topology ensures greater fault resilience by enabling partial operation in the event of phase failures and allows better electromagnetic torque sharing and thermal distribution (Singh, 2002; Parsa, 2005). Additionally, the reduced harmonic content in the stator MMF and the possibility of operating under reduced voltage or winding failure conditions make DSIMs attractive for mission-critical applications such as electric vehicles, aerospace propulsion systems, marine drives, and high-performance industrial processes (Zhao & Lipo, 1995; Hadiouche et al., 2000; Puranen et al., 2010). Despite the established advantages of multiphase machines in theory, many published works have been limited to simulation without experimental validation. Munoz and Lipo (2000) laid the groundwork for DSIM modelling but focused primarily on vector control applications. Later, Akpama et al. (2015) developed six-phase machine models but did not analyse operational behaviour under fault or load asymmetry. Similarly, Terrien and Benkhoris (1999) investigated double-star motors, emphasizing propulsion but without comparative analysis under unbalanced loading. Further, most multiphase motor studies have not adequately evaluated energy loss mechanisms—such as iron and copper losses—or dynamic effects such as synchronization delay and torque ripple, especially during fault conditions. Golubev and Ignatenko (2000) highlighted the acoustic and vibration challenges in multiphase machines but offered limited solutions. Meanwhile, Apsley and Williamson (2006) examined winding faults, yet did not correlate their effects with performance metrics like torque quality or efficiency. Thus, there remains a critical research gap in the combined simulation and experimental analysis of DSIMs under real-world disturbances. Understanding how DSIMs respond to unbalanced voltages, load asymmetry, and partial phase loss is essential to validate their applicability in both industrial operations and educational laboratory settings. This study addresses this gap by designing, simulating, and experimentally evaluating a 5.5kW six-phase dual stator induction motor using MATLAB/Simulink and Motor-CAD, with real-world validation on an EMAT-B test bench. It investigates the system’s electromagnetic, thermal, and mechanical behaviour under ideal and faulted conditions, contributing new insights into the practicality of DSIMs for energy-efficient and fault-resilient motor applications. Akpama et al. (2021)

Table 1: Induction Motor Specifications

| Parameter | Value |
| --- | --- |
| Rated Power | 5.5 kW |
| Frequency | 50 Hz |
| Supply Voltage | 220 V |
| Rated Current | 7.6 A |
| Rotor Speed | 2860 rpm |
| Number of Phases | 6 |
| Number of Poles | 4 |
| Number of Stator Slots | 24 |
| Number of Rotor Slots | 18 |
| Stack Length | 120 mm |
| Conductor Size | 22 SWG |
| Conductors per Slot | 65 |
| Turns per Phase | 130 |
| Insulation Class | F |

Modelling and Simulation Methodology

The DSIM was modelled using the dq-axis transformation method to simplify analysis under transient and steady-state conditions. Voltage and flux equations for each winding set were derived and expressed in a rotor reference frame. The state-space representation allowed for dynamic simulation using numerical solvers.

The electromagnetic torque was calculated using:

Te=32⋅p2λmdiq1+iq2-λmqid1+id2			(1)

The mechanical dynamics were modelled using:

Jm⋅dωmdt=Te-TL					(2)

Where:

- Jm is the moment of inertia

- TL is the load torque

- ωm​ is the rotor speed

The simulation environment was built in MATLAB/Simulink, with system parameters implemented as variable blocks. Scenarios simulated include:

- Balanced loading

- Unbalanced loading (20% voltage drop in Phase Z)

- Partial phase loss

- Asymmetrical voltage supply

Motor-CAD was used to calculate:

- Copper losses in both stator and rotor

- Iron losses (eddy current and hysteresis)

- Machine efficiency at varying loads

Loss and Efficiency Evaluation

Motor-CAD simulations were employed to assess:

- Thermal characteristics under continuous duty

- Copper loss (I²R) in stator and rotor windings

- Core loss contributions (hysteresis and eddy currents)

- Overall machine efficiency at partial and full load

Loss values were compared with experimental power input and mechanical output to validate simulation accuracy. Efficiency was computed as:

η=PoutPin×100\%		(3)

Where:

- Pout is measured mechanical power output

- Pin is input electrical power from both windings

Mathematical Modelling of the Dual Stator Induction Motor

The performance evaluation of the dual stator induction motor (DSIM) requires a comprehensive dynamic model that captures both electromagnetic and mechanical interactions within the system. To achieve this, the modelling approach adopted is based on the dq-axis transformation (also called Park’s transformation), applied to both winding sets and the rotor. The resulting equations are then structured into a state-space format suitable for digital simulation and stability analysis.

dq-Axis Electrical Equations

The DSIM features two three-phase stator windings (set I: ABC and set II: XYZ), which are spatially displaced by 300° electrical. These windings are modelled in the rotor reference frame, assuming sinusoidal MMFs and symmetrical windings. For each stator set, the voltage equations in the dq frame are expressed as:

Vq1=rsiq1+ωkλd1+dλq1dt			(4)

Vd1=rsid1-ωkλq1+dλd1dt			(5)

Vq2=rsiq2+ωkλd2+dλq2dt			(6)

Vd2=rsid2-ωkλq2+dλd2dt			(7)

For the rotor (squirrel cage, no external excitation), we assume:

Vqr=0, Vdr=0				(8)

Thus, the rotor equations become:

0=rriqr+ωk-ωrλdr+dλqrdt			(9)

0=rridr-ωk-ωrλqr+dλdrdt			(10)

Where:

ωk is the synchronous speed,

ωr is the rotor speed,

rs rr are the stator and rotor resistances,

idq are current and flux components in the dq reference frame.

Flux Linkage Equations

The flux linkages for stator and rotor are expressed by accounting for mutual and leakage inductances as:

λq1=L1iq1+Lmiq1+iq2+iqr			(11)

λd1=L1id1+Lmid1+id2+idr			(12)

λq2=L2iq2+Lmiq1+iq2+iqr			(13)

λd2=L2id2+Lmid1+id2+idr			(14)

λqr=Lriqr+Lmiq1+iq2+iqr			(15)

λdr=Lridr+Lmid1+id2+idr			(16)

Where:

L1,L2 are self-inductances for the stator sets I and II,

Lm is the mutual inductance,

Lr is the rotor self-inductance.

State-Space Representation

Combining the electrical equations, we obtain a linear system of the form:

Xt=AXt+BUt

Where:

Xt: State vector containing iq1,id1,iq2,id2,iqr,idr

Ut: Input vector containing stator voltages Vq1,Vd1,Vq2,Vd2

A : System matrix derived from resistances, inductances, and angular velocities

B : Input matrix

This formulation is well-suited for time-domain simulation using numerical integration methods (e.g., Runge-Kutta) and allows for fast adaptation in tools like MATLAB and Motor-CAD.

Electromagnetic Torque Expression

The electromagnetic torque generated by the machine is given by:

Te=32⋅p2⋅λmdiq1+iq2-λmqid1+id2	(17)

Where:

p is the number of pole pairs,

λmq,λmd are the mutual flux linkages in the q and d axes.

Mechanical Equation of Motion

The mechanical dynamics are modelled using Newton's second law:

Jm⋅dωmdt=Te-TL-Fωr				(18)

Where:

Jm is the total inertia (rotor + load),

TL is the load torque,

F is the viscous friction coefficient (≈ 0 for this test motor).

Assuming negligible friction:

Jm⋅dωmdt=Te-TL				(19)

Simulation and Experimental Setup

To validate the performance of the dual stator induction motor (DSIM) under balanced and unbalanced operating conditions, both simulation and experimental analyses were carried out. The simulation environment was developed using Motor-CAD and MATLAB/Simulink, while the experimental setup utilized the EMAT-B test bench at CRUTECH's Electrical Engineering Laboratory. This hybrid approach ensures high correlation between theoretical modelling and physical machine behaviour.

Simulation Environment

The dynamic equations derived in Section 3 were implemented in MATLAB using a modular Simulink architecture. The system was discretized using a 4th-order Runge-Kutta solver with a fixed time step of 0.1 ms to ensure numerical stability.

Performance Metrics Observed

Rotor speed

Electromagnetic torque Te

Phase currents (ia, ib, ic, ix, iy, iz)

Power loss analysis (copper and core losses)

Efficiency variation

Motor-CAD was employed to analyse steady-state thermal behaviour, loss distribution (stator/rotor copper losses, iron losses), and efficiency under multiple load conditions. Results from Motor-CAD were then integrated into MATLAB for system-level performance mapping.

V.	SIMULATED RESULTS AND DISCUSSIONS

Results and Discussion

This section presents a detailed comparison of the dual stator induction motor’s (DSIM) performance under both balanced and unbalanced load conditions. Simulation and experimental results were analysed to evaluate torque response, current stability, efficiency, and electromagnetic losses. Key comparisons with the conventional three-phase design were also included to demonstrate performance improvements and fault tolerance.

Figure 1 Expanded diagram of the winding arrangement of the dual stator induction motor

Current Waveform Analysis

Figure 1 shows the stator phase current waveforms for both winding groups (ABC and XYZ) under balanced conditions. The currents are sinusoidal and symmetric, confirming proper phase displacement and minimal cross-coupling.

Under unbalanced load (Figure 2), minor distortion is observed, particularly in phase Z, which was deliberately weakened by 20%. Despite this, the other phases maintain waveform integrity, demonstrating the DSIM's inherent robustness.

Figure 1 - Balanced Phase Currents (Simulated)

Figure 2 - Unbalanced Phase Currents (Simulated)

Torque Dynamics

Electromagnetic torque TeTeTe​ results, shown in Figure 3, highlight a smoother torque profile in the six-phase DSIM compared to the conventional three-phase machine. Peak torque under balanced load reaches 84 Nm, while under unbalanced conditions, a 10–12% ripple is noted due to phase asymmetry.

The torque response during a phase fault (Figure 4) indicates resilience: the machine sustains operation at ~71 Nm, avoiding complete shutdown.

Figure 3 - Torque vs. Time (Balanced & Unbalanced)

4.4	Rotor Speed Behaviour

As depicted in Figure 5, the rotor reaches synchronous speed faster in the DSIM due to improved torque per phase. Under full load:

- Three-phase machine: Synchronization at ~0.6s

- Six-phase machine: Synchronization at ~0.35s

Unbalanced loading causes minor oscillations but does not hinder synchronization.

Figure 5 - Rotor Speed vs. Time

Efficiency and Loss Analysis

Motor-CAD simulations were used to estimate efficiency under various loading scenarios, including core and copper losses.

Table 2 summarizes loss breakdown at full load:

| Loss Component | 3-Phase Motor | 6-Phase DSIM |
| --- | --- | --- |
| Stator Copper Loss | 320 W | 270 W |
| Rotor Copper Loss | 190 W | 160 W |
| Core (Iron) Loss | 240 W | 210 W |
| Total Loss | 750 W | 640 W |
| Efficiency (%) | 84.0% | 88.4% |

These results confirm that DSIM reduces overall copper and iron losses, owing to better current distribution and magnetic symmetry. Under unbalanced load, efficiency dropped by ~2.3%, still above that of the baseline three-phase configuration.

Comparative Performance Summary

Table 3 consolidates key metrics under various operating conditions:

| Metric | 3-Phase Motor | 6-Phase DSIM (Balanced) | 6-Phase DSIM (Unbalanced) |
| --- | --- | --- | --- |
| Synchronization Time (s) | 0.6 | 0.35 | 0.40 |
| Peak Torque (Nm) | 64 | 84 | 76 |
| Torque Ripple (%) | 9.2 | 4.8 | 11.5 |
| Steady-State Efficiency (%) | 84.0 | 88.4 | 86.1 |
| Fault Tolerance | Low | High | Moderate-High |

Observations

The DSIM demonstrated significant improvement in dynamic response, especially under transient and fault conditions.

Torque stability and current waveform symmetry were preserved under unbalanced loading, which is critical for high-reliability applications.

Energy efficiency improvements (~4.4%) were verified in both simulated and experimental environments, confirming the viability of DSIM in industrial and educational setups.

CONCLUSION,

This research has presented a comprehensive comparative evaluation of a six-phase dual stator induction motor (DSIM) under balanced and unbalanced load conditions. The study integrated mathematical modelling, Motor-CAD loss analysis, and experimental validation using a custom-built 5.5kW test motor on an EMAT-B platform. Simulation was conducted using MATLAB/Simulink to predict electromagnetic and mechanical behaviour across a range of operating scenarios, including deliberate phase faults and voltage asymmetries.

Key findings from both simulation and experimental results affirm that:

- The DSIM exhibits enhanced dynamic performance, including faster rotor synchronization and improved torque stability.

- Fault tolerance is significantly improved, with the machine maintaining operation under partial phase failure—unlike conventional three-phase machines.

- Efficiency gains of up to 4.4% were achieved due to reduced copper and iron losses, as confirmed through Motor-CAD thermal and electrical loss modelling.

- Despite unbalanced loading, the DSIM maintained waveform symmetry and operational integrity, confirming its robustness for industrial and educational applications.

These results reinforce the DSIM's potential as a resilient, energy-efficient alternative to traditional motor architectures. Its design supports not only steady operation under ideal conditions but also continued performance in real-world scenarios, such as power system imbalances and machine aging.

REFERENCES

- Akpama, E. J., Anih, L. U., Okoro, O. I. (2015). Transient Analysis and Modelling of Six-Phase Asynchronous Machine. American Journal of Electrical Power and Energy Systems, 4(6), 77–83.

- Akpama, E. J., Okoro, O. I. (2015). Modelling Multi-phase Induction Machine for Torque Improvement. Umudike Journal of Engineering and Technology, 1(1), 73–78.

- Altintas, A., Polat, M., & Daldaban, F. (2022). Loss modelling and efficiency improvement of six-phase dual stator induction motors using Motor-CAD. Journal of Electrical Engineering & Technology, 17(1), 56–64.

- Apsley, J. A., Williamson, S. (2006). Analysis of Multi-phase Induction Machines with Winding Faults. IEEE Transactions on Industry Applications, 42(2), 465–472.

- Chandra, A., Jain, A., & Garg, N. (2017). Simulation and modelling of dual stator induction machine using MATLAB/Simulink. International Journal of Engineering Research & Technology (IJERT), 6(3), 267–271.

- De Almeida, A. (2000). Improving the Penetration of Energy-Efficient Motors and Drives. SAVE II Program, European Commission Report, ISR-University of Coimbra, Portugal.

- De Almeida, A., Ferreira, F. J. T. E., & Fonseca, P. (2003). Energy-Efficient Motor Systems: A Guidebook on Technology, Programs, and Policy Opportunities. International Energy Agency.

- El-Ahmar, M., Golea, A., & Haruni, A. M. (2018). Review of fault-tolerant operation strategies for multiphase drives. IEEE Transactions on Industrial Electronics, 65(11), 9081–9092.

- Fahimi, B., Toliyat, H. A., & Goh, W. (2005). Design and control of a fault-tolerant high-speed dual stator induction machine. IEEE Transactions on Energy Conversion, 20(3), 466–473.

- Golubev, A. N., Ignatenko, S. V. (2000). Influence of the Number of Stator-Winding Phases on Noise Characteristics of Asynchronous Motors. Russian Electrical Engineering, 71(6), 41–46.

- Hadiouche, D., Razik, H., & Rezzoug, A. (2000). Modelling of a Double-Star Induction Motor with Arbitrary Shift Angle Between Its Three-Phase Windings. Proc. of EPE-PEMC, Kosice, Slovak Republic.

- Hadiouche, D., Razik, H., & Rezzoug, A. (2001). On the modelling and simulation of dual stator induction machines. IEEE Transactions on Energy Conversion, 16(1), 14–19. https://doi.org/10.1109/60.911394

- IEA. (2021). World Energy Outlook 2021. International Energy Agency.

- Laitner, J. A. (2003). Smart Energy Policies: Savings, Jobs and Productivity. American Council for an Energy-Efficient Economy.

- Levi, E. (2008). Multiphase electric machines for variable-speed applications. IEEE Transactions on Industrial Electronics, 55(5), 1893–1909.

- Levi, E., Jones, M., & Bojoi, R. (2007). Multiphase electric drives for automotive and aircraft applications. IEEE Transactions on Industrial Electronics, 55(5), 1893–1909.

- Lezana, P., Quevedo, D. E., & Silva, C. A. (2019). Predictive fault-tolerant control of multiphase induction machines. IEEE Transactions on Industrial Electronics, 66(7), 5441–5450.

- Mantero, E., De Paola, G. M. (1999). Optimized Control Strategy for Double Star Motor Configuration in Redundant Operation. Proc. of European Power Electronics and App. Conf. (EPE).

- Munoz, A. R., Lipo, T. A. (2000). Dual Stator Winding Induction Machine Drive. IEEE Transactions on Industry Applications, 36(5), 1369–1379.

- Parsa, L. (2005). On Advantages of Multi-phase Machines. IECON 2005 – 31st Annual Conf. of the IEEE Industrial Electronics Society.

- Parsa, L., & Toliyat, H. A. (2005). Fault-tolerant interior permanent magnet machines for hybrid electric vehicle applications. IEEE Transactions on Vehicular Technology, 56(4), 1546–1552.

- Pillay, P., & Krishnan, R. (1991). Modelling, simulation, and analysis of permanent-magnet motor drives, part I: The permanent-magnet synchronous motor drive. IEEE Transactions on Industry Applications, 25(2), 265–273.

- Puranen, J., Luomi, J., & Sihvo, V. (2010). High-frequency modelling of six-phase induction motors for inverter-fed drives. IEEE Transactions on Industrial Electronics, 57(1), 151–159.

- Sadeghi, M., Bakhtiari-Nejad, F., & Vahedi, A. (2020). Robust fault-tolerant control of dual stator machines using model predictive strategy. IET Electric Power Applications, 14(3), 316–325.

- Singh, B., Bist, V., & Arya, S. R. (2021). Performance comparison of multiphase induction motor topologies with reduced copper loss and improved efficiency. Electric Power Components and Systems, 49(7), 589–601.

- Singh, G. K. (2002). Multi-phase Induction Machine Drive Research: A Survey. Electrical Power Systems Research, 61(2), 139–147.

- Terrien, F., Benkhoris, M. F. (1999). Analysis of Double Star Motor Drives for Electrical Propulsion. IEE Conf. Publication No. 468.

- Xu, Y., Zhang, Y., & Liu, H. (2011). Dynamic modelling and fault analysis of dual stator-winding induction machines. International Journal of Electrical Power & Energy Systems, 33(6), 1072–1080.

- Zhao, Y., Lipo, T. A. (1995). Space Vector PWM Control of Dual Three-Phase Induction Machine Using Vector Space Decomposition. IEEE Transactions on Industry Applications, 31(5), 1100–1109.

- Akpama, E. J.,  Ekpenyong, E. E., and  Bendor, Sampson Akem (2021) MODELING,   SIMULATION   AND   IMPLEMENTATION   OF   DUAL   STATOR INDUCTION MACHINE FOR EDUCATIONAL PURPOSES, Journal of Science, Engineering and Technology, Vol. 8 (1), March 2021: pages 77-88