PERFORMANCE EVALUATION OF DRIVEN PILES IN LAGOS NIGERIA

ABSTRACT: Driven pile foundations are essential for supporting structures in regions with weak and compressible soils, particularly in rapidly developing coastal cities like Lagos, Nigeria. The city's fast-paced urbanization and increasing infrastructural demand have made deep foundation systems indispensable in overcoming the geotechnical challenges posed by variable soil conditions. These challenges include high groundwater levels, compressible clays, and spatially inconsistent subsoil profiles, which can lead to excessive settlement and structural instability if not adequately addressed. This study presents a comprehensive performance evaluation of driven piles installed in Ilubirin, Lagos, based entirely on full-scale field testing. A total of 30 precast reinforced concrete piles, each measuring 360 mm × 360 mm in cross-section and driven to depths between 31.0 m and 36.5 m, were subjected to static axial load tests using the Kentledge method. The safe working load for the piles was 700 kN; however, test loads were applied up to 3150 kN. Settlements under these loads ranged from 8 mm to 19 mm, while post-loading recovery (rebound) was minimal, between 0.00 mm and 0.13 mm. The results showed that all tested piles performed well under the applied loads, exhibiting stable load-settlement behavior with no signs of plunging failure. This confirmed effective load transfer to the subsoil, primarily through end-bearing resistance mobilized at the dense sand strata. The consistency in performance across all piles indicates uniform subsoil conditions and demonstrates the reliability of driven piles in Lagos's coastal environment. The study also highlights that most piles carried loads well above their safe working capacity without exceeding allowable settlement limits, suggesting a conservative margin in current design practice. Overall, the findings validate the use of driven precast piles in complex soil environments like Lagos and emphasize the importance of field verification through load testing. These insights are critical for geotechnical engineers and construction professionals working in coastal and estuarine regions where soft subsoils prevail, offering guidance for safe and optimized deep foundation design.

Keywords: Driven piles, static load test, foundation performance, settlement behavior, coastal soils, Lagos subsoil, end-bearing capacity, pile-soil interaction.

# INTRODUCTION

Driven pile foundations have long been recognized as a fundamental solution for transmitting structural loads to deeper, more stable strata in areas with weak or problematic surface soils. These piles, which are pre-manufactured and installed by mechanical driving, are widely utilized in both onshore and offshore projects due to their high load-bearing capacity, rapid installation, and effectiveness across diverse geotechnical conditions (Meyerhof, 1976; Randolph & Wroth, 1979). In the Nigerian context, particularly in coastal urban centres like Lagos, the use of driven piles has become increasingly prevalent. This trend is fuelled by rapid urban expansion, rising demand for high-rise developments, and the need to overcome the challenges posed by difficult subsoil conditions (Adeleye et al., 2014). Lagos, located in the southwestern part of Nigeria along the Atlantic coast, is characterized by complex subsurface geology, a high groundwater table, and highly compressible clayey deposits (Bolarinwa & Ola, 2016). Much of the city is built on young alluvial and lagoonal soils, which tend to be heterogeneous and often exhibit poor engineering properties. These include low bearing capacity, high moisture content, and considerable settlement potential. Such unfavourable conditions make shallow foundations unsuitable for most medium- to high-rise structures, increasing the reliance on deep foundations to ensure structural safety and serviceability.

Driven piles, when properly installed, provide a practical and efficient solution to these geotechnical challenges. They interact with the surrounding soil primarily through shaft friction and end-bearing resistance (Fellenius, 2001). The efficiency of these resistance mechanisms depends on several factors including pile geometry, installation energy, soil type, and depth of embedment. However, in coastal regions like Lagos where the subsurface is stratified and often includes soft clay layers interbedded with silty sands, the behavior of driven piles becomes more complex and site-specific (Akpila et al., 2019). This study was undertaken to evaluate the real-world performance of driven piles installed in Ilubirin, Lagos, using full-scale static axial load testing. A total of 30 precast reinforced concrete piles were tested across three project sites within the area. The piles, each measuring 360 mm × 360 mm in cross-section and installed to depths between 31.0 m and 36.5 m, were subjected to axial loads using the Kentledge method. Measurements of load versus settlement were recorded, and the performance of each pile was assessed based on stability, settlement behavior, and overall structural adequacy under the applied loads.

The significance of this study lies in its contribution to understanding the in-situ behavior of driven piles in Lagos’s coastal soil environment. By focusing solely on the outcomes of field load tests, the research provides empirical evidence of how driven piles interact with layered and weak soils typical of the region. It also offers practical insights for construction engineers, geotechnical designers, and infrastructure developers tasked with selecting appropriate foundation systems in similar geologic conditions. The scope of the research is limited to the structural performance evaluation of driven piles using static load testing results obtained from Ilubirin. No predictive modeling or analytical estimations were performed. Instead, the study emphasizes load-settlement measurements, test pile responses, and field observations to draw conclusions about pile behavior. The outcome is a localized performance-based assessment that reflects the realities of foundation construction in Lagos and similar tropical coastal environments.

# LITERATURE REVIEW

Pile foundations are essential for the stability of structures constructed on weak or compressible soils. Driven piles, in particular, are extensively used where surface soils cannot safely support loads from superstructures. Their ability to transfer loads to deeper, more stable strata makes them invaluable in areas like Lagos, Nigeria, where soil conditions are highly variable and often problematic (Meyerhof, 1976; Randolph et al., 1979).

Driven piles are prefabricated structural elements that are driven into the ground using mechanical equipment. Their effectiveness arises from the development of both skin friction along the shaft and end-bearing resistance at the tip (Eslami et al., 1997). Other pile types include bored piles and continuous flight auger (CFA) piles, which differ in installation technique and performance based on soil conditions. While bored piles minimize vibration and noise during installation and are ideal in urban areas, CFA piles reduce soil disturbance and are suited for saturated or soft soils (Doherty et al., 2011).

In Lagos, driven piles are commonly used due to the compressible clay layers, high groundwater table, and the need for rapid construction schedules. These conditions demand pile foundations that can provide reliable support with minimal settlement (Adeleye et al., 2014). However, the performance of driven piles in such conditions depends on accurate design and appropriate construction methods.

2.1 Geological and Soil Properties in Lagos

Lagos is situated in a coastal region characterized by alluvial and marine deposits comprising silty clay, loose sand, and peat. The soils often exhibit high compressibility and low shear strength, particularly in the soft clay zones that dominate much of the subsurface profile (Afolabi et al., 2018). These properties necessitate the use of pile foundations to ensure structural safety.

The soil variability across Lagos presents a challenge to engineers. For instance, while lateritic soils provide good frictional resistance when dry, they become plastic and lose strength upon saturation. Conversely, sandy soils, though less compressible, are prone to liquefaction under dynamic loading. Such heterogeneity requires site-specific geotechnical investigations and tailored pile design strategies (Olugbenga, 2018).

2.2 Performance Challenges and Soil-Pile Interaction

Driven piles derive capacity primarily from skin friction and end bearing. In soft clay, the skin friction is typically low, making end bearing the dominant mode of load transfer. In contrast, in sandy soils, friction contributes significantly to the load-bearing capacity (Eslami et al., 1997). However, improper pile installation in such soils can reduce the effectiveness of these mechanisms.

Soil-pile interaction is influenced by several factors including pile material, soil density, installation method, and stress history. Figure 1 in the thesis illustrates the load transfer mechanism in axially loaded piles, where the total capacity Qu​ is the sum of base resistance Qb​ and shaft resistance Qs​. The importance of understanding this interaction is underlined by Fellenius (2001), who notes that variability in skin resistance can lead to significant overestimation or underestimation of pile performance.

2.3 Empirical Methods Based on SPT and CPT Data

Empirical methods simplify the design process by correlating pile capacity directly to in-situ test results, particularly the Standard Penetration Test (SPT) and Cone Penetration Test (CPT). These methods are attractive due to their ease of application but must be used with caution in soils unlike those from which the methods were derived (Shooshpasha et al., 2013).

Aoki and De’Alencar (1975) proposed equations for calculating shaft and base resistance using average SPT values near the pile base:

qb=aNb ; qs​=kNs​

Where aaa and kkk are constants depending on soil type. For sands, a=14a = 14a=14, k=1k = 1k=1; for clays, a=60a = 60a=60, k=0.2k = 0.2k=0.2.

Meyerhof (1976) introduced another popular method using the average SPT value 10D above and 4D below the pile base. Similarly, Decourt (1995) proposed:

qb=kbNb ; qs​=α(208Ns+10)​

For CPT-based predictions, Aoki and De’Alencar (1975) and Penpile (1978) suggested equations such as:

rt= qcfb ; rs=sFs∙qc

Where qc is the cone resistance, s is sleeve friction, and fb​, Fs​ are empirical constants.

These empirical approaches, though convenient, often yield inaccurate results in Lagos soils due to variations in soil formation, strength, and moisture conditions not considered in the original model derivations (Alilahi & Adampira, 2015).

2.4 Load Testing: The Benchmark for Performance

Static pile load testing remains the most reliable method for determining pile performance. It involves the direct measurement of settlement under incremental loads, thus reflecting true field behavior. Though costly and time-consuming, load testing is critical for validating design assumptions and calibrating analytical or empirical methods (Baca et al., 2019; Eslami et al., 2013).

According to Zhaohui and Robert (1998), uncertainties in pile design stem from assumptions regarding loading conditions, pile behavior under load, and subsoil reaction. Load testing helps resolve these uncertainties by capturing the real-time interaction between the pile and surrounding soil.

2.5 Environmental and Sustainability Considerations

Driven piles, while structurally efficient, can cause environmental disturbances such as noise, vibration, and soil displacement. These effects are particularly relevant in densely populated urban settings like Lagos. Press-in piling methods and vibration monitoring have been suggested as strategies to mitigate environmental impact (Preiss & Adam, 2012; Matsumoto et al., 1995).

Sustainable practices also include the use of recycled materials, optimization of pile length, and use of performance-based design rather than prescriptive methods. This shift supports both environmental goals and improved structural performance (Johnston, 2016).

# MATERIALS AND METHOD

This study was designed to assess the performance of driven pile foundations under real-world geotechnical conditions in Ilubirin, Lagos, a typical coastal zone characterized by soft clay, silty sand, and high groundwater levels. The methodology integrates soil investigation, pile installation records, field load testing, laboratory analysis, and theoretical computations. The aim was to analyze pile capacity using both field measurements and analytical estimates, and to compare them in order to evaluate predictive accuracy and pile behavior.

3.1 Study Location and Geotechnical Context

The research was carried out at three construction sites in the Ilubirin area, located along the Lagos Lagoon front. The choice of these sites was based on their challenging soil profiles, existing construction activity, and availability of driven pile records. The sites are geologically characterized by alluvial sediments composed of soft clay, silty clay, and sand layers, typically found at depths ranging from 0 to 40 meters. The presence of a high water table and weak near-surface soils necessitated the use of deep foundations shown in Figure 1.

Figure 1: Study Area Map

3.2 Field Investigation and Subsoil Characterization

At each site, three boreholes were drilled to depths ranging from 35 to 40 meters using rotary drilling equipment. Standard Penetration Tests (SPT) were performed at 1.5-meter intervals to determine in-situ soil resistance. The blow counts were recorded and corrected for overburden pressure and field procedures. Cone Penetration Tests (CPT) were also conducted to obtain continuous resistance profiles for comparative assessment with the SPT results.

Soil samples collected from the boreholes were preserved and transported to the laboratory for classification and strength testing. These included natural moisture content, Atterberg limits, grain size analysis, bulk density, and unconfined compressive strength (UCS) for cohesive soils. The data obtained helped to define the input parameters for pile capacity prediction and analysis. A generalized soil stratification for one of the study sites is presented in Table 3.

Table 1: Generalized Soil Profile at Site A

| Depth (m) | Soil Description | SPT (N-value) | Moisture Content (%) | Clay Content (%) |
| --- | --- | --- | --- | --- |
| 0 - 5 | Soft silty clay | 4 | 62 | 58 |
| 5 - 12 | Very soft clayey silt | 6 | 49 | 41 |
| 12 - 20 | Silty sand (medium) | 12 | 32 | 18 |
| 20 - 36 | Dense sand with gravel | 28 | 18 | 10 |

3.2.1 Pile Details and Installation

Precast reinforced concrete piles of square cross-sections (350 mm × 350 mm and 400 mm × 400 mm) were used across the sites. The pile lengths ranged from 31 m to 36.5 m. Driving was executed using a hydraulic hammer, and the pile penetration rate, number of blows per meter, and set per blow were recorded to assess driving resistance. Pile records were reviewed to identify any anomalies during driving, such as sudden loss of resistance or soil plug formation. The driving resistance provided preliminary insights into soil stratification and potential changes in soil strength with depth.

3.2.2 Static Load Testing

Static load tests were conducted on selected piles at each site using the Kentledge method. A reaction system was constructed using precast concrete blocks to provide counterweight. A calibrated hydraulic jack was placed between the test pile and the reaction beam, and incremental axial loads were applied in accordance with load test standards. The load was applied in stages, typically in increments of 25% of the proposed working load, until the maximum test load (usually 200% of the working load) was reached. Each load increment was held for 30 minutes, and settlements were recorded at 5-minute intervals using dial gauges mounted on a reference beam supported independently of the test pile.

Table 2: Sample Load-Settlement Observation (Test Pile TP-1)

| Load Applied (kN) | Settlement (mm) |
| --- | --- |
| 0 | 0.00 |
| 200 | 1.14 |
| 400 | 2.73 |
| 600 | 4.35 |
| 800 | 6.88 |
| 1000 | 9.42 |
| 1200 | 13.11 |
| 1400 | 17.25 |

The resulting load-settlement curves were plotted to evaluate the load capacity, identify the point of failure or plunging, and determine the allowable load using criteria such as the 6 mm offset method or 10% of pile diameter rule.

3.3 Data Analysis and Comparison

The final phase of the study involved a systematic comparison between the pile capacities obtained through field load testing and those predicted using analytical and empirical methods. The objective was to evaluate the accuracy, consistency, and reliability of the theoretical approaches in predicting pile behavior under real geotechnical conditions in Ilubirin, Lagos.

Static load test results were considered the benchmark or “true” capacity of the piles since they represent actual performance under load. For each test pile, the ultimate capacity was derived from the load-settlement curve using both the Davisson offset method and the 10% diameter criterion. These measured values were then compared with the capacities estimated analytically using total and effective stress methods, as well as empirically through SPT-based correlations.

Graphical representations were developed to enhance interpretation. Bar charts were plotted to illustrate the predicted vs. measured capacities for all test piles. Scatter plots of predicted capacity against measured capacity helped visualize the degree of alignment between methods and actual performance. The closer the data points were to the 45-degree line (representing perfect prediction), the more accurate the method.

Through this comparative analysis, it was possible to identify which methods performed best under local soil conditions. It also allowed for the evaluation of method sensitivity to soil variability, pile length, and embedment depth. The findings informed conclusions on which design methods were most appropriate for predicting pile behavior in Lagos, and highlighted the importance of incorporating site-specific calibration into pile design practice.

# RESULTS AND DISCUSSIONS

### 4.1 Assessment of Pile Load Tests Pile Capacity

Analysis of Pile load tests were carried out using six methods that are frequently used by engineers. The methods includes: Brinch Hansen, Tangent, Decourt, Shen, Chin-Kondner and Abd Elsamee. The predicted pile capacity of pile load tests based on these methods are as shown in Table 3 and 4.

Table 3:  SITE A Ultimate Pile Capacity from Pile Load Test Predictive methods

| Pile ref. | Brinch Hansen | Shen | Chin - Kondner | Decourt | Tangent | Abd Elsamee |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) |
| PLA1 | 2041 | 1250 | 2817 | 2836 | 1200 | 840 |
| PLA2 | 1976 | 1100 | 2500 | 2388 | 1180 | 781 |
| PLA3 | 2282 | 1150 | 2381 | 2365 | 900 | 682 |
| PLA4 | 2440 | 1200 | 2803 | 2710 | 1360 | 698 |
| PLA5 | 2531 | 1400 | 2941 | 2764 | 1850 | 599 |
| PLA6 | 1757 | 1050 | 2439 | 2471 | 1080 | 586 |
| PLA7 | 3536 | 1300 | 3225 | 3224 | 1900 | 635 |
| PLA8 | 1725 | 1150 | 3333 | 3607 | 1130 | 760 |
| PLA9 | 2440 | 1100 | 2500 | 2485 | 1200 | 630 |
| PLA10 | 2384 | 1050 | 2337 | 2336 | 1300 | 581 |

Table 4:  SITE B Ultimate Pile Capacity from Pile Load Test Predictive methods

| Pile ref. | Brinch Hansen | Shen | Chin - Kondner | Decourt | Tangent | Abd Elsamee |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) |
| PLB1 | 2795 | 1250 | 3115 | 2997 | 1490 | 533 |
| PLB2 | 1725 | 950 | 2273 | 2297 | 1200 | 683 |
| PLB3 | 2440 | 1000 | 2500 | 2485 | 1450 | 574 |
| PLB4 | 2214 | 1050 | 2500 | 2385 | 1550 | 662 |
| PLB5 | 3727 | 950 | 2500 | 2552 | 1200 | 881 |
| PLB6 | 2384 | 1200 | 2502 | 2387 | 1552 | 816 |
| PLB7 | 2357 | 950 | 3572 | 2153 | 2400 | 698 |
| PLB8 | 2565 | 850 | 3333 | 2389 | 1200 | 857 |
| PLB9 | 1813 | 850 | 2142 | 2141 | 1555 | 1059 |
| PLB10 | 2384 | 850 | 2337 | 2344 | 1380 | 790 |

Table 5:  SITE C Ultimate Pile Capacity from Pile Load Test Predictive methods

| Pile ref. | Brinch Hansen | Shen | Chin - Kondner | Decourt | Tangent | Abd Elsamee |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) |
| PLC1 | 2282 | 800 | 2564 | 2555 | 1450 | 1032 |
| PLC2 | 2041 | 900 | 2500 | 2387 | 1600 | 918 |
| PLC3 | 2635 | 850 | 2564 | 2510 | 1300 | 1173 |
| PLC4 | 2635 | 800 | 2857 | 2864 | 1600 | 875 |
| PLC5 | 2887 | 1050 | 3226 | 2754 | 2200 | 864 |
| PLC6 | 1768 | 900 | 2045 | 2054 | 1400 | 833 |
| PLC7 | 2440 | 870 | 2347 | 2353 | 1150 | 1190 |
| PLC8 | 2887 | 1150 | 3125 | 3176 | 1550 | 957 |
| PLC9 | 2500 | 950 | 2500 | 2486 | 1500 | 1043 |
| PLC10 | 2282 | 950 | 2564 | 2600 | 1250 | 770 |

The assessment of pile capacity across Sites A, B, and C using six widely recognized interpretative methods reveals valuable insights into the overall performance of driven piles in Ilubirin. The use of Brinch Hansen, Tangent, Decourt, Shen, Chin-Kondner, and Abd Elsamee methods provided a broad analytical basis for evaluating ultimate pile capacity derived from static load tests.

From the results presented in Table 3 to 5, pile capacities for Site A ranged from 581 kN to 3607 kN, with an average capacity of 1824 kN and a coefficient of variation (COV) of 0.48. For Site B, capacities ranged from 533 kN to 3727 kN with a slightly lower COV of 0.46. Site C showed the most consistent data set, with pile capacities ranging from 770 kN to 3326 kN and a COV of 0.42. These coefficients of variation indicate a generally low dispersion in the dataset across all sites, suggesting a relatively uniform subsurface condition and predictable pile behavior when installed to similar depths and specifications.

Among the methods used, the Chin-Kondner and Decourt approaches consistently provided the highest pile capacity estimates, likely due to their sensitivity to the non-linear portions of the load-settlement curves. In contrast, the Tangent and Abd Elsamee methods yielded the lowest capacity values, indicating a more conservative interpretation of the pile behavior. This variation in interpretative outcomes emphasizes the importance of selecting appropriate load test interpretation techniques based on site conditions and pile response characteristics.

Overall, the consistency in results across sites, particularly the low variation in Site C, confirms the structural adequacy and reliability of driven piles in the coastal soils of Lagos. These findings support the effectiveness of driven pile systems and highlight the relevance of full-scale testing and multi-method interpretation for informed geotechnical decision-making.

Overall, the multi-method interpretation underscores the robust performance of driven piles across all three sites. The relatively low variation in results and consistent trends among higher- and lower-bound methods reinforce the structural adequacy of the piles and highlight the importance of selecting multiple interpretation techniques for evaluating pile capacity from load tests.

4.2 Pile Load Tests Pile Capacity Comparison with the Average Capacity

In the analysis and design of piles, since there are many predictive methods and emperical relationships through which the ultimate capacity of piles can be determined, it will be misleading to make use of just one method. In view of this, getting the average value from all the methods will not only be safer and economical but will help the designer in making a good judgement on the method whose value is closest to the average value amongst all the methods considered. The ratio of individual ultimate capacity for each predictive method to that of the average ultimate capacity (QU/QUave) was determined with the methods having values of (QU/QUave) closest to 1 being the nearest to the average capacity as shown in Table 6.

Table 6: Ratio of individual ultimate capacity for each predictive method to that of the average ultimate capacity (QU/QUave)

| Pile ref. | Brinch | Shen | Chin | Decourt | Tangent | Abd Elsamee |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qu/Qu(ave) | Qu/Qu(ave) | Qu/Qu(ave) | Qu/Qu(ave) | Qu/Qu(ave) | Qu/Qu(ave) |
| PLA1 | 1.11 | 0.68 | 1.54 | 1.55 | 0.66 | 0.46 |
| PLA2 | 1.19 | 0.66 | 1.51 | 1.44 | 0.71 | 0.47 |
| PLA3 | 1.40 | 0.71 | 1.46 | 1.45 | 0.55 | 0.42 |
| PLA4 | 1.31 | 0.64 | 1.50 | 1.45 | 0.73 | 0.37 |
| PLA5 | 1.26 | 0.70 | 1.46 | 1.37 | 0.92 | 0.30 |
| PLA6 | 1.12 | 0.67 | 1.56 | 1.58 | 0.69 | 0.37 |
| PLA7 | 1.54 | 0.56 | 1.40 | 1.40 | 0.82 | 0.28 |
| PLA8 | 0.88 | 0.59 | 1.71 | 1.85 | 0.58 | 0.39 |
| PLA9 | 1.41 | 0.64 | 1.45 | 1.44 | 0.70 | 0.37 |
| PLA10 | 1.43 | 0.63 | 1.40 | 1.40 | 0.78 | 0.35 |
| Average | 1.27 | 0.65 | 1.50 | 1.49 | 0.71 | 0.38 |
| PLB1 | 1.38 | 0.62 | 1.53 | 1.48 | 0.73 | 0.26 |
| PLB2 | 1.13 | 0.62 | 1.49 | 1.51 | 0.79 | 0.45 |
| PLB3 | 1.40 | 0.57 | 1.44 | 1.43 | 0.83 | 0.33 |
| PLB4 | 1.28 | 0.61 | 1.45 | 1.38 | 0.90 | 0.38 |
| PLB5 | 1.89 | 0.48 | 1.27 | 1.30 | 0.61 | 0.45 |
| PLB6 | 1.32 | 0.66 | 1.38 | 1.32 | 0.86 | 0.45 |
| PLB7 | 1.17 | 0.47 | 1.77 | 1.06 | 1.19 | 0.35 |
| PLB8 | 1.37 | 0.46 | 1.79 | 1.28 | 0.64 | 0.46 |
| PLB9 | 1.14 | 0.53 | 1.34 | 1.34 | 0.98 | 0.66 |
| PLB10 | 1.42 | 0.51 | 1.39 | 1.39 | 0.82 | 0.47 |
| Average | 1.35 | 0.55 | 1.49 | 1.35 | 0.83 | 0.43 |
| PLC1 | 1.28 | 0.45 | 1.44 | 1.43 | 0.81 | 0.58 |
| PLC2 | 1.18 | 0.52 | 1.45 | 1.38 | 0.93 | 0.53 |
| PLC3 | 1.43 | 0.46 | 1.39 | 1.37 | 0.71 | 0.64 |
| PLC4 | 1.36 | 0.41 | 1.47 | 1.48 | 0.83 | 0.45 |
| PLC5 | 1.33 | 0.49 | 1.49 | 1.27 | 1.02 | 0.40 |
| PLC6 | 1.18 | 0.60 | 1.36 | 1.37 | 0.93 | 0.56 |
| PLC7 | 1.41 | 0.50 | 1.36 | 1.36 | 0.67 | 0.69 |
| PLC8 | 1.35 | 0.54 | 1.46 | 1.48 | 0.72 | 0.45 |
| PLC9 | 1.37 | 0.52 | 1.37 | 1.36 | 0.82 | 0.57 |
| PLC10 | 1.31 | 0.55 | 1.48 | 1.50 | 0.72 | 0.44 |
| Average | 1.32 | 0.50 | 1.43 | 1.40 | 0.82 | 0.53 |

Across all sites (A, B, and C), the Brinch Hansen method consistently returned values closest to the average, with Qu/Quave ratios of 1.27, 1.35, and 1.32, respectively. This indicates that the method provides moderate and stable estimates that do not deviate significantly from the overall trend. In contrast, Chin-Kondner and Decourt methods frequently overshot the average values, with average Qu/Quave ratios ranging from 1.43 to 1.50, making them the most optimistic in capacity estimation. On the other end, the Abd Elsamee method produced the most conservative estimates with Qu/Quave averages of 0.38 (Site A), 0.43 (Site B), and 0.53 (Site C), indicating underestimation in all cases.

This spread demonstrates that while certain methods may align more closely with average behavior (e.g., Brinch Hansen), others like Chin-Kondner and Abd Elsamee may require caution, particularly in critical designs where over- or underestimation can impact structural safety or lead to uneconomical designs. The findings reinforce the value of using multiple interpretative approaches during pile load analysis and suggest that incorporating average-based evaluations provides a safer, more rational foundation for decision-making in Lagos’s variable soil conditions.

4.3 Pile Load Test Capacity Comparison with the Failure Load

The piles tested to failure were very few. However, the failure loads on the piles were taken as the load at a settlement of 10% of the pile diameter. Table 7 shows the various values of the predicted ultimate capacity of piles as well as the failure load at each location while Table 8 presents the average relative accuracies as quantified using the relative errors of all the predictive methods. The negative sign indicates that the predicted value is less than the measured failure load.

The 100% line on the histogram shows a basis for comparing the predicted values and measured values of the pile capacity. In this case, the methods having Qp/Qm lower than the 100% threshold is an evidence of under prediction while those with values higher than the 100% line are associated with over prediction of the ultimate pile capacity.

Table 7: Predicted Pile load tests results Comparison with Failure Loads

| Pile ref. | Brinch Hansen | Shen | Chin - Kondner | Decourt | Tangent | Abd Elsamee | Failure Load |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) |
| PLA1 | 2041 | 1250 | 2817 | 2836 | 1200 | 840 | 1750 |
| PLA2 | 1976 | 1100 | 2500 | 2388 | 1180 | 781 | 1500 |
| PLA3 | 2282 | 1150 | 2381 | 2365 | 900 | 682 | 1250 |
| PLA4 | 2440 | 1200 | 2803 | 2710 | 1360 | 698 | 1500 |
| PLA5 | 2531 | 1400 | 2941 | 2764 | 1850 | 599 | 2450 |
| PLA6 | 1757 | 1050 | 2439 | 2471 | 1080 | 586 | 1500 |
| PLA7 | 3536 | 1300 | 3225 | 3224 | 1900 | 635 | 2450 |
| PLA8 | 1725 | 1150 | 3333 | 3607 | 1130 | 760 | 1250 |
| PLA9 | 2440 | 1100 | 2500 | 2485 | 1200 | 630 | 1500 |
| PLA10 | 2384 | 1050 | 2337 | 2336 | 1300 | 581 | 1750 |
| PLB1 | 2795 | 1250 | 3115 | 2997 | 1490 | 533 | 1750 |
| PLB2 | 1725 | 950 | 2273 | 2297 | 1200 | 683 | 1250 |
| PLB3 | 2440 | 1000 | 2500 | 2485 | 1450 | 574 | 1250 |
| PLB4 | 2214 | 1050 | 2500 | 2385 | 1550 | 662 | 1750 |
| PLB5 | 3727 | 950 | 2500 | 2552 | 1200 | 881 | 2450 |
| PLB6 | 2384 | 1200 | 2502 | 2387 | 1552 | 816 | 1500 |
| PLB7 | 2357 | 950 | 3572 | 2153 | 2400 | 698 | 2800 |
| PLB8 | 2565 | 850 | 3333 | 2389 | 1200 | 857 | 2000 |
| PLB9 | 1813 | 850 | 2142 | 2141 | 1555 | 1059 | 1250 |
| PLB10 | 2384 | 850 | 2337 | 2344 | 1380 | 790 | 1500 |
| PLC1 | 2282 | 800 | 2564 | 2555 | 1450 | 1032 | 1750 |
| PLC2 | 2041 | 900 | 2500 | 2387 | 1600 | 918 | 1750 |
| PLC3 | 2635 | 850 | 2564 | 2510 | 1300 | 1173 | 1500 |
| PLC4 | 2635 | 800 | 2857 | 2864 | 1600 | 875 | 1750 |
| PLC5 | 2887 | 1050 | 3226 | 2754 | 2200 | 864 | 2800 |
| PLC6 | 1768 | 900 | 2045 | 2054 | 1400 | 833 | 1500 |
| PLC7 | 2440 | 870 | 2347 | 2353 | 1150 | 1190 | 1250 |
| PLC8 | 2887 | 1150 | 3125 | 3176 | 1550 | 957 | 2800 |
| PLC9 | 2500 | 950 | 2500 | 2486 | 1500 | 1043 | 1500 |
| PLC10 | 2282 | 950 | 2564 | 2600 | 1250 | 770 | 1200 |

The comparison of predicted capacities with measured failure loads revealed significant variability among methods (Table 7). Brinch Hansen, Chin-Kondner, and Decourt consistently over-predicted (Qp/Qm > 1.0), with Chin-Kondner reaching up to 1.53. Conversely, Tangent, Shen, and Abd Elsamee methods under-predicted capacities, with Abd Elsamee yielding the lowest Qp/Qm ratios (40.2%–94.7%). Relative accuracy rankings placed the Tangent method highest (-20.99%), indicating better alignment with failure loads. Shen and Abd Elsamee, though conservative, showed higher dispersion. The findings confirm the necessity of evaluating both accuracy and consistency when interpreting pile test results.

4.4 Pile Load Test Capacity Comparison with the Failure Load

Design, execution of pile foundation is not a cheap venture, adequate efforts must be mobilized to ensure that the final decision made on the capacity of the piles for any structure to be executed is not only safe but coat effective. Table 8 shows the factor of safety obtained for all the predictive methods in all locations considered in the research, with the average for each method also stated. The factor of safety was obtained through the ratio of the ultimate capacity of all predictive methods to the design load.

Table 8: Factor of safety from predictive pile load test methods

| Pile ref. | Brinch Hansen | Shen | Chin - Kondner | Decourt | Tangent | Abd - Elsamee |
| --- | --- | --- | --- | --- | --- | --- |
| PLA1 | 2.92 | 1.79 | 4.02 | 4.05 | 1.71 | 1.20 |
| PLA2 | 2.82 | 1.57 | 3.57 | 3.41 | 1.69 | 1.12 |
| PLA3 | 3.26 | 1.64 | 3.40 | 3.38 | 1.29 | 0.97 |
| PLA4 | 3.49 | 1.71 | 4.00 | 3.87 | 1.94 | 1.00 |
| PLA5 | 3.62 | 2.00 | 4.20 | 3.95 | 2.64 | 0.86 |
| PLA6 | 2.51 | 1.50 | 3.48 | 3.53 | 1.54 | 0.84 |
| PLA7 | 5.05 | 1.86 | 4.61 | 4.61 | 2.71 | 0.91 |
| PLA8 | 2.46 | 1.64 | 4.76 | 5.15 | 1.61 | 1.09 |
| PLA9 | 3.49 | 1.57 | 3.57 | 3.55 | 1.71 | 0.90 |
| PLA10 | 3.41 | 1.50 | 3.34 | 3.34 | 1.86 | 0.83 |
| PLB1 | 3.73 | 1.67 | 4.15 | 4.00 | 1.99 | 0.71 |
| PLB2 | 2.46 | 1.36 | 3.25 | 3.28 | 1.71 | 0.98 |
| PLB3 | 3.49 | 1.43 | 3.57 | 3.55 | 2.07 | 0.82 |
| PLB4 | 3.16 | 1.50 | 3.57 | 3.41 | 2.21 | 0.95 |
| PLB5 | 5.32 | 1.36 | 3.57 | 3.65 | 1.71 | 1.26 |
| PLB6 | 3.41 | 1.71 | 3.57 | 3.41 | 2.22 | 1.17 |
| PLB7 | 3.37 | 1.36 | 5.10 | 3.08 | 3.43 | 1.00 |
| PLB8 | 3.66 | 1.21 | 4.76 | 3.41 | 1.71 | 1.22 |
| PLB9 | 2.59 | 1.21 | 3.06 | 3.06 | 2.22 | 1.51 |
| PLB10 | 3.41 | 1.21 | 3.34 | 3.35 | 1.97 | 1.13 |
| PLC1 | 3.26 | 1.14 | 3.66 | 3.65 | 2.07 | 1.47 |
| PLC2 | 2.92 | 1.29 | 3.57 | 3.41 | 2.29 | 1.31 |
| PLC3 | 3.76 | 1.21 | 3.66 | 3.59 | 1.86 | 1.68 |
| PLC4 | 3.76 | 1.14 | 4.08 | 4.09 | 2.29 | 1.25 |
| PLC5 | 4.12 | 1.50 | 4.61 | 3.93 | 3.14 | 1.23 |
| PLC6 | 2.53 | 1.29 | 2.92 | 2.93 | 2.00 | 1.19 |
| PLC7 | 3.49 | 1.24 | 3.35 | 3.36 | 1.64 | 1.70 |
| PLC8 | 4.12 | 1.64 | 4.46 | 4.54 | 2.21 | 1.37 |
| PLC9 | 3.57 | 1.36 | 3.57 | 3.55 | 2.14 | 1.49 |
| PLC10 | 3.26 | 1.36 | 3.66 | 3.71 | 1.79 | 1.10 |
| Average | 3.41 | 1.47 | 3.82 | 3.66 | 2.05 | 1.14 |

Table 10 presents the factor of safety (FOS) for each predictive method relative to a consistent design load. Results show that Chin-Kondner yielded the highest average FOS of 3.82, followed by Decourt (3.66) and Brinch Hansen (3.41), indicating more conservative design outputs. On the contrary, Tangent (2.05), Shen (1.47), and Abd Elsamee (1.14) produced significantly lower FOS, reflecting under-prediction tendencies. These results reinforce the importance of choosing an interpretation method that not only ensures safety but also provides cost-effective design margins, especially in variable subsoil conditions typical of Lagos. Conservative estimates must be balanced against economic feasibility.

4.5 Assessment of Pile Capacity Based on Dynamic Formulae

Six methods that are widely used were considered in assessing the pile capacity using dynamic formula. The methods are : Janbu, Gates, Navy – Mckay, Danish. Eytelwein and Modified ENR. Table 9 reflects the ultimate pile capacity of the piles in the location considered for the research as gotten from the dynamic formulamethods mentioned earlier.

Table 9: Dynamic Formula Ultimate Capacity of Pile Foundation

| PILE REF | GATES | MODIFIED ENR | DANISH | NAVY - MCKAY | EYTELWEIN | JANBU |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) |
| PLA1 | 694 | 751 | 1026 | 711 | 354 | 683 |
| PLA2 | 771 | 975 | 1211 | 979 | 505 | 735 |
| PLA3 | 816 | 1106 | 1299 | 1126 | 574 | 699 |
| PLA4 | 836 | 1201 | 1370 | 1263 | 665 | 730 |
| PLA5 | 795 | 1045 | 1259 | 1056 | 542 | 718 |
| PLA6 | 822 | 1124 | 1308 | 1142 | 578 | 686 |
| PLA7 | 988 | 1794 | 1646 | 2040 | 1033 | 511 |
| PLA8 | 734 | 861 | 1120 | 840 | 425 | 714 |
| PLA9 | 792 | 1036 | 1253 | 1046 | 537 | 719 |
| PLA10 | 655 | 682 | 965 | 659 | 342 | 711 |
| PLB1 | 799 | 1029 | 1236 | 1010 | 497 | 661 |
| PLB2 | 747 | 889 | 1140 | 864 | 432 | 698 |
| PLB3 | 868 | 1269 | 1387 | 1305 | 650 | 625 |
| PLB4 | 759 | 917 | 1160 | 891 | 444 | 689 |
| PLB5 | 1088 | 2279 | 1793 | 2813 | 1400 | 375 |
| PLB6 | 884 | 1332 | 1425 | 1390 | 696 | 618 |
| PLB7 | 870 | 1282 | 1397 | 1326 | 664 | 631 |
| PLB8 | 1071 | 2196 | 1775 | 2675 | 1339 | 399 |
| PLB9 | 738 | 860 | 1116 | 828 | 412 | 691 |
| PLB10 | 833 | 1136 | 1304 | 1134 | 559 | 640 |
| PLC1 | 572 | 513 | 790 | 473 | 238 | 600 |
| PLC2 | 600 | 559 | 841 | 516 | 259 | 624 |
| PLC3 | 447 | 342 | 579 | 311 | 158 | 457 |
| PLC4 | 662 | 698 | 980 | 675 | 351 | 716 |
| PLC5 | 641 | 657 | 939 | 634 | 332 | 707 |
| PLC6 | 619 | 603 | 886 | 571 | 293 | 666 |
| PLC7 | 438 | 328 | 562 | 293 | 147 | 435 |
| PLC8 | 681 | 726 | 1005 | 691 | 348 | 692 |
| PLC9 | 630 | 621 | 904 | 584 | 296 | 664 |
| PLC10 | 412 | 300 | 523 | 266 | 132 | 402 |

Results reveal notable variability across methods. The Danish method consistently yielded the highest pile capacities, followed closely by Navy–Mckay and Modified ENR, while Gates, Eytelwein, and Janbu methods produced more conservative estimates. For instance, PLA7 reached 2040 kN with Navy–Mckay but only 511 kN with Janbu. This disparity underscores the influence of method assumptions on results and highlights the need for caution when selecting dynamic formulas. Danish and Navy–Mckay may overestimate capacity if energy losses or soil damping are not well accounted for during driving.

4.6 Dynamic Formulae  Pile Capacity Comparison with Failure Load

When compared with the failure load, the Danish method has the best performance with reference to the relative accuracy. Table 10 shows the ultimate pile capacity of the different methods alongside the failure load while table 11 is a reflection of the relative accuracy of all the methods. Comparison of the estimated pile capacity with the measured capacity is presented in Figure 2. The graph shows that all the dynamic formulae underestimated the measured pile capacity as indicated by the threshold line on the graph.

Table 10 Dynamic Formula Ultimate Capacity of Pile Foundation

| PILE REF | GATES | MODIFIED ENR | DANISH | NAVY - MCKAY | EYTELWEIN | JANBU | FAILURE LOAD |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) | Qu (kN) |
| PLA1 | 694 | 751 | 1026 | 711 | 354 | 683 | 1750 |
| PLA2 | 771 | 975 | 1211 | 979 | 505 | 735 | 1500 |
| PLA3 | 816 | 1106 | 1299 | 1126 | 574 | 699 | 1250 |
| PLA4 | 836 | 1201 | 1370 | 1263 | 665 | 730 | 1500 |
| PLA5 | 795 | 1045 | 1259 | 1056 | 542 | 718 | 2450 |
| PLA6 | 822 | 1124 | 1308 | 1142 | 578 | 686 | 1500 |
| PLA7 | 988 | 1794 | 1646 | 2040 | 1033 | 511 | 2450 |
| PLA8 | 734 | 861 | 1120 | 840 | 425 | 714 | 1250 |
| PLA9 | 792 | 1036 | 1253 | 1046 | 537 | 719 | 1500 |
| PLA10 | 655 | 682 | 965 | 659 | 342 | 711 | 1750 |
| PLB1 | 799 | 1029 | 1236 | 1010 | 497 | 661 | 1750 |
| PLB2 | 747 | 889 | 1140 | 864 | 432 | 698 | 1250 |
| PLB3 | 868 | 1269 | 1387 | 1305 | 650 | 625 | 1250 |
| PLB4 | 759 | 917 | 1160 | 891 | 444 | 689 | 1750 |
| PLB5 | 1088 | 2279 | 1793 | 2813 | 1400 | 375 | 2450 |
| PLB6 | 884 | 1332 | 1425 | 1390 | 696 | 618 | 1500 |
| PLB7 | 870 | 1282 | 1397 | 1326 | 664 | 631 | 2800 |
| PLB8 | 1071 | 2196 | 1775 | 2675 | 1339 | 399 | 2000 |
| PLB9 | 738 | 860 | 1116 | 828 | 412 | 691 | 1250 |
| PLB10 | 833 | 1136 | 1304 | 1134 | 559 | 640 | 1500 |
| PLC1 | 572 | 513 | 790 | 473 | 238 | 600 | 1750 |
| PLC2 | 600 | 559 | 841 | 516 | 259 | 624 | 1750 |
| PLC3 | 447 | 342 | 579 | 311 | 158 | 457 | 1500 |
| PLC4 | 662 | 698 | 980 | 675 | 351 | 716 | 1750 |
| PLC5 | 641 | 657 | 939 | 634 | 332 | 707 | 2800 |
| PLC6 | 619 | 603 | 886 | 571 | 293 | 666 | 1500 |
| PLC7 | 438 | 328 | 562 | 293 | 147 | 435 | 1250 |
| PLC8 | 681 | 726 | 1005 | 691 | 348 | 692 | 2800 |
| PLC9 | 630 | 621 | 904 | 584 | 296 | 664 | 1500 |
| PLC10 | 412 | 300 | 523 | 266 | 132 | 402 | 1200 |

Table 11 Dynamic Formula Relative Accuracy of Ultimate Capacity of Pile Foundation

| Methods | Relative Accuracy (%) | Rank |
| --- | --- | --- |
| Gates | -57.80 | 4 |
| Modified ENR | -44.82 | 3 |
| Danish | -35.17 | 1 |
| Navy - Mckay | -42.91 | 2 |
| Eytelwein | -71.18 | 6 |
| Janbu | -64.18 | 5 |

| Methods | Mean | Standard Deviation | COV |
| --- | --- | --- | --- |
| Gates | 0.42 | 0.10 | 0.23 |
| ENR Modified | 0.56 | 0.24 | 0.43 |
| Danish | 0.65 | 0.19 | 0.29 |
| Navy - McKay | 0.58 | 0.28 | 0.48 |
| Eytelwein | 0.29 | 0.14 | 0.47 |
| Janbu | 0.36 | 0.04 | 0.12 |

Table 12 Measure of dispersion of Dynamic Formula of Ultimate Capacity of Pile Foundation

Figure 2: Percentage of  Qp/Qm based on the Dynamic Formulae Methods

The comparison between dynamic formula-based pile capacities and measured failure loads reveals a consistent trend of underestimation across all six methods analyzed. As shown in Table 10, the estimated capacities from Gates, Modified ENR, Danish, Navy–McKay, Eytelwein, and Janbu were generally lower than the measured failure loads. Among these, the Danish method exhibited the best predictive alignment, with a relative accuracy of −35.17% (Table 11), ranking first among the methods. This was followed by Navy–McKay (−42.91%) and Modified ENR (−44.82%), while Eytelwein and Janbu returned the least accurate predictions at −71.18% and −64.18%, respectively.

Figure 2 illustrates that all Qp/Qm ratios fell below the 100% line, confirming underprediction. Although conservative estimates are safer in practice, excessive deviation from actual failure loads may lead to overly cautious designs and increased construction costs. From a statistical reliability standpoint, Janbu had the lowest coefficient of variation (COV = 0.12), indicating consistent but low capacity estimates (Table 12). Conversely, Navy–McKay (COV = 0.48) and Eytelwein (COV = 0.47) showed the highest dispersion, highlighting variability in their application. Overall, while dynamic formulas provide quick field estimates, they should be supplemented with full-scale testing or site-specific calibration to improve reliability in coastal soils like Lagos.

# CONCLUSION

This study was conducted to assess the in-situ performance of driven precast concrete piles installed in the Ilubirin area of Lagos, Nigeria. The objectives were to evaluate ultimate pile capacities using static load tests, compare these with outcomes from commonly used predictive methods, analyze pile-soil interaction behavior, and identify the most reliable approaches for pile design in the region. From full-scale static load testing on 30 piles, the ultimate pile capacities ranged from 533 kN to 3727 kN, with measured settlements between 8 mm and 19 mm, and minimal rebound (0.00 to 0.13 mm), indicating stable performance under high loading conditions. This confirmed the effectiveness of driven piles in transferring load into competent bearing strata. In evaluating predictive methods, the Brinch Hansen method produced values closest to the average across all sites with a Qu/Quₐᵥₑ of 1.27, while Chin-Kondner and Decourt overestimated capacities (Qu/Quₐᵥₑ ≈ 1.50). The Tangent, Shen, and Abd Elsamee methods underpredicted capacity, with Abd Elsamee having the lowest average ratio of 0.38. Relative accuracy analysis ranked Tangent highest (−20.99% error) and Decourt lowest (69.51% error).

Among dynamic methods, the Danish method gave the best performance with a relative accuracy of −35.17%, while Eytelwein performed worst (−71.18%). All dynamic methods underestimated failure loads. Static analysis using API and UPD methods showed that API underpredicted by 30% (FOS ≈ 1.8), while UPD overpredicted by 100% (FOS ≈ 4.4). The API method, despite being conservative, aligns more closely with actual field behavior. Driven piles performed excellently in Lagos subsoils, particularly when tip embedment reached dense sand layers. The study recommends the Brinch Hansen and API methods—with appropriate safety factors—for preliminary designs, while full-scale load testing remains essential for verifying actual pile behavior in variable tropical soil conditions.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

REFERENCE

Adeleye, A. S., Olagunju, A. E., & Obalum, D. C. (2014). Soil properties and engineering behavior of Lagos marine clay. International Journal of Engineering Research and Applications, 4(6), 86–93.

Afolabi, O., & Agbede, I. (2018). A study of the geotechnical properties of some clay soils in Lagos State, Nigeria. Journal of Engineering Research and Reports, 5(2), 1–8.

Almeida, M. S. S., Fernando, A. B. D., & Lunne, T. (1996). Use of the piezocone test to predict the axial capacity of driven and jacked piles in clay. Canadian Geotechnical Journal, 33(1), 23–41.

Atkinson, J. (2008). Rules of thumb in geotechnical engineering. In Proceedings of the 18th NZGS Geotechnical Symposium on Soil-Structure Interaction (pp. 1-10).

Bajpayee, T. S., Hinshaw, G. A., & Randolph, D. (2008). The use of recycled steel in driven piles: Performance and environmental benefits. Journal of Materials in Civil Engineering, 20(5), 421–429. https://doi.org/10.1061/(ASCE)0899-1561(2008)20:5(421)

Broms, B. B. (1964). Lateral resistance of piles in cohesionless soils. Journal of Soil Mechanics and Foundations Div, 90(3), 27–63.

BS 8004. (1986). Code of practice for foundations. British Standards Institution.

Clarke, J., Long, M. M., & Hamilton, J. (1993). The axial tension test of an instrumented pile in overconsolidated clay at Tilbrook Grange. Proceedings of Large-Scale Pile Tests in Clay, 362–380.

Gavin, K., & Lehane, B. (2003). The shaft capacity of driven piles in sand. Canadian Geotechnical Journal, 40(1), 36–45.

Johnston, D. (2016). Green Guide to Specification: An Environmental Profiling System for Building Materials and Components. IHS BRE Press.

Khalaf, F. I., Gharib, I. M., & Al-Hashash, M. Z. (1984). Types of characteristics of the recent surface deposits of Kuwait, Arabian Gulf. Journal of Arid Environments, 7(2), 9–33.

Matsumoto, T., Michi, Y., & Hirano, T. (1995). Performance of axially loaded steel pipe piles driven in soft rock. Journal of Geotechnical Engineering, 121(4), 305–315.

Meyerhof, G. G. (1976). Bearing capacity and settlement of pile foundations. Journal of Geotechnical Engineering Division, 102(3), 195–228.

O'Brien, A. T., Belshaw, D. M., & Davies, P. (2004). Drilled shaft design and construction for large bridges. International Journal of Geoengineering Case Histories, 1(2), 45–60.

O’Neill, M. W., & Reese, L. C. (1999). Drilled Shaft: Construction Procedures and Design Methods. Federal Highway Administration.

Olagunju, A. E. (2019). Performance assessment of pile foundations in Lagos State. International Journal of Civil Engineering and Technology, 10(1), 178–189.

Olaoye, O. J., Ojo, J. A., & Adeyemo, A. A. (2014). An assessment of the geotechnical properties of soils in Lagos State for foundation design. Journal of Emerging Trends in Engineering and Applied Sciences, 5(5), 46–51.

Olayinka, A. I., Afolabi, O., & Adeyemo, A. (2019). Geotechnical properties of lateritic soils in Lagos State, Nigeria. Journal of Civil Engineering and Construction Technology, 10(5), 42–50.

Olugbenga, O. (2018). Assessment of pile foundation failures in Lagos, Nigeria. International Journal of Geotechnical Engineering, 12(1), 28–36.

Wang, Y., Wang, Z., & Zhao, D. (2017). Assessment of vibrations induced by pile driving and its impact on adjacent structures. International Journal of Structural Engineering, 15(3), 144–157.

Wroth, C. P. (1984). The design of pile foundations. In Piling: A practical guide (pp. 1–20). Thomas Telford.