AI-Based Tolerance Optimization for Cost Reduction

ABSTRACT

Manufacturing industries are under constant pressure to deliver high-quality products while simultaneously minimizing production costs. In the pursuit of achieving this balance, one of the most critical yet often overlooked factors is the proper selection of dimensional tolerances. Tolerances define the  allowable  variation  in  a  part’s  geometry  and  directly  influence  its  functionality,  assembly compatibility, and manufacturability. While tighter tolerances may enhance performance and precision, they inherently demand higher levels of machining accuracy, stricter quality control, and more advanced manufacturing processes — all of which contribute to a substantial increase in production costs.

Therefore, a key optimization challenge lies in identifying tolerance grades that are sufficiently tight to ensure mechanical performance and assembly integrity, yet not excessively stringent to inflate manufacturing expenses unnecessarily. This becomes especially important in components like shafts and hubs that require precise fits —  either clearance or  interference —  based on  their functional requirements.

To address this challenge, the present study investigates the use of Artificial Intelligence (AI), particularly Genetic Algorithms (GAs), as a decision-support tool for optimal tolerance selection. GAs are a class of evolutionary algorithms that mimic natural selection to iteratively evolve better solutions in complex, multi-objective  optimization  problems.  In  this  context, the  GA  is  used  to  explore  and  evaluate combinations  of International Tolerance  (IT) grades for both  shaft  and hub  components, considering both  functional fit constraints and associated cost indices.

The methodology adheres to the ISO  286  standard, which provides globally accepted guidelines for tolerance classification and limit fits. A Python-based simulation environment was developed to implement the GA, enabling the system to autonomously propose cost-effective tolerance combinations that ensure either minimum clearance or interference, depending on the application.

This AI-based approach introduces a paradigm shift in tolerance selection — transitioning from static, experience-based  decision-making  to  dynamic,  data-driven  optimization.  The  outcome  is  a more intelligent and scalable framework that supports engineers in achieving functionally robust and economically  viable designs, ultimately contributing to  leaner and more competitive  manufacturing operations.

Keywords: Tolerance optimization, Genetic algorithm, Manufacturing cost reduction, ISO 286,  Artificial

Intelligence, Shaft-hub fit, Dimensional tolerancing, Clearance and interference fit

1. INTRODUCTION

Tolerancing is a fundamental aspect of mechanical design that defines the permissible limits of variation in the dimensions of a component. These dimensional variations are critical as they directly influence the  assembly  fit,  functional  performance,  reliability,  and overall  manufacturability  of  the  product. Traditionally,  mechanical  engineers  rely  on  standardized  tolerance  charts, engineering  handbooks (such as ISO and ANSI fits), and their domain expertise to assign appropriate tolerance grades. While this manual approach is well-established and widely practiced, it often results in conservative decisions—engineers tend to specify unnecessarily tight tolerances to ensure functionality and safety margins (Siqueira et al., 2021; Raza, S., & Ding, 2019).  This  conservative  design  practice,  known   as over-engineering,  can significantly  escalate manufacturing complexity and cost, especially when high-precision machining or inspection techniques are involved.

Recognizing the limitations of traditional methods, this research proposes a novel, data-driven approach to  tolerance  optimization (Miah et al., 2022).  Specifically,  the  study leverages Artificial  Intelligence  (AI)  techniques— namely Genetic Algorithms (GAs)—to systematically determine the optimal tolerance grades for mating parts such as shafts and holes. Genetic Algorithms are adaptive heuristic search algorithms inspired by the principles of natural selection and genetics (Martins & Ning, 2021). They are particularly effective for solving complex, nonlinear,  and multi-objective  optimization  problems,  making  them suitable  for  tolerance  selection where trade-offs between fit, function, and cost must be considered.

The proposed AI-based framework evaluates a wide range of possible tolerance combinations and iteratively evolves toward an optimal solution. The objective function of the GA balances two conflicting goals: (1) ensuring that the resulting tolerances satisfy specific fit conditions (e.g., clearance, transition, or interference  fits),  and (2) minimizing  the associated  manufacturing  cost, which  typically  increases with tighter tolerances. By  encoding tolerance values into a population of  candidate solutions, the algorithm simulates the processes of selection, crossover, and mutation to explore the design space efficiently. The final output is a set of recommended tolerance grades that provide the desired functional fit at the lowest feasible cost.

This approach introduces a paradigm shift in mechanical design practice—from reliance on subjective decision-making to an intelligent, optimization-driven methodology. The integration of AI tools not only enhances design efficiency but also aligns with the broader goals of digital manufacturing and Industry

4.0 by enabling smarter, data-informed engineering decisions.

2. PROBLEM STATEMENT

While the ISO 286  standard provides a comprehensive framework for defining fit classifications—such as the commonly used H7/h6 for hole-shaft assemblies—it primarily serves as a reference for achieving desired geometric fits (clearance, transition, or interference) between mating parts (Ben Amor et al., 2021). However, one of the significant limitations of this standard is its static nature; it does not incorporate dynamic or contextual factors such as real-time manufacturing capabilities, material-specific machinability, process tolerance limits, or cost-performance trade-offs. As a result, tolerance decisions made purely based on ISO  286 charts often fail to reflect the practical realities of production environments.

In industrial practice, engineers frequently adopt overly conservative tolerances to guarantee assembly reliability and mitigate the risk of functional failure (Guo et al., 2023). While this safety-first approach ensures fit integrity, it inadvertently introduces several downstream issues in manufacturing:

	Increased   Machining   and  Inspection   Time:  Tighter   tolerances   require   more  precise machining operations, which are not  only slower but  also demand higher levels of process control. This often leads to extended cycle times and bottlenecks in production lines.

	Higher Rejection Rates:  Even minor  deviations  outside  narrow tolerance bands can render components unacceptable, leading to increased scrap rates, rework, and material wastage.

	Elevated Tooling and Labor Costs: Achieving and verifying precise tolerances necessitates specialized tooling, higher-quality raw materials, skilled labor, and advanced metrology equipment—all of which contribute to increased production costs.

Given these challenges, there is a compelling need for an intelligent, context-aware system that goes beyond traditional standards. Such a system should be capable of recommending tolerance grades that not  only  satisfy  the  required  functional  fit  but   also  take  into  account  manufacturability,  process capabilities,  and cost-effectiveness.  By  integrating  design  and manufacturing  considerations,  this approach can help  eliminate  unnecessary over-engineering  and support lean,  efficient  production workflows.

3. METHODOLOGY

3.1 System Input Parameters

To ensure that the tolerance optimization process is tailored to the specific engineering application, the system begins by accepting a set of user-defined input parameters that define the mechanical context and functional requirements of the fit. These include:

Nominal Diameter of the Shaft/Hub: This is the baseline size of the mating components, such as 50 mm, around which tolerance zones will be defined. The nominal size determines the appropriate range of IT (International Tolerance) grades according to ISO 286.

Fit  Type: The user specifies the desired type of fit between the components—either a clearance fit, where free relative motion is desired (e.g., rotating shafts), or an interference fit, where a tight, press-fit engagement is required (e.g., gears on shafts).

Material Selection: Materials such as steel, aluminum, or composites can influence manufacturability and achievable tolerance bands due to differences in machinability, thermal expansion, and structural rigidity.

Component Function: Functional context—such as whether the part is a high-speed rotating shaft, a structural press-fit joint, or  a sliding assembly—helps refine tolerance selection by  determining the acceptable range of clearance or interference.

This set of inputs ensures that the optimization algorithm operates within the boundaries of real-world design intent and manufacturing constraints.

3.2 Tolerance Data and Cost Model

The tolerance optimization process is grounded in the ISO 286  standard, which defines standardized IT grades that determine the width of tolerance zones for holes and shafts. As the precision of tolerances increases (i.e., lower IT grades), the associated cost of manufacturing also increases due to the need for tighter machining controls, specialized tooling, and increased inspection rigor.

The following table summarizes the tolerance width and associated cost index for selected IT grades: Table 1: Tolerance Grade Table

| Tolerance Grade (IT) | Tolerance Width (mm) | Cost Index |
| --- | --- | --- |
| IT6 | 0.010 | 5 |
| IT7 | 0.016 | 4 |
| IT8 | 0.025 | 3 |
| IT9 | 0.040 | 2 |
| IT10 | 0.063 | 1 |

Assumption:  The cost index  is  inversely  proportional  to the tolerance  width;  thus, tighter  tolerances (e.g., IT6) are associated with higher manufacturing costs due to increased machining precision and quality assurance requirements.

This cost model is embedded into the optimization algorithm, allowing it to weigh the trade-off between functionality (fit requirements) and economic efficiency (cost index).

3.3 Optimization Using Genetic Algorithm (GA)

A Genetic Algorithm (GA) is employed as the optimization engine due to its robustness in solving multi- objective, non-linear, and combinatorial problems. In this context, the GA explores the best combination of IT grades for the shaft and hub that achieves the specified fit type while minimizing the manufacturing cost.

Key Components of the GA:

Chromosome  Representation:  Each  individual  (or  chromosome) represents  a  potential  solution, encoded as a pair of IT grades—one for the shaft and one for the hub.  For  example, a chromosome could be (IT8 shaft, IT7 hub).

Population Size: The algorithm maintains a population of 20 individuals per generation.

Generations: The GA evolves the population over 30  generations to converge towards an optimal or near-optimal solution.

Fitness Function: The fitness of each individual is evaluated based on:

Whether the resulting fit  satisfies the functional requirement (i.e., maintains minimum clearance or interference).

The total  manufacturing cost, computed as the sum of the cost indices  of the selected IT grades for shaft and hub.

Constraint Conditions:

For clearance fits: a minimum clearance of 0.005 mm is required (i.e., Hub Lower – Shaft Upper ≥ 0.005 mm).

For interference fits: a minimum interference of 0.002 mm is required (i.e., Shaft Lower – Hub Upper ≥

0.002 mm).

Any  individual  violating  these constraints  is  assigned  a large  penalty  (e.g., infinite  fitness  value), effectively removing it from consideration.

Fig. 1: Housing, Bearing, Shaft Assembly

Drawing Side View

Fig. 2: Housing, Bearing, Shaft Assembly

Drawing Front  View

Fig. 3: Housing, Bearing, Shaft Assembly Model

4. GENETIC ALGORITHM  PROCESS

The GA follows a structured five-step iterative process:

Step 1: Initialization

A random initial population of 20 individuals is generated. Each individual is a unique pair of IT grades for the shaft and the hub,  sampled from the range IT6 to IT10.

Step 2: Fitness Evaluation

For each individual in the population:

The upper and lower dimensional limits for both  shaft and hub  are computed using the IT grades and the nominal diameter.

The resulting clearance or interference is calculated: Clearance = Hub Lower Limit – Shaft Upper Limit Interference = Shaft Lower Limit – Hub Upper Limit

If the clearance/interference meets the specified minimum threshold, the fitness is computed as: Fitness=Cost Index shaft + Cost Index hub

If constraints are violated, a penalty value is assigned, excluding the individual from selection.

Step 3: Selection

The top  50%  of individuals (those with valid fits and the lowest total cost) are selected as parents for the  next  generation.  This   selection   pressure  ensures   convergence  towards  cost-optimal   and functionally valid solutions.

Step 4: Crossover and Mutation

Genetic diversity is maintained through recombination:

Crossover: New individuals are created by randomly combining IT grades from two parent individuals.

Mutation: With a small probability (e.g., 5–10%), one of the IT grades in the chromosome is randomly replaced with a new value from the permissible range (IT6–IT10), allowing the algorithm to explore new regions of the solution space.

Step 5: Iteration

The newly formed population undergoes the same evaluation, selection, and reproduction process for

30   generations.  With  each  generation,  the  algorithm  converges toward a  solution  that  satisfies functional fit requirements while minimizing the associated manufacturing cost.

5. RESULTS AND INTERPRETATION

To validate the effectiveness of the proposed Genetic Algorithm-based tolerance optimization system, a representative sample case was evaluated. The input parameters were carefully chosen to reflect a typical mechanical engineering application involving a rotating shaft in a clearance fit scenario. The following input data was provided to the system:

Tolerance Recommendation System

Enter nominal shaft/hub diameter in mm (e.g., 50): 50

Enter fit type (clearance/interference): clearance

Enter the material used (e.g., steel, aluminum, etc.): steel

Enter the function of the component (e.g., rotating shaft, press-fit gear): rotating shaft

Based on ISO 286  standards and general engineering practice:

- Nominal Diameter: 50.0  mm

- Fit Type: Clearance

- Material: Steel

- Function: Rotating Shaft

Recommended Tolerance Grades:

- Shaft: h8

- Hub: H7

Reference: ISO 286-1:2010 and ISO 286-2:2010 for ISO system of limits and fits.

Recommended Tolerances (Based on Nominal Size):

Shaft Tolerance: h9

Hub Tolerance: H8

Optimized Tolerancing Result

Material: Steel

Function: Rotating Shaft

Shaft Tolerance Grade: IT6 (10.00 microns) Shaft Limits: +0.005 mm / -0.005 mm

Hub Tolerance Grade: IT10 (63.00 microns) Hub Limits: +0.032 mm / -0.032 mm

Total Manufacturing Cost Index: inf

Fit Type: Clearance — Functional constraints satisfied

Fig. 4: Input Data

Fig. 5: Recommended Output

6. BENEFITS OF THE PROPOSED SYSTEM

The  proposed  AI-based   tolerance   optimization   system  offers   several   practical   advantages  over traditional methods:

	Cost Reduction: By avoiding unnecessary tight tolerances, the system minimizes machining, inspection, and tooling costs, leading to more economical production.

	Automation: The system eliminates the need for manual look-ups and expert-driven decisions by automating the selection of tolerance grades based on functional and cost constraints.

	Flexibility: It accommodates various  design  scenarios—different  diameters,  materials  (e.g., steel, aluminum), and functional roles (e.g., rotating shafts, press-fits).

	Scalability: The framework  can be extended to  include  surface  finish,  geometric  tolerances (e.g., roundness), and manufacturing process constraints, making it adaptable for advanced design-for-manufacturing applications.

7. LIMITATIONS AND FUTURE  SCOPE

Table 2: Limitations and Future Improvement Table

| Limitation | Future Improvement |
| --- | --- |
| Limited to shaft-hub cylindrical fits | Extend to complex geometries and assemblies |
| Mock material properties | Integrate database for material-based tolerance selection |
| Static cost model | Dynamic cost estimation using real-time production data |
| No machine/process constraints | Include CNC capabilities, inspection limits |

8. CONCLUSION

This  study highlights  the effective  application  of Artificial Intelligence  (AI) techniques—specifically, Genetic  Algorithms  (GAs)—in  the domain  of  tolerance  selection  for  mechanical  assemblies.  The proposed system addresses  a long-standing  challenge  in  manufacturing  design:  how  to  balance functional fit requirements with the economic realities of production.

Traditional  reliance  on  standardized  tolerance  charts and conservative  engineering  judgment  often leads  to  over-engineered  solutions,  where tolerances  are tighter  than necessary.  This  results  in increased machining complexity, extended cycle times, and elevated production costs. By contrast, the AI-driven approach presented in this work enables intelligent, data-informed decision-making that aligns closely with both  design intent and manufacturability.

The genetic  algorithm  serves as  a powerful  optimization  engine  capable  of  navigating  the large combinatorial space of tolerance pairings (e.g., shaft and hub),  while considering constraints such as minimum clearance or interference, material selection, and process capability. The algorithm efficiently identifies tolerance combinations that satisfy all functional requirements without incurring unnecessary precision—and by extension, unnecessary cost.

By bridging the gap between theoretical ISO  tolerance standards and practical, cost-sensitive design, this system empowers engineers to implement leaner, smarter, and more sustainable manufacturing practices. Moreover, its flexible and scalable framework opens the door  for integration into CAD/CAM systems, product lifecycle management (PLM), and Industry 4.0 digital design workflows.

In summary, this study demonstrates the tangible value of AI-based optimization in mechanical design and manufacturing,  marking  a step forward  toward  fully  intelligent  and cost-effective  engineering decision support systems.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

9. REFERENCES

1.    ISO 286-1:2010 — Geometrical product specifications (GPS) — ISO code system for tolerances on linear sizes — Part 1: Basis of tolerances, deviations and fits.

2.    ISO 286-2:2010 — Geometrical product specifications (GPS) — ISO code system for tolerances on linear sizes — Part 2: Tables of standard tolerance classes.

3.    Goldberg, D.E.,  Genetic Algorithms in Search, Optimization and Machine Learning, Addison- Wesley, 1989.

4.    Qin, S., Liu, D., & Yu, H. (2021). "Intelligent Design Tolerancing in Industry 4.0", IEEE Access, vol. 9, pp. 44258–44269.

Gao, R., Wang, L., Teti, R., et al. (2015). "Cloud-enabled prognosis for manufacturing", CIRP Annals, vol. 64, no. 2, pp. 749–772.

5. Siqueira, B. R., Ferrari, F. C., Souza, K. E., Camargo, V. V., & de Lemos, R. (2021). Testing of adaptive and context‐aware systems: approaches and challenges. Software Testing, Verification and Reliability, 31(7), e1772.

6. Raza, S., & Ding, C. (2019). Progress in context-aware recommender systems—An overview. Computer Science Review, 31, 84-97.

7. Miah, M. H., Zhang, J., & Singh Chand, D. (2022). Knowledge creation and application of optimal tolerance distribution method for aircraft product assembly. Aircraft Engineering and Aerospace Technology, 94(3), 431-436.

8. Martins, J. R., & Ning, A. (2021). Engineering design optimization. Cambridge University Press.

9. Ben Amor, S., Zongo, F., Louhichi, B., Brailovski, V., & Tahan, A. (2021, August). Classification of dimensional deviation in additive manufacturing LPBF process for AlSi10Mg alloy according to ISO 286 and ANSI B4. 2. In International Design Engineering Technical Conferences and Computers and Information in Engineering Conference (Vol. 85376, p. V002T02A044). American Society of Mechanical Engineers.

10. Guo, F., Hou, Y., Xiao, Q., Zhang, X., Xiao, S., & Wang, Z. (2023). Reliability improvement on assembly accuracy with maximum out-of-tolerance probability analysis and prior precise repair optimization. Advanced Engineering Informatics, 55, 101866.

ABBREVIATIONS

AI: Artificial Intelligence

GA: Genetic Algorithm

IT: International Tolerance

ISO: International Organization for Standardization

PLM: Product Lifecycle Management

CNC: Computer Numerical Control

GPS: Geometrical Product Specifications

APPENDIX Tolerance info IT_grades = {

"IT6": 0.010, "IT7": 0.016, "IT8": 0.025, "IT9": 0.040, "IT10": 0.063

}

tolerance_cost = { "IT6": 5,

"IT7": 4, "IT8": 3, "IT9": 2, "IT10": 1

}

Input

print("--- Tolerance Recommendation System (Mock API Mode) ---")

part_diameter = float(input("Enter nominal shaft/hub diameter in mm (e.g., 50): "))

fit_type = input("Enter fit type (clearance/interference): ").lower()

material = input("Enter the material used (e.g., steel, aluminum, etc.): ").lower()

function = input("Enter the function of the component (e.g., rotating shaft, press-fit gear): ").lower()

ChatGPT Response chatgpt_recommendation = f"""

Based on ISO 286  standards and general engineering practice:

- Nominal Diameter: {part_diameter} mm

- Fit Type: {fit_type.title()}

- Material: {material.title()}

- Function: {function.title()}

Recommended Tolerance Grades:

- Shaft: h8

- Hub: H7

Reference: ISO 286-1:2010 and ISO 286-2:2010 for ISO system of limits and fits. """

print("\n--- ChatGPT Recommendation (Mocked) ---")

print(chatgpt_recommendation)

Initial Recommendation (Standard ISO-based Example)

def recommend_tolerance(diameter, fit):

if diameter < 30: shaft = "h7" hub  = "H7"

elif diameter < 50:

shaft = "h8" hub  = "H7"

else:

shaft = "h9" hub  = "H8"

return shaft, hub

rec_shaft, rec_hub = recommend_tolerance(part_diameter, fit_type) print("\nRecommended Tolerances (Based on Nominal Size):") print(f"Shaft Tolerance: {rec_shaft}")

print(f"Hub Tolerance: {rec_hub}")

Optimization Section min_clearance = 0.005 min_interference = 0.002 population_size = 20 generations = 30 mutation_rate = 0.2

def generate_individual():

return [random.choice(list(IT_grades.keys())), random.choice(list(IT_grades.keys()))]

def fitness(individual):

shaft_tol = IT_grades[individual[0]] hub_tol = IT_grades[individual[1]] shaft_upper = part_diameter + (shaft_tol / 2) shaft_lower = part_diameter - (shaft_tol / 2) hub_upper = part_diameter + (hub_tol / 2) hub_lower = part_diameter - (hub_tol / 2) clearance = hub_lower - shaft_upper interference = shaft_lower - hub_upper

if fit_type == "clearance" and clearance < min_clearance:

return float('inf')

elif fit_type == "interference" and interference < min_interference:

return float('inf')

total_cost = tolerance_cost[individual[0]] + tolerance_cost[individual[1]]

return total_cost

def select(population):

sorted_pop = sorted(population, key=fitness)

return sorted_pop[:int(len(sorted_pop) / 2)]

def crossover(parent1, parent2):

return [random.choice([parent1[0], parent2[0]]), random.choice([parent1[1], parent2[1]])]

def mutate(individual):

if random.random() < mutation_rate:

individual[random.randint(0, 1)] = random.choice(list(IT_grades.keys()))

return individual

population = [generate_individual() for _ in range(population_size)]

for _ in range(generations): selected = select(population) children = []

while len(children) < population_size: parent1 = random.choice(selected) parent2 = random.choice(selected) child = crossover(parent1, parent2) child = mutate(child) children.append(child)

population = children

best = min(population, key=fitness)

shaft_tol = best[0]

hub_tol = best[1]

shaft_val = IT_grades[shaft_tol] hub_val = IT_grades[hub_tol] total_cost = fitness(best)

shaft_upper = part_diameter + (shaft_val / 2) shaft_lower = part_diameter - (shaft_val / 2) hub_upper = part_diameter + (hub_val / 2) hub_lower = part_diameter - (hub_val / 2)

print("\n--- Optimized Tolerancing Result ---") print(f"Material: {material.title()}") print(f"Function: {function.title()}")

print(f"Shaft Tolerance Grade: {shaft_tol} ({shaft_val*1000:.2f} microns)")

print(f"   Shaft Limits: +{(shaft_val/2):.3f} mm / -{(shaft_val/2):.3f} mm")

print(f"Hub Tolerance Grade: {hub_tol} ({hub_val*1000:.2f} microns)") print(f"   Hub Limits: +{(hub_val/2):.3f} mm / -{(hub_val/2):.3f} mm") print(f"Total Manufacturing Cost Index: {total_cost}")

print(f"Fit Type: {fit_type.title()} — Functional constraints satisfied")