# Original Research Article

PSO-GWO-Based Adaptive Control for Bridge Cranes with Input Delay

ABSTRACT

As key equipment in the field of engineering construction and logistics transport, bridge cranes play an important role in material handling operations. The problem of load swinging is often encountered during operations. This phenomenon significantly affects work efficiency and may cause serious safety accidents. In addition, the inherent flexibility of the hoist rope introduces time delays in control inputs, further complicating anti-swing control. Therefore, this paper employs the Padé approximation to convert the delayed system of the bridge crane into a delay-free equivalent model, addressing the challenge of input time delays. Based on this equivalent model, an adaptive controller is designed to simultaneously achieve load anti-swing control and trolley positioning. Moreover, the controller performance is optimized through a hybrid particle swarm optimization-grey wolf optimizer (PSO-GWO) algorithm. Finally, the effectiveness of the control method is verified by digital experiments. The results indicate that the proposed control method improves both positioning accuracy and anti-swing response speed with modest control effort.

Keywords: Bridge crane; Input delay; Adaptive control; Padé approximation

1. INTRODUCTION

Bridge cranes serve as essential machinery in industrial material transport, extensively employed across diverse logistical settings including port operations, manufacturing facilities, and storage areas. The underactuated nature and highly coupled nonlinear dynamics of the bridge crane system pose significant challenges in achieving precise positioning and payload swing suppression. To solve these problems, researchers have developed several effective control methods, particularly model predictive control, adaptive sliding mode control, and intelligent optimization approaches. These methods aim to achieve rapid and accurate positioning, effectively reduce swing angle and ensure system stability. Such methods not only advance the control theory of underactuated systems, but also offer practical solutions for engineering applications. Li et al. [1] proposed an  output feedback control strategy for nonlinear bridge cranes based on Takagi-Sugeno fuzzy modelling. A fuzzy state estimator is designed to approximate non-measurable system states, while the virtual-desired synthesis approach converts the original trajectory tracking task into a stabilization problem. Simulation results achieve zero positioning error and complete elimination of residual load swing. Compared with model predictive control and standard  controller, the proposed approach achieves 50% and 67% faster convergence, respectively. Tan et al. [2] proposed a composite control framework of deep deterministic policy gradient (DDPG) algorithm with super-twisting extended state observer (STESO) for underdriven bridge crane systems. The controller design is simplified by constructing equivalent state variables and the finite time convergence estimation and compensation is achieved by STESO, which is combined with the DDPG algorithm to effectively eliminate the tracking error. Digital and physical experiments validate the hybrid control architecture's comprehensive capabilities in adaptive disturbance rejection and dynamic robustness enhancement. Li et al. [3] developed an accurate Lagrange model of a 7-degree-of-freedom bridge crane to design the first nonlinear closed-loop controller without linearization. Based on the energy analysis method, the friction uncertainty and actuator dynamics are effectively solved, while the synergistic control of bridge car displacement and cable length is realized. Experimental validation performed on a dedicated test platform confirms enhanced performance and improved robustness of the proposed control method compared to existing nonlinear control approaches. For crane systems with flatness properties, Kim et al. [4] developed an adaptive unscented Kalman filter (AUKF) to estimate unmodeled friction and unmeasurable states, utilizing condition-selective scaling to enhance filter convergence. Simulation experiments confirm that the method improves the accuracy of friction estimation and state observation. Nguyen et al. [5] combined the T-S descriptor fuzzy modelling with input shaping (IS) technique to design the tracking controller for variable cable length bridge cranes. The system nonlinearity is handled by fuzzy modelling, parallel distributed compensated integral control is used to improve the tracking performance, and the controller parameters are solved based on LMI. Simulations verify the effectiveness of the method in terms of perturbation suppression. Zhao et al. [6] developed an advanced control approach for 3D double-pendulum bridge crane systems. Adaptive tracking control without velocity sensing is achieved by composite displacement signal and mechanical energy function design. Experimental results demonstrate the satisfactory robustness and regulation precision of the control scheme when handling uncertain model parameters. Nishimoto et al. [7] design two-dimensional crane anti-swing controller based on positional command type, using a leaky integrator (leaky integrator) to achieve high-pass filtered integral damping of the swing angle. Effectively solves non-smooth signal tracking and actuator overshoot problems for human-machine collaboration and autonomous systems. Alyazidi [8] established an adaptive Kalman filtering framework to collect data and design a linear-quadratic optimal tracking controller for 3D crane systems with variable hoist rope lengths, wind disturbances and time-varying delays. Precise trajectory following of the system output to the reference input is attained by optimizing the reward function, with simulation tests confirming controller efficacy under time-varying delays. Zhao et al. [9] proposed a state feedback control strategy for bridge cranes based on a three-rule T-S fuzzy model to solve the input time lag and actuator saturation problem. The dynamic response is enhanced by designing a decay rate acceleration mechanism to establish time-lag dependent stability conditions. The controller parameters are solved by Linear Matrix Inequality (LMI) for accurate load positioning and pendulum suppression. The validity of the method under time-lag-saturation constraints is experimentally verified.

Existing studies have explored the anti-swing and anti-jamming performance of bridge cranes, but relatively few studies have addressed the input delay problem. Therefore, this paper addresses the delay problem generated by the crane due to the flexible characteristics of the hoist rope, uses the Padé approximation to approximate the delayed state, and designs an adaptive controller for the approximated equivalent delay-free model. Furthermore, control performance is enhanced by a hybrid particle swarm optimization-grey wolf optimizer (PSO-GWO) algorithm, while simulation experiments confirm the effectiveness of the control method.

METHODLOGY

2. Bridge crane system

As a critical piece of material handling equipment, the bridge crane is essential for load lifting and transportation in industrial manufacturing facilities, warehouses, and construction sites. Its structure mainly consists of bridge, trolley, hoist ropes and Lifting hook. However, during crane operation, the load may occur swing motions. Under certain conditions, this problem substantially impairs system performance, diminishing operational efficiency and potentially resulting in equipment failure or personnel hazards. In addition, control command transmission to the load exhibits time delays due to the inherent flexibility of the hoist ropes. The input delay phenomenon significantly degrades both the response speed and anti-disturbance performance of conventional control strategies, thereby exacerbating challenges in anti-swing control systems. Therefore, this section establishes an equivalent delay-free dynamic model for the input-delayed bridge crane system and analyzes the performance.

2.1 Bridge crane system model

The linear model of a 2D bridge crane can be expressed as

where  is the mass of the trolley and the bridge,  is the mass of the load,  is the displacement of the trolley,  is the driving force applied by the motor to the trolley through the transmission mechanism,  is the coefficient of friction,  is the length of the hoist rope,  is the swing angle of the load,  is the acceleration of gravity.

The elastic deformation of hoist ropes generates inherent time delays between control input and load dynamic response in bridge crane systems. The state vector is taken as  and the control input is taken as , then the system state space equation is

where  is the delay time, with

, , .

This input delay characteristic will significantly reduce the stability margin of the control system and greatly increase the difficulty and complexity of the controller design. To address the control challenge posed by time delay, this paper employs the Padé approximation method to convert the transcendental delay function into a rational fraction representation. This transformation allows the original transcendental terms describing the delay dynamics to be expressed as finite-dimensional polynomial ratios. Following the methodologies in [10] and [11], the transfer function of the time-delay element can be expressed as

Using a first-order Padé approximation, the time-delay element can be approximated as

where  is a delay-free instruction for the controller,  is the input after the approximate time delay.

Then,  can be written as

Defining a state variable and performing the inverse Laplace transform yields the following relation

Substituting  into  yields the state-space representation of the approximated time-delay element

Finally, using the Padé approximation method,  can be approximated as .

Therefore, by substituting  into , the equivalent delay-free model of the system is obtained as

with , , .

This method converts the original time-delay system into an equivalent augmented system without containing explicit delay components. The controller designed for this augmented system ensures the control input  achieves equivalent performance to the original delayed control input . This method employs a unified control framework to circumvent the challenges of direct time-delay term processing, achieving coordinated control of the system's primary and delayed dynamics.

2.2 Controllability and observability of the system

To ensure the feasibility of the controller design, this section verifies the controllability and observability of this equivalent model. According to the theory of linear systems, the canonicality of a system is determined by the rank of the canonicality matrix . If  ( is the system state dimension), then the system is fully controllable. The system is fully observable if the rank of the observability matrix  is equal to .

For the delay-free equivalent model established in this paper, both theoretical analysis and numerical verification confirm that the state-space representation satisfies  in the system dynamics. Therefore, the system is both fully controllable and observable in the neighborhood of the equilibrium point.

3. Controller Design

3.1 Design of Adaptive Controller

Adaptive control is an intelligent control strategy that can adjust its control parameters online according to the actual operating state of the system [12]. Its core theoretical advantage lies in strictly ensuring the global stability of the closed-loop system and dynamically adjusting the key parameters based on the tracking error. In this section, an adaptive controller is designed for the bridge crane system, with demonstrated capabilities in parameter self-tuning and Lyapunov stability. The controller simultaneously ensures accurate target positioning and load anti-swing control. The design methodology of the proposed controller is as follows.

The state space equation for the reference model is

For system , the controller is designed to ensure the system state  tracks the reference state . The output of the controller is

where  is the feedback matrix and  is the feedforward gain matrix.

Substituting  into , the system state equation can be expressed as

The generalized state error is defined as

The time derivative of the error is given by

When the plant dynamics coincide with those of the reference model, the adaptive parameters converge to their ideal values , .

Substituting  into  yields

with , .

The Lyapunov function is chosen as

where ,  and  are positive definite symmetric matrices.

The time derivative of  is given by

with , , , .

The adaptive law is designed as

This adaptive law ensures , thereby guaranteeing asymptotic system stability through the controller design method.

3.2 Optimization of the control parameter

In conventional adaptive controller design, the selection of the weight matrix  typically relies on empirical tuning by designers. This approach is not only time-consuming but also fails to guarantee optimal control performance, potentially compromising both convergence rate and system robustness. To solve this problem, this paper uses PSO-GWO hybrid algorithm to determine . The PSO-GWO algorithm combines the global search capability of PSO with the local fine-tuning advantages of GWO to perform efficient search in complex, non-convex parameter spaces [13]. Therefore, in this paper, the PSO-GWO algorithm proposed in [14] is used to indirectly adjust the dynamic characteristics of the adaptive law by using the weight matrix  as the optimization objective, thus improving the system performance. The flowchart of the algorithm is as follows

Fig. 1. The flowchart of PSO-GWO algorithm

Firstly, the position update coefficient  and the random perturbation coefficient  are calculated

where  is the convergence factor and ,  are random numbers in the range .

The relative position of the current individual to the leader of the pack is then calculated to guide the direction of subsequent movements

where  is the inertia weight coefficient.

Updating the velocity and position of the particle, the update equation is

where , , and  correspond to the positions of , , and  wolves in the current iteration, respectively.

The fitness function of the algorithm is:

where  and  are the weight coefficients.

The fitness function enhances tracking precision by minimizing time-weighted squared error and control input energy. The time-squared error term imposes a stronger penalty for persistent errors in the later stages of the response, both accelerating system convergence and significantly reducing steady-state errors, while the control energy term constrains the amplitude of the input signal, preventing the actuator from saturating while reducing energy consumption. Selecting this fitness function can balance the relationship between tracking error and control volume by adjusting the weight coefficients  and , which is convenient to adjust the optimization direction according to the actual engineering requirements, and finally get the controller parameters with better comprehensive performance.

The control block diagram of the whole closed-loop system is as follows

Fig. 2. Control block diagram of the proposed system

By combining the PSO-GWO hybrid algorithm with an adaptive control algorithm, the weighting matrix  was optimized, leading to a significant improvement in the system's control performance. This approach not only avoids the subjectivity and complexity inherent in traditional manual parameter tuning, but also proposes an efficient optimization methodology for designing critical parameters in model reference adaptive control systems, showing both practical utility and research significance. Meanwhile, the proposed algorithm maintains the positive definiteness of , thereby ensuring . Therefore, the system is asymptotically stable.

4. results and discussion

This section verifies the effectiveness of the proposed control method through digital experiments. The values of each parameter of the system during the experiments are as follows: , , , , , . The change curves of the displacement and swing angle of the trolley are shown in Fig 3. The change curves of control signal during the control process are shown in Fig 4.

(a)                                                                     (b)

Fig. 3. The variation curves of trolley position and swing angle

(a)                                            (b)                                              (c)

Fig. 4. Control signal curves of different controllers: (a) Adaptive control, (b) PSO-based adaptive control, (c) PSO-GWO-based adaptive control.

The result curves demonstrate that the proposed control method effectively achieves both positioning and anti-swing control for the bridge crane system with input delays. Compared to other control methods, the PSO-GWO-based adaptive controller enables the trolley to reach the specified position precisely within 4 seconds. At the same time, the swing angle of the load is able to converge quickly, converging to 0 degree within 6 seconds and maintaining in a stable state. Therefore, this method improves the dynamic response efficiency and positioning accuracy of the system.

Fig. 4(a) shows the control signal curve from the classical adaptive control strategy. This control signal variations degrade positioning accuracy, extend stabilization duration, and promote wear in mechanical components. Fig. 4(b) presents the control signal curve from the PSO-based adaptive control strategy. Compared with classical adaptive control, this strategy generates control signals with higher amplitude and a smoother curve. Such characteristics enable precise positioning, reduce stabilization time, and decrease wear on actuators like motors. Fig. 4(c) displays the PSO-GWO-based adaptive control signal curve.

Compared with the two preceding approaches, the control signal generated by this control method not only has a moderate amplitude, but also exhibits enhanced smoothness in its variation process. This directly reduces the frequency of action and regulation of actuators such as drive motors, effectively reduces the energy loss of the system and the mechanical wear caused by large-scale regulation, and has a certain degree of engineering practicality.

5. Conclusion

In this paper, the control problem of accurate positioning of the trolley and efficient suppression of the load swing angle is studied in depth for the bridge crane system with input delay characteristics. Firstly, modelling and transformations are carried out using the Padé approximation to establish a delay-free equivalent model of the system and analyze its controllability and observability. Then, an adaptive controller based on PSO-GWO algorithm is designed. This controller is able to adjust the control parameters by using the powerful global search and optimization capability of PSO-GWO. The method not only significantly saves the time spent on manual parameter selection in conventional methods, but also ensures better control performance through its global optimization seeking capability. Finally, the effectiveness of the proposed method is fully verified by digital experiments. Compared with conventional methods, the proposed controller achieves precise trolley positioning within 4 seconds. Simultaneously, it ensures rapid swing angle convergence, stabilizing at zero degrees within 6 seconds. In addition, the control signal exhibits smaller amplitude and smoother variations.

Although this paper verifies the control performance through digital experiments, physical experiments are still required, particularly to verify the control method's stability under harsh conditions including load disturbances, environmental disturbances and parameter uncertainties. This complete validation will demonstrate practical potential.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References

- Li, C., Xia, Y., & Wang, W. (2021).  Output-Feedback Anti-Swing Control for a Nonlinear Overhead Crane System With Disturbances Based on T-S Fuzzy Model. IEEE Access, 9, 135571-135584.

- Tan, P., Liu. J., Sun, M., & Chen, Z. (2025). Disturbance Compensation-Based Deep Reinforcement Learning Control Strategy for Underactuated Overhead Crane Systems: Design and Experiments. IEEE Transactions on Intelligent Transportation Systems, 26(4), 4380-4390.

- Li, G., Ma, X., & Li, Y. (2024). Adaptive Anti-Swing Control for 7-DOF Overhead Crane With Double Spherical Pendulum and Varying Cable Length. IEEE Transactions on Automation Science and Engineering, 21(4), 5240-5251.

- Kim, J., Kiss, B., Kim, D., & Lee, D. (2021). Tracking Control of Overhead Crane Using Output Feedback With Adaptive Unscented Kalman Filter and Condition-Based Selective Scaling. IEEE Access, 9, 108628-108639.

- Nguyen, H. -P., Bui, N. -T. & Nguyen, T. -v. -A. (2024). Tracking Control Based on Takagi-Sugeno Fuzzy Descriptor Model for Overhead Crane Combined With Input Shaping. IEEE Access, 12, 127507-127521.

- Zhao, B., Ouyang, H., & Iwasaki, M. (2022). Motion Trajectory Tracking and Sway Reduction for Double-Pendulum Overhead Cranes Using Improved Adaptive Control Without Velocity Feedback. IEEE/ASME Transactions on Mechatronics, 27(5), 3648-3659.

- Nishimoto, R., & Kikuuwe, R. (2023). Position-Commanding Anti-Sway Controller for 2-D Overhead Cranes Under Velocity and Acceleration Constraints. IEEE Access, 11, 35069-35079.

- Alyazidi, N. M. (2023). Optimal Tracking Control for 3D Overhead Crane with Varying Time Delay and Windy Disturbance. 2023 20th International Multi-Conference on Systems, Signals & Devices (SSD), Mahdia, Tunisia, 2023, 844-849.

- Zhao, Y., & Gao, H. (2012). Fuzzy-Model-Based Control of an Overhead Crane With Input Delay and Actuator Saturation. IEEE Transactions on Fuzzy Systems, 20(1), 181-186.

- Niu, X., Ye, H., Liu, Y., & Liu, X. (2013). Padé approximation based method for computation of eigenvalues for time delay power system. 2013 48th International Universities' Power Engineering Conference (UPEC), Dublin, Ireland, 2013, 1-4.

- Sharma, D., Mishra, S., & Firdaus, A. (2019). Computation of Delay Margin in a Power System Having Open Channel Communication Based Automatic Generation Control using Padé Approximation. 2019 IEEE International Conference on Environment and Electrical Engineering and 2019 IEEE Industrial and Commercial Power Systems Europe (EEEIC / I&CPS Europe), Genova, Italy, 2019, 1-6.

- Ioannou, P. A., & Sun, J. (1995). Robust adaptive control. Englewood Cliffs, NJ: Prentice Hall.

- Mahapatra, S., Badi, M., & Raj, S. (2019). Implementation of PSO, it’s variants and Hybrid GWO-PSO for improving Reactive Power Planning. 2019 Global Conference for Advancement in Technology (GCAT), Bangalore, India, 2019, 1-6.

- Li, Z., Zuo, J., Lv, R., Liu, Z., & Wang, Y. (2021). A Hybrid Multi-group Co-evolution Intelligent Optimization Algorithm: PSO-GWO. 2021 IEEE International Conference on Electrical Engineering and Mechatronics Technology (ICEEMT), Qingdao, China, 2021, 607-611.