Ameliorative Effects of Nano Zinc Oxide on Spent Engine Oil-Polluted Soil and Growth Performance of Ficus benjamina

Abstract

Background: Spent engine oil affects soil physico-chemical parameters and plant productivity but the interactive effects of n-ZnO  in soils polluted with SEO and impacts on growth performance of plants in SEO polluted soil have not been fully explored.

Aim: The study aims to assess the effects of  n-ZnO on physicochemical properties of soil polluted with spent engine oil and the growth performance of F. benjamina.

Materials and Methods: A 12-month Screen house experiment was conducted in the Institute of Ecology and Environmental Studies, Obafemi Awolowo University, Ile Ife, Nigeria using sandy loam soil treated with six levels of SEO (0, 0.5, 1.0, 1.5, 2.0, and 2.5 L/kg) with and without 50 ppm n-ZnO arranged in a completely randomized design Pre- and post-harvest soil physicochemical parameters (pH, total nitrogen, organic carbon, phosphorus, exchangeable bases, cation exchange capacity, and moisture content) were analyzed using standard methods. Plant growth parameters such as height and number of leaves were monitored bi-weekly. Data were analyzed using appropriate statistical methods.

Results: Results indicated that SEO contamination significantly reduced soil pH, phosphorus availability, exchangeable bases, and moisture content, but  increased the total nitrogen and organic carbon due to hydrocarbon residues. High concentrations of SEO (≥2.0 L/kg) led to stunted growth and causing up to 100% plant mortality. At 0.5 L/kg SEO without amendment, F. benjamina showed a 20.8% reduction in height and a 23.9% reduction in leaf number compared to the control. However, the application of 50 ppm n-ZnO mitigated these adverse effects. Notably, F. benjamina grown in 0.5 L/kg SEO amended with n-ZnO  increased the plant  height by 16.1%  and number of leaves by 6.7%, while  at 1.0 and 1.5 L/kg SEO, n-ZnO amendment led to increases in height by 19.8% and 0.7%, and in leaf number by 21.8% and 37.8%  respectively in relation to the plants in the SEO unamended  soils. The observed ameliorative effects of n-ZnO were attributed to its high reactivity, increased nutrient bioavailability, and stimulation of  soil microbial activity, which improved soil fertility and plant resilience.

Conclusion: This study concludes that n-ZnO is a promising amendment for remediating SEO-contaminated soils and enhancing plant growth. The findings underscore the potential of integrating nanotechnology into sustainable environmental management practices. Further studies are recommended to assess long-term ecological impacts and the potential bioaccumulation of nanoparticles in plants under field conditions

Keywords: Spent engine oil, Nano zinc oxide; soil physicochemical properties; Soil amendment, phytotoxicity;  Ficus benjamina.

- INTRODUCTION

Soil ecosystem is an essential component of life and man depends on it for food and natural resources while plants depend on it for their growth (Adewole and Uchegbu, 2010, Akpan et al., 2024).  However, the pressure on developing countries to keep space with the global industrial revolution has led to a sharp increase in the importation of used vehicles (commonly referred as  ‘Tukunbo’), engines and heavy duty machinery  (Nwosu et al., 2008). This has resulted in proliferation of auto-mechanic workshops across urban and peri-urban areas with attendant effects including the release of a wide range of organic and inorganic pollutants into the soil ecosystem through the indiscriminate discharge of Spent Engine Oil (SEO) into the environment. with attendant effects on soil health (Gautam et al., 2012).

The term spent oil is used to describe engine oils, transmission oils, and industrial oils (hydraulic and cutting oils) after use (Rauckyte et al., 2006).  This oil has been used for its intended purpose and is contaminated by physical or chemical contaminants such as iron, steel, copper, zinc, lead, barium, cadmium, sulfur, dirt particles and ash that enter the oil as the engine wears (Okebalama et al., 2024). Spent oil has a mixture of various chemicals such as aliphatic hydrocarbons, aromatic hydrocarbons, polychlorinated biphenyls, chlorodibenzofurans, lubricating additives, decomposition products and heavy metal contaminants that come from engine parts as they wear out (Adedokun and Ataga, 2007; Nwite and Alu, 2015; Umana et al., 2016).

Upon entering the soil, the interaction of the SEO with the soil may alter its  properties due to attachment or adsorption to soil components, which can affect soil ecosystem (Vu et al., 2015; Vu and Mulligan, 2023). These pollutants accumulate in the soil, leading to long-term contamination which may affect soil health and plant productivity (Shahzad et al., 2024).  Moreover, contamination of surface soil alters its chemical properties, thereby affecting soil–plant interactions, water retention, soil atmosphere, and the bioavailability of soil nutrients  and by extension, affecting soil health (Nwankwo et al., 2025).

Soils polluted with SEO affect soil health due to poor aeration, immobilization of soil nutrients and lowering of soil pH (McGrath et al., 1995; Okebalama et al., 2024). This is because oil in soil creates unsatisfactory conditions for plant growth, probably due to insufficient aeration of the soil, by displacement of air from spore spaces by oil and increase the demand for oxygen by the activities of oil-decomposing micro-organisms (Gudin and Syrat, 1975; Rowell, 1977; De Jong, 1980; Odjegba and Sadiq 2002).

Several studies have shown that spent engine oil in soil affect soil physicochemical properties (Chijioke and chinenye, 2016; Muhammad et al., 2016; Nwite and Alu, 2015; Okonokhua et al., 2007; Tanimu, et al., 2019). However, the effects of spent oil on soil physicochemical properties depend to a large extent on the levels and concentrations of the oil.

Studies have also shown that SEO in the soil affect plant germination, growth, flowering and fruitings, however dependent on plant species and the concentration of SEO. According to Adenipekun et al. (2008), spent engine oil at all concentrations delayed the germination of Corchorus olitorius by 2 days compared to control. Also, both the number of leaves and plant height decreased with increasing concentration  of SEO.  Osuagwu and Nwofia (2014) and Osuagwu and Iwuoha (2015) attributed the effects of SEO on poor germination and growth rates of plants to volatile fraction of oil which has high wetting capacity and penetrating power; to enter the seed coat, deprieve the seed of water and oxygen, thereby destroying the embryo. Also, as oil contaminated soil samples become compact, it leads to poor wetting ability, poor water content, poor aeration and increase in the amount of toxic substances which result in decreased in germination and growth of plants (Agbogidi and Etuotor, 2012). Vegetative growth reduction and low biomass could be attributed to the total effect of oil on the stomata number per unit area of the leaf which resulted to decrease in the photosynthetic products (Agbogidi and Ilondu, 2013). Spent oil in the soil-plant micro environment affects soil chemistry including nutrient release and reduction in the amount of water available for plant (Osuagwu et al., 2017).

Nanotechnology is an emerging technology in the field of environmental management and agriculture, manipulating materials at the nanoscale, with at least one dimension less than 100 nanometers for diverse applications and these engineered nanoparticles (ENPs) are increasingly used in agriculture and environmental management (Akpan  et al., 2025). Studies have revealed that ENPs have effects of the physio-chemical properties of the soil (Vu and Mulligan, 2023; Ogunneye et al., 2014; Akpan et al., 2025).

Zinc oxide nanoparticle are among the most commonly used ENPs. It has a wide range of applications, including in sunscreen products, paint, and nano-fertilizers (Sharifan and Ma, 2017). Zinc oxide nanoparticles are considered to be effective in slow release of bioavailable zinc in soil due to their enhanced reactivity as compared to their bulk counterparts (Deka, 2019). Studies reveal that n-ZnO reduce the toxicity of heavỷ metals in plants. Raliyd et al. (2015) reported that co-presence of n-ZnO showed significant increase in shoot and root growth as well as biomass rate in Solanum  lycopersicum. According to Priyanka et al. (2021),  ZnO nanoparticles play a significant role in enhạncing the seedlings growth characteristics via regulation of Cd and Pb metal tolerance potential by reduction of oxidative stress as well as increase in photosynthetic pigment contents of plants under heavy metals stress.

Ficus benjamina (weeping fig)  is one of the most beautiful and common park ornamental plants belonging to the family of Moraceae which is generally broader and tall with a beautiful dense umbrella-like canopy of pendant branches that cascade down to the ground (Akpan et al., 2025). Some previous studies reports the medicinal properties of F. benjamina for antimicrobial, pro-wound healing, anti-inflammatory and effects on cancer cell lines proliferation (Dahan et al., 2025). According to Prasad et al. (1999) and  Pilon-Smits, (2005)  F. benjamina has a tendency to tolerate certain pollutants such as heavy metals. Application of n-ZnO may probably enhance phytoremediation capability of organic and inorganic compounds.  Ficus benjamina can host a range of endophytic and rhizospheric microorganisms, which may play a role in its phytoremediation potential in the rhizospheric degradation (Lugtenberg and Kamilova, 2009).

Studies  show that n-ZnO  improves soil fertility and enhances plant growth (Akpan et al., 2025), but there is still dearth of information and understanding of the interactive effects of n-ZnO  in soils polluted with SEO and impacts on growth performance of Ficus benjamina in SEO polluted soil. Given these concerns, the present study investigates the effects of  n-ZnO in physicochemical properties of soil polluted with SEO and the growth performance of F. benjamina.

2.0	MATERIALS AND METHODS

Nano Zinc Oxide (ZnO) Water Dispersion was purchased from LIWEI Nano tech Co., Ltd,. Kaohsiung City, Taiwan (mean particle size of 30 nm) and Ficus benjamina seedlings were raised at Parks and Garden Unit, Obafemi Awolowo University, Ile Ife, Nigeria. Spent engine oil was obtained from auto-mechanic workshops in Ile-Ife metropolis.

2.1	Screenhouse Experimental Set-up

Screenhouse experiment was conducted at the Institute of Ecology and Environmental Studies, Obafemi Awolowo University, Ile-Ife, Nigeria.  Screenhouse experiment involved twelve months in  spent engine oil (SEO) contaminated soils amended with n-ZnO cultivated with Ficus benjamina. The established tolerable concentration of n-ZnO (50 ppm) (Akpan et al., 2025) was used as soil amendment in SEO contaminated soils. Thirty six (36) plastic pots were filled with 5 kg of air-dried soil samples from relatively undisturbed natural vegetation. Six different concentrations (0, 0.5, 1.0, 1.5, 2.0, 2.5 L/kg) of SEO were introduced into the pots containing the soil samples in triplicates. Eighteen (18) pots containing the SEO were then amended with the established 50 ppm of n-ZnO.  The amended soil samples were watered and left for two weeks for equilibration. Three-week-old seedlings of Ficus benjamina were transplanted into treated soils. The experiment was set up using a Complete Randomized Design (CRD) and was monitored for 12 months. Plant height and number of leaves were measured fortnightly with effect from second week after planting. At the termination of the experiment, plants were harvested while post-harvest soil analysis was carried out using standard methods.

2.2	Determination of Physiochemical Parameters of Experimental Soil Samples

All soil samples were air-dried at room before being ground and sieved through a 2 mm stainless steel sieve for the determination of various parameters. Soil samples from each treatment were analyzed for their physicochemical properties.

Soil pH was determined using the ASTM D 4972 method (ASTM, 2013) using a calibrated pH meter. Soil particle size distribution was determined using the Bouyoucos hydrometer method (Rowell, 2014). Cation Exchange Capacity (CEC) was determined following the USEPA (2000) method. Exchangeable cation was analyzed using a flame atomic absorption spectrometer. Total nitrogen was analyzed using the alkaline permanganate method (Subbiah & Asija, 1956). Organic carbon was determined using the Walkley and Black (1934) wet oxidation method. Soil moisture content was determined using the gravimetric method (ASTM, 2016). Available phosphorus was measured using the Olsen et al. (1954) method. The transmittance was measured at 660 nm using a spectrophotometer.

2.3	Statistical Analysis

All data collected were subjected to normality testing using the Shapiro-Wilk test. Data sets that met the assumptions of normality were analyzed using one-way analysis of variance (ANOVA). Differences between treatment means were assessed using Duncan’s Multiple Range Test (DMRT) at a significance level of p ≤ 0.05

3.0	RESULTS AND DISCUSSION

3.1	Physical and Chemical Parameters of Experimental Soil before Contamination

The baseline physical and chemical properties of the experimental soil were analyzed before the experiment to ascertain its suitability for supporting plant growth and to provide reference values for post-experiment comparisons as presented in Table 1. The soil was classified as sandy loam, based on its particle size distribution (sand: 76%, silt: 10%, clay: 14%). This texture is generally favorable for plant growth due to its balanced water retention and drainage properties. Nwite and Alu (2015) and Akpan et al. (2025) reported similar textural classes in comparable agro-ecological settings.

The soil pH measured 6.5, falls within the optimal range (6.0–7.0) for most agricultural crops (Nichols, 2022). This slightly acidic condition supports the availability of essential nutrients and optimal microbial activity. The organic carbon content was 2.6%, indicative of moderate soil fertility, with implications for microbial biomass, aggregate stability, and cation exchange dynamics (Lefèvre et al., 2017). While this value suggests some organic enrichment, it remains slightly below levels typically associated with highly fertile soils. The total nitrogen content was 0.26%, which is within the moderate range for most cultivated soils adequate for supporting early vegetative growth. The available phosphorus content was 70.3 mg/kg, significantly above the critical threshold of 10 mg/kg required for optimal crop growth (Enwezor et al., 1982; FAO, 1976). This suggests that phosphorus availability was not a limiting factor in this soil.

Soil moisture content was 42.66% at the time of sampling, indicating suitable water availability for seed germination and initial plant establishment under controlled or field conditions. The soil’s cation exchange capacity (CEC) was 12.87 Cmol/kg, indicating moderate nutrient retention capacity typical of sandy loam soils. Among the exchangeable bases, calcium (5.6 Cmol/kg) and magnesium (6.15 Cmol/kg) were dominant, reflecting a fertile soil profile. Potassium and sodium were measured at 0.31 Cmol/kg and 0.18 Cmol/kg, respectively. These values are characteristic of productive, unpolluted soils, as reported by Sesan et al. (2013). The dominance of calcium and magnesium in the exchange complex also supports plant vigor and physiological function, particularly in cell wall stability and chlorophyll synthesis.

Although the organic carbon and total nitrogen levels could be considered moderately low relative to temperate soils, they are within the expected range for tropical sandy loam soils and comparable to those reported in similar ecological contexts (Landon, 1991; Nwite & Alu, 2015). The overall fertility status, texture, and base saturation indicate that the soil is suitable for plant growth and experimental evaluation. Comparable soil properties have been used effectively in related studies by Amin (2011), Sesan et al. (2013), and Akpan et al. (2025), further affirming the suitability of the soil for the present investigation.

3.2	Physical and Chemical Properties of Screen-house Soils after Harvest

Table 2 presents the post-harvest physicochemical properties of soils treated with various concentrations of spent engine oil (SEO) and SEO amended with nano zinc oxide (n-ZnO). Overall, the results demonstrate the interactive effects of SEO contamination and n-ZnO amendment on soil texture, nutrient dynamics, and moisture retention in relation to the growth performance of Ficus benjamina.

Table 1:  Physical and chemical properties of soil before planting

| Soil Properties | Values |
| --- | --- |
| Soil pH | 6.5 ±0.10 |
| Organic Carbon (%) | 3.6 ±0.00 |
| Sand (%) | 68 ±0.00 |
| Silt (%) | 10 ±0.00 |
| Clay (%) | 14 ±0.00 |
| Total N (%) | 0.26 ±0.20 |
| Available P (mg/kg) | 70.3 ±0.20 |
| Moisture (%) | 42.6 ±0.10 |
| K (Cmol/kg) | 0.31 ±0.13 |
| Mg (Cmol/kg) | 6.15 ±0.00 |
| Na (Cmol/kg) | 0.18 ±0.12 |
| Ca (Cmol/kg) | 5.6 ±0.10 |

Soil texture remained relatively stable across all treatments. The sand fraction in uncontaminated soils was 75.0%, closely matching the pre-planting value of 76%, confirming the sandy loam classification. SEO-contaminated soils without n-ZnO showed a decline in sand content (70–72%) and a corresponding increase in clay (up to 17%), suggesting potential compaction and structural changes caused by SEO. In contrast, n-ZnO-amended soils retained higher sand content (75–78%) and lower clay levels (12–15%), indicating that n-ZnO mitigated the texture-altering effects of SEO. The results suggest that SEO  treatments may have influenced particle size redistribution but with no effects on the overall soil texture. These observations are consistent with findings by Adesodun (2004), Okonokhua et al. (2007) and Nwite et al. (2020), who noted that short-term petroleum hydrocarbon contamination does not significantly alter soil texture due to its dependence on parent material. Akpan et al. (2025) emphasised that soil texture is mainly influenced by parent material and not easily affected by short-term n-ZnO exposure.

Total nitrogen (TN) increased significantly (p < 0.05) in both SEO and SEO+n-ZnO-treated soils. The control soil recorded a TN of 0.57%, whereas SEO-only soils ranged from 1.35% to 3.36%. The highest TN value (3.64%) was observed in SEO+n-ZnO-treated soil at 1.5 L/kg SEO. However, nitrogen levels declined at higher SEO concentrations (2.0–2.5 L/kg), possibly indicating microbial inhibition or nutrient immobilization at elevated contamination levels. Similarly, organic carbon (OC) content increased from 3.30% in control to 5.63% in SEO-only and 6.98% in SEO+n-ZnO-treated soils. These increases may result from the hydrocarbon-rich nature of SEO, which introduces organic residues that temporarily boost microbial activity and carbon inputs. These findings align with previous studies by Osaigbovo et al. (2013), Iren and Ediene (2017), Nwite and Alu (2015). The authors reported elevated organic carbon and nitrogen levels in soils contaminated with SEO. Chijioke and Chinenye (2016) attributed such increases in OC to the hydrocarbon content of spent engine oil.  Okonokhua et al. (2007) reported that the elevated levels of  C and N  of the SEO contaminated soils  result from crude oil, from which the engine oil is produced, which contains principal elements such as oxygen, nitrogen and sulphur other than hydrogen and carbon.  However, the present findings contrast with reports by Kayode et al. (2009) and Devatha et al. (2019), who documented reductions in nitrogen and carbon content at higher levels of SEO contamination. This discrepancy may reflect differences in soil type, climatic conditions, microbial community structure, or the duration of exposure in various studies. The enhanced total nitrogen and organic carbon observed in this study may thus be due to the input of organic residues from the oil, which can temporarily increase microbial biomass and organic matter content.

Furthermore, the application of n-ZnO consistently improved both nitrogen and carbon concentrations in the soil across most treatments, possibly due to its catalytic properties and increased nutrient availability through enhanced microbial activity. According to Al Momani et al. (2024) and Akpan et al. (2025), TN and OC increased at lower n-ZnO level (10–50 ppm) in the soil due to microbial stimulation at low doses of n-ZnO.

Although SEO contamination reduced soil pH slightly, differences across treatments were not statistically significant (p > 0.05). Control soil had a pH of 6.6, while SEO-treated soils ranged from 6.44 to 6.40. The pH decline aligns with findings by Okonokhua et al. (2007), Osaigbovo (2013), and Nwachukwu et al. (2020), who attributed reduced pH to hydrocarbon-induced acidification and microbial degradation However, contradictory results in the literature (for instance, Odjegba & Atebe, 2007; Swapna et al., 2021; Mbah et al., 2010) have reported pH increases in hydrocarbon-contaminated soils, possibly due to enhanced cation exchange or alkaline base accumulation. The application of n-ZnO mildly intensified acidification, likely due to Zn²⁺ ion release via nanoparticle oxidation and proton exchange (Verma et al., 2022; Vu et al., 2023). Akpan et al. (2025) reported that soil pH decreased from 6.5 in control soil to 6.3 and 5.3 in soil amended with 50 ppm and 300 ppm of n-ZnO respectively within 12 weeks of exposure.

Available phosphorus (AP) decreased significantly with SEO contamination. The control soil recorded 66.4 mg/kg AP, while SEO-only treatments ranged from 9.78 to 53.52 mg/kg. SEO+n-ZnO treatments showed improved phosphorus availability (16.77–60.27 mg/kg) compared to SEO-only soils, although still below control levels. The decline in phosphorus may result from microbial immobilization during hydrocarbon degradation or chemical interactions, potentially immobilizing phosphorus (Okonokhua et al., 2007; Adams & Duncan, 2022; Uquetan et al., 2017). According to FMARD (2002), Nwite and Alu (2015), Iren and Ediene (2017), and Akpan et al. (2024), SEO contamination inhibit nutrient dynamics, particularly phosphorus. Devatha et al. (2019) also found that the concentration of available phosphorus in soil decreased proportionally with increasing SEO levels.  The recovery of phosphorus with n-ZnO application supports findings by Verma et al. (2022) and Tarafdar & Claassen (2003), who noted that zinc enhances phosphatase activity and microbial P mobilization at low concentrations. However, Bala et al. (2019) and Lv et al. (2022) warned that excessive zinc may antagonize phosphorus availability due to ionic competition or complex formation. While available phosphorus was generally reduced by SEO contamination but improved with n-ZnO amendment, however, the values remained lower than in control soils, and inconsistencies in the response to n-ZnO suggest a complex interaction between zinc, hydrocarbons, and soil phosphorus. Further research is needed for to ascertain the underlying mechanisms responsible for these trends.

Exchangeable cations (Na, Mg, K, and Ca) declined progressively with increasing SEO concentrations. For instance, Na dropped from 3.33 Cmol/kg in control to 0.66 Cmol/kg at 2.5 L/kg SEO. Similar trends were observed for Mg (30.0 to 5.92 Cmol/kg), K (24.5 to 8.41 Cmol/kg), and Ca (38.10 to 14.76 Cmol/kg). The impact was more severe at the highest level of SEO as supported by Nwite et al. (2020). These reductions reflect the displacement or adsorption interference caused by hydrocarbon coatings on soil particles (Swapna, 2021; Uheghu et al., 2012). Application of n-ZnO ameliorated these effects, increasing cation concentrations and CEC across most treatments. Improved CEC in n-ZnO soils is likely due to enhanced surface reactivity and ion exchange properties of the nanoparticles (Rezaei & Baya, 2022; Liu & Dong, 2020; Akpan et al., 2025). Nevertheless, the response was not uniform for all cations, suggesting complex interactions between soil type, SEO, and n-ZnO concentrations.

Soil moisture content was highest in the control (45.82%) and decreased significantly with increasing SEO levels (6.88% at 2.5 L/kg). This supports previous findings that oil pollutants reduce soil porosity and water retention by clogging soil pores (Khamehchiyan et al., 2007; Kayode et al., 2009; Nwite and Alu, 2015). According to Swapna, (2021) and  Adenipekun et al.(2011), the adverse impact of SEO on moisture content is attributed to pore blockage and reduced water permeability.

However, n-ZnO addition improved soil moisture, with values ranging from 7.69% to 30.55%. This suggests a mitigating role of n-ZnO in sustaining moisture retention under oil-contaminated conditions. These findings are consistent with those of Bayat et al. (2017), who reported that n-ZnO enhances water retention and soil structure.

Table 2:    Physicochemical Parameters of Soils after Harvest of F. benjamina

| Contaminant level of SEO (L/kg) | Sand (%) | Silt (%) | Clay (%) | TN (%) | OC (%) | pH | AP (mg/kg | Moisture (%) | CEC (Cmol/kg) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Na | Mg | K | Ca |
| Uncontaminated soil | 75.00 | 12.00 | 13.00 | 0.57d | 3.30e | 6.60 | 65.66a | 45.50a | 3.33a | 30.00a | 24.50a | 38.10a |
| 0.5 | 72.00 | 13.00 | 15.00 | 1.35c | 5.34c | 6.44 | 53.52b | 28.53b | 1.77b | 29.26b | 13.23b | 33.72c |
| 1 | 70.00 | 13.00 | 17.00 | 2.44b | 5.31c | 6.41 | 43.24c | 11.33c | 1.43d | 22.41c | 13.05c | 37.41b |
| 1.5 | 70.00 | 13.00 | 17.00 | 2.47b | 5.63a | 6.41 | 35.37d | 11.13c | 1.56c | 13.01d | 11.33d | 23.67d |
| 2 | 70.00 | 13.00 | 17.00 | 2.47b | 5.50b | 6.40 | 17.40e | 10.53d | 1.00e | 10.65e | 8.41e | 23.01e |
| 2.5 | 70.00 | 13.00 | 17.00 | 3.36a | 3.01d | 6.40 | 9.78f | 6.88e | 0.66f | 7.69f | 5.92f | 14.76f |
| Contaminant +n-ZnO (50 ppm) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0.0 (control) | 75.00 | 12.00 | 13.00 | 0.58c | 3.38e | 6.60 | 66.40a | 45.82a | 3.32d | 29.65a | 26.75a | 37.19a |
| 0.5 | 75.00 | 12.00 | 13.00 | 2.23b | 6.24b | 6.20 | 60.27b | 30.55b | 4.81a | 25.25b | 15.20b | 33.47b |
| 1 | 78.00 | 10.00 | 12.00 | 3.37a | 6.96a | 6.20 | 47.75c | 18.37c | 4.33b | 24.50b | 14.42b | 30.41c |
| 1.5 | 77.00 | 10.00 | 13.00 | 3.64a | 6.98a | 6.00 | 42.38d | 12.12d | 3.43c | 15.01c | 14.03b | 21.61d |
| 2 | 78.00 | 10.00 | 12.00 | 3.53a | 5.81c | 6.20 | 29.45e | 11.40e | 2.82e | 11.55d | 9.05c | 19.30e |
| 2.5 | 75.00 | 10.00 | 15.00 | 3.35a | 4.12d | 6.00 | 16.77f | 7.69f | 1.94f | 9.71d | 6.01c | 16.73f |

Means of   replicates (SE) followed by different letters in the same column are significantly different (p < .05) according to Duncan’s new multiple range test.

3.3	Growth Performance of F. benjamina

Figures 1–27 illustrate the bi-weekly growth pattern of Ficus benjamina cultivated for 12 months in soil contaminated with varying concentrations of spent engine oil (SEO), with and without nano zinc oxide (n-ZnO) amendment. The results indicate a progressive decline in plant growth parameters—specifically height and number of leaves as the concentration of SEO increased.

At lower contamination (0.5 L/kg SEO), the plants demonstrated moderate tolerance, with growth values significantly higher than at higher concentrations. Specifically, by the time of harvest, plants in uncontaminated soil recorded an average height of 85.7 cm and 164 leaves, while those in 0.5 L/kg SEO-contaminated soil had 67.9 cm in height and 124.7 leaves representing a 20.8% decrease in height and a 23.9% decrease in number of leaves compared to the control. At 1.0 L/kg SEO, plant height and number of leaves decreased  by 60.6%  and 64.5%  respectively in relation to the control. Similarly,   increasing the SEO concentration to  1.5 L/kg resulted in substantial growth reduction  to 29.5 cm in height and 37 leaves,  representing 65.6% and 77.4% reductions, respectively. Notably, plants in 2.5 L/kg SEO died by week 4, while those in 2.0 L/kg died at week 8, confirming the phytotoxicity of high concentrations of SEO. These findings align with previous studies. For instance, Donald et al. (2016) observed complete inhibition of germination and growth of Capsicum frutescens, C. chinense, and C. annum in 3%–9% SEO-contaminated soils. Similarly, Ohanmu et al. (2014) reported a delayed lethal effect of crude oil-contaminated soils on Zea mays, becoming evident 3–6 weeks after planting. Anoliefo and Vwioko (1995) also reported premature mortality in Lycopersicon esculentum exposed to all concentrations of spent lubricating oil. According to Johnpaul (2021),  contamination of soil with spent engine oil caused growth retardation in tomato (Lycopersicomesculentum)  pepper (Capsicum annum L.). For example, Ogboghodo et al. (2004) reported that  SEO inhibited the germination and growth of maize at high pollution levels while the growth of okro and fluted pumpkin seedlings were highly reduced with an increase in crude oil concentrations (Asuquo et al., 2001). Нe response of plants to oil contamination depends upon the level of contamination

However, the application of 50 ppm n-ZnO significantly improved growth performance. F. benjamina in 0.5 L/kg SEO + 50 ppm n-ZnO  recorded the highest growth performance across all treatments, attaining 99.5 cm in height and 175 leaves, which represents a 16.1% increase in height and a 6.7% increase in number of leaves over the control. This indicates that n-ZnO not only mitigated the toxicity of SEO but also enhanced overall plant performance beyond baseline. At 1.0 L/kg SEO + n-ZnO, plant height and leaf number improved to 40.5 cm and 71 leaves, reflecting a 19.8% increase in height and a 21.8% increase in leaf number compared to the same SEO level without n-ZnO. Similarly, at 1.5 L/kg SEO + n-ZnO, plant growth improved to 29.7 cm and 51 leaves, corresponding to a 0.7% increase in height and a 37.8% increase in leaves over the unamended SEO treatment at the same concentration. However, even with n-ZnO, high SEO levels (2.0 and 2.5 L/kg) remained lethal to plants. These results reinforce the phytotoxic nature of elevated SEO concentrations and demonstrate the partial but concentration-dependent ameliorative effects of nano zinc oxide.

The observed improvement in plant growth under n-ZnO-amended conditions may be attributed to the nutrient-enhancing and stress-mitigating properties of zinc nanoparticles (Dimkpa et al., 2019; Faizan et al., 2021; Thounaojam et al., 2021). Zinc is a vital micronutrient involved in various physiological processes such as enzyme activation, protein synthesis, and membrane integrity (Broadley et al., 2007; Solanki, 2021). Nano-scale formulations offer greater zinc bioavailability due to their high surface-area-to-volume ratio and solubility (Beig et al., 2023). Khan and Bano (2016) reported that nanoparticle supplementation improved plant resilience, nutrient uptake, and stress tolerance. Similarly, Pillai and Kottekottil (2016) found that nano-materials enhanced plant adaptability to both in situ and ex situ stress conditions. Moreover, n-ZnO amendment appeared to enhance soil quality. Soils amended with nano-zinc oxide exhibited higher concentrations of total nitrogen, organic carbon, and exchangeable bases such as calcium, magnesium, sodium, and potassium—essential nutrients for plant growth. These findings corroborate the report by El-Sharkawy et al. (2022), who demonstrated significant improvements in soil nutrient profiles and plant performance under nano-zinc oxide treatments.

The results indicated that F. benjamina showed clear sensitivity to spent engine oil contamination, with growth inhibition and plant mortality at higher pollutant concentrations. However, amendment with nano-zinc oxide significantly enhanced growth performance, improved soil nutrient status, and increased survival rates, demonstrating its potential as a phytoprotective and soil-restorative agent in SEO-polluted environments.

Fig 1: Plant growth at week 1		                            Fig 2: Plant growth at week at  week 3

Fig 3: Plant growth at week 5			  Fig 4: Plant growth at week 7

Fig 1-4: Plant Height and Number of Leaves of Ficus benjamina in Spent Engine Oil Contaminated Soil at Week 1 to 7

Fig 5: Plant growth at week 9		Fig 6:  Plant growth at week 11

Fig 7: Plant growth at week 13		Fig 8: Plant growth at week 15

Figs 5- 10: Plant Height and Number of Leaves of Ficus benjamina in Spent Engine Oil Contaminated Soil at Week 9 to 15

Fig 9: Plant growth at week 17		Fig 10: Plant growth at week 19

Fig 11: Plant growth at week 21		Fig 12: Plant growth at week 23

Figs 11-14:  Plant Height and Number of Leaves of Ficus benjamina in Spent Engine Oil Contaminated Soil at Week 17 to 23

Fig 13: Plant growth at week 25				Fig 14: Plant growth at week

Fig 15: Plant growth at week 29				Fig 16: Plant growth at week 31

Figs 13-16: Plant Height and Number of Leaves of Ficus benjamina in Spent Engine Oil Contaminated Soil at Week 25 to 31

Fig 17: Plant growth at week 33			Fig 18: Plant growth at week 35

Fig 19: Plant growth at week 37				Fig 20: Plant growth at week 39

Figs 20-21: Plant Height and Number of Leaves of Ficus benjamina in Spent Engine Oil Contaminated Soil at Week 33 to 39

Fig 21: Plant growth at week 41			Fig 22: Plant growth at week 43

Fig 23: Plant growth at week 45			      Fig 24: Plant growth at week 47

Figs 23-26: Plant Height and Number of Leaves of Ficus benjamina in Spent Engine Oil Contaminated Soil at Week 41 to 47

Fig 25: Plant growth at week 49			  Fig 26: Plant growth at week 51

Fig 27: Plant growth at week 52

Figs 25-27: Plant Height and Number of Leaves of Ficus benjamina in Spent Engine Oil Contaminated Soil at Week 49 to 53

4.0	CONCLUSION

The study demonstrated that spent engine oil (SEO) contamination significantly alters soil physicochemical properties, resulting in reduced nutrient availability, compromised soil texture, and diminished moisture retention. These adverse changes were directly linked to the poor growth performance and mortality of Ficus benjamina at higher SEO concentrations, underscoring the phytotoxic effects of hydrocarbon pollutants on soil-plant systems. However, the incorporation of nano zinc oxide (n-ZnO) at 50 ppm effectively mitigated several deleterious impacts of SEO. The n-ZnO amendment improved soil fertility indicators—such as phosphorus, potassium, calcium, and magnesium levels—as well as moisture content and organic matter balance. Notably, n-ZnO-treated soils supported significantly better plant growth, even under moderate SEO pollution, indicating its potential as a remediation agent. These findings highlight the dual utility of n-ZnO in restoring soil quality and enhancing plant resilience in polluted environments. Future research should explore long-term ecological impacts, the bioaccumulation potential of n-ZnO, and its field-scale application in contaminated landscape.

The study however recommended  that future field-based studies should  be conducted to validate these the findings beyond controlled screenhouse conditions. Stake holders in the Environment should develop low-cost, community-friendly phytoremediation models using accessible nanoparticles and ornamentals to rehabilitate degraded urban and peri-urban soils. Also, while the study showed short-term benefits, long-term environmental risks and bioaccumulation of nanoparticles in soil and plant systems remain unclear. Therefore, further research should focus on  understanding fate and transport of n-ZnO in the environment, and evaluating risks to soil microbiota, food chains, and groundwater.  It is further recommended that comparative studies across different plant species be conducted to identify plants best suited for phytoremediation in oil-polluted environments when supported by nanoparticle amendments.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

REFERENCES

.

Adam, G. and Duncan, H. J. (2002). Influence of Diesel Fuel on Seed Germination. Environmental Pollution., 120:363-370.

Adenikin, B. A., Olanipon, D. G. and Osungbemir, W. B. (2023). Phytoremediation Potential of Gliricidia sepium (Jacq) walp in Spent Engine Oil Polluted Soil: Effects on Growth and Soil Physico-chemical Properties. Journal of Plant Research, 37(3). DOI: 10.1007/s42535-023-00691-5

Adedokun, O. M. and Ataga A. E (2007). Effects of Amendments and Bio Augmentation of Soil Polluted with Crude Oil, Automotive Gasoline Oil, and Spent Engine Oil on the Growth of Cowpea (Vignaungiculata L. Walp). Scientific Research and Essay, 2, 147-14

Adenipekun C. O., Oyetunji O. J. and Kassim L. S. (2008). Effect of Spent Engine on the Growth Parameter and Chlorophyll Content of Corehorous olitorius Linn. Environmentalist, 28:446-450.

Adenipekun, C. O., Olanrewaju, O. O. and Ogunjobi, A. A. (2011). Bioaccumulation of Heavy Metals and Nutrient Content Supplementation by Two White Rot Fungi in Crude Oil Polluted Soils. Researcher, 3(5):13-19.

Adesodun, J. K. (2004). Bioremediation of an Alfisol Contaminated with Spent Oil and its 	Quality 	Assessment using Micro-morphological Analysis. Ph.D Thesis, 	University of 	Nigeria, 	Nsukka, Nigeria.

Adewole, M. B. and Uchegbu, L. U. (2010). Properties of Soils and Plants Uptake within the Vicinity of Selected Automobile Workshops in Ile-Ife Southwestern, Nigeria. Ethiopian Journal of Environmental Studies and Management, 3(3).

Agbogidi, O. M. and Enujeke, E. C. (2012). Effects of Spent Motor Oil on Soil Physicochemical Properties and Growth of Arachis hypogea L. Global Journal of Biological Science and Biotechnology 1(7): 68 – 74.

Agbogidi, O. M. and Eruotor, P. G. (2012). Morphological Changes due to Spent Engine Oil Contamination and its Heavy Metal components of Jatropha curcas Linn.  Seedlings. Proceedings of the International Conference on Bioscience, Biotechnology and Healthcare Sciences. Singapore. pp. 88-93.

Agbogidi, O. M. and Ilondu, E. M. (2013). Effects of Spent Engine Oil on the Germination and Seedling Growth of Moringa oleifera (Lam.) Scholarly Journal of Agricultural Science, 3(6):239-243

Akpan N., Awotoye O.O., Oladeji O. F and Omotoso A. J. (2024). An Assessment of Heavy-Metal 	Accumulation in Soils in Selected Auto Mechanic Workshops Using Pollution 	Indices in 	Osun-State, Nigeria. International Journal of Advanced Multidisciplinary 	Reviewed Studies, 	4(1):724-733 2024.

Akpan, N. J.,  Awotoye, O. O.,  Odoh,  P. C., Adewusi, A.A.,   Oladeji, F.O. and  Ogundare, J. F (2025).		 Effects of 	Nano Zinc Oxide (n-ZnO) on Soil Physicochemical Properties, Micronutrient 	Dynamics, and the Growth 		Performance of Ficus Benjamina. Asian Soil Research 	Journal 9 (3):125-36. 	https://doi.org/10.9734/asrj/2025/v9i3195

Ali, H., Khan, E., & Ilahi, I. (2019). Environmental Chemistry and Ecotoxicology of Hazardous 	Heavy 	Metals: Environmental Persistence, Toxicity, and Bioaccumulation. Journal of 		chemistry, 2019(1), 6730305.

Al-Momani, H., Aolymat, I., Ibrahim, L., Albalawi, H., Al Balawi, D. A., Albiss, B. A., ... & Alghweiri, S. 	(2024). Low-dose zinc oxide nanoparticles trigger the growth and biofilm formation of 	Pseudomonas aeruginosa: a hormetic response. BMC microbiology, 24(1), 290.

Amadi A, Abbey SA and Nma A, 1996. Chronic effect of Oil Spill on Soil Properties 	and 	Micro 	Flora of a Rainforest Ecosystem in Nigeria. Water, Air  and Soil 	Pollution 	86:1-11.

Anoliefo, G. O. and Vwioko, D. E. (1995). Effects of Spent Lubricating Oil on the Growth of Capsicum 	annum and Lycopersicon esculentum, Environmental Pollution 88: 361–364.

AOAC (1995). Official Methods of Analysis of the Association of Official Analytical Chemists . Association of Official Analytical Chemists..

ASTM International (2013).  Standard Test Method for pH of Soils," (D4972 Method). American Society for Testing and Materials

ASTM International (2015). Standard Test Method for Particle-Size Analysis of Soils (D422-63) 	American Society for Testing and Materials

ASTM International. (2013). Standard test method for pH of soils (D4972 Method). American 	Society 	for Testing and Materials.

ASTM International. (2016). Standard Test Methods for Laboratory Determination of Water 	(Moisture) Content of Soil and Rock by Mass (ASTM D2216-10) . American Society for 	Testing 	and Materials.

Asuquo, F. E.; Ibanga, I. J. & Hungafa, N. 2001. Effects of Qua Iboe Crude Oil Contamination on 	Germination and 	Growth of Okra (Abeloschus esculintus L.) and Fluted pumpkin (Telflaric 	occidentalis L.) Proceeding of the 27th Annual Conference of the Soil Science Society of Nigeria. 	Nov. 5 – 9, 2001. University of Calabar, 	Nigeria.

Aucelio, R. Q., Martins de Souza, R., Calixto de Campos, R. Miekeley, N. N. and Porto da Silveira. C. L. (2007).  The determination of Trace Metals in Lubricating Oils by Atomic Spectrometry. Spectrochimica Acta Part B: Atomic Spectroscopy 62(9):  952-961.

Ayodele, R. I. and Afolarin O. (2020). Decontamination of Automobile Workshop Soils Containing Heavy Metals and PAHs using Chelating Agents. International Journal of Environmental Pollution and Remediation, 8: 37-45, DOI: 10.11159/ijepr.2020.005

Bala, A., Majebi, O. J., Ebhodaghe, O. F., Ufuoma, U. and Anwuli, E. R. (2019). Levels of Heavy Metals in Soil Sample from Active Automobile Workshops in Benin City. International Journal of Environmental Chemistry, 3(1), 1-7.

Beig, B., Niazi, M. B. K., Jahan, Z., Haider, G., Zia, M., Shah, G. A., and Hayat, A. (2023). Development and Testing of Zinc Sulfate and Zinc Oxide Nanoparticle-coated Urea Fertilizer to Improve N and Zn use Efficiency. Frontiers in Plant Science, 13, 1058219.

Billings, S. A., Lajtha, K., Malhotra, A., Berhe, A. A., Graaff, M. ‐A., Earl, S. and Wieder, W. (2021). Soil Organic Carbon is not just for Soil Scientists: Measurement Recommendations for Diverse Practitioners. Ecological Applications,31(3). doi:10.1002/eap.2290.

# Broadley, M. R., White, P. J., Hammond, J. P., Zelko, I., & Lux, A. (2007). Zinc in plants. New 	phytologist, 173(4), 677-702.

Buraka, T., Elias, E. and Lelago, A. (2022). Soil Organic Carbon and its' Stock Potential in Different Land-use Types along Slope Position in Coka Watershed, Southern Ethiopia. Heliyon, 8(8).

Burnete, S and Pilasluck, C. (2005). Trade and Environment: A Historical Perspective. Studies in Business and Economics, 10(2) 17-31.

Chinenye, C. and Chijioke, O. (2016). Effects of Waste Engine Oil on the Microflora and Physico-Chemical Quality of Soils from Two West African Countries, Asian Journal of Environment & Ecology, 1(2):1-14

Dahan, A., Yarmolinsky, L., Budovsky, A., Khalfin, B., & Ben-Shabat, S. (2025). Therapeutic 	potential of Ficus benjamina: Phytochemical identification and investigation of 	antimicrobial, 	anticancer, pro-wound-healing, and anti-inflammatory properties. Molecules, 30(9), 1961

De Jong, E. (1980). The Effect of a Crude Oil Spill on Cereals, Environmental Pollution Series A, Ecological and Biological,  22 (3): 187-196

Deka, H. (2019). Role of Nanoparticles in Heavy Metal Bioremediation. In Bioremediation of 	Industrial Waste for Environmental Safety (pp. 91-111). IGI Global.

Devatha, C. P., Vishnu Vishal, A. and Purna Chandra Rao, J. (2019). Investigation of Physical and Chemical Characteristics on Soil due to Crude Oil Contamination and its Remediation. Applied Water Science, 9: 1-10.

# Dimkpa, C. O., Singh, U., Bindraban, P. S., Elmer, W. H., Gardea-Torresdey, J. L., & White, J. 	C. 	(2019). Zinc oxide nanoparticles alleviate drought-induced alterations in sorghum 		performance, nutrient acquisition, and grain fortification. Science of the Total 	Environment, 688, 926-934.

Echiegu, E. A., Amadi, A. S., Ugwuishiwu, B. O. and Nwoke, O. A. (2022). Effect of Spent Engine Oil Contamination on the Soil Properties in Selected Automobile Mechanic Villages in Enugu, Enugu State, Nigeria. Environmental Quality Management, 31(3), 209-218.

El-Sharkawy, M., Mahmoud, E., Abd El-Aziz, M. and Khalifa, T. (2022). Effect of Zinc Oxide Nanoparticles and Soil Amendments on Wheat Yield, Physiological Attributes and Soil Properties Grown in the Saline–sodic Soil. Communications in Soil Science and Plant Analysis, 53(17), 2170-2186.

Enwezor, W. O., Udo, E. J., Usoroh, N. J., Ayotade, K. A., Adepetu, J. A., Chude, V. O., & Udegbe, C. I. 	(1982). Fertility 	capability classification for Nigerian soils. Soil Science Division, Faculty of 	Agriculture, University of Ibadan, Nigeria.

# Faizan, M., Yu, F., Chen, C., Faraz, A., & Hayat, S. (2021). Zinc Oxide Nanoparticles Help to 	Enhance Plant Growth and Alleviate Abiotic Stress: A review. Current Protein and Peptide		 Science, 22(5), 362-375.

FAO. (1976). A framework for land evaluation (Food and Agriculture Organizarion Soil Bulletin No. 32). 	Rome.

Federal Ministry of Agriculture and Rural Development (2002). Fertilizer Use and Management Practice for Crops in Nigeria. In: Aduayi, E.A, Chude, V.O., Adebusyi, B.A. and Oloyiwola, S.O. (eds). Federal Fertilizer Department, Abuyi. P 188.

FMARD (Federal Ministry of Agriculture and Rural Development) (2002). Fertilizer Use and Management Practice for Crops in Nigeria. In: Aduayi, E.A, Chude, V.O., Adebusyi, B.A. and Oloyiwola, S.O. (eds). Federal Fertilizer Department, Abuyi. P 188.

Gautam, S. P., Srivastava, V. and Agarwal, V. C. (2012). Use of Glass Wastes as Fine Aggregate in Concrete.  Journal of Academic and Industrial Research, 1(6)

Gudin, C. and  Syratt, Y. J. (1975). Biological Aspects of Land Rehabilitation Following Hydrocarbon Contamination. Environmental Pollution,  8 (2): 107-112.

Insook Lee, K. B., Hyunhee, K., Sunghyun, K., Jaisoo, K., Youngseok, K., Yoonyoung, C and Bumhan, B. (2007). Phytoremediation of Soil Co-contaminated with Heavy Metals and TNT using Four Plant Species. Journal of Environmental Science and Health, Part A: Toxic/Hazardous Substances and Environmental Engineering, 42(13):  2039-2045

Iren, O. B. and Ediene, V. F. (2017). Effect of Spent Engine Oil Discharge on Soil Properties in Selected Automobile Workshop in Calabar, Cross River State, Nigeria. International Journal of Innovative Science Engineering Technology 4(2):391–401.

Iren, O. B. and Ediene, V. F. (2017). Effect of Spent Sngine Oil discharge on Soil Properties in Selected Automobile Workshop in Calabar, Cross River State, Nigeria. International Journal of Innovative Science, Engineering and Technology, 4(2), 391-401.

Jia, W., Cheng, L., Tan, Q., Liu, Y., Dou, J., Yang, K. and Ding, A. (2023). Response of the Soil Microbial Community to Petroleum Hydrocarbon Stress Shows a Threshold Effect: Research on Aged Realistic Contaminated Fields. Frontiers in Microbiology, 14, 1188229.

# Johnpaul, A. (2021). Assessment of Phytotoxic Effect of Spent Engine Oil on Germination and 	Early 	Seedling Growth of Some Legumes in Imo State, Southeastern Nigeria. Research and Review in 	Biosciences. 20217;16(6):111.

Jones, J. B., and Jacobsen, J. S. (2001). Soil Testing and Plant Analysis. Madison, WI: Soil Science Society of America, Inc.

Kayode J, Oyedeji AA and Olowoyo O, 2009. Evaluation of the Effect of Pollution with 	Spent 	Lubricant Oil on the Physical and Chemical Properties of Soil. The Pacific Journal of Science and 	Technology 10(1): 387-291.

Kayode, J., Ige, S. A., Adeyemo, A. I. and Abayomi, O. S. (2009). Effect of Used Engine Oil on Some Growth Parameters of Abelmoschus esculentus L. Moorich and Capsicum annuum L. New York Science Journal, 2(4), 10-14.

Khamehchiyan, M.,   Charkhabi, A, H. and Tajik, M. (2007). Effects of Crude Oil Contamination on Geotechnical Properties of Clay and Sandy Soils. Engineering Geology,  89 (3–4): 220-229

Lackoova, L., Pokrývková, J., Kozlovsky Dufková, J., Policht-Latawiec, A., Michałowska, K. and Dąbrowska, J. (2021). Long-term Impact of Wind Erosion on the Particle Size Distribution of Soils in the Eastern Part of the European Union. Entropy, 23(8), 935.

Landon, J. R. (Ed.). (1991). Booker Tropical Soil Manual: A Handbook for Soil Survey and Agricultural 	Land Evaluation in the Tropics and Subtropics. Longman Scientific & Technical Publications, 	Harlon

Laura, L. S. and Jodi, J. (2009). Understanding Soil Cation Exchange Capacity. University of Wyoming Cooperative Extension Service, 1093.

# Lefèvre, C., Rekik, F., Alcantara, V., & Wiese, L. (2017). Soil Organic Carbon: The Hidden 	Potential (pp. ix+-77).

Liu, D. M. and Dong, C. (2020). Recent Advances in Nano-carrier Immobilized Enzymes and their Applications. Process Biochemistry, 92, 464-475.

Lugtenberg, B. and Kamilova, F. (2009). Plant-growth-promoting Rhizobacteria. Annual Review of Microbiology

Lv, H., Ji, C., Ding, J., Yu, L. and Cai, H. (2022). High Levels of Zinc Affect Nitrogen and Phosphorus Transformation in Rice Rhizosphere Soil by Modifying Microbial Communities. Plants, 11(17), 2271.

Mbah C. N, Nwite J. N, Njoku C. and Nweke I. A. (2010). Response of Maize (Zea mays L.) to Different Rates of Wood Ash Application in Ultisol with Organic Wastes and its Effect on Acid Ultisol in Southeast Nigeria. Africa  Journal of Agriculture and Research, 5(7): 580-593.

McGrath, S., Chaudri, A. M. and Griller,  K. E. (1995). Long Term Effects of Metals in Sewage Sludge on Soils, Micro-organisms and Plants. Journal of Industrial Microbiology 14: 94-104.

Mohamed, R. M., Al-Gheethi, A. A., Noramira, J., Chan, C. M., Hashim, M. A. and Sabariah, M. (2018). Effect of Detergents from Laundry Greywater on Soil Properties: A Preliminary Study. Applied Water Science, 8, 1-7.

Muhammad, I., Panaullah, G. M., Hossain, M. B., Loeppert, R. H. and Panaullah, G. M. (2016). Arsenic Toxicity to Rice (Oryza sativa L.) in Bangladesh. Plant and Soil, 317(1-2), 31-39.

# Nichols, K. (2022). The Importance of pH and Liming Material,University of Maryland

Njoku, K. L. and Nwani, S. O. (2022). Phytoremediation of Heavy Metals Contaminated Soil Samples Obtained from Mechanic Workshop and Dumpsite using Amaranthus spinosus. Scientific African, 17, e01278.

Nwachukwu, M. O., Azorji, J. N., Adjero, L. A., Green, M. C., Igwe, C. E. and Nnadozie, R. I. A. (2020). Influence of Spent Engine Oil Pollution and Organic Amendment on Soil Physicochemical Properties, Microbial Population and Growth of Capsicum annuum (L.). Asian Soil Research Journal, 3(1), 17-25.

Nwankwo, C. E. I., Okeke, E. S., Umeoguaju, F. U., Ejeromedoghene, O., Adedipe, D. T., & 	Ezeorba, T. P. C. (2025). Addressing Emerging Contaminants in Agriculture Affecting 	Plant–	soil Interaction: a Review on Bio-based and Nano-enhanced Strategies for Soil Health 	and 	Global Food Security (GFS). Discover Toxicology, 2(1), 1-30.

Nwite, J. N. and Alu, M. O. (2015). Effect of Different Levels of Spent Engine Oil on Soil Properties, Grain Yield of Maize and its Heavy Metal Uptake in Abakaliki, Southeastern Nigeria. Journal of Soil Science and Environmental Management, 6(4), 92-99

Nwite. J. N., Enyioko, C. O. and Nwogbaga, A. C. (2020). Impact of used Motor Oil on Soil Properties and Yield Indices of Corn in Izzi, Ebonyi State, Nigeria. Asian Journal Agriculture and Biology, 8(3):291-298. DOI:  https://doi.org/10.35495/ajab.2018.08.263

Nwosu, F. O., Olu-Owolabi, B. I., Adebowale, K. O. and Leke, L. (2008). Comparative Investigation of Wear Metals in Virgin and Used Lubricant Oils. Terrestrial and Aquatic Environmental Toxicology 2 (1):38–43.

Nyarko, H. D., Okpokwasili, G. C., Joel, O. F. and Galyuon, I. A. K. (2019). Effect of Petroleum Fuels and Lubricants on Soil Properties of Auto-mechanic Workshops and Garages in Cape Coast Metropolis, Ghana. Journal of Applied Sciences and Environmental Management, 23(7), 1287-1296.

Odjegba , V. J. and  Sadiq, A. O. (2002). Effects of Spent Engine Oil on the Growth Parameters, Chlorophyll and Protein levels of Amaranthus hybridus L. Environmentalist, 22:23–28.

Ofori, S. A., Cobbina, S. J. and Doke, D. A. (2020). The Occurrence and Levels of Polycyclic 	Aromatic Hydrocarbons (PAHs) in African Environments—A Systematic Review. 	Environmental 	Science and Pollution Research, 27, 32389-32431

Ogboghodo, I. A., Iruaga, E. K., Osemwota, I. O., & Chokor, J. U. (2004). An assessment of the effects of 	crude oil pollution on soil properties, germination and growth of maize (Zea mays) using two 	crude types–Forcados light and Escravos light. Environmental Monitoring and Assessment, 96(1),		 143-152.

Oghenerume, P., Eduok, S., Ita, B.,  John, O. and Bassey, I. (2020). Influence of Zinc Oxide			 Nanoparticles on Soil Physicochemical Properties and Arachis hypogea Rhizosphere 	Microbial Community. International Journal of Plant & Soil Science, 32(8): 88-100

Oguh, C. E. and Obiwulu, E. N. O. (2020). Human Risk on Heavy Metal Pollution and Bioaccumulation Factor in Soil and Some Edible Vegetables around Active Auto-mechanic Workshop in Chanchaga Minna Niger State, Nigeria. Annals of Ecology and Environmental Science, 4(1), 12-22.

Ogunneye, A.L.; Omoboyowa, D.A.; Sonibare, A.L.; Adebusuyi, A.J. and Faniran, T.P. (2014). Hepatotoxic and Nephrotoxic Effects of Petroleum Fumes on Petrol Attendants in Ibadan, Nigeria. Nigeria Journal of Basic and Applied. Science, 22, 57–62

Oguntimehin, I. and Ipinmoroti, K.O. (2008). Profile of Heavy Metals from Automobile Workshops in Akure, Nigeria. Journal of Evironmental Science and Technology 	1(7):19 – 26

Ohanmu,  E. O., Bako, S. P. and  Adelanwa, M.A. (2014). Effect of Crude Oil Polluted Soil on the Growth and Survival of Pepper (Capsicum annuum L.). Annals Experimental Biology, 2(4):5-10

Okebalama, C. B., Onwurah, C. L., Jidere, C. M., & Okolo, C. C. (2024). Disposal of Spent Oil 	into 	Soils around Auto Parts Markets Impacts Heavy Metal Concentrations and Poses a Potential 	Ecological Risk. Environmental Systems Research, 13(1), 15.

Okeke, O., Ezeh, E., Okeke, H. C., Aniobi, C. C. and Akagha, C. I. (2020). Comparison of Soil Samples from Selected Anthropogenic Sites within Enugu Metropolis for Physicochemical Parameters and Heavy Metal Levels Determination. Journal of Environmental Protection, 11(10), 848-861.

Okonokhua, B. O., Ikhajiagbe, B., Anoliefo, G. O. and Emede, T. O. (2007). The Effects of Used Engine Oil on Soil Properties and Growth of Maize (Zea mays L.). Journal of Applied Sciences and Environmental Management, 11(3), 147-152.

Oloruntoba, E. O. and Ogunbunmi, T. O. (2020). Impact of Informal Auto-Mobile Mechanic Workshops Activities on Groundwater Quality in Ibadan, Nigeria. Journal of Water Resource and Protection, 12: 590-606. https://doi.org/10.4236/jwarp.2020.127036

Olsen, S.R., Cole, C.V., Watanabe, F.S. and Dean, L.A. (1954). Estimation of Available  Phosphorus in Soils by Extraction with Sodium Bicarbonate. USDA Circular 939. US Government Printing Office, Washington DC

Osaigbovo AU, Law-Ogbomo KE, Agele SO (2013) Effects of Spent Engine Oil Polluted Soil 	and 	Organic Amendment on Soil Chemical Properties, Micro-flora on Growth and 	Herbage of 	Telfairia occidentalis. Bayero Journal of  Pure and Applied Science, 	6(1):72–78

Osaigbovo, A. U., Law-Ogbomo, K. E.1. and Agele, S. O. (2014). Effects of Spent Engine Oil Polluted Soil and Organic Amendment on Soil Chemical Properties, Micro-Flora on Growth and Herbage of Telfairia occidentalis (HOOK F). Bayero Journal of Pure and Applied Sciences, 6(1): 72 – 78.

Osuagwu, A. and Nwofia, G. E. (2014). Effect of Spent Engine Oil on the Germination Ability of Eleven Accessions of African Yam Bean Seeds (Sphenostylis sternocarpa Hochst ex A. Rich) Harms. IOSR Journal of Agriculture and Veterinary Science 7(1):59-62

Osuagwu, A. N. and Iwuoha, C. L. (2015). Assessment of Spent Engine Oil on the Germination Ability of Cajanus cajan, Vigna subterranean and Phaselous vulgaris. International Conference on Advances in Agricultural, Biological and Environmental Sciences (AABES-2015) London (UK).

Osuagwu, A. N., Ndubuisi, P. and Okoro, C. K. (2017). Effect of Spent Engine Oil Contaminated Soil on 	Arachis hypogea (L.), Zea mays (L.) and Vigna unguiculata (L.) 	Walp. International Journal of 	Advanced and Agricultural Research, 5:76-81

Pillai, H. P., & Kottekottil, J. (2016). Nano-phytotechnological Remediation of Endosulfan using Zero 	valent 	Iron 	Nanoparticles.   7(05), 734.

Pilon-Smits, E. (2005). Phytoremediation. Annual Review of Plant Biology, 56: 15–39

Prasad, M. N. V., Hagemeyer, J., Dietz, K. J., Baier, M. and Krämer, U. (1999). Free Radicals and Reactive Oxygen Species as Mediators of Heavy Metal Toxicity in Plants. Heavy Metal Stress in Plants: from Molecules to Ecosystems, 73-97.

Priyanka, N., Geetha, N., Manish, T., Sahi, S. V., and Venkatachalam, P. (2021). Zinc Oxide Nanocatalyst Mediates Cadmium and Lead Toxicity Tolerance Mechanism by Differential Regulation of Photosynthetic Machinery and Antioxidant Enzymes Level in cotton Seedlings. Toxicology Reports, 8: 295-302.

Raliya, R., & Tarafdar, J. C. (2013). ZnO Nanoparticle Biosynthesis and its Effect on Phosphorous-	Mobilizing Enzyme 	Secretion and Gum Contents in Clusterbean (Cyamopsis tetragonoloba 	L.). Agricultural Research, 2, 48–57.

Rauckyte-Zak, T., Hargreaves, D. J. and Pawlak, Z. (2006). Determination of Heavy Metals and Volatile Aromatic Compounds in Used Engine Oils and Sludges, Fuel 85(4):481-485

Rezaei S., and Bayat H. 2022. Effect of Zinc Oxide Nano-Particles (ZnO) on some Physical and Chemical 	Properties of Soils with Different Textures. Journal of Water and Soil 36(3). 	DOI: .22067/JSW.2022.75234.1143

Rowell, D. L. (2014). Soil science: Methods and applications (2nd ed.). Routledge.

Rowell, M. J. (1977). The Effect of Crude Oil Spills on Soils. A Review of Literature. In: The Reclamation of Agricultural Soils after Oil Spills, Toogood, J.A. (Ed.). University of Alberta, Edmonton, pp: 1-3

Russell, M., Robinson, C. D., Walsham, P., Webster, L. and Moffat, C. F. (2011). Persistent Organic Pollutants and Trace Metals in Sediments Close to Scottish Marine Fish Farms.	 Aquaculture, 319(1-2), 262-271.

Sesan, A., Aransiola, U. J. J. I., & Olabisi, P. A. (2013). Phytoremediation of Lead Polluted Soil by Glycine 	max L. 	Journal of Applied and Environmental Soil Science.

Shahzad, A., Zahra, A., Li, H. Y., Qin, M., Wu, H., Wen, M. Q., & Zafar, S. (2024). Modern 	perspectives of 	heavy metals alleviation from oil contaminated soil: A 	review. Ecotoxicology 	and Environmental Safety, 282, 116698.

Sharifan, H. and  Ma, L. Q. (2017). Environmental Transformations and Ecological Effects of 	Engineered Nanomaterials: A Review. Environmental Pollution, 231: 381-394.

Sheteiwy, M. S., Shaghaleh, H., Hamoud, Y. A., Holford, P., Shao, H., Qi, W. and Wu, T. (2021). Zinc Oxide Nanoparticles: Potential Effects on Soil Properties, Crop Production, Food Processing, and Food Quality. Environmental Science and Pollution Research, 28(28), 36942–36966. doi:10.1007/s11356-021-14542-w.

# Solanki, M. (2021). The Zn as a vital micronutrient in plants.  Journal of Microbiology, Biotechnology and 	Food Sciences, 11(3), e4026-e4026.

Subbiah, B.V. and Asija, G.L. (1956). A Rapid Procedure for the Estimation of Available Nitrogen in Soils. Current Science, 25, 259-260

Swapna, A. A., Vijayammal, R. and Radha, D. S. (2021). Effects of Spent Engine Oil on Soil Characteristics and Selected Phytochemicals in Amaranthus hybridus. Research Square: https://doi.org/10.21203/rs.3.rs-939651/v1

Tanimu, J., Michael, G. I.  and James, P. A. (2019). Effect of Contamination of Soil with Used Engine oil on Some Soil Properties and Microbial Growth in Wukari, North Eastern Nigeria. East African Scholars Journal of Agriculture and Life Science 2 (6):358-363.

Tarafdar, J. C., & Claassen, N. (2003). Zinc and phosphorus mobilization in soils. Biology and 	Fertility 	of Soils, 38(2), 102 107.

# Thounaojam, T. C., Thounaojam, T. M., & Upadhyaya, H. (2021). Role of Zinc Oxide 	Nanoparticles in 	Mediating Abiotic Stress Responses in Plant. In Zinc-based Nanostructures for 	Environmental 	and Agricultural Applications (pp. 323-337). Elsevier.

Uchegbu, F. O, Akubugwo, E. I, Iweala, E. J and Uhegbu, O. C. (2012). Impact of Spent Engine Oil on 	Soil and the Growth of Zea mays L. Seeds. Scientific Journal of Environmental Science 1(1): 1-8.

Umana, E. J., Akwaji, P. I. and Markson, A. A (2016). Bioremediation of Spent Engine Oil Contaminated Soil by using Fungus, Penicillium sp. International Letters of Natural Sciences,. 59: 82-91.

Uquetan, U. I., Osang, J. E., Egor, A. O., Essoka, P. A., Alozie, S. I., and  Bawan, A. M. (2017). A Case Study of the Effects of Oil Pollution on Soil Properties and Growth of Tree Crops in Cross River State, Nigeria. International Research Journal of Pure and Applied Physics, 5(2), 19-28.

USEPA (2000). Electrokinetic and Phytoremediation Insitu Treatment of Metal Contaminated Soil: State-of-the-Practice. Draft for Final Review. United States Environmental Protection Agency, Office of Solid Waste and Emergency Response Tecnology Innovation. Washington D.C.

Verma, Y., Singh, S.K. and Jatav, H.S. (2022). Interaction of Zinc Oxide 	Nanoparticles with			 Soil: Insights into the Chemical and Biological Properties. Environmental Geochemistry			 and Health 44, 221–234  https://doi.org/10.1007/s10653-021-00929-8

# Vu, K. A.  and Mulligan, C. N. (2023). An Overview on the Treatment of Oil Pollutants in 	Soil Using 	Synthetic and Biological Surfactant Foam and 	Nanoparticles. 	International Journal of 	Molecular Science, 24(3), 	1916;   https://doi.org/10.3390/ijms24031916

Vu, K.A. and Mulligan, C.N. (2023). An Overview on the Treatment of Oil Pollutants in Soil Using 	Synthetic and Biological Surfactant Foam and Nanoparticles.International  Journal of Molecular 	Science (24) 1916. https://doi.org/10.3390/ 	ijms24031916

Vu, K.A.; Tawfiq, K.and Chen, G. (2015). Rhamnolipid Transport in 	Biocharamended 	Agricultural Soil. Water, Air and Soil Pollution. 226, 256–	264

Wang, Y., Li, Y. and Yao, X. (2017). Nano-ZnO Modified with Persulfate for the 	Photocatalytic 	Degradation of Atrazine under Visible Light. Journal of 	Nanoscience and 	Nanotechnology, 17(10), 7392-7399.