INTEGRATED GRAVITY AND MAGNETIC DERIVATIVE MODELLING FOR STRUCTURAL CONTROL OF GOLD MINERALIZATION IN NORTH-CENTRAL NIGERIA

Abstract

Structural controls play a critical role in the localization of gold mineralization within Precambrian basement terrains. In North-Central Nigeria, the combination of complex lithology, tectonic structures, and hydrothermal processes necessitates an integrated approach for mineral targeting. The study area, approximately 72,600 square kilometres, constituting about 7.86% of the country’s landmass, covers part of Niger State, the Federal Capital Territory (FCT), and a few other towns in Kaduna, Plateau, and Nasarawa, adjoining the FCT. This study applies derivative modelling of gravity and magnetic datasets to delineate structural features controlling gold mineralization. Bouguer gravity anomaly, tilt derivative, and vertical derivative maps were integrated with magnetic total field intensity, analytic signal, and first vertical derivative to reveal major and minor lineament trends. Structural orientations were compared with known gold occurrences, showing strong associations with NE–SW and NW–SE fault systems. The results highlight structurally favourable zones for gold exploration; particularly along deep-seated lineaments interpreted as conduits for hydrothermal fluid migration. The integration of geophysical derivatives with geological data improves the accuracy of mineral targeting and supports the development of predictive models for mineral prospectivity.

Keywords: Mineral Prospectivity, Gravity, Magnetic Derivatives, Lineaments, Structural Control, North-Central Nigeria, Gold Exploration

- Introduction

Gold mineralization in Precambrian basement terrains is commonly controlled by structural features such as faults, fractures, and shear zones (Ajibade et al., 2008). Geophysical methods, especially gravity and magnetic surveys, are powerful tools for mapping these features because variations in density and magnetic susceptibility often reflect structural and lithological contrasts (Telford et al., 1990). These subsurface features influence ore deposition as mineral exploration in crystalline basement terrains often relies on identifying geological structures that serve as conduits for mineralizing fluids (Groves et al., 2016). In North-Central Nigeria, several gold occurrences are hosted in structurally complex schist belts and granitic intrusions (Obaje, 2009; Akintola et al., 2022). However, much of the region remains underexplored.

Previous studies in Nigeria have demonstrated the effectiveness of integrating geophysical datasets for mineral exploration (Anakwuba et al., 2014; Oden et al., 2019). Recent studies like Augie et al. (2024), Salako et al. (2024), and akin have delineated geologic structures like faults, fractures, and lineaments, which may be potential hosts to minerals such as gold, using geophysical datasets. Gravity methods provide insight into subsurface density variations, and on the other hand, magnetic surveys detect variations in the magnetic properties of rocks, both of which can be enhanced through derivative filtering techniques to highlight structural features. This study focuses on integrating gravity and magnetic derivatives techniques to delineate structures controlling gold mineralization in North-Central Nigeria, contributing to improved mineral prospectivity mapping in the region.

- Location and Geological Setting

The study area bounded by latitude 8o30’00”– 10o00’00” North of the equator and longitude 5o00’00” – 9o00’00” East of the Greenwich meridian (Figure 1) lies within the North-Central part of Nigeria, underlain majorly by the Precambrian Basement Complex comprising migmatite-gneiss complexes, schist belts, gneisses, granites, and metavolcanic sequences or Pan-African granitoids (Rahaman, 1988; Obaje, 2009; Bolarinwa et al., 2021). These rocks are intruded by quartz veins and pegmatites, often associated with gold mineralization. Structurally, The North central region is traversed by N-S, NE-SW, and NW-SE trending structures as documented in articles like Ogunmola et al. 2015, Daniel et al., 2019, Mustapha et al., 2020, Ahmed et al., 2020, Ebele et al., 2021, Arogundade et al., 2022, and Egbelehulu and Abu (2024), many of which are believed to control gold mineralization. Historical artisanal gold mining has been documented in locations such as Minna, Wushishi, Paikoro, and Keffi, indicating the presence of auriferous quartz veins and hydrothermally altered zones.

Figure.1: The geological map of the study area (Extracted from the USGS World Geologic Maps website)

- Geophysical Data Analysis Techniques

In geophysical data analysis, analysing magnetic and gravity data involves various techniques aimed at interpreting the signals received from these different geophysical methods. The techniques to be used include:

3.1     Reduction to the Equator (RTE)

The RTE is a geophysical data processing technique primarily used in the interpretation of magnetic data. The Earth's magnetic field is dipolar; that is, its intensity and direction vary with latitude. This variation can complicate the analysis of magnetic anomalies because the anomalies' shapes and amplitudes are influenced by the inclination and declination of the Earth's magnetic field at the survey location. RTE mitigates these effects by transforming the observed magnetic data as if it were measured at the magnetic equator, where the field is horizontal. Application of the RTE simplifies the interpretation of magnetic data by providing a more consistent anomaly shape and enhances the visibility of geological structures

3.2 First Vertical Derivative (FVD)

The FVD technique is particularly effective in identifying subtle variations in the subsurface, such as faults, mineral deposits, and hydrocarbon reservoirs. It highlights the sharpness of an anomaly in geophysical data, making it easier to identify geological boundaries or structures. One recent study by Ishola et al. (2020) utilised magnetic data to map the subsurface structure of Ewekoro region, Southwestern Nigeria. The vertical derivative of the potential field enhances the shallow features while attenuating the deep-sited features, and sharpens the shorter wavelength at the shallow subsurface region (Salako et al. 2024; Aliyu et al., 2021).

FVD= ∂Z∂z = Zx,y,z+∆z-Z(x,y,z)∆z 		(1)

Where:

Z is the measured geophysical anomaly (e.g., magnetic or gravity field data), ∂Z∂z is the first vertical derivative, and ∆z is the vertical distance over which the derivative is computed

3.3   Analytical Signal (AS)

Analytical Signal is a measure of the rate of change of a geophysical field calculated by taking the square root of the sum of the squared derivatives of the field in each direction highlighting changes in the data’s amplitude and locating anomalies

AS=∂T/∂x2+∂T/∂y2+∂T/∂z2		       (2)

where; AS constitute the amplitude of the total gradient of the signal at the x,y plane, T  is the measured magnetic susceptibility, ∂T/∂x+∂T/∂y+∂T/∂z  are the vertical and orthogonal derivatives along the z, y and x planes, respectively.

3.4  Tilt derivative (TD)

Tilt derivative (TD) technique is one that can be applied to potential field (magnetic or gravity) data to enhance features and reveal structural trends and edges of anomalous bodies. The TD is a geometrical function that uses the vertical and horizontal derivatives of a field. The technique involves calculating the tilt angle of the field and then taking the derivative of the tilt angle to the horizontal coordinate x and vertical coordinate y. Abdelrady et al. (2023) used the TD to identify several anomalies that are associated with gold deposits and also to map the distribution of subsurface structures in an area. The TD is computed from the FVD of the magnetic or gravity anomaly and is given by the formula:

TD=(∂Z∂y)(∂Z∂x)2+(∂Z∂y)2              			      (3)

where; Z is the magnetic or gravity anomaly data, ∂Z∂x and ∂Z∂y are the first vertical derivatives in the x and y directions, respectively.

3.5  Euler deconvolution

Euler deconvolution technique, based on Euler's homogeneity equation is used to locate the source of anomalies in magnetic and gravity data. The Euler's homogeneity equation relates the curvature of the magnetic or gravity field to the location of the source of the anomaly. The technique involves fitting a set of magnetic or gravity data to Euler's homogeneity equation and solving for the location of the source of the anomaly  (Reid et al., 1990).

x-x0∂T∂x+y-y0∂T∂y+z-z0∂T∂z=ηβ-T		      (4)

where β is the regional value of the total field, (x0, y0, z0 ) is the position of the source which produces the total field T, and η, defines the structural index. Identified structural geometry and depth is controlled by the structural index; η (Reid et al., 1990).

- Materials and Methods

  - Data source and Acquisition

Total Magnetic Intensity (TMI) data covering 24 sheets were obtained from the Nigerian Geological Survey Agency (NGSA) and the sheets are Fashe (161), Akerre(162), Zungeru (163), Minna (164), Bishini (165), Kachia (166), Kafanchan (167), Naragatu 168), Mokwa (182) , Egbako (183), Bida (184), Paiko (185), Abuja (186), Gitata (187), Jema’a (188), Kurra (189), Lafiagi (203), Pategi (204), Baro (205), Gulu (206), Kuje (207), Keffi (208), Akwanga (209), and Wamba (210). The data were captured for NGSA by Fugro Airborne surveys between 2007 and 2009 as part of the nationwide airborne geophysical surveys and were acquired along a series of NE-SW profiles with a flight line spacing of 500 and terrain clearance of 80 m. The Bouguer gravity anomaly was obtained from the Bureau Gravimetric International (BGI) World Gravity Map archives. Ancillary data including geological map and mineral occurrence points were extracted from the USGS earth explorer website and Nigeria Mineral resources and Metallogenic map (available on the NGSA website) respectively.

#### 4.2 Data Processing and Analysis

Using the 2010 International Geomagnetic Reference Field, irregularities caused by regional features were removed from the magnetic data. RTE trailed to mathematically transform the magnetic data to simulate how it would appear if measured at the magnetic equator. Hence, positioning the magnetic anomalies above their source bodies.  The RTE map was then subjected to further filtering FVD, AS, TD to improve data interpretation. The FVD was carried out on the RTE map to identify subtle variations in the subsurface, such as lineaments as it enhances near-surface anomalies, The AS was computed from the RTE data to highlights changes in the data’s amplitude, and to detect edges of magnetic sources. Euler deconvolution map of structural index of 1 (SI=1) was obtained to reveal the locations and depths of magnetic sources within the study region. The gravity Bouguer map for the area was processed as in the case of aeromagnetic data interpretation. Regional-residual separation was carried out to isolate the regional component of the gravity field from the local anomalies, derivative (vertical, and tilt) was computed.  The gravity derivatives were computed from the TD which enhance subtle structures and FVD which sharpens shallow density features. As in the case of the magnetic analysis, the Euler deconvolution map of structural index of 1 (SI=1) was computed for the gravity data to obtain depths to source bodies.

All datasets (FVD, TD, AS, and Euler deconvolution) were georeferenced to UTM Zone 32N, WGS84 datum, and resampled to a 30 m spatial resolution using bilinear interpolation. Structural features from each derivative map were extracted and digitized. Structural orientation analysis was finally carried out to compare interpreted structures with known gold occurrences.

### 5. Results

#### 5.1 Gravity Data Interpretation

The Bouguer residual anomaly map (Figure 2) showcases values ranging from -29.4705 mGal in the low-density zones to 50.7770 mGal over high-density basement outcrops. The highest anomalies correspond to the migmatite–gneiss complex and Pan-African granitoids, particularly around Wushishi-Minna- Paikoro corridor, while the lowest anomalies occur along fault-bounded troughs around Kauru, Tafan Jema’a and Tagagi regions, interpreted as zones of deep weathering and alteration.

The gravity FVD map (Figure 3) highlights variations in density contrast within the upper crust revealing subsurface structural features affirming the high density (about 300 mGal/m) observed around the Minna area. The high positive values shown in pink, red, and orange tones correspond to zones of steep density gradients, often associated with faults, fractures, and intrusive bodies which are key features that can control hydrothermal fluid flow and mineralization. Conversely, areas of low values (blue and green) typically represent more homogeneous geological zones or deeper, less variable basement regions.

The gravity TD map (Figure 4) effectively enhances density contrasts, revealing continuous NE–SW and NW-SE trending anomalies, with tilt values ranging from -1.3712 radians to 1.1993 radians with many exceeding ±0.3 radians in tilt angle. These trends extend for more than 25 km in some cases, suggesting regionally persistent structural corridors. The Minna–Kampala–Paikoro corridor, with elevated tilt values, suggests significant crustal breaks or lithological contacts. The Wamba–Jema’a–Keffi region, which shows moderate to high tilt responses, likely corresponding to sheared or fractured terrains, possibly associated with hydrothermal activity and mineralizing fluids. The Lapai–Tagagi–Gurdi area, characterized by linear high tilt anomalies trending in NE–SW and NW–SE directions. These structural trends coincide with regional tectonic fabrics and could act as conduits for mineralization.

The gravity Euler deconvolution map (Figure 5) showcases colour-coded Euler solutions represent varying depths to geological contacts from blue dots (<250 m) representing shallow sources to magenta dots (>2000 m) representing deep sources. The clustering of Euler solutions across the study area delineates zones of intense structural deformation. Notably, regions such as Minna–Paikoro–Abaji, Malagum–Tafan Jema’a, and Wamba–Keffi exhibit a high density of shallow to intermediate depth solutions, suggesting the presence of near-surface fault zones, fracture corridors, and lithological boundaries. The clustering of Euler solutions across the study area delineates zones of intense structural deformation. Notably, regions such as Minna–Paikoro–Abaji, Malagum–Tafan Jema’a, and Wamba–Keffi exhibit a high density of shallow to intermediate depth solutions, suggesting the presence of near-surface fault zones, fracture corridors, and lithological boundaries. The eastern and southeastern regions around Wamba, Malagum, and Kachia are marked by deeper solutions (>1000 m), indicative of subsurface

Figure 2: Bouguer gravity residual map

Figure 3: Gravity First Vertical Derivative (FVD) map

Figure 4: Gravity Tilt Derivative (TD) Map

Figure 5: Gravity Euler Deconvolution map of structural Index of one (SI=1)

intrusions or major crustal structures, potentially acting as conduits for deep-seated hydrothermal fluids.

#### 5.2 Magnetic Data Interpretation

From magnetic RTE map (Figure 6), the magnetic field is seen to range from -43.94 nT to 113.58 nT.  Low magnetic field intensities (< 61.26 nT) is predominant covering about 70% of the study area. High magnetic intensity (> 100 nT) is observed majorly at the central region of the study area covering parts of the Precambrian (basement) rocks as evident on the geology map (Figure.1)

The magnetic FVD map (Figure 7) accentuates shallow magnetic sources, with anomaly magnitudes exceeding 0.04 nT/m in mineralized shear zones. Zones of reduced magnetic intensity observed around places like Bida and Lapai, suggests magnetite destruction during mineralizing events.The magnetic AS map (Figure 8) records amplitudes ranging from -0.105 to 0.169 nT/m, with the highest responses over mafic intrusions near Wushishi, Bosso, Minna, Paikoro, and Abaji regions and lower amplitudes over altered regions like Keffi, Abuja, and Lapai.

The magnetic TD (Figure 9) map highlights variations in the magnetic field associated with geological structures as visible also in Figure 7 and 8. Structures like faults and contacts are precisely identified with the colour scale representing the tilt derivative values, ranging from 1.63 (dark blue) to 1.55 (pink). Except at the central, northern and small other areas like the south west corner of the study area, over 60% of the study area is pronounced by warm colours, suggesting the presence of deeper and less magnetic materials. In the magnetic Euler deconvolution map (Figure 10), At the central part of the map is a notable cluster with a mix of depths (< 1000 m). These shallower anomalies (blue and green points) correspond to surface-level features observed around regions like Minna, Paikoro, Bosso, Abaji, Munya, Wushishi, Rafi. The deeper anomalies (pink and red points) notable majorly as the linear trends running northwest to south indicates complex geological structures favourable for mineral deposition. These deep dense clusters of points represented by the pink and red points (> 1000 m) indicates deeper geological structures that could be associated with mineralization processes.

#### 5.3 Integrated Structural Analysis

Integration of gravity and magnetic derivatives (Figure 11) identifies at least 48 major lineaments, with dominant NE–SW orientations (45% of total) and secondary NW–SE trends (33%). The structural intersections cluster particularly in Minna–Paikoro axis can be said to be associated with gold occurrences as the Abaji corridor as documented by Ayuba and Abu 2024 is said to host multiple artisanal gold workings. The northern Lapai zone is characterized by deep-seated faults inferred from gravity lows coupled with magnetic breaks. Overlaying the structural derivative map on the mineral occurrence map (Figure 12) and metallogenic map (Figure 13), over 72% of known gold occurrences fall within 2 km of these mapped structures,

Figure 6: Reduction to Equator (RTE) map

Figure 7: Magnetic First Vertical Derivative (FVD) map

Figure 8: Magnetic Analytical Signal map

Figure 9: Magnetic Tilt Derivative (TD) Map

Figure 10: Euler Deconvolution map of Structural Index of one (SI = 1)

Figure 11: Gravity and Magnetic structural derivatives extracted FVD, AS and Tilt Derivaratives

Figure 12: Gravity and Magnetic structural derivative overlayed on Mineral resources map of the study area (extracted from the NGSA online map archives)

Figure 13: Gravity and Magnetic structural derivative overlayed on Metallogenic map of the study area (extracted from the NGSA online map archives)

with many located at triple junctions where three or more faults converge. These structural relationships strongly suggest that mineralization is controlled by fault permeability and associated hydrothermal fluid pathways.

### 6.  Discussion

The integrated gravity and magnetic interpretations reveal structural and lithological controls that are consistent with the known metallogenic framework of North-Central Nigeria (Figure 12 and 13). The high Bouguer anomalies (> +50 mGal) observed over migmatite–gneiss complexes and Pan-African granitoids likely reflect dense crystalline basement rocks, which in some cases act as rigid blocks that focus strain along their margins. These margins are often sites of shear zone development, providing pathways for mineralizing fluids, as also documented by Odeyemi et al. (1999) and Anakwuba et al. (2014) in similar Nigerian schist belt settings.

Conversely, gravity lows (< –30 mGal), particularly in Paikoro and Wushishi, correspond to deeply weathered or altered zones where hydrothermal fluids have altered the host rocks and reduced bulk density. Such density reduction, coupled with low magnetic amplitudes (≤ 0.1 nT/m), suggests magnetite destruction due to sulphidation processes — a common feature in orogenic gold systems (Groves et al., 1998).

The tilt derivative and first vertical derivative maps highlight structurally coherent NE–SW and NW–SE trends. The dominance of NE–SW structures (45%) align with regional transcurrent shear zones formed during the Pan-African orogeny (~600 Ma). These shear zones are known to be major gold-bearing corridors across West Africa (Leube et al., 1990; Feybesse et al., 2006). The NW–SE trends (33%) represent cross-cutting faults that act as dilation zones or secondary conduits for gold-bearing fluids.

A key finding from this study is that 72% of documented gold occurrences are located within 2 km of major lineaments, with the highest density of occurrences at structural intersections. This is significant because such junctions often localize ore due to enhanced permeability and fluid focusing (Robert et al., 1997). The Paikoro axis is one of the regions documented by Jonah and Mallam (2024) to possess a high K/Th ratio. High K/Th ratio and strong gravity gradients, indicate intense alteration adjacent to deep-seated faults, and these conditions are favourable for gold deposition.

### 7. Conclusion

This study demonstrates that the integration of gravity and magnetic derivative modelling provides an effective framework for delineating structural controls on gold mineralization in North-Central Nigeria. The results reveal that NE–SW and NW–SE trending faults, identified from tilt and vertical derivative maps, act as primary conduits for hydrothermal fluids, with structural intersections being the most prospective zones. The spatial association between negative gravity anomalies, low magnetic responses, and documented gold occurrences underscores the importance of alteration-related density and magnetic susceptibility reduction in mineralizing processes. Approximately 72% of known gold occurrences were found within 2 km of major lineaments, highlighting the significance of structural proximity in exploration targeting.

# Competing interests disclaimer

Authors have declared that they have no known competing financial interests OR non-financial interests, or personal relationships that could have appeared to influence the work reported in this paper.

REFERENCES

Abdelrady, M., Moniem, M. A., Alarfi, S. S., Abdelrady, A., Othman, A., Mohammed M. A. A., & Mohamed, A., (2023). Geophysical investigation for the identification of subsurface features influencing mineralization zones. Journal of King Saud University-Science, 35(7),  https://doi.org/10.1016/j.jksus.2023.102809

Ahmed, S. A., Nuhu, M. W., Hadiza, L. M., Mohammed, A. M., & Apeh, A. G., (2020). Reconnaissance geochemical and geophysical exploration for gold at Iri gold field, North central Nigeria.  Scientia Bruneiana, 18(2), pages 19-31.

Ajibade, A. C., Anyanwu, N. P. C., Okoro, A. U., & Nwajide, C. S. (2008). The geology of the Minna area (Bulletin No. 43). Nigerian Geological Survey Agency.

Akinduko, O. K., Salako, K. A., Alhassan, U. D., Lawrence, J. O., Alkali, A., Aliyu, S. B., Abdullahi, M., Adewumi, T., & Damidami, L., (2022). Aero-magnetic and Aero-radiometric prospecting for Gold Mineralisation over Ilesha and its Environs Southwest, Nigeria. Asian Journal of Geological Research (AJOGER), 5(4), pp. 1-16; Article no. AJOGER 96016

Akintola, O. A., Olatunji, A. S., & Adepoju, K. A. (2022). Geology and mineral resources of North-Central Nigeria: A review. Journal of African Earth Sciences, 191, 104520. https://doi.org/10.1016/j.jafrearsci.2022.104520

Aliyu, S. B., Adetona, A. A., Rafiu, A. A., Ejepu, J., Adewumi, T., (2021). Delineating and interpreting the gold veins within Bida and Zungeru area, Niger State, Nigeria, using aeromagnetic and radiometric data. Pakistan Journal of Geology (PJG), 5(2), pp 41-50. DOI: 10.2478/pjg-2021-0006

Anakwuba, E. K., Chinwuko, A. I., Onwuemesi, A. G., & Oha, I. A. (2014). Interpretation of aeromagnetic data over Maiduguri and environs for structural and mineral prospectivity analysis. International Journal of Science and Technology, 3(1), 67–79.

Arogundade A. B., Awoyemi, M.O., Hammed, O.S. Falade, S.C., & Ajama O. D. (2022). Structural Investigation of Zungeru-Kalangai Fault Zone, Nigeria using Aeromagnetic and Remote Sensing Data. Heliyon, 8(3), e09055. https://doi.org/10.1016/j.heliyon.2022.e09055

Augie, A. I., Salako, K. A., Rafiu, A. A. & Jimoh, M. O. (2024). Integrated Geophysical Investigation for Potential Gold Mineralised Zone within Lower Part of Zuru Schist Belts, NW Nigeria. Nigerian Journal of Physics (NJP), 33(2). ISSN online: 3027-0936ISSN print: 1595-0611. DOI: https://doi.org/10.62292/njp.v33i2.2024.212, Special Issue, June 2024

Ayuba Bwamba Jonah, & Abu Mallam (2024). Integrated interpretationof Aeromagnetic and Aero-radiometric data to delineate structures and hydrothermal Alteration zones associated with gold Mineralization in parts of North-Central Nigeria. Asian Journal of Geological Research (AJOGER) 7(2): 136-153

BGI - Bureau Gravimetrique International / International Gravimetric Bureau, World Gravity Map (WGM) (2012). WGM2012 maps visualization/extraction – BGI

Bolarinwa, A. T., Oyebanjo, O. A., & Ojo, A. M. (2021). Structural framework and geochemical characteristics of mineralized zones in Nigeria. Minerals, 11(12), 1396. https://doi.org/10.3390/min11121396

Daniel, E., Jimoh, R., & Lawal, K. (2019). Delineation of Gold Mineral Potential Zone Using High Resolution Aeromagnetic Data Over Part of Kano State, Nigeria. Journal of Geology & Geophysics 8:464. 10.35248/2381-8719.464

Ebele, J. E., Ofoegbu, C. O. & Nur, A. (2021). Interpretation of high-resolution aeromagnetic and radiometric data for delineation of mineral potential zones over Abuja and Environs, North-Central Nigeria. Arabian Journal of Geosciences, 14(1947) https://doi.org/10.1007/s12517-021-07915-5. Corpus ID: 237412613

Egbelehulu, P., & Abu, M. (2024). Integration of ground magnetic method and whole-rock analysis for solid mineral prospecting in a part of Abuja, Nigeria. Pakistan Journal of Geology (PJG), 8(1), 67-73. DOI: http://doi.org/10.26480/pjg.01.2024.67.73

Feybesse, J. L., Billa, M., Guerrot, C., Duguey, E., Lescuyer, J. L., Milesi, J. P., & Bouchot, V. (2006). The Paleoproterozoic Ghanaian province: Geodynamic model and ore controls, including regional stress modeling. Precambrian Research, 149(3–4), 149–196. https://doi.org/10.1016/j.precamres.2006.06.003

Groves, D. I., Goldfarb, R. J., Gebre-Mariam, M., Hagemann, S. G., & Robert, F. (1998). Orogenic gold deposits: A proposed classification in the context of their crustal distribution and relationship to other gold deposit types. Ore Geology Reviews, 13(1–5), 7–27. https://doi.org/10.1016/S0169-1368(97)00012-7

Groves, D. I., Goldfarb, R. J., Santosh, M., & Zhang, L. (2016). The conjunction of factors that lead to formation of giant gold provinces and deposits in non-arc settings. Geoscience Frontiers, 7(3), 303–314. https://doi.org/10.1016/j.gsf.2015.07.001

Ishola, K. S., Akerele, P. O., Folarin, O., Adeoti, L., Adegbola, R. B., Adeogun, O. Y. (2020). Application of aeromagnetic data to map subsurface structural features in Ewekoro, Southwestern Nigeria. Modeling Earth Systems and Environment. pp. 1 - 12 https://doi.org/10.1007/s40808-020-00812-y

Leube, A., Hirdes, W., Mauer, R., & Kesse, G. O. (1990). The Early Proterozoic Birimian Supergroup of Ghana and some aspects of its associated gold mineralization. Precambrian Research, 46(1–2), 139–165.

Mustapha S., Suleman S., Iliyasu S. R., Udensi E. E., Sanusi Y. A., Dahuwa D. and Abba L (2020) Interpretation of Aeromagnetic Data and Landsat Imagery over the Nigerian Younger Granites in and around Kafanchan Area, North-Central Nigeria. FUDMA Journal of Sciences, 4(4), pp 323 – 333.  DOI: https://doi.org/10.33003/fjs-2020-0404-489

Nigeria Geological Survey Agency (NGSA) – Geological Maps. https://ngsa.gov.ng/geological-maps/

Obaje, N. G. (2009). Geology and Mineral Resources of Nigeria. Springer.

Oden, M. I., Okpamu, E. A., & Amah, E. A. (2012). Comparative analysis of fracture lineaments in Oban and Obudu areas, Southeastern Nigeria. Journal of Geography and Geology, 4(2), 36–47. https://doi.org/10.5539/jgg.v4n2p36

Ogunmola, J. K., Gajere, E. N., Ayolabi, E. A., Olobaniyi, S. B., Jeb, D. N., Agene, I. J. (2015). Structural study of Wamba and Environs, North-central Nigeria using Aeromagnetic data and NigeriaSat-X image. Journal of African Earth Sciences, 111(1), pp. 307-321. https://doi.org/10.1016/j.jafrearsci.2015.07.028

Rahaman, M. A. (1988). Recent advances in the study of the Basement Complex of Nigeria. In Precambrian Geology of Nigeria (pp. 11-41). Geological Survey of Nigeria.

Reid, A. B., Allsop, J. M., Granser, H., Millet, A. J. & Somerton, I. W. (1990). Magnetic interpretation in three dimensions using Euler deconvolution. Geophysics, 55(1), pp 80-91.

Robert, F., Poulsen, K. H., Cassidy, K. F., & Hodgson, C. J. (1997). Gold deposits and their geological classification. In R. V. Kirkham, W. D. Sinclair, R. I. Thorpe, & J. M. Duke (Eds.), Mineral deposit modeling (pp. 199–214). Geological Association of Canada.

Salako, K.A., Adetona, A. A., Rafiu, A. A., Augie, A. I., Jimoh, M. O., Alkali, A., Muriana, R. A. Lawrence, J. O., (2024). Integrated geophysical Investigation for gold mineralization potential over southern parts of Kebbi state and its Environs, northwestern Nigeria. Heliyon 10 (2024) e34093 https://doi.org/10.1016/j.heliyon.2024.e34093.

Telford, W. M., Geldart, L. P., & Sheriff, R. E. (1990). Applied geophysics (2nd ed.). Cambridge University Press. https://doi.org/10.1017/CBO9781139167932

U,S. Geological Survey, 2024, World Geologic Maps, accessed May 6, 2024 at https://www.usgs.gov/media/images/world-geologic-maps-data-downloads