Beeswax as a Bioindicator of Heavy Metal and Pesticide Residue in Commercial Apiaries

ABSTRACT

The bioactive compounds, minerals, and trace elements that constitute honey products can be contaminated by heavy metals in the environment, which directly affect the quality of bee products. Heavy metal contamination may disrupt the bee colony. This study aims to assess beeswax as a bioindicator of heavy metal and pesticide residues in commercial apiaries. This study sought to assess the concentration of heavy metal and pesticide residue from beeswax collected from Awka, Amizi, Ezzamgbo and Saki. The study was carried out at the Springboard Research Centre, Awka, Anambra State. The honey beeswax samples were collected from four different states in Nigeria: Saki in Saki West L.G.A of Oyo State, which falls within the geographical coordinates of Ifite Awka in Anambra State, Amizi in Abia State and Ezzamgbo in Ebonyi State. Atomic Absorption Spectrophotometer (AAS) and GC-FID were used for analysis and quantification of samples. From the study, beeswax from Awka had the highest concentration of Cadmium and Iron, with values (1.211 mg/kg) and Iron (0.193 mg/kg), while the Saki area of Oyo was observed to have the highest concentration of Zinc and Chromium, with the concentration of 2.215 mg/kg. and 0.034 mg/kg, respectively. Ezzamgbo had the least concentration of all the metals analysed. Generally, there was a significant difference between the total mean concentrations of all heavy metals sampled (P<0.05) and the recommended standard by FAO/WHO, except in cadmium (P>0.05). Different mean pesticides residue were checked and Ezzamgbo recorded the highest values of Chlorophyrifos (0.108 mg/kg), Dichlorovos (0.464 mg/kg), Dicophol (0.282 mg/kg), Dieldrin (0.064 mg/kg), Profeneros (0.321 mg/kg) and T-nonachlor (0.103 mg/kg) and beeswax sampled from Saki had the highest values of  Dianzinon (0.003 mg/kg) and  Endosulfan (0.032 mg/kg) while 2_4 DDT (0.243 mg/kg), Heptachlor (0.321 mg/kg) and Lindane (0.318 mg/kg) were highest in Amizi. Statistically, there was a significant difference in the levels of the pesticide residue components recovered from the beeswax sampled in the various locations at P<0.05, except in g-chlordane and Profeneros (P>0.05).  The result further showed that all concentrations of the various components of the pesticide residues were below the recommended limit by FAO except 2_4 DDT, Carbofuran, Dichlorovos, Dicophol, Heptachlor and Lindane. The findings of this study posit that while beeswax from Awka and Saki had the highest metal concentration, Ezzamgbo and Amizi were more contaminated with Pesticide residue.  From this study, it is evident that heavy metals are prevalent in Awka and Saki, which is predisposed to metals could be due to much industrialisation. Beeswax from Ezzamgbo had the least contraction of metals. The government should develop adequate measures to mitigate the pollution caused by pesticides.

Keyword: Heavy Metals, Pesticide Residues, Beeswax, Beekeepers

INTRODUCTION

“Several insects in nature create wax, but bees and other Apoidea are responsible for the production of wax that is greatly appreciated and utilized by humans” (Papa et al., 2022). “Bees produce beeswax, which is used to make honeycombs. It is biodegradable, sustainable, compostable, eco-friendly” (Gupta & Anjali,2023; Saralaya et al.,2021). “Beeswax is produced by the two most widely cultivated bee species, Apis mellifera and A. cerana. Beeswax is produced by younger worker bees (between the ages of 12 and 18 days, when they have completed their nursing duties) and is a complex material that is available in liquid form” (Giampieri et al., 2022). “Beeswax is made by mixing beeswax, pollen, and enzymes with resin from tree buds. It is a naturally occurring product secreted from worker bees that has varied uses in modern day” (Nong et al.,2023). Therefore, “Beeswax can then be used as a substitute for wax. There are several practical applications for beeswax, which include embalming, the preservation of papyrus, the conservation of artworks, the manufacture of candles and cult artefacts, as well as its usage as a medicinal component and adhesive ingredient” (Gupta and Anjali, 2023). “As well as being a film-forming material, beeswax has also been reported to exhibit several therapeutic properties, e.g., wound healing, anti-stress, anti-inflammatory, antioxidant, and antimicrobial activities” (Hosseini et al.,2023; Tu et al.,2024). Also, “it acts as a UV radiation barrier to keep the skin from being corroded or scratched, and it keeps moisture from escaping. Additionally, beeswax serves as a source of nest-mate identification clues for honey bee colonies because of its unique chemical composition” (Dumitru et al., 2022). “Beeswax is commonly used in the food business as a food additive and to extend the shelf life of goods, and it is also widely employed in the pharmaceutical and nutraceutical sectors” (Gupta and Anjali, 2023). Although there have been some improvements, and concerns regarding contamination and adulteration of high-quality beeswax.

“The health risks associated with beeswax stem from the presence of pesticide residues (including active substances and their metabolites), antibacterial substances (including antibiotics), and heavy metals” (Piven et al., 2020). “Beeswax may be used to detect the presence of metals and pesticides in soil, water, and plants as a bio-indicator of pollution. Due to their unique lipid-based chemical composition, beeswax combs act as a reservoir for a variety of environmental pollutants and toxins” (Ngat et al., 2020; Ćirić et al., 2021). The bioactive compounds, minerals and trace elements that constitute honey products can be contaminated by heavy metals occurring in the environment, directly affecting the quality of bee products. Heavy metal contamination may disrupt the activity of the bee colony. “These substances come mainly from the environment, whose cleanliness depends largely on human activity” (Oymen et al., 2022). “Antibacterial substances are used to prevent and fight diseases in broods and bees, and pesticides are meant to protect crop plants against fungi (fungicides), insects (insecticides), and weeds (herbicides). The use of pesticides in agriculture is essential to obtaining high yields, but results in the contamination of the soil, water, air, and also the flowers from which bees collect nectar and pollen, the natural components of honey” (Oymen et al., 2022). “These agents negatively affect both bees and people, causing changes in the endocrine and nervous systems” (Hernández et al., 2016). “Heavy metals can have a variety of hazardous consequences for consumers and bees and depending on how polluted the air, water, and soil” (Ullah et al., 2022).

“Long-term exposure to high levels of heavy metals may cause kidney damage, high lead levels in the blood, and carcinogenic risks for adults and children” (Mao et al. 2019). “Heavy metals affect bee development, brood rearing, adult bee lifespan, pollen, nectar, and honey storage, and consumption behaviours” (Burden et al. 2019). Since “heavy metals can accumulate over a long period of time, beekeepers regularly remove old, dark, and damaged combs during the active beekeeping season and transport them to the craft unit for processing into fresh wax foundations” (Svečnjak et al. 2019). “Heavy metals are transported to the hive as a result of bee physiological activity or human activity during routine beekeeping procedures” (Borsuk et al. 2021). This is now one of the most important challenges relating to beeswax quality, accounting for around 60% of all problems. These challenges include assessing the extent of contamination of beeswax by pesticides and heavy metals. Therefore, this study seeks to assess beeswax as a bioindicator of heavy metal and pesticide residue in commercial apiaries.

## MATERIALS AND METHODS

Study Area and Duration of Study

The study was carried out at Springboard Research Centre, Awka, Anambra State, which fall within the geographical coordinates of latitudes of 6.2220° N and 7.0821° E (Longitude) and stretches in an east-west direction along Enugu-Onitsha expressway and about 5km in a North-South orientation. It experiences dry and wet seasons dry and wet season and a mean annual rainfall of 2.169.8 mm, and the highest month of rainfall is September, having an average temperature of 29°C and 34°C.

Collection of Beeswax

The honey beeswax sample were collected from four different states in Nigeria: Saki in Saki west L.G.A of Oyo State which fall within the geographical coordinates (Latitude: 8.6033° N, Longitude: 3.1378° E), Ifite Awka in Anambra State (Latitude: 6.2404° N,   Longitude: 7.0902° E), Amizi in Abia State (Latitude: 5.4682° N, Longitude: 7.5024° E) and Ezzamgbo in Ebonyi State (Latitude: 6.3994° N and Longitude: 7.9484° E).

Procedures for Digestion for Heavy Metal Analysis

Two grams (2g) of the sample was weighed into a digestion flask and add 20ml of the acid mixture aqua regia (65ml conc NHO3; 8ml perchloric acid and 2ml conc H2SO4). Heat the flask until a clear digest is obtained. Dilute the digest with distilled water to the 100ml mark.

Working Principle of Atomic Absorption Spectrophotometer (AAS)

The atomic absorption spectrometer's working principle is built basically on the sample being aspirated into the flame and atomised when the AAS's light beam from the monochromator is directed through the flame onto the detector that measures the amount of light absorbed by the atomised element in the flame when ignited. Metal elements on their own have distinct characteristic wavelengths in the source hollow cathode lamp, composed of the elements/metals to be analysed. The amount of energy of the characteristic wavelength absorbed in the flame is directly proportional to the concentration of the element in the tested sample. The sample is aspirated into the oxidising air-acetylene flame. When the aqueous sample is aspirated, the sensitivity for 1% absorption is observed. Soxhlet Extraction of oil from the Beeswax samples 250ml round bottom boiling flask was dried in an oven at 105-110oC for about 15 minutes and transferred into a desiccator and allowed to cool. The 250 ml flask was filled with n-hexane solvent. The weighed sample (20g) was inserted into the thimble of the Soxhlet apparatus with cotton wool underneath to serve as a filter. The apparatus was assembled on the boiling flask of the Soxhlet apparatus and allowed to stand on an electric hot plate at a temperature of 60-75℃, and allowed to reflux about 4 times for five repeated extractions. Extract from the flask was collected and emptied into a rotatory evaporator at a temperature of 40-60℃ to separate the n-hexane solvent from the extracted oil. The extracted oil was collected and stored in a container.

3.3.1 Preparation of sample for Pesticide Residues

Five millilitres 5ml of the oil sample was pipette into a beaker containing 20ml of N-hexane, shaken very well and poured into a separating funnel and allowed to stand for 30 minutes. The N-hexane layer was collected and stored in a sample vial for pesticide residue Analysis with GC-FID.

Quantification with GC-FID

The analysis of the PCB profile was performed on a BUCK M910 Gas chromatography equipped with a flame ionisation detector. A RESTEK 15-meter MXT-1 column (15mx 250u mx0.15um) was used. The injector temperature was with splitless injection of 2µl of sample, and the oven was operated initially at 200℃, it was heated to 330°C at a rate of 3°c min, and was kept at this temperature for 5 minutes. the detector operated at a temperature of 320℃. PCB components were determined by the ratio between the area and mass of the internal standard and the area of the identified compounds. The concentration of the different amino acids expressed in ug/ml and percentage. Fixed Setting: In addition, the injector and detector temperatures were set. The detectors are generally held at the high end of the oven temperature range to minimise the risk of analyte precipitation. All of these parameters were set to the correct values, but double check all the instrument: Buck 530 gas chromatograph were equipped with an on-column, automatic injector, Flame Ionization detector, HP88 capillary column (100m x 0.25μm film thickness,) CA, USA Detector Temperature A:250°C Injector temperature 22°C Integrator chart speed: 2cm/min Set the OVEN TEMP to 180C and was allowed to warm up. When the instrument was ready, the "NOT READY” light turned off, and the Run began. 1 microliter sample was injected onto column A using proper injection technique.

Statistical Analysis

The data collected from the study was subjected to Analysis of variance using the SPSS version software 25 program and the statistical significance was estimated at 95% confidence level. The sample mean were separated using Turkey-HSD test.

RESULTS

Heavy Metal Concentrations of Beeswax Samples Collected From Different Locations

The results of the mean heavy metal concentrations of four beeswax samples collected from different locations in Nigeria are presented in Table 1. Table 1 showed that zinc concentration was highest in Saki (2.215 mg/kg) while least in Ezzamgbo (0.320 mg/kg). Cadmium concentration was highest in Awka (1.211 mg/kg) while Saki had the least (0.028 mg/kg). However, iron was highest in Awka (0.193 mg/kg) while Ezzamgbo had the least (0.020 mg/kg). Chromium concentration was highest in Saki (0.034 mg/kg) while Ezzamgbo had the least (0.020 mg/kg).  Statistically, there was a significant difference in the iron concentrations among the four locations (P<0.05) while no significant difference exists in the zinc, cadmium and chromium (P>0.05). The result in Table 1 further showed that the total mean concentrations of zinc exceeded the permissible limit by FAO/WHO, while those of cadmium, iron, chromium and nickel were below the recommended standard. There was a significant difference between the total mean concentrations of all heavy metals sampled (P<0.05) and the recommended standard by FAO/WHO, except in cadmium (P>0.05).

Table 1: Mean concentrations of heavy metals in beeswax sampled from four locations

| Locations | Mean concentration of Heavy metals (mg/kg) ±SD |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Zinc | Cadmium | Iron | Chromium | Nickel |
| Amizi | 0.439a±0.211 | 0.267a±0.071 | 0.115bc±0.017 | 0.024a±0.015 | ND |
| Awka | 2.046a±1.06 | 1.211a±0.663 | 0.193c±0.024 | 0.024a±0.004 | ND |
| Ezzamgbo | 0.320a±0.113 | 0.444a±0.191 | 0.020a±0.028 | 0.020a±0.006 | ND |
| Saki | 2.215a±1.207 | 0.028a±0.025 | 0.062a±0.005 | 0.034a±0.004 | ND |
| Total | 1.255±1.122 | 0.487±0.542 | 0.097±0.071 | 0.025±0.009 | ND |
| Permissible Limit (mg/kg) | 0.3 | 0.3 | 5.0 | 0.10 | 0.10 |
| Remark | NS | S | S | S | S |

Columns sharing similar superscripts are not significantly different at P>0.05

NS= Not satisfactory; S= satisfactory; ND= Not Detected

Pesticide Residue in Beeswax Samples Collected from Four Locations in Nigeria

The results of the mean pesticide residue concentrations of four beeswax samples collected from different locations in Nigeria are presented in Table 2. Table 2 showed that the concentration of 2_4 DDT (0.243 mg/kg), Heptachlor (0.321 mg/kg) and Lindane (0.318 mg/kg) were highest in Amizi. Aldrin (0.344 mg/kg), carbofuran (0.435 mg/kg), and g-chlordane (0.059 mg/kg) were highest in Awka. The result further showed that Ezzamgbo recorded the highest values of Chlorophyrifos (0.108 mg/kg), Dichlorovos (0.464 mg/kg), Dicophol (0.282 mg/kg), Dieldrin (0.064 mg/kg), Profeneros (0.321 mg/kg) and T-nonachlor (0.103 mg/kg) while beeswax sampled from Saki had the highest values of Dianzinon (0.003 mg/kg) and Endosulfan (0.032 mg/kg). Statistically, there was a significant difference in the levels of the pesticide residue components recovered from the beeswax sampled in the various locations at P<0.05, except in g-chlordane and Profeneros (P>0.05).  Table 2 further showed that all concentrations of the various components of the pesticide residues were below the recommended limit by the FAO limit except 2_4 DDT, Carbofuran, Dichlorovos, Dicophol, Heptachlor and Lindane.

Table 2: Pesticide residue in Beeswax Samples Collected from Four Locations in Nigeria

| Locations | Mean concentration of pesticide residue of beeswax sampled from different locations(mg/kg) ±SD |  |  |  | Total | Limit | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Awka | Amizi | Ezzamgbo | Saki |  |  |  |
| 2_4- DDT | 0.126b±0.001 | 0.243d±0.002 | 0.225c±0.001 | 0.001a±0.000 | 0.149±0.103 | 0.01 | NS |
| Aldrin | 0.344b±0.063 | 0.03a±0.001 | 0.085a±0.015 | 0.000a±0.000 | 0.115±0.147 | 0.2 | S |
| Carbofuran | 0.435d±0.002 | 0.001a±0.000 | 0.108b±0.001 | 0.319c±0.001 | 0.216±0.183 | 0.06 | NS |
| Chlorophyrifos | 0.000a±0.000 | 0.000a±0.000 | 0.108b±0.002 | 0.000a±0.000 | 0.027±0.050 | 1 | S |
| Dianzinon | 0.000a±0.000 | 0.000a±0.000 | 0.000a±0.000 | 0.003b±0.001 | 0.001±0.001 | 0.01 | S |
| Dichlorovos | 0.099a±0.002 | 0.203b±0.014 | 0.464c±0.047 | 0.108ab±0.001 | 0.218±0.159 | 0.1 | NS |
| Dicophol | 0.000a±0.000 | 0.002a±0.001 | 0.282b±0.051 | 0.000a±0.000 | 0.071±0.132 | 0.002 | NS |
| Dieldrin | 0.016a±0.007 | 0.030b±0.002 | 0.064c±0.000 | 0.012a±0.000 | 0.030±0.022 | 0.2 | S |
| Endosulfan | 0.005a±0.004 | 0.000 a±0.000 | 0.000a±0.000 | 0.032b±0.000 | 0.009±0.014 | 5 | S |
| g-chlordane | 0.059a±0.068 | 0.000a±0.000 | 0.000a±0.000 | 0.012a±0.001 | 0.018±0.037 | 0.1 | S |
| Heptachlor | 0.000a±0.000 | 0.321c±0.001 | 0.000a±0.000 | 0.084b±0.000 | 0.101±0.141 | 0.01 | NS |
| Lindane | 0.002a±0.001 | 0.318b±0.002 | 0.000a±0.000 | 0.000a±0.000 | 0.08±0.147 | 0.02 | NS |
| Profeneros | 0.154a±0.069 | 0.150a±0.174 | 0.321a±0.000 | 0.235a±0.001 | 0.215±0.103 | 0.5 | S |
| T-nonachlor | 0.000a±0.000 | 0.011a±0.000 | 0.103b±0.009 | 0.013a±0.000 | 0.032±0.044 | 0.04 | S |

Rows sharing similar superscripts are not significantly different at P>0.05

NS= Not satisfactory; S= satisfactory

DISCUSSION

One of the most prevalent heavy metals in the Earth's crust, zinc (Zn) is said to be a vital trace metal needed for life's development. It is a cofactor of important proteins and mediates the control of a number of immunomodulatory processes (Hussain et al., 2022). Zinc is necessary for many body processes, but too much of it has also been linked to immune system and gastrointestinal flora changes (Skalny et al., 2021). Higher amounts of zinc can be harmful to cells, which frequently leads to the disruption of vital biological processes by misbinding with other metals to block protein thiols (Marchetti, 2013; Chandrangsu et al., 2017). All of the beeswax samples in this study contained zinc, but Saki appeared to have the highest concentration (2.215 ±1.207), followed by Awka (2.046 ±1.06). This is higher than Omran et al. (2019), who reported that zinc contents ranged from 0.185 to 0.241 mg/kg. The greatest Zn values in four regions were 3.975, 8.266, 7.052, and 1.122 (mg/g), according to Aljedani et al. (2020). Except for the Zn concentration (1.122 mg/g) in the fourth region reported by Aljedani et al. (2020), which is lower than the maximum Zn concentration in this study, the Zn concentration reported by Aljedani et al. (2020) was greater than the highest Zn concentration observed in this study.

The mean iron level in the current study was between 0.193 to 0.020 mg/kg, which is lower than the range of 2.068 to 5.041 ppm reported by Hassona et al. (2023). Additionally, the iron level was also 56.470 to 285 ppm (Gajger et al., 2019). The Fe concentration from the several ambient sources ranged between 1.080 and 334 µg/g (Formicki et al. 2013) and 5.972 and 18,516 mg/g (Aljedani, 2020). All of which exceeded the iron content found in our investigation. Fe is necessary for the synthesis of haemoglobin, the transportation of oxygen, and the action of a number of enzymes. Fe metabolism abnormalities, which can range from anaemia to Fe overload and possibly neurological disease, are among the most common ailments in humans (Abbaspour et al. 2014).

Although cadmium is naturally occurring in the environment, it is also produced by industrial and agricultural processes such as smelting copper and nickel, burning fossil fuels, and making phosphate fertilisers, PVC products, and colour pigments (Gnechi et al., 2020). Cadmium exposure mostly happens through consuming tainted food and water, but it can also happen through inhalation and smoking (Taha et al., 2018). Cadmium is considered carcinogenic to humans by the International Agency for Research on Cancer (IARC, 2023). Numerous cancers, including those of the liver, kidneys, breast, lung, and prostate, may be linked to exposure to this metal (Gnechi et al., 2020). In this study cadmium was found in every sample, with Awka having the highest concentration (1.211 mg/kg) of all of the study samples. This study's maximum cadmium concentration was lower than that of Aljedani et al. (2020), who found a maximum cadmium concentration of 0.075 mg/kg. Additionally, the Cd content found in this study was higher than the Cd value (0.194) mg/kg reported by Omran et al. (2019). The city's location, which includes multiple main roads with piles of corroded metals from accident vehicles and mechanic shops, may be linked to the high content of cadmium. This could be the cause of the metal poisoning of the honey. No nickel was found in any of the samples; this could be because materials that could pollute those locations are not easily manufactured there.

The study's findings reveal that Ezzamgbo had the highest levels of T-nonachlor (0.103 mg/kg), Profeneros (0.321 mg/kg), Dieldrin (0.064 mg/kg), Dichlorovos (0.464 mg/kg), Dicophol (0.282 mg/kg), and Chlorophyrifos (0.108 mg/kg), all of which were equal to the EU threshold value. The range of atrazine results obtained falls within the range reported by (Blasco et al., 2003). Farmers in Ebonyi State frequently use these herbicides, particularly in Ezzamgbo, where rice and cassava are produced in considerable quantities. All other results exceeded the European Union's maximum pesticide residue level of 0.01 mgkg−1 in foods. In Ngwa village, lindane is a pesticide that is frequently used on plants and to control insects in both urban and rural regions. All of the results were higher than the 0.01 mg·kg−1 that the European Union recommends, which suggests that they were used excessively on farms and have the potential to endanger ecosystem health. Report on analysis of 2_4 DDT (0.243 mg/kg), Heptachlor (0.321 mg/kg) and Lindane (0.318 mg/kg) were highest in Amizi.

CONCLUSION

Contamination of beeswax by pesticides is very harmful to the body. When beeswax is contaminated, it could pose a threat to both bees and humans. From this study, it is evident that heavy metals are prevalent in Awka and Saki, which is predisposed to metals could be due to much industrialisation. Beeswax from Ezzamgbo had the least contraction of metals. Ezzamgbo had the highest concentration of pesticides. This may have been caused by many agricultural practices, for which the area is known among the four locations. The government should develop adequate measures to mitigate the issue of pollution caused by pesticides.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

REFERENCES

Abbaspour, N., Hurrell, R. and Kelishadi, R. (2014). Review On Iron and Its Importance for Human Health. J Res Med Sci 19:164–174.

Aljedani, D. M. (2020). Revealing Some Elements and Heavy Metals in Honeybee and Beeswax Samples Collected from Different Environments. Entomology and Applied Science Letters, 7:89-101.

Blasco, C., Fernández, M., Pena, A., Lino, C., Silveira, M. I., Font, G. and Picó, Y. (2003). Assessment of Pesticide Residues In Honey Samples From Portugal And Spain. Journal of Agricultural and Food Chemistry, 51(27), 8132–8138.

Chandrangsu, P., Rensing C. and Helmann J. D. (2017). Metal Homeostasis and Resistance in Bacteria. Nature Reviews Microbiology, 15:338–350.

Formicki G, Greń A, Stawarz R, Zyśk, B. and Gał, A. (2013). Metal content in honey, propolis, wax, and bee pollen and implications for metal pollution monitoring. Pol J Environ Stud 22:99–106

Genchi, G., Sinicropi, M.S., Lauria, G., Carocci, A. and Catalano, A. (2020). The Effects of Cadmium Toxicity. Int J Environ Res Public Health. May 26;17(11):3782.

Hassona, N. M. and El-Wahed, A. A. A. (2023). Heavy Metal Concentrations of Beeswax (Apis mellifera L.) at Different Ages. Bulletin of Environmental Contamination and Toxicology, 111(3): 26.

Hussain S, Khan M, Sheikh TMM, Mumtaz MZ, Chohan TA, Shamim S, Liu Y. Zinc Essentiality, Toxicity, and Its Bacterial Bioremediation: A Comprehensive Insight. Front

IARC https://monographs.iarc.who.int/list-of-classifications (2023), Accessed 11th Mar 2023

Marchetti, C. (2013). Role of Calcium Channels in Heavy Metal Toxicity. International Scholarly Research Notices, 1–9.

Omran, N.S., Omar, M.O.M., Hussein, M.H. and Mm, A.A. (2019). Heavy Metals Concentrations in Bee Products Collected from Contaminated and Non-Contaminated Areas from Upper Egypt Governorates. International Journal of Research Studies in Biosciences, 7(2), 24-31.

Skalny, A.V., Aschner, M., Lei, X.G., Gritsenko, V.A., Santamaria, A. and Alekseenko, S. I. (2021). Gut Microbiota as A Mediator of Essential and Toxic Effects of Zinc in The Intestines and Other Tissues. International Journal of Molecular Sciences, 22:13074–13090.

Taha, M. M., Mahdy-Abdallah, H., Shahy, E. M., Ibrahim, K. S. and Elserougy, S. (2018). Impact of Occupational Cadmium Exposure On Bone in Sewage Workers. International Journal of Occupational and Environmental Health, 24(3-4), 101–108.

Gupta, G., & Anjali, K. (2023, February). Environmentally friendly beeswax: properties, composition, adulteration, and its therapeutic benefits. In IOP Conference Series: Earth and Environmental Science (Vol. 1110, No. 1, p. 012041). IOP Publishing.

Saralaya, S., BS, J., Thomas, N. S., & SM, S. (2021). Bee wax and honey—a primer for OMFS. Oral and Maxillofacial Surgery, 25(1), 1-6.

Nong, Y., Maloh, J., Natarelli, N., Gunt, H. B., Tristani, E., & Sivamani, R. K. (2023). A review of the use of beeswax in skincare. Journal of Cosmetic Dermatology, 22(8), 2166-2173.

Hosseini, S. F., Mousavi, Z., & McClements, D. J. (2023). Beeswax: A review on the recent progress in the development of superhydrophobic films/coatings and their applications in fruits preservation. Food chemistry, 424, 136404.

Tu, X., Du, C., He, Y., Yang, J., Chen, J., Jin, Q., ... & Chen, W. (2024). Determination of bisphenols in beeswax based on sugaring out–assisted liquid–liquid extraction: Method development and application in survey, recycling and degradation studies. Chemosphere, 351, 141274.