Study the economic analysis of Amaranthus for different lining materials of farm ponds in Konkan region.

Abstract:

The Konkan region’s specific challenges, such as high rainfall and poor water retention in soil, farm ponds with durable and efficient linings are indispensable for maximizing the benefits of rainwater harvesting. The effectiveness of farm ponds depends significantly on the choice of lining material. Lining ensures that water stored during the rainy season remains available for agricultural use throughout the year. With climate change causing erratic rainfall patterns, lined ponds provide a more reliable water storage solution, supporting sustainable agriculture and improving farmers' resilience against water scarcity. The study evaluate the economic viability of different materials, helping farmers choose the best options based on durability, cost, and maintenance requirements. This study is necessary for selecting durable and eco-friendly materials. The study contributes to study the economics of lining material. The field experiment is conducted during Rabi season of 2017-18, at the Instructional Farm, Department of Soil and Water Conservation Engineering, College of Agricultural Engineering and Technology, Dr. Balasaheb Sawant Konkan Krishi Vidyapeeth, Dapoli. In treatment 3 lining material had mechanical properties like puncture resistance, busting strength, ash content, seam strength and carbon black content showed better results and low costas compared to lining material used in treatment 1 and treatment 2. The benefit cost (B.C.) ratio was worked out by dividing gross returns with to total expenditure incurred for production. Standard procedure adopted to calculate per hector cost of cultivation for Amaranths by following the recommended package of practices. The lowest input cost of Rs. 1,64,226 and a total cost of Rs. 2,85,385. The gross returns reached Rs. 5,82,500, yielding net returns of Rs. 4,18,274 at input cost and Rs. 2,97,115 at total cost. The B:C ration was highest in Treatment 3.

Keywords:  B:C ratio, Amaranth,. Lining material, farm ponds

- Introduction:

Rainwater harvesting is a vital strategy for addressing water scarcity, particularly in the Konkan where rainfall pattern is not erratic (Ertop et al., 2023; Huang et al., 2021). Therefore, lined farm ponds have become a critical water management tool, particularly in the Konkan region. Lined farm ponds have emerged as an efficient solution for harnessing rainwater, offering multiple benefits like water storage and sustainability (Teston et al., 2022). The Konkan region’s specific challenges, such as high rainfall and poor water retention in soil, farm ponds with durable and efficient linings are indispensable for maximizing the benefits of rainwater harvesting. The effectiveness of farm ponds depends significantly on the choice of lining material (García-Ávila et al., 2023). Lining ensures that water stored during the rainy season remains available for agricultural use throughout the year (Mfitumukiza et al., 2022; Obieluem et al., 2023). With climate change causing erratic rainfall patterns, lined ponds provide a more reliable water storage solution, supporting sustainable agriculture and improving farmers' resilience against water scarcity (Mahale et al., 2011).

Farmers in the Konkan region can optimize rainwater harvesting, improve water conservation, and enhance agricultural productivity by lining farm ponds, addressing both immediate and long-term water management needs (Shelar et al., 2023). The lining materials for farm ponds are a crucial investment in agricultural water management. Their costs, durability, and handling requirements vary widely, influencing the economics of farm pond construction and maintenance (Gawai et al., 2024).

By ensuring reliable irrigation water, the project can improve crop yields, reduce dependency on erratic monsoons, and boost farmer incomes. Using eco-friendly and durable linings reduce soil erosion, preserve natural water bodies, and promote sustainable farming practices. The study evaluate the economic viability of different materials, helping farmers choose the best options based on durability, cost, and maintenance requirements. This study is necessary for selecting durable and eco-friendly materials. The study contributes to study the economics of lining material.

2. MATERIALS AND METHODS

The experimental site is situated at 17° 45” 13.1” N latitude and 73° 10‟ 47.4” E longitude with altitude of 250 m. Experimental layout of Farm ponds shown in Fig.1. The location is in coastal belt of Konkan region of Maharashtra state. The field experiment is conducted during 2017-18, at the Instructional Farm, Department of Soil and Water Conservation Engineering, College of Agricultural Engineering and Technology, Dr. Balasaheb Sawant Konkan Krishi Vidyapeeth, Dapoli. Climatic condition are humid with average annual rainfall at Dapoli is 3635 mm (Mandale, 2016) and average minimum to maximum atmospheric temperatures varies from 8 °C to 36.0 °C. The topography of the experimental layout is level. The slope of land is less than 1%. The soil type of the experimental field is lateritic.

2.1 Farm pond layout and constuction

15 farm ponds of same shape, size and capacity were constructed in same soil type. Out of 15 ponds, five ponds were lined with black lining material, five ponds were lined with blue lining material and remaining five pond were lined with HDPE material. The fifteen farm ponds were constructed of size 4 m x 2 m x 2 m. The space between two farm pond is 2 m. The volume of water stored in each pond is 16 m3. Layout the experiment is given in the Fig. .1

T1 - Black lining

T2 - Blue lining

T3 - HDPE lining

Fig. 1 Experimental layout of Farm ponds.

  - Experimental details and layout

The experiment involves three different treatments (T1, T2, T3) with 10 beds per treatment. Each plot measures 3 m x 2 m, total 30 plots (10 beds per treatment). Including all replications there were 30 treatment combinations and arranged on an area of 36.5 m × 7 m.

## 2.3   Cost analysis

Cost analysis of amaranthus was consisted of cost of production, gross monetary returns, net income and benefit cost ratio of amaranthus cultivation for three years and average of three years.

### 2.4   Cost of production

The cost of production was worked out for each treatment. The variable cost includes paid out cost on hired human labour, seeds, fertilizers, water charges, supervision charges, interest on working capital and interest on fixed capital, depreciation, repair and maintenance for installation of drip irrigation system.

### 2.5 Gross monetary returns

The gross monetary return per hectare was worked out by considering the amaranthus yield from different treatments and prevailing market prices of amaranthus.

### 2.6 Net income

The net income is worked out by subtracting the cost of production from gross monetary returns in each treatment.

### 2.7 Benefit Cost ratio

The benefit cost (B.C.) ratio was worked out by dividing gross returns with to total expenditure incurred for production. Standard procedure adopted to calculate per hector cost of cultivation for Amaranths by following recommended package of practices.

3 RESULTS AND DISCUSSIOON

Among ten replications of each treatment in Rabi season for crop yield having a plot size of 6 sq. meters, the data presented in Table 1. Treatment 1 (T1) produced the highest yield of 15.45 kg in Replication 1, with its lowest yield being 10.10 kg in Replication 6. The average yield for T1 across all replications was 12.82 kg, equating to 2.14 kg per square meter. Treatment 2 (T2) exhibited a range of yields, with the highest yield of 14.77 kg in Replication 1 and the lowest yield of 11.36 kg in Replication 4. The average yield for T2 was 13.37 kg or equal to 2.23 kg per square meter. Treatment 3 (T3) showed more consistency, with the highest yield of 15.84 kg in Replication 3 and the lowest yield of 10.67 kg in Replication 7. The average yield for T3 was 13.98 kg, translating to 2.33 kg per square meter.

The average yield per treatment bed of amaranthus in 2017-18 (Rabi season) was analyzed statistically. This indicates that the variation among the treatment is not statistically significant.

Table 1: Yield of amaranthus in Rabi season for year 2017-18

| Sr. No | Plot size 3m x 2m | Total yield of crop in Kg |  |  |
| --- | --- | --- | --- | --- |
|  |  | Treatment |  |  |
|  | Replication | T1 | T2 | T3 |
| 1 | 1 | 15.45 | 14.77 | 14.37 |
| 2 | 2 | 14.54 | 13.15 | 15.10 |
| 3 | 3 | 13.90 | 12.99 | 15.84 |
| 4 | 4 | 11.19 | 11.36 | 14.95 |
| 5 | 5 | 13.54 | 12.86 | 14.78 |
| 6 | 6 | 10.10 | 12.42 | 13.80 |
| 7 | 7 | 10.71 | 14.34 | 10.67 |
| 8 | 8 | 12.57 | 13.67 | 12.42 |
| 9 | 9 | 13.98 | 14.13 | 13.13 |
| 10 | 10 | 12.25 | 13.98 | 14.78 |
|  | Total | 128.23 | 133.67 | 139.84 |
|  | Mean | 12.82 | 13.37 | 13.98 |

Table 2: Per hectare cost of Cultivation of Amaranthus Treatment 1 for 2017-2018 (Rabi season)

| Sr. No | Particulars | Unit | Quantity | Rate (Rs) | Amount (Rs) |
| --- | --- | --- | --- | --- | --- |
| 1 | Labour - Male / Female | Days | 170 | 201 | 34170 |
| 2 | Machine (Tractor) | Hours | 10 | 600 | 6000 |
| 3 | Seed material | Kg. | 2.5 | 1000 | 2500 |
| 4 | F.Y.M. | Tonne | 10 | 4000 | 40000 |
| 5 | Fertilizers |  |  |  |  |
|  | i) Urea | Kg. | 130 | 7.00 | 910 |
|  | ii) Single Super Phosphate | Kg. | 188 | 9.00 | 1692 |
|  | iii) Muriate of potash | Kg. | 50 | 20.00 | 1000 |
| 6 | Plant protection |  |  |  | 2000 |
| 7 | Irrigation material |  |  |  | 70000 |
| 8 | Cost of Lining material and excavation |  |  |  | 13159 |
|  | Input Cost |  |  |  | 171431 |
| 9 | Depreciation on implements and Machinery |  |  |  | 400 |
| 10 | Land revenue and other cesses |  |  |  | 50 |
| 11 | Interest on working capital (@ 6% for 6 Months) |  |  |  | 5058 |
| 12 | Interest on fixed capital |  |  |  | 600 |
| 13 | Rental value of land (1/6th of the gross value - Land revenue) |  |  |  | 86575 |
| 14 | Electricity cost for pump operation |  |  |  | 1500 |
| 15 | Supervision charges (@10% input cost) |  |  |  | 17143 |
|  | Total Cost |  |  |  | 285384 |
| 16 | Yield and Gross returns | t/ha | 21.40 |  |  |
|  | i) Main Product (1 bundle = 0.4 kg) | bundle | 53500 | 10 | 535000 |
|  | Gross returns |  |  |  | 535000 |
| 17 | Net returns at: |  |  |  |  |
|  | i) Input cost |  |  |  | 363569 |
|  | ii) Total cost |  |  |  | 249616 |
| 19 | B:C Ratio at Input Cost |  |  |  | 3.12 |
| 20 | B:C Ratio at Total Cost |  |  |  | 1.87 |

Table 3: Per hectare cost of Cultivation of Amaranthus Treatment 2 for 2017-2018 (Rabi season)

| Sr. No. | Particulars | Unit | Qty. | Rate (Rs) | Amount (Rs) |
| --- | --- | --- | --- | --- | --- |
| 1 | Labour - Male / Female | Days | 170 | 201 | 34170 |
| 2 | Machine (Tractor) | Hours | 10 | 600 | 6000 |
| 3 | Seed material | Kg. | 2.5 | 1000 | 2500 |
| 4 | F.Y.M. | Tone | 10 | 4000 | 40000 |
| 5 | Fertilizers |  |  |  |  |
|  | i) Urea | Kg. | 130 | 7.00 | 910 |
|  | ii) Single Super Phosphate | Kg. | 188 | 9.00 | 1692 |
|  | iii) Muriate of potash | Kg. | 50 | 20.00 | 1000 |
| 6 | Plant protection |  |  |  | 2000 |
| 7 | Irrigation material |  |  |  | 70000 |
| 8 | Cost of Lining material and excavation |  |  |  | 14933 |
|  | Input Cost |  |  |  | 173205 |
| 9 | Depreciation on implements and Machinery |  |  |  | 400 |
| 10 | Land revenue and Other cesses |  |  |  | 50 |
| 11 | Interest on working capital (@ 6% for 6 Months) |  |  |  | 5196 |
| 12 | Interest on fixed capital |  |  |  | 600 |
| 13 | Rental value of land (1/6th of the gross value - Land revenue) |  |  |  | 93283 |
| 14 | Electricity cost for pump operation |  |  |  | 1500 |
| 15 | Supervision charges (@10% input cost) |  |  |  | 17321 |
|  | Total Cost |  |  |  | 291555 |
| 16 | Yield and Gross returns | t/ha | 22.40 |  |  |
|  | i) Main Product (1 bundle = 0.4 kg) | bundle | 56000 | 10 | 560000 |
|  | Gross returns |  |  |  | 560000 |
| 17 | Net returns at: |  |  |  |  |
|  | i) Input cost |  |  |  | 386795 |
|  | ii) Total cost |  |  |  | 268445 |
| 19 | B:C Ratio at Input Cost |  |  |  | 3.23 |
| 20 | B:C Ratio at Total Cost |  |  |  | 1.92 |

Table 4: Per hectare cost of Cultivation of Amaranthus Treatment 3 for 2017-2018 (Rabi season)

| Sr. No | Particulars | Unit | Qty. | Rate (Rs) | Amount (Rs) |
| --- | --- | --- | --- | --- | --- |
| 1 | Labour - Male / Female | Days | 170 | 201 | 34170 |
| 2 | Machine (Tractor) | Hours | 10 | 600 | 6000 |
| 3 | Seed material | Kg. | 2.5 | 1000 | 2500 |
| 4 | F.Y.M. | Tone | 10 | 4000 | 40000 |
| 5 | Fertilizers |  |  |  |  |
|  | i) Urea | Kg. | 130 | 7.00 | 910 |
|  | ii) Single Super Phosphate | Kg. | 188 | 9.00 | 1692 |
|  | iii) Muriate of potash | Kg. | 50 | 20.00 | 1000 |
| 6 | Plant protection |  |  |  | 2000 |
| 7 | Irrigation material |  |  |  | 70000 |
| 8 | Cost of Lining material and excavation |  |  |  | 6154 |
|  | Input Cost |  |  |  | 164426 |
| 9 | Depreciation on implements and Machinery |  |  |  | 400 |
| 10 | Land revenue & Other cesses |  |  |  | 50 |
| 11 | Interest on working capital (@ 6% for 6 Months) |  |  |  | 4933 |
| 12 | Interest on fixed capital |  |  |  | 600 |
| 13 | Rental value of land (1/6th of the gross value - Land revenue) |  |  |  | 97033 |
| 14 | Electricity cost for pump operation |  |  |  | 1500 |
| 15 | Supervision charges (@10% input cost) |  |  |  | 16443 |
|  | Total Cost |  |  |  | 285385 |
| 16 | Yield & Gross returns | t/ha | 23.30 |  |  |
|  | i) Main Product (1 bundle = 0.4 kg) | bundal | 58250 | 10 | 582500 |
|  | Gross returns |  |  |  | 582500 |
| 17 | Net returns at: |  |  |  |  |
|  | i) Input cost |  |  |  | 418074 |
|  | ii) Total cost |  |  |  | 297115 |
| 19 | B:C Ratio at Input Cost |  |  |  | 3.54 |
| 20 | B:C Ratio at Total Cost |  |  |  | 2.04 |

### Benefit Cost ratio for three Treatment over the Rabi  season of 2017-2018

During the Rabi season of 2017–2018, three different treatments were evaluated for the cultivation of Amaranthus, each demonstrating varying levels of profitability and resource efficiency. The analysis considered input costs, total costs, gross and net returns, and benefit-cost ratios to assess economic viability.

Treatment 1 involved an input cost of Rs. 1,71,431 per hectare, covering labour, machinery, seed material, fertilizers, plant protection, irrigation, and lining materials. When depreciation, interest, land rental, and supervision charges were added, the total cost rose to Rs. 2,85,384. The gross returns from the yield were Rs. 5,35,000, resulting in net returns of Rs. 3,63,569 at input cost and Rs. 2,49,616 at total cost. The benefit-cost ratios stood at 3.12 (input cost) and 1.87 (total cost), indicating a profitable outcome.

Treatment 2 incurred a slightly higher input cost of Rs. 1,73,205 per hectare, with a total cost of Rs. 2,91,555. The gross returns were Rs. 5,60,000, leading to net returns of Rs. 3,86,795 at input cost and Rs. 2,68,445 at total cost. The benefit-cost ratios were 3.23 (input cost) and 1.92 (total cost) respectively, showing improved profitability and efficiency compared to Treatment 1.

Treatment 3 demonstrated the most favorable results, with the lowest input cost of Rs. 1,64,226 and a total cost of Rs. 2,85,385. The gross returns reached Rs. 5,82,500, yielding net returns of Rs. 4,18,274 at input cost and Rs. 2,97,115 at total cost. The benefit-cost ratios were highest among the three treatments, at 3.54 (input cost) and 2.04 (total cost) respectively.

CONCLUSION

The B:C ration was highest in Treatment 3. Adaptable linings contribute to efficient water management, reduced maintenance costs, and enhanced agricultural productivity, making them an essential component of sustainable farming practices. By preventing seepage and contamination, lined ponds help in creating sustainable and environmental friendly agricultural systems.

# COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, manuscript.

REFERENCES

Mahale, D. M., Gharde, K. D., Bhange, H. N., Idate S. S. and Palkar, S. M. (2011) Rainwater Harvesting Through Lined Farm Pond for Sustainable Development of Konkan Region. Journal of the Indian Society of Coastal Agricultural Research Vol. 29(1), 45-53.

Mandale, V. (2016). Trend analysis of rainfall in Konkan region of Maharashtra. Unpublished Thesis College of Agricultural Engg. and Tech. Dr. B.S.K.K.V. Dapoli, Maharashtra.

Teston, A., Piccinini Scolaro, T., Kuntz Maykot, J., & Ghisi, E. (2022). Comprehensive environmental assessment of rainwater harvesting systems: a literature review. Water, 14(17), 2716.

García-Ávila, F., Guanoquiza-Suárez, M., Guzmán-Galarza, J., Cabello-Torres, R., & Valdiviezo-Gonzales, L. (2023). Rainwater harvesting and storage systems for domestic supply: An overview of research for water scarcity management in rural areas. Results in Engineering, 18, 101153.

Shelar, R., Maji, S., Singh, A. K., & Yarazari, S. P. (2023). Climate Change Awareness and Vulnerability among Farming Communities of Konkan Region, Maharashtra, India. Int. J. Environ. Clim. Change, 13(11), 163-175.

Gawai, M. P., Waghmode, B. D., Patil, P. D., Sagvekar, V. V., & Sawant, V. P. (2024). Production oriented survey (POS) on different aspects of rice cultivation and farmers practices in Konkan region of Maharashtra state. J Agric Res Technol, 49(3), 426-430.

Ertop, H., Kocięcka, J., Atilgan, A., Liberacki, D., Niemiec, M., & Rolbiecki, R. (2023). The importance of rainwater harvesting and its usage possibilities: Antalya example (Turkey). Water, 15(12), 2194.

Huang, Z., Nya, E. L., Rahman, M. A., Mwamila, T. B., Cao, V., Gwenzi, W., & Noubactep, C. (2021). Integrated water resource management: Rethinking the contribution of rainwater harvesting. Sustainability, 13(15), 8338.

Mfitumukiza, D., Barasa, B., Sseviiri, H., Nyarwaya, A., Mwesigwa, G., & Kiggundu, N. (2022). Rainwater harvesting technologies: adoption, maintenance, and limitations among smallholder farmers in drought prone areas of Uganda. African Journal of Rural Development, 7(1), 111-132.

Obieluem, U. H., Uzuegbu, J. O., & Ugwuagbo, W. (2023). Evolving Trends in Rain Water Harvesting and Storage System in Nsukka Area, South-east, Nigeria. African Renaissance, 20(3), 233.