Modelling of Regulated Water Management on Water Use Efficiency and Economics of Wheat (Triticum aestivum L.)

ABSTRACT

Wheat is the most extensively grown cereal crop in the world, playing a crucial role in ensuring food security. Climate change impacts, soil degradation, pest and disease outbreaks, and market volatility all pose challenges to modern wheat cultivation, affecting yields and profitability. Heat stress and erratic rainfall, exacerbated by climate change, are major concerns nowadays. Therefore, the present field experiment was laid out in wheat crop during rabi 2021-22 at Irrigation and Water Management Farm of Department of Soil Science, CSK HPKV, Palampur to study the effect of regulated deficit irrigation on the water use efficiency and economics of wheat (Triticum aestivum L.). The treatments consisted of selected combination of four deficit levels viz. high (50% ETC), medium (70% ETC), low (90% ETC) and free from deficit (100% ETC) and three phases of application viz. phase I (CRI and jointing), phase II (flowering) and phase III (dough stage). A total of eight combinations viz. FFF: Free from deficit at all three phases; LLL: Low deficit at all three phases; MMM: Medium deficit at all three phases; FMM: Free from deficit at phase I, Medium at phase II, Medium at phase III; HFM: High at phase I, Free from deficit at phase II, Medium at phase III; MFH: Medium at phase I, Free from deficit at phase II, High at phase III; MFM: Medium at phase I, Free from deficit at phase II, Medium at phase III; HFH: High at phase I, Free from deficit at phase II and High at phase III, were tested under randomized block design and replicated three times. The soil of the experimental site was silty clay loam in texture, acidic in reaction, medium in organic carbon, available potassium, high in available phosphorus and low in available nitrogen. Results showed that highest irrigation water use efficiency was recorded under MMM (41.1 kg ha-1 mm-1), followed by FMM (40.5 kg ha-1 mm-1) both being statistically at par and lowest irrigation water use efficiency was recorded under HFM (27.5 kg ha-1 mm-1) followed by HFH (27.7 kg ha-1 mm-1). Results showed that among all regulated water deficit regimes the adoption of regulated water deficit regimes of LLL (90% ETC at CRI & Jointing, flowering and dough stage) resulted in water productivity comparable to water deficit free cultivation (FFF) with a water saving of 11 per cent under limited water availability conditions, in wheat crop. The net returns (₹ 72303 ha-1), benefit-cost ratio (2.35) and returns per unit of water use (₹ 229.89 mm-1) were higher under no deficit condition (FFF), but it was closely related to net returns (₹ 67603 ha-1), benefit-cost ratio (2.26) and returns per unit water use (₹ 222.81 mm-1) of low deficit at all three stages (LLL). The lowest net return (₹ 33465 ha-1), benefit cost ratio (1.30) and returns per unit water use (₹ 116.61 mm-1) was observed under HFH followed by HFM.

Keywords: Wheat, Water Management, Food security, Water Use Efficiency

1. INTRODUCTION

“According to an analysis of food demand projections, by 2050, global food demand will increase by 35% to 56% relative to 2010” (Van Dijk et al., 2021). “Wheat is most probably the major crop contributing to global food security, providing 20% of global dietary energy and protein intake” (Naik et al. 2024; Gooding and Shewry, 2022, Salaria et al. 2024a). “Assuming the proportion of wheat in global diets remains similar in future, an additional 224-359 Mt will be needed by 2050 to meet the increased demand” (Kettlewell et al. 2023). More than 35% of the world’s population depends on wheat as staple food. It is the main source of plant-based protein, with a protein content of around 13% which was higher than most of the major cereals (Sharma and Sharma, 2025). Wheat is globally cultivated on 219 million hectares, producing 808.4 million tonnes (FAOSTAT, 2025). Wheat production across the globe is facing numerous challenges nowadays. Climate change has emerged as a major threat, with rising temperatures, erratic rainfall, and increasing frequency of droughts and heatwaves significantly affecting crop growth and yield. Water scarcity, driven by over-extraction of groundwater and inefficient irrigation practices, further limits production. Low input use efficiency, particularly in fertilizer and water management, and limited access to advanced technologies and climate-resilient crop varieties hinder yield improvement. Addressing these issues is essential to ensure the future viability of wheat cultivation globally. For effective decision making in agriculture, forecasts of weather parameters found useful and profitable whereas application of rainfall and temperature forecasts proved to be beneficial in saving irrigation crops (Rana et al., 2013a).

Wheat is the second major food crop after rice in India. The country accounts for 13.33% of global wheat production, covering 30.4 million hectares area and producing 107.7 million tonnes with an average productivity of 3.46 t/ha (Salaria et al. 2024b; FAOSTAT, 2025). Wheat yield remains lower than the global average due to a combination of factors, including climate change, soil degradation, water scarcity, and pests and diseases. Climate change, with its associated impacts like rising temperatures and uneven rainfall, directly affects crop growth and productivity. Additionally, unsustainable agricultural practices contribute to soil degradation, reducing soil fertility and impacting yield (Rana et al. 2025; Choudhary et al. 2024).

In Himachal Pradesh only 21.8% of wheat area is under irrigation, and it is grown on 319.4 thousand hectares area, producing 609.3 thousand tonnes with an average yield of 1.90 t/ha (Department of Agriculture, 2022-23; Salaria et al. 2024a) which is significantly lower than national average. Agriculture utilizes 70-80% of available freshwater, making water scarcity the most critical limiting factor in wheat production (Saharan et al. 2024). Scientific studies revealed decreasing rainfall trends and increasing maximum and minimum temperatures in Himachal Pradesh (mountainous state) and with an increase in temperature, crop water requirements will also increase in future for 1, 2, and 3 o C rise in temperature compared to current climate (Rana et al. 2012; Pareek et al. 2021). Apart from significant water demand for wheat, the water supply for crop production has been advocated to be reduced due to changes in climatic conditions in mountains over the last three decades. (Rana et al., 2014).

“As population is increasing at an alarming rate, increasing yield per unit area to increasing yield per unit of water to meet the demands of rapidly growing population is the need of the hour” (Sharma et al. 2024). Agronomic practices and field adaptations has the potential to mitigate the negative effects of climate change on crop production systems in mountainous areas (Rana et al., 2021; Sharma et al. 2025). Traditional methods of irrigations in wheat farming include fixed schedules or flood irrigation which often lead to inefficient water application, resulting in excess water losses through evaporation, deep percolation and runoff. Moreover, indiscriminate water use contributes to declining groundwater tables and reduced irrigation efficiency. The decreasing availability of water for irrigation and the global rise in production costs emphasize the need to apply techniques that improve water use efficiency, such as Regulated Deficit Irrigation (RDI) (Chai et al. 2016). In this concern, “regulated deficit irrigation (RDI) is a water-saving irrigation technique that involves strategically reducing water application during specific crop growth stages while ensuring adequate water supply during other critical periods. This approach focuses on supplying water at critical growth stages such as crown root initiation (CRI), tillering, flowering, and grain filling, thereby optimizing both water use efficiency and crop performance. This method can significantly improve water productivity and potentially increase crop yields, especially in water-stressed areas. Deficit irrigation reduces water consumption while minimizing yield loss” (Panda et al., 2003).

Water use efficiency (WUE) is defined as the amount of biomass or grain yield produced per unit of water used, and it is used as a key indicator of sustainable water management. By aligning irrigation schedules with critical growth phases and employing deficit irrigation techniques when necessary, regulated water management can enhance WUE without significantly compromising yield. Additionally, such practices may reduce production costs by limiting excessive water application and associated inputs like energy or labour, thus improving the economics of wheat cultivation. In regions facing water scarcity, site-specific deficit irrigation strategies can help maximize water use efficiency and boost overall productivity.

2. MATERIALS AND METHODS

A field investigation was carried out during the rabi season of 2021-2022 at the Irrigation and Water Management Farm of the Department of Soil Science, Chaudhary Sarwan Kumar Himachal Pradesh Krishi Vishvavidyalaya (CSK HPKV), Palampur. The experimental site is situated at 32°06′39.1″ N latitude and 76°32′10.5″ E longitude, at an elevation of 1290 meters above mean sea level in the north-western Himalayas in Kangra district. During the crop growth period, which lasted from October 2021 to April 2022, the mean weekly minimum and maximum temperatures varied from 2.4°C to 16.8°C and 10.8°C to 31.3°C, respectively. During the season, relative humidity ranged from 33.6% to 85.9%.

The soil at the experimental site was silty clay loam with an acidic pH of 5.4, soil organic carbon content of 0.73%, and a bulk density of 1.27 Mg m⁻³, whereas the available nitrogen, phosphorus, and potassium content were 270.5 kg ha⁻¹, 29.3 kg ha⁻¹, and 175.4 kg ha⁻¹, respectively.

The experiment was laid in a Randomized Block Design (RBD), with eight irrigation treatments replicated three times for a total of 24 plots. The wheat variety HPW 236 was sown on 26th October 2021 with all recommended agronomic practices, except for the irrigation treatments, which were modified as per the experimental design to investigate regulated deficit irrigation. The eight treatments involved different combinations of water deficit levels applied at three critical growth phases: Phase I (Crown Root Initiation and Jointing), Phase II (Flowering), and Phase III (Dough stage). Water deficit levels included high (50% of crop evapotranspiration, ETc), medium (70% ETc), low (90% ETc), and no deficit (100% ETc). The treatment combinations were as follows: T1 - FFF (no deficit in all phases), T2 - LLL (low deficit throughout), T3 - MMM (medium deficit throughout), T4 - FMM (no deficit at Phase I, medium deficit at Phases II and III), T5 - HFM (high deficit at Phase I, no deficit at Phase II, medium deficit at Phase III), T6 - MFH (medium at Phase I, no deficit at Phase II, high at Phase III), T7 - MFM (medium deficit at Phases I and III, no deficit at Phase II), and T8 - HFH (high deficit at Phases I and III, no deficit at Phase II). These treatments were designed to study the effects of deficit irrigation on water use efficiency and economics of wheat under water-limited conditions common to rainfed hill agriculture.

Water use efficiency indices were computed using consumptive water use, water productivity, irrigation water use efficiency and returns per unit water use.

Consumptive water use (mm) was calculated using formula given by FAO:

Total water use = Effective rainfall + Total water applied through irrigation + Δ S

CROPWAT 8.0 software was used which utilizes the FAO Penman-Monteith method for calculating reference evapotranspiration (ETO). This method is widely recommended by the FAO for estimating ETO from meteorological data due to its physically-based approach and ability to closely approximate grass ETo.

The mean weekly meteorological data of the crop during rabi season was recorded at Meteorological Observatory of the Department of Agronomy, College of Agriculture, CSK Himachal Pradesh Krishi Vishvavidyalaya, Palampur and it this data was used as input in CROPWAT 8.0 software to calculate reference crop evapotranspiration (ETO).

Rainfall data and ET data was calculated using Penman Monteith Modified method.

For determination of the crop water requirements CWR, crop evapotranspiration ETC was calculated under standard conditions as follows:

ETC = ETO x KC

where:

ETC = crop evapotranspiration, mm day-1;

ETO = reference crop evapotranspiration, mm day-1;

KC   = crop coefficient

The values of ETC and CWR are identical, where by ETC refers to the amount of water lost through evapotranspiration and CWR refers to the amount of water that is needed to compensate for the loss. ETC calculated from climatic data by directly integrating the effect of crop characteristics into ETO.  (Noreldin et al. 2015)

Water productivity (WP) (kg m-3) was computed as follows:

WP (kg m-3) = Grain Yield (kg ha-1)Total water use (mm)

Irrigation water use efficiency (kg ha-1 mm-1) was worked out as:

IWUE = Yield (kg ha-1)Irrigation water use (mm)

Returns per unit water use was calculated as:

Returns per unit water use = Net return (₹ ha-1)Total water use mm

Economics for different treatments were calculated. The cost of cultivation (₹ ha-1) was calculated by adding all costs involved in each operation and input. The treatment-wise grain and straw yields were multiplied with their respective market prices. The gross returns (₹ ha-1) were calculated by adding returns received from grain and straw yields. The treatment-wise net returns (₹ ha-1) were obtained by subtracting the cost of cultivation from the gross returns of the respective treatments. The benefit to cost ratio was obtained by dividing net returns by the cost of cultivation.

3. RESULTS AND DISCUSSION

3.1 Influence of regulated water levels on water use efficiency indices of wheat crop

The regulated water levels had significant effect on various water use efficiency indices of wheat (Table 1). Under the regulated regime of FFF (100% ETC at all three phases: CRI & jointing, flowering and dough), higher water productivity (WP) was observed, followed by LLL (90% ETC at all three phases) though both were statistically same. Lower WP was observed under HFH and HFM when atleast one high water deficit (50% ETC) was imposed although both remained statistically at par. Water productivity is function of yield and water use. Higher WP with more irrigation amount may be attributed to greater and easier availability of water for absorption by the plant roots (Bhunia et al. 2005). As a result, WP increased along with irrigation schedule. It was recorded that the yields increased with higher irrigation levels and total consumptive use also increased in the same ratio under full irrigation regime FFF, followed by the low deficit regime LLL. The findings are similar to Shivani et al. (2001). Increased root growth has been linked to higher water use efficiency and wheat grain yield (Liao et al. 2004), as evidenced by higher root biomass observed with more water applied.

Regulated water levels had a significant effect on irrigation water use efficiency (IWUE). Higher IWUE was observed under regulated regime of MMM, in which a medium deficit (70% ETC) was imposed throughout all phases, followed by FMM, in which no water deficit was imposed (100% ETC) at CRI and jointing, followed by a medium deficit (70% ETC) at flowering, dough, and MFM, in which a medium deficit (70% ETC) at CRI and jointing and dough and full irrigation (100% ETC) at flowering. However, the IWUE observed under MMM conditions remained at par with FMM and MFM. Lower IWUE was observed under HFH regulated regime, which imposed a high deficit (50% ETC) at CRI & jointing, dough and it was free from deficit (100% ETC) at flowering, which was followed by the HFM regulated regime wherein high deficit (50% ETC) was imposed at CRI and jointing, free from deficit (100% ETC) at flowering and medium deficit (70% ETC) at dough stage. Latter two remained at par with one another. It could be due to the fact that the increase in grain yield was much lower than the increase in total irrigation used. At various stages of growth, yield increased in response to full irrigation (FFF) or low deficit (LLL) treatments, but the amount of water used increased as well, reducing efficiency. However, it is clear that increasing irrigation levels increased crop water uptake and evaporation losses from the soil surface. The results are similar to those of Waghmare (2002), who found that the higher water use efficiency was achieved with only two irrigations (CRI and blooming) and the lower was achieved with six. Rizk and Sherif (2014), Kirda (2002), and Kang et al. (2002) all reported similar results.

Regulated water levels had a significant influence on returns per unit water use. The FFF and LLL regulated regimes provided higher returns per unit water use (₹ 229.89 mm-1 and ₹ 222.81 mm-1, respectively), which were comparable to MFM, FMM, MFH, and HFM. The HFH regulated regime provided lower returns per unit water use (₹116.61 mm-1) compared to HFM, MFH, and MMM regimes. However, HFH and HFM remained at par with one another. Higher returns per unit water use recorded under FFF and LLL can be attributed to the higher yield produced under these regulated water regimes.

3.2 Influence of regulated water levels on economic studies of wheat crop

Wheat economics have been influenced by various regulated deficit regimes. (Table 2). The highest cost of cultivation, net returns, gross returns and B:C ratio was recorded under FFF (free from deficit throughout) water regime, which was followed by LLL water regime. The lowest cost of cultivation, net returns, gross returns and B:C ratio was recorded under HFH water regulated regime, which was followed by HFM water regulated regime. The highest cost of cultivation was ₹30717 ha-1, which was observed under regulated regime of no deficit FFF, followed by low deficit LLL and MFM. The HFH regulated regime resulted in the lowest cultivation cost (₹ 25677 ha-1) due to reduced labour requirements for irrigation, harvesting, and threshing operations. Maximum gross returns (₹ 103020 ha-1) were recorded under the regulated regime of no deficit (FFF) which was followed by LLL and MFM. HFH resulted in the lowest gross returns (₹ 59142 ha-1) due to lower grain and straw yields. Maximum net returns (₹72303 ha-1) were observed under deficit free regime FFF. All other regulated regimes produced lower net returns than the deficit free FFF regime. Minimum net returns (₹ 33465 ha-1) were obtained under regulated water deficit regime of HFH. Results are in close proximity with Kanwal et al. (2020), who stated that giving 4 irrigations instead of 2 or 3 irrigations had high yield potential. Gangwar and Lodhi (2018) found that the highest number of irrigations in wheat resulted in highest net returns. Water deficit free regime FFF recorded highest benefit-cost ratio (2.35) due to higher net returns. It was followed by LLL, MFM and FMM regulated regimes. The least benefit-cost ratio (1.30) was recorded under HFH conditions which was followed by HFM. Similar results are reported by Kanwal et al. (2020), who stated that giving 4 irrigations instead of 2 or 3 irrigations had high economic benefit.

4. CONCLUSION

In wheat crop, under limited water availability, adoption of regulated water deficit regime of 90% ETC (low deficit) at CRI & jointing, flowering and dough stage gave yield and water productivity similar to water deficit free conditions (FFF) with a water saving of 11 per cent. The net returns (₹ 72303 ha-1), benefit-cost ratio (2.35) and returns per unit of water use (₹ 229.89 mm-1) were higher under FFF (water deficit free condition at all phases), but it was closely matched by net returns (₹ 67603 ha-1), benefit-cost ratio (2.26) and returns per unit of water use (₹ 222.81 mm-1) of LLL regulated water regime (90% ETC at all stages).

Table 1 Influence of regulated water levels on water use efficiency indices of wheat crop

| Regulated water levels | Water productivity (kg m-3) | Irrigation water use efficiency (kg ha-1 mm-1) | Returns per unit water use (₹ mm-1) |
| --- | --- | --- | --- |
| T1 (FFF) | 10.56 | 35.6 | 229.89 |
| T2 (LLL) | 10.31 | 37.3 | 222.81 |
| T3 (MMM) | 9.35 | 41.1 | 199.40 |
| T4 (FMM) | 9.73 | 40.5 | 208.94 |
| T5 (HFM) | 6.88 | 27.5 | 128.20 |
| T6 (MFH) | 7.51 | 32.1 | 147.19 |
| T7 (MFM) | 9.79 | 38.4 | 210.59 |
| T8 (HFH) | 6.46 | 27.7 | 116.61 |
| SEm± | 0.22 | 0.9 | 6.59 |
| CD (P=0.05) | 0.68 | 2.8 | 19.98 |

Regulated water levels: High (50%), medium (70%), low (90%), free from deficit (100%) ETC

Stages of deficit irrigation: Phase I (CRI and jointing), Phase II (Flowering), Phase III (Dough)

*FFF: Free from deficit at all three phases; LLL: Low deficit at all three phases; MMM: Medium deficit at all three phases; FMM: Free from deficit at phase I, Medium at phase II, Medium at phase III; HFM: High at Phase I, Free from deficit at Phase II, Medium at Phase III; MFH: Medium at Phase I, Free from deficit at Phase II, High at Phase III; MFM: Medium at Phase I, Free from deficit at Phase II, Medium at Phase III; HFH: High at Phase I, Free from deficit at Phase II and High at Phase III

Table 2 Influence of regulated water levels on economics of wheat crop

| Regulated water levels | Cost of cultivation (₹ ha-1) | Gross returns (₹ ha-1) | Net returns (₹ ha-1) | B:C Ratio |
| --- | --- | --- | --- | --- |
| T1 (FFF) | 30717 | 103020 | 72303 | 2.35 |
| T2 (LLL) | 29877 | 97480 | 67603 | 2.26 |
| T3 (MMM) | 27357 | 84523 | 57166 | 2.09 |
| T4 (FMM) | 28197 | 88969 | 60772 | 2.16 |
| T5 (HFM) | 26797 | 64905 | 38108 | 1.42 |
| T6 (MFH) | 27077 | 71195 | 44118 | 1.63 |
| T7 (MFM) | 29037 | 92966 | 63929 | 2.20 |
| T8 (HFH) | 25677 | 59142 | 33465 | 1.30 |

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

Bhunia, S. R., Chauhan, R. B. S. & Yadav, B. S. (2005). Effect of nitrogen and irrigation on water use moisture extraction pattern, nutrient uptake and yield of fennel (Foeniculum vulgare). Indian Journal of Agronomy, 50(1), 73-76.

Chai, Q., Gan, Y., Zhao, C., Xu, H.L., Waskom, R.M., Niu, Y. & Siddique, K.H.M. (2016). Regulated deficit irrigation for crop production under drought stress. Agronomy for Sustainable Development, 36, 1–21.

Choudhary, R., Kumar, R., Rana, B. B., Choudhary, M., Gurjar, S. N., Choudhary, J., & Choudhary, K. (2024). Effects of cow-based preparations and mulching on weed management and nodulation in chickpea under intercropping system. Journal of Experimental Agriculture International, 46(11), 571-579.

Department of Agriculture. (2022-23). Statement showing crop-wise area, production and productivity of principal crops in Himachal Pradesh. Himachal Pradesh.  Available:https://agriculture.hp.gov.in/en/production-2/

Gangwar, H. K. & Lodhi, M. D. (2018). Effect of nitrogen levels and number of irrigations on growth and yield of wheat. International Journal of Current Microbiology and Applied Sciences, 7(3), 3663-3673.

Gooding, M. J., & Shewry, P. R. (2022). Wheat: Environment, food and health. John Wiley & Sons https://doi.org/10.1002/9781119652601

Kang, S., Zang, L., Liang, Y., Hu, X., Cai, H. & Gu, B. (2002). Effect of limited irrigation on yield and water use efficiency of winter wheat in the Loess Plateau of China. Agriculture Water Management, 55, 203-216.

Kanwal, T., Maryam, H., Ahmad, R., Ahmad, S., Ali, A., Hussain, B. & Tasleem, M. W. (2020). Effect of irrigation regimes on growth and yield of wheat (Triticum aestivum L.); economic analysis. International Research Journal of Advanced Science, 1(2), 53-59.

Kettlewell, P., Byrne, R. & Jeffery, S. (2023). Wheat area expansion into northern higher latitudes and global food security. Agriculture, Ecosystems and Environment, 351, 1-2.

Kirda, C. (2002). Deficit irrigation scheduling based on plant growth stages showing water stress tolerance. Food and Agricultural Organization of the United Nations, Deficit Irrigation Practices, Water Reports, 22, 102.

Liao, M., Fillery, I. R. P. & Palta, J. A. (2004). Early vigorous growth is a major factor influencing nitrogen uptake in wheat. Functional Plant Biology, 31, 121-129.

Naik, M., Rana, M. C., Sharma, G. D., Sharma, T., & Rana, B. B. (2024). Effect of diverse cropping systems on crop indices and resource use efficiency under natural farming. Himachal Journal of Agricultural Research, 279-283.

Noreldin, T., Ouda, S., Mounzer, O. & Abdelhamid, M.T. (2015). CropSyst model for wheat under deficit irrigation using sprinkler and drip irrigation in sandy soil. Journal of Water and Land Development, 26: 57-64.

Panda, R.K., Behera, S.K. & Kashyap, P.S. (2003). Effective management of irrigation water for wheat under stressed conditions. Agricultural Water Management, 63 (1), 37–56.

Pareek, B., Rana, R. S., Manuja, S., Sandal, S. K., Kalia, V., & Singh, K. K. (2021). Modeling crop water requirement using weather model under elevated temperature scenarios for scaling up quality of weather based agro advisory. Journal of Crop and Weed, 17(2), 80-86. https://doi.org/10.22271/09746315.2021.v17.i2.1455

Rana, B. B., Sharma, V., Manuja, S., Sharma, S. K., Singh, A., Sharma, R., & Chauhan, G. (2025). Evaluation of herbicide efficacy on growth indices of maize cultivated under conservation agriculture system. Plant Archives, 25(1), 2808-2817.

Rana, R. S., Bhagat, R. M., Singh, M. M., Kalia, V., Singh, S., & Prasad, R. (2012). Trends in climate variability over Himachal Pradesh. Journal of Agrometeorology, 14(1), 35-40. https://doi.org/10.54386/jam.v14i1.1378

Rana, R. S., Kalia, V., Singh, S., Randhawa, S. S., Chauhan, R., Katoch, A., ... & Upadhyay, S. K. (2021). Climate vulnerability assessment of farming systems in Himachal Pradesh, Indian Himalayas. Mountain Research and Development, 41(4), 50-60. https://doi.org/10.1659/MRD-JOURNAL-D-20-00056.1

Rana, R. S., Sood, R., Rana, A. & Shekhar, J. (2013). Validation of medium range weather forecasts in sub-temperate and sub-humid climate of western Himalayas. Indian Journal of Agricultural Sciences, 83(12), 1357-1363.

Rana, R., Singh, S., Chander, N., Sood, R., Sharma, R., & Aditya, A. (2014). Impacts of changes in climate on mountain water resources of Himachal Pradesh. Mausam, 65(2), 153-160. https://doi.org/10.54302/mausam.v65i2.956

Rizk, A. H. & Sherif, M. M. (2014). Effect of soil moisture depletion on the yield of wheat under sprinkler irrigation at Toshka Area, Egypt. Middle East Journal of Agriculture Research, 3(4), 981-987.

Saharan, S., Singh, J., Sharma, R., Singh, A., Narwal, K., Rana, B. B., ... & Prashar, D. (2024). Revitalizing Rainfed Agriculture: The Transformative Potential of Watershed Development. International Journal of Plant & Soil Science, 36(7), 969-987.

Salaria, S., Rana, R. S., Chauhan, G., Sharma, T., & Pathania, R. (2024). Effect of sowing dates and wheat cultivars on agro-meteorological indices of wheat under conditions of north-western Himalayas, India. International Journal of Plant & Soil Science, 36(10), 282-289. https://doi.org/10.9734/ijpss/2024/v36i105077

Salaria, S., Rana, R. S., Sharma, T., Chauhan, G., Sandal, S. K., Kalia, V., ... & Priyanka. (2024). Optimizing Sowing Windows for Wheat (Triticum aestivum L.) in the Sub-temperate Climate of the North-western Himalayas of India. International Journal of Plant & Soil Science, 36(9), 911-918. https://doi.org/10.9734/ijpss/2024/v36i95042

Sharma, K. & Sharma, P. (2025). Wheat as a Nutritional Powerhouse: Shaping Global Food Security. In: Triticum - The Pillar of Global Food Security. IntechOpen. DOI:10.5772/intechopen.1009499

Sharma, T., Chauhan, P. S., Patel, M., Singh, A., Kaur, M., Chauhan, G., ... & Walia, A. (2025). Carbon negative biofuels: A step ahead of carbon neutrality. Biofuels, 1-21.

Sharma, T., Singh, J., Madaik, S., Kumar, P., Singh, A., Rana, B. B., & Chauhan, G. (2024). Organic input incorporation for enhancing sustainability and economic viability of cowpea in North-Western Himalayan region. Frontiers in Agronomy, 6, 1458603.

Shivani, Verma, U. N., Pal, S. K., Thakur, R. & Kumar, S. (2001). Production potential and water use efficiency of wheat (Triticum aestivum) cultivars under different dates of seeding and irrigation levels. Indian Journal of Agronomy, 46(4), 659-664.

Van Dijk, M., Morley, T., Rau, M. L., Saghai, Y. (2021). A meta-analysis of projected global food demand and population at risk of hunger for the period 2010-2050. Nature Food, 2, 494-501. https://doi.org/10.1038/s43016-021-00322-9.

Waghmare, J. D. (2002). Growth, yield, consumptive use and water use efficiency of wheat variety AKW -107 under late sown conditions. Thesis Abstracts, 28(3), 186-187.