ASSESSMENT OF HEAVY METAL CONTAMINATION IN SOILS OF GASHAKA LGA, NIGERIA USING GEO-ACCUMULATION INDEX

Abstract
Anthropogenic activities such as mining, improper waste disposal, and agricultural practices have contributed significantly to heavy metal contamination in soils, posing substantial ecological and public health risks. This study assessed the levels of heavy metal contamination in soils across four locations: Lambangudu, Maijankasa, Gidan Kara Bodel, and Quentin Gate in Gashaka Local Government Area (LGA), Taraba State, Nigeria. Using the Geo-accumulation Index (Igeo), soil samples were analysed for nine heavy metals: cadmium (Cd), lead (Pb), cobalt (Co), copper (Cu), chromium (Cr), zinc (Zn), nickel (Ni), manganese (Mn), and iron (Fe). The results revealed that Mn, Pb, and Fe exhibited extreme contamination (Igeo > 6) across all sites, indicating strong anthropogenic influence, likely from artisanal mining and unregulated waste disposal. Ni and Co also recorded high contamination levels, falling within the heavily to extremely contaminated class (Igeo Class 5), while Zn showed moderate contamination. Cd, Cu, and Cr remained within uncontaminated levels (Igeo Class 0), suggesting limited anthropogenic inputs for these metals. The findings highlight serious ecological threats and potential public health hazards due to the bioaccumulative nature of these metals. Urgent remediation, policy enforcement, and public health interventions are recommended to mitigate the risks in these vulnerable communities.

Keywords: Geo-accumulation index; Heavy metals; Soil contamination; Artisanal mining; Environmental risk; Gashaka LGA; Taraba State; Nigeria

1. Introduction

The accelerated pace of industrialisation, urbanization, and unsustainable agricultural practices has led to increased concerns over anthropogenic contamination of soils, especially in developing regions like sub-Saharan Africa. Among the most pressing issues is heavy metal pollution, which poses a significant threat to both ecological systems and human health (Adelekan & Abegunde, 2020). In Nigeria, the proliferation of artisanal and small-scale mining (ASM), indiscriminate waste disposal, and application of agrochemicals have been associated with elevated concentrations of heavy metals in agricultural and residential soils (Obi et al., 2022).

Gashaka Local Government Area (LGA) in Taraba State, Nigeria, is endowed with mineral-rich terrains and has witnessed increased human activities, including mining, which potentially exacerbate heavy metal pollution. Although the area plays a crucial role in local economic sustenance, the environmental and public health consequences of soil contamination remain under-investigated. Soil, being a dynamic reservoir of essential and non-essential elements, can serve both as a sink and a source of pollutants. Heavy metals such as lead (Pb), cadmium (Cd), nickel (Ni), chromium (Cr), and manganese (Mn) tend to persist in soils, bioaccumulate, and enter the food chain, thereby impacting biodiversity and human health (Chibuike & Obiora, 2021; Zhang et al., 2023).

To adequately assess contamination levels and risk potentials, various indices have been developed. Among these, the geo-accumulation index (Igeo) introduced by Müller remains one of the most widely used tools for evaluating the extent of anthropogenic influence on soil heavy metal concentrations. The Igeo accounts for natural background levels and helps in distinguishing between natural and human-induced contamination (Keshavarzi & Kumar, 2020).

This study, therefore, evaluates the spatial variability and severity of soil contamination by heavy metals in selected sites within Gashaka LGA, namely Lambangudu, Maijankasa, Gidan Kara Bodel, and Quentin Gate. By employing the geo-accumulation index, the research seeks to identify the pollution classes of individual metals and determine the ecological risks associated with these contaminants. The outcome of this assessment is critical for informing local policies, guiding remediation efforts, and protecting community health.

2. Materials and Methods

2.1 Study Area

The study was conducted in Gashaka Local Government Area, located in the Central Senatorial Zone of Taraba State, northeastern Nigeria. This region lies within the coordinates 7.3167° N, 11.6167° E. The region is characterised by a mixture of savannah and montane ecosystems, with an elevation ranging from 300 to 1,800 meters. The area experiences a tropical climate with distinct wet and dry seasons and supports various forms of land use, including agriculture, grazing, and mining. ASM activities in the region focus mainly on gold, lead, and associated sulfide ores.

Study Locations in the Central Regions

Figure 1: Map of the Study Area (Taraba State Government)

2.3 Sampling Design and Sample Collection

Soil samples were collected from four mining sites: Lambangudu, Maijankasa, Gidan Kara Bodel, and Quenty Gate. At each site, nine composite soil samples were obtained using a grid-based sampling method. Samples were collected from the top 0–20 cm of soil using a stainless steel auger to avoid metal contamination. Each composite sample was made by mixing sub-samples from 3–5 points within a 10-meter radius. Samples were stored in pre-cleaned polyethylene bags, labelled, and transported to the laboratory under controlled conditions.

2.4 Sample Preparation and Analysis

In the laboratory, soil samples were air-dried at room temperature, homogenized, and passed through a 2 mm plastic sieve. Approximately 1 gram of each sieved soil sample was digested using a mixture of concentrated nitric acid (HNO₃) and perchloric acid (HClO₄) following the protocol outlined by the American Public Health Association (APHA, 2012). The digested samples were filtered and analyzed for cadmium (Cd), lead (Pb), cobalt (Co), copper (Cu), chromium (Cr), zinc (Zn), nickel (Ni), manganese (Mn), and iron (Fe) using a Flame Atomic Absorption Spectrophotometer (AAS; model AA-7000, Shimadzu Corp.).

2.5 Geo-accumulation Index (Igeo) Calculation

The level of heavy metal contamination was assessed using the Geo-accumulation Index (Igeo), which is calculated according to the equation developed by Müller (1969):

Igeo= log2Cn1.5 X Bn

Where:

Cn is the measured concentration of the element in the soil sample,

Bn is the background concentration of the element,

1.5 is a constant factor accounting for background matrix variability.

Background values were derived from global geochemical averages for uncontaminated soils, following Alloway (2013). The Igeo values were then classified into seven categories ranging from class 0 (uncontaminated) to class 6 (extremely contaminated).

2.6 Quality Assurance and Control

All glassware and containers were acid-washed and rinsed with distilled water before use. Reagent blanks, duplicate samples, and certified reference materials (CRM) were included in the analysis to ensure data accuracy and precision. Analytical results showed recovery rates between 90% and 105%, with relative standard deviations below 10%.

3. Results

Table 1: Geo-accumulation Index of Soil Samples from Lambangudu

| Metal | Igeo | Class | Pollution Level |
| --- | --- | --- | --- |
| Cd | 0.0000 | 0 | Uncontaminated |
| Pb | 8.3574 | 6 | Extremely contaminated |
| Co | 5.0589 | 5 | Heavily to extremely contaminated |
| Cu | 0.0000 | 0 | Uncontaminated |
| Cr | 0.0000 | 0 | Uncontaminated |
| Zn | 1.7640 | 1 | Uncontaminated to moderately contaminated |
| Ni | 5.5328 | 5 | Heavily to extremely contaminated |
| Mn | 13.6502 | 6 | Extremely contaminated |
| Fe | 8.2760 | 6 | Extremely contaminated |

The results from Lambangudu reveal alarming levels of Pb, Mn, and Fe, indicating intense environmental contamination, likely due to ore processing and dumping of mining waste. Similar levels have been associated with health hazards, including neurotoxicity and groundwater pollution (Table 1).

Table 2: Geo-accumulation Index of Soil Samples from Maijankasa

| Metal | Igeo | Class | Pollution Level |
| --- | --- | --- | --- |
| Cd | 0.0000 | 0 | Uncontaminated |
| Pb | 8.2084 | 6 | Extremely contaminated |
| Co | 4.8524 | 4 | Heavily contaminated |
| Cu | 0.0000 | 0 | Uncontaminated |
| Cr | 0.0000 | 0 | Uncontaminated |
| Zn | 1.5021 | 1 | Uncontaminated to moderately contaminated |
| Ni | 5.3847 | 5 | Heavily to extremely contaminated |
| Mn | 13.6304 | 6 | Extremely contaminated |
| Fe | 8.2675 | 6 | Extremely contaminated |

Soils from Maijankasa show similar trends, with Pb, Mn, and Fe again being extremely contaminated. The high Igeo values for Ni and Co suggest serious pollution, consistent with artisanal extraction and improper tailings disposal (Table 2).

Table 3: Geo-accumulation Index of Soil Samples from Gidan Kara Bodel

| Metal | Igeo | Class | Pollution Level |
| --- | --- | --- | --- |
| Cd | 0.0000 | 0 | Uncontaminated |
| Pb | 8.3775 | 6 | Extremely contaminated |
| Co | 5.6756 | 5 | Likely heavily to extremely contaminated |
| Cu | 0.0000 | 0 | Uncontaminated |
| Cr | 0.0000 | 0 | Uncontaminated |
| Zn | 1.7797 | 1 | Uncontaminated to moderately contaminated |
| Ni | 5.6797 | 5 | Heavily to extremely contaminated |
| Mn | 13.5617 | 6 | Extremely contaminated |
| Fe | 8.2498 | 6 | Extremely contaminated |

The contamination levels at Gidan Kara Bodel closely mirror those from Lambangudu and Maijankasa. Co, Ni, and Zn levels reinforce the pattern of intense soil contamination in the Central Taraba mining corridor (Table 3).

Table 4: Geo-accumulation Index of Soil Samples from Quenty Gate

| Metal | Igeo | Class | Pollution Level |
| --- | --- | --- | --- |
| Cd | 0.0000 | 0 | Uncontaminated |
| Pb | 8.2403 | 6 | Extremely contaminated |
| Co | 5.4075 | 5 | Heavily to extremely contaminated |
| Cu | 0.0000 | 0 | Uncontaminated |
| Cr | 0.0000 | 0 | Uncontaminated |
| Zn | 1.4697 | 1 | Uncontaminated to moderately contaminated |
| Ni | 5.8997 | 5 | Heavily to extremely contaminated |
| Mn | 13.5628 | 6 | Extremely contaminated |
| Fe | 8.2742 | 6 | Extremely contaminated |

Quenty Gate presents one of the highest cumulative contamination profiles, with extreme values for Mn, Fe, and Pb, and significant Co and Ni pollution. These values are indicative of both ore type and poor mining practices, as also observed in similar contexts (Table 4).

4. Discussion

The results from the four sampling sites in Gashaka LGA; Lambangudu, Maijankasa, Gidan Kara Bodel, and Quentin Gate reveal substantial anthropogenic heavy metal pollution, with particular concern for manganese (Mn), lead (Pb), iron (Fe), cobalt (Co), and nickel (Ni). The use of the Geo-accumulation Index (Igeo) effectively quantified contamination levels and classified soils from unpolluted to extremely contaminated. Consistent findings across the sites for Mn, Pb, and Fe in the extremely contaminated class (Class 6) suggest that these metals have a widespread and uniform source, likely related to artisanal mining, smelting activities, and possibly industrial or e-waste disposal also reported in similar studies in mining-dense regions (Adeyeye et al., 2022; Al-Khashman & Shawabkeh, 2021).

Mn recorded the highest Igeo values in all four sites (Igeo > 13), placing it in the extreme contamination class. While manganese is an essential micronutrient for plants and humans, excessive exposure is neurotoxic, particularly through dust inhalation or ingestion from crops grown in Mn-rich soils (Zhang et al., 2021). The elevated Mn levels mirror findings from Kogi and Zamfara States, Nigeria, where manganese toxicity has been linked to informal mining and ore processing (Ogunyemi et al., 2022). Additionally, Mn contamination affects microbial activities and soil enzyme functioning, thereby degrading overall soil fertility (Ighalo et al., 2023). The similar Mn contamination across all sites supports the notion that anthropogenic activities are systematically affecting soils in Gashaka LGA.

Pb levels in all locations also exceeded the Igeo Class 6 threshold (>8), suggesting extreme contamination. Lead is a non-essential, highly toxic heavy metal, known for its persistence in soils and high potential for bioaccumulation. Chronic exposure to lead affects neurological development, particularly in children, and has been linked to kidney disease, reproductive toxicity, and behavioral disorders (WHO, 2022). The consistently high Pb levels in the study sites align with reports from similar mining and smelting communities in Plateau and Taraba States, where lead concentrations have exceeded WHO and FAO safety limits for agricultural soils (Abubakar et al., 2021). Mining residues, battery dumps, and the use of lead-based agrochemicals could explain the contamination pattern.

Ni and Co levels were classified as heavily to extremely contaminated (Igeo Class 5) across all sites. Ni was particularly elevated at Quentin Gate (Igeo = 5.90), raising concerns over its carcinogenic potential. Long-term exposure to Ni has been associated with respiratory tract infections, skin dermatitis, and organ toxicity (Yuan et al., 2020). Similarly, high Co levels (up to Igeo = 5.68) at Gidan Kara Bodel underscore the geochemical risk posed by cobalt exposure. Cobalt toxicity may affect cardiovascular and endocrine systems and can lead to DNA damage if bioavailable (Oboh et al., 2023). These results corroborate findings in Tanzanian gold mining areas, where Co and Ni were also dominant pollutants (Kasambara et al., 2020). The high Co and Ni readings point to metalliferous waste disposal, unregulated ore washing, and tailings leachate as possible pollution sources.

Iron, while essential for plants and animals, can become toxic at elevated concentrations. All four locations had Fe levels categorized as extremely contaminated (Class 6). Fe overload in soils can alter pH and redox conditions, affecting the solubility and mobility of other metals (Fashola et al., 2020). Elevated iron can also interfere with phosphorus uptake in plants, thereby impairing crop yields and soil balance. Notably, similar extreme Fe contamination levels have been reported in Nasarawa and Ebonyi States, both of which also host informal mining operations. This correlation further supports the anthropogenic source hypothesis (Adelekan & Abegunde, 2020).

Interestingly, Cd, Cr, and Cu levels were consistently in the uncontaminated class (Class 0) across all sites. This suggests limited usage or deposition of these metals in the area. Unlike Pb and Ni, cadmium is often linked to phosphate fertilisers, batteries, and plastics, which may not be prevalent in the study locations (Nguyen et al., 2023).

The absence of significant Cr and Cu levels indicates either their absence in the local geology or effective environmental attenuation mechanisms like adsorption or precipitation. However, their presence should still be monitored due to their high mobility under acidic conditions. Zn levels ranged between Igeo = 1.47 and 1.78, placing them in the uncontaminated to moderately contaminated class (Class 1). Although Zn is essential for plant and microbial growth, excessive concentrations may impair soil enzyme activity and induce phytotoxic effects (Chibuike & Obiora, 2021). The Zn levels recorded are not yet alarming, but could increase with continuous anthropogenic pressure. Zn contamination is often attributed to corrosion of galvanized materials and leaching from dumpsites, making its gradual rise in the region plausible (Obi et al., 2022).

5. Conclusion

This study confirms that soils in Lambangudu, Maijankasa, Gidan Kara Bodel, and Quenty Gate in Gashaka LGA are severely polluted with heavy metals due to artisanal mining activities. The application of the Geo-accumulation Index highlights the urgent need for comprehensive environmental monitoring programs, soil and water remediation strategies, public health assessments and community awareness and enforcement of environmental regulations and rehabilitation policies.

Disclaimer (Artificial intelligence)

Author(s) hereby declares that NO generative AI technologies such as Large Language Models (ChatGPT, manuscript.

6. References

Abubakar, S. A., Garba, S. T., & Bako, A. (2021). Assessment of heavy metals in agricultural soils and vegetables in Plateau State, Nigeria. African Journal of Environmental Science and Technology, 15(5), 189–198. https://doi.org/10.5897/AJEST2021.3034

Adelekan, B. A., & Abegunde, K. D. (2020). Heavy metal contamination in soils and water in the vicinity of small-scale mining sites in Nigeria. Environmental Monitoring and Assessment, 192(5), 1–13. https://doi.org/10.1007/s10661-020-08314-3

Adeyeye, A. O., Akinola, M. O., & Oyelami, C. A. (2022). Soil heavy metal pollution and health risk assessment in mining-impacted areas of Nigeria. Environmental Research, 209, 112826. https://doi.org/10.1016/j.envres.2022.112826

Al-Khashman, O. A., & Shawabkeh, R. A. (2021). Heavy metal contamination in roadside soils: Case study of mining towns. Environmental Earth Sciences, 80(15), 1–11. https://doi.org/10.1007/s12665-021-09810-0

American Public Health Association (APHA). (2021). Standard Methods for the Examination of Water and Wastewater (23rd ed.). APHA, AWWA, WEF.

Chen, Y., Xu, W., Huang, Y., & Zhang, C. (2021). Health risk assessment of heavy metals in contaminated soil and bioaccumulation in crops: A case study from mining areas. Environmental Pollution, 286, 117555. https://doi.org/10.1016/j.envpol.2021.117555

Chibuike, G. U., & Obiora, S. C. (2021). Heavy metal polluted soils: Effect on plants and bioremediation methods. Environmental Toxicology and Pharmacology, 84, 103636. https://doi.org/10.1016/j.etap.2021.103636

Ekpo, A. P., Udofia, S. I., & Okon, A. U. (2021). Phytoremediation potentials of selected plants on heavy metal-contaminated soils in Nigeria. Nigerian Journal of Soil Science, 31(2), 84–92.

Fashola, M. O., Ngole-Jeme, V. M., & Babalola, O. O. (2020). Review of remediation techniques for heavy metal-contaminated soils. Environmental Technology & Innovation, 17, 100576. https://doi.org/10.1016/j.eti.2019.100576

Ighalo, J. O., Adeniyi, A. G., & Olasehinde, P. I. (2023). Heavy metals in Nigerian soils: Sources, health effects, and remediation approaches. Journal of Environmental Chemical Engineering, 11(2), 109254. https://doi.org/10.1016/j.jece.2023.109254

Kasambara, A., Ngetha, C., & Mtewa, A. (2020). Heavy metals in mining soils and potential ecological risks: Case study in Tanzania. Toxicology Reports, 7, 678–685. https://doi.org/10.1016/j.toxrep.2020.04.001

Keshavarzi, B., & Kumar, V. (2020). A review on assessment of heavy metals in the soil using geo-accumulation index and other pollution indices. Environmental Geochemistry and Health, 42, 1513–1538. https://doi.org/10.1007/s10653-019-00470-7

Müller, G. (1969). Index of geoaccumulation in sediments of the Rhine River. Geojournal, 2(3), 108–118.

Nguyen, H. T., Le, Q. V., & Vo, D. T. (2023). Cadmium and lead accumulation in soil and plants in relation to fertilizer application. Science of the Total Environment, 877, 162835. https://doi.org/10.1016/j.scitotenv.2023.162835

Obi, A., Ezeh, H. N., & Umeuduji, J. E. (2022). Assessment of heavy metals in agricultural soils around mining sites in Taraba State, Nigeria. Nigerian Journal of Soil and Environmental Research, 20(1), 34–42.

Oboh, O., Akinola, M. O., & Dauda, K. T. (2023). Ecological and human health risk assessment of heavy metals in soil from artisanal mining sites in Nigeria. Toxicology Reports, 10, 324–336. https://doi.org/10.1016/j.toxrep.2023.02.003

World Health Organization (WHO). (2022). Lead poisoning and health. https://www.who.int/news-room/fact-sheets/detail/lead-poisoning-and-health

Yuan, G., Wang, L., & Wang, W. (2020). Effects of nickel contamination on soil health and human exposure. Ecotoxicology and Environmental Safety, 189, 110022. https://doi.org/10.1016/j.ecoenv.2019.110022

Zhang, L., Wang, Y., Liu, X., & Feng, Y. (2021). Distribution and risk assessment of heavy metals in soil from mining regions in West Africa. Science of the Total Environment, 757, 143749. https://doi.org/10.1016/j.scitotenv.2020.143749