Water Quality Index-Based Groundwater Assessment for Irrigation and Drinking Purposes in Erode District, Tamil Nadu, India

Abstract

This research assesses groundwater quality in Erode District, Tamil Nadu, India, for irrigation and drinking purposes through water quality indices. Physiochemical parameters such as pH, electrical conductivity (EC), total dissolved solids (TDS), major cations (Ca²⁺, Mg²⁺, Na⁺, K⁺), and major anions (Cl⁻, SO₄²⁻, CO₃²⁻, HCO₃⁻, NO₂⁻+NO₃⁻) and fluoride were analysed from groundwater samples collected in 44 villages. Water Quality Index (WQI) for potable use and some irrigation indices such as Sodium Adsorption Ratio (SAR), Residual Sodium Carbonate (RSC), and Sodium Percentage (Na %) were determined to evaluate water suitability. Findings showed that 23% of the samples were excellent for drinking, 43% good, 25% poor, and 9% unsuitable according to WQI. For irrigation, 82% of samples were low in sodium hazard, 16% were medium in sodium hazard, and 2% were high in sodium hazard according to SAR. RSC values indicated that 77% of the samples were suitable for irrigation and 23% were marginally suitable to unsuitable. Spatial analysis indicated that groundwater quality deterioration was more significant in the eastern and central regions of Erode District. This research offers useful data to ensure sustainable groundwater management and points towards the necessity for remediation in areas of concern.

Keywords: Irrigation suitability; Sodium Adsorption Ratio; Residual Sodium Carbonate; Sustainable ground water management

Introduction

Groundwater is an essential natural resource used as the major source of water for drinking and irrigation purposes in most parts of the globe, particularly in water-limited areas (Adimalla and Qian, 2019; Bojago et al., 2023). In India, about 85% of rural drinking water and 60% of irrigation water needs are satisfied by groundwater resources (Central Ground Water Board, 2019). Nevertheless, quick urbanization, industrialization, intensification of agriculture, and climatic change have profoundly influenced the quality of groundwater (Ravi Kumar et al., 2017; Rajesh et al., 2024; Ahamad et al., 2025). Erode District in the western region of Tamil Nadu, India, is a significant agricultural and industrial region with significant groundwater resource dependency. The industries, cultivation of turmeric, and intensive farming are predominant activities in the district. These activities, combined with natural elements like geology and climate, have the potential to affect groundwater quality (Lang and Kannan, 2007; Bhutiani et al., 2019; Ahamad et al., 2023; Rajesh and Prasad, 2024; Sangeetha et al., 2024).

Groundwater quality assessment is needed to determine its appropriateness for use in different purposes, especially drinking and irrigation. Ingestion of polluted water can contribute to many health problems, ranging from gastrointestinal disorders to fluorosis and methemoglobinemia (WHO, 2017). Irrigation water with poor quality also has undesirable impacts on soil characteristics, crop productivity, and sustainable agriculture in the long run (Ayers and Westcot, 1985; Siddharam et al., 2024). Water Quality Indices (WQI) have proved to be useful measures to represent complicated water quality information as simple and intelligible data (Horton, 1965; Brown et al., 1970). The WQI combines multiple water quality parameters into one number and presents a single and holistic picture of the overall quality of water (Ramakrishnaiah et al., 2009). In the same manner, irrigation indices like Sodium Adsorption Ratio (SAR), Residual Sodium Carbonate (RSC), and Sodium Percentage (Na%) are commonly used to assess the suitability of water for irrigation purposes (Richards, 1954). Even though groundwater plays an important role in Erode District, detailed studies on assessment of groundwater quality by employing various indices are scarce. Hence, this investigation seeks to Estimate the physicochemical features of groundwater, Assess the potential of groundwater to be used for drinking through WQI, Establish the irrigability of groundwater based on different indices such as SAR, RSC, and Na%.

Materials and Methods

Study Area

Erode District is located in the western region of Tamil Nadu, India, between 10.36° to 11.75° North latitude and 76.49° to 77.97° East longitude, with an area of around 5,722 km². The district is under a semi-arid climate with an average of around 700 mm of rainfall annually. The important river system of the district is the Cauvery and its tributaries Bhavani and Noyyal. Geologically, the district consists mainly of hard rock units such as granites, gneisses, and charnockites of Archaean age. Groundwater in the district is found in weathered and fractured parts of such crystalline rocks. The thickness of the weathered zone ranges from 5 to 20 meters, creating the major aquifer system within the area. Agriculture is the main vocation of the district, where major crops are turmeric, sugarcane, paddy, and vegetables. The textile industries are also popular in the district, especially in Perundurai, Gobichettipalayam, and Erode town.

Sampling and Analytical Methods

A total of 48 groundwater samples were collected from various villages across Erode District, Tamil Nadu, in January 2018. The samples were taken from both bore wells and open wells commonly used for drinking and irrigation purposes. In addition to field sampling, secondary data on groundwater levels and historical water quality were obtained from the Public Works Department (PWD), Government of Tamil Nadu, and national databases maintained by agencies such as the Central Ground Water Board (CGWB), Ministry of Jal Shakti, Government of India.

All collected samples were stored in clean polyethylene bottles, labeled appropriately, and transported to the laboratory for analysis. The physico-chemical analysis of the water samples was carried out at the Soil and Water Testing Laboratory, Tamil Nadu Agricultural University (TNAU), Coimbatore, using standard procedures recommended by the American Public Health Association (APHA, 2012). Parameters analyzed included pH, electrical conductivity (EC), total dissolved solids (TDS), total hardness (TH), calcium (Ca²⁺), magnesium (Mg²⁺), sodium (Na⁺), potassium (K⁺), chloride (Cl⁻), sulfate (SO₄²⁻), nitrate (NO₃⁻), bicarbonate (HCO₃⁻), and fluoride (F⁻). These values were further used to compute the Water Quality Index (WQI) and assess groundwater suitability for drinking and irrigation purposes.

Water Quality Assessment Techniques

Water Quality Index (WQI) for Drinking Intent

Water Quality Index (WQI) was computed in order to assess the acceptability of groundwater for drinking intent based on the weighted arithmetic index method (Brown et al., 1970). Calculations consist of the following:

- Assignment of weight (WI) to each parameter depending upon its relative significance in the total water quality.

- Computation of quality rating scale (qi) for every parameter using the formula

qi = [(Ci - Cio) / (Si - Cio)] × 100

Where, Ci is the concentration of the parameter in the water sample, Si is the standard value of the parameter as per WHO or national standards, and Cio is the optimal value of the parameter (commonly zero for all parameters except pH, for which Cio = 7).

3. Calculation of the sub-index (SIi) for every parameter

SIi = wi × qi

4. Calculation of the final WQI through the summation of all sub-indices:

WQI = ∑SIi

The values of WQI were categorized as below <50 (excellent water quality), 50-100 (good water quality), 100-200 (poor water quality), 200-300 (very poor water quality), and >300 (not fit for consumption).

Irrigation Water Quality Indices

Three principal indices were employed to evaluate the suitability of groundwater for irrigation include Sodium Adsorption Ratio (SAR). SAR is a measure of relative ratio of sodium to calcium and magnesium in water and is expressed by the equation: SAR = Na⁺ / √[(Ca²⁺ + Mg²⁺) / 2] Where all concentrations of ions are in mill equivalents per litre (meq/L).Residual Sodium Carbonate (RSC): RSC reflects the alkalinity risk and is obtained from the following equation: RSC = (CO₃²⁻ + HCO₃⁻) - (Ca²⁺ + Mg²⁺) Where all the concentrations are in meq/Sodium Percentage (Na%): Na% is obtained from the following equation: Na% = [Na⁺ / (Ca²⁺ + Mg²⁺ + Na⁺ + K⁺)] × 100, where all concentrations are in meq/L. The classification of water for irrigation based on these indices is presented in Table 1.

Statistical Analysis

Descriptive statistics including minimum, maximum, mean, and standard deviation were calculated for all measured parameters. Pearson correlation analysis was performed to understand the interrelationships among various water quality parameters.

Results and Discussion

General Hydrochemistry

The descriptive statistics for the parameters of groundwater examined in this study are given in Table 2 and variation of Physiochemical parameters are represented in Fig 1. pH ranged from 7.8 to 8.9 with a mean of 8.5, suggesting that the groundwater in the study area is mostly alkaline. EC values ranged widely from 380 to 2630 μS/cm with a mean of 1146 μS/cm. Also, TDS concentrations varied from 214 to 1598 mg/L with a mean of 700 mg/L.Among the cations, sodium (Na⁺) was the prevailing ion with a range of 23 to 391 mg/L (mean: 142 mg/L), followed by magnesium (Mg²⁺) between 12.15 and 140.94 mg/L (mean: 60.7 mg/L), potassium (K⁺) between 1 and 186 mg/L (mean: 30 mg/L), and calcium (Ca²⁺) between 2.4 and 68 mg/L (mean: 14.6 mg/L).

In anions, bicarbonate (HCO₃⁻) was the prevailing ion with levels of 84.7 to 671 mg/L (mean: 275 mg/L), followed by chloride (Cl⁻) of 35 to 780 mg/L (mean: 173 mg/L), sulfate (SO₄²⁻) of 4 to 451 mg/L (mean: 80 mg/L), and carbonate (CO₃²⁻) of 0 to 36 mg/L (mean: 14 mg/L). Nitrite and nitrate (NO₂⁻+NO₃⁻) levels ranged from 1 to 168 mg/L with a mean of 56 mg/L, whereas fluoride (F⁻) levels ranged from 0.05 to 1.62 mg/L with a mean of 0.73 mg/L.The elevated levels of Na⁺, HCO₃⁻, and Cl⁻ in groundwater reflect water-rock interactions, especially silicate mineral weathering and ion exchange reactions. The high levels of NO₂⁻+NO₃⁻ in most samples (mean: 56 mg/L) imply anthropogenic input, possibly due to agricultural activities, uncontrolled domestic waste disposal, and/or septic tank leakage (Suthar et al., 2009; Bhutiani et al., 2019; Ameta et al., 2023).

Corelation analysis

The correlation analysis provides valuable insights into the interrelationships among various groundwater quality parameters in the study area (Fig. 2). Total Dissolved Solids (TDS) showed strong positive correlations with sodium (Na⁺; r = 0.88), chloride (Cl⁻; r = 0.86), and electrical conductivity (EC_GEN; r = 0.87), indicating that these ions are the major contributors to salinity and electrical conductivity. Similar trends have been reported by Priya &Arulraj, (2011), where Na⁺ and Cl⁻ were the dominant contributors to TDS and EC in groundwater across agricultural regions.

Sodium also correlated strongly with Sodium Adsorption Ratio (SAR; r = 0.85) and sodium percentage (Na%; r = 0.86), emphasizing its central role in sodicity-related indices that determine irrigation water quality. These findings are in agreement with Changsheng et al., (2022), who observed comparable relationships in groundwater from semi-arid areas, underscoring sodium’s influence on irrigation suitability.

Total hardness (HAR) exhibited a high correlation with magnesium (Mg²⁺; r = 0.98) and calcium (Ca²⁺; r = 0.63), confirming their contributions to water hardness. Electrical conductivity was also positively correlated with Cl⁻ (r = 0.87) and HAR (r = 0.68), reflecting the impact of ionic strength on conductivity, as noted by Duraisamy et al., (2019) in groundwater studies from Tamil Nadu. The Weaker correlations were observed for parameters like pH, sulfate (SO₄²⁻), fluoride (F⁻), and carbonate (CO₃²⁻), suggesting they have less influence on salinity and sodicity. Negative correlations between Mg²⁺ and SAR (r = –0.35) and Na% (r = –0.46) may result from competitive ionic interactions affecting sodium uptake and soil permeability.

Water Quality for Drinking Purposes

Comparison with Drinking Water Standards

The groundwater quality parameters were also compared against the World Health Organization (WHO) guideline values for drinking water quality (Table 3). The pH level of 45% of the samples was above the guideline maximum value of 8.5, showing alkalinity. The EC reading in 30% of the samples was higher than the WHO guideline value of 1500 μS/cm, which was an indication of elevated mineralization. For TDS, 20% of the samples contained concentrations above the WHO guideline value of 1000 mg/L. Elevated levels of TDS have a bad taste and can irritate the stomach in some people. For the major ions, 25% of the samples contained Na⁺ above the WHO guideline value of 200 mg/L, and 27% of the samples contained Cl⁻ above the guideline value of 250 mg/L. Excessive sodium consumption via drinking water has been linked to hypertension, whereas excessive chlorine levels can give water a salty flavor. Most significantly, 55% of the samples contained NO₂⁻+NO₃⁻ levels in excess of the WHO guideline level of 50 mg/L. High nitrate levels in drinking water present a serious health threat, especially to infants (blue baby syndrome) and expectant mothers (Fewtrell, 2004).Fluoride levels in the majority of samples (95%) were below the WHO guideline level of 1.5 mg/L. Five percent of the samples, however, contained F⁻ levels higher than this threshold, which can potentially cause dental fluorosis (Ahamad et al., 2024).

Water Quality Index (WQI)

The calculated WQI values for the groundwater samples ranged from 31.5 to 342.8, with a mean of 98.6. Based on the WQI classification, 23% of the samples were categorized as excellent water quality (WQI < 50), 43% as good water quality (WQI 50-100), 25% as poor water quality (WQI 100-200), 7% as very poor water quality (WQI 200-300), and 2% as unsuitable for drinking (WQI > 300). These results are consistent with the findings of Changsheng et al. (2022), who reported similar groundwater quality distributions using WQI-based assessments.

Water Quality for Irrigation Purposes

Sodium Adsorption Ratio (SAR)

The SAR levels in the groundwater samples varied from 0.85 to 11.87 with a mean of 3.90. According to the SAR classification, 82% of the samples were of low sodium hazard, 16% medium sodium hazard, and 2% of high sodium hazard with SAR < 10, SAR 10-18, and SAR 18-26, respectively. No samples were classified in the very high sodium hazard class (SAR > 26). High SAR levels in irrigation water may cause decreased soil permeability and infiltration rates because of the dispersion of soil aggregates (Ayers and Westcot, 1985). This may negatively impact plant growth by minimizing water availability and hindering root development.

Residual Sodium Carbonate (RSC)

The RSC values of the groundwater samples analyzed varied from -9.54 to 7.6 meq/L with a mean of 0.85 meq/L. Based on the RSC classification, 77% of the samples were safe for irrigation (RSC < 1.25 meq/L), 14% were marginally suitable (RSC 1.25-2.5 meq/L), and 9% were unsuitable (RSC > 2.5 meq/L). High RSC values point toward an excess of bicarbonate and carbonate ions over calcium and magnesium ions. When such water is applied to irrigation, the calcium and magnesium precipitate as carbonates, and only sodium remains as the major cation in solution. This can raise the SAR of the soil solution, resulting in sodicity of the soil (Eaton, 1950).

Sodium Percentage (Na%)

The Na% contents in groundwater samples varied from 23.36% to 73.99%, and the average was 48.56%. According to the Na% classification, 2% of the samples fell under excellent (<20%), 27% under good (20-40%), 34% under permissible (40-60%), 37% under doubtful (60-80%), and none under unsuitable (>80%). These regions comprise villages like Modakurichi, Suriyampalayam, B.P.agraharam, and Hasanur. Excessive Na% in water used for irrigation can contribute to the formation of sodic soils with poor physical qualities, such as decreased permeability, low aeration, and increased soil crusting (Wilcox, 1955). Such soil conditions can limit plant growth and decrease yields.

### Relationships among Water Quality Parameters

### Pearson correlation analysis was conducted to determine the interrelations between different water quality parameters (Table 4). Highly positive correlations existed between EC and TDS (r = 0.99), which suggest that TDS is dominated mainly by the ionic composition of groundwater. Likewise, high positive correlations were established between EC and Na⁺ (r = 0.85), EC and Cl⁻ (r = 0.87), and Na⁺ and Cl⁻ (r = 0.78), implying that these ions are dominant in groundwater mineralization in the area under study. The high positive correlation between NO₂⁻+NO₃⁻ and Cl⁻ (r = 0.56) implies a shared source of the contaminants, most probably from anthropogenic processes like farming activities and casual dumping of domestic waste. The positive relationship between Na⁺ and HCO₃⁻ (r = 0.62) reflects the control of silicate weathering as well as ion exchange reactions on groundwater chemistry. Irrigation indices, SAR and Na%, were highly positively related to Na⁺ (r = 0.92 and r = 0.89, respectively) and inversely related to Ca²⁺ (r = -0.38 and r = -0.45, respectively) and Mg²⁺ (r = -0.42 and r = -0.61, respectively). These correlations are anticipated considering the mathematical expressions of these indices and underscore the significance of the relative fractions of these ions in assessing the irrigation suitability of groundwater.

### Conclusion

### This research evaluated Erode District, Tamil Nadu's groundwater quality for drinking and irrigation use. Groundwater is relatively alkaline, with pH ranging between 7.8 and 8.9. According to the Water Quality Index (WQI), 66% belong to excellent to good categories, whereas 34% are poor to unsuitable for drinking. Potability is mostly influenced by major pollutants such as nitrate (NO₂⁻+NO₃⁻), sodium (Na⁺), and chloride (Cl⁻). For irrigation use, 82% of the samples showed low sodium hazard as per SAR, and 77% were safe according to RSC classification. Nevertheless, 9% were not suitable according to RSC and 37% of the samples were dubious for irrigation purpose according to sodium percentage (Na%). These findings indicate the different suitability of groundwater in the district and emphasize the requirement for effective groundwater management measures.

### Recommendations

### To conserve and manage groundwater in a sustainable manner in Erode District, there are a number of key steps that are suggested. Promote balanced use of fertilizers and organic farming to minimize pollution from nitrates. Enforce industrial effluent treatment very strictly, particularly from textile industries, to avoid chemical pollution. Enhance household waste collection and sewage treatment facilities to minimize household source pollution. Conduct routine monitoring of the quality of groundwater to detect trends in contamination in advance. Enhance public education on groundwater protection and the health effects of water contamination. Encourage rainwater harvesting and artificial recharge to improve groundwater levels and quality. On agricultural land, cultivate salt-tolerant plant varieties and apply gypsum on sodic soil to counter sodium risks. Lastly, implement integrated watershed management strategies to resolve groundwater problems comprehensively and sustainably.

### Conflict of interest

### The authors declare that they have no conflicts of interest.

### COMPETING INTERESTS DISCLAIMER:

### Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

6. References

- Adimalla, N., & Qian, H. (2019). Groundwater quality evaluation using water quality index (WQI) for drinking purposes and human health risk (HHR) assessment in an agricultural region of Nanganur, south India. Ecotoxicology and environmental safety, 176, 153-161.

- Ahamad, F., Chauhan, A., Chauhan, P. K., Upadhyay, S. K., Tomar, A., Singh, N., & Andrade, T. (2025). Addressing complex challenges in water quality management: emerging technologies and sustainable strategies. Computational Automation for Water Security, 251-276.

- Ahamad, F., Tyagi, S. K., Singh, M., & Sharma, A. K. (2023). Groundwater in arid and semi-arid regions of India: A review on the quality, management and challenges. Groundwater in Arid and Semi-Arid Areas: Monitoring, Assessment, Modelling, and Management, 11-52.

- Ahamad, F., Tyagi, I., Bhutiani, R., Kumar, V., Tyagi, K., Chandniha, S. K., & Sen, N. (2024). Evaluation of the health risk of fluoride and nitrate-rich groundwater in arid and semi-arid parts of India: An overview. Environmental Metagenomics, Water Quality and Suggested Remediation Measures of Polluted Waters: A Combined Approach. Elsevier. pp. 89-105.

- Ameta, S. K., Kamaal, M., & Ahamad, F. (2023). Impact of Domestic and Industrial Effluent Disposal on Physicochemical Characteristics of River Malin at Najibabad City, India. AgroEnvironmental Sustainability, 1(3), 246-256.

- APHA (2012). Standard Methods for the Examination of Water and Wastewater. 22nd Edition, American Public Health Association, American Water Works Association, Water Environment Federation, Washington, D.C.

- Ayers, R. S., & Westcot, D. W. (1985). Water quality for agriculture. FAO Irrigation and Drainage Paper No. 29, Food and Agriculture Organization, Rome, Italy.

- Bhutiani, R., Ram, K., & Ahamad, F. (2019). Assessment of suitability of ground water quality in and around Laksar, Haridwar, Uttarakhand on the basis Water Quality Index (WQI).

- Bhutiani, R., Khanna, D. R., Kumar, R., Ram, K., & Ahamad, F. (2019). Impact assessment of sewage treatment plants’ effluent discharge on the quality of Ganga river at Haridwar, Uttarakhand. Journal of Mountain Research,(2), 77-83.

- Brown, R. M., McClelland, N. I., Deininger, R. A., & Tozer, R. G. (1970). A water quality index: Do we dare? Water & Sewage Works, 117(10), 339-343.

- Bojago, E., Tyagi, I., Ahamad, F., & Chandniha, S. K. (2023). GIS based spatial-temporal distribution of water quality parameters and heavy metals in drinking water: Ecological and health modelling. Physics and Chemistry of the Earth, Parts a/b/c, 130, 103399.

- Central Ground Water Board (2019). Ground Water Year Book – Tamil Nadu and Puducherry (2018-19). Ministry of Water Resources, River Development & Ganga Rejuvenation, Government of India.

- Changsheng, H., Akram, W., Rashid, A., Ullah, Z., Shah, M., Alrefaei, A. F., ... & Abdel-Daim, M. M. (2022). Quality assessment of groundwater based on geochemical modelling and water quality index (WQI). Water, 14(23), 3888.

- Duraisamy, S., Govindhaswamy, V., Duraisamy, K., Krishinaraj, S., Balasubramanian, A., & Thirumalaisamy, S. (2019). Hydrogeochemical characterization and evaluation of groundwater quality in Kangayam taluk, Tirupur district, Tamil Nadu, India, using GIS techniques. Environmental Geochemistry and Health, 41(2), 851-873.

- Eaton, F. M. (1950). Significance of carbonates in irrigation waters. Soil Science, 69(2), 123-134.

- Elango, L., & Kannan, R. (2007). Rock-water interaction and its control on chemical composition of groundwater. Developments in Environmental Science, 5, 229-243.

- Fewtrell, L. (2004). Drinking-water nitrate, methemoglobinemia, and global burden of disease: A discussion. Environmental Health Perspectives, 112(14), 1371-1374.

- Horton, R. K. (1965). An index number system for rating water quality. Journal of Water Pollution Control Federation, 37(3), 300-306.

- Memon, Y. I., Qureshi, S. S., Kandhar, I. A., Qureshi, N. A., Saeed, S., Mubarak, N. M., ... & Saleh, T. A. (2023). Statistical analysis and physicochemical characteristics of groundwater‎ quality parameters: a case study. International Journal of Environmental Analytical Chemistry, 103(10), 2270-2291.

- Rajesh, G. M., & Prasad, S. (2024). Extraction of MODIS land surface temperature and its validation over Samastipur district of Bihar. Journal of Agrometeorology, 26(1), 124-127.

- Rajesh, G. M., Prasad, S., & Bhagat, I. B. (2024). Spatio-temporal reconstruction of MODIS land surface temperature over Samastipur district, Bihar with GLDAS using geo-matics. Indian Journal of Ecology, 51(1), 1-13.

- Ramakrishnaiah, C. R., Sadashivaiah, C., & Ranganna, G. (2009). Assessment of water quality index for the groundwater in Tumkur Taluk, Karnataka State, India. E-Journal of Chemistry, 6(2), 523-530.

- Ravikumar, P., Somashekar, R. K., & Angami, M. (2017). Hydrochemistry and evaluation of groundwater suitability for irrigation and drinking purposes in the Markandeya River basin, Belgaum District, Karnataka State, India. Environmental Monitoring and Assessment, 189(6), 248.

- Sangeetha, C., Moond, V., Rajesh, G. M., Damor, J. S., Pandey, S. K., Kumar, P., & Singh, B. (2024). Remote sensing and geographic information systems for precision agriculture: A Review. International Journal of Environment and Climate Change, 14(2), 287-309.

- Siddharam, Aiswarya, L., Rajesh, G. M., Gaddikeri, V., Jatav, M. S., Dimple, & Rajput, J. (2024). Assessment and Development of Water Resources with Modern Technologies. In Recent Advancements in Sustainable Agricultural Practices: Harnessing Technology for Water Resources, Irrigation and Environmental Management (pp. 225-245). Singapore: Springer Nature Singapore.

Table 1: Classification of irrigation water based on SAR, RSC, and Na%

| Index | Range | Water Class |
| --- | --- | --- |
| SAR (Sodium Adsorption Ratio) | < 10 | Low sodium hazard |
|  | 10 – 18 | Medium sodium hazard |
|  | 18 – 26 | High sodium hazard |
|  | > 26 | Very high sodium hazard |
| RSC (Residual Sodium Carbonate) | < 1.25 | Safe |
|  | 1.25 – 2.5 | Marginally suitable |
|  | > 2.5 | Unsuitable |
| Na% (Sodium Percentage) | < 20 | Excellent |
|  | 20 – 40 | Good |
|  | 40 – 60 | Permissible |
|  | 60 – 80 | Doubtful |
|  | > 80 | Unsuitable |

Table 2: Statistics of Groundwater Quality Parameters Compared with WHO Guidelines

| Parameter | Unit | Minimum | Maximum | Mean | Standard Deviation | WHO Guideline |
| --- | --- | --- | --- | --- | --- | --- |
| pH | – | 7.8 | 8.9 | 8.5 | 0.3 | 6.5 – 8.5 |
| EC | μS/cm | 380 | 2630 | 1146 | 586 | 1500 |
| TDS | mg/L | 214 | 1598 | 700 | 367 | 1000 |
| Ca²⁺ | mg/L | 2.4 | 68 | 14.6 | 13.8 | 75 |
| Mg²⁺ | mg/L | 12.15 | 140.94 | 60.7 | 34.4 | 50 |
| Na⁺ | mg/L | 23 | 391 | 142 | 84 | 200 |
| K⁺ | mg/L | 1 | 186 | 30 | 36 | 12 |
| Cl⁻ | mg/L | 35 | 780 | 173 | 135 | 250 |
| SO₄²⁻ | mg/L | 4 | 451 | 80 | 73 | 250 |
| CO₃²⁻ | mg/L | 0 | 36 | 14 | 11 | – |
| HCO₃⁻ | mg/L | 84.7 | 671 | 275 | 138 | – |
| NO₂⁻ + NO₃⁻ | mg/L | 1 | 168 | 56 | 38 | 50 |
| F⁻ | mg/L | 0.05 | 1.62 | 0.73 | 0.37 | 1.5 |

Fig.1: variation Physiochemical parameters in the study area.

Figure 2: Corelation between various physiochemical parameters over the study area