High Resolution Object-Based Mapping of Shallow Water Benthic Habitats Using Drone Imagery in the Gorontalo Bay Marine Conservation Area, Dulanga Beach, Gorontalo Regency

.

ABSTRACT

Aims: This study aimed to produce accurate and efficient maps of shallow-water benthic habitats in support of marine conservation area designation, addressing the scarcity of detailed spatial data in Gorontalo Province. Study design: Experimental field-based remote sensing study using unmanned aerial vehicle (UAV) imaging and object-based image analysis (OBIA). Place and Duration of Study: Conducted in the Coastal Conservation Area of Dulanga Beach, Gorontalo Regency, Indonesia, between June and July 2025. Methodology: A drone platform (Autel Evo II Pro V3) was used to acquire imagery at four time intervals (08:00, 10:00, 12:00, and 14:00 Ante Meridiem (AM) local time) under two lens angle configurations (90° and 45°). Data were processed through an OBIA workflow. At level 1, contextual editing delineated principal objects based on spatial context. At level 2, supervised classification was applied using the support vector machine (SVM) algorithm, with field survey data serving as reference inputs. A spatial segmentation scale of 50 was tested for optimal delineation of six benthic habitat classes. Results: The optimal acquisition occurred at 10:00 AM local time with a 90° lens angle. Under these conditions, SVM classification with segmentation scale 50 yielded an overall accuracy of 91.75% and a Kappa coefficient of 0.8999, indicating very high concordance with field validation. Conclusion: Object-based drone remote sensing provides a highly accurate and efficient method for benthic habitat mapping in shallow waters. The findings support improved spatial planning and management of marine conservation areas in Gorontalo Province.

Keywords: Benthic habitats; drone; Dulanga Beach; mapping; OBIA.

1. INTRODUCTION

The Blue Economy concept has emerged as a strategic approach to promoting sustainable marine development by balancing economic growth with marine ecosystem conservation. One of its key pillars is the designation of marine protected areas (MPAs), as legally mandated by the Indonesian Ministry of Marine Affairs and Fisheries through Ministerial Regulation No. 14 of 2016 and reinforced by Regulation No. 31 of 2020 concerning the governance of marine conservation areas. The Indonesian Ministry of Marine Affairs and Fisheries has set a target of marine conservation areas to cover 30% of the total national marine area by 2045. By the end of 2024,  the total area of marine conservation areas had reached 29.9 million hectares, equivalent to 9.2% of Indonesia's marine area (KKP, 2024). In Gorontalo Province, conservation areas have been included in the 2024 Regional Spatial Plan, covering more than 159,000 hectares. One of the legally established MPAs is the Gorontalo Bay Marine Conservation Area, designated under Ministerial Decree No. 127 of 2023, with a total area of 76,580.48 hectares encompassing core zones, limited-use zones, and other functional zones.

The success of marine conservation area management largely depends on the availability of accurate spatial data, especially regarding the condition of benthic habitats in shallow waters. Official conservation area designation documents generally only include general zoning information without detailed habitat classifications. However, in-depth spatial information is crucial for formulating ecosystem-based zoning policies and for more precise monitoring of environmental dynamics. Benthic habitat mapping is critical because it clearly explains underwater ecosystems' distribution, extent, and condition (Purkis et al., 2019). This information serves as a vital scientific foundation for informed conservation decision-making.

Remote sensing technology has become the primary method for mapping benthic habitats. Yet, regarding shallow waters, the limited resolution of satellite images often makes it hard to distinguish finer habitat details. As technology advances, drones (unmanned aerial vehicles/UAVs) offer an alternative solution that can produce high-resolution images and high operational flexibility in coastal areas (Klemas, 2015). In addition to providing detailed visual results, drones also enable object-based image analysis (OBIA), which can improve the accuracy of habitat classification. Nababan et al. (2021) also found that taking images at a 45° angle can help minimize visual disruptions like sunlight reflections (sunglint), a common obstacle when mapping shallow-water habitats.

This study focuses on Dulanga Beach, one of the limited-use zones in the Gorontalo Bay Conservation Area (Gorontalo Provincial Regulation No. 2 of 2024), which ecologically has ideal characteristics such as water clarity, minimal human disturbance, and high accessibility. To date, no drone-based benthic habitat mapping studies have been conducted in this area or other coastal areas of Gorontalo. This study aims to evaluate the quality and accuracy of benthic habitat classification results based on drone imagery, verified through field data (ground truthing) based on drone flight time and sensor angle. As an initial study, this research is expected to fill the spatial data gap at Dulanga Beach while also serving as a foundation for developing drone-based benthic habitat mapping methods across the broader coastal areas of Gorontalo.

2. methodology

2.1 Location and Time of Research

Data collection for this study was conducted from June to July 2025 at Pantai Dulanga, located in Gorontalo Regency, Indonesia. Geographically, the study area is situated between 0°29’33.97” N–123°1’44.04” E and 0°29’42.71” N–123°1’47.09” E. The study site map is presented in Fig. 1.

Fig. 1. Study area map and sampling locations

2.2 Data and Equipment

The data used in this study were drone data acquired on 14–15 June 2025 and ground truth data. The drone was an Autel Evo II Pro V3 with an RGB (red, green, blue) camera lens. The application used to operate the drone was the Autel Explorer application installed on the drone's remote control. The equipment used for field data collection included a Garmin 78s Global Positioning System (GPS), snorkelling and scuba diving gear, square transects, an underwater camera, underwater writing tools, and waterproof plastic. Data processing and analysis were conducted using an Intel Core i7 laptop with 16 GB Random Access Memory (RAM), equipped with software such as: Archmap 10.8, Quantum Geographic Information System (QGIS), Ecognition Developer, Agisoft PhotoScan, Global Mapper, and Microsoft Office.

2.3 Data Collection

Benthic habitat data collection in this study used stratified random sampling, in which the study area was divided into several strata based on substrate characteristics such as sand, mud, live coral reefs, dead reefs, seagrass, rubble, and algae. After stratification, sample points were randomly selected from each stratum to ensure proportional representation of all habitat types (Congalton & Green, 2019). Field data was collected through direct visual observation and documentation using 30x30 cm² square transect photographs (Siregar, 2010), with a camera and a Garmin 78s handheld GPS device to record precise location coordinates. Transect size adjustments were made to ensure spatial accuracy based on the accuracy of the GPS device used. A total of 189 observation points were collected for benthic habitat data, divided into two parts: (1) 94 points as accuracy test data from image classification results and (2) 95 points as thematic layer data in the image classification process.

2.4 Drone Image Acquisition

The drone flight path planning uses a grid pattern that has proven effective in ensuring systematic area coverage and facilitating the image mosaicking to produce high-quality visual output (Øie et al., 2024). Image acquisition was carried out at an altitude of 100 meters with a maximum speed of 14 m/s, and frontlap and sidelap settings of 80% each. Area coverage is adjusted to the conditions of the research location and the drone's battery capacity, which affects flight duration. The entire aerial photography acquisition process was carried out in June 2025. Environmental conditions, such as wind speed, currents, waves, cloud cover, and sunlight intensity, influence the quality of aerial images drones produce. The best image results are obtained when the water is calm, the sky is clear, the wind speed is low, and no sunlight is reflected on the water surface (sunglint). Sunglint can obscure important benthic features in high-resolution UAV images, compromising coral reef classification accuracy (Qin et al., 2024). Additionally, sunglint can cause white spots in images, which may lead to errors in the classification of benthic habitats (Nugroho et al., 2024). In this study, drone image acquisition was carried out by dividing the acquisition time into four categories to evaluate the optimal time for producing high-quality images, namely 08:00, 10:00, 12:00, and 14:00 Ante Meridiem (AM) local time. Additionally, the camera lens tilt angle was divided into a 90° vertical angle and a 45° inclined angle to assess their impact on image quality and benthic visibility.

2.5 Pre-processing of Drone Images (Digital Orthophotos)

Digital orthophotos aim to produce images that have accurate spatial representation and uniform scale (Wolf & Dewitt, 2000). The process begins with image alignment, where Agisoft Metashape processing software identifies features or tie points between overlapping images. This process forms an initial three dimensions (3D) spatial structure through the Structure from Motion (SfM) method, which produces sparse point clouds (Westoby et al., 2012). Next, a dense point cloud is formed, representing the surface structure in greater detail. Following this, a digital surface model (DSM) is created to represent the elevation of the surface captured by the camera, which is then used in the orthorectification process. The geometrically corrected images are then combined through a mosaicking process, producing a final product in the form of an orthomosaic or orthophoto, which is a single high-resolution image with a geographic coordinate system that can be used for further spatial analysis (Turner et al., 2012).

2.6 Image Segmentation

Segmentation is a critical stage in the OBIA approach, aimed at simplifying imagery by grouping pixels with similar spectral and spatial characteristics into meaningful objects (Mastu, 2018; Lillesand et al., 2015). In this study, segmentation was performed using the Multiresolution Segmentation (MRS) algorithm developed by Baatz (2000), employing a region-growing method based on homogeneity criteria. Three key parameters in MRS (shape, compactness, and scale) were used to control the form and size of the resulting segments. The shape parameter adjusts the influence of shape versus color, compactness regulates the compactness of the segments, and scale determines the level of segmentation detail, which is adjusted according to classification requirements (Benfield et al., 2007). This study applied a two-level segmentation approach: Level 1 for reef-scale delineation and Level 2 for benthic habitat class segmentation. At Level 1, a scale parameter of 200 is used to distinguish between land, shallow water, and deep water, referring to Nababan et al. (2021). Meanwhile, at Level 2, the segmentation process is carried out with a more detailed scale, namely 50, referring to the approach proposed by Nugroho et al. (2024). This scale is recommended for accurately delineating dominant benthic cover classes in shallow-water environments, aligning with aquatic habitat mapping classification principles.

2.7 Image Classification

Benthic habitat classification is an essential step in shallow water mapping, which aims to group image objects based on spectral and spatial characteristics to produce accurate thematic maps. Benthic habitat classification typically relies on several environmental factors, such as the type of sea floor, the main biological cover, landform features, depth, and how much waves and currents influence the area (Roelfsema et al., 2020). In practice, the process starts by picking out parts of the image that look visually consistent. These areas are then grouped based on their similarities and given names that describe the kind of habitat they represent (Green et al., 2000; Lillesand et al., 2015). These areas are then grouped based on shared characteristics and assigned habitat labels that reflect their ecological identity (Green et al., 2000; Lillesand et al., 2015). In this study, the OBIA approach was used as a drone image classification method because it can combine spectral and spatial information simultaneously (Blaschke, 2010). Classification was performed at two levels, where level 1 used the contextual editing method with threshold values obtained through trials, and level 2 used the Support Vector Machine (SVM) algorithm based on field data (Mastu, 2018). The SVM algorithm implemented through the eCognition Developer 8.7 software was developed by Cortes & Vapnik (1995) and is known as one of the most effective machine learning algorithms for optimally distinguishing classes through a separating function (hyperplane).

2.8 Accuracy Assessment

Accuracy assessment of the drone image classification results was conducted to evaluate the reliability of the classification method used in mapping shallow-water benthic habitats. The method applied was the confusion matrix, a standard approach in remote sensing for comparing classified image outputs with ground-reference data (Prayuda, 2014). According to the Indonesian National Standard (Badan Standardisasi Nasional, 2011), the minimum acceptable accuracy level for benthic habitat maps is 60%.

The key components of a confusion matrix include Overall Accuracy (OA), Producer Accuracy (PA), User Accuracy (UA), and the Kappa coefficient. Accuracy metrics were calculated using the following formulas (Congalton & Green, 2019):

| OA= i=1knii .................................................................................................................. | (1) |
| --- | --- |

| PA= njjn+j ......................................................................................................................... | (2) |
| --- | --- |

| UA= niini+ ......................................................................................................................... | (3) |
| --- | --- |

Where k is the number of rows in the matrix, nii is the number of observations in the column i and row i; njj is the number of observations in the column j and row j; ni+ is the marginal total of the row i; and n+j is the marginal total of the column j. Kappa values ​​range from 0 to 1, where the level of agreement strength is interpreted according to the categories proposed by Fleiss (1981), as shown in Table 1. The calculation of the kappa value can be explained as follows (Green et al, 2000):

| K=N i=1knii- i=1k(ni+n+i)N2-i=1k(ni+n+i) ................................................................................................. | (4) |
| --- | --- |

Where K is the kappa coefficient, N is the total observations; k is the number of row in the matrix; nii is the number of observations in the column i and row i; ni+ is the marginal total of the row i; and n+i is the marginal total of the column i.

Table 1.	Kappa index categories

| Indeks Kappa | Category |
| --- | --- |
| < 0 0.1 – 0.2 0.21 – 0.4 0.41 – 0.6 0.61 – 0.8 0.81 - 1 | Poor Slight Fair Moderate Substantial Almost perfect |

Source: Fleiss (1981)

3. results and discussion

3.1 UAV Image Acquisition Performance

This study's aerial photography acquisition process produced 1,521 photographs, with image quality and quantity influenced by technical parameters such as flight altitude, overlap and sidelap values, area coverage, and environmental conditions during flight. These factors directly affected the ground sampling distance (GSD), which determines the spatial resolution and precision of the resulting imagery. Variations in camera tilt angle, specifically between 90° (nadir) and 45° (oblique), significantly impacted acquisition duration, number of images captured, and the level of different benthic habitat details in Tables 2 and 3. Imaging performed at angles that are too low or too high results in uneven lighting intensity, thereby reducing visual quality, particularly in detecting small benthic objects (Roelfsema et al., 2020).

Other literature also shows that drone stability and image capture time must be designed precisely, especially for coastal areas with high reflectance, such as sandy expanses and shallow coral reefs. Studies by Joyce et al. (2018) and Casella et al. (2017) highlights the importance of carefully planned flight missions that consider flight altitude, sun angle, and camera configuration to avoid acquisition failure in tropical marine environments. Therefore, to produce high-quality, reliable spatial data for benthic habitat mapping, the timing of acquisition, camera angle, and control over atmospheric conditions are critical technical steps determining the success of data acquisition.

Table 2.		Acquisition results (lens angle 90°)

| Components | Acquisition Times (AM local time) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 08:00 | 10:00 | 12:00 | 14:00 |
| Overlap | 80% |  |  |  |
| Sidelap | 80% |  |  |  |
| Flight height | 100 m |  |  |  |
| Flight Time (hours:minute:second) | 0:12:55 | 0:14:45 | 0:16:15 | 0:16:48 |
| Number of photos | 163 | 196 | 264 | 281 |
| GSD (cm/pixel) | 3.35 | 2.30 | 2.35 | 2.38 |

Table 3.	Acquisition Results (Lens Angle 45°)

| Components | Acquisition Times (AM local time) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 08:00 | 10:00 | 12:00 | 14:00 |
| Overlap | 80% |  |  |  |
| Sidelap | 80% |  |  |  |
| Flight height | 100 m |  |  |  |
| Flight Time (hours:minute:second) | 0:15:27 | 0:14:25 | 0:12:30 | 0:16:29 |
| Number of photos | 223 | 183 | 156 | 275 |
| GSD (cm/pixel) | 3.29 | 2.64 | 3.12 | 2.85 |

3.2 Orthophoto

The drone orthomosaic generated using Agisoft Metashape is presented in Fig. 2, illustrating significant visual differences across acquisition times and camera tilt angles. Specifically, variations between vertical (90° nadir) and oblique (45°) lens orientations influenced the spatial representation of features, scene coverage, and the degree of geometric distortion observed in the mosaics (James & Robson, 2012). Notably, orthophotos captured at 08:00 using a 90° angle were visually compromised by sun glint, solar reflections on the water surface, which caused image misalignments and inconsistencies in the final mosaic (Doukari et al., 2021).

The presence of sun glint is known to produce inaccurate reflectance values and hinder the image classification process and visual interpretation of benthic habitats (Chong et al., 2021). Conversely, oblique imaging at 45° tends to produce varying GSD, where the front part of the image has smaller pixel sizes than the back part, which can affect spatial homogeneity in orthophotography results (Hohle, 2008). Furthermore, according to Turner et al. (2012), the quality of orthophotography is highly dependent on the accuracy of camera modeling and tie point density in the SfM process, which, if not adequately controlled, can cause local deformation in the mosaic. Therefore, the selection of acquisition time, camera angle, and field data support is crucial in producing accurate and representative orthophotos for spatial analysis of benthic habitats.

Fig. 2. Orthophoto results from drone acquisition at a 90° and 45° lens angle at 08:00; 10:00; 12:00; and 14:00 AM local time

3.3 Benthic Habitat Classification Scheme

Based on field observations at 189 sampling points, six dominant shallow-water benthic habitat classes were identified in the study area: algae (A), live coral (LC), dead coral (DC), sand (S), seagrass (SG), and rubble (R). This classification scheme was established by referring to the most dominant habitat components in each square transect unit, which were identified visually during the field survey. The frequency of occurrence of each habitat class was as follows: algae (30 points), living coral (42 points), dead coral (35 points), sand (24 points), seagrass (39 points), and rubble (22 points). Of the total observation points, 94 points were used as validation data for testing the accuracy of the classification results, while 95 points were used as thematic layer data in the object-based classification process using eCognition software.

This process aligns with the benthic habitat classification approach that combines remote sensing data with field verification to improve the thematic accuracy of habitat maps (Roelfsema et al., 2020). According to Blaschke (2010), object-based classification supported by thematic layer data can improve accuracy and reliability in mapping complex habitats such as coral reef systems and seagrass beds. In addition, field test point-based validation is an integral part of avoiding classification bias and ensuring thematic consistency between imagery and ecological reality (Congalton & Green, 2019).

3.4 Image Segmentation and Drone-Based Classification

Image segmentation in this study used the MRS method. This approach breaks the image into objects by looking at how similar the colors and shapes are, based on scale, compactness, and shape. It tends to work better when the sea surface is calm and there are strong color contrasts; those conditions make it easier to separate benthic features. Segmentation can struggle when the water is choppy or the image colors are too similar (Doukari & Topouzelis, 2022). Furthermore, Wahidin et al. (2015) also note that the segmentation scale plays a significant role in shaping the form, size, and number of identified objects. If the segmentation value is too small, oversegmentation can occur, whereby objects that should be one are classified as several segments. Conversely, a segmentation scale that is too large causes undersegmentation, whereby several different objects are combined into one segment. The classification results show significant spatial variation in habitat class distribution between recording angles of 90° and 45° and different acquisition times (Fig. 3).

Fig. 3. Benthic habitat classification using 90° and 45° lens angle SVM algorithm

Based on the results of benthic habitat classification using the SVM algorithm, it was found that differences in the angle of the aerial photography lens and the time of acquisition significantly affected the results of habitat cover identification, both in terms of area and spatial distribution. The classification results at 10:00 AM local time, with lens angles of 90° and 45° and a segmentation scale of 50 showed significant differences in the estimated area of each benthic habitat class, as shown in Tables 4 and 5. These variations are primarily attributed to differences in light refraction angles, which are influenced by the sun’s position and atmospheric conditions at the time of image capture, ultimately affecting the visibility of the seafloor (Roelfsema et al., 2020). The amount of light that reaches the camera sensor also plays a key role, as it impacts the quality of reflectance data and how well different features can be distinguished during spatial classification (Joyce et al., 2018). As noted by Eugenio et al. (2017), factors such as sunlight intensity and water clarity significantly affect the accuracy of mapping underwater habitats using drone-based imagery. Therefore, the planning of acquisition timing, lens angle, and segmentation parameter adjustments must be carefully designed to ensure that image classification can accurately and informatively represent benthic habitat conditions.

Table 4.	The area of ​​benthic habitat at 10:00 AM local time at a drone lens angle of 90° using the SVM algorithm with a segmentation scale of 50

| Benthic Class | Benthic Habitat Area (m2) |
| --- | --- |
| A LC DC SG S R | 270.32 1,112.56 727.58 4,377.66 1,985.01 450.31 |

Table 5.	The area of ​​benthic habitat at 10:00 AM local time at a drone lens angle of 45° using the SVM algorithm with a segmentation scale of 50

| Benthic Class | Benthic Habitat Area (m2) |
| --- | --- |
| A LC DC SG S R | 385.12 762.11 1,813.51 6,217.98 910.25 228.14 |

Based on Table 4, the results of benthic habitat classification using the SVM algorithm at the time of recording at 10:00 AM local time. with a drone lens angle of 90° and a segmentation scale of 50, the three dominant habitat classes are seagrass (SG) covering an area of 4,377.66 m2, sand (S) covering 1,985.01 m², and live coral (LC) covering 1,112.56 m². Conversely, the classes with the smallest coverage are rubble (R) and algae (A), covering only 450.31 m² and 270.32 m², respectively. Different results were found in the image acquisition with a drone lens angle of 45° (Table 5), where the dead coral (DC) class dominated with an area of 1,813.51 m², seagrass (SG) with an area of 6,217.98 m², and sand (S) with an area of 910.25 m². In contrast, the smallest areas were found in the algae (A) and sand (S) classes. The differences in spatial distribution and area of each habitat class reflect the significant influence of the drone lens angle and segmentation parameters on the classification results. According to Blaschke (2010) and Peña et al. (2014), segmentation parameters and lens angle changes can affect the object extraction process and the overall habitat categorization results. Additionally, variations in object appearance in images with oblique and perpendicular angles also affect the spectral and spatial characteristics recorded, thereby impacting the performance of classification algorithms (Roelfsema et al., 2020; Eugenio et al., 2017). As explained by Peña et al. (2014), the success of object-based classification in marine ecosystems is greatly influenced by the suitability between the segmentation scale and habitat heterogeneity, so the selection of segmentation parameters must be adjusted to the ecological characteristics of the study area and supported by adequate field data.

3.5 Accuracy Assessment

The validation or accuracy testing process is critical in determining the optimal segmentation parameters to make the classification results more representative and accurate (Blaschke, 2010). In addition, the accuracy of the classification results is also influenced by the number of classes, the number of field data samples, sampling techniques such as square transect size, and errors in data collection and interpretation of benthic habitat cover. Benthic habitat classification using the SVM algorithm with six habitat classes showed significant variations in accuracy levels based on the angle of image capture, acquisition time, and segmentation scale used. The results show that at a drone lens angle of 90°, the highest accuracy was obtained at 10:00 AM local time with a segmentation scale of 50 and an Overall Accuracy (OA) of 91.75%. Conversely, the lowest accuracy at the same angle occurred at 8:00 AM local time with an OA of 73.19% at a segmentation scale of 50. At a recording angle of 45°, the highest OA value was obtained at 14:00 AM local time with a segmentation scale of 50, 76.28%, while the lowest value of 67.01% was obtained at 12:00 AM local time, as shown in Fig. 4. These results support the findings of Green et al. (2000) that OA values above 60% are considered to meet the ideal standard for mapping benthic habitats in shallow waters.

Significant improvements in accuracy at specific times and configurations reflect the algorithm's sensitivity to lighting quality, substrate visibility, and acquisition condition stability. Mastu et al. (2018) noted that SVM provides high accuracy in drone-based benthic habitat mapping with an OA value of 81.1% in classifying linear and non-linear data through transformation to high-dimensional space and using hyperplanes as class separators. A similar study by Nababan et al. (2021) also reported accuracies of 81.1% and 77.4% in classifying 12 and 9 benthic habitat classes using drone imagery, while Wahidin et al. (2015) and Sugara et al. (2017) reported accuracies of 73% and 78%, respectively, on Landsat 8 Operational Land Imager (OLI) and WorldView-2 imagery. This study is consistent with other literature indicating that classification success is highly dependent on the combination of appropriate algorithms, suitable segmentation parameters, and the quality of field reference data (Congalton & Green, 2019; Roelfsema et al., 2020), making it essential to design a comprehensive accuracy testing scheme for benthic habitat mapping in tropical coastal areas.

Fig. 4. Accuracy evaluation of drone camera angles (90° and 45°) across acquisition times

The kappa coefficient is a statistical metric used to assess how closely classification results match reference data, considering agreement that’s not due to chance. It is an important indicator for evaluating the quality of benthic habitat map classification. Based on the classification results using the SVM algorithm at the time of acquisition at 10:00 and a drone lens angle of 90°, the highest Overall Accuracy (OA) was obtained at a segmentation scale of 50, which was 91.75%. Table 6 shows that the Kappa coefficient obtained is 0.89. According to Fleiss’s (1981) classification of Kappa indices, this value falls into the “almost perfect” category (0.81–1), indicating that the classification results using SVM have a high level of reliability.

This study aligns with Congalton & Green (2019), who state that a kappa value close to 1 indicates a high degree of classification agreement with actual data. In the context of benthic habitat mapping, evaluation using the kappa coefficient is crucial as it provides an objective assessment of the consistency of complex multi-class classifications, such as tropical shallow-water habitats with high heterogeneity (Roelfsema et al., 2020). A study by Huanfeng et al. (2020) also showed that the use of the SVM algorithm on underwater habitats produced high kappa values compared to other classification methods, thereby reinforcing the relevance of using SVM in high-resolution image-based mapping.

Table 6. Kappa coefficient results using the SVM algorithm

| Drone Flight Times (Local Time) | Drone Camera Angles | Kappa Coefficient |
| --- | --- | --- |
| 08:00 | 90° | 0.67 |
|  | 45° | 0.68 |
| 10:00 | 90° | 0.89 |
|  | 45° | 0.66 |
| 12:00 | 90° | 0.71 |
|  | 45° | 0.59 |
| 14:00 | 90° | 0.82 |
|  | 45° | 0.70 |

4. Conclusion

The mapping of shallow-water benthic habitats was effectively conducted using an object-based image classification approach (Object-Based Image Analysis/OBIA) with the Support Vector Machine (SVM) algorithm applied to high-resolution drone imagery. The optimal acquisition time was identified as 10:00 AM local time, with a nadir (90°) camera angle and a segmentation scale of 50, achieving an Overall Accuracy (OA) of 91.75% and a Kappa coefficient of 0.89, indicating a classification result within the “almost perfect” category. This study confirms that the appropriate combination of image acquisition timing, drone camera angle, and a robust classification algorithm significantly enhances the accuracy of benthic habitat mapping.

DISCLAIMER (ARTIFICIAL INTELLIGENCE)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc) and text-to-image generators have been used during writing or editing of this manuscript.

References

ArcGIS Development Team. (2022). ArchGIS Geographic Information System. Open Source Geospatial Foundation Project. https://www.arcgis.com/index.html

Baatz, M., & Schäpe, A. (2000). Multiresolution segmentation: an optimization approach for high quality multi-scale image segmentation. Proceedings of Angewandte Geographische Informationsverarbeitung XII, 12-23.

Badan Standardisasi Nasional. (2011). Pemetaan habitat perairan laut dangkal (Bagian 1): Pemetaan terumbu karang dan padang lamun. Jakarta: Badan Standardisasi Nasional

Benfield, S. L., Guzman, H. M., Mair, J. M., & Young, J. A. T. (2007). Mapping the distribution of coral reefs and associated sublittoral habitats in Pacific Panama: a comparison of optical satellite sensors and classification methodologies. International Journal of Remote Sensing, 28(22), 5047-5070. https://doi.org/10.1080/01431160701258062

Blaschke, T. (2010). Object based image analysis for remote sensing. ISPRS Journal of Photogrammetry and Remote Sensing, 65(1), 2-16. https://doi.org/10.1016/j.isprsjprs.2009.06.004

Casella, E., Collin, A., Harris, D., Ferse, S. C. A., Bejarano, S., Parravicini, V., & Rovere, A. (2017). Mapping coral reefs using consumer-grade drones and structure from motion photogrammetry techniques. Coral Reefs, 36(1), 269-275. https://doi.org/10.1007/s00338-016-1522-0

Chong, W. S., Zaki, N. H. M., Hossain, M. S., Muslim, A. M., & Pour, A. B. (2021). Introducing Theil-Sen estimator for sun glint correction of UAV data for coral mapping. Geocarto International, 37(15), 4527-4556. https://doi.org/10.1080/10106049.2021.1892206

Congalton, R. G., & Green, K. (2019). Assessing the accuracy of remotely sensed data: principles and practices (3rd ed.). Boca Raton: CRC Press. https://doi.org/10.1201/9780429052729

Cortes, C., & Vapnik, V. (1995). Support-vector networks. Machine Learning, 20(3), 273-297. (https://doi.org/10.1007/BF00994018)

Doukari, M., Batsaris, M., Topouzelis, K. (2021). UASea: A Data Acquisition Toolbox for Improving Marine Habitat Mapping. Drones, 5, 73. https://doi.org/10.3390/drones5030073.

Doukari, M., & Topouzelis, K. (2022). Overcoming the UAS limitations in the coastal environment for accurate habitat mapping. Remote Sensing Applications: Society and Environment, 26, 1-13. https://doi.org/10.1016/j.rsase.2022.100726

Eugenio, F., Marcello, J., Martin, J., & Rodríguez-Esparragón, D. (2017). Benthic habitat mapping using multispectral high-resolution imagery: Evaluation of shallow water atmospheric correction techniques. Lensas, 17(11), 1-23. https://doi.org/10.3390/s17112639

Fleiss, J. L. (1981). Statistical methods for rates and proportions (2nd ed.). New York: John Wiley & Sons.

Green, E. P., Mumby, P. J., Edwards, A. J., & Clark, C. D. (2000). Remote sensing handbook for tropical coastal management. In A. J. Edwards (Ed.). Coastal Management Sourcebooks 3. Paris: UNESCO Publishing.

Hohle, J. (2008). Photogrammetric measurements in oblique aerial images. Photogrammetrie, Fernerkundung, Geoinformation, 1, 7-14.

Huanfeng, S., Jiang, Y., Li, T., Cheng, Q., Zeng, C. & Zhang, L. (2020). Deep learning-based air temperature mapping by fusing remote sensing, station, simulation and socioeconomic data, Remote Sensing of Environment, 240, 1-14. https://doi.org/10.1016/j.rse.2020.111692

James, M. R., & Robson, S. (2012). Straightforward reconstruction of 3D surfaces and topography with a camera: Accuracy and geoscience application. Journal of Geophysical Research: Earth Surface, 117(F3), 1-17. https://doi.org/10.1029/2011JF002289

Joyce, K. E., Duce, S., Leahy, S. M., Leon, J., & Maier, S. W. (2018). Principles and practice of acquiring drone-based image data in marine environments. Marine and Freshwater Research, 70(7), 952-963. https://doi.org/10.1071/MF17380

Kementerian Kelautan dan Perikanan. (2016). Peraturan Menteri Kelautan dan Perikanan Nomor 14 Tahun 2016 tentang Kriteria dan Kategori Kawasan Konservasi Perairan untuk Wisata Alam Perairan. Jakarta: Kementerian Kelautan dan Perikanan

Kementerian Kelautan dan Perikanan. (2020). Peraturan Menteri Kelautan dan Perikanan Nomor 31 Tahun 2020 tentang Pengelolaan Kawasan Konservasi. Jakarta: Kementerian Kelautan dan Perikanan

Kementerian Kelautan dan Perikanan. (2023). Keputusan Menteri Kelautan dan Perikanan Nomor 127 Tahun 2023 tentang Kawasan Konservasi di Perairan di Wilayah Teluk Gorontalo Provinsi Gorontalo. Jakarta: Kementerian Kelautan dan Perikanan RI.

Klemas, V. V. (2015). Coastal and environmental remote sensing from unmanned aerial vehicles: An overview. Journal of Coastal Research, 31(5), 1260–1267. (https://doi.org/10.2112/JCOASTRES-D-15-00005.1)

Lillesand, T. M., Kiefer, R. W., & Chipman, J. W. (2015). Remote sensing and image interpretation (7th ed.). New York: John Wiley & Sons.

Longley, P. A., Goodchild, M. F., Maguire, D. J., & Rhind, D. W. (2015). Geographic information systems and science (4th ed.). New York: John Wiley & Sons.

Mastu, L. O. K. (2019). Pemetaan habitat bentik berbasis objek menggunakan citra ubmanned aerial vehicle (UAV) dan satelit Sentinel-2 di perairan Pulau Wangi-Wangi Kabupaten Wakatobi [Tesis]. IPB University, Bogor.

Mumby, P. J., & Harborne, A. R. (1999). Development of a systematic classification scheme of marine habitats to facilitate regional management and mapping of Caribbean coral reefs. Biological Conservation, 88(2), 155-163. https://doi.org/10.1016/S0006-3207(98)00108-6

Nababan, B., Mastu, L. O. K., Idris, N. H., & Panjaitan, J. P. (2021). Shallow-water benthic habitat mapping using drone with object based image analyses. Remote Sensing, 13(21), 1-23. https://doi.org/10.3390/rs13214452

Nugroho, A., Nababan, B., Panjaitan, J. P., & Agus, S. B. (2024). Object-based benthic habitat mapping using drones in the waters of Gili Labak Island, Sumenep. Jurnal Kelautan, 17(1), 29-42. https://doi.org/10.21107/jk.v17i1.24518

Øie, K., Gundersen, H., Nøddebo, R., Edward, J., Salberg, A., Esam, M., Buls, T., Bekkby, T., & Hancke, K. (2024). MethodsX drone and ground-truth data collection, image annotation and machine learning : A protocol for coastal habitat mapping and classification. MethodsX, 13, 1-19. https://doi.org/10.1016/j.mex.2024.102935

Pemerintah Provinsi Gorontalo. (2024). Peraturan Daerah Provinsi Gorontalo tentang Rencana Tata Ruang Wilayah Provinsi Gorontalo Tahun 2024-2043. Gorontalo: Pemerintah Provinsi Gorontalo.

Peña, J. M., Gutiérrez, P. A., Torres-Sánchez, J., Hervás-Martínez, C., & López-Granados, F. (2014). Object-based image classification of agricultural systems using machine learning algorithms. Remote Sensing, 6(6), 5019-5041. https://doi.org/10.3390/rs6065019

Purkis, S. J., Gleason, A. C. R., Purkis, C. R., Dempsey, A. C., Renaud, P. G., Faisal, M., Saul, S., & Kerr, J. M. (2019). High-resolution habitat and bathymetry maps for 65,000 sq. km of Earth’s remotest coral reefs. Coral Reefs, 38(3), 467-488. https://doi.org/10.1007/s00338-019-01802-y

Prayuda, B. (2014). Panduan teknis pemetaan habitat dasar perairan laut dangkal. pemetaan habitat dasar perairan laut dangkal. Jakarta: COREMAP CTI LIPI. https://www.scribd.com/document/332370130/p2olipi-pdf

Qin, J., Li, M., Zhao, J., Li, D., Zhang, H., & Zhong, J. (2024). Advancing sun glint correction in high-resolution marine UAV RGB imagery for coral reef monitoring. ISPRS Journal of Photogrammetry and Remote Sensing, 207, 298-311. https://doi.org/10.1016/j.isprsjprs.2023.12:007

Roelfsema, C. M., Kovacs, E. M., Ortiz, J. C., Callaghan, D. P., Hock, K., Mongin, M., Johansen, K., Mumby, P. J., Wettle, M., Ronan, M., Lundgren, P., Kennedy, E. V, & Phinn, S. R. (2020). Habitat maps to enhance monitoring and management of the Great Barrier Reef. Coral Reefs, 39(4), 1039-1054. https://doi.org/10.1007/s00338-020-01929-3

Siregar, V. P. (2010). The bottom substrate shallow water mapping using the Quick Bird satellite imagery. E-Jurnal Ilmu dan Teknologi Kelautan Tropis, 2(1), 19-30.

Sugara, A., Siregar, V. P., & Agus, S. B. (2020). Benthic habitat classification of shallow water using Worldview-2 Imagery with in-situ and drone data. Jurnal Ilmu dan Teknologi Kelautan Tropis, 12(1), 135-150. https://doi.org/10.29244/jitkt.v12i1.26448

Turner, D., Lucieer, A., & Watson, C. (2012). An automated technique for generating georectified mosaics from ultra-high resolution UAV imagery, based on Structure from Motion (SfM) point clouds. Remote Sensing, 4(5), 1392–1410. https://doi.org/10.3390/rs4051392

Wahidin, N., Siregar, V. P., Nababan, B., & Jaya, I. (2015). Object-based image analysis for coral reef benthic habitat mapping with several classification algorithms. Procedia Environmental Sciences, 24, 222–227. (https://doi.org/10.1016/j.proenv.2015.03.029)

Westoby, M. J., Brasington, J., Glasser, N. F., Hambrey, M. J., & Reynolds, J. M. (2012). ‘Structure-from-Motion’ photogrammetry: A low-cost, effective tool for geoscience applications. Geomorphology, 179, 300–314. https://doi.org/10.1016/j.geomorph.2012.08.021

- Wolf, P. R., & Dewitt, B. A. (2000). Elements of Photogrammetry with Applications in GIS (3rd ed.). McGraw-Hill.

Definitions, Acronyms, Abbreviations

| % | Percent |
| --- | --- |
| < | Less than |
| A | Algae |
| AM | Ante Meridiem |
| cm | Centimeter |
| DC | Dead coral |
| DSM | Digital surface model |
| et al. | Et alia |
| GPS | Global positioning system |
| GSD | Ground sampling distance |
| LC | Live coral |
| m | Meter |
| MPAs | Marine protected areas |
| MRS | Multi-resolution segmentation |
| OA | Overall accuracy |
| OBIA | Object-based image analysis |
| OLI | Operational Land Imager |
| PA | Producer accuracy |
| QGIS | Quantum Geographic Information System |
| R | Rubble |
| RAM | Random Access Memory |
| RGB | Red, green, blue |
| S | Sand |
| SfM | Structure from motion |
| SG | Seagrass |
| SVM | Support vector machine |
| UA | User accuracy |
| UAVs | Unmanned aerial vehicles |
| 3D | Three dimensions |