Original Research Article

# Approach to Urban Tropical Surface Runoff Using SCS-CN via Google Earth Engine: Application to the Gourou Watershed (Abidjan, Côte d’Ivoire)

# ABSTRACT

The Gourou watershed, located in the metropolitan region of Abidjan, is highly vulnerable to flooding due to rapid urbanization and the increasing frequency of extreme rainfall events. This study presents a detailed quantification of runoff processes through the application of the Soil Conservation Service Curve Number (SCS-CN) method, combined with a spatialized analysis integrating pedological characteristics (soil textures) and land use dynamics.

Our approach relies on an innovative processing chain implemented in Google Earth Engine, enabling high-resolution land use classification (overall accuracy: 99.58%; Kappa index: 0.97), spatial interpolation of soil data (based on 19 sampling points), computation of Curve Numbers (CN) through matrix cross- referencing of soil texture and land use classes, and hydrological modeling using the SCS equation.

The analysis reveals alarming hydrological indicators, including a significant reduction in effective infiltration, an average runoff rate of 42.6% (runoff coefficient = 0.42), a total runoff volume of 827,113 m³ for cumulative rainfall of 1,941,220 m³, and widespread imperviousness, with 77.8% of the watershed surface consisting of built-up areas or bare soils (CN > 78). These conditions contribute to the intensification of flood peaks due to the rapid concentration of surface flows.

Keywords: SCS-CN method, Runoff, Curve Number (CN), Urban flooding, Hydrological risk, Google Earth Engine (GEE)

# INTRODUCTION

Surface runoff is a complex process influenced by several factors, including precipitation intensity and duration, soil type, land cover, vegetation cover, topographic slope, and hydrographic network density (Ponce, 1996; Mishra, 2003). Within a watershed, this dynamic transform rainfall into concentrated flows, feeding streams and increasing flood risks—particularly in urban areas where soil impermeabilization exacerbates the phenomenon (Brooks, 2013). Extreme events, such as intense rainfall, can trigger sudden floods with catastrophic consequences, as observed in the Gourou watershed, where rainfall totals of 125 mm (June 21, 2022) and 80 mm (June 18, 2023) caused devastating flooding (Njeuguet, 2024).

Accurate estimation of surface runoff remains a major challenge, especially in ungauged watersheds, where the lack of hydrometric data limits classical modeling approaches. Among available methods, the Soil Conservation Service Curve Number (SCS-CN) method, developed by the USDA (1986), stands out for its simplicity and effectiveness. It relies on a limited number of parameters, primarily the Curve Number (CN), which integrates land cover, soil type, and

antecedent moisture conditions (Hawkins, 2009). Originally designed for rural watersheds, this method has been successfully adapted for urban and forested environments, offering valuable flexibility for hydrological risk assessment (Giridhar, 2014).

However, traditional application of the SCS-CN method requires laborious manual processing of geospatial data, which may introduce delays and uncertainties. The emergence of cloud-based platforms such as Google Earth Engine (GEE) has revolutionized this approach by enabling large-scale automated computations. Thanks to access to up-to-date satellite catalogs (e.g., Landsat, Sentinel) and distributed computing power, GEE facilitates rapid extraction of key parameters (land cover, vegetation index, slope) and dynamic runoff calculations.

This study proposes an innovative methodology for mapping surface runoff and identifying high- risk flood zones in the Gourou watershed by combining the SCS-CN method with the capabilities of Google Earth Engine.

# Study area

The Gourou watershed (Fig.1) is located in the Autonomous District of Abidjan (Côte d’Ivoire). It lies between latitudes 5°10' and 5°25' North and longitudes 3°59'10" and 4°2'30" West. This watershed covers an area of approximately 27.5 km² (BRL (2015), extending along a north-south axis about 9 km in length and 3 km in width from east to west. It is bounded to the east by the Boulevard Latrille, to the west by the Adjamé– Anyama  railway  line,  to  the  north  by  the

municipality of Abobo, and to the south by the Bay of Cocody and the Plateau municipality (Coulibaly et al, 2011). Its main drainage canal corresponds to the former bed of the “Gougou” river, as named in the Atchan (Ébrié) language. All surface water within the watershed converges toward its outlet located at the Indénié junction, at the intersection of the municipalities of Plateau, Cocody, and Adjamé, before flowing into the Ébrié Lagoon.

# Materials and Methods

  - Materials

## Data Used

Fig 1. Gourou watershed

The data used in this study (Table 1) include rainfall data, remote sensing imagery, and pedological (soil) data.

Table 1. Types of data used in the study

Data type and format	Source / Sensor	Key characteristics	Use in the study

Rainfall data (Raster - GeoTIFF)

Satellite	image (Raster - GeoTIFF)

Pedological	data (CSV)

Climatic data (2022) https://www.chc.ucsb. edu/data/chirps

Landsat 8 – USGS EarthExplorer http://earthexplorer.us gs.gov

Field campaign (19 sampling points)

- Wettest month: June (avg. 266 mm)

-	June	2022:	424	mm

- Selected event: June 16–17, 2022

(73.05 mm)

- Spatial	resolution:	30	m

- Date:	January	20,	2022

- Bands: B4 (0.636–0.673 µm), B5 (0.851–0.879 µm), B6 (1.566–1.651

µm)

- Soil texture analyzed in laboratory

- Sites covering various land cover types and geomorphological units

Selection of extreme event for simulating runoff using the SCS- CN method

Land cover mapping (LCM) used to assign CN values based on soil texture

Classification of soil hydrologic groups for CN calculation

DEM	(Raster	- GeoTIFF)

ALOS	PALSAR

https://search.asf.alas ka.edu

Spatial	resolution:

- Elevation model derived from radar interferometry

Determination of slope and flow direction for assessing runoff coefficient and topographic factors

12.5 m

# Methods

The methodology adopted in this study combines the SCS-CN model with Google Earth Engine’s (GEE) spatial analysis tools to estimate surface runoff at the watershed scale.

# Soil Classification and Hydrologic Group Assignment

Estimation of the runoff coefficient (Cr) in the SCS- CN method critically depends on soil physical properties, particularly texture, which strongly influences infiltration capacity (Shadeed, 2010). To this end, a soil sampling campaign was conducted within the Gourou watershed, targeting

19 georeferenced sites selected for their representativeness in terms of land cover and geomorphological variability (Fig.2). Soil samples were collected using a manual auger at depths ranging from 30 to 100 cm. Each sample, packaged in quantities of approximately 500 g, was transported to the laboratory of the Centre de Recherche en Environnement (CRE) at Nangui Abrogoua University. Granulometric analyses were performed in accordance with the AFNOR NF X31-107.500 standard to determine the respective percentages of sand, silt, and clay.

Fig 2. Selected soil sampling points

These granulometric data were then used to determine the textural class of each sampling point based on the USDA soil texture triangle. A spatial interpolation was performed using the Inverse Distance Weighting (IDW) method within Google Earth Engine to generate a continuous map of soil texture across the entire watershed.

Based on the identified textural classes, each soil

type was assigned to a hydrologic group (A, B, C, or D) in accordance with USDA (1986) classification guidelines. This grouping is based on estimated soil infiltration rates, defined as follows:

- Group A (Code 1): Highly permeable soils with high infiltration rates (> 25 mm/h); typically, sandy soils.

- Group B (Code 2): Moderately permeable soils with moderate infiltration rates (12.5–25 mm/h); typically, loamy sand or sandy loam.

- Group C (Code 3): Soils with low infiltration rates (2.5–12.5 mm/h); typically, sandy clay loam or silty clay loam.

- Group D (Codes 3 or 4): Soils with very low infiltration rates (< 2.5 mm/h); typically, clay, silty clay, or clay loam.

This hydrologic soil classification serves as a critical input for assigning Curve Number (CN) values in the runoff modeling process.

# 3.3. Land Cover Mapping Using Landsat 8

Land cover is a key parameter in determining the Curve Number (CN). To produce a recent and accurate land cover map, a supervised classification was performed using a cloud-free Landsat 8 OLI/TIRS image from the dry season of 2022, with a spatial resolution of 30 m. The processing was carried out in ENVI 5.3 software, following several preprocessing steps: radiometric and atmospheric corrections, selection of relevant spectral bands (Bands 2 to 7 of Landsat 8), and definition of land cover classes based on ground truth data collection. Four main classes were identified: Class 1: Built-up areas / bare soil, Class 2: Dense Forest, Class 3: Degraded Forest, Class 4:

Wetlands / croplands. The Maximum Likelihood Classification (MLC) algorithm was used for image classification. After classification, the resulting land cover map was imported into Google Earth Engine for spatial integration and assignment of CN values based on standard USDA (1986) guidelines (Table 2).

Table 2: Assignment of CN values based on land cover (USDA, 1986)

Code	Land Cover	CN

- Built-up areas / bare soil	92

- Dense forest	55

- Degraded forest	65

4	Wetlands / croplands	75

# 3.4. Presentation of the SCS-CN Method

The SCS-CN (Soil Conservation Service – Curve Number) method, developed by the United States Department of Agriculture (USDA), is one of the

Q = (P−Ia)2

(P− Ia)+S

where:

(Eq 1)

most widely used approaches for estimating direct surface runoff generated by a rainfall event, expressed as depth (mm). It is based on an empirical relationship between total precipitation

(P) and runoff volume (Q), taking into account the physical characteristics of the watershed: land cover, soil texture, antecedent moisture conditions (AMC), and, indirectly, topographic slope.

The fundamental equation of the SCS-CN method is:

- Q: runoff (mm)

- P: total precipitation (mm)

- S: potential maximum retention (mm)

- Ia: initial abstraction (mm)

The initial abstraction (Ia) represents all losses occurring before runoff begins, including vegetation interception, depression storage, initial evaporation, and early infiltration (Mishra, 2003). In practice, Ia is commonly estimated as 20% of

the maximum retention S:

𝐼𝑎 = 0.2 * S (Eq 2)

Substituting this relationship into Equation (6) yields the simplified form widely used in hydrological applications:

(P−Ia)2

permeability, ranging from highly permeable sandy soils to compacted clay soils; and the antecedent moisture condition (AMC), which represents the initial soil moisture (AMC I: dry, AMC II: average, AMC III: saturated). In this study, the “Poor” condition corresponds to AMC III, while the “Good” condition corresponds to AMC II.

Q =

(P+0.8*S)

(Eq 3)

Although the SCS-CN method does not explicitly

The value of S is directly related to the Curve Number (CN), an empirical parameter ranging from 0 to 100, according to the formula:

S = 25400 − 254 (Eq 4)

CN

The Curve Number (CN) provides an integrated measure of the hydrological properties of a watershed. Its value increases with runoff potential: a high CN (close to 100) indicates an impermeable, highly urbanized, or saturated soil that generates significant runoff, whereas a low CN (close to 0) corresponds to a highly permeable soil or one with dense vegetation cover, thereby favoring infiltration.

Three main factors determine the CN: land use/land cover, which governs vegetation cover and surface imperviousness; the hydrologic soil group (A to D), which reflects soil texture and

incorporate slope, topography plays a critical role in runoff generation by shortening infiltration time and accelerating overland flow. For this reason, CN values were adjusted according to three slope classes: < 3% (flat, promoting infiltration), 3–8% (moderate), and > 8% (steep slopes, considerably enhancing runoff). Incorporating slope into the analysis improves the spatial accuracy of runoff estimation, particularly in heterogeneous catchments such as the Gourou basin, where topography exerts a strong influence on water dynamics.

Table 3 presents the Curve Number (CN) values for the different combinations of soil type, aggregated land use (e.g., cultivated land and fallows grouped as agricultural areas; built-up zones, paved roads, and bare soils grouped as artificial surfaces), slope, and hydrologic condition.

Table 3. Curve Number (CN) for different combinations of soil, land use, slope, and hydrologic conditions (provided in USDA–NRCS Technical Release 55 (TR-55))

Soil class (texture)

| Land use | Slope (%) | Hydrologic condition | A | B | C | D |
| --- | --- | --- | --- | --- | --- | --- |
|  | <3% | Poor Good | 63 60 | 74 70 | 81 78 | 82 81 |
| Agricultural areas | 3–8% | Poor Good | 65 63 | 76 75 | 84 83 | 88 87 |
|  | >8% | Poor Good | 72 67 | 81 78 | 88 85 | 91 89 |

|  | <3% | Poor Good | 25 22 | 55 53 | 70 65 | 77 74 |
| --- | --- | --- | --- | --- | --- | --- |
| Dense vegetation | 3–8% | Poor Good | 41 25 | 63 55 | 75 70 | 81 77 |
|  | >8% | Poor Good | 47 41 | 68 63 | 80 75 | 84 81 |
|  | <3% | Poor Good | 73 59 | 83 74 | 88 82 | 90 86 |
| Artificial surfaces | 3 à 8 % | Poor Good | 73 59 | 83 70 | 88 75 | 90 81 |
|  | >8% | Poor Good | 73 59 | 81 70 | 88 81 | 91 82 |

# Estimation of Average Rainfall and Associated Hydrological Volumes

To assess the hydrological response of the Gourou watershed, a quantitative analysis of rainfall and surface runoff was conducted using remote sensing data processed on the Google Earth Engine (GEE) platform. The primary objective was to estimate the total volume of rainfall and runoff, followed by the calculation of the runoff coefficient, a key indicator of the watershed’s water balance dynamics. The methodology involved several steps, each grounded in fundamental hydrological principles and spatially explicit data processing, as follows:

# Calculation of Total Rainfall Volume (Vp)

The total rainfall volume over the watershed was computed using the following equation:

𝑉𝑃 = 𝑃 × 𝐴 (Eq 5)

𝑉𝑃 is the total rainfall volume (m³),

P is the rainfall depth (mm), derived from satellite- based precipitation datasets (CHIRPS),

A is the watershed area (m²), obtained from watershed delineation.

Since rainfall depth is generally expressed in millimeters, it was converted into meters (by dividing by 1,000) to ensure unit consistency when multiplied by the watershed area, yielding a volume in cubic meters (m³).

# Calculation of Runoff Volume per Pixel (V_Q)

Runoff depth (QQQ) was estimated using the Soil Conservation Service Curve Number (SCS-CN) method, applied pixel by pixel across the watershed. The runoff volume generated in each pixel was then calculated as follows:

where:

𝑉Q

= Q×𝑆 (Eq 6)

1000

where:

𝑉𝑸 is the runoff volume (m³),

Q is runoff depth (mm),

S is the area of each pixel (m²), defined by the spatial resolution of the input imagery (30 m × 30 m for Landsat data).

This measure provides insights into the efficiency of water retention versus runoff generation in the watershed, reflecting land cover, soil type, topography, and antecedent moisture conditions.

Runoff Index (𝐼𝑸/𝑃)

The runoff index, defined as the ratio between the total runoff volume and the total rainfall volume, was calculated as:

Division by 1,000 converts millimeters to meters, ensuring dimensional consistency.

𝐼Q/𝑃

= 𝑉𝘘 (Eq 9)

𝑉𝘗

# Calculation of Total Runoff Volume (V_Qtotal, m³)

The total runoff volume over the entire watershed was obtained by summing the individual pixel contributions:

This dimensionless parameter serves as a simplified measure of watershed hydrological response and is commonly used in comparative studies across different climatic and geomorphological contexts.

𝑉Q𝑡𝑜𝑡𝑎𝑙

= ∑ 𝑉Q

(Eq 7)

# Overview of Google Earth Engine

Google Earth Engine (GEE) is a powerful cloud-based

This summation was efficiently implemented using GEE’s pixel-wise reduction functions, enabling rapid computation across large spatial domains.

# Runoff Percentage (%Q)

The proportion of rainfall contributing to surface runoff was determined as:

%Q = (𝑉𝘘) × 100 (Eq 8)

𝑉𝘗

geospatial platform developed by Google, designed for the processing, visualization, and analysis of large- scale spatial datasets. It provides access to an extensive catalog of satellite, climate, and environmental data free of charge. In the context of watershed runoff assessment, GEE offers several advantages, including direct access to spatial datasets (satellite imagery, rainfall, elevation, land use, etc.), automated large- scale and long-term calculations, and rapid visualization of results through interactive maps.

# Results and discussion

  - Results

## Estimation of Mean Precipitation

The climatological analysis based on CHIRPS (Climate Hazards Group InfraRed Precipitation with Station data) for the year 2022 reveals an exceptional annual rainfall, with a total accumulation of 1743.55 mm, corresponding to a monthly average of 145.30

mm. This trend confirms the highly rainy character of the year 2022 in the Abidjan region. June stands out as the wettest period, with a monthly total of 498.22 mm (Fig.3), which corresponds to a daily average precipitation of 14.12 mm.

This rainfall peak largely exceeds the values observed during the other months of the year, highlighting a strong seasonal variability. More specifically, the period from June 16 to 17, 2022, was marked by an intense rainfall event, with a maximum daily value of

73.05 mm.

This extreme episode caused major flooding in several districts of Abidjan, notably Abobo, Adjamé, and Cocody, thereby illustrating the increased vulnerability of the Gourou watershed to extreme precipitation events.

Variation of precipitation in 2022

600

500

400

300

200

100

0

MOIS

Série1

Fig 3. Variation of precipitation in 2022

precipitation

Soil Classification (Texture)

The results presented in Fig. 4 reveal a marked textural diversity, with a predominance of sandy and sandy loam soils, followed by sandy clay soils. Indeed, sandy and sandy loam textures alone account for 42.1% of the watershed area, and are mainly distributed in the central-eastern part of the Gourou basin. These soils are generally well- drained, promoting infiltration and limiting surface runoff.

In contrast, clayey textures (clay, sandy clay, and silty clay), which are concentrated in the northwestern and southern sections of the basin, cover 36.85% of the watershed. These soils tend to retain more water, thereby generating higher runoff during intense rainfall events. Finally, silty textures, mostly located in the central zone, represent approximately 15.79% of the Gourou watershed.

## Land Use/Land Cover in 2022

Land use analysis for the year 2022 was conducted using a Landsat 8 image, classified with a remarkably high level of accuracy (Overall

Fig 4. Soil texture map of the Gourou watershed

Accuracy = 99.58%; Kappa Coefficient = 0.97), thereby confirming the reliability of the classification process. Fig.5 illustrates the spatial distribution of the different land use classes within the Gourou watershed.

The results highlight a clear predominance of artificial surfaces, with the Built-up/Bare soil class covering 20.88 km², corresponding to 77.79% of the total area. This high proportion reflects an advanced level of urbanization and deforestation, which considerably reduces the extent of natural surfaces capable of regulating hydrological flows. Degraded forest represents the second most extensive class, covering 5.25 km² (19.56%), thereby indicating significant anthropogenic pressure on forest ecosystems. This fragmentation of vegetative cover promotes erosion and reduces soil infiltration capacity.

By contrast, dense forest, a remnant of the original vegetation formations, accounts for only

0.48 km² (1.79%) of the basin area. Finally, wetlands and agricultural lands, which play a crucial role in hydrological regulation, are highly marginalized, with a surface area of only 0.23 km² (0.86%). The scarcity of these natural buffer zones exacerbates the risk of rapid runoff during flood events.

## Curve Number (CN) Values

The Curve Number (CN), a key parameter of the SCS-CN method (Soil Conservation Service Curve Number), is classically categorized into five hydrological groups reflecting soil permeability and land use/land cover characteristics (Fig.6).

Spatial analysis of CN distribution across the Gourou watershed reveals particularly concerning patterns. Approximately 77% of the watershed area exhibits CN values greater than 78, with 57% falling within the critical range of 83–88. These elevated values are primarily associated with road infrastructure (fully impervious surfaces), industrial zones (artificialized areas), and dense unplanned settlements (compacted soils). Such zones generate direct runoff exceeding 70% of rainfall, thereby fostering rapid flood peaks and accelerating soil erosion processes.

Conversely, only 15% of the watershed area presents CN values ranging from 55 to 67, which are linked to forests, permanent grasslands (high infiltration capacity), and residual wetlands. These  natural  spaces,  although  crucial  for

Fig 5. Land use/land cover of the Gourou watershed in 2022

mitigating surface runoff and regulating hydrological dynamics, occupy a surface area far too limited to offset the widespread dominance of impervious land covers.

Fig 6. Curve Number (CN) map of the Gourou watershed

## Runoff Coefficient Mapping

The mapping of the runoff coefficient, derived using the SCS-CN method from an intense rainfall event of 73 mm that occurred on 16–17 June 2022, reveals a contrasting hydrological response within the Gourou watershed. Three runoff classes were identified (Fig.7): low runoff areas (4–13%), mainly corresponding to dense forest environments or well-drained sandy soils; moderate runoff areas (13–27%), associated with loamy soils or croplands with intermediate infiltration capacity, predominantly located in the south-eastern and central parts of the basin; and high runoff areas (27–44%), largely concentrated in urbanized sectors or clayey soils where surface sealing promotes rapid overland flow. The overall runoff index obtained is 0.42, corresponding to a runoff rate of 42.6%, which represents a runoff volume of 827,113.53 m³ out of a total rainfall volume of 1,941,220.80 m³. These results confirm the dominant influence of urbanization (77.8% of the surface occupied by settlements or bare soils) on the hydrological dynamics of the basin, as

reflected in reduced infiltration capacity, increased erosion processes, and the rapid concentration of surface flows that trigger frequent flood events.

Fig 7. Runoff coefficient map for P = 73 mm

# Discussion

The runoff study conducted in the Gourou watershed highlights a strong vulnerability to flooding, resulting from the combination of three major factors: increasing urbanization, marked pedological diversity, and high rainfall variability. The year 2022, which was particularly wet with an annual cumulative rainfall of 1743.55 mm, was dominated by a peak in June representing nearly 30% of the annual precipitation. The extreme event of June 16–17 (73.05 mm in 24 hours) triggered major floods, a phenomenon also observed in other West African cities exposed to intense tropical rainfall regimes (Yéro et al., 2019; Oloukoi et al., 2014).

From a pedological perspective, granulometric analyses reveal a dominance of sandy soils (42.1%), which are favorable to infiltration but highly sensitive to saturation during heavy rainfall, and clayey or loamy textures (52.64%), with low permeability that accentuate runoff, particularly in the central-eastern and north-eastern parts of the

watershed. These findings are consistent with those reported in a study of the Akouédo River basin, where compact clay soils were strongly associated with high runoff coefficients (Ouattara et al., 2017). In parallel, land-use mapping highlights pronounced artificialization: 77.79% of the surface area is impervious (settlements/bare soil), while dense forests and wetlands, which are essential for hydrological regulation, account for only 1.79% and 0.86% of the basin, respectively. This profile is similar to that observed in urban watersheds of Central and West Africa, particularly in Yaoundé, Korhogo, and Abomey- Calavi (Kouadio et al., 2021).

The application of the SCS-CN method reveals that 77% of the basin exhibits Curve Number (CN) values greater than 78, indicating a high propensity for runoff. The calculated runoff rate reaches 42.6%, representing a volume of more than 827,000 m³ for 73 mm of rainfall, thereby confirming the hydrological impact of imperviousness. These results are comparable to

those obtained by Koffi et al. (2016) in the Marahoué basin, where runoff rates of 35 to 45% were observed under degraded forest cover.

# Conclusion

This study analyzed the dynamics of runoff in the Gourou watershed (Abidjan) using an integrated approach combining soil texture, land use, and the SCS-CN method. The results highlight a marked spatial variability of the runoff coefficient (Cr), ranging from 4.13% to 44.35% for a rainfall event of 73 mm, representative of the region’s extreme events. The findings reveal that the predominance of urbanized areas and bare soils (77.79%), combined with a very limited residual forest cover (1.79%), leads to widespread impermeabilization, resulting in Curve Number (CN) values exceeding 78 over 77% of the watershed. This configuration promotes high runoff rates, surpassing 70% of total precipitation.

Furthermore, the spatial distribution of soil textures accentuates hydrological contrasts: sandy and sandy-loam soils (42.1%), concentrated in the central-western sector, tend to limit runoff (Cr < 13%), whereas clay soils (36.85%), predominant in the northwest, south, and central-east, significantly increase surface flows. This spatial heterogeneity contributes to the polarization of flood-prone areas, particularly in densely urbanized northeastern sectors, where the combined effect of high population density and low soil permeability generates elevated coefficients (27–44%). These results, consistent with the flash floods observed in June 2022, confirm the structural vulnerability of the basin and underline the urgency of integrating land-use planning and sustainable management strategies to strengthen the hydrological resilience of the territory.

In this context, it is necessary to promote integrated stormwater management measures, such as the implementation of green infrastructure (infiltration swales, retention basins, green roofs), the restoration of wetlands, and the preservation of remaining forest patches, which play an essential buffering role in hydrological regulation. At the same time, urban planning

should incorporate non-buildable zones in the most vulnerable sectors and improve soil permeability through the use of permeable materials. Finally, awareness-raising and community engagement are crucial to foster local ownership of solutions and ensure the long-term sustainability of interventions.

References

Brooks, E. S. (2013). Urban hydrology: Impacts of impervious surfaces on stormwater runoff. Journal of Hydrology: Regional Studies, 1(1), 1–

15. https://doi.org/10.1016/j.ejrh.2013.08.001

Funk, C. C., Peterson, P. J., Landsfeld, M. F.,

Pedreros, D. H., Verdin, J. P., Rowland, J. D., ... & Husak, G. J. (2015). The climate hazards infrared precipitation with stations—a new environmental record for monitoring extremes. Scientific	Data,	2,	150066.

https://doi.org/10.1038/sdata.2015.66

Giridhar, R. (2014). Application of the SCS-CN method for urban catchments: A review. Journal of Hydrologic Engineering, 19(4), 747–758. https://doi.org/10.1061/(ASCE)HE.1943- 5584.0000863

Hawkins, R. H. (2009). The SCS curve number method: How it began and what happened since. In Proceedings of the 2009 National Water Conference (pp. 1–10). American Society of Civil Engineers. https://doi.org/10.1061/41044(344)1

Koffi, K., Yéo, K. N., & Kouakou, A. (2016). Estimation of surface runoff in the Marahoué basin (Côte d’Ivoire) using the SCS-CN method. International Journal of Environmental Science and Technology, 13(8), 2075–2084. https://doi.org/10.1007/s13762-016-1047-9

Kouadio, K. A., Konan, K. T., & N’Guessan, A. (2021). Urbanization and hydrological vulnerability in West African cities: Comparative analysis of Yaoundé, Korhogo and Abomey- Calavi. Hydrology Research, 52(5), 1123–1140. https://doi.org/10.2166/nh.2021.145

Mishra, S. K. (2003). Soil Conservation Service Curve Number (SCS-CN) methodology: An update. University of Florida, Department of

Agricultural	and	Biological	Engineering. https://www.ces.ncsu.edu/depts/ae/faculty/mishr a/SCS-CN%20Methodology.pdf

Njeuguet, E. (2024). *Analyse des inondations dans le district d’Abidjan : impacts des précipitations extrêmes (2022-2023)*. Journal of African Hydrology, 12(1), 112-130

Oloukoi, M. D., Fofana, M., & Traoré, S. (2014). Flood risk assessment in urban areas of West Africa: Case study of Abidjan and Ouagadougou. Natural Hazards, 74(2), 1255–1272. https://doi.org/10.1007/s11069-014-1245-0

Ouattara, A., Traoré, O., & Koné, I. (2017). Influence of soil properties on surface runoff generation in the Akouédo River basin (Côte d’Ivoire). Journal of Water Resource and Protection,	9(11),	1187–1202. https://doi.org/10.4236/jwarp.2017.911072

Ponce, V. M. (1996). Engineering hydrology: Principles and practices. Prentice Hall .

Shadeed, M. (2010). Soil texture and its influence on infiltration and runoff: A review. International Journal of Soil Science, 5(2), 85–96. https://doi.org/10.3923/ijss.2010.85.96

United States Department of Agriculture, Natural Resources Conservation Service (USDA-NRCS). (1986). National engineering handbook, Section 4: Hydrology. Technical Release No. 55 (TR-55). https://www.nrcs.usda.gov/wps/portal/nrcs/detail/ national/home/?cid=nrcs143_014226

Yéro, A., Diallo, A., & Sanogo, A. (2019). Extreme rainfall events and flash floods in West African urban centers: Trends and impacts. Environmental Monitoring and Assessment, 191(10),  651.  https://doi.org/10.1007/s10661-

019-7835-4 .