Original Research Article

Study of estimation of iron content in water using conductometric method

ABSTRACT

Iron content is one of the important parameters of water quality. Permanganometric method is a very common technique to estimate Fe2+ ion. However, this technique fails to estimate Fe3+ ion but water sample contains both Fe2+ and Fe3+ ions. Colourimetric method is popular to estimate iron content in a given water sample. However, it has some limitations e.g., the method is sensitive to pH and temperature. Furthermore, the method is applicable to a limited concentration range. The present work proposed a novel technique, potentiometric method, to estimate iron content in a given water sample. The method is based on change in ionic conductance of Fe2+ ion during formation of iron-phenanthroline complex. A calibration curve is obtained while plotting change in ionic conductance values against known concentrations of Fe2+ ion. Using this calibration curve unknown concentration of Fe2+ ion in a given sample water can be determined by measuring corresponding change in ionic conductance of Fe2+ ion in presence of 1,10 phenanthroline. This novel technique is applicable in a wide concentration range. Furthermore, the method is independent of pH and temperature.

Keywords:	Mohr’s salt, Sulphuric acid, 1,10 Phenanthroline, Hydroxylamine hydrochloride, Fe-Phenanthroline complex, Potentiometer, Conductance.

__________________________________________________________

INTRODUCTION

Iron is an essential element for growth. However, presence of excess iron in drinking water causes disturbances in normal functionality of organs, causing health concern. So, iron content is one of the important parameters to specify drinking water quality. Excess iron content in water poses a threat on agricultural sector also. There are different methods to determine iron content in a given water sample, e.g., spectrophotometric method, atomic absorption spectrometric method, volumetric titration method. All these methods are based on ferrous iron (Fe2+). As iron remains both as Fe2+ and Fe3+ forms in water, it is required that a reducing agent be used to convert Fe3+ to Fe2+ ion for accurate estimation of iron content. Different analytical methods for estimation of iron are well documented in the literature.1,2 Baltzis et al. developed an optical sensor to determine speciation of iron (Fe2+/ Fe3+) using deforoxamine reagent.3 Several authors reported that UV-VIS spectrophotometric method is a useful tool to determine iron content in different water samples.4-6, Pepper et al. reported a solvent extraction method to determine ferrous and ferric iron in aqueous samples.7 It is documented that ferrous iron can be estimated in presence of ferric iron using F⁻ as complexing agent, which masks Fe3+ ion.8 Estimation of total iron by WCRM technique was reported by Lin et al.9 Gonsalves et al. reported that iron content can be determined using UV-VIS spectrometry of hydrazine complex of Fe2+ ion10. It is documented that ferrous iron content in drug samples, containing ferrous fumerate, can be determined using redox titration method.11. Kirakosyan et al. reported that ferric iron in drug samples can be estimated successfully by complexometric titration method using EDTA12. Korte et al., documented that iron content in environmental water samples can be estimated by FIA-TLS method13. Saleh and Nassir documented a new merging-zone flow injection system for estimation of iron content in water samples14.

The present work puts forth a novel technique to estimate the iron content in a water sample using potentiometric method, which helps measuring the change in ionic conductance of Fe²⁺ ion due to formation of Fe-Phenanthroline complex. This information is capitalised to obtain a calibration curve using several Mohr’s salt solutions with known concentration of Fe²⁺ ion. Using this calibration curve iron content of an unknown water sample can be determined. This novel technique is neither temperature-sensitive nor pH-sensitive and believed to be applicable in wide range of iron concentration in different water samples.

Materials and Sample solutions

Apparatus

(a) Conductivity meter (EI-make).

(b) 10 ml Pipette.

(c) 100 ml Glass beakers.

(d) 100 ml Volumetric flask.

(e) 10 ml Measuring cylinder.

(f) 100 ml Reagent bottles.

Chemicals

1. Details of chemicals and Mohr’s salt solutions, used in this work, are shown in the Table 1 and Table 2 respectively.

Table 1  	Details of chemicals used in this research work

| Chemicals | Molecular Weight | Purpose |
| --- | --- | --- |
| Mohr’s salt | 392 | To provide Fe2+ ion in aqueous solution |
| Sulphuric acid | 98 | To stabilize Mohr’s salt solution |
| Hydroxylamine hydrochloride | 69.5 | Reducing agent converting Fe3+ ion to Fe2+ ion |
| 1.10 Phenanthroline monohydrate | 198 | Complexing agent |

2. Two types of Mohr’s salt solutions are prepared for this present work.

(a) Low concentration Mohr’s salt solution.

(b) High concentration Mohr’s salt solution.

Table 2  	Concentrations of Mohr’s salt solutions

| Mohr’s salt solutions |  |  |  |
| --- | --- | --- | --- |
| Low concentration samples |  | High concentration samples |  |
| Serial No. | Concentration (N/100) | Serial No. | Concentration (N/100) |
| 1 | 0.025 | 1 | 0.16 |
| 2 | 0.034 | 2 | 0.25 |
| 3 | 0.040 | 3 | 0.34 |
| 4 | 0.050 | 4 | 0.50 |
| 5 | 0.067 | 5 | 0.65 |
| 6 | 0.075 | 6 | 0.80 |
| 7 | 0.094 | 7 | 1.00 |
| 8 | 0.100 |  |  |

Experimental methods

1. Sample preparation

All the samples are prepared at room temperature 25℃.

Hydroxylamine hydrochloride solution

1.04 gm  LR grade Hydroxylamine hydrochloride is dissolved in 50 ml double distilled water so that concentration of the solution is maintained at 0.3 M.

Mohr’s salt solutions of known concentrations (for calibration)

50 mg LR grade Mohr’s salt is dissolved in 50 ml distilled water in a 100 ml volumetric flask. 1 ml  4N H2SO4 is added into it. 5 ml Hydroxylamine hydrochloride, as prepared above, is added into it. Double distilled water is added into it till the level reaches 100 mark. So, 100 ml N50 Mohr’s salt solution is prepared. pH of the solution is measured as 4.25. This is considered as mother solution, from which 15 different Mohr’s salt solutions, as mentioned in the Table II, are prepared using double distilled water and stored in 15 different tagged reagent bottles. All these 15 solutions are required to prepare two calibration curves, one for low concentration samples and another for high concentration samples.

Mohr’s salt solutions of unknown concentrations

Two other Mohr’s salt solutions of concentrations [100 ml 0.084 (N⁄100) and 100 ml 0.2 (N⁄100)] are prepared separately. Few drops of  4N H2SO4 is added in each solution. 1 ml freshly prepared hydroxylamine hydrochloride solution is added in each solution. These two solutions are marked as unknown solutions.

1,10 Phenanthroline solution

40 mg  1,10 phenanthroline crystals is dissolved in 80 ml slightly warm double distilled water with stirring. Concentration of the solution is 2.5 (M/100) or 2.5×10-2 M.

2. General procedure

(a) Conductivity-meter is calibrated. Conductance is expressed in μS.

(b) Low concentration samples, as mentioned in the Table 2, are considered first. 10 ml of the 1st solution, i.e., 0.025 (N/100) is taken in a 100 ml beaker using 10 ml pipette. Double distilled water is added to it up to the mark 80 ml. The solution is placed under the conductivity-meter and the electrode-set is dipped slowly into it. Stir the solution slowly and wait till the conductance reading reaches a constant value. Take the reading of conductance value. The electrode-set is taken out and washed properly with double distilled water.

(c) The step (b) is repeated for the remaining 7 low concentration samples (Table 2).

(d) So 8 blank readings, i.e., without addition of 1,10 phenanthroline solution, are collected and noted down. The readings are shown in the Table 3.

(e) 10 ml of the 1st solution, i.e., 0.025 (N/100) is taken in a 100 ml beaker using 10 ml pipette. 5 ml freshly prepared 1,10phenanthroline solution is added into it. The solution turns intense red-orange in colour. Stir the solution gently. Double distilled water is added to it up to the mark 80 ml. The solution is placed under the conductivity-meter and the electrode-set is dipped slowly into it. Stir the solution slowly and wait till the conductance reading reaches a constant value. Take the reading of conductance value. The electrode-set is taken out and washed properly with double distilled water.

(f) The step (e) is repeated for the remaining 7 low concentration samples (Table 2).

(g) So 8 readings with addition of 1,10 phenanthroline solution are collected and noted down. The readings are shown in the Table 3.

(h) Step (b) is repeated for the high concentration samples (mentioned in the Table 2) without addition of 1,10 phenanthroline solution.

(i) So 7 blank readings, i.e., without addition of 1,10 phenanthroline solution, are collected and noted down. The readings are shown in the Table 4.

(j) Step (e) is repeated for the high concentration samples (mentioned in the Table 2) with addition of freshly prepared 1,10 phenanthrioline solution in each case.

(k) So 7 readings, i.e., with addition of 1,10 phenanthroline solution, are collected and noted down. The readings are shown in the Table 4.

RESULTS AND DISCUSSION

Iron in Fe2+ state forms a complex with 1,10phenanthroline (represented as phen) as given in equation (1).

Fe2++3Phen  Fephen32+ ⋯⋯⋯⋯⋯⋯(1)

The complex appears as intense red-orange colour. The size of Fephen32+ ion is expected to be much larger than that of Fe2+ ion and hence ionic conductance of Fephen32+ ion is believed to be less than that of  Fe2+ ion. In presence of phen ligand Fe2+ mostly remains as Fephen32+. So, it is expected that ionic conductance of any solution, containing Fe2+ ion, should decrease in presence of phen ligand. Using this concept calibration curve for standard Mohr’s salt solutions can be obtained by measuring decrease in conductance value of Fe2+ ion in presence of phen ligand.  As iron usually remains in Fe3+ state in water sample, it is mandatory to use a reducing agent to convert Fe3+ ion to Fe2+ion. In this method hydroxylamine hydrochloride is used as reducing agent.

1. Low concentration samples

There are 8 low concentration Mohr’s salt solutions and one unknown concentration, listed in the Table 2. The details of their conductance values are shown in the Table 3.

Table 3   Conductance values for low concentration Mohr's salt solutions

| Serial No. | Concentration N100 | Conductance (Blank value) C1 in μS | Conductance (with 1,10 phenanthroline) C2 in μS | Diff. in conductance ∆C in μS |
| --- | --- | --- | --- | --- |
| 1 | 0.025 | 45.5 | 34.1 | 11.40 |
| 2 | 0.034 | 66.0 | 44.4 | 21.60 |
| 3 | 0.040 | 80.0 | 51.3 | 28.70 |
| 4 | 0.050 | 103.0 | 61.3 | 41.70 |
| 5 | 0.067 | 133.2 | 80.5 | 52.70 |
| 6 | 0.075 | 168.6 | 104.6 | 64.00 |
| 7 | 0.094 | 201 | 117.6 | 83.40 |
| 8 | 0.100 | 217 | 127 | 90.00 |
| 9 | Unknown | 186.4 | 111.4 | 75.00 |

∆C values are plotted against concentrations of Mohr’s salt solution. The plot is shown in the Fig.1, which is called calibration curve for low concentration Mohr’s salt solutions. Mohr’s salt solution of unknown concentration shows ∆C value of 75 μS (TABLE III). According to the Fig.1, the corresponding unknown concentration of Mohr’s salt solution is 0.085 (N/100). However, the actual value is 0.084 (N/100).

Figure 1    Calibration curve for low concentration Mohr's salt solutions.

2. High concentration samples

There are 7 high concentration Mohr’s salt solutions and one unknown concentration, listed in the Table 2. The details of their conductance values are shown in the Table 4.

Table 4   Conductance values for high concentration Mohr's salt solutions

| Serial No. | Concentration N100 | Conductance (Blank value) C1 in μS | Conductance (with 1,10 phenanthroline) C2 in μS | Diff. in conductance ∆C in μS |
| --- | --- | --- | --- | --- |
| 1 | 0.16 | 359 | 199 | 160 |
| 2 | 0.25 | 518 | 278 | 240 |
| 3 | 0.34 | 709 | 379 | 330 |
| 4 | 0.50 | 1000 | 565 | 435 |
| 5 | 0.65 | 1325 | 790 | 535 |
| 6 | 0.80 | 1773 | 1123 | 650 |
| 7 | 1.00 | 2010 | 1220 | 790 |
| 8 | Unknown | 423 | 243 | 180 |

∆C values are plotted against concentrations of Mohr’s salt solution. The plot is shown in the Fig.2, which is called calibration curve for high concentration Mohr’s salt solutions. Mohr’s salt solution of unknown concentration shows ∆C value of 180 μS (TABLE IV). According to the Fig.2, the corresponding unknown concentration of Mohr’s salt solution is 0.19 (N/100). However, the actual value is 0.20 (N/100).

Figure 2    Calibration curve for high concentration Mohr's salt solution

#### Conclusions

1. Iron content of a given water sample can be estimated accurately using potentiometric method.

2. This novel technique of iron estimation obviates any use of external indicators, eliminating human errors.

3. Both high and low concentrations of Fe2+ ion in a given water sample can be estimated accurately using this potentiometric method.

4. This novel technique of iron estimation is expected to explore future research works based on conductivity-meter.

DISCLAIMER (ARTIFICIAL INTELLIGENCE)

The Authors hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

CONFLICT-OF-INTERST STATEMENT

The authors hereby declare that no conflict-of-interest exists while performing the experiment or writing this article.

REFERENCES

1. D. Merli, A. Profumo, C. Dossi,   Journal of Pharmaceutical Analysis,  2(6), pp.450-453  (2012) (https://dx.doi.org/10.1016/j.jpha.2012.05.003)

2. V.B. Mohite,  Asian Journal of Research in Chemistry  4(3), pp.348-361 (2011) (https://ajrconline.org/AbstractView.aspx?PID=2011-4-3-4)

3. D. Baltzis, D., A. Tsiasioti, C.K. Zacharis, P.D. Tzanavaras, P. D., Chemosensors 11(12) pp.577 (2023) ( https://dx.doi.org/10.3390/chemosensors11120577)

4. V.S. A. Devi , V.K. Reddy,  International Journal of Analytical Chemistry Vol.2012(Special issue), pp.1-12 (2012) (https://dx.doi.org/ 10.1155/2012/981758)

5. S. Balasubramanian, A. Udayabhanu, P.S. Kumar, P. Muthamiselvi, C. Eswari, A. Vasantavada, A. Kapoor, A.,  International Journal of Environmental Analytical Chemistry  103(11), pp.2480-2497 (2021)  (https://dx.doi.org/10.1080/03067319.2021.1893711)

6. B. Peng, Y. Shen, Z. Gao, M. Zhou, Y. Ma, S. Zhao, S., Food Chemistry  176, pp.288-293 (2015) (https://dx.doi.org/10.1016/j.foodchem.2014.12.084)

7. S. E. Pepper, M. Borkowski, M. K. Richmann, D. T. Reed, D. T.,  Analytica Chimica Acta  663(2) , pp.172-177 (2010)  (https://dx.doi.org/10.1016/j.aca.2010.01.056)

8. J. Zhu, X. Yang, F. Fan, Y. Li,   Applied Water Science  Vol.8, pp.228 (2018) (https://dx.doi.org/10.1007/s13201-018-0876-6)

9. L. Lin, X. Liu,  Biosystems Engineering  220, pp.233-242 (2022) (https://dx.doi.org/10.1016/j.biosystemseng.2022.06.004)

10. A.A. Gonsalves, C.R.M. Araújo,  C.X. Galhardo, M.O.F. Goulart, F.C. Abreu,  American Journal of Analytical Chemistry  Vol.2(7), pp.776-782 (2011) (https://dx.doi.org/10.4236/ajac.2011.27089)

11. M. A. Balarabe, A. Z. Folashade, A. Z., World Journal of Applied Chemistry  Vol.4(3), pp.42-44 (2019) 42 (https://dx.doi.org/10.11648/j.wjac.20190403.13)

12. V. Kirakosyan, A. Davinyan,  Der Pharma Chemica  Vol.13, pp.57-62 (2021) (https://www.derpharmachemica.com/archive.html) issue 5

13. D. A. Korte, G. Tomsic, A. Bratkic, H. Budasheva, M. Franko, L. Goljat, Acta Chimica Slovenica  Vol.66(4), pp.814-820 (2019)  (https://dx.doi.org/10.17344/acsi.2018.4825).

14. A.S. Farhood, D.N. Taha,  Analytical Science and Technology  35(5), pp.218-227 (2022)   (https://dx.doi.org/10.5806/AST.2022.35.5.218)