Original Research Article

ENHANCING MECHANIZATION OF PADDY CULTIVATION FOR SMALL SCALE FARMERS

# Abstract

Power tiller operated machines may play vital role in paddy cultivation for small scale farming system as the average holding size is close to 1 ha and more than 86% of India’s farming community are small and marginal farmers. Present efforts were carried out for the development and testing of power tiller operated strip-till multi-crop planter for the mechanization of paddy cultivation in small holder farming community. With increasing the level of mechanization by adopting appropriate matching implements for paddy cultivation in small farming community, the production and productivity could be increased by reducing cost of cultivation and increasing input use efficiency. Efforts are being made to adopt conservation tillage practices with power tiller operated matching implements for small holder farming focusing on paddy cultivation. The developed planter was tested under three different tillage practices as conventional tillage (CT), strip tillage (ST) and zero tillage (ZT). Three types of rotary blades viz. L-type, J-type and C-type were used in strip-tilling operation. The developed machine was evaluated in terms of field capacity, fuel consumption, draft, seed germination and crop yield. Average paddy yield in CT was maximum (2672 kg ha-1) followed by ST (2556 kg ha-1) and ZT (2442 kg ha-1). The cost of cultivation was maximum for CT followed by ZT and ST.

Key words: paddy; mechanization; small holder; power tiller; matching implements;  conservation tillage; strip tillage; zero tillage

1. Introduction

Rice (Oryza sativa L.) is one of the most important food crops in the world, and staple food for more than 60% of the global population (Kaur and Singh, 2017; Kumar and Karmakar, 2025). In India, rice is the important food crop covering about one-fourth of the total cropped area and providing food to about half of the Indian population (Sankaralingam, 2020). The total rice cropped area is 161.5 million hectares in the world. In India, rice is grown on an area of 43.5 m ha with a total production of 105.5 mt and productivity of 2.4 t/ha (Tiwari et al., 2020). Transplanted paddy require low seed rate (50 kg ha-1) as compared to other methods like drilling (100 kg ha-1) and broadcasting (130 kg ha-1) (Kumar and Thomas, 2019). Basically in India establishment of rice depends on the availability of moisture, climatic condition, age of the variety, availability of inputs & human labour. Rice is a labour-intensive crop and requires about 80-90 labour days per acre (Nadaf et al., 2023). Timely availability of labour and water for various activities of rice is becoming a problem.

Mechanization for paddy cultivation is done for different farm operations right from field preparation, direct seeding, transplanting to harvesting and threshing by using various tractor drawn implements. Marginal and small land holders are not able to adopt mechanization due to lack of appropriate power tiller matching implements and also lack of awareness to use power tillers effectively. Kumar and Karmakar (2024) studied the performance of various power tiller operated matching implements for different farm operations in conservation agriculture for various crops. The mechanization level of India depicts that the farm power availability has crossed 2.6 kW ha-1 (Rathinavel et al., 2024). Power tiller operated machines may play vital role for small scale farming system as the average holding size is close to 1 ha and more than 86% of India’s farming community are small and marginal categories (Kumar and Karmakar, 2025a).

Kumar and Thomas (2019a) conducted an experiment to analyse the four bar linkage mechanism of a power tiller operated 8-row rice transplanter. Position analysis, velocity analysis and acceleration analysis were done from both analytical as well as graphical methods. The acceleration and velocity of transplanting finger at different crank angles were evaluated. The maximum acceleration and velocity were found to be 3866.5 cm s-2 and 280.32 cm s-1 respectively, at 125 rpm crank speed. The angle of fixed link with the horizontal was 37.41⁰. Total distance travelled by the finger in the vertical direction was found to be 20.05 cm.

Kumar and Thomas (2019) conducted an experiment to develop a manually operated 2-row rice transplanter with a modified four-bar linkage mechanism of power tiller operated 8-row rice transplanter. The average hill-to-hill spacing, depth of planting, no. of seedlings per hill and height of seedlings were found to be 21.68 cm, 6.98 cm, 3.68 and 20.21 cm, respectively. The missing hills and floating hills were found to be 8.54% and 3.66%, respectively. The actual field capacity and the efficiency of the developed rice transplanter were found to be 0.14 ha day-1 (8 hrs) and 85.5%, respectively.

Modi et al. (2022) developed a tractor operated mat type nursery seeder for the preparation of mat type nursery for mechanized transplanting of paddy with paddy transplanter. It was observed that mat type nursery seeder saves 60-65% cost and 90-95% labour as compared to the traditional method of nursery raising.

Maklavani et al. (2025) investigated that agricultural mechanization can enhance the technical efficiency, energy consumption, and environmental sustainability of paddy cultivation compared to conventional farming practices. Results showed that mechanized farming systems gives higher productivity and lower energy use, while also reducing environmental impacts such as greenhouse gas emissions, water consumption, and soil erosion.

2. Materials and Methods

2.1 Development of strip-till multi-crop planter

Power tiller operated strip-till multi-crop planter was developed which consists of furrow opener, seed and fertilizer boxes, seed metering mechanisms, seed and fertilizer tubes, ground drive wheel, furrow covering unit, strip-tilling unit, chain & sprockets, set of bevel gears etc. The power tiller VST Shakti 130DI was used as the prime mover.

2.2 Performance evaluation of the developed multi-crop planter

The performance of the developed strip-till multi-crop planter was evaluated in the laboratory as well as in the field with paddy of variety GB1.

2.2.1 Experimental site

Field investigations were carried out during the cropping season of 2022-23 at Instructional Farm of Bidhan Chandra Krishi Viswavidyalaya (BCKV), Nadia, West Bengal, India. The research work was a part of the PhD research project named “Power Tiller Operated Strip-till Multi-crop Planter for Conservation Agriculture”. The homogeneous soil of the field was clay loam type. The design of experiment was split plot with three replications and the whole experiments were carried out in accordance to the design. The crop was cultivated according to three different tillage practices viz. conventional tillage (CT), strip tillage (ST) and zero tillage (ZT).

2.2.2 Experimental details

The entire field of area 26 x 62 m2 was divided equally into three main plots of 8 x 62 m2 area comprising of 3 different tillage practices as conventional tillage (CT), strip tillage (ST) and zero tillage (ZT). The paddy (GB1) was sown in Kharif season. For field preparation, various tractor drawn tillage implements were used in CT and ST whereas no tillage was done in case of ZT. The crop was sown with the developed power tiller operated strip-till multi-crop planter.

2.2.3 Crop establishment technique

The developed power tiller operated strip-till multi-crop planter was used for sowing of paddy in CT and ZT without strip tilling and in ST with strip tilling. The VST Shakti 130DI was selected as the matching prime mover for the development of strip-till multi-crop planter. The row to row and plant to plant spacing were 20 cm and 15 cm, respectively. Inclined plate type seed metering mechanism was used to maintain seed to seed spacing of the crop. The metering plates were designed on the basis of engineering properties of paddy (GB1) seeds. Separate seed boxes were used for rows and row to row spacing was adjustable. The number of rows of the power tiller operated multi-crop planter for paddy were four.

The multi-crop planter was calibrated before sowing operation in the field. The inclined type seed metering plates with cells made on the periphery of the circular plates was used for paddy seeds.

The recommended dose of fertilizer (RDF, kg ha-1) was (N:P2O5:K2O) 50:20:20 for paddy. Urea (46 % N), Di-ammonium Phosphate (DAP) (18 % N and 46 % P2O5) and Muriate of Potash (MOP) (60 % K2O) were used to apply to meet N:P2O5:K2O dose. The amount of nitrogen (N) was calculated from Urea (46 % N) and DAP (18 % N) to meet out required dose. The amount of P2O5 and K2O were calculated from DAP (46 % P2O5) and MOP (60 % K2O), respectively to meet out the required dose.

3. Results and discussion

The performance of the developed power tiller operated strip-till multi-crop planter was evaluated in both laboratory and field conditions with paddy (GB1) seeds. The machine was calibrated five times with a fixed number of revolutions of ground drive wheel (5, 10, 15, 20, and 25). The average seed rate was found to be 16.42 kg ha-1.

The sand bed method was used to evaluate the performance of the machine in the laboratory. The average seed to seed spacing, number of multiples, missing hills and visible damage were found as 15.40 cm, 4.48%, 1.99% and 1.49%, respectively. The field performance of the developed planter was undertaken in terms of field capacity, draft, fuel consumption, seed germination and crop yields.

3.1 Effective field capacity

The average EFC at 1.5 km h-1 forward speed for paddy sowing was 0.105 ha h-1. And when forward speed increased to 2.5 km h-1, the average EFC increased to 0.161 ha h-1 as shown in Fig. 1 (a). The percent increase in EFC from 1.5 km h-1 to 2.5 km h-1 forward speed was 53.33%. With the increase in operating depth, the average EFC was increased for C-type rotary blade. The average EFC for L-type blade was minimum at 5 cm operating depth and maximum at 6 cm operating depth. The average EFC for J-type rotary blade was minimum at 4 cm operating depth and maximum at 5 cm operating depth as illustrated in Fig. 1 (b). This was due to minimum time losses during turning, adjustment, seed refilling etc.

| (a) | (b) |
| --- | --- |

Fig. 1 Effective field capacity at different forward speed and operating depth for different rotary blades

3.2 Fuel consumption

It was observed that the fuel consumption decreases with increase in forward speed of the machine for all types of rotary blades used in strip-tilling. The fuel consumptions at 1.5 km h-1 forward speed for paddy were 13.29, 13.09 and 12.48 L ha-1 for L-type, J-type and C-type rotary blades, respectively. With increase in forward speed to 2.5 km h-1, the fuel consumptions were 12.21, 12.11 and 11.78 L ha-1 for L-type, J-type and C-type rotary blades, respectively as shown in Fig. 2 (a). The average fuel consumption at 1.5 km h-1 forward speed was 12.96 L ha-1. With increase in forward speed to 2.5 km h-1, the average fuel consumption decreased to 12.03 L ha-1. The percentage decrease in fuel consumption with increase in forward speed from 1.5 to 2.5 km h-1 was 7.18%. It was observed that with increase in operating depth from 4 to 6 cm, the average fuel consumption was decreased for L-type and C-type rotary blades and for J-type rotary blade, it was minimum at 5 cm operating depth and maximum at 6 cm operating depth as illustrated in Fig.2(b).

| (a) | (b) |
| --- | --- |

Fig. 2 Fuel consumption of the machine at different forward speed and operating depth for different rotary blades

3.3 Wheel slip

It was observed that the average slip percentage of power tiller wheel was 8.96±1.97% in paddy cultivation. The slip percentage was decreased with increase in forward speed as shown in Fig..3(a). The average slip at 1.5 km h-1 forward speed was 9.92% for paddy sowing. At 2.5 km h-1 forward speed, the average slip percentage was 7.51%. When the forward speed increased from 1.5 km h-1 to 2.5 km h-1, the slip percentage was decreased by 24.29%. It was also found that slip percentage decreases with increase in operating depth as illustrated in Fig..3(b). The average slip percentage at 4 cm operating depth was 9.92%. At 6 cm operating depth, the average slip percentage was 7.54%. It was seen that the maximum slip percentage was for L-type rotary blade. The slip percentage for L-type blade was 10.47%. The minimum slip percentage was 6.27% for C-type blade at 6 cm operating depth.

3.4 Draft

The draft of the machine was increased with increase in operating depth and forward speed for all types of rotary blades as illustrated in Fig. 4 (a & b). The drafts at 1.5 km h-1 forward speed were 167, 160 and 160 N for L-type, J-type and C-type rotary blades, respectively. With increase in forward speed to 2.5 km h-1, the drafts were 230, 277 and 237 N for L-type, J-type and C-type rotary blades, respectively. With increase in operating depth from 4 cm to 6 cm, the draft increases for all types of rotary blades used. The maximum draft (318 N) was found at 6 cm operating depth for J-type rotary blade and minimum draft (133 N) was found at 4 cm operating depth for L-type rotary blade.

| (a) | (b) |
| --- | --- |

Fig. 3 Slip at different forward speed and operating depth for different rotary blades

| (a) | (b) |
| --- | --- |

Fig. 4 Draft of the machine at different forward speed and operating depth for different rotary blades

3.5 Germination of seeds

The average germination of paddy seeds was observed for strip tillage operation with different input parameters as illustrated in Fig..5. It was observed that the average seed germination of paddy were 84.01±2.21%, 83.57±2.84% and 84.81±1.23%, respectively for L-type, J-type and C-type rotary blade tilled plots.

It was seen from the Fig..5(a) that with increase in forward speed from 1.5 km h-1 to 2.5 km h-1, the seed germination was more or less same for all rotary blades. With increase in operating depth from 4 cm to 6 cm, the seed germination was almost same at 4 cm and 6 cm operating depth for all types of rotary blade but at 5 cm operating depth, the seed germination was minimum for J-type of rotary blade as shown in Fig. 5(b).

| (a) | (b) |
| --- | --- |

Fig. 5 Seed germination at different forward speed and operating depth for three types of rotary blades

3.6 Yield of paddy

The average yield variation of paddy under strip tillage plots with forward speeds and operating depths are presented in Fig..6. It is observed from Fig. 6 (a) that the average yield of paddy was maximum (2628 kg ha-1) under strip tilled plot operated by L-type rotary blade at 1.5 km h-1 forward speed and minimum (2286 kg ha-1) operated by J-type rotary blade at 2.5 km h-1 forward speed.

From Fig..6 (b), it was seen that the average yield of paddy was maximum (2804 kg ha-1) under strip tilled plot operated by L-type rotary blade at 4 cm operating depth and minimum (2149 kg ha-1) operated by J-type rotary blade at 5 cm operating depth.

The average yield of paddy under strip tillage operation for various input parameters and the yield under conventional tillage and zero tillage are presented in Fig..7. From Fig. .7(a), it was seen that the average paddy yield was maximum for conventional tillage plots (2672 kg ha-1) followed by strip tilled plots (2556 kg ha-1) operated at 1.5 km h-1 forward speed and zero tilled plot (2442 kg ha-1).

From Fig..7(b), it was observed that the average paddy yield of all the strip tilled plots operated at 4 cm operating depth was maximum (2658 kg ha-1) followed by 6 cm (2442 kg ha-1) and 5 cm (2406 kg ha-1) operating depth.

Similarly, from Fig..7(c), it was seen that the average paddy yield of all the strip tilled plots operated by L-type rotary blade was maximum (2575 kg ha-1) followed by C-type (2543 kg ha-1) and J-type (2389 kg ha-1) rotary blade.

| (a) | (b) |
| --- | --- |

Fig..6 Paddy yield variation with (a) forward speed and (b) operating depth for different rotary blades

| (a) | (b) |
| --- | --- |

(c)

Fig..7 Paddy yield variation of strip tilled plot with (a) forward speed (b) operating depth and (c) type of rotary blade and comparison with yield under zero tillage and conventional tillage

3.7 Cost of production for paddy

From Fig..8, it was found that the cultivation cost of paddy was maximum under CT (58230 INR ha-1) followed by ZT (46559 INR ha-1) and ST (45784 INR ha-1). The gross return was observed as maximum for CT (85130 INR ha-1) and minimum for ZT (77802 INR ha-1) whereas net return was maximum for ST (33936 INR ha-1) and minimum for CT (26899 INR ha-1). The cultivation cost under CT was maximum due to higher operational charge and electricity charge used during irrigation. On the other hand, the gross return and net return were maximum under strip tillage because of lower input resources used for cultivation and higher crop production. The B:C ratio were observed as 0.74, 0.67 and 0.46 for ST, ZT and CT, respectively (Table 1).

Fig. 8 Cost of cultivation and returns under different tillage practices for paddy

Table 1 Cost of cultivation and returns under different tillage practices

| Crop | Tillage | Cost of cultivation, INR ha-1 | Gross return, INR ha-1 | Net return, INR ha-1 | B:C ratio |
| --- | --- | --- | --- | --- | --- |
| Paddy (GB1) | ST | 45784.27 | 79720.73 | 33936.46 | 0.74 |
|  | ZT | 46558.93 | 77802.12 | 31243.19 | 0.67 |
|  | CT | 58230.48 | 85129.92 | 26899.44 | 0.46 |

4. Conclusions:

Sowing of seeds is a very crucial and time bound farm operation for better crop production and productivity. With the considerations of small and marginal farm holders, and the advantages of strip tillage (ST) over conventional tillage (CT) and zero tillage (ZT), the present work was carried out to develop a power tiller operated strip-till multi-crop planter for conservation agriculture. Conventional tillage requires large amount of energy as well as high operating cost. The operating time from tillage to sowing is also high in CT as compared to combined tillage and ZT. The loss of natural resources from CT could be minimized with the adoption of reduced tillage or combined tillage. The study shows that power tiller operated strip-till multi-crop planter can be used effectively to compensate conventional and zero tillage practices especially for small and marginal farm holdings.

5. References

Baral, S. S., Budhe, V. K., Khurana, R. and Swarnkar, R. (2019). Development of tractor drawn electronic multi-crop planter cum fertilizer applicator for precision farming. International Journal of Advances in Agricultural Science and Technology, 6(10): 1-16.

Bisen, S., Victor, V. M. and Chandraker, A. K. (2020). Performance evaluation of three row animal drawn multi crop planter cum herbicide applicator. International Journal of Applied and Pure Science and Agriculture, 6(12): 33-40.

Hoque, M. A. and Karim, M. R. (2015). Up-scaling and evaluation of BARI inclined plate planter. The Agriculturists, 13(2): 1-8.

Kaur, J. and Singh, A. (2017). Direct seeded rice: prospects, problems/constraints and researchable issues in India. Current Agriculture Research Journal, 5(1): 13-32.

Kumar, S. and Karmakar, S. (2024). Comparative study of power tiller operated machines for small holders farming in conservation agriculture. Journal of Experimental Agriculture International, 46 (12): 128-136.

Kumar, S. and Karmakar, S. (2025). Engineering properties of maize, groundnut and paddy seeds for the design of seed metering mechanism of multi-crop planter. Journal of Scientific Research and Reports, 31 (7): 766-773.

Kumar, S. and Karmakar, S. (2025a). Mechanization of groundnut cultivation for small holder farming community. Journal of Scientific Research and Reports, 31 (7): 725-734.

Kumar, S. and Thomas, E. V. (2019). Development of manually operated 2-row rice transplanter. Agricultural Engineering Today, 43(1): 34-38.

Kumar, S. and Thomas, E. V. (2019a). Analysis of four-bar linkage mechanism of a power tiller operated 8-row rice transplanter. Agricultural Engineering Today, 43(3): 24-31.

Maklavani, F. H., Parashkoohi, M. G., Zamani, D. M. and Afshari, H. (2025). Investigating the impact of integrating land consolidation with agricultural mechanization on the technical, energy, and environmental dimensions of paddy production. Journal of Agricultural Engineering, 56: 1721.

Modi, R. U., Manes, G. S., Mahal, J. S., Dixit, A. K. and Singh, A. (2022). Mechanized sowing of mat type paddy nursery for mechanical transplanting. Indian Farming, 72(4): 10-12.

Nadaf, S., Praveen, T. and Arshad, S. (2023). Design and fabrication of rice planting machine. Journal of Nonlinear Analysis and Optimization, 14(1): 373-380.

Nandini, Y., Moses, S. C. and Aalam, R. N. (2018). Design of manually operated multi-crop inclined plate planter for farm women. International Journal of Current Microbiology and Applied Sciences,7(6): 931-940.

Rathinavel, S., Kavitha, R., Mohankumar, A. P., Suthakar, B. and Surendrakumar, A. (2024). Mechanization challenges in selected district agricultural systems in India. Journal of Experimental Agriculture International, 46(9): 122-129.

Sahu, M. and Verma, A. (2017). Design and field experiments of precise seed metering mechanism with paddy seeds in Chhattisgarh. The Ecoscan: An International Quarterly Journal of Environmental Sciences, 11(1): 37 – 43.

Sankaralingam, V. K. (2020). Technologies to enhance the productivity and profitability in mechanised semi dry rice cultivation. Journal of Pharmacognosy and Phytochemistry, 9(4): 234-237.

Tiwari, R. K., Mehta, C. R., Sathpathy, S. K., Jekendra, Y., Patle, R. and Sirmour, A. (2020). Seeding and transplanting equipment for rice production for last four decades – a review. Indian Journal of Hill Farming, 33(2): 226-231.