# Original Research Article

Temperature Data-driven Predictive Control for Wafer Rapid Thermal Processing

.

ABSTRACT

Rapid Thermal Processing (RTP) is a key technology for semiconductor manufacturing. However, the strong coupling, nonlinearities, and multiple disturbances inherent in RTP render accurate physical modeling extremely challenging. This paper studied the application of subspace predictive control (SPC) to a wafer RTP system driven by standard tungsten halogen lamps. This control algorithm does not require an exact RTP physical model and employs data-driven method for temperature control. The subspace identification algorithm is employed to construct a subspace-based predictor for the RTP system. The pseudo-random binary series (PRBS) is designed as the excitation signal of RTP system to obtain the input and output data. To design such PRBS to persistently excite the system, several pre-experiments for determining its settling time and maximum frequency are needed to be designed beforehand. The designed PRBS excitation signal produces a highly periodic RTP output, which proves the effectiveness of the input and output data. Finally, The temperature control of RTP system is realized by SPC. Experimental results show that the achieved wafer temperature uniformity and control accuracy satisfy the required RTP performance metrics.

Keywords: Rapid thermal processing; Subspace predictive control; Data-driven method; Pseudo-random binary series

1. INTRODUCTION

Precise temperature control across wafers and wafer stacks is essential for multiple semiconductor-manufacturing processes. Rapid Thermal Processing (RTP) is widely employed in semiconductor manufacturing to precisely tailor material properties—enhancing or modulating conductivity, dielectric constant, and density while eliminating contaminants(Fair et al.; Singh et al.; Borisenko et al.; Campbell et al.). As device sizes in ultra-large-scale integration (ULSI) continue to shrink, semiconductor manufacturing technology faces increasingly formidable challenges. It is necessary to constantly update the technology to improve the accuracy and uniformity of wafer temperature in RTP system.

There are many mature control strategies for RTP temperature control system. Xiao et al. (2015) extracted the dominant modes using the Karhunen–Loeve (K-L) based Galerkin’s method to construct a reduced-order model and designed an iterative learning controller. Yuan et al. (2024) design a feedforward decoupling PID control based on a five-order coupled thermal model. Ling et al. (2010) proposed a scheme that combines feedforward compensation with multiplexed model predictive control (MPC). In addition, Xiao et al. (2018) applied synergetic control to the rapid thermal processing (RTP) system. In contrast, model predictive control (MPC) offers distinct advantages for temperature regulation in RTP due to its inherent capabilities for handling constrained, multivariable, and multi-objective optimization.

However, the practical implementation of MPC is heavily reliant on the availability of an accurate mathematical model, whose precision is critical to ensuring closed-loop stability and overall control performance (Morari et al., 1999). The controlled object RTP device itself belongs to a typical multi-input multi-output, strong coupling distributed nonlinear time-varying process (Fair et al., 2012). The physical properties of the wafer vary with temperature in a highly uncertain manner. Consequently, maintaining temperature uniformity across the wafer surface within a strict tolerance of 5 ℃ is essential for normal process operation (Dassau et al., 2006). It is worth noting that RTP is a highly integrated system with a variety of superimposed physical fields, such as temperature, pressure, magnetic fields, and cooling circulation systems. Therefore, the correlation between channels results in strongly coupled characteristics(Anderson et al., 2004). Under field conditions, it is necessary to consider the uniformity between output temperatures under the condition of limiting according to the actual equipment safety driving index. The above contents all reflect that it is difficult to obtain the temperature model of RTP process.

With the development of information technology and control theory, it has become possible to measure and store data during the operation of the equipment (Berberich et al., 2022; Campi et al., 2002). These data contain implicit information about the system, making it possible to design controllers on this basis, an approach known as data-driven control (DDC) ( Hou et al., 2010). The subspace predictive control (SPC) algorithm proposed by De Moor and Favoreel directly utilizes input–output data to regulate the plant (Favoreel et al., 1999). The emergence of the SPC algorithm effectively solves the problem that it is difficult to obtain an accurate mathematical model through mechanism modeling. The SPC algorithm can directly obtains the mathematical model in the form of state space according to the collected input and output data, and the subspace predictor equation of the identified intermediate result can be used as the prediction model. Based on least-squares principles, the SPC admits a simple recursive implementation with excellent numerical properties. Zhao et al. (2022) applied data-driven predictive control to counteract the strong time-varying nonlinearity of selective catalytic reduction (SCR) systems, achieving effective reduction of nitrogen-oxide and ammonia emissions while meeting real-time constraints. Vajpayee et al. (2020) crafted a disturbance-observer-augmented subspace predictive controller and deployed it for agile load-following in pressurized-water reactor nuclear power plants. Carlet et al. (2022) systematically reviewed the paradigm shift from model-based to fully data-driven predictive control and experimentally compared the two approaches on benchmark systems. They applied it to the current control of synchronous motor drives and demonstrated that the data-driven methods can achieve performance comparable to the model-based benchmark.

In this paper, a data-driven subspace predictive control algorithm is developed for RTP system. The control algorithm employs subspace identification to derive a predictor that serves as the multi-step output equation. Pre-experiments need to be designed to obtain the subspace predictor. The pseudo-random binary series (PRBS) matched with the RTP system is studied and designed as the excitation voltage of the pre-experiment. To design such PRBS to persistently excite the system, several pre-experiments for determining its settling time and maximum frequency are needed to be designed beforehand. Finally, the effectiveness of SPC algorithm for RTP wafer temperature control is verified by numerical simulation.

This paper is organized as follows. Section 2 reviews the SPC fundamentals. Section 3 designs the specific parameters of PRBS. Section 4 verifies the effectiveness of SPC on RTP temperature control by simulation. Section 5 concludes the paper.

2. SRC ALGORITHM

The linear system can be described by the following process form

,                                                                       (1)

.                                                                         (2)

where , , and  are the input, output and state of the system. The matrices , , , and  are the state, input, output, and feed-through gain matrices of the system.

It is assumed that the input excitation and output temperature measured in the pre-experimental stage are

,                                                                        (3)

,                                                                       (4)

where  is the length of the collected data. This data can be decomposed into  prediction problems, each prediction horizon is . It is assumed that the measurements of inputs and outputs ,  for are available. The data block Hankel matrices for  are defined as

,                                                              (5)

.                                                            (6)

The subscripts  and  denote past and future matrices respectively. Each block element in the Equation (5) and Equation (6) is a column vector of inputs, . Similarly data Hankel matrices for output  , represented as  and   can be written.

,                                                                    (7)

.                                                                 (8)

Also, we define the state sequence as

,                                                                        (9)

.                                                                    (10)

Equation (1) and Equation (2) recursively obtain the predicted output matrix

,                                                                         (11)

where  is the extended observability matrix, is the block Toeplitz matrices containing the impulse response of the system to the deterministic input respectively, denoted by

,.

The prediction of the output can be written as

,                                                                        (12)

where ,  is the output of the data-driven predictor model,  is the subspace matrix that corresponds to the past input and output data, and  is the subspace matrix that corresponds to the future input data. These predictor matrices can be computed approximately by solving the following least-squares problem

.                                                                    (13)

Subspace identification uses linear algebra tools QR decomposition and SVD decomposition to solve the above least squares problem. The optimal predicted value  for the future output increment  can be described as the orthogonal projection from the column space  onto the column space composed of  and .

.                                                                               (14)

The numerical realization of this projection can be obtained by QR decomposition

,                                                               (15)

where  is a lower triangular matrix and  is an orthogonal matrix. Solve equation (14)

.                                                                                (16)

Obtain the parameter matrix

,                                                            (17)

where  and . The element matrices in Equation (12) is analyzed as follows

.                                     (18)

The subspace matrix of Equation (12) is analyzed in detail. The first row of the subspace matrix  is the ARX model of the system. When , the subspace matrix  serves as the impulse response model for the system's deterministic inputs.

The target signal of the control output  is defined as . Within the prediction horizon, the objective of the predictive control system is to make the predicted output as close as possible to the target signal. We assume the target signal remains constant throughout the optimization window and define

.                                                                          (19)

In addition to ensuring output tracking of the desired reference trajectory, it is critical to mitigate excessive variations in the control input. This objective can be achieved by incorporating soft constraints into the performance index function formulation. Therefore, SPC solves the following constrained quadratic programming (QP) problem at each sampling instant

,                                                             (20)

where  and  are extended weight matrices, which are composed of weight matrices  and .  is prediction horizon and  is control horizon, and  and  are defined as being equal to . The definitions of  and  are

(21)

Use Equation (20) to solve . It should be noted that only the first value in the current value can be selected to calculate the next control. This rule must be followed in every subsequent iteration.

The SPC control algorithm establishes sliding data windows  and  by collecting past input and output data at time , and calculates the current control amount based on past data. Since the classical QP problem is used to solve the optimal control quantity, it is proved that the SPC algorithm also has the ability to deal with constraints.

3. EXCITATION SIGNAL DESIGN

3.1 RTP System

RTP is an inherently dynamic and nonlinear process. The system exhibits interesting properties: different component time constants, radiation-dominated nonlinear behavior, nonlinear lamps, power supply effects, sensor number and placement, lamp number and placement, and large temperature variations. To quantify the response time and its temperature dependence, we consider a simplified nonlinear model incorporating radiative  and conductive  heat transfer terms (Ebert et al. 2004).

,                                                                (22)

where , , , and .  is the wafer temperature and  is the control voltage.

To estimate the subspace predictor for the nonlinear RTP system at a specific environmental temperature, the input voltages for the lamps are chosen as mutually independent PRBS. Owing to their favourable statistical properties and strong resemblance to white noise, PRBS signals ensure high estimation accuracy. Consequently, they serve as suitable reference control signals for the RTP system, generating uncorrelated sequences of uniform amplitude.

3.2 Calculate The Settling Time

The transient time  of the input is measured based on the time interval during which the step response first enters and remains within a specified range around the steady-state value. This range is defined to ensure the response does not exceed the set boundaries. By observing the pulse signal output, the transition process time of the single-input system can be accurately determined. The step signal is well applied to the observation transient time. A step signal with amplitude of 1 is input to the system, and  is 10.1s from fig. 1.

Fig. 1. The step response of the wafer RTP system

To get accurate results, we need to design the valid input in advance that not only meets the stable output but also meets the requirements of the field equipment for the process. It has been pointed out above that the production of chips requires strict uniformity of heating.

3.3 The Maximum Working Frequency

The Maximum Working Frequency  is measured by observing the tracking ability of the system to the square wave signal. It corresponds to the open-loop cutoff frequency, beyond which signals are significantly attenuated. In digital signal processing, this cutoff frequency serves as a critical threshold. Any signal component with a frequency higher than this value is assigned zero magnitude, a practice essential for preventing aliasing and maintaining system stability.  defines the upper limit of effective response frequency of the system and determines that the excitation signal must have sufficient bandwidth. The excitation signal must be able to continuously excite the system so that its frequency components should cover all important dynamic modes of the system.

A square wave with a duty cycle of 50 % is used as the input signal, which is applied to the system from its low frequency band and gradually increases the frequency, while observing the output response. In the case of continuous experiments, the input response under the 50 % duty cycle pulse ( 6.7Hz ) excitation is as follows.

Fig. 2. The pulse response of the wafer RTP System

3.4 PRBS parameters

Select appropriate shift pulse period , cycle count , and PRBS structural parameters to comprehensively capture the dynamic characteristics of the RTP process. The following constraints should be met

,                                                                             (23)

.                                                                          (24)

In order to ensure that the PRBS can excite all the dynamic frequency bands of the system, the maximum frequency must cover the  of the system. The period length of PRBS must be greater than the stabilization time of the system to ensure that the system can fully respond and enter a steady state. The  obtained in this way can ensure that the collected data contains the complete dynamic information of the system.

3.5 PRBS design

This paper will take a classic engineering case as an example. When PRBS is selected as the excitation signal of the pre-experiment, the amplitude, the shift pulse period  and the cycle count  of PRBS need to be determined. We choose the amplitude is 1. According to the analysis of Figure 1 and Figure 2, the system transient time  10.1s and the maximum operating frequency  6.7Hz were measured, 0.04s and  255 were calculated.

Fig. 3. PRBS

Fig. 4. Output excitation response

Fig. 3 and Fig. 4 show the input  and output  data in the pre-experiment. As visually presented in Figure 3, the random fluctuations of the PRBS signal over one cycle offer a clear manifestation of its wide-ranging distribution in the time domain. The pseudo-random nature of the signal guarantees continuous and sufficient excitation of all dynamic modes within the system. Fig. 4 illustrates the dynamic behavior of the system when stimulated by the aforementioned PRBS. The output response of three cycles is selected according to experience. It can be seen from the fig. 4 that the output has good repeatability, which proves that the designed PRBS has a good excitation effect. Its shape is determined by the system's inherent dynamic characteristics, such as inertia and delay.

4. simulation

4.1 Simulation verification

The controller parameters  and  are set to 5,  = 15 and  = 1. Fig. 5 shows the temperature control of the single wafer RTP system by the SPC control algorithm. The wafer temperature in the figure can well track the reference input. In terms of steady-state error, the wafer temperature is 25.013. The error is far less than the engineering relative error of 0.5. Fig. 6 shows the control voltage of the single wafer RTP system.

Fig. 5. The control temperature of the wafer RTP System

Fig. 6. The control voltage of the wafer RTP System

Fig. 7. The control temperature of the wafer RTP System

In order to verify the anti-interference ability of the control system, step disturbance is added after the temperature is stable. It can be seen from Fig. 7 that the control algorithm has a good effect on suppressing disturbance.

The common RTP equipment in engineering belongs to a typical multi-input multi-output, strong coupling distributed nonlinear time-varying process. This paper further considers a three-in-three-out linear RTP model supplementary verification (Franklin et al. 2002).

,                                                                           (25)

,                                                                           (26)

where

,

is a unit matrix,  is a zero matrix. The linear model is used for supplementary verification, and the PRBS is designed as the first-order model. It should be noted that when designing the PRBS parameters for the third-order system, a step signal must be used as the input in sequence to ensure the most suitable value of  is found. To determine the maximum operating frequency, the input square wave is swept upward from a low frequency until, among the three output waveforms, at least one exhibits no obvious jitter. Finally, three unrelated sequences with the same amplitude are obtained. Fig. 8 shows three unrelated PRBS excitation signals. The excitation signal of one cycle of PRBS is demonstrated. Fig. 9 shows the excitation response of MIMO RTP system. The response curve shown in this figure has good periodicity, which proves the effectiveness of the PRBS design.

Fig. 8. The PRBS of the MIMO wafer RTP System

Fig. 9. The excitation response of the MIMO wafer RTP System

Fig. 10. The control temperature of the MIMO wafer RTP System

Fig. 8 shows the temperature control of the MIMO wafer RTP system by the SPC control algorithm. The average error between the three output temperatures and the target temperature is 0.008. The average error of the three output temperatures is 0.007, which has good temperature uniformity. Fig. 9 shows the control voltage for the MIMO wafer RTP system.

Fig. 11. The control voltage of the MIMO wafer RTP System

5. conclusion

The previous studies have shown that establishment of physical model of RTP is difficult due to the strong coupling, nonlinearities, and multiple disturbances inherent in RTP. In this paper, a data-driven predictive control strategy is developed for RTP system through subspace method. In the pre-experiment stage, a PRBS excitation signal was designed to acquire the input and output data necessary for the subspace identification method. The step signal and square wave signal are designed as the input signal of RTP system respectively, and the relevant parameters of PRBS are obtained. The collected data is calculated to obtain the subspace predictor. The subspace predictor is used to replace the multi-step predictive output equation in classical MPC. Simulation results obtained from both first-order and third-order RTP systems validate the effectiveness and anti-disturbance ability of the data-driven predictive control algorithm across different model orders.

Although the effectiveness of the control algorithm for RTP system is verified by simulation, environmental factors need to be considered in practical engineering applications. The later research work will apply the control algorithm in practical engineering to verify the practical application ability of the algorithm.

DISCLAIMER (ARTIFICIAL INTELLIGENCE)

Authors hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Competing interests

Authors have declared that they have no known competing financial interests or non-financial interests or personal relationships that could have appeared to influence the work reported in this paper.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References

Fair, R. B. (Ed.). (2012). Rapid thermal processing: science and technology. Academic Press.

Singh, R. (2001). Rapid Thermal Processing for future semiconductor devices. Role of Rapid Thermal Processing in the development of disruptive and non-disruptive technologies for Semiconductor and Manufacturing in the 21st century, Ise-Shima, Japan.

Borisenko, V. E., & Hesketh, P. J. (2013). Rapid thermal processing of semiconductors. Springer Science & Business Media.

Campbell, S. A. (2008). Fabrication engineering at the micro-and nanoscale. (No Title).

T. Xiao and H. -X. Li, (2015). "Learning Control Approach for Thermal Regulation of Rapid Thermal Processing System," 2015 IEEE International Conference on Systems, Man, and Cybernetics, Hong Kong, China, pp. 334-340.

G. Yuan, Q. Hu, Y. Wang, C. Zhou and Z. Shi, (2024) "A MIMO Decoupling Algorithm for Rapid Heating Systems of Wafers," 2024 IEEE/CIC International Conference on Communications in China (ICCC), Hangzhou, China, pp. 1715-1720.

K. V. Ling, W. K. Ho, B. F. Wu, A. Lo and H. Yan, (2010) "Multiplexed MPC for Multizone Thermal Processing in Semiconductor Manufacturing," in IEEE Transactions on Control Systems Technology, vol. 18, no. 6, pp. 1371-1380.

Xiao T, Li X D, Wang S. (2018) Dominant-modes-based sliding-mode observer for estimation of temperature distribution in rapid thermal processing system[J]. IEEE Transactions on Industrial Informatics, 15(5): 2673-2681.

Morari, M., & Lee, J. H. (1999). Model predictive control: past, present and future. Computers & Chemical Engineering, 23(4-5), 667-682.

Dassau, E., Grosman, B., & Lewin, D. R. (2006). Modeling and temperature control of rapid thermal processing. Computers & chemical engineering, 30(4), 686-697.

Anderson, C. V., & Tamma, K. K. (2004). An overview of advances in heat conduction models and approaches for prediction of thermal conductivity in thin dielectric films. International Journal of Numerical Methods for Heat & Fluid Flow, 14(1), 12-65.

Berberich, J., Köhler, J., Müller, M. A., & Allgöwer, F. (2022). Linear tracking MPC for nonlinear systems—Part II: The data-driven case. IEEE Transactions on Automatic Control, 67(9), 4406–4421.

Campi, M. C., Lecchini, A., & Savaresi, S. M. (2002). Virtual reference feedback tuning: a direct method for the design of feedback controllers. Automatica, 38(8), 1337-1346.

Hou, Z., & Jin, S. (2010). A novel data-driven control approach for a class of discrete-time nonlinear systems. IEEE Transactions on Control Systems Technology, 19(6), 1549-1558.

Favoreel, W., De Moor, B., Van Overschee, P., & Gevers, M. (1999, June). Model-free subspace-based LQG-design. In Proceedings of the 1999 American control conference (Cat. No. 99CH36251) (Vol. 5, pp. 3372-3376). IEEE.

Zhao, J., Liu, J., Sun, H., Hu, Y., Sun, Y., & Xie, F. (2022). Data-driven predictive control with switched subspace matrices for an SCR system. IEEE Access, 10, 107616-107629.

Vajpayee, V., Becerra, V., Bausch, N., Banerjee, S., Deng, J., Shimjith, S. R., & Arul, A. J. (2020, June). Disturbance observer-based subspace predictive control of a pressurized water-type nuclear reactor. In 2020 7th International Conference on Control, Decision and Information Technologies (CoDIT) (Vol. 1, pp. 1012-1017). IEEE.

Carlet, P. G., Favato, A., Bolognani, S., & Dörfler, F. (2022). Data-driven continuous-set predictive current control for synchronous motor drives. IEEE Transactions on Power Electronics, 37(6), 6637-6646.

Ebert, J. L., de Roover, D., Porter, L. L., Lisiewicz, V. A., Ghosal, S., Kosut, R. L., & Emami-Naeini, A. (2004, June). Model-based control of rapid thermal processing for semiconductor wafers. In Proceedings of the 2004 American Control Conference (Vol. 5, pp. 3910-3921). IEEE.

Franklin, G. F., Powell, J. D., Emami-Naeini, A., & Powell, J. D. (2002). Feedback control of dynamic systems (Vol. 4). Upper Saddle River: Prentice hall.