Opinion Article

Application of Matter-element Theory for Systematic Safety Risk Grading on Expressways

Abstract: Traditional risk assessment methods for expressways cannot accurately evaluate in a dynamic manner and have certain limitations. The model established by the matter-element extension method can handle uncertainty and fuzziness dynamically and is a new type of mathematical model. This paper takes road alignment, subgrade and pavement, traffic safety facilities, and traffic environment as evaluation indicators, and uses the matter-element extension method to establish an evaluation model and calculate the results. The model effect is verified by the Xin-Yi Expressway. According to the calculation, the Xin-Yi Expressway is divided into 14 sections of low-risk level (Level I), 16 sections of relatively low-risk level (Level II), and 0 sections of general and high-risk levels. This paper finds that the safety risk classification model established by the matter-element extension method has certain feasibility and applicability and can dynamically evaluate the safety risk indicators of road sections based on traffic factors.

Keywords: Traffic safety; Matter-element extension model; Safety risk classification

0 Introduction

With the development of transportation industry, the number of highway safety accidents has been increasing year by year, causing many casualties and property losses. Therefore, it is very necessary to classify the safety of highways. By classifying the risk and safety of highways, the relationship between traffic factors and accidents can be analyzed, potential safety risks and key factors of highways can be identified, and the safety level of highways can be improved and enhanced. It can also lay a good foundation for the refined design of traffic safety facilities.

In recent years, scholars at home and abroad have conducted in-depth research on the safety classification of highways. Liu Tangzhi et al. [2] used the entropy weight method to conduct safety risk assessment on highway interchanges, dividing each section of the highway interchange into risk levels and providing corresponding measures and suggestions. He Yulong [3] conducted a study on traffic factors such as weather and road linearity, and used the theory of uncertain measures to evaluate the safety of a highway in Hunan Province, verifying the feasibility of the model theory. Cheng Wei et al. [4] constructed a tree augmented Bayesian network prediction model based on 1939 traffic accidents, and quantitatively analyzed the factors through data fusion to identify key causal factors; Li Xun [5] established a highway safety evaluation model based on safety level classification and combined it with six highways in Henan Province to determine risk factors. Then, the safety level of highways was classified using the rank sum ratio method. Xue Feng [6] proposed a tunnel safety risk assessment method based on Analytic Hierarchy Process and cloud model, with tunnels as the research object. Quyang et al. [7] constructed a tunnel conflict prediction model by setting parameters such as clear distance between tunnels, traffic volume, number of lanes, and distance between vehicles, and using negative binomial regression method to effectively evaluate the safety risk status between adjacent tunnels. Gedefaye Geremew [8] Assessing and predicting crash dynamics with and without road safety measures on the Dejen to Bahir Dar highway in Ethiopia，It was found that the safety facilities in different environments exhibited different behaviors after a vehicle collision. Irene Nandutu [9]and others employed drone technology and computer vision algorithms to identify and detect wildlife fences and related features, analyzed the impact of traffic safety facilities on wildlife crossing, and established an evaluation model.

In summary, most existing research on highway safety risk assessment uses traditional methods to explore the impact of various traffic factors on the safety level of highways. Without dynamic exploration of the relationship between traffic factors, there are certain limitations. The expressway is a complex system composed of "people, vehicles, roads, and environment", where various traffic factors interact with each other and have a certain degree of uncertainty and ambiguity. Among them, human factors are more subjective, while cars, roads, and environment are objective factors. Objective factors dominate the causes of traffic accidents, with road environment being the most critical factor. Therefore, this article establishes an evaluation index system based on factors such as road linearity, roadbed and pavement, traffic safety facilities, and traffic environment. Based on the Xinyi Expressway, the material element extension method is used to classify the risk and safety of the expressway. This method uses correlation functions to transform actual problems into formal problem models for scientific analysis. This method is both easy to operate and ensures the effectiveness of the results. This article first divides the Xinyi Expressway into five types of sections: straight, curved, curved slope, longitudinal slope, and tunnel. Based on the safety impact factors of the expressway, a safety risk classification index system is established from four levels: road alignment, roadbed and pavement, safety facilities, and traffic environment. Combined with standard specifications, the grading and quantification score standards of the indicators are determined. The entropy weight method and combination weight method are used to weight the indicators, and the matter element extension theory is used to establish a safety risk assessment model, realizing the quantification of micro section safety risks of the Xinyi Expressway.

1.Section division and risk assessment of expressway

1.1 Division of Expressway Sections

The methods for dividing highway sections can be divided into fixed length method and non fixed length method. The fixed length method is a method of manually defining the division length, which is simple to operate, but has the disadvantages of low accuracy and strong subjectivity; The non fixed length method uses a certain attribute of the road (traffic volume, horizontal and vertical alignment indicators, number of lanes, etc.) as the basis for division, and the length of the divided road sections is different, with strong characteristics and good persuasiveness.

This section of the text divides the road based on the "Safety Evaluation Specifications for Highway Projects" (JTG B05—2015). According to the indicators of horizontal and vertical alignment, the expressway is divided into five types of sections: straight, ascending slope, horizontal and curved, turning slope, and tunnel. The division criteria are shown in Figure 1.

Figure 1: Road segment division criteria

1.2 Establishment of safety risk indicator system

Based on the analysis of factors affecting highway safety and combined with existing research results on safety risk assessment, we select and discard objective safety influencing factors, and choose four aspects: road alignment, subgrade and pavement, safety facilities, and traffic environment to construct a safety risk assessment index system, as shown in Figure 2.

Figure 2: Indicator System for Highway Safety Risk Assessment

1.3 Quantification and value standard of evaluation indicators

To scientifically and reasonably assess the safety risks of individual sections of expressways, the indicator grading standards are established by referencing expressway design specifications, standards, and research findings. Each indicator is classified into four levels: low, lower, medium, and high, using interpolation or extrapolation methods. The lowest quantifiable score is 0, and the highest is 100. The grading and quantifiable scoring standards for expressway safety risk assessment indicators are shown in Table 1.

Table 1 Grading and Quantitative Scoring Criteria for Evaluation Indicators

|  | Description | Small | Smaller | Moderate | High risk | According |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0~25 | 25~50 | 50~75 | 75~100 |  |
| Radius of the horizontal curve | 80 | >1500 | 1500~700 | 700~400 | <400 | ①② |
|  | 100 | >3000 | 3000~1500 | 1500~700 | <700 |  |
|  | 120 | >4000 | 4000~2000 | 2000~700 | <700 |  |
| Longitudinal slope length | 80 | 200~900 | 900~1000 | 1000~1100 | >1100 | ①② |
|  | 100 | 250~800 | 800~900 | 900~1000 | >1000 |  |
|  | 120 | 300~700 | 700~800 | 800~900 | >900 |  |
| Longitudinal slope | 80 | <2.5 | 2.5~4 | 4~5 | >5 | ①② |
|  | 100 | <2 | 2~3 | 3~4 | >4 |  |
|  | 120 | <1.5 | 1.5~2 | 2~3 | >3 |  |
| Parking sight distance | —— | >240 | 240~180 | 180~120 | <120 | - ② |
| Bridge and Tunnel Safety | Compliance with specifications | 0~25 | 25~50 | 50~75 | 75~100 | - ②⑦ |
| Roadbed cross section | Has it changed | Excellent | Satisfactory | Average | Poor | - ⑦ |
| Roadside safety clearance zone | —— | 2.5~2 | 2~1.5 | 1.5~1 | <1 | - ⑦ |
| Ultra-high satisfaction rate | —— | 100~90 | 90~80 | 80~70 | 70~0 | - ⑦ |
| Traffic signs and markings | Level of facility perfection | 0~25 | 25~50 | 50~75 | 75~100 | ④⑤ |
| Crashworthiness level of guardrail | Crash resistance performance | 500~400 | 400~280 | 280~160 | 160~0 | - ④ |
| Sight guidance facility | Visual range guidance facility | 0~25 | 25~50 | 50~75 | 75~100 | - ⑥ |
| Other facilities | —— | 0~25 | 25~50 | 50~75 | 75~100 | - ⑥ |
| Speed difference | Speed difference between vehicle models | <15 | 15~30 | 30~45 | >45 | ②⑦ |
| Roadside environment | Roadside hazards | 0~25 | 25~50 | 50~75 | 75~100 | - ⑦ |
| Proportion of large vehicles | Proportion of large vehicles | 0~15 | 15~25 | 25~35 | >35 | ②⑦ |
| Climate conditions | Number of adverse weather days | <15 | 15~25 | 25~35 | >35 | ②⑦ |

Note: ① JTG D20-2017 Highway Route Design Specification; ② JTG B05-2015 Safety Evaluation Specification for Highway Projects; ③ JTG B05-01-2013 Safety Performance Evaluation Standards for Highway Guardrails; ④ JTG D81-2017 Design Specification for Highway Traffic Safety Facilities; ⑤ DB45/T 954-2020 Specification for the Installation of Traffic Signs and Markings on Highways; ⑥ Design Guidelines for Traffic Safety Facilities in Special Sections of CQJTGT D09-2021 Expressway; ⑦ JTG 5210-2018 Highway Technical Condition Evaluation Standards.

2. Construction of Security Risk Assessment Model

2.1 Introduction to Matter Element Extension Method

The matter element extension method is a method proposed by Chinese scholar Cai Wen to solve contradictory problems. Its principle is to represent actual problems with ordered triplets of "things（）, features（） and values（）", and use correlation functions to transform actual problems into formal problem models. Through the model, the problem-solving process is described to achieve the goal of resolving contradictions.

When using the matter element extension method for analysis work, it is first necessary to construct a multi index comprehensive evaluation index system based on the evaluation purpose, divide the level intervals of individual indicators, determine the classical domain, section domain, and the matter element matrix to be evaluated, convert the correlation function into the correlation degree of each indicator, establish a set of evaluation levels for each indicator, and then integrate the weights of the indicators of the evaluation object into the calculation to obtain the weighted correlation degree. According to the principle of maximum correlation degree, the level interval of the evaluation object is determined.

2.2 Determination of Evaluation Index Weights

To ensure the scientific validity of the evaluation results, the index weights are calculated using the entropy weight method. The calculation process of entropy weight method and combination weight is as follows.

(1) Entropy weight method

Entropy is a physical quantity derived from thermodynamics that characterizes the conversion of heat into work, which was later introduced into information theory as a measure of information uncertainty. The more evenly distributed the information in a system, the lower the entropy, the more chaotic the information distribution, and the higher the entropy. The entropy weight method can objectively determine the degree of impact of each indicator on system security, making the weight values of the indicators more objective. The specific calculation steps are as follows:

1) Score () factors by () experts to form a () - order original evaluation matrix.

2) Standardization of indicators:

3) Calculate the feature weight of the () th factor in the () th section:

4) Calculate the information entropy of the () th evaluation indicator:

In the formula:

5) Calculate entropy weight:

(2) Comprehensive weight calculation

() is the subjective weight of each indicator in the Analytic Hierarchy Process, () is the objective weight of each indicator in the Entropy Weight Method, and () is the comprehensive weight of each indicator. The calculation formula is as follows:

In the formula, () represents the subjective weight of each indicator in the Analytic Hierarchy Process; () is the objective weight of each indicator in the entropy weight method;

() is the coefficient of variation.

The comprehensive weight varies with the changes. Based on existing research results and combined with the actual situation of the evaluation index system in this article, we take:

2.3 Construction of Matter Element Extension Evaluation Model

(1) Determine the Matter Element Matrix

The research object of the material element analysis system for highway safety risks is the object (), the characteristics of each evaluation index (), and the evaluation values of each index as the quantity value (), forming an ordered triplet called the material element matrix, which is specifically represented as:

(2) Determine the classical domain () and () domain

1) Determine the Classic Domain ()

The classical domain for evaluating matter elements ()is the value range divided into four levels of evaluation indicators I, II, III, and IV,

2) Determine the domain

The domain of the evaluated object element () is the range of values for all evaluation indicator levels, expressed as follows:

(3) Determine the correlation function

The correlation function represents the degree of correlation between the value of the evaluated material element and a certain range of values for a certain level, allowing the evaluation index value to be () and the interval to be ()

The correlation function between the evaluation index characteristics () and the interval () of the object to be evaluated is as follows:

Where:

If  ，；Just .

(4) Determine correlation and evaluation level

1) Determine correlation degree

The correlation degree of each level of indicator () is calculated as follows:

2) Determination of comprehensive evaluation level

The magnitude of the calculated correlation value determines the level of the evaluated object element, and the formula is as follows:

3) Calculate the evaluation level characteristic value

Using the level evaluation method, calculate the characteristic value of the evaluation level for assessing the safety risk of the road section. The formula is as follows:

Table 2 : By using Matelab software for calculation, the corresponding relationship between risk level and characteristic values can be obtained

| Risk level | I | II | III | IV |
| --- | --- | --- | --- | --- |
| Risk characteristic | Small | Smaller | Moderate | High risk |
| value | <1.5 | 1.5~2.5 | 2.5~3.5 | >3.5 |

3 Engineering Case Applications

3.1 Overview of Xinyi Expressway

The Xinyi Expressway is one of the newly added planned routes in China's "Henan Province Expressway Network Adjustment Plan". The project starts from the Lianhuo Expressway, connects to the Luolu Expressway to the south, then connects to the Luoluan Expressway to the east, and intersects with the Erguang Expressway to the end. The total length of the route is 81.246 kilometers, and its geographical location is shown in the map.

Figure 3 Geographical location map of Xinyi Expressway

3.2 Division of evaluation sections for Xinyi Expressway

Based on the criteria for road section division and the quantitative standards for evaluation indicators in Table 1, the scores of evaluation indicators for each road section were compiled through analysis of the design data and field investigation of the Xinyi Expressway. The specific situation is shown in Table 3.

Table 3 Evaluation score of risk indicators for road sections

| Starting station number | End station number | Road alignmentB1 |  |  |  |  | Roadbed and pavementB2 |  |  | Safety facilitiesB3 |  |  |  | Traffic environment B4 |  |  |  | Road section type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | B11 | B12 | B13 | B14 | B15 | B21 | B22 | B23 | B31 | B32 | B33 | B34 | B41 | B42 | B43 | B44 |  |
| K0+000 | K3+622 | 33 | 29 | 11.3 | 20.1 | 37.5 | 37.5 | 20.8 | 18.3 | 12.5 | 75 | 12.5 | 12.5 | 20 | 12.5 | 36.3 | 38.5 | Sraight and flat |
| K3+622 | K4+655 | 67 | 25 | 71.3 | 24.7 | 37.5 | 37.5 | 22.1 | 17.1 | 37.5 | 50 | 37.5 | 12.5 | 26.1 | 37.5 | 36.3 | 38.5 | Curve |
| K4+655 | K5+218 | 67 | 81 | 71.3 | 30.4 | 12.5 | 12.5 | 20.3 | 21.9 | 37.5 | 50 | 37.5 | 12.5 | 20 | 37.5 | 36.3 | 38.5 | Curved slope |
| K5+218 | K5+455 | 50 | 72.5 | 61.3 | 20.5 | 12.5 | 12.5 | 19.5 | 18.3 | 12.5 | 25 | 37.5 | 12.5 | 20 | 62.5 | 36.3 | 38.5 | Longitudinal slope |
| K5+455 | K6+190 | 50 | 13 | 11.3 | 35.1 | 12.5 | 12.5 | 20.7 | 17.1 | 12.5 | 75 | 37.5 | 12.5 | 20 | 37.5 | 36.3 | 38.5 | Longitudinal slope |
| K6+190 | K11+900 | 35 | 86 | 61.1 | 38.2 | 12.5 | 62.5 | 23.1 | 24.1 | 12.5 | 50 | 12.5 | 12.5 | 20 | 12.5 | 40.6 | 38.5 | Sraight and flat |
| K11+900 | K12+800 | 50 | 50 | 42.8 | 19.5 | 12.5 | 37.5 | 23.4 | 20.3 | 12.5 | 50 | 37.5 | 12.5 | 25.9 | 37.5 | 40.6 | 38.5 | Longitudinal slope |
| K12+800 | K13+500 | 50 | 20 | 61.8 | 16.4 | 12.5 | 12.5 | 19.8 | 25 | 12.5 | 50 | 12.5 | 12.5 | 20 | 12.5 | 40.6 | 38.5 | Sraight and flat |
| K13+500 | K14+300 | 10 | 50 | 14.3 | 46.9 | 37.5 | 37.5 | 23.5 | 23.1 | 12.5 | 25 | 37.5 | 12.5 | 25.2 | 37.5 | 40.6 | 38.5 | Longitudinal slope |
| K14+300 | K15+473 | 46 | 75 | 65.6 | 43 | 37.5 | 37.5 | 23.2 | 23.1 | 12.5 | 50 | 37.5 | 12.5 | 25.3 | 12.5 | 40.6 | 38.5 | Sraight and flat |
| K15+473 | K21+800 | 37 | 77 | 63.4 | 41.4 | 37.5 | 87.5 | 21.4 | 19.7 | 12.5 | 50 | 37.5 | 12.5 | 20 | 12.5 | 38.4 | 38.5 | Sraight and flat |
| K21+800 | K22+640 | 27 | 23 | 13.4 | 44.9 | 37.5 | 12.5 | 20.8 | 22.1 | 12.5 | 50 | 12.5 | 12.5 | 25.9 | 12.5 | 38.4 | 38.5 | Longitudinal slope |
| K22+640 | K25+500 | 59 | 87 | 65.6 | 33.4 | 37.5 | 62.5 | 20.6 | 21.7 | 12.5 | 50 | 37.5 | 12.5 | 20 | 12.5 | 38.4 | 38.5 | Sraight and flat |
| K25+500 | K26+000 | 53 | 32 | 42.3 | 38.4 | 12.5 | 12.5 | 24.1 | 20.6 | 12.5 | 50 | 12.5 | 12.5 | 23.0 | 37.5 | 38.4 | 38.5 | Longitudinal slope |
| K26+000 | K27+680 | 50 | 83 | 68.1 | 26.7 | 12.5 | 37.5 | 21.9 | 24.4 | 12.5 | 50 | 12.5 | 12.5 | 20 | 12.5 | 38.4 | 38.5 | Sraight and flat |
| K27+680 | K28+500 | 10 | 37.5 | 46.5 | 36.9 | 12.5 | 62.5 | 12.7 | 18.9 | 12.5 | 50 | 12.5 | 12.5 | 25.9 | 12.5 | 36.3 | 38.5 | Longitudinal slope |
| K28+500 | K31+882 | 10 | 41 | 68.1 | 41.1 | 37.5 | 62.5 | 22.8 | 20.2 | 12.5 | 25 | 37.5 | 12.5 | 20 | 12.5 | 36.3 | 38.5 | Sraight and flat |
| K31+882 | K35+460 | 10 | 82 | 27.5 | 24.9 | 37.5 | 12.5 | 18.4 | 20.2 | 62.5 | 50 | 12.5 | 12.5 | 20 | 62.5 | 33.8 | 38.5 | Tunnel |
| K35+460 | K36+131 | 15 | 87 | 27.5 | 16.2 | 62.5 | 37.5 | 19.4 | 18.2 | 37.5 | 50 | 12.5 | 37.5 | 20 | 12.5 | 33.8 | 38.5 | Sraight and flat |
| K36+131 | K36+461 | 27 | 19 | 30.8 | 23.3 | 37.5 | 37.5 | 23 | 18.2 | 62.5 | 50 | 12.5 | 12.5 | 24.1 | 37.5 | 33.8 | 38.5 | Tunnel |
| K36+461 | K38+962 | 29 | 86 | 28.8 | 35.7 | 37.5 | 37.5 | 21.8 | 17.7 | 62.5 | 50 | 12.5 | 12.5 | 20 | 37.5 | 33.8 | 38.5 | Tunnel |
| K38+962 | K41+262 | 21 | 77 | 20.8 | 44.8 | 62.5 | 37.5 | 22.1 | 20.4 | 12.5 | 50 | 12.5 | 12.5 | 22.0 | 12.5 | 33.8 | 38.5 | Sraight and flat |
| K41+262 | K57+200 | 10 | 81 | 65.3 | 34.3 | 37.5 | 87.5 | 22 | 24.8 | 12.5 | 25 | 12.5 | 12.5 | 20 | 12.5 | 32.0 | 38.5 | Sraight and flat |
| K57+200 | K57+900 | 10 | 27 | 47.7 | 48.6 | 12.5 | 12.5 | 20.1 | 17.6 | 12.5 | 50 | 12.5 | 12.5 | 24.6 | 37.5 | 36.1 | 38.5 | Longitudinal slope |
| K57+900 | K70+860 | 39 | 89 | 65.3 | 30.1 | 37.5 | 87.5 | 20.5 | 24.6 | 12.5 | 50 | 37.5 | 12.5 | 20 | 12.5 | 36.1 | 38.5 | Sraight and flat |
| K70+860 | K71+800 | 10 | 50 | 48.7 | 30 | 37.5 | 37.5 | 17.8 | 22 | 12.5 | 25 | 12.5 | 12.5 | 23.0 | 62.5 | 31.6 | 38.5 | Longitudinal slope |
| K71+800 | K72+900 | 10 | 24 | 63.8 | 32.3 | 37.5 | 37.5 | 22.5 | 23.2 | 12.5 | 50 | 37.5 | 12.5 | 20 | 12.5 | 31.6 | 38.5 | Sraight and flat |
| K72+900 | K74+575 | 10 | 23 | 55 | 16.2 | 37.5 | 37.5 | 17.3 | 18.6 | 12.5 | 50 | 37.5 | 12.5 | 29.1 | 62.5 | 31.6 | 38.5 | Longitudinal slope |
| K74+575 | K76+100 | 10 | 24 | 63.8 | 19.9 | 12.5 | 12.5 | 17.8 | 20.1 | 12.5 | 50 | 37.5 | 12.5 | 20 | 62.5 | 31.6 | 38.5 | Longitudinal slope |
| K76+100 | K84+100 | 50 | 19 | 55 | 18.1 | 12.5 | 62.5 | 21.3 | 17.6 | 12.5 | 50 | 37.5 | 12.5 | 20 | 12.5 | 31.6 | 38.5 | Sraight and flat |

3.3 Weight Calculation of Evaluation Indicators

The entropy weight method constructs an evaluation matrix based on the ratings of five industry experts for each evaluation indicator. The weight values () of the indicators are calculated using equations 1-4. Then, the Analytic Hierarchy Process and entropy weight method are combined to obtain the comprehensive weight () of the evaluation indicators using equation 5. The detailed weight values of each indicator are shown in the table.

Table 4 Weight Values of Safety Risk Assessment Indicators

| Criterion layer | AHP | Indicator level | AHP | AHP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Wi |  | Wi | Combined | Wj |  |
| B1 | 0.472 | B11 | 0.307 | 0.145 | 0.034 | 0.090 |
|  |  | B12 | 0.232 | 0.109 | 0.071 | 0.090 |
|  |  | B13 | 0.240 | 0.113 | 0.079 | 0.096 |
|  |  | B14 | 0.110 | 0.052 | 0.052 | 0.052 |
|  |  | B15 | 0.112 | 0.053 | 0.096 | 0.075 |
| B2 | 0.108 | B21 | 0.311 | 0.033 | 0.043 | 0.038 |
|  |  | B22 | 0.493 | 0.053 | 0.042 | 0.048 |
|  |  | B23 | 0.196 | 0.021 | 0.037 | 0.029 |
| B3 | 0.164 | B31 | 0.293 | 0.048 | 0.089 | 0.038 |
|  |  | B32 | 0.416 | 0.068 | 0.085 | 0.048 |
|  |  | B33 | 0.185 | 0.030 | 0.060 | 0.029 |
|  |  | B34 | 0.107 | 0.018 | 0.047 | 0.033 |
| B4 | 0.256 | B41 | 0.300 | 0.077 | 0.098 | 0.087 |
|  |  | B42 | 0.389 | 0.100 | 0.057 | 0.079 |
|  |  | B43 | 0.188 | 0.048 | 0.058 | 0.053 |
|  |  | B44 | 0.124 | 0.032 | 0.052 | 0.042 |

3.3 Weight Calculation of Evaluation Indicators

Refer to section 1.3 for the quantitative value standards of evaluation indicators, and use the numerical values of each indicator to establish a material element matrix for the safety risk assessment of the Xinyi Expressway. The classical domain is:

The domain is:

According to the weight of each evaluation index calculated in Table 4, the correlation degree of the divided road sections is calculated, and the evaluation level of each road section is obtained based on the risk grading standard defined in Table 2. The risk assessment results of each road section of Xinyi Expressway are shown in Table 5

Table 5 Safety Risk Assessment Results of Various Sections of Xinyi Expressway

| Starting station number | End station number | I | II | III | IV |  | risk level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Small | Smaller | Moderate | High risk |  |  |
| K0+00 | K3+622 | 0.068 | -0.104 | -0.493 | -0.636 | 1.494 | I |
| K3+622 | K4+655 | -0.206 | 0.096 | -0.240 | -0.489 | 1.970 | Ⅱ |
| K4+655 | K5+218 | -0.179 | -0.079 | -0.277 | -0.447 | 2.179 | Ⅱ |
| K5+218 | K5+455 | 0.081 | -0.150 | -0.471 | -0.622 | 1.583 | Ⅱ |
| K5+455 | K6+190 | 0.081 | -0.150 | -0.471 | -0.622 | 1.583 | Ⅱ |
| K6+190 | K11+900 | -0.051 | -0.168 | -0.354 | -0.504 | 1.477 | I |
| K11+900 | K12+800 | -0.075 | 0.043 | -0.322 | -0.548 | 1.809 | Ⅱ |
| K12+800 | K13+500 | 0.104 | -0.215 | -0.412 | -0.631 | 1.423 | I |
| K13+500 | K14+300 | 0.001 | 0.014 | -0.420 | -0.614 | 1.407 | I |
| K14+300 | K15+473 | -0.151 | -0.023 | -0.256 | -0.487 | 1.698 | Ⅱ |
| K15+473 | K21+800 | -0.139 | -0.159 | -0.298 | -0.466 | 1.425 | I |
| K21+800 | K22+640 | 0.101 | -0.092 | -0.489 | -0.659 | 1.606 | Ⅱ |
| K22+640 | K25+500 | -0.157 | -0.121 | -0.257 | -0.441 | 1.774 | Ⅱ |
| K25+500 | K26+000 | -0.018 | 0.014 | -0.373 | -0.588 | 1.544 | Ⅱ |
| K26+000 | K27+680 | -0.044 | -0.202 | -0.376 | -0.509 | 1.472 | I |
| K27+680 | K28+500 | 0.080 | -0.114 | -0.440 | -0.636 | 1.438 | I |
| K28+500 | K31+007 | -0.014 | -0.061 | -0.375 | -0.595 | 1.493 | I |
| K31+007 | K35+460 | -0.087 | -0.157 | -0.324 | -0.514 | 1.756 | Ⅱ |
| K35+460 | K36+131 | -0.050 | -0.120 | -0.401 | -0.517 | 1.642 | Ⅱ |
| K35+460 | K36+461 | -0.072 | 0.067 | -0.349 | -0.583 | 1.801 | Ⅱ |
| K36+461 | K39+090 | -0.166 | 0.032 | -0.329 | -0.474 | 1.830 | Ⅱ |
| K39+090 | K41+262 | -0.008 | -0.164 | -0.381 | -0.564 | 1.473 | I |
| K41+262 | K57+200 | -0.029 | -0.213 | -0.439 | -0.530 | 1.448 | I |
| K57+200 | K57+900 | 0.062 | -0.101 | -0.448 | -0.632 | 1.338 | I |
| K57+900 | K70+860 | -0.154 | -0.092 | -0.357 | -0.437 | 1.514 | Ⅱ |
| K70+860 | K71+800 | -0.023 | -0.066 | -0.355 | -0.589 | 1.620 | Ⅱ |
| K71+800 | K72+900 | 0.010 | -0.071 | -0.391 | -0.614 | 1.311 | I |
| K72+900 | K74+575 | -0.039 | -0.055 | -0.327 | -0.581 | 1.472 | I |
| K74+575 | K76+100 | 0.048 | -0.202 | -0.368 | -0.618 | 1.687 | Ⅱ |
| K76+100 | K84+100 | 0.052 | -0.160 | -0.380 | -0.607 | 1.375 | I |

4 Conclusion

This article proposes a safety risk classification model using the matter element extension method to address the dynamic inaccuracy of the highway safety risk classification method. By establishing a highway safety risk assessment index system B based on road linear B1, roadbed and pavement B2, traffic safety facilities B3, and traffic environment B4, the material element extension method is used to calculate the highway safety risk assessment standards. Based on the case study of Xinyi Expressway, a traffic safety configuration table for each section is provided. Using the matter element extension method as a model, the safety risk levels of each section of Xinyi Expressway are calculated, which are divided into 14 low-risk (Level I) sections, 16 low-risk (Level II) sections, and 0 general risk and high-risk levels.

# COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

References

- The Ministry of Transport of the People's Republic of China Statistical Bulletin on the Development of the Transportation Industry in 2022 [R]. Beijing: Ministry of Transport of the People's Republic of China, 2023

- Liu Tangzhi, Bai Jingrong, Nie Shulei, etc Research on Safety Evaluation of Interchange Exit Ramp Based on Entropy Weight Matter Element [J]. Journal of Chongqing Jiaotong University (Natural Science Edition), 2019, 38 (10): 95-101

- He Yulong, Chen Bingshuo, Gao Pengwang, etc Highway Traffic Safety Evaluation Based on Unascertained Measure Theory [J]. Journal of Chongqing Jiaotong University (Natural Science Edition), 2021, 40 (2): 8-12

- Cheng Wei, Ma Mingwei, Zhang Xiaolong Prediction of Severity and Cause Analysis of Highway Traffic Accidents Based on Bayesian Networks [J]. Journal of Chongqing Jiaotong University (Natural Science Edition), 2023, 42 (7): 87-95

- Li Xun Highway safety evaluation and risk identification based on safety level classification [D]. Xi'an: Chang'an University, 2020

- Xue Feng, Hu Ping, Huang Yu Highway Tunnel Traffic Safety Evaluation Based on Composite Model [J]. Highway Traffic Technology, 2021, 38 (07): 85-95Ouyang P ,Wu J ,Xu C , et al. Traffic safety analysis of inter-tunnel weaving section with conflict prediction models[J]. Journal of Transportation Safety & Security,2020,14(4).

- Pengying Ouyang,Jiaming Wu,Chengcheng Xu,Lu Bai & Xuefeng Li.(2020).Traffic safety analysis of inter-tunnel weaving section with conflict prediction models.Journal of Transportation Safety & Security,14(4),1-25.

- Geremew G .Assessing and predicting crash dynamics with and without road safety measures on the Dejen to Bahir Dar highway in Ethiopia[J].Scientific Reports,2025,15(1):32055-32055.

- Irene Nandutu, et al."Enhancing highway security and wildlife safety: Mitigating wildlife–vehicle collisions with deep learning and drone technology."Journal of Intelligent Systems 34.1(2025):20240219-20240219.