ASSESMENT OF CANCER RISK IN ORGANS OF THE  ABDOMINOPELVIC REGION FROM COMPUTED TOMOGRAPHY (CT) SCAN RADIATION

.

ABSTRACT

Aims The research aims to determine and analyze the effective dose value received by the patient's abdominopelvic from CT-scan radiation, determine the effective dose value of several organs at risk around the abdominopelvic organ, conduct a cancer potential test on the abdominopelvic organ and the organs at risk around the region Place and Duration of Study: Radiology Installation of Prof. Dr. dr. I Gusti Ngurah Hospital, Jl. Diponegonegoro, Dauh Puri Klod, Kec. West Denpasar., Denpasar City, Bali 80113 between (Juanuary 2023 until January 2024) Methodology: This research used Canon Brand CT-Scan, Type: XXG-012A, Serial No.: 8AB1862432, surveymeter as radiation dose measurement, and alcohol as a standard washing tool. The variables in this study are large dose of X-ray radiation in the laboratory and Radiology Clinic of STIKes Widya Cipta Husada Malang. The radiation dose was measured at each measurement point. Results: The potential for cancer is carried out by analyzed the amount of Dose Length Product (DLP) produced by Treatment Planning System getting a maximum effective dose of 5,230 mGy. Cm for female patients and 5,880 mGy. Cm for men, while Bapeten issued a threshold provision of 13,275 mGy. Cm. The potential is quite small. This is also proven by the t-test on the effective dose of DLP obtained by 2.756 results stating statistics (t count <t table). The large dose received by workers in the Laboratory and Radiology Clinic is 0.009 0.009 mSv / year. Conclusion: The potential for cancer is statistically small or the results of the effective dose analysis do not exceed the BAPETEN threshold. The large dose received by workers in the Laboratory and Radiology Clinic is still within the safe limit and in accordance with BAPETEN provisions. The results of the study of the potential for cancer in the abdopelvic organs and critical organs around it the TPS results are still small but it is recommended to conduct tests on other organs because other organs have different weight factors such as the thorax, head organs or also the cervix.

Keywords: Ct-Scan, Radiology, DLP, Cancer

1. INTRODUCTION

The development of radiation utilization in the medical world is very rapid and the discovery of increasingly sophisticated and modern radiation devices allows the use of X-ray radiation for medical diagnosis and therapy to be very extensive and developed. Wilhelm Conrad Roentgen's discovery of X-ray radiation in 1895 became the starting point for the use of X-rays in the medical world. This discovery was a revolution in the world of medicine, one of these uses is because X-rays can produce images or images of body organs that make it very easy to diagnose abnormalities in human organs (Satwika, 2020). In hospitals, many now have X-ray instruments, one of which is Computed Tomography Scanning (CT. Scan.). CT. Scan is an instrument that can display transparent images of a human organ object. This can medically provide important information about the internal organs of the human body without surgery (Ibrahim et al., 2018). The CT Scan scanning process is an organ that will be known for its abnormalities is exposed to radiation from a source on the instrument. The process of exposure or radiation to the organ to be known is a neighboring organ or other organ that is close to the surrounding area. The organ is called a risk organ or Organ At Risk (OAR). The value or amount of radiation exposed to the main organ has a comparable amount of radiation energy released by the CT Scan. The amount of radiation is called the radiation dose.

Each organ or anatomical part of the human body has a level of sensitivity to radiation exposure. So the radiation dose must be monitored and become something that is very important and routinely studied. The level of organ sensitivity to radiation is more specifically called the effective dose.

DRLs are an optimization technique in radiology that ensure patients get the right radiation doses without affecting the image quality (S.M. Kabeer et al,  2024). The special authority given to issue effective dose values ​​in Indonesia is BAPETEN through the regulation of the Head of the Nuclear Energy Regulatory Agency (BAPETEN) concerning the Indonesian Diagnostic Guidelines or Indonesian Diagnostic Reference Level (I-DRL). The effective dose was introduced by the International Commission of Radiological Protection (ICRP) to provide a summation of radiation doses to tissues and organs for radiological protection. For a non-uniform irradiation, the effective dose is designed to provide an estimate of the corresponding uniform whole-body dose that would result in the same stochastic detriment (Brenner & Huda, 2008). This effective dose, if it exceeds the set threshold, will cause various adverse effects on the human body such as damage to body tissue cells or genetic damage in the form of mutations in reproductive cells (Yogantara, 2020).

This fact is urgent to be studied, one of which is the abdo pelvis organs and the risk organs around them. Based on the results of the study, the abdo pelvis organs are organs that are often the subject of CT-Scan exposure. The abdo pelvis is a cavity in the human body that includes the abdomen (stomach) to the pelvis (hips). This part often experiences abnormalities due to the influence of food composition or excessive movement. In addition, if a scan is performed to see the abnormality, many organs in the abdominal cavity are affected. It is very necessary and urgent to conduct research on the potential for cancer testing of the abdominal organs and several of their critical organs and critical organs.

2. material and methods

This research was conducted at the Radiology Installation of Prof. Dr. dr. I Gusti Ngurah Hospital, Jl. Diponegonegoro, Dauh Puri Klod, Kec. West Denpasar., Denpasar City, Bali 80113. The tools and materials used were Canon Brand CT-Scan, Type: XXG-012A, Serial No.: 8AB1862432 and alcohol as a standard washing tool. The variables used in this study were the independent variables in the form of the abdopelvis and the risk area around the abdopelvis, the dependent variables in the form of CTDIvol and DLP values ​​which would be analyzed to obtain their respective effective doses, and the control variables were a maximum voltage of 35 kV/ and a current of 600 mA.

Figure 1. Research flow diagram

Radiation dose measurement using a surveymeter. A room plan is needed in this study to facilitate the measurement of the dose coming out of the X-ray machine. The laboratory plan can be seen in Figure 1. The variables observed in this study are the large dose of X-ray radiation in the laboratory and Radiology Clinic of STIKes Widya Cipta Husada Malang. The radiation dose will be measured at each measurement point.

3. results and discussion

From the research that has been conducted using a CT-Scan machine, the CT-Scan scan data in the form of CTDIvol and DLP dose values ​​will be converted into effective dose values ​​by multiplying the data by the weighting factor issued by the authority body. These results will be tested with a comparison test so that the percentage of cancer potential that will be obtained as a recommendation is obtained.

Table 1. Results of the Treatment Planning System (TPS) and CT exposure

| NO | FEMALE |  |  |
| --- | --- | --- | --- |
|  | Initial | CTDIvol (mGy) | DLP (mGy. Cm) |
| 1 | Wn | 7,3 | 340,9 |
| 2 | Hn | 7,2 | 294,9 |
| 3 | Rn | 7,2 | 300 |
| 4 | Ga | 7,2 | 280,6 |
| 5 | Rn | 7,2 | 302,1 |
| 6 | Pn | 7,2 | 305,7 |
| 7 | Iw | 7,2 | 305,7 |
| 8 | Sn | 7,2 | 320 |
| 9 | Ss | 7,2 | 317,1 |
| 10 | Mk | 7,2 | 317,8 |
| 11 | Sd | 7,2 | 322,8 |
| 12 | Ta | 7,2 | 321,5 |
| 13 | Pu | 7,2 | 345,2 |
| 14 | Dn | 7,2 | 348,8 |
| 15 | Wn | 7,2 | 330,4 |
| 16 | Pn | 7,2 | 340,9 |
| 17 | Ks | 7,2 | 330,1 |
| 18 | Yn | 7,2 | 329,2 |
| 19 | Pj | 7,2 | 327 |
| 20 | Pu | 7,2 | 345,2 |
| 21 | Sk | 7,2 | 317,1 |
| 22 | Km | 7,2 | 317,8 |
| 23 | Si | 7,2 | 322,8 |
| 24 | It | 7,2 | 321,5 |
| 25 | Pm | 7,2 | 345,2 |
| 26 | Dk | 7,2 | 348,8 |
| 27 | Wk | 7,2 | 330,4 |
| 28 | Wi | 7,2 | 340,9 |
| 29 | Hk | 7,2 | 294,9 |
| 30 | Dr | 7,2 | 300 |
| Average |  | 7,203 | 322,193 |

The results of the TPS data above were analyzed for homogeneity (evenness) coming out of the CT machine. The scan used was tested for homogeneity against the CT X-ray output. The results of the homogeneity test can be seen in table 2 and from the  results of the study, data was obtained as in Table 2.

Table 2. Results of homogeneity tests for male and female patients

| Gender | Skewness |  |  | Kurtosis |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Statistic | Error Std. | Ratio | Statistic | Error Std. | Ratio |
| Female | -.353 | .427 | .827 | -.557 | .883 | .631 |
| Male | 1.081 | .427 | 2.531 | 2.642 | .833 | 3.171 |

CT scan data. The scan results in the form of DLP values ​​from TPS results are analyzed for the potential for abdovelvis organ cancer by calculating the patient's effective dose. The cancer risk in each gender is taken from the effective dose at the minimum, maximum and average DLP. The cancer risk can be seen in table 3 below.

Table 3. Effective doses for each gender based on DLP

| NO | FEMALE |  |  | MALE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Initial | CTDIvol (mGy) | DLP (mGy. Cm) | Initial | CTDIvol (mGy) | DLP (mGy.cm) |
| 1 | Wn | 7,3 | 340,9 | Si | 7,2 | 306,4 |
| 2 | Hn | 7,2 | 294,9 | Ri | 7,2 | 310,7 |
| 3 | Rn | 7,2 | 300 | Wg | 7,2 | 345,2 |
| 4 | Ga | 7,2 | 280,6 | Ig | 7,2 | 334,4 |
| 5 | Rn | 7,2 | 302,1 | Sn | 7,2 | 345,2 |
| 6 | Pn | 7,2 | 305,7 | In | 7,2 | 336,6 |
| 7 | Iw | 7,2 | 305,7 | Kg | 7,2 | 347,3 |
| 8 | Sn | 7,2 | 320 | Km | 7,2 | 347,3 |
| 9 | Ss | 7,2 | 317,1 | Mn | 7,2 | 351,5 |
| 10 | Mk | 7,2 | 317,8 | Dn | 7,2 | 348,8 |
| 11 | Sd | 7,2 | 322,8 | Jk | 7,2 | 351,5 |
| 12 | Ta | 7,2 | 321,5 | Sn | 7,2 | 358,1 |
| 13 | Pu | 7,2 | 345,2 | In | 7,2 | 358,1 |
| 14 | Dn | 7,2 | 348,8 | Si | 7,2 | 359,5 |
| 15 | Wn | 7,2 | 330,4 | Ji | 7,2 | 449,2 |
| 16 | Pn | 7,2 | 340,9 | Hn | 7,2 | 391,8 |
| 17 | Ks | 7,2 | 330,1 | Na | 7,2 | 391,8 |
| 18 | Yn | 7,2 | 329,2 | Si | 7,2 | 388 |
| 19 | Pj | 7,2 | 327 | Sk | 7,2 | 359,5 |
| 20 | Pu | 7,2 | 345,2 | Sj | 7,2 | 370,3 |
| 21 | Sk | 7,2 | 317,1 | St | 7,2 | 306,4 |
| 22 | Km | 7,2 | 317,8 | Nh | 7,2 | 391,8 |
| 23 | Si | 7,2 | 322,8 | Wk | 7,2 | 345,2 |
| 24 | It | 7,2 | 321,5 | Ic | 7,2 | 334,4 |
| 25 | Pm | 7,2 | 345,2 | Sm | 7,2 | 345,2 |
| 26 | Dk | 7,2 | 348,8 | Ni | 7,2 | 336,6 |
| 27 | Wk | 7,2 | 330,4 | Kj | 7,2 | 347,3 |
| 28 | Wi | 7,2 | 340,9 | Ss | 7,2 | 306,4 |
| 29 | Hk | 7,2 | 294,9 | Rk | 7,2 | 310,7 |
| 30 | Dr | 7,2 | 300 | Gm | 7,2 | 345,2 |
| Average |  | 7,203 | 322,193 | Average | 7,2 | 352,266 |

The cancer risk in the abdominopelvic organs is known by reviewing the provisions issued by the competent body for this purpose, namely BAPETEN (BAPETEN No. 121/K/V, 2021). Calculation of effective dose without regard to gender: Equation 2.1 DLP is 885 mGy.cm, conversion coefficient k is 0.015 mSv/mGy.cm. So the effective dose (E) recommended by BAPETEN is as follows:

E = DLP × k

E =885 mGy.cm × 0.015 mSv⁄(mGy.cm)

E =13.275 mSv

Table 4. Potential for abdovelvis cancer based on effective, maximum, minimum and average doses.

| Type/Unit | Effective dose (E) for each gender |  |
| --- | --- | --- |
|  | Female (mGy.cm) | Male (mGy.cm) |
| E maximum | 5.230 | 5.880 |
| E minimum | 4.210 | 4.600 |
| E average | 4.830 | 5.283 |
| E bapeten | 13.275 | 13.275 |

Figure 2. Cancer Potential based on Effective Dose of DLP from TPS results

Cancer risk is also analyzed using one sample t-test statistics. The t-test is conducted to determine whether the effective dose value of the patient in the abdomen pelvis is still permitted or in accordance with the threshold value set by BAPETEN. The t-test is shown in table 5.

Table 5. Results of the t-test for effective dose values ​​in the abdopelvis section

| Gender | n | df (n-1) | t-value | t-table |
| --- | --- | --- | --- | --- |
| Male | 30 | 29 | -169.778 | 2,756 |
| Female | 30 | 29 | -95.943 | 2,756 |

The t-test results show a t-table value of 2.756, this result is greater than both t-count values, both patient genders or with statistical writing (t-count <t-table). So the potential for cancer is statistically small or the results of the effective dose analysis do not exceed the BAPETEN threshold. Critical organs in the abdominopelvic area include the liver, kidneys and gonads. The potential for cancer in this OAR organ can be done by testing the effective dose value calculated from the CTDIvol obtained from the TPS. The effective dose value of the OAR is converted into millisieverts (mSv) where 1 mSv = 1 mGy. The results of the OAR organ analysis are as in Table 6.

Table 6. Critical Organ Cancer Potential Test (OAR) around Abdopelvis

| Critical Organ | Average |  |
| --- | --- | --- |
|  | Male | Female |
| Liver | 0.2882 | 0.2882 |
| Kidney | 0.8646 | 0.8646 |
| Gonad | 0.5764 | 0.5764 |

From the table above it can be translated as Figure 3 below

Figure 3. Potential for Critical Organ Cancer (OAR) around the abdopelvis

The potential for cancer is seen in the kidney organ in both female and male patients at 0.8646 mSv. This result, although still below the BAPETEN regulation, needs to be a concern for monitoring the Quality Control and Quality Insurance of this aircraft.

4. Conclusion

Based on the research that has been done, the results of the research have provided an overview of the need for an effective dose test on the abdovelvis organ from CT Scan exposure. There are several things that can be concluded that the potential for cancer is carried out by analyzing the amount of Dose Length Product (DLP) produced by TPS getting a maximum effective dose of 5,230 mGy. Cm for female patients and 5,880 mGy. Cm for men, while Bapeten issued a threshold provision of 13,275 mGy. Cm. The potential is quite small. This is also proven by the t-test on the effective dose of DLP obtained by 2.756 results stating statistics (t count <t table). So the potential for cancer is statistically small or the results of the effective dose analysis do not exceed the BAPETEN threshold. The large dose received by workers in the Laboratory and Radiology Clinic is 0.009 0.009 mSv / year and is still within the safe limit and in accordance with BAPETEN provisions, besides the potential for cancer in the OAR organ is obtained maximum in the kidney organ in both female and male patient types of 0.8646 mSv, and it is better in this study to clarify the calculation of the room conditions of the Laboratory and Radiology Clinic so that it is truly in accordance with the standards. The results of the study of the potential for cancer in the abdopelvic organs and critical organs around it the TPS results are still small but it is recommended to conduct tests on other organs because other organs have different weight factors such as the thorax, head organs or also the cervix. Testing at the dose rate is very necessary to maintain the QI and QC of the aircraft used.

Ethical Approval:

As per international standards or university standards written ethical approval has been collected and preserved by the author(s).

REFERENCES

BAPETEN. (2021). Decree No. 121/K/V on the Determination of Indonesian Diagnostic Reference Levels for X-ray CT Scan and General Radiography Modalities. Jakarta: BAPETEN. https://jdih.bapeten.go.id/unggah/dokumen/peraturan/1056-full.pdf

Bushberg, J.T., Seibert, J.A., Leidholt, E.M., & Boone, J.M. (2012). The Essential Physics of Medical Imaging. Baltimore: Williams and Wilkins.

Fitriyani, F., Suharyana, Muhtarom, & Riyatun. (2017). Effect of Focus Film Distance on Radiation Beam Accuracy in X-Ray Simulator Device at Radiotherapy Installation of Dr. Moewardi Hospital Surakarta. Sebelas Maret University, Surakarta.

Ibrahim, A.A., Bualkar, A., & Halmar, H. (2018). Estimasi Dosis Efektif Pasien Bagian Abdomen dari Hasil Pemeriksaan CT-Scan Merek Siemens SOMATOM [Estimation of Effective Dose to Abdomen Patients from Siemens SOMATOM CT Scan Examination]. POSITRON, 8(2), 39–42. https://jurnal.untan.ac.id/index.php/jpositron/article/view/25213

Kristinayanti, N.K.D., Iffah, M., Jeniyanthi, N.P.R., & Astina, K.Y. (2019). Analysis of Scan Length Effect on Dose Length Product Value in Multislice CT Scan at Mangusada Hospital Badung. IMEJING Journal, 3(1), 3.

Lestari, D.P. (2017). Kondisi Faktor Ekpose ( kV, mA.s ) pada Radiografi Setelah Di Lakukan Adjusment Kalibrasi Pesawat Mobile. [Exposure Factor Conditions (kV, mA.s) on Radiography After Mobile Aircraft Calibration Adjustment] (Undergraduate Thesis). University of North Sumatra, Medan. https://repositori.usu.ac.id/handle/123456789/3171

Nanlohy, M.B. (2011). Faktor koreksi geometri dalam pengukuran dosis pada phantom dengan menggunakan metode CTDI di udara dan CTDI pada phantom. [Geometry Correction Factors in Dose Measurement on Phantom Using CTDI Method in Air and on Phantom] (Undergraduate Thesis). University of Indonesia, Depok. https://lib.ui.ac.id/detail?id=20289512

Nurhayati, A.Y., Nariswari, N.N., Rahayuningsih, B., & Hariadi, Y.C. (2019). Analisis Variasi Faktor Eksposi dan Ketebalan Irisan Terhadap CTDI dan Kualitas Citra Pada Computed Tomography Scan. [Analysis of Exposure Factor Variation and Slice Thickness on CTDI and Image Quality in CT Scan]. BERKALA SAINSTEK, 7(1), 7–12. https://bst.jurnal.unej.ac.id/index.php/BST/article/view/9913

Retnoningsih, D.S., Anam, C., & Setiabudi, W. (2012). Studi Uniformitas Dosis Radiasi CT Scan pada Fantom Kepala yang Terletak pada Sandaran Kepala. [Study of CT Scan Radiation Dose Uniformity on Head Phantom Positioned on Headrest]. Journal of Science and Mathematics, 20(2), 41–42. https://ejournal.undip.ac.id/index.php/sm/article/view/8007

Rosyida, N. (2016). Measurement of External Radiation Dose Rate in the Radiotherapy Area of Dr. Saiful Anwar Hospital Malang. Brawijaya University, Malang.

Rusli, M. (2017). Uji Keamanan Radiasi Paparan Rontgen Gigi di Atro Radiologi RS Muhammadiyah Makassar [Radiation Safety Test of Dental X-Ray Exposure at Atro Radiology Muhammadiyah Hospital Makassar] (Undergraduate Thesis). Hasanuddin University, Makassar. https://www.scribd.com/document/533658551/radiologi

Soediatmoko, E. (2011). Estimasi dosis radiasi pada pemeriksaan CT scan kepala = Radiation dose estimation in head CT scan examination. [Estimation of Radiation Dose in Head CT Scan Examination](Undergraduate Thesis). University of Indonesia, Depok. https://lib.ui.ac.id/detail?id=20290765&lokasi=lokal