# Original Research Article

Seasonal Wind Energy Forecasting for Sustainable Electricity Generation in Northern Nigeria

.

ABSTRACT

The study evaluates the seasonal wind energy resources for sustainable electricity generation across some selected states in Northern Nigeria, motivated by the country’s persistent electricity shortages despite abundant renewable resources, highlighting the need for diversification into renewable energy. This study presents one of the first state-level, field-based wind energy assessments across Kaduna, Katsina, Kano, Bauchi, Yobe, Kogi, and Abuja. Wind speed data was obtained by in-situ measurement approach. The wind speed data (2024-2025) were collected seasonally from Wet to Dry seasons. The wind speed data was measured at a 10m height for an interval of 30mints from 6:00am to 7:00pm for selected months and analyzed on daily, seasonal, and annual timescales. The energy output was estimated using the wind power equation and power density analysis. A 16.21% loss adjustment accounted for turbine downtime, wake effects, and transmission inefficiencies, while results were scaled to 1% of state land mass to assess deployment feasibility. Results show strong seasonal variability, with dry-season peaks (e.g., Katsina, 178.09 kWh in February; Yobe, 147.43 kWh in January; Kano, 141.38 kWh in December) and wet-season lows (e.g., Kaduna, 47.40 kWh in June; Kogi, 35.14 kWh in July). Annual net energy output ranged from 37,597.52 kWh in Katsina to 15,143.86 kWh in Kogi. Scaled land-mass analysis identified Yobe (178 GWh/year), Bauchi (152 GWh/year), and Kaduna (143 GWh/year) as level 1 states for utility-scale wind farms, while Kogi and Abuja were classified as level 3, suitable for hybrid mini-grids. The study concludes that wind resources in Northern Nigeria are substantial but unevenly distributed. It provides empirical, loss-adjusted, and land-scaled estimates that can guide renewable energy planning, hybridization with solar photovoltaic, and infrastructure expansion in line with Nigeria’s sustainable energy goals.

Keywords: Wind energy; wind power equation; power density analysis; loss adjustment; Renewable energy planning; Land mass scaling; Northern Nigeria.

1. INTRODUCTION

Energy plays an important role in national development, serving as a driver of industrial growth, socio economic advancement, and improved quality of life. In Nigeria, the demand for electricity far exceeds supply, resulting in chronic shortages, low per-capital electricity consumption, and frequent grid collapses (Oyedepo, 2012; Sambo, 2009). The Energy sector in Nigeria relies on conventional fossil fuels, namely oil and gas, which account for more than 80% of the nation’s primary energy supply (SAO Admin, 2024). The National Renewable Energy and Energy Efficiency Policy (NREEP) is seen as the key framework guiding renewable energy development in Nigeria (Adeyemi, 2024), Nigeria has several opportunities to advance renewable energy policymaking, which includes abundant renewable resources i.e., Wind and Solar, growing energy demand, international support and job creation. The Nation’s vast renewable resources can contribute significantly (Adeyemi, 2024).

Wind energy is seen as a clean and abundant resource, which requires attention. In the context of the worldwide shift towards cleaner and more efficient energy systems, understanding the localized potential of wind energy becomes a necessary element.  This comprehension is not only optimizing on energy generation but also propels sustainable development efforts, aligning with the transition toward cleaner energy models (Tan et al., 2022). The criterion of a nation's economic advancement is intricately linked to its capability to generate and distribute power, an indispensable pre-requisite for societal growth and national progress (Lawal et al., 2023). In addition, Northern Nigeria possesses vast land areas that remain underutilized, creating opportunities for utility-scale deployment. If just 1% of each state’s land area were allocated to wind farms, the resulting energy potential would be substantial, as demonstrated in recent assessments. Thus, a systematic evaluation of wind energy resources across multiple states provides a strong foundation for strategic energy planning in Nigeria.

Several studies have delved into evaluating local wind power sites, with multiple investigations conducted in Nigeria focusing on assessing wind resources. For instance, the urgent need to address climate change, environmental concerns and ensure a secure and diversified energy supply has prompted extensive research into various methods and technologies within the realm of renewable energy. Attabo et al. (2023) examined the potential for wind energy and the economic viability of establishing wind farms along Nigeria's coast and in offshore locations and revealed that offshore areas possess considerably greater wind power potential compared to coastal regions. Labuschagne & Kamper (2021), studied the wind speed characteristics and energy potential in five selected locations in the southwestern part of Nigeria were investigated using monthly mean wind speed data of 51 years obtained from the Nigeria Meteorological Agency. The data were subjected to the 2-parameter-Weibull and other statistical analyses. The outcome showed that the wind speed measured at a height of 10 m ranged from 1.3 to 13.2 m/s while the modal wind speed ranges from 3.0 to 5.9 m/s. 83.6% of the data were found to be greater than 3.0 m/s. The average monthly wind speed ranged from 2.72 to 7.72 m/s. Nymmphas & Teliat (2024), In this study, five numerical methods, Weibull-two (W), Mixture Weibull (MW), Gamma (G), lognormal (LN) and normal (N), were used to analyze wind speed data from twelve stations measured at 10 m height in order to determine the best method for estimating Weibull parameters for each station and region. Kassem et al., (2024), The study location was Jeje, Nigeria and data on wind speed were gathered from a variety of sources, including EAR5, EAR5 Ag, EAR5 Land, CFSR, and MERRA-2 at a height of 10 m. The Weibull distribution function, commonly employed to evaluate wind energy potential, was utilized. A maximum wind power density value of 15.75W/m2 was obtained when the MERRA-2 dataset was implemented. The results indicate that large-scale wind turbines are not a viable alternative in this area. Thus, the performance of six wind turbines, expressed by output power with a cut in speed ranging between 1 and 1.5 m/s, was investigated. The results demonstrated that the AWI-E1000Tis the most efficient wind turbine under consideration. In addition, it has been shown that each considered turbine can be installed in this area based on the data acquired from the MERRA-2 and CFSR. Amadi et al., (2024). Incorporating wind energy into Nigeria's energy mix requires robust policy backing, enhanced infrastructure, and active cooperation between the government and private sector participants.

The overall recent study on wind energy in Nigeria has been based on coastal and offshore assessment, the reviewed work ended at Weibull fitting or just mere speed prediction, without extending to realistic power and energy yield estimation. The forecasting validation is weak, review on reanalysis dataset without ground data validation and techno economic scaling and land use assessments for inland regions where not used, as such this present study addresses a combining field measurement, energy yield, validation metrics and techno-economic analysis.

Therefore, addressing the inadequate comprehensive, state-by-state wind energy potential evaluation that will guide seasonal wind energy forecasting for sustainable electricity generation in Northern Nigeria are based on the following objectives (i) measure and analyze wind speed data across selected states, (ii) estimate daily, seasonal, and annual wind energy output, (iii) adjust results for losses due to turbine downtime, wake effects, and transmission inefficiencies, and (iv) scale energy potential to 1% of each state’s land mass to assess feasibility for large-scale deployment.

2. MATERIALS AND METHODS

2.1 Research Design

This study adopts a quantitative research design that integrates empirical wind speed data with analytical modeling techniques to estimate the total energy generation potential from wind resources. The design is suitable because it allows for structured analysis of meteorological datasets and the application of established wind energy conversion equations to determine potential daily and annual energy outputs.

2.2 Study Area

The research focuses on selected states in Northern Nigeria namely Kaduna, Katsina, Kano, Bauchi, Yobe, Kogi, and the Federal Capital Territory (FCT) Abuja. These locations were chosen due to their varying geographical, climatic, and meteorological conditions, which affect wind regimes and energy potential. Northern Nigeria lies within the savannah and semi-arid belt with a Lat 40 to 14N and 30 E to150E, which generally experiences higher wind speeds than southern regions.

2.3 Data Collection

The instrument used for the course of this research is a hand held (pocket-sized) digital anemometer, GM816 model, 65MPH range, battery included. The instrument helped in wind speed data collection at a height 10m for an interval of 30-minute from 6:00 AM to 7:00 PM 13hrs daily for a duration of 3days each during the Dry and Wet season respectively for selected months (Katsina: February 2025 and October 2024, Kano: August 2025 and December 2024, Kaduna: April 2025 and January 2025, Bauchi & Yobe: January 2025 and August 2024 respectively, Kogi and Abuja: July 2025 and March 2025 respectively), across 2024 and 2025. Each dataset represents possible wind speed scenarios reflecting diurnal variations in different seasons.

2.4 Data Processing and Analysis

The data were analyzed in three major steps:

- Wind Power Density Calculation

The available power in the wind was estimated using the fundamental wind power equation:

P=12ρAv3					            (1)

where: P = power (W), ρ = air density (1.225 kg/m³ assumed at standard conditions), A = swept area of the turbine (m²) and v = wind speed (m/s).

A=π(D2)2 (m)2 						(2)

Power from wind (watts) before rating limit (amount of power generated from a wind turbine before reaching its rated capacity or max power output)

PvW=12ρAηtotv3 					(3)

- Energy Conversion to kWh

Daily energy output was calculated by integrating the available power over the measurement period (0.5 hours for energy per interval), then converted to kilowatt-hours (kWh). The yearly output was estimated by multiplying daily averages by 365 days.

Pkw=P(w)1000 						(4)

Ekwh=Pkw×0.5 					(5)

Annual Energy=Daily Energy ×365 days / year 	(6)

- Scaling with Land Area

Since large-scale deployment was considered, the potential was estimated using 1% of each state’s land mass, assuming uniform distribution of turbines within the allocated land (Shaaban & Petinrin, 2014).

The installed power density = 3.0 MW per km² (i.e., 3000 kW/km²) a reasonable onshore wind-farm density accounting for spacing. Turbine rated capacity = 250 kW (same turbine used in our interval calculations). The turbines per km² = 3000 / 250 = 12 turbines/km². Daily total on 1% land = (1% area in km²) × (12 turbines/km²) × (avg daily kWh per turbine) and Yearly total = daily × 365.

- Loss and Availability Factors

To account for practical limitations, the following correction factors were applied: Turbine availability: 98%, Electrical losses: 95% and Wake and array effects: 90%

The combined multiplier was:

Net Output=Gross Output×0.98×0.95×0.90		(7)

Thus, all final results represent realistic net energy potentials.

2.5 Tools

Microsoft Excel was used for dataset processing, calculations, and visualization. Tables and graphical plots (daily vs yearly energy) were generated for better interpretation of the results.

3. results and discussion

The results of the wind energy potential assessment conducted in selected states of Northern Nigeria are presented below:

3.1 Present clear seasonal differences (dry vs. wet season)

The Tables 1 below shows the average wind speed (m/s) for each selected states, indicating the time and the seasonal differences for dry and wet season respectively. As seen in the tables the wind speed changes occur throughout the day and it is observed that the wind speed starts lower in the morning and increases in the afternoon.

Table 1. Seasonal Differences

| States | Bauchi |  | Yobe |  | Kogi |  | FCT Abuja |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time (mins) | Wind Speed (m/s) |  | Wind Speed (m/s) |  | Wind Speed (m/s) |  | Wind Speed (m/s) |  |
|  | Dry | Wet | Dry | Wet | Dry | Wet | Dry | Wet |
| 6:00 | 1.93 | 1.88 | 2.27 | 1.63 | 1.43 | 1.13 | 1.77 | 1.93 |
| 6:30 | 2.20 | 2.10 | 2.27 | 1.57 | 1.63 | 1.17 | 2.20 | 1.20 |
| 7:00 | 1.97 | 2.09 | 2.73 | 1.27 | 1.60 | 1.23 | 2.30 | 1.53 |
| 7:30 | 2.27 | 2.16 | 2.83 | 1.83 | 2.23 | 1.47 | 2.03 | 1.87 |
| 8:00 | 2.53 | 2.64 | 3.13 | 2.00 | 2.30 | 1.73 | 2.30 | 2.30 |
| 8:30 | 2.23 | 2.28 | 3.57 | 2.40 | 2.53 | 1.83 | 2.20 | 2.50 |
| 9:00 | 2.67 | 2.82 | 4.50 | 2.50 | 2.87 | 2.03 | 2.60 | 2.77 |
| 9:30 | 2.83 | 3.01 | 4.37 | 2.50 | 2.60 | 2.27 | 2.67 | 2.37 |
| 10:00 | 3.00 | 3.10 | 4.37 | 2.67 | 2.77 | 2.50 | 2.93 | 2.53 |
| 10:30 | 3.10 | 3.03 | 4.50 | 3.57 | 2.83 | 2.47 | 2.77 | 2.43 |
| 11:00 | 3.73 | 3.81 | 4.67 | 3.20 | 3.33 | 2.77 | 3.57 | 3.30 |
| 11:30 | 4.03 | 4.11 | 4.90 | 3.00 | 3.17 | 2.97 | 3.53 | 2.67 |
| 12:00 | 3.97 | 4.16 | 5.30 | 3.70 | 3.70 | 3.20 | 3.67 | 3.33 |
| 12:30 | 4.27 | 4.42 | 5.10 | 4.03 | 3.93 | 3.17 | 4.27 | 3.90 |
| 13:00 | 4.87 | 4.82 | 5.03 | 3.97 | 3.80 | 3.47 | 4.03 | 3.47 |
| 13:30 | 4.67 | 4.52 | 5.23 | 4.30 | 3.97 | 3.43 | 4.17 | 3.53 |
| 14:00 | 5.17 | 4.99 | 5.27 | 4.40 | 4.13 | 3.30 | 4.43 | 3.70 |
| 14:30 | 4.93 | 4.74 | 5.07 | 3.80 | 3.83 | 2.87 | 4.63 | 3.40 |
| 15:00 | 4.90 | 4.90 | 4.83 | 3.93 | 3.97 | 2.90 | 4.60 | 3.63 |
| 15:30 | 5.10 | 4.97 | 4.40 | 4.03 | 3.90 | 3.23 | 4.83 | 3.83 |
| 16:00 | 5.07 | 5.02 | 4.60 | 3.60 | 3.90 | 2.97 | 4.53 | 4.47 |
| 16:30 | 4.73 | 4.68 | 4.93 | 3.67 | 3.60 | 2.90 | 4.03 | 4.03 |
| 17:00 | 4.63 | 4.74 | 4.03 | 3.50 | 3.77 | 2.80 | 4.20 | 4.10 |
| 17:30 | 4.23 | 4.21 | 4.20 | 3.43 | 3.47 | 2.63 | 3.83 | 4.13 |
| 18:00 | 4.00 | 4.03 | 4.13 | 3.23 | 3.17 | 2.40 | 3.57 | 3.87 |
| 18:30 | 3.73 | 3.68 | 3.20 | 2.47 | 2.70 | 2.33 | 3.43 | 3.60 |
| 19:00 | 3.67 | 3.76 | 3.07 | 2.57 | 2.40 | 2.17 | 3.10 | 3.33 |

Table 1: Seasonal Differences (CONTD)

| States | Kaduna |  | Katsina |  | Kano |  |
| --- | --- | --- | --- | --- | --- | --- |
| Time (mins) | Wind Speed (m/s) |  | Wind Speed (m/s) |  | Wind Speed (m/s) |  |
| Time | Dry | Wet | Dry | Wet | Dry | Wet |
| 6:00 | 2.10 | 1.57 | 2.30 | 1.60 | 2.17 | 2.10 |
| 6:30 | 2.20 | 1.50 | 2.30 | 1.63 | 2.30 | 1.53 |
| 7:00 | 2.30 | 1.63 | 2.47 | 1.70 | 2.47 | 1.43 |
| 7:30 | 2.53 | 1.80 | 2.70 | 1.93 | 2.47 | 1.73 |
| 8:00 | 2.57 | 1.87 | 3.07 | 2.17 | 2.37 | 2.10 |
| 8:30 | 2.67 | 2.13 | 3.17 | 2.37 | 2.53 | 2.37 |
| 9:00 | 3.40 | 2.47 | 3.50 | 2.60 | 3.13 | 2.37 |
| 9:30 | 3.40 | 2.50 | 3.80 | 2.77 | 3.37 | 2.47 |
| 10:00 | 3.73 | 2.57 | 4.20 | 3.10 | 3.07 | 2.73 |
| 10:30 | 3.77 | 2.87 | 4.37 | 3.27 | 3.70 | 2.87 |
| 11:00 | 4.03 | 3.20 | 4.67 | 3.83 | 4.17 | 2.93 |
| 11:30 | 4.33 | 3.30 | 4.70 | 3.83 | 4.17 | 3.53 |
| 12:00 | 4.70 | 3.43 | 4.87 | 4.30 | 4.23 | 3.23 |
| 12:30 | 4.80 | 3.70 | 5.17 | 4.27 | 4.63 | 3.83 |
| 13:00 | 4.93 | 3.67 | 5.27 | 4.17 | 4.93 | 3.90 |
| 13:30 | 5.17 | 3.80 | 5.33 | 4.23 | 5.37 | 4.13 |
| 14:00 | 5.20 | 3.87 | 5.57 | 4.20 | 5.53 | 4.33 |
| 14:30 | 5.23 | 3.63 | 6.00 | 4.63 | 5.67 | 4.00 |
| 15:00 | 5.03 | 3.60 | 6.00 | 4.50 | 5.50 | 4.30 |
| 15:30 | 4.80 | 3.40 | 5.97 | 3.97 | 5.37 | 4.40 |
| 16:00 | 4.87 | 3.47 | 5.70 | 3.43 | 5.10 | 4.27 |
| 16:30 | 4.40 | 3.23 | 5.60 | 3.17 | 5.27 | 4.03 |
| 17:00 | 4.17 | 2.83 | 5.33 | 2.67 | 4.83 | 3.63 |
| 17:30 | 3.70 | 2.67 | 4.77 | 2.40 | 4.43 | 3.33 |
| 18:00 | 3.30 | 2.70 | 4.17 | 2.10 | 4.23 | 3.03 |
| 18:30 | 2.93 | 1.90 | 3.77 | 1.87 | 3.90 | 3.07 |
| 19:00 | 2.70 | 1.53 | 3.03 | 1.47 | 3.43 | 2.67 |

Based on the table 1  result shows that wind speed is generally higher in dry season with a peak value between 12:00pm and 15:00pm while wet season are a little bit lower. The state with the highest wind speed is Yobe while Kogi shows lowest potential for wind energy.

3.3 Daily and Seasonal Wind Energy Generation

Table 2, shows a clear seasonal dependence of wind energy generation across the studied states. Dry season months: January to February and December, consistently record higher average daily energy outputs, i.e., Katsina (178.09 kWh in February), Yobe (147.43 kWh in January), Kano (141.38 kWh in December). In comprising with wet season months: June to August and October shows massive reductions: Katsina: 67.74 kWh in October, Kaduna: 47.40 kWh in June, Kogi: 35.14 kWh in July. This pattern aligns with the climatological characteristics of Northern Nigeria, where the dry season is dominated by Harmattan winds driven by the northeasterly trade winds, while the wet season is characterized by humid, lower-velocity southwesterlies.

Importantly, intra-seasonal variability is observed: for example, Kaduna records moderate generation in April: 121.32 kWh but drops sharply in June: 47.40 kWh, reflecting transitional wind regimes. Fig.1. Shows the winds alone, which favors Katsina, Yobe and Kano

Table 2. the representative daily wind energy outputs for selected states across dry, wet, and transitional seasons.

| State | Season (Month) | Day 1 (kWh) | Day 2 (kWh) | Day 3 (kWh) | Average Daily (kWh) |
| --- | --- | --- | --- | --- | --- |
| Katsina | Feb (Dry) | 193.86 | 170.56 | 169.85 | 178.09 |
| Katsina | Oct (Wet) | 66.72 | 65.84 | 70.66 | 67.74 |
| Kano | Aug (Wet) | 72.19 | 70.87 | 66.41 | 69.82 |
| Kano | Dec (Dry) | 143.86 | 134.07 | 146.22 | 141.38 |
| Kaduna | Apr (Transition) | 131.97 | 109.45 | 122.53 | 121.32 |
| Kaduna | Jun (Wet) | 57.15 | 38.97 | 46.08 | 47.40 |
| Bauchi | Jan (Dry) | 117.29 | 107.86 | 115.97 | 113.71 |
| Bauchi | Aug (Wet) | 65.49 | 66.52 | 69.53 | 67.18 |
| Yobe | Jan (Dry) | 150.94 | 143.19 | 148.17 | 147.43 |
| Yobe | Aug (Wet) | 65.00 | 68.10 | 62.78 | 65.29 |
| Kogi | Mar (Transition) | 73.50 | 57.46 | 60.72 | 63.89 |
| Kogi | Jul (Wet) | 33.39 | 24.85 | 47.18 | 35.14 |
| Abuja | Mar (Transition) | 83.10 | 88.60 | 85.80 | 85.83 |
| Abuja | Jul (Wet) | 65.90 | 62.58 | 66.54 | 65.01 |

Fig.1. Peak daily wind energy output

This implies that energy planners must account for seasonal intermittency when integrating wind into hybrid systems, possibly requiring storage or complementary solar Photo Voltaic (PV) during wet months as shown in table 2.

3.4 Annual Energy Potential and Loss Adjustment

Table 3, estimates the annual gross energy output ranges from 18,073.58 kWh in Kogi to 44,863.98 kWh in Katsina. After accounting for the 16.21% loss factor, effective annual output reduces by ~6,000–7,000 kWh for high performing states, and ~3,000 kWh for low-performing states.

Table 3. Summarizes the average daily and annual wind energy generation, including the impact of the 16.21% system loss factor.

| State | Avg. Daily Energy (kWh) | Annual (kWh) | Annual after Loss (kWh) |
| --- | --- | --- | --- |
| Katsina | 122.92 | 44,863.98 | 37,597.52 |
| Kano | 105.60 | 38,545.22 | 32,297.04 |
| Kaduna | 84.36 | 30,790.79 | 25,799. 60 |
| Bauchi | 90.44 | 33,011.82 | 27,660. 60 |
| Yobe | 106.36 | 38,822.01 | 32,528.96 |
| Kogi | 49.52 | 18,073.58 | 15,143.86 |
| Abuja (FCT) | 75.42 | 27,528.30 | 23,065.96 |

This implies that large scale wind farms should be strategically located in the northeast and northwest axis, while central states may only support small scale, localized wind projects.

Fig. 2. Annual wind energy potential against after loss

Fig. 2. This confirms that northeastern and northwestern states i.e., Katsina, Yobe and Kano have superior long term wind resources compared to the northcentral states i.e., Kogi and Abuja.

3.4 Wind Energy Scaling by Land Mass

Tab 4, shows the scaling of 1% of each state’s land mass with a substantial difference in daily net outputs emerge. Yobe has 486,590 kWh/day and Bauchi has 417,394.5 kWh/day which leads the group due to their large land mass coupled with high wind speeds. Kaduna has 391,030.8 kWh/day which also shows high potential, despite having relatively moderate average wind speeds, due to its large geographical extent. In comprising with Abuja which has 55,358.31 kWh/day and produces the least, constrained by its small land mass (73 km² at 1%).

Fig.3. shows emphasizes on wind energy planning, must go beyond raw wind speed and consider territorial scalability.

Table 4. Estimated Daily Wind Energy Output for 1% Land Mass

| State | Land Mass (km²) | 1% Land Mass (km²) | Daily Net Energy (kWh/day) |
| --- | --- | --- | --- |
| Katsina | 24,192 | 242 | 299096.5 |
| Kano | 20,131 | 201 | 213419.2 |
| Bauchi | 45,893 | 459 | 417394.5 |
| Yobe | 45,502 | 455 | 486590 |
| Kogi | 29,833 | 298 | 148378.3 |
| Kaduna | 46,053 | 461 | 391030.8 |
| FCT Abuja | 7,315 | 73 | 55358.31 |

(Note: Figures represent net output after applying availability and loss factor = 0.8379).

The above table implies that the Land availability is a critical scaling factor. Even if a state like Kano has decent wind potential, its smaller land area limits total capacity compared to Bauchi or Yobe.

Fig. 3. Darly Net Energy Output for 1% land mass

  - Yearly Net Energy Potential for 1% Land Mass

Extrapolation to yearly output shows massive differences across states: Yobe: 177,605,342.8 kWh/year (≈178 GWh/year), Bauchi: 152,348,976.2 kWh/year (≈152 GWh/year), Kaduna: 142,726,231 kWh/year (≈143 GWh/year) and Kogi & Abuja: <55 GWh/year

Tab 4, shows that Yobe alone could generate 3 – 4× more annual wind energy than Abuja with the same percentage of land utilization. These findings highlight regional wind energy dominance in northeastern and northwestern states, driven by both meteorological and geographical advantages. Fig.4. makes emphasis on spatial temporal synergy of these factors that Nigerian’s wind energy priority states such as Yobe, Bauchi and Kaduna, with Katsina and Kano as strategic supplements.

Table 5: Estimated Yearly Wind Energy Output for 1% land utilization are presented

| State | Daily Net Energy (kWh/day) | Yearly Net Energy (kWh/year) |
| --- | --- | --- |
| Katsina | 299096.5 | 109170228.3 |
| Kano | 213419.2 | 77897994.45 |
| Bauchi | 417394.5 | 152348976.2 |
| Yobe | 486590 | 177605342.8 |
| Kogi | 148378.3 | 54158072.71 |
| Kaduna | 391030.8 | 142726231 |
| FCT Abuja | 55358.31 | 20205783.21 |

The above table implies that in designing a national or regional energy policy, Yobe, Bauchi, and Katsina should be prioritized as wind energy hubs, while other states can be integrated into distributed hybrid networks (solar + wind + grid + Small Modular Reactor SMR).

Fig.4. Yearly net Wind Energy Output for 1% Land Mass

  - Comparative Discussion of States

The high potential states are Yobe, Bauchi, Kaduna this are suitable for utility scale wind farms, moderate potential states are Katsina, Kano are also suitable for medium scale grid-connected wind projects and the low potential states are Kogi, Abuja which are suitable for localized, hybrid microgrids. This classification provides a strategic roadmap for staged wind development, consistent with recommendations by the Energy Commission of Nigeria.

4. CONCLUSION

This study concludes that Northern Nigeria possesses substantial wind energy potential, but its exploitation requires state-specific and season-sensitive strategies. Wind resources are stronger in the dry season which varies significantly across states and larger states like Bauchi and Yobe which holds greater cumulative potentials. The hybridization with solar Photovoltaic (PV) and potential nuclear small modular reactors (SMRs) can significantly strengthen Nigeria’s energy mix intermittency.

4.1 Contribution to Knowledge

This research makes the following contributions:

- Provides one of the first state-level, field-based wind energy assessments in Northern Nigeria.

- Introduces 1% land mass scaling methodology, demonstrating how land availability dramatically affects cumulative energy potential.

- Establishes a level framework for state prioritization, offering policymakers a clear roadmap for wind energy deployment.

- Validates the importance of seasonal analysis in renewable energy planning, underscoring the necessity of hybrid system integration

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1. Microsoft office Grammarly for grammar checking and paraphrasing

2. NOTE: AI listed above has been removed

3.

REFERENCES

Adeyemi, A. (2024). Promoting renewable energy through a decentralised electricity regulatory framework – An analysis of Nigeria’s Electricity Act 2023. Journal of Sustainable Development Law and Policy (The), 15(3), 399–421. https://doi.org/10.4314/jsdlp.v15i3.15

Amadi, H. N., Madu, M. C., Ojuka, O. E., & Igbogidi, O. N. (2024). Renewable energy in Nigeria: Prospects and challenges. European Journal of Advances in Engineering and Technology, 11(4), 51–60.

Attabo, A. A., Ajayi, O. O., Oyedepo, S. O., & Afolalu, S. A. (2023). Assessment of the wind energy potential and economic viability of selected sites along Nigeria’s coastal and offshore locations. Frontiers in Energy Research, 11. https://doi.org/10.3389/fenrg.2023.1186095

Kassem, Y., Camur, H., & Apreala, T. (2024). Assessment of wind energy potential for achieving Sustainable Development Goal 7 in the rural region of Jeje, Nigeria. Engineering, Technology & Applied Science Research, 14(4), 14977–14987. https://doi.org/10.48084/etasr.17812

Labuschagne, C. J., & Kamper, M. J. (2021). Wind generator impedance matching in small-scale passive wind energy systems. IEEE Access, 9, 22558–22568.

Lawal, O. A., Jimoh, A. A., Abdulkadir, Z. A., Bello, A. B., Balogun, M. O., & Atanda, A. R. (2023, March). Mitigating the challenge of energy crisis via energy audit and efficiency measures: A case of a household in Nigeria. In 2023 7th International Conference on Green Energy and Applications (ICGEA) (pp. 218–223). IEEE.

Nymphas, E. F., & Teliat, R. O. (2024). Evaluation of the performance of five distribution functions for estimating Weibull parameters for wind energy potential in Nigeria. Scientific African, 23, e02037. https://doi.org/10.1016/j.sciaf.2023.e02037

Oyedepo, S. O. (2012). Energy and sustainable development in Nigeria: The way forward. Energy, Sustainability and Society, 2(1), 15.

SAO Admin. (2024, May 14). Opportunities and challenges in navigating Nigeria’s energy transition - SAO Energy. SAO Energy. https://saoenergy.com/opportunities-and-challenges-in-navigating-nigerias-energy-transition/

Sambo, A. S. (2009). Strategic developments in renewable energy in Nigeria. International Association for Energy Economics, 16(4), 15–19.

Shaaban, M., & Petinrin, J. O. (2014). Renewable energy potentials in Nigeria: Meeting rural energy needs. Renewable and Sustainable Energy Reviews, 29, 72–84. https://doi.org/10.1016/j.rser.2013.08.078

Tan, J. D., Chang, C. C. W., Bhuiyan, M. A. S., Nisa’Minhad, K., & Ali, K. (2022). Advancements of wind energy conversion systems for low-wind urban environments: A review. Energy Reports, 8(1), 3406–3414.