Geochemical Barriers and Mineral Precipitation for Groundwater Remediation: Advances in Natural and Engineered Systems

ABSTRACT

Groundwater across North America is still vulnerable to widespread contamination by metals and metalloids, left over from historic-industrial and mine activities. This review explores natural and engineered techniques employing geochemical barriers and precipitates to immobilize contaminants. Natural attenuating processes like arsenic removal by iron hydroxide at Elizabeth City, NC, and carbonate buffering at Sudbury Basin and Faro Mine, YK, highlight inherent systems' capacities to attenuate pollutant mobility. However, these are subject to stable geochemical environments and long-term management, especially with respect to increasing climate variability and its effect on hydrological regimes. Engineered remedies like zero-valent permeable reactive iron barriers at Denver Federal Center, CO, and lead immobilization by phosphate amendment at Butte, MT, show high removal capacities; however, engineered remedies are limited by clogging and side reactions and by evolving regulatory standards. New techniques like nanoscale hydroxides and biomineralization through sulfate-reducing bacteria hold promise for inexpensive and sustainable remediation, though large-scale validation is required. The remediation guidelines enacted by U.S. EPA and by Canadian agencies set cleanup goals and monitoring schemes and cost-benefit analyses show passive systems to have long-term economic and societal advantage even at large initial characterization costs. Future remediation work needs to incorporate adaptive management schemes and by planning for climatic change can ensure sustainable aquifer protection.

Keywords: Groundwater contamination, geochemical barriers, mineral precipitation, natural attenuation, zero-valent iron

1. INTRODUCTION

Groundwater, a vital natural resource, faces widespread contamination from anthropogenic activities, particularly industrial discharges and historical mining operations. The presence of dissolved metals and metalloids in groundwater poses significant risks to human health and ecological systems due to their toxicity, persistence, and potential for bioaccumulation (Edo et al., 2024; Kumar et al., 2023). Effective remediation of these contaminated aquifers is a complex undertaking, requiring a deep understanding of hydro geochemistry and the development of sustainable treatment strategies (Warner et al., 2023).

1.1 Groundwater Contamination Landscape in North America

Groundwater contamination remains one of the most pressing environmental challenges across North America, demanding robust and sustainable remediation strategies. In the United States, the Environmental Protection Agency (EPA) oversees thousands of hazardous waste sites under the Superfund program, reflecting the scale of historical pollution (Holifield, 2004). As of June 2024, the National Priorities List documented 1,340 active sites, with an additional 39 awaiting formal listing. These include landfills, abandoned industrial facilities, and mines many burdened by heavy metal contamination resulting from past waste management practices (U.S. Environmental Protection Agency, n.d.-p).

Canada faces a parallel legacy, particularly linked to its long-standing reliance on resource extraction. Recent estimates suggest that approximately 450 abandoned mines require remediation, while broader assessments place the number of orphaned or abandoned sites closer to 10,000 (Natural Resources Canada, 2023; Natural Resources Canada, n.d.-b). Many of these sites lack comprehensive closure or management plans, prolonging their ecological and safety risks.

The prevalence of such sites across both nations underscores that industrial contamination is not an isolated or temporary problem, but a systemic and persistent challenge with far-reaching consequences (Tremblay & Hogan, 2016). Addressing these legacies is not only an environmental necessity but also a socio-economic imperative. In the United States, Superfund cleanups are explicitly tied to community revitalization and local economic development (U.S. Environmental Protection Agency, n.d.-p), while in Canada, remediation programs prioritize collaboration with Indigenous and Northern communities directly affected by contaminated lands (Natural Resources Canada, n.d.-a).

1.2 Fundamental Principles of Mineral Precipitation for Metal Immobilization

Mineral precipitation stands as a cornerstone mechanism in the immobilization of dissolved metals within contaminated groundwater. This process fundamentally involves the transformation of soluble metal ions into insoluble mineral phases, thereby significantly reducing their mobility and bioavailability in the aqueous environment (Ortiz-Castillo et al., 2021; U.S. Environmental Protection Agency, n.d.-h). Understanding the specific geochemical conditions that favor these transformations is critical for designing and optimizing remediation strategies. Three primary precipitation mechanisms are widely recognized for their role in metal immobilization:

Hydroxide Formation: This mechanism relies on the alteration of groundwater pH, typically through the addition of hydroxide compounds or direct pH adjustment, to convert soluble heavy metal ions into insoluble metal hydroxides (Client's Research Brief, 2025; Ortiz-Castillo et al., 2021).

Carbonate Precipitation: Carbonate precipitation involves the formation of calcium carbonate (CaCO₃) or other insoluble metal carbonates (e.g., PbCO₃, ZnCO₃) (Client's Research Brief, 2025). A significant advancement in this area is Microbial-Induced Calcium Carbonate Precipitation (MICP), a biogeochemical remediation method that leverages biological activity to drive carbonate formation (Konstantinou, & Wang, 2023). Ureolytic bacteria can hydrolyze urea, producing CO₂, NH₃, and OH⁻, which raise alkalinity and drive carbonate precipitation.

Sulfide Formation: The precipitation of sulfides, usually in reducing (anoxic) environments, results in the formation of highly insoluble metal sulfides (Client's Research Brief, 2025; Encyclopedia.pub, n.d.; Ortiz-Castillo et al., 2021; U.S. Environmental Protection Agency, n.d.-h). The sulfides are significantly less soluble compared to their corresponding hydroxides, and this facilitates effective metal removal (Encyclopedia.pub, n.d.; U.S. Environmental Protection Agency, n.d.-h; Licht, 1988; Ortiz-Castillo et al., 2021; Vidu et al., 2020). This method is also less prone to interference by chelating compounds that hinder hydroxide-based precipitation (Vidu et al., 2020).

Metal precipitation relies on environmental conditions, pH for carbonates/hydroxides and reducing conditions for sulfides thus, aquifer geochemistry is key to immobilization. Eh–pH diagrams, solubility constants, and kinetics yield an estimate of feasibility (Encyclopedia.pub, n.d.; Lin et al., 2021). Though chemical, precipitation is increasingly associated with biology. MICP prefers the formation of carbonate (Zhang et al., 2023), whereas SRB is central in sulfide-based immobilization (Hu et al., 2024; Xu & Chen, 2020; Mafane et al., 2025).

Remediation strategies balance natural attenuation (NA) and engineered methods. NA is cheaper and less invasive (Federal Remediation Technologies Roundtable, n.d.-a; Pradhan et al., 2020; Newell et al., 2021), yet its use with metals/radionuclides is limited by complexity (Boehm et al., 2020). Institutionalized Monitored Natural Attenuation (MNA) formalizes NA with required management (Federal Remediation Technologies Roundtable, n.d.-b; U.S. Environmental Protection Agency, n.d.-h). Engineered barriers, on the other hand, introduce reactive materials to form in-situ treatment zones (Muller et al., 2021).

2. NATURAL ATTENUATION SYSTEMS: HARNESSING GEOCHEMICAL PROCESSES

Natural attenuation (NA) systems leverage intrinsic geochemical and biological processes within an aquifer to reduce the mobility and concentration of contaminants (Federal Remediation Technologies Roundtable, n.d.-b; Newell et al., 2021; U.S. Environmental Protection Agency, n.d.-h).

2.1 Iron-Mediated Precipitation Barriers

Iron minerals, particularly iron hydroxides and oxyhydroxides, play a crucial role in the natural attenuation of various metal and metalloid contaminants through adsorption and co-precipitation (Mashale et al., 2025). Their ubiquitous presence in many aquifer systems makes them natural reactive media.

2.1.1 Elizabeth City, NC: Arsenic Immobilization

The Elizabeth City, North Carolina, site serves as a compelling illustration of successful, long-term natural attenuation of arsenic (As) mediated by iron. At this location, natural iron hydroxide (Fe(OH)₃) precipitation achieved a 92% removal of arsenic from groundwater, with monitoring over a decade confirming its stability and continued effectiveness (Client's Research Brief, 2025; U.S. Environmental Protection Agency, n.d.-d). The Elizabeth City site has been a subject of detailed investigations by the U.S.

The case of Elizabeth City highlights a critical geochemical duality: Iron compounds serve to be stabilizers as well as arsenic sources in groundwater. Iron hydroxides immobilize arsenic by adsorption as well as co-precipitation, with efficiencies up to 92% (Client's Research Brief, 2025). Reductive dissolution of iron oxides is a significant route for arsenic release, however When Fe(III) oxides, strong arsenic binders are microbially reduced to soluble Fe(II) in carbon-rich environments, structural changes lead to arsenic desorption and mobilization (Hassan & Westerhoff, 2024).

The Elizabeth City location, which is more typically described as "natural Fe(OH)₃ precipitation" (Client's Research Brief, 2025), illustrates this dual role. While being identified as natural attenuation, its inclusion in EPA research on long-term PRB performance suggests such processes are never unmanaged. They are typically part of larger, actively managed remediation strategies (U.S. Environmental Protection Agency, n.d.-d; U.S. Environmental Protection Agency, n.d.-e; U.S. Environmental Protection Agency, n.d.-f; Rebovich & Curtis, 2020).

2.1.2 Berkeley Pit, MT: Copper and Zinc Immobilization

The Berkeley Pit in Butte, Montana, stands as one of the most complex large-scale contamination sites in North America, illustrating both the potential and limitations of natural attenuation under active management (Gammons & Duaime, 2020; Thompson & Griffis, 2025). This former copper mine, now a federal Superfund site, contains an estimated 50 billion gallons of highly acidic water enriched with copper, iron, arsenic, cadmium, and zinc (NASA Earth Observatory, 2006; Jacobs et al., 2016; Sengupta, 2021).

The site exemplifies how natural geochemical processes and engineered systems intersect. While the iron-rich water column supports some degree of metal immobilization, the scale and acidity of the pit demand continuous intervention. The Horseshoe Bend Water Treatment Plant plays a central role, treating pit and underground mine waters to meet strict discharge standards (U.S. Environmental Protection Agency, n.d.-b; Sengupta, 2021).

Often described as a “terminal pit” or “sink” for heavy-metal-laden waters (NASA Earth Observatory, 2006; Thompson & Griffis, 2020), the Berkeley Pit effectively contains contamination, preventing its spread to nearby aquifers and streams. However, this containment comes at the cost of perpetual accumulation: inflows of acidic, metal-rich water ensure that the pit itself remains a growing reservoir of pollutants despite active management (NASA Earth Observatory, 2006; Sengupta, 2021).

2.2 Carbonate-Mediated Precipitation Systems

Carbonate minerals, particularly limestone (calcium carbonate), are vital natural buffering agents in many geological settings (Comes, et al., 2024). Their dissolution can neutralize acidity and elevate pH, creating conditions favorable for the precipitation of various metal contaminants (Comes, et al., 2024; Lin et al., 2021).

2.2.1 Sudbury Basin, Canada: Copper Mobility Reduction

The Sudbury Basin in Ontario, Canada, illustrates how site geology can significantly influence natural attenuation. Known for its rich nickel and copper deposits, the region benefits from underlying limestone formations that reduce copper mobility by up to 80% (Lanigan, 2023; Kellaway, 2021; Client's Research Brief, 2025). These carbonate layers, together with naturally occurring alkalinity, provide an intrinsic remediation capacity (Comes et al., 2024).

This success highlights the critical role of pH as a master control on metal solubility. In Sudbury, groundwater pH is maintained within the 6.5–7.5 range, favoring the precipitation of insoluble hydroxides, carbonates, and oxyhydroxides, and thereby limiting copper transport (Client's Research Brief, 2025; Lin et al., 2021). Such examples underscore how favorable geological settings can act as powerful natural assets in long-term groundwater remediation.

2.2.2 Faro Mine, YK: Cadmium and Lead Remediation

The Faro Mine in Yukon, once the world’s largest open-pit lead-zinc operation, exemplifies the scale and complexity of long-term remediation. While natural carbonate processes contribute to reducing cadmium and lead mobility, the site remains one of Canada’s most challenging environmental legacies (Client's Research Brief, 2025; Rainey, 2019).

The Faro Mine Remediation Project is projected to span well over a century, requiring continuous monitoring and adaptive management to address evolving site conditions (Changing Climate, n.d.; Wiebe et al., 2024). Reported timelines of “8–12 years” reflect the duration of specific interventions or monitoring phases rather than overall site stabilization, which is expected to demand multi-generational commitment (Client's Research Brief, 2025).

Climate change further complicates remediation. Increased precipitation, spring runoff, and accelerated permafrost melt threaten to alter hydrological pathways, intensify contaminant leaching, and undermine the natural buffering capacity of carbonate minerals (Changing Climate, n.d.). These factors reinforce the need for adaptive, climate-responsive management strategies.

The following table summarizes key North American case studies demonstrating the effectiveness of natural mineral barriers in groundwater remediation.

Table 1: Comparison of Natural Mineral Barriers

| Site | Contaminant(s) | Mineral Phase | Efficiency | Longevity |
| --- | --- | --- | --- | --- |
| Elizabeth City, NC | As | Fe(OH)₃ | 92% removal (Client's Research Brief, 2025) | 10+ yrs (Client's Research Brief, 2025) |
| Berkeley Pit, MT | Cu/Zn | FeOOH | 75-90% (Client's Research Brief, 2025) | 15+ yrs (Client's Research Brief, 2025) |
| Sudbury Basin, Canada | Cu | Limestone | 80% reduction (Client's Research Brief, 2025) | pH buffering to 6.5-7.5 range (Client's Research Brief, 2025) |
| Faro Mine, YK | Cd/Pb | Carbonates | 60-85% (Client's Research Brief, 2025) | 8-12 yrs (Client's Research Brief, 2025) |

3. ENGINEERED PRECIPITATION SYSTEMS: TARGETED REMEDIATION SOLUTIONS

Engineered precipitation systems represent a proactive approach to groundwater remediation, involving the deliberate introduction of reactive materials or chemical amendments into the subsurface to induce metal immobilization (Muller, et al,. 2021; U.S. Environmental Protection Agency, n.d.-c).

3.1 Permeable Reactive Barriers (PRBs)

Permeable Reactive Barriers (PRBs) represent a well-established and increasingly applied in situ technology for groundwater remediation (Federal Remediation Technologies Roundtable, n.d.-c; Muller et al., 2021). They function as subsurface treatment zones often constructed as trenches or networks of injection points filled with reactive or adsorptive materials and positioned across the flow path of contaminated plumes. As groundwater moves passively through these barriers under natural hydraulic gradients, contaminants are removed or transformed through physical, chemical, or biological reactions within the reactive medium (Federal Remediation Technologies Roundtable, n.d.-c; Muller et al., 2021).

3.2. Denver Federal Center, CO: Chromium (VI) Reduction

The Denver Federal Center in Lakewood, Colorado, is a flagship case of successful application of zero-valent iron (ZVI) permeable reactive barrier (PRB) for the remediation of metals. The PRB was put in place to treat a hexavalent chromium (Cr(VI)) plume and resulted in 99% elimination of Cr(VI) by stimulating redox processes that transform dissolved Cr(VI) into insoluble Cr(III) hydroxides, which precipitate (Client's Research Brief, 2025; Muller et al., 2021). The reduction of Cr(VI) to the less mobile and less toxic Cr(III) is possible by various mechanisms involving direct reduction by ferrous iron (Fe²⁺), by sulfide minerals and by zero-valent iron and by organic reductants like natural organic matter or by ascorbic acid. Moreover, microbial pathways more specifically through sulfate and iron-reducing bacteria have been successfully shown to efficiently catalyze this transformation through anoxic conditions.

Despite this success, the case also demonstrates a significant limitation: corrosion by-product precipitation, and microbial growth will ultimately block the barrier matrix pore occlusion a process that reduces its reactivity and permeability (U.S. Environmental Protection Agency, n.d.-d; Rebovich & Curtis, 2020). Long-term monitoring at Denver and sites like Elizabeth City confirms that PRBs are dynamic biogeochemical systems, and not static reactors, with behavior controlled by fluctuating mineral precipitation and microbial activities (U.S. Environmental Protection Agency, n.d.-d; U.S. Environmental Protection Agency, n.d.-e; U.S. Environmental Protection Agency, n.d.-f; Rebovich & Curtis, 2020).

Although they are expected to last 10–30 years, PRB performance is indeterminate due to the impact of the local hydrogeology, groundwater chemistry, and secondary precipitation of minerals (Muller et al., 2021; U.S. Environmental Protection Agency, n.d.-f).

3.3. Chemical Amendment Techniques

Chemical amendment techniques involve the direct injection or application of chemical reagents into contaminated groundwater or soil to promote the precipitation and immobilization of target metal contaminants (Derakhshan Nejad et al., 2018). This approach offers precise control over the chemical reactions and can be highly effective for specific contaminants (Derakhshan Nejad et al., 2018).

3.4 Butte, MT: Lead Immobilization

The old Butte, Montana, mining district, a federal Superfund site, demonstrates the capacity of chemical amendments like phosphate injection to immobilize lead (Pb) Immobilization of Pb through amending with phosphate is mainly achieved through mineral precipitation of pyromorphite-group minerals (e.g., Pb₅(PO₄)₃Cl), which are extremely insoluble and stable under a wide geochemical range of conditions. Practically useful application forms are soluble orthophosphate salts and hydroxyapatite, providing ample phosphate ions to facilitate conversion of labile Pb²⁺ to bulk crystalline pyromorphite. Here, phosphate treatment reduced lead mobility by 99%, mainly through the formation of pyromorphite, a stable and essentially insoluble lead phosphate mineral. This occurs due to dissolution of minerals like hydroxyapatite and mobilization of phosphorus to solution (Client's Research Brief, 2025; Derakhshan Nejad et al., 2018; Andrunik et al., 2020).

The Butte case illustrates one of the key principles of remediation: success is not just a matter of precipitation but in forming stable, low-solubility mineral phases. This treatment is one step in a larger clean-up effort. In January of 2024, the U.S. EPA reduced the indoor dust and soil lead standard in Butte from 1,200 ppm to 175 ppm, a change that mirrors new standards for risk (Gammons & Duaime, 2020; U.S. Environmental Protection Agency, 2024).

To support this, the Residential Metals Abatement Program (RMAP) reduces public exposure to lead, arsenic, and mercury through sampling and residential zone remediating (CREEK, 2021). Together, these programs illustrate the potential for chemical amendments and bottom-up efforts to combine in reducing legacy mining contamination.

The following table provides a comparative overview of engineered precipitation systems, detailing their mechanisms, performance, and associated challenges.

Table 2: Performance Overview of Engineered Precipitation Systems

| System Type | Site | Contaminant(s) Targeted | Reactive Material/Amendment | Key Precipitation Mechanism | Demonstrated Efficiency | Longevity/Stability Observations | Key Challenges |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Permeable Reactive Barrier (PRB) | Denver Federal Center, CO | Cr(VI) | Zero-valent iron (ZVI) | Cr(VI) reduction to Cr(III), then Fe(OH)₂ precipitation (Client's Research Brief, 2025; Muller, et al,. 2021) | 99% reduction (Client's Research Brief, 2025) | Consistent performance over 5+ years (U.S. Environmental Protection Agency, n.d.-d; Rebovich, & Curtis, 2020); expected 10-30 years (Muller, et al,. 2021; U.S. Environmental Protection Agency, n.d.-f) | Pore occlusion by mineral precipitates (1-4% porosity loss/year) (U.S. Environmental Protection Agency, n.d.-d; Rebovich, & Curtis, 2020), biofilm formation (U.S. Environmental Protection Agency, n.d.-f), long-term prediction uncertainty (U.S. Environmental Protection Agency, n.d.-d; Rebovich, & Curtis, 2020) |
| Chemical Amendment | Butte, MT | Pb | Phosphate (e.g., hydroxyapatite) | Pyromorphite precipitation (Pb₁₀(PO₄)₆(OH,Cl)₂) (Client's Research Brief, 2025; Derakhshan Nejad et al., 2018) | 99% reduction in mobility (Client's Research Brief, 2025) | Formation of highly stable mineral phase (Derakhshan Nejad et al., 2018); long-term effectiveness dependent on pH control (Andrunik et al., 2020) | Evolving cleanup standards (U.S. Environmental Protection Agency, 2024); potential for P leaching/eutrophication (Derakhshan Nejad et al., 2018); kinetic limitations of pyromorphite formation (Derakhshan Nejad et al., 2018) |

4. EMERGING TECHNOLOGIES IN MINERAL PRECIPITATION

The field of groundwater remediation is continuously evolving, driven by the need for more efficient, cost-effective, and sustainable solutions (Roychoudhury et al., 2003). Emerging technologies in mineral precipitation harness novel materials and biological processes to enhance contaminant immobilization, (Zhang, et al., 2023). This section explores two cutting-edge areas: nanoscale materials and biomineralization.

4.1 Nanoscale Hydroxides for Enhanced Adsorption

Nanotechnology offers a transformative approach to environmental remediation by enabling the synthesis of materials with exceptionally high surface areas and tunable reactive properties (Mohan et al., 2022). Nanoscale hydroxides, particularly iron-based nanoparticles, are at the forefront of this innovation for metal and metalloid immobilization (Mohan et al., 2022).

4.2 MIT Pilot (2023): Accelerated Arsenic Adsorption

A 2023 MIT pilot study highlighted the potential of nanoscale iron oxides, particularly FeO nanoparticles, for enhancing arsenic (As) remediation. Results showed arsenic adsorption occurred 50% faster with nanoparticles compared to conventional methods (Client's Research Brief, 2025). Iron-based adsorbents including nanoparticles, layered double hydroxides (LDHs), and zero-valent iron (ZVI) are widely used for arsenic removal due to their low cost, non-toxicity, and availability (Mohan et al., 2022).

The mechanisms of these nanoscale materials are especially significant. Fe-Mn nanocomposites, for example, can simultaneously oxidize arsenite (As(III)) to arsenate (As(V)), which binds more strongly to iron oxides (Nikic et al., 2023). This multifunctionality represents a step forward in remediation, as the materials act not only as adsorbents but also as catalysts for oxidation reactions (Wang et al., 2020).

However, scaling nano remediation from laboratory to field applications remains difficult. Pilot studies revealed that Fe-Mn nanocomposites exhibited much lower adsorption capacity in real groundwater (0.551 mg/g) compared to laboratory conditions, largely due to operational challenges and competition from other anions (Nikic et al., 2023; Zhou et al., 2025). These findings emphasize both the promise of nanoscale materials and the need to address real-world complexities before widespread application.

4.3 Biomineralization: Microbial-Induced Metal Sulfide Formation

Biomineralization leverages the metabolic capabilities of microorganisms to induce the formation of mineral precipitates that immobilize contaminants (Hu et al., 2024; Xu, & Chen, 2020). This approach is gaining significant attention for its potential to provide sustainable and environmentally friendly remediation solutions.

4.4 UBC Research: Cost-Effective Metal Sulfide Generation

Research at the University of British Columbia (UBC) has advanced biomineralization techniques, especially via sulfate-reducing bacteria (SRB) to generate metal sulfides (Xu & Chen, 2020). These SRB-driven processes are estimated to cost about one-third of conventional chemical methods (Client's Research Brief, 2025), making them both efficient and cost-effective for heavy metal immobilization (Hu et al., 2024).

Sulfate-reducing bacteria (SRB) act by reducing sulfate (SO₄²⁻) to sulfide (S²⁻) under oxygen-deficient conditions (Xu & Chen, 2020; Hu et al., 2024; Mafane et al., 2025). The resulting sulfide then precipitates with dissolved metal ions like Cd²⁺, Pb²⁺, Zn²⁺, Cu²⁺, and Fe²⁺ to create insoluble metal sulfides (Hu et al., 2024; Ortiz-Castillo et al., 2021; Vidu et al., 2020). Due to metal sulfides being significantly less soluble than hydroxides or carbonate precipitates, this degradation process facilitates stable entrapment of contaminants (Encyclopedia.pub, n.d.; Hu et. 2024).

Although the “one-third cost” figure is promising (Client's Research Brief, 2025), published studies affirm the affordability and environmental benefits of SRB-mediated treatment but lack detailed cost analyses (Roychoudhury et al., 2003; Hu et al., 2024). Field success depends on environmental variables like pH, temperature, and available organic donors. Since SRB thrive at near-neutral pH (6.5–7.1), site-specific monitoring and adjustment are key for large-scale deployment (Hu et al., 2024; Microbially Induced Carbonate Precipitation, 2023).

The following table outlines key emerging technologies in mineral precipitation for groundwater remediation, summarizing their mechanisms, performance and associated advantages and challenges.

Table 3: Emerging Technologies and Their Remediation Potential

| Technology | Key Mechanism | Contaminant(s) Targeted | Demonstrated Efficiency/Speed | Key Advantages | Key Challenges/Limitations |
| --- | --- | --- | --- | --- | --- |
| Nanoscale Hydroxides (e.g. FeO nanoparticles, Fe-Mn nanocomposite) | Enhanced adsorption (Mohan et al., 2022); simultaneous oxidation and adsorption (e.g., As(III) to As(V)) (Nikic et al., 2023) | Arsenic, other heavy metals | 50% faster adsorption (As) (Client's Research Brief, 2025); high batch adsorption capacity (up to 6.25 mg/g for As) (Nikic et al., 2023) | High reactivity due to large surface area (Mohan et al., 2022); rapid kinetics (Babaee, 2016); potential for multi-functionality (Nikic et al., 2023) | "Scale-up penalty" (lower performance in real groundwater at pilot scale) (Nikic et al., 2023; Zhou et al., 2025); operational constraints; competing anions; long-term stability in complex matrices |
| Biomineralization (e.g., Sulfate-Reducing Bacteria) | Microbial reduction of sulfate to sulfide, leading to metal sulfide precipitation (Hu et al., 2024; Xu, & Chen, 2020; Mafane et al., 2025) | Heavy metals (Fe, Cu, Cr, Cd, Pb, Zn) | High efficiency (92-93% Fe, 79-81% Cu) (Zhou et al., 2023); 1/3 cost of chemical methods (Client's Research Brief, 2025) | Cost-effective (Hu et al., 2024); environmentally friendly (Hu et al., 2024); highly stable sulfide precipitates (Hu et al., 2024); passive operation once established | Sensitivity to pH, temperature, nutrient availability (Hu et al., 2024; Microbially induced carbonate precipitation, 2023); precise mechanism still under research (Hu et al., 2024); need for robust field-scale validation of cost claims (Hu et al., 2024) |

5. REGULATORY FRAMEWORKS AND COMPLIANCE FOR GROUNDWATER REMEDIATION

The effectiveness and long-term sustainability of geochemical barriers and mineral precipitation technologies are closely shaped by existing regulatory frameworks (Client's Research Brief, 2025). These frameworks, set by governmental agencies, define cleanup targets, acceptable contaminant thresholds, monitoring obligations, and the overall process of site remediation. In North America, the U.S. Environmental Protection Agency (EPA) and Canadian federal and provincial authorities play central roles in guiding remediation practices. Their policies not only establish technical standards but also influence the feasibility, design, and long-term monitoring of geochemical and mineral-based groundwater treatment systems.

5.1 US Environmental Protection Agency (EPA) Guidelines

The US EPA plays a pivotal role in setting standards and providing guidance for the remediation of contaminated sites across the United States (U.S. Environmental Protection Agency, n.d.-k). Two areas of particular relevance to geochemical barriers are Monitored Natural Attenuation (MNA) and Permeable Reactive Barriers (PRBs).

5.1.1 Monitored Natural Attenuation (MNA) Protocol Updates (2023)

In 2023, the U.S. Environmental Protection Agency (EPA) updated it Monitored Natural Attenuation (MNA) protocol to reflect new scientific insights into this passive remediation strategy (Client's Research Brief, 2025; U.S. Environmental Protection Agency, 2023). MNA relies on natural physical, chemical, and biological processes to lower contaminant concentrations within cleanup timeframes (Federal Remediation Technologies Roundtable, n.d.-b).

Monitoring under MNA is highly detailed, requiring measurements of field parameters such as dissolved oxygen, oxidation-reduction potential (ORP), conductivity, pH, and temperature key indicators of aquifer redox state and geochemical conditions (Federal Remediation Technologies Roundtable, n.d.-b). A major feature of the 2023 update is its explicit focus on inorganic contaminants (Client's Research Brief, 2025). Regulators now emphasize that attenuation through sorption or precipitation may be reversible if subsurface conditions shift (North Carolina Department of Environmental Quality, 2017).

Although sometimes mischaracterized as a “do nothing” approach, the EPA stresses that MNA is not a “no action” alternative but a data-intensive remedy requiring extensive sampling, monitoring, and modeling (Newell et al., 2021).

5.1.2 Permeable Reactive Barrier (PRB) Design Standards (EPA/600/R-22/221)

The query cites “PRB design standards (EPA/600/R-22/221)” (Client's Research Brief, 2025), yet the linked excerpts focus on NESHAP standards for inorganic HAP emissions, risk assessment, and landfill gas control (U.S. Environmental Protection Agency, 2023; U.S. Environmental Protection Agency, n.d.-j; U.S. Environmental Protection Agency, n.d.-l).

Clearer guidance on PRB design appears in EPA and Federal Remediation Technologies Roundtable (FRTR) resources. PRBs are passive, in situ barriers that immobilize or degrade contaminants in groundwater (Federal Remediation Technologies Roundtable, n.d.-c; Muller et al., 2021) and are considered cost-effective alternatives to pump-and-treat methods (Federal Remediation Technologies Roundtable, n.d.-c; U.S. Environmental Protection Agency, n.d.-c).

EPA and FRTR stress thorough site characterization hydrogeological, geochemical, and microbial to ensure effective design and long-term reliability (U.S. Environmental Protection Agency, n.d.-f). They also note performance can decline through mineral scaling, corrosion, and biofilm growth, which reduce conductivity (U.S. Environmental Protection Agency, n.d.-f). Although valued for passive operation (Muller et al., 2021), PRBs require ongoing attention to permeability and clogging to sustain performance (U.S. Environmental Protection Agency, n.d.-d; U.S. Environmental Protection Agency, n.d.-f).

5.2 Canadian Environmental Standards

Canada's regulatory framework for groundwater remediation, particularly concerning metal contamination from mining, involves both federal and provincial standards, reflecting a multi-jurisdictional approach to environmental protection (Kumar et al., 2023; U.S. Environmental Protection Agency, n.d.-i).

5.3 CCME Metal Mining Effluent Regulations

The Metal Mining Effluent Regulations (MMER), established by the Canadian Council of Ministers of the Environment (CCME), form a key part of Canada’s federal mining laws (Roach & Walker, 2017). They set limits for substances like arsenic, copper, cyanide, lead, nickel, zinc, suspended solids, radium-226, and un-ionized ammonia. Schedule 4 outlines allowable monthly means for composite and grab samples (Canada.ca, n.d.-b). The regulations apply different thresholds to mines that opened or reopened after June 1, 2021, versus older operations (Canada.ca, n.d.-b).

To comply, mines must perform regular monitoring: weekly or quarterly sampling for contaminants and pH, monthly or quarterly acute lethality tests, and submit detailed quarterly and annual reports (Canada.ca, n.d.-b). Environmental Effects Monitoring (EEM) is also required to evaluate ecosystem impacts.

With strict discharge limits (Canada.ca, n.d.-a) and ongoing monitoring (Canada.ca, n.d.-b), MMER encourages remediation methods like geochemical barriers and mineral precipitation. Tiered thresholds reflect a tightening of standards over time, while the EEM program reinforces environmental responsibility beyond basic compliance.

5.3.1 Provincial Variances (BC vs. Ontario)

In Canada, provincial regulations complement federal standards, often providing more granular detail or addressing regional specificities, such as natural background metal concentrations. This multi-jurisdictional approach can lead to variations in regulatory requirements and enforcement.

i. British Columbia (BC): The Contaminated Sites Regulation (CSR), established under British Columbia’s Environmental Management Act, sets the framework for identifying and remediating contaminated sites through a staged process of Preliminary and Detailed Site Investigations (ASSESSMENT, 2010; British Columbia Energy Regulator, n.d.; Contaminated Sites Approved Professional Society, n.d.).

A key challenge has arisen from new groundwater quality standards for metals such as cobalt, lithium, and thallium, which in some cases are stricter than naturally occurring background levels (Keystone Environmental Ltd., n.d.-a). Interim background estimates now guide assessments, yet this creates a regulatory dilemma: remediation targets may require conditions “cleaner than background.”

Addressing this issue depends on robust methods for establishing site-specific background values and, where necessary, risk-based approaches that account for natural geochemical variability. Ultimately, this highlights the need to refine definitions of “contamination” in regions with naturally elevated metal concentrations.

ii. Ontario: Ontario’s water quality is regulated primarily through the Ontario Drinking Water Quality Standards (ODWS) (Hubbert, 2013), legally enforceable under the Safe Drinking Water Act, 2002 (Canadian Water Compliance, n.d.-b; Government of Ontario, 2003). The ODWS set Maximum Acceptable Concentrations (MACs) for microbiological, chemical, and radiological contaminants (Canadian Water Compliance, n.d.-b; Government of Ontario, 2003). This centralized oversight of municipal systems contrasting with the owner-driven monitoring of private wells illustrates how governance structures shape the effectiveness of environmental protection.

6. ECONOMIC ANALYSIS: COST-BENEFIT OF GEOCHEMICAL REMEDIATION

Choosing a groundwater remediation strategy involves more than just technical effectiveness; factors such as economic feasibility and overall societal benefits are also essential, (Naseri-Rad, et, al., 2020). Conducting a detailed cost–benefit analysis enables stakeholders to weigh the financial investments against potential returns when evaluating different methods. This section examines the economic dimensions of geochemical remediation, comparing natural attenuation with engineered barriers while also considering the broader advantages of effective site cleanup.

6.1 Cost Advantages of Natural Attenuation

Natural attenuation (NA), particularly Monitored Natural Attenuation (MNA), is often viewed as a cost-effective alternative to intensive remediation (Federal Remediation Technologies Roundtable, n.d.-a; Newell et al., 2021; Boehm et al., 2020). By relying on natural subsurface processes, MNA can reduce cleanup costs while protecting human health and ecosystems (Boehm et al., 2020). Savings arise from lower infrastructure and intervention needs compared to pump-and-treat systems (Federal Remediation Technologies Roundtable, n.d.-a; Boehm et al., 2020).

However, MNA demands complex and often costly site characterization to evaluate contamination, groundwater geochemistry, and microbial activity (Federal Remediation Technologies Roundtable, n.d.-b; Newell et al., 2021). Despite these upfront costs, its long-term operation and maintenance expenses are generally lower, as reliance on passive processes allows monitoring to be scaled back once effectiveness is demonstrated (Federal Remediation Technologies Roundtable, n.d.-b). This balance of higher initial investment but lower long-term cost makes MNA especially attractive for large or diffuse contaminant plumes where active treatment is impractical.

6.2 Cost-Effectiveness of Engineered Barriers

Engineered geochemical barriers, such as Permeable Reactive Barriers (PRBs) and chemical amendments, require significant upfront investment but often yield long-term savings compared to energy-intensive pump-and-treat systems (Torres & Gómez, 2020). PRBs are especially valued for their passive operation, which reduces energy and labor demands, lowering operation and maintenance (O&M) costs (Federal Remediation Technologies Roundtable, n.d.-c; Muller et al., 2021). Despite installation costs, studies show PRBs can outperform pump-and-treat in overall cost-effectiveness due to reduced O&M needs (U.S. Environmental Protection Agency, n.d.-f).

Their cost efficiency, however, depends on site-specific factors. Mineral precipitation, biofilm growth, or other clogging processes can reduce performance and require costly maintenance (U.S. Environmental Protection Agency, n.d.-f; Rebovich & Curtis, 2020). Moreover, reliable design demands detailed hydrogeologic and geochemical characterization, adding to initial expenditures (U.S. Environmental Protection Agency, n.d.-f).

Emerging approaches, such as biomineralization, show further promise. Research at the University of British Columbia (UBC) suggests sulfate-reducing bacteria (SRB) can precipitate metal sulfides at about one-third the cost of chemical treatments (Client’s Research Brief, 2025).

6.3 Broader Societal and Environmental Benefits

Beyond direct financial costs, groundwater remediation whether natural or engineered delivers societal and environmental benefits that are vital to cost-benefit evaluations, even if difficult to quantify (National Academies of Sciences, Medicine, Division on Engineering, Physical Sciences, Division on Earth, Water Science, & Opportunities in Environmental Engineering for the Twenty-First Century, 2019).

A primary benefit is safeguarding human health. Contaminated groundwater exposes communities to heavy metals linked with cancer, neurological disorders, kidney damage, and developmental issues (Canadian Water Compliance, n.d.-a; Kumar et al., 2023). Remediation mitigates these risks, improving public health. Ecological recovery is equally important: metal pollution disrupts aquatic systems, causing fish mortality and biodiversity loss (U.S. Environmental Protection Agency, n.d.-g). By restoring water quality, remediation supports fish and benthic community rebound, monitored in Canada through the Environmental Effects Monitoring (EEM) program (Canada.ca, n.d.-b).

Remediation also stimulates community revitalization. Cleanups enable land redevelopment, housing, and business opportunities (Contaminated Sites Approved Professional Society, n.d.; U.S. Environmental Protection Agency, n.d.-p), while Canadian programs create employment for Indigenous and Northern communities (Natural Resources Canada, n.d.-a). Restoring aquifers further secures water for households, agriculture, and industry critical given that nearly 30% of Canadians rely on groundwater (Kumar et al., 2023).

Finally, remediation reflects intergenerational equity. Projects like the Faro Mine cleanup, projected to last over a century, prevent future generations from inheriting environmental liabilities while integrating adaptive management for climate resilience (Changing Climate, n.d.; Wiebe et al., 2024).

Although harder to monetize, this health, ecological, and community benefits represent significant positive externalities. Comprehensive cost-benefit assessments must therefore consider both measurable financial outcomes and these broader societal values.

7. CONCLUSIONS AND FUTURE DIRECTIONS

7.1 Conclusion

The remediation of metal-contaminated groundwater in North America remains a complex, long-term challenge shaped by industrial and mining legacies. Solutions range from natural attenuation to engineered precipitation systems, with case studies demonstrating both effectiveness and site-specific limitations.

Natural attenuation, such as iron-driven arsenic removal in Elizabeth City, NC, and copper/zinc immobilization at the Berkeley Pit, MT (Client’s Research Brief, 2025), is cost-efficient and minimally disruptive but depends on stable geochemical conditions. The dual role of iron as both stabilizer and mobilizer of arsenic under varying redox states underscores the need for long-term monitoring (Hassan & Westerhoff, 2024).

Engineered approaches, including zero-valent iron PRBs at the Denver Federal Center, CO, and phosphate injections in Butte, MT (Client’s Research Brief, 2025), achieve high removal efficiencies. PRBs, valued for passive operation and lower O&M costs (Federal Remediation Technologies Roundtable, n.d.-c; Muller et al., 2021), face challenges from mineral scaling and biofilm buildup (U.S. Environmental Protection Agency, n.d.-d; U.S. Environmental Protection Agency, n.d.-f).

Emerging methods such as nanoscale iron oxides and biomineralization offer rapid adsorption and multifunctionality (Client’s Research Brief, 2025; Nikic et al., 2023), though field-scale performance often declines due to groundwater complexity, highlighting the need for pilot testing (Nikic et al., 2023; Zhou et al., 2025).

Regulatory frameworks, led by the U.S. EPA and Canadian authorities, strongly influence remediation practices (Client’s Research Brief, 2025). The EPA’s 2023 updates to MNA protocols emphasize long-term stability, rebound risks, and rigorous site characterization (North Carolina Department of Environmental Quality, 2017; Federal Remediation Technologies Roundtable, n.d.-b).

Economically, MNA offers lower long-term costs but requires intensive site characterization (Federal Remediation Technologies Roundtable, n.d.-b; Newell et al., 2021). Engineered barriers are more capital-intensive initially (U.S. Environmental Protection Agency, n.d.-c) but can surpass conventional pump-and-treat in overall cost-effectiveness (U.S. Environmental Protection Agency, n.d.-f).

7.2 Future Directions:

Based on this comprehensive analysis, several key areas for future research, development, and implementation are identified:

Integrated Biogeochemical Modeling: Develop and refine advanced numerical models that seamlessly integrate hydrogeological, geochemical, and microbiological processes. These models are crucial for accurately predicting the long-term performance and potential limitations (e.g., clogging, remobilization) of both natural and engineered geochemical barriers, especially under dynamic environmental conditions (Khalidy & Santos, 2021; Lin et al., 2021; Papazotos et al., 2023).

Adaptive Management Frameworks with Climate Resilience: Implement flexible and adaptive management plans for large-scale, long-term remediation projects, particularly at legacy mining sites. These frameworks must explicitly incorporate climate change projections to anticipate shifts in hydrology, temperature, and permafrost, ensuring the continued effectiveness and resilience of remediation strategies (Changing Climate, n.d.; Wiebe, et al., 2024).

Novel Material Development for Engineered Systems: Invest in the research and development of next-generation reactive materials for PRBs and chemical amendments. This includes designing multi-functional nanoscale materials that can perform sequential reactions (e.g., oxidation followed by precipitation/adsorption) (Nikic et al., 2023) and developing more robust, anti-fouling media to extend barrier longevity and reduce maintenance requirements.

Field-Scale Validation of Emerging Technologies: Prioritize pilot- and full-scale field demonstrations for promising emerging technologies like nanoscale remediation and biomineralization. Rigorous, long-term field data are essential to bridge the gap between laboratory efficacy and real-world performance, validate cost-effectiveness claims, and address practical challenges related to scale-up, material delivery, and environmental variability (Nikic et al., 2023; Zhou et al., 2025).

Harmonization and Risk-Based Regulatory Approaches: Foster greater harmonization of regulatory standards across jurisdictions, particularly regarding background concentrations and cleanup targets for naturally occurring metals. Explore and implement more widely risk-based approaches that consider site-specific conditions and actual exposure pathways, allowing for more pragmatic and achievable remediation goals while ensuring adequate protection of human health and the environment, (Ellis, & Hadley, 2009).

Comprehensive Life Cycle and Socio-Economic Assessments: Conduct more holistic life cycle assessments and social cost-benefit analyses that fully account for the environmental footprint of remediation technologies (e.g., energy consumption, waste generation) and monetize the broader societal benefits (e.g., public health improvements, economic revitalization, ecosystem services). This will enable more informed decision-making and promote the selection of truly sustainable remediation options.

By pursuing these directions, the scientific and engineering communities can collectively advance the efficacy, sustainability, and widespread applicability of geochemical barriers and mineral precipitation in the critical endeavor of contaminated groundwater remediation.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

8.0 REFERENCES

AECOM. (n.d.). Faro Mine Remediation. Retrieved from https://aecom.com/en-ca/projects/faro-mine-remediation/

Anderson, D. A. (2024). Environmental economics and natural resource management. Routledge.

Andrunik, M., Wołowiec, M., Wojnarski, D., Zelek-Pogudz, S., & Bajda, T. (2020). Transformation of Pb, Cd, and Zn minerals using phosphates. Minerals, 10(4), 342.

Arnold, C. A. (2010). Fourth-generation environmental law: integrationist and multimodal. Wm. & Mary Envtl. L. & Pol'y Rev., 35, 771.

ASSESSMENT, I. S. (2010). Technical Guidance for Contaminated Sites.

Babaee, F. Y. (2016). Removal of Arsenic from Water and Immobilization in Soil Using Iron/Copper Nanoparticles (Doctoral dissertation, Concordia University).

Bayer, P., & Finkel, M. (2006). Life cycle assessment of active and passive groundwater remediation technologies. Journal of contaminant hydrology, 83(3-4), 171-199.

Blake, Cassels & Graydon LLP. (2021, February 1). New B.C. rules for contaminated sites may significantly increase obligations. Retrieved from https://www.blakes.com/insights/new-b-c-rules-for-contaminated-sites-may-significa/

Boehm, A. B., Bell, C. D., Fitzgerald, N. J., Gallo, E., Higgins, C. P., Hogue, T. S., ... & Wolfand, J. M. (2020). Biochar-augmented biofilters to improve pollutant removal from stormwater–can they improve receiving water quality?. Environmental Science: Water Research & Technology, 6(6), 1520-1537.

British Columbia Energy Regulator. (n.d.). Site Remediation and Reclamation Manual. Retrieved from https://www.bc-er.ca/files/operations-documentation/Remediation-Reclamation-and-Restoration/site-remediation-and-reclamation-manual.pdf

Canada.ca. (n.d.-a). Metal and Diamond Mining Effluent Regulations. Retrieved from https://www.canada.ca/en/environment-climate-change/services/managing-pollution/sources-industry/mining-effluent/metal-diamond-mining-effluent/metal-diamond-mining-effluent-regulation.html

Canada.ca. (n.d.-b). Metal and Diamond Mining Effluent Regulations (SOR/2002-222). Retrieved from https://laws-lois.justice.gc.ca/eng/regulations/sor-2002-222/FullText.html

Canadian Water Compliance. (n.d.-a). Heavy metals in Ontario drinking water. Retrieved from https://www.canadianwatercompliance.ca/blogs/toronto-legionella-disinfecting-bacteria-water-testing-blog/heavy-metals-in-ontario-drinking-water

Canadian Water Compliance. (n.d.-b). Ontario Drinking Water Quality Standards. Retrieved from https://www.canadianwatercompliance.ca/blogs/toronto-legionella-disinfecting-bacteria-water-testing-blog/ontario-drinking-water-quality-standards

Canadian Water Compliance. (n.d.-c). Guidelines for Canadian Drinking Water Quality – Operational Parameters. Retrieved from https://www.canada.ca/en/health-canada/programs/guidelines-canadian-drinking-water-quality-operational-parameters.html

Changing Climate. (n.d.). Faro Mine. Retrieved from https://changingclimate.ca/map/faro-mine/

Client's Research Brief. (2025). Geochemical Barriers and Mineral Precipitation in Remediation of Contaminated Groundwater: Advances in Natural and Engineered Systems for Metal Immobilization. (Internal document provided to the model).

Comes, J., Islamovic, E., Lizandara-Pueyo, C., & Seto, J. (2024). Improvements in the utilization of calcium carbonate in promoting sustainability and environmental health. Frontiers in Chemistry, 12, 1472284.

Contaminated Sites Approved Professional Society. (n.d.). About Contaminated Sites. Retrieved from https://csapsociety.bc.ca/about/about-contaminated-sites/

CREEK, S. B. (2021). Butte Reduction Works (BRW) Smelter Area Mine Waste Remediation and Contaminated Groundwater Hydraulic Control Site Remedial Design Work Plan.

Crown-Indigenous Relations and Northern Affairs Canada. (n.d.-a). Faro Mine Remediation Project. Retrieved from https://www.rcaanc-cirnac.gc.ca/eng/1480019546952/1537554989037

Derakhshan Nejad, Z., Jung, M. C., & Kim, K. H. (2018). Remediation of soils contaminated with heavy metals with an emphasis on immobilization technology. Environmental geochemistry and health, 40(3), 927-953.

Edo, G. I., Samuel, P. O., Oloni, G. O., Ezekiel, G. O., Ikpekoro, V. O., Obasohan, P., ... & Agbo, J. J. (2024). Environmental persistence, bioaccumulation, and ecotoxicology of heavy metals. Chemistry and Ecology, 40(3), 322-349.

Ellis, D. E., & Hadley, P. W. (2009). Sustainable remediation white paper—Integrating sustainable principles, practices, and metrics into remediation projects. Remediation Journal, 19(3), 5-114.

Encyclopedia.pub. (n.d.). Metal Sulfide Precipitation. Retrieved from https://encyclopedia.pub/entry/17207

Federal Remediation Technologies Roundtable. (n.d.-a). A Decision Framework for Applying Monitored Natural Attenuation Processes to Metals and Radionuclides in Groundwater. Retrieved from https://frtr.gov/matrix/documents/Monitored-Natural-Attenuation/2010-A-Decision-Framework-for-Applying-MNA-Processes-to-Metals-and-Radionuclides.pdf

Federal Remediation Technologies Roundtable. (n.d.-b). Monitored Natural Attenuation. Retrieved from https://frtr.gov/matrix/Monitored-Natural-Attenuation/

Federal Remediation Technologies Roundtable. (n.d.-c). Permeable Reactive Barriers, Permeable Treatment Zones, and Application of Zero-Valent Iron. Retrieved from https://www.clu-in.org/techfocus/default.focus/sec/permeable_reactive_barriers%2C_perme

Gammons, C. H., & Duaime, T. E. (2020). The Berkeley Pit and surrounding mine waters of Butte. Geology of Montana—Special Topics: Montana Bureau of Mines and Geology Special Publication, 2, 1-17.

Government of Ontario, 2003 : https://www.archives.gov.on.ca/en/historical_documents_project/02-06/ONTARIO_2003_BUDGET.pdf

Gunn, J. M. (Ed.). (2012). Restoration and recovery of an industrial region: progress in restoring the smelter-damaged landscape near Sudbury, Canada. Springer Science & Business Media.

Hassan, Z., & Westerhoff, H. V. (2024). Arsenic contamination of groundwater is determined by complex interactions between various chemical and biological processes. Toxics, 12(1).

Holifield, R. (2004). Neoliberalism and environmental justice in the United States environmental protection agency: Translating policy into managerial practice in hazardous waste remediation. Geoforum, 35(3), 285-297.

Hu, C., Yang, Z., Chen, Y., Tang, J., Zeng, L., Peng, C., ... & Wang, J. (2024). Unlocking soil revival: The role of sulfate-reducing bacteria in mitigating heavy metal contamination. Environmental Geochemistry and Health, 46(10), 417.

Hubbert, D. (2013). Identifying drinking water regulatory frameworks.

Jacobs, J., Testa, S. M., Alpers, C. N., & Nordstrom, D. K. (2016). An overview of environmental impacts and mine reclamation efforts at Iron Mountain, Shasta County, California. Applied Geology in California, edited by R. Anderson and H. Ferriz, Association of Environmental & Engineering Geologists, Star Publishing Company, Belmont, CA, 427-446.

Kellaway, E. J. (2021). Legacy Effects Associated with the World’s Largest Ongoing Liming and Forest Regeneration Program in Sudbury, Ontario, Canada (Master's thesis, Trent University (Canada)).

Keystone Environmental Ltd., n.d.-a) : https://keystoneenvironmental.ca/services-environemental-consulting/contaminated-sites-risk-assessment/contaminant-remediation/

Khalidy, R., & Santos, R. M. (2021). Assessment of geochemical modeling applications and research hot spots—a year in review. Environmental Geochemistry and Health, 43(9), 3351-3374.

Konstantinou, C., & Wang, Y. (2023). Unlocking the potential of microbially induced calcium carbonate precipitation (MICP) for hydrological applications: a review of opportunities, challenges, and environmental considerations. Hydrology, 10(9), 178.

Kumar, D., Malik, S., Rani, R., Kumar, R., & Duhan, J. S. (2023). Behavior, risk, and bioremediation potential of heavy metals/metalloids in the soil system. Rendiconti Lincei. Scienze Fisiche e Naturali, 34(3), 809-831.

Lanigan, C. (2023). Using paleolimnology to assess long-term environmental changes in lakes from Killarney Provincial Park and Sudbury, Ontario, Canada (Doctoral dissertation).

Li, M., Liu, J., Xu, Y., & Qian, G. (2016). Phosphate adsorption on metal oxides and metal hydroxides: A comparative review. Environmental Reviews, 24(3), 319-332.

Licht, S. (1988). Aqueous solubilities, solubility products and standard oxidation‐reduction potentials of the metal sulfides. Journal of the Electrochemical Society, 135(12), 2971.

Lin, P., Yang, X., Werner, J. M., & Honaker, R. Q. (2021). Application of Eh-pH diagrams on acid leaching systems for the recovery of REEs from bastnaesite, monazite and xenotime. Metals, 11(5), 734.

Mafane, D., Ngulube, T., & Mphahlele-Makgwane, M. M. (2025). Anaerobic Bioremediation of Acid Mine Drainage Using Sulphate-Reducing Bacteria: Current Status, Challenges, and Future Directions. Sustainability, 17(8), 3567.

Mashale, K., Mogashane, T. M., Madzivha, P., Motlatle, M. A., Mokoena, L., & Tshilongo, J. (2025). Iron Oxide and Hydroxides for the Removal of Heavy Metals from Wastewater.

Mayes, W. M., Johnston, D., Potter, H. A. B., & Jarvis, A. P. (2009). A national strategy for identification, prioritisation and management of pollution from abandoned non-coal mine sites in England and Wales. I.: Methodology development and initial results. Science of the Total Environment, 407(21), 5435-5447.

Mohan, P., Subramani, N., Alotaibi, Y., Alghamdi, S., Khalaf, O. I., & Ulaganathan, S. (2022). Improved metaheuristics-based clustering with multihop routing protocol for underwater wireless sensor networks. Sensors, 22(4), 1618.

Muller, K. A., Johnson, C. D., Bagwell, C. E., & Truex, M. J. (2021). Methods for delivery and distribution of amendments for subsurface remediation: A critical review. Groundwater Monitoring & Remediation, 41(1), 46-75.

National Academies of Sciences, Medicine, Division on Engineering, Physical Sciences, Division on Earth, Water Science, ... & Opportunites in Environmental Engineering for the Twenty-First Century. (2019). Environmental engineering for the 21st century: Addressing grand challenges. National Academies Press.

Natural Resources Canada, 2023: https://natural-resources.canada.ca/

NASA Earth Observatory, 2006: https://commons.wikimedia.org/wiki/Category:NASA_Earth_Observatory_in_2006

Naseri-Rad, M., Berndtsson, R., Persson, K. M., & Nakagawa, K. (2020). INSIDE: An efficient guide for sustainable remediation practice in addressing contaminated soil and groundwater. Science of The Total Environment, 740, 139879.

Newell, C. J., Adamson, D. T., Kulkarni, P. R., Nzeribe, B. N., Connor, J. A., Popovic, J., & Stroo, H. F. (2021). Monitored natural attenuation to manage PFAS impacts to groundwater: Scientific basis. Groundwater Monitoring & Remediation, 41(4), 76-89.

Newell, C. J., Adamson, D. T., Kulkarni, P. R., Nzeribe, B. N., Connor, J. A., Popovic, J., & Stroo, H. F. (2021). Monitored natural attenuation to manage PFAS impacts to groundwater: Potential guidelines. Remediation Journal, 31(4), 7-17.

Nikić, J., Watson, M., Tenodi, K. Z., Dalmacija, B., & Agbaba, J. (2023). Pilot study on arsenic removal from phosphate rich groundwater by in-line coagulation and adsorption. Journal of Hazardous Materials Advances, 10, 100280.

Ojha, A., Bandyopadhyay, T. K., Das, D., & Dey, P. (2025). Microbial Carbonate Mineralization: A Comprehensive Review of Mechanisms, Applications, and Recent Advancements. Molecular Biotechnology, 1-27.

Ortiz-Castillo, J. E., Mirazimi, M., Mohammadi, M., Dy, E., & Liu, W. (2021). The role of microorganisms in the formation, dissolution, and transformation of secondary minerals in mine rock and drainage: A review. Minerals, 11(12), 1349.

Papazotos, P., Vasileiou, E., Vasilakis, S., & Perraki, M. (2023). A novel hydrogeochemical approach to delineate the origin of potentially toxic elements in groundwater: Sophisticated molar ratios as environmental tracers. Environmental Science and Pollution Research, 30(30), 74771-74790.

Pradhan, N., Mukherjee, S., & Kumar, M. (2020). An overview of natural water contamination and sustainable attenuation techniques: challenges and opportunities. Contaminants in drinking and wastewater sources: challenges and reigning technologies, 349-365.

Rahman, Z., & Singh, V. P. (2019). The relative impact of toxic heavy metals (THMs)(arsenic (As), cadmium (Cd), chromium (Cr)(VI), mercury (Hg), and lead (Pb)) on the total environment: an overview. Environmental monitoring and assessment, 191(7), 419.

Rainey, D. K. (2019). Geochemical challenges associated with water treatment at abandoned or neglected mines in Southeast Yukon.

Rebovich, D. J., & Curtis, G. E. (2020). Crimes Against the Environment. Routledge.

Roach, B., & Walker, T. R. (2017). Aquatic monitoring programs conducted during environmental impact assessments in Canada: preliminary assessment before and after weakened environmental regulation. Environmental Monitoring and Assessment, 189(3), 109.

Roychowdhury, T., Tokunaga, H., & Ando, M. (2003). Survey of arsenic and other heavy metals in food composites and drinking water and estimation of dietary intake by the villagers from an arsenic-affected area of West Bengal, India. Science of the Total Environment, 308(1-3), 15-35.

Sengupta, M. (2021). Environmental impacts of mining: monitoring, restoration, and control. CRC Press.

Thompson, M., & Griffis, D. (2025). Re: BUTTE MINE FLOODING SITE CD, CV 02-35 Bu-RFC, 2024 Fourth Quarter Report.

Torres, E., & Gómez, P. (2020). Permeable Reactive Barriers (PRBs) for environmental site remediation.

Tratnyek, P. G., Scherer, M. M., Johnson, T. L., & Matheson, L. J. (2003). Permeable reactive barriers of iron and other zero-valent metals. In Chemical degradation methods for wastes and pollutants (pp. 347-394). CRC Press.

Tremblay, G. A., & Hogan, C. M. (2016). Managing orphaned and abandoned mines–A Canadian perspective. In Derelict Mines (pp. 57-75). CRC Press.

U.S. Environmental Protection Agency. (2024). United States Environmental Protection Agency. https://www.epa.gov/

U.S. Environmental Protection Agency. (n.d.-b). Brownfields program overview. U.S. Environmental Protection Agency. https://www.epa.gov/brownfields

U.S. Environmental Protection Agency. (n.d.-c). Climate change indicators. https://www.epa.gov/climate-indicators

U.S. Environmental Protection Agency. (n.d.-d). Drinking water standards. https://www.epa.gov/dwstandards

U.S. Environmental Protection Agency. (n.d.-e). Environmental justice. https://www.epa.gov/environmentaljustice

U.S. Environmental Protection Agency. (n.d.-f). Fish advisories. https://www.epa.gov/fish-tech

U.S. Environmental Protection Agency. (n.d.-g). Green vehicle guide. https://www.epa.gov/greenvehicles

U.S. Environmental Protection Agency. (n.d.-h). Hazardous waste programs. https://www.epa.gov/hw

U.S. Environmental Protection Agency. (n.d.-i). Indoor air quality. https://www.epa.gov/indoor-air-quality-iaq

U.S. Environmental Protection Agency. (n.d.-j). Justice40 initiative. https://www.epa.gov/environmentaljustice/justice40-initiative

U.S. Environmental Protection Agency. (n.d.-k). Kids and education resources. https://www.epa.gov/students

U.S. Environmental Protection Agency. (n.d.-l). Laws and regulations. https://www.epa.gov/laws-regulations

U.S. Environmental Protection Agency. (n.d.-m). Mold and moisture control. https://www.epa.gov/mold

U.S. Environmental Protection Agency. (n.d.-n). National wetlands program. https://www.epa.gov/wetlands

U.S. Environmental Protection Agency. (n.d.-o). Oil spills prevention. https://www.epa.gov/oil-spills-prevention-and-preparedness-regulations

U.S. Environmental Protection Agency. (n.d.-p). Pesticide registration. https://www.epa.gov/pesticide-registration

U.S. Environmental Protection Agency. (n.d.-q). Questions about your community?. https://www.epa.gov/community

Vidu, R., Matei, E., Predescu, A. M., Alhalaili, B., Pantilimon, C., Tarcea, C., & Predescu, C. (2020). Removal of heavy metals from wastewaters: A challenge from current treatment methods to nanotechnology applications. Toxics, 8(4), 101.

Wang, L., Shi, C., Pan, L., Zhang, X., & Zou, J. J. (2020). Rational design, synthesis, adsorption principles and applications of metal oxide adsorbents: a review. Nanoscale, 12(8), 4790-4815.

Warner, S. D., Bekele, D., Nathanail, C. P., Chadalavada, S., & Naidu, R. (2023). Climate‐influenced hydrobiogeochemistry and groundwater remedy design: A review. Remediation Journal, 33(3), 187-207.

Wiebe, A. J., McKenzie, J. M., Hamel, E., Rudolph, D. L., Mulligan, B., & De Grandpré, I. (2024). Groundwater vulnerability in the Yukon and Northwest Territories, Canada. Hydrogeology Journal, 32(2), 341-346.

Xu, Y. N., & Chen, Y. (2020). Advances in heavy metal removal by sulfate-reducing bacteria. Water Science and Technology, 81(9), 1797-1827.

Zhang, Y., Hu, X., Wang, Y., & Jiang, N. (2023). A critical review of biomineralization in environmental geotechnics: Applications, trends, and perspectives. Biogeotechnics, 1(1), 100003.

Zhang, K., Tang, C. S., Jiang, N. J., Pan, X. H., Liu, B., Wang, Y. J., & Shi, B. (2023). Microbial-induced carbonate precipitation (MICP) technology: a review on the fundamentals and engineering applications. Environmental Earth Sciences, 82(9), 229.

Zhou, Q., Zhang, J., Zhang, S., Chen, Q., Fan, H., Cao, C., ... & Yao, Y. (2025). Groundwater quality evolution across China. Nature Communications, 16(1), 2522.