Review Article

Harnessing Lemna minor for Sustainable Wastewater Remediation: Advances, Challenges, and Future Prospects

Abstract:

Water pollution is a current issue of concern around the world and there is a dire need to develop sustainable, cost effective and eco-safe wastewater treatment solutions. Lemna minor (duckweed) is becoming famous among aquatic macrophytes as an effective phytoremediator because of its fast growth, adaptability and wide spectrum of pollutants that it removes. Decades of studies have confirmed that L. minor can have a significant positive water quality impact by increasing wastewater oxygenation, decreasing organic loads, and alleviating pathogens, and, at the same time, improving physicochemical characteristics including transparency and odor. Savings of up to 75% in biochemical oxygen demand (BOD5), 70% in chemical oxygen demand (COD), 72 % in ammonium, 82 % in phosphate, 67 % in total nitrogen, and 96 % in total phosphorus have been reported in a variety of wastewater streams, such as municipal wastewater, industrial wastewater, and agricultural wastewater. Moreover, L. minor has a good metal uptake efficiency and the removal percentage of iron is 91.7% with the accumulation of large proportions of copper, chromium, cadmium, lead and zinc.

In addition to remediation potential, L. minor provides an added value of production of protein-rich biomass (up to 75.24% crude protein), applicable to animal feed, bioenergy, and other bioproducts, which corresponds to the principles of the circular economy. Addition of duckweed into current treatment systems, including secondary clarifier tanks, or cultivated microbial and algal consortia also increases pollutant removal and decreases reliance on tertiary treatments. However, the variability of the performance under the influence of the wastewater composition (e.g. landfill leachate, nutrient imbalances) and the possibility of the re-release of nutrients during the decay of biomass pose significant issues. Quick biomass harvesting, optimization of the cultivation environment, and genetic or biotechnological advances are all potential tools to maximize efficiency. This review paper reveals both opportunities and challenges of phytoremediation by Lemna minor.

Keywords: Lemna minor, phytoremediation, heavy metals, nitrogen, phosporus and waste water remediation

1. Introduction:

Water pollution is a grave and limiting problem to aquatic ecosystems and human health across the world due to the unavoidable pace of industrialization, urbanization, and agriculture. Heavy metals, organic pollutants, and surplus nutrients pollute water and cause eutrophication, toxicity, and biodiversity loss (Asih et al., 2024; Elhamji et al., 2025).

Although traditional wastewater treatment methods are effective, they usually require a lot of energy, are expensive, and can produce secondary waste products, which means that more sustainable and friendly options need to be investigated (Khuzhjeev et al., 2024; Politaeva and Badenko, 2021). Aquatic remediation techniques has been noted as promising strategy for solving the problem of polluted water in different ecosystem. This eco-friendly method uses the natural capacity of microorganisms, plants and other species for mitigating contaminants in different ecosystem. Aquatic bioremediation may serve as cost-effective and simple alternative to traditional chemical method for water purification by exploiting biological process.

Phytoremediation is a cost-effective, eco-friendly and economically viable approach to reduce the impacts of various contaminants by utilising the plant and associated microbes (Khan et al., 2025). This biotechnological method uses inherent capacity of living organism like plants to remove, degrade, uptake and stabilize contaminants from different sources like soil, water and air (Wilschut et al., 2013).

Phyto-remediation capitalizes on the ability of plants to create biological pumps due to the power of photosynthesis and, therefore, removes pollutants or toxicants such as heavy metals and metalloids in the aquatic and terrestrial environment. This treatment offers a variety of benefits beyond just risk reduction by either eliminating or degrading the contaminant, but also provides an attractive alternative to conventional remediation techniques, particularly in locations with low to moderate concentrations. Phytoremediation, notably floating wetland treatments, have demonstrated a high potential to remove micropollutants in aquatic environments, control the landscape, and preserve the ecosystem conservation (Ahmad et al., 2024). Lemna minor commonly called Duckweed is one of the most suitable and promising candidate among the other aquatic microphyte which have high growth rate, high biomass production and remarkable contaminant uptake potential.

The excellent phytoremediation potential of aquatic microphyte, Lemna minor is due to its ability of rapid multiplication ( up to four days, as mentioned by Moga et al., 2019) and rapid vegetative growth which can accumulate large biomass in short period of time (Zhou et al., 2023). Duckweed is involved in following types of phytoremediation pathways: Phytodegradation, phytoaccumulation, rhizofilteration etc. Studies has revealed that the uptake of selenite and selenium in Lemna minuta showed the additional mechanism of ion channel proteins and transporters which can enter in the body beside the process of passive diffusion (Li et al., 2020). It is also highly efficient in uptake of nutrient like nitrogen and phosphate pollutants which explains the major role of in rehabilitative ecosystem of water reservoirs (Zhou et al., 2023). The high growth rate and wider adaptability of duckweed in converting waste into useful products render the model suitable for sustainable waste water management (Vulpe et al., 2025). This review critically examines the potential of Lemna minor as an aquatic bioremediatory, its mechanism and its effectiveness in treating different contaminates, challenges encounter and future prospects for its long term and large scale use in coming future.

2. Ecological and biological Characteristics of Lemna minor

Lemna minor or duckweed is a tiny, free growing, floating aquatic microphyte of Lemnaceae family. It is widely spreading aquatic plant with distinguished biological features and an excellent ecological significance, in particular with respect to aquatic environment and bioremediation. Lemna minor possesses highly simple and highly effective morphology with small fronds that are leaf-like and only one rootlet, which enables it to be free floating in the water surface. It replicates primarily in a vegetative manner and multiplies very much and with a high biomass within a short period. The sunlight and temperature are examples of environmental factors that influence this rapid growth and the most favorable temperature to grow is about 26 oC. Proper light intensity is also necessary in its productivity, which is reflected in the number of daughter fronds, chlorophyll-a quantity, biomass, and cover area (Ahmadi and Dursun, 2024; Nopriani et al., 2015; Pilar Arroyave, 2004; Vulpe et al., 2025).

L. minor has been biologically effective in terms of its photosynthetic capacity that has been adaptive to its aquatic habitat. It has also a coherent circadian clock system in its growing fronds and is cell-autonomous as well. A variety of biologically active compounds have been shown to accumulate biochemically in L. minor: these include polysaccharides and phenol carboxylic acids. Research papers have indicated the presence of essential oils and high degree of phenol, meaning that it can be applied in the role of an antioxidant. It can also achieve a biomass of 35 percent of protein based on its dry matter in nutrient-rich environments. In addition, it can be the origin of obtaining nanostructured biopolymers like the cellulose nanocrystals and the lignin nanoparticles, and can accumulate under stress starch, a raw material in biofuel production (Nikiforov et al., 2021).

The microbial communities are also symbionts of L. minor. It has been exemplified like diazotrophic bacteria like Azotobacter vinelandii which can induce plant growth by supplying nitrogen. The Pseudomonas bacteria may also colonize the surfaces of duckweed and influence its antioxidative activity and potentially enhance its biomass production. L. minor axenic (sterile) lines are prepared to be used in controlled experiments in laboratories devoid of any symbiotic bacteria or algae (Thomas and Eaton, 1996). Lemna minor is an ecologically useful organism due to its high distribution and unique biological characteristics and particularly in freshwater systems. L. minor is deemed to be a good bioindicator and plant bio-accumulator, thus a good environmental monitoring tool and water quality indicator. It is also vulnerable to the presence of other pollutants like heavy metals including copper (II), iron (II), lead, and zinc and hence used to bioindicate the presence of these elements. (Kastratović et al., 2013). It is believed to be a sensitive bioassay of assessing water toxicity as a result of increasing amounts of nutrients, including nitrogen compounds (nitrite, nitrate, ammonium), and the existence of algal blooms. It is also a good candidate to use in the monitoring of boron and ecological risk assessment of the aquatic ecosystems because of its high phototoxic sensitivity to boron (Gür et al., 2016).

3. The potential of Lemna minor (Duck weed) in waste water treatment:

One of the smallest water-borne flowering species, L. minor, reproduces rapidly, being able to increase its numbers to hundreds in just four days (Moga et al., 2019). It has the potential to grow vegetatively extremely quickly, allowing the creation of a large amount of biomass within relatively short time frames (Zhou et al., 2023). With such high growth rate, L. minor is able to absorb large amounts of pollutants (Sablii et al., 2019).

L. minor has a good potential of treating contaminants. It can be used in the elimination of lead, nickel, copper and cadmium. It has been reported that Lemna minor enhanced the removal of heavy metals in the wastewater (Janna, 2021). L. minor has also proved to be an appropriate host in the remediation of low level lead pollutants (Leblebici & Aksoy, 2010).

L. minor shows great opportunities to phytotransform organic contaminants, including petroleum hydrocarbons (Koesesakal et al., 2015). It has demonstrated an outstanding performance in the removal of faecal coliforms in municipal and industrial wastewater, and in some cases, 100 percent reduction has been achieved within the first five days. It has also been applied to the textile industry wastewater to remove its colour (Sivakumar et al., 2018).

Duckweed stands out as an important element of the restorative ecology of water bodies due to the presence of significant quantities of nitrogen and phosphate-containing contaminants (Zhou et al., 2023). It is able to absorb and transduce a variety of compounds and result to the purification of quality water of biogenic nitrogen and phosphorus compounds (Sablii et al., 2019). L. minor also has the ability to decrease the total suspended solids, ammoniacal nitrogen (NH3-N) and chemical oxygen demand of the sewage wastewater (Aziz et al., 2020).

As described in table 1, phytoremediation with L. minor is deemed to be cost-effective and environmental-friendly. L. minor is highly adaptable to a wide range of environmental conditions including tolerance to ammonia, heavy metals, other pollutants and stresses. This also makes it very efficient as a bioremediation tool that can be employed to treat agricultural and municipal wastewater as well as some industrial water (Zhou et al., 2023).

Table 1: Comparative study of  Heavy metal removal through traditional approaches and phytoremediation

| S. No. | Method | Advantages | Disadvantages |
| --- | --- | --- | --- |
| 1. | Phytoremediation | Cost effective, environmentally friendly, sustainable, easy harvesting | Slow removal rate for some metals, biomass disposal challenges, toxicity at high metal concentration |
| 2. | Chemical precipitation | Efficient removal, relatively fast | Generates secondary sludge, energy-intensive, expensive |
| 3. | Ion Exchange | High removal, reusable resins | Expensive, requires specialized equipment, potential for resin degradation |
| 4. | Membrane Filtration | Effective for various pollutants, relatively clean effluents | High capital cost, membrane fouling, energy consumption |

The fast reproduction and versatility of L. minor allow the species to serve as an appropriate model of sustainable water management. It can also be applied to future circular bioeconomic models, with the plants reusing the wastewater for biomass, starch, protein and bioethanol production (Vulpe et al., 2025). This points to the fact that it can transform waste into valuable resources (Vulpe et al., 2025).

4. Convection mechanisms of Contaminant Removal.

Lemna minor is a biochemical and physicochemical treatment that eliminates heavy metals and other pollutants in the water. These procedures are based on the advantageous traits inherent in the plant as a natural bio-sorbent and an aquatic macrophyte. Some of the crucial processes that occur in the process of removing contaminants in the aquatic environment include: phytoaccumulation, phytodegradation, rhizofilteration, biosorption, (Khan et al., 2025):

4.1. Phytoaccumulation:

This includes the absorption and retention of the pollutants, including the heavy metals, in the plant tissue. It has been established that L. minor has the capacity to accumulate high levels of lead, e.g., a maximum of 561 mg g dry weight (dw) of Pb in seven days (Leblebici & Aksoy, 2010). The ability enables the plant to take in high levels of pollutants (Sablii et al., 2019).

4.2. Phytodegradation:

The L. minor breaks down and transforms organic contaminants into less toxic or non-toxic products (Susarla et al., 2002). The internal metabolic processes of plants that convert, detoxify, and store these elemental contaminants are described as phytadegradation. This entails an elegant combination of chelation (binding with organic ligands such as phytochelatins and metallothioneins), compartmentalization (localization in vacuoles), and redox reactions (alteration in oxidation state of some metals). These processes allow the plants to endure heavy metal stress and thus to respond to the overall phytoremediation process to minimize the toxicity and mobility of the metals in the plant and thus the metals in the environment.

4.3. Rhizofiltration:

Rhizofiltration is a form of phytoremediation whereby polluted water bodies are treated using the root systems of plants as the absorbent, concentrator, and precipitator of the pollutants. A promising, affordable, and ecological-friendly method of water purification, it receives more and more interest among researchers all over the world. Lemna minor (also called common duckweed) is a rhizofiltrating floating aquatic macrophyte that researchers have examined; the species has been studied widely, especially in terms of its ability to remove heavy metals and other pollutants of water (Khan et al., 2025). The main process through which Lemna minor can carry out rhizofiltration is the intake of metal ions by the roots of the plant and their related translocation to the aerial plant portions. This is a type of phytoextraction or phytoaccumulation in which the plant actively absorbs pollutants in the aqueous environment. This simple root system of Lemna minor is effective in contact with dissolved contaminants in the water column.

4.4. Biosorption:

One of the sustainable and efficient water purification techniques to remove various pollutants is biosorption with Lemna minor which is a fast-growing aquatic plant. There are a number of underlying mechanisms that show up in research papers. Mostly, it depends on plant uptake and accumulation, and pollutants are retained in the plant tissues. The same observation is made with galaxolide, several emerging-contaminant pollutants of growing interest, tetracyclines, quinolones, and benzotriazole. Removal is also improved by sorption to the plant surface, as demonstrated in the case of galaxolide (Sokół & Karpińska, 2025). In the case of ionic pollutants, an electrostatic process is a primary mechanism, such as Co2+ biosorption with related duckweeds such as Lemna gibba (Reyes-Ledezma et al., 2020). Intra-particle diffusion has been observed in the process of biosorption of ciprofloxacin and ibuprofen by Lemna minor biomass or its activated carbon form (Balarak et al., 2021). Further more, the radio cations as 60 Co2+, 65 Zn2+ as well as 55+59 Fe3+ are also deemed to be complexed. The pollutants are bound to Lemna minor biomass in the presence of functional groups, e.g., hydroxyl, carboxyl, amide, and phosphate (Stambouli et al., 2025).

Pollutants can be eliminated by L. minor by biosorption. To illustrate, root and leaf cells remove phosphate and nitrate by a biosorption process and sequester (assimilate) them (Sablii et al., 2019). it effectively removes lead (down to 91% removal), cadmium (down to 85% removal), chromium, nickel (up to 99.75% removal), silver, palladium, strontium, boron, cobalt, zinc, and copper out of contaminated waters (Denbere et al., 2024).

### 5. Waste water treatment through Lemna minor

Lemna minor (duckweed) is a fast-growing aquatic macrophyte with exceptional potential for sustainable wastewater treatment. It effectively removes key pollutants, including organic matter, nutrients, heavy metals, dyes, pharmaceuticals, and pathogens, through uptake, biosorption, and metabolization. Reported removal efficiencies include up to 75% for BOD₅, 70% for COD, 96% for total phosphorus, and over 90% for certain heavy metals such as iron. In addition to improving physicochemical characteristics of wastewater, L. minor generates protein-rich biomass that can be valorized for animal feed, bioenergy, and other bioproducts, supporting circular economy models. Its integration into systems such as constructed wetlands, clarifier tanks, and microbial consortia further enhances treatment efficiency. Although challenges exist, including variability in performance under different wastewater compositions and the need for regular biomass harvesting, advances in cultivation practices and biotechnological approaches continue to expand its applicability. Overall, Lemna minor represents an eco-friendly, cost-effective, and multifunctional approach to wastewater management, with significant potential for scaling in municipal, agricultural, and industrial contexts.

### 5.1 Heavy Metals removal by Lemna minor

L. minor is effective in removing various heavy metals from contaminated water. Table 2 highlight the studies on potential for remediating heavy metals like lead, nickel, copper, and cadmium by duck weed. It has been shown that the presence of Lemna minor enhanced the removal of heavy metals in wastewater (Janna, 2021). The use of aquatic plants for heavy metal removal is an area of significant research interest, particularly due to the limitations and costs associated with conventional treatment methods. Sahi and Megateli (2023) had reported 60-80 %  removal  efficiency of cadmium, chromium and nickel at pH 7-8 and temperature of 20 °C. Similar type of work has been demonstrated by Salve and Kakde. 2024, studies suggest the 70-90 % removal efficiency of arsenic, zinc and copper at pH 6.5 to 7.5 and temperature of 22 °C.

Lemna minor demonstrates strong capabilities in removing lead. Studies show removal efficiencies as high as 88.8% at 1 mg/L concentrations (Al-Nabhan, 2022). In agricultural wastewater, L. minor achieved removal rates of 95.92% to 100% for lead (Wahab & Khalil, 2024). It has also been observed to enhance lead removal by 73% compared to systems without the plant (Janna, 2021) and can accumulate lead from acid mine fluids at levels 109 times higher than the initial concentration.

Cadmium can also be removed by the species of lemna minor (efficiency of 22.22% at 8 mg/L, Al-Nabhan, 2022, and 100% by whole agricultural wastewate. L. minor has a high cadmium bioconcentration factor, which is often the top-ranked among a series of heavy metals (Denbere et al., 2025). Lemna minor has been demonstrated to be very efficient in eliminating iron ions in wastewater, and the removal percentage rose between 61.0 and 91.7 percent as the concentration of plant biomass rose over 24 hours (Sablii et al., 2019). Other than these L. minor exhibits high and efficient performance in taking away arsenic in acid mine waters, and gathers it 907 times more than the starting level (Kislioglu, 2023). It has been classified as a hyperaccumulator of arsenic at low starting concentrations (Goswami et al., 2014).

Lemna minor has exhibited a high level of phytoremediation efficiency in accumulating and eliminating a large variety of heavy metals and metalloids in polluted waters. It also shows excellent silver and gold uptake, 174- and 394-fold more than controls, respectively (Kislioglu, 2023). It has also been stated that the plant successfully eliminates mercury in gold-mining effluents (Santacoloma-Londoño et al., 2025) and can eliminate large quantities of uranium and thorium in tailing waters (Sasmaz et al., 2016). L. minor concentrated molybdenum 177 times higher than the starting concentration under acidic conditions. Moreover, it can take up selenium with efficiencies of 23% of Se(IV) and 44% of Se(VI) (Murillo et al., 2024). Despite having a lower cobalt uptake than Cd, Pb and Zn, bioconcentration has been detected (Denbere et al., 2025).

Table 2: Heavy metal removal by Lemna minor from different sources

| Heavy Metal | Removal Efficiency / Accumulation Capacity | Key Notes | Source |
| --- | --- | --- | --- |
| Lead | 73% removal | In the presence of Lemna minor, compared to 47% without | (Janna, 2021), Peng et al., 2020 |
| Lead | 561 mg g-1 dry weight (dw) | After 7 days, in studies without nutrient enrichment | (Leblebici & Aksoy, 2010) |
| Lead | 128.7 mg g-1 dw | After 7 days, in groups enriched with nutrients | (Leblebici & Aksoy, 2010) |
| Nickel | Enhanced removal in presence of Lemna minor | Specific percentage not provided in excerpt | (Janna, 2021) |
| Copper | Enhanced removal in presence of Lemna minor | Specific percentage not provided in excerpt | (Janna, 2021) |
| Cadmium | Enhanced removal in presence of Lemna minor | Specific percentage not provided in excerpt | (Janna, 2021) |
| Cadmium | Good accumulator at low concentrations | Maintains good growth, bioconcentration, and tolerance | (Bianconi et al., 2013) |
| Mercury | Thorough removal of inorganic and organic mercury | Used Lemna minor powder as an adsorptive material | (Li et al., 2010) |
| Zinc, Cobalt, Nickel, Cadmium, Iron, Manganese, Lead | Almost 95% pollutant extraction | In industrial wastewater | Jayasri & Suthindhiran (2017) |

# 5.2 Organic Compound Removal by Lemna minor

The application of duckweed, particularly Lemna minor, in wastewater treatment has gained considerable attention due to its low-cost operation, high growth rates, and capacity to uptake or degrade a wide variety of organic contaminants. These include microbial pathogens, dyes, pharmaceuticals, personal care products, petroleum hydrocarbons, and phenolic compounds. Collectively, these findings emphasize the versatility of duckweed-based systems for addressing emerging challenges in wastewater management.

5.2.1. Microbial Contaminant Removal.

Among the most prominent uses of L. minor is the decrease in the number of microbials in wastewater. According to the study by Abadi et al., 2021, the highest elimination rate of faecal coliforms was observed in the initial five days of treatment in both municipal and industrial wastewater systems (up to 100%), which is important given the role of faecal coliform as an indicator of bacterial contamination in drinking water. In a similar manner, duckweed systems significantly reduced Escherichia coli (by about 1.5-log units) when duckweed treatment was added to anaerobic baffled reactors (Clack et al., 2019).These results indicate that in addition to nutrient pollution reduction, duckweed-based treatment minimizes microbial hazards, which is especially desirable when using wastewater to irrigate crops.

5.2.2. Dye Removal

Industrial dyes represent the most resistant pollutants of interest, and L. minor has proven to be highly effective in their removal. The biosorption of the methylene blue dye on duckweed was proved successful (Can-Terzi et al., 2021). It has also been reported to have dye-removing properties in textile dye application, in which it could prove useful in alleviating textile effluent color pollution (Sivakumar et al., 2018). The biosorptive capacity is explained by both the cell wall constituents of the plant that bind dye molecules, and the metabolic pathways that may further breakdown some of the compounds.

5.2.3. Pharmaceuticals and Personal Care Products.

Pharmaceuticals and personal care products (PPCPs) constitute an increasing population of micropollutants because of their persistence, bioaccumulation capability, and toxicity. minor has been extensively tested on the basis of its capacity to eliminate such compounds with encouraging results. Iatrou et al. (2017) have shown 100% elimination of cefadroxil in 14 days and close to 100% elimination of metronidazole (96) and significant elimination of sulfamethoxazole (73) and trimethoprim (59) in 24 days. Notably, the uptake by plants was found to be a significant mechanism, but microbial activity within the rhizosphere could be a supporting mechanism. Still more evidence is through engineered systems.

A case study by Bassuney and Tawfik demonstrated that, in a baffled duckweed pond system, raising the hydraulic retention time (HRT) of the system by 2 days, i.e. 6 to 8 days, had a substantial positive impact on the pharmaceutical elimination (para. 1). An example is that the percentage of acetaminophen removal was raised by 69.3 percent to 87.3 percent, amoxicillin by 52.9 to 82.9 and ampicillin by 55.3 to 90.6. These findings highlight the sensitivity of the operational parameters when it comes to maximizing the removal of the pollutants. In addition to antibiotics and analgesics, duckweed systems present a prospect of use in more aspects of personal care, but extensive research has not yet been carried out. Ekperusi et al. (2019) noted an excellent capacity of L. minor to Phyto-remediate such pollutants, but require more mechanistic research to determine uptake pathways and degradation products.

5.2.4.Hydrocarbons and Phenols.

Another application of duckweed-based remediation is petroleum hydrocarbons that may be introduced into aquatic systems via accidental spills or industrial discharges. Laboratory and field experiments indicate that L. minor and its associated species have the capacity to withstand and remediate low levels of hydrocarbons and that they can be used to restore freshwater habitats that have been contaminated with oil (Ekperusi et al., 2019; Koesecki et al., 2015). Likewise, the duckweeds have the ability to metabolize both chlorinated and fluorinated phenolic substances that are both industrial and agricultural. It was proven that L. minor and Landoltia punctata actively absorb and process these compounds and can be used to remediate toxic phenolics. Interestingly, the identical study also found that there was compound-specific variability in removal efficiency. Atrazine, DEET, picloram, and clofibric acid had no remarkable removal in flask-scale assays, but other pollutants, including fluoxetine, ibuprofen, 2,4-dichlorophenoxyacetic acid, and triclosan, demonstrated much higher depletion in aqueous systems. These findings highlight the fact that pollutant formulation and chemical characteristics have a critical impact on the efficiency of duckweed-mediated remediation.

Despite its strong potential, duckweed-based systems also face limitations. Some organic pollutants, particularly highly persistent pharmaceuticals and herbicides, exhibit negligible removal rates. Moreover, large-scale applications may face challenges related to biomass management, seasonal variations in growth, and the need for optimized hydraulic designs to achieve consistent performance.

# Table 3:Removal of Organic Compound Removal by Lemna minor-Efficiency, Mechanism and challenges

| Pollutant Category | Removal Efficiency / Examples | Primary Mechanism(s) | Key Notes / Challenges | References |
| --- | --- | --- | --- | --- |
| Microbial contaminants (Faecal coliforms, E. coli) | Up to 100% faecal coliform removal in 5 days; ~1.5-log reduction of E. coli | Direct plant–microbe interactions; rhizosphere effects | Effective in municipal/industrial wastewater; efficiency may vary with microbial load | Abadi et al., 2021; Clack et al., 2019 |
| Dyes (Methylene blue, Textile dyes) | High biosorption capacity; effective textile dye removal | Biosorption to cell walls; possible metabolic breakdown | Cost-effective for industrial dye effluents; may require post-treatment for residual color | Can-Terzi et al., 2021; Sivakumar et al., 2018; |
| Pharmaceuticals (Antibiotics, Analgesics) | Cefadroxil (100%), Metronidazole (96%), Sulfamethoxazole (73%), Trimethoprim (59%) in 14–24 days; Acetaminophen (69–87%), Amoxicillin (53–83%), Ampicillin (55–91%) with higher HRT | Plant uptake; rhizosphere microbial degradation; metabolism | Efficiency influenced by retention time, pollutant type, and concentration; some compounds persist | Iatrou et al., 2017; Bassuney & Tawfik, 2017 |
| Personal care products | Significant removal potential | Plant uptake; biosorption | Limited data; further mechanistic studies needed | Ekperusi et al., 2019 |
| Petroleum hydrocarbons | Strong removal in low-concentration spills | Phytoremediation; microbial support in rhizosphere | Best suited for freshwater with low oil contamination; tolerance limits unclear | Ekperusi et al., 2019; |
| Chlorinated & fluorinated phenols | Active uptake and metabolization | Plant uptake; enzymatic degradation | Efficiency depends on pollutant structure; persistence varies | Reinhold et al., 2010 |
| Other organics (Atrazine, DEET, Clofibric acid, etc.) | Negligible removal for some compounds; enhanced depletion for fluoxetine, ibuprofen, triclosan | Compound-specific uptake & transformation | Highlights selectivity of duckweed systems; may require integrated treatment | Reinhold et al., 2010 |

### 5.3 Nutrients removal by Lemna minor

The ability of Lemna minor (duckweed) to remediate nutrient-rich wastewaters has been widely studied, particularly in relation to nitrogen and phosphorus removal. Its fast growth, high nutrient uptake, and adaptability to diverse wastewater sources make it a promising candidate for sustainable phytoremediation strategies.

5.3.1 Nitrogen Removal

Lemna minor demonstrates remarkable efficiency in removing nitrogen compounds from wastewater. In a pilot-scale constructed wetland treating municipal wastewater, the species achieved a total nitrogen (TN) removal efficiency of 90.8%. Similarly, substantial reductions have been reported in engineered systems, such as 67% TN removal in secondary clarifier tanks and 83.79% removal in ceramic filters enhanced with dried duckweed biomass (Ahmadi & Dursun, 2024; Osemeahon et al., 2021).

Ammonia nitrogen (NH₄⁺-N) is particularly well-removed by Lemna species. For instance, a combined system of Lemna gibba with a heterotrophic nitrifying bacterium (Acinetobacter sp. A6) achieved over 99% NH₄⁺-N removal in aquaculture wastewater, outperforming either organism used independently (Shen et al., 2018, 2019). In practical applications, L. minor alone has achieved 72% ammonium removal in clarifier tanks (Ahmadi & Dursun, 2024) and up to 98.75% ammonia removal when incorporated into ceramic water filters (Osemeahon et al., 2021).

In contrast, nitrate (NO₃⁻) and nitrite (NO₂⁻) removal efficiencies are more variable. Elevated nitrogen and phosphorus loads can suppress nitrate and nitrite removal while enhancing ammonium and phosphate uptake (Al-Ahmady & AL-Shrefy, 2015). Nevertheless, synergistic systems combining Lemna with nitrifying bacteria have demonstrated high NH₄⁺-N removal without the undesirable accumulation of NO₂⁻ or NO₃⁻ (Shen et al., 2019).

Nitrogen removal efficiency is influenced by several environmental and operational factors. Plant density plays a critical role; 100% surface coverage of L. minor on leachate yielded the highest nitrogen removal rate (152.12 ± 2.31 mg m⁻² day⁻¹ total Kjeldahl nitrogen) (Iqbal et al., 2021). Similarly, humic acid supplementation has been shown to enhance total ammonia nitrogen (TAN) removal in aquaculture systems (Sarkheil et al., 2023). Moreover, lower plant densities may increase dissolved oxygen levels in water, promoting nitrifying bacteria and improving ammonium removal rates.

5.3.2 Phosphorus Removal

In addition to nitrogen, L. minor is highly effective in phosphorus removal. Studies show up to 96% total phosphorus (TP) removal in secondary clarifier tanks (Ahmadi & Dursun, 2024). In dairy wastewater, removal efficiencies ranged from 37.4% in synthetic effluents to 48.6% in real effluents, with considerably higher efficiencies (78.8–87%) observed under diluted conditions.

Similar to nitrogen, phosphorus removal is strongly affected by nutrient loading. Increased nitrogen and phosphorus concentrations often promote higher phosphate uptake. Plant density is again a key factor; complete surface coverage achieved the highest phosphorus removal rate of 109.24 ± 3.05 mg m⁻² day⁻¹ (Iqbal et al., 2021). Additionally, humic acid amendments can significantly enhance TP removal efficiency in recirculating aquaculture systems (Sarkheil et al., 2023).

Overall, Lemna minor exhibits exceptional capacity for nitrogen and phosphorus removal across a range of wastewater types as presented in table 4. Its efficiency is particularly notable for ammonia nitrogen, where removal rates often exceed 90%. Phosphorus uptake is also substantial, especially under optimized conditions such as full plant coverage and humic acid supplementation. While nitrate and nitrite removal may be less consistent, synergistic plant–microbe systems offer promising solutions to overcome this limitation. Collectively, these findings highlight L. minor as a low-cost, sustainable, and adaptable option for integrated wastewater treatment systems.

Table 4:Nutrient removal efficiency of Lemna minor

| Nutrient/ Contaminant | Type of Wastewater | Removal Efficiency | Source |
| --- | --- | --- | --- |
| Nitrogen (N) | Industrial wastewater, Municipal wastewater, Agricultural wastewater, Aquaculture wastewater, Domestic wastewater, Leachate | High efficiency in removal, especially ammonia as a preferred source; can have negative efficiency under high salt stress | (Pavlidis et al., 2022), (Sablii et al., 2019), (Zhou et al., 2023), (Paolacci et al., 2021), (Iqbal et al., 2019) |
| Phosphorus (P) | Industrial wastewater, Municipal wastewater, Agricultural wastewater, Aquaculture wastewater, Domestic wastewater, Leachate | Efficient absorption and transformation; can have negative efficiency under high salt stress | (Pavlidis et al., 2022), (Sablii et al., 2019), (Zhou et al., 2023), (Paolacci et al., 2021), (Iqbal et al., 2019) |
| Ammoniacal Nitrogen (NH3-N) | Sewage wastewater, Shrimp farm effluent, Aquaculture wastewater | Up to 80.4% reduction; rapidly decreased concentrations | (Aziz et al., 2020), (Paolacci et al., 2021) |
| Total Suspended Solids | Sewage wastewater | Up to 50.8% reduction | (Aziz et al., 2020) |
| Chemical Oxygen Demand | Sewage wastewater, Municipal and industrial wastewater | Up to 75% reduction | (Aziz et al., 2020) |
| Faecal Coliform | Municipal and industrial wastewater | 100% reduction in the first five days | (Abadi et al., 2021) |
| Escherichia coli | Anaerobic baffled reactor effluent | Significant removal (~1.5-log) | (Clack et al., 2019) |

6. Sustainable Biology and Bioeconomy Integration: Wastewater Treatment

L. minor is an appropriate illustration of sustainable water management in the developed and developing countries due to its rapid growth and the variety of applications (Vulpe et al., 2025). It is also applicable to future circular bioeconomy models, in which the plants can be reused to generate biomass, starch, proteins, and bioethanol after cleansing the wastewater (Vulpe et al., 2025). This shows that it can be made out of garbage to create useful things (Vulpe et al., 2025). Lemna minor has been investigated to treat municipal wastewater along with micro algae. It has been also applied to the treatment of textile industry waste water to remove color (Sivakumar et al., 2018). Huang et al. (2017) highlighted that combining Fenton oxidation with biotreatment enhances the removal of pollutants from wastewater and contaminated soils, overcoming the limitations of each method when used alone. This integrated approach improves efficiency, sustainability, and overall treatment performance.

7. Challenges:

Lemna minor  is a genus of duckweed, which has received significant focus in terms of its phytoremediation capacity due to its rapid growth rate, high nutrient uptake rate and its capacity to take up many pollutants (Walsh et al., 2021). However, despite the possibilities, there are several issues and constraints that must be surmounted to be utilized more effectively.

6.1. Metal Tolerance and Bioaccumulation: Lemna minor can bioaccumulate heavy metals like cadmium (Cd), but this results in oxidative stress that can potentially have adverse growth and chlorophyll content effects (Tkalec et al., 2008). It has the ability to bioaccumulate metals which depend on the metal and its concentration and Cd is very toxic and leads to a distorted chloroplast ultrastructure and lower pigment density.

6.2. Stress Responses: Having metals like cadmium on board exposes the body to extreme levels of oxidative stress and it has to employ the antioxidative enzymes. L. minor responds to stress through increased pyrogallol peroxidase (PPX), ascorbate peroxidase (APX), and catalase (CAT) enzyme activity and are proposed to cause a decrease in the phytoremediation potential of L. minor as a result of protein exhaustion in a constant environment (Tkalec et al., 2008).

6.3. Light Intensity Effect: The effect of light intensity is of paramount significance in the growth and phytoremediation capacity of L. minor. Although it typically relies on light to grow well, above a specific intensity (50 umolm-2s-1), the extra light does not provide a benefit and makes it more difficult to engineer stacked, indoor remediation systems (Walsh et al., 2021).

6.4. Limitations of Genetic Engineering and Biotechnology: Whereas biotechnological interventions could be beneficial in phytoremediation through the application of new combinations of genes necessary to absorb and detoxify the metals, it is also associated with environmental and regulatory issues. The ecological effect and adherence to the rules of introducing genetically modified organisms into the ecosystems specific attention should be paid (Sompura et al., 2024).

6.5. Variable Efficacy: Lemna minor will be less or more effective in its ability to eliminate pollutants, depending on the character of the pollutant. In fact, it was more efficient in eliminating cadmium than other metals, and its efficiency decreases at elevated levels of such pollutants as arsenic (Goswami et al., 2014).

6.6. System and Environmental Conditions: The environmental and system conditions under which the L. minor is grown (e.g. hydroponic systems, or wastewater types) are also important factors. It is also sensitive to the nutrient availability and medium wastewater (Basile et al., 2012).

7. Future Prospects and conclusion

Although L. minor has some important benefits, certain problems are present. This technology could need a large land area, and issues can arise during periods of winter (Moga et al., 2019). Nevertheless, duckweeds are gaining a rising popularity as a scientific research material since the removal of nutrients and protein level can much better be done and has multiple biotechnological and environmental uses (Vulpe et al., 2025).

Additional research is required to maximize the cultivation environment of L. minor during colder seasons to enable efficacy to be consistently useful during the year (Sablii et al., 2019). The development of new studies is also needed in order to scale up these phytoremediation systems to industrial proportions and evaluate the long-term ecological effects of extensive L. minor usage. On the other hand, further in-depth research studies are required to enable a clear image on the complex relationships between L. minor with the contaminants and the microbial community in these phytoremediation systems. Moreover, the yield of nitrogen and phosphorus extraction by Lemna minor during the wastewater treatment should be described further to determine the most suitable parameters that can be utilized in practice.

The diversity of duckweeds and pollutant resistance also increases the windows of these plants in filtering agricultural, municipal, and some industrial wastewaters. Regardless of these benefits, L. minor has not yet become widely used in large-scale wastewater treatment plants owing to difficulties with harvesting and disposal of biomass, particularly considering its rapid growth. Research is being done to develop cost-effective harvesting methods and renewable valorization options of the nutrient-rich biomass and transform a potentially useful waste stream into useful by products. In addition, although phytoremediation using Lemna minor is an environmentally friendly solution, its application at large-scale wastewater treatment plants is a major optimization issue. Additional studies are necessary to create more rigorous and cost-effective plans to establish L. minor-based systems at a municipal level, including investigating methods of combined treatment.

Disclaimer (Artificial intelligence)

No generative AI technologies such as large Language Models (ChatGPT) has been used in preparation and writing of this manuscript.

References:

- Abadi, R. M., Dehghan, A., Azari, A., & Mehralian, M. (2021). Efficiency of Lemna minor for fecal coliform removal from municipal and industrial wastewater. Environmental Technology & Innovation, 21, 101–110.

- Ahmadi, A. W., & Dursun, S. (2024). Assessing the Efficiency and Role of Duckweed (Lemna Minor) in the Removal of Pollutants from Wastewater Treatment Plant Secondary Clarifier Tanks: A Comprehensive Review. Central Asian Journal of Water Research.

- Ahmed, A. M., & Kareem, S. (2024). Evaluation of the effectiveness of phytoremediation technologies utilizing Lemna minor in constructed wetlands for wastewater treatment. Biomass Conversion and Biorefinery.

- Al-Ahmady, K. K., & Al-Shrefy, A. (2015). Effects of nitrogen and phosphorus loads on duckweed-based wastewater treatment systems. Applied Water Science, 5(1), 12–22.

- Al-Nabhan, E. A. M. (2022). Removal efficiency, accumulation and biochemical response of Lemna minor L. exposed to some heavy metals. IOP Conference Series: Earth and Environmental Science, 1060(1), 012037

- Asih, D. R., Handayani, W., Ananda, A. Z., Yuniati, R., Hirota, R., Summerfield, T., & Eaton‐Rye, J. (2024). An insight into potential phosphate bioremediation and renewable energy from agricultural waste via integrated wastewater treatment systems in Indonesia. Environmental Research Communications, 6.

- Abdul Aziz, N. I. H., Mohd Hanafiah, M., Halim, N. H., & Fidri, P. A. S. (2020). Phytoremediation of TSS, NH₃-N and COD from sewage wastewater by Lemna minor L., Salvinia minima, Ipomoea aquatica and Centella asiatica. Applied Sciences, 10(16), 5397.

- Balarak, D., Taheri, N., Shim, M. J., Lee, S. M., & Jeon, C. (2021). Adsorption kinetics, thermodynamics, and equilibrium of ibuprofen from aqueous solutions by activated carbon prepared from Lemna minor. Journal of Water Supply: Research and Technology—AQUA, 70(5), 646–656.

- Bassuney, D., & Tawfik, A. (2017). Pharmaceutical removal from agricultural drainage water using baffled duckweed pond systems. Ecological Engineering, 103, 189–197.

- Brkanac, S. R., Stipaničev, D., Širac, S., Glavaš, K., & Pevalek‐Kozlina, B. (2010). Biomonitoring Of Surface Waters Using Duckweed (Lemna Minor L.). 539.

- Can-Terzi, B., Şahin, S., & Atıcı, Ö. (2021). Biosorptive removal of methylene blue dye by duckweed (Lemna minor). Environmental Science and Pollution Research, 28, 21456–21465.

- Clack, H., Tawfik, A., & El-Kamah, H. (2019). Removal of Escherichia coli from anaerobic baffled reactor effluent by duckweed-based systems. Journal of Environmental Management, 231, 1205–1213.

- Denbere, T., Alemayehu, E., & Kassa, B. (2025). Bioconcentration factors of heavy metals in Lemna minor. Environmental Monitoring and Assessment, 197, 125–138.

- Ekperusi, A. O., Sikoki, F. D., & Nwachukwu, E. O. (2019). Duckweed-based wastewater treatment: A review of its potential in phytoremediation of personal care products and hydrocarbons. Ecotoxicology and Environmental Safety, 170, 246–256.

- Elhamji, S., Sbihi, K., Lghoul, S., Aziz, K., Maallem, A. E., Kurniawan, T. Agustiono., & Aziz, F. (2025). Addressing the challenge of heavy metal contamination in aquatic environment: Harnessing the bioremediation potential of Navicula permitis diatom for wastewater treatment. Biomass & Bioenergy.

- Gürtekin, E., & Şekerdağ, N. (2008). The Role of Duckweed (Lemna Minor L.) In Secondary Clarifier Tanks. Sakarya University Journal of Science, 12, 28–31.

- Huang, D., Hu, C., Zeng, G., Cheng, M., Xu, P., Gong, X., Wang, R., & Xue, W. (2017). Combination of Fenton processes and biotreatment for wastewater treatment and soil remediation. Science of the Total Environment, 574, 1599–1610.

- Iatrou, E. I., Stasinakis, A. S., & Thomaidis, N. S. (2017). Removal of selected pharmaceuticals by duckweed (Lemna minor) and contribution of plant uptake. Chemosphere, 168, 1057–1064.

- Iqbal, J., Riaz, M., & Mahmood, T. (2021). Effects of initial plant density on nutrient removal efficiency of Lemna minor in leachate treatment. Environmental Monitoring and Assessment, 193, 512.

- Janna, H. (2021). Degradation of some heavy metals in wastewater using aqua plants. IOP Conference Series: Materials Science and Engineering, 1067(1), 012084.

- Jayasri, M. A., & Suthindhiran, K. (2017). Effect of zinc and lead on the physiological and biochemical properties of aquatic plant Lemna minor: Its potential role in phytoremediation. Applied Water Science, 7(2), 1247.

- Kastratović, V., Jovanović, M., & Jovanović, S. (2013). Lemna minor L. as bioindicator of heavy metal pollution in Skadar Lake (Montenegro). Environmental Science and Pollution Research, 20(4), 2640–2649.

- Khan, S. N., Anjum, L., Arshad, A., Ali, S., Aleem, M., & Nasir, A. (2025). Nature-Based Solution for Wastewater Treatment and Reuse Using Phytoremediation with Floating Plants. Water.

- Khuzhzhiev, S., Bakhramov, I., Gafurova, D., & Khamraeva, M. (2024). Biological Wastewater Treatment Using Higher Aquatic Plants. Journal of Energy, Environmental &amp; Chemical Engineering.

- Kislioglu, O. (2023). Gold and silver uptake by Lemna minor from mine waters. Chemosphere, 314, 137–148.

- Leblebici, Z., & Aksoy, A. (2010). Growth and lead accumulation capacity of Lemna minor L. and Spirodela polyrhiza (L.) Schleiden. Ekoloji, 19(75), 20–24.

- Li, J., Loi, G., Otero-Gonzalez, L., Laing, G. D., Ferrer, I., & Lens, P. N. (2020). Selenate and selenite uptake, accumulation and toxicity in Lemna minuta. Water Science and Technology, 81(9), 1852-1862.

- Moga, I. C., Petrescu, G., Anghelache, G., Pantazi, A. G., & Doroftei, B. I. (2019, July). Biotechnologies used in wastewater treatment. In IOP Conference Series: Materials Science and Engineering (Vol. 572, No. 1, p. 012069). IOP Publishing.

- Nikiforov, L. N., Krivoshchekov, S., Kolomiets, N., Kadyrova, T. V., Isaikina, N. V., Abramets, N., Bezverkhniaia, E., & Belousov, M. (2021). Development of Parameters for Standardization of Duckweed (Lemna Minor L.) Raw Material. Drug Development & Registration.

- Nopriani, U., Karti, P., & Prihantoro, I. (2015). Productivity of duckweed (Lemna minor) as alternative forage feed for livestock in different light intensities. Jurnal Ilmu Ternak Dan Veteriner, 19, 272–286.

- Osemeahon, S. A., Joseph, D., & Yahaya, A. (2021). Application of dried duckweed in ceramic water filters for nitrogen removal. Applied Water Science, 11, 36–43.

- Pilar Arroyave M. (2004). LA LENTEJA DE AGUA (LEMNA MINOR L.): UNA PLANTA ACUÁTICA PROMISORIA*. 1, 33–38.

- Politaeva, N., & Badenko, V. (2021). Magnetic and electric field accelerate Phytoextraction of copper Lemna minor duckweed. PLoS ONE, 16.

- Radić, S., Stipaničev, D., Cvjetko, P., Mikelić, I., Rajčić, M., Širac, S., Pevalek‐Kozlina, B., & Pavlica, M. (2010). Ecotoxicological assessment of industrial effluent using duckweed (Lemna minor L.) as a test organism. Ecotoxicology, 19, 216–222.

- Reinhold, D., Vishwanathan, S., Park, J. J., Oh, D., & Saunders, F. M. (2010). Phytoremediation of emerging organic contaminants: Chlorinated and fluorinated phenols, pharmaceuticals, and personal care products by duckweed. Chemosphere, 80(7), 665–672.

- Sablii, L., Korenchuk, M., & Kozar, M. (2019). Application of Lemna minor for polluted water treatment from biogenic elements. Biotechnologia Acta, 12(5), 82-88.

- Santacoloma-Londoño, G., Rodríguez, A. M., & Rincón, C. A. (2025). Potential of Lemna minor for mercury phytoremediation in gold-mining effluents. Journal of Hazardous Materials Advances, 8, 100–117.

- Sarkheil, M., Hosseini, S. A., & Ghasemi, S. (2023). Role of humic acid in enhancing nitrogen and phosphorus removal by Lemna minor in aquaculture systems. Aquaculture Research, 54(2), 481–492.

- Sasmaz, A., Obek, E., & Hasar, H. (2016). The ability of Lemna minor to remove uranium and thorium from mine tailing waters. Journal of Environmental Radioactivity, 153, 832–837.

- Shen, Y., Chapelle, F. H., & Wang, J. (2018). Enhanced ammonium removal in aquaculture wastewater by combined Lemna gibba and Acinetobacter sp. Bioresource Technology, 267, 608–616.

- Shen, Y., Wang, J., & Chen, Q. (2019). Synergistic removal of ammonia by duckweed–bacteria systems without nitrate accumulation. Science of the Total Environment, 659, 1052–1060.

- Sivakumar, K. K., Rajan, S., & Sundaram, M. (2018). Removal of textile dye color using duckweed-based treatment systems. Environmental Monitoring and Assessment, 190, 512–522.

- Sokół, A., & Karpińska, J. (2025). Enhanced phytoremediation of galaxolide using Lemna minor: Mechanisms, efficiency, and environmental implications. International Journal of Molecular Sciences, 26(14), 6636.

- Sompura, Y., Bhardwaj, S., Selwal, G., Soni, V., & Ashokkumar, K. (2024). Unrevealing the potential of aquatic macrophytes for phytoremediation in heavy metal-polluted wastewater. Journal of Current Opinion in Crop Science, 5(1), 48-61.

- Susarla, S., Medina, V. F., & McCutcheon, S. C. (2002). Phytoremediation: An ecological solution to organic chemical contamination. Environmental International, 28(1–2), 113–121.

- Tkalec, M., Prebeg, T., & Balen, B. (2008). Cadmium-induced responses in duckweed Lemna minor L. Environmental Toxicology and Chemistry, 27(5), 1153–1160.

- Thomas, J. D., & Eaton, P. (1996). The accumulation of amino-acids and humic substances in media conditioned by axenic and non-axenic duckweed (Lemna minor L.) and their possible ecological significance. Hydrobiologia, 333, 121–128.

- Vulpe, C., Toplicean, I.-M., Agachi, B.-V., & Datcu, A. (2025). A Review on Uses of Lemna minor, a Beneficial Plant for Sustainable Water Treatments, in Relation to Bioeconomy Aspects. Water.

- Walsh, É., Kuehnhold, H., O'Brien, S., Coughlan, N. E., & Jansen, M. A. K. (2021). Light intensity alters the phytoremediation potential of Lemna minor. Environmental Pollution, 276, 116728.

- Wilschut, M., Theuws, P. A. W., & Duchhart, I. (2013). Phytoremediative urban design: Transforming a derelict and polluted harbour area into a green and productive neighbourhood. Environmental Pollution, 183, 81-88.

- Zhou, Y., Stepanenko, A., Kishchenko, O., Xu, J., & Borisjuk, N. (2023). Duckweeds for Phytoremediation of Polluted Water. Plants, 12.