Empirical Statistical Modelling of Angle of Internal Friction using SPT-N Value, Median Grain Size (D50), and Effective Overburden Stress

Abstract: The study aimed to create statistical models that relate the angle of internal friction to predictor factors including median grain size (D50), effective overburden stress, and Standard Penetration Test (SPT-N) values for soil deposits at Rumuibekwe Bridge in the Niger Delta. A total of 16 SPT tests were conducted within depths of 0–30 m, and soil samples were collected at each stratum. Laboratory analyses included sieve tests for D50 and consolidated drained triaxial tests for internal friction angle determination. The soils were classified mainly as fine to medium sands with variable fines content, reflecting the heterogeneous depositional profile of the Niger Delta. Statistical regression models, both simple and multiple, were developed to connect predictor variables with the response variable. Effective overburden stress emerged as the most significant predictor (R² = 0.921), outperforming SPT-N (R² = 0.807), while D50 exhibited weak predictive capability. Model validation confirmed that effective stress–based models provide more reliable predictions of friction angle than conventional SPT-only correlations. The findings highlight the importance of localized, stress-dependent models for geotechnical design in complex soil environments.

Keywords: friction angle, SPT-N values, effective stress, median grain size, multiple regression, Niger Delta

1. INTRODUCTION

A crucial design parameter for the majority of geotechnical design processes, including earth structure design, slope stability analysis, and bearing capacity calculation, is the angle of internal friction (Ullah et al., 2025). Laboratory experiments like the triaxial test or the direct shear box test are still the most accurate way to determine and estimate the angle of internal friction of soil (Brown & Hettiarachchi, 2008; Ullah et al., 2025). Engineers now use empirical correlation for estimate, nevertheless, due to a number of issues, including laboratory error and maintaining the soil's condition prior to the experiment. Additionally, the expense of the test and the number of man hours needed, particularly when doing several tests, have led engineers to search for a quicker and less costly way to estimate the angle of friction. Currently, the angle of friction is determined by comparing the friction angle to the N-values from the Standard Penetration Test. Since the beginning of the 20th century, Standard Penetration Test (SPT) N-values have been the main in-situ testing parameter for characterizing soil. Several correlations have been established to estimate shear strength parameters from penetration resistance (Peck et al., 1953; Ohsaki et al., 1959; Hatanaka & Uchida, 1996; Cubrinovski & Ishihara, 1999).

The angle of internal friction is often modeled using just SPT-N values (Peck et al., 1957; Ohsaki et al., 1959; Hatanaka & Uchida, 1996). Nevertheless, these models have not been very successful, particularly when applied to localized or regional soil (Sahab et al., 2022). In order to increase the accuracy of the angle of internal friction prediction, additional predictive factors must be included. When applied to certain geological conditions, the addition of factors beyond SPT-N values has shown notable gains in correlation accuracy and dependability (Sahab et al., 2022). Granular soils' mechanical behavior is significantly influenced by grain size features, especially median grain size (D50). Particle features influence friction angle mobilization in sandy soils, and laboratory investigations have shown a strong correlation between shear strength and particle size distribution (Li, 2013; Wang et al., 2013). According to Wang et al. (2013), the angle of internal friction rises as the soil's median size increases. Nevertheless, there is still little integration of grain size data into SPT-based friction angle correlations, especially for regional geological contexts. Another factor that affects the angle of internal friction is effective overburden stress (Liao & Whitman, 1986).

Because of its complicated geological past and varied depositional settings, the Niger Delta area poses particular difficulties for the development of geotechnical correlations. An angle of friction should be estimated using a localized model due to the variance in the soil profile in the Niger Delta area (Kennedy & TrustGod, 2024). The goal of the study is to create statistical models for estimating the angle of internal friction for sandy soils in the Niger Delta by utilizing a number of predictor factors, such as SPT-N values, median particle size (D50), and effective overburden stress.

2. MATERIALS AND METHOD

2.1 Study Area

The site of the Rumuibekwe Bridge in the Niger Delta, as seen in Figure 1, is the study's main emphasis. The Rumuibekwe Bridge is located in Rivers State's Obio/Akpor Local Government Area. Latitude 4.843N and longitude 7.0575E are the location's geographic coordinates. The site is rather flat, with elevations typically below 15.24 meters above mean sea level. The area has a tropical monsoon climate, with typical annual temperatures of around 28.64°C and 2,000–2,900 millimeters of precipitation per year.

Figure 1: Map of study Area

2.2 Soil Parameters and Laboratory Characterization

Before analyzing soil parameters, it was important to classify and describe the soils tested in this study. The subsoils at Rumuibekwe Bridge were heterogeneous, reflecting the depositional complexity of the Niger Delta. According to the Unified Soil Classification System (USCS), the soils ranged from poorly graded fine sands (SP) to silty sands (SM), with occasional clay-sized particles. The median grain size (D50) varied from 0.002 mm to 2.008 mm, representing clay to medium sand fractions, while the mean D50 was 0.52 mm, indicating that fine sand dominated the profile. The soils were generally compacted with Standard Penetration Test (SPT-N) values ranging from very loose (N < 4) to dense sands (N > 30). Effective overburden stress across the strata ranged from 56.00 kPa to 276.55 kPa, confirming significant variation in stress conditions with depth.

The angle of internal friction (φ′) was determined through Consolidated Drained (CD) triaxial compression tests, in line with ASTM D4767 standards. This test was selected because it allows for full dissipation of pore pressures, thereby producing shear strength values that represent effective stress conditions typical of sandy soils. The CD triaxial results provided reliable estimates of the friction angle, which were then correlated with field-based SPT results. Together, these data formed the foundation for developing statistical models that connect SPT-N values, effective overburden stress, and grain size characteristics to soil shear strength behavior.

2.3 Data Collection and Laboratory Testing

By performing the Standard Penetration Test (SPT) in accordance with ASTM D1586 guidelines, the N-values for different subsoil layers were determined. 16 subsurface strata, ranging in depth from 0 to 30 meters, were subjected to SPT, and the SPT-N values for each layer were noted. A total of sixteen samples were taken from Rumuibekwe Bridge in order to get further soil parameters at each tier. After being collected, the dirt was placed in an airtight container and brought to the lab for analysis. Programs for laboratory testing included sieve and triaxial tests. Standard sieve analytical methods were carried out in accordance with ASTM D6913 to acquire median grain size (D50) values, and consolidated drained triaxial compression tests were conducted in accordance with ASTM D4767 to determine the angle of internal friction.

2.4 Data Analysis

A major component of the data analysis approach was the creation of statistical models. Descriptive and correlational analyses of the soil characteristics were conducted, however. To compile the values of the soil parameters derived from the field and laboratory tests, descriptive statistics were used. Using Pearson correlation as a basis for feature selection, the connection between the soil metrics was examined. To create prediction models for the angle of internal friction, multiple linear regression analysis was utilized. The response variable for the model development was the angle of internal friction, while the predictor factors were the median grain size, effective overburden pressure, and SPT-N values. Root mean square error (RMSE), mean squared error (MSE), adjusted R2, and coefficient of determination (R2) were used to evaluate the model's performance. Three independent data sets that weren't utilized to build the model were evaluated.

3. RESULTS

3.1 Geotechnical Parameters for Rumuibekwe Bridge

At Rumuibekwe Bridge, Table 1 displays the findings of the descriptive statistic for the soil parameters derived from 16 samples. N-values for the standard penetration test showed a mean of 20.25±11.43, with a broad range of 4 to 36. Meyerhoff (1956) defined compact soil as having an SPT-N value between 10 and 30 and cohesionless soil with SPT values less than 4 as primarily very loose soil. Across the sampling depths, the soil at Rumuibekwe Bridge showed notable variation in soil density conditions, according to the SPT-N values. The mean angle of internal friction was 32.25±3.75°, which is normal for sandy soils and ranged from 26° to 38°. With a mean of 163.98 kPa, effective overburden stress varied from 56.00 to 276.55 kPa. Diverse particle size distributions were found within the soil layers, as evidenced by the median grain size (D50), which varied significantly from 0.002 to 2.008 mm with a mean of 0.52±0.67 mm. As per the Unified Soil Classification System (USCS), the D50 values indicate a diverse soil profile that includes medium sand (D50 = 2.008 mm) and fine-grained soils (D50 = 0.002 mm, which correspond to clay-sized particles). The conditions are primarily fine sand, as shown by the median value of 0.20mm. The subsoils at Rumuibekwe Bridge are primarily fine particles that are compacted, according to the soil conditions. This indicates that clay soils make up the majority of the subsurfaces at that site.

Table 1: Parameter Statistics by Location

| Parameter | Count | Mean | Median | Std Dev | Min | Max |
| --- | --- | --- | --- | --- | --- | --- |
| SPT-N | 16 | 20.25 | 23.5 | 11.43 | 4 | 36 |
| Friction Angle (°) | 16 | 32.25 | 32.5 | 3.75 | 26 | 38 |
| Effective Stress (kPa) | 16 | 163.98 | 160.77 | 73.75 | 56.00 | 276.55 |
| D50 (mm) | 16 | 0.52 | 0.20 | 0.67 | 0.002 | 2.008 |

3.2 Correlation Analysis

The association between the soil characteristics and the outcome shown in Table 2 was determined using Pearson correlation. Several of the soil characteristics and the angle of internal friction showed remarkably significant linear connections, according to the Pearson correlation study. The strongest association between friction angle and effective overburden pressure (r = 0.96, p < 0.001) was found, indicating that the soils' shear strength is essentially stress-dependent. The theoretical underpinnings of employing penetration resistance as an indication of shear strength features were supported by the very significant connection between SPT-N and friction angle (r = 0.90, p < 0.001). The median grain size and the angle of internal friction did, however, exhibit a somewhat favorable correlation.

Table 2: Pearson Correlation Matrix

| Variables | Effective Overburden pressure | SPT-N | D50 | Ø |
| --- | --- | --- | --- | --- |
| Effective Overburden pressure | 1.00 |  |  |  |
| SPT-N | 0.96 | 1.00 |  |  |
| D50 | 0.53 | 0.61 | 1.00 |  |
| Ø | 0.96 | 0.90 | 0.49 | 1.00 |
| Values in bold are different from 0 with a significance level alpha=0.05 |  |  |  |  |

3.3 Development of Models

The model was developed in stages. First, a straightforward linear regression model was created that connected each soil parameter to the angle of internal friction. A multiple linear regression model was created that established a relationship between the angle of internal friction and all soil characteristics. To assess each parameter's predictive power for estimating friction angle, three straightforward linear regression models were created. Table 3 displays the model's goodness of fit results. With a coefficient of determination R2 = 0.921, the effective overburden pressure model showed remarkable performance, explaining 92.1% of the variance in friction angle through effective stress alone. The model was extremely significant (F = 128.479, p < 0.0001) and had a root mean square error of 1.091°. Additionally, the model showed that the angle of internal friction increases by 0.047° for every 1KPa rise in the effective stress. At R2 = 0.807, the SPT-N model had statistically significant predictive performance, accounting for 80.7% of the variance in the friction angle with an RMSE of 1.708°. According to the model, the angle of internal friction will rise by 0.286° for every unit change in the SPT-N value. With R2 = 0.242 and an RMSE of 3.383°, the median grain size model performed noticeably worse, accounting for just 24.2% of the variance in the friction angle. At a significance level of 10%, this model demonstrated a significant association, but at a 5% level, it was not significant (F = 3.507, p = 0.088). The outcome demonstrated the median grain size's poor predictive power. Given the heterogeneous character of the soil profile and the large range of particle sizes (from medium sand to clay-sized particles), the poor prediction accuracy of D50 may be explained by the complex interactions that are not sufficiently reflected by median grain size alone (Table 4).

Table 3: Goodness of Fit Statistics for Simple Linear Regression Models

| Statistic | Effective Overburden Pressure | SPT-N | D50 |
| --- | --- | --- | --- |
| Observations (n) | 13 | 13 | 13 |
| Degrees of Freedom | 11 | 11 | 11 |
| R² | 0.921 | 0.807 | 0.242 |
| Adjusted R² | 0.914 | 0.789 | 0.173 |
| MSE | 1.19 | 2.918 | 11.443 |
| RMSE | 1.091 | 1.708 | 3.383 |
| F-statistic | 128.479 | 45.89 | 3.507 |
| p-value (model) | < 0.0001 | < 0.0001 | 0.088 |
| Durbin-Watson | 2.677 | 2.787 | 1.489 |

Table 4: Model Parameters for Simple Linear Regression Models

| Model | Intercept |  | Slope Coefficient |  |  |  | Equation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Estimate | p-value | Estimate | Std. Error | t-value | p-value |  |
| Effective Overburden Pressure | 24.683 | < 0.0001 | 0.047 | 0.004 | 11.335 | < 0.0001 | Ø = 24.683 + 0.047×EOP |
| SPT-N | 26.661 | < 0.0001 | 0.286 | 0.042 | 6.774 | < 0.0001 | Ø = 26.661 + 0.286×SPT-N |
| D50 | 31.392 | < 0.0001 | 2.59 | 1.383 | 1.873 | 0.088 | Ø = 31.392 + 2.590×D50 |

The best single-parameter model performed marginally worse than the multiple linear regression model that included all three predictor variables, with R2 = 0.929 and adjusted R2 = 0.905. A little improvement over the effective overburden pressure model alone, the model described 92.9% of the variance in the friction angle with an RMSE of 1.146°. However, analysis of individual parameter significance within the multiple regression model revealed important insights into parameter interactions. In predicting friction angle, the effective overburden pressure coefficient (0.062°/kPa) continued to play a dominant role, being highly significant (t = 3.860, p = 0.004). On the other hand, when paired with the effective overburden pressure, SPT-N and D50 were both statistically invalid, suggesting little predictive potential. A D50 coefficient of 0.108°/mm (t = 0.178, p = 0.863) and an SPT-N coefficient of -0.107°/unit (t = -0.959, p = 0.363) were found. Multicollinearity effects may be shown by the nonsignificant influence of both the SPT-N and D50, where the effective overburden pressure parameter fully captures the predictive information found in all three measures (Table 5).

The SPT-N values were the second most significant predictor of the angle of internal friction, after the effective overburden pressure, as shown by the Pareto chart in Figure 2. But only the effective overburden pressure was statistically significant, according to the figure.

Table 5: Goodness of Fit Statistics for Multiple Linear Regression Models

| Observations | 13.000 |
| --- | --- |
| Sum of weights | 13.000 |
| DF | 9.000 |
| R² | 0.929 |
| Adjusted R² | 0.905 |
| MSE | 1.312 |
| RMSE | 1.146 |
| DW | 2.231 |

Table 6: Model Parameters for Multiple Linear Regression Models

| Source | Value | Standard error | t | Pr > ¦t¦ | Lower bound (95%) | Upper bound (95%) |
| --- | --- | --- | --- | --- | --- | --- |
| Intercept | 24.282 | 0.928 | 26.170 | < 0.0001 | 22.183 | 26.381 |
| Effective Overburden pressure | 0.062 | 0.016 | 3.860 | 0.004 | 0.026 | 0.099 |
| SPT-N | -0.107 | 0.112 | -0.959 | 0.363 | -0.361 | 0.146 |
| D50 | 0.108 | 0.606 | 0.178 | 0.863 | -1.263 | 1.478 |

Figure 2: Pareto for important predictors for angle of internal friction

3.4 Model Evaluation

To assess prediction accuracy in real-world scenarios, three separate test samples were used for model validation. The validation performance of the effective overburden pressure model was superior, with mean absolute error (MAE) = 0.84°, mean absolute percentage error (MAPE) = 2.9%, and RMSE = 0.88°. With an RMSE of 0.91°, MAE of 0.81°, and MAPE of 2.9%, the multiple linear regression model demonstrated comparable performance, indicating that the practical prediction accuracy was only marginally improved by the increased complexity of many factors.

The SPT-N model performed much worse than the stress-based model, although it nevertheless demonstrated satisfactory validation performance with RMSE = 1.1°, MAE = 1.09°, and MAPE = 3.8%. Due to its poor validation performance (RMSE = 2.99°, MAE = 2.64°, and MAPE = 9.4%), the D50 model was found to be of limited use in predicting friction angles in the diverse soil profile found at Rumuibekwe Bridge. Grain size effects are less constant across different soil types than stress-dependent behavior, and the validation results confirm that effective overburden pressure is the main predictive variable for friction angle estimate in Niger Delta mixed soil deposits.

Table 7: Model Predictions

| Test | EOP | SPT-N | D50 | Observed Ø | Pred Ø from EOP SLR | Pred Ø from SPT-N SLR | Pred Ø FROM D50 SLR | Pred Ø from MLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 92.4 | 9 | 0.1 | 28 | 29.03 | 29.23 | 31.6 | 29.06 |
| Test 2 | 145 | 19 | 0.1 | 31 | 31.48 | 32.09 | 31.67 | 31.23 |
| Test 3 | 92.1 | 8 | 0.1 | 28 | 29.01 | 28.95 | 31.66 | 29.14 |

Note. EOP = Effective Overburden Pressure; SPT-N = Standard Penetration Test value; D50 = Median particle size; Ø = Angle of internal friction; SLR = Simple Linear Regression; MLR = Multiple Linear Regression

Table 8: Model Performance Metrics on Test Data

| Model | RMSE (°) | MAE (°) | MAPE (%) | Bias (°) | Max Error (°) |
| --- | --- | --- | --- | --- | --- |
| EOP | 0.88 | 0.84 | 2.9 | 0.838 | 1.026 |
| SPT-N | 1.1 | 1.09 | 3.8 | 1.093 | 1.235 |
| D50 | 2.99 | 2.64 | 9.4 | 2.642 | 3.656 |
| Multiple | 0.91 | 0.81 | 2.9 | 0.809 | 1.144 |

Note. RMSE = Root Mean Square Error; MAE = Mean Absolute Error; MAPE = Mean Absolute Percentage Error; Bias = Mean prediction error; Max Error = Maximum absolute error.

Table 9: Model Performance Ranking Summary

| Model | RMSE Rank | MAE Rank | MAPE Rank | Overall Score |
| --- | --- | --- | --- | --- |
| Multiple Regression Model | 2 | 1 | 1 | 4 |
| EOP SLR | 1 | 2 | 2 | 5 |
| SPT-N SLR | 3 | 3 | 3 | 9 |
| D50 SLR | 4 | 4 | 4 | 12 |

4. DISCUSSION

With an outstanding performance (R2 = 0.921) that outperforms conventional SPT-N based correlations, the results show that effective overburden stress is the primary predictor of friction angle for the heterogeneous soil deposits at Rumuibekwe Bridge. SPT-N values are often regarded as the main parameter for estimating friction angle, although this conclusion contradicts standard geotechnical practice. Stress levels and soil properties have a significant impact on the relationship between SPT-N and friction angle (Hatanaka & Uchida, 1996). Because Niger Delta deltaic deposits are complex and have varied soil profiles that range from medium sand to clay-sized particles, median grain size alone is insufficient to accurately depict mechanical behavior. This is shown in the D50 model's low performance (R2 = 0.242). Its practical value for engineering applications is demonstrated by the effective overburden pressure model's exceptional validation performance (RMSE = 0.88°, MAPE = 2.9%). The majority of slope stability and foundation design assessments may be completed with this level of precision, which is a significant improvement over conventional correlations with generally greater errors. Because of their location-specificity, these correlations highlight how crucial regional calibration is to the establishment of geotechnical correlations. In contrast to relationships that are generally employed, empirical correlations between SPT N-values and soil characteristics show better performance when created for particular geological settings (Cubrinovski & Ishihara, 1999).

Limitations

This study developed statistical models linking Standard Penetration Test (SPT) values, effective overburden stress, and median particle size (D50) to the angle of internal friction in Niger Delta soils. While the findings are valuable, several limitations should be noted.

- The first limitation concerns sample size. Only sixteen samples from the Rumuibekwe Bridge site were analyzed. Although sufficient for preliminary modeling, this number may not fully represent the variability of Niger Delta soils, which are known for their heterogeneity. A larger dataset covering multiple sites would provide stronger validation and improve generalizability.

- A second limitation relates to multicollinearity among predictor variables. Effective overburden stress emerged as the dominant factor, while SPT-N and D50 lost significance in combined models, suggesting redundancy. Although discussed qualitatively, no diagnostic measures such as Variance Inflation Factor (VIF) were reported. Including such tests in future studies would improve the robustness of regression models.

- Another limitation is the absence of uncertainty quantification. Predictions were presented as point estimates without confidence intervals or error bounds. Given the variability of soils, incorporating uncertainty analysis would enhance the reliability of the models and provide clearer guidance for engineering applications.

- The regional scope of the study also limits applicability. The models reflect conditions specific to Rumuibekwe Bridge and may not transfer directly to other parts of the Niger Delta or beyond. Broader validation across sites and environments is needed to extend their relevance.

- Finally, the study did not include a detailed comparison with established correlations such as those by Peck et al. (1957) or Ohsaki et al. (1959). Benchmarking against classical models would have highlighted more explicitly the advantages of the proposed approach.

5. CONCLUSION

Developing a statistical model that can accurately forecast the angle of internal friction based on soil properties and SPT-N values was the goal of this work. At Rumuibekwe Bridge, statistical models were created to forecast the friction angle for Niger Delta soils based on SPT-N values, median particle size, and effective overburden stress. In comparison to conventional SPT-based correlations, the basic linear effective overburden pressure model performed quite well (R2 = 0.921, RMSE = 1.091°). Due to multicollinearity effects, the multiple regression model (R2 = 0.929) only marginally improved the single-parameter effective stress model; only the effective overburden pressure remained statistically significant (p = 0.004). For geotechnical applications in the Niger Delta, the results highlight the significance of location-specific correlations and stress-dependent design methodologies.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

REFERENCES

Anbazhagan, P., & Sitharam, T. G. (2010). Relationship between shear wave velocity and SPT-N for different types of soils in India. Geotechnical and Geological Engineering, 28(1), 119-137. https://doi.org/10.1007/s10706-009-9285-4

Arifuzzaman, A., & Anisuzzaman, M. (2022). An initiative to correlate the SPT and CPT data for an alluvial deposit of Dhaka city. International Journal of Geo-Engineering, 13(5), 1-13. https://doi.org/10.1186/s40703-021-00170-3

Asci, M., Kurtulus, C., Kaplanvural, I., & Mataracioglu, M. O. (2014). Correlation of SPT-CPT data from the subsidence area in Golcuk, Turkey. Soil Mechanics and Foundation Engineering, 51(6), 268-272. https://doi.org/10.1007/s11204-015-9294-y

Chen, J. (2004). Axial behavior of drilled shafts in gravelly soils [Doctoral dissertation, Cornell University].

Cubrinovski, M., & Ishihara, K. (1999). Empirical correlation between SPT N-value and relative density for sandy soils. Soils and Foundations, 39(5), 61-71. https://doi.org/10.3208/sandf.39.5_61

Demir, S., & Sahin, E. K. (2022). Comparison of tree-based machine learning algorithms for predicting liquefaction potential using canonical correlation forest, rotation forest, and random forest based on CPT data. Soil Dynamics and Earthquake Engineering, 154, 107130. https://doi.org/10.1016/j.soildyn.2021.107130

Demir, S., & Sahin, E. K. (2023). An investigation of feature selection methods for soil liquefaction prediction based on tree-based ensemble algorithms using AdaBoost, gradient boosting, and XGBoost. Neural Computing and Applications, 35, 3173-3190. https://doi.org/10.1007/s00521-022-07856-4

Dos Santos, M. D., & Bicalho, K. V. (2017). Proposals of SPT-CPT and DPL-CPT correlations for sandy soils in Brazil. Journal of Rock Mechanics and Geotechnical Engineering, 9(6), 1152-1158. https://doi.org/10.1016/j.jrmge.2017.08.001

Fernando, H., Nugroho, S. A., Suryanita, R., & Kikumoto, M. (2021). Prediction of SPT value based on CPT data and soil physical properties using ANN with and without data normalization. International Journal of Artificial Intelligence Research, 5(2), 123-131. https://doi.org/10.29099/ijair.v5i2.192

Ghali, M., Chekired, M., & Karray, M. (2018). Framework to improve the correlation of SPT-N and geotechnical parameters in sand. Acta Geotechnica, 13(6), 1445-1458. https://doi.org/10.1007/s11440-018-0745-3

Hatanaka, M., & Uchida, A. (1996). Empirical correlation between penetration resistance and internal friction angle of sandy soils. Soils and Foundations, 36(4), 1-9. https://doi.org/10.3208/sandf.36.4_1

Hettiarachchi, H., & Brown, T. (2009). Use of SPT blow counts to estimate shear strength properties of soils: Energy balance approach. Journal of Geotechnical and Geoenvironmental Engineering, 135(6), 830-834. https://doi.org/10.1061/(ASCE)GT.1943-5606.0000016

Jarushi, F., AlKaabi, S., & Cosentino, P. (2015). A new correlation between SPT and CPT for various soils. International Journal of Geological and Environmental Engineering, 9(2), 101-107.

Kalantary, F., Ardalan, H., & Nariman-Zadeh, N. (2009). An investigation on the Su-SPT-N correlation using GMDH type neural networks and genetic algorithms. Engineering Geology, 104(1-2), 144-155. https://doi.org/10.1016/j.enggeo.2008.09.006

Kara, O., & Gunduz, Z. (2010). Correlation between CPT and SPT in Adapazari, Turkey. In Proceedings of the 2nd International Symposium on Cone Penetration Testing (pp. 1-8). CPT'10.

Kennedy, C., & TrustGod, J. A. (2024). Comparison of SPT-N correlations for different geological formations in the Niger Delta region of Nigeria. Research Square. https://doi.org/10.21203/rs.3.rs-4602554/v1.

Khan, Z., Yamin, M., Attom, M., & Al Hai, N. (2022). Correlations between SPT, CPT, and Vs for reclaimed lands near Dubai. Geotechnical and Geological Engineering, 40, 4109-4120. https://doi.org/10.1007/s10706-022-02143-4

Kim, M., Okuyucu, O., Ordu, E., Ordu, S., Arslan, Ö., & Ko, J. (2022). Prediction of undrained shear strength by the GMDH-type neural network using SPT-value and soil physical properties. Materials, 15(18), 6385. https://doi.org/10.3390/ma15186385

Khodaparast, M., Rajabi, A. M., & Derakhshan, M. (2020). Development of practical correlations between cone penetration resistance and SPT values for various types of soils. Iranian Journal of Science and Technology, Transactions of Civil Engineering, 44(Suppl 1), 471-481. https://doi.org/10.1007/s40996-019-00319-2

Kulhawy, F. H., & Mayne, P. W. (1990). Manual on estimating soil properties for foundation design (Report No. EL-6800). Electric Power Research Institute.

Kumar, R., Bhargava, K., & Choudhury, D. (2016). Estimation of engineering properties of soils from field SPT using random number generation. Transactions of the Indian National Academy of Engineering, 1(2), 77-84. https://doi.org/10.1007/s41403-016-0012-6

Li, Y. (2013). Effects of particle shape and size distribution on the shear strength behavior of composite soils. Bulletin of Engineering Geology and the Environment, 72(3), 371-381.

Liao, S. S. C., & Whitman, R. V. (1986). Overburden correction factors for SPT in sand. Journal of Geotechnical Engineering, 112(3), 373-377. https://doi.org/10.1061/(ASCE)0733-9410(1986)112:3(373)

Lingwanda, M. I., Larsson, S., & Nyaoro, D. L. (2015). Correlations of SPT, CPT and DPL data for sandy soil in Tanzania. Geotechnical and Geological Engineering, 33, 1221-1233. https://doi.org/10.1007/s10706-015-9897-1

Mayne, P. W., & Kulhawy, F. H. (1982). K0-OCR relationships in soil. Journal of the Geotechnical Engineering Division, 108(6), 851-872. https://doi.org/10.1061/AJGEB6.0001306

Meyerhof, G. G. (1956). Penetration tests and bearing capacity of cohesionless soils. Journal of the Soil Mechanics and Foundation Division, ASCE, 82(SM1), 1-19. https://doi.org/10.1061/JSFEAQ.0000074

Meyerhof, G. G. (1957). Discussion on research on determining the density of sands by spoon penetration testing. In Proceedings of the 4th International Conference on Soil Mechanics and Foundation Engineering (Vol. 3, pp. 110-115). Butterworths Scientific Publications.

Mujtaba, H., Farooq, K., Sivakugan, N., & Das, B. M. (2018). Evaluation of relative density and friction angle based on SPT-N values. KSCE Journal of Civil Engineering, 22(2), 572-581. https://doi.org/10.1007/s12205-017-1899-5

Peck, R. B., Hanson, W. E., & Thornburn, T. H. (1974). Foundation engineering handbook. John Wiley & Sons.

Phoon, K. K., & Kulhawy, F. H. (1999). Characterization of geotechnical variability. Canadian Geotechnical Journal, 36(4), 612-624. https://doi.org/10.1139/t99-038

Robertson, P. K., & Campanella, R. G. (1983). Interpretation of cone penetration tests: Sands and clays. Canadian Geotechnical Journal, 20, 719-745. https://doi.org/10.1139/t83-079

Sabab, A. B., Islam, M. S., & Haque, M. N. (2022). Machine learning approach to predict the friction angle of silty sand from standard penetration test. Engineering Research Express, 4(3), 035018. https://doi.org/10.1088/2631-8695/ac7f49

Schmertmann, J. H. (1970). Static cone to compute static settlement over sand. Journal of the Soil Mechanics and Foundations Division, 96(3), 1011-1043. https://doi.org/10.1061/JSFEAQ.0001418

Schmertmann, J. H. (1975). Measurement of in-situ shear strength. In Proceedings of the ASCE Specialty Conference on In Situ Measurement of Soil Properties (Vol. 2, pp. 57-138). American Society of Civil Engineers.

Terzaghi, K., & Peck, R. B. (1967). Soil mechanics in engineering practice (2nd ed.). John Wiley & Sons.

Ullah, S., Khan, L., Hussain, B., Ullah, B., Ullah, I., Rahman, M. U., Khan, M. A., & Ahmad, I. (2025). Machine learning approach for the prediction of angle of internal friction of granular soils using SPT records. Engineering Research Express, 7(1), 015502. https://doi.org/10.1088/2631-8695/add858

Wang, J. J., Zhang, H. P., Tang, S. C., & Liang, Y. (2013). Effects of particle size distribution on shear strength of accumulation soil. Journal of Geotechnical and Geoenvironmental Engineering, 139(11), 1994-1997.

Wolff, T. F. (1989). Pile capacity prediction using parameter functions. In Predicted and observed axial behavior of piles (ASCE Geotechnical Special Publication No. 23, pp. 96-106). American Society of Civil Engineers.