Cold Plasma Technology in Poultry Meat: A Review

Abstract
 	Cold plasma (non-thermal plasma) is an emerging surface decontamination technology that produces reactive oxygen and nitrogen species (RONS), UV photons and charged particles which inactivate microorganisms at near-ambient temperatures. For poultry meat, cold plasma (including in-package atmospheric cold plasma, dielectric barrier discharge, plasma jets and plasma-activated water) has demonstrated the ability to reduce pathogenic and spoilage organisms, extend refrigerated shelf life and provide options for chemical-free processing while largely preserving sensory and nutritional quality when properly optimised (Noriega et al., 2011; Dirks et al., 2012; Moutiq et al., 2020). This review summarises mechanisms, system types, poultry-specific applications, effects on meat quality, challenges (oxidation, surface complexity, scale-up, regulation) and future directions for industrial adoption. Key knowledge gaps and research priorities for integrating cold plasma into poultry processing chains are highlighted.

Keywords: Cold plasma, Atmospheric cold plasma, Dielectric barrier discharge, Plasma-activated water, Poultry meat, chicken, Microbial decontamination, Shelf life

1. Introduction

Poultry meat is a globally consumed protein but remains a common vehicle for foodborne pathogens such as Salmonella, Campylobacter and spoilage bacteria (Noriega et al., 2011). Traditional interventions (thermal pasteurisation, chlorinated washes) have limitations  heat damages product quality and some chemical washes face regulatory/consumer resistance. Non-thermal surface technologies that achieve microbial reduction while preserving sensory quality are therefore highly attractive; among these, cold plasma has emerged as a promising approach for poultry meat processing (Dirks et al., 2012; Wang et al., 2016). This review focuses specifically on cold plasma applications for poultry and poultry products, summarising experimental evidence and practical considerations.

2. Principles and Equipment Types

Cold plasma is a partially ionised gas containing reactive species (O·, OH·, O₃, NOx), ions, electrons and UV that are generated by applying an electric field to a working gas (air, O₂, N₂, He, Ar) (Niemira, 2012; Akhtar et al., 2022). Because treatments are non-thermal, product temperature remains near ambient, limiting heat damage (Niemira, 2012). Major cold plasma delivery formats used in poultry research include:

• Dielectric barrier discharge (DBD)

Planar or roll-to-roll DBD systems generate plasma across a gap and have been used for direct treatment of chicken surfaces and in-package applications (Dirks et al., 2012; Zhang et al., 2022).
• In-package atmospheric cold plasma (IPCP)

Plasma generated within sealed trays to treat products without opening packaging (Wang et al., 2016; Moutiq et al., 2020).
• Plasma jet / remote plasma

Directed plasma plume for localized treatment, used for skin and small area treatments (Noriega et al., 2011).
• High-Voltage Atmospheric Cold Plasma (HVACP)

Used for eggs and shell surface decontamination and tested for poultry-related surfaces (Chen et al., 2014; Illera et al., 2022).
• Plasma-Activated Water (PAW)

Liquid generated by plasma treatment of water; used for dipping/washing carcasses and cuts (Kang et al., 2019; Große-Peclum et al., 2023).

Key controllable parameters are applied voltage/power, treatment time, working gas composition and exposure mode; these strongly determine microbial efficacy and any quality impacts (Dirks et al., 2012; Xiang et al., 2018).

3. Antimicrobial Efficacy on Poultry Surfaces and Products

3.1 Pathogen reductions on raw chicken

Early work demonstrated that dielectric barrier and atmospheric cold plasma can significantly reduce Campylobacter and Salmonella on chicken breasts and skin (Dirks et al., 2012). Dirks et al. reported log reductions that depended on inoculum level and exposure time, with breast muscle typically showing greater reductions than skin because skin topography shelters bacteria (Dirks et al., 2012; Noriega et al., 2011). Subsequent studies confirmed similar pathogen reductions using DBD or in-package ACP systems (Wang et al., 2016; Moutiq et al., 2020).

3.2 Spoilage organisms and shelf-life extension

In-package ACP treatments at optimized settings reduced natural microflora and extended refrigerated shelf life of chicken breast by reducing psychrotrophs and Pseudomonas spp. (Moutiq et al., 2020; Wang et al., 2016). Studies on braised chicken and ready-to-eat poultry products reported that DBD or IPCP treatments (with appropriate headspace gases) delayed spoilage and extended shelf life by days to weeks depending on initial quality and packaging (Zhang et al., 2022; Zhuang et al., 2024).

3.3 Plasma-Activated Water (PAW) for poultry

PAW dipping has been evaluated for poultry carcass washing and shows promise: Kang et al. (2019) reported ~1.05 log CFU/g reduction of Pseudomonas deceptionensis on chicken breasts after PAW immersion. PAW formulations and buffer systems (e.g., TRIS-buffered PAW) have been optimised to improve antimicrobial activity while moderating quality impacts (Große-Peclum et al., 2023; Qian et al., 2021).

3.4 Ready-to-eat poultry products and processed meat

Cold plasma has been applied to sliced cooked chicken sausages and RTE poultry slices with reported reductions in total viable counts and extended shelf life under refrigerated storage, with plasma generally outperforming UV in some trials (Zeraatpisheh et al., 2022; Zeraatpisheh et al., 2022 (PMC)). These studies indicate plasma’s utility for post-cooking surface recontamination control.

4. Effects on Poultry Meat Quality (Physicochemical & Sensory)

4.1 Colour and appearance

Colour changes depend on gas composition, power and exposure. Some studies reported minimal changes when IPCP was combined with modified atmospheres, while higher power/time caused reductions in redness (a*) and changes in lightness (L*) or yellowness (b*) (Wang et al., 2016; Zhang et al., 2022; Xiang et al., 2018).

4.2 Lipid and protein oxidation

RONS generated by CP can oxidise lipids and proteins. Many poultry studies report that properly optimised CP or PAW treatments do not cause significant increases in TBARS or protein carbonyls, but over-exposure can accelerate oxidation and cause off-flavours (Moutiq et al., 2020; Abdel-Naeem et al., 2022; Qian et al., 2021).

4.3 Texture, water-holding capacity and cooking loss

In most poultry trials with moderate CP doses, texture and WHC remain within acceptable limits; PAW has been shown in model systems to alter gel properties of chicken myofibrillar proteins beneficially in some cases (Qian et al., 2021), but immersion PAW can also change surface moisture dynamics and appearance (Kang et al., 2019).

4.4 Sensory acceptance

Consumer panels in several studies registered no major differences in taste/texture for optimised treatments (Moutiq et al., 2020; Zeraatpisheh et al., 2022). However, when treatments were too intense or PAW conditions poorly chosen, sensory scores declined (Kang et al., 2019; Große-Peclum et al., 2023).

5. Factors Influencing Efficacy on Poultry Products

Several factors influence the efficacy of cold plasma treatments on poultry products. Surface topology plays a key role, as skin with feather follicles and fat provides micro-niches that shelter bacteria, making it more difficult to decontaminate compared to smooth muscle surfaces (Noriega et al., 2011; Dirks et al., 2012). The packaging headspace is also critical, with modified atmospheres such as high CO₂ or O₂/CO₂ mixtures enhancing the antimicrobial effect of in-package cold plasma and extending shelf life synergistically (Wang et al., 2016; Zhuang et al., 2024). The choice of working gas affects outcomes as well: air plasma delivers strong antimicrobial action but generates more NOx and oxidative changes, while inert gases like argon or helium reduce oxidation but are less cost-effective (Noriega et al., 2011; Zhang et al., 2022). Treatment power and time must be carefully balanced, since higher intensities achieve greater microbial inactivation but also increase the risk of product quality deterioration (Xiang et al., 2018; Moutiq et al., 2020). Finally, product temperature and humidity during treatment influence the persistence of reactive species and their reaction pathways, with humidity shown to enhance the generation of certain RONS (Akhtar et al., 2022).

6. Combination (Hurdle) Approaches

Multiple studies demonstrate synergy when cold plasma is combined with natural antimicrobials (essential oils), MAP, mild heat, or PAW, enabling lower plasma intensity while maintaining efficacy (Zhang et al., 2022; Große-Peclum et al., 2023; Zeraatpisheh et al., 2022). For example, CP + rosemary oil slowed microbial growth better than either alone in poultry fillets (AvMJ / 2024 study).

7. Safety, Regulatory and Practical Considerations

7.1 Formation of by-products and oxidation

RONS can form oxidation products; thorough compositional and toxicological evaluations are required to ensure treated meat has no harmful residues (Akhtar et al., 2022; Jayasena et al., 2023).

7.2 Equipment, energy and cost

Industrial adoption requires scalable, energy-efficient designs and safe high-voltage handling in processing plants (Moutiq et al., 2020; Illera et al., 2022). In-package systems are attractive because they allow closed-system treatment with reduced recontamination risk.

7.3 Validation and standardisation

Regulatory bodies need validated protocols, and industry must define critical control parameters, labeling rules and QA procedures (Jayasena et al., 2023).

7.4 Consumer acceptance

Transparent communication (chemical-free, non-thermal, safety data) and sensory assurance are critical to foster acceptance (Jayasena et al., 2023; Große-Peclum et al., 2023).

8. Future Research Needs and Perspectives

Future research on cold plasma in poultry should prioritise the development of standardised treatment matrices, enabling cross-laboratory comparisons on poultry muscle, skin and whole birds to facilitate reliable meta-analyses (Dirks et al., 2012; Moutiq et al., 2020). At the same time, mechanistic studies are needed to better understand how reactive oxygen and nitrogen species (RONS) interact with meat lipids, myoglobin and proteins, thereby defining safe exposure windows and minimising quality deterioration (Xiang et al., 2018; Qian et al., 2021). Moving beyond the laboratory, scale-up pilot trials should focus on in-package ACP and automated dielectric barrier discharge tunnels for treating whole carcasses and ready-to-eat poultry products (Illera et al., 2022). Integrating cold plasma with other hurdle strategies such as modified atmosphere packaging and natural antimicrobials may allow lower treatment intensities while preserving efficacy. Equally important are toxicology and by-product monitoring studies, which can establish safety margins for any oxidative or nitrative compounds formed during processing. Finally, consumer research and regulatory dialogue will be critical to address public perceptions, inform labelling requirements and shape regulatory acceptance pathways for industrial adoption of this promising technology.

9. Conclusion

Cold plasma technology (including DBD, IPCP, plasma jets and PAW) shows significant potential for improving microbial safety and shelf life of poultry meat while preserving quality when optimised. A growing body of poultry-specific evidence demonstrates reductions of pathogens and spoilage organisms and shelf-life gains, but practical industrial adoption hinges on addressing oxidation control, surface heterogeneity (skin), equipment scaling and regulatory acceptance. With coordinated research, pilot-scale demonstrations and risk assessments, cold plasma could become a standard, chemical-free intervention in modern poultry processing.

References

- Abdel-Naeem HHS, et al. Decontamination of chicken meat using dielectric barrier discharge cold plasma technology: The effect on microbial quality, physicochemical properties, topographical structure and sensory attributes. Food Sci Hum Wellness. 2022; (article).

- Dirks BP, Dobrynin D, Fridman G, Mukhin Y, Fridman A, Quinlan JJ. Treatment of raw poultry with nonthermal dielectric barrier discharge plasma to reduce Campylobacter jejuni and Salmonella enterica. J Food Prot. 2012;75(1):22–28.

- Fröhling A, Schlüter O, et al. (Selected study on CP and meat color/quality in packaged products). Food Control. 2019; (article).

- Gavahian M, et al. Prospective applications of cold plasma for processing meat products: review and poultry examples. Trends Food Sci Technol. 2019; (article).

- Große-Peclum V, et al. Using TRIS-buffered plasma-activated water to reduce pathogenic microorganisms on poultry carcasses with evaluation of physicochemical and sensory parameters. Foods. 2023;12(5):1113.

- Illera AE, et al. High-voltage atmospheric cold plasma treatment of refrigerated chicken eggs for control of Salmonella Enteritidis contamination on egg shell. Innov Food Sci Emerg Technol. 2022; (article).

- Jayasena DD, et al. Innovative application of cold plasma technology in meat and its products: A review. Food Sci Anim Resour. 2023;43(6):1087–1110.

- Kang C, Xiang Q, et al. (see reference 5) — included as poultry-specific PAW study.

- Kang CD, Xiang QS, Zhao DB, Wang WJ, Niu LY, Bai YH. Inactivation of Pseudomonas deceptionensis CM2 on chicken breasts using plasma-activated water. J Food Sci Technol. 2019;56(11):4938–4945.

- Ma R, et al. (PAW and meat applications) Food Sci Technol. 2016; (article).

- Moutiq R, Misra NN, Mendonça A, Keener K. In-package decontamination of chicken breast using cold plasma technology: Microbial, quality and storage studies. Meat Sci. 2020;159:107942.

- Niemira BA. Cold plasma decontamination of foods. Annu Rev Food Sci Technol. 2012;3:125–142.

- Noriega E, Shama G, Laca A, Díaz M, Kong MG. Cold atmospheric gas plasma disinfection of chicken meat and chicken skin contaminated with Listeria innocua. Food Microbiol. 2011;28(7):1293–1300.

- Oliveira M, et al. Plasma-activated water: A cutting-edge technology driving food safety and quality (review, includes poultry studies). Trends Food Sci Technol. 2022; (article).

- Qian J, et al. Plasma-activated water-induced formation of compact chicken myofibrillar protein gels with intrinsically antibacterial activity. Food Hydrocoll. 2021; (article) — (PMC PMID:33631612).

- Wang J, Zhuang H, Hinton A Jr, Zhang J. Influence of in-package cold plasma treatment on microbiological shelf life and appearance of fresh chicken breast fillets. Food Microbiol. 2016;60:142–146.

- Xiang Q, Kang C, Niu L, Zhao D, Li K, Bai Y. Antibacterial activity and membrane damage mechanism of plasma-activated water against Pseudomonas deceptionensis CM2. LWT — Food Sci Technol. 2018;96:395–401.

- Xiang Q, Ma R, et al. (related PAW mechanism studies and LWT) — LWT. 2018; 96:395–401.

- Zeraatpisheh F, et al. (see reference 8) — RTE poultry sausage study.

- Zeraatpisheh F, et al. Investigation of effect of cold plasma on microbial load and physicochemical properties of ready-to-eat sliced chicken sausage during 60 days of storage. Foods. 2022;11: (PMC9525496).

- Zhang YL, et al. In-package cold plasma treatment of braised chicken: Voltage effects on microbial growth and quality. Food Sci Hum Wellness. 2022;11:845–853.

- Zhuang H, Rothrock MJ, Wang J, et al. Effects of in-package cold plasma treatment on poultry breast meat packaged in high CO₂ atmosphere. Poult Sci. 2024;103(10):104085.