# Original Research Article

Generate Wastewater in Coastal Regions of Ba Ria – Vung Tau, Southern of Vietnam

.

ABSTRACT

Bà Rịa–Vũng Tàu areas, a rapidly urbanizing and industrializing coastal region of Vietnam, faces increasing challenges from wastewater generation and pollutant discharges. This study quantified and projected pollutant loads from major sources, including domestic, industrial, port, aquaculture, agricultural, livestock, and stormwater activities, for the baseline year 2023 and future scenarios to 2030 and 2035. Results demonstrate that domestic wastewater is the largest and most consistent contributor, with volumes exceeding 91,000 m³/day in 2023 and projected to surpass 100,000 m³/day by 2030, all of which will be treated in centralized plants. Industrial wastewater volumes are expected to increase by more than 50% by 2035, though pollutant concentrations are anticipated to decline under stricter discharge standards. Agriculture and livestock are projected to decline sharply due to land use policies, while aquaculture expansion, particularly cage culture of marine fish, is expected to become a dominant nutrient source. Ports and stormwater runoff represent localized but important pollution risks. Emerging concerns, including microplastics and heavy metals, require urgent attention. The findings underscore the need for integrated wastewater governance, advanced treatment technologies, aquaculture zoning, and safe reuse strategies to balance economic growth with ecological sustainability and public health protection in coastal Vietnam.

Keywords: Wastewater management; Nutrient loading; Pollutant loads; Eutrophication; Aquaculture sustainability; Coastal water quality

1. INTRODUCTION

The Ba Ria – Vung Tau area (BRVT) in Vietnam faces pressing wastewater management challenges that threaten marine ecosystems, public health, and alignment with Sustainable Development Goals (SDGs). Rapid industrialization, urbanization, and inadequate wastewater infrastructure contribute to a growing crisis that demands an integrated understanding of its environmental and socio-economic implications. BRVT occupies a coastal region of strategic ecological and economic importance, hosting oil terminals, industrial parks, aquaculture farms, and tourism centers. However, its industrial areas are also major sources of marine pollution. Research shows that wastewater discharges from these areas deteriorate water quality, disrupt marine habitats, and undermine the livelihoods of communities dependent on fisheries and tourism [1]. Case studies from the Thi River illustrate how industrial effluents contaminate estuarine and coastal waters, leading to biodiversity loss and altered ecosystem services [2,3].

A key driver of this problem is the lack of adequate wastewater treatment facilities. Many industrial plants release untreated or partially treated wastewater directly into the environment, elevating levels of heavy metals and hazardous substances [4]. The Thi Vai estuary, for example, exhibits pronounced heavy-metal accumulation, worsened by high sedimentation rates and local hydrodynamics [5]. These findings highlight the urgent need for effective regulatory frameworks, monitoring programs, and the adoption of technological innovations in wastewater treatment.

Public health implications are equally concerning. Untreated wastewater not only harms marine life but also exposes coastal communities to health risks such as gastrointestinal and pollution-related diseases [6]. Vulnerable populations living near industrial areas often lack resources to mitigate exposure, which impedes progress toward SDG 6 - clean water and sanitation [7]. As wastewater management problems persist, the burden on healthcare systems increases, underscoring the need for collaborative interventions. These challenges also threaten Vietnam’s progress toward SDG 14, which emphasizes the conservation and sustainable use of marine resources. Persistent contamination undermines ecosystem resilience and constrains economic activities such as aquaculture, fisheries, and tourism. Strategic environmental assessments - particularly for port and industrial areas - can help identify intervention points and improve stakeholder collaboration [8].

Innovative solutions are therefore essential. Ecological practices in industrial processes, the adoption of green technologies, and locally adapted wastewater treatment systems can significantly reduce pollutant loads [9,10]. Community participation and public awareness initiatives are equally critical, enabling affected populations to advocate for better policies and adaptive management strategies in the face of climate change and pollution [11].

Continuous monitoring of water quality in critical basins, combined with scientific investigation, provides the foundation for evidence-based decision-making [12]. Addressing wastewater management challenges in BRVT thus requires a multidimensional approach that integrates marine ecosystem preservation, public health protection, and sustainable development. Lessons from this area may also inform broader strategies for other coastal regions worldwide facing similar threats [13].

2. material and methods

2.1 Materials

This article draws upon data collected from socio-economic survey campaigns conducted under the project “Study on Environmental Carrying Capacity for Building the Master Plan of Sustainable Development of Aquaculture in BRVT” (2021–2024). Additional data sources include the Statistical Yearbook [14], the Master Planning of BRVT for the Period 2021–2030, with a Vision to 2050 [15], as well as key provincial policy documents. These include Decision No. 1073/QĐ-UBND (30 March 2022) of the People’s Committee of BRVT on the “Implementation Plan of the Prime Minister’s Decision No. 1664/QĐ-TTg approving the Scheme for Marine Aquaculture Development by 2030, with a Vision to 2045 in BRVT”, and Decision No. 3678/QĐ-UBND (30 November 2022) on the “Action Plan for the Implementation of the Prime Minister’s Decision No. 985/QĐ-TTg (16 August 2021) issued the National Program for Aquaculture Development for the Period 2021–2030 in BRVT”.

2.1 Assessment and forecasting of generating waste sources

The estimation and projection of waste loads from various sources were generated and conducted following the framework illustrated in Figure 1.

Fig. 1. Flowchart for Calculation and forecasting of waste load of sources [16,17]

Generally, the total waste load (CT) is calculated according to Equation (1):

CT=CTi				(1)

Where, CTi represents the total waste load from different sources, including industrial and port activities (CTIP), domestic activities (CTDA), agriculture (CTAC) and stormwater-runoff-generated wastewater. The waste load from each source can be estimated using one of the following equations (2-4).

CTi=CTi×Vi				(2)

or 		CTi=CSPi×MSPi			(3)

or		CTi=Si×CSTi				(4)

where, Vi: Total volume of wastewater generated from production activity type i (m³/year); CTi: Average concentration of wastewater from production activity type i (kg/m³); MSPi: Total output of product type i (tons/year); CSPi: Average waste generation coefficient per unit of product type i (kg/tones); Si: Production area of type i (m²); CSTi: Average waste generation coefficient per unit area of production activity type i (kg/m²/year). Wastewater component includes Biochemical Oxygen Demand (BOD), Chemical Oxygen Demand (COD), Total Suspended Solids (TSS), Total Nitrogen (TN), and Total Phosphorus (TP).

2.2.1 Domestic wastewater load coefficient

According to the Vietnam National Standard TCXDVN 33:2006 on Water supply - Distribution system and facilities - Design standard, the water supply standard per capita is 200-270 L person-1·day-1 for medium and small cities and towns, small industrial zones, 80-150 L person-1·day-1 for towns, industrial and agricultural centers, industrial and fishing centers, rural residential places. The corresponding wastewater generation factor is assumed to be 80% of the supplied water volume. In the study area, domestic wastewater is partly treated in centralized treatment plants, achieving effluent quality standards of National Technical Regulation QCVN 14:2008/BTNMT and QCVN 40:2011/BTNMT (Table 1a). For wastewater not connected to centralized systems, preliminary treatment is carried out through household septic tanks, with treatment efficiency ranging from 30% to 70%, depending on specific conditions (Table 1b).

Table1. Pollutant concentrations in domestic wastewater

a) Treated domestic wastewater from centralized treatment systems

| No. | Index | Unit | QCVN 14:2008/BTNMT | QCVN 40:2011/BTNMT | Use value |
| --- | --- | --- | --- | --- | --- |
| 1 | BOD | mg/L | 30 | 30 | 30 |
| 2 | COD | mg/L |  | 75 | 75 |
| 3 | TSS | mg/L | 50 | 50 | 50 |
| 4 | TN | mgN/L |  | 20 | 20 |
| 5 | TP | mgP/L |  | 4 | 4 |

b) Generated factors for domestic wastewater domestic wastewater not connected to centralized treatment systems and corresponding treatment efficiencies [18].

| Index | Per capita pollutant load |  | Cumulative removal efficiency (%) |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Unit | Value | Filtration chamber | Primary sedimentation tank | Biological treatment |
| BOD | kg/person/year | 10-25 (17.5) | 0-10 | 10-30 | 50-80 |
| COD | kg/person/year | 20-35 (27.5) | 0-10 | 10-20 | 30-60 |
| TSS | kg/person/year | 20-30 (25) | 0-10 | 50-70 | 70-95 |
| TN | kg/person/year | 4 | 0-10 | 20-40 | 20-50 |
| TP | kg/person/year | 0.5 – 1.1 (0.8) | 0-10 | 10-20 | 10-30 |

2.2.2 Pollutant load coefficients of industrial and port wastewater

According to the Vietnam National Standard TCXDVN No. 33:2006, the water consumption standards for industrial production purpose are defined as follows: (1) 45 m³ ha-1·day-1 for industries such as beer, milk, canned food, food processing, paper, textile; and (2) 22 m³ ha-1·day-1 for other industries. In this study, the water supply coefficient for port activities was set at 22 m³ ha-1·day-1, whereas the average for industrial land was calculated as (22+45)/2 = 33.5 m³ ha-1·day-1. The corresponding wastewater generation coefficient is assumed to be 80% of the supplied water volume. Pollutant concentrations in port wastewater were determined from field survey data collected at representative ports within the study area. For industrial wastewater, pollutant concentrations were assumed to comply with the National Technical Regulation QCVN 40:2011/BTNMT, Column A and Column B (current conditions), and with Column A standards projected for 2030 and 2035 (Table 2).

Table 2. 	Concentrations of pollutants in industrial wastewater

| No. | Index | Unit | QCVN 40:2011/BTNMT |  |
| --- | --- | --- | --- | --- |
|  |  |  | Column B | Column A |
| 1 | BOD | mg/L | 50 | 30 |
| 2 | COD | mg/L | 150 | 75 |
| 3 | TSS | mg/L | 100 | 50 |
| 4 | TN | mg/L | 40 | 20 |
| 5 | TP | mg/L | 6 | 4 |

2.2.3 Pollutant load coefficient of agricultural wastewater

Pollutant load coefficient in crop cultivation is applied based on the average fertilizer residual coefficient, as presented in Table 3, whereas pollutant load coefficient in livestock farming was applied according to Table 4.

For aquaculture activities, including extensive and improved extensive shrimp farming, as well as cage culture in rivers, different pollutant load coefficients are applied. In improved extensive shrimp farming, commercial feed is used with a feed conversion ratio (FCR) of 0.5; therefore, no significant waste generation is assumed. Similarly, oyster cage culture (estuarine oysters and Pacific oysters) is also considered to generate negligible waste. In contrast, fish cage culture involving species such as grouper (Epinephelus spp.), cobia (Rachycentron canadum), Asian seabass (Lates calcarifer), red drum (Sciaenops ocellatus), and snubnose pompano (Trachinotus blochii) can generate measurable pollutant loads, depending on the type of feed applied. Trash fish, typically containing 11.5–16.9% protein and 0.4% phosphorus, or formulated feed, containing 35–42% protein and more than 1% phosphorus, are used in these systems. The corresponding pollutant load coefficients for cultured species are summarized in Table 5.

Table 3. 	Pollutant load coefficient of fertilizer application in agriculture

| No. | Crop category | Fertilizer utilization coefficient (kg/ha) |  | Fertilizer residual coefficient (%) |
| --- | --- | --- | --- | --- |
|  |  | Range | Mean |  |
| 1 | Food crops |  |  |  |
|  | Urea | 60 - 210 | 119 | 65% |
|  | Phosphate fertilizer | 56 - 90 | 71 | 60% |
| 2 | Annual industrial crops |  |  |  |
|  | Urea | 36 - 300 | 94 | 65% |
|  | Phosphate fertilizer | 64 - 150 | 96 | 60% |
| 3 | Perennial industrial crops |  |  |  |
|  | Urea | 28 - 300 | 120 | 65% |
|  | Phosphate fertilizer | 24 - 120 | 70 | 60% |

Source: Decision No. 18/2022/QĐ-UBND dated September 22, 2022

Table 4. 	Pollution load coefficient in livestock farming

| No | Index | Unit | Buffalo and Cow | Pig | Poultry |
| --- | --- | --- | --- | --- | --- |
| 1 | BOD | kg/Ind/year | 164 | 32.9 | 4.01 |
| 2 | COD | kg/Ind/year | 233.60 | 73.00 | 2.73 |
| 3 | TSS | kg/Ind/year | 1.204 | 73 | 6.65 |
| 4 | TN | kg/Ind/year | 43.8 | 7.3 | 2.05 |
| 5 | TP | kg/Ind/year | 11.3 | 2.3 | 0.156** |

Source: * [19], **: [20]

Table 5. 	Pollution load coefficient in cage aquaculture

| Objects | Unit | Grouper | Cobia | Seabass | Red drum | Snubnose pompano |
| --- | --- | --- | --- | --- | --- | --- |
| Seed | cm/individual | >12 | >18 | >12 | >10 | >8 |
|  | g/individual | 60-70 | 65 | 20-30 | 10 | 10 |
| Density | individual /m3 | 25 | 3 | 25 | 25 | 25 |
| Feed |  |  |  |  |  |  |
| Protein | % | >42 | Trash fish 11.5-16.9 | >35 | >35 | >35 |
| Total N | g/kg | 67.2 | 22.72 | 56 | 56 | 56 |
| Total P | g/kg | 10 | 4 | 10 | 10 | 10 |
| PCR |  | 2 | >12 | 1.5 | 1.5 | 2.3 |
| Culture period | Month | 12 | 9 | 10 | 10 | 12 |
| Survival rate | % | 60 | 70 | 70 | 70 | 70 |
| Harvest size | kg/ individual | 1.5 | 5 | 1 | 1 | 1 |
| Production | kg/m3 | >22.5 | >10.5 | >17.5 | >17.5 | >17.5 |
| Total N | g/kg | 31.08 | 32 | 31.36 | 29.952 | 68.8 |
| Total P | g/kg | 4 | 4 | 4 | 4 | 4 |
| Produced waste load |  |  |  |  |  |  |
| N load | g/m3 | 2,195.28 | 2,492.37 | 884.45 | 931.14 | 1,027.46 |
| P load | g/m3 | 340.50 | 455.95 | 185.94 | 189.88 | 328.48 |
| N load | g/kg | 97.57 | 237.37 | 50.54 | 53.21 | 58.71 |
| P load | g/kg | 15.13 | 43.42 | 10.63 | 10.85 | 18.77 |
| BOD5* | g/kg | 152.45 | 370.89 | 78.97 | 83.14 | 91.74 |

Sources: [20], for livestock and aquaculture waste, TN/BOD = 0.64 and COD/BOD = 2.3

2.2.4 Runoff pollution coefficient for rainfall-induced flows

Stormwater runoff was estimated based on the Rational method [21], as equation (5):

Q=0.001×A×C×I			(5)

where Q is the runoff discharge (m³/year), A is the catchment area (m²), C is the runoff coefficient, and I is the mean annual rainfall (mm/year).

According to the Statistical Yearbook [14], the total area of the study region, including Vũng Tàu City (15,090 ha), Bà Rịa City (9,104) and Phú Mỹ Town (33,302 ha), is 57,496 ha (equivalent to 574,960,000 m²). Based on typical runoff coefficients [22], values range from 0.20–0.52 for agricultural land, 0.10–0.30 for forest land, and 0.30–0.75 for urban land (including residential, special-use, institutional, and defense land). For the calculation, average of 0.36, 0.20, and 0.53 were selected for agricultural, forested, and urban areas, respectively. The pollutant load coefficient for stormwater runoff is presented in Table 6.

Table 6. 	Runoff pollution coefficient for rainfall-induced flows

| Parameters | BOD | COD | TSS | TN | TP |
| --- | --- | --- | --- | --- | --- |
| Mean concentration (g/m3) | 9,863 | 18,665 | 8,563 | 0,292 | 0,071 |

Source: [23]

3. results and discussion

According to the Master Planning of BRVT for the Period 2021–2030, with a Vision to 2050 [15], the major pollution load sources in the coastal zone of BRVT (including Bà Rịa City, Vũng Tàu City, and Phú Mỹ Town) are derived from socio-economic activities and include: (1) domestic wastewater; (2) industrial and seaport activities; and (3) other sources such as agriculture, aquaculture, and rainfall-induced flows.

3.1 Pollutant load from domestic wastewater

In 2023, Vũng Tàu City was classified as a Class I urban center, Bà Rịa City as Class II, and Phú Mỹ Town as Class III. According to the Statistical Yearbook [14], the total provincial population was 1,187,501, comprising 723,373 urban and 464,128 rural residents. The combined population of the three major urban centers (Vũng Tàu, Bà Rịa, and Phú Mỹ) was 668,390. Based on the provincial socio-economic development plan, by 2030 Vũng Tàu will remain a Class I urban center, whereas Bà Rịa and Phú Mỹ will be upgraded to Class II. Assuming an average natural population growth rate of approximately 1% per year, the population of the three urban areas is projected to reach 716,605 by 2030, with 619,814 in urban and 96,790 in rural areas. Under the plan, all domestic wastewater will be collected and treated in centralized municipal wastewater treatment plants. By 2035, with the same growth rate, the population is expected to increase to 753,159, with full collection and treatment of domestic wastewater to meet regulatory standards.

The municipal wastewater treatment infrastructure currently includes the Vũng Tàu Urban Wastewater Treatment Plant (capacity: 44,000 m³/day), the Long Sơn–Gò Găng Urban Wastewater Treatment Plant (9,500 m³/day), the Bà Rịa City Wastewater Treatment Plant (12,000 m³/day), and the Phú Mỹ Town Wastewater Treatment Plant (29,700 m³/day). The total design capacity of the system is therefore 95,200 m³/day.

The pollutant load from domestic wastewater in 2023 is presented in Table 7, whereas the estimated pollutant loads for the projection scenarios are summarized in Table 8 and 9. In 2023, the total domestic wastewater generated in the three urbans was estimated at 91,343 m³/day, of which 86,283 m³/day was treated in centralized systems and 5,059 m³/day treated locally via household septic tanks. By 2030, the domestic wastewater volume is projected to reach 102,892 m³/day, with complete collection and centralized treatment.

Table 7. 	Calculation and projection of domestic wastewater load in the study area

| Index | Unit | Vung Tau |  | Ba Ria |  | Phu My |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Urban | Rural | Urban | Rural | Urban | Rural |
| 2023 |  |  |  |  |  |  |  |
| Population | Person | 354,260 | 14,422 | 82,383 | 33,266 | 141,469 | 42,590 |
| Water supply standard | L/person/day | 200 | 150 | 150 | 100 | 150 | 100 |
| Domestic water supply | m3/day | 70,852 | 2,163 | 12,357 | 3,327 | 21,220 | 4,259 |
| Domestic wastewater | m3/day | 56,682 | 1,731 | 9,886 | 2,661 | 16,976 | 3,407 |
| Wastewater treatment capacity | m3/day | 53,900 |  | 12,000 |  | 29,700 |  |
| Centrally treated wastewater | m3/day | 53,900 |  | 12,000 |  | 20,383 |  |
| Household-level pretreated wastewater | m3/day | 4,512 |  | 547 |  | 0 |  |
| 2030 |  |  |  |  |  |  |  |
| Population | Person | 379,815 | 15,462 | 88,326 | 35,666 | 151,674 | 45,662 |
| Water supply standard | L/person/day | 200 | 150 | 200 | 150 | 150 | 100 |
| Domestic water supply | m3/day | 75,963 | 2,319 | 17,665 | 5,350 | 22,751 | 4.566 |
| Domestic wastewater | m3/day | 60,770 | 1,855 | 14,132 | 4,280 | 18,201 | 3,653 |
| Wastewater treatment capacity | m3/day | 62,626 |  | 18,412 |  | 21,854 |  |
| 2035 |  |  |  |  |  |  |  |
| Population | Person | 415,440 |  | 130,316 |  | 207,402 |  |
| Water supply standard | L/person/day | 200 |  | 200 |  | 200 |  |
| Domestic water supply | m3/day | 83,088 |  | 26,063 |  | 41,480 |  |
| Domestic wastewater | m3/day | 66,470 |  | 20,851 |  | 33,184 |  |
| Wastewater treatment capacity | m3/day | 66,470 |  | 20,851 |  | 33,184 |  |

Table 8: Calculation of domestic wastewater volume in 2023

| Index | Unit | Vung Tau |  | Ba Ria |  | Phu My | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Centralized treatment | Decentralized treatment | Centralized treatment | Decentralized treatment | Centralized treatment |  |
| Total wastewater volume | m3/day | 53,900 | 4,512 | 12,000 | 547 | 20,383 | 91,343 |
| BOD | Tones/year | 590 | 139 | 131 | 30 | 223 | 1,114 |
| COD | Tones/year | 1,476 | 131 | 329 | 28 | 558 | 2,521 |
| TSS | Tones/year | 984 | 477 | 219 | 103 | 372 | 2,154 |
| TN | Tones/year | 393 | 38 | 88 | 8 | 149 | 676 |
| TP | Tones/year | 79 | 4 | 18 | 1 | 30 | 131 |

Table 9.	Forecast of domestic wastewater load in the study area in 2030, with a projection to 2035

| Index | Unit | Vung Tau | Ba Ria | Phu My | Total |
| --- | --- | --- | --- | --- | --- |
| 2030 |  |  |  |  |  |
| Total wastewater volume | m3/day | 62,626 | 18,412 | 21,854 | 102,892 |
| BOD | Tones/year | 686 | 202 | 239 | 1,127 |
| COD | Tones/year | 1,714 | 504 | 598 | 2,817 |
| TSS | Tones/year | 1,143 | 336 | 399 | 1,878 |
| TN | Tones/year | 457 | 134 | 160 | 751 |
| TP | Tones/year | 91 | 27 | 32 | 150 |
| 2035 |  |  |  |  |  |
| Total wastewater volume | m3/day | 66,470 | 20,851 | 33,184 | 120,505 |
| BOD | Tones/year | 728 | 228 | 363 | 1320 |
| COD | Tones/year | 1820 | 571 | 908 | 3299 |
| TSS | Tones/year | 1213 | 381 | 606 | 2199 |
| TN | Tones/year | 485 | 152 | 242 | 880 |
| TP | Tones/year | 97 | 30 | 48 | 176 |

3.2 Pollutant load from industrial activities

As of 2023, BRVT had 13 industrial parks and 11 industrial clusters in operation [14]. According to the provincial development plan, the number of industrial parks is expected to increase to 24 and industrial clusters to 16 by 2030 [15]. Within the study area—comprising Vũng Tàu City, Bà Rịa City, and Phú Mỹ Town - there are currently nine industrial parks and six industrial clusters, covering a total area of 4,834.55 hectares, and most industrial zones and clusters are equipped with wastewater collection and treatment facilities (as National Technical Regulation QCVN 40:2011/BTNMT - Column B). By 2030, the number of industrial parks is projected to reach 14 and industrial clusters eight, with a combined industrial land area of 7,227.56 hectares; and by 2035, the total industrial land area is expected to expand further to 7,572.56 hectares (Table 10), and all wastewater are collected and treated as National Technical Regulation QCVN 40:2011/BTNMT - Column A.

Table 10. 	Land area allocated for industrial activities in the study area

| No | Area | Current industrial area (ha) | Industrial land use index to 2030 (ha) | Industrial land use index to 2035 (ha) |
| --- | --- | --- | --- | --- |
| I | Industrial Zone Area | 4,727.57 | 6,992.57 | 7,337.57 |
| 1 | Phu My | 4,727.57 | 6,142.57 | 6,487.57 |
| 2 | Vung Tau |  | 850.00 | 850.00 |
| II | Industrial cluster area | 106.98 | 234.99 | 234.99 |
| 1 | Vung Tau | 14.05 | 34.99 | 34.99 |
| 2 | Ba Ria | 62.93 | 115 | 115 |
| 3 | Phu My | 30 | 85 | 85 |
|  | Total | 4,834.55 | 7,227.56 | 7,572.56 |

The estimated pollutant loads from industrial wastewater are summarized in Tables 11. The total wastewater volume generated in 2023 was 47,321 × 10³ m³/year, which is projected to increase to 70,700 × 10³ m³/year in 2030 and 74,075 × 10³ m³/year in 2035, representing increases of 49.41% and 56.54% relative to the baseline year 2023, respectively.

Table 11. Estimated pollutant loads from industrial wastewater for 2023, 2030, and 2035

| Index | Unit | Vung Tau | Ba Ria | Phu My | Total |
| --- | --- | --- | --- | --- | --- |
| 2023 |  |  |  |  |  |
| Area | ha | 14.05 | 62.93 | 4,757.57 | 4,837.55 |
| Water supply demand | 1000 m3/year | 172 | 769 | 58.173 | 59.151 |
| Wastewater | 1000 m3/year | 137 | 616 | 46.539 | 47.321 |
| BOD | Ton/year | 6.87 | 30.78 | 2,326.93 | 2,366.05 |
| COD | Ton/year | 20.62 | 92.34 | 6,980.78 | 7,098.14 |
| TSS | Ton/year | 13.74 | 61.56 | 4,653.85 | 4,732.09 |
| TN | Ton/year | 5.50 | 24.62 | 1,861.54 | 1,892.84 |
| TP | Ton/year | 0.82 | 3.69 | 279.23 | 283.93 |
| 2030 |  |  |  |  |  |
| Area | ha | 884.99 | 115.00 | 6,227.57 | 7,227.56 |
| Water supply demand | 1000 m3/year | 10.821 | 1.406 | 76.148 | 88.375 |
| Wastewater | 1000 m3/year | 8.657 | 1.125 | 60.918 | 70.700 |
| BOD | Ton/year | 259.71 | 33.75 | 1,827.54 | 2,121.00 |
| COD | Ton/year | 649.27 | 84.37 | 4,568.86 | 5,302.50 |
| TSS | Ton/year | 432.85 | 56.25 | 3,045.90 | 3,535.00 |
| TN | Ton/year | 173.14 | 22.50 | 1,218.36 | 1,414.00 |
| TP | Ton/year | 34.63 | 4.50 | 243.67 | 282.80 |
| 2035 |  |  |  |  |  |
| Area | ha | 884.99 | 115.00 | 6,572.57 | 7,572.56 |
| Water supply demand | 1000 m3/year | 10.821 | 1.406 | 80.366 | 92.593 |
| Wastewater | 1000 m3/year | 8.657 | 1.125 | 64.293 | 74.075 |
| BOD | Ton/year | 259.71 | 33.75 | 1,827.54 | 2,121.00 |
| COD | Ton/year | 649.27 | 84.37 | 4,568.86 | 5,302.50 |
| TSS | Ton/year | 432.85 | 56.25 | 3,045.90 | 3,535.00 |
| TN | Ton/year | 173.14 | 22.50 | 1,218.36 | 1,414.00 |
| TP | Ton/year | 34.63 | 4.50 | 243.67 | 282.80 |

3.3 Pollutant loads from port activities

BRVT currently has 79 ports and inland waterway terminals, with the number projected to increase to 123 by 2030 [15]. Specifically, Vũng Tàu City currently has 16 ports, planned to expand to 29 by 2030; Phú Mỹ Town has 23 ports, projected to reach 43 by 2030; and Bà Rịa City has one port, planned to increase to four by 2030. By 2035, the scale of the port system along the Dinh and Thị Vải rivers is expected to remain largely unchanged relative to 2030.

Port operations do not generate industrial wastewater but do produce domestic wastewater. Field surveys indicate that eight operational seaports within the study area are equipped with domestic wastewater collection and treatment systems, with a combined capacity of approximately 900 m³/day. On average, each port treatment system operates at a capacity of 112.5 m³/day.

Other potential sources of pollution, including cargo handling, bilge water, and ship-cleaning wastewater, are highly dependent on cargo type and handling methods and are therefore difficult to quantify. Consequently, water quality monitoring around ports is used as the primary approach to assess the environmental impacts of port activities. Survey results (Table 12) indicate that water quality in the vicinity of seaports generally complies with the standards specified in the National Technical Regulation QCVN 10:2023/BTNMT, although risks of organic pollution persist.

Table 12: Port Environmental Quality (mg/L) in the Study Area

| Seaport | Month | BOD 5 | COD | TN | TP | As | Cd | Hg | Pb | Zn | Hydro Carbon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Con Dao pier | 6 | 9.88 | 34.48 | 14.96 | 0.95 | 0.004 | 0.001 | LOD | 0.001 | 0.009 | 0,038 |
| Con Dao pier | 6 | 14.54 | 35.76 | 19.79 | 1.42 | 0.003 | 0.001 | LOD | 0.002 | 0.012 | 0,072 |
| Cat Lo port | 6 | 48.96 | 157.92 | 22.39 | 2.61 | 0.005 | 0.001 | LOD | 0.002 | 0.010 | 0,219 |
| Cat Lo port | 6 | 31.89 | 81.76 | 13.03 | 0.60 | 0.003 | 0.001 | LOD | 0.002 | 0.007 | 0,105 |
| Ha Loc Oil and Service Port | 6 | 10.05 | 34.64 | 10.39 | 0.81 | 0.004 | LOD | LOD | 0.001 | 0.009 | 0,053 |
| Cat Lo port | 12 | 14.21 | 17.16 | 12.31 | 1.56 | 0.003 | LOD | LOD | 0.002 | 0.010 | 0,110 |
| Cat Lo port | 12 | 8.06 | 21.33 | 2.02 | 0.31 | 0.005 | LOD | LOD | 0.001 | 0.008 | 0,453 |
| Cat Lo port | 12 | 16.51 | 35.56 | 53.29 | 9.87 | 0.006 | 0.001 | LOD | 0.004 | 0.016 | 1,010 |
| Ha Loc port | 12 | 2.06 | 4.48 | 4.21 | 0.36 | 0.002 | LOD | LOD | LOD | 0.002 | 0,616 |
| Con Dao pier | 12 | 3.07 | 9.89 | 1.88 | 0.34 | 0.002 | 0.001 | LOD | 0.008 | 0.003 | 0,371 |
| Con Dao pier | 12 | 2.30 | 8.50 | 3.10 | 0.23 |  |  |  |  |  |  |
| QCVN 10:2023 |  |  |  |  |  | 0.02 | 0.005 | 0.0005 | 0.05 | 0.1 | 5.0 |
| QCVN 8:2023 (level B) |  | 6 | 15 | 1.5 | 0.3 |  |  |  |  |  |  |
| QCVN 40:2011 (Column A) |  | 30 | 75 | 20 | 4 | 0.05 | 0.05 | 0.005 | 0.1 | 3 | 5 |
| QCVN 40:2011 (Column B) |  | 50 | 150 | 40 | 6 | 0.1 | 0.1 | 0.01 | 0.5 | 3 | 10 |

Noted: LOD: Limit of detection (For Hg, LOD <0.0001 mg/L, for Cd and Pb, LOD<0.001 mg/L)

The calculated and projected domestic wastewater generated from port activities is presented in Table 13. The results indicate that the total volume of treated domestic wastewater was 1,642 × 10³ m³/year in 2023 and is projected to increase to 3,121 × 10³ m³/year by 2030. This represents an approximate 90% increase in associated pollutant loads relative to 2023.

Table 13. Estimation and projection of pollutant load from port-generated domestic wastewater

| Index | Unit | Vung Tau | Ba Ria | Phu My | Total |
| --- | --- | --- | --- | --- | --- |
| In 2023 |  |  |  |  |  |
| Number of ports | Port | 16 | 1 | 23 | 40 |
| Domestic wastewater | 1000 m3/year | 657 | 41 | 944 | 1642 |
| BOD | Ton/year | 19.71 | 1.23 | 28.33 | 49.28 |
| COD | Ton/year | 49.28 | 3.08 | 70.83 | 123.19 |
| TSS | Ton/year | 32.85 | 2.05 | 47.22 | 82.13 |
| TN | Ton/year | 13.14 | 0.82 | 18.89 | 32.85 |
| TP | Ton/year | 2.63 | 0.16 | 3.78 | 6.57 |
| In 2030, vision to 2035 |  |  |  |  |  |
| Number of ports | Port | 29 | 4 | 43 | 76 |
| Domestic wastewater | 1000 m3/year | 1191 | 164 | 1766 | 3121 |
| BOD | Ton/year | 35.72 | 4.93 | 52.97 | 93.62 |
| COD | Ton/year | 89.31 | 12.32 | 132.43 | 234.06 |
| TSS | Ton/year | 59.54 | 8.21 | 88.28 | 156.04 |
| TN | Ton/year | 23.82 | 3.29 | 35.31 | 62.42 |
| TP | Ton/year | 4.76 | 0.66 | 7.06 | 12.48 |

3.4 Pollutant loads from agriculture

3.4.1 Crop cultivation

In 2023, the total agricultural cultivation areas in Vũng Tàu City, Bà Rịa City, and Phú Mỹ Town were 159 ha, 8,244 ha, and 5,270 ha, respectively [14]. However, crop cultivation is no longer implemented in Vũng Tàu and Bà Rịa cities, whereas vegetable, ornamental plant, and flower production is maintained in Tan Hải and Châu Pha communes [15]. Urea fertilizer contains 20–21% nitrogen, whereas superphosphate contains 15–17% P₂O₅, equivalent to 3.28–3.71% phosphorus, with an average of 3.49%. Based on these values, pollutant loads from crop cultivation were calculated as shown in Table 14. TN and TP generate in 2023 were 198.32 tones N/year and 23.23 tones P/year, respectively, decreasing to 18.05 tones N/year and 2.90 tones P/year by 2030, and further to 12.64 tones N/year and 2.03 tones P/year by 2035 (Table 15).

Table 14: Calculation of pollutant loads from crop cultivation in the study area for the year 2023

| Category | Index | Unit | Vung Tau | Ba Ria | Phu My | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Food crops | Area | ha | 0 | 2,404 | 477 | 2,881 |
|  | Urea | Ton/year | 0 | 185.96 | 36.87 | 223 |
|  | Phosphate fertilizer | Ton/year | 0 | 102.41 | 20.30 | 123 |
|  | TN | Ton/year | 0 | 38.12 | 7.56 | 45.68 |
|  | TP | Ton/year | 0 | 3.57 | 0.71 | 4.28 |
| Annual industrial crops | Area | ha | 115 | 3,573 | 2,059 | 5,747 |
|  | Urea | Ton/year | 7.05 | 218.30 | 125.79 | 351 |
|  | Phosphate fertilizer | Ton/year | 6.64 | 205.80 | 118.58 | 331 |
|  | TN | Ton/year | 1.44 | 44.75 | 25.79 | 71.98 |
|  | TP | Ton/year | 0.23 | 7.18 | 4.14 | 11.55 |
| Perennial industrial crops | Area | ha | 43 | 2,267 | 2,734 | 5,044 |
|  | Urea | Ton/year | 3.37 | 176.79 | 213.28 | 393 |
|  | Phosphate fertilizer | Ton/year | 1.82 | 95.19 | 114.84 | 212 |
|  | TN | Ton/year | 0.69 | 36.24 | 43.72 | 80.65 |
|  | TP | Ton/year | 0.06 | 3.32 | 4.01 | 7.39 |
| Total | Area | ha | 159 | 8,244 | 5,270 | 13,672 |
|  | Urea | Ton/year | 10 | 581 | 376 | 967 |
|  | Phosphate fertilizer | Ton/year | 8 | 403 | 254 | 666 |
|  | TN | Ton/year | 2.14 | 119.12 | 77.07 | 198.32 |
|  | TP | Ton/year | 0.30 | 14.08 | 8.86 | 23.23 |

Table 15	Forecast of pollutant loads from agricultural crop cultivation in Phú Mỹ Town for the years 2030 and 2035

| Crop category | Index | Unit | 2030 | 2035 |
| --- | --- | --- | --- | --- |
| - Annual industrial crops | Area | ha | 1,441 | 1,009 |
|  | Urea | Ton/year | 88.05 | 61.64 |
|  | Phosphate fertilizer | Ton/year | 83.01 | 58.11 |
|  | TN | Ton/year | 18.05 | 12.64 |
|  | TP | Ton/year | 2.90 | 2.03 |

3.4.2 Livestock farming

Livestock farming in the study area includes buffaloes, cattle, and poultry. The total numbers of livestock in 2023 are presented in Table 16. Livestock production will only be maintained in Phu My Town until 2030, limited to existing farms until the end of their operating period [15]. The scale of livestock farming in this locality is expected to decrease by approximately 50 % compared with current levels, and by 2035 livestock farming will no longer be practiced. Vung Tau City and Ba Ria City are no longer permitted to conduct livestock operations. Thus, the total waste loads generated from livestock activities in the study area were estimated as shown in Tables 16 and 17. By 2035, since livestock farming will no longer be maintained, waste loads from livestock activities are expected to be negligible

Table 16.	Calculation of pollutant loads from livestock activities for 2023

| Index | Area | Quantity | BOD5 | COD | TSS | TN | TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unit |  | Ind. | Ton/year | Ton/year | Ton/year | Ton/year | Ton/year |
| Buffalo and Cow | Vung Tau | 712 | 116.77 | 166.32 | 857.25 | 31.19 | 8.05 |
|  | Ba Ria | 6,150 | 101.17 | 224.48 | 224.48 | 22.45 | 7.07 |
|  | Phu My | 12,011 | 24.05 | 16.4 | 39.94 | 12.31 | 0.94 |
| Pig | Vung Tau | 4,344 | 712.42 | 1,014.76 | 5,230.18 | 190.27 | 49.09 |
|  | Ba Ria | 18,143 | 596.9 | 1,324.44 | 1,324.44 | 132.44 | 41.73 |
|  | Phu My | 53,270 | 213.35 | 145.43 | 354.25 | 109.2 | 8.31 |
| Poultry | Vung Tau | 31 | 5.07 | 7.22 | 37.2 | 1.35 | 0.35 |
|  | Ba Ria | 181 | 5.95 | 13.21 | 13.21 | 1.32 | 0.42 |
|  | Phu My | 1,619 | 6.48 | 4.42 | 10.77 | 3.32 | 0.25 |
| Total | Vung Tau |  | 834.25 | 1,188.30 | 6,124.63 | 222.81 | 57.48 |
|  | Ba Ria |  | 704.03 | 1,562.13 | 1,562.13 | 156.21 | 49.22 |
|  | Phu My |  | 243.88 | 166.24 | 404.95 | 124.83 | 9.5 |
|  | Total |  | 1,782.16 | 2,916.67 | 8,091.70 | 503.85 | 116.2 |

Table 17. 	Forecast of pollutant loads from livestock activities in Phú Mỹ Town for 2030

| Index | Unit | Buffalo and Cow | Pig | Poultry | Total |
| --- | --- | --- | --- | --- | --- |
| Quantity | Ind. | 6,006 | 26,635 | 810 | 121.94 |
| BOD5 | Tone/year | 12.03 | 106.67 | 3.24 | 83.12 |
| COD | Tone/year | 8.20 | 72.71 | 2.21 | 202.47 |
| TSS | Tone/year | 19.97 | 177.12 | 5.38 | 62.42 |
| Total N | Tone/year | 6.16 | 54.60 | 1.66 | 4.75 |
| Total P | Tone/year | 0.47 | 4.16 | 0.13 | 121.94 |

3.4.3 Aquaculture activities

Brackish and marine aquaculture activities in rivers and coastal waters were recorded by the Fisheries Sub-Department in 2023. These activities include cage farming of marine fish and bivalve mollusk culture. The total area devoted to mollusk farming was 205 ha, distributed across the Dinh River, Cha Va River, Cua Lap River, and Con Dao District (100 ha, out of this study area). Cage farming occupied 174 ha, comprising 12,702 cages with a total rearing volume of 1,631,680 m³. Survey results indicate that cage farming is concentrated mainly in the Cha Va River. The total production of brackish and marine aquaculture was 17,848 tones, including 1,554 tones finfish (656 tones from pond culture and the remainder from cage farming in rivers and coastal waters) and 6,126 tones of other aquatic species, approximately 50% of which was produced in the Dinh and Cha Va Rivers.

According to Decision No. 1073/QĐ-UBND of the People’s Committee of BRVT dated 30 March 2022, which approves the implementation plan of the Prime Minister’s Decision No. 1664/QĐ-TTg on marine aquaculture development until 2030 with a vision to 2045, the aquaculture area in rivers and coastal waters is planned to reach 150 ha by 2025 and more than 200 ha by 2030. Specifies coastal farming zones including Rang–Cha Va–Mui Giui, the Dinh River (from Go Gang Bridge to Co May Bridge), the Co May–Cua Lap River (expansion area), the Mo Nhat River, and Phuoc Hai Town [15].

According to the Assessment of the Current Status of Marine Aquaculture and Identification of Potential Areas for Development in Vietnam (Institute of Fisheries Economics and Planning, 2024), the potential area for marine aquaculture is 1,337 ha in the Cha Va estuary, 320 ha in Ganh Rai Bay, and 260 ha in Con Dao District (out of this study area). Based on these findings, the maximum marine and coastal farming area in the Dinh and Cha Va Rivers was estimated at 50 ha in 2023, increasing to 100 ha by 2030. Decision No. 795/QĐ-UBND of the People’s Committee of BRVT dated 26 March 2021 set the approved cage culture area in the Dinh and Cha Va Rivers at 48.15 ha, consistent with the provincial plan, with expansion expected within the designated water surface by 2030.

By 2030, the projected expansion of aquaculture will primarily occur in bivalve mollusk farming, with partial conversion of existing cage culture areas. Assuming that mollusk production in the Dinh and Cha Va Rivers accounted for 50% of total mollusk production in 2023 (approximately 3,063 t) and cage fish production was 898 t (1,554 minus 656 t), mollusk output is projected to double to approximately 6,100 t by 2030, while cage fish production is expected to increase by 50% to 1,347 t. With an annual growth rate of 5%, cage fish production is projected to reach 1,719 t by 2035.

The main cultured fish species include grouper (Epinephelus spp.), cobia (Rachycentron canadum), sea bass (Lates calcarifer), American red drum (Sciaenops ocellatus), and golden pompano (Trachinotus blochii). Based on Table 5, nitrogen and phosphorus emissions from marine fish farming range from 50.92 to 239.67 g N/kg of fish and 10.67 to 43.71 g P/kg of fish, with average values of 100.21 g N/kg and 19.85 g P/kg. Accordingly, pollutant loads generated from marine fish farming were estimated as shown in Table 18.

Table 18. 	Calculation and projection of pollutant loads from marine fish aquaculture in the study area for 2023, with projections to 2030-2035

| Index | Unit | Load standards | Unit | For 2023 | For 2030 | For 2035 |
| --- | --- | --- | --- | --- | --- | --- |
| Aquaculture products |  |  | Ton | 898 | 1.347 | 1.719 |
| BOD | g/kg | 155.44 | Ton/year | 139.58 | 209.37 | 267.20 |
| COD | g/kg | 357.51 | Ton/year | 321.04 | 481.56 | 614.55 |
| TN | g/kg | 99.48 | Ton/year | 89.33 | 134.00 | 171.01 |
| TP | g/kg | 19.76 | Ton/year | 17.74 | 26.62 | 33.97 |

3.5 Pollutant load from stormwater runoff

With an average annual rainfall of 1,500 mm, the stormwater runoff volume through the study area is estimated in Table 19. The results of the calculated pollutant load generated from stormwater runoff in the study area are presented in Table 20

Table 19: Estimated stormwater runoff volumes from rainfall in the study area

| Location | Agricultural land |  | Forestry land |  | Urban land |  | Total (m3/year) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A(ha) | Q (m3/year) | A(ha) | Q (m3/year) | A(ha) | Q (m3/year) |  |
| Vung Tau | 2,289 | 12,360,600 | 2,242 | 6,726,000 | 4,745 | 37,722,750 | 56,809,350 |
| Ba Ria | 4,085 | 22,059,000 | 334 | 1,002,000 | 2,416 | 19,207,200 | 42,268,200 |
| Phu My | 11,904 | 64,281,600 | 4,791 | 14,373,000 | 12,777 | 101,577,150 | 180,231,750 |

Table 20: Estimated pollutant loads in in stormwater runoff

| Pollutant parameter | TSS | COD | BOD5 | TN | NH4 | NO3 | TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average concentration (g/m³) | 8.563 | 18.665 | 9.863 | 0.292 | 0.075 | 0.081 | 0.071 |
| Vung Tau (ton/year) | 486.46 | 1.060.35 | 560.31 | 16.59 | 4.26 | 4.60 | 4.03 |
| Ba Ria (ton/year) | 361.94 | 788.94 | 416.89 | 12.34 | 3.17 | 3.42 | 3.00 |
| Phu My (ton/year) | 1.543.32 | 3.364.03 | 1.777.63 | 52.63 | 13.52 | 14.60 | 12.80 |
| Total (ton/year) | 2.391.73 | 5.213.31 | 2.754.83 | 81.56 | 20.95 | 22.62 | 19.83 |

3.6 Discussions

The quantification and projection of pollutant loads in BRVT demonstrate the growing complexity of wastewater management in a rapidly urbanizing and industrializing coastal region. Interactions among domestic growth, industrial activities, aquaculture intensification, agricultural restructuring, and stormwater runoff generate significant pressures on aquatic ecosystems and public health, underscoring the need for integrated and adaptive management strategies.

Domestic wastewater remains the dominant source of pollution. In 2023, urban areas produced approximately 91,343 m³/day of wastewater, with 95% treated in centralized plants. By 2030, volumes are projected to exceed 100,000 m³/day, with full treatment coverage anticipated. Compared with other coastal regions where untreated discharges remain common [7], the province is progressing, though increasing volumes continue to strain existing systems.

Industrial wastewater is expanding alongside rapid industrialization. Discharges are projected to rise from 47.3 × 10³ m³/year in 2023 to over 74.1 × 10³ m³/year by 2035. Regulatory standards are expected to tighten, shifting from Column B to Column A of National Technical Regulation QCVN 40:2011/BTNMT, potentially reducing pollutant concentrations if compliance is ensured. Innovative treatment approaches - membrane bioreactors, advanced oxidation processes, and anaerobic membrane systems - offer opportunities to enhance efficiency and resource recovery [7,24]. For the seafood processing sector, central to the local economy, these technologies can reduce organic pollution whereas supporting water reuse and competitiveness in export markets [25]. However, industrial effluents containing heavy metals such as mercury and cadmium remain a serious risk, requiring stricter monitoring and adoption of cleaner production [1,26].

Agriculture and livestock are declining contributors due to structural transformation. In 2023, crop cultivation emitted about 198.3 tones N/year and 23.2 tones P/year, but by 2035 these are projected to fall to 12.6 tones N/year and 2.0 tones P/year. Livestock is being phased out in most areas, further reducing nutrient sources. Nevertheless, wastewater reuse in irrigation offers opportunities under freshwater scarcity, provided risks from pathogens, pharmaceuticals, and heavy metals are adequately managed [27,28].

Aquaculture, by contrast, is rapidly intensifying. Cage and mollusk farming produced nearly 18,000 tones in 2023, with mollusk production expected to double and cage fish output projected to rise by 50% by 2030. With emission factors averaging 100 g N/kg fish and 20 g P/kg fish, aquaculture is emerging as a dominant nutrient source. International experience warns that poorly managed aquaculture leads to eutrophication and biodiversity decline [29,30], requiring capacity-based planning and sustainable feed management.

Ports and stormwater runoff contribute localized pressures. Port wastewater remains modest (~900 m³/day in 2023) but will expand with port infrastructure growth to 2030. Stormwater runoff, calculated by the Rational method [21], carries hydrocarbons, suspended solids, and nutrients, with climate change likely to intensify impacts. Adoption of green infrastructure solutions can buffer these risks.

Further, emerging contaminants such as microplastics are increasingly detected in coastal waters, with implications for seafood safety and human health [31,32]. Addressing these requires integrated monitoring alongside conventional wastewater governance.

Therefore, BRVT faces escalating wastewater challenges across sectors. Effective governance will depend on stricter regulatory enforcement, advanced treatment technology adoption, aquaculture zoning, and the safe reuse of treated water. The province’s ability to integrate economic growth with ecological sustainability will determine its success in safeguarding marine ecosystems and public health.

4. Conclusion

This study highlights the complex dynamics of wastewater generation and pollutant loads in BVRT under conditions of rapid socio-economic transformation. By 2035, wastewater volumes from domestic and industrial activities will increase substantially, while pollutant loads from agriculture and livestock are expected to decline due to structural shifts in land use and farming systems. Conversely, the expansion of aquaculture represents an emerging and potentially dominant source of nutrient pollution, underscoring the need for spatial planning and carrying capacity assessments. Advanced wastewater treatment technologies, including membrane bioreactors, biological nutrient removal, and anaerobic membrane systems, present opportunities to enhance treatment efficiency, enable water reuse, and reduce long-term costs. However, addressing persistent contaminants such as heavy metals and microplastics requires more comprehensive approaches that integrate monitoring, regulatory enforcement, and cleaner production practices.

Sustainable wastewater management in BRVT demands a coordinated governance framework that balances environmental protection with industrial and agricultural development objectives. Aligning treatment strategies with national and provincial plans, while fostering collaboration among government, industry, and research institutions, will be critical to safeguarding water quality, public health, and marine ecosystem resilience. Such an approach will position the province as a model of sustainable coastal development in Vietnam.

References

1. Tran, C.T. & Nguyen, P.Q.P. Some Main Causes of Marine Pollution in Vietnam. European Journal of Engineering and Technology Research. 2019; 4: 170-175. Doi: 10.24018/ejeng.2019.4.3.1214.

2. Phuong, T.H., Tho, N. & Hung, N.T. Environmental Impacts of Port and Industrial Development Along the Thi Vai River. Journal of Environmental Impact Assessment. 2009; 18: 435-443.

3. Costa-Böddeker, S., Hoelzmann, P., Thuyên, L.X., Huy, H.D., Nguyen, H.A., Richter, O. & Schwalb, A. Ecological risk assessment of a coastal zone in Southern Vietnam: Spatial distribution and content of heavy metals in water and surface sediments of the Thi Vai Estuary and Can Gio Mangrove Forest. Marine Pollution Bulletin. 2017; 114: 1141-1151. Doi: https://doi.org/10.1016/j.marpolbul.2016.10.046.

4. Hung Phu, L., Hong Ngoc, P., Trong Dung, L., Hong Thu, N., Van The, H., Tuan Linh, V.T., Minh Hieu, N. & Minh Thu, P. Heavy metals assessment for sustainable management in Estuaries of Ba Ria-Vung Tau Province. Vietnam Journal of Marine Science and Technology. 2024; 24: 17-32. Doi: 10.15625/1859-3097/18976.

5. Costa-Böddeker, S., Thuyên, L.X., Hoelzmann, P., de Stigter, H.C., van Gaever, P., Huy, H.Đ.c. & Schwalb, A. The hidden threat of heavy metal pollution in high sedimentation and highly dynamic environment: Assessment of metal accumulation rates in the Thi Vai Estuary, Southern Vietnam. Environmental Pollution. 2018; 242: 348-356. Doi: https://doi.org/10.1016/j.envpol.2018.05.096.

6. Nguyen, G.T. & Truong, D.H. Risks of Surface Water Pollution in Southern Vietnam. Civil Engineering Journal. 2023; 9: 2725-2735. Doi: 10.28991/CEJ-2023-09-11-06.

7. Pham, Q.-N., Nguyen, N.-H., Ta, T.-T. & Tran, T.-L. Vietnam’s Water Resources: Current Status, Challenges, and Security Perspective. Sustainability. 2023; 15: 6441.

8. Quynh, L.X., Hieu, V.V., Hung, N.T., Lan, T.D. & Hens, L. Strategic Environmental Assessment for Port Areas: Application to the Ports of Hai Phong and Vung Tau in Vietnam. Strategic Environmental Assessment for Port Areas (Quynh, L.X., Hieu, V.V., Hung, N.T., Lan, T.D., Hens, L., Bhasin, V. & Susanne, C. eds). 2014: 80-96.

9. Dore, G., Brylski, P., Nygard, J. & Tran, P.T.T. Review and analysis of the pollution impacts from Vietnamese manufacturing sectors. World Bank. Washington, USA. 2008.

10. Meon, G. & Phuoc, N.V. BMBF-VNU-joint research project “Environmental and Water Protection Technologies of Coastal Zones in Vietnam” (EWATEC-COAST). EWATEC‐COAST: Technologies for Environmental and Water Protection of Coastal Regions in Vietnam (Band 1). Contributions to 4th International Conference for Environment and Natural Resources — ICENR 2014. Ho Chi Minh City, Vietnam, 17-18 June 2014, Cuvillier-Verlag, Gottingen. 2014: 1-20.

11. Schmidt-Thome, P., Nguyen, T.H., Pham, T.L., Jarva, J. & Nuottimäki, K. Impacts of Climate Change on the Ba Ria–Vung Tau Province. Climate Change Adaptation Measures in Vietnam: Development and Implementation. Springer International Publishing. Cham. 2015: 45-68. Doi: 10.1007/978-3-319-12346-2_4.

12. Tong, N.X., Hoa, N.K., Tram, N.T.T. & Khang, L.T.P. Water Quality Index, Heavy Metals, and Endocrine Disruptors in the Saigon River Basin: Pollution Assessment and Correlation Analysis. Environmental Quality Management. 2025; 34: e70063. Doi: https://doi.org/10.1002/tqem.70063.

13. Huynh, P., Tran, T.M.H. & Huynh, T.N.H. Environmental pollution along the coast of Binh Thuan province, caused by the discharge of wastewater into the main river basins from the mainland. IOP Conference Series: Earth and Environmental Science. 2023; 1278: 012032. Doi: 10.1088/1755-1315/1278/1/012032.

14. BRVT Statistics Office. Ba Ria - Vung Tau Statistics Office 2023. Statistics Publidhing House. Ha Noi, Vietnam. 2024.

15. UBND BRVT. General report: Master Plan of Ba Ria-Vung Tau province for the 2021-2030 period, with a vision toward 2050. People's Committee of Ba Ria - Vung Tau. 2023: 1037.

16. Thu, P.M. & Huan, N.H. Quantitative pollutant discharge into Thuy Trieu – Cam Ranh bay. Vietnam Journal of Science and Technology 2014; 52: 98-104.

17. Thu, P.M., Huan, N.H. & Bui Hong Long. Assessment of environmental capacity in Thuy Trieu – Cam Ranh waters. Vietnam Journal of Marine Science and Technology. 2013; 13: 371-381.

18. UNEP. Pollutants from land-based resources in the Mediterranean. UNEP Regional Seas Reports and Studies No. 32. 1984.

19. WHO. Assessment of sources of air, water, and land pollution : a guide to rapid source inventory techniques and their use in formulating environmental control strategies. In: Economopoulos, A.P. ed. Geneva: World Health Organization. 1993.

20. Diego-McGlone, M.L.S., Smith, S.V. & Nicolas, V.F. Stoichiometric Interpretations of C:N:P Ratios in Organic Waste Materials. Marine Pollution Bulletin. 2000; 40: 325-330. Doi: https://doi.org/10.1016/S0025-326X(99)00222-2.

21. Chow, V.T., Maidment, D.R. & Mays, L.W. Applied Hydrology. McGraw-Hill. Singapore. 1998.

22. Luo, H., Luo, L., Huang, G., Liu, P., Li, J., Hu, S., Wang, F., Xu, R. & Huang, X. Total pollution effect of urban surface runoff. Journal of Environmental Sciences. 2009; 21: 1186-1193. Doi: https://doi.org/10.1016/S1001-0742(08)62402-X.

23. Nguyễn Văn Hồng & Phan Thùy Linh. Nghiên cứu tính toán tải lượng của các chất ô nhiễm trên sông Sài Gòn. Tạp chí Khí tượng Thủy văn. 2013; 12/2013: 6-11.

24. Phat, V.V., Thi Thai Hang, T., Thuan, N.C., Dao, N.A., Van Tri, D., Le Luu, T., Thuan, T.H., Van Tuyen, N. & Quang, C.X. Treatment of seafood processing wastewater using an anaerobic membrane bioreactor (AnMBR) for water reuse: A comparison between flat sheet and hollow fiber ultrafiltration membranes. Journal of Environmental Chemical Engineering. 2025; 13: 116066. Doi: https://doi.org/10.1016/j.jece.2025.116066.

25. Vinh Thi, N., Le Hung, A., Trung Minh, D., Thang Anh, N. & Tan Thanh, L. Energy efficiency evaluation of a centralised wastewater treatment plant in an industrial zone of former Binh Duong province, Vietnam. Vietnam Journal of Science, Technology and Engineering. 2025; 10.31276/VJSTE.2024.0117,  Doi: 10.31276/VJSTE.2024.0117.

26. Bui, L.T. & Tran, D.L.T. Evaluation of the Role of Self-cleaning Capacity on Marine Environmental Carrying Capacity: A Case of Ganh Rai Bay, Vietnam. Archives of Environmental Contamination and Toxicology. 2023; 85: 212-228. Doi: 10.1007/s00244-023-00989-0.

27. Huỳnh, M.S., Hiếu, N.M. & Thụ, P.M. Evaluation of primary production and growth rate phytoplankton in estuaries of Ba Ria – Vung Tau Province. Journal of Fisheries Science and Technology. 2024; 4(2024): 116-126. Doi: https://doi.org/10.53818/jfst.04.2024.522.

28. Uyen, B.T.U., Ngan, D.K., Huy, N.N., Hai, N.T.H., Trieu, N.H. & Thuy, N.T. Application of response surface methodology for optimization of phosphate and ammonia removal from domestic wastewater by modified steel slag. Science, Engineering and Health Studies. 2024; 18: 24020009. Doi: 10.69598/sehs.18.24020009.

29. Minh-Thu, P., The, H.V., Ben, H.X., Hieu, N.M., Phu, L.H., Dung, L.T., Ngoc, P.H., Linh, V.T.T., Mien, P.T., Ha, T.T., Thang, N.T.X., Vinh, H.T. & Viet Ha, D. Eutrophication Monitoring for Sustainable Development in Nha Trang Marine Protected Area, Vietnam. Sustainability. 2025; 17: 5128.

30. Troell, M., Naylor, R.L., Metian, M., Beveridge, M., Tyedmers, P.H., Folke, C., Arrow, K.J., Barrett, S., Crépin, A.-S., Ehrlich, P.R., Gren, Å., Kautsky, N., Levin, S.A., Nyborg, K., Österblom, H., Polasky, S., Scheffer, M., Walker, B.H., Xepapadeas, T. & de Zeeuw, A. Does aquaculture add resilience to the global food system? Proceedings of the National Academy of Sciences. 2014; 111: 13257-13263. Doi: doi:10.1073/pnas.1404067111.

31. Doan, T.O., Duong, T.T., Pham, L.A., Nguyen, T.M., Pham, P.T., Nguyen, T.A.N., Hoang, T.Q., Phuong, N.N., Le, T.P.Q., Le, N.D., Cao, T.N., Le, T.T., Hoang, T.T.H., Nguyen, X.C., Hoang, V.T.K. & Gasperi, J. Microplastics in wastewater and the role of local wastewater treatment stations in controlling microplastic pollution: a case study from Vietnam. Environmental Monitoring and Assessment. 2025; 197: 420. Doi: 10.1007/s10661-025-13882-1.

32. Nhon, N.T.T., Nguyen, N.T., Hai, H.T.N., Minh, T.H. & Hien, T.T. Microplastic pollution in coastal surface seawater of Southern Vietnam. Environmental Monitoring and Assessment. 2024; 196: 1077. Doi: 10.1007/s10661-024-13243-4.