Health Risk Evaluation of Heavy Metals Via Dermal Contact (Swimming)

in Qua Iboe River, Nigeria

Abstract

This research was carried out to assess human health risk through dermal exposures to heavy metals (Cadmium, Chromium, Copper, Nickel, Vanadium, Lead and Zinc) in Qua Iboe River, Nigeria. Water samples were collected from five sampling stations in wet and dry seasons for the analysis of physicochemical parameters and heavy metals. Physicochemical parameters were determined using standard methods. Heavy metals were determined using atomic absorption spectrometer. Raw data obtained were subjected to statistical analysis using Statistical Package for Social Science. Levels of the physicochemical parameters in the water samples in both seasons were all within permissible limits for water quality standards set up by World Health Organisation among other regulatory bodies. Temperature ranged from 27.0 – 29.0 oC; pH 6.5 - 7.2; electrical conductivity, 119.13 – 133.33 µS/cm. dissolved oxygen 5.53 – 6.13 mg/L; biochemical oxygen demand, 2.04 – 2.05 mgL. Human health risk assessment of the heavy metals via dermal exposures in the river revealed little of no significant health effect as the hazard quotient values for the analysed heavy metals were all less than unity (1) (HQ <1). Hazard quotients for the heavy metals analysed are as shown, Cadmium 3.31E-4; Chromium, 4.65E-5; Nickel, 1.98E-6; Copper, 6.35E-6; Vanadium, 1.99E-6; Lead, 9.52E-5 and Tin, 1.35E-4. Although the measured values were all within permissible limits, some levels were close to maximum limits in locations of tremendous anthropogenic activities. Since hazard quotient (HQ) < 1 indicates no significant health risk. It therefore means that there is no significant human health risk of heavy metals through dermal contact. Elevated levels of some heavy at some locations, implies a likelihood of occurrence of actual risk in the future if the trend is not curbed. Hence routine monitoring of these these metals in the river is strongly advocated for sustainability of the river water for use by the present generation without compromising its use for future generations.

Keywords: Heavy metals, Qua Iboe River, dermal risk, Hazard quotient, ASS, statistical analysis

1.   Introduction

Water is one form of matter that constitutes a major component of living organisms. Essentiality of water to life cannot be overemphacised. Deficiency of water in living things has numerous adverse health implications. ‘‘Prolonged deficiency can result in extinction of some species of aquatic organisms. Evidence of the water deficiency in plants is obvious through leaves shading in dry season as a result of high rate of dehydration of soil moisture due to increase in temperature. It is worthy of note that although water is essential to life, not all kinds of water are suitable for life sustainability’’ [Akpan, 2024, Ido et al., 2023]. ‘‘According to World Health Organisation, (WHO 2019), water whether used for the purpose of drinking, food production, irrigation, domestic utility has an important impact on health. Therefore standards to maintain the quality of surface water are very important. It is in the light of this that WHO and other national and international regulatory organisations established standards for portable water human consumption. Investigations on toxicity of heavy metals to aquatic organisms revealed that physicochemical parameters have a considerable influence on the toxicity and accumulation of metals in organisms’’ [Obasi et al., 2012]. Most water quality parameters that are frequently monitored to ascertain the use to which it cpould be put include temperature, pH, dissolved oxygen (DO), biochemical oxygen demand (BOD), total dissolved solids (TDS), hardness, electrical conductivity (EC), organic matter, etc. ‘‘Water quality is usually described according to its physical, chemical and biological characteristics. Interference in ecological balance of water quality in aquatic ecosystems results from activities capable of causing severe damage to aquatic lives directly or indirectly. These activities include land clearing, land utilization, stream and river dredging, building of factories and industries, transportation of all forms, provision of electricity, drilling of all types, oil exploration and utilization and a host of other activities carried out by humans aimed at solving their social and economic problems’’ [Udosen, 2019]. ‘‘Water quality is continually threatened due to increasing inputs of the end products of these activities and this attracts greater attention of government in the control of water pollution. This is because the contaminants in air, soil or on land ultimately end up in the aquatic ecosystem through local precipitation, water run-off and leaching of rocks and solid wastes’’ [Uwah et al., 2020; Uwah et al.,2021].

‘‘Heavy metals occur naturally in the earth crust at varying concentrations (background levels). Sources of heavy metals into the environment include natural geochemical sources such as volcanic eruptions, weathering of rocks and forest fires while the anthropogenic sources include almost all activities carried by humans in their environments’’ [Wokoma, 2014].  ‘‘The anthropogenic sources of heavy metals contribute to the bulk of heavy metals into aquatic environment. Sources of heavy metals found in marine and estuarine environment are attributed majorly to industrial uses and storm water runoffs which represent significant sources of these metals’’ [Elkady et al., 2015]. ‘‘Another important source of most toxic metals in coastal waters is discharge of municipal waste water through submarine outfalls. Waste effluents from industries which contain tanning and dyeing, cement, insecticides, plastics, fertilizers, steel rolling, motor vehicles, refineries, petrochemicals and various food processing plants, enter the environment through both their main outfall and numerous discharges’’ [Nsi et al., 2020; Ubon et al., 2021]. ‘‘Tetraethyl lead which is used as an anti-knock agent in gasoline and its emission from steel plants increased the potential for elevated levels into the atmosphere over the natural sources. Some toxic metal pollutants can also reach the aquatic environment in different forms and phases and through different routes’’ [Lawson, 2011; Uwah et al., 2013]. ‘‘Scientifically, it has been established that heavy metals are non-biodegradle and persist for a longer time in the environment and although some of them are essential in trace amounts for plants and animals, some are toxic but the exact toxicity varies considerably. The toxicity also depends on chemical forms as well as the exposed area. Generally, metals produce their toxicity by forming complexes or ligands with organic compounds. These modified biological molecules lose their ability to function properly and result in ill health of the affected cells. When metals bind to these cells, they may form inactive enzymes systems and distort protein structure. The sensitivity of organisms to metal toxicity varies widely with species of plants and animals.’’ (Ahmed et al., 2010). Their accumulation in tissues and organs of the organisms and consequent transfer to humans through food chain is of great concern in pollution studies. Heavy metals levels in aquatic environments are very important in pollution studies and setting up of water quality criteria. ‘‘Researchers have reported that in many parts of Nigeria, especially rural areas where there is no availabibility of safe and portable water supply for drinking and other domestic purposes, the rural dwellers depend on rivers, streams, ponds, and lakes for drinking and bathing (swimming) as well as on aquatic animals for foods which have the tendency to bioaccumulate these metals in their tissues and organs. In their results, they stated that harzard quotient values of the heavy metals studied due to dermal absorption were all less than unity (1) indicating low risk [Uwah et al., 2020]. Results of determination of water quality index and suitability of urban river for municipal water supply in Lagos, Nigeria investitigated by [Akinteyon et al., 2011] also revealed human risk of the heavy metals studied due to dermal absorption indicated low risk as the hazard quotient values of the metals were all less than unity. Evaluation of heavy metal contamination in water, soil and around the poen landfill site Mogla Bazar in Sylhet, Bangladesh by [Alam et al., 2020] revealed that dermal absorption of the heavy metals in the river indicated low risk. Over the years, riverine communities of Qua Iboe River have embarked on sand minning, agricultural activities and have been swimming (bathing) in the river. It has been established through researches that these are some of the potential sources of exposure to heravy metals through dermal absorption. Other prevailing anthropogenic activities within the study area capable of interfering with the ecological balance of the river water quality include lumbering, road construction, fishing, transportation, automobile works, including numerous upstream and downstream industries among others. Therefore information on risk exposure to these heavy metals in the river through dermal absorption through this reseach will serve as a basis for exposure control hence this research.

2. Materials and Methods

Study Area

The study area is Qua Iboe River (QIR) which drains through Oruk Anam, Akwa Ibom State, Nigeria. It lies between the latitude of 04o28’31.0’’ and 07o10’12.4’’N and longitude of 06o65’41.2’’ and 06o52’50.5’’E.

Figure 1: Map of study area

Predominant human activities in the study area include agriculture, fishing, industrial activities, sand mining, lumbering and auto-mobil among others.

Water Samples Collection

Water samples for dissolved oxygen determination were fixed on site with 2 mL of Winkler solutions 1 and 11. The samples for trace metals analysis were collected with HNO3 pre-rinsed (1L) polyethylene containers and 5 mL of concentrated nitric acid (HNO3) to a pH < 2 was added to maintain the integrity of the samples.

Determination of Water quality Parameters:

Temperature

Temperature measurement was done in-situ using a multi parameter meter Hach Sension 156 model 51935-10-11. This was achieved by dipping the instrument to about 5 cm of the surface water for about five minutes to obtain a stabilised level before taking reading.

Determination of pH

Levels of pH in water samples were determined at the site using pH meter with model HACH SENSION 3. The instrument used was standardised with two buffer solutions of pH 4.0 (potassium hydrogen phthalate) and pH 6.85 (a mixture of potassium di-hydrogen phosphate and di-sodium hydrogen phosphate buffer). [Ido et al., 2023; Akpan 2024; Akpan et al., 2024].

Electrical Conductivity (EC)

Levels of EC were measured with conductivity Scan Meter model 1560. The Meter probe was calibrated by immersing it into a solution of known electrical conductivity and then rinsed with deionised water for each measurement. [Uwah et al., 2020; Ido et al., 2023, Udosen et al., 2019].

Dissolved Oxygen (DO) by Winkler’s Method

In this method, oxygen is usually fixed on site. ‘‘Aliquot of 100 mL was withdrawn from the treated water sample and 5 mL manganese sulphate, 5 mL alkali-iodide-azide, 1 mL of tetraoxosulphate (VI) acid and 1 mL of starch indicator were added to the solution and stirred. The solution was then titrated with 0.025 M sodium thiosulphate (Na2S2O3. 5H2O) solution until the colour changed from blue to colourless indicating end point’’ [Etuk et al., 2020; Ubon et al., 2021; Akpan 2024]. The level of DO in the sample was obtained using Equation 1.

DO (mg/L) = Volume of 0.025 M Na2S2O3.5H2O.           		                       Equation 1

Biochemical Oxygen Demand (BOD5)

In the determination of BOD5, the water samples were kept in an incubator at 20oC in the dark for five days after which the DO test was repeated and BOD5 calculated by taking the difference between the initial and final concentrations of oxygen present after incubation [El-Zeiny et al., 2018; Uwah et al., 2020; Ido et al., 2021]. DO was measured using equation 2.

BOD5 (mg/L) = DO (initial) – DO (final). 			                                 Equation 2

Where: BOD5 = Biochemical Oxygen Demand after 5 days, DO (initial) = dissolved oxygen before incubation, DO (final) = dissolved oxygen after 5 days.

Total Dissolved Solids (TDS)

Total dissolved solid was determined by first taking the weight of an empty crucible. The water sample (50 mL) was put into the weighed crucible and heated on a hot plate until the sample evaporated. The residue and the crucible were dried, cooled and reweighed [Akpan et al., 2024, ]. TDS was determined using Equation 3.

TDS	=	W2 - W1  					          	             Equation 3

W2	=	Weight of crucible and residue

W1	=	Weight of empty crucible

Chloride

Mohr’s method which employs silver nitrate (AgNO3) as titrant and potassium chromate (K2CrO4) as the end point indicator was used. The chloride ions present in the sample was precipited as white silver chloride. ‘‘Water samples (50 mL) each was put in a clean conical flask. K2CrO4 indicator (2 drops) were added and titrated with 0.02 M AgNO3 until the colour changed from yellow to brick red indicating the presence of chromate’’ [Akpan, 2024; Uwah et al., 2021; Udosen 2019].  The concentration of chloride in the sample was calculated using Equation 4.

Concentration of Cl- (mg/L)      =                     Vol.  of AgNO3  X 1000  X 35.550 cm3 of sample used                    Equation 4

Sulphate

‘‘The filtered water sample (100 mL) was measured into 250 mL conical flask. Dilute HCl was added in drops till acidic to litmus and evaporated to about 50 mL; the solution was boiled while barium chloride was added until all the sulphate was precipitated. It was digested in water bath at 40oC until the precipitate had settled. The precipitate was filtered into a preweighed ashless filter paper and washed a number of times with hot water until the filtrate was chloride free (AgNO3 test). The filter paper and sample were transferred into a preweighed crucible and was ashed in the furnace at 500oC for 1 hour. The content was transferred into the desicator for cooling and the weight of the precipitate alone was obtained by difference’’ [Ido et al., 2023; Uwah et al., 2021; Akpan et al., 2024]. The concentration of sulphate in the sample was calculated using Equation 5.

SO4 (mg/L) =   (mgBaSO4 x 411.5) (ml of sample)     					           Equation 5

Phosphate

Phosphate determination using Ultraviolet/Visible (UV/V) spectrophotometer was carried out to indicate the suitable absorbance measurement with appropriate wavelength using a 5 cm cell. The sample (10 mL) was poured into 100 mL measuring cylinder and 5 mL of ammonium molybedate added. The sample solutions were then measured into the 5 cm cell. The cell was properly cleaned to dryness before setting it into the UV/V spectrophotometer. Absorbance for each sample solution was read after 30 seconds at a phosphate wavelength of 420 nm. Phosphate level in the sample was estimated using Equation 6 [Akpan, 2024, Akpan et al., 2024]

PO43- (mg/L) = Ab x Df        				                            Equation       6

Where: Ab = PO43- absorbance from UV/V spectrophotometer, Df = dilution factor concentration, Df for PO43- = 55.56. Therefore, PO43- conc. (mg/L) = Ab x 55.56

Determination of Heavy Metals in Water

The concentrations of heavy metals were determined using UNICAM solar 969 Atomic Absorption Spectrophotometer (AAS). The water samples (100 mL each) were filtered using filter paper No.1. The filtrate was acidified with HNO3 (10 mL) and 50% HCl solution (10 mL). It was evaporated to near dryness on an electric hot plate. The solution was transferred to a 100 mL volumetric flask and made up to mark with deionised water. A blank was also prepared the same way with the omission of the sample using deionised water. The samples were aspirated into the air/acetylene flame.

Exposure Assessment of Heavy Metals in Water through Dermal absorption

=

EXPdermal      	=  	  CW x SA x KP x ABSd x ET x EF x ED x CF       BW x AT  	              	Equation 7

Where: EXPdermal   = exposure dose through dermal absorption (µg /kg/day), CW = mean concentration of trace metal in water, SA = exposed skin area (cm3),  KP = dermal permeability coefficient, ABSd = dermal absorption factor, ET = exposure time (h/day), EF = exposure frequency, ED = exposure duration over a life time, CF = conversion factor (1/cm3), BW = average body weight of the exposed person (70 kg for normal adult). AT = average time (days).

Quality Control and Assurance

The implementation of laboratory quality assurance and quality laboratory methods, including the use of standard operating procedures, calibrations with standards, analysis of reagent blanks and analysis of replicates guaranteed the quality of the analytical data.

Statistical Analysis:

The generated data were processed using Statistical Package for Social Sciences (SPSS).

3. Results and Discussion

Comparation of results of physicochemical parameters of Qua Iboe River in both wet and dry seasons with water quality standards are presented in Table 1. Results in Tables 2 and 3 show Principal Component Analysis (PCA) for physicochemical parameters in water samples during wet and dry season respectively. Results in Table 4 shows Comparison of the levels of trace metals in water for wet and dry seasons with standard permissible limits set by regulatory bodies.

Table 1: Water Quality Parameters of QIR, Oruk Anam for Wet and Dry Season    compared with Water quality standards

|  |  | Inen Nsai | Umani | Ikot Okoro | Nung Ikot | Ibesit | Mean | WHO | USPH | NSDWQ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp | Wet | 27.01 | 29.02 | 30.08 | 30.10 | 29.20 | 29.20±0.49 | - | - | - |
|  | Dry | 29.01 | 29.67 | 30.90 | 30.01 | 30.34 | 30.34±0.24 |  |  |  |
| pH | Wet | 6.50 | 6.90 | 6.80 | 6.30 | 6.40 | 6.50±0.57 | 6.9-8.5 | 6.5-8.5 | 6.5-8.5 |
|  | Dry | 7.20 | 7.13 | 7.30 | 7.40 | 7.41 | 7.20±0.06 |  |  |  |
| TDS | Wet | 70.50 | 73.50 | 96.80 | 96.88 | 97.73 | 87.08±0.02 | 500 | 500-1500 | 500 |
|  | Dry | 86.00 | 92.67 | 107.00 | 123.00 | 124.00 | 106.50±0.01 |  |  |  |
| EC | Wet | 119.13 | 110.40 | 119.63 | 121.18 | 123.36 | 118.74±2.24 | 300 | 300 | 1000 |
|  | Dry | 133.33 | 132.70 | 145.00 | 167.33 | 167.67 | 149.21±15.57 |  |  |  |
| DO | Wet | 6.13 | 6.10 | 8.90 | 10.01 | 11.50 | 8.58±0.17 | - | - | - |
|  | Dry | 5.53 | 6.00 | 7.30 | 9.50 | 10.40 | 7.74±0.26 |  |  |  |
| BOD | Wet | 2.05 | 2.60 | 3.09 | 3.10 | 4.50 | 3.06±0.01 | - | - | - |
|  | Dry | 2.04 | 2.50 | 3.02 | 3.05 | 4.01 | 2.92±0.01 |  |  |  |
| NO3- | Wet | 1.32 | 1.26 | 1.63 | 1.67 | 1.74 | 1.52±0.20 | 40-50 | - | 50 |
|  | Dry | 1.32 | 1.45 | 1.85 | 1.97 | 2.04 | 1.61±0.31 |  |  |  |
| NO2- | Wet | 0.01 | 0.02 | 0.02 | 0.02 | 0.05 | 0.02±0.14 | 3 | - | 0.2 |
|  | Dry | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02±0.01 |  |  |  |
| PO43- | Wet | 0.23 | 0.22 | 0.24 | 0.26 | 0.31 | 0.25±0.25 | 0.3 | 0.3 | - |
|  | Dry | 0.23 | 0.23 | 0.30 | 0.32 | 0.33 | 0.28±0.10 |  |  |  |
| SO42- | Wet | 5.13 | 5.01 | 5.23 | 5.26 | 5.29 | 5.18±0.12 | 200-250 | 250 | 1000 |
|  | Dry | 5.18 | 5.91 | 6.05 | 6.14 | 6.15 | 5.20±0.55 |  |  |  |
| Cl- | Wet | 19.01 | 19.43 | 21.92 | 26.50 | 35.20 | 24.4±1.04 | 200-600 | 250 | 250 |
|  | Dry | 18.20 | 18.41 | 19.50 | 25.30 | 34.60 | 23.20±5.06 |  |  |  |

Temperature

As shown in Table 1, temperature of the water samples ranged between 27.0 and 30.0oC across the sampling sites with a mean of 29.2± 0.49 for wet season and between 29.0 and 32.0oC with a mean of 30.3± 0.24 for dry season. Comparatively, levels recorded in all the sampling locations in dry season were higher than levels recorded in respective locations in wet season which could be ascribed to dilution effect. Temperature range recorded in this study in both seasons was higher than levels reported by [Uwah et al. 2021]. The temperature range is within the WHO permissible limit of 29.0 – 40.0oC. This temperature range supports the survival of aquatic organisms.

Hydrogen Ion (pH)

Hydrogen ion concentrations recorded in water samples ranged between 6.3 - 6.9 with the mean of 6.5± 0.5 and between 7.1 - 7.4 with the mean of 7.2± 0.06 in wet and dry seasons respectively.  The mean levels of pH in this study were consistent with levels reported by [Ido et al. 2023, Ubon, et al., 2020] and were within standards of 6.5 and 8.5 set up WHO, USPH and NSDWQ.

Total Dissolved Solids (TDS)

Levels of TDS in water samples ranged from 70.50 - 97.73 mg/L with mean level of 87.08± 0.02 mg/L and between 86.00 - 124.00 mg/L with a mean level of 87.08 ± 0.02 mg/L during wet and dry seasons respectively. The taste of water could be affected by the concentration of dissolved solids. In addition, it affects turbidity as well as density of water [Akpan, 2024, Etuk et al., 2020]. Levels of TDS in water samples reported in this study were below maximum permissable limit of 500 mg/L given by regulatory bodies.

Electrical Conductivity (EC)

Levels of EC in water samples ranged from 110.40 - 123.36 µS/cm with a mean level of 118.74 ± 2.24µS/cm and from 132.70 - 167.67 µS/cm with a mean level of 149.21±15.57 µS/cm for wet and dry seasons respectively. Comparatively, higher levels were recorded in dry season than wet season in all locations and could be ascribed to dilution and concentration effects in wet and dry seasons respectively. Fresh water organisms can only survive within a certain range of EC. Levels recorded across all sampling locations were lower than levels reported by [Nsi et al., 2020] but were consistent with levels reported by [Ubon et al., 2021] In general, levels recorded in this study were below maximum limits of 250µS/cm recommended for drinking water by WHO among others.

Dissolved Oxygen (DO)

Levels of DO recorded in water samples ranged between 6.10 - 11.50 mg/L with a mean level of 8.53 ± 0.17 mg/L and from 5.53 - 10.40 mg/L with a mean level of 7.74±0.26 mg/L in wet and dry seasons respectively. As expected, levels recorded in dry season were lower than levels obtained in wet season in all locations due to effect of higher temperatures. Comparatively, DO levels recorded in this study  were higher than that reported by [Ido et al. 2023] but were below a permissible limit of 13.0 – 14.0  mg/L stipulated by WHO (2011). ‘‘Without dissolved oxygen, there will be no survival of aquatic life forms in any water body. Adequate dissolved oxygen is necessary for good water quality. Excessive levels can cause harm to aquatic life such as gas bubble disease; vessels blood flow blockage resulting in death among others’’ [Akpan et al., 2024; Udosen, 2019].

Biochemical Oxygen Demand

Levels of BOD in water samples in this study ranged from 2.05 - 4.50 mg/L with mean level of 3.06 ± 0.01 mg/L and between 2.03 - 4.01 mg/L with mean level of 2.92±0.01 mg/L in wet and dry seasons respectively. Levels of BOD recorded in this study were higher than levels reported by [Akpan et al. 2024; Ubon et al., 2021] but were within standards for drinking water.

Chloride

Chloride levels in water samples ranged from 19.01 - 35.20 mg/L with mean levels of 24.41±1.04 mg/L and from 18.20 - 34.60 mg/L with mean level of 23.20±5.06 mg/L in wet and dry seasons respectively. The result revealed no significant differences in the levels for both seasons. The results obtained were higher than those recorded by [Etuk et al., 2020] but within WHO permissible limits of 250 mg/L for drinking water.

Nitrate

Nitrate levels in water samples ranged from 1.26 - 1.74 mg/L with mean levels of 1.52±0.20 mg/L and between 1.32 - 2.04 mg/L with mean levels of 1.61±0.31 mg/L for wet and dry seasons respectively. The results revealed no significant difference in the levels for both seasons. The levels of nitrate in this study were similar to that recorded by Wokoma (2014) and were below maximum permissible limit of 50 mg/L set up by WHO for drinking water.

Sulphate

Sulphate levels in water samples in this study ranged between 5.01 - 5.29 mg/L with mean levels of 5.18±0.12 mg/L and between 5.18 - 6.15 mg/L with mean levels of 5.20±0.55 mg/L in wet and dry seasons respectively. Sulphate levels obtained in all locations in both seasons were within recommended limits for drinking water and were also consistent with levels reported by [Ido et al., 2023; Ubon et al., 2021].

Phosphate

Phosphate levels in water samples ranged from 0.22 - 0.31 mg/L with mean level of 0.25±0.25 and from 0.23 - 0.33 mg/L with mean levels of 0.28±0.10 mg/L for wet and dry seasons respectively. These levels were below the WHO permissible limit of 250 mg/L and were consistent with levels reported by [Ubon et al., 2021 Akpan et al., 2024; Uwah et al., 2020].

Nitrite

Nitrite levels in water samples in this study ranged between 0.01 - 0.05 mg/L with a mean level of 0.02±0.14 mg/L from 0.01 - 0.02 mg/L with mean level of 0.02±0.01 mg/L during wet and dry seasons respectively. The results obtained were consistent to levels recorded by [Uwah et al. 2013] and are within the WHO permissible limits for drinking water standards.

[[

Table 4: Levels (mg/L) of heavy metals in surface water of Qua Iboe River for wet and dry seasons

| Heavy Metals |  | IN | UM | IO | NI | IB | WHO | USPH NSDWR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cd | Wet | 0.02 ±0.01 | 0.02±0.01 | 0.03 ±0.01 | 0.05 ±0.01 | 0.05±0.01 | 0.005 | 0.005 |
|  | Dry | 0.03 ±0.01 | 0.01 ±0.01 | 0.06 ±0.01 | 0.07±0.01 | 0.08±0.01 |  |  |
| Cr | Wet | 0.01 ±0.01 | 0.08 ±0.01 | 0.03 ±0.01 | 0.07 ±0.01 | 0.06±0.01 | 0.05 | 0.05 |
|  | Dry | 0.01 ±0.01 | 0.01 ±0.01 | 0.04 ±0.01 | 0.08 ±0.01 | 0.09±0.01 |  |  |
| Ni | Wet | 0.01 ±0.00 | 0.02± 0.01 | 0.01±0.00 | 0.02 ±0.01 | 0.0 3±0.00 | 0.1 | -- |
|  | Dry | 0.02 ±0.01 | 0.03 ±0.01 | 0.01 ±0.01 | 0.04±0.01 | 0.04±0.01 |  |  |
| Cu | Wet | 0.02±0.01 | 0.05±0.01 | 0.07 ±0.01 | 0.08±0.01 | 0.08±0.01 | 1.5 | 1.3 |
|  | Dry | 0.03±0.01 | 0.07 ±0.00 | 0.08±0.01 | 0.10 ±0.00 | 0.10±0.01 |  |  |
| V | Wet | 0.01±0.01 | 0.02±0.01 | 0.02 ±0.01 | 0.03±0.01 | 0.03±0.01 | 0.02 |  |
|  | Dry | 0.02±0.00 | 0.03±0.00 | 0.04±0.00 | 0.04 ±0.01 | 0.04±0.01 |  |  |
| Pb | Wet Dry | 0.00±0.00 | 0.00±0.00 | 0.00±0.00 | 0.01±0.00 | 0.01±0.01 | 0.05 | 0.015 |

IN = Inen Nsai, UM = Umani, IO = Ikot Okoro, NI = Nung Ikot, IB = Ibesit, STD = Standard deviation.

Table 5:  Risk Assessment (in terms of HQ) due to Hermal Exposures to Heavy Metals in

Qua Iboe River

Wet Season                                                                                  Dry Season

| TM | RfDDermal | Ibesit | Nung Ikot | Ikot Okoro | Umani | Inen Nsai | Ibesit | Nung Ikot | Ikot Okoro | Umani | Inen Nsai |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cd | 1.00E-5 | 3.31E-4 | 3.30E-4 | 2.64E-4 | 1.95E-4 | 1.32E-4 | 3.96E-4 | 3.95E-4 | 3.31E-4 | 1.98E-4 | 1.32E-4 |
| Cr | 6.00E-5 | 4.65E-5 | 4.63E-5 | 3.17E-6 | 6.61E-5 | 6.60E-6 | 5.30E-6 | 5.28E-5 | 2.63E-4 | 1.32E-6 | 1.32E-6 |
| Ni | 5.40E-4 | 1.98E-6 | 3.32E-6 | 1.32E-6 | 1.32E-6 | 6.61E-7 | 2.65E-6 | 2.62E-6 | 2.61E-6 | 1.98E-6 | 1.32E-6 |
| Cu | 1.20E-2 | 6.35E-6 | 6.34E-6 | 5.55E-6 | 3.96E-6 | 1.58E-6 | 7.94E-6 | 2.65E-6 | 5.29E-6 | 4.63E-6 | 1.98E-6 |
| V | 1.00E-2 | 1.99E-6 | 1.98E-6 | 1.33E-6 | 1.32E-6 | 6.61E-7 | 2.66E-6 | 2.65E-6 | 2.64E-6 | 1.98E-6 | 1.33E-6 |
| Pb | 5.25E-4 | 9.52E-5 | 6.60E-4 | 6.59E-6 | 6.58E-4 | 6.66E-4 | 9.78E-5 | 6.65E-5 | 6.78E-4 | 6.59E-4 | 6.68E-4 |
| Sn Hazard Index(HI) | 1.00E-5 | 1.35 E-4 6.186-4 | 1.34E-4 1.82E-3 | 1.33E-4 4.15E-4 | 6.61E-4 1.58E-3 | 11.99E-6 8.20E-4 | 1.99E-6 5.14E-4 | 1.98E-6 5.27E-4 | 1.34E-6 4.26E-3 | 1.33E-6 8.68E-4 | 6.61E-5 8.72E-4 |

HM = Heavy Metals

Table 6: Overall risks due to dermal exposures to Heavy metals in water

of Qua Iboe River via swimming for dry and wet seasons.

Locations

Ibesit          Nung Ikot    Ikot Okoro        Umani      Inen Nsai

| Hazard Indices (H1) | 1.06E-3 | 1.60E-3 | 1.18E-3 | 2.39E-3 | 1.67E-3 |
| --- | --- | --- | --- | --- | --- |

The calculated dermal absorbed doses (DADs) and estimated risks due to dermal exposures to heavy metals in surface water through swimming were presented in Tables 5 and 6 respectively. The levels of DAD were higher in Ibesit (downstream) than Inen Nsai (upstream), which could be attributed to downstream flow of the river as well as area with much anthropogenic activities along the study area as the end products and the after effects of such activities were usually washed into the surface water when it rains resulting in elevated levels of the heavy metals. DAD values in water were higher in dry season than wet season due to concentration effect of the metals in dry season. The results of this study were consistent with the results reported by [Uwah et al., 2021; Ido et al., 2023; Ubon et al., 2020]. As indicated in Tables 5 and 6, health risk of the heavy metals via dermal exposures revealed little of no significant risk as calculated HQ values were all less than 1 (i.e HQ < 1) across all the locations and seasons. The calculated HQ values (HIs) were notably observed to increase gradually in the sections of the river with increase in anthropogenic activities such as Nung Ikot. By extension, this is a reflection of the likelihood of occurrence of actual risk in the future if the trend is not curbed.

4. Conclusion

The following conclusions were drawn based on the statistical analysis of the results.The results of the physicochemical parameters obtained in water samples from the river showed varriations across all the sampling locations and between seasons. The levels of physicochemical parameters in the water samples from the river were higher in downstream than upstream as well as in dry season than wet season. A similar trend was also obtained in levels of heavy metals in the water samples across the sampling locations and between seasons. This is a reflection of the impacts of varying levels of anthropogenic activities across the sampling locations. Risk assessment of the heavy metals in the river water through dermal absorption (swimming) indicated low. It could be concluded therefore that although the levels of these parameters were all within safety limits given by WHO among other regulatory bodies, there is likelihood of occurrence of actual risk in the future if the trend is not curbed. This study has been able to establish the contamination levels, sources and the associated health risk of some heavy metals hence, contributing effectively to knowledge in the area of environmental chemistry. Data generated will serve as references to other researchers around the world.

References

- Alinnor I, Alagoa A. Trace Metals Distribution in Fish, Sediment and Water Samples

from Nkisa River Nigeria. British Journal of Applied Science and Technology. 2014;

4(20): 2901 - 2913

- Akan JC, Abdulraham FI, Ochanya AE, Askira YK. Heavy metals in sediment from River Ngida Maiduguri Metropolis, Borno State. Nigeria. Journal of Environment Chemistry and Ecotoxicology. 2010; 2(9): 131 – 140.

- Akintoye OA, Obi CN, Etim OA, Olorundam DT, Ukata SU, Harrison U. Seasonal variations of the physiccochemical Characteristics of surface Water in Etche River, Niger Delta Area of Nigeria. Journal of Environmental Science, Technology and Food Technology. 2014; 8(7): 1 - 7.

- Akpan NA, Udombeh RB, Mfon BU. Investigation of the Quality of Physicochemical

Parameters in Water Samples from Qua Iboe River, Ikot Ekpene Stretch, Akwa Ibom

State, Nigeria.  Asian Journal of Geological Research. 2024; 7(1): 31–40.

5.  Akpan NA, Udombeh RB, Ukpong MB, Iboroakam EU. Effects of Human Activities

On Trace Metals in Qua Iboe River, Ikot Ekpene Stretch, Akwa Ibom State, Nigeria.

International Journal of Science Research Archive. 2024; 11(01): 2120-2128.

6.  Akpan NA. Human Health Risk Assessment of Some Trace Metals through Fish

Consumption from Qua Iboe River, Oruk Anam, Nigeria. Asian Journal of Environment and Ecology. 2024; 23(5): 19 – 29.

- El-Zeiny M, El-Hamid H, El-Alfy M. Anthropogenic impacts on water

quality of River Nile and marine Environment, Rosetta branch using geospatial

analyses. Journal of Environmental Science. 2018; 4(7): 89 – 101.

- Etuk BA, Udiong DS, Akpakpan AE. Human Health Risk Assessment of Trace Metals

in Water from Cross River Estuary, Niger Delta, Nigeria. Asian Journal of Chemical

Sciences. 2020; 7(3):1-11

- Ido UH, Akpan NA, Udombeh RB, Udoidiong OM. Bioaccumulation of Trace Metals in Fish From Issiet River, Uruan, Nigeria. Science Journal of Analytical Chemistry. 2023; 11(4):40 – 48.

- Namrata S. Physicochemical properties of polluted water of River Ganga at Varanasi. International Journal of Energy and Environment. (2010); 1(5): 562 – 578.

- Ubon UU, Ekwere IO, Ikpe EE, Obadimu C. Assessment of physicochemical properties and water quality index of borehole water in Mkpat Enin Local Government Area, International Journal of Chemistry Studies. 2021; 5(2):49-58.

- Ubon UU, Ikpe EE, Ekwere IO, Uwanta EJ. Physicochemical and Polycyclic Aromatic

Hydrocarbons (PAHs) Analysis in Soil and sediment from Vicinty of Ikot Akpaden, Akwa Ibom State, Nigeria. Asian Journal of Biology. 2023; 19(1): 16-24.

- Udosen ED. Concepts in Environmental Chemistry. Nigeria Anikzo Global Ventures. 2015; 132p.

- Udosen ED. Enviro-chemical Pollutants: The Unwelcome yet Unavoidable Human companions, the 68th Inaugural Lecture, University of Uyo, University of press Ltd. 2019; 102p.

- Udosen ED, Offiong NO, Alade IG. Human health risk assessment of trace metals due to dietary intake of some edible fish species collected from Enyong Creek, Itu. Book of proceedings of 37th Annual International Conference, workshop and Exhibition, Akwa Ibom State. 2014; 1224-1231.

- Udosen ED, Offiong NO, Edem S, Edet JB. Distribution of trace metals in surface water and sediments of Imo River Estuary (Nigeria): Health risk assessment, seasonal and physicochemical variability, Journal of Environmental Chemistry and Ecotoxicology. 2016; 8(1): 22 – 25.

- Uwah I E, Solomon FD, Rebecca A, Etiuma P, Unyime EE. Evaluation of status of heavy metals pollution of sediments in Qua Iboe river estuary and associated creeks South – Eastern, Nigeria. Journal of Environmental Pollution. 2013; 2(4):120 – 132.

- Uwah EI, Edem EM, Udosen IE, Udosen ED, Udoidiong OM. Quantification of pollutant levels in water, sediment and winkles in Akani Obio Uruan River, Nigeria. Science Journal of Analytical Chemistry. 2020; 8(2): 43-47.

- Uwah EI, Rapheal U, Okon RU, Essien DU, Udoidiong OM. Atomic absorption spectrophotometric determination of elements in water, fish and sediment of Atabong River, Nigeria. Science of Journal of Analytical Chemistry. 2021; 9(4): 73-75.

- Wala C, Hart AI, Babatunde BB, Zabbey N. Assessment of human health risk from heavy metal loads in freshwater dam Nigeria radiate from the Nun River, Niger Delta. Journal of Environmental and Health Science. 2016; 6(9): 186 – 256.